3GPP TSG-RAN WG4 Meeting #98-e		R4-2101241

E-meeting, 25 January – 5 February, 2021

Agenda item:	7.6.2

Source:	Intel Corporation

Title:	Discussion on PDCCH-WUS requirements

Document for:	Discussion

# Introduction

In RAN4 #97-e meeting WF on power saving demodulation was approved [1].

- Test parameters for PDCCH - Reuse the test parameters for PDCCH in DRX-ON in 38.101-4 - For FR1 - For FDD: Test No. 3 in Table 5.3.2.1.1-1 and Test No. 3 in Table 5.3.3.1.1-1 - For TDD: Test No. 2 in Table 5.3.2.2.1-1 and Test No. 2 in Table 5.3.3.2.1-1 - For FR2 - For TDD: Test No. 1-2 in Table 7.3.2.2.1-1 - Applicability rule - Apply the applicability rule that UE already fulfilling PDCCH-WUS performance requirement can skip the corresponding Rel-15 test(s) - The number of search space for PDCCH-WUS - 1 search space for PDCCH-WUS

In this paper we provide view on demodulation requirements for NR power saving WI.

# Discussion

One of the open questions is payload and aggregation level for PDCCH-WUS configuration for each test. The following options are listed in agreed WF [1] for each test:

- Option 1:12 bits, AL=8

- Option 2: 36 bits, AL=16

In Figure 1 and Figure 2 we provide simulation results for FR1 FDD and TDD scenarios.

| 2Rx, PDCCH-WUS: 12 bits, AL 8 | 2Rx, PDCCH-WUS: 36 bits, AL 16 |
| --- | --- |
| 4Rx, PDCCH-WUS: 12 bits, AL 8 | 4Rx, PDCCH-WUS: 36 bits, AL 16 |
| Figure 1. FR1 FDD simulation results |  |

| 2Rx, PDCCH-WUS: 12 bits, AL 8 | 2Rx, PDCCH-WUS: 36 bits, AL 16 |
| --- | --- |
| 4Rx, PDCCH-WUS: 12 bits, AL 8 | 4Rx, PDCCH-WUS: 36 bits, AL 16 |
| Figure 2. FR1 TDD simulation results |  |

From FR1 results we can observe that both options of PSCCH-WUS configuration are suitable for definition of power saving requirement under assumption that existing Normal PDCCH requirements can be reused.

In Figure 3 we provide simulation results for FR2 scenarios for the following PDCCH-WUS configurations:

- Option 1: 12 bits, AL=8, 1 symbol CORSET duration

- Option 2: 36 bits, AL=8, 1 symbol CORSET duration

- Option 3: 36 bits, AL=16, 2 symbols CORSET duration

| PDCCH-WUS: 12 bits, AL 8, 1 symbol | PDCCH-WUS: 36 bits, AL 8, 1 symbol |
| --- | --- |
| PDCCH-WUS: 36 bits, AL 16, 2 symbols |  |
| Figure 3. FR2 simulation results |  |

From these results we can observe that all PDCCH-WUS configuration options do not impact the Normal PDCCH performance.

Taking into account, that all considered options of PDCCH-WUS configuration don’t have impact on Normal PDCCH performance. We suggest to consider configuration with higher payload (i.e. 36 bits).

Proposal 1:	Use the following PDCCH-WUS configurations for Rel-16 Power saving requirements:

- FR1 FDD and TDD: 36 bits, AL 16

- FR2: 36 bits, AL 8

# Conclusion

In this paper we provided view on demodulation requirements for power saving and made the following proposals:

Proposal 1:	Use the following PDCCH-WUS configurations for Rel-16 Power saving requirements:

- FR1 FDD and TDD: 36 bits, AL 16

- FR2: 36 bits, AL 8

# References

- R4-2017677 “WF on power saving demodulation”, CATT, RAN4 #97-e, November 2020.