3GPP TSG-RAN WG4 Meeting #98-e		R4-2101250

E-meeting, 25 January – 5 February, 2021

Agenda item:	7.10.1.1

Source:	Intel Corporation

Title:	Discussion on FR2 DL 256QAM UE demodulation requirements

Document for:	Discussion

# Introduction

In the previous meeting WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM was agreed [1].

- Propagation condition - Introduce test case with TDLD30-75 based on the assumption that we can complete the work for introducing TDL-D channel model into specification in RAN4#98e. If no conclusion for introducing TDL-D channel model in RAN4#98e, then RAN4 will adopt TDLA30-300 instead of TDLD30-75. - In RAN4#98e, companies provide simulation results (including 70%TP ideal and impairment SNR points and TP curves) in the template provided by Intel. - Specification of TDLD30 channel model into TS38.101-4 - Option 1: derived using only Steps 1-5 from methodology in Section B.2.1 of TS 38.101-4 for Rayleigh components Tap #; Delay; Power in [dB]; Fading distribution. 1; 0; -0.2; LOS path. 0; -12.4; Rayleigh. 2; 20; -21; Rayleigh. 3; 40; -16.7; Rayleigh. 4; 55; -18.3; Rayleigh. 5; 80; -21.9; Rayleigh. 6; 120; -27.8; Rayleigh. 7; 240; -23.6; Rayleigh. 8; 285; -24.8; Rayleigh. 9; 290; -30.0; Rayleigh. 10; 375; -27.7; Rayleigh. - Option 2: Tap #; Delay; Power in [dB]; Fading distribution. 1; 0; -0.2; LOS path. 0; -13.5; Rayleigh. 2; 2; -18.8; Rayleigh. 3; 18; -21; Rayleigh. 4; 40; -22.8; Rayleigh. 5; 42; -17.9; Rayleigh. 6; 54; -18.3; Rayleigh. 7; 78; -21.9; Rayleigh. 8; 122; -27.8; Rayleigh. 9; 238; -23.6; Rayleigh. 10; 282; -24.8; Rayleigh. 11; 292; -30; Rayleigh. 12; 376; -27.7; Rayleigh. - Companies are encouraged to check if the above option 1 is agreeable and provide feedback in RAN4 draft reflector during 16th - 27th Nov after the meeting.

In this paper we provide our view on UE demodulation requirements and simulation results for FR2 256QAM.

# Discussion

One of the open questions from the previous meeting is definition of simplified TDL-D channel model.

In TS 38.101-4 Section B.2.1 procedure for generation of simplified channel model is defined. This procedure was used for generation of NLOS channel model. Therefore, first we need to check whether all steps defined in this section are applicable for generation of simplified LOS channel model.

Based on our understanding, Steps 1-8 can be applied either to NLOS or LOS type of channels. And for LOS channel, all simplification steps can be applied only to taps with Rayleigh distribution. As for Step 9, we think that this step is only applicable to NLOS channel model because the strongest tap for all original NLOS TDL models is 0 dB. Same time, for original LOS channels all taps have non-zero power. Therefore, step with re-normalization of taps is not required for LOS channels. In tables below we provide the information about output of each step.

|  | Step 1 |  | Step 2 |  | Step 3 |  | Step 4 |  | Step 5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tap # | Normalized delay | Power | Normalized delay | Power | Delay | Power | Delay | Power | Delay | Power |
| 1 | 0 | -0.2 | 0 | -0.2 | 0 | -0.2 | 0 | -0.2 | 0 | -0.2 |
|  | 0 | -13.5 | 0 | -13.5 | 0 | -13.5 | 0 | -13.5 | 0 | -12.377 |
| 2 | 0.035 | -18.8 | 0.035 | -18.8 | 1.05 | -18.8 | 0 | -18.8 | 20 | -21 |
| 3 | 0.612 | -21 | 0.612 | -21 | 18.36 | -21 | 20 | -21 | 40 | -16.683 |
| 4 | 1.363 | -22.8 | 1.363 | -22.8 | 40.89 | -22.8 | 40 | -22.8 | 55 | -18.268 |
| 5 | 1.405 | -17.9 | 1.405 | -17.9 | 42.15 | -17.9 | 40 | -17.9 | 80 | -21.9 |
| 6 | 1.804 | -20.1 | 1.775 | -22.9 | 53.25 | -22.9 | 55 | -22.9 | 120 | -27.8 |
| 7 | 2.596 | -21.9 | 1.804 | -20.1 | 54.12 | -20.1 | 55 | -20.1 | 240 | -23.6 |
| 8 | 1.775 | -22.9 | 2.596 | -21.9 | 77.88 | -21.9 | 80 | -21.9 | 285 | -24.8 |
| 9 | 4.042 | -27.8 | 4.042 | -27.8 | 121.26 | -27.8 | 120 | -27.8 | 290 | -30.0 |
| 10 | 7.937 | -23.6 | 7.937 | -23.6 | 238.11 | -23.6 | 240 | -23.6 | 375 | -27.7 |
| 11 | 9.424 | -24.8 | 9.424 | -24.8 | 282.72 | -24.8 | 285 | -24.8 |  |  |
| 12 | 9.708 | -30.0 | 9.708 | -30.0 | 291.24 | -30.0 | 290 | -30.0 |  |  |
| 13 | 12.525 | -27.7 | 12.525 | -27.7 | 375.75 | -27.7 | 375 | -27.7 |  |  |

|  | Step 6 |  | Step 7 |  | Step 8 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tap # | Delay | Power | Delay | Power | Delay | Power |
| 1 | 0 | -0.2 | 0 | -0.2 | 0 | -0.2 |
|  | 0 | -12.377 | 0 | -12.4 | 0 | -12.4 |
| 2 | 20 | -21 | 20 | -21 | 20 | -21 |
| 3 | 40 | -16.683 | 40 | -16.7 | 40 | -16.7 |
| 4 | 55 | -18.268 | 55 | -18.3 | 55 | -18.3 |
| 5 | 80 | -21.9 | 80 | -21.9 | 80 | -21.9 |
| 6 | 120 | -27.8 | 120 | -27.8 | 120 | -27.8 |
| 7 | 240 | -23.6 | 240 | -23.6 | 240 | -23.6 |
| 8 | 285 | -24.8 | 285 | -24.8 | 285 | -24.8 |
| 9 | 290 | -30.0 | 290 | -30.0 | 290 | -30.0 |
| 10 | 375 | -27.7 | 375 | -27.7 | 375 | -27.6 |
| 11 |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |

As the next step, we analyse the performance difference between original and simplified channel models. In Figure 1 we provide the simulation results. In Table 1 we provide the list of parameters used for link level analysis.

Table 1. Link level simulation assumptions.

| Parameter | Value |
| --- | --- |
| CBW | 50 MHz |
| SCS | 120kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-D 30ns delay spread, 35Hz Doppler frequency (Original and Simplified) |
| MCS | MCS 20 in TS 38.214 Table 5.1.3.1-2 |
| Antenna configuration | 2x2 for Rank1 and Rank2, Low correlation |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | KPT-RS: 2 (every 2 RBs), LPT-RS: 1 (every 1 symbol) |
| Impairments | TX: EVM = 3% RX: No (Ideal) |

Figure 1. FR2 256QAM performance analysis.

In Table 2 summary of link level simulation results is presented.

Table 2. PDSCH performance summary

| Channel model | SNR for 70% of max T-put |  |
| --- | --- | --- |
|  | Alignment results | Impairment results |
| TDL-D Original | 17.6 | 20.1 |
| TDL-D Simplified | 17.6 | 20.1 |

From these results we can observe that performance for Original and Simplified channel models is same. Therefore, we can use this simplified channel model, for FR2 256QAM requirements definition.

Proposal 1:	Use the following channel model for FR2 256QAM requirements:

| Tap # | Delay | Power | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -12.4 | Rayleigh |
| 2 | 20 | -21 | Rayleigh |
| 3 | 40 | -16.7 | Rayleigh |
| 4 | 55 | -18.3 | Rayleigh |
| 5 | 80 | -21.9 | Rayleigh |
| 6 | 120 | -27.8 | Rayleigh |
| 7 | 240 | -23.6 | Rayleigh |
| 8 | 285 | -24.8 | Rayleigh |
| 9 | 290 | -30.0 | Rayleigh |
| 10 | 375 | -27.6 | Rayleigh |

# Conclusion

In this paper we provided view on FR2 256QAM demodulation requirements definition and made the following proposals:

Proposal 1:	Use the following channel model for FR2 256QAM requirements:

| Tap # | Delay | Power | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -12.4 | Rayleigh |
| 2 | 20 | -21 | Rayleigh |
| 3 | 40 | -16.7 | Rayleigh |
| 4 | 55 | -18.3 | Rayleigh |
| 5 | 80 | -21.9 | Rayleigh |
| 6 | 120 | -27.8 | Rayleigh |
| 7 | 240 | -23.6 | Rayleigh |
| 8 | 285 | -24.8 | Rayleigh |
| 9 | 290 | -30.0 | Rayleigh |
| 10 | 375 | -27.6 | Rayleigh |

# References

- R4-2017536 “WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM”, China Telecom, RAN4 #97-e, November 2020.