3GPP TSG-RAN WG4 Meeting #98e		R4-2101257

Electronic, 25th January – 5th February, 2021

Agenda item:	7.9.4.2.2

Source:	Intel Corporation

Title:	Simulation results for multi-DCI based multi-TRP Tx scheme

Document for:	Discussion

# Introduction

In RAN4 #97e meeting WF on PDSCH demodulation requirements and general aspects of NR eMIMO Rel-16 was agreed[1]. All test parameters for multi-DCI based Tx scheme were finalized. In this paper we provide simulation results for corresponding scenario.

# Discussion

## Simulation results

In Figure 1 simulation results for agreed multi-DCI based multi-TRP Tx scenarios are provided for FDD and TDD duplex modes. The simulation assumptions can be found in [2]. The summary of demodulation performance at 70% at max achievable throughput is provided in Table 1 for ideal and impairment results as well.

| Figure 1. Demodulation performance of multi-DCI based Multi-TRP non-overlapping Tx scheme |  |
| --- | --- |

Table 1. Demodulation performance at 70% @max achievable throughput

| Duplex mode | Antenna configuration | Ideal results, dB | Impairment results, dB |
| --- | --- | --- | --- |
| FDD | 2x2 | 17.1 | 19.6 |
|  | 2x4 | 11 | 13.5 |
| TDD | 2x2 | 17.1 | 19.6 |
|  | 2x4 | 11 | 13.5 |

# Conclusion

In this paper we have provided simulation results to define performance requirements for multi-DCI based multi-TRP Tx scheme.

# References

- R4-2017529 “WF on NR eMIMO PDSCH requirement”, Intel, RAN4 #97-e, November 2020

- R4-2017530 “Simulation assumption for PDSCH requirement with single-DCI and Multi-DCI transmission schemes”, Ericsson, Intel, RAN4 #97-e, November 2020