3GPP TSG-RAN WG4 Meeting #98-e	     R4-2101330

# Electronic Meeting, Jan. 25th – Feb. 5th, 2021

Title: 	Simulation results on URLLC UE demodulation requirements with higher BLER

Source: 	Huawei, HiSilicon

Agenda item:	7.8.1.2.1

Document for:	Discussion

Background

In this paper, the simulation results for URLLC UE demodulation requirements are presented based on the agreed parameters on previous e-meetings.

High reliability for FR2

FR2 PDSCH high reliability with higher BLER

Simulation assumption

For FR2 PDSCH high reliability with higher BLER, the performance was simulated with proposed parameters.

Table 2-1: Simulation assumption for FR2 PDSCH performance requirements

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Frequency range |  |  | FR2 |
| Duplex mode |  |  | TDD |
| TDD pattern |  |  | DDSU (S=13:1:0) |
| Bandwidth/SCS |  |  | 100 MHz/ 120 kHz |
| Antenna configuration |  |  | 2x2, ULA low |
| Channel model |  |  | TDLA30-75 |
| PDSCH configuration | Mapping type |  | Type A |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
|  | PDSCH aggregation factor |  | 2 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 2 |
| Maximum number of HARQ transmissions |  |  | 4 |
| MCS Table |  |  | Table 3, MCS13/16/19 |
| Test metric |  |  | 1% BLER |

Simulation results:

FR2 PDSCH performance for TDD was simulated.

Figure 2-1: PDSCH performance for FR2 PDSCH repetition Type A

The SNR values at 1% BLER are shown in table below:

| Antenna configuration | 2x2 |  |
| --- | --- | --- |
| MCS | Real SNR (dB) | Implemented SNR (dB) |
| 13 | -6.9 | -5.4 |
| 16 | -3.8 | -2.3 |
| 19 | -1.9 | -0.4 |

Low latency

PDSCH mapping Type B for FR2

Simulation assumption

The test parameters according to the discussion of previous meetings are concluded in table below:

Table 3-1: Test parameters for FR2 PDSCH performance requirements

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR2 |
| Duplex mode |  | TDD |
| Antenna configuration |  | 2x2, ULA low |
| PDSCH configuration | Mapping type | Type B |
|  | Starting symbol (S) | 1 |
|  | Length (L) | 2 |
|  | PDSCH aggregation factor | 1 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 0 |
| Maximum number of HARQ transmission |  | 1 |
| Number of HARQ process |  | 4 |
| Propagation condition |  | TDLA30-75 |
| MCS Table |  | Table 1, [MCS 4] |
| SCS and BW |  | 120KHz/100MHz |
| TDD Pattern |  | DDDSU, S=10:2:2 |
| Frequency domain resource |  | Full BW |
| Testing metric |  | Throughput: 70% |

Simulation results:

PDSCH mapping Type B performance for FR2 was simulated based on the parameter assumption in Table 3-1.

Figure 3-1: FR2 PDSCH performance for Mapping Type B

The SNR value at 70% maximum TP is -2.5dB:

| Antenna configuration | 2x2 |  |
| --- | --- | --- |
| Duplex mode | Real SNR (dB) | Implemented SNR (dB) |
| TDD | -2.5 | -1 |

Pre-emption indication:

Simulation assumption

The agreed parameters are concluded in table below.

Table 3-2: Simulation assumption for UE PDSCH pre-emption performance requirements

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD, TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | FDD: 4 TDD: 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information and TDD pattern |  |  | TBD |
| CBW/SCS |  |  | FDD: 10/15 TDD: 40/30 |
| RB allocation |  |  | Full bandwidth |
| MIMO layer |  |  | Rank 1 |
| Propagation condition |  |  | TDLA30-10 |
| Test metric |  |  | 70% TP |

Simulation results

According to the agreed parameters for URLLC and parameters in table above for eMBB, simulations were setup for FDD with Rx configuration of 2x2 for MCS13 and MCS16. The results are shown in figure below, simulations include 1). Throughput vs SNR for eMBB UE without pre-emption. 2). Throughput vs SNR for eMBB UE with pre-emption of 20% probability with pre-emption and without buffer flush. 3). Throughput vs SNR for eMBB UE with pre-emption of 20% probability with pre-emption and buffer flushing.

Figure 3-2: PDSCH performance for pre-emption with 20% probability for MCS13 and MCS16 from Table 1 (FDD 2x2)

The SNR values of these setup for 70% of maximum throughput are shown in table below:

| Antenna configuration | 2x2 |  |
| --- | --- | --- |
| MCS from Table 1 | MCS13 | MCS16 |
| SNR difference between with and without buffer flush | <0.5 dB | 2dB |

Reference

- R4-2017509, “Way forward for NR UE URLLC performance requirements”, Intel Corporation.