3GPP TSG-RAN WG4 Meeting #98-e    	R4-2101332

# Electronic Meeting, Jan. 25th – Feb. 5th, 2021

Title: 	Simulation results on URLLC FR2 PUSCH demodulation requirements with higher BLER

Source: 	Huawei, HiSilicon

Agenda item:	7.8.1.2.2

Document for:	Discussion

Background

In this paper, URLLC BS FR2 PUSCH demodulation requirements for high reliability and low latency are provided based on the agreed parameters.

FR2 PUSCH high reliability with higher BLER

Simulation assumption

The agreed simulation assumption are shown in table below:

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| SCS/BW |  | Both of 60 kHz and 120 KHz for both of 50 MHz and 100 MHz |
| Uplink-downlink allocation for TDD (Note 1) |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| Antenna configuration |  | 1x2, Low |
| Number of layers |  | 1 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0,3,0,3 (Note 2) |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Additional DM-RS position | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0}, {0, 1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | B |
|  | Starting symbol | 0 |
|  | Allocation length | 10 |
|  | PUSCH aggregation factor | n8 (Note 3) |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hoping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Propagation condition and correlation matrix |  | TDLA30-300 Low |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| Testing metric |  | Target BLER: 10-2 (Calculate the target BLER after all transmission) |
| Note 1: The same requirements are applicable to TDD with different UL-DL patterns and different aggregation factor configurations under assumption that two effective transmissions of the transport block are generated. Note 2: The effective RV sequence is {0,2,3,1} with slot aggregation Note 3: The intention of this configuration is to have two effective transmissions of the transport block. To achieve this for the standard TDD pattern captured in this table, a value of n8 is necessary. |  |  |

Simulation results

The simulation results of 60 kHz and 120 kHz for both of 50 MHz and 100 MHz are shown in below:

As the test metrics is 1% BLER, the real SNR and implemented SNR are listed in table below:

| Bandwidth | 50MHz |  | 100MHz |  |
| --- | --- | --- | --- | --- |
| SCS | Real SNR (dB) | Implemented SNR (dB) | Real SNR (dB) | Implemented SNR (dB) |
| 60 kHz | -14.1 | -12.1 | -15.3 | -13.3 |
| 120 kHz | -13.8 | -11.8 | -14.6 | -12.6 |

FR2 PUSCH mapping Type B

Simulation assumption

The simulation parameters were agreed during the last meeting and they are shown in table below:

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| SCS/BW |  | Both of 60 kHz and 120 KHz for both of 50 MHz and 100 MHz |
| Uplink-downlink allocation for TDD |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| Antenna configuration |  | 1x2, Low |
| Number of layers |  | 1 |
| MCS |  | MCS5 (Table 3) |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Additional DM-RS position | 0 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0}, {0, 1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | B |
|  | Starting symbol | 0 |
|  | Allocation length | 4 |
|  | PUSCH aggregation factor | 1 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hoping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Propagation condition and correlation matrix |  | TDLA30-300 Low |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| Testing metric |  | 70% maximum throughput |
| Note 1: The same requirements are applicable to TDD with different UL-DL patterns and different aggregation factor configurations under assumption that two effective transmissions of the transport block are generated. Note 2: The effective RV sequence is {0,2,3,1} with slot aggregation Note 3: The intention of this configuration is to have two effective transmissions of the transport block. To achieve this for the standard TDD pattern captured in this table, a value of n8 is necessary. |  |  |

Simulation results

The simulation results for FR2 PUSCH Mapping Type B of 60 kHz and 120 kHz for both of 50 MHz and 100 MHz are shown in figure below:

Figure 3-1: Throughput vs SNR for FR2 PUSCH mapping Type B with symbol length of 4

The real SNR and the implemented SNR values are listed in table below:

| Bandwidth | 50MHz |  | 100MHz |  |
| --- | --- | --- | --- | --- |
| SCS | Real SNR (dB) | Implemented SNR (dB) | Real SNR (dB) | Implemented SNR (dB) |
| 60 kHz | -5.9 | -3.9 | -7.6 | -5.6 |
| 120 kHz | -6.8 | -4.8 | -7.6 | -5.6 |

Reference

[1] R4-2017675, Way forward on NR URLLC BS performance requirements, #97bis, Huawei, Hisilicon.