3GPP TSG-RAN4 Meeting #98-e	R4-2101448

# Online, , 25th Jan 2021 - 5th Feb 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0159 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: FRC for eMIMO sDCI/mDCI-based PDSCH transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, HiSilicon, Intel, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Perf |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | FRC for PDSCH demodulation requirement with sDCI/mDCI-based PDSCH transmission is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new FRCs for PDSCH demodulation requirements with mDCI-based transmission - R.PDSCH.1-3.3 and R.PDSCH.1-3.4 FDD: mDCI based transmission in FDD - R.PDSCH.2-3.3 and R.PDSCH.2-3.4 TDD: mDCI based transmission in TDD Addition of new FRCs for PDSCH demodulation requirements with sDCI-based SDM transmission - R.PDSCH.1-3.2 FDD: sDCI based SDM scheme in FDD - R.PDSCH.2-3.2 TDD: sDCI based SDM scheme in TDD Addition of new FRCs for PDSCH demodulation requirements with sDCI-based FDM Scheme A transmission - R.PDSCH.1-2.5 FDD: sDCI based FDM Scheme A in FDD - R.PDSCH.2-2.5 TDD: sDCI based FDM scheme A in TDD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The eMIMO feature sDCI/mDCI-based transmission scheme is not verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2.1.1, A.3.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521-4 ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

#### A.3.2.1.1	Reference measurement channels for SCS 15 kHz FR1

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

Table A.3.2.1.1-2: PDSCH Reference Channel for FDD (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-2.1 FDD | R.PDSCH.1-2.2 FDD | R.PDSCH.1-2.3 FDD | R.PDSCH.1-2.4 FDD | R.PDSCH.1-2.5 FDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 | 10 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 52 | 52 | 52 | 52 | 52 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 | 19 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAMLowSE |
| MCS index |  | 13 | 13 | 13 | 13 | 19 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 | 0.54 |
| Number of MIMO layers |  | 1 | 2 | 3 | 4 | 2 |
| Number of DMRS REs |  | 12 | 12 | 24 | 24 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 13064 | 26120 | 35856 | 48168 | 29704 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 2 | 4 | 5 | 6 | 4 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 10, 11 | Bits | 26208 | 52416 | 71136 | 94848 | 52416 |
| For Slots i = 1,…, 9, 12, …, 19 | Bits | 27456 | 54912 | 74880 | 99840 | 54912 |
| Max. Throughput averaged over 2 frames | Mbps | 12.411 | 24.814 | 34.063 | 45.760 | 28.219 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.1.1-3: PDSCH Reference Channel for FDD (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-3.1 FDD | R.PDSCH.1-3.2 FDD | R.PDSCH.1-3.3 FDD | R.PDSCH.1-3.4 FDD |  |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |  |
| Number of allocated resource blocks | PRBs | 52 | 52 | 26 (Note 3) | 26 (Note 4) |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 |  |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 19 | 19 | 19 | 19 |  |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |  |
| Target Coding Rate |  | 0.51 | 0.51 | 0.51 | 0.51 |  |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |  |
| Number of DMRS REs |  | 12 | 24 | 24 | 24 |  |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | Bits | 42016 | 37896 | 18960 | 18960 |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A |  |
| For Slots i = 1,…, 19 | CBs | 5 | 5 | 3 | 3 |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 10, 11 | Bits | 78624 | 71136 | 35568 | 35568 |  |
| For Slots i = 1,…, 9, 12, …, 19 | Bits | 82368 | 74880 | 37440 | 37440 |  |
| Max. Throughput averaged over 2 frames | Mbps | 39.915 | 36.001 | 18.012 | 18.012 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: PDSCH is scheduled in PRB numbers from 0 to 26. Note 4: PDSCH is scheduled in PRB numbers from 27 to 51. |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

#### A.3.2.2.2	Reference measurement channels for SCS 30 kHz FR1

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

Table A.3.2.2.2-2: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-2.1 TDD | R.PDSCH.2-2.2 TDD | R.PDSCH.2-2.3 TDD | R.PDSCH.2-2.4 TDD | R.PDSCH.2-2.5 TDD |
| Channel bandwidth | MHz | 40 | 40 | 40 | 40 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 106 | 106 | 106 | 106 | 106 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 | 4 | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 31 | 31 | 31 | 31 | 31 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAMLowSE |
| MCS index |  | 13 | 13 | 13 | 13 | 19 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 | 0.54 |
| Number of MIMO layers |  | 1 | 2 | 3 | 4 | 2 |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | 12 | 12 | 6 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 24 | 24 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 8456 | 16896 | 22032 | 29192 | 19464 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 26632 | 53288 | 73776 | 98376 | 60456 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6}for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 2 | 3 | 3 | 4 | 3 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 4 | 7 | 9 | 12 | 8 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 20, 21 | Bits | 53424 | 106848 | 144008 | 193344 | 106848 |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 17808 | 35616 | 45792 | 61056 | 35616 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 55968 | 111936 | 152640 | 203520 | 111936 |
| Max. Throughput averaged over 2 frames | Mbps | 37.644 | 75.318 | 104.004 | 138.646 | 85.508 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-3: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-3.1 TDD | R.PDSCH.2-3.2 TDD | R.PDSCH.2-3.3 TDD | R.PDSCH.2-3.4 TDD |  |
| Channel bandwidth | MHz | 40 | 40 | 40 | 40 |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |  |
| Allocated resource blocks | PRBs | 106 | 106 | 53 (Note 3) | 53 (Note 4) |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 | 4 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 |  |
| Allocated slots per 2 frames |  | 31 | 31 | 31 | 31 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 19 | 19 | 19 | 19 |  |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |  |
| Target Coding Rate |  | 0.51 | 0.51 | 0.51 | 0.51 |  |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 12 | 12 | 12 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 24 | 24 | 24 |  |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 27144 | 23040 | 11528 | 11528 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 83976 | 77896 | 38936 | 38936 |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 | 24 | 24 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6}for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 4 | 3 | 2 | 2 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 10 | 10 | 5 | 5 |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 20, 21 | Bits | 160272 | 145008 | 72504 | 72504 |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 53424 | 45792 | 22896 | 22896 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 167904 | 152640 | 76820 | 76820 |  |
| Max. Throughput averaged over 2 frames | Mbps | 118.796 | 109.768 | 54.869 | 54.869 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: PDSCH is scheduled in PRB numbers from 0 to 52. Note 4: PDSCH is scheduled in PRB numbers from 53 to 106. |  |  |  |  |  |  |

------------------------------------------------------------- End of change ------------------------------------------------------------