3GPP TSG-RAN WG4 meeting #98-e	R4-2101450

Electronic Meeting, 25 January – 5 February 2021

Agenda Item:	7.9.4.2.1

Source:	Ericsson

Title:	Simulation results of sDCI-based SDM transmission

Document for:	Information

# 1	Introduction

RAN4#97-e agreed with the simulation assumption of mDCI/sDCI-based SDM transmission schemes [1]. This contribution provides our alignment and impairment simulation results according to the simulation assumption.

# 2	Simulation results

## 2.1	Single-DCI based SDM transmission

Figure 1 shows our ideal simulation results of single-DCI based SDM transmission. Table 1 and Table 2 summarize the SNR levels to achieve 70% of maximum throughput.

| Simulation cases | Duplex mode | BW / SCS | MCS / MCS Table | Propagation condition | Antenna configuration | Frequency offset / Time offset | Metric |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3-1 | FDD | 10MHz / 15kHz | MCS19 (64QAM 0.5) MCS Table 1 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 1a: 200Hz / -0.5us Test 1b: 0Hz / 2us | 70% of max Tput |
| 3-2 | FDD | 10MHz / 15kHz | MCS19 (64QAM 0.5) MCS able 1 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 1a: 200Hz / -0.5us Test 1b: 0Hz / 2us | 70% of max Tput |
| 4-1 | TDD | 40MHz / 30kHz | MCS19 (64QAM 0.5) MCS Table 1 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 1a: 300Hz / -0.25us Test 1b: 0Hz / 1us | 70% of max Tput |
| 4-2 | TDD | 40MHz / 30kHz | MCS19 (64QAM 0.5) MCS Table 1 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 1a: 300Hz / -0.25us Test 1b: 0Hz / 1us | 70% of max Tput |

| (a) FDD SCS=15kHz | (b) TDD SCS=30kHz |
| --- | --- |

Figure 1	Simulation results of Single-DCI based SDM transmission.

Table 1	Summary of simulation results for single-DCI based SDM transmission (Test 1a, SNR to achieve 70% of maximum throughput (dB)).

|  | FDD SCS=15kHz 2Rx | FDD SCS=15kHz 4Rx | TDD SCS=30kHz 2Rx | TDD SCS=30kHz 4Rx |
| --- | --- | --- | --- | --- |
|  | TO = -0.5us FO = 200Hz | TO = -0.5us FO = 200Hz | TO = -0.25us FO = 300Hz | TO = -0.25us FO = 300Hz |
| Alignment results | 14.2 | 7.9 | 15.1 | 8.2 |
| Impairment results | 16.2 | 9.9 | 17.1 | 10.2 |

Table 2	Summary of simulation results for single-DCI based SDM transmission (Test 1b, SNR to achieve 70% of maximum throughput (dB)).

|  | FDD SCS=15kHz 2Rx | FDD SCS=15kHz 4Rx | TDD SCS=30kHz 2Rx | TDD SCS=30kHz 4Rx |
| --- | --- | --- | --- | --- |
|  | TO = 2us FO = 0Hz | TO = 2us FO = 0Hz | TO = 1us FO = 0Hz | TO = 1us FO = 0Hz |
| Alignment results | 13.2 | 6.9 | 13.6 | 7.1 |
| Impairment results | 15.2 | 8.9 | 15.6 | 9.1 |

# 3	Summary

This contribution provided our alignment and impairment simulation results for PDSCH with multi-TRP transmission. We would like to consider these results for the final PDSCH demodulation requirements.

# References

- R4-2017530, “Simulation assumption for PDSCH requirement with single-DCI and Multi-DCI transmission schemes”, Ericsson, Intel.