3GPP TSG-RAN WG4 meeting #98-e	R4-2101451

Electronic Meeting, 25 January – 5 February 2021

Agenda Item:	7.9.4.2.2

Source:	Ericsson

Title:	Simulation results of mDCI-based transmission

Document for:	Information

# 1	Introduction

RAN4#97-e agreed with the simulation assumption of mDCI/sDCI-based SDM transmission schemes [1]. This contribution provides our alignment and impairment simulation results according to the simulation assumption.

# 2	Simulation results

## 2.1	Multi-DCI based transmission

Figure 1 shows our ideal simulation results of multi-DCI based transmission. Table 1 summarizes the SNR levels to achieve 70% of maximum throughput.

| Simulation cases | Duplex mode | BW / SCS | MCS / MCS Table | Propagation condition (Note 1) | Antenna configuration | Frequency offset / Time offset | Metric |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | FDD | 10MHz / 15kHz | MCS19 (64QAM 0.5) MCS Table 1 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 2a: 200Hz / -0.5us | 70% of max Tput |
| 1-2 | FDD | 10MHz / 15kHz | MCS19 (64QAM 0.5) MCS Table 1 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 2a: 200Hz / -0.5us | 70% of max Tput |
| 2-1 | TDD | 40MHz / 30kHz | MCS19 (64QAM 0.5) MCS Table 1 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 2a: 300Hz / -0.25us | 70% of max Tput |
| 2-2 | TDD | 40MHz / 30kHz | MCS19 (64QAM 0.5) MCS Table 1 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 2a: 300Hz / -0.25us | 70% of max Tput |
| Note 1: MIMO faders are independent for each TRP transmission. |  |  |  |  |  |  |  |

| (a) FDD SCS=15kHz | (b) TDD SCS=30kHz |
| --- | --- |

Figure 1	Simulation results of Multi-DCI based transmission.

Table 1	Summary of simulation results for multi-DCI based transmission (SNR to achieve 70% of maximum throughput (dB)).

|  | FDD SCS=15kHz 2Rx | FDD SCS=15kHz 4Rx | TDD SCS=30kHz 2Rx | TDD SCS=30kHz 4Rx |
| --- | --- | --- | --- | --- |
| Alignment results | 16.4 | 10.4 | 16.9 | 10.6 |
| Impairment results | 18.4 | 12.4 | 18.9 | 12.6 |

# 3	Summary

This contribution provided our alignment and impairment simulation results for PDSCH with multi-TRP transmission. We would like to consider these results for the final PDSCH demodulation requirements.

# References

- R4-2017530, “Simulation assumption for PDSCH requirement with single-DCI and Multi-DCI transmission schemes”, Ericsson, Intel.