3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2101463

# Electronic Meeting, Jan. 25-Feb. 5, 2021

Source: 	vivo

Title: 	Simulation results for R17 RLM/BFD relaxation

Agenda Item:	11.9.2

Document for:	Information

Introduction

In RAN4 97e, the WF R4-2017269 was agreed in [1] and the corresponding evaluation assumptions are agreed in [2]. This contribution provides evaluation results based on [2], and further discussion can be found in [3].

Evaluation results on mobility performance impact

In last RAN4 meeting, the evaluation assumptions for mobility impact listed in [2] are agreed. Based on [2], we provide our results as below on

- RLF triggering latency: Increased latency in RLF triggering is evaluated, if the period of RLM measurements are extended K times and the baseline latency is the RLF triggering latency in R15.

- Delta SINR: Increased delta SINR compared to R15, while the delta SINR is the difference between estimated SINR and ideal SINR. Note that this is slightly different from the agreed delta SINR definition in [2].

Moreover, the definition of estimated SINR is further clarified as follows. There are two type of estimated SINR:

•	Estimated SINR without one-shot estimation error: Ideal L1 measurement with L1 filtering using N samples, i.e. L1 one-shot estimation error is NOT considered. N equals to 10 for RLM and 5 for BFD.

•	Estimated SINR with one-shot estimation error: Realistic L1 measurement with L1 filtering using N samples, i.e. L1 one-shot estimation error is considered. N equals to 10 for RLM and 5 for BFD.

Evaluation results for increased RLF triggering latency

In this section the evaluation results for increased RLF triggering latency are provided. Some additional assumptions are listed in Table 1.

Table 1 Additional assumptions based on [2] for increased RLF triggering latency evaluation

| Parameters | Value |
| --- | --- |
| Frequency Range | FR1 |
| Relaxation factor(K) | 2, 4, 8 |
| DRX cycle length | 40ms |
| Moving across Boarder | Option 1: Wrap around |
| BS array configuration | (M,N,P,Mg,Ng) = Option 2: (2,4,2,1,1) (dH,dV) = Option 2: (0.5, 0.9)λ |
| BS beam set | For direction of analog beam steering (in LCS): Azimuth angle φi = [-45, -15, 15, 45] degree Zenith angle θj = [90,115] degree |
| Q_out | -10dB |
| Q_in | -1.5dB |
| n310 | 1 |
| T310 | 1000 ms |
| n311 | 1 |
| w/wo one-shot estimation error | Both with and without one-shot estimation error are evaluated. |
| Intra-cell beam switching | Ideal beam switching |

In the evaluations, a SINR relaxation threshold is configured. A UE is allowed to relax RLM measurements K times when SINR is above threshold X dB. For a UE in relaxed RLM measurement mode, when measured SINR falls below the threshold X dB, the UE falls back to normal RLM measurement, i.e. without any relaxation. By evaluating the SINR variation from -6dB to 6 dB, a%-tile increased latency less than Yms is plotted in the below figures, while a is in {95, 99} and K is in {2,4,8}.

95%-tile increased latency, without one-shot estimation error

Figure 1 95%-tile increased RLF trigger latency (compared to R16) less than Yms if UE is allowed to relax RLM measurements K times when SINR is above threshold XdB. The one-shot estimation error is not considered

99%-tile increased latency, without one-shot estimation error

Figure 2 99%-tile increased RLF trigger latency (compared to R16) less than Yms if UE is allowed to relax RLM measurements K times when SINR is above threshold XdB. The one-shot estimation error is not considered

From Figure 1 and Figure 2 the 95%-tile and 99%-tile increased RLF triggering latency compared to R16 are shown. Based on above evaluation results we can make the following observations:

Observation 1  If a UE is only allowed to relax RLM when SINR is above a proper SINR threshold, and falls back to normal measurement when SINR is below such threshold, then the impact to increased RLF triggering latency with 99%-tile probability can be less than (K-1) × DRX_cycle, while K is the relaxation factor.

With 40ms DRX_cycle, the R16 RLF latency can be calculated as

oos latency + T310 = 40 × 1.5 × 10 + 1000 = 1600 [ms]

Then the increased percentage of RLF latency can be derived as

For K=2, 40/1600 = 2.5%

For K=4, 120/1600 = 7.5%

For K=8, 280/1600 = 17.5%

Observation 2  If 40ms DRX cycle is considered and a UE is only allowed to relax RLM when SINR is above a proper SINR threshold, the RLF latency increases no more than only 2.5% when K=2, 7.5% when K=4 and 17.5% when K=8, with 99%-tile probability.

Regarding how to set the threshold for relaxation, it is also observed that it can be optimized by setting different thresholds according to different relaxation factor and/or mobility state. However, it is also feasible to set a slightly conservative threshold, e.g. 0 dB for K=4 to mitigate the impact of UE mobility state.

Observation 3  The SINR threshold for relaxation can be set by leaving enough margin to accommodate different mobility scenarios.

95%-tile increased latency, with one-shot estimation error

Figure 3 95%-tile increased RLF trigger latency (compared to R16) less than Yms if UE is allowed to relax RLM measurements K times when SINR is above threshold XdB. The one-shot estimation error is considered

99%-tile increased latency, with one-shot estimation error

Figure 4 99%-tile increased RLF trigger latency (compared to R16) less than Yms if UE is allowed to relax RLM measurements K times when SINR is above threshold XdB. The one-shot estimation error is considered

From Figure 3 to Figure 4, the impact of one-shot SINR estimation error is shown. Compared to Figure 1 and Figure 2, we see the low SINR region is significantly impacted by the SINR estimation error. However, the medium or high SINR region is not impacted. If enough SINR margin is left for the threshold, then it is still feasible to relax RLM.

Observation 4  The one-shot SINR estimation error mainly impacts low SINR region, and it is still feasible to relax RLM if enough SINR margin is left for the relaxation threshold.

Evaluation results for Delta SINR

In this section the evaluation results for Delta SINR are provided. Some additional assumptions are listed in Table 2.

Table 2 Additional assumptions based on [2] for Delta SINR evaluation

| Parameters | Value |
| --- | --- |
| Frequency Range | FR1 |
| Relaxation factor(K) | 2, 4, 8 |
| DRX cycle length | 40ms |
| Moving across Boarder | Option 1: Wrap around |
| BS array configuration | (M,N,P,Mg,Ng) = Option 2: (2,4,2,1,1) (dH,dV) = Option 2: (0.5, 0.9)λ |
| BS beam set | For direction of analog beam steering (in LCS): Azimuth angle φi = [-45, -15, 15, 45] degree Zenith angle θj = [90,115] degree |
| w/wo one-shot estimation error | Without one-shot estimation error |
| Intra-cell beam switching | Ideal beam switching |

.

For Figure 5 below, as we considered 40ms DRX cycles, Delta SINR is the estimation error of L3 SS-SINR based on 10 samples in the last 15 x K DRX cycles, which is compared with ideal SINR. Note that ideal SINR estimation is considered.

Delta SINR = (estimated SINR – ideal SINR) 	[dB].

K is the relaxation factor. As summarized in Table 3, the Delta SINR span, including the increased span(inc. span) over R15 baseline, is provided.

Figure 5 CDF of Delta SINR under Umi scenario with different relaxation factor K

Table 3 Summary of Delta SINR spans under different scenarios

|  | 95%-5% Delta SINR span [dB] |  | 99%-1% Delta SINR span [dB] |  |
| --- | --- | --- | --- | --- |
| Scenario | 3km/h | 30km/h | 3km/h | 30km/h |
| K=1 | 1.8 | 12.7 | 9.6 | 22.8 |
| K=2 (inc. span) | 3.8 (+2) | 15.5 (+2.8) | 13.3 (+3.7) | 24.3 (+1.5) |
| K=4 (inc. span) | 6.8 (+5) | 17.8 (+5.1) | 16.6 (+7.0) | 27.1 (+4.3) |
| K=8 (inc. span) | 10.3 (+8.5) | 20.1 (+7.4) | 20.1 (+10.5) | 30 (+7.2) |

Based on above evaluation results in Table 3, we have following observations:

Observation 5  The conclusion of Delta SINR approach is aligned with the increased RLF latency approach. If the increased delta SINR margin is considered as no more than 11dB for K=8, then the same threshold at around 0dB can be obtained for RRM relaxation

Evaluation results on power saving gain

As stated in [1], TR 38.840 is the starting point for power saving gain analysis, and evaluation assumptions in [2] are only considered as options that not precluded. In our understanding, the purpose of this evaluation is to identify the scenarios (Note: not corner cases) that can achieve power saving gain when RLM/BFD are relaxed. Of course the power saving gain is not significant in all scenarios.

By considering PDCCH-WUS in R16 baseline, we further evaluate 3 types of traffic with 40 ms DRX cycle to observe the power saving gain

- FTP with 200ms arrival time: This is aligned with TS 38.840. The reason for evaluating 40 ms DRX cycle with it is

  - As indicated in the WID scope, the relaxation mainly considered short DRX cycles, and of course limited mobility impact should be guaranteed. For long DRX cycle there would be very limited room for relaxation.

  - Adjustment of DRX cycle length is normally based on apriori statistics of the traffic load at cell level, and therefore network may not always able to adjust DRX cycle length according the traffic of a given UE.

- FTP with 40ms arrival time: This is the intensive eMBB scenario discussed in RAN1.

- VoIP model: This is aligned with TS 38.840 and [3]. Note that semi-persistent scheduling is not considered.

The power saving gain is summarized in Table 4.

Table 4 Power saving gain results with DRX = 40 ms and PDCCH-WUS configured.

| Baseline | K = 2 | K = 4 | K = 8 |
| --- | --- | --- | --- |
| FTP with 200ms arrival time | 16.43% | 20.01% | 25.82% |
| FTP with 40ms arrival time | 11.63% | 13.73% | 16.67% |
| VoIP | 10.84% | 13.34% | 16.09% |

A more detailed illustration is provided in Table 5, while the power consumption is evaluated based on different DRX start offsets relative to RLM-RS. Since the RLM/BFD offset is not necessarily aligned with DRX, additional 15%~27% power saving gain can be achieved if PDCCH-WUS is configured and RLM/BFD is relaxed from 1x to 5x.

Table 5 Evaluation of different DRX start offset when PDCCH-WUS is configured

Based on above results, we have the following observations

Observation 6  To optimise the case where data packet arrives with interval of around 100ms to 200ms, and 40 ms DRX cycle is considered, relaxation of RLM/BFD may further achieve power saving gain on top of R16 power saving techniques. If PDCCH WUS is configured and relaxing RLM-RS measurements 2x/4x/8x, 15~ 26% additional gain can be achieved.

Observation 7  For intensive eMBB or VoIP traffic, relaxing RLM measurements 2x/4x/8x, can also achieve 10% to 17% power saving gain.

Observation 8  The DRX on-duration offset to the SSB may have impact on power saving gain.

Conclusion

Based on evaluation results, we have the following observations

Observation 1  If a UE is only allowed to relax RLM when SINR is above a proper SINR threshold, and falls back to normal measurement when SINR is below such threshold, then the impact to increased RLF triggering latency with 99%-tile probability can be less than (K-1) × DRX_cycle, while K is the relaxation factor.

Observation 2  If 40ms DRX cycle is considered and a UE is only allowed to relax RLM when SINR is above a proper SINR threshold, the RLF latency increases no more than only 2.5% when K=2, 7.5% when K=4 and 17.5% when K=8, with 99%-tile probability.

Observation 3  The SINR threshold for relaxation can be set by leaving enough margin to accommodate different mobility scenarios.

Observation 4  The one-shot SINR estimation error mainly impacts low SINR region, and it is still feasible to relax RLM if enough SINR margin is left for the relaxation threshold.

Observation 5  The conclusion of Delta SINR approach is aligned with the increased RLF latency approach. If the increased delta SINR margin is considered as no more than 11dB for K=8, then the same threshold at around 0dB can be obtained for RRM relaxation

Observation 6  To optimise the case where data packet arrives with interval of around 100ms to 200ms, and 40 ms DRX cycle is considered, relaxation of RLM/BFD may further achieve power saving gain on top of R16 power saving techniques. If PDCCH WUS is configured and relaxing RLM-RS measurement 2x/4x/8x, 15~ 26% additional gain can be achieved.

Observation 7  For intensive eMBB or VoIP traffic, relaxing RLM measurements 2x/4x/8x, can also achieve 10% to 17% power saving gain.

Observation 8  The DRX on-duration offset to the SSB may have impact on power saving gain.

- References

[1]  R4-2017269	WF on NR UE Power Saving Enhancements, MediaTek.  RAN4 #97e

[2]  R4-2017306  Evaluation assumptions for R17 RLM/BFD relaxation, vivo, MediaTek.  RAN4 #97e

[3]  R4-2101462  Discussion on R17 RLMBFD relaxation, vivo.  RAN4 #98e