3GPP TSG RAN4 Meeting #98e	R4-2101521

Electronic Meeting, 25th Jan – 5th Feb 2021

Title: 	Adding 90 and 100MHz bandwidth for band n40

Source: 	Huawei, HiSilicon

Agenda item:	9.21.2

Document for:	Approval

Introduction

In the last plenary meeting, proposals for adding new existing BWs are approved in the revised WID RP-202062 [1]. It is copied below that for every proposal, RAN4 considers to specify the corresponding RF requirements:

- Analyze and specify requirements: - Reference sensitivity and associated RB allocation. - When needed: - MPR (relative bandwidth criteria) - Additional Maximum Power Reduction (A-MPR) - NS signalling. - Any other RF requirement which might be relevant.

In last meeting, the REFSENS values proposed in R4-2016861 was acceptable by the group. In this paper, we provide further discussion on other remaining issues.

Introducing 90 and 100MHz UE channel bandwidth for band n40

To add 90 and 100MHz UE channel bandwidth to n40 requires thorough review on the general UE RF requirements specified for band n40 in TS 38.101-1. In the below table, we list the corresponding UE RF requirements that belong to band n40 in the current spec, and we also identify the potential specification needed to accommodate 90 and 100MHz UE channel bandwidth.

| Item | Sub-item | Appliance | Note |
| --- | --- | --- | --- |
| Tx |  |  |  |
| MOP | PC3 23dBm; PC2 26dBm | Yes to both 90 and 100MHz |  |
| MPR | General MPR | Yes |  |
|  | Delta MPR | 0dB for 90 1dB for 100 | 1dB delta MPR specification is proposed according to our measurements |
| AMPR | None | NA |  |
| Emissions | SEM | Generally applied |  |
|  | ACLR |  |  |
|  | Spurious |  |  |
| Others |  | Generally applied |  |
| Rx |  |  |  |
| REFSENS | 30k SCS REFSENS | Calculation based on smaller bandwidth, value scaled with bandwidth/RB allocations |  |
|  | 60k SCS REFSENS | Same logic as 30k SCS |  |
| Others |  | Generally applied |  |

MPR for 100MHz UE channel bandwidth

In the above table, we propose 1dB delta MPR specification on n40 for 100MHz UE channel bandwidth.

According to the agreement in [2], the limitation for maximum UE channel bandwidth is the relative bandwidth, which is the key to ensure the MPR performance (Memory effects, bandwidth limitation, and output matching Q). The agreement for maximum channel bandwidth criteria in [2] is as follows:

In order to introduce new channel bandwidths in the future we propose to introduce a criteria to decide if the general MPR applies or additional MPR should be evaluated:

- If max UL CH BW rel ≤3% for FDD bands and ≤4% for TDD bands general MPR requirement applies

- If max UL CH BW rel >3% for FDD bands and >4% for TDD bands additional band and channel bandwidth specific MPR should be evaluated

According to equation of maximum relative channel bandwidth as following,

max UL CH BW rel = (2*max CH BW) / (FUL low + FUL high)

we can calculate the maximum channel bandwidth for NR band n40, which is 94MHz and close to 100MHz.

According to the agreement above, we see the possibility to define 100MHz UE channel bandwidth, if the proper deltaMPR is defined. According to the results of series of measurements carried out upon PA, and RFIC, 0.5dB~1dB delta is found for ACLR evaluation and no difference for EVM evaluation.

It would be OK when 100MHz channel bandwidth is utilized if the transmission power is reduced slightly and properly, while when the higher transmission power is needed the smaller BWP or per-UE channel bandwidth can be configured.

And it seems feasible to specify 90MHz channel bandwidth even without bandwidth specific deltaMPR.

On the other hand, 100 MHz is the whole band for band n40. The attenuation of one example B40 filter is shown in blow figure. The filer can provide considerable attenuation at adjacent channel due to co-existence with ISM band. Similar observation was made in [7].

Based on the discussion above, we propose,

Proposal 1: DeltaMPR is not needed for 90 MHz and 100MHz UE channel bandwidth for n40.

Co-existence with WIFI or Bluetooth

The interference avoidance for in-device coexistence between LTE band 40 and 2.4 GHz ISM band had been studied in Rel-11 SI [6]. The co-existence are guaranteed by implementation measures, such FDM solutions, TDM solutions and power control solutions. We believe the situation on the co-existence keep unchanged for NR.

Table 4-1 shows a typical attenuation in the band 40 bandpass filter and Table 4-2 shows a typical attenuation in the ISM bandpass filter.

Table 4-1: Typical attenuation in the Band 40 bandpass filter

| Freq. (MHz) | 2402-2418 | 2418-2436 | 2436-2480 |
| --- | --- | --- | --- |
| Typical attenuation | 10 | 39 | 45 |

Table 4-2: Typical attenuation in the ISM bandpass filter

| Freq. (MHz) | 2300-2370 | 2370-2380 | 2380-2400 |
| --- | --- | --- | --- |
| Typical attenuation | 50 | 40 | 4 |

As shown in Table 4-1 and 4-2, the attenuation is small to the adjacent interference. Due to band 40 and 2.4GHz ISM band are adjacent, the whole n40 band will suffer large blocking from WIFI or BT TX in the frequency range 2400~2418, and correspondingly, the WIFI or BT will suffer blocking from n40 TX in upper edge frequency 2380~2400. We can observe that no new issue is introduced for the case 100 MHz CBW. The co-existence are guaranteed by implementation measures in frequency domain, time domain, power domain or their combinations.

Conclusions

In this paper, we discuss the corresponding RF requirements for the approved proposals: adding 90 and 100MHz for band n40.

Proposal 1: DeltaMPR is not needed for 90 MHz and 100MHz UE channel bandwidth for n40.

References

[1] RP-202062, Revised Basket WID on Rel-17 adding channel bandwidth support to existing NR bands, Ericsson

[2] Skyworks, Huawei, Qorvo, Broadcom, R4-1801008, “WF on maximum channel bandwidth criteria”, 3GPP TSG RAN4 Meeting #94e.

[3] R4-2015297, draftCR to 38101-1 to add 90 and 100MHz BW for band n40, Huawei, HiSilicon

[4] R4-2015298, draftCR to 38104 to add 90MHz BW for band n40, Huawei, HiSilicon

[5] R4-2016861, draftCR to 38101-1 to add 90 and 100MHz BW for band n40, Huawei, HiSilicon

[6] TR 36.816, Study on signalling and procedure for interference avoidance for in-device coexistence

[7] R4-2014593, n40 MPR and Interference for Additional Channel Bandwidths, Skyworks Solutions, Inc.