3GPP TSG-RAN WG4 Meeting #98-e                                                    	R4-2101563

Online, 25th January - 5th February, 2021

Source:	Vodafone

Title:	TP for TR 37.717-41-11: DC_1-7-8-20_n28

Agenda item:			9.6.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-41-11 to include DC_1-7-8-20_n28.

The TIB,c and RIB values provided are derived from the relevant subsets already present in 36101 and 38101-3.

# 2. Reference

# 3. Text Proposal

<Start of Text Proposal>

### 5.1.x	DC_1-7-8-20 _n28

#### 5.1.x.1	Configuration for EN-DC

Table 5.1.x.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-8A-20A _n28A | DC_1A_n28A DC_7A_n28A DC_8A_n28A DC_20A_n28A |

#### 5.1.x.2	∆TIB and ∆RIB values

Table 5.1.x.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-8A-20A _n28A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |

Table 5.1.x.2.-1: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-7A-8A-20A _n28A | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |

#### 5.1.1.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

<End of Text Proposal>