3GPP TSG-RAN WG4 Meeting # 98-e                                                              R4-2101609

Electronic Meeting, 25th January– 5th Febuary, 2021

Source:	Huawei, HiSilicon, CMCC

Title:	TP for TR 37.717-00-00 for SUL_n79A-n97A

Agenda item:	9.8.2

Document for:	Approval

Background

The WID for SUL band combinations was updated in RAN #90 meeting. This contribution provides a TP for SUL_n79A-n97A for TR 37.717-00-00.

Discussion

Table 1 summarize the MSD analysis results for SUL_n79-n97, due to 2nd harmonic interference.

Table 1 The MSD analysis results for SUL_n79-n97, due to 2nd harmonic interference

|  | primary |  | diversity |  |
| --- | --- | --- | --- | --- |
| parameter | value | H2 level | value | H2 level |
| n97 PA output power, dBm | 27 |  |  |  |
| n97 Post PA Filter IL, dB | 2.5 |  |  |  |
| antenna switch IL, dB | 0.7 |  |  |  |
| Diplexer IL,dB | 0.8 |  |  |  |
| n97 PA H2, Attenuation,dBc | 40 |  |  |  |
| n97 PA H2, dBm |  | -13 |  |  |
| n97 Post PA Filter,dB | 25 |  |  |  |
| n97 H2 output at Post PA Filter,dBm |  | -38 |  |  |
| n97 H2 output at antenna switch,dBm |  | -38.7 |  |  |
| Diplexer Rejection&isolation,dB | 10 | -48.7 |  |  |
| Antenna isolation, dB |  |  | 10 | -58.7 |
| antenna switch IL, dB | 0.7 | -49.4 | 0.7 | -59.4 |
| n41 filter IL, dB | 2 | -51.4 | 2 | -61.4 |
| PA to LNA PCB isolation, dB | 70 | -83 | 70 | -83 |
| Composite H2 at LNA port,dBm |  | -51.3969965 |  | -61.37006 |
| SNR requirement for QPSK(dB) | -1 |  |  |  |
| REFSENSE (referred to antenna)(40MHz BW) |  | -52.3969965 |  | -62.37006 |
| combined REFSENS(40MHz BW), dBm |  | -62.7864404 |  |  |
| Margin,dB |  | 2.5 |  |  |
| combined REFSENS(10MHz BW)after considering Margin, dBm |  | -60.2864404 |  |  |
| Noise Floor(40M), dBm |  | -89.7 |  |  |
| MSD |  | 29.41355963 |  |  |

Text Proposal

---Start of changes---

## 5.x	SUL_n79A-n97A

### 5.x.1	Operating bands

Table 5.x.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n79-n97 | n79, n97 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.x.2	Channel bandwidths per operating band

Table 5.x.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUL_n79A-n97A | SUL_n79A-n97A | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  | n97 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |

### 5.x.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

### 5.x.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.x.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for SUL_n79-n97.

Table 5.x.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n79 | UL/DL frequency range | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 | 17600 | 20000 |
| n97 | UL frequency range | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 | 9200 | 9600 |

The 2nd harmonic interference of band n97 may fall into Rx band of n79.

### 5.x.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.x.5-1: Supplementary uplink configuration for reference sensitivity

|  | NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n79 | n97 | 15 |  |  |  |  |  |  | 270 | 270 | 270 |  | 270 |  | 270 |

The MSD exception for SUL_n79-n97 are specified below.

Table 5.X.5-1: Reference sensitivity exceptions due to cross band isolation

| UL band |  | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n97 |  | n411,2 |  |  |  |  |  |  | 29.4 | 28.4 | 27.6 | 26.3 |  | 25.3 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for SUL_n78-n80, SUL_n78-n86. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.X.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation

| UL band | DL band | 5 MHz (NRB) | 10 MHz (NRB) | 15 MHz (NRB) | 20 MHz (NRB) | 25 MHz (NRB) | 30MHz (NRB) | 40 MHz (NRB) | 50 MHz (NRB) | 60 MHz (NRB) | 80 MHz (NRB) | 90 MHz (NRB) | 100 MHz (NRB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n97 | n41 |  |  |  |  |  |  | 100 | 135 | 160 | 216 |  | 270 |
| NOTE 1: 15 kHz SCS is assumed for UL band. NOTE 2: The UL configuration applies regardless of the channel bandwidth of the low band NOTE 3: Unless stated otherwise, UL resource blocks shall be centered within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.x.6	∆TIB and ∆RIB values

For SUL_n79-n97, the TIB,c and RIB,c values are given in the tables below.

Table 5.x.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| SUL_n79-n97 | n79 | 0.8 |
|  | n97 | 0.3 |

Table 5.x.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| SUL_n79-n97 | n79 | 0.5 |

---End of changes---

Reference

[1]	RP-202611, “Revised WID on Band combinations for SA NR Supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP)”, Huawei, HiSilicon