3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2101637

# Electronic Meeting, Jan. 25-Feb. 5, 2021

Agenda item:	7.1.5.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on remaining issues for random access for NR-U

Document for:	Discussion

- Introduction

In the last RAN4#97e meeting, the RRM requirements for NR-U were discussed with the agreements captured in the WF [1]. For random access in NR-U, we further provide our views on the remaining issues in this paper.

- Discussion

In the last RAN4#97e meeting, the core requirements for random access were discussed with following agreements:

RAN4#97e Agreement Specification structure for section 6.2.2A 6.2.2A Random access with CCA 6.2.2A.1 Introduction 6.2.2A.2 Requirements [at least for 4-step RA] FFS: whether 6.2.2A.2 covers only 4-step RA or (if RAN4 will specify requirements for 2-step for NR-U) 6.2.2A.2 is further split to cover 4-step and 2-step RA or a separate section on the same level (e.g. 6.2.2A.3) is introduced for 2-step RA requirements 4-step RA type The requirements are not the same as in Rel-15 The requirements will include requirements for contention-based and non-contention based RA 2-step RA type FFS: RAN4 will define in Rel-16 NR-U RA requirements for 2-step RA RAN4 will investigate and discuss in RAN4#98-e the impacts of 2-step RA on other sections/requirements

It was disused in the last meeting the current requirement for Random access for NR-U shall be modified to reflect the corresponding LBT operations. As pointed out in [2], the UE behavior may be different depending on configuration of lbt-FailureRecoveryConfig according to TS 38.321. From our understanding, it is necessary to update the corresponding requirements for reflect these behavior for NR-U.

Another issue is whether to define RA requirements for 2-step RA in Rel-16 NR-U. It could be observed that from RAN2 spec that the corresponding part for 2-step is defined with NR-U. However, as discussed in the last RAN4 meeting, all the defined RRM requirements for NR-U are based on Rel-15 requirements. There requirements defined in Rel-16 were never considered in NR-U during previous discussion. Whether to define the requirements for Rel-16 features for NR-U at these stage should be taken very cautiously also for other Rel-16 features.

Proposal 1: Whether to define the requirements for Rel-16 features for NR-U should be taken cautiously. Whether to define 2-step RACH requirements for NR-U shall be discussed also for other Rel-16 features/requirements.

- Conclusions

Proposal 1: Whether to define the requirements for Rel-16 features for NR-U should be taken cautiously. Whether to define 2-step RACH requirements for NR-U shall be discussed also for other Rel-16 features/requirements.

References

[1] R4-2017080. WF on NR-U RRM Core Requirements, Ericsson

[2] R4-2015386. Random access requirements in NR-U, Nokia, Nokia Shanghai Bell