3GPP TSG-RAN WG4 Meeting #98-e	R4-2101678

Online, 25th Jan– 05th Feb, 2021

Title: 	Discussion on testability of pathloss-RS activation delay

Source: 	Huawei, HiSilicon

Agenda item:	7.9.3.3.3

Document for:	Discussion

Introduction

In RAN4#97e meeting, the testability for pathloss-RS activation delay was discussed and a PHR based test method was suggested in [1]. In this contribution, we will investigate the testability of PHR based pathloss-RS activation test.

Discussion

In last RAN4 meeting, it is mentioned to use the different PHR values to reflect the pathloss-RS change for testing pathloss-RS activation delay. Then, the different PHR values for the old pathloss-RS and target pathloss-RS need to be configured.

In TS38.213, the UE power headroom reports are defined as below:

The types of UE power headroom reports are the following. A Type 1 UE power headroom that is valid for PUSCH transmission occasion on active UL BWP of carrier of serving cell . A Type 3 UE power headroom that is valid for SRS transmission occasion on active UL BWP of carrier of serving cell . … If a UE determines that a Type 1 power headroom report for an activated serving cell is based on an actual PUSCH transmission then, for PUSCH transmission occasion on active UL BWP of carrier of serving cell , the UE computes the Type 1 power headroom report as [dB] where , , , , , and are defined in Clause 7.1.1.

It can be observed that UE determines the power headroom report for PUSCH transmission based on 7 input values. The input  is the UE configured maximum output power. The input  is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks for PUSCH transmission. The input  is the downlink pathloss estimate in dB calculated by the UE using pathloss-RS index qd and equal to (referenceSignalPower – higher layer filtered RSRP). For these 3 inputs, it only involves one or two parameters for setting its value. However, for the other 4 inputs, it involves multiple parameters for setting its value, which can be listed in Table 1.

Table 1: Parameters for PUSCH transmission power control

| Inputs |  | Involved parameters |
| --- | --- | --- |
| (= + ) | j=0 | preambleReceivedTargetPower, msg3-DeltaPreamble or msgADeltaPreamble, |
|  | j=1 | p0-NominalWithoutGrant, p0-PUSCH-Alpha in ConfiguredGrantConfi, |
|  | j>1 | p0-NominalWithGrant, p0 in P0-PUSCH-AlphaSet, |
|  | j=0 | msgA-Alpha or msg3-Alpha |
|  | j=1 | p0-PUSCH-Alpha in ConfiguredGrantConfig |
|  | j>1 | alpha in P0-PUSCH-AlphaSet |
|  |  | , where is provided by deltaMCS |
|  |  | , and for PUSCH with UL-SCH data Where, C is a number of transmitted code blocks, Kr is a size for code block r, and NRE is a number of resource elements. |
|  |  | , and for CSI transmission in a PUSCH without UL-SCH data Where, Qm is the modulation order and R is the target code rate. |
|  | w/o tpc-Accumulation | Where, is a TPC command value included in a DCI format |
|  | w/ tpc-Accumulation |  |

Observation 1: The value of power headroom report is determined by plenty of parameters, and the calculation on values of all the power control parameters will be very complicated in order to setting a certain power headroom report value.

If any one of the power control related parameters is changed, then the power headroom report for PUSCH transmission may be updated. It can be observed that only the input is related to the pathloss-RS index. In order to directly reflect the pathloss-RS switching by the power headroom report change, all the power control related parameters except the RSRP value of pathloss-RS need to be the same values during the whole test.

Observation 2: Only when all the power control parameters except pathloss-RS RSRP are not changed during the whole test, the pathloss-RS switching could be directly reflected by the power headroom report change.

Some power control parameters are included in RRC message PUSCH-PowerControl or PUSCH-ConfigCommon. In TS38.331, the RRC meassages PUSCH-PowerControl and PUSCH-ConfigCommon are defined as follows:

PUSCH-PowerControl ::= SEQUENCE { tpc-Accumulation ENUMERATED { disabled } OPTIONAL, -- Need S msg3-Alpha Alpha OPTIONAL, -- Need S p0-NominalWithoutGrant INTEGER (-202..24) OPTIONAL, -- Need M p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF P0-PUSCH-AlphaSet OPTIONAL, -- Need M pathlossReferenceRSToAddModList SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS OPTIONAL, -- Need N pathlossReferenceRSToReleaseList SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS-Id OPTIONAL, -- Need N twoPUSCH-PC-AdjustmentStates ENUMERATED {twoStates} OPTIONAL, -- Need S deltaMCS ENUMERATED {enabled} OPTIONAL, -- Need S sri-PUSCH-MappingToAddModList SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControl OPTIONAL, -- Need N sri-PUSCH-MappingToReleaseList SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControlId OPTIONAL -- Need N } P0-PUSCH-AlphaSet ::= SEQUENCE { p0-PUSCH-AlphaSetId P0-PUSCH-AlphaSetId, p0 INTEGER (-16..15) OPTIONAL, -- Need S alpha Alpha OPTIONAL -- Need S } PUSCH-ConfigCommon ::= SEQUENCE { groupHoppingEnabledTransformPrecoding ENUMERATED {enabled} OPTIONAL, -- Need R pusch-TimeDomainAllocationList PUSCH-TimeDomainResourceAllocationList OPTIONAL, -- Need R msg3-DeltaPreamble INTEGER (-1..6) OPTIONAL, -- Need R p0-NominalWithGrant INTEGER (-202..24) OPTIONAL, -- Need R ... }

The RRC messages PUSCH-PowerControl and PUSCH-ConfigCommon are semi-static indicated and could be configured as fixed values during the whole test. However, some other power control parameters are included in DCI format and will be dynamic indicated. It will be difficult to set these parameters as fixed values, such as TPC command value. The input is the PUSCH power control adjustment and determined by the TPC command value. Even UE does not perform pathloss-RS switching, the power headroom report value may also be updated due to TPC command change.

Observation 3: The power control parameters included in DCI format are dynamic indicated and would be difficult to be set as fixed values during the whole test.

According to the analysis in this contribution, we suggest not to introduce the pathlaoss-RS activation delay test.

Proposal: In considering of the complexity and feasibility, it is suggested not to introduce the test for verifying pathloss-RS activation delay requirements.

Conclusions

This contribution provides the discussion on the testability of PHR based pathloss-RS activation test. The following are provided:

Observation 1: The value of power headroom report is determined by plenty of parameters, and the calculation on values of all the power control parameters will be very complicated in order to setting a certain power headroom report value.

Observation 2: Only when all the power control parameters except pathloss-RS RSRP are not changed during the whole test, the pathloss-RS switching could be directly reflected by the power headroom report change.

Observation 3: The power control parameters included in DCI format are dynamic indicated and would be difficult to be set as fixed values during the whole test.

Proposal: In considering of the complexity and feasibility, it is suggested not to introduce the pathloss-RS activation test.

Reference

- R4-2017286, “Email discussion summary for [97e][216] NR_eMIMO_RRM”, Samsung

- R4-2017375, “WF on NR eMIMO RRM Performance requirements”, Samsung