3GPP TSG RAN WG4 Meeting #98-e	R4-2101768

Electronic Meeting, January 25 – February 5, 2020

Agenda item:			7.14.2.1.1

Source:	vivo

Title:	Simulation results for CSI-RSRP measurement accuracy

Document for:	Information

- Introduction

In the RAN4#97-e meeting, the updated simulation assumption for CSI-RS based measurement are provided in [1].

In this contribution, we provide simulation results for CSI-RSRP measurement accuracy.

# Discussion

Simulation results for CSI-RSRP measurement accuracy are provided in Tables 1, 2, 3, 4 for different configurations in both FR1 and FR2. CDF curves are further provided in Annex.

Table 1. simulation results of CSI-RSRP accuracy under {4GHz, SINR = -6dB, 48PRB, D=3, 15KHz SCS}

| delay |  |  | 0 |  |  | CP/2 |  |  | CP |  |  | 2CP |  |  | 3us |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| channel | sample | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -2.44 | 0.33 | 2.20 | -2.31 | 0.37 | 2.15 | -2.88 | 0.07 | 1.90 | -3.92 | -0.43 | 1.59 | -2.78 | 0.27 | 2.03 |
|  | 3 | -1.24 | 0.19 | 1.32 | -1.15 | 0.23 | 1.28 | -1.62 | -0.13 | 1.13 | -2.45 | -0.57 | 0.77 | -1.52 | 0.10 | 1.24 |
|  | 5 | -1.07 | 0.15 | 0.97 | -0.87 | 0.16 | 1.05 | -1.37 | -0.13 | 0.77 | -1.87 | -0.61 | 0.35 | -1.18 | 0.04 | 0.98 |
| EPA5 | 1 | -2.48 | 0.06 | 2.15 | -3.07 | 0.05 | 2.20 | -3.59 | -0.22 | 2.12 | -4.29 | -0.79 | 1.95 | -3.47 | 0.01 | 2.14 |
|  | 3 | -1.41 | 0.04 | 1.32 | -1.67 | 0.02 | 1.26 | -1.93 | -0.33 | 1.13 | -2.60 | -0.87 | 0.81 | -1.90 | -0.15 | 1.34 |
|  | 5 | -1.13 | 0.06 | 1.07 | -1.30 | 0.06 | 1.09 | -1.86 | -0.32 | 0.82 | -2.19 | -0.86 | 0.56 | -1.47 | -0.12 | 1.00 |
| ETU30 | 1 | -2.53 | 0.09 | 2.29 | -2.84 | 0.00 | 2.13 | -3.21 | -0.22 | 1.91 | -4.25 | -0.77 | 1.80 | -3.14 | -0.08 | 2.01 |
|  | 3 | -1.54 | 0.10 | 1.30 | -1.52 | -0.04 | 1.29 | -2.00 | -0.27 | 1.11 | -2.74 | -0.77 | 0.66 | -1.67 | -0.17 | 1.24 |
|  | 5 | -1.11 | 0.02 | 0.95 | -1.30 | 0.02 | 1.08 | -1.45 | -0.26 | 0.80 | -2.11 | -0.83 | 0.38 | -1.29 | -0.15 | 0.83 |
| ETU70 | 1 | -2.43 | 0.14 | 2.30 | -2.84 | 0.03 | 2.18 | -3.22 | -0.22 | 1.89 | -4.10 | -0.71 | 1.78 | -3.17 | -0.10 | 1.95 |
|  | 3 | -1.34 | 0.09 | 1.43 | -1.60 | -0.04 | 1.22 | -2.13 | -0.36 | 0.94 | -2.49 | -0.80 | 0.75 | -1.83 | -0.16 | 1.15 |
|  | 5 | -1.16 | 0.06 | 1.10 | -1.40 | -0.05 | 1.01 | -1.59 | -0.31 | 0.67 | -2.24 | -0.80 | 0.42 | -1.53 | -0.18 | 0.92 |

Table 2. simulation results of CSI-RSRP accuracy under {4GHz, SINR = -6dB, 48PRB, D=3, 30KHz SCS}

| delay |  |  | 0 |  |  | CP/2 |  |  | CP |  |  | 2CP |  |  | 3us |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| channel | sample | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| AWGN | 1 | -2.31 | 0.38 | 2.21 | -2.30 | 0.43 | 2.23 | -3.07 | 0.06 | 1.93 | -3.74 | -0.40 | 1.69 | -3.09 | 0.01 | 1.89 |
|  | 3 | -1.19 | 0.22 | 1.37 | -1.35 | 0.23 | 1.42 | -1.78 | -0.10 | 1.12 | -2.38 | -0.60 | 0.72 | -1.74 | -0.21 | 1.00 |
|  | 5 | -0.82 | 0.16 | 1.09 | -0.87 | 0.16 | 1.19 | -1.36 | -0.19 | 0.78 | -2.10 | -0.64 | 0.46 | -1.34 | -0.28 | 0.74 |
| EPA5 | 1 | -2.39 | 0.18 | 2.31 | -2.95 | 0.10 | 2.19 | -3.32 | -0.19 | 1.97 | -3.92 | -0.75 | 1.64 | -3.19 | -0.40 | 1.93 |
|  | 3 | -1.14 | 0.18 | 1.47 | -1.49 | 0.10 | 1.33 | -1.70 | -0.27 | 0.98 | -2.63 | -0.85 | 0.74 | -2.02 | -0.42 | 1.04 |
|  | 5 | -0.80 | 0.14 | 1.15 | -1.03 | 0.08 | 1.07 | -1.65 | -0.29 | 0.82 | -2.21 | -0.87 | 0.19 | -1.70 | -0.47 | 0.65 |
| ETU30 | 1 | -2.80 | 0.16 | 2.15 | -3.18 | -0.08 | 2.01 | -3.46 | -0.38 | 1.76 | -4.58 | -0.84 | 1.46 | -3.72 | -0.48 | 1.65 |
|  | 3 | -1.52 | 0.02 | 1.29 | -1.61 | -0.14 | 1.21 | -2.20 | -0.38 | 0.88 | -2.89 | -0.86 | 0.57 | -2.35 | -0.51 | 0.80 |
|  | 5 | -1.01 | 0.03 | 1.00 | -1.28 | -0.15 | 0.99 | -1.69 | -0.39 | 0.66 | -2.27 | -0.94 | 0.17 | -1.93 | -0.62 | 0.47 |
| ETU70 | 1 | -2.81 | 0.06 | 2.07 | -3.05 | -0.02 | 2.15 | -3.28 | -0.31 | 1.84 | -4.46 | -0.86 | 1.55 | -3.86 | -0.41 | 1.83 |
|  | 3 | -1.56 | 0.00 | 1.20 | -1.62 | -0.08 | 1.15 | -2.04 | -0.34 | 1.02 | -2.85 | -0.90 | 0.62 | -2.35 | -0.56 | 1.02 |
|  | 5 | -1.14 | -0.03 | 1.01 | -1.18 | -0.11 | 0.87 | -1.71 | -0.42 | 0.70 | -2.48 | -0.95 | 0.19 | -1.92 | -0.56 | 0.62 |

Table 3. simulation results of CSI-RSRP accuracy under {30GHz, SINR = -6dB, 48PRB, D=3, 60KHz SCS}

| delay |  |  | 0 |  |  | CP/2 |  |  | CP |  |  | 2CP |  |  | 3us |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| channel | sample | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| TDL-A 83Hz | 1 | -2.43 | 0.08 | 2.19 | -2.33 | 0.11 | 2.30 | -3.06 | -0.21 | 2.15 | -3.82 | -0.73 | 1.81 | -4.88 | -1.18 | 1.54 |
|  | 3 | -1.25 | 0.07 | 1.23 | -1.28 | 0.05 | 1.35 | -1.65 | -0.23 | 1.01 | -2.43 | -0.80 | 0.88 | -2.99 | -1.20 | 0.34 |
|  | 5 | -1.02 | 0.02 | 1.20 | -1.02 | 0.11 | 1.07 | -1.48 | -0.27 | 0.85 | -2.00 | -0.86 | 0.63 | -2.42 | -1.18 | 0.16 |
| TDL-C 830Hz | 1 | -2.24 | 0.10 | 2.30 | -2.45 | 0.05 | 2.01 | -3.03 | -0.26 | 1.92 | -3.76 | -0.76 | 1.86 | -4.43 | -1.17 | 1.55 |
|  | 3 | -1.33 | 0.09 | 1.40 | -1.39 | 0.06 | 1.29 | -1.76 | -0.24 | 1.01 | -2.42 | -0.76 | 0.87 | -3.02 | -1.23 | 0.41 |
|  | 5 | -1.17 | 0.14 | 1.02 | -1.14 | 0.00 | 1.09 | -1.37 | -0.26 | 0.76 | -2.05 | -0.74 | 0.55 | -2.79 | -1.28 | 0.15 |

Table 4. simulation results of CSI-RSRP for {30GHz, SINR = -6dB, 48PRB, D=3, 120KHz SCS}

| delay |  |  | 0 |  |  | CP/2 |  |  | CP |  |  | 2CP |  |  | 3us |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| channel | sample | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| TDL-A 83Hz | 1 | -2.77 | 0.09 | 2.58 | -2.64 | 0.07 | 2.07 | -3.18 | -0.27 | 2.11 | -4.32 | -0.76 | 1.87 | -8.99 | -3.21 | 0.75 |
|  | 3 | -1.63 | 0.07 | 1.32 | -1.27 | -0.02 | 1.28 | -2.08 | -0.27 | 1.06 | -2.52 | -0.78 | 0.63 | -6.53 | -3.33 | -0.73 |
|  | 5 | -1.26 | 0.08 | 1.09 | -1.16 | -0.04 | 0.96 | -1.67 | -0.32 | 0.89 | -2.12 | -0.80 | 0.46 | -6.27 | -3.28 | -1.08 |
| TDL-C 830Hz | 1 | -2.50 | 0.06 | 2.23 | -2.48 | 0.00 | 2.05 | -3.07 | -0.31 | 1.85 | -4.11 | -0.87 | 1.55 | -8.68 | -3.18 | 0.33 |
|  | 3 | -1.64 | 0.03 | 1.27 | -1.34 | 0.00 | 1.38 | -1.89 | -0.31 | 0.88 | -2.74 | -0.95 | 0.61 | -6.45 | -3.29 | -1.24 |
|  | 5 | -1.12 | 0.08 | 1.06 | -1.12 | -0.08 | 1.00 | -1.44 | -0.42 | 0.70 | -2.30 | -0.89 | 0.33 | -6.13 | -3.29 | -1.63 |

Following observations can be made based on simulation results above.

Observation 1. If timining delay between two cells is smaller than CP, the absolute accuary of CSI-RSRP measurement with 5 samples is within ±2 dB in both FR1 and FR2.

Observation 2. If timining delay between two cells is smaller than 2CP, the absolute accuary of CSI-RSRP measurement with 5 samples is within ±3 dB in both FR1 and FR2.

Observation 3. If timining delay between two cells is smaller than 3us, the absolute accuary of CSI-RSRP measurement with 5 samples is within ±2 dB in FR1 only.

Observation 4. If timining delay between two cells is smaller than CP, the absolute accuary of CSI-RSRP measurement with 3 samples is worse than ±2 dB in both FR1 and FR2.

# Summary

In this contribution we provided simulation results for CSI-RSRP measurement accuracy. Following observations are made.

Observation 1. If timining delay between two cells is smaller than CP, the absolute accuary of CSI-RSRP measurement with 5 samples is within ±2 dB in both FR1 and FR2.

Observation 2. If timining delay between two cells is smaller than 2CP, the absolute accuary of CSI-RSRP measurement with 5 samples is within ±3 dB in both FR1 and FR2.

Observation 3. If timining delay between two cells is smaller than 3us, the absolute accuary of CSI-RSRP measurement with 5 samples is within ±2 dB in FR1 only.

Observation 4. If timining delay between two cells is smaller than CP, the absolute accuary of CSI-RSRP measurement with 3 samples is worse than ±2 dB in both FR1 and FR2.

- References

- R4-2017230	Updated link-level simulation assumptions for CSI-RS based L3 measurements, CATT

# Annex

CDF curves for CSI-RSRP measurement accuracy under 15kHz SCS and 60kHz SCS are provided in the following figures.

For 15kHz SCS,

- Delay = 0

- Delay = CP

- Delay = 2CP

- Delay = 3us

For 60kHz SCS,

- Delay = 0

- Delay = CP

- Delay = 2CP

- Delay = 3us