3GPP TSG-RAN WG4 Meeting #98-e	R4-2101851

Electronic Meeting, 25th January - 5th February, 2021

Source:	ZTE Corporation

Title:	Discussion on 2Tx UL switching between two bands

Agenda Item:	11.2.2.3

Document for:	Discussion

# Introduction

The RF enhancement for FR1 in Rel-17 proposes a new case for UL Tx switching [1], which the Tx switching is performed between one carrier at Band A and two contiguous carriers at Band B, for both SUL and UL CA.

And in RAN4#97e, whether or not to mandating 2-layer MIMO support for this case is still open [2]:

- 2-layer MIMO support - FFS whether to Mandate 2-layer PUSCH transmission in all the carriers supporting 2Tx, including the two carriers on band B in 1Tx-2Tx switching scenario and the three carriers on band A+B for 2Tx-2Tx switching scenario.

In order to conclude this open issue, there is one scenario which is not clarified in the scope.

# Discussion

In the case of switching between carrier #1 at Band A and two contiguous carriers (#2 and #3) at Band B, there could be three modes. And in each mode, the two RF chains are active at the same time with different configurations.

Fig. 1 shows the three modes:

Fig. 1, three modes for Tx switching between two bands

In Mode #1, two RF chains are used for the same carrier #1 at Band A, either in UL-MIMO or transparent TxD. In Mode #2, two RF chains are used for two contiguous carriers (#2 and #3), either in UL-MIMO or transparent TxD. While in Mode #3, one RF chain is used for carrier #2 with single Tx, and the other RF chain is for carrier #3 also with single Tx.

We understand that the switching between Mode #1 and Mode #2 is the main focus of the WID [1]. However, according to the scope of the latest WID, it is not clear whether or not Mode #3 is included or not. Since in Mode #3 UL-MIMO support is not required at all, it should be clarified before we make any decision on mandating 2-layer MIMO support for all carriers.

Observation: It is not clear whether the mode with each of the two contiguous carriers at Band B transmitted in single Tx is included in the scope of the WID.

Proposal : RAN4 to clarify whether or not the observed mode (Mode #3) is included in the working scope of the WID before deciding optionality of 2-layer MIMO support for all three carriers.

# Conclusion

In this paper, we have the following observation and proposal for 2Tx switching between two bands:

Observation: It is not clear whether the mode with each of the two contiguous carriers at Band B transmitted in single Tx is included in the scope of the WID.

Proposal : RAN4 to clarify whether or not the observed mode (Mode #3) is included in the working scope of the WID before deciding optionality of 2-layer MIMO support for all three carriers.

# Reference

RP-202799, New WID proposal: RF requirements enhancement for NR frequency range 1 (FR1) in Rel-17, Huawei, HiSilicon, China Telecom

R4-2017815, WF on RF requirements for Rel-17 Tx switching enhancement, China Telecom