3GPP TSG-RAN WG4 Meeting #98-e	R4-2101860

# Online, 25–Jan --  5-Feb, 2021

Source:	Ericsson

Title:	MRTD and MTTD in non-contiguous CA in FR1

Agenda item:	7.19.5

Document for:	Approval

# 1	Introduction

In this contribution we discuss the colocation requirement for FR1 non-contiguous CA, based on the contribution from Huawei in [1].

# 2	Discussion

As pointed out in [1], release-16 has defined several intra-band non-contiguous band combinations. The band combinations are listed in TS 38.101-1 [2].

Table 5.2A.1-2: Intra-band non-contiguous CA operating bands in FR1 NR CA Band; NR Band (Table 5.2-1). CA_n3(*); n3. CA_n7(*); n7. CA_n25(*); n25. CA_n41(*); n41. CA_n48(*); n48. CA_n66(*); n66. CA_n77(*); n77. CA_n78(*); n78. NOTE 1: The minimum requirements only apply for non simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 2: The notation CA_nX(*) in this table indicates intra-band non-contiguous CA for band nX. The configurations for each band are in 5.5A.2..

In TS 38.104 [3] the TAE requirement for intra-band con-contiguous carrier aggregation is stated as: “For intra-band non-contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 3µs”. A TAE requirement of 3 µs is well suited for non-colocated deployment. A TAE requirement of 3 µs is already used for inter-band carrier aggregation, which can be deployed non-colocated.

Proposal 1: Allow non-colocated deployment for intra-band con-contiguous carrier aggregation for FR1 in realse-16.

When it comes to the actual MRTD and MTTD values we need a UE reference architecture to start from in order to anchor requirement analysis. When non-contiguous carrier aggregation was introduced, for the first time, the first step was to agree a reasonable reference UE architecture. The reference architecture as not mandatory, but, if used, then fulfilment of the non-contiguous carrier aggregation requirement set was ensured. In TR 36.823, the UE reference architecture was [3, Figure 6.2.3.1-1, 6.2.2.1-1]:

Figure 1: Reference receiver architecture

Figure 2: Reference transmitter architecture

Figure 1 and Figure 2 show that a reasonable assumption for an intra-band non-contiguous carrier aggregation is to have the same FR1 MRTD and MTTD as inter-band carrier aggregation, since each band is managed separately.

Proposal 2: MRTD and MTTD for intra-band non-contiguous carrier aggregation in FR1, are the same MRTD and MTTD for inter-band carrier aggregation in FR1.

# 3	Summary

Proposal 1: Allow non-colocated deployment for intra-band con-contiguous carrier aggregation for FR1 in realse-16.

Proposal 2: MRTD and MTTD for intra-band non-contiguous carrier aggregation in FR1, are the same MRTD and MTTD for inter-band carrier aggregation in FR1.

# References

[1]	R4-2015478, Discussion on MRTD/MTTD requirements for FR1 intra-band NCCA, RAN4#97-e, Huawei

[2]  	TS 38.101-1, User Equipment (UE) radio transmission and reception. Part 1: Range 1 Standalone, V16.5.0

[3]  	TR 36.823,  Carrier aggregation enhancements (Release 11) V11.0.1.