3GPP TSG-RAN WG4 Meeting #98-e	R4-2101913

Electronic Meeting, 25 January – 5 February 2021

Source:	Ericsson, Rogers

Title:	TP for TR 37.717-21-11: Including band combinations DC_12-66_n41

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.717-21-11 to add DC_12A-66A_n41A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_12-66_n41

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 12-66_n41 | CA_12-66 | n41 |  |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_12A-66A_n41A | DC_12A_n41A DC_66A_n41A | CA_12A-66A | n41A |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_12-66_n41, the TIB,c and RIB,c values are reused from DC_66_n41-n71 and are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-66_n41 | 12 | 0.6 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-66_n41 | 12 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

### 5.1.x.4	REFSENS requirements

There is IMD2 impact from UL 66_n41 affecting DL band 12. The MSD value is derived from DC_7A-13A_n66A.

The IMD3 impact from UL 66_n41 affecting DL band 12 is too close to the band edges to allocate test points and can therefore be disregarded.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A-66A_n41A | 12 | 712 | 5 | 25 | 742 | 31 | IMD2 |
|  | 66 | 1773 | 5 | 25 | 2173 | N/A | N/A |
|  | n41 | 2515 | 5 | 25 | 2515 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-202613, Dual Connectivity (DC) of 2 bands LTE inter-band CA  (2DL/1UL) and 1 NR band (1DL/1UL), Huawei, HiSilicon