3GPP TSG-RAN WG4 Meeting #98-e														R4-2101931

Electronic meeting, January 25th – 2nd February 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	On BS RF aspects for the lower 6GHz NR unlicensed operation

Agenda item:	9.35.3

Document for:	Approval

# Introduction

During RAN#89-e a WID was approved for a lower 6GHz NR unlicensed operation intended for deployment in Europe. The start of the WID was delayed pending administrations in Europe have had unlicensed operation in the range 5925 to 6425 MHz for consultation. The result of this consultation is that at the November 2020 meeting the ECC with ECC Decision (20)01 “on the harmonized use of the frequency bands 5945 to 6425 MHz for the implementation of Wireless Access Systems including Radio Local Area Networks (WAS/RLANs)” approved unlicensed operation in the range 5945 MHz to 6425 MHz. Given the ECC decision the work was at RAN#90 approved to start in RAN4#98 [1].

NR-Unlicensed operation is standardized in Rel-16 [3] with the definition of band n96 covering the spectrum range 5925 MHz -7125 MHz which is currently applicable in the USA only subject to FCC Report and Order FCC 20-51. The ECC decision covers the range 5945 MHz to 6425 MHz which is therefor a sub-set of the already defined band n96.

In this contribution, we discuss how the frequency range 5945 MHz to 6425 MHz can be enabled for unlicensed operation.

# Discussion

The objectives for the core part from the WID [1] relevant for the BS are given as:

- Depending on the details of the European regulatory requirements, determine whether they are best handled by relevant updates (if any) of band n96 or whether a new band is needed.

  - If a new band is needed, determine the band plan for unlicensed operation in the range 5945-6425 MHz

- Define or update (if needed) system parameters such as channel bandwidths and channel arrangements

- Define or update (if needed) transmitter and receiver characteristics requirements for the UE

These objectives of the WID are discussed further in the following sections.

#### Is it needed to define or update system parameters?

When defining system parameters as channel bandwidths, channel and synchronization raster for band n96 great care was taken to ensure alignment to other technologies intended deployed in the same unlicensed spectrum. The same should apply to the overlapping frequency range now adopted for unlicensed operation in Europe.

Observation 1: The same efforts for channel arrangement alignment to other technologies intended deployed in the frequency range as used for the design of n96 should be applied.

The raster design used for band n96 follows the approach, also adopted earlier for band n46, which is aligned to the Harmonised Standard for access to radio spectrum in the 6 GHz range [2]. This implicitly means alignment to e.g. WiFi channel arrangements.

The difference from band n96, intended for US deployments, and the frequency range included in this work is that band n96 starts at 5925 MHz while the EU range starts at 5945 MHz and that the US range goes to 7125 MHz while the EU range stops at 6425 MHz. Even though the starting frequency is different for the US and EU ranges the channel and synchronization raster are still aligned since the 20 MHz shift have already been accounted for in the design of band n96. The main difference is that for the US range will have additional available raster points corresponding to the upper range of band n96 (i.e. 6425 MHz – 7125 MHz).

Based on the previous there should be no need to modify or add additional channel bandwidths, channel or synchronization raster points as already defined for band n96.

Observation 2: There is no need to modify or add additional channel bandwidths, channel or synchronization raster points as already defined for band n96.

Restricting the use of the channels defined for n96 above 6425 MHz to only allow the lower range for operation in Europe can be done by modifying the notes in TS 38.104 Table 5.4.2.3-1 Table 5.4.3.3-1 or simply to relay on the fact that BS operation in shared spectrum is subject to regional requirements as specified in section 4.5 of 38.104.

Observation 3: The European utilization of band n96 could be restricted done by modifying the notes in 38.104 Table 5.4.2.3-1 Table 5.4.3.3-1.

#### Is it needed to define or update transmitter and receiver characteristics requirements?

In the Harmonised Standard for access to radio spectrum in the 6 GHz range [2] slightly different deployments definitions have been used as compared to the FCC Report and Order FCC 20-51. This means the allowed transmit power is different dependent on the region. This have however already been addressed in TS 38.104 Section 6.2 Base station output power, were the following is stated:

In addition, for operation with shared spectrum channel access operation, the BS may have to comply with the applicable BS power limits established regionally, when deployed in regions where those limits apply and under the conditions declared by the manufacturer.

The above statement means the operator of a BS are to ensure conformance to regional requirements for allowed outpower before deployment of the BS. Hence, there is no need for specific output power requirements for the frequency range 5945 MHz to 6425 MHz for unlicensed operation in Europe

Observation 4: There is no need to modify or add additional requirements for output power besides the ones already defined for band n96.

For unwanted emission requirements band n96 follows the Harmonised Standard for access to radio spectrum in the 6 GHz range which means the European requirements already fulfilled.

Observation 5: There is no need to modify or add additional unwanted emission requirements besides the ones already defined for band n96.

The receiver spurious emissions basic limits for n96 already defined in TS 38.104 section 7.6.2 is fulfilling the requirements of  the Harmonised Standard for access to radio spectrum in the 6 GHz [2].

Observation 6: There is no need to modify or add receiver requirements besides the ones already defined for band n96.

#### New band or reuse n96?

Based on the previous sections it seems the most straight forward approach for fulfilling the objectives of the WID is to introduce unlicensed operation in the European range 5945 MHz - 6425 MHz by utilizing the already defined band n96.

A limiting factor is Note 4 now included in Table 5.2-1 of TS 38.104 stating that This band is applicable in the USA only subject to FCC Report and Order FCC 20-51. Perhaps Note 3, already defined, is more fitting which stats that This band is restricted to operation with shared spectrum channel access as defined in 37.213. It should be emphasised that reusing n96 for European unlicensed operation does not preclude defining a new band either fully or partially overlapping n96 for normal NR operation.

Observation 7: If band n96 is to be reused Note 4 in Table 5.2-1 of TS 38.104 shall be modified.

In concussion, it is proposed to proceed with enabling the frequency range 5945 MHz to 6425 MHz for unlicensed operation in Europe by utilizing band n96 already defined.

Proposal 1: Introduce the 5945 MHz to 6425 MHz frequency range for unlicensed operation in Europe using  already defined band n96 with relevant modifications.

# Conclusion

In this contribution, we discuss the 6 GHz band for unlicensed operation in Europe. We have made following observations and proposals:

Observation 1: The same efforts for channel arrangement alignment to other technologies intended deployed in the frequency range as used for the design of n96 should be applied.

Observation 2: There is no need to modify or add additional channel bandwidths, channel or synchronization raster points as already defined for band n96.

Observation 3: The European utilization of band n96 could be restricted done by modifying the notes in 38.104 Table 5.4.2.3-1 Table 5.4.3.3-1.

Observation 4: There is no need to modify or add additional requirements for output power besides the ones already defined for band n96.

Observation 5: There is no need to modify or add additional unwanted emission requirements besides the ones already defined for band n96.

Observation 6: There is no need to modify or add receiver requirements besides the ones already defined for band n96.

Observation 7: If band n96 is to be reused Note 4 in Table 5.2-1 of TS 38.104 shall be modified.

Proposal 1: Introduce the 5945 MHz to 6425 MHz frequency range for unlicensed operation in Europe using  already defined band n96 with relevant modifications.

# References

- RP-202592, WID on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia, Nokia Shanghai Bell

- Draft EN 303 687 V0.0.11, ETSI BRAN

- RP-200312 Revised WID on NR-based Access to Unlicensed Spectrum, Ericsson