3GPP TSG-RAN WG4 Meeting #98-e				  											R4-2101934 
E-meeting, 21st Jan – 5th Feb, 2021

Source:				Nokia, Nokia Shanghai Bell

Title:					NTN - HAPS simulation assumptions for co-existence study

Agenda item: 	11.8.3.1

Document for:	Discussion and Approval

# Introduction

In the NTN WID [1], RAN4 objectives are defined as follows.

### 4.1.4 RAN4 Study the framework how NTN core requirements are defined. Specify the following requirements [RAN4] (Note 1) - UE RRM core requirements - Study and identify which bands may be potentially relevant to NTN including: - Analysis of regulations in the spectrum considered - Adjacent channel co-existence - Considering the potential bands to be used as example for the WID: - Specify needed generic RF core requirements for the network and the UE such that adjacent channel co-existence scenarios are met and performance of other RF parameters (RX performance, TX signal quality etc.) are subject to acceptable minimum requirements - Investigate and specify UE timing & frequency pre compensation accuracy requirements as needed [RAN4]. Note 1: It is assumed that this work item will be frequency agnostic and therefore we can consider that NTN can operate in FR1 or FR2 ranges. Defining NR bands for NTN should be included as part of dedicated Rel-17 RAN4 led work items including an analysis of regulations in spectrum considered, which bands 3GPP should specify, as well as potential co-existence between NR terrestrial and satellite

With respect to HAPS, the WID added this description in RAN#90-e:

In the context of this work item, HAPS refers to a high altitude platform system for which at least the service link (HAPS – UE) operates a 3GPP specified NR mobile service in allocated spectrum which regulation allows. If needed, the terminology “HAPS” may be revisited.

Email discussions during RAN#90-e also concluded that [2]

“ Proposal 5: The scope and work load associated to adjacent channel co-existence study(ies) between HAPS and TN in existing 3GPP band(s) allowed by regulation for HAPS as IMT BS operation shall be clarified before being considered to be part of the Rel-17 NR-NTN-solutions WI.”

Furthermore, the following agreements were made in the last RAN4 meeting [3]:

- 3GPP RAN4 should provide/conduct relative independent adjacent channel coexistence studies to develop RF requirements for NTN.

- NTN RF requirements shall be specified assuming no impact on TN RF requirements.

- For coexistence studied, both NTN/NTN and NTN/TN in adjacent channels should be considered.

- RAN4 need to consider how to take into account the heterogeneous cell patterns of NTN and TN networks assuming that they serve the same areas.

- Use TR 38.821 as a baseline/starting point, as long as HAPS is still included, and NTN study aligns with WID in RAN.

- Further discuss if and which HAPS scenarios should be considered by RAN4 as part of the WI NR-NTN-solutions.

From standardization perspective, HAPS operational scenario is not as challenging as LEO and GEO satellites due to its shorter propagation path and lower mobility. Our view about HAPS adjacent channel co-existence is presented in [4].

In this contribution, we further discuss HAPS simulation scenarios and assumptions.

# Discussion

## Simulation methodology

Co-existence between HAPS and terrestrial mobile networks (TN) can be studied based on system level simulations following similar methodology in [5]. The impact of co-channel interference from the aggressor network is examined in terms of the throughput loss in the victim network. We propose reusing the same 5% throughput loss criteria as in [5] to determine the adjacent channel requirements. Since HAPS is expected to support the same UE devices used for terrestrial networks, there should be no impact to the UE, and the existing UE adjacent channel requirements apply to the HAPS co-existence scenarios. The ACLR and ACS requirements for HAPS should therefore be derived from the 5% throughput loss and the existing ACLR and ACS parameters for the UEs. The ACLR value is derived when HAPS is an aggressor. The ACS value is derived when HAPS is a victim.

Proposal 1: 	Existing UE adjacent channel requirements apply to HAPS and TN co-existence scenarios.

Proposal 2: 	Determine the adjacent channel requirements for HAPS based on a 5% throughput degradation in the victim network in HAPS and TN co-existence scenarios.

As the co existence studies should focus on the worst case scenario we propose to use frequency reuse factor 1.

Proposal 3:	RAN4 to use frequency reuse factor 1 for co-existence studies.

## Co-existence scenarios

HAPS may cover a large area with multiple cells. Study of adjacent channel interference (ACI) between HAPS and TN needs to consider the separation distance between the two networks. A cellular layout of 7 sites, 21 cells with a certain ISD may be used to model TN. This scenario is illustrated in Figure 1, where HAPS is operating near a well-covered cellular network. The topology of the scenario can be characterized by these parameters:

- HAPS coverage radius

- HAPS number of cells

- HAPS and TN separation distance (2D, center-to-center)

- TN number of cells

- TN inter-site distance

Another scenario worth considering is when HAPS coverage overlaps spotty TN cells, which are typically seen near remote villages or towns. In that case, the isolated TN coverage is noise limited and ACI from HAPS may have a greater impact. A simple topology of this scenario is shown in Figure 2. For the co-existence study, we should consider the scenarios of both densely deployed TN and sparsely deployed TN.

Proposal 4: 	Consider both densely deployed TN and sparsely deployed TN in HAPS-TN co-existence scenarios.

Since HAPS may operate in the terrestrial bands of paired and unpaired spectrum, both FDD and TDD need to be simulated. In the TDD mode, HAPS and TN are most likely unsynchronized, so HAPS downlink transmission may inflict TN uplink reception and TN downlink transmission may impact HAPS uplink reception.

Proposal 5: 	Adjacent channel interference in co-existence scenarios should be studied for both FDD and TDD.

Figure 1.  HAPS and TN co-existence scenario 1

Figure 2.  HAPS and TN co-existence scenario 2

## Simulation assumptions

HAPS antenna design is a key element that impacts simulation results and must be included in the simulation assumptions. A reference design proposed for HIBS (HAPS as IMT base stations) study in ITU WP-5D [6] is shown in Figure 3. The antenna array is composed of seven antenna panels (six side panels and one downward facing panel). Antenna elements on each panel are co-phased to form one beam in two linear polarizations to serve one cell. There are a total of seven cells in two layers, one cell in the 1st layer and six cells in the 2nd layer. Some parameters of the reference array are listed in Table 1 for information.

Proposal 6: 	Adopt a reference HAPS antenna array for adjacent channel co-existence simulations.

| (a) | (b) |
| --- | --- |

Figure 3. Example of HAPS antenna array and cell layout

Table 1. Example HAPS antenna parameters for IMT bands below 2.7 GHz

| Number of cells | 7 |
| --- | --- |
| Antenna array configuration (row x column) | 2 x 2 for 1st layer cell 4 x 2 for 2nd layer cell |
| Antenna polarization | Linear ±45° |
| Element gain | 8 dBi |
| Element HPBW horizontal/vertical | 65° for both H/V |
| Element front-to-back ratio horizontal/vertical | 30 dB for both H/V |
| Element spacing horizontal/vertical | 0.5 wavelength for both H/V |
| Antenna panel tilt | 90° for 1st layer cell 23° for 2nd layer cell |
| Tx power per antenna panel | 46 dBm |

Other simulation assumptions depend on the frequency band and terrestrial network environment. Table 2 lists examples of simulation assumptions for Band n1 in Urban Macro and Rural Macro terrestrial environments. The TN network topology may include both the 7-site scenario of Figure 1 and the isolated 1-site scenario of Figure 2. Different separation distances between TN and HAPS may need to be simulated since the antenna gains of HAPS and TN in the interfering links vary with the distance dependent elevation angles. While a proper mix of indoor/outdoor UEs typical of the terrestrial environment should be used for TN, we may assume that the UEs connected to HAPS are all outdoor UEs to minimize the outage rate in the simulations. Details of the simulation assumptions should be determined considering the selected scenarios and the terrestrial bands for HAPS deployment.

Proposal 7: 	Determine simulation assumptions based on the HAPS co-existence scenarios and the terrestrial bands for HAPS deployment.

Table 2. Example system level simulation parameters for rural scenario

| Frequency band | Band n1 (UL: 1920-1980 MHz, DL: 2110-2170 MHz) |  |
| --- | --- | --- |
| Terrestrial environment | Urban Macro | Rural Macro |
| Channel bandwidth | 20 MHz | 20 MHz |
| HAPS altitude | 20 Km | 20 Km |
| HAPS coverage radius | 100 Km | 100 Km |
| HAPS and TN coverage center distance | 0, 20, 40 Km | 0, 20, 40 Km |
| HAPS channel model | NTN urban [7] | NTN rural [7] |
| HAPS indoor UE percentage | 0% | 0% |
| TN inter-site distance | 1 Km | 2 Km |
| TN BS antenna height | 25 m | 35 m |
| TN BS transmit power | 46 dBm | 46 dBm |
| TN BS antenna array configuration | 8 x 8 x 2 (M=8, N=8, P=2) 0.5 wavelength spacing | 8 x 1 x 2 (M=8, N=1, P=2) 0.5 wavelength spacing |
| TN BS antenna downtilt | 10⁰ | 6⁰ |
| TN channel model | UMa model [8] | RMa model [8] |
| TN BS element gain | 8 dBi | 8 dBi |
| TN BS element HPBW horizontal/vertical | 65⁰ for H/V | 65⁰ for H/V |
| TN BS element radiation pattern | As in Table 7.3-1 of [8] | As in Table 7.3-1 of [8] |
| TN indoor UE percentage | 70% | 50% |
| Body loss | 4 dB | 4 dB |
| UE antenna gain | -3 dBi | -3 dBi |
| UE antenna array | 1 x 1 x 2 (M=1, N=1, P=2) | 1 x 1 x 2 (M=1, N=1, P=2) |
| UE transmit power | 23 dBm | 23 dBm |

# Conclusion

We have discussed simulation methodology, scenarios, and assumptions for HAPS adjacent channel co-existence study. Our view can be summarized in the following proposals.

Proposal 1:	Existing UE adjacent channel requirements apply to HAPS and TN co-existence scenarios.

Proposal 2: 	Determine the adjacent channel requirements for HAPS based on a 5% throughput degradation in the victim network in HAPS and TN co-existence scenarios.

Proposal 3:	RAN4 to use frequency reuse factor 1 for co-existence studies.

Proposal 4: 	Consider both densely deployed TN and sparsely deployed TN in HAPS-TN co-existence scenarios.

Proposal 5: 	Adjacent channel interference in co-existence scenarios should be studied for both FDD and TDD.

Proposal 6: 	Adopt a reference HAPS antenna array for adjacent channel co-existence simulations.

Proposal 7: 	Determine simulation assumptions based on the HAPS co-existence scenarios and the terrestrial bands for HAPS deployment.

# References

- RP-202908, Thales, “Solutions for NR to support non-terrestrial networks (NTN)”, 3GPP TSG RAN meeting #90-e, December 7-11, 2020.

- RP-202907, Thales (Email discussion moderator), “Moderator's summary of email discussion   [90E][27][R17_NTN_bands&scope],” 3GPP TSG RAN meeting #90-e, December 7-11, 2020.

- R4-2017600, “WF on NTN solutions”, 3GPP TSG-RAN WG4 meeting #97-e, November 2-13, 2020.

- R4-2101935, NTN - HAPS adjacent channel coexistence, Nokia

- 3GPP TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects.”

- SoftBank, Loon LLC, Nokia, Ericsson, “Proposed deployment and system characteristics of HIBS in the working document towards a preliminary draft new Report ITU-R M.[HIBS-CHARACTERISTICS],” ITU WP-5D contribution, Sep. 28, 2020.

- 3GPP TR 38.811, “Study on New Radio (NR) to support non-terrestrial networks.”

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz.”