3GPP TSG-RAN WG4 Meeting#98-e                                 R4-2101950

E-meeting,25th Jan – 5th Feb, 2021

Agenda item:	10.1.3

Source: 	ZTE Corporation

Title: 	TP to TR 38.921 summary of simulation results

Document for:	Approval

Introduction

Since RAN4#98e meeting should be last meeting for this ITU-R related WID based the latest status report RP-202275 . Until this meeting, extensive simulation results among interested companies have already been submitted, therefore this TP is to capture the summary of simulation results in the TR.

Discussion

TP to TR 38.921

< START OF CHANGE>

4.3	Co-existence simulation results

### 4.3.1	Urban Macro scenario

#### 4.3.1.1	Downlink

For Urban macro scenarios, simulation results for DL throughput loss with baseline assumption ACLR=45dBc at BS and ACS=33dBc at UE are summarized in Table 4.3.1.1-1/2.

Table 4.3.1.1-1: DL throughput loss of victim UE for 6425-7125MHz

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-2016134 | AAS based BS | Average throughput loss in % (7GHz) | 1.65 | 1.89 | 2.10 | 2.33 | 2.63 | 2.99 |
|  |  | 5%-tile throughput loss in % (7GHz) | 4.39 | 5.18 | 5.98 | 7.07 | 7.87 | 9.06 |
| Nokia R4-2014476 | AAS based BS | Average throughput loss in % (7GHz) | 1.11 | 1.28 | 1.48 | 1.70 | 1.94 | 2.21 |
|  |  | 5%-tile throughput loss in % (7GHz) | 6.22 | 6.23 | 6.24 | 7.51 | 8.99 | 9.23 |
| Huawei R4-2015678 | AAS based BS | Average throughput loss in % (7GHz) | 1.04 | 1.39 | 1.47 | 1.77 | 2.30 | 2.39 |
|  |  | 5%-tile throughput loss in % (7GHz) | 3.00 | 4.63 | 4.96 | 6.37 | 6.78 | 7.16 |

Table 4.3.1.1-1a: DL throughput loss of victim UE for 6425-7125MHz

|  | ACIR (dB) | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson R4-2015897 | Throughput lost at Average | 2.7 | 2.4 | 2.1 | 1.8 | 1.6 | 1.4 | 1.2 | 1.0 | 0.9 | 0.8 | 0.7 |
|  | Throughput lost at 5%-tile | 13.1 | 10.6 | 10.0 | 8.3 | 6.6 | 6.1 | 5.5 | 4.7 | 3.1 | 2.2 | 2.1 |
| CATT R4-2016777 | Throughput lost at Average | 2.945446 | 2.615785 | 2.318919 | 2.050108 | 1.808206 | 1.592518 | 1.399160 | 1.226961 | 1.073971 | 0.936971 | 0.814444 |
|  | Throughput lost at 5%-tile | 11.345302 | 10.038643 | 8.745891 | 7.593149 | 6.594121 | 5.813514 | 5.023313 | 4.235969 | 3.589495 | 3.063706 | 2.576903 |
| Qualcomm R4-2016236 | Throughput lost at Average |  |  | 3.0285 | 2.6700 | 2.3630 | 2.0876 | 1.8414 | 1.6318 | 1.4200 |  |  |
|  | Throughput lost at 5%-tile |  |  | 12.3347 | 10.6516 | 9.2041 | 7.9086 | 6.7639 | 5.8122 | 4.9061 |  |  |

Table 4.3.1.1-2: DL throughput loss of victim UE for 10-10.5GHz

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-2016134 | AAS based BS | Average throughput loss in % (7GHz) | 1.52 | 1.68 | 1.93 | 2.12 | 2.31 | 2.62 |
|  |  | 5%-tile throughput loss in % (7GHz) | 3.36 | 3.79 | 4.79 | 5.78 | 6.41 | 7.63 |
| Nokia R4-2014476 | AAS based BS | Average throughput loss in % (7GHz) | 1.03 | 1.20 | 1.38 | 1.60 | 1.83 | 2.09 |
|  |  | 5%-tile throughput loss in % (7GHz) | 4.38 | 4.38 | 4.86% | 5.23 | 6.01 | 7.40 |
| Huawei R4-2015678 | AAS based BS | Average throughput loss in % (7GHz) | 1.21 | 1.22 | 1.30% | 1.72 | 1.99 | 2.44 |
|  |  | 5%-tile throughput loss in % (7GHz) | 3.04 | 3.33 | 4.14% | 5.06 | 6.41 | 8.67 |

Table 4.3.1.1-2a: DL throughput loss of victim UE for 10-10.5GHz

|  | ACIR (dB) | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson R4-2015897 | Throughput lost at Average | 2.1 | 1.9 | 1.7 | 1.4 | 1.3 | 1.1 | 0.9 | 0.8 | 0.7 | 0.6 | 0.5 |
|  | Throughput lost at 5%-tile | 11.8 | 10.0 | 9.6 | 9.4 | 7.8 | 6.3 | 4.8 | 4.7 | 4.2 | 3.9 | 3.5 |
| CATT R4-2016777 | Throughput lost at Average | 2.874567 | 2.547846 | 2.252212 | 1.986030 | 1.748135 | 1.537007 | 1.348903 | 1.181463 | 1.032390 | 0.899829 | 0.782843 |
|  | Throughput lost at 5%-tile | 9.975828 | 8.600968 | 7.468747 | 6.454207 | 5.566782 | 4.750499 | 4.068893 | 3.494199 | 2.988947 | 2.552896 | 2.076688 |
| Qualcomm R4-2016236 | Throughput lost at Average |  |  | 2.788 | 2.4497 | 2.1611 | 1.9031 | 1.6734 | 1.4787 | 1.2825 |  |  |
|  | Throughput lost at 5%-tile |  |  | 11.0936 | 9.6118 | 8.3690 | 7.2691 | 6.2895 | 5.4504 | 4.5726 |  |  |

#### 4.3.1.2	Uplink

For Urban macro scenarios, simulation results for UL throughput loss with baseline assumption ACLR=30dBc at UE and ACS=46dBc at BS are summarized in Table 4.3.1.2-1/2.

Table 4.3.1.2-1: UL throughput loss of victim BS for 6425-7125MHz

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-2016135 | AAS based BS | Average throughput loss in % (7GHz) | 0.99 | 1.09 | 1.23 | 1.40 | 1.55 | 1.76 |
|  |  | 5%-tile throughput loss in % (7GHz) | 2.72 | 2.84 | 5.28 | 6.01 | 6.71 | 7.20 |
| Ericsson R4-2015898 | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Nokia R4-2014477 | AAS based BS | Average throughput loss in % (7GHz) | 1.08% | 1.24% | 1.43% | 1.65% | 1.89% | 2.15% |
|  |  | 5%-tile throughput loss in % (7GHz) | 1.01% | 1.68% | 3.14% | 3.98% | 5.76% | 8.08% |
| Huawei R4-2015679 | AAS based BS | Average throughput loss in % (7GHz) | 0.64% | 0.69% | 0.69% | 0.92% | 1.01% | 1.07% |
|  |  | 5%-tile throughput loss in % (7GHz) | 3.8% | 4.3% | 5.0% | 6.2% | 7.6% | 8.7% |
| CATT R4-2016778 | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Qualcomm R4-2016601 | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |

Table 4.3.1.2-1a: UL throughput loss of victim BS for 6425-7125MHz

|  | ACIR (dB) | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson R4-2015898 | Throughput lost at Average | 1.5 | 1.3 | 1.1 | 1.0 | 0.8 | 0.7 | 0.6 | 0.5 | 0.5 | 0.4 | 0.3 |
|  | Throughput lost at 5%-tile | 6.3 | 6.2 | 5.5 | 4.3 | 2.3 | 1.6 | 1.5 | 1.4 | 0.8 | 0.6 | 0.5 |
| CATT R4-2016778 | Throughput lost at Average | 1.489821 | 1.301164 | 1.134213 | 0.986689 | 0.856389 | 0.741773 | 0.641199 | 0.553419 | 0.476770 | 0.409968 | 0.351849 |
|  | Throughput lost at 5%-tile | 7.283069 | 6.142583 | 5.227730 | 4.495608 | 3.745409 | 3.051767 | 2.429319 | 1.621495 | 0.968716 | 0.671695 | 0.526234 |
| Qualcomm R4-2016601 | Throughput lost at Average | 1.5 | 1.2 | 1.0 | 0.9 | 1.0 | 0.9 | 0.8 | 0.6 | 0.5 |  |  |
|  | Throughput lost at 5%-tile | 4.3 | 3.4 | 2.4 | 2.2 | 2.4 | 2.2 | 1.8 | 1.5 | 1.3 |  |  |

Table 4.3.1.2-2: UL throughput loss of victim BS for 10-10.5GHz

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-2016135 | AAS based BS | Average throughput loss in % (7GHz) | 0.79 | 0.87 | 1.11 | 1.23 | 1.41 | 1.59 |
|  |  | 5%-tile throughput loss in % (7GHz) | N/A | N/A | N/A | N/A | N/A | N/A |
| Ericsson R4-2015898 | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Nokia R4-2014477 | AAS based BS | Average throughput loss in % (7GHz) | 0.90% | 1.04% | 1.21% | 1.39% | 1.60% | 1.84% |
|  |  | 5%-tile throughput loss in % (7GHz) | 1.98% | 2.27% | 2.28% | 3.90% | 4.00% | 6.41% |
| Huawei R4-2015679 | AAS based BS | Average throughput loss in % (7GHz) | 0.53% | 0.59% | 0.61% | 0.76% | 0.91% | 1.05% |
|  |  | 5%-tile throughput loss in % (7GHz) | 1.7% | 2.3% | 3.3% | 4.0% | 5.3% | 6.4% |
| CATT R4-2016778 | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Qualcomm R4-2016601 | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |

|  | ACIR (dB) | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson R4-2015898 | Throughput lost at Average | 1.3 | 1.1 | 1.0 | 0.8 | 0.7 | 0.6 | 0.5 | 0.5 | 0.4 | 0.3 | 0.3 |
|  | Throughput lost at 5%-tile | 3.0 | 2.3 | 1.3 | 1.0 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.2 |
| CATT R4-2016778 | Throughput lost at Average | 1.535229 | 1.345764 | 1.177367 | 1.027196 | 0.894277 | 0.777219 | 0.673935 | 0.582684 | 0.502769 | 0.432740 | 0.371237 |
|  | Throughput lost at 5%-tile | 9.496269 | 7.816610 | 6.444894 | 5.273296 | 4.180206 | 3.433665 | 3.082907 | 2.691289 | 2.325946 | 2.024768 | 1.625941 |
| Qualcomm R4-2016601 | Throughput lost at Average | 1.5 | 1.2 | 1.0 | 0.9 | 0.8 | 0.6 | 0.5 | 0.4 |  |  |  |
|  | Throughput lost at 5%-tile | 4.0 | 3.5 | 2.9 | 2.6 | 2.2 | 2.0 | 1.6 | 1.4 |  |  |  |

Table 4.3.1.2-2a: UL throughput loss of victim BS for 10-10.5GHz

### 4.3.2	Indoor scenario

#### 4.3.2.1	Downlink

For indoor scenarios, simulation results for DL throughput loss with baseline assumption ACLR=45dBc at BS and ACS=33dBc at UE are summarized in Table 4.3.2.1-1/1a/2/2a.

Table 4.3.2.1-1: DL throughput loss of victim UE for 6425-7125MHz

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) | 0.1371 | 0.1724 | 0.2168 | 0.2726 | 0.3425 | 0.4302 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.1429 | 0.1798 | 0.2263 | 0.2847 | 0.3123 | 0.3427 |
| Ericsson R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |

Table 4.3.2.1-1a: DL throughput loss of victim UE for 6425-7125MHz

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) | 0.3043 | 0.3817 | 0.4775 | 0.5947 | 0.7378 | 0.9123 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.0054 | 0.0067 | 0.0085 | 0.0107 | 0.0135 | 0.1421 |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |

Table 4.3.2.1-2: DL throughput loss of victim UE for 10-10.5GHz

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) | 0.1561 | 0.1956 | 0.2447 | 0.3058 | 0.3815 | 0.4750 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.1096 | 0.1380 | 0.1736 | 0.2369 | 0.3323 | 0.4823 |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |

Table 4.3.2.1-2a: DL throughput loss of victim UE for 10-10.5GHz

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) | 0.3188 | 0.3979 | 0.4950 | 0.6133 | 0.7587 | 0.9337 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.0481 | 0.0553 | 0.0645 | 0.0760 | 0.0969 | 0.1978 |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |

#### 4.3.2.2	Uplink

For indoor scenarios, simulation results for UL throughput loss with baseline assumption ACLR=30dBc at UE and ACS=46dBc at BS are summarized in Table 4.3.2.2-1/1a/2/2a.

Table 4.3.2.2-1: UL throughput loss of victim BS for 6425-7125MHz

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) | 0.1433 | 0.1794 | 0.2245 | 0.2805 | 0.3499 | 0.4356 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.0078 | 0.0098 | 0.0123 | 0.0155 | 0.0212 | 0.0246 |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |

Table 4.3.2.2-1a: UL throughput loss of victim BS for 6425-7125MHz

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) | 0.1414 | 0.1770 | 0.2211 | 0.2759 | 0.3437 | 0.4275 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.0033 | 0.0042 | 0.0053 | 0.0378 | 0.0880 | 0.1511 |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |

Table 4.3.2.2-2: UL throughput loss of victim BS for 10-10.5GHz

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) | 0.1510 | 0.1892 | 0.2367 | 0.2957 | 0.3689 | 0.4593 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.0214 | 0.0347 | 0.0362 | 0.0382 | 0.0407 | 0.1147 |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | Omni based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |

Table 4.3.2.2-2a: UL throughput loss of victim BS for 10-10.5GHz

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) | 0.1536 | 0.1920 | 0.2395 | 0.2983 | 0.3709 | 0.4602 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.0900 | 0.0904 | 0.0910 | 0.0918 | 0.0927 | 0.0938 |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |
| Company R4-20xxx | AAS based BS | Average throughput loss in % (7GHz) |  |  |  |  |  |  |
|  |  | 5%-tile throughput loss in % (7GHz) |  |  |  |  |  |  |

< End OF CHANGE>