3GPP TSG-RAN WG4 Meeting#98-e                                R4-2101954

E-meeting,25th Jan – 5th Feb, 2021

Agenda item:	10.1.4

Source: 	ZTE Corporation

Title: 	TP to TR 38.921 Antenna configurations

Document for:	Approval

Introduction

In the RAN4#97e meeting, simulation assumption for small cell indoor scenarios is agreed, this TP is to capture the antenna configuration for small indoor scenario.

TP to TR 38.921

< START OF CHANGE>

### 6.1.5	Maximum output power

The maximum output power will be provided in the antenna parameter table. It was agreed to be aligned with antenna characteristics.

The Total Radiated Power for two polarizations was agreed as shown in table 6.1.5-1 below.

Table 6.1.5-1: The total Radiated power

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban | Indoor |
| --- | --- | --- | --- | --- |
| Total Radiated Power for two polarizations (dBm) | 46 | 46 | 37 | 24 |

< Next of CHANGE>

### 8.1.2		Array antenna parameters

In Table 8.1.2-1, base station array antenna parameters for different deployment scenarios is listed. Element parameters have been selected to produce correct element peak gain determined by calculating the directivity from a given geometry including beam widths. The element directivity can be calculated based on the pattern described by Table 8.1.2-1 in dBi as:

GE,max=DE,max-LE	(8.1.2-1)

, where the peak directivity DE,max is calculated from given values on 3dB, 3dB, dh and dv as:

DE,max=10log104πAlinθ,φmax-ππ0πAlinθ,φsinθdθdφ	 (8.1.2-2)

, where Alin(,) is defined in linear scale as:

Alinθ,φ=10Aθ,φ10	(8.1.2-3)

Table 8.1.2-1: BS array antenna parameters

| Parameter | Small cell Indoor (Note 7) | Macro Sub-urban (Note 4) | Macro Urban (Note 4) | Micro Urban (Note 5) |
| --- | --- | --- | --- | --- |
| Am (dB) | 30 | 30 | 30 | 30 |
| SLAv (dB) | 30 | 30 | 30 | 30 |
| 3dB (deg.) | 90 | 90 | 90 | 90 |
| 3dB (deg.) | 90 | 65 | 90 | 90 |
| GE,max (dBi) | 5.5 | 6.4 | 5.5 | 5.5 |
| LE (dB) | 2.0 | 2.0 | 2.0 | 2.0 |
| (M, N) | (4, 4) | (16, 8) | (16, 8) | (8,8) |
| dh (m) | 0.5l | 0.5 | 0.5 | 0.5 |
| dv (m) | 0.5l | 0.7 | 0.5 | 0.5 |
| Horizontal coverage range (deg.) | N/A | +/- 60 | +/- 60 | +/- 60 |
| Vertical coverage range (deg.) | N/A | 90 to 100 | 90 to 120 | 90 to 120 |
| Conducted power (before ohmic loss) per antenna element (dBm) | 9 | 22 | 22 | 16 |
| Mechanical downtilt (deg.) | N/A (Note 6) | 6 | 10 | N/A |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: The vertical coverage range includes the mechanical downtilt. Note 4: The conducted power per element assumes 16x8x2 elements (i.e. power per H/V polarized element). Note 5: The conducted power per element assumes 8x8x2 elements (i.e. power per H/V polarized element). Note 6: Boresight direction is perpendicular to the ceiling. Note 7: For small cell indoor case, omni antenna could be also applicable. |  |  |  |  |

< End OF CHANGE>