3GPP TSG-RAN WG4 Meeting #98-e	R4-2101987

# E-meeting, 25nd Jan, 2021 – 5th Feb, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-1 | CR | 0242 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.145-1: introduction of 35MHz and 45MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_35MHz_45MHz_BW-Core |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | BS RF requirements for 35MHz and 45MHz is missing in the existing TS 37.145-1, therefore corresponding RF requirements should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Summary the change: Tx: output dynamics , ACLR requirements - Rx: REFSENS, dynamic range, receiver intermodulation, ICS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | BS conformance testing requirements for 35MHz and 45MHz is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.4.5.3, 7.2.5.4, 7.3.5.4, 7.7.5.1.1, 7.7.5.1.2, 7.8.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.104, 37.105 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.141-1/2, 37.145-1/2, TS 37.141 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

6.3.4.5.3	NR

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level in table 6.3.4.5.3-1.

Table 6.3.4.5.3-1: NR TAB connector total power dynamic range

| NR channel bandwidth (MHz) | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
|  | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 5 | 13.5 | 10 | N/A |
| 10 | 16.7 | 13.4 | 10 |
| 15 | 18.5 | 15.3 | 12.1 |
| 20 | 19.8 | 16.6 | 13.4 |
| 25 | 20.8 | 17.7 | 14.5 |
| 30 | 21.6 | 18.5 | 15.3 |
| 35 | 22.7 | 19.6 | 16.4 |
| 40 | 22.9 | 19.8 | 16.6 |
| 45 | 23.8 | 20.7 | 17.6 |
| 50 | 23.9 | 20.8 | 17.7 |
| 60 | N/A | 21.6 | 18.5 |
| 70 | N/A | 22.3 | 19.2 |
| 80 | N/A | 22.9 | 19.8 |
| 90 | N/A | 23.4 | 20.4 |
| 100 | N/A | 23.9 | 20.9 |

NOTE 1:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

NOTE 2:	Additional test requirements for the Error Vector Magnitude (EVM) at the lower limit of the dynamic range are defined in clause 6.5.4.

<Next of change>

6.6.3.5.3.1A	NR test requirement

For NR, the test requirement is specified in tables 6.6.3.5.3.1A-1 and applies outside the Base Station RF Bandwidth or Maximum Radio Bandwidth.

For a TAB connector operating in non-contiguous spectrum, the ACLR also applies for the first adjacent channel inside any sub-block gap with a gap size as indicated in table 6.6.3.5.3.1A-2. The ACLR requirement for the second adjacent channel applies inside any sub-block gap with a gap size as indicated in table 6.6.3.5.3.1A-2. The CACLR test requirement in clause 6.6.3.5.3.4 applies in sub block gaps for the frequency ranges defined in table 6.6.3.5.3.4-2.

For a multi-band TAB connector, the ACLR also applies for the first adjacent channel inside any Inter RF Bandwidth gap with a gap size as indicated in table 6.6.3.5.3.1A-2. The ACLR requirement for the second adjacent channel applies inside any Inter RF Bandwidth gap with a gap size as indicated in table 6.6.3.5.3.1A-2. The CACLR test requirement in clause 6.6.3.5.3.4 applies in sub block gaps for the frequency ranges defined in table 6.6.3.5.3.4-2.

The requirement applies during the transmitter on period.

The ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency and a filter centred on the adjacent channel frequency according to the tables below.

For operation in paired or unpaired spectrum, the ACLR shall be higher than the value specified in table 6.6.3.5.3.1A-1.

Table 6.6.3.5.3.1A-1: Base Station ACLR limit

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel [MHz] | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
|  | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44.2 dB |
| 5, 10, 15, 20 | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44.2 dB |
|  | BWChannel /2 + 2.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44.2 dB (NOTE 3) |
|  | BWChannel /2 + 7.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44.2 dB (NOTE 3) |
|  | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 43.8 dB |
| 25, 30, 35, 40, 45, 50, 60, 70, 80,90, 100 | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 43.8 dB |
|  | BWChannel /2 + 2.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 43.8 dB (NOTE 3) |
|  | BWChannel /2 + 7.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 43.8 dB (NOTE 3) |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: The requirements are applicable when the band is also defined for E-UTRA or UTRA. |  |  |  |  |

For operation in non-contiguous paired or unpaired spectrum, the ACLR shall be higher than the value specified in table 6.6.3.5.3.1A-2.

Table 6.6.3.5.3.1A-2: Base Station ACLR limit in non-contiguous spectrum or multiple bands

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel [MHz] | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies [MHz] | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | Wgap ≥ 15 (Note 3) Wgap ≥ 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44.2 dB |
|  | Wgap ≥ 20 (Note 3) Wgap ≥ 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) |  |  |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Wgap ≥ 60 (Note 4) Wgap ≥ 30 (Note 3) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 43.8 dB |
|  | Wgap ≥ 80 (Note 4) Wgap ≥ 50 (Note 3) | 30 MHz | 20 MHz NR (Note 2) |  |  |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 5, 10, 15, 20 MHz. NOTE 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |

<Next of change>

6.6.3.5.3.2	MSR UTRA FDD test requirement

For UTRA FDD, the test requirement is specified in TS 25.141 [18], clause 6.5.2.2.5, and applies outside the Base Station RF Bandwidth or Maximum Radio Bandwidth.

For a TAB connector operating in non-contiguous spectrum, ACLR requirement also applies for the first adjacent channel, inside any sub-block gap with a gap size Wgap ≥ 15 MHz. The ACLR requirement for the second adjacent channel applies inside any sub-block gap with a gap size Wgap ≥ 20 MHz. The CACLR test requirement in clause 6.6.3.5.6.2 applies in sub block gaps for the frequency ranges defined in table 6.6.3.5.6.2-1.

For a multi-band TAB connector ACLR requirement also applies for the first adjacent channel, inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 15 MHz. The ACLR requirement for the second adjacent channel applies inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 20 MHz. The CACLR requirement in clause 6.6.3.5.6.2 applies in Inter RF Bandwidth gaps for the frequency ranges defined in table 6.6.3.5.6.2-1.

6.6.3.5.3.3	MSR UTRA TDD test requirement

For UTRA TDD, the test requirement is specified in clause 6.6.3.5.5, and applies outside the Base Station RF Bandwidth or Maximum Radio Bandwidth.

6.6.3.5.3.4	Cumulative ACLR requirement in non-contiguous spectrum

The following test requirement applies for sub-block or Inter RF Bandwidth gap sizes listed in table 6.6.3.5.3.4-1 for UTRA FDD, UTRA TDD or E-UTRA operation or table 6.6.3.5.3.4-1A for NR operation:

-	Inside a sub-block gap within an operating band for a BS operating in non-contiguous spectrum.

-	Inside an Inter RF Bandwidth gap for a BS operating in multiple bands, where multiple bands are mapped on the same antenna connector.

The Cumulative Adjacent Channel Leakage power Ratio (CACLR) in a sub-block gap or the Inter RF Bandwidth gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap; and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The requirement applies to adjacent channels of E-UTRA or UTRA or NR carriers allocated adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap. In case of mixed UTRA and E-UTRA (but not NR) carriers on either side of the gap, table 6.6.3.5.3.4-1 is applicable. In case of mixed NR and E-UTRA or UTRA or NR only carriers on either side of the gap, table 6.6.3.5.3.4-1A is applicable. The assumed filter for the adjacent channel frequency is defined in tables 6.6.3.5.3.4-1 and 6.6.3.5.3.4-1A and the filters on the assigned channels are defined in table 6.6.3.5.3.4-2.

NOTE:	If the RAT on the assigned channel frequencies is different, the filters used are also different.

The CACLR for E-UTRA and UTRA carriers located on either side of the sub-block gap or the Inter RF Bandwidth gap shall be higher than the value specified in table 6.6.3.5.3.4-1.

Table 6.6.3.5.3.4-1: CACLR in non-contiguous spectrum or multiple bands for UTRA and E-UTRA only

| Band Category | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies | adjacent channel centre frequency offset below or above the sub-block edge or the Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | CACLR limit |
| --- | --- | --- | --- | --- | --- |
| BC1, BC2 | 5 MHz ≤ Wgap < 15 MHz | 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| BC1, BC2 | 10 MHz ≤ Wgap < 20 MHz | 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| BC3 | 5 MHz ≤ Wgap < 15 MHz | 2.5 MHz | 5 MHz E-UTRA | Square (BWConfig) | 44.2 dB |
| BC3 | 10 MHz < Wgap < 20 MHz | 7.5 MHz | 5 MHz E-UTRA | Square (BWConfig) | 44.2 dB |
| NOTE: For BC1 and BC2 the RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104 [2], with a chip rate as defined in this table. |  |  |  |  |  |

Table 6.6.3.5.3.4-1A: CACLR in non-contiguous spectrum or multiple bands for E-UTRA and NR combinations

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel [MHz] | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies [MHz] | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | CACLR limit |
| --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | 5 ≤ Wgap < 15 (Note 3) 5 ≤ Wgap < 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44.2 dB |
|  | 10 < Wgap < 20 (Note 3) 10 ≤ Wgap < 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) |  |  |
| 25, 30, 35, 40, 45, 50, 60, 70, 80,90, 100 | 20 ≤ Wgap < 60 (Note 4) 20 ≤ Wgap < 30 (Note 3) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 43.8 dB |
|  | 40 < Wgap < 80 (Note 4) 40 ≤ Wgap < 50 (Note 3) | 30 MHz | 20 MHz NR (Note 2) |  |  |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the BS channel bandwidth of the UTRA, E-UTRA or NR carrier transmitted at the other edge of the gap is 5, 10, 15, 20 MHz. NOTE 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |

Table 6.6.3.5.3.4-2: Filter parameters for the assigned channel for UTRA, E-UTRA only combinations

| RAT of the carrier adjacent to the sub-block or Inter RF Bandwidth gap | Filter on the assigned channel frequency and corresponding filter bandwidth |
| --- | --- |
| E-UTRA | E-UTRA of same BW |
| UTRA FDD | RRC (3.84 Mcps) |
| NOTE: The RRC filter shall be equivalent to the transmit pulse shape filter defined in TS 25.104 [2], with a chip rate as defined in this table. |  |

<Next of change>

7.2.5.4	NR operation

For NR, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A in TS 38.104 [36] with parameters specified in table 7.2.5.4-1 for Wide Area BS, in table 7.2.5.4-2 for Medium Range BS and in table 7.2.5.4-3 for Local Area BS.

Table 7.2.5.4-1: NR Wide Area BS reference sensitivity levels

| BS channel bandwidth | Sub-carrier spacing | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
| (MHz) | (kHz) |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15 | 15 | G-FR1-A1-1 | -101 | -100.7 | -100.2 |
| 10, 15 | 30 | G-FR1-A1-2 | -101.1 | -100.8 | -100.3 |
| 10, 15 | 60 | G-FR1-A1-3 | -98.2 | -97.9 | -97.4 |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 | -94.6 | -94.3 | -93.8 |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -94.9 | -94.6 | -94.1 |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -95 | -94.7 | -94.2 |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.2.5.4-2: NR Medium Area BS reference sensitivity levels

| BS channel bandwidth | Sub-carrier spacing | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
| (MHz) | (kHz) |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15 | 15 | G-FR1-A1-1 | -96 | -95.7 | -95.2 |
| 10, 15 | 30 | G-FR1-A1-2 | -96.1 | -95.8 | -95.3 |
| 10, 15 | 60 | G-FR1-A1-3 | -93.2 | -92.9 | -92.4 |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 | -89.6 | -89.3 | -88.8 |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -89.9 | -89.6 | -89.1 |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -90 | -89.7 | -89.2 |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.2.5.4-3: NR Local Area BS reference sensitivity levels

| BS channel bandwidth | Sub-carrier spacing | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
| (MHz) | (kHz) |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0GHz |
| 5, 10, 15 | 15 | G-FR1-A1-1 | -93 | -92.7 | -92.2 |
| 10, 15 | 30 | G-FR1-A1-2 | -93.1 | -92.8 | -92.3 |
| 10, 15 | 60 | G-FR1-A1-3 | -90.2 | -89.9 | -89.4 |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 | -86.6 | -86.3 | -85.8 |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -86.9 | -86.6 | -86.1 |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -87 | -86.7 | -86.2 |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The relationship between Minimum Requirements and Test Requirements is defined in clause 4.1 and the explanation of how the Minimum Requirement has been adjusted by the Test Tolerance is given in Annex C.

<Next of change>

7.3.5.4	NR operation

For NR, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A in TS 38.104 [36] with parameters specified in table 7.3.5.4-1 for Wide Area BS, in table 7.3.5.4-2 for Medium Range BS and in table 7.3.5.4-3 for Local Area BS.

Table 7.3.5.4-1: Wide Area BS dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -70.4 | -82.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 |  |  |
| 10 | 15 | G-FR1-A2-1 | -70.4 | -79.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 |  |  |
|  | 60 | G-FR1-A2-3 | -68.1 |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.4 | -77.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 |  |  |
|  | 60 | G-FR1-A2-3 | -68.1 |  |  |
| 20 | 15 | G-FR1-A2-4 | -64.2 | -76.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 25 | 15 | G-FR1-A2-4 | -64.2 | -75.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 30 | 15 | G-FR1-A2-4 | -64.2 | -74.4 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 35 | 15 | G-FR1-A2-4 | -64.2 | -73.7 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 40 | 15 | G-FR1-A2-4 | -64.2 | -73.1 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 45 | 15 | G-FR1-A2-4 | -64.2 | -72.6 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 50 | 15 | G-FR1-A2-4 | -64.2 | -72.1 | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 60 | 30 | G-FR1-A2-5 | -64.2 | -71.3 | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 70 | 30 | G-FR1-A2-5 | -64.2 | -70.7 | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 80 | 30 | G-FR1-A2-5 | -64.2 | -70.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 90 | 30 | G-FR1-A2-5 | -64.2 | -69.5 | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| 100 | 30 | G-FR1-A2-5 | -64.2 | -69.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5.4-2: Medium Range BS dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -65.4 | -77.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 |  |  |
| 10 | 15 | G-FR1-A2-1 | -65.4 | -74.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 |  |  |
|  | 60 | G-FR1-A2-3 | -63.1 |  |  |
| 15 | 15 | G-FR1-A2-1 | -65.4 | -72.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 |  |  |
|  | 60 | G-FR1-A2-3 | -63.1 |  |  |
| 20 | 15 | G-FR1-A2-4 | -59.2 | -71.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 25 | 15 | G-FR1-A2-4 | -59.2 | -70.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 30 | 15 | G-FR1-A2-4 | -59.2 | -69.4 | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 35 | 15 | G-FR1-A2-4 | -59.2 | -68.7 | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 40 | 15 | G-FR1-A2-4 | -59.2 | -68.1 | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 45 | 15 | G-FR1-A2-4 | -59.2 | -67.6 | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 50 | 15 | G-FR1-A2-4 | -59.2 | -67.1 | AWGN |
|  | 30 | G-FR1-A2-5 | 59.8 |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 60 | 30 | G-FR1-A2-5 | -59.2 | -66.3 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 70 | 30 | G-FR1-A2-5 | -59.2 | -65.7 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 80 | 30 | G-FR1-A2-5 | -59.2 | -65.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 90 | 30 | G-FR1-A2-5 | -59.2 | -64.5 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| 100 | 30 | G-FR1-A2-5 | -59.2 | -64.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5.4-3: Local Area BS dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -62.4 | -74.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -63.1 |  |  |
| 10 | 15 | G-FR1-A2-1 | -62.4 | -71.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -63.1 |  |  |
|  | 60 | G-FR1-A2-3 | -60.1 |  |  |
| 15 | 15 | G-FR1-A2-1 | -62.4 | -69.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -63.1 |  |  |
|  | 60 | G-FR1-A2-3 | -60.1 |  |  |
| 20 | 15 | G-FR1-A2-4 | -56.2 | -68.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 25 | 15 | G-FR1-A2-4 | -56.2 | -67.2 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 30 | 15 | G-FR1-A2-4 | -56.2 | -66.4 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 35 | 15 | G-FR1-A2-4 | -56.2 | -65.7 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 40 | 15 | G-FR1-A2-4 | -56.2 | -65.1 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 45 | 15 | G-FR1-A2-4 | -56.2 | -64.6 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 50 | 15 | G-FR1-A2-4 | -56.2 | -64.1 | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 60 | 30 | G-FR1-A2-5 | -56.2 | -63.3 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 70 | 30 | G-FR1-A2-5 | -56.2 | -62.7 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 80 | 30 | G-FR1-A2-5 | -56.2 | -62.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 90 | 30 | G-FR1-A2-5 | -56.2 | -61.5 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| 100 | 30 | G-FR1-A2-5 | -56.2 | -61.1 | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The relationship between Minimum Requirements and Test Requirements is defined in clause 4.1 and the explanation of how the Minimum Requirement has been adjusted by the Test Tolerance is given in Annex C.

<Next of change>

7.7.5	Test requirements

7.7.5.1	MSR operation

7.7.5.1.1	General intermodulation test requirement

Interfering signals shall be a CW signal and an E-UTRA or UTRA signal, as specified in annex A of TS 37.141 [16].

The requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For multi-band TAB connector, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the UTRA/E-UTRA interfering signal centre frequency offset from the RF bandwidth edge. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals coupled to the TAB connector, using the parameters in table 7.7.5.1.1-1 and 7.7.5.1.1-2, the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in clause 7.2.5.3.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.1.

-	For any measured UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.2.

-	For any NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 38.104 [36], clause 7.2.

Table 7.7.5.1.1-1: General intermodulation requirement

| Base Station Type | Mean power of interfering signals (dBm) | Wanted Signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -48 +y (Note 6) | PREFSENS +x dB (Note 2, 5) |  |
| Medium Range BS | -44 +y (Note 6) | PREFSENS +x dB (Note 3, 5) | See Table 7.7.5.1.1-2 |
| Local Area BS | -38 +y (Note 6) | PREFSENS +x dB (Note 4, 5) |  |
| NOTE 1: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see clause 7.2. NOTE 2: For WA BS not supporting NR, "x" is equal to 6 NOTE 3: For MR BS not supporting NR, "x" is equal to 6 in case of UTRA wanted signals, 9 in case of E-UTRA wanted signals. NOTE 4: For LA BS not supporting NR, "x" is equal to 12 in case of E-UTRA wanted signals, 6 in case of UTRA wanted signals. NOTE 5: For a BS that transmits NR in some configurations and does not transmit UTRA in any configuration, x is equal to 6. NOTE 6: For a BS not supporting NR, "y" is equal to zero for all BS classes. For a BS supporting NR and not supporting UTRA; "y" is equal to -4 for the WA BS class, -3 for the MR BS class and -6 for the LA BS class. |  |  |  |

Table 7.7.5.1.1-2: Interfering signals for intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge | Interfering signal centre frequency offset from the Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±2.0 (BC1 and BC3) / ±2.1 (BC2) | CW |
|  | ±4.9 | 1.4MHz E-UTRA signal |
| E-UTRA 3 MHz | ±4.4 (BC1 and BC3) / ±4.5 (BC2) | CW |
|  | ±10.5 | 3MHz E-UTRA signal |
| UTRA FDD and E-UTRA 5 MHz | ±7.5 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| E-UTRA 10 MHz | ±7.375 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| E-UTRA 15 MHz | ±7.25 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| E-UTRA 20 MHz | ±7.125 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| 1.28 Mcps UTRA TDD | ±2.3 (BC3) | CW |
|  | ±5.6 (BC3) | 1.28Mcps UTRA TDD signal |
| NR 5 MHz | ±7.5 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 10 MHz | ±7.465 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 15 MHz | ±7.43 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 20 MHz | ±7.395 | CW |
|  | ±17.5 | 5MHz E-UTRA signal |
| NR 25 MHz | ±7.465 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 30 MHz | ±7.43 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 35 MHz | ±7.44 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 40 MHz | ±7.45 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 45 MHz | ±7.415 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 50 MHz | ±7.35 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 60 MHz | ±7.49 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 70 MHz | ±7.42 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 80 MHz | ±7.44 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 90 MHz | ±7.46 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 100 MHz | ±7.48 | CW |
|  | ±25 | 20MHz E-UTRA signal |

<Next of change>

7.7.5.1.2	General narrowband intermodulation test requirement

Interfering signals shall be a CW signal and an E-UTRA 1RB signal, as specified in annex A of TS 37.141 [16].

The requirement is applicable outside the Base Station RF Bandwidth or maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF bandwidth edges or maximum Radio Bandwidth edges.

For a TAB connector operating in non-contiguous spectrum within each supported operating band, the requirement applies in addition inside any sub-block gap in case the sub-block gap is at least as wide as the channel bandwidth of the E-UTRA interfering signal in table 7.7.5.1.2-2. The interfering signal offset is defined relative to the sub-block edges inside the gap.

For a multi-band TAB connector, the requirement applies in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the E-UTRA interfering signal in table 7.7.5.1.2-2. The interfering signal offset is defined relative to the RF bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted signal at the assigned channel frequency and two interfering signals coupled to the Base Station antenna input, using the parameters in table 7.7.5.2-1 and 7.7.5.2-2, the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in clause 7.2.5.3.

-	For any NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 38.104 [36], clause 7.2.5.4.

-	For any measured UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.1.

-	For any measured UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in clause 7.2.5.2.

Table 7.7.5.1.2-1: General narrowband intermodulation requirement

| Base Station Type | Mean power of interfering signals (dBm) | Wanted Signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -52 |  |  |
| Medium Range BS | -47 | PREFSENS +x dB (Note) | See Table 7.7.5.1.2-2 |
| Local Area BS | -44 |  |  |
| NOTE: PREFSENS depends on the RAT, the BS class and on the channel bandwidth, see clause 7.2 in TS 37.104 [12]. "x" is equal to 6 in case of NR, E-UTRA or UTRA wanted signals. |  |  |  |

Table 7.7.5.1.2-2: Interfering signals for narrowband intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge or sub-block edge | CW or 1RB interfering signal centre frequency offset from the Base Station RF Bandwidth edge or sub-block edge inside a gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±970 (BC1 and BC3) / ±790 (BC2) | 1.4 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA or E-UTRA 3 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±960 (BC1 and BC3) / ±780 (BC2) | 3.0 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 5 MHz | ±360 | CW |
|  | ±1060 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 10 MHz | ±325 | CW |
| (NOTE 2) | ±1240 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 15 MHz | ±380 | CW |
| (NOTE 2) | ±1600 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 20 MHz | ±345 | CW |
| (NOTE 2) | ±1780 | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| UTRA FDD | ±345 (BC1 and BC2) | CW |
|  | ±1780 (BC1 and BC2) | 5MHz E-UTRA signal, 1 RB (NOTE 1) |
| 1.28Mcps UTRA TDD | ±190 (BC3) | CW |
|  | ±970 (BC3) | 1.4 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 5 MHz | ±360 | CW |
|  | ±1420 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 10 MHz | ±370 | CW |
|  | ±1960 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 15 MHz | ±380 | CW |
| (NOTE 2) | ±1960 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 20 MHz | ±390 | CW |
| (NOTE 2) | ±2320 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 25 MHz | ±325 | CW |
| (NOTE 2) | ±2350 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 30 MHz | ±335 | CW |
| (NOTE 2) | ±2350 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 35 MHz (Note 2) | ±345 | CW |
|  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 40 MHz | ±355 | CW |
| (NOTE 2) | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 45 MHz (Note 2) | ±365 | CW |
|  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 50 MHz | ±375 | CW |
| (NOTE 2) | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 60 MHz | ±395 | CW |
| (NOTE 2) | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 70 MHz | ±415 | CW |
| (NOTE 2) | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 80 MHz | ±435 | CW |
| (NOTE 2) | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 90 MHz | ±365 | CW |
| (NOTE 2) | ±2530 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 100 MHz | ±385 | CW |
| (NOTE 2) | ±2530 | E-UTRA signal, 1 RB (NOTE 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the Base Station RF Bandwidth edge. NOTE 2: This requirement shall apply only for an E-UTRA FRC A1-3 or NR G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. |  |  |

<Next of change>

7.8.5	Test requirements

For each measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in [Annex A] with parameters specified in table 7.8.5-1 for AAS BS of Wide Area BS, class in Table7.8.5-2 for AAS BS of Local Area BS class and in table 7.8.5-3 for AAS BS of Medium Range BS class.

Table 7.8.5-1: Wide Area BS in-channel selectivity for E-UTRA channels

| E-UTRA channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |
| 1.4 | A1-4 in clause A.1 | -105.5 | -105.1 | -87 | 1.4 MHz E-UTRA signal, 3 RBs |
| 3 | A1-5 in clause A.1 | -100.7 | -100.3 | -84 | 3 MHz E-UTRA signal, 6 RBs |
| 5 | A1-2 in clause A.1 | -98.6 | -98.2 | -81 | 5 MHz E-UTRA signal, 10 RBs |
| 10 | A1-3 in clause A.1 | -97.1 | -96.7 | -77 | 10 MHz E-UTRA signal, 25 RBs |
| 15 | A1-3 in clause A.1 (Note) | -97.1 | -96.7 | -77 | 15 MHz E-UTRA signal, 25 RBs (Note) |
| 20 | A1-3 in clause A.1 (Note) | -97.1 | -96.7 | -77 | 20 MHz E-UTRA signal, 25 RBs (Note) |
| NOTE: Wanted and interfering signal are placed adjacently around FC. |  |  |  |  |  |

Table 7.8.5-2: Local Area BS in-channel selectivity for E-UTRA channels

| E-UTRA channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |
| 1.4 | A1-4 in clause A.1 | -97.5 | -97.1 | -79 | 1.4 MHz E-UTRA signal, 3 RBs |
| 3 | A1-5 in clause A.1 | -92.7 | -92.3 | -76 | 3 MHz E-UTRA signal, 6 RBs |
| 5 | A1-2 in clause A.1 | -90.6 | -90.2 | -73 | 5 MHz E-UTRA signal, 10 RBs |
| 10 | A1-3 in clause A.1 | -89.1 | -88.7 | -69 | 10 MHz E-UTRA signal, 25 RBs |
| 15 | A1-3 in clause A.1 (Note) | -89.1 | -88.7 | -69 | 15 MHz E-UTRA signal, 25 RBs (Note) |
| 20 | A1-3 in clause A.1 (Note) | -89.1 | -88.7 | -69 | 20 MHz E-UTRA signal, 25 RBs (Note) |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, this reference measurement channel and interfering signal are not applied for Band 46 and Band 49. |  |  |  |  |  |

Table 7.8.5-3: Medium Range BS in-channel selectivity for E-UTRA channels

| E-UTRA channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |
| 1.4 | A1-4 in clause A.1 | -100.5 | -100.1 | -82 | 1.4 MHz E-UTRA signal, 3 RBs |
| 3 | A1-5 in clause A.1 | -95.7 | -95.3 | -79 | 3 MHz E-UTRA signal, 6 RBs |
| 5 | A1-2 in clause A.1 | -93.6 | -93.2 | -76 | 5 MHz E-UTRA signal, 10 RBs |
| 10 | A1-3 in clause A.1 | -92.1 | -91.7 | -72 | 10 MHz E-UTRA signal, 25 RBs |
| 15 | A1-3 in clause A.1 (Note) | -92.1 | -91.7 | -72 | 15 MHz E-UTRA signal, 25 RBs (Note) |
| 20 | A1-3 in clause A.1 (Note) | -92.1 | -91.7 | -72 | 20 MHz E-UTRA signal, 25 RBs (Note) |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, this reference measurement channel and interfering signal are not applied for Band 46. |  |  |  |  |  |

For NR channels, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A of TS 38.104 [36] with parameters specified in Table 7.8.5-4 for Wide Area BS, in Table 7.8.5-5 for Medium Range BS and in Table7.8.5-6 for Local Area BS. The characteristics of the interfering signal is further specified in annex A of TS 38.104 [36].

Table 7.8.5-4: Wide Area BS in-channel selectivity for NR channels

| NR channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |  |  |
| 5 | 15 | G-FR1-A1-7 | -99.2 | -98.8 | -98.1 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 PRBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -97.3 | -96.9 | -96.2 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 PRBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -90.9 | -90.5 | -89.8 | -71.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 PRBs |
| 5 | 30 | G-FR1-A1-8 | -99.9 | -99.5 | -98.8 | -81.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 5 PRBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -97.4 | -97 | -96.3 | -78.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 10 PRBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -91.2 | -90.8 | -90.1 | -71.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 50 PRBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -96.8 | -96.4 | -95.7 | -78.4 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 PRBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -91.3 | -90.9 | -90.2 | -71.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 PRBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [41]. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |  |

Table 7.8.5-5: Medium Range BS in-channel selectivity for NR channels

| NR channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |  |  |
| 5 | 15 | G-FR1-A1-7 | -94.2 | -93.8 | -93.1 | -76.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 PRBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -92.3 | -91.9 | -91.2 | -72.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 PRBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -85.9 | -85.5 | -84.8 | -66.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 PRBs |
| 5 | 30 | G-FR1-A1-8 | -94.9 | -94.5 | -93.8 | -76.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 5 PRBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -92.4 | -92 | -91.3 | -73.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 10 PRBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -86.2 | -85.8 | -85.1 | -66.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 50 PRBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -91.8 | -91.4 | -90.7 | -73.4 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 PRBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -86.3 | -85.9 | -85.2 | -66.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 PRBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [41]. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |  |

Table 7.8.5-6: Local area BS in-channel selectivity for NR channels

| NR channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |  |  |
| 5 | 15 | G-FR1-A1-7 | -91.2 | -90.8 | -90.1 | -73.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 PRBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -89.3 | -88.9 | -88.2 | -69.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 PRBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -82.9 | -82.5 | -81.8 | -63.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 PRBs |
| 5 | 30 | G-FR1-A1-8 | -91.9 | -91.5 | -90.8 | -73.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 5 PRBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -89.4 | -89 | -88.3 | -70.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 10 PRBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -83.2 | -82.8 | -82.1 | -63.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 50 PRBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -88.8 | -88.4 | -87.7 | -70.4 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 PRBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -83.3 | -82.9 | -82.2 | -63.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 PRBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [41]. The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

<End of change>