3GPP TSG-RAN WG4 Meeting #98-e		                    R4-2102049

Online, 25th Jan – 5th Feb, 2021

Source: 	Keysight Technologies

Title: 	47GHz band TT for NR BS RF requirement
Agenda Item:	9.25.4.1

Document for:	Agreement

# Introduction

In RAN4#97e, it was proposed and discussed about BS conformance test TT (test tolerance) value and MU (Measurement Uncertainty) value, and WF[1] document developed. In BS RF requirement, test tolerance values need to be set for setting conformance requirement. In currently defined TT values for existing band, there are some values which defined by TT=MU. With this calculation, in order to set TT, MU analysis is needed. Ideally, these MU values should be studied/experimented from actual system to set these values, however, there was no such analysis done in 3gpp RAN4 for frequency range of n262 band. Then values needed to be discussed with estimated number. This document proposes approach and number with some background.

Also importantly, for receiver testing, because there is no vector Signal Generator available in market with this frequency range, use of mixer needs to be considered. This means MU budget table used for existing band needs to be modified.

# Description

When OTA study was conducted and TR document[4][5] created, it covered up to 40GHz in FR2 frequency. For n262 (47.2GHz – 48.2GHz), values in this TR document as well as existing TT values in TS document (up to n259 band) can’t be reused because too much of frequency difference for upper frequency side in mmWave.  For example;

- There is more pathloss which degrade SNR at receiver

- Mis-match uncertainty increases with higher frequency

- Various uncertainty changes/increases for OTA related terms

- Measurement Equipment uncertainty increases with higher frequency

While ideal situation is to conduct study and experimental measurement of various chamber and OTA method, unfortunately, there is not enough time under given this WI timeframe. However, what needs to be done is to set TT values, therefore this document proposes extrapolation method to calculate “Estimated MU” for setting TT value.

Please note that this same method is used for setting TT value for UE n259 requirement with calculating “Estimated MU” values for setting TT values. Also note that this method is only for setting TT value. For setting actual MU values, it should be done through study and actual measurement of test system and method.[2][3]

Proposed Method

Linear extrapolation is as following figure 1.

Figure 1. Illustration of proposed linear extrapolation approach to estimate MU value for n262

Tx test Estimated MU value

Here is calculated “Estimated MU” values for n262.

Table 1. Estimated MU values to set TT values for n262, for Tx test cases (in dB)

| Tx test | 24.25 ~ 29.5GHz | 37 ~ 40GHz | Estimated MU 47.2~48.2GHz |
| --- | --- | --- | --- |
| EIRP | 1.7 | 2 | 2.2 |
| EIRP extreme | 3.1 | 3.3 | 3.5 |
| Output Power | 2.1 | 2.4 | 2.6 |
| Tx Off power | 2.9 | 3.3 | 3.6 |
| Relative ACLR | 2.3 | 2.6 | 2.8 |

For some Tx test cases which has same MU/TT values for both frequency range (24.25~29.5G and 37~40G), there is no difference in values for n262 with extrapolation approach. This is risky for values in this high frequency range therefore to add some margin based on above tables, 0.2dB

Table 2. Estimated MU values to set TT values for n262, for Tx test cases where two values are the same (in dB)

| Tx test | 24.25 ~ 29.5GHz | 37 ~ 40GHz | Estimated MU 47.2 ~ 48.2Ghz |
| --- | --- | --- | --- |
| Absolute ACLR | 2.7 | 2.7 | 2.9 |
| OBUE (close) | 2.7 | 2.7 | 2.9 |

Rx test Estimated MU value

For Receive testing, there is very important difference from Tx testing, which is currently there is no vector Signal Generator available in market with this frequency range. This means, there is no Signal Generator can generate 5G NR modulated signal with this frequency range.

This means, currently used MU budget table for receiver testing is not usable because it is using Signal Generator MU value for Stimulus and Modulated interferer signal.

Important observation for Receiver testing

- There is no vector Signal Generator available in market with this n262 frequency range

- Ideally, if there was estimated MU for such frequency range supported Signal Generator, which would be ideal, however there is no such value available. There are multiple possible contributions for estimating signal level accuracy, such as flatness, components level loss, etc. it’s not easy and straight forward process to do, actual measurement is needed with real equipment (which don’t exist).

- Therefore, generating 5G NR modulated stimulus and interference signal, it is necessary to assume use of mixer to upconvert modulated signal to n262 frequency range.

Table 3, Estimated MU value to set TT value for n262, for Rx tests, EIS (in dB)

| From Agreed EIS value for 37GHz < f < 40GHz value (note 1) | 1.22449 |
| --- | --- |
| Mixer uncertainty (note 2) | 2.25 |
| Combined uncertainty 1 sigma value | 2.561616 |
| Expanded uncertainty (1 sigma x 1.96) | 5.020767 |
| Add estimated uncertainty contribution 0.2 from other terms | 5.220767 |
| Proposed MU for EIS (2 digit) for 47.2GHz < f < 48.2GHz | 5.2 |

(note 1, value is from [5], from table 10.2.7-2, value 2.4 divided by 1.96 to make it as 1 sigma value)

(note 2, mixer uncertainty (UID A1-22, A2-20) is from [5] table 12.2.2.4-1, 12.2.3.3-1

for Tx spurious measurement for 40GHz ~60GHz)

Table 4, Estimated MU value to set TT value for n262, for Rx tests, ACS, IBB, In-channel selectivity (in dB)

| MU-EIS (value from table 3 above, combined uncertainty 1 sigma) | 2.561616 |
| --- | --- |
| MU-TE for modulated interferer with Mixer uncertainty combined by RSS (note 2) | 2.423324 |
| MU PA (from [5] table 10.5.4-3) | 0.2 |
| ACLR effect (from [5] table 10.5.4-3) | 0.4 |
| Combined standard uncertainty (1 sigma) | 3.554 |
| Expanded uncertainty (1 sigma x 1.96) | 6.966 |
| Add estimated uncertainty contribution 0.2 from other terms | 7.167 |
| Expanded uncertainty - Proposed MU for ACS, IBB, In-channel selectivity (2 digit) for 47.2GHz < f < 48.2GHz | 7.2 |

(Note, this table is based from TR37.941 v15.2.0[5], table 10.5.4-3 MU for ACS/IBB/In-channel selectivity)

(Note2, value in table 10.5.4-3 is 0.9, mixer uncertainty (2.25) combined by RSS ( ((0.9)^2+(2.25)^2)^0.5)

Table 5, Estimated MU value to set TT value for n262, for Rx tests, IM

| MU-EIS (value from table 3 above, combined uncertainty 1 sigma) | 2.561616 |
| --- | --- |
| MU-TE for modulated interferer with Mixer uncertainty combined by RSS (note 2) | 2.423324 |
| MU CW interferer (CW Signal Generator) (note 3) | 1.28 |
| MU PA (from [5] table 10.5.4-3) | 0.2 |
| ACLR effect (from [5] table 10.5.4-3) | 0.4 |
| Combined standard uncertainty (1 sigma) | 3.778 |
| Expanded uncertainty (1 sigma x 1.96) | 7.405 |
| Add estimated uncertainty contribution 0.2 from other terms | 7.605 |
| Expanded uncertainty - Proposed MU for ACS, IBB, In-channel selectivity (2 digit) for 47.2GHz < f < 48.2GHz | 7.6 |

(Note, this table is based from TR37.941 v15.2.0[5], table 10.6.4-2 MU for Rx IM)

(Note2, value in table 10.5.4-3 is 0.9, mixer uncertainty (2.25) combined by RSS ( ((0.9)^2+(2.25)^2)^0.5)

(Note3, MU TE for CW signal generator level uncertainty in this frequency range is 1.28)

# Proposal

This document proposes, “Estimated MU” values which calculated by extrapolation method for setting TT values.

Here is overall TT values table with “Estimated MU” values

Table 6. updated table of Table C.1-2 in TS38.141-2 v16.6.0[6], Derivation of test requirements (FR2 OTA transmitter tests) under Annex C Test tolerances and derivation of test requirements

| Test | Minimum requirement in TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 Radiated transmit power | See TS 38.104 [2], clause 9.2 | Normal conditions: 1.7 dB, 24.25GHz < f ≦ 29.5GHz 2.0 dB, 37GHz < f ≦ 43.5GHz 2.2 dB, 47.2GHz < f ≦48.2GHz Extreme conditions: 3.1 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz 3.5 dB, 47.2GHz < f ≦48.2GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.3 OTA base station output power | See TS 38.104 [2], clause 9.3 | 2.1 dB, 24.25GHz < f ≦ 29.5GHz 2.4 dB, 37GHz < f ≦ 43,5GHz 2.6 dB, 47.2GHz < f ≦48.2GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.4 OTA output power dynamics | See TS 38.104 [2], clause 9.4 | 0.4 dB | Formula: Total power dynamic range – TT |
| 6.5.1 OTA transmitter OFF power | See TS 38.104 [2], clause 9.5.2 | 2.9 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz 3.6 dB, 47.2GHz < f ≦48.2GHz | Formula: Minimum Requirement + TT |
| 6.6.2 OTA frequency Error | See TS 38.104 [2], clause 9.6.1 | 12 Hz | Formula: Frequency Error limit + TT |
| 6.6.3 OTA Modulation quality (EVM) | See TS 38.104 [2], clause 9.6.2 | 1 % | Formula: EVM limit + TT |
| 6.6.4 OTA time alignment error | See TS 38.104 [2], clause 9.6.3 | 25 ns |  |
| 6.7.2 OTA occupied bandwidth | See TS 38.104 [2], clause 9.7.2 | 0 Hz | Formula: Minimum Requirement + TT |
| 6.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR) | See TS 38.104 [2], clause 9.7.3 | Relative: 2.3 dB, 24.25GHz < f ≦ 29.5GHz 2.6 dB, 37GHz < f ≦43.5GHz 2.8 dB, 47.2GHz < f ≦48.2GHz Absolute: 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦43.5GHz 2.9 dB, 47.2GHz < f ≦48.2GHz | Formula: Relative limit - TT Absolute limit +TT |
| 6.7.4 OTA operating band unwanted emissions | See TS 38.104 [2], clause 9.7.4 | 0 MHz f < 0.1*BWcontiguous 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦43.5GHz 2.9 dB, 47.2GHz < f ≦48.2GHz 0.1*BWcontiguous f < fmax 0 dB For co-existence with Earth Exploration Satellite Service 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.2 General transmitter spurious emissions requirements Category A | See TS 38.104 [2], clause 9.7.5.3.2 | 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.2 General transmitter spurious emissions requirements Category B | See TS 38.104 [2], clause 9.7.5.3.2 | 0 dB | Formula: Minimum Requirement + TT |
| 6.7.5.4 OTA transmitter spurious emissions, additional requirements | See TS 38.104 [2], clause 9.7.5.3.3 | For co-existence with Earth Exploration Satellite Service 0 dB | Formula: Minimum Requirement + TT |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

Table 7. updated table of Table C.2-2 in TS38.141-2 v16.6.0[6], Derivation of test requirements (FR2 OTA receiver tests) under Annex C Test tolerances and derivation of test requirements

| Test | Minimum requirement in TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 7.3 OTA reference sensitivity level | See TS 38.104 [2], clause 10.3 | 2.4 dB, 24.25 GHz < f ≦ 33.4 GHz 2.4 dB, 37 GHz < f ≦ 43.5 GHz 5.2 dB, 47.2GHz < f ≦48.2 GHz | Formula: EISREFSENS+ TT |
| 7.5.1 OTA adjacent channel selectivity | See TS 38.104 [2], clause 10.5.1 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.5.2 In-band blocking | See TS 38.104 [2], clause 10.5.2 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.6 OTA out-of-band blocking | See TS 38.104 [2], clause 10.6 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged |
| 7.7 OTA receiver spurious emissions | See TS 38.104 [2], clause 10.7 | 0 dB | Formula: Minimum Requirement + TT |
| 7.8 OTA receiver intermodulation | See TS 38.104 [2], clause 10.8 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.9 OTA in-channel selectivity | See TS 38.104 [2], clause 10.9 | 3.4 dB, 24.25 GHz < f ≦ 33.4 GHz 3.4 dB, 37 GHz < f ≦ 43.5 GHz 7.2 dB, 47.2GHz < f ≦48.2 GHz | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

# References

[1] R4-2016881 WF on BS MU/TT for n262, Nokia, Nokia Shanghai Bell, Ericsson

[2] R5-200316 On AP#85.28: n259 MU Estimate, Keysight Technologies

[3] R5-200816 Estimate of Measurement Uncertainty for n259, Rohde & Schwarz

[4]	TR38.817-02 v15.7.0

[5] TR37.941 v15.2.0

[6] TS38.141-2 v16.6.0