3GPP TSG-RAN WG4 #98-e	R4-2102084

Electronic Meeting, January 25nd – February 5th, 2021

Agenda item:	7.8.1.2.1

Source:	MediaTek Inc.

Title:	Simulation results for URLLC pre-emption

Document for:	Discussion

- Introduction

Based on the simulation assumptions and agreements in [1], we provide our simulation results for URLLC pre-emption.

- Simulation results and discussion

We provide the simulation results for MCS4, MCS 13 and MCS 16 based on the agreed simulation assumptions. Besides, Table 1 show the SNR values to achieve the 70% of maximum throughput.

Figure 1. PDSCH performance for pre-emption with MCS4

Figure 2. PDSCH performance for pre-emption with MCS13

Figure 3. PDSCH performance for pre-emption with MCS16

Table 1. SNR values at 70% throughput for FDD

| MCS | Pre-emption probability | SNR @ 70% throughput [dB] |  |
| --- | --- | --- | --- |
|  |  | Without HARQ buffer Flushing | With HARQ buffer Flushing |
| 4 | 10% | -2.09 | -2.09 |
|  | 20% | -1.36 | -1.38 |
| 13 | 10% | 6.19 | 6.15 |
|  | 20% | 8.86 | 8.31 |
| 16 | 10% | 9.41 | 9.35 |
|  | 20% | NA | NA |

For MCS 16, the SNR loss between “with buffer flushing” and “without buffer flushing” is small for pre-emption probability 10%. Besides, with pre-emption probability 20%, it cannot attain 70% throughput and the slope near 70% is very small. Hence, we propose using MCS 13 to define test cases.

Proposal: Use MCS 13 to define test cases.

- Summary

Our proposal is summarized as below:

Proposal: Use MCS 13 to define test cases.

- Reference

- R4-2017509, “WF on URLLC UE performance requirements with higher BLER”, Intel