# 3GPP TSG-RAN WG4 Meeting # 98-e 			R4-2102098

# Electronic Meeting, Jan. 25-Feb. 5, 2021

Source:	Sony

Title:	Test frequencies for NB-IOT UE in standalone operation

Agenda item:	5.2

Document for:	Discussion

- Background

Band-edge emission at stand-alone and guard-band NB-IoT conditions is regulated by FCC OOB emission requirements. How to test the emission according to the FCC requirement was debated in RAN4 #96-e [1]. It was decided to seek guidance from the FCC on some specific questions which resulted in an LS sent to FCC [2].  In RAN4 #97-e the issue was brought up again [3] and it was decided to wait for the response from FCC [4]. Since no response has been received so far, we bring up this issue again.

This document is a resubmission of [3].

- Discussion

The issue is related to band-edge emission at stand-alone and guard-band NB-IoT conditions that is regulated by FCC OOB emission requirements.

According to 3GPP TS 36.508 [5] the NB-IoT channel in stand-alone and guard-band operating modes may be allocated just at 100kHz offset from the band edges; see for example band 12.

Table 8.1.3.1.1.12-1: NB-IoT standalone Test frequencies for operating band 12

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23011 | 0 | 699.1 | 5011 | -0.5 | 729.1 |
| Mid Range | 23095 | 0 | 707.5 | 5095 | -0.5 | 737.5 |
| High Range | 23179 | 0 | 715.9 | 5179 | -0.5 | 745.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.12-3: NB-IoT guard-band Test frequencies for operating band 12

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 23011 | 0 | 699.1000 | 5011 | 1 | 729.1075 |
| Mid Range | 5 | 23071 | 0 | 705.1000 | 5071 | 1 | 735.1075 |
|  | 10 | 23141 | 0 | 712.1000 | 5141 | 1 | 742.1075 |
| High Range | Any | 23179 | 0 | 715.9000 | 5179 | -2 | 745.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

The tables refer to a NB-IoT channel of 180kHz, so the first NB-IoT tone (15kHz or 3.75kHz) will be only about 10kHz from the band edges where the FCC regulation applies.

There was a question in last meeting whether the rules apply to the 3GPP band 12, the FFC band 12 or only A block. That is, does the “100 kHz” refer to 698.9-699MHz and 716-716.1MHz or 697.9-698 MHz and 716-716.1MHz or 697.9-698 MHz and 704.704.1MHz? If the answer is that it refers to the 3GPP band 12 there is a problem. This is the way it is tested today.

Observation 1:	TS 36.104 test conditions (test frequencies) for both stand-alone and guard-band NB-IoT operation may conflict with FCC band-edge spectrum emission requirements.

A proposed way to avoid the violation of the FCC regulation would be to omit operation at the first and last NB-IoT EARFCNs in stand-alone and guard-band modes (in the above example NUL=23011 and NUL=23179).

Observation 2:		100 kHz offset for NB-IoT network deployments may solve the violation of the FCC regulation.

We, therefore, propose an LS should be sent to RAN5 to exclude first and last EARFCNs

Proposal 1:	Send an LS to RAN5 with proposal to exclude the first and last EARFCNs in TS 36.104 test frequencies for both stand-alone and guard-band IoT operation modes for all frequency bands were FCC regulation applies.

The change of test frequencies, of course, must be accompanied with a deployment regime which require 100 kHz offset for NB-IoT network deployments as discussed above.

- Conclusion

In this contribution, we have discussed testing of NB-IOT. The following observations and proposals are made.

Observation 1:	TS 36.104 test conditions (test frequencies) for both stand-alone and guard-band NB-IoT operation may conflict with FCC band-edge spectrum emission requirements.

Observation 2:		100 kHz offset for NB-IoT network deployments may solve the violation of the FCC regulation.

Proposal 1:	Send an LS to RAN5 with proposal to exclude the first and last EARFCNs in TS 36.104 test frequencies for both stand-alone and guard-band IoT operation modes for all frequency bands were FCC regulation applies.

- References

R4-2011845,	“Email discussion summary for [96e][105] LTE UE RF maintenance”, Skyworks Solution Inc.

R4-2011913,	“LS on NB-IoT testing issues”, RAN WG4

R4-2015807,	“Test frequencies for NB-IOT UE in standalone operation”, Sony

R4-2016949,	“Email discussion summary for [97e][105] LTE UE RF maintenance”, Skyworks Solution Inc.

TS 36.508, 	“Common test environments for User Equipment (UE) conformance testing”, Section 8.1.3

TS 36.104,	 “Base Station (BS) radio transmission and reception”, Section 5.6