3GPP TSG-RAN WG4 Meeting #98-e	R4-2102239

Electronic Meeting, January 25 –February 5, 2021

Source: 	Ericsson

Title: 	Updated simulation assumptions for evaluating UE power saving for RLM and BM

Agenda Item:	11.9.2

Document for:	Approval

# Introduction

This contribution provides updated evaluation assumptions for R17 RLM/BFD relaxation. The motivation is presented in Appendix in section 4.

# System level simulation assumptions for mobility impact analysis from RLM/BFD relaxation

The intention of this evaluation is to justify the mobility impact of RLM/BFD relaxation under different scenarios. For FR1 and FR2, the evaluation assumption is provided in Table I.

Table I Evaluation assumptions for mobility impact of RLM/BFD relaxation in FR1 and FR2

| Parameters | Dense urban (UMi) in FR1 | Dense urban (UMi) in FR2 |
| --- | --- | --- |
| UE speed | 3km/h; 30km/h |  |
| Relaxation factor (X) | 2, [3], 4, [8] Note: For DRX cycle <=320 ms, “1.5” factor is considered in the R15/R16 baseline. | 2, [3], 4, [8] Note: For DRX cycle <=320 ms, “1.5” factor is considered in the R15/R16 baseline. |
| DRX cycle length | 20ms, 40ms |  |
| Layout | Single layer: Macro layer: Hex. Grid |  |
| Moving across Boarder | Option 1: Wrap around as defined in 36.839 [1] Option 2: Bouncing back as defined in 36.839 [1] |  |
| Inter-BS distance | 200m | 200m |
| Carrier frequency and SCS | 4GHz and 30kHz SCS for SSB | 30GHz and 120kHz SCS for SSB |
| Simulation bandwidth | 20MHz (for CORESET0 BW) | 80MHz (for CORESET0 BW) |
| N factor (# of RX beams for FR2) | N.A. | 8 for SSB, 1 for CSI-RS |
| SSB multiplex with Data | No |  |
| LoS probability | According to Table 7.4.2-1 in TR 38.901 [2] |  |
| Pathloss and shadow fading std. | Refer to UMi model in TR38.901 [2] |  |
| Shadow fading correlation distance | 10 for LOS and 13 for NLOS |  |
| BS Tx power (PA power) | 44dBm | 40dBm |
| UE receiver noise figure | 9dB | 13dB |
| UE distribution | 100% outdoor |  |
| BS antenna height | 10m |  |
| UE antenna height | 1.5m |  |
| BS antenna element gain + connector loss | Follow 38.802 [3] Table A.2.1-4 |  |
| UE antenna model | Follow 38.802 [3] Table A.2.1-4 | Follow 38.802 [3] Table A.2.1-4 (M,N,P,Mg,Ng) = Option 1: (1,2,2,1,2) Option 2: (2,2,2,1,2) Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) Note: only one of panels should be selected |
| UE beam sweeping | No | Yes. # of beams: N = 8 |
| BS antenna down-tilt | 15 degree |  |
| BS array configuration | (M,N,P,Mg,Ng) = Option 1: (8,8,2,1,1) Option 2: (2,4,2,1,1) (dH,dV) = Option 1: (0.5, 0.8)λ Option 2: (0.5, 0.9)λ +45°, -45° polarization | (M,N,P,Mg,Ng) = Option 1: (4,8,2,1,1) Option 2: (4,8,2,2,2) (dH,dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (4.0, 2.0)λ +45°, -45° polarization |
| BS beam set | For direction of analog beam steering (in LCS): Azimuth angle φi = [-45, -15, 15, 45] degree Zenith angle θj = [90,115] degree Note: Other options are not precluded. | For direction of analog beam steering (in LCS): Azimuth angle φi = [-60, -42.86, -25.71, -8.57, 8.57, 25.71, 42.86, 60] degree Zenith angle θj = [90, 100, 110, 120] degree Note: Other options are not precluded. |

The UE performs signal and interference measurements based on RLM-RS (i.e. SSB or CSI-RS) to derive a downlink link quality metric, and SINR is considered here for simplicity.

The performance metric of this evaluation can be

- increased latency in RLF triggering (for RLM)

- increased latency in beam failure detection and the initiation of beam recovery procedure (for BFD)

- the CDF curves of Delta SINR comparison between R15 an R17.

- Delta SINR = (estimated SINR_R17– estimated SINR_R15) 	[dB].

As one reference, the definition of Delta SINR in R15 is

•	Delta SINR = MAX (ABS(estimated SINR – ideal SINR) CDF=5%), ABS(estimated SINR – ideal SINR) CDF=95%)  	[dB].

# Evaluation assumptions for power saving benefits from RLM/BFD relaxation

This evaluation is going to provide power consumption comparison between relaxed RLM/BFD and R15/R16 baseline. The evaluation is based on power model in annex of TR 38.840 [4] and some further evaluation assumptions are provided in Table II as follows.

Table II Evaluation assumptions for power saving gain of RLM/BFD relaxation

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| longDRXCycle | 40ms for VoIP traffic models; 160ms for FTP Model 3 Note: Other options are not precluded | 40ms for VoIP traffic models; 160ms for FTP Model 3 Note: Other options are not precluded |
| OnDurationTimer | 4ms for VoIP traffic models; 8ms for FTP Model 3 Note: Other options are not precluded | 4ms for VoIP traffic models; 8ms for FTP Model 3 Note: Other options are not precluded |
| Drx_InactiveTimer | 4ms for VoIP traffic models; 20ms for FTP Model 3 Note: Other options are not precluded | 4ms for VoIP traffic models; 20ms for FTP Model 3 Note: Other options are not precluded |
| Slot length | 0.5ms | 0.125ms |
| Relaxation factor (X) | 2, [3], 4, [8] Note: For DRX cycle <=320 ms, “1.5” factor is considered in the R15/R16 baseline. | 2, [3], 4, [8] Note: For DRX cycle <=320 ms, “1.5” factor is considered in the R15/R16 baseline. |
| PDCCH-WUS configuration | Option 1: Yes WUS offset: 4 slot No wake-up if WUS is missed Option 2: No | Option 1: Yes WUS offset: 4 slot No wake-up if WUS is missed Option 2: No |
| Traffic Model | Option 1: FTP Model 3 0.1Mbytes packet size 200ms mean inter-arrival time (Note: Other options are not precluded) Option 2: VoIP traffic models in TR 38.840 | Option 1: FTP Model 3 0.1Mbytes packet size 200ms mean inter-arrival time (Note: Other options are not precluded) Option 2: VoIP traffic models in TR 38.840 |
| RLM-RS periodicity | SSB: 20 ms CSI-RS: TBD | SSB: 20 ms CSI-RS: TBD |
| RRM-RS periodicity | SSB(SMTC periodicity): 20ms Note: Other options are not precluded. | SSB(SMTC periodicity): 20ms Note: Other options are not precluded. |
| UL transmission | Interested companies are encouraged to consider this and claim the detailed assumptions when delivering results. |  |
| BW of RLM/BFD-RSs | Interested companies are encouraged to consider this and claim the detailed assumptions when delivering results. |  |
| RS for L1 measurements | Interested companies are encouraged to consider this and claim the detailed assumptions when delivering results. |  |

# Appendix

In order to evaluate the outcome of the SINR error performance due to applying different scaling factors, and to compare the results presented by different companies the fundamental performance metric should be the aligned. One such metric that is currently missing in the evaluation assumptions is the percentile (e.g. 90%-tile, 95%-tile, or 99%-tile) at which the SINR error should be derived. 5%- and 95% percentile are typically used in RAN4 evaluations, and therefore the performance metric for delta SINR in the evaluations assumptions should be updated to include the percentile as follows:

•	Delta SINR = MAX (ABS(estimated SINR – ideal SINR) CDF=5%), ABS(estimated SINR – ideal SINR) CDF=95%)  	[dB].

# References

[1]  TR 36.839	v11.1.0

[2]  TR 38.901	v15.1.0

[3]  TR 38.802	v14.2.0

[4]  TR 38.840	v16.0.0

[5]  TR 37.910	v16.1.0