3GPP TSG-RAN WG4 Meeting#98                	R4-2102264

E-meeting, January 25th – February 5th, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Measurement Performance Requirements test for MR-DC

Agenda item:	7.5.3.1.2

Document for:	Discussion

Introduction

In order to enable RAN5 to develop the necessary RF testcases for LTE_NR_CA_DC in Rel-16, RAN4 need to specify the necessary accuracy testcases in a similar manner as was done for Rel-16 for euCA. Hence, RAN4 need to develop the necessary performance requirement test cases.

This paper shortly discuss what additions are needed in RAN4, such that RAN5 can develop the necessary test cases for idle mode CA/DC measurements.

Discussion

According to 38.331, for Rel-16 NR early measurement reporting scheme UE will perform the idle mode measurements in idle mode, according to the EMR configuration which is provided either in system information or RRC release message. UE shall only report the EMR measurement results for the cells which at least meet the configured threshold (qualityThreshold). This means that the UE will perform measurements, but shall only report them to network, if the measurements fulfil quality criteria (as per network configuration):

TS 38.331 (NR Rel-16):

5.7.8 Idle/inactive Measurements …. #### 5.7.8.2a Performing measurements … 5> store the derived cell measurement results as indicated by reportQuantities for cells applicable for idle/inactive measurement reporting within the measReportIdleNR in VarMeasIdleReport in decreasing order of the cell sorting quantity, i.e. the best cell is included first, as follows: 6> if qualityThreshold is configured: 7> include the measurement results from the cells applicable for idle/inactive measurement reporting whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThreshold; 6> else: 7> include the measurement results from all cells applicable for idle/inactive measurement reporting; … #### MeasIdleConfig The IE MeasIdleConfig is used to convey information to UE about measurements requested to be done while in RRC_IDLE or RRC_INACTIVE. MeasIdleConfig information element … MeasIdleCarrierNR-r16 ::= SEQUENCE { carrierFreq-r16 ARFCN-ValueNR, ssbSubcarrierSpacing-r16 SubcarrierSpacing, frequencyBandList MultiFrequencyBandListNR OPTIONAL, -- Need R measCellListNR-r16 CellListNR-r16 OPTIONAL, -- Need R reportQuantities-r16 ENUMERATED {rsrp, rsrq, both}, qualityThreshold-r16 SEQUENCE { idleRSRP-Threshold-NR-r16 RSRP-Range OPTIONAL, -- Need R idleRSRQ-Threshold-NR-r16 RSRQ-Range OPTIONAL -- Need R } OPTIONAL, -- Need R ssb-MeasConfig-r16 SEQUENCE { nrofSS-BlocksToAverage-r16 INTEGER (2..maxNrofSS-BlocksToAverage) OPTIONAL, -- Need S absThreshSS-BlocksConsolidation-r16 ThresholdNR OPTIONAL, -- Need S smtc-r16 SSB-MTC OPTIONAL, -- Need S ssb-ToMeasure-r16 SSB-ToMeasure OPTIONAL, -- Need S deriveSSB-IndexFromCell-r16 BOOLEAN, ss-RSSI-Measurement-r16 SS-RSSI-Measurement OPTIONAL -- Need S } OPTIONAL, -- Need S beamMeasConfigIdle-r16 BeamMeasConfigIdle-NR-r16 OPTIONAL, -- Need R ... } …

As RAN2 specified the quality threshold as criteria for UE to report the measurements performed in idle, this aspect needs to be reflected also in RAN4 specifications. Therefore, in a similar manner as for euCA Rel15 in Rel-16, RAN4 need to define the measurement performance test case to ensure that SS-RSRP/Q measurement accuracy is within the specified limits.

RAN4 has defined the requirements for absolute accuracy for both idle mode serving cell and inter-frequency carrier measurements, but there are no test cases to ensure that the accuracy of the measurements are within these limits. The purpose of such test is to verify that the UE SS-RSRP/Q measurement accuracies are within the specified limits.

Define measurement performance test cases for SS-RSRP and SS-RSRQ measurement accuracies for LTE-NR_DC_CA.

Define intra-frequency measurement performance test cases for SS-RSRP and SS-RSRQ.

Define inter-frequency measurement performance test cases for SS-RSRP and SS-RSRQ.

Define inter-RAT measurement performance test cases for SS-RSRP and SS-RSRQ.

In [1] the reference LTE CR is provided.

Conclusion

In this paper we discuss missing tests for accuracy requirements for the SS-RSRP and SS-RSRQ measurements.

- Define measurement performance test cases for SS-RSRP and SS-RSRQ measurement accuracies for LTE-NR_DC_CA.

Define intra-frequency measurement performance test cases for SS-RSRP and SS-RSRQ.

Define inter-frequency measurement performance test cases for SS-RSRP and SS-RSRQ.

Define inter-RAT measurement performance test cases for SS-RSRP and SS-RSRQ.

References

- R4-2017062, CR on performance requirements tests for euCA, Nokia, Nokia Shanghai Bell