3GPP TSG RAN4 Meeting #98-e	R4-2102399

# Electronic, 25th January – 5th February

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.124 | CR | 0032 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.124: Correction of FR1 radiated spurious emissions (R16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Bureau Veritas, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-02-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are several problems in radiated spurious emissions: - The frequency range 9kHz~30MHz is not testable for radiated measurement - Measurement uncertainty for 12.75~26GHz is missing - Table number in NOTE below table 8.2.5-1 is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following changes are made: - Remove test frequency range 9kHz~30MHz. - Mark measurement uncertainty for 12.75~26GHz as TBD. - Correct the table numbers in NOTE below table 8.2.5-1 is incorrect. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The radiated spurious emission requirements are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4, 8.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Unchanged Text Skipped>

### 8.2.4	Limits

Unless otherwise stated, the radiated spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in table 8.2.4-0 from the edge of the channel bandwidth. The radiated spurious emission limits in table 8.2.4-1 apply for all transmitter band configurations (NRB) and channel bandwidths.

The references for these requirements are ITU-R SM 329 [5], SM.1539 [18] and TS 38.101-1 [3] for FR1.

The frequency boundary and reference bandwidths for the detailed transitions of the limits between the requirements for out of band emissions and spurious emissions are based on ITU-R SM 329 [5].

These requirements are only applicable for frequencies in the spurious domain. The limits are specified in table 8.2.4-1 for UE equipment supporting operations in FR1 only.

Table 8.2.4-0: Boundary between NR out of band and general spurious emission domain

| Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| BWChannel | BWChannel + 5 |

Table 8.2.4-1: Radiated spurious emissions requirements for UE equipment supporting operations in FR1 only

| Frequency range | Maximum level (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz ≤ f < 1000 MHz | -36 | 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 | 1 MHz | 4 |
|  | -25 | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 | 1 MHz | 2 |
| NOTE 1: Applies for Band that the upper frequency edge of the UL Band more than 2.69 GHz. NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz. NOTE 3: As specified in TS 38.101-1 [3]: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. NOTE 4: As specified in TS 38.101-1 [3]: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. |  |  |  |

NOTE:	Void.

### 8.2.5	Interpretation of the measurement results

The interpretation of the results recorded in a test report for the radiated emission measurements described in the present document shall be as follows:

-	the measured value related to the corresponding limit will be used to decide whether an equipment meets the requirements of the present document;

-	the value of the measurement uncertainty for the measurement of each parameter shall be included in the test report;

-	the recorded value of the measurement uncertainty shall be, for each measurement, equal to or lower than the figure in table 8.2.5-1.

Table 8.2.5-1 specifies the Maximum measurement uncertainty of the Test System. The Test System shall enable the equipment under test to be measured with an uncertainty not exceeding the specified values. All tolerances and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95% is the measurement uncertainty tolerance interval for a specific measurement that contains 95% of the performance of a population of test equipment.

Table 8.2.5-1: Maximum measurement uncertainty

| Parameter | Uncertainty |
| --- | --- |
| Effective radiated RF power between 30 MHz and 180 MHz | 6 dB |
| Effective radiated RF power between 180 MHz and 12.75 GHz | 3 dB |
| Effective radiated RF power between 12.75 GHz and 26 GHz | TBD |

NOTE:	If the Test System for a test is known to have a measurement uncertainty greater than that specified in table 8.2.5-1, this equipment can still be used, provided that an adjustment is made follows:
Any additional uncertainty in the Test System over and above that specified in table 8.2.5-1 is used to tighten the Test Requirements - making the test harder to pass. This procedure will ensure that a Test System not compliant with table 8.2.5-1 does not increase the probability of passing an EUT that would otherwise have failed a test if a Test System compliant with table 8.2.5-1 had been used.

### <End of Changes>