3GPP TSG-WG RAN4 Meeting #98-e	R4-2102476

# Electronic meeting, 25th January – 5th February, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.141 | CR | 0965 | rev | - | Current version: | 10.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.104: Correction to Band 24 requirements (Rel-10) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_B24_mod-Perf |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are two regulatory updates related to BS operation in Band 24 which need to be reflected in 36.104: - Regulations limits the downlink power to 9.8 dBW/MHz and limits transmission between 1526 – 1536 MHz - OOBE emission limits have been modified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Band 24 downlink transmission limitation is added in Table 4.4-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 24 requirements would be not aligned with the latest regulation updates. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: since there is no Band 24 product, proposed changes are from Rel’10. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Resubmission of endorsed Draft CR R4-2016891 |  |  |  |  |  |  |  |  |

## 4.4	Operating bands and band categories

MSR requirements are applicable for band definitions and band numbering as defined in the specifications TS 45.005 [6], TS25.104 [3], TS 25.105 [4] and TS 36.104 [5]. For the purpose of defining the MSR BS requirements, the operating bands are divided into three band categories as follows:

-	Band Category 1 (BC1): Bands for E-UTRA FDD and UTRA FDD operation

-	Band Category 2 (BC2): Bands for E-UTRA FDD, UTRA FDD and GSM/EDGE operation

-	Band Category 3 (BC3): Bands for E-UTRA TDD and UTRA TDD operation

NOTE:	For UTRA TDD, requirements in the present document cover the 1.28 Mcps UTRA TDD option.

The paired and unpaired bands for the three Band Categories are shown in Table 4.4-1 and 4.4-2, together with the corresponding E-UTRA, UTRA and GSM/EDGE band designations. In the present specification, the operating band of an MSR Base Stations is designated using the E-UTRA band number according to the tables.

Table 4.4-1: Paired bands in E-UTRA, UTRA and GSM/EDGE.

| MSR and E-UTRA Band number | UTRA Band number | GSM/EDGE Band designation | Uplink (UL) BS receive UE transmit |  |  | Downlink (DL) BS transmit UE receive |  |  | Band category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | I | - | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | 1 |
| 2 | II | PCS 1900 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | 2 |
| 3 | III | DCS 1800 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | 2 |
| 4 | IV | - | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | 1 |
| 5 | V | GSM 850 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz | 2 |
| 6 (1) | VI | - | 830 MHz | – | 840 MHz | 875 MHz | – | 885 MHz | 1 (1) |
| 7 | VII | - | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | 1 |
| 8 | VIII | E-GSM | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | 2 |
| 9 | IX | - | 1749.9 MHz | – | 1784.9 MHz | 1844.9 MHz | – | 1879.9 MHz | 1 |
| 10 | X | - | 1710 MHz | – | 1770 MHz | 2110 MHz | – | 2170 MHz | 1 |
| 11 | XI | - | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | 1 |
| 12 | XII | - | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | 1 |
| 13 | XIII | - | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | 1 |
| 14 | XIV | - | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | 1 |
| 15 | XV | - | Reserved |  |  | Reserved |  |  |  |
| 16 | XVI | - | Reserved |  |  | Reserved |  |  |  |
| 17 | - | - | 704 MHz | – | 716 MHz | 734 MHz | – | 746 MHz | 1 (2) |
| 18 | - | - | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | 1 (2) |
| 19 | XIX | - | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | 1 |
| 20 | XX | - | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | 1 |
| 21 | XXI | - | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | 1 |
| 22 | XXII | - | 3410 MHz | – | 3490 MHz | 3510 MHz | – | 3590 MHz | 1 |
| 23 | - | - | 2000 MHz | – | 2020 MHz | 2180 MHz | – | 2200 MHz | 1 (2) |
| 24 (3) | - | - | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | 1 (2) |
| 25 | XXV | - | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | 1 |
| NOTE 1: The band is for UTRA only. NOTE 2: The band is for E-UTRA only. NOTE 3: Void NOTE 4: Void NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: Void NOTE 9: Void NOTE 10: DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48. |  |  |  |  |  |  |  |  |  |