3GPP TSG-RAN WG4 Meeting #98-e	R4-2102501

# Electronic Meeting, Jan. 25-Feb. 5, 2021

Agenda Item:	10.1.1

Source: 		Qualcomm Incorporated

Title: 	Discussion on UE SEM for 6.425-7.125GHz and 10-10.5GHz

Document for:	Approval

# Introduction

In RAN#90-e meeting, the SI on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz was agreed to postpone by one quarter[1]. In RAN4 #97-e meeting, UE parameters including UE ACLR, ACS and  blocking were agreed in [2] but UE SEM is FFS.

In this paper, we provide our views on UE SEM for 6.425-7.125GHz and 10-10.5GHz.

# Discussion

The agreements on UE ACLR for 6.425-7.125GHz and 10-10.5GHz are 26dB and 24dB respectively [2]. Compared with PC3 ACLR in FR1 [3], 4dB is relaxed for 6.425-7.125GHz and 6dB is relaxed for 10-10.5GHz. In [4], Out of band emission specified in clause 6.5.2.2 of [4] is proposed to apply for both 6.425-7.125GHz and 10.0-10.5GHz frequency ranges. We should note that emission mask in clause 6.5.2.2 of [4]  is aligned with ACLR for PC3 UE in FR1, i.e., 30dB. Therefore, we need to check if NR FR1 emission mask.is still appropriate for the agreed ACLR, i.e., 26dB and 24dB. Due to lack of 6GHz and 10GHz PA model, the following simulation is conducted with 3.5GHz PA model. The SEM measurements results at FOOB1.5MHz for 20MHz CBW with full allocation is shown as Table 1 and Figure 1. Note that if we further consider smaller CBW or partial allocation, then SEM emission would be higher. For example, based on the spectral utilization, the peak SEM emission measured at the FOOB edge using MBW = 1% of channel BW will be for the 40MHz case using the 8 RBs at the edge of the channel BW. For this case peak SEM emission will fail due to increasing the power for relaxed ACLR.

Table 1: SEM at FOOB-1.5MHz dBm/MHz, CBW=20MHz

| Rel. P, dB | ACLR, dBc | SEM, dBm/MHz |
| --- | --- | --- |
| 0 | -30 | -16.8 |
| 0.75 | -27 | -12.9 |
| 1 | -26 | -11.6 |

Figure 1: SEM, ACLR vs power relative to MPR1, CBW=20MHz

Observation 1: The general NR FR1 SEM is not aligned with ACLR values that were agreed for 6.425-7.125GHz and 10-10.5GHz.

Observation 2: Relaxation is needed for general NR SEM to algin with agreed ACLR.

Some simulation was done with modified PA model for 3.5GHz. Through the preliminary simulation results, it can be seen that the SEM mask should be relaxed at the FOOB edge ± 0-1 by at least 3dB (-13dBm/1% BW to -10dBm/1% BW) to avoid additional MPR.

Proposal 1: The SEM mask for 6.425-7.125GHz and 10-10.5GHz should be relaxed at the FOOB edge ± 0-1 by at least 3dB (-13dBm/1% BW to -10dBm/1% BW) to avoid additional MPR.

Table 2: SEM for 6.425-7.125GHz and 10-10.5GHz

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 | [-10] | [-10] | [-10] | [-10] | [-10] | [-10] | [-10] |  |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  |  |  |  | -24 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 | -25 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  | -25 |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  | -25 |  |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  | -25 |  |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  | -25 |  |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  | -25 |  |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  | -25 |  |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  | -25 |  |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  | -25 |  |  |  |  |  |
| ± 65-70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 70-75 |  |  |  |  |  |  |  |  |  | -25 |  |  |  |  |
| ± 75-80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  |  | -25 |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  |  |  |  | -25 |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  |  |  | -25 |  |

# Conclusion

In this paper, we provide our views on UE SEM for 6.425-7.125GHz and 10-10.5GHz. We have the flowing observations and proposals:

Observation 1: The general NR FR1 SEM is not aligned with ACLR values that were agreed for 6.425-7.125GHz and 10-10.5GHz.

Observation 2: Relaxation is needed for general NR SEM to algin with agreed ACLR.

Proposal 1: The SEM mask for 6.425-7.125GHz and 10-10.5GHz should be relaxed at the FOOB edge ± 0-1 by at least 3dB (-13dBm/1% BW to -10dBm/1% BW) to avoid additional MPR.

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 | [-10] | [-10] | [-10] | [-10] | [-10] | [-10] | [-10] |  |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  |  |  |  | -24 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 | -25 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  | -25 |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  | -25 |  |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  | -25 |  |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  | -25 |  |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  | -25 |  |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  | -25 |  |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  | -25 |  |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  | -25 |  |  |  |  |  |
| ± 65-70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 70-75 |  |  |  |  |  |  |  |  |  | -25 |  |  |  |  |
| ± 75-80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  |  | -25 |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  |  |  |  | -25 |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  |  |  | -25 |  |

# 3.	References

- RP-202276, Revised SI on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz, Ericsson

- R4-2016905, WF on BS and UE parameters for 6.425-7.125 and 10.0-10.5 GHz, Nokia, CATT, Ericsson, ZTE, Huawei, Qualcomm

- 38.101-1

- R4-2015676, TP on UE IMT technology related parameters, Huawei, HiSilicon