3GPP TSG-RAN4 Meeting #98-e	R4-2102572

# Online, , 25th Jan 2020 – 5th	Feb 2021

Source: 	Huawei

Title: 	Regulatory aspects for the 600MHz range in APT region

Agenda Item:	12.4.2

Document for:	Discussion and agreement

# Introduction

During RAN#90e meeting, LS from APT (Asia-Pacific Telecommunity) was received in RP-202143 and related new SID on 600MHz range for APT was approved in RP-202924.

In this contribution we provide the inputs on the regulatory overview of 600 MHz range. Related TP to TR is provided for approval.

# Discussion

The 470 - 694 MHz frequency range is allocated to the broadcasting service and mobile service on a co-primary basis in Region 3. The frequency band 470 - 698 MHz, or parts thereof, was identified by WRC-15 in 7 countries in Region 3 through footnote No. 5.296A for use by those administrations as listed wishing to implement terrestrial IMT systems. In addition, there was interest from other significant markets to do the same. Elsewhere, USA, Mexico and several other countries in Region 2 also identified this band for IMT through footnotes 5.295 and 5.308A. It is noted that resolves 2 of Resolution 224 (Rev.WRC-15) encourages “administrations to take into account results of the existing relevant ITU-R studies, when implementing IMT applications/systems”.

The regulatory landscape overview for all regions in frequency range 470 – 694 MHz is provided in table below, based on ITU radio regulations.

Table: Frequency bands in the range 470 – 694 MHz in radio regulation

As captured in SID, since the regulatory study of frequency range around 600 MHz is for Region 3, the outcomes of this study item will not impact any requirements defined for US 600 MHz band, i.e. band n71.

It is proposed to capture the above information in the TR.

# Conclusions

Based on the discussion, it is proposed to agree in the following proposals:

Proposal 1: approve the attached TP to TR on regulatory aspects.

# References

[1]		RP-202143	LS on Frequency arrangements for IMT in the band 470-703MHz, Asia-Pacific Telecommunity (APT), RAN#90e

[2]		RP-202924	New SID: Study on extended 600MHz NR band, RAN#90e

# TP to TR 38.xxx (Regulatory aspects)

Based on the discussion above, the text proposal below is proposed for approval.

------------------------------ Modified section ------------------------------

# x	Regulatory landscape

## x.1	General

The 470 - 694 MHz frequency range is allocated to the broadcasting service and mobile service on a co-primary basis in Region 3 [1]. The frequency band 470 - 698 MHz, or parts thereof, was identified by WRC-15 in 7 countries in Region 3 through footnote No. 5.296A for use by those administrations as listed wishing to implement terrestrial IMT systems. In addition, there was interest from other significant markets to do the same. Elsewhere, USA, Mexico and several other countries in Region 2 also identified this band for IMT through footnotes 5.295 and 5.308A. It is noted that resolves 2 of Resolution 224 (Rev.WRC-15) encourages “administrations to take into account results of the existing relevant ITU-R studies, when implementing IMT applications/systems”.

The regulatory landscape overview for all regions in frequency range 470 – 694 MHz is provided in table x.1-1, based on ITU radio regulation [3].

Table x.1-1: Frequency bands in the range 470 – 694 MHz in radio regulation [3]

NOTE: 	Since regulatory study of frequency range around 600 MHz is for Region 3, the outcomes of this study item will not impact any requirements defined for US 600 MHz band, i.e. band n71.

5.149		In making assignments to stations of other services to which the bands: 13 360-13 410 kHz, 25 550-25 670 kHz, 37.5-38.25 MHz, 73-74.6 MHz in Regions 1 and 3, 150.05-153 MHz in Region 1, 322-328.6 MHz, 406.1-410 MHz, 608-614 MHz in Regions 1 and 3, 1 330-1 400 MHz, 1 610.6-1 613.8 MHz, 1 660-1 670 MHz, 1 718.8-1 722.2 MHz, 2 655-2 690 MHz, 3 260-3 267 MHz, 3 332-3 339 MHz, 3 345.8-3 352.5 MHz, 4 825-4 835 MHz, 4 950-4 990 MHz, 4 990-5 000 MHz, 6 650-6 675.2 MHz, 10.6-10.68 GHz, 14.47-14.5 GHz, 22.01-22.21 GHz, 22.21-22.5 GHz, 22.81-22.86 GHz, 23.07-23.12 GHz, 31.2-31.3 GHz, 31.5-31.8 GHz in Regions 1 and 3, 36.43-36.5 GHz, 42.5-43.5 GHz, 48.94-49.04 GHz, 76-86 GHz, 92-94 GHz, 94.1-100 GHz, 102-109.5 GHz, 111.8-114.25 GHz, 128.33-128.59 GHz, 129.23-129.49 GHz, 130-134 GHz, 136-148.5 GHz, 151.5-158.5 GHz, 168.59-168.93 GHz, 171.11-171.45 GHz, 172.31-172.65 GHz, 173.52-173.85 GHz, 195.75-196.15 GHz, 209-226 GHz, 241-250 GHz, 252-275 GHz are allocated, administrations are urged to take all practicable steps to protect the radio astronomy service from harmful interference. Emissions from spaceborne or airborne stations can be particularly serious sources of interference to the radio astronomy service (see Nos. 4.5 and 4.6 and Article 29). (WRC-07)

5.291	Additional allocation: in China, the band 470-485 MHz is also allocated to the space research (space-to-

Earth) and the space operation (space-to-Earth) services on a primary basis subject to agreement obtained under No. 9.21 and subject to not causing harmful interference to existing and planned broadcasting stations.

5.291A	Additional allocation: in Germany, Austria, Denmark, Estonia, Liechtenstein, the Czech Rep., Serbia and

Switzerland, the frequency band 470-494 MHz is also allocated to the radiolocation service on a secondary basis. This use is limited to the operation of wind profiler radars in accordance with Resolution 217 (WRC-97). (WRC-15)

5.296		Additional allocation: in Albania, Germany, Angola, Saudi Arabia, Austria, Bahrain, Belgium, Benin,

Bosnia and Herzegovina, Botswana, Bulgaria, Burkina Faso, Burundi, Cameroon, Vatican, Congo (Rep. of the), Côte d'Ivoire, Croatia, Denmark, Djibouti, Egypt, United Arab Emirates, Spain, Estonia, Eswatini, Finland, France, Gabon, Georgia, Ghana, Hungary, Iraq, Ireland, Iceland, Israel, Italy, Jordan, Kenya, Kuwait, Lesotho, Latvia, Lebanon, Libya, Liechtenstein, Lithuania, Luxembourg, North Macedonia, Malawi, Mali, Malta, Morocco, Mauritius, Mauritania, Moldova, Monaco, Mozambique, Namibia, Niger, Nigeria, Norway, Oman, Uganda, the Netherlands, Poland, Portugal, Qatar, the Syrian Arab Republic, Slovakia, the Czech Republic, Romania, the United Kingdom, Rwanda, San Marino, Serbia, Sudan, South Africa, Sweden, Switzerland, Tanzania, Chad, Togo, Tunisia, Turkey, Ukraine, Zambia and Zimbabwe, the frequency band 470-694 MHz is also allocated on a secondary basis to the land mobile service, intended for applications ancillary to broadcasting and programme-making. Stations of the land mobile service in the countries listed in this footnote shall not cause harmful interference to existing or planned stations operating in accordance with the Table in countries other than those listed in this footnote. (WRC-19)

5.296A	In Micronesia, the Solomon Islands, Tuvalu and Vanuatu, the frequency band 470-698 MHz, or portions thereof, and in Bangladesh, Maldives and New Zealand, the frequency band 610-698 MHz, or portions thereof, are identified for use by these administrations wishing to implement International Mobile Telecommunications (IMT) – see Resolution 224 (Rev.WRC-19). This identification does not preclude the use of these frequency bands by any application of the services to which they are allocated and does not establish priority in the Radio Regulations. The mobile allocation in this frequency band shall not be used for IMT systems unless subject to agreement obtained under No. 9.21 and shall not cause harmful interference to, or claim protection from, the broadcasting service of neighbouring countries. Nos. 5.43 and 5.43A apply. (WRC-19)

5.298	Additional allocation: in India, the band 549.75-550.25 MHz is also allocated to the space operation

service (space-to-Earth) on a secondary basis.

5.305	Additional allocation: in China, the band 606-614 MHz is also allocated to the radio astronomy service on a primary basis.

5.313A	The frequency band, or portions of the frequency band 698-790 MHz, in Australia, Bangladesh, Brunei

Darussalam, Cambodia, China, Korea (Rep. of), Fiji, India, Indonesia, Japan, Kiribati, Lao P.D.R., Malaysia, Myanmar (Union of), New Zealand, Pakistan, Papua New Guinea, the Philippines, the Dem. People’s Rep. of Korea, Solomon Islands, Samoa, Singapore, Thailand, Tonga, Tuvalu, Vanuatu and Viet Nam, are identified for use by these administrations wishing to implement International Mobile Telecommunications (IMT). This identification does not preclude the use of these frequency bands by any application of the services to which they are allocated and does not establish priority in the Radio Regulations. (WRC-19)

5.305	Additional allocation: in China, the band 606-614 MHz is also allocated to the radio astronomy service on a primary basis.

5.306	Additional allocation: in Region 1, except in the African Broadcasting Area (see Nos. 5.10 to 5.13), and

in Region 3, the band 608-614 MHz is also allocated to the radio astronomy service on a secondary basis.

5.307 Additional allocation: in India, the band 608-614 MHz is also allocated to the radio astronomy service on a primary basis.

5.313		(SUP - WRC-97)

5.313A 	The frequency band, or portions of the frequency band 698-790 MHz, in Australia, Bangladesh, Brunei

Darussalam, Cambodia, China, Korea (Rep. of), Fiji, India, Indonesia, Japan, Kiribati, Lao P.D.R., Malaysia, Myanmar (Union of), New Zealand, Pakistan, Papua New Guinea, the Philippines, the Dem. People’s Rep. of Korea, Solomon Islands, Samoa, Singapore, Thailand, Tonga, Tuvalu, Vanuatu and Viet Nam, are identified for use by these administrations wishing to implement International Mobile Telecommunications (IMT). This identification does not preclude the use of these frequency bands by any application of the services to which they are allocated and does not establish priority in the Radio Regulations. (WRC-19)

5.317		Additional allocation: in Region 2 (except Brazil, the United States and Mexico), the frequency band

806-890 MHz is also allocated to the mobile-satellite service on a primary basis, subject to agreement obtained under No. 9.21. The use of this service is intended for operation within national boundaries. (WRC-15)

5.317A 	The parts of the frequency band 698-960 MHz in Region 2 and the frequency bands 694-790 MHz in Region 1 and 790-960 MHz in Regions 1 and 3 which are allocated to the mobile service on a primary basis are identified for use by administrations wishing to implement International Mobile Telecommunications (IMT) – see Resolutions 224 (Rev.WRC-19), 760 (Rev.WRC-19) and 749 (Rev.WRC-19), where applicable. This identification does not preclude the use of these frequency bands by any application of the services to which they are allocated and does not establish priority in the Radio Regulations. (WRC-19)

5.320	Additional allocation: in Region 3, the bands 806-890 MHz and 942-960 MHz are also allocated to the

mobile-satellite, except aeronautical mobile-satellite (R), service on a primary basis, subject to agreement obtained under No. 9.21. The use of this service is limited to operation within national boundaries. In seeking such agreement, appropriate protection shall be afforded to services operating in accordance with the Table, to ensure that no harmful interference is caused to such services.

Editor’s note: refer to related WRC resolutions

## x.2	ITU Region 3

Editor’s note: refer to the APT LS; any APT specific regulatory matters

ITU Region 3 specific regulatory aspects for the frequency range 470 – 694 MHz are provided in this clause.

Referring to the APT LS, AWG-26 agreed to undertake the study to revise the APT/AWG/REP-79 [4] to develop frequency arrangements in the band 470-703 MHz for APT Members that wish to implement both the APT700 and a 600 MHz frequency arrangements that is optimal for APT Members.

------------------------------ End of modified section ------------------------------