3GPP TSG-RAN4 Meeting #98-e	R4-2102573

# Online, , 25th Jan 2020 – 5th	Feb 2021

Source: 	Huawei

Title: 	Initial considerations on the coexistence studies for 600MHz SI

Agenda Item:	12.4.3

Document for:	Discussion

# Introduction

During RAN#90e meeting, LS from APT (Asia-Pacific Telecommunity) was received in RP-202143 and related new SID on 600MHz range for APT was approved in RP-202924.

In this contribution we provide initial thoughts on the potential co-existence studies required for the extended 600MHz band in Region 3.

# Discussion

Referring to the SID in RP-202924, one of the objectives was defined as follows:

Co-existence study for the frequency range of 612-652/663-703 MHz such as with DTV (if needed)

Furthermore, the LS in RP-202143 captured some information on the expected co-existence scenario for the option B1:

Co-existence with existing services Co-existence with the following services will need study: - Broadcast television below 612 MHz and size of guard band required. We expect this will be similar to band n71 - Radio Astronomy: Radio astronomy is a receive only service that uses highly sensitive receivers. We expect IMT services using this band will need geographical separation with radio astronomy service receivers, but this will an Administration specific issue. - Unlicensed white space devices and Wireless microphones: Wireless microphones are permitted to operate even with band 71 in part of the duplex gap. This has certain limits proposed in 3GPP TR 36 755. These limits will not change. This proposal is not making any changes to the duplex gap.

The above text refers to the TR 36.755 (US 600 MHz Band for LTE) which was studying on the (later introduced) band 71/n71:

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |

One shall notice that that frequency arrangement option B1 is an extension of the reverse duplex based NR band n71. Option B1 extends band n71 upwards and downwards by 5 MHz.

Observation 1: before (potential) analysis on co-existence scenarios starts, RAN4 shall first conclude on the preferred frequency arrangement (B1, B2 or other) for the extended 600MHz band in Region 3.

Observation 2: TR 36.755 shall be used as the starting point for the (potential) co-existence studies in this SI, with consideration of Region 3 regulations on adjacent services.

Finally, it shall be noted that for the option B2, there was no RAS co-existence issue identified in the LS.

# Conclusions

In this contribution, initial observations were collected for the (potential) co-existence studies for the extended 600MHz band in Region 3. Further study is required.