3GPP TSG-RAN4 Meeting #98-e	R4-2102580

# Online, , 25th Jan 2020 – 5th	Feb 2021

Source: 	Huawei

Title: 	Consideration of duplex mode for the frequency bands for NR repeater

Agenda Item:	11.11.1

Document for:	Discussion

# Introduction

During RAN#90e meeting, new WID on the NR Repeaters was approved in RP-202927 [1].

In this contribution we provide analysis on the duplex mode for the NR Repeater frequency bands.

# Discussion

# Scope of the work for NR Repeater

Referring to the WID in RP-202927, the following objectives were specified for the Rel-17 NR Repeater in RAN4:

### 4.1 Objective of SI or Core part WI or Testing part WI Normative work phase objective [RAN4] - Specify RF and EMC requirements for NR repeaters - Consider FR1 (FDD and TDD) and FR2 (TDD) bands For the above objective, the leveraging of RF specifications for LTE repeater and IAB should be sought. It is assumed that the repeater does not perform adaptive beamforming towards the UE. It is assumed that the TDD repeater meets both BS and UE emission requirements (or the more stringent absolute level in dBm) in all slots. The following constraint may be considered for TDD repeaters, as needed, to contain the workload associated with this project: - TX EIRP/TRP/power of Repeater not exceeding any UE power class defined in the band Note: FR1 FDD repeaters testing is assumed to be conducted. This assumption can be revisited in RAN#91.

Observation 1: NR Repeater WID recognizes both FDD and TDD bands to be considered.

# Duplex mode considerations for E-UTRA repeater

Referring to the TS 36.106 (E-UTRA; FDD Repeater radio transmission and reception) section 5.5, the operating bands considered for the E-UTRA repeater were reusing those defined for E-UTRA base station, with the obvious limitation to the FDD duplex mode. Among the bands defined for E-UTRA repeater were bands 29 and 32. Based on information in TS 36.104, those are SDL bands for the CA operation:

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |  |  |
| 29 | N/A | 717 MHz | – | 728 MHz | FDD (NOTE 2) |
| 32 | N/A | 1452 MHz | – | 1496 MHz | FDD (NOTE 2) |
| NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |

As can be seen in the table above, E-UTRA specifications are denoting SDL bands with the “FDD” Duplex Mode. Observation 2: one can notice that SDL bands were included in the E-UTRA repeater specification, even though not explicitly called as “SDL”.

This is different than in the NR specifications, which are clearly distinguishing FDD, TDD, SDL and SUL. This can be found in the TS 38.104 (NR BS), as extracted below for some selected example bands:

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL |
| n76 | N/A | 1427 MHz – 1432 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n80 | 1710 MHz – 1785 MHz | N/A | SUL |
| n81 | 880 MHz – 915 MHz | N/A | SUL |
| n82 | 832 MHz – 862 MHz | N/A | SUL |
| n83 | 703 MHz – 748 MHz | N/A | SUL |
| n84 | 1920 MHz – 1980 MHz | N/A | SUL |
| n86 | 1710 MHz – 1780 MHz | N/A | SUL |
| n89 | 824 MHz – 849 MHz | N/A | SUL |
| n90 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n91 | 832 MHz – 862 MHz | 1427 MHz – 1432 MHz | FDD2 |
| n92 | 832 MHz – 862 MHz | 1432 MHz – 1517 MHz | FDD2 |
| n93 | 880 MHz – 915 MHz | 1427 MHz – 1432 MHz | FDD2 |
| n94 | 880 MHz – 915 MHz | 1432 MHz – 1517 MHz | FDD2 |
| n951 | 2010 MHz – 2025 MHz | N/A | SUL |
| NOTE 1: This band is applicable in China only. NOTE 2: Variable duplex operation does not enable dynamic variable duplex configuration by the network, and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band. |  |  |  |

NOTE: please note that SDL band 32 was not defined for NR.

Observation 3: NR specification explicitly distinguishes various duplex modes, including FDD, TDD, SDL and SUL.

# Conclusions

Based on the above discussion, the following is proposed:

Proposal 1: NR Repeater specification shall consider all the duplex modes considered in the NR work so far, i.e. FDD, TDD, SDL, SUL.

# References

[1]		RP-202927	New WID proposal for NR Repeaters, RAN#90-e

[2]		3GPP TS 36.106		E-UTRA; FDD Repeater radio transmission and reception