3GPP RAN WG4 Meeting #98e		R4-2102591

Electronic meeting, Jan. 25 - Feb. 5, 2021

Agenda item:	9.25.1.3

Source:	Apple Inc.

Title:	Peak EIS and EIS Spherical Coverage for band n262

WI/SI:	NR_47GHz_Band-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

The work item [1] agreed on the introduction of NR 47 GHz (47.2 to 48.2 GHz). Last meeting the WF [2] summarized the companies positions for minimum peak EIRP and REFSENS. The calculation of mean value scoped down the number to 16.5 mW and 16.1 dBm for min Peak EIRP and to -79.3 mW and -79.9 dBm for the REFSENS. In this contribution we share our view for the REFSENS and include our analysis for the EIS spherical coverage.

# 2	Discussion

For the estimation of the peak EIS we need to consider the impact on the Rx component when supporting a higher carrier frequency such as 47 GHz. In the receiver, the LNA is a key component since its NF dominates the overall noise performance of the receiver. The LNA should provide a wideband impedance matching to the antenna, gain, good linearity and low noise figure (NF). These parameters become more challenging when supporting higher carrier frequency and the impact is reflected in subsection 2.1.

## 2.1	REFSENS for n262

Collecting all the parameters for the REFSENS for n262, we have estimated the REFSENS budget -78.98 dBm/50 MHz.

| Parameter | Unit | Proposal |
| --- | --- | --- |
| Band number |  | n262 |
| Frequency range | GHz | 47.2 - 48.2 |
| SNR Requirement | dB | -1,00 |
| Bandwidth | MHz | 50,00 |
| Thermal Noise | dBm/Hz | -174,00 |
| Noise Figure | dB | 16,00 |
| # of antennas in an array |  | 4,00 |
| Average antenna element gain | dBi | 3,75 |
| Realized antenna array gain |  | 8,27 |
| Diversity Gain | dB | 0,00 |
| Antenna gain roll-off loss versus frequency | dB | -1,50 |
| Mismatch and transmission line loss |  | -3,30 |
| Beamforming Loss | dB | -2,00 |
| Total Insertion Loss | dB | -11,30 |
| Form factor Integration losses |  | -6,00 |
| REFSENS | dBm | -78,98 |

Proposal 2:	The single-band peak EIS requirement for band n262 is -78.98 dBm/50 MHz, -75.98 dBm/100 MHz, -72.98 dBm/200 MHz, -69.98 dBm/400 MHz

## 2.2	EIS Spherical Coverage

The WF for Spherical coverage encouraged companies to provide their view for the ‘gain drop’ along the 50th %tile direction and to consider 13.4 dB as potential value for the ‘gain drop’ for band n262. In our simulation for the spherical coverage CDF for n262 we have included n259 and the ideal model as references for comparison.

|  | Parameters; Ideal Reference Model; Band n259 model; Band n262 model. Amplitude error δak; 0; 1; 1.5. Phaser error δpk; 0˚; 15˚; 30˚. RSRP error ∆k; 0 dB; 0.67 dB; 2 dB. |
| --- | --- |

The simulation setup includes 2 panels with 4x1 array polarized patch antennas (as described in TR 38.803). The codebook considers 9 beams with 15-degree steps. The Table above provides a summary of parameters used as input for the simulation of the plotted curve of the spherical coverage CDF.

The simulation results show that for band n262 the gain drop between the value of the 100%-tile of the CDF and the 50%-tile of the CDF is -12 dB, whereas for band n259 the gain drop is -11.3 dB. The results between band n262 and n259 gives a delta of 0.7 dB. Since the mutual coupling, and the form factor integration were not part of simulation, we expect a deviation of around 1 dB to 1.5 dB from the simulation results. The outcome considering the delta will give a gain drop of around -13.5 dB for band n262 and around 12.8 dB for band n259.

Based on our simulation, we support the proposal from the WF to consider 13.4 dB as gain drop for the EIRP spherical coverage. Therefore, we propose the given values from Table 1 for the EIS at 50%-tile for power class 3, taking into account the peak EIS requirement of -78.98 dBm/50 MHz.

Table 1: EIS spherical coverage for power class 3

| Operating band | EIS at 50th %-tile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -77.4 | -74.4 | -71.4 | -68.4 |
| n258 | -77.4 | -74.4 | -71.4 | -68.4 |
| n259 | -71.9 | -68.9 | -65.9 | -62.9 |
| n260 | -73.1 | -70.1 | -67.1 | -64.1 |
| n261 | -77.4 | -74.4 | -71.4 | -68.4 |
| n262 | -65,6 | -62,6 | -59,6 | -56,6 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in Annex E.2.1. |  |  |  |  |

Proposal 2:	RAN4 shall consider for EIS spherical coverage for power class 3 for band n262 as provided in Table 1.

# 3	Conclusions

This contribution provides our view on the peak EIS considering the key parameters for the receiver, as well as the simulation results for the spherical CDF for band n262. In summary, we have made the following observations and proposals:

Proposal 1:	The single-band peak EIS requirement for band n262 is -78.98 dBm/50 MHz, -75.98 dBm/100 MHz, -72.98 dBm/200 MHz, -69.98 dBm/400 MHz

Proposal 2:	RAN4 shall consider for EIS spherical coverage for power class 3 for band n262 as provided in Table 1.

# 4	References

RP-201232, “New WID: introduction of NR 47 GHz band.”, T-Mobile USA, Dish Network, July 2020

R4-2016879, “WF on UE RF requirements of n262”, Qualcomm, Nokia, Sony, 3GPP RAN4 Meeting #97-e, November 2020