3GPP RAN WG4 Meeting #98-e		R4-2102891

Electronic meeting, 25th Jan – 5th Feb 2021

Agenda item:	8

Source:	Qualcomm Incorporated

Title:	UE feature on Simultaneous dormant BWP switching

Release:	Rel-16

Document for:	Decision

# 1	Introduction

During the previous RAN4#97e meeting, RAN4 made the following agreement [1].

Agreement:

- Option 1: Introduce a capability D’ for dormant BWP switching of multiple SCells that is separate from corresponding capability D for active BWP switching.

In order to reflect the agreed UE capability into the feature list accordingly, the following suggest is also included in the approved WF [1].

- For RAN4#98-e

Interested companies are encouraged to provide contributions addressing the late agreement:

  - Issue 1-2-3: Delay requirement for switching of multiple SCells between dormancy and non-dormancy

Introduce a capability D’ for dormant BWP switching of multiple SCells that is separate from corresponding capability D for active BWP switching.

# 2 Rel-16 RRM enhancements

As per the approved WF [1], we propose the follwoing:

Proposal 1: Add the following UE capability for simultaneous dormant BWP switching to the feature list.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6. LTE_NR_DC_CA_enh | 6-[X] | Dormant BWP switching on multiple CCs RRM requirements | Incremental delay for BWP switch processing on additional SCells in timer/DCI based simultaneous dormant BWP switching on multiple SCells | RAN1 feature 18-4 and 18-4a | Yes | N/A | There may be additional unclear BWP switching delay if network trigger dormant BWP switching on multiple SCells simultaneously. | Per UE | No | No | N/A | For component 2), the candidate values are: ● {100us, 200us} for UE indicates type1 in bwp-SwitchingDelay ● {200us, 400us, 800us, 1000us} for UE indicates type 2 in bwp-SwitchingDelay The total BWP switching delay will be captured in TS38.133 UE needs to indicate either of the candidate values in case it supports dormant BWP | Optional with capability signalling |

# 3	Conclusions

Proposal 1: Add the following UE capability for simultaneous dormant BWP switching to the feature list.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6. LTE_NR_DC_CA_enh | 6-[X] | Dormant BWP switching on multiple CCs RRM requirements | Incremental delay for BWP switch processing on additional SCells in timer/DCI based simultaneous dormant BWP switching on multiple SCells | RAN1 feature 18-4 and 18-4a | Yes | N/A | There may be additional unclear BWP switching delay if network trigger dormant BWP switching on multiple SCells simultaneously. | Per UE | No | No | N/A | For component 2), the candidate values are: ● {100us, 200us} for UE indicates type1 in bwp-SwitchingDelay ● {200us, 400us, 800us, 1000us} for UE indicates type 2 in bwp-SwitchingDelay The total BWP switching delay will be captured in TS38.133 UE needs to indicate either of the candidate values in case it supports dormant BWP | Optional with capability signalling |

# 4	References

R4-2017123, Way Forward on RRM Core requirements maintenance in MR-DC RRM 2, Ericsson, 3GPP RAN4#97e, Nov. 2020