3GPP TSG-RAN WG4 Meeting #98e 										   																					R4-2103857

Electronic Meeting, January 25th – February 5th, 2021

Agenda item:			7.4.3.4.3

Source:	Nokia, Nokia Shanghai Bell

Title:	WF on IAB-MT test coverage and IAB node test burden reduction

Document for:	Approval

# Introduction

During RAN4#98-e test coverage and test burden of IAB-Nodes were discussed in multiple contributions

- R4-2101962, ZTE Corporation

- R4-2102014, Nokia, Nokia Shanghai Bell

- R4-2102321, Ericsson

- R4-2102420, Huawei

Based on these inputs, discussion took place in thread [307]. This WF aims to capture the agreements reached during the discussion and set direction for the future.

# Way Forward

## Way Forward for different RF architectures

Test burden discussion shall concentrate on shared RF architecture. Test reduction cannot be applied when IAB-DU and IAB-MT are of different class.

## Way Forward on additional manufacturer declarations

A new declaration or multiple new declarations are needed. The following options exist

Option 1: It is declared that RF HW is shared between IAB-DU and IAB-MT

Option 2: It is declared that RF HW is the same between IAB-DU and IAB-MT

For both options, further study shall take place on additional side conditions. Study includes but is not limited to:

- relationship to other declarations, for example:

  - do output powers of IAB-MT and IAB-DU need to be declared to be equal?

  - do maximum steering directions of IAB-MT and IAB-DU need to be declared to be equal?

Only when the side conditions are fulfilled, it is possible to declare shared or same RF HW. The reduction is allowed only when declaration for shared or same RF HW is done. Detailed declaration wording shall be agreed in TP drafting phase.

## Way Forward on test reduction

To intention of test reduction is to minimize duplicated testing as much as possible when it does not bring value.

When test reduction is applied, combined test coverage of IAB-MT and IAB-DU needs to be at least as large as it is for IAB-DU when test reduction is not applied. At least following aspects can be considered:

- number of conformance directions

- number of RF channels

- Is IAB-MT or IAB-DU requirement more challenging than the other

- How to consider the TT impact is FFS (for the case of which IAB-DU and IAB-MT is different).

Test reduction cannot be applied for requirements which are defined for only IAB-DU or only IAB-MT.

As each requirement is unique, it is recommended to agree test coverage per requirement. Tables are provided below to help provide structured proposals. The content of the tables is provided as an example, and is not for approval.

Example table for radiated Tx requirements, the content is not for approval.

| Tx requirement |  | Classification | Number of | Number of conformance directions for IAB-MT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | conformance directions for IAB-DU |  | RF channels for IAB-DU | RF channels for IAB-MT | Comments |
| Radiated transmit power |  | Directional | 3 | 2 | B, T | M | Conformance directions for IAB-DU and IAB-MT shall be non-overlapping. Coverage of total number of directions and channels is equal to gNB test coverage. |
| OTA BS output power |  | TRP |  |  | B, T | M |  |
| OTA output power dynamics |  | Directional | 1 |  | M |  |  |
| OTA transmitter OFF power |  | Co-location | See clause 4.12 |  | M |  |  |
|  |  | Directional | 1 |  | M |  |  |
| OTA transient period |  | Co-location | See clause 4.12 |  | M |  |  |
|  |  | Directional | 1 |  | M |  |  |
| OTA modulation quality |  | Directional | 5 |  | B, T |  |  |
| OTA frequency error |  | Directional | 1 |  | B, T |  |  |
| OTA time alignment error |  | Directional | 1 | N/A | M | N/A | Test reduction not possible as requirement is not defined for IAB-MT |
| OTA occupied bandwidth |  | Directional | 1 |  | M |  |  |
| OTA ACLR |  | TRP | See annex I |  | B, T |  |  |
| OTA operating band unwanted emission |  | TRP | See annex I |  | B, M, T |  | It is sufficient to test only IAB-DU as the emission performance of CP-OFDM signal does not depend on whether the underlying data content is UL or DL. |
| OTA transmitter spurious emission | General requirement | TRP | See annex I |  | B below Tx signal frequency, T above Tx signal frequency |  |  |
|  | Additional spurious emissions | TRP | See annex I |  | B below Tx signal frequency, T above Tx signal frequency |  |  |
|  | Co-location with other base stations | Co-location | See clause 4.12 |  | M |  |  |
| OTA transmitter intermodulation |  | Co-location | See clause 4.12 |  | M |  | Requirement is applicable only for FR1. |

Example table for radiated Rx requirements, the content is not for approval.

| Rx requirement |  | Classification | Applicability levels |  | Number of conformance directions for IAB-DU | Number of conformance directions for IAB-MT | RF channels for IAB-DU | RF channels for IAB-MT | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR1 | FR2 |  |  |  |  |  |
| OTA sensitivity |  | Directional | Minimum EIS | N/A | 3 | 2 | M | M | Conformance directions for IAB-DU and IAB-MT shall be non-overlapping. |
| OTA reference sensitivity |  | Directional | OTA REFSENS | OTA REFSENS | 5 |  | B, M, T |  |  |
| OTA Dynamic range |  | Directional | OTA REFSENS | N/A | 1 | N/A | M | N/A | Test reduction not possible as requirement is not defined for IAB-MT |
| OTA adjacent channel selectivity |  | Directional | minSENS | OTA REFSENS | 1 |  | M |  |  |
| OTA in-band blocking |  | Directional | OTA REFSENS and minSENS | OTA REFSENS |  | 5 |  | M | It is sufficient to test only IAB-MT as CP-OFDM interfering signal has higher PAR and therefore the test case is more challenging |
| OTA out-of-band blocking | General requirement | Directional | minSENS | OTA REFSENS | M |  | M |  |  |
|  | Co-location with other base stations | Co-location | minSENS | N/A | M |  | M |  |  |
| OTA receiver spurious emissions |  | TRP | See clause 7.7 | See clause 7.7 | See annex I |  | B below Rx signal frequency, T above Rx signal frequency |  |  |
| OTA receiver intermodulation |  | Directional | OTA REFSENS and minSENS | OTA REFSENS | 1 |  | M |  | Requirement is specified only in FR1 |
| OTA in-channel selectivity |  | Directional | minSENS | OTA REFSENS | 1 |  | M |  |  |

Example table for conducted Tx requirements, the content is not for approval.

| Tx requirement |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | RF channels for IAB-DU | RF channels for IAB-MT | Comments |
| Maximum output power |  | B, T | M |  |
| Output power dynamics |  | M |  |  |
| Transmitter OFF power |  | M |  |  |
|  |  | M |  |  |
| Transient period |  | M |  |  |
|  |  | M |  |  |
| Modulation quality |  | B, M, T |  |  |
| Frequency error |  | B, M, T |  |  |
| Time alignment error |  | M | N/A | Test reduction not possible as requirement is not defined for IAB-MT |
| Occupied bandwidth |  | M |  |  |
| ACLR |  | B, M, T |  |  |
| Operating band unwanted emission |  | B, M, T |  |  |
| Transmitter spurious emission | General requirement | B below Tx signal frequency, T above Tx signal frequency |  |  |
|  | Additional spurious emissions | B below Tx signal frequency, T above Tx signal frequency |  |  |
|  | Co-location with other base stations | B below Tx signal frequency, T above Tx signal frequency |  |  |
| OTA transmitter intermodulation |  | M |  |  |

Example table for conducted Rx requirements, the content is not for approval.

| Rx requirement |  | RF channels for IAB-DU | RF channels for IAB-MT | Comments |
| --- | --- | --- | --- | --- |
| Reference sensitivity |  | B, M, T |  |  |
| Dynamic range |  | M | N/A | Test reduction not possible as requirement is not defined for IAB-MT |
| Adjacent channel selectivity |  | M |  |  |
| In-band blocking |  | M |  | It is sufficient to test only IAB-MT as CP-OFDM interfering signal has higher PAR and therefore the test case is more challenging |
| Out-of-band blocking | General requirement | M |  |  |
|  | Co-location with other base stations | M |  |  |
| Receiver spurious emissions |  | M |  |  |
| Receiver intermodulation |  | M |  |  |
| In-channel selectivity |  | M | N/A | Test reduction not possible as requirement is not defined for IAB-MT |

## Way Forward on capturing test reduction to TS

Agreements shall be done in TP drafting phase. At least following options exist for further consideration:

Option 1: Test reduction applicability with side conditions is captured in general clauses (clause 4)

Option 2: Test reduction is captured in initial conditions of each test

Option 3: Test reduction is captured in test procedures