3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2103980

# Electronic Meeting, Jan. 25-Feb. 5, 2021

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: Demod HARQ buffer soft combining test cases for NR V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL |  |  |  |  |  |  | Date: |  | 2021/01/15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add the demod HARQ buffer soft combining test cases for Rel-16 NR V2X |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rel-16 NR V2X demod HARQ buffer soft combining test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The demod HARQ buffer soft combining test cases for Rel-16 NR V2X not be added. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.6 (New), A.6.6 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 38.521-4 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections>>>

### 11.1.6	HARQ buffer soft combining test

#### 11.1.6.1 2Rx requirement

##### 11.1.6.1.1 Minimum requirement

The purpose of this test is to verify the maximum number of HARQ processes per TTI supported by the V2X UE.

The minimum requirement is specified in Table 11.1.6.1.1-2 with the test parameters specified in Table 11.1.6.1.1-1.

Table 11.1.6.1.1-1: Test Parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Communication resource pool configuration |  |  | As specified in Table A.x-x (Configuration xxx) |
| Active cell(s) |  |  | None |
| Active Sidelink UE(s) |  |  | Sidelink UE i, 0 ≤ i ≤ n (Note 1) |
| Sidelink UE i, 0 ≤ i ≤ n | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | Time gap between initial transmission and retransmission | Slots | [n (Note 2)] |
|  | Timing offset (Note 3) | s | [0] |
|  | Frequency offset (Note 4) | Hz | [0] |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 |
| PSFCH resource period |  | Slots | 1 |
| Note 1: When n = 16 or 24, sidelink UEs transmit one by one circularly for every slot; when n=32, the first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel and the 32nd UE transmits signal in the first slot and in the second subchannel; when n=48, the first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel, the next 17 UEs transmit signal one by one circularly for every slot and in the second subchannel; when n=64, first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel, the next 31 UEs transmit signal one by one circularly for every slot and in the second subchannel, the last 2 UEs transmit signal one by one circularly for every slot in the third subchannel. Note 2: n is the number of HARQ process UE can support (based on IE harq-RxProcessSidelink) Note 3: Time offset of Sidelink UE receive signal is with respect to GNSS reference timing. Note 4: Frequency offset of Sidelink UE is with respect to GNSS reference frequency. |  |  |  |

Table 11.1.6.1.1-2: Minimum performance

| Test num. | Bandwidth | PSSCH Reference channel | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | PSSCH BLER (%) | SNR (dB) of PSSCH |
| 1 | 20 MHz | CD.x | AWGN | 5 | [TBD] |

<<< End of changed sections>>>

<<< Start of changed sections>>>

# A.6.6 Reference measurement channels for HARQ buffer soft combining test

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.PSSCH 2-x.y |
| Channel bandwidth | MHz | 20 |
| Subchannel size | RB | 10 |
| PSSCH Symbols per slot |  | 7 |
| SCI-2 payload | Bits | 35 |
| BetaOffsets2ndSCI |  | 2.5 |
| Modulation |  | [TBD] |
| Transport Block Size | Bits | [TBD] |
| Transport block CRC | Bits | [TBD] |
| Number of PSSCH transmissions |  | 2 |
| Binary Channel Bits | Bits | [TBD] |

## <<< End of changed sections>>>