3GPP TSG-RAN WG4 Meeting#98-bis             	R4-2104406

E-meeting, 12th – 20th  April, 2021

Title:	TP to TR 37.717-21-11: DC_20-40_n1

Source:	Nokia, Telefonica

Agenda item:	7.4.2

Document for:	Approval

# 1	Introduction

This is a TP into TR 37.717-21-11 to introduce DC_20-40_n1.

*********************** Start of TP ***************************************

## 5.X	DC_20-40_n1

### 5.X.1	Operating bands for DC

Table 5.X.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_20-40_n1 | CA_20-40 | n1 | No |

### 5.X.2	Configurations for DC

Table 5.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_20A-40A_n1A DC_20A-40C_n1A | DC_20A_n1A DC_40A_n1A | CA_20A-40A CA_20A-40C | n1 |

### 5.X.3	Co-existence studies

Based on co-existence studies of Band 20 + Band n1 captured in 37.863-01-01 there is no IMD interfering band 40

And based on co-existence studies of Band 40 + Band n1 captured in 37.863-01-01, MSD shall be considered since

-	4th order IMD generated by dual uplink of the two bands may fall into own Rx of band 20.

### 5.X.4	∆TIB and ∆RIB values

It is proposed to re-use relaxation values from DC_1-20_n38 which is very similar.

Table 5.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-40-n1 | 20 | 0.3 |
|  | 40 | 0.5 |
|  | n1 | 0.5 |

Table 5.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20-40-n1 | 20 | 0 |
|  | 40 | 0 |
|  | n1 | 0 |

### 5.X.5	REFSENS requirements

It is proposed to re-use IMD4 MSD value from DC_1A_n8A-n40A which is very similar.

Table 5.X.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A-40A_n1A DC_20A-40C_n1A | 20 | 841 | 5 | 25 | 800 | 8.0 | IMD4 |
|  | 40 | 2330 | 5 | 25 | 2330 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |

*********************** End of TP ***************************************