3GPP TSG-RAN WG4 Meeting#98-bis             	R4-2104425

E-meeting, 12th – 20th  April, 2021

Title:	TP to TR 37.717-11-11: DC_71A_n71A

Source:	Nokia, T-Mobile USA

Agenda item:	7.3.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 37.717-11-11to introduce BCS1 for DC_71A_n71A

****************************Start of the TP ****************************

### 6.X.1	DC_71A_n71A

#### 6.X.1.1	Operating bands for DC

Table 6.X.1.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_71_n71 | 71 | n71 | yes |

#### 6.x.1.2	Configuration for DC

Table 6.X.1.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ENDC configuration | Uplink ENDC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_71A_n71A | DC_71A_n71A2 | 5,10,15, 20 | 5,10,15,20 |  | 30 | 1 |
|  |  |  | 5,10,15, 20 | 5,10,15, 20 |  |  |
| Note 2: Only single switched UL is supported. |  |  |  |  |  |  |

#### 6.x.1.3	Maximum output power for DC

Already specified in TS 38.101-3.

#### 6.x.1.4	Spurious emission band UE co-existence for DC

Already specified in TS 38.101-3.

#### 6.1.1.5	MSD analysis for DC

As new channel bandwidths are introduced MSD definition for 30 MHz aggregated BW is needed.

Table 6.69.5-1: MSD for the DC_71A_n71A configuration

| DC configuration | Aggregated bandwidth |  | Wgap / (MHz) | UL NR allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | NR | E-UTRA |  |  |  |  |
| DC_71A_n71A | 5 MHz | 5 MHz | 5 < Wgap ≤ 25 | 5 | 4.0 | FDD |
|  |  |  | 0 < Wgap ≤ 5 | 20 | 0 |  |
|  | 10 MHz | 5 MHz | 5 < Wgap ≤ 20 | 513 | 4.6 |  |
|  |  |  | 0 < Wgap ≤ 5 | 2013 | 2.3 |  |
|  | 10 MHz | 10 MHz | 5 < Wgap ≤ 15 | 513 | 4.3 |  |
|  |  |  | 0 < Wgap ≤ 5 | 2013 | 3.2 |  |
|  | 15 MHz | 10 MHz | 5 < Wgap ≤ 10 | 514 | 22.2 |  |
|  |  |  | 0 < Wgap ≤ 5 | 2015 | 5.2 |  |
|  | 20 MHz | 10 MHz | Wgap = 5 | 514 | [22.2] 16 |  |
|  | 10 MHz | 20 MHz |  | 514 | TBD16 |  |
|  | 15 MHz | 15 MHz |  | 514 | TBD16 |  |
| NOTE 13: 13 Uplink resource block starts at RB position 9 for SCS=15KHz. NOTE 14: 14 Uplink resource block starts at RB position 2 for SCS=15KHz. NOTE 15: 15 Uplink resource block starts at RB position 19 for SCS=15KHz. NOTE 16: 16 Applicaple for BCS1 |  |  |  |  |  |  |

#### 6.x.1.6	∆TIB and ∆RIB values

Already specified in TS 38.101-3.

****************************End of the TP ****************************