3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2104436

Online Meeting, 12 April – 20 April, 2021

Third Generation Partnership Project (3GPP™)

Meeting Report
for
TSG RAN WG4
meeting: 98-e

Electronic Meeting, Online, 25/01/2021 to 05/02/2021

Report generated on Monday, 2021-02-15 14:23  UTC

Contents:

1	Opening of the E-meeting	14

2	Approval of the agenda	14

3	Letters / reports from other groups / meetings	15

4	Rel-15 New radio access technology	18

4.1	System Parameters Maintenance [NR_newRAT-Core]	18

4.2	UE RF requirements maintenance [NR_newRAT]	20

4.2.1	[FR1] Maintenance for 38.101-1 [NR_newRAT-Core]	20

4.2.2	[FR2] Maintenance for 38.101-2 [NR_newRAT-Core]	32

4.2.3	Maintenance for 38.101-3 [NR_newRAT-Core]	40

4.3	UE EMC requirements maintenance [NR_newRAT-Core]	48

4.4	BS RF requirements maintenance [NR_newRAT-Core]	51

4.4.1	General [NR_newRAT-Core]	51

4.4.2	TX/RX requirements maintenance (38.104) [NR_newRAT-Core]	53

4.4.3	MSR specifications maintenance [NR_newRAT-Core/Perf]	58

4.5	BS conformance testing Maintenance [NR_newRAT-Perf]	65

4.5.1	General [NR_newRAT-Perf]	65

4.5.2	Conducted conformance testing (38.141-1) [NR_newRAT-Perf]	69

4.5.3	Radiated conformance testing (38.141-2) [NR_newRAT-Perf]	70

4.5.4	eAAS specifications maintenance [NR_newRAT-Core/Perf]	75

4.6	BS EMC requirements Maintenance [NR_newRAT-Core]	79

4.6.1	Core requirements [NR_newRAT-Core]	79

4.6.2	Performance requirements [NR_newRAT-Perf]	79

4.7	RRM core requirements maintenance (38.133/36.133) [NR_newRAT-Core]	81

4.8	RRM perf. requirements maintenance (38.133/36.133) [NR_newRAT-Perf]	95

4.9	Demodulation and CSI requirements maintenance (38.101-4/38.104)[NR_newRAT-Perf]	121

4.9.1	UE demodulation requirements[NR_newRAT-Perf]	121

4.9.2	CSI requirements [NR_newRAT-Perf]	123

4.9.3	BS demodulation requirements [NR_newRAT-Perf]	124

4.10	Positioning specs maintenance (36.171, 37.171 and 38.171) [NR_newRAT-Perf or TEI]	128

4.11	Testability Maintenance (38.810) [FS_NR_test_methods]	129

5	LTE maintenance (up to Rel15) [WI code or TEI]	129

5.1	BS RF requirements [WI code or TEI]	138

5.2	UE RF requirements [WI code or TEI]	139

5.3	RRM requirements  [WI code or TEI]	143

5.4	Demodulation and CSI requirements  [WI code or TEI]	143

5.4.1	UE demodulation and CSI requirements [WI code or TEI]	143

5.4.2	BS demodulation requirements [WI code or TEI]	143

6	Rel-16 Work Items for LTE	143

6.1	Additional MTC enhancements for LTE [LTE_eMTC5]	143

6.1.1	RF core requirements maintenance [LTE_eMTC5-Core]	143

6.1.2	RRM requirements maintenance [LTE_eMTC5-Core/Perf]	143

6.2	Additional enhancements for NB-IoT [NB_IOTenh3]	146

6.2.1	RF core requirements maintenance [NB_IOTenh3-Core]	146

6.2.2	RRM requirements maintenance [NB_IOTenh3-Core/Perf]	146

6.3	Even further Mobility enhancement in E-UTRAN [LTE_feMob]	147

6.3.1	RRM core requirements maintenance [LTE_feMob-Core]	147

6.3.2	RRM perf. requirements [LTE_feMob-Perf]	149

6.3.2.1	General [LTE_feMob-Perf]	149

6.3.2.2	Test cases [LTE_feMob-Perf]	149

6.4	R16 LTE maintenance [WI code]	151

6.4.1	BS RF requirements [WI code]	151

6.4.2	UE RF requirements [WI code]	151

6.4.3	RRM requirements [WI code]	152

6.4.4	Demodulation and CSI requirements [WI code]	153

6.4.4.1	UE demodulation and CSI requirements [WI code]	153

6.4.4.2	BS demodulation requirements [WI code]	153

7	Rel-16 non-spectrum related work items for NR	153

7.1	NR-based access to unlicensed spectrum [NR_unlic]	153

7.1.1	System parameters maintenance [NR_unlic-Core]	156

7.1.2	UE RF requirements maintenance [NR_unlic-Core]	159

7.1.2.1	Transmitter characteristics [NR_unlic-Core]	159

7.1.2.2	Receiver characteristics [NR_unlic-Core]	160

7.1.3	BS RF requirements maintenance  [NR_unlic-Core]	160

7.1.3.1	General  [NR_unlic-Core]	160

7.1.3.2	Transmitter characteristics  [NR_unlic-Core]	160

7.1.3.3	Receiver characteristics  [NR_unlic-Core]	161

7.1.4	BS conformance testing [NR_unlic-Perf]	162

7.1.4.1	General  [NR_unlic-Perf]	163

7.1.4.2	Transmitter characteristics  [NR_unlic-Perf]	163

7.1.4.3	Receiver characteristics  [NR_unlic-Perf]	165

7.1.5	RRM core requirements maintenance (38.133) [NR_unlic-Core]	165

7.1.5.1	General [NR_unlic-Core]	171

7.1.5.2	RRC connection mobility control [NR_unlic-Core]	173

7.1.5.3	SCell activation/deactivation (delay and interruption) [NR_unlic-Core]	175

7.1.5.4	Active TCI state switching [NR_unlic-Core]	178

7.1.5.5	RLM [NR_unlic-Core]	178

7.1.5.6	Beam management [NR_unlic-Core]	179

7.1.5.7	Measurement requirements [NR_unlic-Core]	181

7.1.5.8	Measurement capability and reporting criteria [NR_unlic-Core]	182

7.1.5.9	Timing [NR_unlic-Core]	183

7.1.5.10	Other requirements  [NR_unlic-Core]	184

7.1.6	RRM perf. requirements (38.133) [NR_unlic-Perf]	187

7.1.6.1	General [NR_unlic-Perf]	196

7.1.6.2	Common RRM test configuration [NR_unlic-Perf]	198

7.1.6.3	Test cases [NR_unlic-Perf]	200

7.1.6.3.1	General [NR_unlic-Perf]	200

7.1.6.3.2	RRC IDLE, cell re-selection [NR_unlic-Perf]	201

7.1.6.3.3	HO delay and interruptions [NR_unlic-Perf]	202

7.1.6.3.4	RRC Re-establishment [NR_unlic-Perf]	203

7.1.6.3.5	RRC Connection Release with Redirection [NR_unlic-Perf]	204

7.1.6.3.6	Timing (transmit timing and TA)  [NR_unlic-Perf]	204

7.1.6.3.7	BWP switching delay and interruptions [NR_unlic-Perf]	206

7.1.6.3.8	PSCell addition/release (delay and interruption)  [NR_unlic-Perf]	207

7.1.6.3.9	Interruptions [NR_unlic-Perf]	208

7.1.6.3.10	RLM [NR_unlic-Perf]	208

7.1.6.3.11	Beam management [NR_unlic-Perf]	209

7.1.6.3.12	Intra-frequency, inter-frequency and inter-RAT measurement requirements [NR_unlic-Perf]	210

7.1.6.3.13	Accuracy requirements for NR-U intra-frequency, inter-frequency and inter-RAT measurements [NR_unlic-Perf]	212

7.1.7	Demodulation and CSI requirements (38.101-4/38.104) [NR_unlic-Perf]	214

7.1.7.1	General [NR_unlic-Perf]	214

7.1.7.2	UE demodulation requirements [NR_unlic-Perf]	221

7.1.7.3	CSI requirements [NR_unlic-Perf]	222

7.1.7.4	BS demodulation requirements [NR_unlic-Perf]	223

7.1.7.4.1	General [NR_unlic-Perf]	223

7.1.7.4.2	PUSCH requirements [NR_unlic-Perf]	223

7.1.7.4.3	PUCCH requirements [NR_unlic-Perf]	225

7.1.7.4.4	PRACH requirements [NR_unlic-Perf]	227

7.2	NR mobility enhancement  [NR_Mob_enh]	228

7.2.1	RRM requirements maintenance (38.133) [NR_Mob_enh-Core/Perf]	228

7.3	5G V2X with NR sidelink  [5G_V2X_NRSL]	232

7.3.1	System parameters maintenance  [5G_V2X_NRSL-Core]	232

7.3.2	UE RF requirements maintenance [5G_V2X_NRSL-Core]	232

7.3.2.1	Transmitter characteristics  [5G_V2X_NRSL-Core]	233

7.3.2.2	Receiver characteristics  [5G_V2X_NRSL-Core]	235

7.3.3	Concurrent operation maintenance (scenarios, requirements, etc) [5G_V2X_NRSL-Core]	235

7.3.3.1	Transmitter characteristics  [5G_V2X_NRSL-Core]	235

7.3.3.2	Receiver characteristics  [5G_V2X_NRSL-Core]	237

7.3.4	RRM core requirements maintenance (38.133) [5G_V2X_NRSL-Core]	237

7.3.5	RRM perf. requirements (38.133) [5G_V2X_NRSL-Perf]	238

7.3.5.1	General [5G_V2X_NRSL-Perf]	239

7.3.5.2	L1 SL-RSRP measurement accuracy [5G_V2X_NRSL-Perf]	239

7.3.5.3	Test cases [5G_V2X_NRSL-Perf]	239

7.3.5.3.1	UE transmit timing [5G_V2X_NRSL-Perf]	239

7.3.5.3.2	Initiation/Cease of SLSS Transmission [5G_V2X_NRSL-Perf]	239

7.3.5.3.3	Selection / Reselection of V2X Synchronization Reference Source  [5G_V2X_NRSL-Perf]	239

7.3.5.3.4	L1 SL-RSRP measurements [5G_V2X_NRSL-Perf]	239

7.3.5.3.5	Congestion control measurements [5G_V2X_NRSL-Perf]	239

7.3.5.3.6	Interruptions [5G_V2X_NRSL-Perf]	240

7.3.5.3.7	Resource Pre-emption [5G_V2X_NRSL-Perf]	240

7.3.5.3.8	Resource Re-evaluation [5G_V2X_NRSL-Perf]	240

7.3.5.3.9	Others  [5G_V2X_NRSL-Perf]	240

7.3.6	Demodulation requirements (38.101-4) [5G_V2X_NRSL-Perf]	240

7.3.6.1	General [5G_V2X_NRSL-Perf]	240

7.3.6.2	Single link test  [5G_V2X_NRSL-Perf]	247

7.3.6.2.1	PSSCH demodulation test [5G_V2X_NRSL-Perf]	248

7.3.6.2.2	PSCCH demodulation test  [5G_V2X_NRSL-Perf]	249

7.3.6.2.3	PSBCH demodulation test [5G_V2X_NRSL-Perf]	250

7.3.6.2.4	PSFCH demodulation test  [5G_V2X_NRSL-Perf]	251

7.3.6.3	Multiple link test [5G_V2X_NRSL-Perf]	252

7.3.6.3.1	Power imbalance requirement [5G_V2X_NRSL-Perf]	253

7.3.6.3.2	HARQ soft buffer combing test  [5G_V2X_NRSL-Perf]	254

7.3.6.3.3	PSFCH decoding capability test [5G_V2X_NRSL-Perf]	254

7.3.6.3.4	PSCCH/PSSCH decoding capability  [5G_V2X_NRSL-Perf]	256

7.3.6.3.5	Others  [5G_V2X_NRSL-Perf]	256

7.4	Integrated Access and Backhaul for NR [NR_IAB]	256

7.4.1	General [NR_IAB-Core]	258

7.4.1.1	System parameters maintenance [NR_IAB-Core]	261

7.4.1.2	Others  [NR_IAB-Core]	261

7.4.2	RF requirements maintenance [NR_IAB-Core]	262

7.4.2.1	Transmitter characteristics [NR_IAB-Core]	262

7.4.2.1.1	Tx Power related requirements  [NR_IAB-Core]	262

7.4.2.1.2	Transmitted signal quality [NR_IAB-Core]	263

7.4.2.1.3	Unwanted emissions [NR_IAB-Core]	264

7.4.2.1.4	Others [NR_IAB-Core]	264

7.4.2.2	Receiver characteristics [NR_IAB-Core]	265

7.4.2.2.1	Sensitivity and dynamic range requirements  [NR_IAB-Core]	265

7.4.2.2.2	In-band selectivity and blocking requirements  [NR_IAB-Core]	265

7.4.2.2.3	Others [NR_IAB-Core]	266

7.4.3	RF conformance testing [NR_IAB-Perf]	267

7.4.3.1	General and work plan [NR_IAB-Perf]	267

7.4.3.2	Common test issues for conducted and radiated conformance testing [NR_IAB-Perf]	277

7.4.3.2.1	Test configurations [NR_IAB-Perf]	277

7.4.3.2.2	Test models [NR_IAB-Perf]	278

7.4.3.2.3	Others [NR_IAB-Perf]	279

7.4.3.3	Conducted conformance testing [NR_IAB-Perf]	280

7.4.3.3.1	Transmitter characteristics [NR_IAB-Perf]	280

7.4.3.3.2	Receiver characteristics [NR_IAB-Perf]	281

7.4.3.3.3	Other test issues  [NR_IAB-Perf]	281

7.4.3.4	Radiated conformance testing [NR_IAB-Perf]	282

7.4.3.4.1	Transmitter characteristics [NR_IAB-Perf]	282

7.4.3.4.2	Receiver characteristics [NR_IAB-Perf]	282

7.4.3.4.3	Other test issues  [NR_IAB-Perf]	283

7.4.4	RRM core requirements maintenance [NR_IAB-Core]	283

7.4.5	RRM perf. requirements [NR_IAB-Perf]	285

7.4.5.1	General [NR_IAB-Perf]	286

7.4.5.2	Test cases [NR_IAB-Perf]	287

7.4.6	EMC core requirements maintenance [NR_IAB-Core]	290

7.4.6.1	General [NR_IAB-Core]	290

7.4.6.2	Emission requirements [NR_IAB-Core]	290

7.4.6.3	Immunity requirements [NR_IAB-Core]	290

7.4.7	EMC performance requirements [NR_IAB-Perf]	291

7.4.8	Demodulation and CSI requirements  [NR_IAB-Perf]	293

7.4.8.1	General  [NR_IAB-Perf]	293

7.4.8.2	IAB-DU performance requirements [NR_IAB-Perf]	294

7.4.8.3	IAB-MT performance requirements [NR_IAB-Perf]	295

7.5	Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements  [LTE_NR_DC_CA_enh]	295

7.5.1	RF requirements maintenance  [LTE_NR_DC_CA_enh-Core]	300

7.5.2	RRM core requirements maintenance (38.133/36.133) [LTE_NR_DC_CA_enh-Core]	304

7.5.2.1	Early Measurement reporting  [LTE_NR_DC_CA_enh-Core]	304

7.5.2.2	Efficient and low latency serving cell configuration, activation and setup [LTE_NR_DC_CA_enh-Core]	306

7.5.3	RRM perf. requirements (38.133) [LTE_NR_DC_CA_enh-Perf]	313

7.5.3.1	Early Measurement reporting  [LTE_NR_DC_CA_enh- Perf]	313

7.5.3.1.1	Accuracy requirements  [LTE_NR_DC_CA_enh-Perf]	313

7.5.3.1.2	Test cases [LTE_NR_DC_CA_enh-Perf]	314

7.5.3.2	Efficient and low latency serving cell configuration, activation and setup [LTE_NR_DC_CA_enh-Perf]	316

7.5.3.2.1	Test cases for direct SCell activation [LTE_NR_DC_CA_enh-Perf]	316

7.5.3.2.2	Test case for SCell Dormancy [LTE_NR_DC_CA_enh-Perf]	319

7.6	UE power saving in NR  [NR_UE_pow_sav]	323

7.6.1	RRM requirements maintenance (38.133) [NR_UE_pow_sav-Core/Perf]	323

7.6.2	Demodulation and CSI requirements (38.101-4)  [NR_UE_pow_sav-Perf]	328

7.7	NR Positioning Support [NR_pos]	334

7.7.1	RRM core requirements maintenance (38.133) [NR_pos-Core]	334

7.7.1.1	PRS-RSTD measurement requirements [NR_pos-Core]	338

7.7.1.2	PRS-RSRP measurement requirements [NR_pos-Core]	342

7.7.1.3	UE Rx-Tx time difference measurement requirements  [NR_pos-Core]	345

7.7.1.4	Other requirements [NR_pos-Core]	349

7.7.2	RRM perf. requirements (38.133) [NR_pos-Perf]	352

7.7.2.1	General [NR_pos-Perf]	352

7.7.2.2	UE requirements and test cases [NR_pos-Perf]	353

7.7.2.2.1	Measurement accuracy requirements [NR_pos-Perf]	356

7.7.2.2.1.1	PRS RSTD  [NR_pos-Perf]	356

7.7.2.2.1.2	PRS RSRP  [NR_pos-Perf]	358

7.7.2.2.1.3	UE Rx-Tx time difference  [NR_pos-Perf]	360

7.7.2.2.2	Test cases [NR_pos-Perf]	361

7.7.2.2.3	Measurement requirements [NR_pos-Perf]	363

7.7.2.2.4	Accuracy requirements [NR_pos-Perf]	365

7.7.2.2.5	Other [NR_pos-Perf]	366

7.7.2.3	gNB requirements  [NR_pos-Perf]	368

7.7.2.3.1	General [NR_pos-Perf]	368

7.7.2.3.2	SRS-RSRP requirements [NR_pos-Perf]	369

7.7.2.3.3	gNB Rx-Tx time difference requirements [NR_pos-Perf]	371

7.7.2.3.4	UL RTOA requirements [NR_pos-Perf]	372

7.8	Physical layer enhancements for NR URLLC [NR_L1enh_URLLC-Core]	374

7.8.1	Demodulation and CSI requirements (38.101-4/38.104) [NR_L1enh_URLLC-Perf]	374

7.8.1.1	Performance requirements with ultra-low BLER [NR_L1enh_URLLC-Perf]	374

7.8.1.1.1	UE demodulation requirements [NR_L1enh_URLLC-Perf]	376

7.8.1.1.2	CSI requirements  [NR_L1enh_URLLC-Perf]	377

7.8.1.1.3	BS demodulation requirements [NR_L1enh_URLLC-Perf]	380

7.8.1.2	Performance requirements with higher BLER [NR_L1enh_URLLC-Perf]	382

7.8.1.2.1	UE demodulation requirements [NR_L1enh_URLLC-Perf]	386

7.8.1.2.2	BS demodulation requirements [NR_L1enh_URLLC-Perf]	390

7.9	Enhancements on MIMO for NR  [NR_eMIMO]	398

7.9.1	UE RF core requirements maintenance (38.101)  [NR_eMIMO-Core]	400

7.9.2	RRM core requirements maintenance (38.133) [NR_eMIMO-Core]	400

7.9.3	RRM perf. requirements (38.133) [NR_eMIMO-Perf]	404

7.9.3.1	General [NR_eMIMO-Perf]	406

7.9.3.2	L1-SINR measurement accuracy [NR_eMIMO-Perf]	406

7.9.3.3	Test cases [NR_eMIMO-Perf]	408

7.9.3.3.1	L1-SINR measurements [NR_eMIMO-Perf]	408

7.9.3.3.2	BFR for SCell [NR_eMIMO-Perf]	409

7.9.3.3.3	DL/UL beam indication with reduced latency and overhead [NR_eMIMO-Perf]	410

7.9.3.3.4	Others [NR_eMIMO-Perf]	411

7.9.4	Demodulation and CSI requirements (38.101-4) [NR_eMIMO-Perf]	412

7.9.4.1	General [NR_eMIMO-Perf]	412

7.9.4.2	Demodulation requirements [NR_eMIMO-Perf]	418

7.9.4.2.1	Single-DCI based SDM scheme [NR_eMIMO-Perf]	418

7.9.4.2.2	Multi-DCI based transmission scheme  [NR_eMIMO-Perf]	419

7.9.4.2.3	Single-DCI based transmission schemes (URLLC) [NR_eMIMO-Perf]	421

7.9.4.3	CSI requirements [NR_eMIMO-Perf]	423

7.10	Add support of NR DL 256QAM for FR2 [NR_DL256QAM_FR2]	426

7.10.1	Demodulation and CSI requirements (38.101-4)  [NR_DL256QAM_FR2-Perf]	426

7.10.1.1	UE Demodulation requirements [NR_DL256QAM_FR2-Perf]	426

7.10.1.2	CSI requirements [NR_DL256QAM_FR2-Perf]	432

7.10.1.3	SDR requirements [NR_DL256QAM_FR2-Perf]	434

7.11	RF requirements for NR frequency range 1 (FR1) [NR_RF_FR1]	435

7.11.1	RF core requirements maintenance [NR_RF_FR1-Core]	435

7.11.1.1	Intra-band UL CA for FR1 power class 3  [NR_RF_FR1-Core]	436

7.11.1.2	Others [NR_RF_FR1-Core]	437

7.11.2	RRM requirements maintenance (38.133) [NR_RF_FR1-Core/Perf]	441

7.12	NR RF requirement enhancements for frequency range 2 (FR2) [NR_RF_FR2_req_enh]	443

7.12.1	RF core requirements maintenance [NR_RF_FR2_req_enh-Core]	443

7.12.2	RRM requirements maintenance (38.133) [NR_RF_FR2_req_enh-Core]	446

7.13	NR RRM requirement enhancement [NR_RRM_Enh-Core]	446

7.13.1	RRM core requirements maintenance (38.133) [NR_RRM_Enh-Core]	457

7.13.1.1	Multiple Scell activation/deactivation  [NR_RRM_Enh-Core]	457

7.13.1.2	BWP switching on multiple CCs [NR_RRM_Enh-Core]	458

7.13.1.3	Other requirements maintenance [NR_RRM_Enh-Core]	460

7.13.2	RRM perf. requirements (38.133) [NR_RRM_Enh-Perf]	468

7.13.2.1	General [NR_RRM_Enh-Perf]	468

7.13.2.2	Test cases [NR_RRM_Enh-Perf]	469

7.13.2.2.1	SRS carrier switching requirements  [NR_RRM_Enh-Perf]	469

7.13.2.2.2	Multiple Scell activation/deactivation  [NR_RRM_Enh-Perf]	471

7.13.2.2.3	CGI reading requirements with autonomous gap [NR_RRM_Enh-Perf]	471

7.13.2.2.4	BWP switching on multiple CCs [NR_RRM_Enh-Perf]	472

7.13.2.2.5	Inter-frequency measurement requirement without MG [NR_RRM_Enh-Perf]	474

7.13.2.2.6	Mandatory MG patterns  [NR_RRM_Enh-Perf]	475

7.13.2.2.7	UE-specific CBW change [NR_RRM_Enh-Perf]	476

7.13.2.2.8	Spatial relation switch for uplink [NR_RRM_Enh-Perf]	476

7.13.2.2.9	Inter-band CA requirement for FR2 UE measurement capability of independent Rx beam [NR_RRM_Enh-Perf]	477

7.14	NR RRM requirements for CSI-RS based L3 measurement [NR_CSIRS_L3meas]	477

7.14.1	RRM core requirements maintenance (38.133)  [NR_CSIRS_L3meas-Core]	483

7.14.2	RRM perf. requirements (38.133)  [NR_CSIRS_L3meas-Perf]	489

7.14.2.1	General [NR_CSIRS_L3meas-Perf]	490

7.14.2.1.1	CSI-RSRP requirements [NR_CSIRS_L3meas-Perf]	491

7.14.2.1.2	CSI-RSRQ requirements [NR_CSIRS_L3meas-Perf]	493

7.14.2.1.3	CSI-SINR requirements [NR_CSIRS_L3meas-Perf]	495

7.14.2.2	Test cases [NR_CSIRS_L3meas-Perf]	497

7.14.2.2.1	General [NR_CSIRS_L3meas-Perf]	498

7.14.2.2.2	Intra-frequency measurement [NR_CSIRS_L3meas-Perf]	499

7.14.2.2.3	Inter-frequency measurement [NR_CSIRS_L3meas-Perf]	500

7.14.2.2.4	Measurement performance [NR_CSIRS_L3meas-Perf]	502

7.15	NR support for high speed train scenario [NR_HST]	505

7.15.1	RRM requirements maintenance (38.133) [NR_HST-Core/Perf]	505

7.15.2	Demodulation and CSI requirements Maintenance (38.101-4 / 38.104) [NR_HST-Perf]	509

7.15.2.1	UE demodulation and CSI requirements [NR_HST-Perf]	509

7.15.2.2	BS demodulation requirements [NR_HST-Perf]	512

7.16	NR performance requirement enhancement [NR_perf_enh-Perf]	519

7.16.1	UE demodulation and CSI requirements (38.101-4) [NR_perf_enh-Perf]	519

7.16.1.1	NR CA PDSCH requirements [NR_perf_enh-Perf]	522

7.16.1.2	PMI reporting requirements with larger number of Tx ports [NR_perf_enh-Perf]	524

7.16.1.3	FR1 CA and EN-DC power imbalance requirements [NR_perf_enh-Perf]	527

7.16.1.4	NR CA CQI reporting requirements [NR_perf_enh-Perf]	528

7.16.1.5	Release independent [NR_perf_enh-Perf]	528

7.16.2	BS demodulation requirements (38.104) [NR_perf_enh-Perf]	529

7.17	Over the air (OTA) base station (BS) testing TR Maintenance [OTA_BS_testing-Perf]	529

7.18	2-step RACH for NR [NR_2step_RACH-Perf]	531

7.18.1	RRM requirements maintenance (38.133)  [NR_2step_RACH-Core/Perf]	531

7.18.2	BS Demodulation requirements maintenance (38.104) [NR_2step_RACH-Perf]	532

7.18.3	Others [NR_2step_RACH-Perf]	537

7.19	R16 NR maintenance [WI code or TEI16]	537

7.19.1	UE transient period capability [TEI16]	537

7.19.2	Transmit diversity and power class related to UL MIMO [TEI16]	539

7.19.2.1	R16 support of transmit diversity [TEI16]	540

7.19.2.2	Power class related to UL MIMO and other related req. (MPR, SEM, etc) [TEI16 or NR_newRAT-Core]	544

7.19.3	Other UE RF [WI code or TEI16]	544

7.19.4	BS RF [WI code or TEI16]	562

7.19.5	RRM [WI code or TEI16]	565

7.19.6	Demodulation and CSI [WI code or TEI16]	573

7.19.7	NR MIMO OTA test methods (38.827) [FS_NR_MIMO_OTA_test]	573

8	Rel-16 UE feature list	574

9	Rel-17 spectrum related Work Items for NR	577

9.1	NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) [NR_CA_R17_intra]	577

9.1.1	Rapporteur Input (WID/TR/CR) [NR_CA_R17_intra-Core /Perf]	577

9.1.2	UE RF for FR1 [NR_CA_R17_intra-Core]	579

9.1.3	UE RF for FR2 [NR_CA_R17_intra-Core]	580

9.2	NR inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2) [NR_CADC_R17_2BDL_xBUL]	580

9.2.1	Rapporteur Input (WID/TR/CR) [NR_CADC_R17_2BDL_xBUL-Core/Perf]	580

9.2.2	NR inter band CA without any FR2 band(s) [NR_CADC_R17_2BDL_xBUL-Core]	581

9.2.3	NR inter band CA with at least one FR2 band  [NR_CADC_R17_2BDL_xBUL-Core]	591

9.3	DC of 1 LTE band and 1 NR band [DC_R17_1BLTE_1BNR_2DL2UL]	593

9.3.1	Rapporteur Input (WID/TR/CR) [DC_R17_1BLTE_1BNR_2DL2UL-Core/Perf]	593

9.3.2	EN-DC without FR2 band [DC_R17_1BLTE_1BNR_2DL2UL-Core]	593

9.3.3	EN-DC with FR2 band  [DC_R17_1BLTE_1BNR_2DL2UL-Core]	598

9.4	DC of 2 LTE band and 1 NR band [DC_R17_2BLTE_1BNR_3DL2UL]	599

9.4.1	Rapporteur Input (WID/TR/CR) [DC_R17_2BLTE_1BNR_3DL2UL-Core/Perf]	599

9.4.2	EN-DC without FR2 band [DC_R17_2BLTE_1BNR_3DL2UL-Core]	600

9.4.3	DMEN-DC with FR2 band  [DC_R17_2BLTE_1BNR_3DL2UL-Core]	611

9.5	DC of 3 LTE band and 1 NR band [DC_R17_3BLTE_1BNR_4DL2UL]	611

9.5.1	Rapporteur Input (WID/TR/CR) [DC_R17_3BLTE_1BNR_4DL2UL-Core/Perf]	611

9.5.2	EN-DC without FR2 band [DC_R17_3BLTE_1BNR_4DL2UL-Core]	612

9.5.3	EN-DC with FR2 band  [DC_R17_3BLTE_1BNR_4DL2UL-Core]	623

9.6	DC of 4 LTE band and 1 NR band [DC_R17_4BLTE_1BNR_5DL2UL]	624

9.6.1	Rapporteur Input (WID/TR/CR) [DC_R17_4BLTE_1BNR_5DL2UL-Core/Perf]	624

9.6.2	EN-DC without FR2 band [DC_R17_4BLTE_1BNR_5DL2UL-Core]	624

9.6.3	EN-DC with FR2 band  [DC_R17_4BLTE_1BNR_5DL2UL-Core]	629

9.7	DC of x bands (x=1,2, 3, 4) LTE inter-band CA and 2 bands NR inter-band CA  [DC_R17_xBLTE_2BNR_yDL2UL]	629

9.7.1	Rapporteur Input (WID/TR/CR) [DC_R17_xBLTE_2BNR_yDL2UL-Core/Per]	629

9.7.2	EN-DC including NR inter CA without FR2 band [DC_R17_xBLTE_2BNR_yDL2UL-Core]	630

9.7.3	EN-DC including NR inter CA with FR2 band [DC_R17_xBLTE_2BNR_yDL2UL-Core]	654

9.8	Band combinations for SA NR supplementary uplink (SUL)	655

9.8.1	Rapporteur Input (WID/TR/CR) [NR_SUL_combos_R17-Core/Per]	655

9.8.2	UE RF [NR_SUL_combos_R17-Core]	656

9.9	NR Inter-band Carrier Aggregation for 3 bands DL with 1 band UL [NR_CA_R17_3BDL_1BUL]	659

9.9.1	Rapporteur Input (WID/TR/CR) [NR_CA_R17_3BDL_1BUL-Core/Per]	659

9.9.2	UE RF [NR_CA_R17_3BDL_1BUL-Core]	660

9.10	NR Inter-band Carrier Aggregation for 4 bands DL with 1 band UL [NR_CA_R17_4BDL_1BUL]	666

9.10.1	Rapporteur Input (WID/TR/CR) [NR_CA_R17_4BDL_1BUL-Core/Per]	666

9.10.2	UE RF [NR_CA_R17_4BDL_1BUL-Core]	667

9.11	NR Inter-band Carrier Aggregation/Dual connectivity for 3 bands DL with 2 bands UL [NR_CADC_R17_3BDL_2BUL]	668

9.11.1	Rapporteur Input (WID/TR/CR) [NR_CADC_R17_3BDL_2BUL-Core/Per]	668

9.11.2	UE RF [NR_CADC_R17_3BDL_2BUL-Core]	669

9.12	DC of x bands (x=1,2) LTE inter-band CA (xDL/xUL) and y bands (y=3-x) NR inter-band CA [DC_R17_xBLTE_yBNR_3DL3UL]	678

9.12.1	Rapporteur Input (WID/TR/CR) [DC_R17_xBLTE_yBNR_3DL3UL-Core/Per]	678

9.12.2	UE RF [DC_R17_xBLTE_yBNR_3DL3UL-Core]	679

9.13	DC of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL) [DC_R17_xBLTE_3BNR_yDL2UL]	679

9.13.1	Rapporteur Input (WID/TR/CR) [DC_R17_xBLTE_3BNR_yDL2UL -Core/Per]	679

9.13.2	UE RF [DC_R17_xBLTE_3BNR_yDL2UL-Core]	680

9.14	NR inter-band Carrier Aggregation and Dual connectivity for DL 4 bands and 2UL bands [NR_CADC_R17_4BDL_2BUL]	686

9.14.1	Rapporteur Input (WID/TR/CR) [NR_CADC_R17_4BDL_2BUL -Core/Per]	686

9.14.2	UE RF [NR_CADC_R17_4BDL_2BUL -Core]	687

9.15	NR inter-band CA for 5 bands DL with x bands UL (x=1, 2) [NR_CADC_R17_5BDL_xBUL_3DL3UL]	688

9.15.1	Rapporteur Input (WID/TR/CR) [NR_CADC_R17_5BDL_xBUL -Core/Per]	688

9.15.2	UE RF [NR_CADC_R17_5BDL_xBUL -Core]	689

9.16	DC of 5 bands LTE inter-band CA (5DL/1L) and 1 NR band (1DL/1UL) [DC_R17_5BLTE_1BNR_6DL2UL]	689

9.16.1	Rapporteur Input (WID/TR/CR) [DC_R17_5BLTE_1BNR_6DL2UL-Core/Per]	689

9.16.2	UE RF [DC_R17_5BLTE_1BNR_6DL2UL-Core]	690

9.17	DC of x bands (x=2,3,4) LTE inter-band CA (xDL/1UL) and 1 NR FR1 band (1DL/1UL) and 1 NR FR2 band (1DL/1UL) [DC_R17_xBLTE_2BNR_yDL3UL]	690

9.17.1	Rapporteur Input (WID/TR/CR) [DC_R17_xBLTE_2BNR_yDL3UL-Core/Per]	690

9.17.2	UE RF [DC_R17_xBLTE_2BNR_yDL3UL-Core]	691

9.18	SAR schemes for UE power class 2 (PC2) for NR inter-band Carrier Aggregation and supplemental uplink (SUL) configurations with 2 bands UL [NR_SAR_PC2_interB_SUL_2BUL]	691

9.18.1	General and Rapporteur Input (WID/TR/CR) [NR_SAR_PC2_interB_SUL_2BUL-Core/Per]	692

9.18.2	PC2 for inter-band CA [NR_SAR_PC2_interB_SUL_2BUL-Core]	693

9.18.3	PC2 for SUL [NR_SAR_PC2_interB_SUL_2BUL-Core]	695

9.18.4	Others [NR_SAR_PC2_interB_SUL_2BUL-Core]	696

9.19	High power UE (power class 2) for NR inter-band Carrier Aggregation with 2 bands downlink and 2 bands uplink [NR_PC2_CA_R17_2BDL_2BUL]	696

9.19.1	Rapporteur Input (WID/TR/CR) [NR_PC2_CA_R17_2BDL_2BUL-Core/Per]	697

9.19.2	UE RF [NR_PC2_CA_R17_2BDL_2BUL-Core]	697

9.20	High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band [ENDC_UE_PC2_R17_NR_TDD]	699

9.20.1	Rapporteur Input (WID/TR/CR) [ENDC_UE_PC2_R17_NR_TDD -Core/Per]	700

9.20.2	UE RF [ENDC_UE_PC2_R17_NR_TDD -Core]	700

9.21	Adding channel bandwidth support to existing NR bands [NR_bands_R17_BWs]	702

9.21.1	General and Rapporteur Input (WID/TR/CR) [NR_bands_R17_BWs -Core/Per]	704

9.21.2	UE RF requirement [NR_bands_R17_BWs -Core]	705

9.21.2.1	Reference sensitivity  [NR_bands_R17_BWs -Core]	705

9.21.2.2	MPR/A-MPR/NS signaling [NR_bands_R17_BWs -Core]	706

9.21.2.3	others [NR_bands_R17_BWs -Core]	707

9.21.3	BS RF requirement [NR_bands_R17_BWs -Core]	707

9.22	Introduction of channel bandwidths 35MHz and 45MHz for NR [NR_FR1_35MHz_45MHz_BW]	707

9.22.1	General and Rapporteur Input (WID/TR/CR) [NR_FR1_35MHz_45MHz_BW-Core/Per]	708

9.22.2	Spectrum utilization [NR_FR1_35MHz_45MHz_BW-Core]	708

9.22.3	UE RF requirements [NR_FR1_35MHz_45MHz_BW-Core]	709

9.22.4	BS RF requirements [NR_FR1_35MHz_45MHz_BW-Core]	712

9.22.5	Others [NR_FR1_35MHz_45MHz_BW-Core]	716

9.23	Band combinations for Uu and V2X con-current operation [NR_LTE_V2X_PC5_combos]	716

9.23.1	General and Rapporteur Input (WID/TR/CR) [NR_LTE_V2X_PC5_combos-Core/Per]	716

9.23.2	UE RF requirement for concurrent operation between NR Uu band and NR PC5 band [NR_LTE_V2X_PC5_combos-Core]	718

9.23.3	UE RF requirement for concurrent operation between LTE Uu band and NR PC5 band [NR_LTE_V2X_PC5_combos-Core]	718

9.23.4	UE RF requirement for concurrent operation between NR Uu band and LTE PC5 band [NR_LTE_V2X_PC5_combos-Core]	719

9.23.5	UE RF requirement for concurrent operation of LTE/NR CA/DC band combinations + PC5 V2X [NR_LTE_V2X_PC5_combos-Core]	719

9.24	Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258  [NR_FR2_FWA_Bn257_Bn258]	719

9.24.1	UE RF (38.101-2) [NR_FR2_FWA_Bn257_Bn258-Core]	719

9.24.2	RRM Core requirements (38.133) [NR_FR2_FWA_Bn257_Bn258-Core]	723

9.24.3	RRM Perf. requirements (38.133) [NR_FR2_FWA_Bn257_Bn258-Perf]	723

9.24.4	Others  [NR_FR2_FWA_Bn257_Bn258-Core/Perf]	724

9.25	Introduction of NR 47 GHz band [NR_47GHz_Band]	725

9.25.1	UE RF (38.101-2) [NR_47GHz_Band-Core]	725

9.25.1.1	Peak EIRP and EIRP spherical coverage  [NR_47GHz_Band-Core]	725

9.25.1.2	Other UE TX requirements  [NR_47GHz_Band-Core]	726

9.25.1.3	REFSENS and EIS spherical coverage  [NR_47GHz_Band-Core]	728

9.25.1.4	Other UE RX requirements [NR_47GHz_Band-Core]	729

9.25.2	BS RF (38.104) [NR_47GHz_Band-Core]	729

9.25.3	RRM (38.133) [NR_47GHz_Band-Core]	730

9.25.4	Others  [NR_47GHz_Band-Core/Perf]	731

9.25.4.1	BS conformance (38.141)  [NR_47GHz_Band-Perf]	731

9.25.4.2	UE Demod (38.101-4)  [NR_47GHz_Band-Perf]	732

9.25.4.3	BS Demod (38.104)  [NR_47GHz_Band-Perf]	733

9.25.4.4	Others [NR_47GHz_Band-Core/Perf]	734

9.26	Introduction of NR band n24 [NR_band_n24]	734

9.26.1	UE RF (38.101-1) [NR_band_n24-Core]	735

9.26.2	BS RF (38.104) [NR_band_n24-Core]	736

9.26.3	RRM (38.133) [NR_band_n24-Core]	736

9.26.4	Others  [NR_band_n24-Core/Perf]	737

9.27	Introduction of 1.6 GHz NR SUL band with same uplink frequency range of Band 24 [NR_SUL_UL_n24]	739

9.27.1	UE RF (38.101-1) [NR_SUL_UL_n24-Core]	739

9.27.2	BS RF (38.104) [NR_SUL_UL_n24-Core]	740

9.27.3	RRM (38.133) [NR_SUL_UL_n24-Core]	744

9.27.4	Others  [NR_SUL_UL_n24-Core/Perf]	744

9.28	Introduction of NR band n67 [NR_n67]	744

9.28.1	UE RF (38.101-1) [NR_n67-Core]	744

9.28.2	BS RF (38.104) [NR_n67-Core]	745

9.28.3	RRM (38.133) [NR_n67-Core]	745

9.28.4	Others  [NR_n67-Core/Perf]	745

9.29	Introduction of NR band n85 [NR_n85]	745

9.29.1	UE RF (38.101-1) [NR_n85-Core]	746

9.29.2	BS RF (38.104) [NR_n85-Core]	746

9.29.3	RRM (38.133) [NR_n85-Core]	746

9.29.4	Others  [NR_n85-Core/Perf]	746

9.30	Introduction of bandwidth combination set 4 (BCS4) for NR [NR_BCS4]	746

9.30.1	General and Rapporteur Input (WID/TR/CR) [NR_BCS4-Core]	747

9.30.2	UE RF requirements [NR_BCS4-Core]	748

9.30.2.1	MSD  [NR_BCS4-Core]	748

9.30.2.2	Others (in case MPR/A-MPR is needed) [NR_BCS4-Core]	749

9.30.3	Signalling [NR_BCS4-Core]	749

9.31	Band combination specific requirements for NR intra band UL Carrier Aggregation []	750

9.31.1	General and Rapporteur Input (WID/TR/CR) [-Core]	750

9.31.2	PC2 UE RF requirements [-Core]	751

9.31.2.1	Maximum output power  [-Core]	751

9.31.2.2	A-MPR [-Core]	751

9.31.2.3	others [-Core]	751

9.31.3	PC3 UE RF requirements [-Core]	751

9.32	Additional NR bands for UL-MIMO [NR_bands_UL_MIMO_PC3_R17]	751

9.32.1	General and Rapporteur Input (WID/TR/CR) [NR_bands_UL_MIMO_PC3_R17-Core]	751

9.32.2	MPR/A-MPR requirement [NR_bands_UL_MIMO_PC3_R17-Core]	752

9.32.3	Others [NR_bands_UL_MIMO_PC3_R17-Core/Perf]	752

9.33	Down link interruption for band combinations to conduct dynamic Tx Switching [DL_intrpt_combos_TxSW_R17]	752

9.33.1	General and Rapporteur Input (WID/TR/CR) [DL_intrpt_combos_TxSW_R17-Core]	752

9.33.2	Determination of inter-band uplink CA and EN-DC combinations for which DL interruption is not allowed [DL_intrpt_combos_TxSW_R17-Core]	753

9.33.3	Others [DL_intrpt_combos_TxSW_R17-Core/Perf]	755

9.34	High-power UE operation for use cases in Band n77 and n78 [HPUE_PC1_5_n77_n78]	756

9.34.1	General [HPUE_PC1_5_n77_n78-Core]	756

9.34.2	PC1.5 UE RF requirements [HPUE_PC1_5_n77_n78-Core]	757

9.34.2.1	A-MPR [HPUE_PC1_5_n77_n78-Core]	757

9.34.2.2	others [HPUE_PC1_5_n77_n78-Core]	757

9.35	Introduction of lower 6GHz NR unlicensed operation for Europe [NR_6GHz_unlic_EU]	758

9.35.1	General [NR_6GHz_unlic_EU-Core]	759

9.35.2	UE RF requirements [NR_6GHz_unlic_EU-Core]	760

9.35.3	BS RF requirements [NR_6GHz_unlic_EU-Core]	760

9.35.4	Others [NR_6GHz_unlic_EU-Core]	761

10	Reply to ITU-R LS (RP-200042)	761

10.1	Study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz [FS_6425_10500MHz _NR]	761

10.1.1	UE parameters	762

10.1.2	BS parameters	764

10.1.3	Coexistence study	766

10.1.3.1	Simulation assumptions	766

10.1.3.2	Downlink	767

10.1.3.3	Uplink	768

10.1.4	Antenna characteristics	770

10.1.5	Relevant information for the sharing and compatibility studies	771

11	Rel-17 non-spectrum related work items for NR	771

11.1	Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs [NR_MIMO_OTA]	771

11.1.1	General  [NR_MIMO_OTA]	771

11.1.2	Performance Requirements  [NR_MIMO_OTA-Core]	772

11.1.2.1	Performance Requirements for FR1  [NR_MIMO_OTA-Core]	772

11.1.2.2	Performance Requirements for FR2 [NR_MIMO_OTA-Core]	773

11.1.3	Testing methodologies [NR_MIMO_OTA-Core]	774

11.1.3.1	Testing parameters for Performance  [NR_MIMO_OTA-Core]	774

11.1.3.2	Optimization of test methodologies [NR_MIMO_OTA-Core]	774

11.1.3.3	Channel model validation [NR_MIMO_OTA-Core]	775

11.2	RF requirements enhancement for NR frequency range 1 (FR1) [NR_RF_FR1_enh]	776

11.2.1	General and work plan [NR_RF_FR1_enh-Core]	776

11.2.2	RF core requirements [NR_RF_FR1_enh-Core]	777

11.2.2.1	UL MIMO configuration for SUL band configurations  [NR_RF_FR1_enh-Core]	777

11.2.2.2	2Tx switching between carrier 1 and carrier 2 [NR_RF_FR1_enh-Core]	778

11.2.2.3	Tx switching between 1 carrier on band A and 2 contiguous aggregated carriers on band B [NR_RF_FR1_enh-Core]	780

11.2.2.4	HPUE for TDD intra-band contiguous UL CA [NR_RF_FR1_enh-Core]	781

11.2.2.5	HPUE for TDD intra-band non-contiguous UL CA [NR_RF_FR1_enh-Core]	782

11.3	NR RF requirement enhancements for frequency range 2 (FR2)  [NR_RF_FR2_req_enh2]	784

11.3.1	General and work plan [NR_RF_FR2_req_enh2-Core]	789

11.3.2	RF core requirements [NR_RF_FR2_req_enh2-Core]	790

11.3.2.1	Inter-band DL CA enhancements [NR_RF_FR2_req_enh2-Core]	791

11.3.2.1.1	Applicability of CBM/IBM for different CA configurations [NR_RF_FR2_req_enh2-Core]	791

11.3.2.1.2	UE requirements for CA configurations CA_n258A-n260A and CA_n257A-n259A based on IBM [NR_RF_FR2_req_enh2-Core]	793

11.3.2.1.3	UE requirements for CA configurations within the same frequency group based on CBM [NR_RF_FR2_req_enh2-Core]	795

11.3.2.2	Inter-band UL CA [NR_RF_FR2_req_enh2-Core]	795

11.3.2.2.1	UE requirements for CA configuration CA_n257A-n259A based on IBM [NR_RF_FR2_req_enh2-Core]	795

11.3.3	Feasibility study [NR_RF_FR2_req_enh2-Core]	796

11.3.3.1	Inter-band DL CA enhancements [NR_RF_FR2_req_enh2-Core]	797

11.3.3.1.1	Feasibility study for CA configurations within same frequency group based on IBM [NR_RF_FR2_req_enh2-Core]	797

11.3.3.1.2	Feasibility study for CA configurations between different frequency groups based on CBM [NR_RF_FR2_req_enh2-Core]	797

11.3.3.2	Inter-band UL CA [NR_RF_FR2_req_enh2-Core]	798

11.3.3.2.1	Feasibility study for CA configurations within same frequency group based on IBM and CBM [NR_RF_FR2_req_enh2-Core]	798

11.3.3.2.2	Feasibility study for CA configurations between different frequency groups based on CBM [NR_RF_FR2_req_enh2-Core]	798

11.3.4	UL gaps for self-calibration and monitoring [NR_RF_FR2_req_enh2-Core]	798

11.3.4.1	Gap use cases and performance evaluation [NR_RF_FR2_req_enh2-Core]	799

11.3.4.2	Others [NR_RF_FR2_req_enh2-Core]	800

11.3.5	RRM core requirements  [NR_RF_FR2_req_enh2-Core]	801

11.3.5.1	Inter-band DL CA enhancements [NR_RF_FR2_req_enh2-Core]	801

11.3.5.2	Inter-band UL CA [NR_RF_FR2_req_enh2-Core]	802

11.4	Further RRM enhancement for NR and MR-DC  [NR_RRM_enh2]	803

11.4.1	General and work plan [NR_RRM_enh2-Core]	809

11.4.2	RRM core requirements  [NR_RRM_enh2-Core]	809

11.4.2.1	SRS antenna port switching  [NR_RRM_enh2-Core]	809

11.4.2.2	HO with PSCell [NR_RRM_enh2-Core]	811

11.4.2.3	PUCCH SCell activation/deactivation [NR_RRM_enh2-Core]	814

11.5	NR and MR-DC measurement gap enhancements [NR_MG_enh]	816

11.5.1	General and work plan [NR_MG_enh-Core]	821

11.5.2	RRM core requirements  [NR_MG_enh-Core]	821

11.5.2.1	Pre-configured MG pattern(s)  [NR_MG_enh-Core]	821

11.5.2.2	Multiple concurrent and independent MG patterns [NR_MG_enh-Core]	823

11.5.2.3	Network Controlled Small Gap [NR_MG_enh-Core]	826

11.6	Enhancement for NR high speed train scenario in FR1 [NR_HST_FR1_enh-Core]	828

11.6.1	General and work plan [NR_HST_FR1_enh-Core]	828

11.6.2	RRM core requirements  [NR_HST_FR1_enh-Core]	828

11.6.2.1	UE RRM core requirements for CA scenario [NR_HST_FR1_enh-Core]	831

11.6.3	UE demodulation requirements (38.101-4) [NR_HST_FR1_enh-Perf]	833

11.6.3.1	General  [NR_HST_FR1_enh-Perf]	833

11.6.3.2	PDSCH requirements for CA scenarios [NR_HST_FR1_enh-Perf]	834

11.6.3.3	Enhanced transmission schemes  [NR_HST_FR1_enh-Perf]	834

11.7	NR support for high speed train scenario in FR2 [NR_HST_FR2_enh]	835

11.7.1	General and work plan [NR_HST_FR2_enh-Core]	836

11.7.2	High speed train deployment scenario in FR2 [NR_HST_FR2_enh-Core]	837

11.7.3	UE RF core requirements [NR_HST_FR2_enh-Core]	838

11.7.4	RRM core requirements [NR_HST_FR2_enh-Core]	839

11.8	Solutions for NR to support non-terrestrial networks (NTN) [NR_NTN_solutions]	843

11.8.1	General and work plan [NR_NTN_solutions-Core]	843

11.8.2	Use cases, deployment scenarios, and regulatory information [NR_NTN_solutions-Core]	846

11.8.3	Coexistence aspects [NR_NTN_solutions-Core]	847

11.8.3.1	Simulation assumptions [NR_NTN_solutions-Core]	848

11.8.3.2	UE requirements aspects [NR_NTN_solutions-Core]	849

11.8.3.3	BS requirements aspects [NR_NTN_solutions-Core]	849

11.8.4	RRM core requirements [NR_NTN_solutions-Core]	850

11.8.4.1	General [NR_NTN_solutions-Core]	854

11.8.4.2	Timing requirements [NR_NTN_solutions-Core]	855

11.8.4.3	Measurement requirements [NR_NTN_solutions-Core]	856

11.9	UE Power Saving Enhancements [NR_UE_pow_sav_enh]	857

11.9.1	General and work plan [NR_UE_pow_sav_enh-Core]	858

11.9.2	UE measurements relaxation for RLM and/or BFD [NR_UE_pow_sav_enh-Core]	859

11.10	NR Sidelink enhancement [NRSL_enh]	863

11.10.1	General and work plan [NRSL_enh]	863

11.10.2	Spectrum request for SL operation [NRSL_enh-Core]	864

11.10.3	UE RF requirements for NR SL enhancement  [NRSL_enh-Core]	865

11.10.3.1	TX requirements [NRSL_enh-Core]	865

11.10.3.2	RX requirements [NRSL_enh-Core]	865

11.10.4	Partially used SL operation with NR Uu operating bands [NRSL_enh-Core]	866

11.10.4.1	Operating scenarios for partially used SL operation [NRSL_enh-Core]	867

11.10.4.2	Synchronous operation between NR Uu and NR SL in an operating band [NRSL_enh-Core]	868

11.10.4.3	Others [NRSL_enh-Core]	869

11.10.5	High power UE(PC2) for SL [NRSL_enh-Core]	869

11.10.5.1	TX requirements [NRSL_enh-Core]	870

11.10.5.2	RX requirements [NRSL_enh-Core]	871

11.10.6	Other RF/general requirements for New SL enhancement [NRSL_enh-Core]	871

11.11	NR repeater	871

11.11.1	General and work plan [NR_repeaters-Core]	871

11.11.2	Conductive RF core requirements [NR_repeaters-Core]	875

11.11.2.1	Transmitted power related requirements [NR_repeaters-Core]	875

11.11.2.2	Emission requirements [NR_repeaters-Core]	876

11.11.2.3	Others  [NR_repeaters-Core]	876

11.11.3	Radiated RF core requirements	877

11.11.3.1	Transmitted power related requirements [NR_repeaters-Core]	877

11.11.3.2	Emission requirements [NR_repeaters-Core]	878

11.11.3.3	Others  [NR_repeaters-Core]	878

11.11.4	EMC core requirements  [NR_repeaters-Core]	879

12	Rel-17 Study Items for NR	880

12.1	Study on enhanced test methods for FR2 in NR [FS_FR2_enhTestMethods]	880

12.1.1	General [FS_FR2_enhTestMethods]	880

12.1.2	Test methodology for high DL power and low UL power test cases [FS_FR2_enhTestMethods]	881

12.1.3	Polarization basis mismatch  [FS_FR2_enhTestMethods]	884

12.1.4	Enhanced test methods for inter-band (FR2+FR2) CA [FS_FR2_enhTestMethods]	884

12.1.5	Extreme temperature conditions [FS_FR2_enhTestMethods]	885

12.1.6	Enhanced test methods for FR2 DL 256QAM RF [FS_FR2_enhTestMethods]	886

12.1.7	Test time reduction [FS_FR2_enhTestMethods]	886

12.1.8	Testability for band n262  [FS_FR2_enhTestMethods]	888

12.1.8.1	Extension of frequency applicability of permitted methods in 38.810 [FS_FR2_enhTestMethods]	888

12.1.8.2	Extension of frequency applicability of enhancement objectives 1-6 [FS_FR2_enhTestMethods]	888

12.2	Study on supporting NR from 52.6 GHz to 71 GHz [FS_NR_52_to_71GHz]	889

12.2.1	Numerology, Channel BW [FS_NR_52_to_71GHz]	889

12.2.1.1	General [FS_NR_52_to_71GHz]	891

12.2.1.2	General [FS_NR_52_to_71GHz]	892

12.2.1.3	Phase noise  [FS_NR_52_to_71GHz]	893

12.2.2	BS aspect [FS_NR_52_to_71GHz]	894

12.2.3	UE aspect [FS_NR_52_to_71GHz]	896

12.2.4	Others [FS_NR_52_to_71GHz]	896

12.3	Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths [FS_NR_eff_BW_util]	898

12.3.1	General and work plan [FS_NR_eff_BW_util]	899

12.3.2	Input on operator licensed channel bandwidths in FR1 that do not align with existing NR channel bandwidths [FS_NR_eff_BW_util]	900

12.3.3	Evaluation of use of larger channel bandwidths than operator licensed bandwidth [FS_NR_eff_BW_util]	900

12.3.4	Evaluation of use of overlapping UE channel bandwidths (from both UE and network perspective) [FS_NR_eff_BW_util]	900

12.3.4.1	UE perspective  [FS_NR_eff_BW_util]	901

12.3.4.2	Network perspective [FS_NR_eff_BW_util]	902

12.3.5	Others [FS_NR_eff_BW_util]	902

12.4	Study on extended 600MHz NR band [FS_NR_600MHz_ext]	902

12.4.1	General	903

12.4.2	Regulatory study	904

12.4.3	Coexistence study	904

12.4.4	Study on frequency arrangements (such as options B1 and B2)	905

12.4.5	Others	908

12.5	Study on high power UE (power class 2) for one NR FDD band [FS_NR_PC2_UE_FDD]	909

12.5.1	General	910

12.5.2	Scheme(s) to comply with the SAR limits	911

12.5.3	Interference issues	911

12.5.4	UE implementation issues	911

12.5.5	System performance evaluations	912

13	Rel-17 Work Items for LTE	912

13.1	LTE inter-band Carrier Aggregation for 2 bands DL with 1 band UL [LTE_CA_R17_2BDL_1BUL]	913

13.1.1	Rapporteur Input (WID/TR/CR) [LTE_CA_R17_2BDL_1BUL-Core/Perf]	913

13.1.2	UE RF with harmonic, close proximity and isolation issues [LTE_CA_R17_2BDL_1BUL-Core]	914

13.1.3	UE RF without specific issues  [LTE_CA_R17_2BDL_1BUL-Core]	914

13.2	LTE inter-band Carrier Aggregation for 3 bands DL with 1 band UL [LTE_CA_R17_3BDL_1BUL]	915

13.2.1	Rapporteur Input (WID/TR/CR) [LTE_CA_R17_3BDL_1BUL-Core/Perf]	915

13.2.2	UE RF with harmonic, close proximity and isolation issues [LTE_CA_R17_3BDL_1BUL-Core]	916

13.2.3	UE RF without specific issues [LTE_CA_R17_3BDL_1BUL-Core]	918

13.3	LTE inter-band Carrier Aggregation for x bands DL (x=4, 5) with 1 band UL [LTE_CA_R17_xBDL_1BUL]	918

13.3.1	Rapporteur Input (WID/TR/CR) [LTE_CA_R17_xBDL_1BUL-Core]	919

13.3.2	UE RF with 4 LTE bands CA [LTE_CA_R17_xBDL_1BUL-Core]	920

13.3.3	UE RF with 5 LTE bands CA [LTE_CA_R17_xBDL_1BUL-Core]	923

13.4	LTE inter-band Carrier Aggregation for 2 bands DL with 2 band UL [LTE_CA_R17_2BDL_2BUL]	925

13.4.1	Rapporteur Input (WID/TR/CR) [LTE_CA_R17_2BDL_2BUL-Core]	925

13.4.2	UE RF with harmonic, close proximity and isolation issues [LTE_CA_R17_2BDL_2BUL-Core]	925

13.4.3	UE RF without specific issues [LTE_CA_R17_2BDL_2BUL-Core]	925

13.5	LTE inter-band Carrier Aggregation for x bands DL (x= 3, 4, 5) with 2 band UL [LTE_CA_R17_xBDL_2BUL]	925

13.5.1	Rapporteur Input (WID/TR/CR) [LTE_CA_R17_xBDL_2BUL-Core]	925

13.5.2	UE RF with MSD [LTE_CA_R17_xBDL_2BUL-Core]	926

13.5.3	UE RF without MSD [LTE_CA_R17_xBDL_2BUL-Core]	926

13.6	RRM for LTE CA basket WIs  [LTE_CA_R17_xxxx]	928

13.6.1	RRM Core (36.133) [LTE_CA_R17_xxxx-Core]	928

13.6.2	RRM Perf (36.133) [LTE_CA_R17_xxxx-Perf]	928

13.7	New WID on Additional LTE bands for UE category M1&M2 and/or NB1&NB2 in Rel-17 [LTE_bands_R17_M1_M2_NB1_NB2]	928

13.7.1	Rapporteur Input (WID/TR/CR) [LTE_bands_R17_M1_M2_NB1_NB2-Core]	928

13.7.2	RF  [LTE_bands_R17_M1_M2_NB1_NB2-Core]	933

13.7.3	Others  [LTE_bands_R17_M1_M2_NB1_NB2-Perf]	933

13.8	Modification of LTE Band 24 specifications to comply with updated regulatory emission limits [LTE_B24_mod]	934

13.8.1	General and rapporteur input [LTE_B24_mod-Core]	934

13.8.2	UE RF [LTE_B24_mod-Core]	934

13.8.3	BS RF [LTE_B24_mod-Core]	938

13.8.4	RRM and others [LTE_B24_mod-Core/Perf]	942

14	Rel-17 Study Items for LTE	948

14.1	High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands 5 and 12 and NR band n71 [FS_LTE_NR_HPUE_FWVM]	948

14.1.1	General	949

14.1.2	Coexistence study	949

14.1.3	UE RF	950

15	Liaison and output to other groups	950

15.1	R17 related	950

15.2	Others	957

16	Revision of the Work Plan	957

16.1	Simplification of band combinations in RAN4 specifications	957

16.2	R17 new proposals	961

16.2.1	Spectrum related	961

16.2.2	Non-spectrum related	962

16.3	Others	964

17	Any other business	964

18	Close of the E-meeting	964

Annex A: Contribution documents and status	975

A1: List of TDocs	975

Annex B: List of change requests	1190

C1: Incoming liaison statements	1267

C2: Outgoing liaison statements	1269

## 1	Opening of the E-meeting

The Chairman Steven Chen (Apple) opened the meeting on RAN4 reflector on 25/01/2021.

Intellectual Property Rights Policy

The attention of the delegates to the meeting of this Technical Specification Group was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of.

The delegates were asked to take note that they were thereby invited:

-	to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP.

-	to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms.

Statement regarding competition law

The attention of the delegates to the meeting was drawn to the fact that 3GPP activities were subject to all applicable antitrust and competition laws and that compliance with said laws was therefore required by any participant of the meeting, including the Chairman and Vice-Chairmen and were invited to seek any clarification needed with their legal counsel. The leadership would conduct the present meeting with impartiality and in the interests of 3GPP. Delegates were reminded that timely submission of work items in advance of TSG/WG meetings was important to allow for full and fair consideration of such matters.

Meeting arrangements

The meeting was conducted on three parallel sessions; Main session, RRM session and BS RF Test Demod session. The Main session was chaired by RAN4 Chairman Steven Chen (Apple), RRM session was chaired by RAN4 Vice Chairman Andrey Chervyakov (Intel) and BS RF Test Demod session was chaired by RAN4 ViceChairman Haijie Qiu (Samsung). The sessions were further broken down into separate email threads to address specific technical topics lead by assigned discussion moderators. Webinar sessions were used to summarize progress, resolve controversial issues and decide way forward.

Current chairman’s continuation under his new company affiliation (Article 22)

GTW call in the main session, the first 15 minutes, were used to check if there was consensus for current chairman’s Steven Chen (Apple) continuation as RAN4 chairman under his new company affiliation.

The secretary explained that according to 3GPP's working procedures Article 22 if the change of company affiliation is due to a move to another company, then the decision for the chairman to continue in office to the end of the present term shall be made by consensus. If consensus cannot be achieved, a special election shall be held for the office.

Jiansong Gan (Huawei) informed that Huawei would like to challenge Steven Chen (Apple) as chairman. Valentin Gheorghiu (Qualcomm) asked what is the deadline to announce a candidate. The secretary answered that there is no deadline, and a candidate can be announced any time before the election. Haijie Qiu (Samsung) asked what is the meeting for the election and had concerns if the election takes place in the April meeting because the meeting is going to last only seven days. The secretary clarified that according to working procedures chairman election needs to be arranged in the next meeting which is the April meeting.

There was no consensus for the current chairman to continue in the office to the end of his present term and hence chairman election wil lbe arranged in the April meeting. The secretary reminded that all chairman candidates need to provide a letter of recommendation to MCC.

## 2	Approval of the agenda

R4-2100003	RAN4#97-e Meeting Report

Type: report		For: Approval
					Source: ETSI MCC

Decision: 		The document was approved.

R4-2100001	Agenda for RAN4 #98-e

Type: Agenda		For: Approval
					Source: RAN4 Chairman (Apple)

Decision: 		The document was approved.

R4-2100002	RAN4#98-e E-meeting Arrangements and Guidelines

Type: other		For: Discussion
					Source: RAN4 Chairman (Apple)

Decision: 		The document was approved.

## 3	Letters / reports from other groups / meetings

R4-2100004	Test methods for over-the-air TRP field measurements of unwanted emissions from IMT radio equipment utilizing active antennas

Type: LS in		For: Information
					Original outgoing LS: -, to -, cc -
					Source: ITU Radiocommunication Study Groups Working Party 1C

Decision: 		The document was noted.

R4-2100005	LS on updated Rel-16 RAN1 UE features lists for LTE

Type: LS in		For: Information
					Original outgoing LS: R1-2009351, to RAN2, cc RAN4
					Source: RAN1

Decision: 		The document was noted.

R4-2100006	Reply LS on definition of NR V2X con-current operation

Type: LS in		For: Information
					Original outgoing LS: R1-2009491, to RAN4, cc RAN2
					Source: RAN1

Decision: 		The document was noted.

R4-2100007	LS reply on cell-grouping UE capability for synchronous NR-DC

Type: LS in		For: Information
					Original outgoing LS: R1-2009570, to RAN2, cc RAN4
					Source: RAN1

Decision: 		The document was noted.

R4-2100008	Reply LS on DCI-based multiple BWP switch simultaneously

Type: LS in		For: Information
					Original outgoing LS: R1-2009575, to RAN4, cc -
					Source: RAN1

Decision: 		The document was noted.

R4-2100009	Reply LS on UE capability xDD differentiation for SUL/SDL bands

Type: LS in		For: Information
					Original outgoing LS: R1-2009576, to RAN2, cc -
					Source: RAN1

Decision: 		The document was noted.

R4-2100010	LS on updated Rel-16 RAN1 UE features lists for NR

Type: LS in		For: Information
					Original outgoing LS: R1-2009586, to RAN2, RAN4, cc -
					Source: RAN1

Decision: 		The document was noted.

R4-2100011	Reply LS on UE capability for V2X

Type: LS in		For: Information
					Original outgoing LS: R1-2009635, to RAN2, cc RAN4
					Source: RAN1

Decision: 		The document was noted.

R4-2100012	LS on uplink Tx switching

Type: LS in		For: Information
					Original outgoing LS: R1-2009676, to RAN2, cc RAN4
					Source: RAN1

Decision: 		The document was noted.

R4-2100013	Reply LS on Clarification of UE behavior after receiving the MAC CE deactivation command for semi-persistent CSI reporting in NR-U

Type: LS in		For: Information
					Original outgoing LS: 	R1-2009689, to RAN4, cc -
					Source: RAN1

Decision: 		The document was noted.

R4-2100014	LS on PUCCH and PUSCH repetition

Type: LS in		For: Information
					Original outgoing LS: R1-2009784, to RAN4, cc -
					Source: RAN1

Decision: 		The document was noted.

R4-2100015	LS on temporary RS for efficient SCell activation in NR CA

Type: LS in		For: Information
					Original outgoing LS: R1-2009798, to RAN4, cc -
					Source: RAN1

Decision: 		The document was noted.

R4-2100016	LS on Beam switching gaps for Multi-TRP UL transmission

Type: LS in		For: Information
					Original outgoing LS: R1-2009807, to RAN4, cc -
					Source: RAN1

Decision: 		The document was noted.

R4-2100017	Reply LS on power control for NR-DC

Type: LS in		For: Information
					Original outgoing LS: R2-2011246, to RAN4, cc RAN1
					Source: RAN2

Decision: 		The document was noted.

R4-2100018	LS on support of NUL and SUL during DAPS handover

Type: LS in		For: Information
					Original outgoing LS: R1-2009682, to RAN2, cc RAN4
					Source: RAN1

Decision: 		The document was noted.

R4-2100019	Reply LS on IAB-MT feature list

Type: LS in		For: Information
					Original outgoing LS: R2-2011273, to RAN4, cc -
					Source: RAN2

Decision: 		The document was noted.

R4-2100020	LS on ambiguity in deciding TL,C

Type: LS in		For: Information
					Original outgoing LS: R5-206676, to RAN4, cc -
					Source: RAN5

Decision: 		The document was noted.

R4-2100021	LS on Frequency Bands for testing of A-GNSS Sensitivity requirements in NR and LTE

Type: LS in		For: Information
					Original outgoing LS: R5-206900, to RAN4, cc PTCRB, PVG, GCF CAG, CTIA OTA Working Group
					Source: RAN5

Decision: 		The document was noted.

R4-2100022	Use of the 252-296 GHz frequency range by land-mobile service applications

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc -
					Source: ITU-R WP 5A

Decision: 		The document was noted.

R4-2100023	LS on single UL operation

Type: LS in		For: Information
					Original outgoing LS: RP-202932, to RAN2. RAN4, cc -
					Source: RAN

Decision: 		The document was noted.

R4-2100024	Reply LS to APT_LS200918 = RP-202143 on Frequency Arrangements for IMT in the Band 470- 703 MHz

Type: LS in		For: Information
					Original outgoing LS: RP-202934, to Asia-Pacific Telecommunity (APT) Wireless Group, cc RAN4
					Source: RAN

Decision: 		The document was noted.

R4-2100025	LS on BCS reporting and support for intra-band EN-DC band combinations

Type: LS in		For: Information
					Original outgoing LS: RP-202935, to RAN2, RAN4, cc -
					Source: RAN

Decision: 		The document was noted.

R4-2100026	LS on Use of Inclusive Language in 3GPP

Type: LS in		For: Information
					Original outgoing LS: SP-201143, to RAN4, cc -
					Source: SA

Decision: 		The document was noted.

R4-2100027	LS on OTA LTE UE TRP and TRS Requirements

Type: LS in		For: Information
					Original outgoing LS: -, to RAN, GCF Steering Group (SG), GSMA, NGMN, PTCRB, CTIA, cc RAN5, RAN4, GCF Conformance Agreement Group (CAG), GCF Performance Agreement Group (PAG), GSMA Terminal Steering Group (TSG), GSMA Networks Group (NG)
					Source: MSG TFES

Decision: 		The document was noted.

R4-2100028	Reply LS on questions to RAN WGs on dual Radio UE (2Rx/2Tx or 2Rx/1Tx) support for simultaneous communication with both SNPN and PLMN

Type: LS in		For: Information
					Original outgoing LS: R2-2011270, to SA2, cc RAN4
					Source: RAN2

Decision: 		The document was noted.

R4-2100029	Spurious emission limits for AAS BS in 6.425 – 7.125 GHz and 10-10.5 GHz

Type: LS in		For: Information
					Original outgoing LS: -, to RAN4, cc RAN
					Source: CEPT ECC PT1

Decision: 		The document was not treated.

## 4	Rel-15 New radio access technology

### 4.1	System Parameters Maintenance [NR_newRAT-Core]

R4-2102949	Email discussion summary for [98e][101] NR_NewRAT_SysParameters

Type: other		For: discussion
					Source: Moderator (ZTE)

Discussion:

The contribution summarised email discussion thread [98e][101] NR_NewRAT_SysParameters. The discussion thread was moderated by Aijun Cao (ZTE Wistron Telecom AB).

Decision: 		The document was revised to R4-2103291.

R4-2103291	Email discussion summary for [98e][101] NR_NewRAT_SysParameters

Type: other		For: discussion
					Source: Moderator (ZTE)

(Replaces R4-2102949)

Discussion:

The contribution summarised email discussion thread [98e][101] NR_NewRAT_SysParameters. The discussion thread was moderated by Aijun Cao (ZTE Wistron Telecom AB).

Decision: 		The document was noted.

R4-2102197	CR to TS38.101-1: Correction on the Aggregated Channel Bandwidth

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0675  Cat: F (Rel-15)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][101] NR_NewRAT_SysParameters in R4-2103291.

Decision: 		The document was revised to R4-2103116.

R4-2103116	CR to TS38.101-1: Correction on the Aggregated Channel Bandwidth

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0675  rev 1 Cat: F (Rel-15)

					Source: ZTE Corporation

(Replaces R4-2102197)

Discussion:

See email discussion summary for [98e][101] NR_NewRAT_SysParameters in R4-2103291.

Decision: 		The document was agreed.

R4-2102198	CR to TS38.101-1: Correction on the Aggregated Channel Bandwidth

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0676  Cat: A (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][101] NR_NewRAT_SysParameters in R4-2103291.

Decision: 		The document was agreed.

R4-2102199	CR to TS38.101-1: Correction on the Aggregated Channel Bandwidth

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0677  Cat: A (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][101] NR_NewRAT_SysParameters in R4-2103291.

Decision: 		The document was agreed.

R4-2102200	CR to TS38.104: Correction on the Aggregated Channel Bandwidth

Type: CR		For: Agreement
					38.104 v15.12.0	  CR-0292  Cat: F (Rel-15)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][101] NR_NewRAT_SysParameters in R4-2103291.

Decision: 		The document was revised to R4-2103117.

R4-2103117	CR to TS38.104: Correction on the Aggregated Channel Bandwidth

Type: CR		For: Agreement
					38.104 v15.12.0	  CR-0292  rev 1 Cat: F (Rel-15)

					Source: ZTE Corporation

(Replaces R4-2102200)

Discussion:

See email discussion summary for [98e][101] NR_NewRAT_SysParameters in R4-2103291.

Decision: 		The document was agreed.

R4-2102201	CR to TS38.104: Correction on the Aggregated Channel Bandwidth

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0293  Cat: A (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][101] NR_NewRAT_SysParameters in R4-2103291.

Decision: 		The document was agreed.

R4-2102202	CR to TS38.104: Correction on the Aggregated Channel Bandwidth

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0294  Cat: A (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][101] NR_NewRAT_SysParameters in R4-2103291.

Decision: 		The document was agreed.

### 4.2	UE RF requirements maintenance [NR_newRAT]

#### 4.2.1	[FR1] Maintenance for 38.101-1 [NR_newRAT-Core]

R4-2102950	Email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1

Type: other		For: discussion
					Source: Moderator (Nokia)

Discussion:

The contribution summarised email discussion thread [98e][102] NR_NewRAT_UE_RF_Part_1. The discussion thread was moderated by Hisashi Onozawa (Nokia Japan).

Decision: 		The document was revised to R4-2103292.

R4-2103292	Email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1

Type: other		For: discussion
					Source: Moderator (Nokia)

(Replaces R4-2102950)

Discussion:

The contribution summarised email discussion thread [98e][102] NR_NewRAT_UE_RF_Part_1. The discussion thread was moderated by Hisashi Onozawa (Nokia Japan).

Decision: 		The document was noted.

R4-2103118	LS on further clarification of simultaneous Rx/Tx capability

Type: LS out		For: Approval
					to -
					Source: Apple

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was withdrawn.

R4-2103120	Way forward on applicability of additional emission requirement to CA/DC

Type: other		For: discussion
					Source: SoftBank

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was approved.

R4-2103121	Way forward on WF on Improvement of UL RMC tables

Type: other		For: discussion
					Source: Rohde & Schwarz

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was approved.

R4-2100138	Reply LS on ambiguity in deciding TL,C

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was noted.

R4-2100139	CR REL15 on ambiguity in deciding TL,C

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0595  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was not pursued.

R4-2100140	CR REL16 on ambiguity in deciding TL,C

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0596  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was withdrawn.

R4-2100141	CR REL17 on ambiguity in deciding TL,C

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0597  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was withdrawn.

R4-2100164	IBE_mask_almost_contiguous_CR_rel15

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0602  Cat: F (Rel-15)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was not pursued.

R4-2100165	IBE_mask_almost_contiguous_CR_rel16-mirror

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0603  Cat: A (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was withdrawn.

R4-2102510	IBE requirement for almost contiguous allocations

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0693  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was withdrawn.

R4-2100392	CR for TS38 101-1 Rel-15 Correction for definition of P-MPR

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0610  Cat: F (Rel-15)

					Source: CATT

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was agreed.

R4-2100393	CR for TS38 101-1 Rel-16 Correction for definition of P-MPR

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0611  Cat: A (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was agreed.

R4-2100394	CR for TS38 101-1 Rel-17 Correction for definition of P-MPR

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0612  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was agreed.

R4-2100395	CR for TS38 101-1 Rel-16 Correction of condition for MPR and delta MPR

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0613  Cat: F (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was revised to R4-2103122.

R4-2103122	CR for TS38 101-1 Rel-16 Correction of condition for MPR and delta MPR

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0613  rev 1 Cat: F (Rel-16)

					Source: CATT

(Replaces R4-2100395)

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was revised to R4-2103389.

R4-2103389	CR for TS38 101-1 Rel-16 Correction of condition for MPR and delta MPR

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0613  rev 2 Cat: F (Rel-16)

					Source: CATT

(Replaces R4-2103122)

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was agreed.

R4-2100396	CR for TS38 101-1 Rel-17 Correction of condition for MPR and delta MPR

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0614  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was agreed.

R4-2100524	On simultaneous TxRx for NR-DC

Type: discussion		For: Discussion
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was noted.

R4-2101005	On applicability of additional emission requirement to CA/DC

Type: other		For: Approval
					38.101-1 v..
					Source: SoftBank Corp.

Abstract:

This contribution is intended to propose how to handle additional emission requirements in CA/DC context. Note that a part of proposals is related to 38.101-3

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was noted.

R4-2101174	IBE mask for almost contiguous allocations

Type: other		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was noted.

R4-2101713	Correction to applicability of simultaneous RX/TX

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0643  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

CR to correct the applicability of simultaneous RxTx for inter-band CA

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was revised to R4-2103119.

R4-2103119	Correction to applicability of simultaneous RX/TX

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0643  rev 1 Cat: F (Rel-15)

					Source: Ericsson

(Replaces R4-2101713)

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was not pursued.

R4-2101714	Correction to applicability of simultaneous RX/TX

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0644  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

CR to correct the applicability of simultaneous RxTx for inter-band CA

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was withdrawn.

R4-2101715	Correction to the lower limit of Pumax

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0645  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

CR to correct the lower limit (tolerance) of Pumax

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was not pursued.

R4-2101716	Correction to the lower limit of Pumax

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0646  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

CR to correct the lower limit (tolerance) of Pumax

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was withdrawn.

R4-2101717	Draft Reply LS on ambiguity in deciding TL,C

Type: LS out		For: Approval
					to RAN5
					Source: Ericsson

Abstract:

Draft Reply LS to RAN5 with proposals to modify the lower limit of the Pumax

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was noted.

R4-2101743	CR on simultaneous Tx-Rx for CA and SUL

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0649  Cat: F (Rel-15)

					Source: OPPO

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was not pursued.

R4-2101744	CR on simultaneous Tx-Rx for CA and SUL (R16 mirror CR)

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0650  Cat: A (Rel-16)

					Source: OPPO

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was withdrawn.

R4-2101745	CR on simultaneous Tx-Rx for CA and SUL (R17 mirror CR)

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0651  Cat: A (Rel-17)

					Source: OPPO

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was withdrawn.

R4-2101811	Discussion and reply draft LS on ambiguity in deciding TL,C

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was noted.

R4-2101947	Simplification of n70

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0661  Cat: F (Rel-15)

					Source: Dish Network

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was agreed.

R4-2101988	Simplification of n70

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0664  Cat: A (Rel-16)

					Source: Dish Network

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was agreed.

R4-2101992	Simplification of n70

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0665  Cat: A (Rel-17)

					Source: Dish Network

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was agreed.

R4-2101989	CR for 38.307 to delete the redundant information

Type: CR		For: Agreement
					38.307 v15.7.0	  CR-0052  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was not pursued.

R4-2101990	CR for 38.307 to delete the redundant information

Type: CR		For: Agreement
					38.307 v16.5.0	  CR-0053  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was withdrawn.

R4-2101991	CR for 38.307 to delete the redundant information

Type: CR		For: Agreement
					38.307 v17.0.0	  CR-0054  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was withdrawn.

R4-2102091	Improvement of UL RMC tables

Type: discussion		For: Approval
					Source: Rohde & Schwarz

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was noted.

R4-2102143	CR to TS38.101-1: Correction on applicability of minimum requirements

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0667  Cat: F (Rel-15)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was withdrawn.

R4-2102194	CR to TS38.101-1: Correction on applicability of minimum requirements

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0672  Cat: F (Rel-15)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was agreed.

R4-2102195	CR to TS38.101-1: Correction on applicability of minimum requirements

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0673  Cat: A (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was agreed.

R4-2102196	CR to TS38.101-1: Correction on applicability of minimum requirements

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0674  Cat: A (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was agreed.

R4-2102376	CR for TS 38.101-1 correction CR for simultaneous TxRx operation (R15)

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0682  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was not pursued.

R4-2102377	CR for TS 38.101-1: correction CR for simultaneous Tx/Rx operation (R16)

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0683  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was withdrawn.

R4-2102703	CR for TS 38.101-1: correction CR for simultaneous Tx/Rx operation (R17)

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0712  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was withdrawn.

R4-2102595	CR for TS 38.101-1: Cleanup for spurious emissions for UE co-existence table

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0696  Cat: F (Rel-15)

					Source: Apple

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was agreed.

R4-2102597	CR for TS 38.101-1: Correction to FR1 time mask for SRS antenna switching

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0697  Cat: F (Rel-15)

					Source: Apple

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was revised to R4-2103123.

R4-2103123	CR for TS 38.101-1: Correction to FR1 time mask for SRS antenna switching

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0697  rev 1 Cat: F (Rel-15)

					Source: Apple

(Replaces R4-2102597)

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was revised to R4-2103388.

R4-2103388	CR for TS 38.101-1: Correction to FR1 time mask for SRS antenna switching

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0697  rev 2 Cat: F (Rel-15)

					Source: Apple

(Replaces R4-2103123)

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was agreed.

R4-2102598	CR for TS 38.101-1: Correction to FR1 time mask for SRS antenna switching

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0698  Cat: A (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was agreed.

R4-2102599	CR for TS 38.101-1: Correction to FR1 time mask for SRS antenna switching

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0699  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was agreed.

R4-2102658	CR to 38.101-1: UL MIMO requirements update

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0705  Cat: F (Rel-15)

					Source: Qualcomm Incorporated, Lenovo, Motorola Mobility

Abstract:

Make 2L EVM requirement consistent with RAN1

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was revised to R4-2103355.

R4-2103355	CR to 38.101-1: UL MIMO requirements update

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0705  rev 1 Cat: F (Rel-15)

					Source: Qualcomm Incorporated, Lenovo, Motorola Mobility

(Replaces R4-2102658)

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was not pursued.

R4-2102659	CR to 38.101-1: UL MIMO requirements update

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0706  Cat: A (Rel-16)

					Source: Qualcomm Incorporated, Lenovo, Motorola Mobility

Abstract:

Make 2L EVM requirement consistent with RAN1

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was withdrawn.

R4-2102660	CR to 38.101-1: UL MIMO requirements update

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0707  Cat: A (Rel-17)

					Source: Qualcomm Incorporated, Lenovo, Motorola Mobility

Abstract:

Make 2L EVM requirement consistent with RAN1

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was withdrawn.

R4-2102661	On FR1 2L UL EVM Requirement

Type: discussion		For: Agreement
					Source: Qualcomm Incorporated, Lenovo, Motorola Mobility

Abstract:

2L EVM calculation detail, impact to other tests in transmit modulation quality section

Discussion:

See email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 in R4-2103292.

Decision: 		The document was noted.

#### 4.2.2	[FR2] Maintenance for 38.101-2 [NR_newRAT-Core]

R4-2102951	Email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2

Type: other		For: discussion
					Source: Moderator (Apple)

Discussion:

The contribution summarised email discussion thread [98e][103] NR_NewRAT_UE_RF_Part_2. The discussion thread was moderated by James Wang (Apple (UK).

Decision: 		The document was revised to R4-2103293.

R4-2103293	Email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2

Type: other		For: discussion
					Source: Moderator (Apple)

(Replaces R4-2102951)

Discussion:

The contribution summarised email discussion thread [98e][103] NR_NewRAT_UE_RF_Part_2. The discussion thread was moderated by James Wang (Apple (UK).

Decision: 		The document was noted.

R4-2103124	LS on SCell dropping

Type: LS out		For: Approval
					to RAN5
					Source: RAN4

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293. The document was subject for post-meeting email approval.

Decision: 		The document was revised to R4-2103410.

R4-2103410	LS on SCell dropping

Type: LS out		For: Approval
					to RAN5
					Source: RAN4

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293. The document was subject for post-meeting email approval.

Decision: 		The document was approved.

R4-2103124	LS on SCell dropping

Type: LS out		For: Approval
					to RAN5
					Source: RAN4

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293. The document was subject for post-meeting email approval.

Decision: 		The document was approved.

R4-2100085	Clarification on NS_203 support by n258

Type: CR		For: Agreement
					38.101-2 v15.12.0	  CR-0313  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Clarification on NS_203 support by n258

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was revised to R4-2103125.

R4-2103125	Removal of a remaining NS_201 related requirement

Type: CR		For: Agreement
					38.101-2 v15.12.0	  CR-0313  rev 1 Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100085)

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was revised to R4-2103370.

R4-2103370	Removal of a remaining NS_201 related requirement

Type: CR		For: Agreement
					38.101-2 v15.12.0	  CR-0313  rev 2 Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2103125)

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was agreed.

R4-2100086	Removal of a remaining NS_201 related requirement

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0314  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was agreed.

R4-2100087	Removal of a remaining NS_201 related requirement

Type: CR		For: Agreement
					38.101-2 v17.0.0	  CR-0315  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was agreed.

R4-2100109	On NS_203/CA_NS_203 for n258

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution discusses necessity of explicitly describing this information in the specification.

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was noted.

R4-2100586	P_min correction and P_cmax CA correction to apply from all cells

Type: CR		For: Agreement
					38.101-2 v15.12.0	  CR-0320  Cat: F (Rel-15)

					Source: Qualcomm Incorporated

Abstract:

Change "each cell" to "all cells" since pcmax is calculated based on all simultaneous grants from all allocated CC's.

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was revised to R4-2103127.

R4-2103127	P_min correction and P_cmax CA correction to apply from all cells

Type: CR		For: Agreement
					38.101-2 v15.12.0	  CR-0320  rev 1 Cat: F (Rel-15)

					Source: Qualcomm Incorporated

(Replaces R4-2100586)

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was not pursued.

R4-2100587	P_min correction and P_cmax CA correction to apply from all cells

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0321  Cat: A (Rel-16)

					Source: Qualcomm Incorporated

Abstract:

Change "each cell" to "all cells" since pcmax is calculated based on all simultaneous grants from all allocated CC's.

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was withdrawn.

R4-2100588	P_min correction and P_cmax CA correction to apply from all cells

Type: CR		For: Agreement
					38.101-2 v17.0.0	  CR-0322  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

Change "each cell" to "all cells" since pcmax is calculated based on all simultaneous grants from all allocated CC's.

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was withdrawn.

R4-2101201	Further discussion on EESS protection

Type: other		For: Approval
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was noted.

R4-2101523	Discussion on WRC-19 remaining issues

Type: discussion		For: Approval
					Source: OPPO

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was noted.

R4-2101738	Discussion on FR2 equal PSD in CA and draft LS

Type: discussion		For: Approval
					Source: OPPO

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was noted.

R4-2101739	CR on FR2 equal PSD in UL CA

Type: CR		For: Agreement
					38.101-2 v15.12.0	  CR-0329  Cat: F (Rel-15)

					Source: OPPO

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was not pursued.

R4-2101740	CR on FR2 equal PSD in UL CA (R16 mirror CR)

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0330  Cat: A (Rel-16)

					Source: OPPO

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was withdrawn.

R4-2101741	CR on FR2 equal PSD in UL CA (R17 mirror CR)

Type: CR		For: Agreement
					38.101-2 v17.0.0	  CR-0331  Cat: A (Rel-17)

					Source: OPPO

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was withdrawn.

R4-2102662	Discussion on FR2 UE Min. Output Power Requirement

Type: discussion		For: Agreement
					Source: Qualcomm Finland RFFE Oy

Abstract:

Establish consistency in Pmin specs across single CC, CA and UL MIMO configurations

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was noted.

R4-2102663	Completion of beam correspondence requirements for all power classes

Type: discussion		For: Agreement
					Source: Qualcomm Finland RFFE Oy

Abstract:

The beam correspondence requirement for partial beam correspondence UEs (‘bit 0 UE’) was completed for PC3 UEs as part of Rel-15 work. We propose completing the requirement by addressing the other power classes

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was noted.

R4-2102664	CR to 38.101-2 on beam correspondence

Type: CR		For: Agreement
					38.101-2 v15.12.0	  CR-0335  Cat: F (Rel-15)

					Source: Qualcomm, Nokia, Nokia Shanghai Bell, Samsung, Verizon, NTT Docomo, Sony, Ericsson

Abstract:

Completion of missing requirements

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was revised to R4-2103126.

R4-2103126	CR to 38.101-2 on beam correspondence

Type: CR		For: Agreement
					38.101-2 v15.12.0	  CR-0335  rev 1 Cat: F (Rel-15)

					Source: Qualcomm, Nokia, Nokia Shanghai Bell, Samsung, Verizon, NTT Docomo, Sony, Ericsson

(Replaces R4-2102664)

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was agreed.

R4-2102665	CR to 38.101-2 on beam correspondence

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0336  Cat: A (Rel-16)

					Source: Qualcomm, Nokia, Nokia Shanghai Bell, Samsung, Verizon, NTT Docomo, Sony, Ericsson

Abstract:

Completion of missing requirements

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was agreed.

R4-2102666	CR to 38.101-2 on beam correspondence

Type: CR		For: Agreement
					38.101-2 v17.0.0	  CR-0337  Cat: A (Rel-17)

					Source: Qualcomm, Nokia, Nokia Shanghai Bell, Samsung, Verizon, NTT Docomo, Sony, Ericsson

Abstract:

Completion of missing requirements

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was agreed.

R4-2102677	Frequency separation class clarification

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0341  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

This paper is an alignement with the Freq separation class signalling in TS38.331

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was not pursued.

R4-2102678	Frequency separation class clarification

Type: CR		For: Agreement
					38.101-2 v17.0.0	  CR-0342  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

This paper is an alignement with the Freq separation class signalling in TS38.331

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was withdrawn.

R4-2102716	CR on FR2 intra-band UL CA

Type: CR		For: Agreement
					38.101-2 v15.12.0	  CR-0343  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was revised to R4-2103348.

R4-2103348	CR on FR2 intra-band UL CA

Type: CR		For: Agreement
					38.101-2 v15.12.0	  CR-0343  rev 1 Cat: F (Rel-15)

					Source: Huawei, HiSilicon

(Replaces R4-2102716)

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was agreed.

R4-2102815	CR on FR2 intra-band UL CA

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0345  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was agreed.

R4-2103391	CR on FR2 intra-band UL CA

Type: CR		For: Agreement
					38.101-2 v17.0.0	  CR-0346  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was agreed.

R4-2102924	Discussion on FR2 UE Min. Output Power Requirement

Type: discussion		For: Agreement
					Source: Qualcomm Incorporated

Abstract:

Establish consistency in Pmin specs across single CC, CA and UL MIMO configurations

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was withdrawn.

R4-2102925	Completion of beam correspondence requirements for all power classes

Type: discussion		For: Agreement
					Source: Qualcomm Incorporated

Abstract:

The beam correspondence requirement for partial beam correspondence UEs (‘bit 0 UE’) was completed for PC3 UEs as part of Rel-15 work. We propose completing the requirement by addressing the other power classes

Discussion:

See email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 in R4-2103293.

Decision: 		The document was withdrawn.

#### 4.2.3	Maintenance for 38.101-3 [NR_newRAT-Core]

R4-2102952	Email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][104] NR_NewRAT_UE_RF_Part_3. The discussion thread was moderated by Peng Zhang (Huawei).

Decision: 		The document was revised to R4-2103294.

R4-2103294	Email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3

Type: other		For: discussion
					Source: Moderator (Huawei)

(Replaces R4-2102952)

Discussion:

The contribution summarised email discussion thread [98e][104] NR_NewRAT_UE_RF_Part_3. The discussion thread was moderated by Peng Zhang (Huawei).

Decision: 		The document was noted.

R4-2103115	Way forward on BCS reporting and support for intra-band EN-DC band combinations

Type: other		For: discussion
					Source: T-Mobile USA

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was noted.

R4-2103349	Way forward on BCS reporting and support for intra-band EN-DC band combinations

Type: other		For: discussion
					Source: Bell Mobility

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was approved.

R4-2103130	Way forward on UE capability on intraBandENDC-Support

Type: other		For: discussion
					Source: Huawei

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was approved.

R4-2101111	Discussion and reply draft LS on BCS reporting and support for intra-band EN-DC band combinations

Type: discussion		For: Approval
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was revised to R4-2102937.

R4-2102937	Discussion and reply draft LS on BCS reporting and support for intra-band EN-DC band combinations

Type: discussion		For: Approval
					Source: Xiaomi

(Replaces R4-2101111)

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was noted.

R4-2101143	Discussion on the reply to LS on BCS for intra-band EN-DC band combinations

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was noted.

R4-2101718	Correction to applicability of simultaneous RX/TX and single-UL transmission

Type: CR		For: Agreement
					38.101-3 v15.12.0	  CR-0461  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

CR to correct the applicability of simultaneous RxTx for inter-band EN-DC and single-UL

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was not pursued.

R4-2101719	Correction to applicability of simultaneous RX/TX and single-UL transmission

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0462  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

CR to correct the applicability of simultaneous RxTx for inter-band EN-DC and single-UL

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was withdrawn.

R4-2101742	Discussion on simultaneous Tx-Rx

Type: discussion		For: Approval
					Source: OPPO

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was noted.

R4-2101746	CR on simultaneous Tx-Rx for EN-DC

Type: CR		For: Agreement
					38.101-3 v15.12.0	  CR-0464  Cat: F (Rel-15)

					Source: OPPO

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was revised to R4-2103128.

R4-2103128	CR on simultaneous Tx-Rx for EN-DC

Type: CR		For: Agreement
					38.101-3 v15.12.0	  CR-0464  rev 1 Cat: F (Rel-15)

					Source: OPPO

(Replaces R4-2101746)

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was not pursued.

R4-2101747	CR on simultaneous Tx-Rx for EN-DC (R16 CR)

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0465  Cat: F (Rel-16)

					Source: OPPO

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was revised to R4-2103129.

R4-2103129	CR on simultaneous Tx-Rx for EN-DC (R16 CR)

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0465  rev 1 Cat: F (Rel-16)

					Source: OPPO

(Replaces R4-2101747)

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was not pursued.

R4-2101748	CR on simultaneous Tx-Rx for EN-DC (R17 mirror CR)

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0466  Cat: A (Rel-17)

					Source: OPPO

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was withdrawn.

R4-2101750	Discussion on BCS for intra-band EN-DC and draft reply LS

Type: discussion		For: Approval
					Source: OPPO

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was noted.

R4-2101853	Draft reply LS on on BCS reporting and support for intra-band EN-DC band combinations

Type: LS out		For: Approval
					to RAN2, cc RAN
					Source: ZTE Wistron Telecom AB

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was noted.

R4-2102094	CR for 38.101-3 to introduce a new MSD due to the counter intermodulation interference(Rel-15)

Type: CR		For: Agreement
					38.101-3 v15.12.0	  CR-0474  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was revised to R4-2103357.

R4-2103357	CR for 38.101-3 to introduce a new MSD due to the counter intermodulation interference(Rel-15)

Type: CR		For: Agreement
					38.101-3 v15.12.0	  CR-0474  rev 1 Cat: F (Rel-15)

					Source: Huawei, HiSilicon

(Replaces R4-2102094)

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was agreed.

R4-2102095	CR for 38.101-3 to introduce a new MSD due to the counter intermodulation interference(Rel-16)

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0475  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was agreed.

R4-2102096	CR for 38.101-3 to introduce a new MSD due to the counter intermodulation interference(Rel-17)

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0476  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was agreed.

R4-2102375	Further consideration on simultaneous RxTx UE capability

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was noted.

R4-2102378	CR for TS 38.101-3 correction CR for simultaneous TxRx operation (R15)

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0684  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was not pursued.

R4-2102379	CR for TS 38.101-3: correction CR for simultaneous Tx/Rx operation (R16)

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0481  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was withdrawn.

R4-2102717	CR for TS 38.101-3: correction CR for simultaneous Tx/Rx operation (R17)

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0494  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was withdrawn.

R4-2102504	BCS reporting and support for intra-band EN-DC band combination

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was noted.

R4-2102505	CR for 38.101-3 on applicability of minimum requirements for EN-DC (Rel-15)

Type: CR		For: Agreement
					38.101-3 v15.12.0	  CR-0489  Cat: F (Rel-15)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was not pursued.

R4-2102506	CR for 38.101-3 on applicability of minimum requirements for EN-DC (Rel-16)

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0490  Cat: A (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was withdrawn.

R4-2102507	CR for 38.101-3 on applicability of minimum requirements for EN-DC (Rel-17)

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0491  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was withdrawn.

R4-2102559	Clarification of intra-bandENDC-Support

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was noted.

R4-2102593	CR for bug fixing of band combination tables for 38101-3 Rel17

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0492  Cat: F (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was revised to R4-2103345.

R4-2103345	CR for bug fixing of band combination tables for 38101-3 Rel17

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0492  rev 1 Cat: F (Rel-17)

					Source: Apple

(Replaces R4-2102593)

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was agreed.

R4-2102594	CR for bug fixing of band combination tables for 38101-3 Rel16

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0493  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was revised to R4-2103346.

R4-2103346	CR for bug fixing of band combination tables for 38101-3 Rel16

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0493  rev 1 Cat: F (Rel-16)

					Source: Apple

(Replaces R4-2102594)

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was agreed.

R4-2102628	on UE capability for intra-band ENDC

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was noted.

R4-2102388	On BCS reporting and support for intra-band EN-DC band combinations

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was noted.

R4-2102148	Discussion on LS RP-202935 BCS reporting and support for intra-band EN-DC band combinations

Type: discussion		For: Approval
					Source: T-Mobile USA, Bell Mobility, TELUS, Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was noted.

R4-2102149	Reply LS to RP-202935 on BCS reporting and support for intra-band EN-DC band combinations

Type: LS out		For: Approval
					to RAN, RAN2
					Source: RAN4

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was approved.

R4-2103401	Second Reply LS to RP-202935 = R4-2100025 on BCS reporting and support for intra-band EN-DC band combinations

Type: LS out		For: Approval
					to RAN, RAN2
					Source: RAN4

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was approved.

Attachments to this outgoing LS: R4-2103402

R4-2103402	CR for 38.101-3: clarification of intra-band EN-DC BCS applicability

Type: CR		For: Agreement
					38.101-3 v..	  CR-0502  Cat: - (Rel-15)

					Source: : T-Mobile USA

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was agreed.

R4-2103403	CR for 38.101-3: clarification of intra-band EN-DC BCS applicability

Type: CR		For: Agreement
					38.101-3 v..	  CR-0503  Cat: - (Rel-16)

					Source: : T-Mobile USA

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was agreed.

R4-2103404	CR for 38.101-3: clarification of intra-band EN-DC BCS applicability

Type: CR		For: Agreement
					38.101-3 v..	  CR-0504  Cat: - (Rel-17)

					Source: : T-Mobile USA

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was agreed.

### 4.3	UE EMC requirements maintenance [NR_newRAT-Core]

R4-2103742	Email discussion summary for [98e] [303] NR_EMC

Type: other		For: discussion
					Source: Moderator (ZTE)

Discussion:

The contribution summarised email discussion thread [98e][303] NR_EMC. The discussion thread was moderated by Wubin Zhou (ShenZhen Zhongxing Shitong).

Decision: 		The document was revised to R4-2103922.

R4-2103922	Email discussion summary for [98e][303] NR_EMC

Type: other		For: discussion
					Source: Moderator (ZTE)

(Replaces R4-2103742)

Discussion:

The contribution summarised email discussion thread [98e][303] NR_EMC. The discussion thread was moderated by Wubin Zhou (ShenZhen Zhongxing Shitong).

Decision: 		The document was noted.

R4-2103770	WF on single carrier NR FR2 UE EMC requirement

Type: other		For: discussion
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was approved.

R4-2100890	CR to TS38.124 on radiated emissions

Type: CR		For: Agreement
					38.124 v15.4.0	  CR-0029  Cat: F (Rel-15)

					Source: Samsung

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was not pursued.

R4-2100891	CR to TS38.124 on radiated emissions

Type: CR		For: Agreement
					38.124 v16.1.0	  CR-0030  Cat: A (Rel-16)

					Source: Samsung

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was withdrawn.

R4-2101872	on FR2 UE EMC requirement

Type: discussion		For: (not specified)
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was noted.

R4-2102398	CR for TS 38.124: Correction of FR1 radiated spurious emissions (R15)

Type: CR		For: Agreement
					38.124 v15.4.0	  CR-0031  Cat: F (Rel-15)

					Source: Huawei, HiSilicon, Bureau Veritas

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was revised to R4-2103771.

R4-2103771	CR for TS 38.124: Correction of FR1 radiated spurious emissions (R15)

Type: CR		For: Agreement
					38.124 v15.4.0	  CR-0031  rev 1 Cat: F (Rel-15)

					Source: Huawei, HiSilicon, Bureau Veritas,Samsung

(Replaces R4-2102398)

Discussion:

Session chair note: please correct the coverpage error as MCC pointed out!

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was agreed.

R4-2102399	CR for TS 38.124: Correction of FR1 radiated spurious emissions (R16)

Type: CR		For: Agreement
					38.124 v16.1.0	  CR-0032  Cat: A (Rel-16)

					Source: Huawei, HiSilicon, Bureau Veritas, Samsung

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was agreed.

R4-2102576	CR to TS 38.124: correction of the lower frequency range of the RSE, Rel-15

Type: CR		For: Agreement
					38.124 v15.4.0	  CR-0033  Cat: F (Rel-15)

					Source: Huawei

Abstract:

Lower spurious range limit for RSE is corrected to 30 MHz, to align with SM.329 and other UE EMC specifications.

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was not pursued.

R4-2102577	CR to TS 38.124: correction of the lower frequency range of the RSE, Rel-16

Type: CR		For: Agreement
					38.124 v16.1.0	  CR-0034  Cat: A (Rel-16)

					Source: Huawei

Abstract:

Lower spurious range limit for RSE is corrected to 30 MHz, to align with SM.329 and other UE EMC specifications.

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was withdrawn.

### 4.4	BS RF requirements maintenance [NR_newRAT-Core]

#### 4.4.1	General [NR_newRAT-Core]

R4-2103740	Email discussion summary for [98e][301] BSRF_Maintenance

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][301] BSRF_Maintenance. The discussion thread was moderated by Johan Sköld (Ericsson).

Decision: 		The document was revised to R4-2103923.

R4-2103923	Email discussion summary for [98e][301] BSRF_Maintenance

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2103740)

Discussion:

The contribution summarised email discussion thread [98e][301] BSRF_Maintenance. The discussion thread was moderated by Johan Sköld (Ericsson).

Decision: 		The document was noted.

--------------------------GTW agenda on Jan.29th for email thread [301] -----------------------

Topic 1 Japan regulation for 2.5GHz

Summary from moderator based on 1st round discussion:

There were no objections to the overall proposal to introduce the Japanese regulation in LTE and NR specs.

On the specific CRs, several issues were raised in discussions:

- The definition of limits “per cell”, which is not a concept defined in 3GPP. Proposed to consider the use of “sum over antenna connectors”.

- The use of “unless stated differently in regional regulations”, which should be avoided. It was argued that it is better to state specific exceptions.

- Specifically, the general spurious emission limits (Category A) should be left unchanged and should not be regional. Exceptions could be added in relation to the reference point, since it is now 250% of NB.

- Keeping the sentence “In certain regions, additional regional requirements may apply, since it is general and concerns not only Japan.

All CRs need to be revised and the detailed text is for further discussion.

Discussion:

NEC: We are fine to accept all the issues and fixed in the revised CRs. We are using the generic limit not on emission limits.

For LTE spec CRs, the situation is different. We are fine to keep the text in LTE as it’s. The only changes to LTE spec changes is OBUE and spurious emission.

Softbank: Concerning with LTE handling, we have discussion with Nokia together within Japanese party, and the agreement is we would like to avoid the heavy maintenance work on LTE specifications.

Topic #2: AAS capability set and support for NR+UTRA

Summary from moderator for 1st round

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | There is no objection to applying Option 2 in Rel-16. Agreements: As a Way-Forward, applying Option 1 in Rel-15 and Option 2 in Rel-16Seems to be agreeable. |

---------------------------------------End -------------------------------------------

R4-2101016	Support of Japan regulation for 2.5GHz(BWA) in NR BS

Type: other		For: Information
					Source: SoftBank Corp., KDDI Corporation, NEC Corporation

Abstract:

This paper provides background info for the proposals to support n41/n90 in BS for Japan.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was noted.

R4-2102844	CR to 37.105 on NR+UTRA support for AAS

Type: CR		For: Agreement
					37.105 v15.11.0	  CR-0224  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

Presently, it is not explicitly explained in TS 37.105 what RATs and RAT combinations that are not supported by AAS BS. This is clarified by the CR.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was revised to R4-2103785.

R4-2103785	CR to 37.105 on NR+UTRA support for AAS

Type: CR		For: Agreement
					37.105 v15.11.0	  CR-0224  rev 1 Cat: F (Rel-15)

					Source: Ericsson

(Replaces R4-2102844)

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2102845	CR to 37.105 on NR+UTRA support for AAS

Type: CR		For: Agreement
					37.105 v16.6.0	  CR-0225  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

Presently, it is not explicitly explained in TS 37.105 what RATs and RAT combinations that are not supported by AAS BS. This is clarified by the CR.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was withdrawn.

R4-2102846	CR to 37.105 on NR+UTRA support for AAS

Type: CR		For: Agreement
					37.105 v17.0.0	  CR-0226  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Presently, it is not explicitly explained in TS 37.105 what RATs and RAT combinations that are not supported by AAS BS. This is clarified by the CR.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was withdrawn.

#### 4.4.2	TX/RX requirements maintenance (38.104) [NR_newRAT-Core]

R4-2101088	CR to TS 38.104: Additions of regional requirements for n41 in Japan, Rel-15

Type: CR		For: Agreement
					38.104 v15.12.0	  CR-0271  Cat: F (Rel-15)

					Source: NEC, SoftBank, KDDI

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was revised to R4-2103777.

R4-2103777	CR to TS 38.104: Additions of regional requirements for n41 in Japan, Rel-15

Type: CR		For: Agreement
					38.104 v15.12.0	  CR-0271  rev 1 Cat: F (Rel-15)

					Source: NEC, SoftBank, KDDI, Nokia

(Replaces R4-2101088)

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2101089	CR to TS 38.104: Additions of regional requirements for n41 and n90 in Japan, Rel-16

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0272  Cat: F (Rel-16)

					Source: NEC, SoftBank, KDDI

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was revised to R4-2103958.

R4-2103958	CR to TS 38.104: Additions of regional requirements for n41 and n90 in Japan, Rel-16

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0272  rev 1 Cat: F (Rel-16)

					Source: NEC, SoftBank, KDDI, Nokia

(Replaces R4-2101089)

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2101090	CR to TS 38.104: Additions of regional requirements for n41 and n90 in Japan, Rel-17

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0273  Cat: A (Rel-17)

					Source: NEC, SoftBank, KDDI, Nokia

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2101091	CR to TS 38.141-1: Additions of regional requirements for n41 in Japan, Rel-15

Type: CR		For: Agreement
					38.141-1 v15.7.0	  CR-0182  Cat: F (Rel-15)

					Source: NEC, SoftBank, KDDI

Discussion:

Session Chair: Move to this AI from AI 4.5.2 to treat together with the changes to core specification. See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was revised to R4-2103778.

R4-2103778	CR to TS 38.141-1: Additions of regional requirements for n41 in Japan, Rel-15

Type: CR		For: Agreement
					38.141-1 v15.7.0	  CR-0182  rev 1 Cat: F (Rel-15)

					Source: NEC, SoftBank, KDDI, Nokia

(Replaces R4-2101091)

Discussion:

Session Chair: Move to this AI from AI 4.5.2 to treat together with the changes to core specification. See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2101092	CR to TS 38.141-1: Additions of regional requirements for n41 and n90 in Japan, Rel-16

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0183  Cat: F (Rel-16)

					Source: NEC, SoftBank, KDDI

Discussion:

Session Chair: Move to this AI from AI 4.5.2 to treat together with the changes to core specification. See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was revised to R4-2103779.

R4-2103779	CR to TS 38.141-1: Additions of regional requirements for n41 and n90 in Japan, Rel-16

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0183  rev 1 Cat: F (Rel-16)

					Source: NEC, SoftBank, KDDI, Nokia

(Replaces R4-2101092)

Discussion:

Session Chair: Move to this AI from AI 4.5.2 to treat together with the changes to core specification. See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2101093	CR to TS 38.141-1: Additions of regional requirements for n41 and n90 in Japan, Rel-17

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0184  Cat: A (Rel-17)

					Source: NEC, SoftBank, KDDI, Nokia

Discussion:

Session Chair: Move to this AI from AI 4.5.2 to treat together with the changes to core specification. See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2101094	CR to TS 38.141-2: Additions of regional requirements for n41 in Japan, Rel-15

Type: CR		For: Agreement
					38.141-2 v15.8.0	  CR-0277  Cat: F (Rel-15)

					Source: NEC, SoftBank, KDDI

Discussion:

Session Chair: Move to this AI from AI 4.5.3 to treat together with the changes to core specification. See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was revised to R4-2103780.

R4-2103780	CR to TS 38.141-2: Additions of regional requirements for n41 in Japan, Rel-15

Type: CR		For: Agreement
					38.141-2 v15.8.0	  CR-0277  rev 1 Cat: F (Rel-15)

					Source: NEC, SoftBank, KDDI, Nokia

(Replaces R4-2101094)

Discussion:

Session Chair: Move to this AI from AI 4.5.3 to treat together with the changes to core specification. See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2101095	CR to TS 38.141-2: Additions of regional requirements for n41 and n90 in Japan, Rel-16

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0278  Cat: F (Rel-16)

					Source: NEC, SoftBank, KDDI

Discussion:

Session Chair: Move to this AI from AI 4.5.3 to treat together with the changes to core specification. See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was revised to R4-2103781.

R4-2103781	CR to TS 38.141-2: Additions of regional requirements for n41 and n90 in Japan, Rel-16

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0278  rev 1 Cat: F (Rel-16)

					Source: NEC, SoftBank, KDDI, Nokia

(Replaces R4-2101095)

Discussion:

Session Chair: Move to this AI from AI 4.5.3 to treat together with the changes to core specification. See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2101096	CR to TS 38.141-2: Additions of regional requirements for n41 and n90 in Japan, Rel-17

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0279  Cat: A (Rel-17)

					Source: NEC, SoftBank, KDDI, Nokia

Discussion:

Session Chair: Move to this AI from AI 4.5.3 to treat together with the changes to core specification. See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2101994	CR to TS 38.104: EESS protection requirement correction

Type: CR		For: Agreement
					38.104 v15.12.0	  CR-0287  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2101995	CR to TS 38.104: EESS protection requirement correction

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0288  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2101996	CR to TS 38.104: EESS protection requirement correction

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0289  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2101997	CR to TS 38.141-2: EESS protection requirement correction

Type: CR		For: Agreement
					38.141-2 v15.8.0	  CR-0296  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

Session Chair: Move to this AI from AI 4.5.3 to treat together with the changes to core specification. See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2101998	CR to TS 38.141-2: EESS protection requirement correction

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0297  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

Session Chair: Move to this AI from AI 4.5.3 to treat together with the changes to core specification. See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2101999	CR to TS 38.141-2: EESS protection requirement correction

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0298  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

Session Chair: Move to this AI from AI 4.5.3 to treat together with the changes to core specification. See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

#### 4.4.3	MSR specifications maintenance [NR_newRAT-Core/Perf]

R4-2102441	CR to 37.141: Correction to ACLR limit in non-contiguous spectrum (Rel-15)

Type: CR		For: Agreement
					37.141 v15.13.0	  CR-0961  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2102442	CR to 37.141: Correction to ACLR limit in non-contiguous spectrum (Rel-16)

Type: CR		For: Agreement
					37.141 v16.8.0	  CR-0962  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2102443	CR to 37.141: Correction to ACLR limit in non-contiguous spectrum (Rel-17)

Type: CR		For: Agreement
					37.141 v17.0.0	  CR-0963  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2102841	CR to 37.145-1 on Removal of additional limit for Band 7

Type: CR		For: Agreement
					37.145-1 v15.8.0	  CR-0248  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

In CRs to RAN4#97-E, additional limits for Bands 1 and 7 were removed, triggered by an LS from ETSI TFES. Limits for Band 7 however remain in TS 37.145-1. Those are removed with this CR to ensure that all specifications are aligned.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2102842	CR to 37.145-1 on Removal of additional limit for Band 7

Type: CR		For: Agreement
					37.145-1 v16.5.0	  CR-0249  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

In CRs to RAN4#97-E, additional limits for Bands 1 and 7 were removed, triggered by an LS from ETSI TFES. Limits for Band 7 however remain in TS 37.145-1. Those are removed with this CR to ensure that all specifications are aligned.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2102843	CR to 37.145-1 on Removal of additional limit for Band 7

Type: CR		For: Agreement
					37.145-1 v17.0.0	  CR-0250  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

In CRs to RAN4#97-E, additional limits for Bands 1 and 7 were removed, triggered by an LS from ETSI TFES. Limits for Band 7 however remain in TS 37.145-1. Those are removed with this CR to ensure that all specifications are aligned.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2102847	CR to 37.104 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.104 v15.12.0	  CR-0934  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

The OBUE applicability tables headings are ambiguous, and sometimes not complete or fully aligned with the intended applicability. The CR corrects the headings to align.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was revised to R4-2103883.

R4-2103883	CR to 37.104 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.104 v15.12.0	  CR-0934  rev 1 Cat: F (Rel-15)

					Source: Ericsson

(Replaces R4-2102847)

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102848	CR to 37.104 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.104 v16.8.0	  CR-0935  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

The OBUE applicability tables headings are ambiguous, and sometimes not cmoplete or fully aligned with the intended applicability. The CR corrects the headings to align.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102849	CR to 37.104 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.104 v17.0.0	  CR-0936  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

The OBUE applicability tables headings are ambiguous, and sometimes not cmoplete or fully aligned with the intended applicability. The CR corrects the headings to align.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102850	CR to 37.141 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.141 v15.13.0	  CR-0973  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

The OBUE applicability tables headings are ambiguous, and sometimes not cmoplete or fully aligned with the intended applicability. The CR corrects the headings to align.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was revised to R4-2103884.

R4-2103884	CR to 37.141 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.141 v15.13.0	  CR-0973  rev 1 Cat: F (Rel-15)

					Source: Ericsson

(Replaces R4-2102850)

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102851	CR to 37.141 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.141 v16.8.0	  CR-0974  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

The OBUE applicability tables headings are ambiguous, and sometimes not cmoplete or fully aligned with the intended applicability. The CR corrects the headings to align.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102852	CR to 37.141 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.141 v17.0.0	  CR-0975  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

The OBUE applicability tables headings are ambiguous, and sometimes not cmoplete or fully aligned with the intended applicability. The CR corrects the headings to align.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102853	CR to 37.105 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.105 v15.11.0	  CR-0227  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

The OBUE applicability tables headings are ambiguous, and sometimes not cmoplete or fully aligned with the intended applicability. The CR corrects the headings to align.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was revised to R4-2103885.

R4-2103885	CR to 37.105 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.105 v15.11.0	  CR-0227  rev 1 Cat: F (Rel-15)

					Source: Ericsson

(Replaces R4-2102853)

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102854	CR to 37.105 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.105 v16.6.0	  CR-0228  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

The OBUE applicability tables headings are ambiguous, and sometimes not cmoplete or fully aligned with the intended applicability. The CR corrects the headings to align.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was withdrawn.

R4-2102855	CR to 37.105 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.105 v17.0.0	  CR-0229  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

The OBUE applicability tables headings are ambiguous, and sometimes not cmoplete or fully aligned with the intended applicability. The CR corrects the headings to align.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was withdrawn.

R4-2102856	CR to 37.145-1 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.145-1 v15.8.0	  CR-0251  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

The OBUE applicability tables headings are ambiguous, and sometimes not cmoplete or fully aligned with the intended applicability. The CR corrects the headings to align.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was revised to R4-2103886.

R4-2103886	CR to 37.145-1 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.145-1 v15.8.0	  CR-0251  rev 1 Cat: F (Rel-15)

					Source: Ericsson

(Replaces R4-2102856)

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102857	CR to 37.145-1 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.145-1 v16.5.0	  CR-0252  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

The OBUE applicability tables headings are ambiguous, and sometimes not cmoplete or fully aligned with the intended applicability. The CR corrects the headings to align.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was withdrawn.

R4-2102858	CR to 37.145-1 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.145-1 v17.0.0	  CR-0253  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

The OBUE applicability tables headings are ambiguous, and sometimes not cmoplete or fully aligned with the intended applicability. The CR corrects the headings to align.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was withdrawn.

R4-2102859	CR to 37.145-2 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.145-2 v15.9.0	  CR-0288  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

The OBUE applicability tables headings are ambiguous, and sometimes not cmoplete or fully aligned with the intended applicability. The CR corrects the headings to align.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was revised to R4-2103887.

R4-2103887	CR to 37.145-2 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.145-2 v15.9.0	  CR-0288  rev 1 Cat: F (Rel-15)

					Source: Ericsson

(Replaces R4-2102859)

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102860	CR to 37.145-2 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.145-2 v16.6.0	  CR-0289  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

The OBUE applicability tables headings are ambiguous, and sometimes not cmoplete or fully aligned with the intended applicability. The CR corrects the headings to align.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was withdrawn.

R4-2102861	CR to 37.145-2 on OBUE table headings and applicability

Type: CR		For: Agreement
					37.145-2 v17.0.0	  CR-0290  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

The OBUE applicability tables headings are ambiguous, and sometimes not cmoplete or fully aligned with the intended applicability. The CR corrects the headings to align.

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was withdrawn.

### 4.5	BS conformance testing Maintenance [NR_newRAT-Perf]

#### 4.5.1	General [NR_newRAT-Perf]

R4-2103741	Email discussion summary for [98e][302] NR_Conformance_Maintenance

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][302] NR_Conformance_Maintenance. The discussion thread was moderated by Richard Kybett (Huawei).

Decision: 		The document was revised to R4-2103924.

R4-2103924	Email discussion summary for [98e][302] NR_Conformance_Maintenance

Type: other		For: discussion
					Source: Moderator (Huawei)

(Replaces R4-2103741)

Discussion:

The contribution summarised email discussion thread [98e][302] NR_Conformance_Maintenance. The discussion thread was moderated by Richard Kybett (Huawei).

Decision: 		The document was noted.

R4-2103787	WF on adjacent channel co-location

Type: other		For: discussion
					Source: Huawei

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was approved.

-------------------------------GTW agenda on Jan.29th for email thread [303] ---------------------

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 CLTA Height | Tentative agreements: The contents of the table seem acceptable to all with the contents of the note describing the use of the 2 definitions requiring work Candidate options: Option 1: CATT: NOTE 2: Only applicable when the vertical radiating dimension of out-of-band CLTAs is smaller than the test object vertical radiating length. NOTE 3: Applicable when the vertical radiating dimension of out-of-band CLTAs derived by equal vertical beam width method is larger than the test object vertical radiating length. Option 2: Huawei: NOTE 2: Both definitions demonstrate compliance to the requirement, either vertical radiating dimension or vertical beam width definition may be used depending on the availability of CLTA Option 3: Nokia: NOTE 2: Beam width definition may be used in combination with vertical radiating dimension definition and vice versa to determine suitable CLTA. Recommendations for 2nd round: Continue to work on wording of the note. Hopefully we can do this by working on CR, resort to another WF only if we cannot get resolution. Possible approach is to state the height match definition is only used when the test chamber dimensions limit the use of the existing definition. Nokia: First priority is existing one, if not feasible then go with the alternative method? Huawei: Yes Nokia: We are fine with the approach and continue to work on the texts. CATT: General is OK with this approach, need to work together in 2nd for the exact wording. Agreements: Using the approach “State the height match definition is only used when the test chamber dimensions limit the use of the existing definition”. Further work in 2nd round for texting into specifications. |
| Sub topic#1-2 Co-location adjacent operating bands | Tentative agreements: The issue seems to be accepted but the proposed solution is not. Further work needed to find an acceptable solution Candidate options: Recommendations for 2nd round: Continue to discuss, there is some time allocated to NR conformance on Friday GTW meeting. WF is best approach to try to capture companies’ views on a solution. Huawei: We have a WF in last meeting to describe this issue which require companies to find possible solutions to resolve this issue. With AAS, we could do more useful for this scenario. We think we need to find a proper solution to address this issue. For non-AAS, the solution is adding a note using site engineering. For AAS, we can consider similar solution. Nokia: We have a question for clarification. We would like to resolve the issue without the modification of existing requirements. E///: We should maintain the requirements as it’s, we should avoid to change the requirements; also site engineering is deployment issue, usually not specified into spec, we prefer to keep the spec as it is. NEC: This is not related to positioning accuracy requirements. If we can take the emission requirements at it is, then we are open to discuss the candidate solution. The changes into colocation table is not a proper place to address this issue. DoCoMo: We have similar view with E///, we can’t accept the changes to test requirements. Huawei: I understand the current proposed solution from our side is not acceptable for companies. For adjacent system, the requirements are not feasible. We try to offer solutions to make requirements for adjacent system with co-location. There is benefit to keep the note into test specification to make it testable. Further work on 2nd round on possible solutions. |

-------------------------End -------------------------------------

R4-2101568	Further discussion on PN23 sequence generation for NR test models

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we further continue discussion on PN23 sequence generator for NR test models that was initiated during RAN4#97 meeting.

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was noted.

R4-2101569	CR to TS 38.141-1 clarification on PN23 sequence generation

Type: CR		For: Agreement
					38.141-1 v15.7.0	  CR-0192  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was revised to R4-2103789.

R4-2103789	CR to TS 38.141-1 clarification on PN23 sequence generation

Type: CR		For: Agreement
					38.141-1 v15.7.0	  CR-0192  rev 1 Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2101569)

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2101570	CR to TS 38.141-1 clarification on PN23 sequence generation

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0193  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2101571	CR to TS 38.141-2 clarification on PN23 sequence generation

Type: CR		For: Agreement
					38.141-2 v15.8.0	  CR-0288  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was revised to R4-2103790.

R4-2103790	CR to TS 38.141-2 clarification on PN23 sequence generation

Type: CR		For: Agreement
					38.141-2 v15.8.0	  CR-0288  rev 1 Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2101571)

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2101572	CR to TS 38.141-2 clarification on PN23 sequence generation

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0289  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2101878	CR to TS 38.141-1 clarification on PN23 sequence generation

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0194  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2101879	CR to TS 38.141-2 clarification on PN23 sequence generation

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0295  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

#### 4.5.2	Conducted conformance testing (38.141-1) [NR_newRAT-Perf]

R4-2102000	CR to TS 38.141-1: Receiver requirement corrections

Type: CR		For: Agreement
					38.141-1 v15.7.0	  CR-0198  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was revised to R4-2103793.

R4-2103793	CR to TS 38.141-1: Receiver requirement corrections

Type: CR		For: Agreement
					38.141-1 v15.7.0	  CR-0198  rev 1 Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102000)

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2102001	CR to TS 38.141-1: Receiver requirement corrections

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0199  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2102002	CR to TS 38.141-1: Receiver requirement corrections

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0200  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

#### 4.5.3	Radiated conformance testing (38.141-2) [NR_newRAT-Perf]

R4-2100385	Further discussion on out-of-band CLTA definition

Type: other		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was noted.

R4-2100386	CR for TS 38.141-2: Correction on definition for the out-of-band CLTA(Rel-15)

Type: CR		For: Agreement
					38.141-2 v15.8.0	  CR-0263  Cat: F (Rel-15)

					Source: CATT

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was revised to R4-2103794.

R4-2103794	CR for TS 38.141-2: Correction on definition for the out-of-band CLTA(Rel-15)

Type: CR		For: Agreement
					38.141-2 v15.8.0	  CR-0263  rev 1 Cat: F (Rel-15)

					Source: CATT, Nokia, Nokia Shanghai Bell, Huawei

(Replaces R4-2100386)

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2100387	CR for TS 38.141-2: Correction on definition for the out-of-band CLTA(Rel-16)

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0264  Cat: A (Rel-16)

					Source: CATT, Nokia, Nokia Shanghai Bell, Huawei

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2100388	CR for TS 38.141-2: Correction on definition for the out-of-band CLTA(Rel-17)

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0265  Cat: A (Rel-17)

					Source: CATT, Nokia, Nokia Shanghai Bell, Huawei

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2101730	TS 38.141-2: Correction of additional spurious emission limits for bands 50, 51, 75, 76

Type: CR		For: Agreement
					38.141-2 v15.8.0	  CR-0290  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

Correction of the unwanted emission limit as it is not aligned with core specifications

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was postponed.

R4-2103795	TS 38.141-2: Correction of additional spurious emission limits for bands 50, 51, 75, 76

Type: CR		For: Agreement
					38.141-2 v15.8.0	  CR-0290  rev 1 Cat: F (Rel-15)

					Source: Ericsson

(Replaces R4-2101730)

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was withdrawn.

R4-2101731	TS 38.141-2: Correction of additional spurious emission limits for bands 50, 51, 75, 76

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0291  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

Correction of the unwanted emission limit as it is not aligned with core specifications

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was withdrawn.

R4-2101732	TS 38.141-2: Correction of additional spurious emission limits for bands 50, 51, 75, 76

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0292  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Correction of the unwanted emission limit as it is not aligned with core specifications

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was withdrawn.

R4-2102003	CR to TS 38.141-2: Receiver requirement corrections

Type: CR		For: Agreement
					38.141-2 v15.8.0	  CR-0299  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2102004	CR to TS 38.141-2: Receiver requirement corrections

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0300  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2102005	CR to TS 38.141-2: Receiver requirement corrections

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0301  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2102429	Further discussion on CLTA maximum height

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Further discussion on how to define a practical height restriction for the CLTA

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was noted.

R4-2102430	Further discussion on co-location for adjacent bands

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Further discussion on the co-location scenarios for adjacent band systems based on the issues raised I the WF

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was noted.

R4-2102434	CR to TS 38.141-2 - Update CLTA definition, Rel-15

Type: CR		For: Agreement
					38.141-2 v15.8.0	  CR-0306  Cat: F (Rel-15)

					Source: Huawei

Abstract:

CR to update the CLTA definition in the NR conformance specification

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was not pursued.

R4-2102435	CR to TS 38.141-2 - Update CLTA definition, Rel-16

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0307  Cat: A (Rel-16)

					Source: Huawei

Abstract:

CR to update the CLTA definition in the NR conformance specification

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was withdrawn.

R4-2102436	CR to TS 38.141-2 - Update CLTA definition, Rel-17

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0308  Cat: A (Rel-17)

					Source: Huawei

Abstract:

CR to update the CLTA definition in the NR conformance specification

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was withdrawn.

R4-2102897	On Two orthogonal cuts with pattern multiplication procedures and CLTA maximum height

Type: other		For: Discussion
					38.141-2 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution further discusses the open issues related to the two orthogonal cuts with pattern multiplication procedure and CLTA maximum height.

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was noted.

R4-2102898	CR to TS 38.141-2: Updating the orthogonal cut procedure

Type: CR		For: Agreement
					38.141-2 v15.8.0	  CR-0310  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Numerical expressions for the two orthogonal cut procedure for TRP computation are included.

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was not pursued.

R4-2102899	CR to TS 38.141-2: Updating the orthogonal cut procedure

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0311  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Numerical expressions for the two orthogonal cut procedure for TRP computation are included.

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was withdrawn.

#### 4.5.4	eAAS specifications maintenance [NR_newRAT-Core/Perf]

R4-2100389	CR for TS 37.145-2: Correction on definition for the out-of-band CLTA(Rel-15)

Type: CR		For: Agreement
					37.145-2 v15.9.0	  CR-0271  Cat: F (Rel-15)

					Source: CATT

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was not pursued.

R4-2100390	CR for TS 37.145-2: Correction on definition for the out-of-band CLTA(Rel-16)

Type: CR		For: Agreement
					37.145-2 v16.6.0	  CR-0272  Cat: A (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was withdrawn.

R4-2100391	CR for TS 37.145-2: Correction on definition for the out-of-band CLTA(Rel-17)

Type: CR		For: Agreement
					37.145-2 v17.0.0	  CR-0273  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was withdrawn.

R4-2102423	CR to TS 37.145-1, Corrections to conformance requirements, Rel-15

Type: CR		For: Agreement
					37.145-1 v15.8.0	  CR-0243  Cat: F (Rel-15)

					Source: Huawei

Abstract:

Corrections to conformance specification based on errors identified while drafting the European harmonized standard

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was revised to R4-2103791.

R4-2103791	CR to TS 37.145-1, Corrections to conformance requirements, Rel-15

Type: CR		For: Agreement
					37.145-1 v15.8.0	  CR-0243  rev 1 Cat: F (Rel-15)

					Source: Huawei

(Replaces R4-2102423)

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was withdrawn.

R4-2102424	CR to TS 37.145-1, Corrections to conformance requirements, Rel-16

Type: CR		For: Agreement
					37.145-1 v16.5.0	  CR-0244  Cat: A (Rel-16)

					Source: Huawei

Abstract:

Corrections to conformance specification based on errors identified while drafting the European harmonized standard

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was withdrawn.

R4-2102425	CR to TS 37.145-1, Corrections to conformance requirements, Rel-17

Type: CR		For: Agreement
					37.145-1 v17.0.0	  CR-0245  Cat: A (Rel-17)

					Source: Huawei

Abstract:

Corrections to conformance specification based on errors identified while drafting the European harmonized standard

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was withdrawn.

R4-2102426	CR to TS 37.145-2: Corrections to conformance requirements, Rel-15

Type: CR		For: Agreement
					37.145-2 v15.9.0	  CR-0280  Cat: F (Rel-15)

					Source: Huawei

Abstract:

Corrections to conformance specification based on errors identified while drafting the European harmonized standard

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was revised to R4-2103792.

R4-2103792	CR to TS 37.145-2: Corrections to conformance requirements, Rel-15

Type: CR		For: Agreement
					37.145-2 v15.9.0	  CR-0280  rev 1 Cat: F (Rel-15)

					Source: Huawei

(Replaces R4-2102426)

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2102427	CR to TS 37.145-2: Corrections to conformance requirements, Rel-16

Type: CR		For: Agreement
					37.145-2 v16.6.0	  CR-0281  Cat: A (Rel-16)

					Source: Huawei

Abstract:

Corrections to conformance specification based on errors identified while drafting the European harmonized standard

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2102428	CR to TS 37.145-2: Corrections to conformance requirements, Rel-17

Type: CR		For: Agreement
					37.145-2 v17.0.0	  CR-0282  Cat: A (Rel-17)

					Source: Huawei

Abstract:

Corrections to conformance specification based on errors identified while drafting the European harmonized standard

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2102431	CR to TS 37.145-2 - Update CLTA definition, Rel-15

Type: CR		For: Agreement
					37.145-2 v15.9.0	  CR-0283  Cat: F (Rel-15)

					Source: Huawei

Abstract:

CR to update the CLTA definition in the AAS conformance specification

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was revised to R4-2103788.

R4-2103788	CR to TS 37.145-2 - Update CLTA definition, Rel-15

Type: CR		For: Agreement
					37.145-2 v15.9.0	  CR-0283  rev 1 Cat: F (Rel-15)

					Source: Huawei, CATT, Nokia

(Replaces R4-2102431)

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2102432	CR to TS 37.145-2 - Update CLTA definition, Rel-16

Type: CR		For: Agreement
					37.145-2 v16.6.0	  CR-0284  Cat: A (Rel-16)

					Source: Huawei, CATT, Nokia

Abstract:

CR to update the CLTA definition in the AAS conformance specification

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2102433	CR to TS 37.145-2 - Update CLTA definition, Rel-17

Type: CR		For: Agreement
					37.145-2 v17.0.0	  CR-0285  Cat: A (Rel-17)

					Source: Huawei, CATT, Nokia

Abstract:

CR to update the CLTA definition in the AAS conformance specification

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was agreed.

R4-2102895	CR to TS 37.145-2: Updating the orthogonal cut procedure

Type: CR		For: Agreement
					37.145-2 v15.9.0	  CR-0291  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Numerical expressions for the two orthogonal cut procedure for TRP computation are included.

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was not pursued.

R4-2102896	CR to TS 37.145-2: Updating the orthogonal cut procedure

Type: CR		For: Agreement
					37.145-2 v16.6.0	  CR-0292  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Numerical expressions for the two orthogonal cut procedure for TRP computation are included.

Discussion:

See email discussion summary for [98e][302] NR_Conformance_Maintenance in R4-2103924.

Decision: 		The document was withdrawn.

### 4.6	BS EMC requirements Maintenance [NR_newRAT-Core]

#### 4.6.1	Core requirements [NR_newRAT-Core]

#### 4.6.2	Performance requirements [NR_newRAT-Perf]

R4-2100354	CR to TS 38.113 on Performance criteria for transient phenomena, Release 15

Type: CR		For: Agreement
					38.113 v15.12.0	  CR-0031  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

CR updating performance criteria for transient phenomena in TS 38.113 Rel 15

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was not pursued.

R4-2100355	CR to TS 38.113 on Performance criteria for transient phenomena, Release 16

Type: CR		For: Agreement
					38.113 v16.2.0	  CR-0032  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

CR mirroring update in performance criteria for transient phenomena in 38.113 Rel 16

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was withdrawn.

R4-2102127	CR to TS 38.113: Radiated emission test method

Type: CR		For: Agreement
					38.113 v15.12.0	  CR-0033  Cat: F (Rel-15)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was withdrawn.

R4-2102128	CR to TS 38.113: Radiated emission test method

Type: CR		For: Agreement
					38.113 v16.2.0	  CR-0034  Cat: A (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was withdrawn.

R4-2102178	CR to TS 38.113: Radiated emission test method

Type: CR		For: Agreement
					38.113 v15.12.0	  CR-0035  Cat: F (Rel-15)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was revised to R4-2103772.

R4-2103772	CR to TS 38.113: Radiated emission test method

Type: CR		For: Agreement
					38.113 v15.12.0	  CR-0035  rev 1 Cat: F (Rel-15)

					Source: ZTE Corporation

(Replaces R4-2102178)

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was agreed.

R4-2102179	CR to TS 38.113: Radiated emission test method

Type: CR		For: Agreement
					38.113 v16.2.0	  CR-0036  Cat: A (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was agreed.

### 4.7	RRM core requirements maintenance (38.133/36.133) [NR_newRAT-Core]

R4-2103440	Email discussion summary: [98e][201] NR_NewRAT_RRM_Core

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][201] NR_NewRAT_RRM_Core. The discussion thread was moderated by Xizeng Dai (Huawei).

Decision: 		The document was revised to R4-2103683.

R4-2103683	Email discussion summary: [98e][201] NR_NewRAT_RRM_Core

Type: other		For: discussion
					Source: Moderator (Huawei)

(Replaces R4-2103440)

Discussion:

The contribution summarised email discussion thread [98e][201] NR_NewRAT_RRM_Core. The discussion thread was moderated by Xizeng Dai (Huawei).

Decision: 		The document was noted.

GTW session (January 29, 2021)

Issue 1-1-2: Whether/how to count CSSF outside MG and CSSF within MG

Discussion

Nokia: For Alt1/Alt2 we prefer Alt2. Not sure if merging in Alt1 works.

Apple: Support Alt1. Alt2 wording is unclear (capability part). To Nokia – MO configuration for the case of serving or non-serving cell cases.

Huawei: Alt1 and Alt2 are quite similar. Capability part in Alt2 is unclear. The merging conditions need to be discussed.

Nokia: we are quite aligned.

Agreements

- Option 2a: Remove the inter-RAT MOs counted in CSSF outside MG from CSSF within MG, and further discuss allowing existing implementations not to meet the updated requirements.

- CSSF calculation

  - CSSF outside MG

    - to consider merging of intra-frequency MO configured by NR SN and inter-RAT MO configured by LTE MN on the same serving frequency that are measured without MG, based on [MO merging conditions in clause 9.1.3.2 of 38.133].

  - CSSF within MG

    - to consider merging of two MOs configured by LTE MN and NR SN on the same frequency that are measured within MG, based on [MO merging conditions in clause 9.1.3.2 of 38.133].

  - Note: companies can further check the exact MO merging conditions

- Allow requirements relaxation for Rel-15 UEs to avoid compatibility issue

  - Option 1: “longer delays for cell identification and measurement periods derived based on CSSFwithin_gap,i can be expected, if the UE is configured with inter-RAT MO on NR serving CC by E-UTRAN PCell in EN-DC mode”.

Issue 1-2: MO merging related to SSB-ToMeasurement indications

Discussion

MTK: This issue was already agreed in the last meeting. We just want to align the spec and copy it to another section. Union is a mathematical term here. To  E///, this is applicable to DC and CA.

Nokia: suggest to clarify the union term (e.g. add a note)

E///: can we use the Component carrier or Serving cell terms in the CR rather than MO?

Session chair: Technical principle is agreeable. Work to address editorial comments.

Issue 1-4: deactivated SCell measurement for intra-frequency measurement with MG

Discussion

Huawei: For the scenario is without MG. Kp needs to be introduced. The idea is that the measurement on deactivated Scell will be longer than for the activated Scell.

QC: Agree with Huawei.

Apple: Agree with QC and Huawei

Nokia: Agree with principle. Not sure if Kp is the correct scaling factor.

Agreements

- Not to introduce the intra-frequency de-activated SCell measurement requirement with MG

- Introduce a scaling factor for intra-frequency deactivated SCell measurement requirements without gap when SMTC is partially overlapping with MG

  - Option 1: Scaling factor = Kp

  - Other options not precluded

Issue 2-1-1: SSB outside the first active BWP

Discussion

Huawei: To QC and NEC the intention is to clarify that the requirements will not apply for the case SSB is outside the first active BWP.

QC: SSB outside BWP is optional feature. UEs with such support should be able to satisfy the requirements

NEC: Is UE RF BW should be larger than the first active BWP?

QC: This capability does not explicitly imply this? UE can tune RF if needed. The main is that there should be no interruptions.

Huawei: UE may not necessarily use larger BW

Huawei: to QC – can UE with such feature do the measurements without MG? If we follow this logic then UE shall be able to support gapless measurements.

QC: this is not our intention.

E///: support QC.

Huawei: this is optional feature

HW/MTK/Apple: For UEs supporting “SSB outside BWP” some interruption may be needed

E///: UE can keep wider RF BW and interruptions are not needed

HW: this is relevant to the specific UE implementation. This is more like an optimization.

QC: need to double check if tentative agreement is acceptable

Tentative agreements

- Rel-15 SCell activation requirements, except those for SSB-less SCell, apply provided that the SSB of the to-be-activated SCell is within the first active DL BWP of the SCell.

- Further discussion can take place for Rel-16+ whether any differentiation of requirements for UEs supporting “SSB outside BWP” is needed.

Issue 2-2-1: Condition and requirements for SSB-less SCell activation for FR1

Discussion

Power imbalance

CMCC: why do we need power restriction? We don’t have it for FR2

Nokia: we think 6dB is ok. We encourage other companies to check with RF colleagues.

MTK: in FR2 we have same TX beam but in FR1 we don’t have such wording and we changed it to power and time difference

E///: 6dB is ok

CMCC: why 6dB is selected

MTK: 6dB is typical value for AGC control

Agreements

- Reception power difference with the contiguous active serving cell is smaller than or equal to 6dB

- RTD is smaller than or equal to 260ns

1st round email discussion conclusions

Issue 2-1-3: Determination of SSB offset

Agreement

- Add the following condition for SCell activation requirements for FR2

  - SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell

Issue 2-2-2: Condition and requirements for SSB-less SCell activation for FR2

Agreement

- If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE supporting scellWithoutSSB is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms, provided

Issue 4-1: Applicability of RRC based BWP switch delay requirement in Rel-15

Agreement

- Wait for RAN2 reply LS before making further decisions on applicability of RRC based BWP switch delay requirement in Rel-15

================================================================================

R4-2100172	Further discussion on CSSF for R15 EN-DC

Type: discussion		For: Discussion
					38.133 v..
					Source: Apple

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was noted.

R4-2100173	CR on CSSF for EN-DC R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1438  Cat: F (Rel-15)

					Source: Apple

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was revised to R4-2103729.

R4-2103729	CR on CSSF for EN-DC R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1438  rev 1 Cat: F (Rel-15)

					Source: Apple

(Replaces R4-2100173)

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was postponed.

R4-2100174	On SSB-less SCell activation

Type: discussion		For: Discussion
					38.133 v..
					Source: Apple

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was noted.

R4-2100175	CR on FR2 SCell activation requirement R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1439  Cat: F (Rel-15)

					Source: Apple

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was postponed.

R4-2100176	CR on FR2 SCell activation requirement R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1440  Cat: A (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was withdrawn.

R4-2100177	CR on FR2 SCell activation requirement R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1441  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was withdrawn.

R4-2100178	CR on smtc1 and smtc2 differentiation in intra-frequency measurement with MG R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1442  Cat: F (Rel-15)

					Source: Apple

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was postponed.

R4-2100179	CR on smtc1 and smtc2 differentiation in intra-frequency measurement with MG R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1443  Cat: A (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was withdrawn.

R4-2100180	CR on smtc1 and smtc2 differentiation in intra-frequency measurement with MG R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1444  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was withdrawn.

R4-2100233	Interruption requirements due to measurement on SCC in NR-DC (R15)

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1463  Cat: F (Rel-15)

					Source: Apple

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was postponed.

R4-2100851	Discussion on deactivated SCell measurement for intra-frequency measurement with measurement gap

Type: discussion		For: Approval
					Source: CMCC

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was noted.

R4-2100852	CR on deactivated SCell measurement for intra-frequency measurement with measurement gap

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1528  Cat: F (Rel-15)

					Source: CMCC

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was merged.

R4-2100853	CR on deactivated SCell measurement for intra-frequency measurement with measurement gap

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1529  Cat: A (Rel-16)

					Source: CMCC

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was withdrawn.

R4-2101006	CR on Scell activation delay maintenance (R15)

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1537  Cat: F (Rel-15)

					Source: Apple

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was revised to R4-2104043.

R4-2104043	CR on Scell activation delay maintenance (R15)

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1537  rev 1 Cat: F (Rel-15)

					Source: Apple

(Replaces R4-2101006)

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was revised to R4-2104084.

R4-2104084	CR on Scell activation delay maintenance (R15)

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1537  rev 2 Cat: F (Rel-15)

					Source: Apple

(Replaces R4-2104043)

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2101007	CR on Scell activation delay maintenance (R16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1538  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2101008	CR on Scell activation delay maintenance (R17)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1539  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2101050	Remaining issues on RRM in R15

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was noted.

R4-2101051	CR on R15 remaining issues

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1548  Cat: F (Rel-15)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was revised to R4-2103481.

R4-2103481	CR on R15 remaining issues

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1548  rev 1 Cat: F (Rel-15)

					Source: MediaTek inc.

(Replaces R4-2101051)

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2101052	CR on R15 remaining issues

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1549  Cat: A (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2101053	CR on R15 remaining issues

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1550  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2101071	CR on SSB less SCell activation for FR1 for Rel-15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1553  Cat: F (Rel-15)

					Source: NEC

Abstract:

SSB less SCell activation for FR1 is supported in Rel-15 from RAN1/RAN2 perspective. However the requirements for the same are missing in TS 38.133.

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was merged.

R4-2101072	CR on SSB less SCell activation for FR1 for Rel-16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1554  Cat: F (Rel-16)

					Source: NEC

Abstract:

SSB less SCell activation for FR1 is supported in Rel-16 from RAN1/RAN2 perspective. However the requirements for the same are missing in TS 38.133.

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was merged.

R4-2101407	Discussion on RRC based BWP switching

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was postponed.

R4-2101464	CR on the filter for beam failure indications in 38.133

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1605  Cat: F (Rel-15)

					Source: vivo

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was revised to R4-2103484.

R4-2103484	CR on the filter for beam failure indications in 38.133

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1605  rev 1 Cat: F (Rel-15)

					Source: vivo

(Replaces R4-2101464)

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2101465	CR on the filter for beam failure indications in 38.133

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1606  Cat: A (Rel-16)

					Source: vivo

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2101466	CR on the filter for beam failure indications in 38.133

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1607  Cat: A (Rel-17)

					Source: vivo

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2102536	On correction to inter-RAT CSSF

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On correction to inter-RAT CSSF

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was noted.

R4-2102731	CR to remove intra-frequency ECID requirements for NE-DC 36133 R15

Type: CR		For: Agreement
					36.133 v15.12.0	  CR-7058  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was revised to R4-2104044.

R4-2104044	CR to remove intra-frequency ECID requirements for NE-DC 36133 R15

Type: CR		For: Agreement
					36.133 v15.12.0	  CR-7058  rev 1 Cat: F (Rel-15)

					Source: Huawei, HiSilicon

(Replaces R4-2102731)

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2102732	CR to remove intra-frequency ECID requirements for NE-DC 36133 R16

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7059  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2102733	CR to remove intra-frequency ECID requirements for NE-DC 36133 R17

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7060  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2102734	CR to idle more requirements in 36133 R15

Type: CR		For: Agreement
					36.133 v15.12.0	  CR-7061  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2102735	CR to idle more requirements in 36133 R16

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7062  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2102736	CR to idle more requirements in 36133 R17

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7063  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2102737	Discussion on CSSF for inter-RAT measurement, SCell activation delay and cell identification requirements on deactivated SCell in Rel-15

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was noted.

R4-2102738	CR on SCell activation delay, cell idenfication requirements on deactivated SCell and inter-RAT ECID requirements for NE-DC R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1752  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was revised to R4-2103482.

R4-2103482	CR on SCell activation delay, cell idenfication requirements on deactivated SCell and inter-RAT ECID requirements for NE-DC R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1752  rev 1 Cat: F (Rel-15)

					Source: Huawei, HiSilicon

(Replaces R4-2102738)

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was revised to R4-2104090.

R4-2104090	CR on SCell activation delay, cell idenfication requirements on deactivated SCell and inter-RAT ECID requirements for NE-DC R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1752  rev 2 Cat: F (Rel-15)

					Source: Huawei, HiSilicon

(Replaces R4-2103482)

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2102739	CR on SCell activation delay, cell idenfication requirements on deactivated SCell and inter-RAT ECID requirements for NE-DC R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1753  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2102740	CR on SCell activation delay, cell idenfication requirements on deactivated SCell and inter-RAT ECID requirements for NE-DC R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1754  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was agreed.

R4-2102827	Correction to inter-RAT CSSF

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1775  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

Correction to inter-RAT CSSF

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was merged.

R4-2102537	Correction to inter-RAT CSSF

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1728  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

Correction to inter-RAT CSSF

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was withdrawn.

R4-2102538	Correction to inter-RAT CSSF

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1729  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Correction to inter-RAT CSSF

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was withdrawn.

R4-2102872	Cat-F CR to SSB-less SCell activation delay requirement for deactivated FR1 SCell in Rel-15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1776  Cat: F (Rel-15)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was postponed.

R4-2103483	Cat-F CR to SSB-less SCell activation delay requirement for deactivated FR1 SCell in Rel-15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1776  rev 1 Cat: F (Rel-15)

					Source: Qualcomm Incorporated

(Replaces R4-2102872)

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was withdrawn.

R4-2102873	Cat-A CR to SSB-less SCell activation delay requirement for deactivated FR1 SCell in Rel-16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1777  Cat: A (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was withdrawn.

R4-2102874	Cat-A CR to SSB-less SCell activation delay requirement for deactivated FR1 SCell in Rel-17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1778  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][201] NR_NewRAT_RRM_Core in R4-2103683.

Decision: 		The document was withdrawn.

### 4.8	RRM perf. requirements maintenance (38.133/36.133) [NR_newRAT-Perf]

R4-2103441	Email discussion summary: [98e][202] NR_NewRAT_RRM_Perf

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][202] NR_NewRAT_RRM_Perf. The discussion thread was moderated by Muhammad Kazmi (Ericsson).

Decision: 		The document was revised to R4-2103684.

R4-2103684	Email discussion summary: [98e][202] NR_NewRAT_RRM_Perf

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2103441)

Discussion:

The contribution summarised email discussion thread [98e][202] NR_NewRAT_RRM_Perf. The discussion thread was moderated by Muhammad Kazmi (Ericsson).

Decision: 		The document was noted.

R4-2100058	[CR] RRM test case maintenance R15 Cat F

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1416  Cat: F (Rel-15)

					Source: ZTE Corporation

Abstract:

R15 Cat F CR to fix some errors existing in the test cases.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2103485.

R4-2103485	[CR] RRM test case maintenance R15 Cat F

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1416  rev 1 Cat: F (Rel-15)

					Source: ZTE Corporation

(Replaces R4-2100058)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100059	[CR] RRM test case maintenance R16 Cat A

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1417  Cat: A (Rel-16)

					Source: ZTE Corporation

Abstract:

Cat A CR for R16

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100060	[CR] RRM test case maintenance R17 Cat A

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1418  Cat: A (Rel-17)

					Source: ZTE Corporation

Abstract:

Cat A CR for R17

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100067	FR2 PDSCH Reference channel and OCNG for RRM Test cases

Type: discussion		For: Endorsement
					Source: ANRITSU LTD

Abstract:

The downlink dB range for RRM Test cases is very restricted where signals arrive from the UE Spherical coverage direction, and there is not enough range to implement FR2 RRM Spherical Coverage test cases as currently written. The range can be increased by

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was noted.

R4-2100068	Update FR2 Reference channels and OCNG for FR2 RRM Test cases

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1422  Cat: F (Rel-15)

					Source: ANRITSU LTD

Abstract:

Define new PDSCH Reference Measurement Channels occupying the same PRBs as the CORESET, 24RB and 48RB. Define new RMSI CORESET Reference Channel and Control Channel RMC with 48RBs allowing use with 240kHz SSB SCS Test cases.

Update OCNG pattern OP.3 to co

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2103486.

R4-2103486	Update FR2 Reference channels and OCNG for FR2 RRM Test cases

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1422  rev 1 Cat: F (Rel-15)

					Source: ANRITSU LTD

(Replaces R4-2100068)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100069	Update FR2 Reference channels and OCNG for FR2 RRM Test cases

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1423  Cat: A (Rel-16)

					Source: ANRITSU LTD

Abstract:

Define new PDSCH Reference Measurement Channels occupying the same PRBs as the CORESET, 24RB and 48RB. Define new RMSI CORESET Reference Channel and Control Channel RMC with 48RBs allowing use with 240kHz SSB SCS Test cases.

Update OCNG pattern OP.3 to co

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100070	Update FR2 Reference channels and OCNG for FR2 RRM Test cases

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1424  Cat: A (Rel-17)

					Source: ANRITSU LTD

Abstract:

Define new PDSCH Reference Measurement Channels occupying the same PRBs as the CORESET, 24RB and 48RB. Define new RMSI CORESET Reference Channel and Control Channel RMC with 48RBs allowing use with 240kHz SSB SCS Test cases.

Update OCNG pattern OP.3 to co

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100071	FR2 Reference channels for RRM Test cases with 240kHz SSB SCS

Type: discussion		For: Endorsement
					Source: ANRITSU LTD

Abstract:

Some FR2 RRM Test cases in Annex A of TS 38.133 specify a configuration with 240kHz SSB SCS.

RRM test case configurations using 240kHz SSB SCS need different Reference channels which are not currently specified in TS 38.133. This Tdoc identifies new Refer

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was noted.

R4-2100072	CR to FR1 SA SS-SINR measurement TCs

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1425  Cat: F (Rel-15)

					Source: ANRITSU LTD

Abstract:

Update Test Parameters table format to show that TRS config is only for Cell 1

Update Table A.6.7.3.2.2-2 to include gap configuration, Gap pattern ID = 0

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100073	CR to FR1 SA SS-SINR measurement TCs

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1426  Cat: F (Rel-16)

					Source: ANRITSU LTD

Abstract:

Update Test Parameters table format to show that TRS config is only for Cell 1

Align SMTC configuration in Table A.6.7.3.1.2-2 with Rel-15 spec.

Update Table A.6.7.3.1.2-2 to include Time offset with Cell 1.

Update Table A.6.7.3.2.2-2 to include gap confi

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100074	CR to FR1 SA SS-SINR measurement TCs

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1427  Cat: A (Rel-17)

					Source: ANRITSU LTD

Abstract:

Update Test Parameters table format to show that TRS config is only for Cell 1

Align SMTC configuration in Table A.6.7.3.1.2-2 with Rel-15 spec.

Update Table A.6.7.3.1.2-2 to include Time offset with Cell 1.

Update Table A.6.7.3.2.2-2 to include gap confi

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100075	CR on E-UTRA carrier for EN-DC event triggered reporting tests

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1428  Cat: F (Rel-15)

					Source: ANRITSU LTD

Abstract:

Remove “TDD” comment to allow LTE FDD configurations to be tested.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100076	CR on E-UTRA carrier for EN-DC event triggered reporting tests

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1429  Cat: A (Rel-16)

					Source: ANRITSU LTD

Abstract:

Remove “TDD” comment to allow LTE FDD configurations to be tested.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100077	CR on E-UTRA carrier for EN-DC event triggered reporting tests

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1430  Cat: A (Rel-17)

					Source: ANRITSU LTD

Abstract:

Remove “TDD” comment to allow LTE FDD configurations to be tested.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100078	Add missing FR2 Test case setups and Beam assumptions

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1431  Cat: F (Rel-15)

					Source: ANRITSU LTD

Abstract:

Specify setup 1 for FR2 Handover and RRC Connection Release/Redirection and add missing Control Channel RMC.

Reduce allocated RBs to 24 for Inter-RAT event triggered reporting test case A.8.4.2.5 and apply Es only.

Specify setup 1 for Inter-RAT event trig

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100079	Add missing FR2 Test case setups and Beam assumptions

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1432  Cat: A (Rel-16)

					Source: ANRITSU LTD

Abstract:

Specify setup 1 for FR2 Handover and RRC Connection Release/Redirection and add missing Control Channel RMC.

Reduce allocated RBs to 24 for Inter-RAT event triggered reporting test case A.8.4.2.5 and apply Es only.

Specify setup 1 for Inter-RAT event trig

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100080	Add missing FR2 Test case setups and Beam assumptions

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1433  Cat: A (Rel-17)

					Source: ANRITSU LTD

Abstract:

Specify setup 1 for FR2 Handover and RRC Connection Release/Redirection and add missing Control Channel RMC.

Reduce allocated RBs to 24 for Inter-RAT event triggered reporting test case A.8.4.2.5 and apply Es only.

Specify setup 1 for Inter-RAT event trig

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100479	Correction to cell reselection test case

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1494  Cat: F (Rel-15)

					Source: CATT

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100480	Correction to cell reselection test case

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1495  Cat: A (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100481	Correction to cell reselection test case

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1496  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100601	Update of DRX configuration in FR1 Event-triggered Test cases

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1503  Cat: F (Rel-15)

					Source: ANRITSU LTD

Abstract:

Update DRX configuration to avoid problem with time alignment timer expiry.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100602	Update of DRX configuration in FR1 Event-triggered Test cases

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1504  Cat: F (Rel-16)

					Source: ANRITSU LTD

Abstract:

Update DRX configuration to avoid problem with time alignment timer expiry.

This CR is Cat F because Rel-16 includes test cases A.4.6.1.7 and A.6.6.1.7 in addition to those in Rel-15.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100603	Update of DRX configuration in FR1 Event-triggered Test cases

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1505  Cat: F (Rel-17)

					Source: ANRITSU LTD

Abstract:

Update DRX configuration to avoid problem with time alignment timer expiry.

This CR is Cat F because Rel-17 includes test cases A.4.6.1.7 and A.6.6.1.7 in addition to those in Rel-15, and the whole test case A.6.6.1.7 was missing.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100760	Correction on PRACH configuration for FR2 Non-Contention based Random Access in R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1512  Cat: F (Rel-15)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100761	Correction on PRACH configuration for FR2 Non-Contention based Random Access in R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1513  Cat: A (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100762	Correction on PRACH configuration for FR2 Non-Contention based Random Access in R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1514  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100763	Correction on PRACH configuration for Beam Failure Detection and Link Recovery Test in R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1515  Cat: F (Rel-15)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2104049.

R4-2104049	Correction on PRACH configuration for Beam Failure Detection and Link Recovery Test in R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1515  rev 1 Cat: F (Rel-15)

					Source: MediaTek inc.

(Replaces R4-2100763)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100764	Correction on PRACH configuration for Beam Failure Detection and Link Recovery Test in R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1516  Cat: A (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100765	Correction on PRACH configuration for Beam Failure Detection and Link Recovery Test in R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1517  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100766	Correction on PRACH RMC for FR1 CSI-RS based Non-Contention based Random Access for BFR in R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1518  Cat: F (Rel-15)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100767	Correction on PRACH RMC for FR1 CSI-RS based Non-Contention based Random Access for BFR in R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1519  Cat: A (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2100768	Correction on PRACH RMC for FR1 CSI-RS based Non-Contention based Random Access for BFR in R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1520  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101047	CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1545  Cat: F (Rel-15)

					Source: CMCC

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101048	CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1546  Cat: A (Rel-16)

					Source: CMCC

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101049	CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1547  Cat: A (Rel-17)

					Source: CMCC

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101161	Correction on the power of the first preamble for random access in EN-DC and SA in R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1563  Cat: F (Rel-15)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2103487.

R4-2103487	Correction on the power of the first preamble for random access in EN-DC and SA in R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1563  rev 1 Cat: F (Rel-15)

					Source: MediaTek inc.

(Replaces R4-2101161)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101162	Correction on the power of the first preamble for random access in EN-DC and SA in R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1564  Cat: A (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101163	Correction on the power of the first preamble for random access in EN-DC and SA in R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1565  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101164	Correction on the time for Scell activation and CSI-report in R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1566  Cat: F (Rel-15)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2103488.

R4-2103488	Correction on the time for Scell activation and CSI-report in R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1566  rev 1 Cat: F (Rel-15)

					Source: MediaTek inc.

(Replaces R4-2101164)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2104082.

R4-2104082	Correction on the time for Scell activation and CSI-report in R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1566  rev 2 Cat: F (Rel-15)

					Source: MediaTek inc.

(Replaces R4-2103488)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101165	Correction on the time for Scell activation and CSI-report in R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1567  Cat: A (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101166	Correction on the time for Scell activation and CSI-report in R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1568  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101167	Correction on the Noc level in TS38.133 in R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1569  Cat: F (Rel-15)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2103489.

R4-2103489	Correction on the Noc level in TS38.133 in R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1569  rev 1 Cat: F (Rel-15)

					Source: MediaTek inc.

(Replaces R4-2101167)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101168	Correction on the Noc level in TS38.133 in R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1570  Cat: A (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101169	Correction on the Noc level in TS38.133 in R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1571  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101170	Correction on the SS-RSRP difference value for SS-RSRP measurement TC in R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1572  Cat: F (Rel-15)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was postponed.

R4-2103490	Correction on the SS-RSRP difference value for SS-RSRP measurement TC in R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1572  rev 1 Cat: F (Rel-15)

					Source: MediaTek inc.

(Replaces R4-2101170)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was withdrawn.

R4-2101171	Correction on the SS-RSRP difference value for SS-RSRP measurement TC in R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1573  Cat: A (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was withdrawn.

R4-2101172	Correction on the SS-RSRP difference value for SS-RSRP measurement TC in R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1574  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was withdrawn.

R4-2101612	Correction to Aperiodic CSI-RS configurations R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1614  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101613	Correction to Aperiodic CSI-RS configurations R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1615  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101614	Correction to Aperiodic CSI-RS configurations R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1616  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101615	Correction to radio link monitoring test cases R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1617  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101616	Correction to radio link monitoring test cases R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1618  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101617	Correction to radio link monitoring test cases R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1619  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101618	Correction to beam failure recovery test cases R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1620  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2103491.

R4-2103491	Correction to beam failure recovery test cases R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1620  rev 1 Cat: F (Rel-15)

					Source: Huawei, HiSilicon

(Replaces R4-2101618)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2104065.

R4-2104065	Correction to beam failure recovery test cases R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1620  rev 2 Cat: F (Rel-15)

					Source: Huawei, HiSilicon

(Replaces R4-2103491)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101619	Correction to beam failure recovery test cases R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1621  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101620	Correction to beam failure recovery test cases R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1622  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101621	Correction to L1-RSRP reporting delay test cases R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1623  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2103492.

R4-2103492	Correction to L1-RSRP reporting delay test cases R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1623  rev 1 Cat: F (Rel-15)

					Source: Huawei, HiSilicon

(Replaces R4-2101621)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101622	Correction to L1-RSRP reporting delay test cases R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1624  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101623	Correction to L1-RSRP reporting delay test cases R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1625  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101660	Discussion on antenna configurations for 4Rx capable UE

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was noted.

R4-2101661	CR on maintaining Antenna configurations in TS38.133 R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1634  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

Session chair: Cover sheet issue (What is the work item? It reads NR_newRAT-Perf on the cover page but the Tdoc is reserved for NR_RRM_enh-Perf.) Please inform MCC on correct WI code. The correct WI code shall be NR_newRAT-Perf.

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2103493.

R4-2103493	CR on maintaining Antenna configurations in TS38.133 R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1634  rev 1 Cat: F (Rel-15)

					Source: Huawei, HiSilicon

(Replaces R4-2101661)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2104056.

R4-2104056	CR on maintaining Antenna configurations in TS38.133 R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1634  rev 2 Cat: F (Rel-15)

					Source: Huawei, HiSilicon

(Replaces R4-2103493)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101662	CR on maintaining Antenna configurations in TS38.133 R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1635  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101663	CR on maintaining Antenna configurations in TS38.133 R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1636  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101664	CR on test requirements for measurement performance tests R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1637  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

Session chair: Cover sheet issue (What is the work item? It reads NR_newRAT-Perf on the cover page but the Tdoc is reserved for NR_RRM_enh-Perf.) Please inform MCC on correct WI code. The correct WI code shall be NR_newRAT-Perf.

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2103494.

R4-2103494	CR on test requirements for measurement performance tests R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1637  rev 1 Cat: F (Rel-15)

					Source: Huawei, HiSilicon

(Replaces R4-2101664)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101665	CR on test requirements for measurement performance tests R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1638  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101666	CR on test requirements for measurement performance tests R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1639  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101701	Correction on test cases of inter-frequency Measurements R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1653  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2103495.

R4-2103495	Correction on test cases of inter-frequency Measurements R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1653  rev 1 Cat: F (Rel-15)

					Source: Huawei, HiSilicon

(Replaces R4-2101701)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101702	Correction on test cases of inter-frequency Measurements R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1654  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101703	Correction on test cases of inter-frequency Measurements R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1655  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2101704	Correction on NR Pcell FR2 active TCI state switching R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1656  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was postponed.

R4-2101705	Correction on NR Pcell FR2 active TCI state switching R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1657  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was withdrawn.

R4-2101706	Correction on NR Pcell FR2 active TCI state switching R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1658  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was withdrawn.

R4-2102306	CR to TS 38.133: Redundant and incorrect TCI state in tests with TRS (Rel-15)

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1712  Cat: F (Rel-15)

					Source: Rohde & Schwarz

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2104054.

R4-2104054	CR to TS 38.133: Redundant and incorrect TCI state in tests with TRS (Rel-15)

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1712  rev 1 Cat: F (Rel-15)

					Source: Rohde & Schwarz

(Replaces R4-2102306)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102307	CR to TS 38.133: Redundant and incorrect TCI state in tests with TRS (Rel-16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1713  Cat: A (Rel-16)

					Source: Rohde & Schwarz

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102308	CR to TS 38.133: Redundant and incorrect TCI state in tests with TRS (Rel-17)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1714  Cat: A (Rel-17)

					Source: Rohde & Schwarz

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102309	CR to TS 38.133: Corrections to TC A.4.5.7.1 (Rel-15)

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1715  Cat: F (Rel-15)

					Source: Rohde & Schwarz

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2103496.

R4-2103496	CR to TS 38.133: Corrections to TC A.4.5.7.1 (Rel-15)

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1715  rev 1 Cat: F (Rel-15)

					Source: Rohde & Schwarz

(Replaces R4-2102309)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102310	CR to TS 38.133: Corrections to TC A.4.5.7.1 (Rel-16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1716  Cat: A (Rel-16)

					Source: Rohde & Schwarz

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102311	CR to TS 38.133: Corrections to TC A.4.5.7.1 (Rel-17)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1717  Cat: A (Rel-17)

					Source: Rohde & Schwarz

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was endorsed.

R4-2102726	CR on test cases for inter-RAT measurement r15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1749  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102727	CR on test cases for inter-RAT measurement r16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1750  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102728	CR on test cases for inter-RAT measurement r17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1751  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102741	CR on SCell activation TCs R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1755  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2103497.

R4-2103497	CR on SCell activation TCs R15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1755  rev 1 Cat: F (Rel-15)

					Source: Huawei, HiSilicon

(Replaces R4-2102741)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102742	CR on SCell activation TCs R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1756  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102743	CR on SCell activation TCs R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1757  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102869	CR on correcting SSB and RACH configuration in CSI-RS based beam failure detection and link recovery tests

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1404  rev 1 Cat: F (Rel-15)

					Source: Qualcomm CDMA Technologies

(Replaces R4-2017048)

Abstract:

R4-2017048 was endorsed but returned due to a formatting issue. Resubmission to 98e.

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102905	CR on correcting SSB and RACH configuration in CSI-RS based beam failure detection and link recovery tests

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1789  Cat: A (Rel-16)

					Source: Qualcomm CDMA Technologies

Abstract:

Mirror CR for CR 1404 in R4-2102869

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102908	CR on correcting SSB and RACH configuration in CSI-RS based beam failure detection and link recovery tests

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1790  Cat: A (Rel-17)

					Source: Qualcomm CDMA Technologies

Abstract:

Mirror CR of CR 1404 in R4-2102869

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102875	Cat-F CR to addition of TRS Configurations in Rel-15 Test Cases

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1779  Cat: F (Rel-15)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2103498.

R4-2103498	Cat-F CR to addition of TRS Configurations in Rel-15 Test Cases

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1779  rev 1 Cat: F (Rel-15)

					Source: Qualcomm Incorporated

(Replaces R4-2102875)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was revised to R4-2104086.

R4-2104086	Cat-F CR to addition of TRS Configurations in Rel-15 Test Cases

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1779  rev 2 Cat: F (Rel-15)

					Source: Qualcomm Incorporated

(Replaces R4-2103498)

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102876	Cat-A CR to addition of TRS Configurations in Rel-16 Test Cases

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1780  Cat: A (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102877	Cat-A CR to addition of TRS Configurations in Rel-17 Test Cases

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1781  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was agreed.

R4-2102878	Cat-F CR to DRX Configurations in Rel-15

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1782  Cat: F (Rel-15)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was not pursued.

R4-2102879	Cat-A CR to DRX Configurations in Rel-16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1783  Cat: A (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was withdrawn.

R4-2102880	Cat-A CR to DRX Configurations in Rel-17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1784  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][202] NR_NewRAT_RRM_Perf in R4-2103684.

Decision: 		The document was withdrawn.

### 4.9	Demodulation and CSI requirements maintenance (38.101-4/38.104)[NR_newRAT-Perf]

#### 4.9.1	UE demodulation requirements[NR_newRAT-Perf]

R4-2103753	Email discussion summary for [98e][314] Demod_R15_Maintenance

Type: other		For: discussion
					Source: Moderator (Intel)

Discussion:

The contribution summarised email discussion thread [98e][314] Demod_R15_Maintenance . The discussion thread was moderated by Belov Dmitry (Intel).

Decision: 		The document was revised to R4-2103925.

R4-2103925	Email discussion summary for [98e][314] Demod_R15_Maintenance

Type: other		For: discussion
					Source: Moderator (Intel)

(Replaces R4-2103753)

Discussion:

The contribution summarised email discussion thread [98e][314] Demod_R15_Maintenance . The discussion thread was moderated by Belov Dmitry (Intel).

Decision: 		The document was noted.

R4-2101441	Correction of CQI test parameters and FRC for UE demodulation test

Type: CR		For: Agreement
					38.101-4 v15.8.0	  CR-0157  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

This CR fix the errors in CQI test parameters and FRC in UE demodulation tests.

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was revised to R4-2103796.

R4-2103796	Correction of CQI test parameters and FRC for UE demodulation test

Type: CR		For: Agreement
					38.101-4 v15.8.0	  CR-0157  rev 1 Cat: F (Rel-15)

					Source: Ericsson

(Replaces R4-2101441)

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2101442	Correction of CQI test parameters and FRC for UE demodulation test

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0158  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

This CR fix the errors in CQI test parameters and FRC in UE demodulation tests.

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2102824	CR on corrections for LTE-NR Co-existence tests and OCNG pattern

Type: CR		For: Agreement
					38.101-4 v15.8.0	  CR-0167  Cat: F (Rel-15)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was revised to R4-2103797.

R4-2103797	CR on corrections for LTE-NR Co-existence tests and OCNG pattern

Type: CR		For: Agreement
					38.101-4 v15.8.0	  CR-0167  rev 1 Cat: F (Rel-15)

					Source: Qualcomm Incorporated

(Replaces R4-2102824)

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2102828	CR on corrections for LTE-NR Co-existence tests and OCNG pattern

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0168  Cat: A (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

#### 4.9.2	CSI requirements [NR_newRAT-Perf]

R4-2101945	CR on FRC for NR RI requirements (Rel-15)

Type: CR		For: Agreement
					38.101-4 v15.8.0	  CR-0161  Cat: F (Rel-15)

					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was revised to R4-2103798.

R4-2103798	CR on FRC for NR RI requirements (Rel-15)

Type: CR		For: Agreement
					38.101-4 v15.8.0	  CR-0161  rev 1 Cat: F (Rel-15)

					Source: Intel Corporation

(Replaces R4-2101945)

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2101946	CR on FRC for NR RI requirements (Rel-16)

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0162  Cat: A (Rel-16)

					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2102870	CR to 38.101-4 on update to CSI reporting test parameters for Aperiodic reporting (R15)

Type: CR		For: Agreement
					38.101-4 v15.8.0	  CR-0169  Cat: F (Rel-15)

					Source: Apple

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was revised to R4-2103799.

R4-2103799	CR to 38.101-4 on update to CSI reporting test parameters for Aperiodic reporting (R15)

Type: CR		For: Agreement
					38.101-4 v15.8.0	  CR-0169  rev 1 Cat: F (Rel-15)

					Source: Apple

(Replaces R4-2102870)

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2102871	CR to 38.101-4 on update to CSI reporting test parameters for Aperiodic reporting (R16)

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0170  Cat: A (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

#### 4.9.3	BS demodulation requirements [NR_newRAT-Perf]

R4-2100548	CR for 38.141-1: BS demodulation synchronization in test setup

Type: CR		For: Agreement
					38.141-1 v15.7.0	  CR-0170  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Include the sentence “In tests performed with signal generators a synchronization signal may be provided from the BS to the signal generator, to enable correct timing of the wanted signal” from the radiated test specificaiton also in the conducted test sp

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was revised to R4-2103800.

R4-2103800	CR for 38.141-1: BS demodulation synchronization in test setup

Type: CR		For: Agreement
					38.141-1 v15.7.0	  CR-0170  rev 1 Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100548)

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2100549	CR for 38.141-1: BS demodulation synchronization in test setup

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0171  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2100550	CR for 38.141-1: BS demodulation synchronization in test setup

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0172  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2100551	CR for 38.141-1: BS demodulation different channel bandwidths applicability rules

Type: CR		For: Agreement
					38.141-1 v15.7.0	  CR-0173  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Modify the first sentence of the various “Applicability of requirements for different channel bandwidths” clauses, to clarify that test requirements apply for all supported CBWs.

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2100552	CR for 38.141-1: BS demodulation different channel bandwidths applicability rules

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0174  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2100553	CR for 38.141-1: BS demodulation different channel bandwidths applicability rules

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0175  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2100554	CR for 38.141-2: BS demodulation different channel bandwidths applicability rules

Type: CR		For: Agreement
					38.141-2 v15.8.0	  CR-0266  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Modify the first sentence of the various “Applicability of requirements for different channel bandwidths” clauses, to clarify that test requirements apply for all supported CBWs.

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was revised to R4-2103801.

R4-2103801	CR for 38.141-2: BS demodulation different channel bandwidths applicability rules

Type: CR		For: Agreement
					38.141-2 v15.8.0	  CR-0266  rev 1 Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100554)

Discussion:

Session Chair: Pay attention on cover page error pointed out by MCC! See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2100555	CR for 38.141-2: BS demodulation different channel bandwidths applicability rules

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0267  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2100556	CR for 38.141-2: BS demodulation different channel bandwidths applicability rules

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0268  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2100557	On BS demodulation different channel bandwidths applicability rules and synchronization in test setup

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, we have explained and motivated our CRs introducing BS demodulation specification text changes pertaining to the applicability of requirements for different channel bandwidths and synchronization signals in conducted test setups.

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was noted.

R4-2103802	CR for 38.141-2: BS demodulation synchronization in test setup

Type: CR		For: Agreement
					38.141-2 v15.8.0	  CR-0312  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2103803	CR for 38.141-2: BS demodulation synchronization in test setup

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0313  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

R4-2103804	CR for 38.141-2: BS demodulation synchronization in test setup

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0314  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][314] Demod_R15_Maintenance in R4-2103925.

Decision: 		The document was agreed.

### 4.10	Positioning specs maintenance (36.171, 37.171 and 38.171) [NR_newRAT-Perf or TEI]

R4-2103002	Email discussion summary for [98e][154] NR_reply_LS_Part_1

Type: other		For: discussion
					Source: Moderator (Xiaomi)

Discussion:

The contribution summarised email discussion thread [98e][154] NR_reply_LS_Part_1. The discussion thread was moderated by Shengxiang Guo (Xiamo).

Decision: 		The document was revised to R4-2103340.

R4-2103340	Email discussion summary for [98e][154] NR_reply_LS_Part_1

Type: other		For: discussion
					Source: Moderator (Xiaomi)

(Replaces R4-2103002)

Discussion:

The contribution summarised email discussion thread [98e][154] NR_reply_LS_Part_1. The discussion thread was moderated by Shengxiang Guo (Xiamo).

Decision: 		The document was noted.

R4-2103287	Way forward on frequency Bands for testing of A-GNSS Sensitivity requirements in NR and LTE

Type: other		For: Approval
					Source: Apple

Discussion:

See email discussion summary for [98e][154] NR_reply_LS_Part_1 in R4-2103340.

Decision: 		The document was approved.

R4-2100196	Analysis of RF interferences to A-GNSS Sensitivity requirements in NR and LTE

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][154] NR_reply_LS_Part_1 in R4-2103340.

Decision: 		The document was noted.

R4-2101923	Frequency Bands for testing of A-GNSS Sensitivity in EN-DC

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][154] NR_reply_LS_Part_1 in R4-2103340.

Decision: 		The document was noted.

### 4.11	Testability Maintenance (38.810) [FS_NR_test_methods]

## 5	LTE maintenance (up to Rel15) [WI code or TEI]

R4-2103442	Email discussion summary: [98e][203] LTE_RRM_maintenance

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][203] LTE_RRM_maintenance. The discussion thread was moderated by Santhan Thangarasa (Ericsson).

Decision: 		The document was revised to R4-2103685.

R4-2103685	Email discussion summary: [98e][203] LTE_RRM_maintenance

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2103442)

Discussion:

The contribution summarised email discussion thread [98e][203] LTE_RRM_maintenance. The discussion thread was moderated by Santhan Thangarasa (Ericsson).

Decision: 		The document was noted.

R4-2100457	CR on NCSG in 36.133

Type: CR		For: Agreement
					36.133 v15.12.0	  CR-7013  Cat: F (Rel-15)

					Source: CATT

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was merged.

R4-2100458	CR on NCSG in 36.133

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7014  Cat: A (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was withdrawn.

R4-2100459	CR on NCSG in 36.133

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7015  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was withdrawn.

R4-2100813	CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1523  Cat: F (Rel-15)

					Source: CMCC

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was withdrawn.

R4-2100814	CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1524  Cat: F (Rel-16)

					Source: CMCC

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was withdrawn.

R4-2100815	CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1525  Cat: F (Rel-17)

					Source: CMCC

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was withdrawn.

R4-2100873	CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used

Type: CR		For: Agreement
					38.133 v15.12.0	  CR-1530  Cat: F (Rel-15)

					Source: CMCC

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was withdrawn.

R4-2100874	CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1531  Cat: A (Rel-16)

					Source: CMCC

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was withdrawn.

R4-2100875	CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1532  Cat: A (Rel-17)

					Source: CMCC

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was withdrawn.

R4-2101453	CR: Correction of eMTC RLM test cases (Rel-14)

Type: CR		For: Agreement
					36.133 v14.17.0	  CR-7019  Cat: F (Rel-14)

					Source: Ericsson

Abstract:

This CR corrects TBD and removes [] from Rel-14 eMTC early-OOS/early-IS tests.

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was revised to R4-2103499.

R4-2103499	CR: Correction of eMTC RLM test cases (Rel-14)

Type: CR		For: Agreement
					36.133 v14.17.0	  CR-7019  rev 1 Cat: F (Rel-14)

					Source: Ericsson

(Replaces R4-2101453)

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2101454	CR: Correction of eMTC RLM test cases

Type: CR		For: Agreement
					36.133 v15.12.0	  CR-7020  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

This CR corrects TBD and removes [] from Rel-14 eMTC early-OOS/early-IS tests.

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was revised to R4-2103500.

R4-2103500	CR: Correction of eMTC RLM test cases

Type: CR		For: Agreement
					36.133 v15.12.0	  CR-7020  rev 1 Cat: F (Rel-15)

					Source: Ericsson

(Replaces R4-2101454)

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2101455	CR: Correction of eMTC RLM test cases

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7021  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

This CR corrects TBD and removes [] from Rel-14 eMTC early-OOS/early-IS tests.

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2101456	CR: Correction of eMTC RLM test cases

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7022  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

This CR corrects TBD and removes [] from Rel-14 eMTC early-OOS/early-IS tests.

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2102348	CR 36.133 (A.8.16.106) Correction of test case for direct SCell activation at addition

Type: CR		For: Agreement
					36.133 v15.12.0	  CR-7038  Cat: F (Rel-15)

					Source: Ericsson

Abstract:

Addressing a missing MG configuration

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2102349	CR 36.133 (A.8.16.106) Correction of test case for direct SCell activation at addition

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7039  Cat: A (Rel-16)

					Source: Ericsson

Abstract:

Addressing a missing MG configuration

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2102350	CR 36.133 (A.8.16.106) Correction of test case for direct SCell activation at addition

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7040  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Addressing a missing MG configuration

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2102693	Correction to applicability of E-UTRAN E-CID measurements requirements for NE-DC

Type: CR		For: Agreement
					36.133 v15.12.0	  CR-7051  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Correction of referencing errors in applicability for NE-DC

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was revised to R4-2103501.

R4-2103501	Correction to applicability of E-UTRAN E-CID measurements requirements for NE-DC

Type: CR		For: Agreement
					36.133 v15.12.0	  CR-7051  rev 1 Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102693)

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2102694	Correction to applicability of E-UTRAN E-CID measurements requirements for NE-DC

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7052  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Correction of referencing errors in applicability for NE-DC

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2102695	Correction to applicability of E-UTRAN E-CID measurements requirements for NE-DC

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7053  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Correction of referencing errors in applicability for NE-DC

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2102696	Correction to requirements for NCSG patterns

Type: CR		For: Agreement
					36.133 v14.17.0	  CR-7054  Cat: F (Rel-14)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Correction of referencing errors for NCSG patterns

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was revised to R4-2103502.

R4-2103502	Correction to requirements for NCSG patterns

Type: CR		For: Agreement
					36.133 v14.17.0	  CR-7054  rev 1 Cat: F (Rel-14)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102696)

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2102697	Correction to requirements for NCSG patterns

Type: CR		For: Agreement
					36.133 v15.12.0	  CR-7055  Cat: A (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Correction of referencing errors for NCSG patterns

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2102698	Correction to requirements for NCSG patterns

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7056  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Correction of referencing errors for NCSG patterns

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2102699	Correction to requirements for NCSG patterns

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7057  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Correction of referencing errors for NCSG patterns

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2102804	CR on TC for eMTC RSTD measurement R15

Type: CR		For: Agreement
					36.133 v15.12.0	  CR-7074  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was revised to R4-2103503.

R4-2103503	CR on TC for eMTC RSTD measurement R15

Type: CR		For: Agreement
					36.133 v15.12.0	  CR-7074  rev 1 Cat: F (Rel-15)

					Source: Huawei, HiSilicon

(Replaces R4-2102804)

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was revised to R4-2104072.

R4-2104072	CR on TC for eMTC RSTD measurement R15

Type: CR		For: Agreement
					36.133 v15.12.0	  CR-7074  rev 2 Cat: F (Rel-15)

					Source: Huawei, HiSilicon

(Replaces R4-2103503)

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2102805	CR on TC for eMTC RSTD measurement R16

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7075  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2102806	CR on TC for eMTC RSTD measurement R17

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7076  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2102807	CR on CRS muting for eMTC R15

Type: CR		For: Agreement
					36.133 v15.12.0	  CR-7077  Cat: F (Rel-15)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was revised to R4-2103504.

R4-2103504	CR on CRS muting for eMTC R15

Type: CR		For: Agreement
					36.133 v15.12.0	  CR-7077  rev 1 Cat: F (Rel-15)

					Source: Huawei, HiSilicon

(Replaces R4-2102807)

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2102808	CR on CRS muting for eMTC R16

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7078  Cat: A (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

R4-2102809	CR on CRS muting for eMTC R17

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7079  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][203] LTE_RRM_maintenance in R4-2103685.

Decision: 		The document was agreed.

### 5.1	BS RF requirements [WI code or TEI]

R4-2101082	CR to TS 36.104: Additions of regional requirements for band 41 in Japan, Rel-15

Type: CR		For: Agreement
					36.104 v15.10.0	  CR-4922  Cat: F (Rel-15)

					Source: NEC, SoftBank, KDDI

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was revised to R4-2103775.

R4-2103775	CR to TS 36.104: Additions of regional requirements for band 41 in Japan, Rel-15

Type: CR		For: Agreement
					36.104 v15.10.0	  CR-4922  rev 1 Cat: F (Rel-15)

					Source: NEC, SoftBank, KDDI, Nokia

(Replaces R4-2101082)

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2101083	CR to TS 36.104: Additions of regional requirements for band 41 in Japan, Rel-16

Type: CR		For: Agreement
					36.104 v16.8.0	  CR-4923  Cat: A (Rel-16)

					Source: NEC, SoftBank, KDDI, Nokia

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2101084	CR to TS 36.104: Additions of regional requirements for band 41 in Japan, Rel-17

Type: CR		For: Agreement
					36.104 v17.0.0	  CR-4924  Cat: A (Rel-17)

					Source: NEC, SoftBank, KDDI, Nokia

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2101085	CR to TS 36.141: Additions of regional requirements for band 41 in Japan, Rel-15

Type: CR		For: Agreement
					36.141 v15.11.0	  CR-1290  Cat: F (Rel-15)

					Source: NEC, SoftBank, KDDI

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was revised to R4-2103776.

R4-2103776	CR to TS 36.141: Additions of regional requirements for band 41 in Japan, Rel-15

Type: CR		For: Agreement
					36.141 v15.11.0	  CR-1290  rev 1 Cat: F (Rel-15)

					Source: NEC, SoftBank, KDDI, Nokia

(Replaces R4-2101085)

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2101086	CR to TS 36.141: Additions of regional requirements for band 41 in Japan, Rel-16

Type: CR		For: Agreement
					36.141 v16.8.0	  CR-1291  Cat: A (Rel-16)

					Source: NEC, SoftBank, KDDI, Nokia

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2101087	CR to TS 36.141: Additions of regional requirements for band 41 in Japan, Rel-17

Type: CR		For: Agreement
					36.141 v17.0.0	  CR-1292  Cat: A (Rel-17)

					Source: NEC, SoftBank, KDDI, Nokia

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

### 5.2	UE RF requirements [WI code or TEI]

R4-2102953	Email discussion summary for [98e][105] LTE_Maintenance

Type: other		For: discussion
					Source: Moderator (Skyworks)

Discussion:

The contribution summarised email discussion thread [98e][105] LTE_Maintenance. The discussion thread was moderated by Dominique Brunel (Skyworks Solutions Inc.).

Decision: 		The document was revised to R4-2103295.

R4-2103295	Email discussion summary for [98e][105] LTE_Maintenance

Type: other		For: discussion
					Source: Moderator (Skyworks)

(Replaces R4-2102953)

Discussion:

The contribution summarised email discussion thread [98e][105] LTE_Maintenance. The discussion thread was moderated by Dominique Brunel (Skyworks Solutions Inc.).

Decision: 		The document was noted.

R4-2103131	Way forward on LTE band 41 NS04 A-MPR for UL 256 QAM

Type: other		For: discussion
					Source: T-Mobile USA, Huawei, HiSilicon, T-mobile USA, Qualcomm Inc

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was approved.

R4-2100053	CR for missing B48 references in a table and note

Type: CR		For: Agreement
					36.101 v14.17.0	  CR-5713  Cat: F (Rel-14)

					Source: C Spire Wireless

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was agreed.

R4-2100244	CR for missing B48 references in a table and note

Type: CR		For: Agreement
					36.101 v15.13.0	  CR-5713  rev 1 Cat: A (Rel-15)

					Source: C Spire Wireless

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was withdrawn.

R4-2100645	CR for missing B48 references in a table and note

Type: CR		For: Agreement
					36.101 v15.13.0	  CR-5717  Cat: A (Rel-15)

					Source: C Spire Wireless

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was withdrawn.

R4-2100648	CR for missing B48 references in a table and note

Type: CR		For: Agreement
					36.101 v16.8.0	  CR-5718  Cat: A (Rel-16)

					Source: C Spire Wireless

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was agreed.

R4-2100651	CR for missing B48 references in a table and note

Type: CR		For: Agreement
					36.101 v17.0.0	  CR-5719  Cat: A (Rel-17)

					Source: C Spire Wireless

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was agreed.

R4-2101197	Addition of UE co-existence requirements for 40 and n40

Type: other		For: Approval
					Source: NTT DOCOMO, INC., SoftBank Corp., KDDI Corporation, Rakuten Mobile, Inc

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was revised to R4-2103134.

R4-2103134	Addition of UE co-existence requirements for 40 and n40

Type: other		For: Approval
					Source: NTT DOCOMO, INC., SoftBank Corp., KDDI Corporation, Rakuten Mobile, Inc

(Replaces R4-2101197)

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was approved.

R4-2101802	CR for 36.101 to add missing spurious emissions for band 38 UE co-existence (Rel-16)

Type: CR		For: Agreement
					36.101 v16.8.0	  CR-5721  Cat: F (Rel-16)

					Source: Huawei, HiSilicon, DT

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was agreed.

R4-2101803	CR for 36.101 to add missing spurious emissions for band 38 UE co-existence (Rel-17)

Type: CR		For: Agreement
					36.101 v17.0.0	  CR-5722  Cat: A (Rel-17)

					Source: Huawei, HiSilicon, DT

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was agreed.

R4-2102098	Test frequencies for NB-IOT UE in standalone operation

Type: other		For: Discussion
					Source: Sony

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was noted.

R4-2102437	A-MPR for LTE CA_NS_04 256QAM PC2

Type: discussion		For: Approval
					Source: Huawei, HiSilicon

Abstract:

A-MPR values are proposed for LTE CA_NS_04 256QAM PC2 based on simulation results.

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was noted.

R4-2102596	CR for TS 36.101: Cleanup for spurious emissions for UE co-existence table

Type: CR		For: Agreement
					36.101 v15.13.0	  CR-5724  Cat: F (Rel-15)

					Source: Apple

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was revised to R4-2103132.

R4-2103132	CR for TS 36.101: Cleanup for spurious emissions for UE co-existence table

Type: CR		For: Agreement
					36.101 v15.13.0	  CR-5724  rev 1 Cat: F (Rel-15)

					Source: Apple

(Replaces R4-2102596)

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was agreed.

### 5.3	RRM requirements  [WI code or TEI]

### 5.4	Demodulation and CSI requirements  [WI code or TEI]

#### 5.4.1	UE demodulation and CSI requirements [WI code or TEI]

#### 5.4.2	BS demodulation requirements [WI code or TEI]

## 6	Rel-16 Work Items for LTE

### 6.1	Additional MTC enhancements for LTE [LTE_eMTC5]

#### 6.1.1	RF core requirements maintenance [LTE_eMTC5-Core]

#### 6.1.2	RRM requirements maintenance [LTE_eMTC5-Core/Perf]

R4-2103464	Email discussion summary: [98e][225] LTE_eMTC5_RRM

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][225] LTE_eMTC5_RRM. The discussion thread was moderated by Santhan Thangarasa (Ericsson).

Decision: 		The document was revised to R4-2103706.

R4-2103706	Email discussion summary: [98e][225] LTE_eMTC5_RRM

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2103464)

Discussion:

The contribution summarised email discussion thread [98e][225] LTE_eMTC5_RRM. The discussion thread was moderated by Santhan Thangarasa (Ericsson).

Decision: 		The document was noted.

R4-2103657	LS on timing of RSS in neighbor cell for Rel-16 eMTC

Type: LS out		For: Approval
					to RAN1, RAN2
					Source: RAN4

Discussion:

See email discussion summary for [98e][225] LTE_eMTC5_RRM in R4-2103706.

Decision: 		The document was approved.

R4-2101457	Correction Rel-16 eMTC RRM performance requirements

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7023  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

This CR corrects RRM performance requirements defined in Rel-16 eMTC WI.

Discussion:

See email discussion summary for [98e][225] LTE_eMTC5_RRM in R4-2103706.

Decision: 		The document was revised to R4-2103658.

R4-2103658	Correction Rel-16 eMTC RRM performance requirements

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7023  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2101457)

Discussion:

See email discussion summary for [98e][225] LTE_eMTC5_RRM in R4-2103706.

Decision: 		The document was agreed.

R4-2101458	Correction Rel-16 eMTC RRM performance requirements

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7024  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

This CR corrects RRM performance requirements defined in Rel-16 eMTC WI.

Discussion:

See email discussion summary for [98e][225] LTE_eMTC5_RRM in R4-2103706.

Decision: 		The document was agreed.

R4-2102247	Discussions on RRM maintenance issues for Rel-16 eMTC UE

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we discuss the open issues that were identified at last meeting with regard to INACTIVE state operation and RSS based measurement for release 16 eMTC.

Discussion:

See email discussion summary for [98e][225] LTE_eMTC5_RRM in R4-2103706.

Decision: 		The document was noted.

R4-2102296	On RSS measurement requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][225] LTE_eMTC5_RRM in R4-2103706.

Decision: 		The document was noted.

R4-2102796	Discussion on remaining issues in RSS measurement and RRC_Inactive state

Type: LS out		For: Approval
					to RAN1, cc RAN2
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][225] LTE_eMTC5_RRM in R4-2103706.

Decision: 		The document was noted.

R4-2103728	LS on RSS based RSRQ for LTE-MTC

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: RAN4

Discussion:

See email discussion summary for [98e][225] LTE_eMTC5_RRM in R4-2103706.

Decision: 		The document was approved.

R4-2102797	CR on eMTC RRM requirements

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7072  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][225] LTE_eMTC5_RRM in R4-2103706.

Decision: 		The document was revised to R4-2103659.

R4-2103659	CR on eMTC RRM requirements

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7072  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2102797)

Discussion:

See email discussion summary for [98e][225] LTE_eMTC5_RRM in R4-2103706.

Decision: 		The document was agreed.

R4-2102798	CR on eMTC RRM requirements R17

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7073  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][225] LTE_eMTC5_RRM in R4-2103706.

Decision: 		The document was agreed.

### 6.2	Additional enhancements for NB-IoT [NB_IOTenh3]

#### 6.2.1	RF core requirements maintenance [NB_IOTenh3-Core]

#### 6.2.2	RRM requirements maintenance [NB_IOTenh3-Core/Perf]

R4-2103465	Email discussion summary: [98e][226] NB_IOTenh3_RRM

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][226] NB_IOTenh3_RRM. The discussion thread was moderated by Zhongyi Shen (Huawei).

Decision: 		The document was revised to R4-2103707.

R4-2103707	Email discussion summary: [98e][226] NB_IOTenh3_RRM

Type: other		For: discussion
					Source: Moderator (Huawei)

(Replaces R4-2103465)

Discussion:

The contribution summarised email discussion thread [98e][226] NB_IOTenh3_RRM. The discussion thread was moderated by Zhongyi Shen (Huawei).

Decision: 		The document was noted.

R4-2101654	CR on maintenance for measurement on non-anchor carrier for NB-IoT

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7027  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][226] NB_IOTenh3_RRM in R4-2103707.

Decision: 		The document was revised to R4-2103660.

R4-2103660	CR on maintenance for measurement on non-anchor carrier for NB-IoT

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7027  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101654)

Discussion:

See email discussion summary for [98e][226] NB_IOTenh3_RRM in R4-2103707.

Decision: 		The document was agreed.

R4-2101655	CR on maintenance for measurement on non-anchor carrier for NB-IoT

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7028  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][226] NB_IOTenh3_RRM in R4-2103707.

Decision: 		The document was agreed.

R4-2102234	Correction to PUR requirements for NB1 UE

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7033  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

PUR requirements have been aligned between category M1 and NB1 UEs. Regarding the use of relaxation factor N in the measurement validation expression, the description of N is not aligned.

Discussion:

See email discussion summary for [98e][226] NB_IOTenh3_RRM in R4-2103707.

Decision: 		The document was withdrawn.

R4-2102235	Correction to PUR requirements for NB1 UE

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7034  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

PUR requirements have been aligned between category M1 and NB1 UEs. Regarding the use of relaxation factor N in the measurement validation expression, the description of N is not aligned.

Discussion:

Session chair: Cover sheet errors (What is the work item? It reads NB_IOTen3-Perf on the cover page but the Tdoc is reserved for NB_IOTenh3-Core.)

See email discussion summary for [98e][226] NB_IOTenh3_RRM in R4-2103707.

Decision: 		The document was not pursued.

### 6.3	Even further Mobility enhancement in E-UTRAN [LTE_feMob]

#### 6.3.1	RRM core requirements maintenance [LTE_feMob-Core]

R4-2103466	Email discussion summary: [98e][227] LTE_feMob_RRM

Type: other		For: discussion
					Source: Moderator (Nokia)

Discussion:

The contribution summarised email discussion thread [98e][227] LTE_feMob_RRM. The discussion thread was moderated by Delia Chen (Nokia).

Decision: 		The document was revised to R4-2103708.

R4-2103708	Email discussion summary: [98e][227] LTE_feMob_RRM

Type: other		For: discussion
					Source: Moderator (Nokia)

(Replaces R4-2103466)

Discussion:

The contribution summarised email discussion thread [98e][227] LTE_feMob_RRM. The discussion thread was moderated by Delia Chen (Nokia).

Decision: 		The document was noted.

R4-2101207	TDD UL-DL and DL-UL switching in LTE DAPS handover

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Further clarification on DL-to-UL and UL-to-DL switching time

Discussion:

See email discussion summary for [98e][227] LTE_feMob_RRM in R4-2103708.

Decision: 		The document was noted.

R4-2101208	Correction on the synchronous condition for DAPS handover

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7017  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

Add conditions for not expected to transmit / not expected to receive covering both source and target cell. Add autonomous interruption allowance if these conditions are unspecified.

Discussion:

See email discussion summary for [98e][227] LTE_feMob_RRM in R4-2103708.

Decision: 		The document was postponed.

R4-2101209	Correction on the synchronous condition for DAPS handover

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7018  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Add conditions for not expected to transmit / not expected to receive covering both source and target cell. Add autonomous interruption allowance if these conditions are unspecified.

Discussion:

See email discussion summary for [98e][227] LTE_feMob_RRM in R4-2103708.

Decision: 		The document was withdrawn.

R4-2101697	Clarification on asynchronous DAPS handover

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7029  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][227] LTE_feMob_RRM in R4-2103708.

Decision: 		The document was revised to R4-2103661.

R4-2103661	Clarification on asynchronous DAPS handover

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7029  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101697)

Discussion:

See email discussion summary for [98e][227] LTE_feMob_RRM in R4-2103708.

Decision: 		The document was agreed.

R4-2101698	Clarification on asynchronous DAPS handover

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7030  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][227] LTE_feMob_RRM in R4-2103708.

Decision: 		The document was agreed.

#### 6.3.2	RRM perf. requirements [LTE_feMob-Perf]

##### 6.3.2.1	General [LTE_feMob-Perf]

##### 6.3.2.2	Test cases [LTE_feMob-Perf]

R4-2101699	Test cases for inter-frequency DAPS handover

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7031  Cat: B (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][227] LTE_feMob_RRM in R4-2103708.

Decision: 		The document was revised to R4-2103662.

R4-2103662	Test cases for inter-frequency DAPS handover

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7031  rev 1 Cat: B (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101699)

Discussion:

See email discussion summary for [98e][227] LTE_feMob_RRM in R4-2103708.

Decision: 		The document was agreed.

R4-2101700	Test cases for inter-frequency DAPS handover

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7032  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][227] LTE_feMob_RRM in R4-2103708.

Decision: 		The document was agreed.

R4-2102485	CR on 36133 LTE FDD-TDD inter-F sync DAPS HO TCs

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7043  Cat: B (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Test cases for LTE FDD-TDD inter-band inter-F sync DAPS HO.

Discussion:

See email discussion summary for [98e][227] LTE_feMob_RRM in R4-2103708.

Decision: 		The document was agreed.

R4-2102486	CR on 36133 LTE FDD-TDD inter-F sync DAPS HO TCs - Cat-A

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7044  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Mirror CR for Rel17 on test cases for LTE FDD-TDD inter-band inter-F sync DAPS HO.

Discussion:

See email discussion summary for [98e][227] LTE_feMob_RRM in R4-2103708.

Decision: 		The document was agreed.

R4-2102632	Addition of missing async FDD-TDD and TDD-FDD LTE interband interfrquency DAPS handover tests

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7047  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

The side conditions are related to one shot timing adjustment, which was removed. The annex is no more applicable and is removed.

Discussion:

See email discussion summary for [98e][227] LTE_feMob_RRM in R4-2103708.

Decision: 		The document was revised to R4-2103663.

R4-2103663	Addition of missing async FDD-TDD and TDD-FDD LTE interband interfrquency DAPS handover tests

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7047  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2102632)

Discussion:

See email discussion summary for [98e][227] LTE_feMob_RRM in R4-2103708.

Decision: 		The document was agreed.

R4-2102633	Addition of missing async FDD-TDD and TDD-FDD LTE interband interfrquency DAPS handover tests

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7048  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

The side conditions are related to one shot timing adjustment, which was removed. The annex is no more applicable and is removed.

Discussion:

See email discussion summary for [98e][227] LTE_feMob_RRM in R4-2103708.

Decision: 		The document was agreed.

### 6.4	R16 LTE maintenance [WI code]

#### 6.4.1	BS RF requirements [WI code]

#### 6.4.2	UE RF requirements [WI code]

R4-2102604	CR for TS 36.101: Cleanup for spurious emissions for UE co-existence table

Type: CR		For: Agreement
					36.101 v16.8.0	  CR-5725  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was revised to R4-2103133.

R4-2103133	CR for TS 36.101: Cleanup for spurious emissions for UE co-existence table

Type: CR		For: Agreement
					36.101 v16.8.0	  CR-5725  rev 1 Cat: F (Rel-16)

					Source: Apple

(Replaces R4-2102604)

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was agreed.

R4-2102605	CR for TS 36.101: Cleanup for spurious emissions for UE co-existence table

Type: CR		For: Agreement
					36.101 v17.0.0	  CR-5726  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][105] LTE_Maintenance in R4-2103295.

Decision: 		The document was agreed.

#### 6.4.3	RRM requirements [WI code]

R4-2102248	Clarification for performance requirements tests for euCA

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7036  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][227] LTE_feMob_RRM in R4-2103708.

Decision: 		The document was agreed.

R4-2102249	Clarification for performance requirements tests for euCA

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7037  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][227] LTE_feMob_RRM in R4-2103708.

Decision: 		The document was agreed.

#### 6.4.4	Demodulation and CSI requirements [WI code]

##### 6.4.4.1	UE demodulation and CSI requirements [WI code]

##### 6.4.4.2	BS demodulation requirements [WI code]

## 7	Rel-16 non-spectrum related work items for NR

### 7.1	NR-based access to unlicensed spectrum [NR_unlic]

R4-2103743	Email discussion summary for [98e][304] NR_unlic_BSRF_Conformance

Type: other		For: discussion
					Source: Moderator (ZTE)

Discussion:

The contribution summarised email discussion thread [98e][304] NR_unlic_BSRF_Conformance. The discussion thread was moderated by Xue Fei (ZTE).

Decision: 		The document was revised to R4-2103926.

R4-2103926	Email discussion summary for [98e][304] NR_unlic_BSRF_Conformance

Type: other		For: discussion
					Source: Moderator (ZTE)

(Replaces R4-2103743)

Discussion:

The contribution summarised email discussion thread [98e][304] NR_unlic_BSRF_Conformance. The discussion thread was moderated by Xue Fei (ZTE).

Decision: 		The document was noted.

------------------------------GTW agenda of Jan.29th for email thread [304] (45minutes) ------------------

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Issue 1-2 and 2-2: Agreement: It is agreed to remove the NB-IoT note for NR-U. Discussion: Nokia: we can further discuss in 2nd round, and these bands are adjacent, we should have a note into the specification. Nokia: These notes not needed for MSR and AAS specifications. Huawei: There are two issues: Issue 1: MSR specifications: not needed since bands not specified there Issue 2: for 38 series, we need to do further discuss. This should in spurious domain, not in OBUE domain, we would like to further discuss in 2nd round. Issue 2: whether Tx co-location requirement exception for n96 should be applied for n46 and vice versa Agreements: Remove the note of band n96, n46 from TS 37.104 and 37.105 specification Tx co-location requirement exception needed for band n96/n46 in 38.104 |

Issue 3-1: MU and TT for n46 and n96

- Proposals

  - Option 1:  propose to extend NR upper frequency from 6GHz to 7.125GHz; [ZTE, R4-2101982]

  - Option 2: reuse LAA MU requirements for n46 and n96; [Nokia, R4-2101566]

  - Option 3: other

- Recommended WF

  - TBA

Discussion:

ZTE: we didn’t get information from TE vendors.

Nokia: TE vendors’ feedback are encouraged.

I review the history of eLAA phase, we used the values from previous. The difference among option 1 and option 2 maybe no big difference. We think with band n96, more reasonable to use values from LAA as it’s unlicensed operation.

ZTE: Usually we have a calculation tables taking into account all the major contributors for Rel-15 NR which different with LTE LAA.

Question 1: What’s the major contributors?

Question 2: What’s the procedure to derive MU values?

Huawei: We are fine to keep options open, for both options the frequency extend need to be extended from 6GHz to 7.125GHz.

For LTE LAA, we don’t BS type 1-H, this need to be considered when define MU.

Keep above options open, further discuss in 2nd round and future meeting if needed.

TE vendors input are encouraged.

--------------------------------End -----------------------------

R4-2101972	CR to TS 36.104: corrections of NR-U BS RF requirements

Type: CR		For: Agreement
					36.104 v16.8.0	  CR-4925  Cat: F (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was revised to R4-2103973.

R4-2103973	CR to TS 36.104: corrections of NR-U BS RF requirements

Type: CR		For: Agreement
					36.104 v16.8.0	  CR-4925  rev 1 Cat: F (Rel-16)

					Source: ZTE Corporation

(Replaces R4-2101972)

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was agreed.

R4-2101973	CR to TS 36.104: corrections of NR-U BS RF requirements

Type: CR		For: Agreement
					36.104 v17.0.0	  CR-4926  Cat: A (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was agreed.

R4-2101974	CR to TS 37.104: corrections of NR-U BS RF requirements

Type: CR		For: Agreement
					37.104 v16.8.0	  CR-0922  Cat: F (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was revised to R4-2103974.

R4-2103974	CR to TS 37.104: corrections of NR-U BS RF requirements

Type: CR		For: Agreement
					37.104 v16.8.0	  CR-0922  rev 1 Cat: F (Rel-16)

					Source: ZTE Corporation

(Replaces R4-2101974)

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was agreed.

R4-2101975	CR to TS 37.104: corrections of NR-U BS RF requirements

Type: CR		For: Agreement
					37.104 v17.0.0	  CR-0923  Cat: A (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was agreed.

R4-2101976	CR to TS 37.105: corrections of NR-U BS RF requirements

Type: CR		For: Agreement
					37.105 v16.6.0	  CR-0220  Cat: F (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was not pursued.

R4-2101977	CR to TS 37.105: corrections of NR-U BS RF requirements

Type: CR		For: Agreement
					37.105 v17.0.0	  CR-0221  Cat: A (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was withdrawn.

#### 7.1.1	System parameters maintenance [NR_unlic-Core]

R4-2102954	Email discussion summary for [98e][106] NR_unlic_Maintenance

Type: other		For: discussion
					Source: Moderator (Qualcomm)

Discussion:

The contribution summarised email discussion thread [98e][106] NR_unlic_Maintenance. The discussion thread was moderated by Gene Fong (Qualcomm).

Decision: 		The document was revised to R4-2103296.

R4-2103296	Email discussion summary for [98e][106] NR_unlic_Maintenance

Type: other		For: discussion
					Source: Moderator (Qualcomm)

(Replaces R4-2102954)

Discussion:

The contribution summarised email discussion thread [98e][106] NR_unlic_Maintenance. The discussion thread was moderated by Gene Fong (Qualcomm).

Decision: 		The document was noted.

R4-2100511	NR-U wideband operation and intra-carrier guard bands

Type: discussion		For: Decision
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][106] NR_unlic_Maintenance in R4-2103296.

Decision: 		The document was noted.

R4-2100512	Corrections of NR-U wideband operation intra-carrier guard bands

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0619  Cat: F (Rel-16)

					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][106] NR_unlic_Maintenance in R4-2103296.

Decision: 		The document was not pursued.

R4-2100513	Corrections of NR-U wideband operation intra-carrier guard bands

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0620  Cat: A (Rel-17)

					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][106] NR_unlic_Maintenance in R4-2103296.

Decision: 		The document was withdrawn.

R4-2101720	Applicability of minimum requirements for shared spectrum access

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0647  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

CR to add the applicability of minimum requirements for DL (Mode 1) and UL

Discussion:

See email discussion summary for [98e][106] NR_unlic_Maintenance in R4-2103296.

Decision: 		The document was revised to R4-2103135.

R4-2103135	Applicability of minimum requirements for shared spectrum access

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0647  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2101720)

Discussion:

See email discussion summary for [98e][106] NR_unlic_Maintenance in R4-2103296.

Decision: 		The document was not pursued.

R4-2103136	Applicability of minimum requirements for shared spectrum access

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0719  Cat: - (Rel-17)

					Source: Ericsson

Discussion:

See email discussion summary for [98e][106] NR_unlic_Maintenance in R4-2103296.

Decision: 		The document was withdrawn.

R4-2101968	CR to TS 38.104: system parameters maintenance for NR-U

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0281  Cat: F (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][106] NR_unlic_Maintenance in R4-2103296.

Decision: 		The document was revised to R4-2103137.

R4-2103137	CR to TS 38.104: system parameters maintenance for NR-U

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0281  rev 1 Cat: F (Rel-16)

					Source: ZTE Corporation

(Replaces R4-2101968)

Discussion:

See email discussion summary for [98e][106] NR_unlic_Maintenance in R4-2103296.

Decision: 		The document was agreed.

R4-2101969	CR to TS 38.104: system parameters maintenance for NR-U

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0282  Cat: A (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][106] NR_unlic_Maintenance in R4-2103296.

Decision: 		The document was agreed.

R4-2101970	CR to TS 38.101-1: system parameters maintenance for NR-U

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0662  Cat: F (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][106] NR_unlic_Maintenance in R4-2103296.

Decision: 		The document was revised to R4-2103138.

R4-2103138	CR to TS 38.101-1: system parameters maintenance for NR-U

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0662  rev 1 Cat: F (Rel-16)

					Source: ZTE Corporation

(Replaces R4-2101970)

Discussion:

See email discussion summary for [98e][106] NR_unlic_Maintenance in R4-2103296.

Decision: 		The document was agreed.

R4-2101971	CR to TS 38.101-1: system parameters maintenance for NR-U

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0663  Cat: A (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][106] NR_unlic_Maintenance in R4-2103296.

Decision: 		The document was agreed.

#### 7.1.2	UE RF requirements maintenance [NR_unlic-Core]

##### 7.1.2.1	Transmitter characteristics [NR_unlic-Core]

R4-2101932	NR-U - On Intra-cell guardbands

Type: discussion		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][106] NR_unlic_Maintenance in R4-2103296.

Decision: 		The document was noted.

##### 7.1.2.2	Receiver characteristics [NR_unlic-Core]

#### 7.1.3	BS RF requirements maintenance  [NR_unlic-Core]

##### 7.1.3.1	General  [NR_unlic-Core]

##### 7.1.3.2	Transmitter characteristics  [NR_unlic-Core]

R4-2101978	CR to TS 38.104: corrections of NR-U BS Tx requirements

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0283  Cat: F (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was revised to R4-2103888.

R4-2103888	CR to TS 38.104: corrections of NR-U BS Tx requirements

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0283  rev 1 Cat: F (Rel-16)

					Source: ZTE Corporation

(Replaces R4-2101978)

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was agreed.

R4-2101979	CR to TS 38.104: corrections of NR-U BS Tx requirements

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0284  Cat: A (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was agreed.

R4-2102835	CR to TS 38.104 corrections to NR-U BS RF Tx requirements

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0296  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was revised to R4-2103889.

R4-2103889	CR to TS 38.104 corrections to NR-U BS RF Tx requirements

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0296  rev 1 Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102835)

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was agreed.

R4-2102836	CR to TS 38.104 corrections to NR-U BS RF Tx requirements

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0297  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was agreed.

##### 7.1.3.3	Receiver characteristics  [NR_unlic-Core]

R4-2101980	CR to TS 38.104: corrections of NR-U BS Rx requirements

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0285  Cat: F (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was revised to R4-2103975.

R4-2103975	CR to TS 38.104: corrections of NR-U BS Rx requirements

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0285  rev 1 Cat: F (Rel-16)

					Source: ZTE Corporation

(Replaces R4-2101980)

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was agreed.

R4-2101981	CR to TS 38.104: corrections of NR-U BS Rx requirements

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0286  Cat: A (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was agreed.

R4-2102837	CR to TS 38.104 corrections to NR-U BS RF Rx requirements

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0298  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was agreed.

R4-2102838	CR to TS 38.104 corrections to NR-U BS RF Rx requirements

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0299  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was agreed.

#### 7.1.4	BS conformance testing [NR_unlic-Perf]

R4-2101982	CR to TS 38.141-1: introduction of  NR-U BS

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0195  Cat: B (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was postponed.

R4-2101983	CR to TS 38.141-1: introduction of  NR-U BS

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0196  Cat: A (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was withdrawn.

R4-2101984	CR to TS 36.141: introduction of NR-U BS

Type: CR		For: Agreement
					36.141 v16.8.0	  CR-1293  Cat: B (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was postponed.

R4-2101985	CR to TS 36.141: introduction of NR-U BS

Type: CR		For: Agreement
					36.141 v17.0.0	  CR-1294  Cat: A (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was withdrawn.

##### 7.1.4.1	General  [NR_unlic-Perf]

R4-2101566	On NR-U measurement uncertainties for BS conformance tests

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we continue discussion on NR-U measurement uncertainties for BS conformance tests.

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was noted.

R4-2102444	Draft CR to 37.141: Introduction of NR-U co-existence requirements

Type: draftCR		For: Endorsement
					37.141 v17.0.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was postponed.

##### 7.1.4.2	Transmitter characteristics  [NR_unlic-Perf]

R4-2101567	Draft CR to TS 37.107 With NR-U intorduction for performance part

Type: draftCR		For: Endorsement
					37.107 v16.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This is draft CR to TS 37.107 with updates related to NR-U introduction for perfromance part.

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was postponed.

R4-2101733	TS 37.145-2: Tx spurious limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U)

Type: CR		For: Agreement
					37.145-2 v16.6.0	  CR-0276  Cat: B (Rel-16)

					Source: Ericsson

Abstract:

Introduction of tx spurious emission limits for co-existence and co-location with NR-U in bands n46 and n96

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was postponed.

R4-2101734	TS 37.145-2: Tx spurious limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U)

Type: CR		For: Agreement
					37.145-2 v17.0.0	  CR-0277  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Introduction of tx spurious emission limits for co-existence and co-location with NR-U in bands n46 and n96

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was withdrawn.

R4-2101735	TS 38.141-2: Tx spurious limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U)

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0293  Cat: B (Rel-16)

					Source: Ericsson

Abstract:

Introduction of tx spurious emission limits for co-existence and co-location with NR-U in bands n46 and n96

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was postponed.

R4-2101736	TS 38.141-2: Tx spurious limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U)

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0294  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Introduction of tx spurious emission limits for co-existence and co-location with NR-U in bands n46 and n96

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was withdrawn.

##### 7.1.4.3	Receiver characteristics  [NR_unlic-Perf]

R4-2102144	TS 37.145-2: Rx blocking limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U)

Type: CR		For: Agreement
					37.145-2 v16.6.0	  CR-0278  Cat: B (Rel-16)

					Source: Ericsson

Abstract:

Introduction of rx blocking limits for co-existence and co-location with NR-U in bands n46 and n96

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was postponed.

R4-2102145	TS 37.145-2: RX blocking limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U)

Type: CR		For: Agreement
					37.145-2 v17.0.0	  CR-0279  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Introduction of rx blocking limits for co-existence and co-location with NR-U in bands n46 and n96

Discussion:

See email discussion summary for [98e][304] NR_unlic_BSRF_Conformance in R4-2103926.

Decision: 		The document was withdrawn.

#### 7.1.5	RRM core requirements maintenance (38.133) [NR_unlic-Core]

R4-2103444	Email discussion summary: [98e][205] NR_unlic_RRM_1

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][205] NR_unlic_RRM_1. The discussion thread was moderated by Iana Siomina (Ericsson).

Decision: 		The document was revised to R4-2103687.

R4-2103687	Email discussion summary: [98e][205] NR_unlic_RRM_1

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2103444)

Discussion:

The contribution summarised email discussion thread [98e][205] NR_unlic_RRM_1. The discussion thread was moderated by Iana Siomina (Ericsson).

Decision: 		The document was noted.

GTW session (January 26, 2021)

Topic #2: RRC connection mobility control (AI 7.1.5.2)

- Sub-topic 2-1: Random Access requirements - general

  - Issue 2-1-2: Supplementary UL in NR-U requirements

    - Proposal 1 (Nokia): It is not necessary to work on requirements for random access in supplementary uplink in Rel-16 NR-U: this scenario is not in the scope of this work item.

Discussion

    - E///: support Proposal

    - Huawei: we should not preclude SUL. Based on current spec the requirements apply to SUL as well

    - E///: the discuss is about 2step RACH for NR-U. 2step RACH applies for SUL. There is no SUL for NR-U and it should not be discussed.

    - Huawei: support of SUL depends on the BC. It is supported in RAN1.

    - E///: Need to check if there is RF support. We can further check what is going on in RAN1.

    - Nokia: Agree with E/// that we don’t have to work in case in there are no RF requirements. What is Huawei intention (not preclude or include)?

    - Huawei: It is ok for us not have requirement now. In case the RF BCs are defined then we can define it in RRM.

Agreements:

Do not define RRM requirements for random access in supplementary uplink.

Note: The respective requirements can be discussed in the future when the corresponding RF band combinations including NR-U and SUL are introduced and also subject to the outcome of RAN1 discussion.

- Sub-topic 2-2: Random Access requirements – 2-step RA

  - Issue 2-2-1: Whether to define NR-U RA requirements for 2-step RA

    - Proposals

      - Proposal 1 (ZTE, Ericsson, Qualcomm, Nokia): RAN4 to define requirements for 2-step RA in Rel-16 NR-U.

      - Proposal 2 (Huawei/HiSilicon, MTK): Whether to define the requirements for Rel-16 features for NR-U should be taken cautiously. Whether to define 2-step RACH requirements for NR-U shall be discussed also for other Rel-16 features/requirements.

    - Recommended WF

      - RAN4 to define NR-U requirements for 2-step RA.

Discussion

    - MTK: Prefer Proposal 2

    - ZTE: Proposals 1

    - E///: Agree with ZTE that it is already defined in RAN2; 2step RACH requirements were defined in a generic way

    - Nokia, QC: Same view as E/// and ZTE

    - MTK: there are other Rel-16 features (e.g., multiple SCell, multiple BWP switching). Should we define NR-U requirements for these features as well?

      - E///: it is up to RAN4 discussion and decision. Prefer to keep it as a separate discussion.

    - Huawei: no strong view but have same concerns as MTK on how to treat other R16 requirements.

    - Apple: we prefer to define additional features as well

Agreements: Define requirements for 2-step RA in Rel-16 NR-U.

Topic #3: SCell activation/deactivation (AI 7.1.5.3)

- Sub-topic 3-1: Interruptions

  - Issue 3-1-1: General

    - Proposals

      - Proposal 1 (Qualcomm): Interruption requirements (i.e. number of interruptions and starting point of an interruption) on any active cell in the same band as the SCell being activated (intra-band) and any active cell outside this band (inter-band) should be treated separately.

    - Recommended WF

      - Agree to discuss separately:

        - Interruptions on any active cell in the same band with the SCell being activated

        - Interruptions on any active cell outside the band with the SCell being activated

- Discussion

      - QC: interruptions should be discussed from the victim cell perspective

      - MTK: agree with QC

      - Apple: do we consider only 2 CCs or can consider more CCs?

        - QC: no restriction. Different requirements may apply for different cells.

- Agreement

      - Further discuss the following 2 types of interruptions

        - Interruptions on any active cell in the same band with the SCell being activated

        - Interruptions on any active cell outside the band with the SCell being activated

  - Issue 3-1-2: For inter-band CA, interruptions (the number of, and the starting point) on active cells in the same band with the SCell being activated

    - Proposals

      - Proposal 1 (Qualcomm): During inter-band CA, interruption requirements (i.e. number of interruptions and starting point of an interruption) for intra-band CA applies to any active cell, if present, in the same band as the SCell being activated.

      - Proposal 2 (Ericsson): A single interruption applies.

      - Proposal 3: For any active cell in the same band with the SCell being activated, the interruption requirements (i.e. number of interruptions and starting point of an interruption) for intra-band CA applies.

    - Recommended WF

      - Discuss the proposals

- Discussion

      - Huawei: The issue is for inter-band CA. Does P3 mean intra-band CA only?

        - E///: Same question for P3.

      - E///: In case Scell is known then for inter-band CA there will be a single interruption. We follow the legacy requirements.

      - MTK: For Proposal 3 we just add a condition that SCell is in the same band. We need multiple interruption windows.

- Agreements: For inter-band CA

- For any active cell in the same band with the SCell being activated, the interruption requirements (i.e. number of interruptions and starting point of an interruption) for intra-band CA apply.

- For any active cell outside the band with the SCell being activated, the interruption requirements are FFS.

  - Issue 3-1-3: For intra-band CA, additional RF re-retuning for the case when there is already activated SCell

    - Proposals

      - Proposal 1 (Qualcomm): There is no need to specify additional RF re-tuning time or extend the interruption time because of DL LBT failure during intra-band SCell activation.

      - Proposal 2 (Huawei/HiSilicon): For the case when there is already activated SCell within the same band, additional interruption for additional RF tuning is allowed when the SMTC occasion is not available until the first available SMTC occasion.

      - Proposal 3 (ZTE): Additional RF re-tuning time is needed.

    - Recommended WF

      - Discuss the proposals

- Discussion

      - MTK: For intra-band CA the interruption duration is already long. We can extend the interruption time a little bit.

      - QC: Interruption is already long. Even in the worst case UE should be able to retune back.

      - Huawei: We are not talking about multiple interruptions.

      - Chair: continue the discussion

- Sub-topic 3-2: SCell activation/deactivation when sCellDeactivationTimer is NOT configured

  - Issue 3-2-1: Applicability of SCell activation requirements when sCellDeactivationTimer is NOT configured

    - Proposals

      - Proposal 1 (Ericsson, Huawei/HiSilicon, Apple): The SCell activation requirements do not apply, when the sCellDeactivationTimer is not configured

        - Issue addressed by the proposal: the current SCell activation delay requirement in 38.133 has no limit, i.e., it is infinite.

      - Proposal 1a (Ericsson, QC):

        - If Proposal 1 (the requirement is not applicable) is not agreeable, an alternative proposal:

        - The SCell activation requirements for NR-U do not apply when the sCellDeactivationTimer is not configured, when the SCell activation delay exceeds some time (e.g., equivalent or comparable to the longest possible value of sCellDeactivationTimer).

      - Proposal 2 (Nokia, ZTE):

        - Proposal 2a (MTK): SCell activation delay requirements are applicable in Scenario A (CA with NR PCell and NR SCell) with any LBT type and in Scenario B and C (E-UTRAN-NR-U DC/SA NR-U) with LBT type 2C. Requirements are also applicable in all scenarios, if the UE does not experience any UL LBT failures during SCell activation/deactivation.

        - Proposal 2b: SCell activation delay requirements are applicable when sCellDeactivationTimer is not configured also in Scenarios B and C (EN-DC and SA) LBT types other than 2C.

      - Proposal 3 (ZTE): Relative requirements apply, regardless of whether or not the timer is configured.

- Discussion

      - E///: If the timer is not configured, then there is not way to stop the Scell activation procedure from the network or UE perspectives.

      - MTK: Share the concern of no limit. Think that this applies to the case of UL LBT only. Prefer proposal 2a.

        - Nokia/Apple: UL LBT is the problem.

      - Nokia: Not clear what would happen if we say that the requirements do not apply

      - Apple: Agree with Proposal 1.

      - ZTE: Proposal 2 is a reasonable compromise for us

      - QC: Proposal 1a is a better compromise

      - Nokia: For Proposal 1a what is the UE behavior when the SCell activation delay exceeds some time (e.g., equivalent or comparable to the longest possible value of sCellDeactivationTimer)?

      - E///: suggest Proposal 1 + UE behavior is changed and it is FFS ; Proposal 1 + UE behavior is unchanged

- Agreements

- Applicability of SCell activation requirements when sCellDeactivationTimer is NOT configured

        - Option 1 (E///, QC, Apple, Huawei/HiSilicon):

          - The SCell activation requirements for NR-U do not apply when the sCellDeactivationTimer is not configured, when the SCell activation delay exceeds some pre-defined time (e.g., equivalent or comparable to the longest possible value of sCellDeactivationTimer).

        - Option 2 (Nokia, ZTE, MTK):

          - SCell activation delay requirements are applicable in Scenario A (CA with NR PCell and NR SCell) with any LBT type and in Scenario B and C (E-UTRAN-NR-U DC/SA NR-U) with LBT type 2C. Requirements are also applicable in all scenarios, if the UE does not experience any UL LBT failures during SCell activation/deactivation.

          - SCell activation delay requirements are applicable when sCellDeactivationTimer is not configured also in Scenarios B and C (EN-DC and SA) LBT types other than 2C.

        - Option 3 (possible compromise solution):

          - SCell activation delay requirements are applicable in Scenario A (CA with NR PCell and NR SCell) with any LBT type and in Scenario B and C (E-UTRAN-NR-U DC/SA NR-U) with LBT type 2C. Requirements are also applicable in all scenarios, if the UE does not experience any UL LBT failures during SCell activation/deactivation.

          - For all other scenarios the SCell activation requirements for NR-U do not apply when the sCellDeactivationTimer is not configured, when the SCell activation delay exceeds some pre-defined time (e.g., equivalent or comparable to the longest possible value of sCellDeactivationTimer).

1st round email discussion conclusions

Decisions

Issue 2-1-1: General specification structure.

Agreement: The specification structure for clause 6.2.2A shall follow the structure of clause 6.2.2, but unnecessary sections can be omitted (do not use void for this purpose).

Issue 6-1-1: Semi-persistent L1-RSRP measurement reporting

Agreement: Capture the following sentence in TS38.133 9.5A.3.2:

When CCA is used on target frequency, the UE shall stop semi-persistent L1-RSRP measurement reports on PUCCH, when the UE cannot transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command.

Issue 7-2-1: CSSF outside measurement gaps for carrier frequency with CCA

Agreement: CSSF outside gaps (CSSFoutside_gap,i ) should increase by one if one MO configured both RMTC (for RSSI measurement) and SMTC (for SSB-based measurement)

Issue 7-2-2: CSSF within measurement gaps for carrier frequency with CCA

Agreement

  - CSSF within measurement gaps (CSSFwithin_gap,i ) needs also to be adapted to account for inter-frequency RSSI/CO measurements and intra-frequency RSSI/CO measurements with gaps

  - Regarding the CSSF within measurement gaps (CSSFwithin_gap,i ), a MO should be counted twice, if the MO is configured with both RMTC and SMTC which are candidates to be measured in gap j where the measurement object i is also a candidate.

Issue 10-1-1: RRM requirements for PL-RS switch in R16 NR-U

Agreement: Do not include RRM requirements for PL-RS switching in Rel-16 NR-U

Issue 10-2-1: RRM requirements for CGI reading in R16 NR-U

Agreement: Do not include RRM requirements for the Rel-16 CGI reading feature in Rel-16 NR-U.

================================================================================

R4-2103512	WF on NR-U RRM core

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was approved.

##### 7.1.5.1	General [NR_unlic-Core]

R4-2100769	Discussion on terminology for NR-U RRM requirements

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2101636	Discussion on terminology updates for NR-U

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2102518	On SSB availability to meet NR-U requirements in DRX

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On SSB availability to meet NR-U requirements in DRX

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2102519	Terminology updates for NR-U in 38.133

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1726  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

Terminology updates for NR-U in 38.133

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was revised to R4-2103514.

R4-2103514	Terminology updates for NR-U in 38.133

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1726  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2102519)

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

R4-2102520	Terminology updates for NR-U in 38.133

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1727  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Terminology updates for NR-U in 38.133

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

R4-2102521	Terminology updates for NR-U in 36.133

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7045  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

Terminology updates for NR-U in 36.133

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was revised to R4-2103515.

R4-2103515	Terminology updates for NR-U in 36.133

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7045  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2102521)

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

R4-2102522	Terminology updates for NR-U in 36.133

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7046  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Terminology updates for NR-U in 36.133

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

##### 7.1.5.2	RRC connection mobility control [NR_unlic-Core]

R4-2100050	Random access in NR-U

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2100051	[CR] Add Random Access requirements under NR-U

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1415  Cat: B (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was merged.

R4-2101100	[CR] Add Random Access requirements under NR-U (Cat A)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1557  Cat: A (Rel-17)

					Source: ZTE Corporation

Abstract:

Cat A CR corresponding to R4-2100051.

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was withdrawn.

R4-2101424	Remaining open issues on connection mobility control in NR-U

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the remaining open issues on the connection mobility control in NR-U,  that is,  random access procedure and maximum SI acquisition time.

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2101425	CR: Introduction of random access requirements with CCA

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1601  Cat: B (Rel-16)

					Source: Ericsson

Abstract:

This CR provides random access procedure with CCA.

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was revised to R4-2103721.

R4-2103721	CR: Introduction of random access requirements with CCA

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1601  rev 1 Cat: B (Rel-16)

					Source: Ericsson

(Replaces R4-2101425)

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

R4-2101426	CR: Introduction of random access requirements with CCA

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1602  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

This CR provides random access procedure with CCA.

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

R4-2101637	Discussion on remaining issues for random access for NR-U

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2102920	Random Access requirements in NR-U

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

In this paper, we discuss  Random Access requirements in NR-U

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

##### 7.1.5.3	SCell activation/deactivation (delay and interruption) [NR_unlic-Core]

R4-2100065	Remaining issues in SCell activation under NR-U

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2101130	SCell (de)activation requirement applicability when sCellDeactivationTimer is not configured

Type: discussion		For: Discussion
					Source: Nokia

Abstract:

Discussion and LS text proposal about requirement applicability when sCellDeactivationTimer is not configured.

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2101638	CR on requirement maintenance for SCell activation and deactivation for NR-U

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1628  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was merged.

R4-2101639	CR on requirement maintenance for SCell activation and deactivation for NR-U

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1629  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was withdrawn.

R4-2101640	Discussion on remaining issues for SCell activation and deactivation for NR-U

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2102515	On remaining issues for SCell activation in NR-U

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On remaining issues for SCell activation in NR-U

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2102516	Updates in SCell activation in NR-U

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1724  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

Updates in SCell activation in NR-U

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was merged.

R4-2102517	Updates in SCell activation in NR-U

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1725  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Updates in SCell activation in NR-U

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was withdrawn.

R4-2102919	Interruptions during SCell activation in NR-U

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

In this paper, we discuss  remaining open issues interruptions during Scell activation in NR-U

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2102922	CR on Interruptions during Scell activation in NR-U

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1791  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Abstract:

The CR updates clause 8.3A based on agreements related to interruptions during Scell activation requirements.

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was revised to R4-2103516.

R4-2103516	CR on Interruptions during Scell activation in NR-U

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1791  rev 1 Cat: F (Rel-16)

					Source: Qualcomm Incorporated

(Replaces R4-2102922)

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

R4-2102923	CR on Interruptions during Scell activation in NR-U

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1792  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

The CR updates clause 8.3A based on agreements related to interruptions during Scell activation requirements.

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

##### 7.1.5.4	Active TCI state switching [NR_unlic-Core]

R4-2102720	CR on Active TCI state switching for NR-U

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1743  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

R4-2102721	CR on Active TCI state switching for NR-U (cat A)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1744  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

##### 7.1.5.5	RLM [NR_unlic-Core]

R4-2102512	On remaining issues for RLM in NR-U

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On remaining issues for RLM in NR-U

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2102513	Updates in RLM requirements for NR-U

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1722  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

Updates in RLM requirements for NR-U

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was revised to R4-2104047.

R4-2104047	Updates in RLM requirements for NR-U

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1722  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2102513)

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

R4-2102514	Updates in RLM requirements for NR-U

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1723  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Updates in RLM requirements for NR-U

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

##### 7.1.5.6	Beam management [NR_unlic-Core]

R4-2100063	[CR] UE behavior when cannot transmit ACK due to LBT failure for MAC-CE deactivation

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1419  Cat: B (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was withdrawn.

R4-2100064	[CR] UE behavior when cannot transmit ACK due to LBT failure for MAC-CE deactivation (Cat A)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1420  Cat: A (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was withdrawn.

R4-2100066	[CR] UE behavior when cannot transmit ACK due to LBT failure for MAC-CE deactivation

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1421  Cat: F (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was merged.

R4-2101427	Remaining open issues on beam management in NR-U

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the remaining open issues on beam management in NR-U.

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2101428	CR: Beam management requirements with CCA

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1603  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

This CR updates the specification of BFD/CBD and L1-RSRP reporting with CCA

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was revised to R4-2103513.

R4-2103513	CR: Beam management requirements with CCA

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1603  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2101428)

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

R4-2101429	CR: Beam management requirements with CCA

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1604  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

This CR updates the specification of BFD/CBD and L1-RSRP reporting with CCA

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

##### 7.1.5.7	Measurement requirements [NR_unlic-Core]

R4-2100191	On RSSI measurement in NR-U

Type: discussion		For: Discussion
					38.133 v..
					Source: Apple

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2100510	Remaining issues on intra and inter frequency measurements under NR-U

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2100770	Discussion on measurement requirements for NR-U

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2101641	CR on measurement requirements for NR-U

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1630  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

R4-2101642	CR on measurement requirements for NR-U

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1631  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

R4-2101643	Discussion on measurement requirements for NR-U

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2102526	RSSI measurement bandwidth

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

RSSI measurement bandwidth

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was endorsed.

##### 7.1.5.8	Measurement capability and reporting criteria [NR_unlic-Core]

R4-2102238	Maintenance CR for NR-U core requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1692  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

Maintenance CR including changes to the core NR-U requirements.

Discussion:

Session chair: moved from AI 7.1.5.1. See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

R4-2102237	Maintenance CR for NR-U core requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1691  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Maintenance CR including changes to the core NR-U requirements.

Discussion:

Session chair: moved from AI 7.1.5.1. See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

##### 7.1.5.9	Timing [NR_unlic-Core]

R4-2100062	Discussions on UE transmit timing and reference cell under NR-U

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2100188	On reference cell availability for NR-U

Type: discussion		For: Discussion
					38.133 v..
					Source: Apple

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2100189	CR on reference cell availability for NR-U R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1451  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was postponed.

R4-2100190	CR on reference cell availability for NR-U R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1452  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was withdrawn.

R4-2100771	Discussion on timing requirements for NR-U

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2101644	CR on timing requirements for NR-U

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1632  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was postponed.

R4-2101645	CR on timing requirements for NR-U

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1633  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was withdrawn.

R4-2101646	Discussion on timing requirements for NR-U

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2102646	On UE transmit timing under DL LBT failure in reference cell

Type: other		For: Discussion
					Source: Ericsson

Abstract:

The paper discusses open issues on timing

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

##### 7.1.5.10	Other requirements  [NR_unlic-Core]

R4-2101014	On PL-RS switch and CGI reading for NR-U

Type: discussion		For: Discussion
					38.133 v..
					Source: Apple

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2101131	Random access requirements in NR-U

Type: discussion		For: Discussion
					Source: Nokia

Abstract:

Discussion about random access requirements for 2-step and 4-step RACH for NR-U.

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2101132	CR to 38.133 - Introducing NR-U random access requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1558  Cat: B (Rel-16)

					Source: Nokia

Abstract:

CR to introduce RA requirements for 2-step and 4-step RACH.

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Session chair: Cover sheet errors (Is the reserved Tdoc number R4-2101132 correctly spelled on the cover page header? What is the current version? It reads 17.0.0 on the cover page but the Tdoc is reserved for version 16.6.0. What is the CR number? It reads xxxx on the cover page but the Tdoc is reserved for CR number 1558.)

Session chair: tdoc will be postponed due to CR cover sheet issues.

Decision: 		The document was postponed.

R4-2102724	CR on PSCell Addition requirements for NR-U

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1747  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Session chair: Cover sheet errors (What is the impacted specification? It reads 36.133 on the cover page but the Tdoc is reserved for 38.133. What is the current version? It reads 16.8.0 on the cover page but the Tdoc is reserved for version 16.6.0.)

Session chair: tdoc will be postponed due to CR cover sheet issues.

Decision: 		The document was postponed.

R4-2102725	CR on PSCell Addition requirements for NR-U (cat A)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1748  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was withdrawn.

R4-2102823	CR to 38.133 - Introducing NR-U random access requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1774  Cat: A (Rel-17)

					Source: Nokia

Discussion:

See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was withdrawn.

R4-2102641	Analysis of impact of RA with CCA on RRM requirements in NR-U

Type: other		For: Discussion
					Source: Ericsson

Abstract:

The paper discusses impact on NR-U requirements when 2-step and 4-step RA subject to CCA are used

Discussion:

Session chair: moved from AI 7.1.5.2. See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was noted.

R4-2102642	Applicability of RA with CCA on RRM requirements in NR-U in 38.133

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1738  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

The CR on applicability of 2-step and 4-step RA subject to CCA to NR-U RRM requirements in 38.133

Discussion:

Session chair: moved from AI 7.1.5.2. See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

R4-2102643	Applicability of RA with CCA on RRM requirements in NR-U in 38.133

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1739  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

The CR on applicability of 2-step and 4-step RA subject to CCA to NR-U RRM requirements in 38.133

Discussion:

Session chair: moved from AI 7.1.5.2. See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

R4-2102644	Applicability of RA with CCA on RRM requirements in NR-U in 36.133

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7049  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

The CR on applicability of 2-step and 4-step RA subject to CCA to NR-U RRM requirements in 36.133

Discussion:

Session chair: moved from AI 7.1.5.2. See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

R4-2102645	Applicability of RA with CCA on RRM requirements in NR-U in 36.133

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7050  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

The CR on applicability of 2-step and 4-step RA subject to CCA to NR-U RRM requirements in 36.133

Discussion:

Session chair: moved from AI 7.1.5.2. See email discussion summary for [98e][205] NR_unlic_RRM_1 in R4-2103687.

Decision: 		The document was agreed.

#### 7.1.6	RRM perf. requirements (38.133) [NR_unlic-Perf]

R4-2103445	Email discussion summary: [98e][206] NR_unlic_RRM_2

Type: other		For: discussion
					Source: Moderator (Nokia)

Discussion:

The contribution summarised email discussion thread [98e][206] NR_unlic_RRM_2. The discussion thread was moderated by Rafael Paiva (Nokia).

Decision: 		The document was revised to R4-2103688.

R4-2103688	Email discussion summary: [98e][206] NR_unlic_RRM_2

Type: other		For: discussion
					Source: Moderator (Nokia)

(Replaces R4-2103445)

Discussion:

The contribution summarised email discussion thread [98e][206] NR_unlic_RRM_2. The discussion thread was moderated by Rafael Paiva (Nokia).

Decision: 		The document was noted.

GTW session (January 26, 2021)

Issue 1-1-3: SCS for data and SSB

- Option 1: Configure the same SCS for data and SSB.

- Option 2: Allow configuration of different SCS for SSB and data.

Discussion:

MTK: Option 1 is more simple and typical case

E///: Option 1

Agreement: Configure the same SCS for data and SSB.

Issue 1-1-4: Cell configuration SCS and channel bandwidth on carrier frequency with CCA

- Option 1 (Huawei?):

  - 15 kHz SSB SCS with 20 MHz bandwidth

- Option 2 (Nokia):

  - NR with CCA 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode

  - NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode

- Option 3 (Ericsson, Mediatek, QC)

  - RAN4 define NR-U RRM test cases with SCS=30kHz for both SSB and data transmission.

  - FFS bandwidths

Discussion

MTK: Option 3. 40MHz BW.

QC, E///: Option 3 is ok.

Agreement

  - RAN4 define NR-U RRM test cases with SCS=30kHz for both SSB and data transmission and 40 MHz bandwidth.

Issue 1-2-1: Differentiation between FBE and LBE

- Option 1 (Ericsson): If needed, test parameter values for FBE and LBE (e.g., signaling-related) are specified in the same test case (a note to clarify their applicability can be added, if needed).

  - For PCCA, it is the actual value that matters, there is no need to call it “FBE” or “LBE”; if needed, multiple PCCA values can be specified in the same test.

- Option 2 (Qualcomm): Define separate test cases for LBE and FBE whenever an LBT failure dependent requirement is tested.

Discussion

QC: Ok with E/// proposal without sub-bullet

Agreement: If needed, test parameter values for FBE and LBE (e.g., signaling-related) are specified in the same test case (a note to clarify their applicability can be added, if needed).

Issue 1-2-2: General approach for DL LBT/CCA models

How should the DL LBT models be defined?

- Option 1: Describe LBT models as a random process defined

- Option 2 (Huawei): Consider the LBT model as a repetitive pattern of n available SSBs for every m SSB occasions.

- Option 3 (Ericsson): For NR-U, define a parameter for CCA success probability, PCCA, to model the probability of successful attempt for acquiring the channel and transmitting the necessary signals.

Discussion

Ericsson: There are two approaches – deterministic and probabilistic. For NR we suggest to adjust the existing LTE approach.

Huawei: For LAA we use probability model. In case of using probabilistic model, then we cannot guarantee a specific UE behavior for each individual test. Test equipment vendors feedback may be required.

E///: Probability-based model was already used in LTE. To guarantee failure we can set the failure probability to 100%.

MTK: we can have a special approach for special test cases.

Agreement:

DL LBT modelling procedures

Option 1: Probabilistic model. Define a parameter for CCA success probability, PCCA, to model the probability of successful attempt for acquiring the channel and transmitting the necessary signals.

Option 2: Deterministic LBT pattern with a repetitive pattern of n available SSBs for every m SSB occasions

Option 1 approach is used as a baseline approach. Option 2 can be used for selected test cases to guarantee proper UE behavior.

Issue 1-2-3: General approach for defining parameters of LBT models

How should the parameters of DL LBT models be defined?

- Option 1 (Ericsson): The probability parameter PCCA is not a single fixed value in the model but a variable; the value(s) are configured to a relevant setting in each test. The specific PCCA values should be defined among cell-specific test parameters in each test case (the access probability can be different at different BS locations). The CCA model only specifies possible values for PCCA. The possible values for PCCA may be defined as

  - 1): The CCA model specifies a continuous range of possible values (one or more specific values from the range are configured in each test), e.g., PCCA Î [0%, 100%], or

  - 2): The CCA model specifies a discrete set of possible values (one or more specific values from the set are chosen in each test).

- Option 2: Common parameters should be defined for all test cases.

Discussion

Apple: what does “the access probability can be different at different BS locations” mean?

E///: there may be some differentiation for PCell and SCell

MTK, QC, Nokia: we prefer to still have a limited subset of values

E///: agree

Agreement: Probabilistic DL LBT model

- The probability parameter PCCA is not a single fixed value in the model but a variable; the value(s) are configured to a relevant setting in each test.

- The specific PCCA values should be defined among cell-specific test parameters in each test case.

- The CCA model specifies a discrete set of possible values

  - One or more specific values from the set are chosen in each test

  - One value can be chosen as a default one and will apply to most of test cases

- The set of values

  - Option 1: {0%, 25%, 50%, 75%, 100%}

  - Other options are not precluded

Issue 1-3-1a: DL LBT model for LBE operation

- Option 1 (Qualcomm, Nokia): DL-LBE-Model 1:

  - For LBE test cases in non DRX: RAN4 to adopt the following DL LBT model: 1) Define a probability equal to P1 for the transmission of the DRS in the first candidate position. 2) In case of LBT failure for transmission in the first candidate position, define a probability equal to P2 for the transmission in the second candidate position for a given SSB index.

- Option 2 (Ericsson): DL-LBE-Model 2:

  - At least at a low Es/Iot (e.g., Es/Iot<-6 dB), the probability of CCA success is higher for the semi-static channel occupancy compared to that for dynamic channel occupancy:

    - PCCA,semi-static,i > PCCA,dynamic,I, when Es/Iot<X,

    - PCCA,semi-static,i = PCCA,dynamic,I, when Es/Iot≥X,

    - where X=TBD (e.g., X=-6 dB).

Discussion

Nokia: what are the benefits of Option 2?

QC:  we agree with E/// proposal that “the probability of CCA success is higher for the semi-static channel occupancy compared to that for dynamic channel occupancy”

E///: Option 2 is simpler since Option 1 has conditional probability

QC: for LBE we agreed to monitor 2 positions and we don’t need a single probability

HW: for Option 2, does it mean that we’ll transmit SSB only in the 1st candidate position with certain probability?

E///: For Option 1 the probability of the 2nd SSB transmission is conditional and it is more difficult to control it. In our model we have separate probabilities for the first and second conditions

Agreement:

DL LBT model for LBE and FBE operation:

1) Define a probability equal to P1 for the transmission of the DRS in the first candidate position.

2) In case of LBT failure for transmission in the first candidate position, define a probability equal to P2 for the transmission in the second candidate position for a given SSB index.

Different probabilities can be used for LBE and FBE operation

Note: in case significant issues with this model are identified then the model with independent probabilities for LBT failure can be considered

1st round email discussion conclusions

Decisions

Issue 1-1-2: Applicability of NR FDD test configurations.

Agreement: NR FDD test configurations do not apply to the configuration of NR-U cells, but may apply to the configuration of NR cells in NR-U test cases.

Issue 1-1-5: E-UTRA, NR and NR-U configurations

Agreements

Configuration for cells without CCA in NR-U test cases:

- NR cells without CCA:

  - NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode

  - NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode

  - NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode

- LTE cells without CCA:

  - LTE FDD

  - LTE TDD

Issue 1-1-6: PRACH test configuration

Agreements

- For handover and RRC re-establishment cases, RAN4 to assume PRACH configuration 1 and 2 as baseline for NR-U tests, as specified in Annex A.3.8.2 in TS 38.133.

- For the random access test case: RAN4 to discuss the PRACH configuration after the core requirements are defined.

- RAN4 to discuss defining a new test configuration with the new PRACH sequences introduced in NR Rel-16.

Issue 1-1-9: RMCs for PDSCH

Agreements

Define new RMC for PDSCH for slots with RMSI under CCA

- SCS=30kHz

- Reuse the same configuration as RMC for PDSCH for slots with RMSI (i.e., Type A, 24PRB, MCS4, dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1, etc.).

Issue 1-1-11: RMC transmission burst

Agreements

For NR-U RRM tests, RMC is transmitted during the RMC transmission burst:

- The length of the RMC transmission burst in slots is defined as N. The RMC burst transmission format is determined according to the steps below:

    - 1. Select N randomly from a given set of the number of slots S1 = {[1,3,5,8]} with equal probability as the total length of RMC burst transmission format.

    - 2. A uniform random variable from 0 to 1 is generated. If the random variable is less than PCCA_DL, a burst of N fully occupied slots is transmitted. Otherwise, the RMC transmission burst is muted and the muting duration is the same as the number N of slots for determined burst format.

- RMC transmission burst is scheduled outside DBT window.

- RAN4 discuss further the number of slots in S1.

Issue 1-1-12: TDD UL/DL configuration

Agreement: NR-U RRM tests does not configure tdd-UL-DL-ConfigurationCommon using RRC configuration. DL scheduling is configured by DCI 1_1 slot by slot

Issue 1-1-14: Antenna configurations

Agreement: Define new subclause for antenna configurations with unlicensed bands. For 4Rx UE, apply the same applicability rule as Rel-15 RRM test.

Issue 1-3-10: General approach in exceeding Lmax values during RRM tests

Agreement: For the test cases where no particular behaviour to be verified, exceeding Lmax shall be avoided

Issue 1-3-11: List of test cases in which exceeding Lmax values may be considered

Agreement:

- Consider having particular test cases to verify the correct UE behaviour for the following cases:

    - Initiating the measurements on neighbour upon exceeding Mp and Mq in Cell reselection

    - Initiate cell selection procedures for the selected PLMN upon L1 exceeding L1,max in RRC release with redirection

    - Report RSRP_0 upon L1 exceeding L1,max for L1-RSRP measurement

- For SCell activation in NR-U, exceeding Lmax should be avoided.

- For SFTD measurement NR-U, exceeding Lmax should be avoided.

- For intra-frequency and inter- frequency measurement for NR-U, exceeding LPSS/SSS,gaps,max should be avoided..

Issue 1-4-1: Need for an UL LBT model

Agreement:

- UL CCA model is needed for NR

- UL CCA model is not necessary in every test case, but where the requirement depends on UL CCA failures.

Issue 1-4-7: Consistent UL CCA failures

Agreement:

- Consistent UL CCA failures are modelled by means of a low PCCA_UL (e.g., 0%).

- FFS: List of test cases that need to model consistent UL CCA failures

Issue 2-1-4: Test cases on Random access

Agreement:

- Add the following test cases on the test case list for NR-U:

    - Random access to NR-U PCell

    - Random access to NR-U PSCell

- Define random access test cases for 4-step and 2-step RA in Rel-16.

Issue 2-1-6: Test cases on timing

Agreement:

- Add the following test cases on the test case list for NR-U:

    - Timing (timing advance) – NR-U PCell

    - Timing (timing advance) – NR-U PSCell

Issue 2-1-8: Test cases on PSCell addition/release delay

Agreement:

- Do not define the following test case for NR-U:

    - PSCell addition/release delay

      - NR-U PSCell with E-UTRA PCC, unknown

Issue 2-1-13b: Test cases for inter-RAT measurement procedure: RSSI and CO

Agreement:

- Add the following test cases on the test case list for NR-U:

    - Inter-RAT measurement procedure

      - E-UTRA-NR-U RSSI measurements requirements:

        - On NR-U neighbor, with E-UTRA (FDD,TDD) PCC

      - E-UTRA-NR-U CO measurements requirements:

        - On NR-U neighbor, with E-UTRA (FDD,TDD) PCC

Issue 2-1-16b: Test cases for accuracy for NR-U inter-RAT measurements: RSSI and CO

Agreement:

- Add the following test cases on the test case list for NR-U:

    - 1b. E-UTRA-NR-U RSSI measurement accuracy requirements:

      - On NR-U neighbor, with E-UTRA (FDD,TDD) PCC

    - 1c. E-UTRA-NR-U CO measurement accuracy requirements:

      - On NR-U neighbor, with E-UTRA (FDD,TDD) PCC

GTW session (February 05, 2021)

[98e][206] NR_unlic_RRM_2

Issue 1-2-5: DL LBT models parameter variation

- Proposals

  - Option 1: One probability value (per transmitter) applies at any time point during a test; one or more probability values can be configured in the entire test, one value PCCA,i per time interval Ti where i≥1 and the multiple time intervals (when i>1) do not overlap (e.g., PCCA=1.0 in T1 and PCCA=0.75 in T2).

  - Option 2: DL LBT model parameters are kept constant during a test.

  - Option 3 (new): As a baseline approach, DL LBT model parameters are kept constant during a test. In selected test cases parameters may be changed if necessity is identified as:

    - One probability value (per transmitter) applies at any time point during a test; one or more probability values can be configured in the entire test, one value PCCA,i per time interval Ti where i≥1 and the multiple time intervals (when i>1) do not overlap (e.g., PCCA=1.0 in T1 and PCCA=0.75 in T2).

- Discussion

  - E///: model has to be generic. Insist on Option 1.

  - QC: agree with Option 3.

  - MTK: Option 3 as a compromise. Need to see specific test case which requires Option 1.

  - Nokia: Option 3 is ok.

  - Apple: Option 3 is fine.

- Agreements

  - DL CCA model: One probability value (per TRP) applies at any time point during a test; one or more probability values can be configured in the entire test, one value PCCA,i per time interval Ti where i≥1 and the multiple time intervals (when i>1) do not overlap (e.g., PCCA=1.0 in T1 and PCCA=0.75 in T2).

  - As a baseline approach, DL CCA probability (PCCA_DL) is kept constant during a test.

  - In selected test cases DL CCA probability (PCCA_DL) parameters may be changed for different time intervals.

Issue 1-2-6: UL LBT models parameter variation

- Proposals

  - Option 1 (Ericsson): For UL CCA, the modelling approach is based on a probability PCCA_UL,i of successful access during the corresponding time Ti of the time interval i.

  - Option 2: UL LBT model should be kept constant during a test.

  - Option 3 (new): As a baseline approach, UL LBT model parameters are kept constant during a test. In selected test cases parameters may be changed if necessity is identified as:

    - For UL CCA, the modelling approach is based on a probability PCCA_UL,i of successful access during the corresponding time Ti of the time interval i.

- Discussion

  - R&S: is it for test case or for simulations?

    - E///: for test cases

  - R&S: it will take some efforts to implement such model.

- Agreements

  - UL CCA model: One probability value applies at any time point during a test; one or more probability values can be configured in the entire test, one value PCCA,i per time interval Ti where i≥1 and the multiple time intervals (when i>1) do not overlap (e.g., PCCA=1.0 in T1 and PCCA=0.75 in T2).

  - As a baseline approach, UL CCA probability (PCCA_UL) is kept constant during a test.

  - In selected test cases UL CCA probability (PCCA_UL) parameters may be changed for different time intervals.

Issue 1-4-3: UL LBT model

- Proposals

  - Option 1 (new part in bold text): baseline UL LBT model as:

    - Use DL FBE model to transmit a full band/LBT BW OCNG noise pattern in one or more of the scheduled/configured UL resource with probability P.

      - P is FFS

    - The test equipment keeps a count of the number of UL LBT failures it may cause.

    - When the OCNG signal is transmitted, the test equipment does not monitor the UL resource in which the OCNG is transmitted.

    - When the OCNG signal is not transmitted, the test equipment monitors the UL resource for the desired UL signal.

    - Based on whether it receives the signal or not, the test equipment declares the test case pass/fail

    - Consistent UL CCA failures are modelled by means of a low LBT success probability.

  - Option 2: basic principles:

    - For UL CCA, the modelling approach is based on a probability PCCA_UL,i of successful access during the corresponding time Ti of the time interval i.

    - Prior to each UL transmission burst within a time interval i of the test:

      - Generate a uniform random variable p from the range [0, 1].

      - If p<PCCA_UL,i, then the energy generated by the test system in the corresponding portion of UL slot is equal to or below the energy detection threshold [TBD]; otherwise the energy generated by the test system in the portion of UL slot is above the energy detection threshold [TBD].

    - Consistent UL CCA failures are modelled by means of a low PCCA_UL,i (e.g., 0%) during the relevant time interval Ti within the test.

    - In the same time interval i, PCCA_UL,I and PCCA_DL,i can have different values.

- Discussion

  - E///: detailed steps of the model may need further check

  - QC: DL and UL models may be different

R4-2103517	WF on LBT models for NR-U RRM performance requirements

Type: other		For: discussion
					Source: Qualcomm

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2104088.

R4-2104088	WF on LBT models for NR-U RRM performance requirements

Type: other		For: discussion
					Source: Qualcomm

(Replaces R4-2103517)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was approved.

R4-2103518	WF on general test configurations for NR-U RRM performance requirements

Type: other		For: discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was approved.

R4-2103519	WF on NR-U RRM performance test case list

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was withdrawn.

R4-2103520	WF on test configurations for NR-U RRM performance requirements

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was approved.

##### 7.1.6.1	General [NR_unlic-Perf]

R4-2101647	Discussion on RRM performance requirements for NR-U

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2101648	Discussion on RRM test configurations for NR-U

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2102523	Draft Big CR: Introduction of Rel-16 NR-U RRM performance requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

Draft Big CR: Introduction of Rel-16 NR-U RRM performance requirements

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2103532.

R4-2103532	Draft Big CR: Introduction of Rel-16 NR-U RRM performance requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2102523)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2102524	Updated test case list for NR-U

Type: other		For: Approval
					Source: Ericsson

Abstract:

Updated test case list for NR-U

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2103523.

R4-2103523	Updated test case list for NR-U

Type: other		For: Approval
					Source: Ericsson

(Replaces R4-2102524)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was approved.

R4-2102525	NR-U test cases structure

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

NR-U test cases structure

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2103533.

R4-2103533	NR-U test cases structure

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2102525)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was endorsed.

R4-2102921	Performance requirements in NR-U

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

In this paper, we discuss the various topics on performance requirements in NR-U

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

##### 7.1.6.2	Common RRM test configuration [NR_unlic-Perf]

R4-2100772	Discussion on general test setting for NR-U test cases

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2100834	Configurations for NR-U RRM test cases

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2101133	Discussion on NR-U RRM test configurations

Type: discussion		For: Discussion
					Source: Nokia

Abstract:

Discussion about  LBT model and other details of test configurations for NR-U.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2101430	Common test parameters for NR-U RRM tests

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the RMC used for NR-U RRM test cases.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2101431	Draft CR: RMC for NR-U test cases

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

This draft CR define RMCs used for NR-U RRM test cases.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2103521.

R4-2103521	Draft CR: RMC for NR-U test cases

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2101431)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was endorsed.

##### 7.1.6.3	Test cases [NR_unlic-Perf]

###### 7.1.6.3.1	General [NR_unlic-Perf]

R4-2100773	Discussion on RRM test cases in NR-U

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2100833	Scope of test cases for NR-U

Type: discussion		For: Discussion
					Source: ZTE Corporation

Abstract:

This paper discusses the planned scope of NR-U test cases with respect to the potential updates in core requirements.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2101134	NR-U RRM test case list updates

Type: discussion		For: Discussion
					Source: Nokia

Abstract:

Proposal for updates in the NR-U test case list.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2102527	On CCA model in NR-U test cases

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On CCA model in NR-U test cases

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2102528	CCA model in NR-U test cases

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

CCA model in NR-U test cases

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2103522.

R4-2103522	CCA model in NR-U test cases

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2102528)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was endorsed.

###### 7.1.6.3.2	RRC IDLE, cell re-selection [NR_unlic-Perf]

R4-2100839	On test cases for cell reselection under NR-U

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2102243	Introduction of NR-U cell reselection tests

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1693  Cat: B (Rel-16)

					Source: Ericsson

Abstract:

This CR introduces cell reselection test cases for NR-U.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2103531.

R4-2103531	Introduction of NR-U cell reselection tests

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1693  rev 1 Cat: B (Rel-16)

					Source: Ericsson

(Replaces R4-2102243)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2104078.

R4-2104078	Introduction of NR-U cell reselection tests

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1693  rev 2 Cat: B (Rel-16)

					Source: Ericsson

(Replaces R4-2103531)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was endorsed.

R4-2102244	Discussions on cell reselection test cases for NR-U

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we provide views on testing of the reselection requirements for the agreed  test cases.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

###### 7.1.6.3.3	HO delay and interruptions [NR_unlic-Perf]

R4-2100840	On test cases for handover under NR-U

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2101135	Draft TC E-UTRAN - NR-U Handover

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia

Abstract:

First draft pf test case for E-UTRAN - NR-U handover.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was postponed.

R4-2101649	Draft CR of test cases for HO delay and interruption for NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2104060.

R4-2104060	Draft CR of test cases for HO delay and interruption for NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2101649)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was endorsed.

R4-2102242	Discussions on handover test cases for NR-U

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we discuss testing of handover requirements for NR-U.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

###### 7.1.6.3.4	RRC Re-establishment [NR_unlic-Perf]

R4-2101136	Draft TC RRC re-establishment with CCA

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia

Abstract:

First draft of test cases for RRC re-establishment in NR-U.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was postponed.

R4-2102647	RRC re-establishment tests for NR-U

Type: other		For: Discussion
					Source: Ericsson

Abstract:

The paper describes test case on RRC re-establishment in NR-U

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

###### 7.1.6.3.5	RRC Connection Release with Redirection [NR_unlic-Perf]

R4-2100842	Test cases for RRC Connection Release with Redirection in NR-U

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2101650	Draft CR of test cases for RRC release with redirection for NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2104061.

R4-2104061	Draft CR of test cases for RRC release with redirection for NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2101650)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was endorsed.

R4-2102648	RRC connetion release with re-direction tests for NR-U

Type: other		For: Discussion
					Source: Ericsson

Abstract:

The paper describes test case on RRC re-direction in NR-U

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

###### 7.1.6.3.6	Timing (transmit timing and TA)  [NR_unlic-Perf]

R4-2100774	Introduction of test cases for UE transmit timing requirements with CCA

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was postponed.

R4-2100843	Test cases for timing in NR-U

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2102649	Analysis of UE timing tests for NR-U

Type: other		For: Discussion
					Source: Ericsson

Abstract:

The paper describes test cases on UE transmit timing and UE timing advance in NR-U

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2102650	UE timing tests for NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

The CR on test cases on UE transmit timing and UE timing advance in NR-U

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2103529.

R4-2103529	UE timing tests for NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2102650)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2104079.

R4-2104079	UE timing tests for NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2103529)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was endorsed.

###### 7.1.6.3.7	BWP switching delay and interruptions [NR_unlic-Perf]

R4-2100841	On test cases for BWP switching delay and interruptions under NR-U

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2102651	Analysis of test cases on BWP switching with consistent UL LBT failures

Type: other		For: Discussion
					Source: Ericsson

Abstract:

The paper describes test cases on BWP swiching with consistent UL LBT failures in NR-U

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2102652	Test cases on BWP switching with consistent UL LBT failures

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

The CR on test cases on BWP swiching with consistent UL LBT failures in NR-U

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2103530.

R4-2103530	Test cases on BWP switching with consistent UL LBT failures

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2102652)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2104080.

R4-2104080	Test cases on BWP switching with consistent UL LBT failures

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2103530)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was endorsed.

###### 7.1.6.3.8	PSCell addition/release (delay and interruption)  [NR_unlic-Perf]

R4-2100838	Test cases for PSCell addition and release

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2101651	Draft CR of test cases for PSCell addition and release for NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2104062.

R4-2104062	Draft CR of test cases for PSCell addition and release for NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2101651)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was endorsed.

R4-2102370	On PSCell addition release and TCI state activation TCs

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on TCs for PSCell addition/release and for TCI state activation.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

###### 7.1.6.3.9	Interruptions [NR_unlic-Perf]

R4-2102368	On SCell interruptions and SCell (de)activation test cases

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on interruption and SCell (de)activation test cases for NR-U

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2102369	DraftCR Introduction of NR-U SCell interruption and SCell (de)activation tests

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

Interruptions and (de)activation test cases for SCells in NR-U

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2103525.

R4-2103525	DraftCR Introduction of NR-U SCell interruption and SCell (de)activation tests

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2102369)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was endorsed.

###### 7.1.6.3.10	RLM [NR_unlic-Perf]

R4-2102529	On RLM test cases

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On RLM test cases

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2102530	RLM test cases

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

RLM test cases

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2103527.

R4-2103527	RLM test cases

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2102530)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was endorsed.

###### 7.1.6.3.11	Beam management [NR_unlic-Perf]

R4-2101432	Test cases on link recovery and L1-RSRP reporting in NR-U

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the test cases for beam failure recovery and L1-RSRP reporting in NR-U.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2101433	Draft CR: test cases for beam management in NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

This draft CR introduces the test cases for bean failure recovery and L1-RSRP reporting in NR-U.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2103524.

R4-2103524	Draft CR: test cases for beam management in NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2101433)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2104081.

R4-2104081	Draft CR: test cases for beam management in NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2103524)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was endorsed.

###### 7.1.6.3.12	Intra-frequency, inter-frequency and inter-RAT measurement requirements [NR_unlic-Perf]

R4-2100775	Introduction of test cases for  Accuracy for NR-U inter-frequency SS-RSRP measurements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was postponed.

R4-2100776	Introduction of test cases for L1-RSRP measurement accuracy with CCA serving cell

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was postponed.

R4-2100836	Test cases for intra and inter frequency and inter-RAT measurement requirements

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2101137	Draft TC NR-U inter-frequency measurements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia

Abstract:

First draft of test cases for  NR-U inter-frequency measurements.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was postponed.

R4-2101652	Draft CR of test cases for inter-RAT measurement for NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2104063.

R4-2104063	Draft CR of test cases for inter-RAT measurement for NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2101652)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was endorsed.

R4-2102531	On NR-U measurements test cases

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On NR-U measurements test cases

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2102532	NR-U RRM, SFTD, RSSI, and CO measurements test cases

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

NR-U measurements test cases

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2103528.

R4-2103528	NR-U RRM, SFTD, RSSI, and CO measurements test cases

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2102532)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was endorsed.

###### 7.1.6.3.13	Accuracy requirements for NR-U intra-frequency, inter-frequency and inter-RAT measurements [NR_unlic-Perf]

R4-2100837	Test cases for intra and inter frequency and inter-RAT measurement accuracy

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2101015	TCs for RSSI and CO measurement accuracy in NR-U R16

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Apple

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was postponed.

R4-2101653	Draft CR of test cases for intra-frequency measurement accuracy for NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2104064.

R4-2104064	Draft CR of test cases for intra-frequency measurement accuracy for NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2101653)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2104077.

R4-2104077	Draft CR of test cases for intra-frequency measurement accuracy for NR-U

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2104064)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was endorsed.

R4-2102371	On TC for NR-U Inter-RAT SFTD accuracy

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on test case for E-UTRAN - NR inter-RAT SFTD accuracy.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was noted.

R4-2102372	DraftCR 38.133 NR-U Inter-RAT SFTD accuracy TC

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

Test case for E-UTRAN - NR inter-RAT SFTD accuracy.

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was revised to R4-2103526.

R4-2103526	DraftCR 38.133 NR-U Inter-RAT SFTD accuracy TC

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2102372)

Discussion:

See email discussion summary for [98e][206] NR_unlic_RRM_2 in R4-2103688.

Decision: 		The document was endorsed.

#### 7.1.7	Demodulation and CSI requirements (38.101-4/38.104) [NR_unlic-Perf]

##### 7.1.7.1	General [NR_unlic-Perf]

R4-2103755	Email discussion summary for [98e][316] NR_unlic_Demod_UE

Type: other		For: discussion
					Source: Moderator (Qualcomm)

Discussion:

The contribution summarised email discussion thread [98e][316] NR_unlic_Demod_UE. The discussion thread was moderated by Pierpaolo Vallese (Qualcomm).

Decision: 		The document was revised to R4-2103934.

R4-2103934	Email discussion summary for [98e][316] NR_unlic_Demod_UE

Type: other		For: discussion
					Source: Moderator (Qualcomm)

(Replaces R4-2103755)

Discussion:

The contribution summarised email discussion thread [98e][316] NR_unlic_Demod_UE. The discussion thread was moderated by Pierpaolo Vallese (Qualcomm).

Decision: 		The document was noted.

R4-2103805	Way Forward on NR-U UE demodulation requirements

Type: other		For: discussion
					Source: QUALCOMM

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103934.

Decision: 		The document was revised to R4-2103987.

R4-2103987	Way Forward on NR-U UE demodulation requirements

Type: other		For: discussion
					Source: QUALCOMM

(Replaces R4-2103805)

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103934.

Decision: 		The document was approved.

------------------------------GTW Notes for Jan.27th email thread [316] (45 minutes) ----------------

List following treatment orders

General Test Scope (discussion can apply to both PDSCH and CQI)

Issue 3-1-1: Discuss how to define PDSCH requirements for UEs with no support for optional capabilities related to CSI-RS validation (DCI 2-0, csi-RS-Validation With-DCI);

- Proposals

  - Option 1: No applicable test cases for such UEs (Huawei)

  - Option 2: NO TRS

  - Option 3: UE validation

  - Option 4: Always TRS with LBT modelling modifications if needed (MTK,Apple, Qualcomm)

- Recommended WF

  - TBA

Discussion:

Apple: For UE not supporting these optional capabilities, we believe always on TRS is only feasible option.

UE validation is not RAN1 assumption which belongs to UE implementation issues.

No TRS will bring performance degradation.

Huawei: We share similar view as Apple. Not feasible for no TRS and UE validation.

MTK: According to RAN1 LS, UE not mandate to implement UE validation. We support always TRS for test set-up.

E///: We share same understanding with MTK for RAN1 LS. It’s better to check with RAN1 for SSB/TRS detection assumption.

Huawei: Question for option 4, how to employ LBT model with option 4.

QC: we can make exceptions for such specific UEs with LBT modelling.

Apple: Do we need to differentiate test set-up for two kinds of UEs or we can have one generic test set-up applicable for all types of UE?

Huawei: We agree to focus on this question first.

MTK: We have same concern as Apple and Huawei.

Question: Whether to have different test set-up/requirements according to UE capability of supporting CSI-validation features?

- Option 1: One generic LBT modelling for all test cases

  - Option 1a , no applicable test cases for UE which not support CSI-validation features

  - Option 1b: pack SSB and TRS in same discovery burst (check with UE assumption for implementation if any agreements in RAN1 for this case); the same requirements applied for both UE types

  - Option 1c: for UE which not CSI-validation feature, with LBT failure probability as 0. Separate requirements will be introduced.

- Option 2: Different test set-up/[performance requirements] for UE with different UE capability

  - Test set1: LBT modelling which applied for UE support CSI validation feature

  - Test set2: Test set-up for UE not support CSI-validation feature

    - FFS how to enable TRS always on considering LBT failure

Issue 3-1-2: Whether to define a requirement based on the optional capability ‘csi-RS-ValidationWith-DCI’:

- Proposals

  - Option 1: Yes, if Always TRS or No TRS are chosen the Issue 3-1-1 (Qualcomm);

- Recommended WF

  - TBA

Issue 3-1-3: Include PDCCH DCI 2-0 in PDSCH Simulations;

- Proposals

  - Option 1: No  (MediaTek, Ericsson, Huawei);

    - Option 1a: No, define PDSCH tests with DCI 2-0 with lower priority (Qualcomm);

  - Option 2: Yes (Apple);

- Recommended WF

  - TBA

Discussion:

QC: We can define requirements based on CSI-validation capability since UE which support DCI2-0, can also support

the optional capability ‘csi-RS-Validation With-DCI’.

Huawei: We share the view from QC.

MTK: We think both mechanism for CSI-validation. We think we need to first discuss the first issue.

--------------------------------End ------------------------------

R4-2103756	Email discussion summary for [98e][317] NR_unlic_Demod_BS

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][317] NR_unlic_Demod_BS. The discussion thread was moderated by Tricia Li (Huawei).

Decision: 		The document was revised to R4-2103933.

R4-2103933	Email discussion summary for [98e][317] NR_unlic_Demod_BS

Type: other		For: discussion
					Source: Moderator (Huawei)

(Replaces R4-2103756)

Discussion:

The contribution summarised email discussion thread [98e][317] NR_unlic_Demod_BS. The discussion thread was moderated by Tricia Li (Huawei).

Decision: 		The document was noted.

R4-2103806	Way forward on NR-U BS demodulation requirements for general part and PUSCH

Type: other		For: discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was revised to R4-2103988.

R4-2103988	Way forward on NR-U BS demodulation requirements for general part and PUSCH

Type: other		For: discussion
					Source: Huawei, HiSilicon

(Replaces R4-2103806)

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was approved.

R4-2103807	Way Forward on NR-U PUCCH demodulation requirements

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was revised to R4-2103989.

R4-2103989	Way Forward on NR-U PUCCH demodulation requirements

Type: other		For: discussion
					Source: Ericsson

(Replaces R4-2103807)

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was approved.

R4-2103808	Way forward on NR-U PRACH demodulation requirements

Type: other		For: discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was approved.

R4-2103809	Summary of simulation results for NR-U

Type: other		For: discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

------------------------------GTW Notes for Jan.27th email thread [317] (45 minutes) ----------------

Test set-up for PUSCH

Issue 2-1-1:  Bandwidth(s) for requirement definition and test applicability rules

- Proposals

  - Option 1: Define the requirements for single carrier with 20MHz and reuse Rel-15 applicability rule for different channel bandwidths. (Huawei, Samsung, Ericsson)

  - Option 2: Define PUSCH performance requirements for 20MHz, 40MHz, 60MHz and 80MHz and reuse Rel-15 applicability rule for different channel bandwidths. (Nokia)

- Recommended WF

Discussion:

Nokia: If we define with option 2, the same applicable rules can be reused, BS can only be verified by largest bandwidth which BS declared. In fact, the performance among CHBW is similar, but we still lost the test coverage for wider channel bandwidth with option1.

Samsung: For NR-U, 20MHz is basic channel bandwith. For option 1, we also cover test coverage with saving test effort. Similar as NR, we can placed 20MHz RB allocations in the centre of larger CHBW which solve Nokia concern. Larger CHBW only available wideband operation 2, 20MHz is basic CHBW apply for all options.

E///: Share similar view as Samsung.

Huawei: We understand the concern, there is similar situation in NR Rel-15 phase, it’s impossible to introduce requirements covering all the CHBW sets. Another approach is defining requirements as bandwidth agonistic.

Nokia: Does companies means to have 20MHz inside 40MHz CHBW?

Samsung: For 20MHz, 40MHz is both available for 15kHz and 30kHz.

Define the requirements for single carrier with 20MHz and reuse Rel-15 applicability rule for different channel bandwidths:

- Based on BS declaration to choose maximum CHBW BS supported, the RB allocation is as 20MHz within the BS test maximum supported CHBW. (most closed to channel centre following Channel arrangement)

- FFS for the detailed text proposal of test applicable rules

Issue 2-2-1: PUSCH mapping type and test applicability rules

- Proposals

  - Option 1: Only type B (Huawei, Samsung)

  - Option 2: Both type A and type B and reuse Rel-15 applicability rules.(Nokia, Ericsson)

- Recommended WF

Discussion:

Nokia: Type B typically more feasible for NR-U meanwhile type-A still can be used and no test effort based on BS declaration.

E///: The requirements should be based on BS declaration, we shouldn’t exclude both type A and type B . We agree type B more flexible. For FBE scenario, we also see the possibility of using type A.

Samsung: From test effort aspect, option 1 and option 2 no difference. In Rel-15 NR, we already cover both type A and type B. For NR-U, due to LBT, even with type A we can’t guarantee always available symbols in first slots. For LTE LAA, we have similar type as type B. Work load and simulation effort also need to be considered.

Huawei: All the companies have same observations that type B is more typical scenario. For comprise, we can accept with option with test applicable rules.

Samsung: For progress, we can comprise to option 2.

Agreements:

Both type A and type B and reuse Rel-15 applicability rules.

Issue 2-3-1: MCS

- Proposals

  - Option 1: MCS 2 and MCS 20 (Nokia)

  - Option 2: MCS 20 (Huawei, Ericsson, Samsung)

- Recommended WF

Discussion:

Nokia: There are different scenarios for NR-U deployment, for some scenarios with stand-alone, with cell edge, low MCS level would be possible case.

Samsung: The test purpose to verify interlace design with non-contiuous RB allocation. The test purpose already be covered by high MCS level.

E///: We already verify MCS 2 in Rel-15, we don’t need to duplicate the test effort for NR-U.

Huawei: We also think the verification of different modulation orders already covered by Rel-15 test cases, we need to focus the delta of specific feature introduced under this WI, MCS 20 enough.

Agreements: MCS 20.

Issue 2-4-1: RV sequence

- Proposals

  - Option 1: {0,2,3,1} (Nokia, Ericsson)

  - Option 2: {0,2,0,2} (Huawei, Samsung)

- Recommended WF

Discussion:

Nokia: option 2 used in eLAA, why option 2 used in eLAA ?

E///: similar question. These two options have similar performance. Apply option for type A. option 2 for type B.

Samsung: We can consider to keep two options.

Huawei: option 2 aims for multi-TB operation.

Agreement:

Using {0,2,3,1} for RV sequence

Issue 2-5-1: Whether to introduce the requirements for CG-UCI multiplexed on PUSCH with interlace allocation.

- Proposals

  - Option 1: No (Nokia, Samsung)

  - Option 2: Yes (Huawei, Ericsson)

    - Option 2a: Introduce requirement for HARQ-ACK multiplexed on interlacing PUSCH with more than 2 bits information, without CSI-1/2, and the test metric use BLER <=1%.(Ericsson)

    - Option 2b: Introduce performance requirements for CG-UCI multiplexed on PUSCH with interlaced resource allocation and without HARQ-ACK, CSI part 1 and CSI part 2 and use following Table as assumptions: (Huawei)

      - The test cases introduced based on BS declaration (Nokia)

- Recommended WF

Discussion:

Samsung: we believe mapping rules similar as existing NR performance requirements. Then only difference is interlacing we should focus on that.

Nokia: We are fine with option 1 or option 2b with BS declaration basis.

E///: We would more time check the assumption: “CSI cases can be covered the scenario for HARQ-ACK.”

Huawei: We would like to focus on NR-U scenario. CG-UCI would be more specific for NR-U,and what E/// proposed is out of this WI scope.

Issue 2-6-1: Whether to configure CG-UCI for PUSCH test.

- Proposals

  - Option 1: Configure CG-UCI for PUSCH test with interlace structure. (Huawei)

  - Option 2: Not configure CG-UCI for PUSCH test with interlace structure (Samsung, Nokia, Ericsson)

- Recommended WF

Discussion:

Huawei: Using CG-UCI more close to real NW configuration.

Samsung: In Rel-15, we didn’t configure UCI in PUSCH test cases. If test purpose is to verify interlace, we already coverd. For rate-matching can also verified in another test cases, no need to combine them together with a single test case.

Nokia: CG-UCI is not mandatory UE feature, no need to be configured with PUSCH test cases. If need to be verified, we prefer to have a separate dedicated test cases.

E///: Share similar view as Nokia and Samsung, we should not combine them together.

Huawei: We consider NR-U with LBT to improve the performance, CG-UCI together with PUSCH would be more useful case.

We are willing to comprise with respect of majority.

Agreements:  Not configure CG-UCI for PUSCH test with interlace structure.

--------------------------------End ------------------------------

R4-2100995	discussion on general issues in NR-U UE and CSI performance requirements

Type: other		For: Discussion
					Source: Ericsson

Abstract:

discussion on general issues in NR-U UE and CSI performance requirements

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103934.

Decision: 		The document was noted.

R4-2101343	Discussion on LBT transmission burst model

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103934.

Decision: 		The document was noted.

R4-2102082	Discussion on performance requirements for NR-U

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103934.

Decision: 		The document was noted.

##### 7.1.7.2	UE demodulation requirements [NR_unlic-Perf]

R4-2100197	Discussion on demodulation requirements for NR-U

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103934.

Decision: 		The document was noted.

R4-2100996	discussion on NR-U PDSCH demodulation

Type: other		For: Discussion
					Source: Ericsson

Abstract:

discussion on NR-U PDSCH demodulation

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103934.

Decision: 		The document was noted.

R4-2101264	Discussion on NR-U PDSCH requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103934.

Decision: 		The document was noted.

R4-2101344	Discussion on NRU UE performance requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103934.

Decision: 		The document was noted.

R4-2102585	Discussion on NR-U PDSCH Demodulation Performance Tests and related Simulation Assumptions

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

This paper will present our views on the points still under discussion related to NR-U PDSCH Demod Performance Tests and the proposed set of the simulation parameters. The simulation parameters are based on our company’s contribution to the previous meeti

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103934.

Decision: 		The document was noted.

##### 7.1.7.3	CSI requirements [NR_unlic-Perf]

R4-2100997	discussion on NR-U CQI report demodulation

Type: other		For: Discussion
					Source: Ericsson

Abstract:

discussion on NR-U CQI report demodulation

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103934.

Decision: 		The document was noted.

R4-2101345	Discussion on NRU CSI requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103934.

Decision: 		The document was noted.

R4-2102586	Discussion on NR-U CQI Performance Tests

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

This paper will present our views on the points under discussion for NR-U CQI Performance Tests.

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103934.

Decision: 		The document was noted.

##### 7.1.7.4	BS demodulation requirements [NR_unlic-Perf]

###### 7.1.7.4.1	General [NR_unlic-Perf]

R4-2100573	General issues on BS demodulation performance requirements for operation in unlicensed bands

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2100998	discussion on general issues in NR-U BS demodulation requirements

Type: other		For: Discussion
					Source: Ericsson

Abstract:

discussion on general issues in NR-U BS demodulation requirements

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2102270	General issues on BS demodulation performance requirements for operation in unlicensed bands

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was withdrawn.

###### 7.1.7.4.2	PUSCH requirements [NR_unlic-Perf]

R4-2100574	PUSCH Demodulation performance requirements for operation in unlicensed bands

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2100575	NR-U PUSCH simulation results

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2100919	View on PUSCH demodulation requirement for NR-U

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2100999	discussion on NR-U PUSCH demodulation assumptions

Type: other		For: Discussion
					Source: Ericsson

Abstract:

discussion on NR-U PUSCH demodulation assumptions

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2101000	simulation results on NR-U PUSCH demodulation

Type: other		For: Information
					Source: Ericsson

Abstract:

simulation results on NR-U PUSCH demodulation

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2101346	Simualtion results on NRU PUSCH performance requirements

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2101347	Discussion on NRU PUSCH performance requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2102271	PUSCH Demodulation performance requirements for operation in unlicensed bands

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2102272	NR-U PUSCH simulation results

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was withdrawn.

###### 7.1.7.4.3	PUCCH requirements [NR_unlic-Perf]

R4-2100576	PUCCH Demodulation performance requirements for operation in unlicensed bands

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2100577	NR-U PUCCH simulation results

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2100920	View on PUCCH demodulation requirement for NR-U

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2101001	discussion on NR-U PUCCH demodulation assumptions

Type: other		For: Discussion
					Source: Ericsson

Abstract:

discussion on NR-U PUCCH demodulation assumptions

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2101002	simulation results on NR-U PUCCH demodulation

Type: other		For: Information
					Source: Ericsson

Abstract:

simulation results on NR-U PUCCH demodulation

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2101348	Simualtion results on NRU PUCCH performance requirements

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2101349	Discussion on NRU PUCCH performance requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2102273	PUCCH Demodulation performance requirements for operation in unlicensed bands

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was withdrawn.

R4-2102274	NR-U PUCCH simulation results

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was withdrawn.

###### 7.1.7.4.4	PRACH requirements [NR_unlic-Perf]

R4-2100578	PRACH Demodulation performance requirements for operation in unlicensed bands

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2100579	NR-U PRACH simulation results

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2100921	View on PRACH demodulation requirement for NR-U

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2101003	discussion on NR-U PRACH demodulation assumptions

Type: other		For: Discussion
					Source: Ericsson

Abstract:

discussion on NR-U PRACH demodulation assumptions

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2101004	simulation results on NR-U PRACH demodulation

Type: other		For: Information
					Source: Ericsson

Abstract:

simulation results on NR-U PRACH demodulation

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2101265	Discussion on NR-U PRACH requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2101350	Simualtion results on NRU PRACH performance requirements

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2101351	Discussion on NRU PRACH performance requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was noted.

R4-2102275	PRACH Demodulation performance requirements for operation in unlicensed bands

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was withdrawn.

R4-2102276	NR-U PRACH simulation results

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][317] NR_unlic_Demod_BS in R4-2103933.

Decision: 		The document was withdrawn.

### 7.2	NR mobility enhancement  [NR_Mob_enh]

#### 7.2.1	RRM requirements maintenance (38.133) [NR_Mob_enh-Core/Perf]

R4-2103446	Email discussion summary: [98e][207] NR_Mob_enh_RRM

Type: other		For: discussion
					Source: Moderator (Apple)

Discussion:

The contribution summarised email discussion thread [98e][207] NR_Mob_enh_RRM. The discussion thread was moderated by Qiming Li (Intel).

Decision: 		The document was revised to R4-2103689.

R4-2103689	Email discussion summary: [98e][207] NR_Mob_enh_RRM

Type: other		For: discussion
					Source: Moderator (Apple)

(Replaces R4-2103446)

Discussion:

The contribution summarised email discussion thread [98e][207] NR_Mob_enh_RRM. The discussion thread was moderated by Qiming Li (Intel).

Decision: 		The document was noted.

R4-2103534	WF on NR mobility enhancement maintenance

Type: other		For: discussion
					Source: Apple

Discussion:

Apple: Issue 1-1: Further clarification on DL-to-UL and UL-to-DL switching time needs discussion

- Option 1: same as option 1 in R4-2017093: clarify that 13us switching time is allowed between source cell and target cell: (Apple, Huawei, QC, MTK)

- Option 2: Add conditions for not expected to transmit / not expected to receive covering both source and target cell. Autonomous interruption is allowed if these conditions are not met: (Ericsson, Nokia)

Huawei: Option 1

Nokia: Option 2

QC: Option 1 is spec compliant. Option 2 has impact on UE complexity.

MTK: Option 1. Option 2 may result in interruptions which shall be studied

E///: Option 2.

Session chair: Continue discussion. Agreement shall be made in RAN4 #98-bis-e. In case no consensus is reached, no requirements may be specified.

See email discussion summary for [98e][207] NR_Mob_enh_RRM in R4-2103689.

Decision: 		The document was approved.

R4-2101009	CR on maintenance for DAPS handover (R16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1540  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][207] NR_Mob_enh_RRM in R4-2103689.

Decision: 		The document was merged.

R4-2101010	CR on maintenance for DAPS handover (R17)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1541  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][207] NR_Mob_enh_RRM in R4-2103689.

Decision: 		The document was withdrawn.

R4-2101204	TDD UL-DL and DL-UL switching in DAPS handover

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Further clarification on DL-to-UL and UL-to-DL switching time

Discussion:

See email discussion summary for [98e][207] NR_Mob_enh_RRM in R4-2103689.

Decision: 		The document was noted.

R4-2101205	CR on TS38.133 for dual active protocol stack handover

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1575  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

Add conditions for not expected to transmit / not expected to receive covering both source and target cell. Add autonomous interruption allowance if these conditions are unspecified.Correct Ntx-rx and Nrx-tx to 25600 Tc

Discussion:

See email discussion summary for [98e][207] NR_Mob_enh_RRM in R4-2103689.

Decision: 		The document was merged.

R4-2101206	CR on TS38.133 for dual active protocol stack handover

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1576  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Add conditions for not expected to transmit / not expected to receive covering both source and target cell. Add autonomous interruption allowance if these conditions are unspecified.Correct Ntx-rx and Nrx-tx to 25600 Tc

Discussion:

See email discussion summary for [98e][207] NR_Mob_enh_RRM in R4-2103689.

Decision: 		The document was withdrawn.

R4-2101210	CR on TS38.133 for Pcell change

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1577  Cat: F (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][207] NR_Mob_enh_RRM in R4-2103689.

Decision: 		The document was revised to R4-2104050.

R4-2104050	CR on TS38.133 for Pcell change

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1577  rev 1 Cat: F (Rel-16)

					Source: MediaTek inc.

(Replaces R4-2101210)

Discussion:

See email discussion summary for [98e][207] NR_Mob_enh_RRM in R4-2103689.

Decision: 		The document was agreed.

R4-2101211	CR on TS38.133 for Pcell change

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1578  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][207] NR_Mob_enh_RRM in R4-2103689.

Decision: 		The document was agreed.

R4-2101667	Discussion on sync conditions for intra-band DAPS handover

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][207] NR_Mob_enh_RRM in R4-2103689.

Decision: 		The document was noted.

R4-2101668	CR on sync conditions for intra-band DAPS handover R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1640  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][207] NR_Mob_enh_RRM in R4-2103689.

Decision: 		The document was revised to R4-2103535.

R4-2103535	CR on sync conditions for intra-band DAPS handover R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1640  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101668)

Discussion:

See email discussion summary for [98e][207] NR_Mob_enh_RRM in R4-2103689.

Decision: 		The document was postponed.

R4-2101669	CR on sync conditions for intra-band DAPS handover R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1641  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][207] NR_Mob_enh_RRM in R4-2103689.

Decision: 		The document was withdrawn.

### 7.3	5G V2X with NR sidelink  [5G_V2X_NRSL]

R4-2103447	Email discussion summary: [98e][208] 5G_V2X_NRSL_RRM

Type: other		For: discussion
					Source: Moderator (LG Electronics)

Discussion:

The contribution summarised email discussion thread [98e][208] 5G_V2X_NRSL_RRM. The discussion thread was moderated by Yoonoh Yang (LG Electronics Inc.).

Decision: 		The document was revised to R4-2103690.

R4-2103690	Email discussion summary: [98e][208] 5G_V2X_NRSL_RRM

Type: other		For: discussion
					Source: Moderator (LG Electronics)

(Replaces R4-2103447)

Discussion:

The contribution summarised email discussion thread [98e][208] 5G_V2X_NRSL_RRM. The discussion thread was moderated by Yoonoh Yang (LG Electronics Inc.).

Decision: 		The document was noted.

#### 7.3.1	System parameters maintenance  [5G_V2X_NRSL-Core]

#### 7.3.2	UE RF requirements maintenance [5G_V2X_NRSL-Core]

R4-2102955	Email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF

Type: other		For: discussion
					Source: Moderator (LG Electronics)

Discussion:

The contribution summarised email discussion thread [98e][107] 5G_V2X_NRSL_UE_RF. The discussion thread was moderated by Suhwan Lim (LG Electronics France).

Decision: 		The document was revised to R4-2103297.

R4-2103297	Email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF

Type: other		For: discussion
					Source: Moderator (LG Electronics)

(Replaces R4-2102955)

Discussion:

The contribution summarised email discussion thread [98e][107] 5G_V2X_NRSL_UE_RF. The discussion thread was moderated by Suhwan Lim (LG Electronics France).

Decision: 		The document was noted.

R4-2103139	LS on the clarification on the name of parameters for 5G V2X UE

Type: LS out		For: Approval
					to -
					Source: Qualcomm

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was withdrawn.

##### 7.3.2.1	Transmitter characteristics  [5G_V2X_NRSL-Core]

R4-2100280	CR on editorial correction on V2X operation in TS38.101-1 in Rel-16

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0605  Cat: F (Rel-16)

					Source: LG Electronics France

Abstract:

Propovide CR to correct editorial correction for 5G V2X RF requirements in TS38.101-1

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was revised to R4-2103140.

R4-2103140	CR on editorial correction on V2X operation in TS38.101-1 in Rel-16

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0605  rev 1 Cat: F (Rel-16)

					Source: LG Electronics France

(Replaces R4-2100280)

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was agreed.

R4-2100281	CR on editorial correction on V2X operation in TS38.101-1 in Rel-17

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0606  Cat: A (Rel-17)

					Source: LG Electronics France

Abstract:

shadowing CR from R4-2100280

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was agreed.

R4-2100404	CR for TS 38.101-1, General corrections for NR V2X

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0615  Cat: F (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was not pursued.

R4-2100499	CR for TS 38.101-1, General corrections for NR V2X

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0618  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was withdrawn.

R4-2100405	CR for TS 38.101-3, General corrections for NR V2X

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0440  Cat: F (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was revised to R4-2103141.

R4-2103141	CR for TS 38.101-3, General corrections for NR V2X

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0440  rev 1 Cat: F (Rel-16)

					Source: CATT

(Replaces R4-2100405)

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was agreed.

R4-2100500	CR for TS 38.101-3, General corrections for NR V2X

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0445  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was agreed.

R4-2100406	CR for 38.886, General corrections for NR V2X

Type: CR		For: Agreement
					38.886 v16.2.0	  CR-0006  Cat: F (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was agreed.

R4-2102382	CR for 38.101-1 correction of NR V2X FRC parameter

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0685  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was not pursued.

##### 7.3.2.2	Receiver characteristics  [5G_V2X_NRSL-Core]

#### 7.3.3	Concurrent operation maintenance (scenarios, requirements, etc) [5G_V2X_NRSL-Core]

R4-2100403	Discussion on switching period position in ITS band

Type: discussion		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was noted.

##### 7.3.3.1	Transmitter characteristics  [5G_V2X_NRSL-Core]

R4-2100783	Further discussion on switching period in the ITS band

Type: discussion		For: Approval
					Source: vivo

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was noted.

R4-2101870	CR for TS 38.101-3 switching period for V2X con-current operation Rel-16

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0469  Cat: F (Rel-16)

					Source: Xiaomi

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was not pursued.

R4-2101871	CR for TS 38.101-3 switching period for V2X con-current operation Rel-17

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0470  Cat: A (Rel-17)

					Source: Xiaomi

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was withdrawn.

R4-2101876	on switching period

Type: discussion		For: (not specified)
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was noted.

R4-2102380	On SL switching period

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was noted.

R4-2102381	CR for TS 38.101-3 NR V2X switching period

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0482  Cat: B (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF in R4-2103297.

Decision: 		The document was not pursued.

##### 7.3.3.2	Receiver characteristics  [5G_V2X_NRSL-Core]

#### 7.3.4	RRM core requirements maintenance (38.133) [5G_V2X_NRSL-Core]

R4-2101054	CR on V2X interruption

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1551  Cat: B (Rel-16)

					Source: Mediatek Inc., LG Electronics, Qualcomm Inc.

Discussion:

See email discussion summary for [98e][208] 5G_V2X_NRSL_RRM in R4-2103690.

Decision: 		The document was revised to R4-2103536.

R4-2103536	CR on V2X interruption

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1551  rev 1 Cat: B (Rel-16)

					Source: Mediatek Inc., LG Electronics, Qualcomm Inc.

(Replaces R4-2101054)

Discussion:

See email discussion summary for [98e][208] 5G_V2X_NRSL_RRM in R4-2103690.

Decision: 		The document was agreed.

R4-2101055	CR on V2X interruption

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1552  Cat: A (Rel-17)

					Source: Mediatek Inc., LG Electronics, Qualcomm Inc.

Discussion:

See email discussion summary for [98e][208] 5G_V2X_NRSL_RRM in R4-2103690.

Decision: 		The document was agreed.

R4-2101788	CR to 38.133 correction on reselection of V2X synchronization reference source requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1671  Cat: F (Rel-16)

					Source: vivo

Discussion:

See email discussion summary for [98e][208] 5G_V2X_NRSL_RRM in R4-2103690.

Decision: 		The document was agreed.

R4-2101789	CR to 38.133 correction on reselection of V2X synchronization reference source requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1672  Cat: A (Rel-17)

					Source: vivo

Discussion:

See email discussion summary for [98e][208] 5G_V2X_NRSL_RRM in R4-2103690.

Decision: 		The document was agreed.

#### 7.3.5	RRM perf. requirements (38.133) [5G_V2X_NRSL-Perf]

R4-2103537	WF on NR V2X RRM requirements

Type: other		For: discussion
					Source: LG Electronics

Discussion:

See email discussion summary for [98e][208] 5G_V2X_NRSL_RRM in R4-2103690.

Decision: 		The document was approved.

R4-2100638	Big CR-Introduction of NR V2X RRM performance requirements (Rel-16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1506  Cat: B (Rel-16)

					Source: LG Electronics

Abstract:

It is a big CR to introduce NR V2X RRM performance requirements based on endorsed Draft big CR R4-2017105 in RAN4#97e meeting and additional changes .

Discussion:

See email discussion summary for [98e][208] 5G_V2X_NRSL_RRM in R4-2103690.

Decision: 		The document was revised to R4-2103538.

R4-2103538	Big CR-Introduction of NR V2X RRM performance requirements (Rel-16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1506  rev 1 Cat: B (Rel-16)

					Source: LG Electronics

(Replaces R4-2100638)

Discussion:

See email discussion summary for [98e][208] 5G_V2X_NRSL_RRM in R4-2103690. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2100639	Big CR-Introduction of NR V2X RRM performance requirements (Rel-17)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1507  Cat: A (Rel-17)

					Source: LG Electronics

Abstract:

It is a Cat.A big CR to introduce NR V2X RRM performance requirements based on endorsed Draft big CR R4-2017105 in RAN4#97e meeting and additional changes for Rel-17.

Discussion:

See email discussion summary for [98e][208] 5G_V2X_NRSL_RRM in R4-2103690. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

##### 7.3.5.1	General [5G_V2X_NRSL-Perf]

##### 7.3.5.2	L1 SL-RSRP measurement accuracy [5G_V2X_NRSL-Perf]

##### 7.3.5.3	Test cases [5G_V2X_NRSL-Perf]

###### 7.3.5.3.1	UE transmit timing [5G_V2X_NRSL-Perf]

###### 7.3.5.3.2	Initiation/Cease of SLSS Transmission [5G_V2X_NRSL-Perf]

###### 7.3.5.3.3	Selection / Reselection of V2X Synchronization Reference Source  [5G_V2X_NRSL-Perf]

###### 7.3.5.3.4	L1 SL-RSRP measurements [5G_V2X_NRSL-Perf]

###### 7.3.5.3.5	Congestion control measurements [5G_V2X_NRSL-Perf]

R4-2101056	Remaining issues on congestion control test case

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][208] 5G_V2X_NRSL_RRM in R4-2103690.

Decision: 		The document was noted.

R4-2101057	draftCR on congestion control test case

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][208] 5G_V2X_NRSL_RRM in R4-2103690.

Decision: 		The document was revised to R4-2104045.

R4-2104045	draftCR on congestion control test case

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: MediaTek inc.

(Replaces R4-2101057)

Discussion:

See email discussion summary for [98e][208] 5G_V2X_NRSL_RRM in R4-2103690.

Decision: 		The document was endorsed.

###### 7.3.5.3.6	Interruptions [5G_V2X_NRSL-Perf]

###### 7.3.5.3.7	Resource Pre-emption [5G_V2X_NRSL-Perf]

###### 7.3.5.3.8	Resource Re-evaluation [5G_V2X_NRSL-Perf]

###### 7.3.5.3.9	Others  [5G_V2X_NRSL-Perf]

#### 7.3.6	Demodulation requirements (38.101-4) [5G_V2X_NRSL-Perf]

##### 7.3.6.1	General [5G_V2X_NRSL-Perf]

R4-2103757	Email discussion summary for [98e][318] V2X_Demod_Part1

Type: other		For: discussion
					Source: Moderator (LGE)

Discussion:

The contribution summarised email discussion thread [98e][318] V2X_Demod_Part1. The discussion thread was moderated by Jin-yup Hwang (LG Electronics Inc.).

Decision: 		The document was revised to R4-2103927.

R4-2103927	Email discussion summary for [98e][318] V2X_Demod_Part1

Type: other		For: discussion
					Source: Moderator (LGE)

(Replaces R4-2103757)

Discussion:

The contribution summarised email discussion thread [98e][318] V2X_Demod_Part1. The discussion thread was moderated by Jin-yup Hwang (LG Electronics Inc.).

Decision: 		The document was noted.

R4-2103976	Draft Big CR: Introduction of Rel-16 NR V2X demodulation performance requirements

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: LGE

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2103810	WF on single link tests for NR V2X demodulation performance

Type: other		For: discussion
					Source: LGE

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was revised to R4-2103990.

R4-2103990	WF on single link tests for NR V2X demodulation performance

Type: other		For: discussion
					Source: LGE

(Replaces R4-2103810)

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was approved.

R4-2103811	Summary of simulation results for V2X single link demodulation

Type: other		For: discussion
					Source: LGE

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

R4-2103812	Updated simulation assumption for NR V2X single link test cases

Type: other		For: discussion
					Source: Huawei

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was revised to R4-2103991.

R4-2103991	Updated simulation assumption for NR V2X single link test cases

Type: other		For: discussion
					Source: Huawei

(Replaces R4-2103812)

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was approved.

-------------------------------GTW Notes for [318] on Jan.27th (45 minutes) ---------------------------

Sub-topic 1-1 : Test cases for PSSCH demodulation

Issue 1-1-1: Additional test cases based on GNSS sync source

- Proposals

  - Option 1: 16QAM for 260km/h relative velocity (Huawei)

  - Option 2: 64QAM for 30km/h relative velocity (LG, Qualcomm, MediaTek, CATT)

  - Option 3: 16QAM for 260km/h relative velocity and 64QAM for 30km/h relative velocity (Intel, Huawei, CATT)

- Recommended WF

  - Need further discussion.

Discussion:

QC: LLR quality need to be guarantee with high mobility and high modulation order. Considering the existing test coverage from Rel-16 performance requirements, we think low speed with high order and medium speed with medium modulation already been verified. No coverage issue with option2.

LG: Similar view as QC. Option 2 already cover worst cases, also PSCCH already cover the combination in option 1.

Intel: Typically, we shall cover all modulation orders to ensure the performance, with this we are fine with option 3 to cover all the scenarios considering 16QAM with 260km/h could be one of typical scenarios.

MTK: We support option 2, our concern on test effort and work load. Similar view as LG.

Huawei: We share the view from Intel, 16QAM is mandatory and typical scenarios and we should introduce test cases for this scenario.

Issue 1-1-2: Other test cases: 256QAM modulation

- Proposals:

  - Option 1: Do not define 256QAM demodulation requirements in Rel-16 (Intel, Huawei, LG, MediaTek, CATT)

  - Option 2: Define 256QAM demodulation requirements in Rel-16 (Qualcomm)

- Recommended WF

  - Need further discussion

  - But based on majority views, option 1 is recommended, and continue discussion in future release

Discussion:

QC: In order to save work load, we can resue test configuration from high MCS test cases (64QAM/16QAM) with replacement of MCS as 256QAM.

We also bring simulation results, the SNR and performance is achieveable. We see the deployment scenario.

LG: 256QAM is feature which support for this WI, as rapportenur we see the urgency to make conclusion since no consensus over several meetings and we sugguet to postpone the related discussion in future release.

Issue 1-1-3: Other test cases: demodulation based on gNB sync source

- Proposals:

  - Option 1: Do not define demodulation requirements based on gNB sync source in Rel-16 (Huawei, LG, MediaTek)

  - Option 2: Define demodulation requirements based on gNB sync source with applicability rule (Intel, CATT)

- Recommended WF

Need further discussion

Discussion:

Intel: Cocurrent operation is one of typical feature in Rel-16 V2X WI, gNB sync source is mandated for this operation.

For save test effort, we can discuss test applicable rules. And we also think gNB sync source have higher FOE and TOE.

LG: gNSS sync source is mandatory and typical case for rel-16 V2x. As rapportenur we propose to postpone this to future release.

QC: This is performance test case other than functionality test case. From performance aspect, the differencet is FOE and TOE value, the difference is relative small for tracking BW aspect, we think no performance impact.

Regarding test applciable rules, if UE has gNSS based tests, then no need gNB test cases, test applicables should based on mandatory feature.

Huawei: gNB based on sync source is a optional UE feature. The only difference is CFO, we don’t think big difference.

MTK: We agree with Huawei comments.

Agreements: Considering RAN4 work load and timelimation in Rel-16

Issue 1-1-1: Two options

- Option 1: Introduce 64QAM for 30km/h relative velocity for PSSCH (majority supporting option)

- Option 2: Introduce 64QAM for 30km/h relative velocity and 16QAM with 260km/h for PSSCH

Issue 1-1-2: Do not define 256QAM demodulation requirements in Rel-16

- RAN4 can further discuss and define corresponding requirements if needed in Rel-17 timeframe i.e. Rel-17 side-link enhancement WI.

Issue 1-1-3: Do not define demodulation requirements based on gNB sync source in Rel-16

- RAN4 can further discuss and define corresponding requirements if needed in Rel-17 timeframe i.e. Rel-17 side-link enhancement WI.

Intel: we are also supporting to introduce 16QAM test cases.

Huawei: we have concern on not introducing 16QAM test cases.

QC: We also have concern on test effort not only on RAN4 work load.

-----------------------End -----------------------------------

R4-2103758	Email discussion summary for [98e][319] V2X_Demod_Part2

Type: other		For: discussion
					Source: Moderator (Intel)

Discussion:

The contribution summarised email discussion thread [98e][319] V2X_Demod_Part2. The discussion thread was moderated by Belov Dmitry (Intel).

Decision: 		The document was revised to R4-2103928.

R4-2103928	Email discussion summary for [98e][319] V2X_Demod_Part2

Type: other		For: discussion
					Source: Moderator (Intel)

(Replaces R4-2103758)

Discussion:

The contribution summarised email discussion thread [98e][319] V2X_Demod_Part2. The discussion thread was moderated by Belov Dmitry (Intel).

Decision: 		The document was noted.

R4-2103819	WF on multiple link tests for NR V2X demodulation performance

Type: other		For: discussion
					Source: Intel

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was approved.

R4-2103820	Simulation assumptions for NR V2X multiple link test case

Type: other		For: discussion
					Source: Huawei

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was revised to R4-2103993.

R4-2103993	Simulation assumptions for NR V2X multiple link test case

Type: other		For: discussion
					Source: Huawei

(Replaces R4-2103820)

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was approved.

-------------------------------------GTW Notes for [319] on Jan.27th (45 minutes) ----------------------------

Topic #4: PSCCH/PSSCH decoding capability test

Issue 4-1: CBW/SCS

         Previous meeting agreement

o    Need to further check on the supporting CHBW sets on V2X band, mandatory or optional and typical deployment scenarios, RAN4 planned to make decision among 40MHz and 20MHz in RAN4#98e.

         Proposals

o    Option 1 (MTK, CATT, QC, Intel,LG): 40 MHz, 30 kHz with AT command.

o    Option 2 (Huawei): 20 MHz, 30 kHz

- Option 3 (LG, Huawei as compromise, MTK is fine to take this option as a baseline and further discuss): Flexibly CBW depending on psfch-TxNumber capability:

  - 40MHz channel bandwidth is for the UE supporting psfch-TxNumber =n16, 20MHz/30MHz channel bandwidth is for the UE supporting psfch-TxNumber =n8, and 10MHz channel bandwidth is for the UE supporting psfch-TxNumber =n4.

         Recommended WF

o    Collect comments on options above with detailed description of UE capability

Discussion:

QC: From UE capability 40MHz is mandatory supporting on band n47. With option 3, we introduce test cases pending on another UE capability which not test cases purpose. With option 1, psfch-TxNumber =n16 works. For other UE capability FFS.

LG: RAN4 already agreed using PSFCH for reporting to collect the feedback. Which feedback mode we used to AT command vs PSFCH feedback.

Intel: We agreed to use PSFCH instead of AT command. We also realized for this case AT command coulde be another option.

Huawei: In previous meeting, we already agree to use PSFCH instead of AT command which different to LTE phase.

A unified test method preferred to save test complexity. We can agree 40MHz is mandatory feature. We think option 3 reasonable.

QC: We don’t think unified method needed, AT command is TE implementation. Using PSFCH just for  test cases stat. purpose which not really match all the deployment scenarios.

Intel: Agree with QC. We also agree to consider different options for different cases.

MTK: We don’t agree to use AT command following previous agreements. We support option 3.

QC:

For UE which supporting psfch-TxNumber =n16, 40MHz/30kHz can be used.

For UE which supporting psfch-TxNumber =n8 or n4, CHBW is FFS pending on whether AT command can be used or not

Huawei: we think we already have agreements for feedback mode.

Further discuss the feedback mode for PSCCH/PSSCH decoding capability test:

-Option1: AT command with 40 MHz, 30 kHz

- Option2: Using PSFCH with PSFCH with periodicity 1, Flexibly CBW depending on psfch-TxNumber capability:

- 40MHz channel bandwidth is for the UE supporting psfch-TxNumber =n16, 20MHz/30MHz channel bandwidth is for the UE supporting psfch-TxNumber =n8, and 10MHz channel bandwidth is for the UE supporting psfch-TxNumber =n4.

Further check with TE vendors to get their feedback regarding feasibility and test complexity.

Issue 4-2: PSFCH configuration

         Agreement RAN4 #96e

o    Feedback loop for test method

    - PSFCH feedback instead of AT commands can be used to test NR V2X UE’s performance

         Agreement RAN4 #97e

o    Option 1: No PSFCH

o    Option 2: PSFCH with periodicity 1

         Proposals

o    Option 1 (MTK, CATT, Huawei): Set PSFCH periodicity to 1.

o    Option 2 (LGE, Intel, QC): Use AT command for only PSCCH/PSSCH decoding capability test

o    Option 3 (LGE): Use PSFCH feedback for PSCCH/PSSCH decoding capability test and introduce different test configuration depending on capability of psfch-TxNumber

    - For example, 40MHz channel bandwidth is for the UE supporting psfch-TxNumber =n16, 20MHz/30MHz channel bandwidth is for the UE supporting psfch-TxNumber =n8, and 10MHz channel bandwidth is for the UE supporting psfch-TxNumber =n4.

         Recommended WF

o   Collect comments on options above

Topic #5: SDR with active sidelink test

Issue 5-1: Whether to define the SDR test with active sidelink

         Proposals

o    Option 1 (MTK, Huawei, QC, LG): No

o    Option 2 (CATT, Intel): Yes

         Recommended WF

o    Collect more comments on this issue with detailed justification why one or another option is supported.

Discussion:

Intel: We prefer to have test cases for co-current operation.

Agreements:

Not define the SDR test cases with active sidelink in Rel-16 V2X WI.

-RAN4 also realize with such agreements, the co-current operation has not be verified by Rel-16 RAN4 demodulation requirements. RAN4 can further discuss and define corresponding test cases if needed in Rel-17 timeframe i.e. Rel-17 side-link enhancement WI.

Topic #1: Power imbalance test

Issue 1-2: Distance between the two links and UEs allocation

         Previous meeting agreement

o    Option 1: 10 PRBs

o    Option 2: 30 PRBs

o    Companies are encouraged to check the impact of IBE for options above for the next RAN4 meeting.

         Proposals

o    Option 1 (LGE, MTK slight preference): 10 PRBs, UE1 PRBs 0-9, UE2 PRBs 20-29

o    Option 2 (Intel, Huawei, QC, LG, MTK): 20 PRBs, UE1 PRBs 0-9, UE2 PRBs 30-39

o    Option 3 (Huawei, QC, LG, MTK): 20 PRBs, UE1 PRBs 10-19, UE2 PRBs 40-49

         Recommended WF

- Collect comments on options above

Agreements: Option2: 20 PRBs, UE1 PRBs 0-9, UE2 PRBs 30-39

------------------------------------End--------------------------

R4-2101065	Discussion on NR V2X Demod test cases

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was noted.

R4-2101942	Draft CR on General section of NR V2X requirements

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was revised to R4-2103813.

R4-2103813	Draft CR on General section of NR V2X requirements

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: Intel Corporation

(Replaces R4-2101942)

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was endorsed.

##### 7.3.6.2	Single link test  [5G_V2X_NRSL-Perf]

R4-2100407	Discussion on single link demodulation test for NR V2X

Type: discussion		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

R4-2100409	Simulation results of NR V2X single link demodulation test

Type: discussion		For: Approval
					Source: CATT, GOHIGH

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

R4-2100411	DraftCR for 38.101-4, Introduce PSBCH performance requirements for NR V2X

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: CATT, GOHIGH

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was revised to R4-2103814.

R4-2103814	DraftCR for 38.101-4, Introduce PSBCH performance requirements for NR V2X

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: CATT, GOHIGH

(Replaces R4-2100411)

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was endorsed.

R4-2100628	V2X single link demod discussion

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

###### 7.3.6.2.1	PSSCH demodulation test [5G_V2X_NRSL-Perf]

R4-2100656	Draft CR for PSSCH demodulation requirements for NR V2X

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: LG Electronics Inc.

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was revised to R4-2103815.

R4-2103815	Draft CR for PSSCH demodulation requirements for NR V2X

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: LG Electronics Inc.

(Replaces R4-2100656)

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was revised to R4-2103995.

R4-2103995	Draft CR for PSSCH demodulation requirements for NR V2X

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: LG Electronics Inc.

(Replaces R4-2103815)

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was endorsed.

R4-2100657	Discussion and simulation results for PSSCH single link test in NR V2X

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

R4-2101068	Simulation results for NR V2X PSSCH test case

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

R4-2101232	Discussion on NR V2X Single Link PSSCH requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

R4-2101352	Discussion on PSSCH requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

R4-2101353	Simulation results on PSSCH requirements

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

###### 7.3.6.2.2	PSCCH demodulation test  [5G_V2X_NRSL-Perf]

R4-2100658	Discussion and simulation results for PSCCH single link test in NR V2X

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

R4-2101067	Simulation results for NR V2X PSCCH test case

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

R4-2101233	Discussion on NR V2X Single Link PSCCH requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

R4-2101234	Draft CR on NR V2X Single Link PSCCH requirements

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was revised to R4-2103816.

R4-2103816	Draft CR on NR V2X Single Link PSCCH requirements

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: Intel Corporation

(Replaces R4-2101234)

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was endorsed.

R4-2101354	Discussion and simulation results on PSCCH performance requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

###### 7.3.6.2.3	PSBCH demodulation test [5G_V2X_NRSL-Perf]

R4-2100659	Simulation results for PSBCH single link test in NR V2X

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

R4-2101066	Simulation results for NR V2X PSBCH

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

R4-2101235	Simulation results for NR V2X Single Link PSBCH requirements

Type: other		For: Information
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

R4-2101355	Simulation results on PSBCH performance requirements

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

###### 7.3.6.2.4	PSFCH demodulation test  [5G_V2X_NRSL-Perf]

R4-2100661	Simulation results for PSFCH single link test in NR V2X

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

R4-2101069	draftCR on NR V2X PSFCH demodulation requirements

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was revised to R4-2103817.

R4-2103817	draftCR on NR V2X PSFCH demodulation requirements

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: MediaTek inc.

(Replaces R4-2101069)

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was endorsed.

R4-2101236	Simulation results for NR V2X Single Link PSFCH requirements

Type: other		For: Information
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

R4-2101356	Simulation results on PSFCH performance requirements

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][318] V2X_Demod_Part1 in R4-2103927.

Decision: 		The document was noted.

##### 7.3.6.3	Multiple link test [5G_V2X_NRSL-Perf]

R4-2100408	Discussion on multiple link demodulation test for NR V2X

Type: discussion		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was noted.

R4-2100410	Simulation results of NR V2X multiple link demodulation test

Type: discussion		For: Approval
					Source: CATT, GOHIGH

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was noted.

R4-2100629	V2X multiple link demod discussion

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was noted.

R4-2100662	Discussion and simulation results for multiple link tests in NR V2X

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was noted.

###### 7.3.6.3.1	Power imbalance requirement [5G_V2X_NRSL-Perf]

R4-2101237	Discussion on NR V2X Multiple Link Power Imbalance requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was noted.

R4-2101357	Draft CR: Introduction on Power imbalance requirements for NR V2X

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was revised to R4-2103818.

R4-2103818	Draft CR: Introduction on Power imbalance requirements for NR V2X

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: Huawei, HiSilicon

(Replaces R4-2101357)

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was endorsed.

R4-2101358	Discussion on V2X Power imbalance requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was noted.

R4-2100630	V2X demod HARQ buffer CR

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: Qualcomm, Inc.

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was revised to R4-2103980.

R4-2103980	V2X demod HARQ buffer CR

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: Qualcomm, Inc.

(Replaces R4-2100630)

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was endorsed.

R4-2101238	Discussion on NR V2X Multiple Link HARQ soft buffer combing requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was noted.

R4-2101359	Discussion on V2X Soft-buffer test

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was noted.

###### 7.3.6.3.2	HARQ soft buffer combing test  [5G_V2X_NRSL-Perf]

###### 7.3.6.3.3	PSFCH decoding capability test [5G_V2X_NRSL-Perf]

R4-2101239	Discussion on NR V2X Multiple Link PSFCH decoding capability requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was noted.

R4-2101360	Draft CR: Introduction on PSFCH decoding capability requirements for NR V2X

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was revised to R4-2103981.

R4-2103981	Draft CR: Introduction on PSFCH decoding capability requirements for NR V2X

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: Huawei, HiSilicon

(Replaces R4-2101360)

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was endorsed.

R4-2101361	Discussion on V2X PSFCH decoding capability test

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was noted.

R4-2101240	Discussion on NR V2X Multiple Link PSCCH/PSSCH decoding capability requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was noted.

R4-2101362	Draft CR: Introduction on PSSCH/PSCCH decoding capability requirements for NR V2X

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was revised to R4-2103982.

R4-2103982	Draft CR: Introduction on PSSCH/PSCCH decoding capability requirements for NR V2X

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: Huawei, HiSilicon

(Replaces R4-2101362)

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was endorsed.

R4-2101363	Discussion on V2X PSSCH/PSCCH decoding capability test

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was noted.

R4-2101364	Discussion on V2X multilink SDR test

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was noted.

R4-2101943	Discussion on NR V2X Multiple Link SDR requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][319] V2X_Demod_Part2 in R4-2103928.

Decision: 		The document was noted.

###### 7.3.6.3.4	PSCCH/PSSCH decoding capability  [5G_V2X_NRSL-Perf]

###### 7.3.6.3.5	Others  [5G_V2X_NRSL-Perf]

### 7.4	Integrated Access and Backhaul for NR [NR_IAB]

R4-2103448	Email discussion summary: [98e][209] NR_IAB_RRM

Type: other		For: discussion
					Source: Moderator (ZTE)

Discussion:

The contribution summarised email discussion thread [98e][209] NR_IAB_RRM. The discussion thread was moderated by Richie Leo (ZTE).

Decision: 		The document was revised to R4-2103691.

R4-2103691	Email discussion summary: [98e][209] NR_IAB_RRM

Type: other		For: discussion
					Source: Moderator (ZTE)

(Replaces R4-2103448)

Discussion:

The contribution summarised email discussion thread [98e][209] NR_IAB_RRM. The discussion thread was moderated by Richie Leo (ZTE).

Decision: 		The document was noted.

GTW session (February 04, 2021)

[98e][209] NR_IAB_RRM

Issue 2-1-2: TDD configuration

- Proposals

  - Option 1 (Huawei, Ericsson, ZTE, Nokia): Tests can be done for any TDD configuration. TDD pattern and related configurations shall be configurable and left for implementation including

    - DL/UL scheduling related configuration

    - PRACH and SRS configuration

    - SSB/CSI-RS configuration

  - Option 2 (Qualcomm): Don’t leave test configurations to declaration and implementation. Update 38.133 to include that requirements are applicable to multi-TRxP deployments. (R4-2100203)

- Discussion

  - QC: Option 1 means that no tests will be introduced in the specification.

  - ZTE: IAB-MT shall be tested as a NW node. The TDD configuration is mainly driven by operators. Option 1 is aligned with RF sessions.

  - E///: for NW nodes (BS) it is typical to test TDD configuration up to declaration. Exact implemented configuration will depend on operator demand.

  - Huawei: the same requirements would apply (no tightening or relaxation of requirements).

  - QC: is it a common accepted practice in the BS testing that TDD parameters are up to declaration?

    - Huawei: yes (but need to check)

    - E///: At least for IAB Demod the requirements are planned to be defined for any possible configurations

  - QC: still disagree with Option 1.

  - E///: in RF session there are approved principles for IAB testing. The WF says that tests can apply for any TDD configuration.

  - ZTE: for BS RF part – BS can choose any TDD configuration based on declaration; for BS Demod – there is a reference configuration, but same requirements apply to all possible configurations.

  - Session chair: Continue discussion.

Issue 2-2-4: Applicability of test cases

- Proposals

  - Option 1 (Huawei, Ericsson, Nokia, ZTE): Only define performance test cases for LA IAB-MT.

  - Option 2 (Qualcomm): Specify test cases also for WA IAB-MT.

- Discussion

  - QC: We already discussed this in Core part. We already agreed that some requirements would apply to both LA and WA. No additional test cases needed.

  - Huawei: for test cases we may follow a different approach. WA is not typical.

  - E///: WA is not an interesting case for test case definition

  - ZTE: Same view as Huawei and E///. Even if we define Core requirements, it is still possible not to define the Performance requirements.

  - Session chair: Continue discussion

#### 7.4.1	General [NR_IAB-Core]

R4-2103744	Email discussion summary for [98e][305] NR_IAB_RF_Maintenance

Type: other		For: discussion
					Source: Moderator (CATT)

Discussion:

The contribution summarised email discussion thread [98e][305] NR_IAB_RF_Maintenance. The discussion thread was moderated by Huiping Shan (Fiberhome Technologies Group).

Decision: 		The document was revised to R4-2103929.

R4-2103929	Email discussion summary for [98e][305] NR_IAB_RF_Maintenance

Type: other		For: discussion
					Source: Moderator (CATT)

(Replaces R4-2103744)

Discussion:

The contribution summarised email discussion thread [98e][305] NR_IAB_RF_Maintenance. The discussion thread was moderated by Huiping Shan (Fiberhome Technologies Group).

Decision: 		The document was noted.

R4-2103849	WF on IAB-MT EVM measurement in core spec

Type: other		For: discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was approved.

- ------------------------------ GTW Notes for Jan.26th (45 minutes) –Email thread 305) -----------------------------------

EVM measurement procedure

Issue 1-1: Does IAB-MT measurement procedure refer UE spec or follow BS procedure?

- Proposals

  - Option 1: Refer UE R15 spec with some necessary modifications. (CATT)

  - Option 2: The same as BS approach with some necessary modifications if needed. (Ericsson, Nokia, ZTE)

- Recommended WF

  - Option 1

Discussion:

CATT: With option 2, we believe BS approach, there are many deltas compared to IAB-MT, these required lots of changes i.e. equalizer.

Nokia: We prefer option 2, both options required the modifications; in the end the output will be same. We prefer to align DU and MT as much as possible. We think option 2 may not need modifications.

E///: We make some analysis between UE like or BS like approach, we believe no changes needed using BS approach.

The TE for BS conformance can be reused without modifications.

ZTE: We prefer option 2, but we have some different understanding for the necessary changes with BS approach. It’s premature to conclude now. The configurations i.e. physical channels etc.

CATT: We already discussed several meetings, we need to conclude right now. But we would like to further discuss the details to understand which parts need to be modified.

Agreements:

Using BS approach as basis, further discuss on the details required and the modifications not precluded if necessary

Issue 1-2: If all of the UL physical channels should be tested?

- Proposals

  - Option 1: BS approach (CATT, ZTE, Samsung, Ericsson, Nokia, Huawei)

  - Option 2: Following UE that PUSCH, PUCCH, DMRS and PRACH should be tested.

  - Option 3: No need to describe physical channel in EVM measurement procedure in core spec, Test modes will be introduced in conformance spec (Nokia)

- Recommended WF

  - Option 1

Discussion:

Nokia: We have core requirements for averaging EVM, no separate for individual physical channels. We will introduce test modes for requirements.

ZTE: We prefer option 1.

Samsung: we support option 1, we can use BS spec approach to update IAB core spec, EVM (modulation schemes) only specified for PDSCH.

E///: We support option 1 with BS approach.

QC: How to ensure DMRS performance as receiver need to use DMRS for decoding?

Nokia: we are fine with BS approach. For DMRS should not have impact the performance in the end.

QC: For UE side, we verify everything.

E///: From performance wise, existing BS approach already verified it works.

Huawei: We prefer to verify PUSCH only.

Agreement:

Using BS approach:

- Core spec: clarify that the EVM (modulation orders) specify for PUSCH

- Introduce test modes in conformance spec with PUSCH channel

Issue 1-3: Should PTRS be used IAB-MT EVM measurement?

- Proposals

  - Option 1: yes for FR2 and optional (Nokia, Ericsson, Samsung,ZTE,Huawei)

  - Option 2: No, as the current UE spec

- Recommended WF

  - Option 1

Moderator: It’s moderator’s understanding that the proposal is for FR2 not FR1.

Discussion:

QC: Why DMRS not enough? In real NW, we can’t guarantee PTRS always configured.

Samsung: What’s the meaning for optional? Declaration basis?

E///: In FR2 EVM measurement procedure for BS, PTRS can be configured optional.

ZTE: FR2 PTRS is key and considering future with further extending frequency ranges PTRS also important. Current in BS FRC, PTRS also exists.

Nokia: Having PTRS in FR2 optional aligned with BS spec.

Huawei: We support option 1. We can use same BS approach.

Agreement:

Follow BS approach to configure PTRS in FR2 with optional

Further discuss the test mode and how to the clarify the optional in conformance specification if needed

Issue 1-4: How to modify IAB-MT measurement diagram?

- Proposals

  - Option 1: As proposed in R4-2100365 (CATT)

  - Option 2: As proposes in R4-2100826 (CMCC)

  - Option 3: As BS diagram (Ericsson)

  - Option 4: Other proposal

- Recommended WF

  - To be discussed

Discussion:

CATT:  Using BS diagram as basis, further modifications not precluded.

E///: EVM should be same BS and UE including reference points with different methodologies.

Agreements:

Using BS diagram as basis, further modifications not precluded if necessary.

Issue 1-6: The basic EVM measurement interval (if time allowed)

- Proposals

  - Option 1: The same as UE for PUSCH (10 ms) (CATT)

  - Option 2: The same as UE (CMCC)

  - Option 3: The same as BS (Ericsson)

- Recommended WF

  - To be discussed

Discussion:

Agreements: The same as BS approach.

---------------------------------------------End --------------------------------------------------------

##### 7.4.1.1	System parameters maintenance [NR_IAB-Core]

R4-2100368	Draft CR for TS 38.174: Correction of clause 5

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: CATT

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was revised to R4-2103957.

R4-2103957	Draft CR for TS 38.174: Correction of clause 5

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: CATT

(Replaces R4-2100368)

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was endorsed.

##### 7.4.1.2	Others  [NR_IAB-Core]

R4-2100910	Big CR for update on TR38.809

Type: CR		For: Agreement
					38.809 v16.1.0	  CR-0002  Cat: F (Rel-16)

					Source: Samsung

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2103983	Big CR to TR 38.174 - correction to clause 6

Type: CR		For: Agreement
					38.174 v16.1.0	  CR-0011  Cat: F (Rel-16)

					Source: Qualcomm

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102422	draft CR to TR 38.174 - correction to clause 6

Type: draftCR		For: Endorsement
					38.809 v16.1.0
					Source: Huawei

Abstract:

The word  "hannon"Seems to have replaced existing text in some locations making the text unreadable.

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was endorsed.

#### 7.4.2	RF requirements maintenance [NR_IAB-Core]

##### 7.4.2.1	Transmitter characteristics [NR_IAB-Core]

###### 7.4.2.1.1	Tx Power related requirements  [NR_IAB-Core]

R4-2102335	IAB-MT TX dynamic range

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on IAB-MT TX dynamic requirement modification.

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was noted.

R4-2102336	CR on Tx Power related requirements in TS 38.174

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Ericsson

Abstract:

in this CR, The TX power dyanmic reference condition is updated

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was not pursued.

###### 7.4.2.1.2	Transmitted signal quality [NR_IAB-Core]

R4-2100365	Discussion on IAB-MT EVM measurement process

Type: other		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was noted.

R4-2100366	Draft CR for TS 38.174: IAB-MT EVM measurement

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: CATT

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was not pursued.

R4-2100367	Draft CR for TR 38.809: IAB-MT EVM measurement

Type: draftCR		For: Endorsement
					38.809 v16.1.0
					Source: CATT

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was not pursued.

R4-2100826	Discussion on EVM measurement methodology for IAB-MT

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was noted.

R4-2102012	IAB EVM procedure

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was noted.

R4-2102333	IAB-EVM procedure

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on the EVM procedure which should be captured in the TS 38.174.

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was noted.

R4-2102337	CR on Transmitted signal quality in TS 38.174

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Ericsson

Abstract:

in this CR, Add EVM detailed procedure is in Annex D

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was not pursued.

###### 7.4.2.1.3	Unwanted emissions [NR_IAB-Core]

###### 7.4.2.1.4	Others [NR_IAB-Core]

R4-2100369	Draft CR for TS 38.174: Correction of clause 6,7 and 9

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: CATT

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was revised to R4-2103850.

R4-2103850	Draft CR for TS 38.174: Correction of clause 6,7 and 9

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: CATT

(Replaces R4-2100369)

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was endorsed.

R4-2102334	IAB-MT interference signal characteristic

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our proposal on IAB-MT characteristic of interference signal.

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was noted.

R4-2102338	CR on Transmitter characteristics- Others  TS 38.174

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Ericsson

Abstract:

in this CR, Annex F for interference charateristic  is added

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was revised to R4-2103851.

R4-2103851	CR on Transmitter characteristics- Others  TS 38.174

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Ericsson

(Replaces R4-2102338)

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was endorsed.

##### 7.4.2.2	Receiver characteristics [NR_IAB-Core]

###### 7.4.2.2.1	Sensitivity and dynamic range requirements  [NR_IAB-Core]

R4-2102341	CR on Sensitivity and dynamic range requirements  TS 38.174

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Ericsson

Abstract:

in this CR, format is corrected and bracket is removed

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was endorsed.

###### 7.4.2.2.2	In-band selectivity and blocking requirements  [NR_IAB-Core]

R4-2102339	CR on In-band selectivity  and blocking requirements in TS 38.174

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Ericsson

Abstract:

in this CR, more typo is corrected

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was endorsed.

###### 7.4.2.2.3	Others [NR_IAB-Core]

R4-2100909	Draft CR to align the general clause of radiated and conducted requirement

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Samsung

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was not pursued.

R4-2102011	DraftCR to TS 38.174: Receiver requirement corrections

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was endorsed.

R4-2102340	CR on Rx Charateristic other related requirements

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Ericsson

Abstract:

in this CR, OOB requriement is updated

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was revised to R4-2103852.

R4-2103852	CR on Rx Charateristic other related requirements

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Ericsson

(Replaces R4-2102340)

Discussion:

See email discussion summary for [98e][305] NR_IAB_RF_Maintenance in R4-2103929.

Decision: 		The document was endorsed.

#### 7.4.3	RF conformance testing [NR_IAB-Perf]

##### 7.4.3.1	General and work plan [NR_IAB-Perf]

R4-2103745	Email discussion summary for [98e][306] NR_IAB_Conformance_Part1

Type: other		For: discussion
					Source: Moderator (Nokia)

Discussion:

The contribution summarised email discussion thread [98e][306] NR_IAB_Conformance_Part1. The discussion thread was moderated by Toni Lahteensuo (Nokia).

Decision: 		The document was revised to R4-2103930.

R4-2103930	Email discussion summary for [98e][306] NR_IAB_Conformance_Part1

Type: other		For: discussion
					Source: Moderator (Nokia)

(Replaces R4-2103745)

Discussion:

The contribution summarised email discussion thread [98e][306] NR_IAB_Conformance_Part1. The discussion thread was moderated by Toni Lahteensuo (Nokia).

Decision: 		The document was noted.

R4-2103853	WF on IAB-MT test environment

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was approved.

R4-2103854	WF on test configurations, models and Rx FRC

Type: other		For: discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was approved.

R4-2103855	WF on manufacturer declarations

Type: other		For: discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was approved.

R4-2103856	WF on IAB conformance specification work split and drafting guidelines

Type: other		For: discussion
					Source: Nokia

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was approved.

--------------------------------------GTW Notes for Jan.26th Email thread [306] -------------------------

Topics from email thread [306] (1H)

Issue 1-1-1: Synchronization

Based on the contributions there seems to be good alignment to leave synchronization between DUT and test equipment up to implementation

- Proposals:

  - Option 1: synchronization between DUT and test equipment is left up to implementation

  - Option 2: TBA

- Recommended WF

  - Option 1

Discussion:

QC: How we can ensure that IAB-MT can sync with parent properly in real network with test set-up?

Huawei: We are referring to RF requirements with feasible TE implementation.

ZTE: We don’t have strong opinion, for test modes, how to transmit SSB/PTRS, this will increase test conformance spec work-load. We are not sure whether this generic approach applied for every test cases or with some exception cases.

E///: GPS can be used as IAB internal sync source. IAB-MT can be directly sync with IAB_DU if MT and DU both rely with GPS.

Keysight: From TE aspect, IAB-MT have sync with IAB-DU or IAB-MT can have sync source themselves, we don’t have preference on the options, but we would like to see clear common understanding for the sync methods.

QC: The behavior of testing and real network difference. BS is different as BS no need to sync with others.

Nokia: We are not clear which RF requirements mandated to use sync-signals from IAB-DU. We would like to align with BS approach using same test set-up.

ZTE: Add a note in test mode section, IAB-MT sync with IAB-DU with DL signal configuration also allowed.

Agreement:

Using same BS approach (no detailed synchronization configuration in conformance specifications; meanwhile add a note in conformance specs to clarify (IAB-MT sync with IAB-DU with DL signal configuration not precluded).

Issue 1-1-2: Two-way communication in IAB-MT tests

One company proposes not to specify two-way communication for RF tests. One company proposes to write the specification in a manner which does not preclude the existence of DL signals

- Proposals:

  - Option 1: Two-way communication is not specified

  - Option 2: Two-way communication is not specified, specification shall not preclude DL signals to be used e.g. for timing and frequency reference purposes during the test

- Recommended WF

  - TBA

Discussion:

Huawei: We already have agreements; notes can be applied to clarify. But the clarification in general section or happened in many places?

Agreement:

Two-way communication is not specified for RF conformance tests, specification shall not preclude DL signals to be used e.g. for timing and frequency reference purposes during the test.

Companies further work on the clarification notes to conformance specifications for topic 1-1.

Issue 1-1-3: Description of connection/measurement setup in specification annex

One company notes that the specification annexes capturing the measurement setups are informative in UE and BS specification and proposes to use the same practice in IAB specification

- Proposals:

  - Option 1: Flexibility in connection / measurement setup is allowed by keeping the specified setup informative

  - Option 2: TBA

- Recommended WF

  - Option 1

Discussion:

Agreements: Option 1: Flexibility in connection / measurement setup is allowed by keeping the specified setup informative

Issue 1-1-4: MU/TT

Two companies prefer to adopt BS measurement uncertainties and test tolerances by default, but are open to allow modification if using UE test equipment requires this. One company prefers to adopt the higher of UE and BS values, but does not propose specific numbers. It should be noted that UE specifications do not cover radiated testing in FR1, which may cause difficulties if UE MU is considered.

- Proposals:

  - Option 1: BS MU/TT will be adopted

  - Option 2: The higher value of BS and UE MU/TT are adopted

  - Option 3: The higher value of BS and UE MU/TT are adopted only regarding the individual contribution of system simulator

  - Option 4: Chapter 4.1.3 in 38.141-1 can be re-used for IAB.

- Recommended WF

  - TBA

Discussion:

Keysight: We prefer to try to avoid the situation that some of TE functionality can’t work well. We prefer with option 3.

Huawei: Hard to have a generic approach. There are some requirements TT with zero considering the regulatory, but the MU still important.

QC: Due to test set-up difference, UE MU probably can’t easily applied for IAB-MT. Better to check with TE vendors.

Nokia: I agree with Huawei comments, some requirements with TT with zero, we would like to further check as UE side and BS side different. We didn’t detailed values till now. We encourage companies to bring more detailed/ specific values for which cases MU/TT can’t be used from BS side.

Keysight: I plan to bring more details in next meeting; test MU depending on test set-up and RF requirements themselves.

Samsung: What’s the plan of completing conformance part for Rel-16 IAB WI.

Agreements:

Choosing higher values of MU/TT among available methods only regarding the individual contribution of system simulator as generic approach:

- some exceptions not excluded considering regulatory requirements impact.

Issue 2-2-1: Re-use of NR test models (Pending on time)

There is a proposal that NR BS test models can be re-used for both IAB-MT and IAB-DU, but re-using all physical layer parameters may need further consideration

- Proposals: Note: multiple options can be selected

  - Option 1: reuse NR FR1 test models and NR FR2 test models for IAB-DU using NR details for physical channel parameters.

  - Option 2: It is proposed to reuse NR FR1 test models and NR FR2 test models for IAB-MT taking into account respective physical channel parameters, i.e. TM design for IAB-MT shall follow same framework as BS, delta to BS spec to be agreed

  - Option 3: Common physical channel parameters can refer to TS 38.521 with the clarification that only the information for the used CBW is referred

- Recommended WF

  - TBA

Discussion:

ZTE: We support option 1, whether we need to reuse all the test modes for BS specifications? This is only related to current dynamic range conformance test discussion.

Nokia: IAB-DU we could almost all from BS specs, for IAB-MT we need to some update for test modes i.e. boosting.

We need to further discuss whether we need to include full set of test modes for IAB-DU/IAB-MT?

E///: Option 2 is our preference. Some modifications required.

Samsung: We also agree with option1 for IAB-DU which already agreed. For Option 2 IAB-MT we also agree the ieal in general. We prefer to use reference approach for IAB-DU to BS spec with note which part not applicable. For IAB-MT, we prefer to specify the test modes in conformance specs independently.

Nokia: For IAB-DU, option 1 OK. The only question how to draft the specs.

Huawei: We agree with Nokia; we may refer to BS specs to have alignment description. For shared arch, test applicable rules among IAB-DU and IAB-MT need to be further discussed and how to naming.

ZTE: We support option 2 for IAB-MT also. We think not feasible for IAB-MT and IAB-DU with common test modes.

Agreement:

For IAB_DU: reuse NR FR1 test models and NR FR2 test models for IAB-DU using NR details for physical channel parameters.

For IAB-MT: It is proposed to reuse NR FR1 test models and NR FR2 test models for IAB-MT taking into account respective physical channel parameters, i.e. TM design for IAB-MT shall follow same framework as BS, delta to BS spec to be agreed

FFS for the spec drafting of structures for IAB-DU and IAB-MT Test modes

Issue 2-2-2: TDD pattern for IAB-MT (Pending on time)

Legacy BS NR test model includes TDD patterns (in FR1 and FR2) that are different compare to UE test specification. TDD pattern for IAB-MT needs to be agreed.

- Proposals: Note: multiple options can be selected

  - Option 1: Apply BS FR1 TDD configuration for IAB-MT in FR1

  - Option 2: Apply BS FR2 TDD configuration for IAB-MT in FR2

  - Option 3: Update the measurement duration due to changed UL-DL split

  - Option 4: Special slot configuration is left for implementation

- Recommended WF

  - TBA

Discussion:

E///: Whether TDD configurations needed for conformance test cases? TE vendors’ feedback appreciated for the feasible of change TDD pattern.

ZTE: Support option 1 and option 2. We should avoid cross-link interference during test.

Key sight: The equalizer on EVM related TDD pattern, we need to further check.

Samsung: option 1 is OK as FR1 both UE and BS aligned. For FR2, it’s DL heavy pattern. The core requirement is per slot basis if we go with option 2 for FR2 since this conflicted with IAB-MT core requirements.

Huawei：For shared architecture DU and MT can share the same pattern and how impact the test MU/TT need to be further studied.

Nokia: For IAB-DU we use DL heavy pattern and for IAB-MT we use UL heavy pattern.

ZTE: In Rel-15, the TDD pattern come from operators’ demand. Better to align with BS approach.

Agreements:

Apply BS FR1 TDD configuration for IAB-MT in FR1 (Baseline)

FFS for BS FR2 TDD configuration considering available DL/UL slots during tests

-Option 1: BS FR2 TDD configurations

-Other options not excluded

-------------------------------------End ------------------------------------

R4-2103746	Email discussion summary for [98e][307] NR_IAB_Conformance_Part2

Type: other		For: discussion
					Source: Moderator (Samsung)

Discussion:

The contribution summarised email discussion thread [98e][307] NR_IAB_Conformance_Part2. The discussion thread was moderated by Yankun Li (Samsung).

Decision: 		The document was revised to R4-2103931.

R4-2103931	Email discussion summary for [98e][307] NR_IAB_Conformance_Part2

Type: other		For: discussion
					Source: Moderator (Samsung)

(Replaces R4-2103746)

Discussion:

The contribution summarised email discussion thread [98e][307] NR_IAB_Conformance_Part2. The discussion thread was moderated by Yankun Li (Samsung).

Decision: 		The document was noted.

R4-2103857	WF on IAB-MT test coverage and IAB node test burden reduction

Type: other		For: discussion
					Source: Nokia

Discussion:

See email discussion summary for [98e][307] NR_IAB_Conformance_Part2 in R4-2103931.

Decision: 		The document was approved.

R4-2103977	WF on Testing aspect on IAB-MT dynamic range, power control and frequency error

Type: other		For: discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][307] NR_IAB_Conformance_Part2 in R4-2103931.

Decision: 		The document was approved.

--------------------------------------GTW Notes for Jan.26th Email thread [307] -------------------------

Topics from email thread [307] (45 minutes)

Issue 1-1: Dynamic range Test point

- Proposals

  - Option 1:Consider two test points  for IAB-MT dynamic range as below:

  - Maximum output power with full RB allocation and maximum output power(Test point 1 in last meeting WF)

  - single RB allocation with 5/10 dB lower PSD as used in test point 1) (Updated Test point 3 in last meeting WF)

  - Option 2: consider dynamic range on full RB allocation only and update core requirement

  - Option 3:

  - Maximum output power with full RB allocation(Test point 1 in WF)

  - Maximum output power with 1/4 RB allocation(belongs other proposal in WF)

  - Minimum output power (as set by 5/10 dB dynamic range requirement) with full RB allocation.(Test point 2 in WF)

- Recommended WF

  - It’s encouraged companies share preference on the options on test points selection. And it would be appreciated if companies can share opinion on whether Dynamic PSD(X) and constant PSD(Y) we agreed last year should be applied for IAB-MT.

Discussion:

Both X and Y need to be covered in conformance test cases

Option 1(Samsung, Nokia, CATT, ZTE):

- Test points 1: Maximum output power with full RB allocation and maximum output power

- Test points 2: single RB allocation with 5/10 dB lower PSD as used in test point 1)

- Test point 1- test point 2 =  X+Y （+/- uncertainty FFS ）

Option 2 (E///, ZTE):

- Test points 1: Maximum output power with full RB allocation and maximum output power

- Test points 3: Minimum output power (as set by 5/10 dB dynamic range requirement) with full RB allocation.

- Test point 4: Sing PRB transmission with same PSD as test point 1

- Test point 1- test point3 = X (+/- uncertainty FFS)

- Test point 1 – test point4 =Y (+/- uncertainty FFS)

FFS for test applicability along with other conformance requirements

Candidate agreements: Option 1 pending on further check by E/// and make decision in this meeting.

E///: Power accuracy need to be considered. Y can be considered to be verified in power control requirements.

Nokia: Prefer option 1, we can follow majority among option 1 and option2. Not prefer to merger test cases with power control requirements.

Samsung: We prefer option 1 considering test burden, power control only defined for local-IAB-MT, prefer not to merge them together.

ZTE: The power control requirements not applicable for dynamic range requirements.

E///: We prefer option 2.

-------------------------------------End ------------------------------------

--------------------------------------GTW agenda on Jan.29th for email thread [307] (45 minutes) -------------------

| Sub-topic 1-2: power control for local Area IAB- MT | Tentative agreements: Issue 1-2-1: Power control test points 4 companies prefer to have explicit test case defined for power control and - One company prefers option 3 as proponent - Other 3 companies fine with either option 2 or option 3. 2 companies have no strong opinion on whether to have the test since the power control step is rather small 2 companies also see the possibility to merge power control in dynamic range. However, it seems all discussion and input would mainly for relative power control tolerance. But not concrete proposal on aggregated power control tolerance. Issue 1-2-2: Test set-up As pointed by companies, the both options abstracted from contribution should not exclude each other. Furthermore, on 26th Jan GTW meeting it is agreed in [306] as below. It’s assumed the test set-up for this specific requirement can be covered by below common agreement. And further discussion on test set-up for power control can be megered into issue 1-1-2 of [306]. Issue 1-1-2: Two-way communication in IAB-MT tests in [306] Two-way communication is not specified for RF conformance tests, specification shall not preclude DL signals to be used e.g. for timing and frequency reference purposes during the test. Companies further work on the clarification notes to conformance specifications for topic 1-1.. CATT: For relative power control two-way communication not needed. For aggregated power control, I guess we need to discuss whether need command from TE. QC: For generic test set-up, we can follow the agreements we have, for specific test cases we may need to discuss the details. E///: we believe we still can use test modes; the generic approach should be applied as well. ZTE: we also this power control can be covered by power dynamic range. Candidate options: For relative power control accuracy: Considering the majority view, the test condition on this requirement can be discussed based on “Option 3: Partial PRB allocation to be considered in Test model design if to reuse the similar test configuration as UE.” For relative power control accuracy Agreements: Option 3: Partial PRB allocation to be considered in Test model design if to reuse the similar test configuration as UE.” E///: We consider to simplify test mode be reused for dynamic range test cases as well. Nokia: we can further check the possibility for test mode. For aggregated power control accuracy: It’s recommended to provide companies view on conformance testing this requirement. E///: This is to verify the out power capability. We would like to see combine with output power requirements. We want to further check. UE will rely on TPC command, we prefer not mention this and leave it to implementation. Nokia: we are fine to have dynamic range and power control combined together. For shared RF architecture, we see this test case probably less meaning; we are open to discuss the combined with other test cases. We believe power dynamic ranges test cases more important. ZTE: The BS and UE implementation for output power is different. We proposed not needed. QC: We are flexible for this as well as we need to verify IAB-MT transmit min and max power with power changes. CATT: Support ZTE this one can be covered by other test cases. Samsung: We share similar comments with CATT, ZTE and Nokia. For aggregated power control accuracy agreements: NO detailed conformance test cases for this requirement, FFS whether can be jointly verified or covered by dynamic range conformance test cases. |
| --- | --- |
| Sub-topic 1-3: frequency error | Tentative agreements: As commented by companies, the both options abstracted from contribution should be enabled with the target to allow TE flexibility. Furthermore, on 26th Jan GTW meeting it is agreed in [306] as below on synchronization and measurement setup aspect. Topics from email thread [306] Issue 1-1-1: Synchronization Agreement: Using same BS approach (no detailed synchronization configuration in conformance specifications; meanwhile add a note in conformance specs to clarify (IAB-MT sync with IAB-DU with DL signal configuration not precluded). Issue 1-1-3: Description of connection/measurement setup in specification annex Agreements: Option 1: Flexibility in connection / measurement setup is allowed by keeping the specified setup informative. It’s suggested to apply above agreement for frequency error. QC: If we have outside source which provide sync, how to ensure IAB-MT tracking sync properly. E///: We already have discussed in general section, for specific test case of FOE, IAB-MT can use GNSS sync. FOE is verified in EVM procedure in BS side also verified in in-band measurement in EVM procedure in UE side. We have separate discussion for fine-sync in demod AI. We think to take this agreements as generic approach as it is. CATT: How to verify FOE together with EVM. EVM applied for absolute frequency, we need to differentiate IAB-MT and IAB-DU. In future with mobile IAB-MT, then we need to discuss this again. ZTE: As mentioned by E///, SSB and GNSS sync source both allowed. Usually GNSS based can offer better performance better than SSB. Nokia: Mobile IAB-MT is out of scope for this discussion. We have the requirements what’s the step following, we can guarantee the performance. We can work on the details for TP drafting, we can do two measurements for absolute frequency and relative frequency. QC: Requirements on IAB_MT is 0.1 ppm. The point refers to frequency of DL signals, how to guarantee the performance of outsource of sync. Nokia: We have a statement, we are not excluded DL signals; also not exclude other solutions and leave it to implementation. E///: We don’t want to mandate any implementation during test set-up. ZTE: In test mode, we can add a note for the frequency error definition. Agreements: Above agreement from [306] applied for frequency error test as well. QC: We have concern on this agreement. The test set-up is artificial. |

----------------------------End ------------------------

R4-2102321	On IAB test case dependency

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on the test burden reduction.

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

##### 7.4.3.2	Common test issues for conducted and radiated conformance testing [NR_IAB-Perf]

###### 7.4.3.2.1	Test configurations [NR_IAB-Perf]

R4-2100906	View on IAB-MT RF Test configuration

Type: other		For: Approval
					Source: Samsung

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

R4-2101565	Discussion on IAB RF conformance test configurations

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, we continue discussions on details for IAB test configurations.

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

R4-2101960	Discussion on IAB test configurations

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

R4-2102322	IAB Common test issue on test Configuration-Conducted

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on test configuration for IAB RF conformance test work and also propose the TP for the test configuration

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

R4-2102323	IAB Common test issue on test configuration-OTA

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on test configuration for IAB RF conformance test work and also propose the TP for the test configuration

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

###### 7.4.3.2.2	Test models [NR_IAB-Perf]

R4-2100371	Discussion on IAB-MT test model

Type: other		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

R4-2100907	Configuration on reference channel for IAB-MT

Type: other		For: Approval
					Source: Samsung

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

R4-2101564	Discussion on IAB test models for conformance specifications

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, we continue discussions on details for IAB test models.

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

R4-2101961	Discussion on IAB test model

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

R4-2102324	IAB Common test issue on test model-Conducted

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our investigation on how the IAB-MT test model should be defined.

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

R4-2102325	IAB Common test issue on test model-OTA

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our investigation on how the IAB-MT OTA test model should be defined.

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

###### 7.4.3.2.3	Others [NR_IAB-Perf]

R4-2100908	View on IAB manufacturer declaration

Type: other		For: Approval
					Source: Samsung

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

R4-2101418	Testing of IAB-MT Requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

R4-2101962	Discussion on IAB conformance testing

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

R4-2102016	Manufacturer declaration framework

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

R4-2102087	IAB-MT conformance Test setup and Test Equipment choice

Type: discussion		For: Agreement
					Source: Keysight Technologies UK Ltd

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

R4-2102326	On IAB-MT synchronization test setup for conformance testing

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on this aspect for rel-16 IAB RF conformance test work.

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

R4-2102421	IAB declarations format

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discussion on the BS declarations format and its suitability for the IAB nodes.

Discussion:

See email discussion summary for [98e][306] NR_IAB_Conformance_Part1 in R4-2103930.

Decision: 		The document was noted.

##### 7.4.3.3	Conducted conformance testing [NR_IAB-Perf]

###### 7.4.3.3.1	Transmitter characteristics [NR_IAB-Perf]

R4-2100370	Discussion on IAB-MT specfic Tx requirements test

Type: other		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][307] NR_IAB_Conformance_Part2 in R4-2103931.

Decision: 		The document was noted.

R4-2102013	Dynamic range test points

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][307] NR_IAB_Conformance_Part2 in R4-2103931.

Decision: 		The document was noted.

R4-2102327	IAB conducted transmitter test

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our investigation on how the IAB-MT conducted transmitter test could be defined.

Discussion:

See email discussion summary for [98e][307] NR_IAB_Conformance_Part2 in R4-2103931.

Decision: 		The document was noted.

###### 7.4.3.3.2	Receiver characteristics [NR_IAB-Perf]

R4-2102017	Receiver RF testing considerations

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][307] NR_IAB_Conformance_Part2 in R4-2103931.

Decision: 		The document was noted.

R4-2102328	TP example for Conducted receiver characteristic test

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our investigation on how the IAB-MT conducted receiver test could be defined.

Discussion:

See email discussion summary for [98e][307] NR_IAB_Conformance_Part2 in R4-2103931.

Decision: 		The document was noted.

###### 7.4.3.3.3	Other test issues  [NR_IAB-Perf]

R4-2102329	General conducted test conditions and declarations

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on IAB generic conducted test conditions and declarations and the corresponding TP is proposed.

Discussion:

See email discussion summary for [98e][307] NR_IAB_Conformance_Part2 in R4-2103931.

Decision: 		The document was noted.

##### 7.4.3.4	Radiated conformance testing [NR_IAB-Perf]

R4-2102420	IAB conformance test burden

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Investigating the number or test conditions the OTA BS is tested under toSee if this ban be reduced for an IAB node

Discussion:

See email discussion summary for [98e][307] NR_IAB_Conformance_Part2 in R4-2103931.

Decision: 		The document was noted.

###### 7.4.3.4.1	Transmitter characteristics [NR_IAB-Perf]

R4-2102015	Test setup considerations

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][307] NR_IAB_Conformance_Part2 in R4-2103931.

Decision: 		The document was noted.

R4-2102330	On IAB-MT frequency error tests

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our investigation on this frequency error test on IAB-MT specifically.

Discussion:

See email discussion summary for [98e][307] NR_IAB_Conformance_Part2 in R4-2103931.

Decision: 		The document was noted.

###### 7.4.3.4.2	Receiver characteristics [NR_IAB-Perf]

R4-2102331	TP example for OTA receiver characteristic test

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our investigation on how the IAB-MT OTA receiver test could be defined.

Discussion:

See email discussion summary for [98e][307] NR_IAB_Conformance_Part2 in R4-2103931.

Decision: 		The document was noted.

###### 7.4.3.4.3	Other test issues  [NR_IAB-Perf]

R4-2102014	IAB RF conformance testing burden considerations

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][307] NR_IAB_Conformance_Part2 in R4-2103931.

Decision: 		The document was noted.

R4-2102332	General test conditions and declarations-OTA

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on IAB generic OTA test conditions and declarations and the corresponding TP is proposed.

Discussion:

See email discussion summary for [98e][307] NR_IAB_Conformance_Part2 in R4-2103931.

Decision: 		The document was noted.

#### 7.4.4	RRM core requirements maintenance [NR_IAB-Core]

R4-2100041	On requirements involving gap patterns for IAB-MTs

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was noted.

R4-2100042	[CR] IAB Core Maintenance

Type: CR		For: Agreement
					38.174 v16.1.0	  CR-0007  Cat: F (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was revised to R4-2103539.

R4-2103539	[CR] IAB Core Maintenance

Type: CR		For: Agreement
					38.174 v16.1.0	  CR-0007  rev 1 Cat: F (Rel-16)

					Source: ZTE Corporation

(Replaces R4-2100042)

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was revised to R4-2104057.

R4-2104057	[CR] IAB Core Maintenance

Type: CR		For: Agreement
					38.174 v16.1.0	  CR-0007  rev 2 Cat: F (Rel-16)

					Source: ZTE Corporation

(Replaces R4-2103539)

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was endorsed.

R4-2101626	Discussion on RRM core requirements maintenance for IAB

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was noted.

R4-2101627	CR on RRM core requirements maintenance for MG for IAB

Type: CR		For: Agreement
					38.174 v16.1.0	  CR-0008  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was merged.

R4-2102487	discussion on measurement gaps for IAB RRM requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

discussion on measurement gaps for IAB RRM requirements

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was noted.

R4-2102488	CR on removing gap aspects from IAB-MT RRM requirements

Type: CR		For: Agreement
					38.174 v16.1.0	  CR-0009  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

CR on removing gap aspects from IAB-MT RRM requirements

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was merged.

R4-2102635	Analysis of measurement gaps for LA IAB-MT

Type: other		For: Discussion
					Source: Ericsson

Abstract:

The paper analyze gaps for IAB

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was noted.

R4-2102636	Measurement gaps for Local Area IAB-MT

Type: CR		For: Agreement
					38.174 v16.1.0	  CR-0010  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

The CR defines one gap for BM and RLM requirements for LA IAB-MT

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was not pursued.

#### 7.4.5	RRM perf. requirements [NR_IAB-Perf]

R4-2103540	WF on test cases for IAB-MTs

Type: other		For: discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was revised to R4-2104069.

R4-2104069	WF on test cases for IAB-MTs

Type: other		For: discussion
					Source: ZTE Corporation

(Replaces R4-2103540)

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was revised to R4-2104091.

R4-2104091	WF on test cases for IAB-MTs

Type: other		For: discussion
					Source: ZTE Corporation

(Replaces R4-2104069)

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was approved.

R4-2102936	IAB-RRM Conformance Testing

Type: discussion		For: Discussion
					Source: Qualcomm CDMA Technologies

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was noted.

##### 7.4.5.1	General [NR_IAB-Perf]

R4-2100047	Scope and work split of test cases for IAB-MTs

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was noted.

R4-2100253	On general aspects of IAB-MT test cases

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was noted.

R4-2101628	Discussion on RRM test cases for IAB

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was noted.

R4-2101629	draftCR to introduce test configurations for IAB-MT RRM performance test

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was revised to R4-2103541.

R4-2103541	draftCR to introduce test configurations for IAB-MT RRM performance test

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Huawei, HiSilicon

(Replaces R4-2101629)

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was endorsed.

R4-2102489	discussion on IAB RRM test cases

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion the RRM test cases for IAB.

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was noted.

R4-2102640	General principles for IAB RRM test cases

Type: other		For: Discussion
					Source: Ericsson

Abstract:

The paper discusses some general aspects of RRM tests for IAB

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was noted.

##### 7.4.5.2	Test cases [NR_IAB-Perf]

R4-2100046	[draft CR] Test cases for timing for IAB-MT

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was revised to R4-2103542.

R4-2103542	[draft CR] Test cases for timing for IAB-MT

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: ZTE Corporation

(Replaces R4-2100046)

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was endorsed.

R4-2101630	draftCR to introduce test cases for RRC release with redirection for IAB-MT

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was revised to R4-2103543.

R4-2103543	draftCR to introduce test cases for RRC release with redirection for IAB-MT

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Huawei, HiSilicon

(Replaces R4-2101630)

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was endorsed.

R4-2102490	draftCR on IAB RLM test cases

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Introduce the RLM test cases for IAB-MTs.

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was revised to R4-2103544.

R4-2103544	draftCR on IAB RLM test cases

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102490)

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was endorsed.

R4-2102637	Big CR: IAB-MT RRM test cases in 38.174

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Ericsson

Abstract:

The big darft CR on spec structure for IAB-MT RRM test cases in annex of TS 38.174. It was endorsed at RAN#97e

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was revised to R4-2103545.

R4-2103545	Big CR: IAB-MT RRM test cases in 38.174

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Ericsson

(Replaces R4-2102637)

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed

R4-2102638	Analysis of RRC re-establishment tests for LA IAB-MT

Type: other		For: Discussion
					Source: Ericsson

Abstract:

The document describes test cases to verify RRC re-establishment requirements for IAB-MT local areas classe

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was noted.

R4-2102639	RRC re-establishment tests for LA IAB-MT

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Ericsson

Abstract:

The darft CR on IAB-MT RRM test cases on RRC re-establishment for IAB-MT LA class

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was revised to R4-2103546.

R4-2103546	RRC re-establishment tests for LA IAB-MT

Type: draftCR		For: Endorsement
					38.174 v16.1.0
					Source: Ericsson

(Replaces R4-2102639)

Discussion:

See email discussion summary for [98e][209] NR_IAB_RRM in R4-2103691.

Decision: 		The document was endorsed.

#### 7.4.6	EMC core requirements maintenance [NR_IAB-Core]

##### 7.4.6.1	General [NR_IAB-Core]

R4-2103773	WF on IAB EMC RI testing with spatial exclusions

Type: other		For: discussion
					Source: Huawei

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was approved.

R4-2100356	Definition of Exclusion Bands for IAB EMC nodes

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion paper on Exclusion bands for IAB EMC testing

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was noted.

R4-2100357	CR to TS 38.175 on Exclusion Bands

Type: CR		For: Agreement
					38.175 v16.1.0	  CR-0008  Cat: B (Rel-16)

					Source: Ericsson

Abstract:

CR on Excclusion Bands for IAB EMC testing

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was not pursued.

##### 7.4.6.2	Emission requirements [NR_IAB-Core]

##### 7.4.6.3	Immunity requirements [NR_IAB-Core]

R4-2100358	Discussion on Spatial Exclusion for IAB EMC RI test

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion paper on Spatial Exclusion for IAB EMC Radiated Immunity Testing

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was noted.

R4-2100359	CR to TS 38.175 on Spatial Exclusion for IAB EMC Radiated Immunity test

Type: CR		For: Agreement
					38.175 v16.1.0	  CR-0009  Cat: B (Rel-16)

					Source: Ericsson

Abstract:

CR on spatial exclusion for IAB EMC Radiated Immunity testing

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was not pursued.

#### 7.4.7	EMC performance requirements [NR_IAB-Perf]

R4-2100360	CR to TS 38.175 on IAB EMC test configuratins and performance requirements

Type: CR		For: Agreement
					38.175 v16.1.0	  CR-0010  Cat: B (Rel-16)

					Source: Ericsson

Abstract:

CR on IAB EMC Performance requirements

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was revised to R4-2103774.

R4-2103774	CR to TS 38.175 on IAB EMC test configuratins and performance requirements

Type: CR		For: Agreement
					38.175 v16.1.0	  CR-0010  rev 1 Cat: B (Rel-16)

					Source: Ericsson, ZTE

(Replaces R4-2100360)

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was not pursued.

R4-2102126	CR to TS 38.175: Performance criteria for IAB

Type: CR		For: Agreement
					38.175 v16.1.0	  CR-0011  Cat: F (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was withdrawn.

R4-2102177	CR to TS 38.175: Performance criteria for IAB

Type: CR		For: Agreement
					38.175 v16.1.0	  CR-0012  Cat: F (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was not pursued.

R4-2102578	Spatial exclusion zone for IAB node

Type: discussion		For: Agreement
					Source: Huawei

Abstract:

In this contribution we provide analysis of the challenges with the application of the spatial exclusion for the EMC RI testing of the IAB.

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was noted.

R4-2102581	CR to TS 38.175: Spatial Exclusion for EMC RI test for IAB

Type: CR		For: Agreement
					38.175 v16.1.0	  CR-0013  Cat: B (Rel-16)

					Source: Huawei

Abstract:

Based on the discussion paper on the analysis of spatial exclusion challenges, in this CR provides addittional text for the completion of the spatial exclusion feature in the IAB EMC specification.

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was not pursued.

#### 7.4.8	Demodulation and CSI requirements  [NR_IAB-Perf]

##### 7.4.8.1	General  [NR_IAB-Perf]

R4-2103759	Email discussion summary for [98e][320] NR_IAB_Demod

Type: other		For: discussion
					Source: Moderator (Nokia)

Discussion:

The contribution summarised email discussion thread [98e][320] NR_IAB_Demod. The discussion thread was moderated by Axel Mueller (Nokia).

Decision: 		The document was revised to R4-2103935.

R4-2103935	Email discussion summary for [98e][320] NR_IAB_Demod

Type: other		For: discussion
					Source: Moderator (Nokia)

(Replaces R4-2103759)

Discussion:

The contribution summarised email discussion thread [98e][320] NR_IAB_Demod. The discussion thread was moderated by Axel Mueller (Nokia).

Decision: 		The document was noted.

R4-2103921	WF on Rel-16 NR IAB demodulation requirements

Type: other		For: discussion
					Source: Nokia

Discussion:

See email discussion summary for [98e][320] NR_IAB_Demod in R4-2103935.

Decision: 		The document was revised to R4-2103994.

R4-2103994	WF on Rel-16 NR IAB demodulation requirements

Type: other		For: discussion
					Source: Nokia

(Replaces R4-2103921)

Discussion:

See email discussion summary for [98e][320] NR_IAB_Demod in R4-2103935.

Decision: 		The document was approved.

R4-2103979	Simulation assumptions for demodulation performance requirements for IAB-MT

Type: other		For: discussion
					Source: Huawei

Discussion:

See email discussion summary for [98e][320] NR_IAB_Demod in R4-2103935.

Decision: 		The document was noted.

R4-2102105	IAB demodulation general considerations

Type: other		For: Approval
					Source: Ericsson

Abstract:

Proposals for remaining general/test issues

Discussion:

See email discussion summary for [98e][320] NR_IAB_Demod in R4-2103935.

Decision: 		The document was noted.

##### 7.4.8.2	IAB-DU performance requirements [NR_IAB-Perf]

R4-2101262	Views on NR IAB-DU demodulation performance requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][320] NR_IAB_Demod in R4-2103935.

Decision: 		The document was noted.

R4-2101293	Discussion on NR IAB DU demodulation performance requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][320] NR_IAB_Demod in R4-2103935.

Decision: 		The document was noted.

R4-2102092	On NR IAB-DU demodulation requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][320] NR_IAB_Demod in R4-2103935.

Decision: 		The document was noted.

R4-2102106	IAB demodulation DU considerations

Type: other		For: Approval
					Source: Ericsson

Abstract:

Proposals for remaining DU issues

Discussion:

See email discussion summary for [98e][320] NR_IAB_Demod in R4-2103935.

Decision: 		The document was noted.

##### 7.4.8.3	IAB-MT performance requirements [NR_IAB-Perf]

R4-2101263	Views on NR IAB-MT demodulation performance requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][320] NR_IAB_Demod in R4-2103935.

Decision: 		The document was noted.

R4-2101294	Discussion on NR IAB MT demodulation performance requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][320] NR_IAB_Demod in R4-2103935.

Decision: 		The document was noted.

R4-2102097	On NR IAB-MT testing setup and demodulation requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][320] NR_IAB_Demod in R4-2103935.

Decision: 		The document was noted.

R4-2102107	IAB demodulation MT considerations

Type: other		For: Approval
					Source: Ericsson

Abstract:

Proposals for remaining MT issues

Discussion:

See email discussion summary for [98e][320] NR_IAB_Demod in R4-2103935.

Decision: 		The document was noted.

### 7.5	Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements  [LTE_NR_DC_CA_enh]

R4-2103449	Email discussion summary: [98e][210] LTE_NR_DC_CA_RRM_1

Type: other		For: discussion
					Source: Moderator (Nokia)

Discussion:

The contribution summarised email discussion thread [98e][210] LTE_NR_DC_CA_RRM_1. The discussion thread was moderated by Lars Dalsgaard (Nokia).

Decision: 		The document was revised to R4-2103692.

R4-2103692	Email discussion summary: [98e][210] LTE_NR_DC_CA_RRM_1

Type: other		For: discussion
					Source: Moderator (Nokia)

(Replaces R4-2103449)

Discussion:

The contribution summarised email discussion thread [98e][210] LTE_NR_DC_CA_RRM_1. The discussion thread was moderated by Lars Dalsgaard (Nokia).

Decision: 		The document was noted.

GTW session (January 27, 2021)

Topic #1: RRM core requirements maintenance

Issue 1-3-1: Clarification regarding NEUTRA_carrier, while T331 is running

- Proposals

  - Option 1: Clarify that NEUTRA_carrier, while T331 is running, excludes configured E-UTRA carriers indicated to meet high speed requirements.

  - Option 2: other.

Discussion:

Huawei: the intention is not to exclude carriers completely. They were already accounted in a different place. The carriers will still be used for EMR.

Agreement: Clarify that NEUTRA_carrier, while T331 is running, excludes configured E-UTRA carriers indicated to meet high speed requirements.

Issue 1-1-1: SSB index reading time for FR2 EMR

- Proposals

  - Option 1: SSB index reading time for FR2 EMR = 3 (Nokia)

  - Option 2: SSB index reading time for FR2 EMR = 5 (QC, HW, Apple, MTK)

- Discussion

  - Apple/MTK: Option 2

  - Nokia: Option 2 is acceptable

Agreement: SSB index reading time for FR2 EMR = 5

Issue 1-2-1: Conditions for detected cell

- Proposals

  - Option 1: Clarification/change to the cell detected conditions when transitioning from connected to idle mode is needed. (Nokia)

  - Option 2: Clarification/change to the cell detected conditions when transitioning from connected to idle mode is not needed. (QC, HW, MTK)

- Discussion

  - Apple: Option 2. Based on current spec the procedure is clear.

Agreement: Clarification/change to the cell detected conditions when transitioning from connected to idle mode is not needed

Topic #2: RRM performance requirements

Issue 2-1-1: Measurement performance test cases for SS-RSRP and SS-RSRQ measurement accuracies for LTE-NR_DC_CA

- Proposals

  - Option 1: Define measurement performance test cases for SS-RSRP and SS-RSRQ measurement accuracies for LTE-NR_DC_CA

  - Option 2: other.

- Discussion

  - Apple: Accuracy was already verified in many other places incl. IDLE mode. From UE baseband perspective we don’t expect difference. If test is introduced then we prefer to verify delay and accuracy in the same test.

  - Nokia: we had a similar discussion in Rel-15 and existing test cases do not test absolute accuracy. Support Option 1.

  - Apple: need to check LTE spec

  - QC: what is the delay in EMR context?

    - Apple: we mean measurement period

Agreement:

- Define measurement performance test cases for SS-RSRP and SS-RSRQ measurement accuracies for LTE-NR_DC_CA

  - Measurement period and accuracy shall be verified in the same test.

Issue 2-1-2: Define intra-frequency measurement performance test cases for SS-RSRP and SS-RSRQ

- Proposals

  - Option 1: Define intra-frequency measurement performance test cases for SS-RSRP and SS-RSRQ

  - Option 2: Other

- Discussion

  - Huawei: EMR is not applicable for intra-frequency. Is it applicable for the serving cell only?

    - Nokia: Yes.

  - Apple: we think intra-frequency measurements are not in the scope

  - Huawei: serving cell accuracy can be verified in the same test as discussed in 2-1-1

  - Nokia: we are ok. Need to further discuss the details of the test.

  - QC: share same view with Apple/Huawei

  - Session chair: further discuss the serving cell accuracy testing as a part of 2-1-1 test cases

Issue 2-2-1: PCell on NR FR1 and EMR target cell on LTE

- Proposals

  - Option 1: RAN4 do not introduce test case for PCell on NR FR1 and EMR target cell on LTE

  - Option 2: Other

- Discussion

  - Nokia: this is not an uncommon case.

  - QC: ok to define the test case

  - Conclusion: previous agreements can be kept

Issue 2-2-2: PCell on NR FR2 and EMR target cell on NR FR1

- Proposals

  - Option 1: RAN4 do not introduce test case for PCell on NR FR2 and EMR target cell on NR FR1

  - Option 2: Other

- Agreement: do not introduce test case for PCell on NR FR2 and EMR target cell on NR FR1

Issue 2-2-3: PCell on NR RF2 and EMR target cell on NR FR2

- Option 1: RAN4 do not introduce test case for PCell on NR RF2 and EMR target cell on NR FR2

- Agreement: do not introduce test case for PCell on NR FR2 and EMR target cell on NR FR2

Issue 2-2-4: PCell on NR RF2 and EMR target cell on LTE

- Option 1: RAN4 do not introduce test case for PCell on NR RF2 and EMR target cell on LTE

- Agreement: do not introduce test case for PCell on NR FR2 and EMR target cell on LTE

R4-2103450	Email discussion summary: [98e][211] LTE_NR_DC_CA_RRM_2

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][211] LTE_NR_DC_CA_RRM_2. The discussion thread was moderated by Joakim Axmon (Ericsson).

Decision: 		The document was revised to R4-2103693.

R4-2103693	Email discussion summary: [98e][211] LTE_NR_DC_CA_RRM_2

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2103450)

Discussion:

The contribution summarised email discussion thread [98e][211] LTE_NR_DC_CA_RRM_2. The discussion thread was moderated by Joakim Axmon (Ericsson).

Decision: 		The document was noted.

GTW session discussion conclusions (January 27, 2021)

Issue 1-2-1: Incremental delay D’ for SCell dormancy switching on multiple CCs

- Background

  - In previous meeting it was agreed that the UE capability for incremental delay per CC can be different for active BWP switching (of non-dormant BWPs) and SCell dormancy switching, respectively. The agreement was captured in R4-2017213: “Introduce a capability D’ for dormant BWP switching of multiple SCells that is separate from corresponding capability D for active BWP switching.”

  - In earlier discussions it has been mentioned that D’ shall be taken from the same set of values as applies for D, but it has not been discussed whether the set of values for D’ and D shall be the same, or whether the values for D’ shall represent a subset of the values for D.

- Proposals

  - Option 1 (Nokia): Introduce the following incremental delay values:

    - D’ is {100µs, 200µs} for UE indicating type1 in bwp-SwitchingDelay

    - D’ is {200µs} for UE indicating type2 in bwp-SwitchingDelay

  - Option 2 (MediaTek): Introduce the following incremental delay values:

    - D’ is {100µs, 200µs} for UE indicating type1 in bwp-SwitchingDelay

    - D’ is {200µs, 400µs} for UE indicating type2 in bwp-SwitchingDelay

  - Option 3 (vivo): Introduce the following incremental delay values:

    - D’ is {100µs, 200µs} for UE indicating type1 in bwp-SwitchingDelay

    - D’ is {200µs, 400µs, 800µs} for UE indicating type2 in bwp-SwitchingDelay

  - Option 4 (Ericsson, Qualcomm [R4-2102891], Apple, NEC): Introduce the following incremental delay values:

    - D’ is {100µs, 200µs} for UE indicating type1 in bwp-SwitchingDelay

    - D’ is {200µs, 400µs, 800µs, 1000µs} for UE indicating type2 in bwp-SwitchingDelay

- Discussion

  - E///: 4 companies are supporting Option 4 based on email discussion

  - MTK: Not clear why we need a new capability if we want to keep the same values. Some clarifications are encouraged. We can compromise to Option 3. Option 4 harms the feasibility of dormancy SCell switching

  - Nokia: Option 4 is acceptable. Suggest to discuss how we capture applicability (e.g. max number of CCs UE can activate simultaneously)

  - Vivo: Option 4 is same as multiple BWP switching. But the scenario is different since for this capability we have a single DCI and there is some room for optimization.

  - Apple: Prefer Option 4. New UE capability allows more flexibility. There is no ambiguity on how many CCs UE can handle.

  - QC: Option 4. Can compromise to Option 3.

  - E///: Prefer Option 4. Option is ok as well.

- Agreements

  - Introduce the following incremental delay values:

    - D’ is {100µs, 200µs} for UE indicating type1 in bwp-SwitchingDelay

    - D’ is {200µs, 400µs, 800µs, 1000µs} for UE indicating type2 in bwp-SwitchingDelay

Issue 1-2-2: Incremental delay D’ and number of CCs for SCell dormancy switching

- Background

  - For some choices of D’, the time for switching on multiple CCs may exceed the maximum time that can be supported by other functions (e.g. HARQ feedback). See for instance discussions in LS out (R4-2012269) and LS reply (R1-2009575) on the matter.

  - In R4-2102256 it is proposed that, if needed (pending on outcome of Issue 1-2-1), it shall be specified for how many CCs switching can be supported.

  - Please note that the discussion here is solely whether to capture, somewhere, that there may be a limited applicability w.r.t. number of SCCs. Further discussions on whether such limitation shall be captured e.g. in the capability description are to be carried out in email thread [98e][114] R16_UE_ feature, Issue 1-1: New feature simultaneous dormant BWP switching.

- Proposals

  - Option 1 (Nokia): If it is agreed that a UE indicating type2 in bwp-SwitchingDelay can report a D’ larger than 200µs, then it shall be specified that for SCS 120kHz, the maximum number of supported CCs will depend on the reported value D’:

| D’ | 200µs | 400µs | 800µs | 1000µs | 200µs | 400µs | 800µs | 1000µs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DCI received within the first 3 symbols |  |  |  | DCI received after the first 3 symbols |  |  |  |
| #CC | 8 | 5 | 3 | 2 | 8 | 4 | 2 | 2 |

- Recommended WF

  - [Moderator] May need to be coordinated with discussions in thread [98e][218] NR_RRM_Enh_RRM_1 on BWP switching on multiple CCs. Where to capture, if agreed, is to be discussed in thread [98e][114] R16_UE_ feature.

- Discussion

  - E///: recommend a high-level description and discuss jointly with 1-1-2 in [218]

  - QC: there are many parameters. Even high-level description may cause ambiguity.

  - E///: we are also fine with not capturing it at all. Can we agree that this is similar issur of multiple BWP switching

  - Apple: agree to align with [218]. Our preference is not to add a description to RAN4 specs.

  - Nokia: Fine to continue discussion in [218]. We can also add a note that there may be certain restrictions on the number of CCs that can be switched simultaneously.

  - vivo: we are ok to put the description at least high-level but we do not need to put the exact number of CCs.

  - Chair: Continue discussion on a potential high-level note on possible constraints on number of CCs for dormant BWP switching in the thread [218] as a part of issue 1-1-2. Do not capture the exact maximum number of supported CCs. Do not further discuss in [211].

1389

#### 7.5.1	RF requirements maintenance  [LTE_NR_DC_CA_enh-Core]

R4-2102956	Email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][108] LTE_NR_DC_CA_enh_RF. The discussion thread was moderated by Christian Bergljung (Ericsson).

Decision: 		The document was revised to R4-2103298.

R4-2103298	Email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2102956)

Discussion:

The contribution summarised email discussion thread [98e][108] LTE_NR_DC_CA_enh_RF. The discussion thread was moderated by Christian Bergljung (Ericsson).

Decision: 		The document was noted.

R4-2103144	LS to RAN2 on single-uplink operation in more than one band pair of a band combination

Type: LS out		For: Approval
					to RAN2
					Source: RAN4

Discussion:

See email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF in R4-2103298.

Decision: 		The document was approved.

R4-2103145	LS to RAN2 on single-uplink operation

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: Huawei

Discussion:

See email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF in R4-2103298.

Decision: 		The document was revised to R4-2103365.

R4-2103365	LS to RAN2 on single-uplink operation

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: RAN4

(Replaces R4-2103145)

Discussion:

See email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF in R4-2103298. The document was subject for post-meeting email approval.

Decision: 		The document was noted.

R4-2100798	MSD due to wider BW evaluation for DC_28_n5

Type: discussion		For: Approval
					38.101-3 v..
					Source: MediaTek Inc.

Discussion:

See email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF in R4-2103298.

Decision: 		The document was noted.

R4-2100844	CR for 38.101-3 Correction on EN-DC MSD due to cross band isolation for DC_28_n5  (R16)

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0449  Cat: F (Rel-16)

					Source: MediaTek Inc.

Discussion:

See email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF in R4-2103298.

Decision: 		The document was not pursued.

R4-2101820	Further discussion on RF requirements about Multi-RAT Dual-Connectivity

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Abstract:

ENDC TDM when roaming

Discussion:

See email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF in R4-2103298.

Decision: 		The document was noted.

R4-2102044	LS reply to RAN2 on power control for NR-DC

Type: LS out		For: Approval
					to RAN2
					Source: Ericsson

Abstract:

LS reply to RAN2 on power control for NR-DC

Discussion:

See email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF in R4-2103298.

Decision: 		The document was noted.

R4-2102207	CR to TS 38.307 on the definition of the duplex-mode for the band configurations

Type: CR		For: Agreement
					38.307 v15.7.0	  CR-0055  Cat: F (Rel-15)

					Source: ZTE Corporation, CHTTL

Discussion:

See email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF in R4-2103298.

Decision: 		The document was not pursued.

R4-2102208	CR to TS 38.307 on the definition of the duplex-mode for the band configurations

Type: CR		For: Agreement
					38.307 v16.5.0	  CR-0056  Cat: A (Rel-16)

					Source: ZTE Corporation, CHTTL

Discussion:

See email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF in R4-2103298.

Decision: 		The document was withdrawn.

R4-2102209	CR to TS 38.307 on the definition of the duplex-mode for the band configurations

Type: CR		For: Agreement
					38.307 v17.0.0	  CR-0057  Cat: A (Rel-17)

					Source: ZTE Corporation, CHTTL

Discussion:

See email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF in R4-2103298.

Decision: 		The document was withdrawn.

R4-2102408	Missing parent clause for NR-DC PCMAX

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0689  Cat: D (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF in R4-2103298.

Decision: 		The document was revised to R4-2103142.

R4-2103142	Missing parent clause for NR-DC PCMAX

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0689  rev 1 Cat: D (Rel-16)

					Source: Qualcomm Incorporated

(Replaces R4-2102408)

Discussion:

See email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF in R4-2103298.

Decision: 		The document was agreed.

R4-2102409	Missing parent clause for NR-DC PCMAX

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0690  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF in R4-2103298.

Decision: 		The document was agreed.

R4-2102711	Discussion and reply LS on p-NR-FR2

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF in R4-2103298.

Decision: 		The document was revised to R4-2103143.

R4-2103143	Discussion and reply LS on p-NR-FR2

Type: discussion		For: Discussion
					Source: vivo

(Replaces R4-2102711)

Discussion:

See email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF in R4-2103298.

Decision: 		The document was noted.

R4-2103373	Further Reply LS on power control for NR-DC

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: RAN4

Discussion:

See email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF in R4-2103298.

Decision: 		The document was approved.

R4-2102387	On SUO for intra-band EN-DC

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was noted.

R4-2101144	Discussion on the reply to LS on single UL operation

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 in R4-2103294.

Decision: 		The document was noted.

#### 7.5.2	RRM core requirements maintenance (38.133/36.133) [LTE_NR_DC_CA_enh-Core]

##### 7.5.2.1	Early Measurement reporting  [LTE_NR_DC_CA_enh-Core]

R4-2102252	Correction to Idle Mode CA/DC Measurements for Inactive mode

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1698  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was agreed.

R4-2102253	Correction to Idle Mode CA/DC Measurements for Inactive mode

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1699  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was agreed.

R4-2102744	Discussion on remaining issues in EMR requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was noted.

R4-2102745	CR on EMR requirement maintenance in 38.133

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1758  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was agreed.

R4-2102746	CR on EMR requirement maintenance in 38.133 R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1759  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was agreed.

R4-2102747	CR on EMR requirements in 36.133

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7064  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was revised to R4-2103727.

R4-2103727	CR on EMR requirements in 36.133

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7064  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2102747)

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was agreed.

R4-2102748	CR on EMR requirements in 36.133 R17

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7065  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was agreed.

##### 7.5.2.2	Efficient and low latency serving cell configuration, activation and setup [LTE_NR_DC_CA_enh-Core]

R4-2103552	WF on replacement of measCycleSCell in core requirements for Direct SCell activation

Type: other		For: discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was approved.

R4-2100227	Activation time in direct SCell activation

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was noted.

R4-2100228	CR on activation time in direct SCell activation

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1461  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was revised to R4-2103553.

R4-2103553	CR on activation time in direct SCell activation

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1461  rev 1 Cat: F (Rel-16)

					Source: Apple

(Replaces R4-2100228)

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was revised to R4-2104071.

R4-2104071	CR on activation time in direct SCell activation

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1461  rev 2 Cat: F (Rel-16)

					Source: Apple

(Replaces R4-2103553)

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was postponed.

R4-2100229	CR on activation time in direct SCell activation (R17)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1462  Cat: A (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was withdrawn.

R4-2101012	CR on activation time in direct SCell activation (R17)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1543  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was withdrawn.

R4-2101212	Discussion on direct Scell activation and dormancy Scell

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was noted.

R4-2101213	CR on TS38.133 for direct Scell activation

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1579  Cat: F (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was postponed.

R4-2101214	CR on TS38.133 for direct Scell activation

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1580  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was withdrawn.

R4-2101377	Remaining issues for Scell dormancy RRM requirements

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was noted.

R4-2101388	CR for adding capability D

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1593  Cat: F (Rel-16)

					Source: vivo

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was not pursued.

R4-2101389	CR for adding capability D

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1594  Cat: A (Rel-17)

					Source: vivo

Discussion:

Session chair: Cover sheet errors (What is the release? It reads Rel-16 on the cover page but the Tdoc is reserved for Rel-17.). See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was not pursued.

R4-2102254	CR clarifying the UE measurement requirements for an SCell with dormant BWP

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1700  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was agreed.

R4-2102255	CR clarifying the UE measurement requirements for an SCell with dormant BWP

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1701  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was agreed.

R4-2102256	Discussion related to multiple SCell dormancy BWP switch

Type: discussion		For: Approval
					38.133 v..
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was noted.

R4-2102257	Correction to simultaneous DCI based BWP switch delay on multiple CCs

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1702  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was agreed.

R4-2102258	Correction to simultaneous DCI based BWP switch delay on multiple CCs

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1703  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was agreed.

R4-2102351	On incremental delay for dormancy switching on multiple CCs

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion on separate capability on incremental delay for SCell dormancy switching on multiple CCs.

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was noted.

R4-2102352	CR 38.133 (8.2 8.6) Corrections related to SCell dormancy switching

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1718  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

Adding separate capability for incremental delay for SCell dormancy switching on multiple CCs. Removal of brackets for interruption due to RRM measurements on dormant SCell.

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was merged.

R4-2102353	CR 38.133 (8.2 8.6) Corrections related to SCell dormancy switching

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1719  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Adding separate capability for incremental delay for SCell dormancy switching on multiple CCs.

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was withdrawn.

R4-2102366	CR 36.133 Removal of brackets for SCell dormancy

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7041  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

Removal of brackets for interruption due to RRM measurements on dormant SCell.

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was agreed.

R4-2102367	CR 36.133 Removal of brackets for SCell dormancy

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7042  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Removal of brackets for interruption due to RRM measurements on dormant SCell.

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was agreed.

R4-2102749	CR on SCell dormancy switching

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1760  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was revised to R4-2103554.

R4-2103554	CR on SCell dormancy switching

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1760  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2102749)

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was agreed.

R4-2102750	CR on SCell dormancy switching R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1761  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was agreed.

R4-2102881	Cat-F CR to Removal of brackets for SCell Dormancy and Simultaneous DCI based BWP switch delay on multiple CCs in Rel-16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1785  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was merged.

R4-2102882	Cat-A CR to Removal of brackets for SCell Dormancy and Simultaneous DCI based BWP switch delay on multiple CCs in Rel-17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1786  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was withdrawn.

R4-2102883	Cat-F CR to Removal of brackets for SCell Dormancy and Simultaneous DCI based BWP switch delay on multiple CCs in Rel-16 LTE

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7080  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was merged.

R4-2102884	Cat-A CR to Removal of brackets for SCell Dormancy and Simultaneous DCI based BWP switch delay on multiple CCs in Rel-17 LTE

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7081  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was withdrawn.

#### 7.5.3	RRM perf. requirements (38.133) [LTE_NR_DC_CA_enh-Perf]

R4-2103547	WF on Test cases for MR-DC Idle mode CA measurements

Type: other		For: discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was approved.

##### 7.5.3.1	Early Measurement reporting  [LTE_NR_DC_CA_enh- Perf]

###### 7.5.3.1.1	Accuracy requirements  [LTE_NR_DC_CA_enh-Perf]

R4-2102262	Draft Big CR: Introduction of Rel-16 MR-DC EMR RRM performance requirements (TS 38.133)

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was revised to R4-2103548.

R4-2103548	Draft Big CR: Introduction of Rel-16 MR-DC EMR RRM performance requirements (TS 38.133)

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102262)

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2103555	Draft Big CR: Introduction of Rel-16 MR-DC EMR RRM performance requirements (TS 36.133)

Type: draftCR		For: Endorsement
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was withdrawn.

R4-2102263	Draft CR for Removal of brackets for idle mode CA measurement accuracy requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was revised to R4-2103549.

R4-2103549	Draft CR for Removal of brackets for idle mode CA measurement accuracy requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102263)

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was endorsed.

R4-2102751	draftCR on accuracy requirements for EMR 38.133

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was endorsed.

R4-2102752	Big CR: Introduction of Rel-16 MR-DC EMR RRM performance requirements (TS 36.133)

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7066  Cat: B (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was postponed.

R4-2102753	Big CR: Introduction of Rel-16 MR-DC EMR RRM performance requirements (TS 36.133)

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7067  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was withdrawn.

###### 7.5.3.1.2	Test cases [LTE_NR_DC_CA_enh-Perf]

R4-2100232	Test case for EMR with both PCell and target cell in FR1

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Apple

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was revised to R4-2103550.

R4-2103550	Test case for EMR with both PCell and target cell in FR1

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Apple

(Replaces R4-2100232)

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was endorsed.

R4-2102261	Draft CR for Idle Mode measurements of inter-frequency CA candidate cells for early reporting

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was endorsed.

R4-2102264	Measurement Performance Requirements test for MR-DC

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was noted.

R4-2102754	draftCR to introduce TC4 for EMR

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was revised to R4-2103551.

R4-2103551	draftCR to introduce TC4 for EMR

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2102754)

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was endorsed.

R4-2102885	Discussion on Cell Configuration for EMR Test Case

Type: discussion		For: Discussion
					38.133 v..
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1 in R4-2103692.

Decision: 		The document was noted.

##### 7.5.3.2	Efficient and low latency serving cell configuration, activation and setup [LTE_NR_DC_CA_enh-Perf]

R4-2103563	WF on test cases for SCell dormancy

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was approved.

R4-2103556	Draft Big CR: Introduction of Rel-16 MR-DC Direct SCell activation and SCell dormancy RRM performance requirements (TS 38.133)

Type: draftCR		For: Endorsement
					Source: Ericsson

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

###### 7.5.3.2.1	Test cases for direct SCell activation [LTE_NR_DC_CA_enh-Perf]

R4-2100230	Test case for Direct SCell Activation: EN-DC, NR spCell in FR1, SCell in FR1, SCell addition

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Apple

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was revised to R4-2103557.

R4-2103557	Test case for Direct SCell Activation: EN-DC, NR spCell in FR1, SCell in FR1, SCell addition

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Apple

(Replaces R4-2100230)

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was endorsed.

R4-2101073	Draft CR on TC for direct SCell activation during handover in NR SA for FR2

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: NEC

Abstract:

TC for direct SCell activation during handover in NR SA for FR2 is added/specified

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was revised to R4-2103562.

R4-2103562	Draft CR on TC for direct SCell activation during handover in NR SA for FR2

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: NEC

(Replaces R4-2101073)

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was endorsed.

R4-2101215	CR on TS38.133 for direct SCell activation of SCell in FR2 intra-band in ENDC mode (A.5.5.X)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1581  Cat: B (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was revised to R4-2103558.

R4-2103558	CR on TS38.133 for direct SCell activation of SCell in FR2 intra-band in ENDC mode (A.5.5.X)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1581  rev 1 Cat: B (Rel-16)

					Source: MediaTek inc.

(Replaces R4-2101215)

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was endorsed.

R4-2101216	CR on TS38.133 for direct SCell activation of SCell in FR2 intra-band in ENDC mode (A.5.5.X)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1582  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was withdrawn.

R4-2102260	Draft CR for NR FR1 Intra frequency handover with direct SCell activation

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was revised to R4-2103561.

R4-2103561	Draft CR for NR FR1 Intra frequency handover with direct SCell activation

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102260)

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was endorsed.

R4-2102357	On TC3 for Direct SCell Activation

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on test cases for direct SCell activation

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was noted.

R4-2102358	DraftCR 38.133 TC3 Direct SCell activation

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

TC3 for direct SCell activation

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was revised to R4-2103559.

R4-2103559	DraftCR 38.133 TC3 Direct SCell activation

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2102358)

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was endorsed.

R4-2102755	draftCR to introduce TC4 for direct SCell activation

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was revised to R4-2103560.

R4-2103560	draftCR to introduce TC4 for direct SCell activation

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2102755)

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was endorsed.

###### 7.5.3.2.2	Test case for SCell Dormancy [LTE_NR_DC_CA_enh-Perf]

R4-2100231	Test case for SCell Dormancy: EN-DC, NR spCell in FR1, SCell FR1, DCI 2_6 within/after 3 OFDM symbols

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Apple

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was revised to R4-2103564.

R4-2103564	Test case for SCell Dormancy: EN-DC, NR spCell in FR1, SCell FR1, DCI 2_6 within/after 3 OFDM symbols

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Apple

(Replaces R4-2100231)

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was endorsed.

R4-2101074	Draft CR on TC for SCell dormancy in NR SA for FR1

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: NEC

Abstract:

TC for SCell dormancy in NR SA for FR1 is specified

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was revised to R4-2103568.

R4-2103568	Draft CR on TC for SCell dormancy in NR SA for FR1

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: NEC

(Replaces R4-2101074)

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was endorsed.

R4-2101217	CR on TS38.133 for E-UTRAN

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1583  Cat: B (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was revised to R4-2103565.

R4-2103565	CR on TS38.133 for E-UTRAN

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1583  rev 1 Cat: B (Rel-16)

					Source: MediaTek inc.

(Replaces R4-2101217)

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was endorsed.

R4-2101218	CR on TS38.133 for E-UTRAN

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1584  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was withdrawn.

R4-2102259	Draft CR for test case 7 for Dormant SCell BWP switch delay

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was revised to R4-2103570.

R4-2103570	Draft CR for test case 7 for Dormant SCell BWP switch delay

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102259)

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was endorsed.

R4-2102359	On TCs 6 and 8 for SCell dormancy switching

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on test cases for SCell dormancy

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was noted.

R4-2102360	DraftCR 38.133 TCs 6 and 8 SCell dormancy switching

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

TCs 6 and 8 for SCell dormancy

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was revised to R4-2103569.

R4-2103569	DraftCR 38.133 TCs 6 and 8 SCell dormancy switching

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2102360)

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was endorsed.

R4-2102756	Further discussion on test for SCell dormancy

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was noted.

R4-2102757	draftCR to introduce TC3 for SCell dormancy

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was revised to R4-2103566.

R4-2103566	draftCR to introduce TC3 for SCell dormancy

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2102757)

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was endorsed.

R4-2102886	DraftCR on EN-DC NR SpCell in FR1 and 2 NR SCells in FR2 for Dormant SCell switch via DCI 2_6 within and after 3 OFDM symbols

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was revised to R4-2103567.

R4-2103567	DraftCR on EN-DC NR SpCell in FR1 and 2 NR SCells in FR2 for Dormant SCell switch via DCI 2_6 within and after 3 OFDM symbols

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Qualcomm Incorporated

(Replaces R4-2102886)

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was endorsed.

R4-2102887	Test framework for SCell dormancy performance requirements

Type: discussion		For: Discussion
					38.133 v..
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][211] LTE_NR_DC_CA_RRM_2 in R4-2103693.

Decision: 		The document was noted.

### 7.6	UE power saving in NR  [NR_UE_pow_sav]

#### 7.6.1	RRM requirements maintenance (38.133) [NR_UE_pow_sav-Core/Perf]

R4-2103451	Email discussion summary: [98e][212] NR_UE_pow_sav_RRM

Type: other		For: discussion
					Source: Moderator (CATT)

Discussion:

The contribution summarised email discussion thread [98e][212] NR_UE_pow_sav_RRM. The discussion thread was moderated by Yanze Fu (Datang).

Decision: 		The document was revised to R4-2103694.

R4-2103694	Email discussion summary: [98e][212] NR_UE_pow_sav_RRM

Type: other		For: discussion
					Source: Moderator (CATT)

(Replaces R4-2103451)

Discussion:

The contribution summarised email discussion thread [98e][212] NR_UE_pow_sav_RRM. The discussion thread was moderated by Yanze Fu (Datang).

Decision: 		The document was noted.

R4-2103571	LS on RRM relaxation in power saving

Type: LS out		For: Approval
					to -
					Source: CATT

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was withdrawn.

R4-2103573	WF on remaining issue for power saving RRM

Type: other		For: discussion
					Source: CATT

Discussion:

vivo: Misalignment between RAN2/RAN4 specifications. If we don’t update RAN4 specs, then we’ll need to update RAN2 specs.

E///: Keep current RAN4 specification and inform RAN2 to update their spec.

Session chair: For issue “Whether to change “K2* Thigher_priority_search” to “1 hour” in Slide 2, continue discussion in RAN4 #98-bis-e. If no consensus is reached to modify RAN4 specification, then LS to RAN2 shall be sent to inform on mismatch in RAN4 and RAN2 specs.

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was approved.

R4-2100473	Discussion on remaining issues for UE power saving test case

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was noted.

R4-2100482	Correction to cell reselection test case for UE Power saving

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1497  Cat: F (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was revised to R4-2103574.

R4-2103574	Correction to cell reselection test case for UE Power saving

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1497  rev 1 Cat: F (Rel-16)

					Source: CATT

(Replaces R4-2100482)

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was agreed.

R4-2100483	Correction to cell reselection test case for UE Power saving

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1498  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was agreed.

R4-2100727	CR on RRM test cases for NR UE power saving

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was merged.

R4-2101383	CR for removing K2 for R16 UE power saving

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1589  Cat: F (Rel-16)

					Source: vivo

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was postponed.

R4-2101384	CR for removing K2 for R16 UE power saving

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1590  Cat: A (Rel-17)

					Source: vivo

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was postponed.

R4-2101385	CR for modifications on FR1 intra-frequency UE power saving test cases

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1591  Cat: F (Rel-16)

					Source: vivo

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was merged.

R4-2101386	CR for modifications on FR1 intra-frequency UE power saving test cases

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1592  Cat: A (Rel-17)

					Source: vivo

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Session chair: Cover sheet errors (What is the release? It reads Rel-16 on the cover page but the Tdoc is reserved for Rel-17. What is the CR category? It reads F on the cover page but the Tdoc is reserved for category A.)

Session chair: tdoc will be postponed due to CR cover sheet issues.

Decision: 		The document was postponed.

R4-2101624	Correction to relexed cell reselection requirements R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1626  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was merged.

R4-2101625	Correction to relexed cell reselection requirements R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1627  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was withdrawn.

R4-2101834	Correction on inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1673  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was revised to R4-2103572.

R4-2103572	Correction on inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1673  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101834)

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was agreed.

R4-2101881	Correction on inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1690  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was agreed.

R4-2101835	Test case for cell reselection to FR2 intra-frequency NR case for UE configured with relaxed measurement

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1674  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was revised to R4-2103575.

R4-2103575	Test case for cell reselection to FR2 intra-frequency NR case for UE configured with relaxed measurement

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1674  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101835)

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was agreed.

R4-2101836	Test case for cell reselection to FR2 intra-frequency NR case for UE configured with relaxed measurement

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1675  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was agreed.

R4-2102246	Changes to cell reselection tests under power saving

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1695  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

The cell reselection test cases contain square brackets which for the signal levels which are removed. Signal levels are checked and no need to further modify them.

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was postponed.

R4-2103576	Changes to cell reselection tests under power saving

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1695  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2102246)

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was withdrawn.

R4-2102245	Changes to cell reselection tests under power saving

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1694  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

The cell reselection test cases contain square brackets which for the signal levels which are removed. Signal levels are checked and no need to further modify them.

Discussion:

See email discussion summary for [98e][212] NR_UE_pow_sav_RRM in R4-2103694.

Decision: 		The document was withdrawn.

#### 7.6.2	Demodulation and CSI requirements (38.101-4)  [NR_UE_pow_sav-Perf]

R4-2103760	Email discussion summary for [98e][321] NR_UE_pow_sav_Demod

Type: other		For: discussion
					Source: Moderator (CATT)

Discussion:

The contribution summarised email discussion thread [98e][321] NR_UE_pow_sav_Demod. The discussion thread was moderated by Yuexia Song (CATT).

Decision: 		The document was revised to R4-2103936.

R4-2103936	Email discussion summary for [98e][321] NR_UE_pow_sav_Demod

Type: other		For: discussion
					Source: Moderator (CATT)

(Replaces R4-2103760)

Discussion:

The contribution summarised email discussion thread [98e][321] NR_UE_pow_sav_Demod. The discussion thread was moderated by Yuexia Song (CATT).

Decision: 		The document was noted.

R4-2103821	WF on remaining issue for UE power saving

Type: other		For: discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][321] NR_UE_pow_sav_Demod in R4-2103936. Session Chair note: The sub-clauses for requirements structure in slide 4 has been updated as following in agreed CRs: 5.3.2.1.3/5.3.2.2.3/5.3.3.1.3/5.3.3.2.3/7.3.2.2.3  5.1.1.3/7.1.1.3/5.3.2.1.3/5.3.3.1.3/5.3.2.2.3/5.3.3.2.3/7.3.2.2.3

Decision: 		The document was approved.

------------------ GTW agenda for Jan.28th email thread [321] (30miniutes) --------------------------

Issue 1-1-1:  DCI length (excluding 24bits CRS) + Aggregation level  （FR1 FDD）

  - Option 1: 12 bits, AL=8（CATT，vivo, Qualcomm，Apple，intel，MediaTek，Huawei）

  - Option 2: 36bits, AL=16 (CATT，CMCC)

Agreement: Option 1: 12 bits, AL=8

CMCC: We prefer to have separate configuration for duplex mode and the performance is similar but WUS payload flexible and with different DCI size can cover more cases.

Intel: We have same configurations for FR1. We need to align the set-up. Not seen any specific technical issue with same DCI size.

QC: Similar as Intel, the SNR values with AL=8 good enough why NW to configure AL 16 without performance benefits.

CATT: We are fine for both options. Both options can fulfil test purpose, we agree with CMCC for test coverage.

CMCC: The configurations not always same for FDD and TDD; PDCCH configuration is different for PDSCH test cases.

We can agree with option 1 with FDD 10MHz, for FR1 TDD with 40MHz, AL 16 can be used.

Huawei: Share similar view as QC, from UE performance aspect, if UE can achieve performance with low AL 8, then should no problem for AL 16.

CMCC: The payload size also important. We think option 2 in TDD more reasonable.

CATT: We would like to complete this WI in time, even we understand the concern from companies.

Issue 1-1-2:  DCI length (excluding 24bits CRS) + Aggregation level  （FR1 TDD）

  - Option 1: 12 bits, AL=8 (CATT，vivo，Qualcomm，Apple，Intel，CMCC，MediaTek, Huawei)

  - Option 2: 36 bits, AL 16 (CATT)

Agreement: Option 1: 12 bits, AL=8

CMCC: We have concern for this considering test coverage issue, we can comprise for progress.

Issue 1-1-3:  DCI length (excluding 24bits CRS) + Aggregation level  （FR2 TDD）

  - Option 1: 12 bits, AL=8 (Qualcom，Apple，Intel，CMCC，MediaTek, Huawei)

  - Option 2: 36 bits, AL=8 (Intel，CMCC，CATT)

Agreement: 12 bits, AL =8

Issue 1-2-1:  where to capture UE power saving requirements？

  - Option 1: separate section under 2Rx/4Rx requirements

  - Option 2: separate section under 1Tx requirements（QC）

    - Option 2a: put the requirements in 1Tx section without new clauses

  - Option 3: separate section at the level of 2Rx/4Rx requirements （CATT，QC，Apple，vivo，CMCC）

    - E.g. new sub-clause 5.3.4/7.3.4 for power saving

  - Option 4: Create sub-clauses 5.3A/7.3A or 5.6/7.6（Ericsson）

  - Option 5: Create sub-clauses 5.3.2.1.3/5.3.2.2.3/5.3.3.1.3/5.3.3.2.3/7.3.2.2.3  Minimum requirements for power saving （Huawei，CMCC， MediaTek, Intel,QC）

  - Option 6: Restructure of normal PDCCH and joint PDCCH/PDCCH-WUS (CMCC), e.g.

5.3 PDCCH demodulation requirements

5.3.1 PDCCH demodulation requirements for normal PDCCH

5.3.2 PDCCH demodulation requirements for power saving

Intel: We think option 5 aligned with draft structure.

QC: We are fine option 2 and option 5. But for option 5, 1Tx is not feature. But Option 5 not strictly follow the structure properly.

CATT: we think option 5 could work, we put joint performance in parallel with 1Tx. Can we consider option 4?

Samsung (Spec editor): if new section needed for this feature, then this should be paced in deepest level sub-section.

Agreements:

Option 5, Create sub-clauses 5.3.2.1.3/5.3.2.2.3/5.3.3.1.3/5.3.3.2.3/7.3.2.2.3 Minimum requirements for power saving

Session chair note: Whether separate sub-clauses need or not for requirements introduced for specific features in future will be discussed in case by case manner with the consideration of maintaining the consistent of spec structure; if sub-clauses generated, it’s recommended placed in deepest sub-clause level if feasible

Issue 1-2-2:  where to put the applicability rules “UE fulfilling the power saving test cases can skip the corresponding normal PDCCH test cases”

  - Option 1: In General clause （CATT，vivo，Huawei，QC，Apple，CMCC，MediaTek）

  - Option 2: In each section together with the PDCCH/PDCCH-WUS requirements. （CATT，vivo，CMCC）

Agreements: Option 1 in general clause

Companies are encouraged to further work on the texts for applicable rules introduced into specification.

----------------------------End --------------------------------------

R4-2100397	Discussion on power saving demodulation test

Type: discussion		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][321] NR_UE_pow_sav_Demod in R4-2103936.

Decision: 		The document was noted.

R4-2100398	CR for TS38.101-4, test for FR2 PDCCH DCI format 2_6 demodulation

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0129  Cat: B (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][321] NR_UE_pow_sav_Demod in R4-2103936.

Decision: 		The document was revised to R4-2103822.

R4-2103822	CR for TS38.101-4, test for FR2 PDCCH DCI format 2_6 demodulation

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0129  rev 1 Cat: B (Rel-16)

					Source: CATT

(Replaces R4-2100398)

Discussion:

See email discussion summary for [98e][321] NR_UE_pow_sav_Demod in R4-2103936.

Decision: 		The document was agreed.

R4-2100541	Views on UE Power Saving Test Cases

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][321] NR_UE_pow_sav_Demod in R4-2103936.

Decision: 		The document was noted.

R4-2100817	Simulation results summary for R16 power saving demodulation

Type: other		For: Information
					Source: CMCC

Discussion:

See email discussion summary for [98e][321] NR_UE_pow_sav_Demod in R4-2103936.

Decision: 		The document was noted.

R4-2100818	CR for TS38.101-4, test for FR1 TDD PDCCH DCI format 2_6 demodulation

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0131  Cat: B (Rel-16)

					Source: CMCC

Discussion:

See email discussion summary for [98e][321] NR_UE_pow_sav_Demod in R4-2103936.

Decision: 		The document was revised to R4-2103823.

R4-2103823	CR for TS38.101-4, test for FR1 TDD PDCCH DCI format 2_6 demodulation

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0131  rev 1 Cat: B (Rel-16)

					Source: CMCC

(Replaces R4-2100818)

Discussion:

See email discussion summary for [98e][321] NR_UE_pow_sav_Demod in R4-2103936.

Decision: 		The document was agreed.

R4-2100820	Demodulation on UE power saving

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][321] NR_UE_pow_sav_Demod in R4-2103936.

Decision: 		The document was noted.

R4-2101241	Discussion on PDCCH-WUS requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][321] NR_UE_pow_sav_Demod in R4-2103936.

Decision: 		The document was noted.

R4-2101295	Discussion and simulation results on the performance requirements for NR power saving

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][321] NR_UE_pow_sav_Demod in R4-2103936.

Decision: 		The document was noted.

R4-2101387	CR on Fixed reference channel for power saving performance

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0154  Cat: F (Rel-16)

					Source: vivo

Discussion:

See email discussion summary for [98e][321] NR_UE_pow_sav_Demod in R4-2103936.

Decision: 		The document was revised to R4-2103824.

R4-2103824	CR on Fixed reference channel for power saving performance

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0154  rev 1 Cat: F (Rel-16)

					Source: vivo

(Replaces R4-2101387)

Discussion:

See email discussion summary for [98e][321] NR_UE_pow_sav_Demod in R4-2103936.

Decision: 		The document was agreed.

R4-2102085	Simulation results on PDCCH-WUS for power saving

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][321] NR_UE_pow_sav_Demod in R4-2103936.

Decision: 		The document was noted.

R4-2102086	CR for TS38.101-4, test for FR1 FDD PDCCH DCI format 2_6 demodulation

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][321] NR_UE_pow_sav_Demod in R4-2103936.

Decision: 		The document was withdrawn.

R4-2103932	CR for TS38.101-4, test for FR1 FDD PDCCH DCI format 2_6 demodulation

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0171  Cat: F (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][321] NR_UE_pow_sav_Demod in R4-2103936.

Decision: 		The document was agreed.

### 7.7	NR Positioning Support [NR_pos]

#### 7.7.1	RRM core requirements maintenance (38.133) [NR_pos-Core]

R4-2103452	Email discussion summary: [98e][213] NR_pos_RRM_1

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][213] NR_pos_RRM_1. The discussion thread was moderated by Li Zhang (Huawei).

Decision: 		The document was revised to R4-2103695.

R4-2103695	Email discussion summary: [98e][213] NR_pos_RRM_1

Type: other		For: discussion
					Source: Moderator (Huawei)

(Replaces R4-2103452)

Discussion:

The contribution summarised email discussion thread [98e][213] NR_pos_RRM_1. The discussion thread was moderated by Li Zhang (Huawei).

Decision: 		The document was noted.

GTW session (January 28, 2021)

Issue 1-1-1: Consideration on PRS resource muting

- Option 1a (HW, OPPO, vivo)

  - If muting option 1 is applied, the periodicity of a PRS resource is scaled by Nmuting

    - where Nmuting is X * dl-prs-MutingBitRepetitionFactor, and X is the size of NR-MutingPattern-r16 for mutingOption1-r16

- Option 1b (QC, Intel)

  - If muting option 1 is applied, the periodicity of a PRS resource is scaled by Nmuting

where Nmuting is X * dl-prs-MutingBitRepetitionFactor, and X is the number of consecutive zeros in NR-MutingPattern-r16 for mutingOption1-r16

- Option 1c (Intel)

  - X is the maximum number of consecutive zeros in NR-MutingPattern-r16 for mutingOption1-r16

- Option 2 (E///, CATT): LTE approach

- Option 3 (Intel): Do not define requirements for the case of PRS resource muting in Rel-16

Discussion

- Intel: can we focus on the no muting case in Rel-16 and deprioritize the work on muting

- E///: need to discuss no muting first. Concerns on way of working. Option 1 provides unnecessary scaling. Consider capability and MG issue.

- Huawei: we are open to Intel’s suggestion to focus on no muting case in Rel-16. On Option 2 – it may not be applicable. Disagree that NR requirements are much worse than LTE.

- vivo: Muting case requirements need to be defined. The PRS configuration is cell-specific and MG is UE specific. The PRS periodicity originally did not account MG periodicity. Therefore, it is important to have repetitions. Measurement delay depends on configuration.

- QC: Limiting the scope of Rel-16 can be discussed. Type 1 muting needs to be accounted since it affects the amount of resources and Type 2 muting may not need to be taken into account.

- Intel: In case PRS muting is applied, then typically the positioning performance will be better. So, we think that no muting case is the worst case.

- Session chair: continue the discussion

Issue 1-1-2: Consideration on different resource periodicities

- Option 1a (CATT, vivo, QC, Intel)

  - Use the least common multiple of PRS periodicities among all PRS resources in the PFL

- Option 1b (OPPO, QC?)

  - Use the least common multiple of PRS periodicities among all PRS resources in the PFL, where only the PRS resources or resource sets within the MGs should be considered

- Option 1c (Huawei)

  - Use the least common multiple of PRS periodicities after muting among all PRS resources in the PFL, where only the PRS resources or resource sets within the MGs should be considered

  - The measurement requirements apply provided that, the resource periodicities after muting are either <= 160ms for all PRS resources on the PFL, or > 160ms for all PRS resources on the PFL.

- Option 2a (Intel)

  - Use the maximum PRS resource periodicity among all PRS resource in a same positioning frequency layer.

- Option 2b (Ericsson)

  - Clarify in RSTD measurement period requirements that the measured PRS resources shall be contained in at least some MGs

  - No need to restrict PRS periodicity to be a multiple of 5 ms.

  - TPRS,i is the longest PRS periodicity (of PRS resources contained within at least some measurement gaps) on that carrier.

- Option 3 (QC)

  - Measurement requirements would not apply to PRS periodicities equal to 4, 8, 16, 32 and 64 ms in Rel-16

  - Adopt option 1a or option 1b (which are equivalent with above bullet)

- Option 4 (HW)

  - RSTD measurement requirements apply provided that all PRS resources on a PRS frequency layer have same periodicity after muting

Discussion

  - Huawei: For Option 1b there were suggestions to clarify “count only PRS resources that have at least some instances fully contained within the configured MG pattern”

  - QC: Option1a is a basic approach. Option1b is a kind of optimization

  - Intel: Option1a is fine for us.

  - E///: It is not sufficient to say PRS within MG. MGRP needs to be considered. Max(PRS periodicity, MGRP) needs to be considered.

  - QC: it is given that MG is configured. Option 1b is an optimization.

  - Vivo: for the case of LCM (option 1) – all resources can be measured, but for the case Max(PRS periodicity, MGRP) some resources may not be measured. Both can work.

  - E///: Network knows only on the configured PRS. Assumptions between gNB and UE need to be aligned.

  - Huawei/QC/Intel: Option 2 is redundant and MGRP is already considered in the existing requirements.

  - OPPO: our intention is to always exclude PRS resources outside the gap. We are ok with Option 1a as well.

  - E///: for PRS periodicities – are these configured or these are PRS resources in gaps

    - QC: these are configured PRS periodicities

Agreements

  - Use the least common multiple of PRS periodicities among all PRS resources in the PFL

    - FFS: whether only the PRS resources or resource sets configured within the MGs should be considered

Issue 1-3-1: Definition of parameter L_(PRS,i)

- Option 1 (vivo)

  - Lprs is the time duration of available number of PRS symbols or number of slots during Teffect depending on the type that UE used to report {N, T}

- Option 2 (HW, CATT, Intel)

  - Refer to clause 5.1.6.5 of 38.214 for definition of calculation of Lprs.

- Option 3 (Ericsson)

  - LPRS,i is the size of the downlink PRS resource in the time domain defined in TS 38.211 [6] and indicated by the higher-layer parameter dl-PRS-NumSymbols specified in TS 37.355.

Discussion

  - vivo: 38.214 does not include clear definition of Lprs

  - HW: we can clarify 38.133 to provide more clear references to RAN1 specs. Can combine Option 1 and Option 2.

  - E///: there are several issues 1) references 2) notations. For Option 2 we may need to change the notation.

  - QC: we are talking about requirements.

  - HW: we are fine to change the notation if it is confusing. Suggest to focus on Option 2 and further discuss the clarifications.

Agreements

  - Refer to clause 5.1.6.5 of 38.214 for calculation of Lprs.

  - Further clarify the description and notations of Lprs in 38.133 (e.g. account PRS resources within MGs; clarify period of time over which Lprs is counted)

Issue 1-4-1: Basic principle (sum approach v.s. max approach)

- Option 1 (Intel, vivo, QC, HW, Nokia)

  - Measurement period of multiple PRS layers is defined as summation of the measurement period in each frequency layer

  - CSSF is only for the MG sharing between PRS and RRM layers. Count only a single PRS layer for a gap occasion in CSSF calculation for both PRS and RRM layers.

- Option 2a (OPPO)

  - Measurement period of multiple PRS layers is defined as maximum of the measurement period in each frequency layer

  - CSSF is based on Rel-15 CSSF concept and all PRS layers should be counted

  - For long PRS processing time cases, the following scaling factor k should be added

TPRS-RSTD,i=’CSSFPRS,i*k*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast

    - The scaling factor k should be applied to all PRS layer involved in the collision,

      - PRS layer i whose processing time covers other PRS instances and

      - PRS layer j whose PRS instance is covered by the processing time of PRS layer i

    - k is the number of colliding MGs due to long processing time

  - If more than one long-periodicity PRS layers are configured, the same MG competition rules as short-periodicity PRS layers could be reused.

- Option 2b (Ericsson)

  - CSSF is the NR concept which is used for all types of measurements including RRM, scaling based on the number of frequency layers is the LTE concept.

  - Hence, for the overlapping case, CSSF shall be used in the requirements, but  over frequency layers shall be replaced with the max operator:

TRSTD, Total = maxi (TRSTD,i).

Discussion

  - Moderator: no changes of views in the 1st round.

  - Session chair: what happens if we don’t make agreement

    - Huawei: by default Option 1 is assumed in the spec but there may be ambiguity or FFS in some parts of the spec

  - E///: in the last meeting we brought some issues which were not resolved. TRSTD,i becomes the measurement period. What we can be ok with the following equation

    - TRSTD, Total = i’CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast)

  - HW: it is more like terminology issue. Our preference is to keep using TRSTD,i but to clarify the definition

  - QC: do not see much difference between the current equation and new equation

Agreements

  - Measurement period of multiple PRS layers is defined as summation of the measurement period in each frequency layer

  - CSSF is only for the MG sharing between PRS and RRM layers. Count only a single PRS layer for a gap occasion in CSSF calculation for both PRS and RRM layers.

  - FFS how to capture the equations in the specifications

    - Option 1A:

      - TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i

      - TPRS-RSTD,i=’CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast

      - Note: TPRS-RSTD,i is already defined in the specification

    - Option 1B

      - TRSTD, Total = i’CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast)+L-1*maxTeffect,i

      - Note: TPRS-RSTD,i needs to be removed from the specification

R4-2103577	WF on UE PRS measurement requirements

Type: other		For: discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was revised to R4-2104076.

R4-2104076	WF on UE PRS measurement requirements

Type: other		For: discussion
					Source: Huawei, HiSilicon

(Replaces R4-2103577)

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was approved.

##### 7.7.1.1	PRS-RSTD measurement requirements [NR_pos-Core]

R4-2100436	Discussion on PRS RSTD measurement requirements

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2100438	CR on PRS RSTD measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1477  Cat: F (Rel-16)

					Source: CATT

Discussion:

Session chair: Cover sheet errors (What is the work item? It reads NR_pos-Perf but the Tdoc is reserved for NR_pos-Core.). See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was revised to R4-2103578.

R4-2103578	CR on PRS RSTD measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1477  rev 1 Cat: F (Rel-16)

					Source: CATT

(Replaces R4-2100438)

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was agreed.

R4-2100468	CR on PRS RSTD measurement requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1489  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was agreed.

R4-2101272	Discussion on NR PRS RSTD measurement report requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2101526	Further discussion on maintenance for RSTD measurement requirement

Type: discussion		For: Discussion
					Source: OPPO

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2101776	Discussion on PRS RSTD measurement requirements

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2101779	CR to 38.133 correction to PRS RSTD measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1663  Cat: F (Rel-16)

					Source: vivo

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was merged.

R4-2101780	CR to 38.133 correction to PRS RSTD measurement requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1664  Cat: A (Rel-17)

					Source: vivo

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was withdrawn.

R4-2102290	On PRS-RSTD measurement requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2102298	Revision of PRS-RSTD measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1706  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was merged.

R4-2102299	Revision of PRS-RSTD measurement requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1707  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was withdrawn.

R4-2102539	On RSTD measurement requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On RSTD measurement requirements

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2102540	RSTD measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1730  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

RSTD measurement requirements

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was merged.

R4-2102541	RSTD measurement requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1731  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

RSTD measurement requirements

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was withdrawn.

R4-2102548	Correction to CSSF for PRS measurements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1736  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

Correction to CSSF for PRS measurements

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was merged.

R4-2102758	Discussion on remaining issues for RSTD measurement requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Abstract:

Rel-16

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2102759	CR to update RSTD measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1762  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was merged.

R4-2102760	CR to update RSTD measurement requirements R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1763  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was withdrawn.

R4-2102926	Correction to CSSF for PRS measurements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1793  Cat: A (Rel-17)

					Source: Ericsson-LG Co., LTD

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was withdrawn.

R4-2102934	On PRS-RSTD measurement period definition

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on aspects related to PRS-RSTD measurement period definition

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

##### 7.7.1.2	PRS-RSRP measurement requirements [NR_pos-Core]

R4-2101781	CR to 38.133 correction on PRS-RSRP measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1665  Cat: F (Rel-16)

					Source: vivo

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was merged.

R4-2101782	CR to 38.133 correction on PRS-RSRP measurement requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1666  Cat: A (Rel-17)

					Source: vivo

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was withdrawn.

R4-2102300	Revision of PRS-RSRP measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1708  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was merged.

R4-2102301	Revision of PRS-RSRP measurement requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1709  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was withdrawn.

R4-2102542	On PRS-RSRP measurement requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On PRS-RSRP measurement requirements

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2102543	PRS-RSRP measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1732  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

PRS-RSRP measurement requirements

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was revised to R4-2103579.

R4-2103579	PRS-RSRP measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1732  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2102543)

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was revised to R4-2104074.

R4-2104074	PRS-RSRP measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1732  rev 2 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2103579)

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was agreed.

R4-2102544	PRS-RSRP measurement requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1733  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

PRS-RSRP measurement requirements

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was agreed.

R4-2102761	CR to update PRS-RSRP measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1764  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was merged.

R4-2102762	CR to update PRS-RSRP measurement requirements R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1765  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was withdrawn.

##### 7.7.1.3	UE Rx-Tx time difference measurement requirements  [NR_pos-Core]

R4-2100049	UE Rx-Tx measurements

Type: LS out		For: Approval
					to RAN1
					Source: ZTE Corporation

Abstract:

This discussion paper discusses issues related to UE Rx - Tx measurements and provides a draft LS to be sent to RAN1 for clarification on a definition in RAN1 specification.

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2100437	Discussion on UE Rx-Tx time difference measurement requirements

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2100439	CR on UE Rx-Tx time difference measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1478  Cat: F (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was merged.

R4-2100469	CR on UE Rx-Tx time difference measurement requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1490  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was withdrawn.

R4-2101273	Discussion on NR UE RX-TX time difference measurement requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2101527	CR to TS 38.133 on UE Rx-Tx time difference measurements (section 9.9.4)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1608  Cat: F (Rel-16)

					Source: OPPO

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was revised to R4-2103580.

R4-2103580	CR to TS 38.133 on UE Rx-Tx time difference measurements (section 9.9.4)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1608  rev 1 Cat: F (Rel-16)

					Source: OPPO

(Replaces R4-2101527)

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was agreed.

R4-2103581	CR to TS 38.133 on UE Rx-Tx time difference measurements (section 9.9.4)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1799  Cat: - (Rel-17)

					Source: OPPO

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was agreed.

R4-2101528	Discussion on maintenance for UE Rx-Tx time difference measurements

Type: discussion		For: Discussion
					Source: OPPO

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2101777	Discussion on UE RX-TX timing difference measurement requirements

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2101783	CR to 38.133 correction on UE Rx-Tx time difference measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1667  Cat: F (Rel-16)

					Source: vivo

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was merged.

R4-2101784	CR to 38.133 correction on UE Rx-Tx time difference measurement requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1668  Cat: A (Rel-17)

					Source: vivo

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was withdrawn.

R4-2102291	On UE Rx-Tx measurement requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2102302	Revision of UE Rx-Tx measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1710  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was merged.

R4-2102303	Revision of UE Rx-Tx measurement requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1711  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was withdrawn.

R4-2102545	On UE Rx-Tx measurement requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On UE Rx-Tx measurement requirements

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2102546	UE Rx-Tx measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1734  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

UE Rx-Tx measurement requirements

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was merged.

R4-2102547	UE Rx-Tx measurement requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1735  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

UE Rx-Tx measurement requirements

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was withdrawn.

R4-2102763	Discussion on remaining issues for UE Rx-Rx time difference measurement requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2102764	CR to update UE Rx-Tx time differnece measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1766  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was merged.

R4-2102765	CR to update UE Rx-Tx time differnece measurement requirements R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1767  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was withdrawn.

##### 7.7.1.4	Other requirements [NR_pos-Core]

R4-2101529	Discussion on remaining issues for general PRS measurements

Type: discussion		For: Discussion
					Source: OPPO

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2101778	Discussion on CCSF for NR positioning measurements

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2101785	CR to 38.133 correction on CCSF for NR measurements for positioning

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1669  Cat: F (Rel-16)

					Source: vivo

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was revised to R4-2103582.

R4-2103582	CR to 38.133 correction on CCSF for NR measurements for positioning

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1669  rev 1 Cat: F (Rel-16)

					Source: vivo

(Replaces R4-2101785)

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was revised to R4-2104075.

R4-2104075	CR to 38.133 correction on CCSF for NR measurements for positioning

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1669  rev 2 Cat: F (Rel-16)

					Source: vivo

(Replaces R4-2103582)

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was agreed.

R4-2101786	CR to 38.133 correction on CCSF for NR measurements for positioning

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1670  Cat: A (Rel-17)

					Source: vivo

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was agreed.

R4-2102292	On general PRS measurement requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2102766	Discussion on measurement capability and MG for PRS measurement

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was noted.

R4-2102767	CR on CSSF and MG for PRS measurement 38.133

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1768  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was revised to R4-2103583.

R4-2103583	CR on CSSF and MG for PRS measurement 38.133

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1768  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2102767)

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was postponed.

R4-2102768	CR on CSSF and MG for PRS measurement 38.133 R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1769  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was withdrawn.

R4-2102769	CR to introduce new measurement gap patterns for positioning in 36.133

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7068  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was revised to R4-2103584.

R4-2103584	CR to introduce new measurement gap patterns for positioning in 36.133

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7068  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2102769)

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was agreed.

R4-2102770	CR to introduce new measurement gap patterns for positioning in 36.133 R17

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7069  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][213] NR_pos_RRM_1 in R4-2103695.

Decision: 		The document was agreed.

#### 7.7.2	RRM perf. requirements (38.133) [NR_pos-Perf]

##### 7.7.2.1	General [NR_pos-Perf]

R4-2102549	Draft Big CR: Introduction of Rel-16 NR Positioning RRM performance requirements and test cases

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson, Intel

Abstract:

Draft Big CR: Introduction of Rel-16 NR Positioning RRM performance requirements and test cases

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was revised to R4-2103585.

R4-2103585	Draft Big CR: Introduction of Rel-16 NR Positioning RRM performance requirements and test cases

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson, Intel

(Replaces R4-2102549)

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

##### 7.7.2.2	UE requirements and test cases [NR_pos-Perf]

R4-2103453	Email discussion summary: [98e][214] NR_pos_RRM_2

Type: other		For: discussion
					Source: Moderator (Intel Corporation)

Discussion:

The contribution summarised email discussion thread [98e][214] NR_pos_RRM_2. The discussion thread was moderated by Huang Rui (Intel).

Decision: 		The document was revised to R4-2103696.

R4-2103696	Email discussion summary: [98e][214] NR_pos_RRM_2

Type: other		For: discussion
					Source: Moderator (Intel Corporation)

(Replaces R4-2103453)

Discussion:

The contribution summarised email discussion thread [98e][214] NR_pos_RRM_2. The discussion thread was moderated by Huang Rui (Intel).

Decision: 		The document was noted.

GTW session (January 28, 2021)

Sub-topic 2-4 How to define the accuracy requirements with the combinations of PRS BW and other parameters (e.g. comb size, repetition)

- Option 1a (Intel). RSTD accuracy requirements are defined based on PRS configuration parameters of

  - PRS BW defined in number of PRBs

  - PRS repetition factor dl-PRS-ResourceRepetitionFactor * dl-PRS-NumSymbols / dl-PRS-CombSizeN

- Option 1b (Huawei). RSTD accuracy requirements are defined based on PRS configuration parameters of

  - PRS BW defined in number of PRBs

  - PRS SCS

  - PRS repetition factor dl-PRS-ResourceRepetitionFactor * dl-PRS-NumSymbols / dl-PRS-CombSizeN

- Option 2 (vivo): Define RSTD accuracy requirements for

  - multiple PRS BWPs.

  - Combination of {12, 12} for {Comb size, number of PRS symbols} is used as baseline to define accuracy requirements for the case without repetition.

  - Accuracy requirements for number of repetitions for PRS resource can be further specified if significant accuracy gap is identified.

- Option 3(Qualcomm)

  - PRS BW defined in number of PRBs

  - PRS SCS

  - number of PRS symbols per slot equal to PRS comb size.

- Option 4 (Ericsson): The RSTD accuracy requirements

  - Depend on PRS BW defined in number of PRBs

  - apply for:

    - any DL-PRS-ResourceRepetitionFactor≥1 and

    - any LPRS≥2 which is given by the higher-layer parameter dl-PRS-NumSymbols and

    - any comb pattern

- Option 5 (CATT): Define the RSTD accuracy requirements based on PRS configuration parameters of

  - PRS BW defined in number of PRBs

  - PRS repetition factor dl-PRS-ResourceRepetitionFactor

Agreements

  - RSTD accuracy requirements are defined to cover all BW configurations

  - Different RSTD accuracy requirements depend on the following parameters

    - PRS BW defined in number of PRBs

  - Companies are encouraged to bring PRS accuracy simulation results in RAN4 #99e for the identified set of parameters

    - PRS BW defined in number of PRBs

    - PRS SCS

    - dl-PRS-ResourceRepetitionFactor

    - dl-PRS-NumSymbols

    - dl-PRS-CombSizeN

    - Other parameters are FFS

    - Note: the full set of parameters and values to be finalized in RAN4 #98e. Prepare unified format for results collection.

Sub-topic 3-1 PRS-RSRP SINR side condition of #1

- Option 1a (Ericsson): PRS-RSRP SINR side condition #1 is -3 dB.

- Option 1b (Ericsson): both -3 dB and -6 dB could also be acceptable

- Option 2 (CATT, Intel, Huawei, vivo): -6dB

Agreement: PRS-RSRP SINR side condition #1 is [-3] dB. Companies are encouraged to bring simulation results for -3dB and -6dB SINR conditions.

R4-2103586	WF on NR Positioning UE performance requirements

Type: other		For: discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was approved.

R4-2104046	Summary of link level simulation result of RSTD, PRS RSRP and UE DToA

Type: other		For: discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was revised to R4-2104073.

R4-2104073	Summary of link level simulation result of RSTD, PRS RSRP and UE DToA

Type: other		For: discussion
					Source: Intel Corporation

(Replaces R4-2104046)

Discussion:

QC: need to update the results. There was change in units from ns to Tc

Intel: added a not on units in the file

E///: we propose to capture in the chairman notes

Chair: need to change TDOA to UE Rx-Tx

CATT: propose to replace parameters

| DL-PRS-NumSymbols | DL-PRS-CombSizeN | DL-PRS_ResourceRepetitionFactor |
| --- | --- | --- |
| 12 | 4 | 1 |

with parameters

| DL-PRS-NumSymbols | DL-PRS-CombSizeN | DL-PRS_ResourceRepetitionFactor |
| --- | --- | --- |
| 6 | 2 | 2 |

Intel/vivo: prefer not to change

vivo: companies can still bring additional results.

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was revised to R4-2104087.

R4-2104087	Summary of link level simulation result of RSTD, PRS RSRP and UE DToA

Type: other		For: discussion
					Source: Intel Corporation

(Replaces R4-2104073)

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was revised to R4-2104092.

R4-2104092	Summary of link level simulation result of RSTD, PRS RSRP and UE DToA

Type: other		For: discussion
					Source: Intel Corporation

(Replaces R4-2104087)

Discussion:

Agreement: the data format in R4-2104092 will be used for simulation results collection in RAN4 98-bis-e

Agreement: RSTD and UE Rx-Tx measurement accuracy data will be collected in the units of Tc

Session chair: encourage rapporteurs to share clean template with companies before the next meeting.

Session chair: companies can bring additional results for 1 sample case.

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

###### 7.7.2.2.1	Measurement accuracy requirements [NR_pos-Perf]

7.7.2.2.1.1	PRS RSTD  [NR_pos-Perf]

R4-2100440	Discussion on PRS RSTD accuracy requirements

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2100443	Link-level simulation results of PRS RSTD measurement

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2101274	Discussion on NR PRS RSTD measurement accuracy requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2101787	Discussion on PRS RSTD accuracy requirements

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2102293	On PRS-RSTD measurement accuracy requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2102312	Test structure and complexity for NR RSTD test cases

Type: discussion		For: Endorsement
					Source: Rohde & Schwarz

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2102550	On RSTD measurement accuracy requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On RSTD measurement accuracy requirements

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2102771	Discussion on accuracy requirements for RSTD measurement

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2102772	draftCR to introduce accuracy requirements for RSTD measurement

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was revised to R4-2103730.

R4-2103730	draftCR to introduce accuracy requirements for RSTD measurement

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2102772)

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was endorsed.

7.7.2.2.1.2	PRS RSRP  [NR_pos-Perf]

R4-2100061	[draft CR] Test cases for PRS-RSRP measurement accuracy

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was revised to R4-2103733.

R4-2103733	[draft CR] Test cases for PRS-RSRP measurement accuracy

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: ZTE Corporation

(Replaces R4-2100061)

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was postponed.

R4-2100441	Discussion on PRS RSRP accuracy requirements

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2100444	Link-level simulation results of PRS RSRP measurement

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2100446	CR on PRS-RSRP accuracy requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1479  Cat: B (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was revised to R4-2103732.

R4-2103732	CR on PRS-RSRP accuracy requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1479  rev 1 Cat: B (Rel-16)

					Source: CATT

(Replaces R4-2100446)

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was endorsed.

R4-2100470	CR on PRS-RSRP accuracy requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1491  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was withdrawn.

R4-2101275	Discussion on NR PRS RSRP measurement accuracy requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2102551	On PRS-RSRP measurement accuracy requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On PRS-RSRP measurement accuracy requirements

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2102773	Discussion on accuracy requirements for PRS-RSRP measurement

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2102774	Further simulation results for PRS-RSRP

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

7.7.2.2.1.3	UE Rx-Tx time difference  [NR_pos-Perf]

R4-2100442	Discussion on UE Rx-Tx time difference accuracy requirements

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2100445	Link-level simulation results for UE  Rx-Tx time difference measurement

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2101276	Discussion on NR UE RX-TX time difference measurement accuracy requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2102294	On UE Rx-Tx measurement accuracy requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2102552	On UE Rx-Tx measurement accuracy requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On UE Rx-Tx measurement accuracy requirements

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2102553	UE Rx-Tx measurement accuracy requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

UE Rx-Tx measurement accuracy requirements

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was revised to R4-2103731.

R4-2103731	UE Rx-Tx measurement accuracy requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2102553)

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was endorsed.

R4-2102775	Discussion on accuracy requirements for UE Rx-Tx time difference measurement

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

###### 7.7.2.2.2	Test cases [NR_pos-Perf]

R4-2100447	CR on PRS configuration for test cases

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1480  Cat: B (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was merged.

R4-2100449	Discussion on test cases for PRS based measurement

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2100471	CR on PRS configuration for test cases

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1492  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was withdrawn.

R4-2101277	Discussion on NR Positioning test cases configuration

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2101278	[draftCR] CR for PRS configurations for NR Pos RRM tests

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was revised to R4-2103735.

R4-2103735	[draftCR] CR for PRS configurations for NR Pos RRM tests

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Intel Corporation

(Replaces R4-2101278)

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was postponed.

R4-2102295	On design of test cases for NR positioning

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2102554	On positioning test cases

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On positioning test cases

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

R4-2102776	Discussion on RRM test case for UE positioning requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

###### 7.7.2.2.3	Measurement requirements [NR_pos-Perf]

R4-2100448	CR on test case for PRS-RSRP measurement requirements for FR2 in SA

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1481  Cat: B (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was revised to R4-2103734.

R4-2103734	CR on test case for PRS-RSRP measurement requirements for FR2 in SA

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1481  rev 1 Cat: B (Rel-16)

					Source: CATT

(Replaces R4-2100448)

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was postponed.

R4-2100472	CR on test case for PRS-RSRP measurement requirements for FR2 in SA

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1493  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was withdrawn.

R4-2101279	[draftCR] CR for the test case of RSTD measurement requirements reporting in SA

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was revised to R4-2103736.

R4-2103736	[draftCR] CR for the test case of RSTD measurement requirements reporting in SA

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Intel Corporation

(Replaces R4-2101279)

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was postponed.

R4-2102555	TC5 and TC6: UE Rx-Tx time difference measurement requirements for FR1 and FR2 in SA

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

TC5 and TC6: UE Rx-Tx time difference measurement requirements for FR1 and FR2 in SA

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was revised to R4-2103737.

R4-2103737	TC5 and TC6: UE Rx-Tx time difference measurement requirements for FR1 and FR2 in SA

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2102555)

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was postponed.

R4-2102777	draftCR to introduce TC for PRS-RSRP measurement requirements for FR1 in SA

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was revised to R4-2103739.

R4-2103739	draftCR to introduce TC for PRS-RSRP measurement requirements for FR1 in SA

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2102777)

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was postponed.

###### 7.7.2.2.4	Accuracy requirements [NR_pos-Perf]

R4-2102556	TC11 and TC12: UE Rx-Tx time difference measurement accuracy for FR1 and FR2 in SA

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

TC11 and TC12: UE Rx-Tx time difference measurement accuracy for FR1 and FR2 in SA

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was revised to R4-2103738.

R4-2103738	TC11 and TC12: UE Rx-Tx time difference measurement accuracy for FR1 and FR2 in SA

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2102556)

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was postponed.

R4-2102778	draftCR to introduce TC for RSTD measurement accuracy for FR1 and FR2 in SA

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was revised to R4-2104042.

R4-2104042	draftCR to introduce TC for RSTD measurement accuracy for FR1 and FR2 in SA

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2102778)

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was postponed.

###### 7.7.2.2.5	Other [NR_pos-Perf]

R4-2103454	Email discussion summary: [98e][215] NR_pos_RRM_3

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][215] NR_pos_RRM_3. The discussion thread was moderated by Muhammad Kazmi (Ericsson).

Decision: 		The document was revised to R4-2103697.

R4-2103697	Email discussion summary: [98e][215] NR_pos_RRM_3

Type: other		For: Endorsement
					38.133 v16.6.0
					Source: Moderator (Ericsson)

(Replaces R4-2101279)

Discussion:

The contribution summarised email discussion thread [98e][215] NR_pos_RRM_3. The discussion thread was moderated by Muhammad Kazmi (Ericsson).See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was noted.

GTW session (January 28, 2021)

Issue 1-2-1: Antenna configuration in accuracy requirement

- Option 1: E///, Nokia

  - Fixed antenna beams are assumed in gNB for deriving accuracy

- Option 2:

  - Proposal 1: ZTE, HW

    - Accuracy does not depend on antenna beam configuration in gNB, i.e. do not assume fixed gNB antenna beams.

  - Proposal 2: HW

    - Positioning measurement requirements apply for the same RoAoA as OTA reference sensitivity requirements for 1-O and 2-O BS

Discussion

- Nokia: Support Option 1

- Huawei: Option 1 is not clear to us. RX beam is up to gNB implementation. The impact on requirement is unclear.

- QC: the main issue is to understand what options mean

- E///: beam is not supposed to change over measurement. BS does not need to do TX beam sweeping.

- QC: Is it TX or RX beam?

  - Nokia: this is RX beam at gNB side.

- E///: update “Accuracy requirements apply under assumption that gNB does not need to perform RX beam sweeping over the measurement period (i.e. RX beam sweeping to improve accuracy is not precluded)”

- Huawei: do we have any gNB behavior defined in spec? Also, we do not have any gNB measurement period

- Nokia: measurement time can be further clarified.

- ZTE: not sure on intention of Option 1. gNB behavior can be left up to implementation.

- HW: how does gNB make the measurement without RX beam sweeping?

Tentative agreements

- gNB accuracy requirements do not mandate gNB RX beam sweeping

Issue 1-6-1: Optional accuracy for low side condition

- Option 1: Nokia

  - gNB accuracy defined only for high Ês/Iot side condition (i.e. corresponding to serving cell) is mandatory for gNB supporting that measurement.

  - gNB accuracy defined for low Ês/Iot side condition (i.e. corresponding to neighbor cell) is optional even for gNB supporting that measurement.

- Other options not precluded

Discussion

- Huawei: If gNB declares the support of the measurement then it needs to support at least one of high or low side conditions

- Nokia: accuracy levels for low/high side conditions are different. Low side condition is more related to the neighbor cell and not very typical case.

- QC: Nokia’s concerns can be addressed when the requirements are defined.

- E///: agree with Huawei proposal

- QC: there is no requirement to meet the side condition. The question it to meet the positioning requirement when for the particular conditions.

Agreement

- The gNB, which declares the support of positioning measurements, needs to declare the support of measurements for at least either high or low Ês/Iot side conditions

Issue 4-1-1: Reuse of gNB-Rx-Tx time difference accuracy for UL RTOA accuracy

- Can gNB-Rx-Tx time difference accuracy be reused for defining UL RTOA accuracy?

  - Option 1: HW, Nokia

    - Yes

  - Option 2: E///

    - No

Agreement

- Further identify how to derive reference time in the ideal UL-RTOA (e.g. whether it is determined by gNB’s own timing based on LMF configuration).

  - If no consensus reached in the 2nd round, then further send LS to RAN1 and RAN3 to clarify the procedure.

R4-2103587	WF on gNB positioning measurement requirements

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][214] NR_pos_RRM_2 in R4-2103696.

Decision: 		The document was approved.

##### 7.7.2.3	gNB requirements  [NR_pos-Perf]

###### 7.7.2.3.1	General [NR_pos-Perf]

R4-2100048	Beam configuration for gNB measurement accuracy

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2100353	gNB positioning requirements

Type: other		For: Discussion
					Source: Ericsson

Abstract:

gNB positioning general topics and WF statement overview

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2101729	gNB Positioning UL SRS System simulation results for side conditions

Type: other		For: Discussion
					Source: Ericsson

Abstract:

gNB positioning system level results for side conditions

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2102690	On SRS configurations for gNB measurement accuracy requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on set of SRS configurations for gNB measurement accuracy requirements

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2102779	Discussion on general issues for gNB positioning measurement requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

###### 7.7.2.3.2	SRS-RSRP requirements [NR_pos-Perf]

R4-2100450	Discussion on SRS-RSRP measurement requirements

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2101737	gNB SRS-RSRP measurement analysis

Type: other		For: Discussion
					Source: Ericsson

Abstract:

Discussion document for related CR

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2101760	gNB SRS-RSRP measurement

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

Adding SRS-RSRP measurement accuracy requirements table structure

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was revised to R4-2104052.

R4-2104052	gNB SRS-RSRP measurement

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2101760)

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was endorsed.

R4-2102783	Discussion on SRS-RSRP requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

Session chair: moved from AI 7.7.2.3.3. See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2102784	Link level simulation results for SRS-RSRP

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

Session chair: moved from AI 7.7.2.3.3. See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2102785	draftCR to introduce SRS-RSRP requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

Session chair: moved from AI 7.7.2.3.3. See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was postponed.

###### 7.7.2.3.3	gNB Rx-Tx time difference requirements [NR_pos-Perf]

R4-2100451	Discussion on gNB Rx-Tx time difference measurement requirements

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2101761	gNB Rx-Tx measurement analysis

Type: other		For: Discussion
					Source: Ericsson

Abstract:

Discussion document for related CR

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2101790	gNB Rx-Tx measurement

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

Adding Rx-Tx measurement accuracy requirements table structure

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was postponed.

R4-2102691	Link simulation results for gNB Rx-Tx time difference accuracy

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Link simulation results for gNB Rx-Tx time difference accuracy for some agreed SRS configurations

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2102780	Discussion on gNB Rx-Tx time difference requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

Session chair: moved from AI 7.7.2.3.2. See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2102781	Link level simulation results for UL-RTOA and gNB Rx-Tx time difference

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

Session chair: moved from AI 7.7.2.3.2. See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2102782	draftCR to introduce gNB Rx-Tx time difference requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

Session chair: moved from AI 7.7.2.3.2. See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was revised to R4-2104053.

R4-2104053	draftCR to introduce gNB Rx-Tx time difference requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2102782)

Discussion:

Session chair: moved from AI 7.7.2.3.2. See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was endorsed.

###### 7.7.2.3.4	UL RTOA requirements [NR_pos-Perf]

R4-2100452	Discussion on UL RTOA measurement requirements

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2100453	Link-level simulation results for UL timing measurement

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2101801	gNB UL RTOA measurement analysis

Type: other		For: Discussion
					Source: Ericsson

Abstract:

Discussion on WF analysis if RxTx requirements defined should be reused for UL RTOA

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2102692	On UL RTOA requirements for NR positioning

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on introduction of RTOA requirements for NR positioning

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2102786	Discussion on UL-RTOA requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon, CMCC

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was noted.

R4-2102787	draftCR to introduce UL-RTOA requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][215] NR_pos_RRM_3 in R4-2103697.

Decision: 		The document was postponed.

### 7.8	Physical layer enhancements for NR URLLC [NR_L1enh_URLLC-Core]

#### 7.8.1	Demodulation and CSI requirements (38.101-4/38.104) [NR_L1enh_URLLC-Perf]

##### 7.8.1.1	Performance requirements with ultra-low BLER [NR_L1enh_URLLC-Perf]

R4-2103761	Email discussion summary for [98e][322] NR_URLLC_Demod_Part1

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][322] NR_URLLC_Demod_Part1. The discussion thread was moderated by Thomas Chapman (Ericsson).

Decision: 		The document was revised to R4-2103937.

R4-2103937	Email discussion summary for [98e][322] NR_URLLC_Demod_Part1

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2103761)

Discussion:

The contribution summarised email discussion thread [98e][322] NR_URLLC_Demod_Part1. The discussion thread was moderated by Thomas Chapman (Ericsson).

Agreements:

CQI test SNR points:

1x2 cases: SNR = [1/2] dB

1X4 cases: SNR = [-2/-1] dB

Decision: 		The document was noted.

R4-2103897	LS on Test Methodology for UE URLLC Ultra Low BLER CQI requirements

Type: LS out		For: Approval
					to RAN5
					Source: RAN4

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was approved.

R4-2103898	Summary of simulation results for 0.001% BLER UE requirement

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was noted.

R4-2103899	Summary of simulation results for 0.001% BLER BS requirement

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was noted.

------------------------------------------GTW agenda Jan.28th for email thread [322] (30 minutes) -------------------

CQI test cases

Recommendation from moderators based on 1st round discussion:

Assume early termination method for CQI test

Confidence level 99% for CQI test

Do not include X=0.5dB in CQI test

No applicability rule for CQI test

SNR defined for one pair 1dB apart

CQI value must be >=1

Issue 1-5: Number of SNR points for requirement definition

Note: This is a separate issue to whether SNR pairs are defined. Each SNR point may or may not have a pair of SNRs and pass at either. The UE would be required to pass all SNR points.

- Proposals

  - Option 1: One SNR point

  - Option 2: Two SNR points

- Recommended WF

  - TBA

Issue 1-6: Lower bound

- Proposals

  - Option 1: No lower bound

  - Option 2: CQI 0 cannot be reported as median CQI

  - Option 3: Another lower bound that is >CQI0

- Recommended WF

  - TBA

Huawei: We should still test both high coding rate and low coding rate for test coverage.

Intel: For high coding rate, we already verified by existing CQI table test cases.

Issue 1-7: SNR point(s) for 1x2

- Proposals

  - Option 1:

|  | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- |
| FDD (SNR) | -4 | -3 | 7 | 8 |
| TDD (SNR) | -4 | -3 | 7 | 8 |

  - Option 2: One SNR pair, same SNR as used for the FMCS requirement (should this be the upper or lower of the pair ?)

  - Option 3: Other value(s) (please state)

- Recommended WF

  - Discuss further in GTW and 2nd round which SNR values to select (1 pair 1dB apart)

Discussion:

Choosing SNR points corresponding to low coding rate

QC: We can’t take too low SNR points similar reason as fixed MCS test cases.

1x2: 1/2 dB, 1x4: -1/-2 dB

Candidate options: (1X2 test):

Option 1: -4/-3 dB (Huawei)

Option 2: 1/2 dB (QC, Intel)

Option 3: 2/3 dB (Apple)

By default, the 1x4 test will have -3 dB offset compared to 1X2 SNR test points.

----------------------------------End -------------------------------------------------------------

###### 7.8.1.1.1	UE demodulation requirements [NR_L1enh_URLLC-Perf]

R4-2101018	CR to 38.101-4 on FRC table update for URLLC ultra low BLER requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0137  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was agreed.

R4-2102120	CR to TS 38.101-4: Performance requirements for URLLC PDSCH 0.001% BLER

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0164  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

Finalize the requirements

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was revised to R4-2103890.

R4-2103890	CR to TS 38.101-4: Performance requirements for URLLC PDSCH 0.001% BLER

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0164  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2102120)

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was agreed.

###### 7.8.1.1.2	CSI requirements  [NR_L1enh_URLLC-Perf]

R4-2100169	Views on URLLC Ultra-low BLER CSI Reporting Test Cases

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was noted.

R4-2100198	CQI Reporting requirements for URLLC

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was noted.

R4-2100199	CR to 38-101-4 on CQI reporting requirements for URLLC

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0126  Cat: B (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was revised to R4-2103891.

R4-2103891	CR to 38-101-4 on CQI reporting requirements for URLLC

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0126  rev 1 Cat: B (Rel-16)

					Source: Apple

(Replaces R4-2100199)

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was agreed.

R4-2101242	Discussion on URLLC Ultra-low BLER CQI requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was noted.

R4-2101327	Discussion on CSI requireements with ultra low-BLER

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was noted.

R4-2101331	Simulation for CQI reporting test

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was noted.

R4-2101338	CR to TS38.101-4 Applicability rules for URLLC CSI requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0152  Cat: B (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was revised to R4-2103892.

R4-2103892	CR to TS38.101-4 Applicability rules for URLLC CSI requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0152  rev 1 Cat: B (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101338)

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was agreed.

R4-2101944	CR on FRC for Ultra low BLER UE CQI requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0160  Cat: B (Rel-16)

					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was revised to R4-2103893.

R4-2103893	CR on FRC for Ultra low BLER UE CQI requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0160  rev 1 Cat: B (Rel-16)

					Source: Intel Corporation

(Replaces R4-2101944)

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was agreed.

R4-2102116	Discussions on URLLC UE CQI reporting requirements for CQI table 3

Type: other		For: Approval
					Source: Ericsson

Abstract:

Remaining open issues for CQI

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was noted.

R4-2102118	Simulation results on URLLC UE CQI reporting requirements for CQI table 3

Type: other		For: Information
					Source: Ericsson

Abstract:

Results for CQI

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was noted.

###### 7.8.1.1.3	BS demodulation requirements [NR_L1enh_URLLC-Perf]

R4-2100563	CR for 38.104: Ultra high reliability BS demodulation requirements

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0267  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

It was announced that one company has discovered that their simulation results need to be updated.

This CR will incorporate the resulting change of SNR requirements for ultra high reliability using a revision, once the results are available during the mee

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was revised to R4-2103894.

R4-2103894	CR for 38.104: Ultra high reliability BS demodulation requirements

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0267  rev 1 Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100563)

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was agreed.

R4-2100564	CR for 38.104: Ultra high reliability BS demodulation requirements

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0268  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was agreed.

R4-2102122	CR to TS 38.141-1 Update of 0.001% BLER test requirements

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0201  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

Captures updated requirement values

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was revised to R4-2103895.

R4-2103895	CR to TS 38.141-1 Update of 0.001% BLER test requirements

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0201  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2102122)

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was agreed.

R4-2102123	CR to TS 38.141-1 Update of 0.001% BLER test requirements

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0202  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Captures updated requirement values

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was agreed.

R4-2102124	CR to TS 38.141-2 Update of 0.001% BLER test requirements

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0302  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

Captures updated requirement values

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was revised to R4-2103896.

R4-2103896	CR to TS 38.141-2 Update of 0.001% BLER test requirements

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0302  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2102124)

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was agreed.

R4-2102125	CR to TS 38.141-2 Update of 0.001% BLER test requirements

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0303  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Captures updated requirement values

Discussion:

See email discussion summary for [98e][322] NR_URLLC_Demod_Part1 in R4-2103937.

Decision: 		The document was agreed.

##### 7.8.1.2	Performance requirements with higher BLER [NR_L1enh_URLLC-Perf]

R4-2103762	Email discussion summary for [98e][323] NR_URLLC_Demod_Part2

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][323] NR_URLLC_Demod_Part2. The discussion thread was moderated by Lu Bai (CHENGDU TD TECH LTD.).

Decision: 		The document was revised to R4-2103938.

R4-2103938	Email discussion summary for [98e][323] NR_URLLC_Demod_Part2

Type: other		For: discussion
					Source: Moderator (Huawei)

(Replaces R4-2103762)

Discussion:

The contribution summarised email discussion thread [98e][323] NR_URLLC_Demod_Part2. The discussion thread was moderated by Lu Bai (CHENGDU TD TECH LTD.).

Decision: 		The document was noted.

R4-2103955	WF for URLLC UE with higher BLER

Type: other		For: discussion
					Source: Intel

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was approved.

R4-2103956	WF for URLLC BS with higher BLER

Type: other		For: discussion
					Source: Huawei

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was approved.

---------------------------------GTW agenda Jan.28th for email thread [323] (30 minutes) ---------------------

Sub-topic 1-2: UE demodulation requirements for high reliability for FR2

Issue 1-2-1: MCS

- Proposals

  - Option 1: MCS19 from Table 3 (QC)

  - Option 2: MCS17 from Table 3 (Ericsson)

  - Option 3: MCS16 from Table 3 (QC, Apple, Huawei, Ericsson)

  - Option 4: MCS13 from Table 3 (Apple, Intel)

  - Higher or equal to -4 dB for final 2 Rx requirement definition (average ideal SNR alignment result + IM).

- Recommended WF

  - Based on the simulation results submitted by companies, the real SNR for MCS13 is around -5.5dB for most companies and below -6 dB for one company. As MCS16 is acceptable by majority companies, the moderator recommends MCS16 from Table 3 as the agreement.

  - Option 3.

Agreements: MCS16 from Table 3

Sub-topic 2-3: UE demodulation requirements for pre-emption

Open issues and candidate options before e-meeting:

Issue 2-3-1: Simulation results observation (based on R4-2101333):

The gain between with and without buffer flushing is

MCS4 with 10% and 20% probability: For majority companies, the gain is very small, less than 0.1 dB.

MCS13 with 20% probability: the span between companies are very large, larger than 4 dB.

MCS 16 with 20 % probability: For majority companies, the maximum throughput cannot reach 70%.

- Recommended WF

  - Consider MCS13 or MCS16 with 10% probability.

  - As indicated by Intel, the average gain for MCS13 with 10% is 0.6dB, which is not testable.

  - According to the simulation results, MCS16 with 10% is a proper test setup (Ericson, Apple, Intel, Huawei)

  - Please fill your impairment results for MCS16 with 10% probability for both of FDD and TDD. (a new table for impairment results is added in R4-2101333)

Issue 2-3-1a: MCS

- Proposals

  - Option 1: MCS16 from Table 1. (Apple, Intel, Huawei)

  - Option 2: MCS13 from Table 1 (MTC, Ericsson)

  - Option 3: MCS 4 from Table 1 (QC)

- Recommended WF

  - Option 1

Issue 2-3-1b: Pre-emption probability

- Proposals

  - Option 1: 20%. (,

  - Option 2: 10% (Intel, Ericsson, QC, Apple, Huawei)

- Recommended WF

  - Option 2

Agreements: MCS 16 with 10% Pre-emption probability

- Issue 2-3-4:  This issue is proposed as initial transmission and its retransmission is always pre-empted with current simulation assumption. By changing the pre-emption pattern rather than modify the TDD HARQ timeline can solve the problem: Recommended WF

  - Pre-emption pattern for TDD:

10% probability: change from (0 1 0 0 0 0 0 0 0 0) to (0 1 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0).

20% probability: change from (0 1 1 0 0 0 0 0 0 0) to (0 1 1 0 0 0 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0 0 0 1 1 0 0 0)

Agreements: Pre-emption pattern for TDD:

10% probability: change from (0 1 0 0 0 0 0 0 0 0) to (0 1 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0).

Sub-topic 4-1: Rel-16 URLLC PUSCH repetition Type B

Issue 4-2-1: Typical scenarios for PUSCH repetition type B:

For PUSCH repetition Type B, different scenarios existed as shown in figure below, in order to simplifying the discussion, we treat figure 1 as scenario 1, figure 2 as scenario 2 and figure 3 as scenario 3:

Scenario 3:

Scenario 1:

Scenario 2:

4 symbols, 4 nominal repetitions

Slot boundary

4 symbols, 2 repetitions

Slot boundary

14 symbols, 1 nominal repetition

Slot boundary

Note: this case requires S+L>14.

10 symbols, 2 nominal repetitions

Scenario 4:

10 symbols, 1 repetition

Scenario 5:

- Proposals

  - Option 1: Scenario 1 (S+L < 14, K > 1), within slot. (Intel, Samsung, Nokia)

  - Option 2: Scenario 2 (S+L < 14, K >1), across slot boundary. (Huawei, CTC, Ericsson(a simple setup based on scenario 2), Samsung, Nokia, Intel, DoCoMo)

  - Option 3: Scenario 3 (S+L >14, K = 1) (Samsung, Nokia, Intel)

  - Option 4: Scenario 4 (S+L > 14, K >1). (Nokia, Intel)

  - Option 5: Scenario 5 (S+L <14, K = 1) (Samsung)

- Recommended WF

  - Note: As Intel indicated that usually TDD requirements are defined in unified manner, i.e. applicable to different TDD UL/DL patterns. Two consecutive slots are required for option 2-4 scenarios. If option 2-4 is considered, requirements will be applicable to FDD and TDD with certain UL/DL configurations.

Intel: It’s beneficial to cover several scenarios within slot and across slot boundaries. For TDD, with scenario 2-4, test applicable rules with TDD pattern would be issue, does mean this only applied for FDD only?

Requirements will be applicable to FDD and TDD with certain UL/DL configurations.

E///: On the timeline, in SR report of Dec –plenary with 90% completion, and now we open a new scenario and much workload expected. We should respect RAN-P decision with management work load on rest issues. One alternative was to further discuss in Rel-17 performance enhancement WI with more available times.

For the detailed scenarios, in general we need to further evaluate with more feasible test set-up and that will be need more time.

Nokia: We understand Intel raised issue, which not aligned with current BS demod test structure and further study needed.

We don’t think only focus on single slot scenario reflect the real deployment scenarios, also loss test coverage for BS implementation with other cases.

China Telecomm: For the scenario, we need to concentrate on one simple scenario. From BS demod aspect, the repetition with mini-slot, and the repetition across several slots; above two are key purpose to define requirements.

We define requirements in Rel-17 which means these features can be verified in Rel-17, same story for Rel-15 URLLC features.

We still pick one simple scenario in Rel-16 with manageable work load.

Samsung: For timeline, we have similar concern as E/// considering overall RAN4 workload. We need to bring further analysis for the applicable scenarios taking into account Intel mentioned issues. All of these will be time consuming.

Huawei: We already 12 test cases, this is additional one, 90% should be reflect the status.

Scenario 2 is reasonable to be taken as starting points with the purpose as China Telecomm mentioned.

Nokia:

What’s the target? Are you going release by release approach i.e. simple test cases in Rel-16 and more in Rel-17. We think not feasible as this is single feature, we should take this a whole. We also agree with Samsung for this aspect.

E///: We don’t think this will impact implementation timeline even introduced in Rel-17 timeframe.

FRC and TDD pattern need to take more time for further study.

China Telecomm: Response to Nokia, we are thinking pick one test scenario to cover this feature and no further test cases in future release for this feature. In WI summary, scenario 2 is captured in there.

Samsung: ALL the scenarios with in Rel-16 TR, and all of them are feasible and supported. We don’t think scenario 2 is typical scenario considering BS performance.

Option 1: Further discuss and introduce corresponding test cases in Rel-16 URLLC WI with WI extension in next RAN-P

Option 2: No further discussion in Rel-16 URLLC WI, and further discuss this feature and introduce corresponding test cases if needed in Rel-17 timeframe i.e Rel-17 performance enhancement WI.

Note: Final decision for whether this WI can be further extended or whether this feature can be further discussed in Rel-17 WIs would pending on RAN-P decision and confirmation.

-----------------------------End ------------------------------------

###### 7.8.1.2.1	UE demodulation requirements [NR_L1enh_URLLC-Perf]

R4-2100170	Views on URLLC High BLER Demodulation Test Cases

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2100171	Correction CR on URLLC Higher BLER Performance Requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0125  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was merged.

R4-2100200	UE demodulation requirements with higher BLER

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2100201	CR to 38.101-4 on requirements with slot aggreagation in FR2

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0127  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was revised to R4-2103900.

R4-2103900	CR to 38.101-4 on requirements with slot aggreagation in FR2

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0127  rev 1 Cat: F (Rel-16)

					Source: Apple

(Replaces R4-2100201)

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2101243	Discussion on URLLC UE demodulation requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2101244	Simulation results for URLLC UE demodulation requirements

Type: other		For: Information
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2101245	CR on FRC for URLLC UE Higher BLER requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0139  Cat: B (Rel-16)

					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was revised to R4-2103901.

R4-2103901	CR on FRC for URLLC UE Higher BLER requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0139  rev 1 Cat: B (Rel-16)

					Source: Intel Corporation

(Replaces R4-2101245)

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2101328	Discussion on URLLC UE demodulation requirements with higher BLER and low latency

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2101330	Simulation results on UE PDSCH demodulation reuqirements with higher BLER and low latency

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2101333	Summary of simulation results for UE URLLC demodulation performance requirements

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2101337	CR to TS 38.101-4 Correction of UE performance requirements for FR1 URLLC PDSCH repetitions over multiple slots.

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0151  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was revised to R4-2103902.

R4-2103902	CR to TS 38.101-4 Correction of UE performance requirements for FR1 URLLC PDSCH repetitions over multiple slots.

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0151  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101337)

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2102084	Simulation results for URLLC pre-emption

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2102117	Simulation results on UE URLLC demodulation performance requirements with higher BLER

Type: other		For: Information
					Source: Ericsson

Abstract:

Results for high BLER

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2102119	Discussion on UE URLLC demodulation performance requirements with higher BLER

Type: other		For: Approval
					Source: Ericsson

Abstract:

Remaining open issues for high BLER demod

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2102121	CR to TS 38.101-4: Performance requirements for URLLC High BLER feature tests

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0165  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

Needed to finalize pre-emption requirements

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was revised to R4-2103903.

R4-2103903	CR to TS 38.101-4: Performance requirements for URLLC High BLER feature tests

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0165  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2102121)

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2102822	CR on FR1 PDSCH Mapping Type B and Processing Capability 2 Requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0166  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was revised to R4-2103904.

R4-2103904	CR on FR1 PDSCH Mapping Type B and Processing Capability 2 Requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0166  rev 1 Cat: F (Rel-16)

					Source: Qualcomm Incorporated

(Replaces R4-2102822)

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

###### 7.8.1.2.2	BS demodulation requirements [NR_L1enh_URLLC-Perf]

R4-2100560	On NR Rel-16 BS demodulation performance requirements with higher BLER and simulation results

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we have provided our views on PUSCH repetition type B in the context of the NR_L1enh_URLLC WI. We have furthermore provided the last remaining simulation results.

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2100561	CR for 38.104: Low latency FR1 BS demodulation requirements

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0265  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Removal of remaining square brackets, following the stable nature of the simulation summary from last meeting [R4-2015629].

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was revised to R4-2103905.

R4-2103905	CR for 38.104: Low latency FR1 BS demodulation requirements

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0265  rev 1 Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100561)

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2100562	CR for 38.104: Low latency FR1 BS demodulation requirements

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0266  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2100796	On BS demodulation requirements for Rel-16 PUSCH repetition type B

Type: discussion		For: Discussion
					Source: China Telecom

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2100923	Discussion and simulation results for BS URLLC requirement

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2100927	CR on PUSCH repetition type A and PUSCH mapping type B radiated performance requirement for TS 38.104

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0269  Cat: B (Rel-16)

					Source: Samsung

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was revised to R4-2103906.

R4-2103906	CR on PUSCH repetition type A and PUSCH mapping type B radiated performance requirement for TS 38.104

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0269  rev 1 Cat: B (Rel-16)

					Source: Samsung

(Replaces R4-2100927)

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2100928	CR on PUSCH repetition type A and PUSCH mapping type B radiated performance requirement for TS 38.104

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0270  Cat: A (Rel-17)

					Source: Samsung

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2100929	CR on FRC for URLLC BS radiated performance requirement for TS 38.141-2

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0269  Cat: B (Rel-16)

					Source: Samsung

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was revised to R4-2103907.

R4-2103907	CR on FRC for URLLC BS radiated performance requirement for TS 38.141-2

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0269  rev 1 Cat: B (Rel-16)

					Source: Samsung

(Replaces R4-2100929)

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2100930	CR on FRC for URLLC BS radiated performance requirement for TS 38.141-2

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0270  Cat: A (Rel-17)

					Source: Samsung

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2101044	CR for TS 38.141-2  Updates of performance requirements of PUSCH repetition type A and PUSCH mapping type B for URLLC

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0275  Cat: B (Rel-16)

					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was revised to R4-2103908.

R4-2103908	CR for TS 38.141-2  Updates of performance requirements of PUSCH repetition type A and PUSCH mapping type B for URLLC

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0275  rev 1 Cat: B (Rel-16)

					Source: NTT DOCOMO, INC.

(Replaces R4-2101044)

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2101045	CR for TS 38.141-2  Updates of performance requirements of PUSCH repetition type A and PUSCH mapping type B for URLLC

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0276  Cat: A (Rel-17)

					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2101246	Discussion on URLLC BS demodulation requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2101247	Simulation results for URLLC BS demodulation requirements

Type: other		For: Information
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2101248	CR on FR2 requirements for PUSCH mapping Type B with low number of symbols (Rel-16)

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0280  Cat: F (Rel-16)

					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was revised to R4-2103909.

R4-2103909	CR on FR2 requirements for PUSCH mapping Type B with low number of symbols (Rel-16)

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0280  rev 1 Cat: F (Rel-16)

					Source: Intel Corporation

(Replaces R4-2101248)

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2101249	CR on FR2 requirements for PUSCH mapping Type B with low number of symbols (Rel-17)

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0281  Cat: A (Rel-17)

					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2101329	Discussion on URLLC BS demodulation requirements with higher BLER and low latency

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2101332	Simulation results on FR2 PUSCH demodulation reuqirements with higher BLER and low latency

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2101334	Summary of simulation results for BS URLLC demodulation performance requirements

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2101335	CR to TS38.141-2 Correction of BS conformance testing for FR2 URLLC PUSCH repetition Type A (Rel-16)

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0286  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was revised to R4-2103910.

R4-2103910	CR to TS38.141-2 Correction of BS conformance testing for FR2 URLLC PUSCH repetition Type A (Rel-16)

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0286  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101335)

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2101336	CR to TS38.141-2 Correction of BS conformance testing for FR2 URLLC PUSCH repetition Type A  (Rel-17)

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0287  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2101339	CR to TS38.141-1 Correction of BS conformance testing for URLLC demodulation requirements with higher BLER (Rel-16)

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0189  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was revised to R4-2103911.

R4-2103911	CR to TS38.141-1 Correction of BS conformance testing for URLLC demodulation requirements with higher BLER (Rel-16)

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0189  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101339)

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2101340	CR to TS38.141-1 Correction of BS conformance testing for URLLC demodulation requirements with higher BLER  (Rel-17)

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0190  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2101341	CR to TS38.104  Correction of BS performance requirements for URLLC FR1 PUSCH repetition Type A (Rel-16)

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0278  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was revised to R4-2103912.

R4-2103912	CR to TS38.104  Correction of BS performance requirements for URLLC FR1 PUSCH repetition Type A (Rel-16)

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0278  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101341)

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2101342	CR to TS38.104 Correction of BS performance requirements for URLLC FR1 PUSCH repetition Type A  (Rel-17)

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0279  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was agreed.

R4-2102115	Simulation results for BS URLLC high BLER

Type: other		For: Information
					Source: Ericsson

Abstract:

Results for additional bandwidths added last meeting

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was noted.

R4-2102285	Correction on requirements for PUSCH repetition Type A

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0304  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was merged.

R4-2102286	Correction on requirements for PUSCH repetition Type A

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0305  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][323] NR_URLLC_Demod_Part2 in R4-2103938.

Decision: 		The document was withdrawn.

### 7.9	Enhancements on MIMO for NR  [NR_eMIMO]

R4-2103455	Email discussion summary: [98e][216] NR_eMIMO_RRM

Type: other		For: discussion
					Source: Moderator (Samsung)

Discussion:

The contribution summarised email discussion thread [98e][216] NR_eMIMO_RRM. The discussion thread was moderated by Yiyan Zhang (Samsung).

Decision: 		The document was revised to R4-2103698.

R4-2103698	Email discussion summary: [98e][216] NR_eMIMO_RRM

Type: other		For: discussion
					Source: Moderator (Samsung)

(Replaces R4-2103455)

Discussion:

The contribution summarised email discussion thread [98e][216] NR_eMIMO_RRM. The discussion thread was moderated by Yiyan Zhang (Samsung).

Decision: 		The document was noted.

GTW session (February 04, 2021)

[98e][216] NR_eMIMO_RRM

Issue 2-2-1: Additional FR2 margin for CMR+IMR L1-SINR measurement Scenario

- Proposals

  - Option 1: No difference between FR1 and FR2 requirement. (Huawei, MediaTek, Ericsson, Nokia, Samsung)

  - Option 2: Introduce 1dB extra RF margin for CMR+IMR case in FR2. (Apple, Qualcomm)

- Moderator view

  - Technically no reason identified for option 2. The only reason for extra FR2 margin is test error due to implementation.

- Discussion

  - QC: we prefer Option 2. If CMR and IMR come from different directions, then they would experience different antenna gains. Should we account this in the requirements?

  - Apple: CMR and IMR measurements are not done at the same time. Need to account for possible mismatch.

    - Samsung: time difference is very short. It should not cause 1 dB loss.

  - MTK: Option 1. There are already big margins in Annex B to account for different antenna gains.

  - Samsung: CMR/IMR in the test come from the same directions.

  - Nokia: to Apple, what would change over time?

  - Nokia: to QC, for different directions, can we average over long time?

    - QC: UE does not need to make such averaging.

  - Samsung: Signal will come from beam peak direction. Both CMR and IMR come from the beam peak direction.

  - E///: CMR/IMR can be scheduled in the same slot.

- Agreements

  - No additional margin for CMR+IMR L1-SINR measurement is introduced for FR2

  - The requirements are defined under the following assumption

    - CMR/IMR in the test come from the same directions

    - CMR/IMR are scheduled in the same slot

    - Note: capture the respective assumptions in the specification

Issue 2-2-2: Other implementation margins for L1-SINR measurement accuracy

- Proposals

  - For L1-SINR measurement accuracy in FR1/FR2, add a margin of 1dB considering the fading condition.

    - Option 1: Support (MediaTek, Ericsson, Qualcomm)

    - Option 2: Do not support (Samsung)

- Moderator view

  - Some companies think fading channel should be considered in the test. But in our understanding for test case the pathloss will be precisely controlled, then only a very little measurement error will exist. 1dB channel fading in the test is unreasonable.

- Discussion

  - MTK: 1 dB was discussed in Rel-15 for RSRP as well and included.

  - E///: Same view as MTK. Measurement requirement is generic for different propagation conditions.

  - QC: Same view as MTK and E///

  - Nokia: how was 1dB derived? Is it similar between FR1 and FR2?

    - MTK: based on simulation results

  - Samsung: can compromise to 1dB. Does it mean that we should derive the requirements based on the fading channel? The test is still AWGN.

    - E///: It is same methodology as RSRP – accuracy derived based on AWGN and margin added to account for fading conditions.

- Agreements

  - Add 1 dB margin for L1-SINR measurement accuracy in FR1/FR2 to take into account the fading conditions.

#### 7.9.1	UE RF core requirements maintenance (38.101)  [NR_eMIMO-Core]

R4-2102386	CR for TS 38.101-1: correction of Pi/2 BPSK

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0687  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was not pursued.

#### 7.9.2	RRM core requirements maintenance (38.133) [NR_eMIMO-Core]

R4-2100202	RRM Core requirements maintenance for eMIMO

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was noted.

R4-2100203	CR to 38.133 on RRM requirements for multi-TRP (R16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1453  Cat: B (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was postponed.

R4-2100204	CR to 38.133 on RRM requirements for multi-TRP (R17)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1454  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was withdrawn.

R4-2100205	CR to 38.133 on Link Recovery requirements (R16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1455  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was revised to R4-2104055.

R4-2104055	CR to 38.133 on Link Recovery requirements (R16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1455  rev 1 Cat: F (Rel-16)

					Source: Apple, MediaTek

(Replaces R4-2100205)

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was agreed.

R4-2100206	CR to 38.133 on Link Recovery requirements (R17)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1456  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was agreed.

R4-2100207	CR to 38.133 on Pathloss activation delay requirements (R16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1457  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was revised to R4-2103589.

R4-2103589	CR to 38.133 on Pathloss activation delay requirements (R16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1457  rev 1 Cat: F (Rel-16)

					Source: Apple

(Replaces R4-2100207)

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was agreed.

R4-2100208	CR to 38.133 on Pathloss activation delay requirements (R17)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1458  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was agreed.

R4-2100755	Discussion on the scaling factor for SCell beam failure recovery

Type: discussion		For: Discussion
					38.133 v..
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was noted.

R4-2100756	CR on the scaling factor for SCell beam failure recovery in R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1508  Cat: F (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was merged.

R4-2100757	CR on the scaling factor for SCell beam failure recovery in R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1509  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was withdrawn.

R4-2100758	Correction on the measurement restriction for CSI-IM resource in R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1510  Cat: F (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was agreed.

R4-2100759	Correction on the measurement restriction for CSI-IM resource in R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1511  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was agreed.

R4-2100933	CR to TS38.133 on L1-SINR measurement requirement

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1533  Cat: F (Rel-16)

					Source: Samsung

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was revised to R4-2103590.

R4-2103590	CR to TS38.133 on L1-SINR measurement requirement

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1533  rev 1 Cat: F (Rel-16)

					Source: Samsung

(Replaces R4-2100933)

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was agreed.

R4-2100934	CR to TS38.133 on L1-SINR measurement requirement

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1534  Cat: A (Rel-17)

					Source: Samsung

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was agreed.

R4-2101670	Discussion on maintaining issues for L1-SINR measurent requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was noted.

R4-2101671	CR on maintaining L1-SINR measurent requirements Rel-16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1642  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was revised to R4-2103591.

R4-2103591	CR on maintaining L1-SINR measurent requirements Rel-16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1642  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101671)

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was agreed.

R4-2101672	CR on maintaining L1-SINR measurent requirements Rel-17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1643  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was agreed.

R4-2102864	On Rel-16 NR eMIMO multi-TRxP transmissions

Type: other		For: Discussion
					38.133 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Proposal 1: Option 2: no need to revise the spec.

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was noted.

#### 7.9.3	RRM perf. requirements (38.133) [NR_eMIMO-Perf]

R4-2103588	WF on NR eMIMO RRM Performance requirements

Type: other		For: discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was revised to R4-2104068.

R4-2104068	WF on NR eMIMO RRM Performance requirements

Type: other		For: discussion
					Source: Samsung

(Replaces R4-2103588)

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was approved.

R4-2100938	Big CR: Introduction of Rel-16 NR eMIMO RRM performance requirements and test cases

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1535  Cat: B (Rel-16)

					Source: Samsung

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2100939	Big CR: Introduction of Rel-16 NR eMIMO RRM performance requirements and test cases

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1536  Cat: A (Rel-17)

					Source: Samsung

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2103726	L1-SINR simulation results summary

Type: other		For: discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was noted.

##### 7.9.3.1	General [NR_eMIMO-Perf]

R4-2101674	Discussion on IMR configuration for L1-SINR measurement tests

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was noted.

R4-2101675	DraftCR on IMR configuration for L1-SINR measurement tests

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was endorsed.

##### 7.9.3.2	L1-SINR measurement accuracy [NR_eMIMO-Perf]

R4-2100209	RRM Performance requirements for L1-SINR Measurement Accuracy

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was noted.

R4-2100868	Simulation results for the measurement of L1-SINR

Type: discussion		For: Information
					Source: CMCC

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was noted.

R4-2100869	Discussion on L1-SINR measurement accuracy

Type: discussion		For: Approval
					Source: CMCC

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was noted.

R4-2100935	Remaining issues on L1-SINR measurement accuracy requirement

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was noted.

R4-2101445	L1-SINR measurement accuracy

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution shows our simulation results on L1-SINR and discusses the accuracy requirements.

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was noted.

R4-2101673	Discussion on L1-SINR measurement accuracy requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was noted.

R4-2102865	Discussions on Rel-16 NR eMIMO L1-SINR measurement Accuracy

Type: other		For: Discussion
					38.133 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

The document has discussed the remaining open issues on L1-SINR measurements.

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was noted.

R4-2102867	CR to TS 38.133: Adding L1-SINR accuracy requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

The CR provides the text proposal for L1-SINR absolute accuracy requirements for FR1 and FR2, which is in accordance with the agreed WF (R4-2017375) and simulation results.

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was revised to R4-2103592.

R4-2103592	CR to TS 38.133: Adding L1-SINR accuracy requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102867)

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was endorsed.

R4-2102868	CR to TS 38.133: Adding conditions for L1-SINR reporting (Annex B.2)

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

The CR provides the text proposal for the conditions for NR L1-SINR reporting, which are required by the L1-SINR accuracy requirements.

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was postponed.

##### 7.9.3.3	Test cases [NR_eMIMO-Perf]

###### 7.9.3.3.1	L1-SINR measurements [NR_eMIMO-Perf]

R4-2100936	Draft CR on L1-SINR measurement accuracy tests with CSI-RS based CMR and dedicated IMR

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Samsung

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was revised to R4-2103593.

R4-2103593	Draft CR on L1-SINR measurement accuracy tests with CSI-RS based CMR and dedicated IMR

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Samsung

(Replaces R4-2100936)

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was endorsed.

R4-2101676	DraftCR on L1-SINR measurement procedure tests with SSB CMR and dedicated IMR

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was endorsed.

R4-2102912	Draft test case CR on measurement performance of L1-SINR for CSI-RS-based CMR and no dedicated IMR

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Qualcomm CDMA Technologies

(Replaces R4-2014292)

Abstract:

Resubmission of R4-2014292

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was revised to R4-2103598.

R4-2103598	Draft test case CR on measurement performance of L1-SINR for CSI-RS-based CMR and no dedicated IMR

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Qualcomm CDMA Technologies

(Replaces R4-2102912)

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was endorsed.

###### 7.9.3.3.2	BFR for SCell [NR_eMIMO-Perf]

R4-2100754	Introduction of test cases for BFD and link recovery procedure for Scell

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was revised to R4-2103595.

R4-2103595	Introduction of test cases for BFD and link recovery procedure for Scell

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: MediaTek inc.

(Replaces R4-2100754)

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was endorsed.

R4-2101446	Test case for SCell beam failure recovery

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the test case for SCell beam failure recovery.

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was noted.

R4-2101447	Draft CR: Introduction of test case of link recovery with link recovery requests

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

This draft CR introduces the test case for link recovery with LRR.

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was revised to R4-2103596.

R4-2103596	Draft CR: Introduction of test case of link recovery with link recovery requests

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2101447)

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was endorsed.

###### 7.9.3.3.3	DL/UL beam indication with reduced latency and overhead [NR_eMIMO-Perf]

R4-2100044	Test cases for applicable timing for PL RS activated by MAC-CE

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was noted.

R4-2100045	[CR] Test cases for applicable timing for PL RS activated by MAC-CE

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1414  Cat: B (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was withdrawn.

R4-2100054	[draft CR] Test cases for applicable timing for PL RS activated by MAC-CE

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: ZTE Corporation

Abstract:

This CR adds test cases to 38.133 in correspondance to the core requirements for PL RS activation delay.

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was revised to R4-2103597.

R4-2103597	[draft CR] Test cases for applicable timing for PL RS activated by MAC-CE

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: ZTE Corporation

(Replaces R4-2100054)

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was postponed.

R4-2101678	Discussion on testability of pathloss-RS activation delay

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was noted.

###### 7.9.3.3.4	Others [NR_eMIMO-Perf]

R4-2100937	Draft CR on CSI-RS configurations

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Samsung

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was merged.

R4-2101677	DraftCR on L1-SINR measurement accuracy tests with SSB CMR and dedicated IMR

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was revised to R4-2103594.

R4-2103594	DraftCR on L1-SINR measurement accuracy tests with SSB CMR and dedicated IMR

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2101677)

Discussion:

See email discussion summary for [98e][216] NR_eMIMO_RRM in R4-2103698.

Decision: 		The document was endorsed.

#### 7.9.4	Demodulation and CSI requirements (38.101-4) [NR_eMIMO-Perf]

##### 7.9.4.1	General [NR_eMIMO-Perf]

R4-2103763	Email discussion summary for [98e][324] NR_eMIMO_Demod

Type: other		For: discussion
					Source: Moderator (Samsung)

Discussion:

The contribution summarised email discussion thread [98e][324] NR_eMIMO_Demod. The discussion thread was moderated by Yunchuan Yang (Samsung).

Decision: 		The document was revised to R4-2103939.

R4-2103939	Email discussion summary for [98e][324] NR_eMIMO_Demod

Type: other		For: discussion
					Source: Moderator (Samsung)

(Replaces R4-2103763)

Discussion:

The contribution summarised email discussion thread [98e][324] NR_eMIMO_Demod. The discussion thread was moderated by Yunchuan Yang (Samsung).

Session Chair note: The performance part (Demod/CSI) is completed, further results and update on the requirements in specifications if needed can be discussed in Rel-16 maintenance AI in future RAN4 meetings.

Decision: 		The document was noted.

R4-2103825	WF on NR eMIMO demodulation and CSI requirements

Type: other		For: discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was approved.

R4-2103832	Simulation results for single and multi-DCI based transmission scheme

Type: other		For: discussion
					Source: Qualcomm

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

-----------------------------------GTW agenda Jan.28th for [324] (30 minutes) -----------------------

eType II codebook test cases

Issue 2-1-1: Test Metric for eType II codebook

- Proposals

  - Option 1: Only introduce relative TP between following eType-II/random PMI with Type I codebook (Samsung, Huawei, Ericsson, QC, Apple)

  - Option 2 (Nokia): Besides relative TP ratio, introduce one additional test metric as following

- Introduce  a performance requirement for eType II PMI reporting that KNZ≥2ν,, for any rank V= 1,2,3,4

- Introduce an additional test requirement in performance testing if KNZ=4. In this case a UE fails the test if all the following conditions are satisfied with probability p>ϵ (for example ϵ=10-3)

- i2,3,l=[1515], for l=1,2

- kl,(il*+L) mod 2L,f(3)=1, for l=1,2, and one value of f∈{0,1,…,Mν}

- c1,i1*L+1mod 2,f=c2,i2*L+1mod 2,f+8 mod 16

- Recommended WF

Result observations

From the simulation results from all the companies’ results, we can see that eType II show enough performance gap over than Type I with test metric of TP ratio between following eType II and random Type I, which is enough to discriminate UE behaviour to ensure proper UE processing for eType II.

Therefore, it is feasible to introduce proper test requirements to ensure UE reporting eType II properly, i.e UE reporting Type I codebook only will fail the test cases. There is no need additional test metric to check UE reported codebook not only within Type I codebook set.

| Cases | TP ratio (Following PMI/Rand PMI) | Samsung | Qualcomm | Huawei | Ericsson | Apple |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FDD 16x2 | eType II/ Random Type I at 90% point | 6.5 | 6.16 | 7.4 | 2.3 | 3.41 |  |
|  | Type I/ Random Type I at 90% point | 2.1 | 2.19 | 2.1 | 1.5 | 2.01 |  |
| FDD 16x4 | eType II/ Random Type I at 90% point | 6.05 | 3.69 | 7.1 | 2.3 | 2.85 |  |
|  | Type I/ Random Type I at 90% point | 2.01 | 2.12 | 2.12 | 1.7 | 2.11 |  |
| TDD 16x2 | eType II/ Random Type I at 90% point | 4.05 | 4.18 | 5.82 | 3 | 3.49 |  |
|  | Type I/ Random Type I at 90% point | 1.83 | 2.05 | 2.2 | 2.2 | 2.03 |  |
| TDD 16x4 | eType II/ Random Type I at 90% point | 4.15 | 3.1 | 5.73 | 2.4 | 3 |  |
|  | Type I/ Random Type I at 90% point | 1.76 | 2.01 | 2.3 | 2.2 | 2.31 |  |

As recommended WF:

  - Introduce Rel-16 Type II codebook requirements with test metric as following PMI (eType II)/Random PMI (Type I codebook)

Agreement: Introduce Rel-16 Type II codebook requirements with test metric as following PMI (eType II)/Random PMI (Type I codebook)

Issue 2-1-2: MIMO Correlation

- Proposals

  - Option 1: XP Medium (Samsung, QC, Apple, [Huawei])

  - Option 2: XP low (Ericsson)

- Recommended WF

From most of companies’ results, the performance gap between following eType II and Type I is more obvious under MIMO-Medium correlation compared with MIMO-Custom Low correlation. Based on majority view

As recommended WF:

  - Option 1: XP Medium

Agreement: XP medium

Issue 2-1-3: Test point

- Proposals

  - Option 1: 70% of TP (Ericsson)

  - Option 2: 90% of TP (Samsung, Apple, Qc)

  - Option 3: 95% of TP (Huawei)

- Recommended WF

From most of companies results submitted, the performance under 90% relative TP point is more stable to introduce requirement. Based on majority view

As recommended WF:

  - Option 2: 90% of TP

Agreement: 90% of TP

Issue 2-1-4: gamma value

- Proposals

  - Option 1(Samsung): With XP medium and 90% relative TP ratio point preference

- FDD

- 2Rx: 3.0

- 4Rx: 3.0

- TDD

- 2Rx: 2.5

- 4Rx: 2.5

  - Option 2(Ericsson): With XP low and 70% relative TP ratio point preference

- FDD

- 2Rx: 2.5

- 4Rx: 1.9

- TDD

- 2Rx: 2.8

- 4Rx: 1.9

- Recommended WF

| Cases | TP ratio (Following PMI/Rand PMI) | Samsung | Qualcomm | Huawei | Ericsson | Apple |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FDD 16x2 | eType II/ Random Type I at 90% point | 6.5 | 6.16 | 7.4 | 2.3 | 3.41 | 2.2 |
|  | Type I/ Random Type I at 90% point | 2.1 | 2.19 | 2.1 | 1.5 | 2.01 |  |
| FDD 16x4 | eType II/ Random Type I at 90% point | 6.05 | 3.69 | 7.1 | 2.3 | 2.85 | 2.2 |
|  | Type I/ Random Type I at 90% point | 2.01 | 2.12 | 2.12 | 1.7 | 2.11 |  |
| TDD 16x2 | eType II/ Random Type I at 90% point | 4.05 | 4.18 | 5.82 | 3 | 3.49 | 2.2 |
|  | Type I/ Random Type I at 90% point | 1.83 | 2.05 | 2.2 | 2.2 | 2.03 |  |
| TDD 16x4 | eType II/ Random Type I at 90% point | 4.15 | 3.1 | 5.73 | 2.4 | 3 | 2.2 |
|  | Type I/ Random Type I at 90% point | 1.76 | 2.01 | 2.3 | 2.2 | 2.31 |  |

Agreement:

FDD: 16X2 ->[2.2] ,16x4 -> [2.2]

TDD: 16x2->[2.2], 16x4->[ 2.2]

Companies are encouraged to further align the results and aims to introduce requirements with gamma values in [ ] in this meeting, Further update the results and the values in future RAN4 meetings not excluded.

PDSCH requirements

Issue 1-1-1: FRC for single-DCI for FDM scheme A

       Proposals

o   Option 1: Add a note in FRC of single-DCI for FDM scheme A to clarify the TBS determinate

o   Option 2: TBA

       Recommended WF

o   Encourage feedback from companies for solution of FRC definition for FDM scheme A

Agreement:

It’s RAN4 common understanding, TBS determination for FRC of single DCI FDM scheme A based on full resource allocation BW.

-FFS for how to introduce the test set-up into specification with clear differentiation of multi-TRP and single-TRP, companies will further discuss based on drafting CR

Apple: Not obvious how to set-up the tests.

Issue 1-1-2: FRC for single-DCI for inter-slot TDM scheme

       Proposals

o   Option 1: Apply the same FRC table as Rel-16 URLLC aggregation factor 2, with additional note to differentiate Rel-16 URLLC with single TRP transmission and Rel-16 NR eMIMO with single-DCI based inter-slot TDM scheme as

         Option 1a: Note 4: Throughput is calculated under assumption of repetition number 2 (Samsung)

         Option 1b: Note 3: Throughput is calculated under assumption of aggregation factor 2 or repetition number 2 depending on Tx scheme (Intel, Samsung )

o   Option 2: TBA

       Recommended WF

Agreement: Throughput is calculated under the assumption of aggregation factor 2 for URLLC slot aggregation schemes or repetition number 2 for inter-slot repetition scheme.

Issue 1-3-1: Requirements definition for 38.101-4

- Proposals

  - Option 1: SNR = average of IM results +extra margin

- extra margin

- 64QAM 0.8 dB

- 16QAM 0.5 dB

- Recommended WF

  - Define tentative SNR requirements for the agreed test cases as much as possible and update CR including SNR requirements for test cases with []

  - Introduce requirement as table summarized for test cases which the ideal results span among companies’ results within [2.5dB] (mark with blue color), using the same extra margin values

  - For test cases which the span among companies’ results larger [2.5dB] (mark with yellow colour) further checking the details of simulation assumptions and results and extra margin cases by cases if needed

Agreement:

  - Option 1: SNR = average of IM results +extra margin

- extra margin

- 64QAM 0.8 dB

- 16QAM 0.5 dB

Companies are encouraged to further align the results and aims to introduce requirements with SNR values in [ ] in this meeting, Further update the results and the values in future RAN4 meetings not excluded.

----------------------------------------- End ------------------------------------------

R4-2100210	CR to 38.101-4 for eMIMO demod requirements - General and Applicability rule

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0128  Cat: B (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was revised to R4-2103826.

R4-2103826	CR to 38.101-4 for eMIMO demod requirements - General and Applicability rule

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0128  rev 1 Cat: B (Rel-16)

					Source: Apple

(Replaces R4-2100210)

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was agreed.

R4-2100903	Simulation results summary for eMIMO performance requirements

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2101448	CR: FRC for eMIMO sDCI/mDCI-based PDSCH transmission

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0159  Cat: B (Rel-16)

					Source: Ericsson, Huawei, HiSilicon, Intel, Samsung

Abstract:

This CR provides the FRCs used for sDCI/mDCI-based PDSCH transmission.

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was revised to R4-2103830.

R4-2103830	CR: FRC for eMIMO sDCI/mDCI-based PDSCH transmission

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0159  rev 1 Cat: B (Rel-16)

					Source: Ericsson, Huawei, HiSilicon, Intel, Samsung

(Replaces R4-2101448)

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was agreed.

R4-2101449	Simulation assumption for PDSCH requirement with mDCI/sDCI-based transmission schemes

Type: other		For: Information
					Source: Ericsson

Abstract:

This contribution provides the updated simulation assumption for PDSCH requirement with mDCI/sDCI-based transmission schemes.

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was approved.

##### 7.9.4.2	Demodulation requirements [NR_eMIMO-Perf]

###### 7.9.4.2.1	Single-DCI based SDM scheme [NR_eMIMO-Perf]

R4-2100898	Simulation results for Single-DCI based SDM scheme

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2101256	Simulation results for single-DCI based multi-TRP SDM Tx scheme

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2101313	Simulation results of PDSCH requirements for Single-DCI SDM scheme

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was revised to R4-2103960.

R4-2103960	Simulation results of PDSCH requirements for Single-DCI SDM scheme

Type: other		For: Information
					Source: Huawei, HiSilicon

(Replaces R4-2101313)

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2101315	CR for 38.101-4 Introduction of PDSCH requirement with Single-DCI based SDM scheme

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0147  Cat: B (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was revised to R4-2103828.

R4-2103828	CR for 38.101-4 Introduction of PDSCH requirement with Single-DCI based SDM scheme

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0147  rev 1 Cat: B (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101315)

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was agreed.

R4-2101450	Simulation results of sDCI-based SDM transmission

Type: other		For: Information
					Source: Ericsson

Abstract:

This contribution shows our PDSCH simulation results with sDCI-based SDM transmission.

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

###### 7.9.4.2.2	Multi-DCI based transmission scheme  [NR_eMIMO-Perf]

R4-2100899	Simulation results for multi-DCI based transmsision scheme

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2101257	Simulation results for multi-DCI based multi-TRP Tx scheme

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2101312	Simulation results of PDSCH requirements for Multi-DCI transmission scheme

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2101316	CR for 38.101-4 Introduction of PDSCH requirement with Multi-DCI based multi-TRP transmission schemes

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0148  Cat: B (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was revised to R4-2103829.

R4-2103829	CR for 38.101-4 Introduction of PDSCH requirement with Multi-DCI based multi-TRP transmission schemes

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0148  rev 1 Cat: B (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101316)

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was revised to R4-2103986.

R4-2103986	CR for 38.101-4 Introduction of PDSCH requirement with Multi-DCI based multi-TRP transmission schemes

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0148  rev 2 Cat: B (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2103829)

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was agreed.

R4-2101451	Simulation results of mDCI-based transmission

Type: other		For: Information
					Source: Ericsson

Abstract:

This contribution shows our PDSCH simulation results with mDCI-based transmission.

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2102083	Simulation results on PDSCH performance requirements for multi-DCI based multi-TRP transmission

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

###### 7.9.4.2.3	Single-DCI based transmission schemes (URLLC) [NR_eMIMO-Perf]

R4-2100211	Simulation results for multi-DCI based transmission scheme

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2100900	Simulation results for URLLC schemes

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was revised to R4-2103831.

R4-2103831	Simulation results for URLLC schemes

Type: discussion		For: Discussion
					Source: Samsung

(Replaces R4-2100900)

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2101258	Simulation results for single-DCI based multi-TRP Repetition Tx schemes

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2101259	CR to TS 38.101-4: Performance requirements single-DCI based multi-TRP Repetition Tx schemes

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0143  Cat: B (Rel-16)

					Source: Intel Corporation, Samsung, Ericsson, Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was revised to R4-2103827.

R4-2103827	CR to TS 38.101-4: Performance requirements single-DCI based multi-TRP Repetition Tx schemes

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0143  rev 1 Cat: B (Rel-16)

					Source: Intel Corporation, Samsung, Ericsson, Huawei, HiSilicon

(Replaces R4-2101259)

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was agreed.

R4-2101314	Simulation results of PDSCH requirements for Single-DCI URLLC schemes

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2101452	Simulation results of sDCI-based FDM/TDM transmission

Type: other		For: Information
					Source: Ericsson

Abstract:

This contribution shows our PDSCH simulation results with sDCI-based FDMSchemeA and inter-slot TDM.

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

##### 7.9.4.3	CSI requirements [NR_eMIMO-Perf]

R4-2100212	PMI reporting requirements with eType II codebook

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was revised to R4-2103961.

R4-2103961	PMI reporting requirements with eType II codebook

Type: discussion		For: Discussion
					Source: Apple

(Replaces R4-2100212)

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2100585	On PMI reporting requirements for enhanced Type II codebooks

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2100622	Views on CSI Reporting test cases for eMIMO

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was revised to R4-2103962.

R4-2103962	Views on CSI Reporting test cases for eMIMO

Type: discussion		For: -
					Source: Qualcomm Incorporated

(Replaces R4-2100622)

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2100896	Introduction of PMI test cases with Rel-16 eType II codebook

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0135  Cat: B (Rel-16)

					Source: Samsung

Abstract:

Introduction of eType II codebook PMI test cases

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was revised to R4-2102938.

R4-2102938	Introduction of PMI test cases with Rel-16 eType II codebook

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0135  rev 1 Cat: B (Rel-16)

					Source: Samsung

(Replaces R4-2100896)

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was revised to R4-2103833.

R4-2103833	Introduction of PMI test cases with Rel-16 eType II codebook

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0135  rev 2 Cat: B (Rel-16)

					Source: Samsung

(Replaces R4-2102938)

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was agreed.

R4-2100901	Discussion and simulation results for Rel-16 eType II codebook test cases

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2101310	Discussion on the test metric of eType II codebook based PMI reporting test

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2101311	Simulation results for eType II codebook based PMI reporting test

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was revised to R4-2103963.

R4-2103963	Simulation results for eType II codebook based PMI reporting test

Type: other		For: Information
					Source: Huawei, HiSilicon

(Replaces R4-2101311)

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2101323	CR for 38.101-4 Applicablity of PMI reporting test of eType II codebook

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0150  Cat: B (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was revised to R4-2103954.

R4-2103954	CR for 38.101-4 Applicablity of PMI reporting test of eType II codebook

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0150  rev 1 Cat: B (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101323)

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was agreed.

R4-2101443	Simulation results for Rel-16 eType II PMI reporting test

Type: other		For: Information
					Source: Ericsson

Abstract:

This contribution shows our simulation results of Rel-16 eType-II PMI reporting.

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2101444	Evaluation of Rel-16 eType-II PMI reporting test

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the requirements of Rel-16 eType-II PMI reporting.

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-2102282	On PMI reporting requirements for enhanced Type II codebooks

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was noted.

R4-21043963	Delete this!

Type: other		For: Information
					Source: ETSI MCC

Discussion:

See email discussion summary for [98e][324] NR_eMIMO_Demod in R4-2103939.

Decision: 		The document was not treated.

### 7.10	Add support of NR DL 256QAM for FR2 [NR_DL256QAM_FR2]

#### 7.10.1	Demodulation and CSI requirements (38.101-4)  [NR_DL256QAM_FR2-Perf]

##### 7.10.1.1	UE Demodulation requirements [NR_DL256QAM_FR2-Perf]

R4-2103764	Email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod

Type: other		For: discussion
					Source: Moderator (China Telecomm)

Discussion:

The contribution summarised email discussion thread [98e][325] NR_DL256QAM_FR2_Demod. The discussion thread was moderated by Jingzhou Wu (China Telecom Corporation Ltd.).

Decision: 		The document was revised to R4-2103940.

R4-2103940	Email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod

Type: other		For: discussion
					Source: Moderator (China Telecomm)

(Replaces R4-2103764)

Discussion:

The contribution summarised email discussion thread [98e][325] NR_DL256QAM_FR2_Demod. The discussion thread was moderated by Jingzhou Wu (China Telecom Corporation Ltd.).

Session Chair Note: Further discuss on the channel model simplification procedure in future RAN4 meeting not precluded; if needed it can be treated in Rel-16 Performance maintenance phase.

Further discuss on the channel matrix applied for LOS path in future RAN4 meeting not precluded; if needed it can be treated in Rel-16 Performance maintenance phase.

Decision: 		The document was noted.

------------------------------GTW agenda on Jan.28th for email thread (325) (30 minutes) ------------

Topic #3: CQI reporting requirements

Issue 3-1: SNR testing points for FR2 CQI Table 2 test

- Higher SNR point without impairment margin

  - Summary of simulation results on Percentage of reported CQI index > 11 @ SNR 17/18/19 dB

| SNR (dB) |  | 17 | 18 | 19 |
| --- | --- | --- | --- | --- |
| Percentage of reported CQI index > 11 (%) | CTC | 47.00 | 59.00 | 71.00 |
|  | HW | 33.56 | 42.91 | 52.66 |
|  | QC | 22.17 | 29.84 | 39.22 |
|  | ZTE | 28.88 | 62.15 | 87.65 |

  - Option1: 17/18 dB (CTC, ZTE, E///, HW)

  - Option2: 18/19 dB (QC)

Discussion:

QC: We are running to show 256QAM gain table over 64QAM table. If companies no such intention, we are also fine with option1.

Huawei: This results is alignment results, not taking into account impairment. Does this value aims to specify the value into specification.

CTC: We have issue 3-4 for that.

Huawei: It’s not logic to separate the discussion. We need to decide the final test points into specification considering alignment results and margin if any.

Spark: share similar view as QC, we would see the gain over 64QAM.

Huawei: From performance testing, it’s better to verify 256QAM through test cases with high SNR points to ensure the reporting probability of 256QAM during test cases.

CTC: We think the extra margin didn’t consider in existing test cases for 64QAM tables.

QC: SNR point with 64QAM, the impairment margin not big issue. For 256QAM, with much high SNR points, the implementation margin does impact.

E///: Depending on the purpose of such test cases, it’s to verify 256QAM table index, then 17/18 should be OK. If we want to ensure CQI index with reporting 256QAM index in some probability, then margin we can consider. We have concern on test feasibility with test SNR points over than 20dB.

CTC: We are fine extra margin with some note:

The extra margin is introduced due to high SNR points with 256QAM reporting enable.

Huawei: We share similar view as QC.

Options for high SNR points into specification:

Option 1: 17/18dB (CTC, ZTE, E///)

Option 2: 20/21dB (Huawei)

Option 3: 21/22dB (QC)

Agreements:

[20/21] dB

Note: Above SNR test points agreed with the consideration of impairment margin due to high SNR points with 256QAM reporting configuration

- Lower SNR point without impairment margin

  - Summary of simulation results on TP ratio with following CQI and Median CQI @ SNR 6/7/8 dB

| SNR (dB) |  | 6 | 7 | 8 |
| --- | --- | --- | --- | --- |
| TP ratio with following CQI and Median CQI | CTC | 1.23 | 1.37 | 1.23 |
|  | E/// | 1.24 | 1.24 | 1.14 |
|  | HW | 1.16 | 1.24 |  |
|  | QC | 0.93 | 0.98 | 1.10 |
|  | ZTE | 1.27 | 1.13 |  |

  - Option 1: 6/7 dB (Agreed parameter in the last meeting in R4-2017536)

  - Option 2: 7/8 dB (QC)

Discussion:

QC: we are not OK with option 1 based on our simulation results.

Agreements: [7/8] dB

Issue 3-3: Test requirement for FR2 CQI Table 2 test

- Candidate options:

| Test metric |  |  |
| --- | --- | --- |
|  | Option 1 (slightly preferred by CTC) | Option 2 (Existing test requirements in Rel-15 FR2 CQI table 1 Test with 100MHz CBW, E///, CTC, HW, ZTE) |
| α: Minimum percentage (%) of reported CQI not in {median CQI – 1, median CQI, median CQI + 1} | 20% | [2]% |
| γ: Minimum TP ratio of followed CQI and fixed median CQI | 1.1 | [1.05] |
| Minimum PDSCH BLER with followed CQI | 0.02 | [0.01] |

- Check if we can go with option 2 after high and low SNR points in issue 3-1 are decided.

Agreements: Option 2 with values in [ ].

Topic #2: SDR requirements

Issue 2-1: Applicability of SDR requirements for UE capable of 256QAM in certain band(s)

- Candidate options:

  - Option 1: Add following applicability in Table 7.1.1.3-1 and no additional change is needed to the test requirement in clause 7.5A.1 in TS 38.101-4 (CTC)

Table 7.1.1.3-1: Requirements applicability for optional UE features UE feature/capability [14]; Test type; Test list; Applicability notes. 256QAM for PDSCH (pdsch-256QAM-FR2); FR2 TDD; SDR; Clause 7.5A.1; For UE capable of PDSCH 256QAM for certain band(s), the MCS table is configured to ‘64QAM’ for SDR test, i.e., no additional SDR test for UE capable of PDSCH 256QAM feature..

  - Option 2: Add following notes in Table 7.5A.1-3 in the test requirement in clause 7.5A.1 in TS 38.101-4 (HW)

    - Note 1: MCS Index is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12].

    - Note 2: For the band(s) on which UE supporting “Maximum modulation format” of 8, the MCS index is derived from the rows with “Maximum modulation format” of 6.

  - Both option 1 and note 2 in option 2 (Intel)

  - Both option 1 and option 2 (QC)

- Companies to check whether ‘option 1 and note 2 in option 2’ can be agreeable, i.e.,

  - Add applicability of option 1 in Table 7.1.1.3-1: Requirements applicability for optional UE features.

  - Add Note2 in option2 in Table 7.5A.1-3: MCS indexes for indicated UE capabilities.

  - No additional change is needed.

Discussion:

Huawei: We think option 1, plus note 1, note 2 needed.

Intel: Note 1 is reductant, as already capture in tables.

CTC: we share similar view as Intel.

Option 1 + note 2 in option 2, and FFS whether note 1 needed or not

FFS further offline for CR drafting whether note 1 needed or not

-------------------------------End ---------------------

R4-2100880	Offline e-mail discussion summary on the TDLD30 channel simplification

Type: discussion		For: Information
					Source: China Telecom

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

R4-2100881	Simulation results for PDSCH normal demodulation for FR2 DL 256QAM

Type: discussion		For: Discussion
					Source: China Telecom

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

R4-2101250	Discussion on FR2 DL 256QAM UE demodulation requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

R4-2101251	Summary of simulation results for FR2 DL 256QAM demodulation requirements

Type: other		For: Information
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

R4-2101252	CR on simplified TDL-D channel model for FR2 DL 256QAM demodulation requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0140  Cat: B (Rel-16)

					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was revised to R4-2103835.

R4-2103835	CR on simplified TDL-D channel model for FR2 DL 256QAM demodulation requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0140  rev 1 Cat: B (Rel-16)

					Source: Intel Corporation

(Replaces R4-2101252)

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was agreed.

R4-2101296	Simulation results on PDSCH requirements for NR DL 256QAM for FR2

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

R4-2101297	CR on applicability and FRC for PDSCH normal demodulation for DL 256QAM for FR2

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0144  Cat: B (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was agreed.

R4-2101369	Views on 256QAM UE requirements for FR2

Type: discussion		For: Discussion
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

R4-2101419	Simulation results of PDSCH with 256QAM in FR2

Type: other		For: Information
					Source: Ericsson

Abstract:

This contribution shows our PDSCH simulation results with 256QAM in FR2.

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

R4-2101420	Open issues on FR2 256QAM PDSCH demodulation requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discussion the open issues on PDSCH demodulation requirements with 256QAM in FR2.

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

R4-2102373	Simulation results for FR2 256QAM PDSCH Test Cases

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

##### 7.10.1.2	CSI requirements [NR_DL256QAM_FR2-Perf]

R4-2100882	On CQI reporting requirements for FR2 DL 256QAM

Type: discussion		For: Discussion
					Source: China Telecom

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

R4-2100883	Summary of CQI reporting simulation results for FR2 DL 256QAM (TDD)

Type: discussion		For: Information
					Source: China Telecom

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

R4-2100884	CR on adding applicability, requirements and measurement channel for FR2 DL 256QAM CQI reporting test under fading condition

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0133  Cat: B (Rel-16)

					Source: China Telecom

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was revised to R4-2103837.

R4-2103837	CR on adding applicability, requirements and measurement channel for FR2 DL 256QAM CQI reporting test under fading condition

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0133  rev 1 Cat: B (Rel-16)

					Source: China Telecom

(Replaces R4-2100884)

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was agreed.

R4-2101114	Simulation For CQI reporting requirements for FR2 DL 256QAM

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

R4-2101116	CR on demodulation performance requirements for DL 256QAM for FR2

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0138  Cat: F (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was revised to R4-2103834.

R4-2103834	CR on demodulation performance requirements for DL 256QAM for FR2

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0138  rev 1 Cat: F (Rel-16)

					Source: ZTE Corporation

(Replaces R4-2101116)

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was agreed.

R4-2101298	Simulation results on CQI requirements for NR DL 256QAM for FR2

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

R4-2101421	Simulation results of CQI table 2 reporting in FR2

Type: other		For: Information
					Source: Ericsson

Abstract:

This contribution provides our simulation results of CQI reporting with CQI table 2.

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

R4-2101422	Open issues on CQI table 2 reporting test in FR2

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the open issues on CQI table 2 reporting test in FR2.

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

R4-2101848	Simulation results on CQI requirements for NR DL 256QAM for FR2

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

R4-2102406	Views on FR2 256QAM CQI Reporting Test Cases

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

##### 7.10.1.3	SDR requirements [NR_DL256QAM_FR2-Perf]

R4-2100885	Updating on CR for SDR requirements for FR2 DL 256QAM capable band

Type: discussion		For: Discussion
					Source: China Telecom

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was noted.

R4-2101253	CR on applicability rules and FRC for FR2 DL 256QAM CQI requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0141  Cat: B (Rel-16)

					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was revised to R4-2103838.

R4-2103838	CR on applicability rules and FRC for FR2 DL 256QAM CQI requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0141  rev 1 Cat: B (Rel-16)

					Source: Intel Corporation

(Replaces R4-2101253)

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was agreed.

R4-2101299	CR on SDR requirements for DL 256QAM for FR2

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0145  Cat: B (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was revised to R4-2103836.

R4-2103836	CR on SDR requirements for DL 256QAM for FR2

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0145  rev 1 Cat: B (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101299)

Discussion:

See email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod in R4-2103940.

Decision: 		The document was agreed.

### 7.11	RF requirements for NR frequency range 1 (FR1) [NR_RF_FR1]

#### 7.11.1	RF core requirements maintenance [NR_RF_FR1-Core]

R4-2102957	Email discussion summary for [98e][109] NR_RF_FR1_Part_1

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][109] NR_RF_FR1_Part_1. The discussion thread was moderated by Qian Zhang (HiSilicon Technologies Co. Ltd).

Decision: 		The document was revised to R4-2103299.

R4-2103299	Email discussion summary for [98e][109] NR_RF_FR1_Part_1

Type: other		For: discussion
					Source: Moderator (Huawei)

(Replaces R4-2102957)

Discussion:

The contribution summarised email discussion thread [98e][109] NR_RF_FR1_Part_1. The discussion thread was moderated by Qian Zhang (HiSilicon Technologies Co. Ltd).

Decision: 		The document was noted.

R4-2103146	Way forward on intra-band UL CA Pcmax

Type: other		For: discussion
					Source: Huawei

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was approved.

##### 7.11.1.1	Intra-band UL CA for FR1 power class 3  [NR_RF_FR1-Core]

R4-2100162	CA_n7B_REFSENS_CatF_CR

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0600  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was agreed.

R4-2102588	CR CA_n7B REFSENS

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0695  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was agreed.

R4-2101178	Intra-band contiguous ULCA Pcmax Issues

Type: other		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was noted.

R4-2102600	CR for TS 38.101-1: Corrections to intra-band UL NC CA requirements

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0700  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was revised to R4-2103149.

R4-2103149	CR for TS 38.101-1: Corrections to intra-band UL NC CA requirements

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0700  rev 1 Cat: F (Rel-16)

					Source: Apple

(Replaces R4-2102600)

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was agreed.

R4-2102601	CR for TS 38.101-1: Corrections to intra-band UL NC CA requirements

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0701  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was agreed.

R4-2102626	on FR1 intra-band CA separation class

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was noted.

##### 7.11.1.2	Others [NR_RF_FR1-Core]

R4-2100160	Introduction of specific Pcmax requirements for inter-band CA category A-B combos

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0599  Cat: C (Rel-16)

					Source: InterDigital Communications

Abstract:

Introduction of specific Pcmax requirements for inter-band CA category A-B combos.

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was not pursued.

R4-2103147	Introduction of specific Pcmax requirements for inter-band CA category A-B combos

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0717  Cat: B (Rel-17)

					Source: InterDigital Communications, Charter Communications

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was agreed.

R4-2100596	UL Switching and coherent UL MIMO Cat-F CR 38.101-1

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0623  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was not pursued.

R4-2100597	UL Switching and coherent UL MIMO Cat-A CR 38.101-1

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0624  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was withdrawn.

R4-2100792	CR for TS 38.101-1: Correction on 1Tx-2Tx switching between two uplink carriers (Rel-16)

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0629  Cat: F (Rel-16)

					Source: China Telecom

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was agreed.

R4-2100793	CR for TS 38.101-1: Correction on 1Tx-2Tx switching between two uplink carriers (Rel-17)

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0630  Cat: F (Rel-17)

					Source: China Telecom

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was agreed.

R4-2100794	CR for TS 38.101-3: Correction on 1Tx-2Tx switching between two uplink carriers (Rel-16)

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0447  Cat: F (Rel-16)

					Source: China Telecom

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was agreed.

R4-2100795	CR for TS 38.101-3: Correction on 1Tx-2Tx switching between two uplink carriers  (Rel-17)

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0448  Cat: A (Rel-17)

					Source: China Telecom

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was agreed.

R4-2101145	Clarification on timing difference for Tx switching in EN-DC R16

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0456  Cat: F (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was revised to R4-2103353.

R4-2103353	Clarification on timing difference for Tx switching in EN-DC R16

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0456  rev 1 Cat: F (Rel-16)

					Source: MediaTek inc.

(Replaces R4-2101145)

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was agreed.

R4-2101146	Clarification on timing difference for Tx switching in EN-DC R17

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0457  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was agreed.

R4-2101285	CR to 38.101-1 (Rel-16) fall back behaviors

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0639  Cat: F (Rel-16)

					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was not pursued.

R4-2101286	CR to 38.101-1 (Rel-17) fall back behaviors

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0640  Cat: A (Rel-17)

					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was withdrawn.

R4-2103350	Corrections to PCMAX for UL CA

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0718  Cat: F (Rel-15)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was agreed.

R4-2102410	Corrections to PCMAX for UL CA

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0691  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was revised to R4-2103148.

R4-2103148	Corrections to PCMAX for UL CA

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0691  rev 1 Cat: F (Rel-16)

					Source: Qualcomm Incorporated

(Replaces R4-2102410)

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was agreed.

R4-2102411	Corrections to PCMAX for UL CA

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0692  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was agreed.

R4-2102682	on FR1 intra-band UL CA Pcmax

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was noted.

R4-2103097	Way forward on introduction of NR intra-band UL CA as UL configuration in an inter-band combination

Type: other		For: discussion
					Source: Skyworks

Discussion:

See email discussion summary for [98e][109] NR_RF_FR1_Part_1 in R4-2103299.

Decision: 		The document was approved.

#### 7.11.2	RRM requirements maintenance (38.133) [NR_RF_FR1-Core/Perf]

R4-2103456	Email discussion summary: [98e][217] NR_RF_FR1_RRM

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][217] NR_RF_FR1_RRM. The discussion thread was moderated by Jing Han (Huawei).

Decision: 		The document was revised to R4-2103699.

R4-2103699	Email discussion summary: [98e][217] NR_RF_FR1_RRM

Type: other		For: discussion
					Source: Moderator (Huawei)

(Replaces R4-2103456)

Discussion:

The contribution summarised email discussion thread [98e][217] NR_RF_FR1_RRM. The discussion thread was moderated by Jing Han (Huawei).

Decision: 		The document was noted.

R4-2103599	WF on test case for DL interruption due to Tx switching between two uplink carriers

Type: other		For: discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][217] NR_RF_FR1_RRM in R4-2103699.

Decision: 		The document was approved.

R4-2101147	Discussion on interruption test cases for Tx switching

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][217] NR_RF_FR1_RRM in R4-2103699.

Decision: 		The document was noted.

R4-2101148	Update on interruption test cases for Tx switching R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1559  Cat: F (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][217] NR_RF_FR1_RRM in R4-2103699.

Decision: 		The document was revised to R4-2103600.

R4-2103600	Update on interruption test cases for Tx switching R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1559  rev 1 Cat: F (Rel-16)

					Source: MediaTek inc.

(Replaces R4-2101148)

Discussion:

See email discussion summary for [98e][217] NR_RF_FR1_RRM in R4-2103699.

Decision: 		The document was agreed.

R4-2101149	Update on interruption test cases for Tx switching R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1560  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][217] NR_RF_FR1_RRM in R4-2103699.

Decision: 		The document was agreed.

R4-2101710	Correction on test cases of DL interruptions at switching between two uplink carriers

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1659  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][217] NR_RF_FR1_RRM in R4-2103699.

Decision: 		The document was revised to R4-2103601.

R4-2103601	Correction on test cases of DL interruptions at switching between two uplink carriers

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1659  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101710)

Discussion:

See email discussion summary for [98e][217] NR_RF_FR1_RRM in R4-2103699.

Decision: 		The document was agreed.

R4-2101711	Correction on test cases of DL interruptions at switching between two uplink carriers

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1660  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][217] NR_RF_FR1_RRM in R4-2103699.

Decision: 		The document was agreed.

### 7.12	NR RF requirement enhancements for frequency range 2 (FR2) [NR_RF_FR2_req_enh]

#### 7.12.1	RF core requirements maintenance [NR_RF_FR2_req_enh-Core]

R4-2102958	Email discussion summary for [98e][110] NR_RF_FR2_req_enh_Part_4

Type: other		For: discussion
					Source: Moderator (Nokia)

Discussion:

The contribution summarised email discussion thread [98e][110] NR_RF_FR2_req_enh_Part_4. The discussion thread was moderated by Petri Vasenkari (Nokia).

Decision: 		The document was revised to R4-2103300.

R4-2103300	Email discussion summary for [98e][110] NR_RF_FR2_req_enh_Part_4

Type: other		For: discussion
					Source: Moderator (Nokia)

(Replaces R4-2102958)

Discussion:

The contribution summarised email discussion thread [98e][110] NR_RF_FR2_req_enh_Part_4. The discussion thread was moderated by Petri Vasenkari (Nokia).

Decision: 		The document was noted.

R4-2103150	Way forward on addition of new frequency FR2 frequency separation classes

Type: other		For: discussion
					Source: Qualcomm, Huawei, Nokia

Discussion:

See email discussion summary for [98e][110] NR_RF_FR2_req_enh_Part_4 in R4-2103300.

Decision: 		The document was approved.

R4-2100589	P_cmax P_IBE wording refinement and terminology improvement

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0323  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Abstract:

Clarify P_IBE applicability and add A-MPR = 0 condition

Discussion:

See email discussion summary for [98e][110] NR_RF_FR2_req_enh_Part_4 in R4-2103300.

Decision: 		The document was revised to R4-2103358.

R4-2103358	P_cmax P_IBE wording refinement and terminology improvement

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0323  rev 1 Cat: F (Rel-16)

					Source: Qualcomm Incorporated

(Replaces R4-2100589)

Discussion:

See email discussion summary for [98e][110] NR_RF_FR2_req_enh_Part_4 in R4-2103300.

Decision: 		The document was agreed.

R4-2100590	P_cmax P_IBE wording refinement and termonology improvement

Type: CR		For: Agreement
					38.101-2 v17.0.0	  CR-0324  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

Clarify P_IBE applicability and add A-MPR = 0 condition

Discussion:

See email discussion summary for [98e][110] NR_RF_FR2_req_enh_Part_4 in R4-2103300.

Decision: 		The document was agreed.

R4-2100591	Inter-band + intra-band CA FR2 frequency separation class

Type: other		For: Approval
					Source: Qualcomm Incorporated

Abstract:

Add 400 and 600 MHz to Fs

Discussion:

See email discussion summary for [98e][110] NR_RF_FR2_req_enh_Part_4 in R4-2103300.

Decision: 		The document was noted.

R4-2101287	CR to 38.101-2 (Rel-16) SSB based beam correspondence side condition

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0326  Cat: F (Rel-16)

					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][110] NR_RF_FR2_req_enh_Part_4 in R4-2103300.

Decision: 		The document was not pursued.

R4-2101288	CR to 38.101-2 (Rel-17) SSB based beam correspondence side condition

Type: CR		For: Agreement
					38.101-2 v17.0.0	  CR-0327  Cat: A (Rel-17)

					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][110] NR_RF_FR2_req_enh_Part_4 in R4-2103300.

Decision: 		The document was withdrawn.

R4-2102132	Discussion on frequency separation class

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][110] NR_RF_FR2_req_enh_Part_4 in R4-2103300.

Decision: 		The document was withdrawn.

R4-2102183	Discussion on frequency separation class

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][110] NR_RF_FR2_req_enh_Part_4 in R4-2103300.

Decision: 		The document was noted.

R4-2102718	CR on FR2 inter-band DL CA CBM and IBM

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0344  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][110] NR_RF_FR2_req_enh_Part_4 in R4-2103300.

Decision: 		The document was revised to R4-2103369.

R4-2103369	CR on FR2 inter-band DL CA CBM and IBM

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0344  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2102718)

Discussion:

See email discussion summary for [98e][110] NR_RF_FR2_req_enh_Part_4 in R4-2103300.

Decision: 		The document was revised to R4-2103405.

R4-2103405	CR on FR2 inter-band DL CA CBM and IBM

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0344  rev 2 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2103369)

Discussion:

See email discussion summary for [98e][110] NR_RF_FR2_req_enh_Part_4 in R4-2103300. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 7.12.2	RRM requirements maintenance (38.133) [NR_RF_FR2_req_enh-Core]

### 7.13	NR RRM requirement enhancement [NR_RRM_Enh-Core]

R4-2103457	Email discussion summary: [98e][218] NR_RRM_Enh_RRM_1

Type: other		For: discussion
					Source: Moderator (Intel Corporation)

Discussion:

The contribution summarised email discussion thread [98e][218] NR_RRM_Enh_RRM_1. The discussion thread was moderated by Hua Li (Intel).

Decision: 		The document was revised to R4-2103700.

R4-2103700	Email discussion summary: [98e][218] NR_RRM_Enh_RRM_1

Type: other		For: discussion
					Source: Moderator (Intel Corporation)

(Replaces R4-2103457)

Discussion:

The contribution summarised email discussion thread [98e][218] NR_RRM_Enh_RRM_1. The discussion thread was moderated by Hua Li (Intel).

Decision: 		The document was noted.

GTW session (January 29, 2021)

Issue 3-1-1: Whether to define DCI/Timer based FR1+FR2 simultaneous BWP switch test case

- Proposals

  - Option 1 (Intel, Huawei): Yes

  - Option 1a (Ericsson, Nokia): Yes, with Pcell in FR1 and Scell in FR2

  - Option 2 (MTK):

    - Option 1: Define FR1+FR2 simultaneous BWP switch test case in RAN4.

    - Option 2: If TE vendors confirm FR1 demodulation performance can’t be guaranteed in OTA mode, RAN4 deletes multiple BWP switch, and multiple Scell activation test cases which UE needs to correctly demodulate the downlink channels for FR1 in OTA mode.

  - Option 3 (vivo, QC, Apple): No.

- Moderator note: Suggest companies to consider the comment from TE vendor in the 2nd round discussion, which are as follows:

  - If an FR1+FR2 BWP switch test case can be considered as a “functional” test, and if we are OK to specify the test case with FR1 as a link only, same as in A.7.3.1.1 (FR1 to FR2 handover) or in A.7.5.3.2 (FR1+FR2 SCell Act/Deact), then we could define a test case.

  - FR1 is configured with noise-free condition. For FR2 Test cases using FR1 link, the FR1 cells should refer to clause A.3.7A, which states “. The Test System shall provide a stable and noise-free NR FR1 signal”.

- Discussion

  - QC: we have link and measurement antenna in the chamber. Spec does not assume performance verification on FR1. There may be some performance failure on FR1.

  - Apple: Functional test is possible. This test is not fully functional.

  - Vivo: Option 3

  - QC: UE will be able to receive PDCCH. Not sure on reliability of FR2 test environment. May not pass FR2 certification.

  - E///: We prefer to have a test. TE vendors confirmed no issues with the test cases and can provide a stable link.

  - QC: FR1 is not tested as of now.

  - MTK: we can define the test case. If TE vendors identify further issues then it can be handled as a RAN5 issue.

  - Apple: do we have precedence of hybrid of FR1-FR2 test?

  - MTK: for current test cases the FR1 link is functional. Decision depends on TE vendors.

  - QC: FR1 + FR2 testing is not possible

  - QC: 38.810 - no performance verification is supported for FR1

  - Session chair: encourage TE vendors to provide more inputs in the 2nd round regarding the feasibility of the test. Come back in the 2nd round.

Issue 3-1-3: Whether to define RRC based BWP switch on multiple CCs test case

- Proposals

  - Option 1 (vivo, Intel):

- No for both simultaneous and non-simultaneous case

  - Option 2 (Huawei):

- Wait for the conclusion on feasibility of RRC based switch on multiple CCs

  - Option 3 (Ericsson, QC, Apple, MTK, vivo, Huawei, Nokia, Intel):

- To be discussed after receiving RAN2 response followed by the conclusion on their requirements

Agreement: Test cases for RRC based BWP switch on multiple CCs will be discussed after receiving RAN2 response followed by the conclusion on the respective Core requirements

Issue 2-1-1: Whether to consider the pathloss RS for PUCCH spatial relation info switch requirements

- Proposals

  - Option 1 (Apple, QC, Ericsson, Nokia): Yes.

  - Option 2 (MTK, Huawei): No

  - Option 3 (NTT DOCOMO, Intel): Needs further clarification.

Discussion

Apple: Pathloss RS may change during PUCCH spatial relation info switch. We suggest to include PL RS into the requirements.

MTK: not sure if we need to combine 2 independent requirements. For example, during BWP switching we may have UL spatial relation switch. No discussion on combination of multiple requirements is needed.

NTT DOCOMO: Not clear if UL spatial relation switch is related to PL RS change

QC: After further check of MTK comments – PL RS requirements are already specified in other sections. We can simply refer to the PL RS requirements in the UL spatial relation requirements

Huawei: PL RS switching is for UL power control and UL spatial relation switch – it is for spatial direction change. If IE includes both, then the existing PL RS requirements already cover this case. Prefer not to consider complicated cases.

Intel: PL RS is aligned with the DL RS for which PUCCH is associated. They should be in the same TCI chain. If they are in different TCI chain then no requirements. Also prefer not to consider known+unknown combinations.

E///: We are fine to keep these issues separate.

Apple: UL spatial relation requirements do not refer to PL RS at all. We need to specify that additional requirements would apply.

Nokia: If it is already covered by different parts then we can keep it. Meantime we can add clarifications.

Session chair: Continue discussion in the second round. PL RS case needs to be referred in the specification for UL spatial relation switch. It is not expected that detailed requirements will be defined. A reference to the PL RS section can be provided or the requirements can be extended.

GTW session (February 02, 2021)

Issue 3-1-1: Whether to define DCI/Timer based FR1+FR2 simultaneous BWP switch test case

- Proposals

  - Option 1 (Intel, Huawei): Yes

  - Option 1a (Ericsson, Nokia): Yes, with Pcell in FR1 and Scell in FR2

  - Option 2 (MTK):

    - Option 1: Define FR1+FR2 simultaneous BWP switch test case in RAN4.

    - Option 2: If TE vendors confirm FR1 demodulation performance can’t be guaranteed in OTA mode, RAN4 deletes multiple BWP switch, and multiple Scell activation test cases which UE needs to correctly demodulate the downlink channels for FR1 in OTA mode.

  - Option 3 (vivo, QC, Apple): No.

  - Moderator note: Suggest companies to consider the comment from TE vendor in the 2nd round discussion, which are as follows:

    - If an FR1+FR2 BWP switch test case can be considered as a “functional” test, and if we are OK to specify the test case with FR1 as a link only, same as in A.7.3.1.1 (FR1 to FR2 handover) or in A.7.5.3.2 (FR1+FR2 SCell Act/Deact), then we could define a test case.

    - FR1 is configured with noise-free condition. For FR2 Test cases using FR1 link, the FR1 cells should refer to clause A.3.7A, which states “. The Test System shall provide a stable and noise-free NR FR1 signal”.

- 1st round GTW summary

  - Session chair: encourage TE vendors to provide more inputs in the 2nd round regarding the feasibility

- Discussion

  - Anritsu: the existing FR1+FR2 tests require functional link. It means that we cannot test demodulation performance in FR1. It will be more challenging if we want to control SNR or noise level in FR1. Functional link is ok.

  - R&S: agree with Anritsu. In case FR1 has functional link without precise SNR, then it is ok. If we need to include FR1 performance (i.e. control SNR) then we need to have additional test methods studies.

  - MTK: would like to further check. We would like to check if it is possible to set the signal level larger than certain level (e.g. -4dB) to ensure that control channels can be decoded?

  - QC: For noise environment approach TE just transmits useful signal without artificial noise. If we cannot control the DL signal level then we cannot guarantee that UE can decode PDCCH.

  - E///: same understanding as MTK. UE needs to be able to receive PDCCH (e.g. anything > 0dB SNR) and we can check ACK/NACK.

  - Apple: can the performance be verified for the FR1 link? Can we check switching delay requirements or interruption requirements.

  - Anritsu: the TE cannot guarantee UE baseband SNR since it depends on UE antenna. Control requires conducted connection. If we want to guarantee a number then we need to have extension of testability studies. Is there anything new in this test?

  - R&S: the current test guarantee reliable link but current specs do not guarantee the specific signal level. Prefer to refrain from the numbers.

  - Chair: are there any specific values we can guarantee

    - Anritsu, R&S: no specific level

  - Apple: how to guarantee that the link is reliable? What is the definition? Can we guarantee that UE can decode RRC configuration?

  - QC: for certification test we need to have full confidence it can work.

  - Huawei: In Rel-15 we already have FR1 + FR2 tests which are quite similar to the current test setup. Are there any issues with these tests?

    - R&S: So far we have not seen any issues. In all EN-DC test case we have LTE OTA link. Same will happen with FR1. Do not see difference and it is working.

    - Anritsu: When the test gets validated we check that UE can decode the link. It is not in our interest to provide the link which cannot be decoded.

    - QC: we can bring CRs to remove such tests in the future

    - vivo: same view as QC. No performance verification on FR1.

  - E///: Can we check that TE can reliably receive on FR1?

    - Anritsu: need to check

  - Intel: there should be ways to validate that the FR1 link is reliable. We can say in RAN4 spec that the FR1 link is reliable.

  - QC: checked with RAN5 colleagues. The test is not feasible. The proper place would be to discuss it in RAN plenary in the scope of FR1 TRP/TRS item. We object to introduce FR1+FR2 test. Prefer not to include RAN5.

  - Apple: we should address the testability issue first in RAN5.

  - Samsung: Is there any boundary for the testability topic? Should we continue this discussion?

  - E///: agree with Samsung. BWP switching does not require UE to do measurements. We can check ACK/NACK on FR2 only.

  - R&S: we have an issue in configuring the level for FR1.

- Session chair: Tentative agreement

  - Define DCI/Timer based FR1+FR2 simultaneous BWP switch test case in RAN4.

    - Add a note that the test is not expected to be verified in RAN5

    - Companies can further check the FR1 + FR2 testability issues as a part of Rel-16 maintenance

  - QC: object

  - Apple: object. Prefer to have studies first.

- Agreement:

  - Do not define DCI/Timer based FR1+FR2 simultaneous BWP switch test case

- Session chair:

  - Companies are encouraged to further discuss FR1 + FR2 testability issues for existing test cases (e.g. as a part of Rel-16 maintenance)

R4-2103458	Email discussion summary: [98e][219] NR_RRM_Enh_RRM_2

Type: other		For: discussion
					Source: Moderator (ZTE)

Discussion:

The contribution summarised email discussion thread [98e][219] NR_RRM_Enh_RRM_2. The discussion thread was moderated by Richie Leo (ZTE).

Decision: 		The document was revised to R4-2103701.

R4-2103701	Email discussion summary: [98e][219] NR_RRM_Enh_RRM_2

Type: other		For: discussion
					Source: Moderator (ZTE)

(Replaces R4-2103458)

Discussion:

The contribution summarised email discussion thread [98e][219] NR_RRM_Enh_RRM_2. The discussion thread was moderated by Richie Leo (ZTE).

Decision: 		The document was noted.

GTW session (January 29, 2021)

Issue 2-1-1: FR1 + FR2 test cases

- Proposals

  - Option 1: Do not introduce FR1+FR2 SRS carrier switching tests. (Qualcomm, Huawei, vivo, Apple)

  - Option 2: Introduce at least one TC for FR1+FR2 SRS carrier switching. (Ericsson, Nokia)

- Discussion:

  - QC: One concern is testability problem. If the SNR is not provided then we cannot guarantee that UE can decode PDCCH. Also we already have FR1 + FR1 interruption test.

  - Huawei: FR1 and FR2 have different RF components and it may not be possible to borrow the RF chains.

  - vivo: Same view as Huawei. FR1 and FR2 switching is not practical. No necessity to have such test case

  - Nokia: Does this case include 1) switching between FR1 and FR2 or 2) the switching within FR1 and impact on FR2

  - Huawei: for this case we discuss Case 1 – switching between FR1 and FR2. Case 2 is discussed in another issue.

Agreements

Further check the feasibility of SRS carrier-based switching between different frequency ranges

Do not define test cases for SRS carrier-based switching between FR1 and FR2 in Rel-16

Issue 2-1-3: How to configure the test parameter usage

- Proposals

  - Option 1: Set the parameter usage to ‘antennaSwitching’ in SRS configuration. (Apple, Huawei, Qualcomm, vivo)

  - Option 2: Set the parameter usage to ‘antennaSwitching’ in SRS configuration for aperiodic SRS. (Nokia)

- Discussion:

  - Nokia: Based on RAN1 spec ‘antennaSwitching’ applies to aperiodic SRS case only. No restriction for periodic SRS.

  - QC: It should be aperiodic one in accordance to the test description.

Agreement

Set the parameter usage to ‘antennaSwitching’ in SRS configuration

- The parameter applies to aperiodic SRS case only

Issue 2-1-5: TCs for SA interruptions

- Proposals

  - Option 1 (Huawei, Ericsson, vivo, Nokia): The below tests can be added into SA interruptions at NR SRS carrier based switching. The added test cases are only specified for UE capable of/configured with per-FR gap.

    - TC1: PCell in FR1, SCell in FR1, adding an SCell in FR2

    - TC2: PCell in FR2, SCell in FR2, adding an SCell in FR1

  - Option 2 (Qualcomm): Agree on Option 1 if Option 1 in Issue 2-1-1 is agreed

- Discussion:

  - QC: disagree with Option 1

  - Huawei: this is relevant to switching within one FR. We can wait for the conclusion of FR1 + FR2 testing.

  - Session chair: come back in the 2nd round and involve TE vendors on testability questions.

Issue 4-1: Whether the UE is allowed to skip R15 tests

- Proposals

  - Option 1: For the scenario which is without SSB time index detection and when DRX is not used, the Rel-15 MG related test cases can be skipped if UE passes the Rel-16 new introduced MG related test cases for the same scenario. For other scenarios, no Rel-15 test cases can be skipped. (Qualcomm, CMCC, Huawei, ZTE, MediaTek)

  - Option 2: No. R15 test cases on mandatory gap patterns shall be inherited completely to R16 specifications, and R16 UEs shall pass all test cases. (ZTE, CMCC, Ericsson, Nokia)

  - Option 3: (Apple)

    - allow UE to skip some existing test case configured with GP#0 if it has to be verified in the new test case configured with GP#2.

    - allow UE to skip some existing test case configured with GP#13 if it has to be verified in the new test case configured with GP#17.

    - The new test cases configured with GP#2 or GP#17 can be introduced by replacing existing test cases configured with GP#0 or GP#13.

- Discussion:

  - Apple: ok with Option 1

  - E///: we would like to further check and come back on Mon

Tentative agreement: For the scenario which is without SSB time index detection and when DRX is not used, the Rel-15 MG related test cases can be skipped if UE passes the Rel-16 new introduced MG related test cases for the same scenario. For other scenarios, no Rel-15 test cases can be skipped.

Session chair: come back in the 2nd round

Issue 3-1-1: Value for T321 in CGI reading with autonomous gap

Discussion

Apple: ok with Option 1

QC: ok with Option 1. Network needs to be aware that more time is needed in case side condition is not satisfied.

Agreement: Value for T321 in CGI reading with autonomous gap is 200ms.

GTW session (February 04, 2021)

[98e][219] NR_RRM_Enh_RRM_2

Issue 2-1-5: TCs for SA interruptions

- Proposals

  - Option 1 (Huawei, Ericsson, vivo, Nokia): The below tests can be added into SA interruptions at NR SRS carrier based switching. The added test cases are only specified for UE capable of/configured with per-FR gap.

    - TC1: PCell in FR1, SCell in FR1, adding an SCell in FR2

    - TC2: PCell in FR2, SCell in FR2, adding an SCell in FR1

  - Option 2 (Qualcomm): keep previous TC1 and TC2

- Agreements

  - Keep existing TC1 and TC2 definition

Issue: Whether the UE is allowed to skip R15 tests

- Proposals

  - Option 1: For the scenario which is without SSB time index detection and when DRX is not used, the Rel-15 MG related test cases can be skipped if UE passes the Rel-16 new introduced MG related test cases for the same scenario. For other scenarios, no Rel-15 test cases can be skipped. (Qualcomm, CMCC, Huawei, ZTE, MediaTek, Apple)

  - Option 2: No. R15 test cases on mandatory gap patterns shall be inherited completely to R16 specifications, and R16 UEs shall pass all test cases. (ZTE, CMCC, Ericsson, Nokia)

  - Option 3: (Apple)

    - allow UE to skip some existing test case configured with GP#0 if it has to be verified in the new test case configured with GP#2.

    - allow UE to skip some existing test case configured with GP#13 if it has to be verified in the new test case configured with GP#17.

    - The new test cases configured with GP#2 or GP#17 can be introduced by replacing existing test cases configured with GP#0 or GP#13.

- 1st round GTW tentative agreement: For the scenario which is without SSB time index detection and when DRX is not used, the Rel-15 MG related test cases can be skipped if UE passes the Rel-16 new introduced MG related test cases for the same scenario. For other scenarios, no Rel-15 test cases can be skipped.

- Discussion

  - Apple: agree with Option 1

  - Nokia: can consider to reduce the test cases for the new mandatory gap patterns

  - Apple: R16 – 2 new test cases. R15 – more than 9 test cases with GP0.

  - Apple/ZTE: Option 2 is the default assumption.

  - E///: we can discuss separately if R15 test cases can be skipped.

- Session chair: no consensus to consider Rel-15 test case skipping.

R4-2103459	Email discussion summary: [98e][220] NR_RRM_Enh_RRM_3

Type: other		For: discussion
					Source: Moderator (Apple)

Discussion:

The contribution summarised email discussion thread [98e][220] NR_RRM_Enh_RRM_3. The discussion thread was moderated by Jerry Cui (Apple).

Decision: 		The document was revised to R4-2103702.

R4-2103702	Email discussion summary: [98e][220] NR_RRM_Enh_RRM_3

Type: other		For: discussion
					Source: Moderator (Apple)

(Replaces R4-2103459)

Discussion:

The contribution summarised email discussion thread [98e][220] NR_RRM_Enh_RRM_3. The discussion thread was moderated by Jerry Cui (Apple).

Decision: 		The document was noted.

GTW session (January 29, 2021)

Issue 4-1: Power imbalance condition for inter-frequency measurement requirement without MG (perf)

- Proposals

  - Option 1 (Qualcomm, MTK, Huawei): Measurement accuracy requirement doesn’t apply when power imbalance between serving frequency layer and inter-frequency layer on which UE performs measurement without gap is larger than [6] dB.

  - Option 2: Do not define conditions

- Discussion

  - Nokia: this needs to be coordinated with RF session

  - CMCC: Issue raise late. No performance analysis to show the actual impacts. Cannot  agree.

  - QC: have an analytical justification for 6dB value. Current tests do not have issues and we do not ask to revise the test.

  - E///: support CMCC view.

  - Huawei: support QC. In Rel-15 UE used gap to perform measurement. UE switched to different CC and adjusted AGC based on SSB. For Rel-16 UE we have a different implementation and UE can use serving cell power for AGC settings.

- Session chair: No consensus to adopt proposal 1. Proponents can bring more simulation results to justify the proposal. For the TC – companies will check the actual power imbalance in the test setup and come back in the 2nd round.

Issue 6-1: AoA setup in TCs of inter-band CA requirement for FR2 UE measurement capability of independent Rx beam (perf)

- Proposals

  - Option 1 (Qualcomm, Huawei): AoA configuration for both Cell 1 and Cell 2 = Setup 3 (with rough beams)

  - Option 2 (Qualcomm): AoA configuration for both Cell 1 and Cell 2 = Setup 4b, and for Cell 1 a fixed Rx Beam Peak of Cell 1 (with rough beams)

  - Option 1a (to address Anritsu comment): AoA configuration for both Cell 1 and Cell 2 = Setup 3 (with rough beams) and FFS on the other parameter adjustment (e.g. Noc, Es, Es/Iot and etc) in the test cases.

- Agreement: AoA configuration for both Cell 1 and Cell 2 = Setup 3 (with rough beams) and FFS on the other parameter adjustment (e.g. Noc, Es, Es/Iot and etc) in the test cases.

Issue 1-1-1: The RTD (reception timing difference) with contiguous FR1 known cell or active serving cell to the to-be-activated SCell (core)

- Proposals

  - Option 1 (MTK, Apple, Ericsson, QC, HW, NEC): shall be no larger than 260ns.

  - Option 2 (Huawei): shall be no larger than CP/2 with respect to the to-be-activated SCell’s SSB numerology.

- Agreement: The RTD (reception timing difference) with contiguous FR1 known cell or active serving cell to the to-be-activated SCell shall be no larger than 260ns

Issue 1-2-2: Requirement applicability on the other being-activated SCells during the FR1 multiple Scells activation (core)

- Proposals

  - Option 1 (Huawei, Apple, Ericsson, QC, MTK): No requirement apply for other SCells being activated, if no requirements apply for any of the FR1 unknown SCell activated with the same MAC CE.

  - Option 2 (Nokia, NEC): Activation requirement still apply for other SCells being activated, even though no requirements apply for one of the FR1 unknown SCell activated with the same MAC CE.

- Discussion

  - Nokia: see no problem for Option 2.

  - Huawei: UE is not aware whether the side conditions will be met. Also, this is only about unknown SCells where UE will need to apply detection. It should address the question for NEC.

  - NEC: we can still compute the N1 value.

    - Huawei: it can be computed. But it is based on the condition that side condition is met for all SCells. If the side conditions are not met then the requirements will not apply. We try to address other SCells which share the searcher with this special SCell.

  - Nokia: disagree with Huawei that UE does not know the side condition. Also, the condition is per SCell. If UE does not know the condition, then it simply makes the detection.

  - Apple: there may be AGC issue which will affect all CCs.

  - NEC: if there are 4 SCells and side conditions are unknown for 2 SCells the how do we apply requirements (N1 = 2)?

    - Huawei: requirements for all SCells counted in N1 will not apply.

    - NEC: Option 1 is ok under such clarification

  - Huawei: RTD conditions are unknown for UE. UE reuses the values from intra-band.

- Tentative agreement: No requirement apply for other SCells being activated, if no requirements apply for any of the FR1 unknown SCell activated with the same MAC CE.

- Session chair: continue discussion in the 2nd round. Nokia will check if Option 1 is acceptable.

GTW session (February 04, 2021)

[98e][220] NR_RRM_Enh_RRM_3

Issue 7-1: Removal of TCs for multiple SCell activation due to the testability of simultaneous FR1+FR2 links

- Test cases

  - TC1: EN-DC (LTE +FR1) UE to activate 2 FR1 SCCs

  - TC2: EN-DC (LTE +FR1) UE to activate 2 FR2 SCCs

    - Interruption on LTE PCell and FR1 PSCell is tested associated with the activation delay

  - TC3: NR-DC (FR1 +FR2) UE to activate 1 FR1 SCC and 1 FR2 SCC

    - Interruption on FR1 and FR2 serving CCs are tested associated with the activation delay

- Proposals

  - Option 1 (Apple, QC, MTK): Remove the TC2 and TC3 at this stage until the testability of simultaneous FR1+FR2 is clarified.

  - Option 2a (QC): Keep TC2 but remove the interruption test part for FR1 PSCell in TC2 and remove TC3 at this stage until the testability of simultaneous FR1+FR2 is clarified

  - Option 2b (Huawei, Apple, Ericsson, MTK): Keep TC2 but remove the interruption test part for LTE and FR1 PSCell in TC2 and remove TC3 at this stage until the testability of simultaneous FR1+FR2 is clarified.

  - Option 3: FFS whether or which TC shall be removed. But TC1 shall be kept.

  - Option 4: Keep TC1 and TC2 in scope. Further discuss TC3 from testability point of view.

- Discussion

  - Apple: we already made an agreement on Option 2b.

  - Session chair: how can we address “simultaneous FR1+FR2” issue

  - QC: RAN4 or RAN5 should resolve it. It should be a separate SI. Not our area of expertise.

  - Apple: share same view as QC that it should be further discussed. Can have a separate item or send LS to RAN5.

  - E///: share same view QC. Need to look into this separately.

  - R&S: at least we should avoid providing the exact power levels for FR1

  - Apple: we checked RAN5 specs, they still list that applicability of FR1+FR2 tests is marked is FFS. Are there any RAN5 issues?

  - Session chair: Encourage companies to further look into “simultaneous FR1+FR2” issue to define a systematic approach to handle this in the future.

R4-2103619	WF on R16 RRM enhancement part 1

Type: other		For: discussion
					Source: Intel Corporation

Decision: 		The document was approved.

R4-2103609	WF on R16 RRM enhancement part 2

Type: other		For: discussion
					Source: ZTE Corporation

Decision: 		The document was revised to R4-2104070.

R4-2104070	WF on R16 RRM enhancement part 2

Type: other		For: discussion
					Source: ZTE Corporation

(Replaces R4-2103609)

Decision: 		The document was approved.

R4-2103610	LS on CGI reading with autonomous gaps

Type: LS out		For: Approval
					to RAN2
					Source: RAN4

Decision: 		The document was approved.

R4-2103620	WF on Rel-16 RRM enhancement part 3

Type: other		For: discussion
					Source: Apple

Decision: 		The document was approved.

#### 7.13.1	RRM core requirements maintenance (38.133) [NR_RRM_Enh-Core]

##### 7.13.1.1	Multiple Scell activation/deactivation  [NR_RRM_Enh-Core]

R4-2101058	Remaining Issues on multiple SCell Activation

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was noted.

R4-2102788	Discussion on remaining issues in multiple SCell activation

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was noted.

R4-2102789	CR on multiple SCell activation requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1770  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Decision: 		The document was revised to R4-2103621.

R4-2103621	CR on multiple SCell activation requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1770  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2102789)

Decision: 		The document was agreed.

R4-2102790	CR on multiple SCell activation requirements R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1771  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

##### 7.13.1.2	BWP switching on multiple CCs [NR_RRM_Enh-Core]

R4-2101408	Discussion on RRC based BWP switching on multiple CC

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was noted.

R4-2101409	CR on RRC based BWP switching on multiple CCs (Rel-16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1597  Cat: B (Rel-16)

					Source: Intel Corporation

Discussion:

Session chair: Cover sheet errors (What is the CR category? It reads F on the cover page but the Tdoc is reserved for category B.). See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was not pursued.

R4-2101410	CR on RRC based BWP switching on multiple CCs (Rel-17)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1598  Cat: A (Rel-17)

					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was withdrawn.

R4-2101631	CR on interruption requirements of BWP switch on multiple CCs for EN-DC

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7025  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2101632	CR on interruption requirements of BWP switch on multiple CCs for EN-DC

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7026  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2101633	Discussion on requirements maintenance for BWP switch on multiple CCs

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was noted.

R4-2102354	On DCI-based simultaneous BWP switch on multiple CCs

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on clarification of DCI-based BWP switching requirement for switching on multiple CCs, in response to LS reply from RAN1.

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was noted.

R4-2102355	CR 38.133 (8.6.2A) Clarification on DCI-triggered BWP switch on multiple CCs

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1720  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

Adding clarification for DCI-based BWP switching on multiple CCs w.r.t. k0, k1, k2.

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was revised to R4-2103602.

R4-2103602	CR 38.133 (8.6.2A) Clarification on DCI-triggered BWP switch on multiple CCs

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1720  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2102355)

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2102356	CR 38.133 (8.6.2A) Clarification on DCI-triggered BWP switch on multiple CCs

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1721  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

Adding clarification for DCI-based BWP switching on multiple CCs w.r.t. k0, k1, k2.

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2102722	CR on maintenance on BWP switch requirements on multiple CCs

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1745  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2102723	CR on maintenance on BWP switch requirements on multiple CCs (cat A)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1746  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

##### 7.13.1.3	Other requirements maintenance [NR_RRM_Enh-Core]

R4-2100052	CGI reading with autonomous gaps

Type: LS out		For: Approval
					to RAN2
					Source: ZTE Corporation

Abstract:

According to RAN2 discussion summary during RAN2 112-e, they are waiting for RAN4 input on the value of T321. This discussion paper along with a draft LS discusses this issue and provides a draft LS to be sent to RAN2.

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was noted.

R4-2100181	CR on maintenance for inter-band FR2 CA RRM R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1445  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2100182	CR on maintenance for inter-band FR2 CA RRM R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1446  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2100183	CR on UE behavior for UE specific CBW change R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1447  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was revised to R4-2103622.

R4-2103622	CR on UE behavior for UE specific CBW change R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1447  rev 1 Cat: F (Rel-16)

					Source: Apple

(Replaces R4-2100183)

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2100184	CR on UE behavior for UE specific CBW change R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1448  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2100213	Remaining issues for UL spatial relation switching requirements

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was noted.

R4-2100214	CR to 38.133 on UL spatial relation switch requirements (R16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1459  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was not pursued.

R4-2100215	CR to 38.133 on UL spatial relation switch requirements (R17)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1460  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was withdrawn.

R4-2100777	Discussion on Inter-band CA requirement for FR2

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was noted.

R4-2100778	Correction on scheduling availability and measurement restriction on FR2 inter-band CA in R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1521  Cat: F (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was revised to R4-2103623.

R4-2103623	Correction on scheduling availability and measurement restriction on FR2 inter-band CA in R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1521  rev 1 Cat: F (Rel-16)

					Source: MediaTek inc.

(Replaces R4-2100778)

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2100779	Correction on scheduling availability and measurement restriction on FR2 inter-band CA in R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1522  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2101219	CR on TS38.133 for inter-frequency measurement requirement without gap

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1585  Cat: F (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was revised to R4-2103624.

R4-2103624	CR on TS38.133 for inter-frequency measurement requirement without gap

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1585  rev 1 Cat: F (Rel-16)

					Source: MediaTek inc.

(Replaces R4-2101219)

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2101220	CR on TS38.133 for inter-frequency measurement requirement without gap

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1586  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2101689	Correction on interruptions of SRS carrier switching

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1647  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2101690	Correction on interruptions of SRS carrier switching

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1648  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2101691	Clarification on inter-frequency measurement without gap requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1649  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was postponed.

R4-2103625	Clarification on inter-frequency measurement without gap requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1649  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101691)

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was withdrawn.

R4-2101692	Clarification on inter-frequency measurement without gap requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1650  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was withdrawn.

R4-2101694	UL spatial relation switching to an unknown DL RS

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1651  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was revised to R4-2103603.

R4-2103603	UL spatial relation switching to an unknown DL RS

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1651  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101694)

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2101695	UL spatial relation switching to an unknown DL RS

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1652  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2101762	CR to 38.133 correction on SRS carrier based switching core requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1661  Cat: F (Rel-16)

					Source: vivo

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was revised to R4-2103611.

R4-2103611	CR to 38.133 correction on SRS carrier based switching core requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1661  rev 1 Cat: F (Rel-16)

					Source: vivo, Qualcomm, Huawei, HiSilicon, MediaTek Inc., Apple, Nokia

(Replaces R4-2101762)

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was postponed.

R4-2101763	CR to 38.133 correction on SRS carrier based switching core requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1662  Cat: A (Rel-17)

					Source: vivo

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was withdrawn.

R4-2102686	SRS carrier switching interruption CR

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1741  Cat: F (Rel-16)

					Source: Qualcomm, Inc.

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was postponed.

R4-2103612	SRS carrier switching interruption CR

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1741  rev 1 Cat: F (Rel-16)

					Source: Qualcomm, Inc.

(Replaces R4-2102686)

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was withdrawn.

R4-2102687	SRS carrier switching interruption CR

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1742  Cat: A (Rel-17)

					Source: Qualcomm, Inc.

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was withdrawn.

R4-2102791	CR on CGI reading requirements 38.133

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1772  Cat: F (Rel-16)

					Source: Huawei, HiSilicon, MediaTek

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was revised to R4-2103613.

R4-2103613	CR on CGI reading requirements 38.133

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1772  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon, MediaTek

(Replaces R4-2102791)

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2102792	CR on CGI reading requirements 38.133 R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1773  Cat: A (Rel-17)

					Source: Huawei, HiSilicon, MediaTek

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2102793	CR on CGI reading report delay 36.133

Type: CR		For: Agreement
					36.133 v16.8.0	  CR-7070  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

R4-2102794	CR on CGI reading report delay 36.133 R17

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7071  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was agreed.

#### 7.13.2	RRM perf. requirements (38.133) [NR_RRM_Enh-Perf]

##### 7.13.2.1	General [NR_RRM_Enh-Perf]

R4-2100225	Test applicability for mandatory gap patterns

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was noted.

R4-2101416	Big CR: Introduction of Rel-16 NR RRM enhancements WI performance requirements and test cases (Rel-16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1599  Cat: B (Rel-16)

					Source: Intel Corporation, ZTE Corporation, Apple

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was revised to R4-2103608.

R4-2103608	Big CR: Introduction of Rel-16 NR RRM enhancements WI performance requirements and test cases (Rel-16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1599  rev 1 Cat: B (Rel-16)

					Source: Intel Corporation, ZTE Corporation, Apple

(Replaces R4-2101416)

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2101417	Big CR: Introduction of Rel-16 NR RRM enhancements WI performance requirements and test cases (Rel-17)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1600  Cat: A (Rel-17)

					Source: Intel Corporation, ZTE Corporation, Apple

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

##### 7.13.2.2	Test cases [NR_RRM_Enh-Perf]

###### 7.13.2.2.1	SRS carrier switching requirements  [NR_RRM_Enh-Perf]

R4-2100226	TC4: E-UTRAN

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Apple

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was revised to R4-2103614.

R4-2103614	TC4: E-UTRAN

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Apple

(Replaces R4-2100226)

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was endorsed.

R4-2100624	SRS carrier switching discussion

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was noted.

R4-2101688	Discussion on SRS carrier switching test case

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was noted.

R4-2101764	Remaining issues for SRS carrier switching test cases

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was noted.

R4-2101765	CR to 38.133 on SRS configuration

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: vivo

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was revised to R4-2103615.

R4-2103615	CR to 38.133 on SRS configuration

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: vivo

(Replaces R4-2101765)

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was endorsed.

R4-2102533	TC2 - SA interruptions at NR SRS carrier based switching

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

TC2 - SA interruptions at NR SRS carrier based switching

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was revised to R4-2103616.

R4-2103616	TC2 - SA interruptions at NR SRS carrier based switching

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2102533)

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was revised to R4-2104085.

R4-2104085	TC2 - SA interruptions at NR SRS carrier based switching

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2103616)

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was endorsed.

R4-2102634	Test cases for BWP switching on multiple CCs for FR1+FR2

Type: other		For: Discussion
					Source: Ericsson

Abstract:

The paper discusses scenarios for RRM tests for multiple BWP switching on CCs with FR1 and FR2

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was noted.

###### 7.13.2.2.2	Multiple Scell activation/deactivation  [NR_RRM_Enh-Perf]

R4-2101059	draftCR on multiple SCell activation test case

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: MediaTek inc.

Decision: 		The document was revised to R4-2103626.

R4-2103626	draftCR on multiple SCell activation test case

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: MediaTek inc.

(Replaces R4-2101059)

Decision: 		The document was endorsed.

###### 7.13.2.2.3	CGI reading requirements with autonomous gap [NR_RRM_Enh-Perf]

R4-2100623	CGI reading discussion T321 LS

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was noted.

R4-2101766	On CGI reading test cases

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was noted.

R4-2102795	draftCR to CGI reading TC4

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was revised to R4-2103617.

R4-2103617	draftCR to CGI reading TC4

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2102795)

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was endorsed.

###### 7.13.2.2.4	BWP switching on multiple CCs [NR_RRM_Enh-Perf]

R4-2101060	Discussion on multiple BWP switch test case

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was noted.

R4-2101378	Further considerations on test cases for BWP switch over multiple CCs

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was noted.

R4-2101390	CR for test cases for simultaneously DCI/timer based bwp switch over mulitple CCs on FR1 in SA

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1595  Cat: B (Rel-16)

					Source: vivo

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was endorsed.

R4-2103604	CR for test cases for simultaneously DCI/timer based bwp switch over mulitple CCs on FR1 in SA

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1595  rev 1 Cat: B (Rel-16)

					Source: vivo

(Replaces R4-2101390)

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was withdrawn.

R4-2101411	Discussion on test case design for BWP switching on multiple CCs

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was noted.

R4-2101412	Draft CR on DCI-based and Timer-based simultaneous Active BWP Switch on multiple CCs on FR1 in EN-DC (section 4.5.6.x)

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was endorsed.

R4-2101634	Discussion on performance requirements for BWP switch on multiple CCs

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was noted.

R4-2101635	draftCR to introduce Active BWP Switch on multiple CCs TC2

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was revised to R4-2103605.

R4-2103605	draftCR to introduce Active BWP Switch on multiple CCs TC2

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2101635)

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was endorsed.

R4-2102362	On TC4 for simultaneous BWP switch on multiple CCs

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on test case for simultaneous BWP switching on multiple CCs.

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was noted.

R4-2102363	DraftCR 38.133 (A.7.5.X) TC4 BWP switching

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

Abstract:

TC4 for simultaneous BWP switching on multiple CCs

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was revised to R4-2103606.

R4-2103606	DraftCR 38.133 (A.7.5.X) TC4 BWP switching

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Ericsson

(Replaces R4-2102363)

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was endorsed.

###### 7.13.2.2.5	Inter-frequency measurement requirement without MG [NR_RRM_Enh-Perf]

R4-2100627	Inter frequency w/o gap power imbalance discussion

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Decision: 		The document was noted.

R4-2101693	Correction on inter-frequency measurement without gap test case

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Decision: 		The document was endorsed.

###### 7.13.2.2.6	Mandatory MG patterns  [NR_RRM_Enh-Perf]

R4-2100625	Mandatory MG applicability rule discussion

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was noted.

R4-2100626	Mandatory MG applicability rule CR

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Qualcomm, Inc.

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was revised to R4-2103618.

R4-2103618	Mandatory MG applicability rule CR

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Qualcomm, Inc.

(Replaces R4-2100626)

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was postponed.

R4-2100860	Discussion on test cases for mandatory MG patterns

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was noted.

R4-2101922	On test cases for mandatory gap patterns

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was noted.

R4-2102361	Test case applicability for mandatory measurement gaps in R15/R16

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on test case applicability for measurement gaps.

Discussion:

See email discussion summary for [98e][219] NR_RRM_Enh_RRM_2 in R4-2103701.

Decision: 		The document was noted.

###### 7.13.2.2.7	UE-specific CBW change [NR_RRM_Enh-Perf]

R4-2101070	draftCR on the CBW change test case for adding the UL CBW configuration

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: MediaTek inc.

Decision: 		The document was revised to R4-2103627.

R4-2103627	draftCR on the CBW change test case for adding the UL CBW configuration

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: MediaTek inc.

(Replaces R4-2101070)

Decision: 		The document was endorsed.

###### 7.13.2.2.8	Spatial relation switch for uplink [NR_RRM_Enh-Perf]

R4-2101696	Correction on uplink spatial relation switching test case

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was endorsed.

R4-2102265	Test case for RRC based UL spatial relation switch associated with a known DL-RS in SA for periodic SRS

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was revised to R4-2103607.

R4-2103607	Test case for RRC based UL spatial relation switch associated with a known DL-RS in SA for periodic SRS

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102265)

Discussion:

See email discussion summary for [98e][218] NR_RRM_Enh_RRM_1 in R4-2103700.

Decision: 		The document was endorsed.

###### 7.13.2.2.9	Inter-band CA requirement for FR2 UE measurement capability of independent Rx beam [NR_RRM_Enh-Perf]

R4-2101679	DraftCR on SCell activation and deactication delay test for FR2 inter-band CA

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Decision: 		The document was revised to R4-2103628.

R4-2103628	DraftCR on SCell activation and deactication delay test for FR2 inter-band CA

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2101679)

Decision: 		The document was endorsed.

R4-2102888	Updates on Test Configuration for FR2 Inter-band CA IBM UEs

Type: discussion		For: Discussion
					38.133 v..
					Source: Qualcomm Incorporated

Decision: 		The document was noted.

### 7.14	NR RRM requirements for CSI-RS based L3 measurement [NR_CSIRS_L3meas]

R4-2103460	Email discussion summary: [98e][221] NR_CSIRS_L3meas_RRM_1

Type: other		For: discussion
					Source: Moderator (CATT)

Discussion:

The contribution summarised email discussion thread [98e][221] NR_CSIRS_L3meas_RRM_1. The discussion thread was moderated by Qiuge Guo (CATT).

Decision: 		The document was revised to R4-2103703.

R4-2103703	Email discussion summary: [98e][221] NR_CSIRS_L3meas_RRM_1

Type: other		For: discussion
					Source: Moderator (CATT)

(Replaces R4-2103460)

Discussion:

The contribution summarised email discussion thread [98e][221] NR_CSIRS_L3meas_RRM_1. The discussion thread was moderated by Qiuge Guo (CATT).

Decision: 		The document was noted.

GTW session (January 27, 2021)

Issue 2-1-1: The upper bound of timing offset for case 1?

- Proposals

  - Option 1: (CATT, Xiaomi, CMCC, Nokia, OPPO, vivo, Apple, Huawei, Intel, QC)

    - CP.

  - Option 2: (MTK)

    - 0.9*CP.

  - Option 4: (Huawei)

    - CP/2.

- Discussion

  - MTK: need to clarify that this is for RSRP/RSRQ. For the TO = CP we observe some performance degradation.

  - Apple/Huawei/Intel/QC: ok with Option 1.

  - MTK: did companies consider positive/negative TO?

    - vivo: for simulation assumption we considered positive only. However from performance perspective positive/negative should be quite identical.

    - Apple: It depends on how to set the timing reference. UE can adjust its implementation to address different scenarios.

  - MTK: suggest to compromise that timing offset is < CP

  - Nokia: we would like to clarify with MTK on the conditions when the performance degradation is observed. What is the channel model?

    - MTK: it is AWGN

  - Chair: MTK will check. Come back on Fri.

- Tentative agreement

  - For CSI-RSRP and CSI-RSRQ the upper bound of timing offset for case 1 is CP

Issue 2-1-2: The CSI-RSRP measurement accuracy requirements for case 1?

- Proposals

  - Option 1: (CATT, Xiaomi, CMCC, MTK, Qualcomm, OPPO, vivo, Huawei, Intel, Apple)

    - Reuse the accuracy requirements of SS-RSRP measurement.

  - Option 3: (Nokia)

    - A better accuracy than SS-RSRP measurement if using 5 samples.

    - Reuse the accuracy of SS-RSRP measurement if using 3 samples.

- Discussion

  - Nokia: from NW perspective CSI-RSRP consumes more resources, so some benefits in terms of accuracy or measurement delay are desirable

  - CATT: the CSI-RS configuration can be more flexible comparing to SSB. Our original intention is to improve accuracy but based on companies results it seems that SS-RSRP accuracy is acceptable

  - Apple: Option 1

  - vivo: for 48MHz BW the amount of resources for CSI-RS and SSB is comparable. We see limited room for improvement of CSI-RSRP accuracy.

  - CMCC: we also have same views as Nokia that CSI-RSRP should provide some benefits. However, we agreed 48MHz BW and the performance gain is quite limited. So, we are ok with Option 1.

  - Nokia: most companies have shown the better performance in case of using 5 samples.

  - QC: on the motivation - on the cell-edge the SSB SNR can be low and CSI-RS can have better SNR due to beamforming gain.

  - MTK: accuracy includes BB accuracy and RF calibration error. RF error depends on the handled BW. Eventually CSI-RS may not necessarily give better accuracy.

  - Chair: Nokia will check. Come back on Fri.

- Tentative agreement

  - Reuse the accuracy requirements of SS-RSRP and SS-RSRQ measurement

Issue 2-1-3: Whether to define CSI-RS measurement accuracy requirements for case 2?

- Proposals

  - Option 1: (CATT, MTK, Qualcomm, CMCC, Apple, Huawei, OPPO, vivo, Xiaomi, Nokia, NTT DoCoMo)

    - No.

  - Option 2: (Xiaomi, CMCC, Huawei, OPPO)

    - Yes.

- Discussion

  - Apple: Option 1. For Option 2 it depends on a specific timing offset value.

  - Huawei/OPPO/Xiaomi: Can compromise to Option 1

- Agreement

  - Do not define CSI-RS measurement accuracy requirements for case 2

Issue 2-3-1: The upper limit of Es/Iot for CSI-SINR measurement with timing offset(T△)?

- Proposals

  - Option 1: (CATT, QC, OPPO, Huawei, Apple)

    - Es/Iot ≤ 10dB for the case that timing offset is within CP.

  - Option 2: (CMCC, vivo)

    - Es/Iot ≤ 25dB for the case that timing offset is within CP.

  - Option 3: (MTK, vivo)

    - Es/Iot ≤ 25dB for the case 0 ≤T△ ≤CP/2.

  - Option 4: (MTK)

    - Es/Iot ≤ X dB for the case |T△| ≤CP/2, where X is within the range of 0 to 10dB.

  - Option 5: (Huawei)

    - Two set of  Es/Iot based on timing difference:

      - Case 1: Es/Iot ≤ 25dB for the case timing error is <= CP/2;

      - Case 2: Es/Iot ≤ 12dB for the case timing error is <= 1.5*CP;

- Discussion

  - vivo: Option 3

  - MTK: need to discuss jointly with the accuracy. We need to relax the accuracy if we go with Option 1.

  - CMCC: We prefer compromise Es/Iot ≤ 18dB

  - QC: we can put the value into brackets.

  - vivo: the serving cell measurements is important and ok with 18dB

  - Huawei: is 18dB based on CP or CP/2?

    - CMCC: CP

  - Huawei: we can accept 10dB with CP. No confident to use 18dB as baseline.

  - MTK: suggest to collect some table

- Agreement

  - Option 1: Es/Iot ≤ [10] dB for the case that timing offset is within CP.

  - Option 2: Es/Iot ≤ [18] dB for the case that timing offset is within CP/2.

GTW session (February 01, 2021)

Issue 2-1-1: The upper bound of timing offset for case 1?

- Proposals

  - Option 1: (CATT, Xiaomi, CMCC, Nokia, OPPO, vivo, Apple, Huawei, Intel, QC)

    - CP.

  - Option 2: (MTK)

    - 0.9*CP.

  - Option 4: (Huawei)

    - CP/2.

- Discussion

  - MTK: need more time to converge on this issue. Recommend companies to check both positive and negative timing offsets.

  - Chair: are there plans to bring more simulation results?

  - CATT: open to have another simulation round

  - vivo: Is WI planned to be closed in the next meeting? What would be the completion level?

  - OPPO: we can endorse CRs and keep values in []

  - Chair: in case the WI cannot finish, the TUs for Q2 were reserved.

  - CATT: it is better to have a conclusion in this meeting. Are companies open to bring more results?

  - QC: We agree with MTK that it is better to bring more simulation results. In the previous meeting there was some ambiguity on the timing offset model.

  - Apple: even 1 CP can be unrealistic. We are open to check more results.

- Agreement

  - For CSI-RSRP and CSI-RSRQ the upper bound of timing offset for case 1 is TBD

    - Option 1: 1 CP

    - Option 2: 0.9 CP

  - Companies are encouraged to bring additional simulation results for different values of timing offset and evaluate both positive and negative timing offsets.

  - Note: the timing offset value in the test cases can be specified in []

Issue 2-1-2: The CSI-RSRP measurement accuracy requirements for case 1?

- Proposals

  - Option 1: (CATT, Xiaomi, CMCC, MTK, Qualcomm, OPPO, vivo, Huawei, Intel, Apple)

    - Reuse the accuracy requirements of SS-RSRP measurement.

  - Option 3: (Nokia)

    - A better accuracy than SS-RSRP measurement if using 5 samples.

    - Reuse the accuracy of SS-RSRP measurement if using 3 samples.

- Discussion

  - Nokia: this is relevant to 2-1-1. Need to have more simulations

  - CATT: We need to fix either 2-1-1 or 2-1-2. Otherwise the discussion will be endless

  - MTK: Same view as CATT. We need to agree on the accuracy at least.

  - vivo: Accuracy can be decided in this meeting. No much difference between 0.9 and 1 CP

  - Xiaomi: agree with CATT/MTK/vivo

  - Nokia: can compromise

- Agreement

  - Reuse the accuracy requirements of SS-RSRP and SS-RSRQ measurement

R4-2103461	Email discussion summary: [98e][222] NR_CSIRS_L3meas_RRM_2

Type: other		For: discussion
					Source: Moderator (OPPO)

Discussion:

The contribution summarised email discussion thread [98e][222] NR_CSIRS_L3meas_RRM_2. The discussion thread was moderated by Roy Hu (OPPO).

Decision: 		The document was revised to R4-2103704.

R4-2103704	Email discussion summary: [98e][222] NR_CSIRS_L3meas_RRM_2

Type: other		For: discussion
					Source: Moderator (OPPO)

(Replaces R4-2103461)

Discussion:

The contribution summarised email discussion thread [98e][222] NR_CSIRS_L3meas_RRM_2. The discussion thread was moderated by Roy Hu (OPPO).

Decision: 		The document was noted.

GTW session (January 27, 2021)

Sub-topic 1-1: Whether to introduce test case for FDD mode

- Proposals

  - Option 1: No (Qualcomm, OPPO, vivo, MTK, Xiaomi, CATT)

  - Option 2: Yes (Huawei, CATT, Nokia, Apple)

    - If option 2 was agreed, FFS time offset for FDD mode (e.g., 3us, 3ms or others)

- Discussion

  - Nokia: Option 2.

  - Huawei: the main concern shared by companies is that there will be large timing error. For the test we can control and set appropriate value.

  - vivo: we can set any arbitrary value in the test. Not sure what is the likelihood of having synchronous FDD networks

  - Huawei: the current NR design already requires some level of synchronous operation for FDD (e.g. SMTC configuration)

  - Apple: do not see much reasons to limit to FDD

  - OPPO: even if we define requirements, we cannot guarantee in the field that there will be synch FDD

  - MTK: there are no tight requirements for FDD

  - Chair: continue the discussion

Sub-topic 1-2: DRX configuration for Test Cases with DRX

- Proposals

  - FR2:

    - Option 1: DRX.5 = 320ms

    - Option 2: DRX.1 = 40ms

  - FR1:

    - Option 1: DRX.6 = 640ms

    - Option 2: DRX.5 = 320ms

- Recommended WF

  - Following the agreements of DRX configuration for test cases with DRX in last meeting:

    - For test cases with DRX: FR1 with long DRX, FR2 with short DRX

  - Recommended DRX configuration:

    - 320ms (DRX.5 in A.3.3) for FR1

    - 40ms (DRX.1 in A.3.3) for FR2

- Discussion

  - OPPO: 320ms (DRX.5 in A.3.3) for FR1 is the majority view

- Agreement

  - DRX configuration:

    - 320ms (DRX.5 in A.3.3) for FR1

    - 40ms (DRX.1 in A.3.3) for FR2

GTW session (February 05, 2021)

Subtopic 1-1: Whether to introduce test case for FDD mode

- Proposals

  - Option 1: No (Qualcomm, OPPO, vivo, MTK, Xiaomi, CATT)

  - Option 2: Yes (Huawei, Nokia, Apple, CMCC, CATT)

- Discussion

  - QC: ok to have FDD. In the test the time offset is within the CP.

  - vivo: ok to have FDD

- Agreements

  - Introduce test case for FDD mode

    - Use same timing offset model as for TDD

================================================================================

#### 7.14.1	RRM core requirements maintenance (38.133)  [NR_CSIRS_L3meas-Core]

R4-2103629	WF on core part maintenance of CSI-RS based L3 measurement requirements

Type: other		For: discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was approved.

R4-2100242	CR on CSSF with CSI-RS for L3 measurement

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1468  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was withdrawn.

R4-2100243	On remaining core issues of CSI-RS for L3 measurements

Type: discussion		For: Agreement
					Source: Apple

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2100421	Discussion on core part maintenance open issues

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2100422	CR on CSI-RS based L3 measurement

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1470  Cat: F (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was revised to R4-2103631.

R4-2103631	CR on CSI-RS based L3 measurement

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1470  rev 1 Cat: F (Rel-16)

					Source: CATT

(Replaces R4-2100422)

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was agreed.

R4-2100461	CR on CSI-RS based L3 measurement

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1482  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was agreed.

R4-2100716	Discussion on the remaining issues of core requirement for CSI-RS L3 measurement

Type: discussion		For: Discussion
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2100718	CR on core requirement for CSI-RS L3 measurement

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was postponed.

R4-2103632	CR on core requirement for CSI-RS L3 measurement

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Xiaomi

(Replaces R4-2100718)

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was withdrawn.

R4-2104058	CR on core requirement for CSI-RS L3 measurement

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1795  Cat: F (Rel-16)

					Source: Xiaomi

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was agreed.

R4-2104059	CR on core requirement for CSI-RS L3 measurement

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1796  Cat: A (Rel-17)

					Source: Xiaomi

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was agreed.

R4-2101150	Maintenance CR for CSI-RS based L3 measurement requirements R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1561  Cat: F (Rel-16)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was revised to R4-2103633.

R4-2103633	Maintenance CR for CSI-RS based L3 measurement requirements R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1561  rev 1 Cat: F (Rel-16)

					Source: MediaTek inc.

(Replaces R4-2101150)

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was agreed.

R4-2101151	Maintenance CR for CSI-RS based L3 measurement requirements R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1562  Cat: A (Rel-17)

					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was agreed.

R4-2101185	Comments on remaining issues of CSI-RS based L3 measurement core requirements

Type: discussion		For: (not specified)
					38.133 v..
					Source: Qualcomm CDMA Technologies

Abstract:

The remaining issues in the core requirements of CSI-RS based L3 measurements from 97e are discussed.

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2101393	Open issues on the CSI-RS based measurement requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2101394	38.133 CR on the CSI-RS based measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1596  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was revised to R4-2103634.

R4-2103634	38.133 CR on the CSI-RS based measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1596  rev 1 Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2101394)

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was revised to R4-2104093.

R4-2104093	38.133 CR on the CSI-RS based measurement requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1596  rev 2 Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2103634)

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was agreed.

R4-2102612	38.133 CR on the CSI-RS based measurement requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1737  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was agreed.

R4-2101767	Remaining issues on CSI-RS L3 measurement core requirements

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2101837	Discussion on remaining issues for CSI-RS based L3 measurement

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2101838	CR on CSI-RS based intra-frequency scheduling restriction

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1676  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was postponed.

R4-2101839	CR on CSI-RS based intra-frequency scheduling restriction

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1677  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was withdrawn.

R4-2101840	Correction on CSSFoutsidegap

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1678  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was agreed.

R4-2101841	Correction on CSSFoutsidegap

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1679  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was agreed.

R4-2101842	CR on CSI-RS measurement window and intra-frequency measurements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1680  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was revised to R4-2103635.

R4-2103635	CR on CSI-RS measurement window and intra-frequency measurements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1680  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101842)

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was postponed.

R4-2101843	CR on CSI-RS measurement window and intra-frequency measurements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1681  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was withdrawn.

#### 7.14.2	RRM perf. requirements (38.133)  [NR_CSIRS_L3meas-Perf]

R4-2103630	WF on accuracy requirements and test cases of CSI-RS based L3 measurement

Type: other		For: discussion
					Source: CATT, OPPO

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was revised to R4-2104089.

R4-2104089	WF on accuracy requirements and test cases of CSI-RS based L3 measurement

Type: other		For: discussion
					Source: CATT, OPPO

(Replaces R4-2103630)

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was approved.

R4-2101291	Big CR: Introduction of Rel-16 CSI-RS based L3 measurement RRM performance requirements

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1587  Cat: B (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2103653	Big CR: Introduction of Rel-16 CSI-RS based L3 measurement RRM performance requirements

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1794  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was withdrawn.

R4-2103725	Simulation assumptions for CSI-RS based L3 measurement

Type: other		For: discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was approved.

R4-2100717	Discussion on the remaining issues of performance requirement for CSI-RS L3 measurement

Type: discussion		For: Discussion
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was noted.

##### 7.14.2.1	General [NR_CSIRS_L3meas-Perf]

R4-2101203	On the performance requirements of CSI-RS based L3 measurement

Type: discussion		For: (not specified)
					38.133 v..
					Source: Qualcomm CDMA Technologies

Abstract:

Sim results on CSI-RSRP and CSI-SINR

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2101395	Discussion on the performance of CSI-RS based measurements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2101413	Discussion on CSI-RS L3 measurement

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

###### 7.14.2.1.1	CSI-RSRP requirements [NR_CSIRS_L3meas-Perf]

R4-2100423	Updated simulation results for CSI-RSRP measurement

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2100426	Discussion on performance requirement for CSI-RSRP

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2100429	CR on performance requirement for CSI-RSRP

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1471  Cat: B (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was postponed.

R4-2100462	CR on performance requirement for CSI-RSRP

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1483  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was withdrawn.

R4-2100719	CR on CSI-RSRP performance requirement for CSI-RS L3 measurement

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was postponed.

R4-2100861	Discussion on performance requirements for CSI-RS based L3 measurement

Type: discussion		For: Approval
					Source: CMCC

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2100862	Simulation results for CSI-RSRP

Type: discussion		For: Information
					Source: CMCC

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2101152	CSI-RSRP measurement accuracy requirement

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2101396	38.133 draftCR on the CSI-RSRP accuracy requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was revised to R4-2103637.

R4-2103637	38.133 draftCR on the CSI-RSRP accuracy requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2101396)

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was endorsed.

R4-2101397	Simulation results for CSI-RS based measurements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2101532	Discussion on remaining issues for CSI-RS measurement accuracy requirements

Type: discussion		For: Discussion
					Source: OPPO

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2101768	Simulation results for CSI-RSRP measurement accuracy

Type: other		For: Information
					Source: vivo

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2101769	Discussion on CSI-RSRP measurement accuracy requirements

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2102799	Discussion on CSI-RSRP accuracy requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

###### 7.14.2.1.2	CSI-RSRQ requirements [NR_CSIRS_L3meas-Perf]

R4-2100424	Updated simulation results for CSI-RSRQ measurement

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2100427	Discussion on performance requirement for CSI-RSRQ

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2100430	CR on performance requirement for CSI-RSRQ

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1472  Cat: B (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was revised to R4-2103636.

R4-2103636	CR on performance requirement for CSI-RSRQ

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1472  rev 1 Cat: B (Rel-16)

					Source: CATT

(Replaces R4-2100430)

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was endorsed.

R4-2100463	CR on performance requirement for CSI-RSRQ

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1484  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was withdrawn.

R4-2100720	CR on CSI-RSRQ performance requirement for CSI-RS L3 measurement

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was postponed.

R4-2100863	Simulation results for CSI-RSRQ

Type: discussion		For: Information
					Source: CMCC

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2101770	Simulation results for CSI-RSRQ measurement accuracy

Type: other		For: Information
					Source: vivo

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2101771	Discussion on CSI-RSRQ measurement accuracy requirements

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

###### 7.14.2.1.3	CSI-SINR requirements [NR_CSIRS_L3meas-Perf]

R4-2100425	Simulation results for CSI-SINR measurement

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2100428	Discussion on performance requirement for CSI-SINR

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2100431	CR on performance requirement for CSI-SINR

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1473  Cat: B (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was postponed.

R4-2100464	CR on performance requirement for CSI-SINR

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1485  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was withdrawn.

R4-2100721	CR on CSI-SINR performance requirement for CSI-RS L3 measurement

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was postponed.

R4-2100864	Simulation results for CSI-SINR

Type: discussion		For: Information
					Source: CMCC

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2100865	Discussion on side condition for CSI-SINR measurement

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2101153	CSI-SINR measurement accuracy requirement

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2101772	Simulation results for CSI-SINR measurement accuracy

Type: other		For: Information
					Source: vivo

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2101773	Discussion on CSI-SINR measurement accuracy requirements

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2102800	Discussion on CSI-SINR accuracy requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was noted.

R4-2102801	draftCR on CSI-SINR accuracy requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was revised to R4-2103638.

R4-2103638	draftCR on CSI-SINR accuracy requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2102801)

Discussion:

See email discussion summary for [98e][221] NR_CSIRS_L3meas_RRM_1 in R4-2103703.

Decision: 		The document was endorsed.

##### 7.14.2.2	Test cases [NR_CSIRS_L3meas-Perf]

R4-2101292	Big CR: Introduction of Rel-16 CSI-RS based L3 measurement RRM test cases

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1588  Cat: B (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was withdrawn.

R4-2103652	Big CR: Introduction of Rel-16 CSI-RS based L3 measurement RRM test cases

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1800  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was withdrawn.

###### 7.14.2.2.1	General [NR_CSIRS_L3meas-Perf]

R4-2100432	Discussion on test case for CSI-RS based measurement

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was noted.

R4-2101533	CR for CSI-RS L3 RRM tests

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1611  Cat: F (Rel-16)

					Source: OPPO, CATT

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2101774	Remaining issues on CSI-RS L3 measurement test cases

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was noted.

R4-2102802	Discussion on remaining issues in CSI-RS RRM test cases

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was noted.

###### 7.14.2.2.2	Intra-frequency measurement [NR_CSIRS_L3meas-Perf]

R4-2100433	CR on test case for CSI-RS based intra-frequency measurement for FR1 in SA

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1474  Cat: B (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was revised to R4-2103639.

R4-2103639	CR on test case for CSI-RS based intra-frequency measurement for FR1 in SA

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1474  rev 1 Cat: B (Rel-16)

					Source: CATT

(Replaces R4-2100433)

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was endorsed.

R4-2100465	CR on test case for CSI-RS based intra-frequency measurement for FR1 in SA

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1486  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was withdrawn.

R4-2100722	RRM test cases for CSI-RS L3 intra-frequency measurements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was revised to R4-2103640.

R4-2103640	RRM test cases for CSI-RS L3 intra-frequency measurements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Xiaomi

(Replaces R4-2100722)

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was endorsed.

R4-2102820	Draft test case of CSI-RS based intra-frequency test for EN-DC event triggered reporting tests without gap for NR neighbor cell in FR2

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Qualcomm CDMA Technologies

Abstract:

Update the cell time offset for the intra-frequency CSI-RS L3 test to be within a CP.

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was revised to R4-2103641.

R4-2103641	Draft test case of CSI-RS based intra-frequency test for EN-DC event triggered reporting tests without gap for NR neighbor cell in FR2

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Qualcomm CDMA Technologies

(Replaces R4-2102820)

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was endorsed.

###### 7.14.2.2.3	Inter-frequency measurement [NR_CSIRS_L3meas-Perf]

R4-2100434	CR on test case for CSI-RS based inter-frequency measurement for FR1 in SA

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1475  Cat: B (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was revised to R4-2103642.

R4-2103642	CR on test case for CSI-RS based inter-frequency measurement for FR1 in SA

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1475  rev 1 Cat: B (Rel-16)

					Source: CATT

(Replaces R4-2100434)

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was endorsed.

R4-2100466	CR on test case for CSI-RS based inter-frequency measurement for FR1 in SA

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1487  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was withdrawn.

R4-2100723	RRM test cases for CSI-RS L3 inter-frequency measurements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was revised to R4-2103643.

R4-2103643	RRM test cases for CSI-RS L3 inter-frequency measurements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Xiaomi

(Replaces R4-2100723)

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was endorsed.

R4-2101534	CR on EN-DC tests for NR inter-frequency neighbor cell in FR2(PScell in FR2)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1612  Cat: F (Rel-16)

					Source: OPPO

Discussion:

Session chair: Cover sheet errors (What is the revision number? It reads revision number 1 on the cover page but the Tdoc is not reserved for a revision.). See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was revised to R4-2103644.

R4-2103644	CR on EN-DC tests for NR inter-frequency neighbor cell in FR2(PScell in FR2)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1612  rev 1 Cat: F (Rel-16)

					Source: OPPO

(Replaces R4-2101534)

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was endorsed.

###### 7.14.2.2.4	Measurement performance [NR_CSIRS_L3meas-Perf]

R4-2100435	CR on test case for CSI-RSRP measurement accuracy requirements in SA

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1476  Cat: B (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was revised to R4-2103645.

R4-2103645	CR on test case for CSI-RSRP measurement accuracy requirements in SA

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1476  rev 1 Cat: B (Rel-16)

					Source: CATT

(Replaces R4-2100435)

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was endorsed.

R4-2100467	CR on test case for CSI-RSRP measurement accuracy requirements in SA

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1488  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was withdrawn.

R4-2100724	RRM test cases for CSI-RS L3 measurement performance requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was revised to R4-2103646.

R4-2103646	RRM test cases for CSI-RS L3 measurement performance requirements

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Xiaomi

(Replaces R4-2100724)

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was endorsed.

R4-2101154	CR to Update timing offset in test case for CSI-SINR in SA FR2

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was postponed.

R4-2103647	CR to Update timing offset in test case for CSI-SINR in SA FR2

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: MediaTek inc.

(Replaces R4-2101154)

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was withdrawn.

R4-2101535	CR on EN-DC tests for CSI-RSRQ accuracy

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1613  Cat: F (Rel-16)

					Source: OPPO

Discussion:

Session chair: Cover sheet errors (What is the revision number? It reads revision number 1 on the cover page but the Tdoc is not reserved for a revision.) See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was revised to R4-2103648.

R4-2103648	CR on EN-DC tests for CSI-RSRQ accuracy

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1613  rev 1 Cat: F (Rel-16)

					Source: OPPO

(Replaces R4-2101535)

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was endorsed.

R4-2101775	CR to 38.133 correction to test cases for EN-DC CSI-SINR measurement accuracy

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: vivo

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was revised to R4-2103649.

R4-2103649	CR to 38.133 correction to test cases for EN-DC CSI-SINR measurement accuracy

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: vivo

(Replaces R4-2101775)

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was endorsed.

R4-2102803	draft CR to update TC3 and TC12 for CSI-RS accuracy test

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was revised to R4-2103650.

R4-2103650	draft CR to update TC3 and TC12 for CSI-RS accuracy test

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Huawei, HiSilicon

(Replaces R4-2102803)

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was endorsed.

R4-2102825	Draft test case of measurement performance for EN-DC CSI-RSRP measurement accuracy for NR neighbor cell in FR2

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Qualcomm CDMA Technologies

Abstract:

Update the cell time offset for the test cases of measurement performance in the scenario of EN-DC FR2

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was revised to R4-2103651.

R4-2103651	Draft test case of measurement performance for EN-DC CSI-RSRP measurement accuracy for NR neighbor cell in FR2

Type: draftCR		For: Endorsement
					38.133 v16.6.0
					Source: Qualcomm CDMA Technologies

(Replaces R4-2102825)

Discussion:

See email discussion summary for [98e][222] NR_CSIRS_L3meas_RRM_2 in R4-2103704.

Decision: 		The document was endorsed.

### 7.15	NR support for high speed train scenario [NR_HST]

#### 7.15.1	RRM requirements maintenance (38.133) [NR_HST-Core/Perf]

R4-2103462	Email discussion summary: [98e][223] NR_HST_RRM

Type: other		For: discussion
					Source: Moderator (CMCC)

Discussion:

The contribution summarised email discussion thread [98e][223] NR_HST_RRM. The discussion thread was moderated by Jingjing Chen (China Mobile Com. Corporation).

Decision: 		The document was revised to R4-2103705.

R4-2103705	Email discussion summary: [98e][223] NR_HST_RRM

Type: other		For: discussion
					Source: Moderator (CMCC)

(Replaces R4-2103462)

Discussion:

The contribution summarised email discussion thread [98e][223] NR_HST_RRM. The discussion thread was moderated by Jingjing Chen (China Mobile Com. Corporation).

Decision: 		The document was noted.

R4-2100237	R16 HST maintenance: measurement requirements for active SCell in HST

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][223] NR_HST_RRM in R4-2103705.

Decision: 		The document was noted.

R4-2100238	CR on HST core part maintenance

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1466  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][223] NR_HST_RRM in R4-2103705.

Decision: 		The document was revised to R4-2103655.

R4-2103655	CR on HST core part maintenance

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1466  rev 1 Cat: F (Rel-16)

					Source: Apple

(Replaces R4-2100238)

Discussion:

See email discussion summary for [98e][223] NR_HST_RRM in R4-2103705.

Decision: 		The document was agreed.

R4-2100239	CR on HST core part maintenance (R17)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1467  Cat: A (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][223] NR_HST_RRM in R4-2103705.

Decision: 		The document was withdrawn.

R4-2101013	CR on HST core part maintenance (R17)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1544  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][223] NR_HST_RRM in R4-2103705.

Decision: 		The document was agreed.

R4-2104094	CR on introduction of missing HST test cases

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1801  Cat: F (Rel-17)

					Source: CMCC

Discussion:

See email discussion summary for [98e][223] NR_HST_RRM in R4-2103705. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2100485	Correction to cell reselection test case for HST

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1500  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][223] NR_HST_RRM in R4-2103705.

Decision: 		The document was withdrawn.

R4-2100849	CR on HST RRM requirements in connected mode

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1526  Cat: F (Rel-16)

					Source: CMCC

Discussion:

See email discussion summary for [98e][223] NR_HST_RRM in R4-2103705.

Decision: 		The document was revised to R4-2103654.

R4-2103654	CR on HST RRM requirements in connected mode

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1526  rev 1 Cat: F (Rel-16)

					Source: CMCC

(Replaces R4-2100849)

Discussion:

See email discussion summary for [98e][223] NR_HST_RRM in R4-2103705.

Decision: 		The document was agreed.

R4-2100850	CR on HST RRM requirements in connected mode

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1527  Cat: A (Rel-17)

					Source: CMCC

Discussion:

See email discussion summary for [98e][223] NR_HST_RRM in R4-2103705.

Decision: 		The document was agreed.

R4-2101844	Correction on inter-RAT measurement in high speed scenario

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1682  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][223] NR_HST_RRM in R4-2103705.

Decision: 		The document was agreed.

R4-2101845	Correction on inter-RAT measurement in high speed scenario

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1683  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][223] NR_HST_RRM in R4-2103705.

Decision: 		The document was agreed.

R4-2101846	Correction on test cases for inter-RAT cell identification in connected mode for HST

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1684  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

Session chair: Cover sheet errors (What is the work item? It reads NR_RRM_Enh-Perf on the cover page but the Tdoc is reserved for NR_HST-Perf.) See email discussion summary for [98e][223] NR_HST_RRM in R4-2103705.

Decision: 		The document was revised to R4-2103656.

R4-2103656	Correction on test cases for inter-RAT cell identification in connected mode for HST

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1684  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101846)

Discussion:

Session chair: resubmit in the next meeting Session chair: Rel-17 spec issue. See email discussion summary for [98e][223] NR_HST_RRM in R4-2103705.

Decision: 		The document was endorsed.

R4-2101847	Correction on test cases for inter-RAT cell identification in connected mode for HST

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1685  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][223] NR_HST_RRM in R4-2103705.

Decision: 		The document was withdrawn.

R4-2100484	Correction to cell reselection test case for HST

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1499  Cat: F (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][223] NR_HST_RRM in R4-2103705.

Decision: 		The document was agreed.

#### 7.15.2	Demodulation and CSI requirements Maintenance (38.101-4 / 38.104) [NR_HST-Perf]

##### 7.15.2.1	UE demodulation and CSI requirements [NR_HST-Perf]

R4-2103754	Email discussion summary for [98e][315] Demod_R16_Maintenance

Type: other		For: discussion
					Source: Moderator (ZTE)

Discussion:

The contribution summarised email discussion thread [98e][315] Demod_R16_Maintenance. The discussion thread was moderated by Aijun Cao (ZTE).

Decision: 		The document was revised to R4-2103941.

R4-2103941	Email discussion summary for [98e][315] Demod_R16_Maintenance

Type: other		For: discussion
					Source: Moderator (ZTE)

(Replaces R4-2103754)

Discussion:

The contribution summarised email discussion thread [98e][315] Demod_R16_Maintenance. The discussion thread was moderated by Aijun Cao (ZTE).

Decision: 		The document was noted.

R4-2100168	CR on FDD HST Single-Tap and Multipath Fading Requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0124  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103978.

R4-2103978	CR on FDD HST Single-Tap and Multipath Fading Requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0124  rev 1 Cat: F (Rel-16)

					Source: Qualcomm Incorporated

(Replaces R4-2100168)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2100848	CR on HST-SFN requirements for TDD

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0132  Cat: F (Rel-16)

					Source: CMCC

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was not pursued.

R4-2103858	CR on HST-SFN requirements for TDD

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0132  rev 1 Cat: F (Rel-16)

					Source: CMCC

(Replaces R4-2100848)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was withdrawn.

R4-2100854	CR on release independent for Rel.16 NR HST UE demodulation requirements

Type: CR		For: Agreement
					38.307 v15.7.0	  CR-0046  Cat: F (Rel-15)

					Source: CMCC

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2100855	CR on release independent for Rel.16 NR HST UE demodulation requirements

Type: CR		For: Agreement
					38.307 v16.5.0	  CR-0047  Cat: F (Rel-16)

					Source: CMCC

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103859.

R4-2103859	CR on release independent for Rel.16 NR HST UE demodulation requirements

Type: CR		For: Agreement
					38.307 v16.5.0	  CR-0047  rev 1 Cat: F (Rel-16)

					Source: CMCC

(Replaces R4-2100855)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2100856	CR on release independent for Rel.16 NR HST UE demodulation requirements

Type: CR		For: Agreement
					38.307 v17.0.0	  CR-0048  Cat: A (Rel-17)

					Source: CMCC

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101300	CR on update TRS and CSI-RS transmission for HST DPS requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0146  Cat: F (Rel-16)

					Source: Huawei, HiSilicon, Ericsson, Intel Corporation

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101301	Summary of ideal and impairment results for NR HST demodulation requirements

Type: other		For: Information
					Source: Huawei, HiSilicon

Abstract:

Include updated simulation results

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was noted.

R4-2101438	Simulation results for HST-DPS

Type: other		For: Information
					Source: Ericsson

Abstract:

This contribution updates our PDSCH simulation results for HST-DPS scenario.

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was noted.

##### 7.15.2.2	BS demodulation requirements [NR_HST-Perf]

R4-2100380	Summary of ideal and impairment results for NR HST demodulation requirements

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103972.

R4-2103972	Summary of ideal and impairment results for NR HST demodulation requirements

Type: discussion		For: Discussion
					Source: CATT

(Replaces R4-2100380)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was noted.

R4-2100381	CR for TS 38.141-2: Introduction of NR PUSCH UL TA performance requirement

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0261  Cat: F (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was not pursued.

R4-2103860	CR for TS 38.141-2: Introduction of NR PUSCH UL TA performance requirement

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0261  rev 1 Cat: F (Rel-16)

					Source: CATT

(Replaces R4-2100381)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was withdrawn.

R4-2100382	CR for TS 38.141-2: Introduction of NR PUSCH UL TA performance requirement(Rel-17)

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0262  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was withdrawn.

R4-2100558	CR for 38.104: HST PUSCH demodulation requirements

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0263  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Update of square bracketed SNR values according to the simulation summary of last meeting [R4-2017557].

Removal of all remaining square brackets.

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103861.

R4-2103861	CR for 38.104: HST PUSCH demodulation requirements

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0263  rev 1 Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100558)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2100559	CR for 38.104: HST PUSCH demodulation requirements

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0264  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2100922	Updated simulation results for NR HST PUSCH

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was noted.

R4-2100925	CR on UL timing adjustment conducted performance requirement for TS 38.141-1

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0178  Cat: F (Rel-16)

					Source: Samsung

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103862.

R4-2103862	CR on UL timing adjustment conducted performance requirement for TS 38.141-1

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0178  rev 1 Cat: F (Rel-16)

					Source: Samsung

(Replaces R4-2100925)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2100926	CR on UL timing adjustment conducted performance requirement for TS 38.141-1

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0179  Cat: A (Rel-17)

					Source: Samsung

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2100993	remove SNR brackets for HST PUSCH in TS38.141-2

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0273  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

remove SNR brackets for HST PUSCH

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103863.

R4-2103863	remove SNR brackets for HST PUSCH in TS38.141-2

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0273  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2100993)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2100994	remove SNR brackets for HST PUSCH in TS38.141-2

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0274  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

remove SNR brackets for HST PUSCH

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103953.

R4-2103953	remove SNR brackets for HST PUSCH in TS38.141-2

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0274  rev 1 Cat: A (Rel-17)

					Source: Ericsson

(Replaces R4-2100994)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101042	CR for TS 38.141-1 Updates of NR PUSCH performance requirements for HST

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0180  Cat: F (Rel-16)

					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103864.

R4-2103864	CR for TS 38.141-1 Updates of NR PUSCH performance requirements for HST

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0180  rev 1 Cat: F (Rel-16)

					Source: NTT DOCOMO, INC.

(Replaces R4-2101042)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101043	CR for TS 38.141-1 Updates of NR PUSCH performance requirements for HST

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0181  Cat: A (Rel-17)

					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101319	CR for 38.104 Cleanup of performance requirements for NR HST PRACH under fading channel (Rel-16)

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0276  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103865.

R4-2103865	CR for 38.104 Cleanup of performance requirements for NR HST PRACH under fading channel (Rel-16)

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0276  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101319)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101320	CR for 38.141-1 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-16)

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0187  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103866.

R4-2103866	CR for 38.141-1 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-16)

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0187  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101320)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101321	CR for 38.141-2 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-16)

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0284  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103867.

R4-2103867	CR for 38.141-2 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-16)

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0284  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101321)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101324	CR for 38.104 Cleanup of performance requirements for NR HST PRACH under fading channel (Rel-17)

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0277  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101325	CR for 38.141-1 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-17)

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0188  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101326	CR for 38.141-2 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-17)

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0285  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101849	CR to TS 38.104 Update on UL timing adjustment performance requirements

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0280  Cat: F (Rel-16)

					Source: ZTE Wistron Telecom AB

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103868.

R4-2103868	CR to TS 38.104 Update on UL timing adjustment performance requirements

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0280  rev 1 Cat: F (Rel-16)

					Source: ZTE Wistron Telecom AB

(Replaces R4-2101849)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

### 7.16	NR performance requirement enhancement [NR_perf_enh-Perf]

#### 7.16.1	UE demodulation and CSI requirements (38.101-4) [NR_perf_enh-Perf]

R4-2103964	CR to TS 38.104 Update on UL timing adjustment performance requirements

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0300  Cat: A (Rel-17)

					Source: Moderator (China Telecomm)

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was revised.

R4-2103846	Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was noted.

R4-2103847	Summary of simulation results of Rel-15 Type-II UE PMI test

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was noted.

----------------------------------GTW agenda for Jan.28th email thread [326] (30 minutes) -----------------

Issue 2-1: Section numbering for PDSCH CA demodulation requirements

Candidate options:

- Option 1: Update the section numbering for PDSCH CA normal requirements, with “void” for the originally used sections (Intel, QC, E///)

- Option 2: Update the section numbering for PDSCH CA normal requirements, by changing the originally used sections

- Option 3: Keep TS structure as it is. (Intel, HW)

Recommendations for 2nd round:

- Option 2 is precluded since it is not allowed by TS drafting rule.

- Decide whether to go with Option 1 or Option 3 in the GTW.

Samsung (spec editor of 38.101-4): From spec structure consistent aspect, we prefer to maintain the section structure alignment as much as possible, unless we see some specific technical reason, we can consider some exception cases carefully.

QC: When we start with inter-working scenarios, these structure maybe not feasible. There are UEs equipped with 2Rx, and 4Rx on different CCs, from single test aspect, cross different sections needed.

We usually focused on feature by features from RAN4, for CA test cases, readability aspect for CA, placed 2Rx, 4Rx in low level sections.

For single CC test cases, only 2Rx or 4Rx need to be verified.

Huawei: Share similar view as Samsung, in NR Rel-15, we spent lots of time to define the structure with feature basis vs Rx antennas as 3nd level sections; in the end we come up existing structures considering features themselves not stable and involved release by release which make specification not stable and uncontrollable.

E///: For CA, power imbalance test cases, with mixed sections. We also think “void sections“ not good from specification aspect.

FFS for 2nd round discussion, if no consensus reached in this meeting and then no further discussion for renumbering issue.

Issue 2-2-1: Antenna connection for CA tests with 4 RX

Candidate options:

- Option 1, i.e., reuse the same antenna connection for CA tests with 4RX in LTE (CMCC, CTC,  Intel, ZTE)

  - If any of the Pcell and/or the Scells is a 4 RX supported RF band, all 4 RX should be connected with data source from system simulator.

  - If any of the Pcell and/or the Scells is a 2 RX supported RF band, 2 out of the 4 RX should be connected with data source from system simulator, and the other the corresponding 2 RX are connected based on UE declaration with zero input.

- Option 2: Reuse the test applicability rule defined for single carrier for CA tests with 2Rx and 4Rx, i.e. (HW)

  - Only conducted CA tests with 2Rx for UE only supports 2Rx

  - Only conducted CA tests with 4Rx for UE only supports 4Rx

  - Only conducted CA tests with 4Rx for UE supports both 2Rx and 4Rx

  - Not conducted CA test with 4Rx for UE only supports 2Rx

Recommendations for 2nd round:

- The main difference between the two options is on how to test UE supporting 2Rx in some bands and supporting 4Rx in the other bands (see the 3rd bullet in option 2).

- Further check can we go with Option 1 by following majority companies’ view? Any additional clarification questions to Option 1?

QC: For op1, why do we need to test UE with 4Rx for 2Rx test set-up?

Huawei: For op1 with 2nd bullet, it’s misunderstanding. If UE support 2Rx and 4Rx in different CCs, 2Rx CC pass 2Rx and 4Rx CC pass 4Rx requirements.

Intel: how to apply for UE under 2Rx CC? our understanding as captured in option1.

China Telecomm: CA test cases different with single CC cases. Both 2Rx CC and 4Rx CC need to be verified with corresponding requirements.

Huawei: Need to improve the wording for option 1.

Question: How to apply test set-up for UE which support 2Rx and 4Rx in different CCs?

Agreements:

- Reuse the same antenna connection for CA tests with 4RX in LTE (CMCC, CTC,  Intel, ZTE)

  - If any of the Pcell and/or the Scells is a 4 RX supported RF band, all 4 RX should be connected with data source from system simulator.

  - If any of the Pcell and/or the Scells is a 2 RX supported RF band, the corresponding 2 RX are connected based on UE declaration

Further work on the details texts into specification for above agreements.

Topic #3: Rel-15 Type II reporting requirements

Issue 3-1: Test metric for type II codebook

Tentative agreement:

- Following PMI (Type II)/Random PMI (Type I codebook) (gamma values) (Samsung, QC, Apple, HW, CTC, E/// - fine if tighter gamma compared to Type I codebook)

  - E///:  Generally fine with Option 1, if the final gamma of Rel-15 Type-II (TP ratio of following Rel-15 Type-II and random Type-I) becomes tighter than the TP ratio of following Type-I and random Type-I)

Issue 3-2: SNR point for type II PMI codebook

Tentative agreement:

- 90% max TP (Apple, QC, CTC, Samsung, E/// - fine if tighter gamma compared to Type I codebook)

  - E///: Propose to review the simulation summary after the 1st round. If we observe the gamma with Rel-15 Type-II is tighter than gamma with Type-I, we are also fine to set SNR to 90%.

Agreements: 90% max TP

Issue 3-3: MIMO correlation for type II codebook

Tentative agreement:

- XP Medium (Samsung, QC, Apple, CTC, E///, HW)

Agreements: XP Medium

Topic #4: UE power imbalance requirements

Issue 4-1: Test applicability rule for UE power imbalance for EN-DC

Recommendations for 2nd round:

Check if it is agreeable to use the table and notes suggested by E///, with the modified Note 2 from DCM:

|  | Inter-band scenarios are not upported | UE indicates “interBandContiguousMRDC” (Note 1, Note 2) | UE does not indicate “interBandContiguousMRDC” (Note 1, Note 3) |
| --- | --- | --- | --- |
| Intra-band scenarios are not supported | N/A | Clause 9.5B.1.1 is executed for inter-band EN-DC scenarios | Clause 9.5B.1.2 is executed for inter-band EN-DC scenarios |
| UE does not indicate “intraBandENDC-Support” or UE indicates “both” in “intraBandENDC-Support” (Note 4) | Clause 9.5B.1.1 is only executed for intra-band EN-DC scenarios | Clause 9.5B.1.1 is executed for both intra-band and inter-band EN-DC scenarios | Clause 9.5B.1.1 is only executed for intra-band EN-DC scenarios |
| UE indicates “non-contiguous” in “intraBandENDC-Support” (Note 5) | Clause 9.5B.1.2 is only executed for intra-band EN-DC scenarios | Clause 9.5B.1.1 is executed for inter-band EN-DC scenarios | Clause 9.5B.1.2 is executed for both intra-band and inter-band EN-DC scenarios |
| Note 1: Requirements are applicable to intra-band scenarios and only inter-band scenarios from Table 5.5B.4.1-1 of TS 38.101-3 [8] for which Note 4 is applied. Note 2: UE supports both intra-band contiguous and non-contiguous EN-DC requirements for supported inter-band EN-DC combinations Note 3: UE supports intra-band non-contiguous EN-DC requirements for supported inter-band EN-DC combinations Note 4: UE supports intra-band contiguous EN-DC, or both intra-band contiguous and non-contiguous EN-DC for supported intra-band EN-DC combinations Note 5: UE supports only intra-band non-contiguous EN-DC for supported intra-band EN-DC combinations |  |  |  |

------------------------------------End-----------------------

R4-2103765	Email discussion summary for [98e][326] NR_perf_enh_Demod

Type: other		For: discussion
					Source: Moderator (China Telecomm)

Discussion:

The contribution summarised email discussion thread [98e][326] NR_perf_enh_Demod. The discussion thread was moderated by Shan Yang (China Telecom Corporation Ltd.).

Decision: 		The document was revised to R4-2103942.

R4-2103942	Email discussion summary for [98e][326] NR_perf_enh_Demod

Type: other		For: discussion
					Source: Moderator (China Telecomm)

(Replaces R4-2103765)

Discussion:

The contribution summarised email discussion thread [98e][326] NR_perf_enh_Demod. The discussion thread was moderated by Shan Yang (China Telecom Corporation Ltd.).

Decision: 		The document was noted.

##### 7.16.1.1	NR CA PDSCH requirements [NR_perf_enh-Perf]

R4-2100786	Remaining issues on PDSCH CA test applicability

Type: discussion		For: Discussion
					Source: China Telecom

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was noted.

R4-2100816	CR for NR PDSCH FR1 CA 2Rx performance requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0130  Cat: F (Rel-16)

					Source: CMCC

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was agreed.

R4-2100822	Test applicability rule for NR CA PDSCH normal demodulation

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was noted.

R4-2101254	Discussion on NR CA UE demodulation requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was noted.

R4-2101255	CR on applicability rules for Normal NR CA requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0142  Cat: B (Rel-16)

					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was revised to R4-2103841.

R4-2103841	CR on applicability rules for Normal NR CA requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0142  rev 1 Cat: B (Rel-16)

					Source: Intel Corporation

(Replaces R4-2101255)

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was agreed.

R4-2101365	Discussion on NR normal CA performance requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was noted.

R4-2101434	Draft CR: Section numbering for PDSCH CA demodulation requirements

Type: draftCR		For: Endorsement
					38.101-4 v16.3.0
					Source: Ericsson

Abstract:

This draft CR proposes to change the section numbering to align with RAN5 spec.

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was not pursued.

R4-2102818	Specification Structure for CA Test Cases

Type: discussion		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was noted.

##### 7.16.1.2	PMI reporting requirements with larger number of Tx ports [NR_perf_enh-Perf]

R4-2100216	PMI reporting requirements with larger number of TX ports

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was noted.

R4-2100897	Introduction of PMI test cases with Rel-15 Type II codebook

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0136  Cat: B (Rel-16)

					Source: Samsung

Abstract:

Introduction of Type II codebook PMI test cases

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was revised to R4-2102939.

R4-2102939	Introduction of PMI test cases with Rel-15 Type II codebook

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0136  rev 1 Cat: B (Rel-16)

					Source: Samsung

(Replaces R4-2100897)

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was revised to R4-2103843.

R4-2103843	Introduction of PMI test cases with Rel-15 Type II codebook

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0136  rev 2 Cat: B (Rel-16)

					Source: Samsung

(Replaces R4-2102939)

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was agreed.

R4-2100902	Discussion and simulation results for Rel-15 Type II codebook test cases

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was revised to R4-2103844.

R4-2103844	Discussion and simulation results for Rel-15 Type II codebook test cases

Type: discussion		For: Discussion
					Source: Samsung

(Replaces R4-2100902)

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was noted.

R4-2101317	Simulation results for Rel-15 Type II codebook PMI reporting test

Type: other		For: Information
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was noted.

R4-2101318	Discussion on the test point for Rel-15 Type II codebook PMI reporting test with larger Tx ports

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was noted.

R4-2101322	CR for 38.101-4 Applicablity of PMI reporting test with Tx ports larger than 8 and up to 32

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0149  Cat: B (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was revised to R4-2103842.

R4-2103842	CR for 38.101-4 Applicablity of PMI reporting test with Tx ports larger than 8 and up to 32

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0149  rev 1 Cat: B (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101322)

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was agreed.

R4-2101435	Simulation results for Rel-15 Type II PMI reporting test

Type: other		For: Information
					Source: Ericsson

Abstract:

This contribution shows our simulation results of Rel-15 Type-II PMI reporting.

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was noted.

R4-2101436	Evaluation of Rel-15 Type-II PMI reporting test

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the Rel-15 Type-II PMI reporting requirements.

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was noted.

R4-2101437	Correction of title on 16Tx port subband PMI reporting

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0156  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

This CR corrects the wrong sub-clause titles for 16Tx port subband PMI reporting.

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was agreed.

R4-2102821	Views on Type II PMI Reporting Tests

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was noted.

##### 7.16.1.3	FR1 CA and EN-DC power imbalance requirements [NR_perf_enh-Perf]

R4-2101366	Discussion on test applicability rule for UE power imbalance for EN-DC

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was noted.

R4-2101367	CR: Updates to power imbalance for CA

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0153  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was agreed.

R4-2103845	CR: Update on test applicability rule for EN-DC power imbalance

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0172  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Session Chair Note: Note 2 in Table 9.1.1-3 will be updated in the future based on further discussion on the interpretation of “interBandContiguousMRDC” in RF session.”

Decision: 		The document was agreed.

##### 7.16.1.4	NR CA CQI reporting requirements [NR_perf_enh-Perf]

R4-2100886	CR: Adding applicability and requirements for FR1 and FR2 CA CQI reporting test

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0134  Cat: B (Rel-16)

					Source: China Telecom

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was agreed.

##### 7.16.1.5	Release independent [NR_perf_enh-Perf]

R4-2100787	Draft CR for TS 38.307 on UE demodulation performance requirements (Rel-15)

Type: CR		For: Agreement
					38.307 v15.7.0	  CR-0043  Cat: B (Rel-15)

					Source: China Telecom

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was revised to R4-2103839.

R4-2103839	Draft CR for TS 38.307 on UE demodulation performance requirements (Rel-15)

Type: CR		For: Agreement
					38.307 v15.7.0	  CR-0043  rev 1 Cat: B (Rel-15)

					Source: China Telecom

(Replaces R4-2100787)

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was agreed.

R4-2100788	Draft CR for TS 38.307 on UE demodulation performance requirements (Rel-16)

Type: CR		For: Agreement
					38.307 v16.5.0	  CR-0044  Cat: B (Rel-16)

					Source: China Telecom

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was revised to R4-2103840.

R4-2103840	Draft CR for TS 38.307 on UE demodulation performance requirements (Rel-16)

Type: CR		For: Agreement
					38.307 v16.5.0	  CR-0044  rev 1 Cat: B (Rel-16)

					Source: China Telecom

(Replaces R4-2100788)

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was agreed.

R4-2100789	Draft CR for TS 38.307 on UE demodulation performance requirements (Rel-17)

Type: CR		For: Agreement
					38.307 v17.0.0	  CR-0045  Cat: A (Rel-17)

					Source: China Telecom

Discussion:

See email discussion summary for [98e][326] NR_perf_enh_Demod in R4-2103942.

Decision: 		The document was agreed.

#### 7.16.2	BS demodulation requirements (38.104) [NR_perf_enh-Perf]

### 7.17	Over the air (OTA) base station (BS) testing TR Maintenance [OTA_BS_testing-Perf]

R4-2103747	Email discussion summary for [98e][308] OTA_BS_Testing_Maintenance

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][308] OTA_BS_Testing_Maintenance. The discussion thread was moderated by Szydelko Michal (Huawei).

Decision: 		The document was revised to R4-2103943.

R4-2103943	Email discussion summary for [98e][308] OTA_BS_Testing_Maintenance

Type: other		For: discussion
					Source: Moderator (Huawei)

(Replaces R4-2103747)

Discussion:

The contribution summarised email discussion thread [98e][308] OTA_BS_Testing_Maintenance. The discussion thread was moderated by Szydelko Michal (Huawei).

Decision: 		The document was noted.

R4-2102492	Relative calibration approach using reference receiver

Type: discussion		For: Discussion
					37.941 v..
					Source: ROHDE & SCHWARZ

Abstract:

This contribution presents the overall description of a relative calibration approach, eventually applicable to all systems, and serves as technical justification for the corresponding CR to implement this option in TR 37.941.

Discussion:

See email discussion summary for [98e][308] OTA_BS_Testing_Maintenance in R4-2103943.

Decision: 		The document was noted.

R4-2102493	draft CR to TR 37.941: Relative calibration approach

Type: draftCR		For: Endorsement
					37.941 v15.2.0
					Source: ROHDE & SCHWARZ

Discussion:

See email discussion summary for [98e][308] OTA_BS_Testing_Maintenance in R4-2103943.

Decision: 		The document was revised to R4-2103848.

R4-2103848	draft CR to TR 37.941: Relative calibration approach

Type: draftCR		For: Endorsement
					37.941 v15.2.0
					Source: ROHDE & SCHWARZ

(Replaces R4-2102493)

Discussion:

See email discussion summary for [98e][308] OTA_BS_Testing_Maintenance in R4-2103943.

Decision: 		The document was postponed.

R4-2102901	CR to TR 37.941: Updating the orthogonal cut procedure

Type: CR		For: Agreement
					37.941 v15.2.0	  CR-0025  Cat: F (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Numerical expressions for TRP computation are included.

Discussion:

See email discussion summary for [98e][308] OTA_BS_Testing_Maintenance in R4-2103943.

Decision: 		The document was postponed.

R4-2102902	CR to TR 37.941: Updating the orthogonal cut procedure

Type: CR		For: Agreement
					37.941 v16.2.0	  CR-0026  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Numerical expressions for TRP computation are included.

Discussion:

See email discussion summary for [98e][308] OTA_BS_Testing_Maintenance in R4-2103943.

Decision: 		The document was withdrawn.

### 7.18	2-step RACH for NR [NR_2step_RACH-Perf]

#### 7.18.1	RRM requirements maintenance (38.133)  [NR_2step_RACH-Core/Perf]

R4-2103463	Email discussion summary: [98e][224] NR_2step_RACH_RRM

Type: other		For: discussion
					Source: Moderator (ZTE)

Discussion:

The contribution summarised email discussion thread [98e][224] NR_2step_RACH_RRM. The discussion thread was moderated by Richie Leo (ZTE).

Decision: 		The document was noted.

R4-2100115	[CR] Applicability rule for 2-step RA

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1434  Cat: F (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][224] NR_2step_RACH_RRM in R4-2103463.

Decision: 		The document was merged.

R4-2100116	[CR] Applicability rule for 2-step RA (Cat A)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1435  Cat: A (Rel-17)

					Source: ZTE Corporation

Abstract:

This is a cat A CR for Release 17.

Discussion:

See email discussion summary for [98e][224] NR_2step_RACH_RRM in R4-2103463.

Decision: 		The document was withdrawn.

R4-2100580	2-step RACH RRM performance requirements  corrections

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1501  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][224] NR_2step_RACH_RRM in R4-2103463.

Decision: 		The document was agreed.

R4-2100581	2-step RACH RRM performance requirements  corrections

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1502  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][224] NR_2step_RACH_RRM in R4-2103463.

Decision: 		The document was agreed.

R4-2100835	Update the applicability rule for 2-step RA

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][224] NR_2step_RACH_RRM in R4-2103463.

Decision: 		The document was noted.

R4-2102277	2-step RACH RRM performance requirements  corrections

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1704  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][224] NR_2step_RACH_RRM in R4-2103463.

Decision: 		The document was withdrawn.

R4-2102278	2-step RACH RRM performance requirements  corrections

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1705  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][224] NR_2step_RACH_RRM in R4-2103463.

Decision: 		The document was withdrawn.

#### 7.18.2	BS Demodulation requirements maintenance (38.104) [NR_2step_RACH-Perf]

R4-2103873	Updated simulation results collection on BS demodulation requirements for 2-step RACH

Type: other		For: discussion
					Source: ZTE

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was noted.

R4-2100582	Corrections on 2-step RACH demodulation requirements

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0176  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell, Ericsson

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103869.

R4-2103869	Corrections on 2-step RACH demodulation requirements

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0176  rev 1 Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell, Ericsson

(Replaces R4-2100582)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2100583	Corrections on 2-step RACH demodulation requirements

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0177  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell, Ericsson

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2100584	2-step RACH simulation results

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was noted.

R4-2100924	Simulation results for BS 2-step RACH requirement

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was noted.

R4-2100931	CR on MsgA PUSCH radiated performance requirement for TS 38.141-2

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0271  Cat: F (Rel-16)

					Source: Samsung

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103870.

R4-2103870	CR on MsgA PUSCH radiated performance requirement for TS 38.141-2

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0271  rev 1 Cat: F (Rel-16)

					Source: Samsung

(Replaces R4-2100931)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2100932	CR on MsgA PUSCH radiated performance requirement for TS 38.141-2

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0272  Cat: A (Rel-17)

					Source: Samsung

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101302	CR on correction 2-step RACH performance requirements for FR2 in 38.104 (Rel-16)

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0274  Cat: F (Rel-16)

					Source: Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103959.

R4-2103959	CR on correction 2-step RACH performance requirements for FR2 in 38.104 (Rel-16)

Type: CR		For: Agreement
					38.104 v16.6.0	  CR-0274  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

(Replaces R4-2101302)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101303	CR on correction 2-step RACH performance requirements for FR2 in 38.104 (Rel-17)

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0275  Cat: A (Rel-17)

					Source: Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101304	CR on update applicability rule for 2-step RACH in 38.141-1 (Rel-16)

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0185  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103871.

R4-2103871	CR on update applicability rule for 2-step RACH in 38.141-1 (Rel-16)

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0185  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101304)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101305	CR on update applicability rule for 2-step RACH in 38.141-1 (Rel-17)

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0186  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101306	CR on update applicability rule for 2-step RACH in 38.141-2 (Rel-16)

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0282  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was revised to R4-2103872.

R4-2103872	CR on update applicability rule for 2-step RACH in 38.141-2 (Rel-16)

Type: CR		For: Agreement
					38.141-2 v16.6.0	  CR-0282  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2101306)

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2101307	CR on update applicability rule for 2-step RACH in 38.141-2 (Rel-17)

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0283  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was agreed.

R4-2102279	Corrections on 2-step RACH demodulation requirements

Type: CR		For: Agreement
					38.141-1 v16.6.0	  CR-0203  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell, Ericsson

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was withdrawn.

R4-2102280	Corrections on 2-step RACH demodulation requirements

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0204  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell, Ericsson

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was withdrawn.

R4-2102281	2-step RACH simulation results

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][315] Demod_R16_Maintenance in R4-2103941.

Decision: 		The document was withdrawn.

#### 7.18.3	Others [NR_2step_RACH-Perf]

### 7.19	R16 NR maintenance [WI code or TEI16]

#### 7.19.1	UE transient period capability [TEI16]

R4-2102959	Email discussion summary for [98e][111] NR_transient_period

Type: other		For: discussion
					Source: Moderator (CMCC)

Discussion:

The contribution summarised email discussion thread [98e][111] NR_transient_period . The discussion thread was moderated by Zhe Shao (China Mobile Group Device Co.).

Decision: 		The document was revised to R4-2103301.

R4-2103301	Email discussion summary for [98e][111] NR_transient_period

Type: other		For: discussion
					Source: Moderator (CMCC)

(Replaces R4-2102959)

Discussion:

The contribution summarised email discussion thread [98e][111] NR_transient_period . The discussion thread was moderated by Zhe Shao (China Mobile Group Device Co.).

Decision: 		The document was revised to R4-2103362.

R4-2103362	Email discussion summary for [98e][111] NR_transient_period

Type: other		For: discussion
					Source: Moderator (CMCC)

(Replaces R4-2103301)

Discussion:

The contribution summarised email discussion thread [98e][111] NR_transient_period . The discussion thread was moderated by Zhe Shao (China Mobile Group Device Co.).

Decision: 		The document was noted.

R4-2103151	Way forward on transient period

Type: other		For: discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][111] NR_transient_period in R4-2103362.

Decision: 		The document was revised to R4-2103361.

R4-2103361	Way forward on transient period

Type: other		For: discussion
					Source: CMCC

(Replaces R4-2103151)

Discussion:

See email discussion summary for [98e][111] NR_transient_period in R4-2103362.

Decision: 		The document was approved.

R4-2101460	Short Transient Period Testing

Type: other		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][111] NR_transient_period in R4-2103362.

Decision: 		The document was noted.

R4-2101484	CR on introduction of shorter Transient Period Capability

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0641  Cat: B (Rel-16)

					Source: Qualcomm Incorporated

Abstract:

Resubmission of endorsed Draft CR R4-2011766

Discussion:

See email discussion summary for [98e][111] NR_transient_period in R4-2103362.

Decision: 		The document was revised to R4-2103152.

R4-2103152	CR on introduction of shorter Transient Period Capability

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0641  rev 1 Cat: B (Rel-16)

					Source: Qualcomm Incorporated

(Replaces R4-2101484)

Discussion:

See email discussion summary for [98e][111] NR_transient_period in R4-2103362.

Decision: 		The document was agreed.

R4-2101487	CR on introduction of shorter Transient Period Capability

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0642  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][111] NR_transient_period in R4-2103362.

Decision: 		The document was agreed.

R4-2102629	on transient period UE capability

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][111] NR_transient_period in R4-2103362.

Decision: 		The document was noted.

R4-2102684	CR on TS 38.101-1 time mask for shorter transient

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0709  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][111] NR_transient_period in R4-2103362.

Decision: 		The document was not pursued.

R4-2103153	CR on TS 38.101-1 time mask for shorter transient

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0709  rev 1 Cat: F (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2102684)

Discussion:

See email discussion summary for [98e][111] NR_transient_period in R4-2103362.

Decision: 		The document was withdrawn.

#### 7.19.2	Transmit diversity and power class related to UL MIMO [TEI16]

R4-2102960	Email discussion summary for [98e][112] NR_TxD

Type: other		For: discussion
					Source: Moderator (vivo)

Discussion:

The contribution summarised email discussion thread [98e][112] NR_TxD. The discussion thread was moderated by Sanjun Feng (vivo Mobile Communication).

Decision: 		The document was revised to R4-2103302.

R4-2103302	Email discussion summary for [98e][112] NR_TxD

Type: other		For: discussion
					Source: Moderator (vivo)

(Replaces R4-2102960)

Discussion:

The contribution summarised email discussion thread [98e][112] NR_TxD. The discussion thread was moderated by Sanjun Feng (vivo Mobile Communication).

Decision: 		The document was noted.

R4-2103154	Way forward on NR TxD

Type: other		For: discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was revised to R4-2103390.

R4-2103390	Way forward on NR TxD

Type: other		For: discussion
					Source: vivo

(Replaces R4-2103154)

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was approved.

R4-2103155	LS on Signaling scheme of Transparent TxD

Type: LS out		For: Approval
					to RAN2, cc RAN1, RAN5
					Source: vivo

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was revised to R4-2103360.

R4-2103360	LS on Signaling scheme of Transparent TxD

Type: LS out		For: Approval
					to RAN2, cc RAN1, RAN5
					Source: RAN4

(Replaces R4-2103155)

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was approved.

##### 7.19.2.1	R16 support of transmit diversity [TEI16]

R4-2100095	Remaining items on transparent Tx diversity

Type: discussion		For: Approval
					Source: Anritsu corporation

Abstract:

In this contribution we show our views on the following remaining issues.

(1) UE Behavior under Conformance Testing

(2) Signaling for Transparent TxD

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was noted.

R4-2100523	On Tx diversity

Type: discussion		For: Decision
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was noted.

R4-2100592	TxD open items for Rel-16

Type: other		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was noted.

R4-2100594	Introduction of Tx diversity in tor 38101-1 Cat-B CR 38.101-1

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0621  Cat: B (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was withdrawn.

R4-2100595	Introduction of Tx diversity in tor 38101-1 Cat-F CR 38.101-1

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0622  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was withdrawn.

R4-2100914	On the Support of Transparent Tx Diversity in Rel-16

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was noted.

R4-2101108	Discussion on Tx diversity open issues

Type: other		For: Approval
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was noted.

R4-2101289	Remaining Issues on Transparent TxD

Type: discussion		For: Approval
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was noted.

R4-2101721	Support of transparent TxD and the associated EVM requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we propose capabilities for TX diversity and EVM measurements. CDD is also discussed.

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was noted.

R4-2101749	Discussion on UE power class high limit

Type: discussion		For: Approval
					Source: OPPO

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was noted.

R4-2101751	Discussion on Rel-16 TxD

Type: discussion		For: Approval
					Source: OPPO

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was noted.

R4-2101850	EVM for transparent TxD

Type: discussion		For: Discussion
					Source: ZTE Wistron Telecom AB

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was noted.

R4-2102089	Discussion on FR1 EVM measurements

Type: discussion		For: Approval
					Source: Rohde & Schwarz

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was noted.

R4-2102383	On TxD RF requirements

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was noted.

R4-2102384	CR for TS 38.101-1: TxD requirements

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0686  Cat: B (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was revised to R4-2103156.

R4-2103156	CR for TS 38.101-1: TxD requirements

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0686  rev 1 Cat: B (Rel-16)

					Source: Huawei, HiSilicon

(Replaces R4-2102384)

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was endorsed.

R4-2102683	CR for TS 38.101-1 Tx diversity requirements

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0708  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was withdrawn.

R4-2102704	Remaining issues in Transparent Tx Diversity

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was noted.

R4-2102917	On the EVM Definition for Transmit Diversity and Antenna Ports

Type: discussion		For: Approval
					Source: Lenovo, Motorola Mobility

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was noted.

##### 7.19.2.2	Power class related to UL MIMO and other related req. (MPR, SEM, etc) [TEI16 or NR_newRAT-Core]

R4-2100593	Power class due to TxD in Rel-15

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was withdrawn.

R4-2102385	Discussion on Reply LS to RAN5 On EN-DC power class

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was noted.

R4-2102705	Remaining issues in Power class

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][112] NR_TxD in R4-2103302.

Decision: 		The document was noted.

#### 7.19.3	Other UE RF [WI code or TEI16]

R4-2102961	Email discussion summary for [98e][113] NR_R16_Maintenance

Type: other		For: discussion
					Source: Moderator (OPPO)

Discussion:

The contribution summarised email discussion thread [98e][113] NR_R16_Maintenance. The discussion thread was moderated by Jinqiang Xing (OnePlus).

Decision: 		The document was revised to R4-2103303.

R4-2103303	Email discussion summary for [98e][113] NR_R16_Maintenance

Type: other		For: discussion
					Source: Moderator (OPPO)

(Replaces R4-2102961)

Discussion:

The contribution summarised email discussion thread [98e][113] NR_R16_Maintenance. The discussion thread was moderated by Jinqiang Xing (OnePlus).

Decision: 		The document was noted.

R4-2103157	Way forward on UE co-existence between n40 and n41

Type: other		For: discussion
					Source: Huawei

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was approved.

R4-2103158	Way forward on non-default RX-TX Frequency Separation

Type: other		For: discussion
					Source: Qualcomm

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was noted.

R4-2103166	Way forward on requirements for Type 2 UEs supporting inter-band MRDC with overlapping DL

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was approved.

R4-2100112	PC1 and PC3 Updates for Band n14

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0588  Cat: F (Rel-16)

					Source: AT&T

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was revised to R4-2103400.

R4-2103400	PC1 and PC3 Updates for Band n14

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0588  rev 1 Cat: F (Rel-16)

					Source: AT&T

(Replaces R4-2100112)

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303. The document was subject for post-meeting email agreement.

Decision: 		The document was revised to R4-2103400.

R4-2103409	PC1 and PC3 Updates for Band n14

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0588  rev 2 Cat: F (Rel-16)

					Source: AT&T

(Replaces R4-2100112)

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2100119	PC1 and PC3 Updates for Band n14

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0589  Cat: A (Rel-17)

					Source: AT&T

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2100127	CR to TS 38.101-2 on correction to intra-band non-contiguous CA configurations (Rel-16)

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0318  Cat: F (Rel-16)

					Source: ZTE Corporation

Abstract:

This CR is for correction to intra-band non-contiguous CA configurations

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2100128	CR to TS 38.101-2 on correction to intra-band non-contiguous CA configurations (Rel-17)

Type: CR		For: Agreement
					38.101-2 v17.0.0	  CR-0319  Cat: A (Rel-17)

					Source: ZTE Corporation

Abstract:

This CR is for correction to intra-band non-contiguous CA configurations

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2100129	CR to TS 38.101-3 on correction to hanging paragraph in the spec (Rel-16)

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0435  Cat: F (Rel-16)

					Source: ZTE Corporation

Abstract:

This CR is to correct the issues of hanging paragraphs in the spec

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was revised to R4-2103161.

R4-2103161	CR to TS 38.101-3 on correction to hanging paragraph in the spec (Rel-16)

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0435  rev 1 Cat: F (Rel-16)

					Source: ZTE Corporation

(Replaces R4-2100129)

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2100130	CR to TS 38.101-3 on correction to hanging paragraph in the spec (Rel-17)

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0436  Cat: A (Rel-17)

					Source: ZTE Corporation

Abstract:

This CR is to correct the issues of hanging paragraphs in the spec

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2100136	38.101 Void clean up R16

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0593  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was revised to R4-2103343.

R4-2103343	38.101 Void clean up R16

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0593  rev 1 Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100136)

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2100137	38.101 Void clean up R17

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0594  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was revised to R4-2103344.

R4-2103344	38.101 Void clean up R17

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0594  rev 1 Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100137)

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2100148	TS 38.101-3: Addition of missing lower order fallbacks R16

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0437  Cat: B (Rel-16)

					Source: Nokia, AT&T

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was revised to R4-2103162.

R4-2103162	TS 38.101-3: Addition of missing lower order fallbacks R16

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0437  rev 1 Cat: B (Rel-16)

					Source: Nokia, AT&T

(Replaces R4-2100148)

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2103395	TS 38.101-3: Addition of missing lower order fallbacks R16

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0501  Cat: A (Rel-17)

					Source: Nokia, AT&T

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2100149	TS 38.101-3: Addition of missing lower order fallbacks R17

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0598  Cat: A (Rel-17)

					Source: Nokia, AT&T

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was withdrawn.

R4-2100150	TR 37.716-21-11: Addition of missing lower order fallbacks

Type: CR		For: Agreement
					37.716-21-11 v16.0.0	  CR-0001  Cat: B (Rel-16)

					Source: Nokia, AT&T

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2100163	CR_CatF_n47_AMPR_correction

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0601  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was not pursued.

R4-2102557	CR for n47 AMPR - Cat A

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0694  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was withdrawn.

R4-2100797	Discussion on requirement for LTE/NR spectrum sharing and dual connectivity (DSS EN-DC) in band 1/n1

Type: discussion		For: Discussion
					Source: China Telecom

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was noted.

R4-2100846	CR for 38.101-1: Update of missing fallback NR-DC combinations Rel-16

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0632  Cat: F (Rel-16)

					Source: SoftBank Corp., ZTE Corporation

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2100847	CR for 38.101-1: Update of missing fallback NR-DC combinations Rel-17

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0633  Cat: A (Rel-17)

					Source: SoftBank Corp., ZTE Corporation

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2100876	CR for 38.101-1: Update of simultaneous Rx/Tx capability for some NR CA band combinations Rel-16

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0634  Cat: F (Rel-16)

					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was not pursued.

R4-2100877	CR for 38.101-1: Update of simultaneous Rx/Tx capability for some NR CA band combinations Rel-17

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0635  Cat: A (Rel-17)

					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was withdrawn.

R4-2100878	CR for 38.101-3: Update of simultaneous Rx/Tx capability for some EN-DC band combinations Rel-16

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0450  Cat: F (Rel-16)

					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was not pursued.

R4-2100879	CR for 38.101-3: Update of simultaneous Rx/Tx capability for some EN-DC band combinations Rel-17

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0451  Cat: A (Rel-17)

					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was withdrawn.

R4-2101106	CR for 38.101-1 Rel16 corrections on exception requirements on out-of-band blocking for inter-band CA

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0637  Cat: F (Rel-16)

					Source: Xiaomi

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was revised to R4-2103347.

R4-2103347	CR for 38.101-1 Rel16 corrections on exception requirements on out-of-band blocking for inter-band CA

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0637  rev 1 Cat: F (Rel-16)

					Source: Xiaomi

(Replaces R4-2101106)

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2101107	CR for 38.101-1 Rel17 corrections on exception requirements on out-of-band blocking for inter-band CA

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0638  Cat: A (Rel-17)

					Source: Xiaomi

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was withdrawn.

R4-2101175	n40-n41 Coexistence

Type: other		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was noted.

R4-2101176	CR to TS 38.101-3 clarification on the single uplink allowance for DC_3A_n3A

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0458  Cat: F (Rel-16)

					Source: CHTTL

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was revised to R4-2103163.

R4-2103163	CR to TS 38.101-3 clarification on the single uplink allowance for DC_3A_n3A

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0458  rev 1 Cat: F (Rel-16)

					Source: CHTTL

(Replaces R4-2101176)

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2101179	CR to TS 38.101-3 clarification on the single uplink allowance for DC_3A_n3A

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0459  Cat: A (Rel-17)

					Source: CHTTL

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2101722	LS to RAN5 on SCell dropping behavior and verification thereof

Type: LS out		For: Approval
					to RAN5
					Source: Ericsson

Abstract:

LS to RAN5 on the verification of CA MOP and make proposals on the Scell dropping behaviour

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was noted.

R4-2101723	Modification of Pcmax for UL CA with uplink Tx switching capability

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0648  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

CR to modify the Pcmax for CA to accommodate power boosting for switched TX

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was revised to R4-2103159.

R4-2103159	Modification of Pcmax for UL CA with uplink Tx switching capability

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0648  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2101723)

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was not pursued.

R4-2101724	Correction to modified MPR behaviour

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0328  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

CR to correct modified MPR behaviour

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was revised to R4-2103167.

R4-2103167	Correction to modified MPR behaviour

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0328  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2101724)

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was not pursued.

R4-2101725	Requirements Type 2 UEs supporting inter-band MRDC with overlapping DL

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0463  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

CR to add requirements for Type 2 UE and add applicability for Type 1

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was revised to R4-2103164.

R4-2103164	Requirements Type 2 UEs supporting inter-band MRDC with overlapping DL

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0463  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2101725)

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2101804	CR for 38.101-3 to add the missing Tib Rib for DC_2-7-7-66_n78/ DC_2-7-66-66_n78/ DC_2-7-7-66-66_n78 (Rel-16)

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0467  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2101805	CR for 38.101-3 to add the missing Tib Rib for DC_2-7-7-66_n78/ DC_2-7-66-66_n78/ DC_2-7-7-66-66_n78 (Rel-17)

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0468  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2101806	Discussion on spurious emission about UE co-existence between band n40 and n41

Type: other		For: Approval
					Source: Huawei, HiSilicon, CMCC

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was noted.

R4-2101807	CR on spurious emission about UE co-existence between band n40 and n41(Rel-16)

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0652  Cat: F (Rel-16)

					Source: Huawei, HiSilicon, CMCC

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was not pursued.

R4-2101808	CR on spurious emission about UE co-existence between band n40 and n41(Rel-17)

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0653  Cat: A (Rel-17)

					Source: Huawei, HiSilicon, CMCC

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was withdrawn.

R4-2101809	CR for 38.101-1 to introduce PC2 for n40 UL MIMO(Rel-16)

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0654  Cat: F (Rel-16)

					Source: Huawei, HiSilicon, Reliance Jio

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was not pursued.

R4-2101810	CR for 38.101-1 to introduce PC2 for n40 UL MIMO(Rel-17)

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0655  Cat: A (Rel-17)

					Source: Huawei, HiSilicon, Reliance Jio

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was withdrawn.

R4-2101852	CR to TS 38.101-1 Operating bands for DC

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0656  Cat: F (Rel-16)

					Source: ZTE Wistron Telecom AB

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was not pursued.

R4-2101939	CR for 38.101-1 to add missing spurious emissions for band n38 UE co-existence (Rel-16)

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0659  Cat: F (Rel-16)

					Source: Huawei, HiSilicon, DT

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2101940	CR for 38.101-1 to add missing spurious emissions for band n38 UE co-existence (Rel-17)

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0660  Cat: A (Rel-17)

					Source: Huawei, HiSilicon, DT

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102146	CR for 38.101-3: Correction for CA_n66A-n260

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0477  Cat: F (Rel-16)

					Source: T-Mobile USA

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102147	CR for 38.101-3: Correction for CA_n66A-n260A

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0478  Cat: A (Rel-17)

					Source: T-Mobile USA

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102152	CR for 38.101-1: Add CA_n25A-n41(2A)-n71A which was missing in the CR implementation

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0668  Cat: F (Rel-16)

					Source: T-Mobile USA

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102153	CR for 38.101-1: Add CA_n25A-n41(2A)-n71A which was missing in the CR implementation

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0669  Cat: A (Rel-17)

					Source: T-Mobile USA

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102203	CR to TS38.101-1: Correction on configured transmitted power requirement

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0678  Cat: F (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102204	CR to TS38.101-1: Correction on configured transmitted power requirement

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0679  Cat: A (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102205	CR to TS38.101-3: Correction on duty cycle signalling terminology for PC2 inter-band ENDC

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0479  Cat: F (Rel-16)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102206	CR to TS38.101-3: Correction on duty cycle signalling terminology for PC2 inter-band ENDC

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0480  Cat: A (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102395	CR for TS 38.101-3 correction of intra-band contiguous EN-DC for DC_(n)66_R16

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0483  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was not pursued.

R4-2102396	CR for TS 38.101-3 correction of intra-band contiguous EN-DC for DC_(n)66_R17

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0484  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was withdrawn.

R4-2102402	CR for TS 38.101-3: Adding delta TIB and RIB requirement for DC_2-7-7-66_n78 (R16)

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0485  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102403	draft CR for TS 38.101-3: Adding delta TIB and RIB requirement for DC_2-7-7-66_n78 (R17)

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0486  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102562	CR to 38.101-2: correction on UL MIMO

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0333  Cat: F (Rel-16)

					Source: Google Inc.

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102582	CR to 38.101-2: correction on UL MIMO

Type: CR		For: Agreement
					38.101-2 v17.0.0	  CR-0334  Cat: A (Rel-17)

					Source: Google Inc.

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102602	CR for TS 38.101-1: Cleanup for spurious emissions for UE co-existence table

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0702  Cat: F (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was revised to R4-2103160.

R4-2103160	CR for TS 38.101-1: Cleanup for spurious emissions for UE co-existence table

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0702  rev 1 Cat: F (Rel-16)

					Source: Apple

(Replaces R4-2102602)

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102603	CR for TS 38.101-1: Cleanup for spurious emissions for UE co-existence table

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0703  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102685	CR on TS 38.101-1 NS_49

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0710  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102816	CR on TS 38.101-1 NS_49

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0713  Cat: A (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was agreed.

R4-2102903	CR on split band duplexer exceptions to non-default TX-RX separation

Type: CR		For: Agreement
					38.101-1 v15.12.0	  CR-0714  Cat: F (Rel-15)

					Source: Qualcomm Incorporated

Abstract:

Proposes that only default TX-RX separation with the deviation of ?FTX-RX = | (BWDL – BWUL)/2 | for asymmetric BW case is permitted for n28 and n74

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was not pursued.

R4-2102904	Non-default RX-TX Frequency Separation Values and split band duplexers

Type: other		For: Approval
					38.101-1 v..
					Source: Qualcomm Incorporated

Abstract:

Proposes to add a note to table 5.4.4-1: For bands n28 and n74 UE that may support only the default TX-RX frequency separation value with the deviation of ?FTX-RX = | (BWDL – BWUL)/2 | for asymmetric BW case.

Discussion:

See email discussion summary for [98e][113] NR_R16_Maintenance in R4-2103303.

Decision: 		The document was noted.

R4-2102412	Notational amendment and correction to PCMAX for EN-DC

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0487  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][106] NR_unlic_Maintenance in R4-2103296.

Decision: 		The document was revised to R4-2103165.

R4-2103165	Notational amendment and correction to PCMAX for EN-DC

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0487  rev 1 Cat: F (Rel-16)

					Source: Qualcomm Incorporated

(Replaces R4-2102412)

Discussion:

See email discussion summary for [98e][106] NR_unlic_Maintenance in R4-2103296.

Decision: 		The document was agreed.

R4-2102413	Notational amendment and correction to PCMAX for EN-DC

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0488  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][106] NR_unlic_Maintenance in R4-2103296.

Decision: 		The document was agreed.

R4-2102826	CR for correction of Rel-16 Dual Connectivity of 1LTE band (1DL/1UL) and 1NR band (1DL/1UL) with FR1

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0495  Cat: F (Rel-16)

					Source: Verizon Denmark

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was postponed.

#### 7.19.4	BS RF [WI code or TEI16]

R4-2101180	CR to TR 38.820: Correction of antenna model in subclause 7.2.4

Type: CR		For: Agreement
					38.820 v16.0.0	  CR-0002  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

The correction in this CR align the parameter defintion with gives equations

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was revised to R4-2103786.

R4-2103786	CR to TR 38.820: Correction of antenna model in subclause 7.2.4

Type: CR		For: Agreement
					38.820 v16.0.0	  CR-0002  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2101180)

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was agreed.

R4-2102563	CR to TS 37.105: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16

Type: CR		For: Agreement
					37.105 v16.6.0	  CR-0222  Cat: F (Rel-16)

					Source: Huawei, China Unicom

Abstract:

Referring to the Rel-16 WI on MSR_GSM_UTRA_LTE_NR, the MSR BS specification was extended with additional CS configuration (e.g. UTRA+EUTRA+NR).

WID in RP-190642 captured that only MSR BS specifications are to be affected, i.e. TS 37.104, TS 37.141. Relat

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was revised to R4-2103782.

R4-2103782	CR to TS 37.105: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16

Type: CR		For: Agreement
					37.105 v16.6.0	  CR-0222  rev 1 Cat: F (Rel-16)

					Source: Huawei, China Unicom

(Replaces R4-2102563)

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102564	CR to TS 37.105: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-17

Type: CR		For: Agreement
					37.105 v17.0.0	  CR-0223  Cat: A (Rel-17)

					Source: Huawei, China Unicom

Abstract:

Referring to the Rel-16 WI on MSR_GSM_UTRA_LTE_NR, the MSR BS specification was extended with additional CS configuration (e.g. UTRA+EUTRA+NR).

WID in RP-190642 captured that only MSR BS specifications are to be affected, i.e. TS 37.104, TS 37.141. Relat

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102565	CR to TS 37.145-1: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16

Type: CR		For: Agreement
					37.145-1 v16.5.0	  CR-0246  Cat: F (Rel-16)

					Source: Huawei, China Unicom

Abstract:

Referring to the Rel-16 WI on MSR_GSM_UTRA_LTE_NR, the MSR BS specification was extended with additional CS configuration (e.g. UTRA+EUTRA+NR).

WID in RP-190642 captured that only MSR BS specifications are to be affected, i.e. TS 37.104, TS 37.141. Realt

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was revised to R4-2103783.

R4-2103783	CR to TS 37.145-1: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16

Type: CR		For: Agreement
					37.145-1 v16.5.0	  CR-0246  rev 1 Cat: F (Rel-16)

					Source: Huawei, China Unicom

(Replaces R4-2102565)

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102566	CR to TS 37.145-1: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-17

Type: CR		For: Agreement
					37.145-1 v17.0.0	  CR-0247  Cat: A (Rel-17)

					Source: Huawei, China Unicom

Abstract:

Referring to the Rel-16 WI on MSR_GSM_UTRA_LTE_NR, the MSR BS specification was extended with additional CS configuration (e.g. UTRA+EUTRA+NR).

WID in RP-190642 captured that only MSR BS specifications are to be affected, i.e. TS 37.104, TS 37.141. Realt

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102567	CR to TS 37.145-2: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16

Type: CR		For: Agreement
					37.145-2 v16.6.0	  CR-0286  Cat: F (Rel-16)

					Source: Huawei, China Unicom

Abstract:

Referring to the Rel-16 WI on MSR_GSM_UTRA_LTE_NR, the MSR BS specification was extended with additional CS configuration (e.g. UTRA+EUTRA+NR).

WID in RP-190642 captured that only MSR BS specifications are to be affected, i.e. TS 37.104, TS 37.141. Realt

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923.

Decision: 		The document was revised to R4-2103784.

R4-2103784	CR to TS 37.145-2: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16

Type: CR		For: Agreement
					37.145-2 v16.6.0	  CR-0286  rev 1 Cat: F (Rel-16)

					Source: Huawei, China Unicom

(Replaces R4-2102567)

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102568	CR to TS 37.145-2: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-17

Type: CR		For: Agreement
					37.145-2 v17.0.0	  CR-0287  Cat: A (Rel-17)

					Source: Huawei, China Unicom

Abstract:

Referring to the Rel-16 WI on MSR_GSM_UTRA_LTE_NR, the MSR BS specification was extended with additional CS configuration (e.g. UTRA+EUTRA+NR).

WID in RP-190642 captured that only MSR BS specifications are to be affected, i.e. TS 37.104, TS 37.141. Realt

Discussion:

See email discussion summary for [98e][301] BSRF_Maintenance in R4-2103923. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 7.19.5	RRM [WI code or TEI16]

R4-2103443	Email discussion summary: [98e][204] R16_NR_RRM_maintenance

Type: other		For: discussion
					Source: Moderator (Apple)

Discussion:

The contribution summarised email discussion thread [98e][204] R16_NR_RRM_maintenance. The discussion thread was moderated by Yang Tang (Apple).

Decision: 		The document was revised to R4-2103686.

R4-2103686	Email discussion summary: [98e][204] R16_NR_RRM_maintenance

Type: other		For: discussion
					Source: Moderator (Apple)

(Replaces R4-2103443)

Discussion:

The contribution summarised email discussion thread [98e][204] R16_NR_RRM_maintenance. The discussion thread was moderated by Yang Tang (Apple).

Decision: 		The document was noted.

1st round email discussion conclusions

Decisions

Issue 1-1: the legacy serving cell measurement and evaluation requirement in IDLE/INACTIVE mode shall not be changed by introducing SMTC2-LP.

Agreement: the legacy serving cell measurement and evaluation requirement in IDLE/INACTIVE mode shall not be changed by introducing SMTC2-LP

Issue 1-4: revise the maximum interruption requirement in paging reception

Agreement: Revise the maximum interruption requirement in paging reception in TS38.133 section 4.2.2.6 as:

At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc.

R4-2103505	WF on MRTD/MTTD for intra-band non-contiguous CA and EN-DC

Type: other		For: discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was withdrawn.

R4-2100117	[CR] Core maintenance for 38.133

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1436  Cat: F (Rel-16)

					Source: ZTE Corporation

Abstract:

This CR tends to fix some errors existing in the current specification TS 38.133.

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was agreed.

R4-2100118	[CR] Core maintenance for 38.133 (Cat A)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1437  Cat: A (Rel-17)

					Source: ZTE Corporation

Abstract:

This is a cat A CR for Release 17.

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was agreed.

R4-2100185	On R16 IDLE and INACTIVE RRM requirement with SMTC2-LP

Type: discussion		For: Discussion
					38.133 v..
					Source: Apple

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was noted.

R4-2100186	CR on IDLE/INACTIVE RRM requirement with SMTC2-LP R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1449  Cat: B (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was revised to R4-2103506.

R4-2103506	CR on IDLE/INACTIVE RRM requirement with SMTC2-LP R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1449  rev 1 Cat: B (Rel-16)

					Source: Apple

(Replaces R4-2100186)

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was agreed.

R4-2100187	CR on IDLE/INACTIVE RRM requirement with SMTC2-LP R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1450  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was agreed.

R4-2100234	Interruption requirements maintenance in NR-DC (R16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1464  Cat: F (Rel-16)

					Source: Apple

Discussion:

Session chair: Cover sheet errors (What is the work item? It reads NR_newRAT-Core on the cover page but the Tdoc is reserved for TEI16.) See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was revised to R4-2103507.

R4-2103507	Interruption requirements maintenance in NR-DC (R16)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1464  rev 1 Cat: F (Rel-16)

					Source: Apple

(Replaces R4-2100234)

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was agreed.

R4-2100235	Interruption requirements maintenance in NR-DC (R17)

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1465  Cat: A (Rel-16)

					Source: Apple

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was withdrawn.

R4-2101011	Interruption requirements maintenance in NR-DC (R17)

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1542  Cat: A (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was agreed.

R4-2101075	CR for measurement period requirements correction

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1555  Cat: F (Rel-16)

					Source: NEC

Abstract:

Measurement period requirements are corrected

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was revised to R4-2103508.

R4-2103508	CR for measurement period requirements correction

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1555  rev 1 Cat: F (Rel-16)

					Source: NEC

(Replaces R4-2101075)

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was agreed.

R4-2101076	CR for measurement period requirements correction

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1556  Cat: A (Rel-17)

					Source: NEC

Abstract:

Measurement period requirements are corrected

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was agreed.

R4-2101530	Maintenance CR on interruption at EUTRA SRS carrier switching in 38.133

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1609  Cat: F (Rel-16)

					Source: OPPO

Discussion:

Session chair: Cover sheet errors (What is the work item? It reads NR_RRM_Enh-Core on the cover page but the Tdoc is reserved for TEI16. What is the revision number? It reads revision number 1 on the cover page but the Tdoc is not reserved for a revision.)

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was revised to R4-2103509.

R4-2103509	Maintenance CR on interruption at EUTRA SRS carrier switching in 38.133

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1609  rev 1 Cat: F (Rel-16)

					Source: OPPO

(Replaces R4-2101530)

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was agreed.

R4-2103722	Maintenance CR on interruption at EUTRA SRS carrier switching in 38.133

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1797  Cat: A (Rel-17)

					Source: OPPO

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was agreed.

R4-2101531	Maintenance CR on SCell activation delay requirement in TS38.133

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1610  Cat: F (Rel-16)

					Source: OPPO

Discussion:

Session chair: Cover sheet errors (What is the work item? It reads NR_RRM_Enh-Core on the cover page but the Tdoc is reserved for TEI16. What is the revision number? It reads revision number 1 on the cover page but the Tdoc is not reserved for a revision.)

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was revised to R4-2103510.

R4-2103510	Maintenance CR on SCell activation delay requirement in TS38.133

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1610  rev 1 Cat: F (Rel-16)

					Source: OPPO

(Replaces R4-2101531)

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was agreed.

R4-2103723	Maintenance CR on SCell activation delay requirement in TS38.133

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1798  Cat: - (Rel-17)

					Source: OPPO

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was agreed.

R4-2101680	Discussion on MRTD/MTTD requirements for FR1 intra-band CA/DC

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was noted.

R4-2101681	CR on MRTD/MTTD requirements for FR1 intra-band CA/DC R16

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1644  Cat: F (Rel-16)

					Source: Huawei, HiSilicon, LG Uplus, SoftBank Corp.

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was postponed.

R4-2101682	CR on MRTD/MTTD requirements for FR1 intra-band CA/DC R17

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1645  Cat: A (Rel-17)

					Source: Huawei, HiSilicon, LG Uplus, SoftBank Corp.

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was withdrawn.

R4-2101860	MRTD and MTTD in non-contiguous CA in FR1

Type: other		For: Approval
					Source: Ericsson

Abstract:

MRTD and MTTD in non-contiguous CA in FR1. This is based on an issue initiated by Huawei to allow non-colocated NCCA deployments.

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was noted.

R4-2101861	MRTD and MTTD in non-contiguous CA in FR1

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1686  Cat: F (Rel-16)

					Source: Ericsson

Abstract:

MRTD and MTTD in non-contiguous CA in FR1. This is based on an issue initiated by Huawei to allow non-colocated NCCA deployments.

Discussion:

Session chair: Cover sheet errors (What is the CR category? It reads B on the cover page but the Tdoc is reserved for category F.) See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was postponed.

R4-2104051	MRTD and MTTD in non-contiguous CA in FR1

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1686  rev 1 Cat: F (Rel-16)

					Source: Ericsson

(Replaces R4-2101861)

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was withdrawn.

R4-2101862	MRTD and MTTD in non-contiguous CA in FR1

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1687  Cat: A (Rel-17)

					Source: Ericsson

Abstract:

MRTD and MTTD in non-contiguous CA in FR1. This is based on an issue initiated by Huawei to allow non-colocated NCCA deployments.

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was withdrawn.

R4-2102250	Correction of band group notation for FR2

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1696  Cat: F (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was agreed.

R4-2102251	Correction of band group notation for FR2

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1697  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was agreed.

R4-2102889	Cat-F CR to addition of TRS Configurations in Rel-16 Test Case

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1787  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Discussion:

Session chair: Cover sheet errors (What is the release? It reads Rel-15 on the cover page but the Tdoc is reserved for Rel-16.) See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was revised to R4-2103511.

R4-2103511	Cat-F CR to addition of TRS Configurations in Rel-16 Test Case

Type: CR		For: Agreement
					38.133 v16.6.0	  CR-1787  rev 1 Cat: F (Rel-16)

					Source: Qualcomm Incorporated

(Replaces R4-2102889)

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was agreed.

R4-2102890	Cat-A CR to addition of TRS Configurations in Rel-17 Test Case

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1788  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][204] R16_NR_RRM_maintenance in R4-2103686.

Decision: 		The document was agreed.

#### 7.19.6	Demodulation and CSI [WI code or TEI16]

#### 7.19.7	NR MIMO OTA test methods (38.827) [FS_NR_MIMO_OTA_test]

R4-2101821	Uplink Power Control for NR MIMO OTA test

Type: CR		For: Agreement
					38.827 v16.1.0	  CR-0008  Cat: F (Rel-16)

					Source: vivo

Decision: 		The document was agreed.

R4-2101993	Minimum number of slots for FR1 MIMO OTA testing

Type: other		For: Endorsement
					Source: CAICT,vivo,OPPO,Huawei,HiSilicon,xiaomi,Samsung

Decision: 		The document was noted.

R4-2102081	CR to TR38.827 Number of Slots for FR1 MIMO OTA test

Type: CR		For: Agreement
					38.827 v16.1.0	  CR-0009  Cat: F (Rel-16)

					Source: CAICT,vivo,OPPO,Huawei,HiSilicon,xiaomi,Samsung

Decision: 		The document was withdrawn.

R4-2102614	On Remaining Channel Model Topics

Type: other		For: Approval
					Source: Keysight Technologies UK Ltd

Decision: 		The document was noted.

R4-2102615	CR on Channel Model Topics

Type: CR		For: Agreement
					38.827 v16.1.0	  CR-0010  Cat: F (Rel-16)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was agreed.

R4-2102817	CR to TR 38.827 on channel model rotations

Type: CR		For: Agreement
					38.827 v16.1.0	  CR-0011  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Decision: 		The document was not pursued.

R4-2102819	CR to 38.827on base station beamforming configuration

Type: CR		For: Agreement
					38.827 v16.1.0	  CR-0012  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Decision: 		The document was postponed.

R4-2103916	CR to TR38.827 on the direction of the BS strongest beams

Type: CR		For: Agreement
					38.827 v16.1.0	  CR-0013  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

## 8	Rel-16 UE feature list

GTW session (January 27, 2021)

Issue 1-1: New feature simultaneous dormant BWP switching

Agreement:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6. LTE_NR_DC_CA_enh | 6-[X] | Dormant BWP switching on multiple CCs RRM requirements | Incremental delay for BWP switch processing on additional SCells in DCI based simultaneous dormant BWP switching on multiple SCells | RAN1 feature 18-4 or 18-4a | Yes | N/A | There may be additional unclear BWP switching delay if network trigger dormant BWP switching on multiple SCells simultaneously. | Per UE | No | No | N/A | For component 2), the candidate values are: ● {100us, 200us} for UE indicates type1 in bwp-SwitchingDelay ● {200us, 400us, 800us, 1000us} for UE indicates type 2 in bwp-SwitchingDelay The total BWP switching delay will be captured in TS38.133 UE needs to indicate either of the candidate values in case it supports dormant BWP | Optional with capability signalling |

Discussion

- Huawei: Dormant BWP switching cannot be timer-based

  - QC: agree

- Session chair: Agreed capability needs to be captured in the LS to RAN2/1 on UE feature list.

Issue 2-1: New per BC indication of the per-FR gap

- Option 1 (Huawei): The per-BC indication of the per-FR gap to be introduced and the original per-UE indication to be kept.

Discussion

- Huawei: We provide analysis based on the previous meeting discussion

- MTK: We do not understand why it should be linked with band combinations and number of CCs

- QC: Support Huawei’s proposal. Current capability signalling limits the UE implementation in terms of baseband complexity. The problem is that feature gradually gets extended and has impact on BB implementation complexity (e.g. multiple BPW switching, multiple SCell activation, …). Now this is not pure RF capability.

- E///: We are not convinced that per-BC per-FR gap indication is needed. Per-BC indication will complicate gNB scheduler.

- Apple: Agree with E/// and MTK. Current capability is enough. Per-FR gap is not related to BB complexity and is mainly RF constraint. The issues mentioned by QC can be addressed in a different manner.

- Huawei: Agree that per-FR gap has impact on baseband capabilities.

- E///: one conclusion is that this is not a RF issue. We can try to address in a different way. For per-FR gap we can keep it as per-UE.

- Apple: we already have examples when we introduced some limitations to control BB complexity (e.g. searcher limitations, scaling factors…). We can follow a similar approach.

- MTK: Agree with E/// and Apple. Can handle the issues case by case.

- Session chair: Continue discussion. Identify the implementation constraints which may come from per-UE per-FR gap capability. Further capture the conclusion in the WF in the 2nd round.

================================================================================

R4-2103479	Updated RAN4 UE features list for Rel-16

Type: other		For: discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][114] R16_UE_ feature in R4-2103304.

Decision: 		The document was approved.

R4-2103480	LS on updated Rel-16 RAN4 UE features lists for NR and LTE

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: RAN4

Discussion:

See email discussion summary for [98e][114] R16_UE_ feature in R4-2103304.

Decision: 		The document was approved.

Attachments to this outgoing LS: R4-2103479

R4-2102962	Email discussion summary for [98e][114] R16_UE_ feature

Type: other		For: discussion
					Source: Moderator (CMCC)

Discussion:

The contribution summarised email discussion thread [98e][114] R16_UE_ feature. The discussion thread was moderated by Xiaoran Zhang.

Decision: 		The document was revised to R4-2103304.

R4-2103304	Email discussion summary for [98e][114] R16_UE_ feature

Type: other		For: discussion
					Source: Moderator (CMCC)

(Replaces R4-2102962)

Discussion:

The contribution summarised email discussion thread [98e][114] R16_UE_ feature. The discussion thread was moderated by Xiaoran Zhang.

Decision: 		The document was noted.

R4-2103168	Way forward on new per BC indication of the per-FR gap

Type: other		For: discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][114] R16_UE_ feature in R4-2103304.

Decision: 		The document was revised to R4-2103366.

R4-2103366	Way forward on new per BC indication of the per-FR gap

Type: other		For: discussion
					Source: CMCC

(Replaces R4-2103168)

Discussion:

See email discussion summary for [98e][114] R16_UE_ feature in R4-2103304.

Decision: 		The document was approved.

R4-2103367	Updated RAN4 UE features list for Rel-16

Type: other		For: discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][114] R16_UE_ feature in R4-2103304.

Decision: 		The document was approved.

R4-2103368	LS on updated Rel-16 RAN4 UE features lists for NR and LTE

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: RAN4

Discussion:

See email discussion summary for [98e][114] R16_UE_ feature in R4-2103304.

Decision: 		The document was approved.

Attachments to this outgoing LS: R4-2103367

R4-2101155	Discussion on UE feature list

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][114] R16_UE_ feature in R4-2103304.

Decision: 		The document was noted.

R4-2101659	Discussion on per-FR gap capability

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][114] R16_UE_ feature in R4-2103304.

Decision: 		The document was noted.

R4-2102891	UE feature on Simultaneous dormant BWP switching

Type: discussion		For: Discussion
					38.306 v..
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][114] R16_UE_ feature in R4-2103304.

Decision: 		The document was noted.

## 9	Rel-17 spectrum related Work Items for NR

R4-2102963	Email discussion summary for [98e][115] NR_Baskets_Part_1

Type: other		For: discussion
					Source: Moderator (Nokia)

Discussion:

The contribution summarised email discussion thread [98e][115] NR_Baskets_Part_1. The discussion thread was moderated by Iwajlo Angelow (Nokia Italy).

Decision: 		The document was noted.

### 9.1	NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) [NR_CA_R17_intra]

#### 9.1.1	Rapporteur Input (WID/TR/CR) [NR_CA_R17_intra-Core /Perf]

R4-2101883	Revised WID NR Intra-band Rel-17

Type: WID revised		For: Endorsement
					Source: Ericsson

Abstract:

Revised WID NR Intra-band Rel-17

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email approval.

Decision: 		The document was endorsed.

R4-2101886	CR introduction completed band combinations Rel-17 NR Intra-band -

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0657  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

CR introduction completed band combinations Rel-17 NR Intra-band -> 38.101-1

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2101887	CR introduction completed band combinations Rel-17 NR Intra-band -

Type: CR		For: Agreement
					38.101-2 v17.0.0	  CR-0332  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

CR introduction completed band combinations Rel-17 NR Intra-band -> 38.101-2

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was withdrawn.

R4-2101891	TR 38.717-01-01 v0.3.0 Rel-17 NR Intra-band

Type: draft TR		For: Agreement
					38.717-01-01 v0.2.0
					Source: Ericsson

Abstract:

TR 38.717-01-01 v0.3.0 Rel-17 NR Intra-band

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 9.1.2	UE RF for FR1 [NR_CA_R17_intra-Core]

R4-2100152	Draft CR addition of BCS2 for CA_71B

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Nokia, T-Mobile USA

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100707	Discussion on DL CA_n77(3A) support

Type: discussion		For: Discussion
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was noted.

R4-2100708	TP for TR 38.717-01-01: CA_3DL_n77(3A)_1UL_n77A

Type: pCR		For: Approval
					38.717-01-01 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102319	Rel-17 CR 38.101-1 for improvements Intra-band tables

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0680  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

Rel-17 CR 38.101-1 for improvements Intra-band tables

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103033.

R4-2103033	Rel-17 CR 38.101-1 for improvements Intra-band tables

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0680  rev 1 Cat: B (Rel-17)

					Source: Ericsson

(Replaces R4-2102319)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was agreed.

#### 9.1.3	UE RF for FR2 [NR_CA_R17_intra-Core]

### 9.2	NR inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2) [NR_CADC_R17_2BDL_xBUL]

R4-2102964	Email discussion summary for [98e][116] NR_Baskets_Part_2

Type: other		For: discussion
					Source: Moderator (Nokia)

Discussion:

The contribution summarised email discussion thread [98e][116] NR_Baskets_Part_2. The discussion thread was moderated by Johannes Hejselbaek (Nokia Denmark).

Decision: 		The document was noted.

#### 9.2.1	Rapporteur Input (WID/TR/CR) [NR_CADC_R17_2BDL_xBUL-Core/Perf]

R4-2102210	TP for TR38.717-02-01_Removal of the sub-clauses under clause 7

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102222	Revised WID on Rel-17 NR Inter-band CA_DC xUL_2DL (x=1,2)

Type: WID revised		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email approval.

Decision: 		The document was endorsed.

R4-2102223	CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into TS 38.101-1

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2102943	CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into TS 38.101-1

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0715  Cat: - (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102224	CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into TS 38.101-3

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2102944	CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into TS 38.101-3

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0500  Cat: - (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102304	TR 38.717-02-01 v0.3.0

Type: draft TR		For: Agreement
					38.717-02-01 v0.3.0
					Source: ZTE Wistron Telecom AB

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 9.2.2	NR inter band CA without any FR2 band(s) [NR_CADC_R17_2BDL_xBUL-Core]

R4-2100091	Draft CR on CA_n1-n3, CA_n1-n78, CA_n3-n78

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: China Unicom

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was not pursued.

R4-2103034	Draft CR on CA_n1-n3, CA_n1-n78, CA_n3-n78

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: China Unicom

(Replaces R4-2100091)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2100155	Draft CR addition of CA_n41-n71 configurations

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Nokia, T-Mobile USA

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2100156	Draft CR addition of CA_n25A-n41(2A)-n71A

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Nokia, T-Mobile USA

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2100259	draftCR for Rel-17 NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL within FR1

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Verizon Denmark

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103035.

R4-2103035	draftCR for Rel-17 NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL within FR1

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Verizon Denmark

(Replaces R4-2100259)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2100694	Draft CR for TS 38.101-1: Support of n77(2A) for DC_n28-n77

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2100743	Introduction of new BCS to CA_n5A-n66A and CA_n5A-78A

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

CA_n5A-n66A BCS1 and CA_n5A-78A BCS1 are added.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2100942	Draft CR for 38.101-1 to introduce CA_n3A-n78(2A) and CA_n41-n78(2A)

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103036.

R4-2103036	Draft CR for 38.101-1 to introduce CA_n3A-n78(2A) and CA_n41-n78(2A)

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Samsung, KDDI

(Replaces R4-2100942)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2100943	TP for TR 38.717-02-01: CA_n3-n18

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103037.

R4-2103037	TP for TR 38.717-02-01: CA_n3-n18

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, KDDI

(Replaces R4-2100943)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100944	TP for TR 38.717-02-01: CA_n18-n41

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103038.

R4-2103038	TP for TR 38.717-02-01: CA_n18-n41

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, KDDI

(Replaces R4-2100944)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100945	TP for TR 38.717-02-01: CA_n41-n77

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2100947	TP for TR 38.717-02-01: DC_n3-n41

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103039.

R4-2103039	TP for TR 38.717-02-01: DC_n3-n41

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, KDDI

(Replaces R4-2100947)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100948	TP for TR 38.717-02-01: DC_n28-n41

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2100949	TP for TR 38.717-02-01: DC_n41-n77

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2100950	TP for TR 38.717-02-01: DC_n41-n78

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2100973	TP for TR 38.717-02-01: CA_n25-n29

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, TELUS, Bell mobility

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100974	TP for TR 38.717-02-01: CA_n25-n71

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, TELUS, Bell mobility

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103102.

R4-2103102	TP for TR 38.717-02-01: CA_n25-n71

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, TELUS, Bell mobility

(Replaces R4-2100974)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100975	TP for TR 38.717-02-01: CA_n25-n77

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, TELUS, Bell mobility

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103040.

R4-2103040	TP for TR 38.717-02-01: CA_n25-n77

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, TELUS, Bell mobility

(Replaces R4-2100975)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2100976	TP for TR 38.717-02-01: CA_n71-n77

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, TELUS, Bell mobility

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103041.

R4-2103041	TP for TR 38.717-02-01: CA_n71-n77

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, TELUS, Bell mobility

(Replaces R4-2100976)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2100977	TP for TR 38.717-02-01: CA_n13-n25

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, TELUS, Bell mobility

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103042.

R4-2103042	TP for TR 38.717-02-01: CA_n13-n25

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, TELUS, Bell mobility

(Replaces R4-2100977)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100978	TP for TR 38.717-02-01: CA_n13-n66

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, TELUS, Bell mobility

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103043.

R4-2103043	TP for TR 38.717-02-01: CA_n13-n66

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, TELUS, Bell mobility

(Replaces R4-2100978)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101512	DraftCR for 38.101-1 to add BCS1 for  CA_n41A-n66A and BCS0 for CA_n41A-n66(2A)

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Huawei, HiSilicon, Bell Mobility, Telus

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2101599	TP for TR 38.717-02-01: CA_n8A-n20A

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101600	TP for TR 38.717-02-01: CA_n3A-n39A

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2103044	TP for TR 38.717-02-01: CA_n3A-n39A

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Huawei, HiSilicon

(Replaces R4-2101600)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2101601	DraftCR for 38.101-1 to add BCS1 for CA_n1A-n7A

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2101602	DraftCR for 38.101-1 to add BCS1 for CA_n1A-n3A

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103045.

R4-2103045	DraftCR for 38.101-1 to add BCS1 for CA_n1A-n3A

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Huawei, HiSilicon

(Replaces R4-2101602)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103377.

R4-2103377	DraftCR for 38.101-1 to add BCS1 for CA_n1A-n3A

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Huawei, HiSilicon

(Replaces R4-2103045)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2101894	draft CR for CA_n46-n48 and DC_n46-n48 configurations

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Ericsson, Charter Communication

Abstract:

draft CR for CA_n46-n48 and DC_n46-n48 configurations

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103046.

R4-2103046	draft CR for CA_n46-n48 and DC_n46-n48 configurations

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Ericsson, Charter Communication

(Replaces R4-2101894)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2102211	draft CR to TS38.101-3: Adding  CA_n39A-n258A

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2102212	TP for TR 38.716-02-00:CA_n34A-n40A

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2102314	TP for TR 38.717-02-01 to include CA_n7-n77

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Ericsson, Bell Mobility

Abstract:

TP for TR 38.717-02-01 to include CA_n7-n77

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103047.

R4-2103047	TP for TR 38.717-02-01 to include CA_n7-n77

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Ericsson, Bell Mobility

(Replaces R4-2102314)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102315	draft CR to include CA_n7-n78, CA_n25-n41, CA_n41-n77 BCS

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Ericsson, Bell Mobility

Abstract:

draft CR to include CA_n7-n78, CA_n25-n41, CA_n41-n77 BCS's

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2102320	Rel-17 CR 38.101-1 for corrections NR CA 2, 3 and 4 band configuration tables

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0681  Cat: F (Rel-17)

					Source: Ericsson

Abstract:

Rel-17 CR 38.101-1 for corrections NR CA 2, 3 and 4 band configuration tables

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was agreed.

#### 9.2.3	NR inter band CA with at least one FR2 band  [NR_CADC_R17_2BDL_xBUL-Core]

R4-2100092	Draft CR on CA_n1-n258, CA_n3-n258, CA_n78-n258

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: China Unicom

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was not pursued.

R4-2103048	Draft CR on CA_n1-n258, CA_n3-n258, CA_n78-n258

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: China Unicom

(Replaces R4-2100092)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2100154	Draft CR addition of CA_n77-n258 configurations

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Nokia, T-Mobile USA

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2100263	draftCR for Rel-17 NR inter-band CA DC combination for 2 bands DL with up to 2 bands UL between FR1 and FR2

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Verizon Denmark

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2100350	Draft CR for 38.101-3 to add n78-n257 inter-band CA configurations

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: SK Telecom

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2100351	Draft CR for 38.101-3 to add n78C in DC_n78-n257

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: SK Telecom

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2100946	TP for TR 38.717-02-01: CA_n41-n257

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103049.

R4-2103049	TP for TR 38.717-02-01: CA_n41-n257

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, KDDI

(Replaces R4-2100946)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100951	TP for TR 38.717-02-01: DC_n41-n257

Type: pCR		For: Approval
					38.717-02-01 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

### 9.3	DC of 1 LTE band and 1 NR band [DC_R17_1BLTE_1BNR_2DL2UL]

#### 9.3.1	Rapporteur Input (WID/TR/CR) [DC_R17_1BLTE_1BNR_2DL2UL-Core/Perf]

R4-2101097	TR 37.717-11-11 v0.3.0 Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL)

Type: draft TR		For: Agreement
					37.717-11-11 v0.1.0
					Source: CHTTL

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2101098	Revised WID: Rel-17 Dual Connectivity (DC) of 1 band LTE (1DL/1UL) and 1 NR band (1DL/1UL)

Type: WID revised		For: Approval
					Source: CHTTL

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2101099	Big CR for Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL)

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0455  Cat: B (Rel-17)

					Source: CHTTL

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 9.3.2	EN-DC without FR2 band [DC_R17_1BLTE_1BNR_2DL2UL-Core]

R4-2100258	Requirement of band 46 reference sensitivity measurement for NR EN-DC band combinations

Type: discussion		For: Decision
					Source: Verizon Denmark

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103009.

R4-2103009	Requirement of band 46 reference sensitivity measurement for NR EN-DC band combinations

Type: discussion		For: Decision
					Source: Verizon Denmark

(Replaces R4-2100258)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2100260	draftCR for Rel-17Dual Connectivity of 1LTE band (1DL/1UL) and 1NR band (1DL/1UL) within FR1

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Verizon Denmark

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100261	draftCR for Rel-16 Dual Connectivity of 1LTE band (1DL/1UL) and 1NR band (1DL/1UL) with FR1

Type: draftCR		For: Endorsement
					38.101-3 v16.6.0
					Source: Verizon Denmark

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was withdrawn.

R4-2100301	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +1LTE band) within FR1 DC_40A_n78(2A)/DC_40C_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100302	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +1LTE band) within FR1 DC_8A_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100352	TP for DC_21_n28 for TR 37.717-11-11

Type: pCR		For: Approval
					37.717-11-11 v0.1.0
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was withdrawn.

R4-2100732	TP to TR 37.717-11-11 DC_25_n77

Type: other		For: Approval
					37.717-11-11 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, a text proposal to complete 2DL/2UL EN-DC configurations, DC_25A_n77A and DC_25A-25A_n77A, is provided.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103012.

R4-2103012	TP to TR 37.717-11-11 DC_25_n77

Type: other		For: Approval
					37.717-11-11 v..
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100732)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2100733	TP to TR 37.717-11-11 DC_25_n78

Type: other		For: Approval
					37.717-11-11 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, a text proposal to complete 2DL/2UL EN-DC configurations, DC_25A_n78A and DC_25A-25A_n78A, is provided.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103013.

R4-2103013	TP to TR 37.717-11-11 DC_25_n78

Type: other		For: Approval
					37.717-11-11 v..
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100733)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101224	TP for DC_21_n28 for TR 37.717-11-11

Type: pCR		For: Approval
					37.717-11-11 v0.1.0
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103021.

R4-2103021	TP for DC_21_n28 for TR 37.717-11-11

Type: pCR		For: Approval
					37.717-11-11 v0.1.0
					Source: NTT DOCOMO INC.

(Replaces R4-2101224)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101584	TP for TR 37.717-11-11: DC_12_n71

Type: pCR		For: Approval
					37.717-11-11 v0.2.0
					Source: Huawei, HiSilicon, Ericsson, US Cellular

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103026.

R4-2103026	TP for TR 37.717-11-11: DC_12_n71

Type: pCR		For: Approval
					37.717-11-11 v0.2.0
					Source: Huawei, HiSilicon, Ericsson, US Cellular

(Replaces R4-2101584)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101909	TP for TR 37.717-11-11 to include 71A_n41A

Type: pCR		For: Approval
					37.717-11-11 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-11 to include 71A_n41A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103027.

R4-2103027	TP for TR 37.717-11-11 to include 71A_n41A

Type: pCR		For: Approval
					37.717-11-11 v0.2.0
					Source: Ericsson, Rogers

(Replaces R4-2101909)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101910	TP for TR 37.717-11-11 to include 7A_n2A

Type: pCR		For: Approval
					37.717-11-11 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-11 to include 7A_n2A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was noted.

R4-2101911	TP for TR 37.717-11-11 to include 71A_n2A

Type: pCR		For: Approval
					37.717-11-11 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-11 to include 71A_n2A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103028.

R4-2103028	TP for TR 37.717-11-11 to include 71A_n2A

Type: pCR		For: Approval
					37.717-11-11 v0.2.0
					Source: Ericsson, Rogers

(Replaces R4-2101911)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102045	draft CR 38.101-3 adding CA_n7B UL configurations for 1 LTE + 1 NR

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Ericsson

Abstract:

draft CR 38.101-3 adding CA_n7B UL configurations for 1 LTE + 1 NR

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103029.

R4-2103029	draft CR 38.101-3 adding CA_n7B UL configurations for 1 LTE + 1 NR

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Ericsson

(Replaces R4-2102045)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was not pursued.

R4-2102213	draft CR to TS38.101-3: Adding DC_40A_n41C and DC_40A_n41(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

#### 9.3.3	EN-DC with FR2 band  [DC_R17_1BLTE_1BNR_2DL2UL-Core]

R4-2100262	draftCR for Rel-17 Dual Connectivity of 1LTE band (1DL/1UL) and 1NR band (1DL/1UL) including FR2

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Verizon Denmark

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2101896	draft CR 38.101-3 to include 2_n258, 5_n258, 12_n258, 66_n258 configurations

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Ericsson, US Cellular

Abstract:

draft CR 38.101-3 to include 2_n258, 5_n258, 12_n258, 66_n258 configurations

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

### 9.4	DC of 2 LTE band and 1 NR band [DC_R17_2BLTE_1BNR_3DL2UL]

#### 9.4.1	Rapporteur Input (WID/TR/CR) [DC_R17_2BLTE_1BNR_3DL2UL-Core/Perf]

R4-2101508	TR 37.717-21-11 V0.2.0 for DC of 2 LTE band and 1 NR band

Type: draft TR		For: Agreement
					37.717-21-11 v0.3.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was not concluded.

R4-2101509	Revised WID: Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)

Type: WID revised		For: Endorsement
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was not concluded.

R4-2101510	CR on introduction of completed EN-DC of 2 bands LTE and 1 band NR from RAN4#96e and RAN4#97e into TS 38.101-3

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0460  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 9.4.2	EN-DC without FR2 band [DC_R17_2BLTE_1BNR_3DL2UL-Core]

R4-2100145	TP to TR 37.717-21-11: DC_20-40_n78

Type: pCR		For: Approval
					37.717-21-11 v0.2.0
					Source: Nokia, Telefonica

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103006.

R4-2103006	TP to TR 37.717-21-11: DC_20-40_n78

Type: pCR		For: Approval
					37.717-21-11 v0.2.0
					Source: Nokia, Telefonica

(Replaces R4-2100145)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2100299	Discussion of MSD for 3DL2UL DC_1-21_n28 due to UL IMD3 issue

Type: pCR		For: Approval
					37.717-21-11 v0.1.0
					Source: Mediatek India Technology Pvt.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was noted.

R4-2100303	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +2LTE band) within FR1 DC_1A-40A_n78(2A)/DC_1A-40C_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100304	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +2LTE band) within FR1 DC_1A-8A_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100305	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +2LTE band) within FR1 DC_3A-40A_n78(2A)/DC_3A-40C_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100306	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +2LTE band) within FR1 DC_3A-8A_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100307	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +2LTE band) within FR1 DC_7A-40A_n78(2A)/DC_7A-40C_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100308	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +2LTE band) within FR1 DC_7A-8A_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100309	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +2LTE band) within FR1 DC_8A-40A_n78(2A)/DC_8A-40C_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100345	TP for TR 37.717-21-11: DC_3-18_n41

Type: pCR		For: Approval
					37.717-21-11 v0.1.0
					Source: KDDI Corporation

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103010.

R4-2103010	TP for TR 37.717-21-11: DC_3-18_n41

Type: pCR		For: Approval
					37.717-21-11 v0.1.0
					Source: KDDI Corporation

(Replaces R4-2100345)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2100508	TP for DC_1-21_n28 for TR 37.717-21-11

Type: pCR		For: Approval
					37.717-21-11 v0.1.0
					Source: NTT DOCOMO, INC. MediaTek Inc., LG Electronics

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was withdrawn.

R4-2100509	TP for DC_3-21_n28 for TR 37.717-21-11

Type: pCR		For: Approval
					37.717-21-11 v0.1.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was withdrawn.

R4-2100642	TP update for TR 37.717-21-11: EN-DC_1-11_n28

Type: pCR		For: Approval
					37.717-21-11 v0.1.0
					Source: SoftBank Corp., LG Electronics

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103011.

R4-2103011	TP update for TR 37.717-21-11: EN-DC_1-11_n28

Type: pCR		For: Approval
					37.717-21-11 v0.1.0
					Source: SoftBank Corp., LG Electronics

(Replaces R4-2100642)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2100734	TP to TR 37.717-21-11 DC_7-25_n77

Type: other		For: Approval
					37.717-21-11 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, a text proposal to complete 3DL/2UL EN-DC configurations, DC_7A-25A_n77A, DC_7A-7A-25A_n77A, DC_7C-25A_n77A, DC_7C-25A-25A_n77A, DC_7A-25A-25A_n77A and DC_7A-7A-25A-25A_n77A, is provided.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103014.

R4-2103014	TP to TR 37.717-21-11 DC_7-25_n77

Type: other		For: Approval
					37.717-21-11 v..
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100734)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2100735	TP to TR 37.717-21-11 DC_7-25_n78

Type: other		For: Approval
					37.717-21-11 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, a text proposal to complete 3DL/2UL EN-DC configurations, DC_7A-25A_n78A, DC_7A-7A-25A_n78A, DC_7C-25A_n78A, DC_7A-25A-25A_n78A, DC_7A-7A-25A-25A_n78A, and DC_7C-25A-25A_n78A, is provided.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103015.

R4-2103015	TP to TR 37.717-21-11 DC_7-25_n78

Type: other		For: Approval
					37.717-21-11 v..
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100735)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2100736	TP to TR 37.717-21-11 DC_25-66_n77

Type: other		For: Approval
					37.717-21-11 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, a text proposal to complete 3DL/2UL EN-DC configurations, DC_25A-66A_n77A and DC_25A-25A-66A_n77A, is provided.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103016.

R4-2103016	TP to TR 37.717-21-11 DC_25-66_n77

Type: other		For: Approval
					37.717-21-11 v..
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100736)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2100737	TP to TR 37.717-21-11 DC_25-66_n78

Type: other		For: Approval
					37.717-21-11 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, a text proposal to complete 3DL/2UL EN-DC configurations, DC_25A-66A_n78A and DC_25A-25A-66A_n78A, is provided

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103017.

R4-2103017	TP to TR 37.717-21-11 DC_25-66_n78

Type: other		For: Approval
					37.717-21-11 v..
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100737)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2100983	TP for TR 37.717-21-11: DC_2-29_n78

Type: pCR		For: Approval
					37.717-21-11 v0.2.0
					Source: Samsung, TELUS, Bell mobility

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103018.

R4-2103018	TP for TR 37.717-21-11: DC_2-29_n78

Type: pCR		For: Approval
					37.717-21-11 v0.2.0
					Source: Samsung, TELUS, Bell mobility

(Replaces R4-2100983)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2100984	TP for TR 37.717-21-11: DC_29-66_n78

Type: pCR		For: Approval
					37.717-21-11 v0.2.0
					Source: Samsung, TELUS, Bell mobility

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103019.

R4-2103019	TP for TR 37.717-21-11: DC_29-66_n78

Type: pCR		For: Approval
					37.717-21-11 v0.2.0
					Source: Samsung, TELUS, Bell mobility

(Replaces R4-2100984)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101230	TP for DC_1-21_n28 for TR 37.717-21-11

Type: pCR		For: Approval
					37.717-21-11 v0.1.0
					Source: NTT DOCOMO INC. , MediaTek Inc., LG Electronics

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103022.

R4-2103022	TP for DC_1-21_n28 for TR 37.717-21-11

Type: pCR		For: Approval
					37.717-21-11 v0.1.0
					Source: NTT DOCOMO INC. , MediaTek Inc., LG Electronics

(Replaces R4-2101230)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101231	TP for DC_3-21_n28 for TR 37.717-21-11

Type: pCR		For: Approval
					37.717-21-11 v0.1.0
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101511	TP for 37.717-21-11: correction of duplicated TPS for some combinations

Type: pCR		For: Approval
					37.717-21-11 v17.0.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101520	DC_2A-66A-66A_n66A,DC_7A-66A-66A_n66A and DC_7A-7A-66A-66A_n66A

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei, HiSilicon, Bell Mobility, Telus

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2101545	TP for TR 37.717-21-11: DC_8-20_n1

Type: pCR		For: Approval
					37.717-21-11 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103023.

R4-2103023	TP for TR 37.717-21-11: DC_8-20_n1

Type: pCR		For: Approval
					37.717-21-11 v0.1.0
					Source: VODAFONE Group Plc

(Replaces R4-2101545)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101546	TP for TR 37.717-21-11: DC_8-20_n3

Type: pCR		For: Approval
					37.717-21-11 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103024.

R4-2103024	TP for TR 37.717-21-11: DC_8-20_n3

Type: pCR		For: Approval
					37.717-21-11 v0.1.0
					Source: VODAFONE Group Plc

(Replaces R4-2101546)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101547	TP for TR 37.717-21-11: DC_8-20_n28

Type: pCR		For: Approval
					37.717-21-11 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was noted.

R4-2101548	TP for TR 37.717-21-11: DC_8-32_n1

Type: pCR		For: Approval
					37.717-21-11 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101912	draft CR to include DC_2-7_n78 configuration

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Ericsson, Rogers

Abstract:

draft CR to include DC_2-7_n78 configuration

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2101913	TP for TR 37.717-21-11 to include 12A-66A_n41A

Type: pCR		For: Approval
					37.717-21-11 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-21-11 to include 12A-66A_n41A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101914	TP for TR 37.717-21-11 to include 2A-12A_n41A, 2A-2A-12A_n41A

Type: pCR		For: Approval
					37.717-21-11 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-21-11 to include 2A-12A_n41A, 2A-2A-12A_n41A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101915	TP for TR 37.717-21-11 to include 66A-71A_n41A

Type: pCR		For: Approval
					37.717-21-11 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-21-11 to include 66A-71A_n41A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101916	TP for TR 37.717-21-11 to include 2A-71A_n41A, 2A-2A-71A_n41A

Type: pCR		For: Approval
					37.717-21-11 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-21-11 to include 2A-71A_n41A, 2A-2A-71A_n41A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101917	TP for TR 37.717-21-11 to include 7A-12A_n66A

Type: pCR		For: Approval
					37.717-21-11 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-21-11 to include 7A-12A_n66A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101918	TP for TR 37.717-21-11 to include 7A-71A_n66A

Type: pCR		For: Approval
					37.717-21-11 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-21-11 to include 7A-71A_n66A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101919	TP for TR 37.717-21-11 to include 7A-12A_n78A

Type: pCR		For: Approval
					37.717-21-11 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-21-11 to include 7A-12A_n78A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101920	TP for TR 37.717-21-11 to include 12A-66A_n78A

Type: pCR		For: Approval
					37.717-21-11 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-21-11 to include 12A-66A_n78A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101921	TP for TR 37.717-21-11 to include 2A-12A_78A, 2A-2A-12A_78A

Type: pCR		For: Approval
					37.717-21-11 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-21-11 to include 2A-12A_78A, 2A-2A-12A_78A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102046	draft CR 38.101-3 adding CA_n7B UL configurations for 2 LTE + 1 NR

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Ericsson

Abstract:

draft CR 38.101-3 adding CA_n7B UL configurations for 2 LTE + 1 NR

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103030.

R4-2103030	draft CR 38.101-3 adding CA_n7B UL configurations for 2 LTE + 1 NR

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Ericsson

(Replaces R4-2102046)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was not pursued.

R4-2102050	TP for TR 37.717-21-11 to include 7A-71A_n78A

Type: pCR		For: Approval
					37.717-21-11 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-21-11 to include 7A-71A_n78A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102318	draft CR to include DC_2-7_n66 configuration

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Ericsson, Bell Mobility

Abstract:

draft CR to include DC_2-7_n66 configuration

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

#### 9.4.3	DMEN-DC with FR2 band  [DC_R17_2BLTE_1BNR_3DL2UL-Core]

R4-2100696	Draft CR for TS 38.101-3: Support of DC_3-11_n257

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

### 9.5	DC of 3 LTE band and 1 NR band [DC_R17_3BLTE_1BNR_4DL2UL]

#### 9.5.1	Rapporteur Input (WID/TR/CR) [DC_R17_3BLTE_1BNR_4DL2UL-Core/Perf]

R4-2101884	Revised WID LTE 3DL and one NR band Rel-17

Type: WID revised		For: Endorsement
					Source: Ericsson

Abstract:

Revised WID LTE 3DL and one NR band Rel-17

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was not concluded.

R4-2101888	CR introduction completed band combinations LTE 3DL and one NR band -

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0471  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

CR introduction completed band combinations LTE 3DL and one NR band -> 38.101-3

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2101892	TR 37.717-31-11 v0.3.0 Rel-17 DC combinations LTE 3DL and one NR band

Type: draft TR		For: Agreement
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TR 37.717-31-11 v0.3.0 Rel-17 DC combinations LTE 3DL and one NR band

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 9.5.2	EN-DC without FR2 band [DC_R17_3BLTE_1BNR_4DL2UL-Core]

R4-2100146	TP to TR 37.717-31-11: DC_1-20-40_n78

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Nokia, Telefonica

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103007.

R4-2103007	TP to TR 37.717-31-11: DC_1-20-40_n78

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Nokia, Telefonica

(Replaces R4-2100146)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2100310	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_1A-3A-40A_n78(2A)/DC_1A-3A-40C_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100311	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_1A-3A-8A_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100312	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_1A-7A-40A_n78(2A)/DC_1A-7A-40C_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100313	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_1A-7A-8A_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100314	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_1A-8A-40A_n78(2A)/DC_1A-8A-40C_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100315	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_3A-7A-40A_n78(2A)/DC_3A-7A-40C_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100316	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_3A-7A-8A_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100317	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_3A-8A-40A_n78(2A)/DC_3A-8A-40C_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100318	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_7A-8A-40A_n78(2A)/DC_7A-8A-40C_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100652	TP for TR 37.717-31-11: EN-DC_1-8-42_n3

Type: pCR		For: Approval
					37.717-31-11 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2100671	TP for TR 37.717-31-11: EN-DC_1-3-42_n28

Type: pCR		For: Approval
					37.717-31-11 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2100742	Introduction of DC_7-25-66_n77 and DC_7-25-66_n78

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Introduce new config of DC bands, DC_7-25-66_n77 and D_7-25-66_n78

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100985	TP for TR 37.717-31-11: DC_2-29-66_n78

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Samsung, TELUS, Bell mobility

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2103020	TP for TR 37.717-31-11: DC_2-29-66_n78

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Samsung, TELUS, Bell mobility

(Replaces R4-2100985)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was withdrawn.

R4-2101518	DraftCR for 38.101-3 to add DC_7A-7A-13A-66A_n66A, DC_2A-7A-66A-66A_n66A and DC_2A-7A-7A-66A-66A_n66A

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei, HiSilicon, Bell Mobility, Telus

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2101549	TP for TR 37.717-31-11: DC_1-8-20_n28

Type: pCR		For: Approval
					37.717-31-11 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was noted.

R4-2101550	TP for TR 37.717-31-11: DC_7-8-20_n1

Type: pCR		For: Approval
					37.717-31-11 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was noted.

R4-2101551	TP for TR 37.717-31-11: DC_7-8-20_n3

Type: pCR		For: Approval
					37.717-31-11 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was noted.

R4-2101552	TP for TR 37.717-31-11: DC_7-8-32_n1

Type: pCR		For: Approval
					37.717-31-11 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101553	TP for TR 37.717-31-11: DC_7-20-32_n78

Type: pCR		For: Approval
					37.717-31-11 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101554	TP for TR 37.717-31-11: DC_8-20-32_n1

Type: pCR		For: Approval
					37.717-31-11 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was noted.

R4-2101895	TP to TR TR 37.717-31-11 to include 3-20-40_n78

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR TR 37.717-31-11 to include 3-20-40_n78

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102019	TP to TR TR 37.717-31-11 to include DC_1A-7A-28A_n3A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR TR 37.717-31-11 to include DC_1A-7A-28A_n3A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102020	TP to TR 37.717-31-11 to include 2A-12A-66A_n41A, 2A-2A-12A-66A_n41A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 2A-12A-66A_n41A, 2A-2A-12A-66A_n41A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102021	TP to TR 37.717-31-11 to include 2A-66A-71A_n41A, 2A-2A-66A-71A_n41A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 2A-66A-71A_n41A, 2A-2A-66A-71A_n41A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102022	TP to TR 37.717-31-11 to include 2A-7A-12A_n66A, 2A-2A-7A-12A_n66A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 2A-7A-12A_n66A, 2A-2A-7A-12A_n66A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102023	TP to TR 37.717-31-11 to include 2A-2A-5A-7A_n66A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 2A-2A-5A-7A_n66A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102024	TP to TR 37.717-31-11 to include 2A-7A-71A_n66A, 2A-2A-7A-71A_n66A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 2A-7A-71A_n66A, 2A-2A-7A-71A_n66A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102025	TP to TR 37.717-31-11 to include 2A-7A-12A_n78A, 2A-2A-7A-12A_n78A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 2A-7A-12A_n78A, 2A-2A-7A-12A_n78A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102026	TP to TR 37.717-31-11 to include 2A-12A-66A_n78A, 2A-2A-12A-66A_n78A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 2A-12A-66A_n78A, 2A-2A-12A-66A_n78A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102027	TP to TR 37.717-31-11 to include 7A-12A-66A_n78A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 7A-12A-66A_n78A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102028	TP to TR 37.717-31-11 to include 7A-66A-71A_n78A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 7A-66A-71A_n78A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102029	TP to TR 37.717-31-11 to include 2A-7A-71A_n78A, 2A-2A-7A-71A_n78A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 2A-7A-71A_n78A, 2A-2A-7A-71A_n78A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102030	TP to TR 37.717-31-11 to include 2A-7A-66A_n2A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 2A-7A-66A_n2A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102031	TP to TR 37.717-31-11 to include 2A-5A-7A_n2A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 2A-5A-7A_n2A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102032	TP to TR 37.717-31-11 to include 5A-7A-66A_n2A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 5A-7A-66A_n2A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102033	TP to TR 37.717-31-11 to include 2A-7A-71A_n2A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 2A-7A-71A_n2A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102034	TP to TR 37.717-31-11 to include 2A-66A-71A_n2A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 2A-66A-71A_n2A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102035	TP to TR 37.717-31-11 to include 2A-7A-12A_n2A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 2A-7A-12A_n2A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102036	TP to TR 37.717-31-11 to include 7A-66A-71A_n2A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-31-11 to include 7A-66A-71A_n2A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102037	7TP to TR 37.717-31-11 to include 7A-12A-66A_n2A

Type: pCR		For: Approval
					37.717-31-11 v0.2.0
					Source: Ericsson

Abstract:

7TP to TR 37.717-31-11 to include 7A-12A-66A_n2A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102047	draft CR 38.101-3 adding CA_n7B UL configurations for 3 LTE + 1 NR

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Ericsson

Abstract:

draft CR 38.101-3 adding CA_n7B UL configurations for 3 LTE + 1 NR

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103032.

R4-2103032	draft CR 38.101-3 adding CA_n7B UL configurations for 3 LTE + 1 NR

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Ericsson

(Replaces R4-2102047)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was not pursued.

R4-2102051	draft CR to include DC_2-7-66_n71, DC_2-7-66_n78 configurations

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Ericsson, Rogers

Abstract:

draft CR to include DC_2-7-66_n71, DC_2-7-66_n78 configurations

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2102317	draft CR to include DC_2-7-13_n66 configurations

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Ericsson, Bell Mobility

Abstract:

draft CR to include DC_2-7-13_n66 configurations

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2103031	draft CR 38.101-3 adding CA_n7B UL configurations for 3 LTE + 1 NR

Type: draftCR		For: Endorsement
					Source: Ericsson

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was not concluded.

#### 9.5.3	EN-DC with FR2 band  [DC_R17_3BLTE_1BNR_4DL2UL-Core]

R4-2100702	Draft CR for TS 38.101-3: Support of DC_1-3-11_n257 and DC_3-8-11_n257

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

### 9.6	DC of 4 LTE band and 1 NR band [DC_R17_4BLTE_1BNR_5DL2UL]

#### 9.6.1	Rapporteur Input (WID/TR/CR) [DC_R17_4BLTE_1BNR_5DL2UL-Core/Perf]

R4-2101924	Revised Rel-17 WID on DC of 4 bands LTE inter-band CA (4DL1UL) and 1 NR band (1DL1UL)

Type: WID revised		For: Endorsement
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Inclusion of requests provided at RAN4#98

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2101925	CR to introduce new combinations of LTE 4band + NR 1band for TS 38.101-3

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0473  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

Inclusion of approved combinations provided at RAN4#98

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2101926	draft TR 37.717-41-11 v0.3.0

Type: draft TR		For: Agreement
					37.717-41-11 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Inclusion of TPs provided at RAN4#98

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 9.6.2	EN-DC without FR2 band [DC_R17_4BLTE_1BNR_5DL2UL-Core]

R4-2100147	TP to TR 37.717-41-11: DC_1-3-20-40_n78

Type: pCR		For: Approval
					37.717-41-11 v0.2.0
					Source: Nokia, Telefonica

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was revised to R4-2103008.

R4-2103008	TP to TR 37.717-41-11: DC_1-3-20-40_n78

Type: pCR		For: Approval
					37.717-41-11 v0.2.0
					Source: Nokia, Telefonica

(Replaces R4-2100147)

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2100319	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +4LTE band) within FR1 DC_1A-3A-7A-40A_n78(2A)/DC_1A-3A-7A-40C_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100320	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +4LTE band) within FR1 DC_1A-3A-7A-8A_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100321	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +4LTE band) within FR1 DC_1A-3A-8A-40A_n78(2A)/DC_1A-3A-8A-40C_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100322	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +4LTE band) within FR1 DC_1A-7A-8A-40A_n78(2A)/DC_1A-7A-8A-40C_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100323	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +4LTE band) within FR1 DC_3A-7A-8A-40A_n78(2A)/DC_3A-7A-8A-40C_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2100672	TP for TR 37.717-41-11: EN-DC_1-3-8-11_n28

Type: pCR		For: Approval
					37.717-41-11 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2100673	TP for TR 37.717-41-11: EN-DC_1-3-8-11_n77

Type: pCR		For: Approval
					37.717-41-11 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101519	DraftCR for 38.101-3 to add DC_2A-7A-7A-13A-66A_N66a and DC_2A-5A-7A-7A-66A_n66A

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei, HiSilicon, Bell Mobility, Telus

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

R4-2101562	TP for TR 37.717-41-11: DC_1-7-8-20_n3

Type: pCR		For: Approval
					37.717-41-11 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was noted.

R4-2101563	TP for TR 37.717-41-11: DC_1-7-8-20_n28

Type: pCR		For: Approval
					37.717-41-11 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was noted.

R4-2101573	TP for TR 37.717-41-11: DC_1-7-8-20_n78

Type: pCR		For: Approval
					37.717-41-11 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2101574	TP for TR 37.717-41-11: DC_7-8-20-32_n1

Type: pCR		For: Approval
					37.717-41-11 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was noted.

R4-2102038	TP to TR 37.717-41-11 to include 2A-7A-12A-66A_n78A, 2A-2A-7A-12A-66A_n78A

Type: pCR		For: Approval
					37.717-41-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-41-11 to include 2A-7A-12A-66A_n78A,2A-2A-7A-12A-66A_n78A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102039	TP to TR 37.717-41-11 to include 2A-7A-66A-71A_n78A, 2A-2A-7A-66A-71A_n78A

Type: pCR		For: Approval
					37.717-41-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-41-11 to include 2A-7A-66A-71A_n78A, 2A-2A-7A-66A-71A_n78A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102040	TP to TR 37.717-41-11 to include 2A-5A-7A-66A_n2A

Type: pCR		For: Approval
					37.717-41-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-41-11 to include 2A-5A-7A-66A_n2A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102041	TP to TR 37.717-41-11 to include 2A-7A-66A-71A_n2A

Type: pCR		For: Approval
					37.717-41-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-41-11 to include 2A-7A-66A-71A_n2A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

R4-2102042	TP to TR 37.717-41-11 to include 2A-7A-12A-66A_n2A

Type: pCR		For: Approval
					37.717-41-11 v0.2.0
					Source: Ericsson

Abstract:

TP to TR 37.717-41-11 to include 2A-7A-12A-66A_n2A

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was approved.

#### 9.6.3	EN-DC with FR2 band  [DC_R17_4BLTE_1BNR_5DL2UL-Core]

R4-2100704	Draft CR for TS 38.101-3: Support of DC_1-3-8-11_n257

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

### 9.7	DC of x bands (x=1,2, 3, 4) LTE inter-band CA and 2 bands NR inter-band CA  [DC_R17_xBLTE_2BNR_yDL2UL]

#### 9.7.1	Rapporteur Input (WID/TR/CR) [DC_R17_xBLTE_2BNR_yDL2UL-Core/Per]

R4-2100272	TR 37.717-11-21 v0.3.0 TR update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17

Type: draft TR		For: Agreement
					37.717-11-21 v0.3.0
					Source: LG Electronics France

Abstract:

Update TR to capture approved TPs for LTE(xDL/1UL)+ NR(2DL/1UL) DC band combos.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2100275	Revised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) DC in Rel-17

Type: WID revised		For: Endorsement
					Source: LG Electronics France

Abstract:

Revised WID to capture new NR DC combos and update the status for each DC band combos for LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) DC band combos

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2100277	Introduction CR on new NR DC LTE(xDL/1UL)+ NR(2DL/1UL) band combinations in Rel-17

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0438  Cat: B (Rel-17)

					Source: LG Electronics France

Abstract:

Big CR to add new DC combos in TS38.101-3 in Rel-17

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 9.7.2	EN-DC including NR inter CA without FR2 band [DC_R17_xBLTE_2BNR_yDL2UL-Core]

R4-2100278	TP on summary of self-interference analysis for new NR DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: LG Electronics France

Abstract:

propose TP to add the self interference analysis results for the new DC band combos

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100279	MSD anlaysis results for new DC band combinations

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: LG Electronics France

Abstract:

Propose MSD levels for the new DC band combos with self interfered problems

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103050.

R4-2103050	MSD anlaysis results for new DC band combinations

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: LG Electronics France

(Replaces R4-2100279)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100298	Discussion of MSD for 3DL2UL DC_21_n28-n79 due to UL IMD issues

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Mediatek India Technology Pvt.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2100300	TP for TR 37.717-11-21: to update DC_18_n3-n41 with IMD3 MSD

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Mediatek India Technology Pvt.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103051.

R4-2103051	TP for TR 37.717-11-21: to update DC_18_n3-n41 with IMD3 MSD

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Mediatek India Technology Pvt.

(Replaces R4-2100300)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100325	TP for TR 37.717-11-21:DC_3A-40A_n1A-n78A/DC_3A-40C_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103052.

R4-2103052	TP for TR 37.717-11-21:DC_3A-40A_n1A-n78A/DC_3A-40C_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Huawei,HiSilicon

(Replaces R4-2100325)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100326	TP for TR 37.717-11-21:DC_7A-40A_n1A-n78A/DC_7A-40C_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103053.

R4-2103053	TP for TR 37.717-11-21:DC_7A-40A_n1A-n78A/DC_7A-40C_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Huawei,HiSilicon

(Replaces R4-2100326)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100327	TP for TR 37.717-11-21:DC_8A-40A_n1A-n78A/DC_8A-40C_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103054.

R4-2103054	TP for TR 37.717-11-21:DC_8A-40A_n1A-n78A/DC_8A-40C_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Huawei,HiSilicon

(Replaces R4-2100327)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100328	TP for TR 37.717-11-21:DC_3A-7A-40A_n1A-n78A/DC_3A-7A-40C_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103055.

R4-2103055	TP for TR 37.717-11-21:DC_3A-7A-40A_n1A-n78A/DC_3A-7A-40C_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Huawei,HiSilicon

(Replaces R4-2100328)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100329	TP for TR 37.717-11-21:DC_3A-8A-40A_n1A-n78A/DC_3A-8A-40C_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103056.

R4-2103056	TP for TR 37.717-11-21:DC_3A-8A-40A_n1A-n78A/DC_3A-8A-40C_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Huawei,HiSilicon

(Replaces R4-2100329)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100330	TP for TR 37.717-11-21:DC_7A-8A-40A_n1A-n78A/DC_7A-8A-40C_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103057.

R4-2103057	TP for TR 37.717-11-21:DC_7A-8A-40A_n1A-n78A/DC_7A-8A-40C_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Huawei,HiSilicon

(Replaces R4-2100330)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100331	TP for TR 37.717-11-21:DC_3A-7A-8A-40A_n1A-n78A/DC_3A-7A-8A-40C_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103058.

R4-2103058	TP for TR 37.717-11-21:DC_3A-7A-8A-40A_n1A-n78A/DC_3A-7A-8A-40C_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: Huawei,HiSilicon

(Replaces R4-2100331)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100346	TP for TR 37.717-11-21: DC_1-3-18_n3-n41

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: KDDI Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103059.

R4-2103059	TP for TR 37.717-11-21: DC_1-3-18_n3-n41

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: KDDI Corporation

(Replaces R4-2100346)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100347	TP for TR 37.717-11-21: DC_1-3-18_n28-n41

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: KDDI Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103060.

R4-2103060	TP for TR 37.717-11-21: DC_1-3-18_n28-n41

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: KDDI Corporation

(Replaces R4-2100347)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100348	TP for TR 37.717-11-21: DC_1-3-18_n41-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: KDDI Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103061.

R4-2103061	TP for TR 37.717-11-21: DC_1-3-18_n41-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: KDDI Corporation

(Replaces R4-2100348)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100349	TP for TR 37.717-11-21: DC_1-3-18_n41-n78

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: KDDI Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103062.

R4-2103062	TP for TR 37.717-11-21: DC_1-3-18_n41-n78

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: KDDI Corporation

(Replaces R4-2100349)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100503	TP for DC_1_n28-n79 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO, INC. MediaTek Inc., LG Electronics

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2100504	TP for DC_3_n28-n79 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO, INC. MediaTek Inc., LG Electronics

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2100505	TP for DC_21_n28-n77 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO, INC. MediaTek Inc., LG Electronics

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2100506	TP for DC_21_n28-n78 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO, INC. MediaTek Inc., LG Electronics

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2100507	TP for DC_21_n28-n79 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO, INC. MediaTek Inc., LG Electronics

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2100643	TP update for TR 37.717-11-21: EN-DC_11_n3-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100644	TP update for TR 37.717-11-21: EN-DC_11_n28-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100649	TP for TR 37.717-11-21: EN-DC_3-11_n28-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100650	TP for TR 37.717-11-21: EN-DC_8-11_n28-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100653	TP for TR 37.717-11-21: EN-DC_1-11_n3-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100654	TP for TR 37.717-11-21: EN-DC_8-11_n3-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100660	TP for TR 37.717-11-21: EN-DC_1-11_n28-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100663	TP for TR 37.717-11-21: EN-DC_1-42_n3-n28

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100666	TP for TR 37.717-11-21: EN-DC_1-42_n3-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100667	TP for TR 37.717-11-21: EN-DC_8-42_n3-n28

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100668	TP for TR 37.717-11-21: EN-DC_8-42_n3-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100674	TP for TR 37.717-11-21: EN-DC_1-8-11_n3-n28

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100675	TP for TR 37.717-11-21: EN-DC_1-8-11_n3-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100676	TP for TR 37.717-11-21: EN-DC_1-3-11_n28-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100677	TP for TR 37.717-11-21: EN-DC_1-8-11_n28-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100678	TP for TR 37.717-11-21: EN-DC_1-8-42_n3-n28

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100679	TP for TR 37.717-11-21: EN-DC_1-8-42_n3-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100680	TP for TR 37.717-11-21: EN-DC_3-8-11_n28-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100688	TP for TR 37.717-11-21: EN-DC_1-3-42_n28-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100689	TP for TR 37.717-11-21: EN-DC_1-3-8-11_n28-n77

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100706	MSD evaluation for TR 37.717-11-21

Type: discussion		For: Approval
					37.717-11-21 v..
					Source: MediaTek Inc.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2101190	TP for DC_1-21_n28-n77 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101191	TP for DC_1-21_n28-n78 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101192	TP for DC_1-21_n28-n79 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101193	TP for DC_1-3_n28-n79 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101194	TP for DC_3-21_n28-n77 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101195	TP for DC_3-21_n28-n78 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101196	TP for DC_3-21_n28-n79 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101225	TP for DC_1_n28-n79 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO INC., MediaTek Inc., LG Electronics

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101226	TP for DC_3_n28-n79 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO INC., MediaTek Inc., LG Electronics

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101227	TP for DC_21_n28-n77 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO INC. , MediaTek Inc., LG Electronics

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101228	TP for DC_21_n28-n78 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO INC. , MediaTek Inc., LG Electronics

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101229	TP for DC_21_n28-n79 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO INC. , MediaTek Inc., LG Electronics

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103063.

R4-2103063	TP for DC_21_n28-n79 for TR 37.717-11-21

Type: pCR		For: Approval
					37.717-11-21 v0.1.0
					Source: NTT DOCOMO INC. , MediaTek Inc., LG Electronics

(Replaces R4-2101229)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101585	TP for TR 37.717-11-21:DC_8A_n28A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103064.

R4-2103064	TP for TR 37.717-11-21:DC_8A_n28A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Huawei, HiSilicon

(Replaces R4-2101585)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101586	TP for TR 37.717-11-21:DC_1A-8A_n28A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101587	TP for TR 37.717-11-21:DC_3A-8A_n28A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101588	TP for TR 37.717-11-21:DC_7A-8A_n28A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101589	TP for TR 37.717-11-21:DC_1A-3A-8A_n28A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103065.

R4-2103065	TP for TR 37.717-11-21:DC_1A-3A-8A_n28A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Huawei, HiSilicon

(Replaces R4-2101589)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101590	TP for TR 37.717-11-21:DC_1A-7A-8A_n28A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101591	TP for TR 37.717-11-21:DC_3A-7A-8A_n28A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101592	TP for TR 37.717-11-21:DC_1A-3A-7A-8A_n28A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101593	Updated TP for TR 37.717-11-21:DC_3C-20A_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101594	Updated TP for TR 37.717-11-21:DC_3C-7A-20A_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101897	TP for TR 37.717-11-21 to include DC_3A-28A_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson

Abstract:

TP for TR 37.717-11-21 to include DC_3A-28A_n1A-n78A

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101898	TP for TR 37.717-11-21 to include DC_7A-28A_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson

Abstract:

TP for TR 37.717-11-21 to include DC_7A-28A_n1A-n78A

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101899	TP for TR 37.717-11-21 to include DC_3A-7A-28A_n1A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson

Abstract:

TP for TR 37.717-11-21 to include DC_3A-7A-28A_n1A-n78A

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101900	TP for TR 37.717-11-21 to include DC_1A-7A-28A_n3A-n78A

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson

Abstract:

TP for TR 37.717-11-21 to include DC_1A-7A-28A_n3A-n78A

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102048	draft CR 38.101-3 adding CA_n7B UL configurations for x LTE + 2 NR

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Ericsson

Abstract:

draft CR 38.101-3 adding CA_n7B UL configurations for x LTE + 2 NR

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103066.

R4-2103066	draft CR 38.101-3 adding CA_n7B UL configurations for x LTE + 2 NR

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Ericsson

(Replaces R4-2102048)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was not pursued.

R4-2102052	TP for TR 37.717-11-21 to include 2A_n71a-n78a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 2A_n71a-n78a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102053	TP for TR 37.717-11-21 to include 7A_n71a-n78a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 7A_n71a-n78a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102054	TP for TR 37.717-11-21 to include 66A_n71a-n78a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 66A_n71a-n78a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102055	TP for TR 37.717-11-21 to include 2A_n38a-n71a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 2A_n38a-n71a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102056	TP for TR 37.717-11-21 to include 66A_n38a-n71a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 66A_n38a-n71a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102057	TP for TR 37.717-11-21 to include 71A_n38a-n78a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 71A_n38a-n78a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102058	TP for TR 37.717-11-21 to include 5A_n66a-n78a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 5A_n66a-n78a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102059	TP for TR 37.717-11-21 to include 71A_n66a-n78a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 71A_n66a-n78a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102060	TP for TR 37.717-11-21 to include 5A_n38a-n66a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 5A_n38a-n66a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102061	TP for TR 37.717-11-21 to include 71A_n38a-n66a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 71A_n38a-n66a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102062	TP for TR 37.717-11-21 to include 2A_n2a-n38a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 2A_n2a-n38a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102063	TP for TR 37.717-11-21 to include 66A_n2a-n38a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 66A_n2a-n38a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102064	TP for TR 37.717-11-21 to include 12A_n2a-n38a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 12A_n2a-n38a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102065	TP for TR 37.717-11-21 to include 2A_n2a-n41a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 2A_n2a-n41a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102066	TP for TR 37.717-11-21 to include 12A_n2a-n41a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 12A_n2a-n41a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102067	TP for TR 37.717-11-21 to include 71A_n2a-n41a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 71A_n2a-n41a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102068	TP for TR 37.717-11-21 to include 2A_n2a-n78a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 2A_n2a-n78a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103067.

R4-2103067	TP for TR 37.717-11-21 to include 2A_n2a-n78a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

(Replaces R4-2102068)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102069	TP for TR 37.717-11-21 to include 7A_n2a-n78a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 7A_n2a-n78a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102070	TP for TR 37.717-11-21 to include 71A_n2a-n78a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 71A_n2a-n78a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103068.

R4-2103068	TP for TR 37.717-11-21 to include 71A_n2a-n78a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

(Replaces R4-2102070)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102071	TP for TR 37.717-11-21 to include 2A_n66a-n71a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 2A_n66a-n71a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102072	TP for TR 37.717-11-21 to include 7A_n66a-n71a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 7A_n66a-n71a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102073	TP for TR 37.717-11-21 to include 66A_n66a-n71a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 66A_n66a-n71a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102074	TP for TR 37.717-11-21 to include 2A_n2a-n71a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 2A_n2a-n71a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102075	TP for TR 37.717-11-21 to include 7A_n2a-n71a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 7A_n2a-n71a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102076	TP for TR 37.717-11-21 to include 66A_n2a-n71a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 66A_n2a-n71a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102077	TP for TR 37.717-11-21 to include 2A_n2a-n66a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 2A_n2a-n66a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102078	TP for TR 37.717-11-21 to include 7A_n2a-n66a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 7A_n2a-n66a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102079	TP for TR 37.717-11-21 to include 66A_n2a-n66a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 66A_n2a-n66a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103069.

R4-2103069	TP for TR 37.717-11-21 to include 66A_n2a-n66a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

(Replaces R4-2102079)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102080	TP for TR 37.717-11-21 to include 71A_n2a-n66a

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson, Rogers

Abstract:

TP for TR 37.717-11-21 to include 71A_n2a-n66a

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

#### 9.7.3	EN-DC including NR inter CA with FR2 band [DC_R17_xBLTE_2BNR_yDL2UL-Core]

R4-2101901	TP for TR 37.717-11-21 to include DC_1A-3A-7A-28A_n78A-n257A/G/H/I

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: Ericsson

Abstract:

TP for TR 37.717-11-21 to include DC_1A-3A-7A-28A_n78A-n257A/G/H/I

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102214	TP for 37.717-11-21_ DC_39_n40-n258

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102215	TP for 37.717-11-21_ DC_39_n41-n258

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102216	TP for 37.717-11-21_ DC_39_n79-n258

Type: pCR		For: Approval
					37.717-11-21 v0.2.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

### 9.8	Band combinations for SA NR supplementary uplink (SUL)

#### 9.8.1	Rapporteur Input (WID/TR/CR) [NR_SUL_combos_R17-Core/Per]

R4-2100291	Revised WID on Band combinations for SA NR Supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP)

Type: WID revised		For: Endorsement
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2100292	TR 37.717-00-00 v0.3.0

Type: draft TR		For: Agreement
					37.717-00-00 v0.2.0
					Source: Huawei, HiSilicon

Abstract:

To capture the approved TPs in this meeting

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2100293	CR on Introduction of completed SUL band combinations into TS 38.101-1

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0607  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2100294	CR on Introduction of completed SUL band combinations into TS 38.101-3

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0439  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was withdrawn.

#### 9.8.2	UE RF [NR_SUL_combos_R17-Core]

R4-2101603	TP for TR 37.717-00-00 for CA_n3A_SUL_n78A-n80A

Type: pCR		For: Approval
					37.717-00-00 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103070.

R4-2103070	TP for TR 37.717-00-00 for CA_n3A_SUL_n78A-n80A

Type: pCR		For: Approval
					37.717-00-00 v0.2.0
					Source: Huawei, HiSilicon

(Replaces R4-2101603)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101604	TP for TR 37.717-00-00 for CA_n41A_SUL_n79A-n83A

Type: pCR		For: Approval
					37.717-00-00 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103071.

R4-2103071	TP for TR 37.717-00-00 for CA_n41A_SUL_n79A-n83A

Type: pCR		For: Approval
					37.717-00-00 v0.2.0
					Source: Huawei, HiSilicon

(Replaces R4-2101604)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101605	TP for TR 37.717-00-00 for CA_n79A_SUL_n41A-n83A

Type: pCR		For: Approval
					37.717-00-00 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103072.

R4-2103072	TP for TR 37.717-00-00 for CA_n79A_SUL_n41A-n83A

Type: pCR		For: Approval
					37.717-00-00 v0.2.0
					Source: Huawei, HiSilicon

(Replaces R4-2101605)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101606	DraftCR for 38.101-1 to add configuration for SUL_n41C-n95A

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2101607	DraftCR for 38.101-1 to add configuration for SUL_n79C-n95A

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2101608	TP for TR 37.717-00-00 for SUL_n41A-n97A

Type: pCR		For: Approval
					37.717-00-00 v0.2.0
					Source: Huawei, HiSilicon, CMCC

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2103073	TP for TR 37.717-00-00 for SUL_n41A-n97A

Type: pCR		For: Approval
					37.717-00-00 v0.2.0
					Source: Huawei, HiSilicon, CMCC

(Replaces R4-2101608)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2101609	TP for TR 37.717-00-00 for SUL_n79A-n97A

Type: pCR		For: Approval
					37.717-00-00 v0.2.0
					Source: Huawei, HiSilicon, CMCC

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103074.

R4-2103074	TP for TR 37.717-00-00 for SUL_n79A-n97A

Type: pCR		For: Approval
					37.717-00-00 v0.2.0
					Source: Huawei, HiSilicon, CMCC

(Replaces R4-2101609)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101610	TP for TR 37.717-00-00 for SUL_n41A-n98A

Type: pCR		For: Approval
					37.717-00-00 v0.2.0
					Source: Huawei, HiSilicon, CMCC

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103075.

R4-2103075	TP for TR 37.717-00-00 for SUL_n41A-n98A

Type: pCR		For: Approval
					37.717-00-00 v0.2.0
					Source: Huawei, HiSilicon, CMCC

(Replaces R4-2101610)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101611	TP for TR 37.717-00-00 for SUL_n79A-n98A

Type: pCR		For: Approval
					37.717-00-00 v0.2.0
					Source: Huawei, HiSilicon, CMCC

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

### 9.9	NR Inter-band Carrier Aggregation for 3 bands DL with 1 band UL [NR_CA_R17_3BDL_1BUL]

#### 9.9.1	Rapporteur Input (WID/TR/CR) [NR_CA_R17_3BDL_1BUL-Core/Per]

R4-2100492	TR 38.717-03-01 on Rel-17 NR inter-band Carrier Aggregation (CA) for 3 Down Link (DL) / 1 Up Link (UL)

Type: draft TR		For: Agreement
					38.717-03-01 v0.2.0
					Source: CATT

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2100493	Revised WID on Rel-17 NR inter-band CA of 3DL bands and 1UL band

Type: WID revised		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

#### 9.9.2	UE RF [NR_CA_R17_3BDL_1BUL-Core]

R4-2100093	Draft CR on CA_n1-n3-n78

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: China Unicom

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was not pursued.

R4-2103076	Draft CR on CA_n1-n3-n78

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: China Unicom

(Replaces R4-2100093)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2100494	CR on Introducing NR inter-band CA for 3DL Bands and 1UL band for 38.101-1

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0617  Cat: B (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2100495	CR on Introducing NR inter-band CA for 3DL Bands and 1UL band for 38.101-3

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0442  Cat: B (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2100497	Correction on supported channel bandwidth for n79

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0443  Cat: F (Rel-16)

					Source: CATT

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2100498	Correction on supported channel bandwidth for n79

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0444  Cat: A (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2100683	Correction on supported channel bandwidth for CA_n39-n41-n79

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0626  Cat: F (Rel-16)

					Source: CATT, CMCC

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2100684	Correction on supported channel bandwidth for CA_n39-n41-n79

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0627  Cat: A (Rel-17)

					Source: CATT, CMCC

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2100738	TP to TR 38.717-03-01 CA_n5-n25-n77

Type: other		For: Approval
					38.717-03-01 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, a text proposal to complete 3DL/1UL NR CA configuration CA_n5A-n25A-n77A is provided.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103077.

R4-2103077	TP to TR 38.717-03-01 CA_n5-n25-n77

Type: other		For: Approval
					38.717-03-01 v..
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100738)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100739	TP to TR 38.717-03-01 CA_n25-n66-n77

Type: other		For: Approval
					38.717-03-01 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, a text proposal to complete 3DL/1UL NR CA configuration CA_n25A-n66A-n77A is provided.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103078.

R4-2103078	TP to TR 38.717-03-01 CA_n25-n66-n77

Type: other		For: Approval
					38.717-03-01 v..
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100739)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100952	TP for TR 38.717-03-01: CA_n3-n18-n41

Type: pCR		For: Approval
					38.717-03-01 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100953	TP for TR 38.717-03-01: CA_n3A-n28A-n77(2A)

Type: pCR		For: Approval
					38.717-03-01 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103079.

R4-2103079	TP for TR 38.717-03-01: CA_n3A-n28A-n77(2A)

Type: pCR		For: Approval
					38.717-03-01 v0.2.0
					Source: Samsung, KDDI

(Replaces R4-2100953)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100954	TP for TR 38.717-03-01: CA_n3A-n28A-n78(2A)

Type: pCR		For: Approval
					38.717-03-01 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103080.

R4-2103080	TP for TR 38.717-03-01: CA_n3A-n28A-n78(2A)

Type: pCR		For: Approval
					38.717-03-01 v0.2.0
					Source: Samsung, KDDI

(Replaces R4-2100954)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100955	TP for TR 38.717-03-01: CA_n3A-n41A-n77(2A)

Type: pCR		For: Approval
					38.717-03-01 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100956	TP for TR 38.717-03-01: CA_n3A-n41A-n78(2A)

Type: pCR		For: Approval
					38.717-03-01 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100957	TP for TR 38.717-03-01: CA_n28A-n41A-n77(2A)

Type: pCR		For: Approval
					38.717-03-01 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100958	TP for TR 38.717-03-01: CA_n28A-n41A-n78(2A)

Type: pCR		For: Approval
					38.717-03-01 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100979	TP for TR 38.717-03-01: CA_n13-n25-n66

Type: pCR		For: Approval
					38.717-03-01 v0.2.0
					Source: Samsung, TELUS, Bell mobility

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103081.

R4-2103081	TP for TR 38.717-03-01: CA_n13-n25-n66

Type: pCR		For: Approval
					38.717-03-01 v0.2.0
					Source: Samsung, TELUS, Bell mobility

(Replaces R4-2100979)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100980	TP for TR 38.717-03-01: CA_n25A-n29A-n66A

Type: pCR		For: Approval
					38.717-03-01 v0.2.0
					Source: Samsung, TELUS, Bell mobility

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101113	TP for TR 38.717-03-01: support of CA_n1-n78-n257

Type: pCR		For: Approval
					38.717-03-01 v0.1.0
					Source: CHTTL

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101188	TP for CA_n28-n77-n79 for TR 38.717-03-01

Type: pCR		For: Approval
					38.717-03-01 v0.1.0
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101189	TP for CA_n28-n78-n79 for TR 38.717-03-01

Type: pCR		For: Approval
					38.717-03-01 v0.1.0
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101595	DraftCR for 38.101-1 to add BCS1 for CA_n1A-n8A-n78A

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2101596	TP for TR 38.717-03-01: CA_n1A-n8A-n79A

Type: pCR		For: Approval
					38.717-03-01 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101597	Updated TP for TR 38.717-03-01: to add configuration CA_n1A-n78(2A)-n79A and CA_n1A-n78A-n79A_BCS1

Type: pCR		For: Approval
					38.717-03-01 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101598	TP for TR 38.717-03-01: CA_n8A-n78A-n79A/CA_n8A-n78(2A)-n79A

Type: pCR		For: Approval
					38.717-03-01 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102217	TP for TR38.717-03-01_ CA_n8A-n39A-n41A

Type: pCR		For: Approval
					38.717-03-01 v0.2.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

### 9.10	NR Inter-band Carrier Aggregation for 4 bands DL with 1 band UL [NR_CA_R17_4BDL_1BUL]

#### 9.10.1	Rapporteur Input (WID/TR/CR) [NR_CA_R17_4BDL_1BUL-Core/Per]

R4-2101885	Revised WID 4 bands NR CA Rel-17

Type: WID revised		For: Endorsement
					Source: Ericsson

Abstract:

Revised WID 4 bands NR CA Rel-17

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was not concluded.

R4-2101889	CR introduction completed band combinations NR Inter-band 4 bands CA -

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0658  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

CR introduction completed band combinations NR Inter-band 4 bands CA -> 38.101-1

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was withdrawn.

R4-2101890	CR introduction completed band combinations NR Inter-band 4 bands CA -

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0472  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

CR introduction completed band combinations NR Inter-band 4 bands CA -> 38.101-3

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2101893	TR 38.717-04-01 v0.3.0 Rel-17 NR Inter-band 4 bands CA

Type: draft TR		For: Agreement
					38.717-04-01 v0.2.0
					Source: Ericsson

Abstract:

TR 38.717-04-01 v0.3.0 Rel-17 NR Inter-band 4 bands CA

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 9.10.2	UE RF [NR_CA_R17_4BDL_1BUL-Core]

R4-2101904	TP for TR 38.717-04-01 to include CA_n41-n66-n71-n77

Type: pCR		For: Approval
					38.717-04-01 v0.2.0
					Source: Ericsson, T-Mobile US

Abstract:

TP for TR 38.717-04-01 to include CA_n41-n66-n71-n77

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103082.

R4-2103082	TP for TR 38.717-04-01 to include CA_n41-n66-n71-n77

Type: pCR		For: Approval
					38.717-04-01 v0.2.0
					Source: Ericsson, T-Mobile US

(Replaces R4-2101904)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

### 9.11	NR Inter-band Carrier Aggregation/Dual connectivity for 3 bands DL with 2 bands UL [NR_CADC_R17_3BDL_2BUL]

#### 9.11.1	Rapporteur Input (WID/TR/CR) [NR_CADC_R17_3BDL_2BUL-Core/Per]

R4-2102225	Revised WID on Rel-17 NR Inter-band Carrier AggregationDual Connectivity for 3 bands DL with 2 bands UL

Type: WID revised		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2102226	CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into TS 38.101-1

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2102945	CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into TS 38.101-1

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0716  Cat: - (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102227	CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into TS 38.101-3

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2102946	CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into TS 38.101-3

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0497  Cat: - (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102305	TR 38.717-03-02 v0.3.0

Type: draft TR		For: Agreement
					38.717-03-02 v0.3.0
					Source: ZTE Wistron Telecom AB

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 9.11.2	UE RF [NR_CADC_R17_3BDL_2BUL-Core]

R4-2100153	Draft CR Addition of BCS1 for CA_n25A-n71(2A), CA_n41(2A)-n66A and CA_n66A-n71(2A)

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Nokia, T-Mobile USA

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was not pursued.

R4-2103083	Draft CR Addition of BCS1 for CA_n25A-n71(2A), CA_n41(2A)-n66A and CA_n66A-n71(2A)

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Nokia, T-Mobile USA

(Replaces R4-2100153)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2100157	TP to TR 38.717-03-02: CA_n25-n41-n66

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Nokia, T-Mobile USA

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2100158	TP to TR 38.717-03-02: CA_n25-n41-n71

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Nokia, T-Mobile USA

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103084.

R4-2103084	TP to TR 38.717-03-02: CA_n25-n41-n71

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Nokia, T-Mobile USA

(Replaces R4-2100158)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100698	Draft CR for TS 38.101-1: Support of DC_ n3-n28-n77

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2100700	Draft CR for TS 38.101-3: Support of n77(2A) in DC_ n28-n77-n257

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2100740	TP to TR 38.717-03-02 CA_n5-n25-n77

Type: other		For: Approval
					38.717-03-02 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, a text proposal to complete 3DL/2UL NR CA configuration CA_n5A-n25A-n77A is provided.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103085.

R4-2103085	TP to TR 38.717-03-02 CA_n5-n25-n77

Type: other		For: Approval
					38.717-03-02 v..
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100740)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100741	TP to TR 38.717-03-02 CA_n25-n66-n77

Type: other		For: Approval
					38.717-03-02 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, a text proposal to complete 3DL/2UL NR CA configuration CA_n25A-n66A-n77A is provided.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103100.

R4-2103100	TP to TR 38.717-03-02 CA_n25-n66-n77

Type: other		For: Approval
					38.717-03-02 v..
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100741)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100959	TP for TR 38.717-03-02: CA_n3-n28-n78

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103101.

R4-2103101	TP for TR 38.717-03-02: CA_n3-n28-n78

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

(Replaces R4-2100959)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100960	TP for TR 38.717-03-02: DC_n3-n41-n257

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103086.

R4-2103086	TP for TR 38.717-03-02: DC_n3-n41-n257

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

(Replaces R4-2100960)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100961	TP for TR 38.717-03-02: DC_n28-n41-n257

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2100962	TP for TR 38.717-03-02: DC_n41-n77-n257

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2100963	TP for TR 38.717-03-02: DC_n41-n78-n257

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2100964	TP for TR 38.717-03-02: CA_n3-n18-n41

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103087.

R4-2103087	TP for TR 38.717-03-02: CA_n3-n18-n41

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

(Replaces R4-2100964)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100965	TP for TR 38.717-03-02: CA_n3-n28-n41

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2100966	TP for TR 38.717-03-02: CA_n3-n28-n77

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2100967	TP for TR 38.717-03-02: CA_n3-n41-n77

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103088.

R4-2103088	TP for TR 38.717-03-02: CA_n3-n41-n77

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

(Replaces R4-2100967)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100968	TP for TR 38.717-03-02: CA_n3-n41-n78

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103089.

R4-2103089	TP for TR 38.717-03-02: CA_n3-n41-n78

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

(Replaces R4-2100968)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100969	TP for TR 38.717-03-02: CA_n28-n41-n77

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103090.

R4-2103090	TP for TR 38.717-03-02: CA_n28-n41-n77

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

(Replaces R4-2100969)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100970	TP for TR 38.717-03-02: CA_n28-n41-n78

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103091.

R4-2103091	TP for TR 38.717-03-02: CA_n28-n41-n78

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, KDDI

(Replaces R4-2100970)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100981	TP for TR 38.717-03-02: CA_n25-n29-n66

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, TELUS, Bell mobility

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103092.

R4-2103092	TP for TR 38.717-03-02: CA_n25-n29-n66

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, TELUS, Bell mobility

(Replaces R4-2100981)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100982	TP for TR 38.717-03-02: CA_n13-n25-n66

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, TELUS, Bell mobility

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103093.

R4-2103093	TP for TR 38.717-03-02: CA_n13-n25-n66

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Samsung, TELUS, Bell mobility

(Replaces R4-2100982)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101158	MSD evaluation for CA 3DL2UL n1-n77-n79 for TR 38.717-03-02

Type: other		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was noted.

R4-2101198	draft CR to TS 38.101-1 Modification of MSD values for n1-n77-n79 and n1-n78-n79

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2101513	TP for TR 38.717-03-02: CA_n66-n71-n78

Type: pCR		For: Approval
					38.717-03-01 v0.3.0
					Source: Huawei, HiSilicon, Bell Mobility, Telus

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2101514	TP for TR 38.717-03-02: CA_n38A-n66A-n78A

Type: pCR		For: Approval
					38.717-03-01 v0.3.0
					Source: Huawei, HiSilicon, Bell Mobility, Telus

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103094.

R4-2103094	TP for TR 38.717-03-02: CA_n38A-n66A-n78A

Type: pCR		For: Approval
					38.717-03-01 v0.3.0
					Source: Huawei, HiSilicon, Bell Mobility, Telus

(Replaces R4-2101514)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101515	TP for TR 38.717-03-02: CA_n25A-n38A-n78A

Type: pCR		For: Approval
					38.717-03-01 v0.3.0
					Source: Huawei, HiSilicon, Bell Mobility, Telus

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101902	TP for 38.717-03-02 to include n25-n66-n71

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Ericsson, T-Mobile US

Abstract:

TP for 38.717-03-02 to include n25-n66-n71

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103095.

R4-2103095	TP for 38.717-03-02 to include n25-n66-n71

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Ericsson, T-Mobile US

(Replaces R4-2101902)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101903	TP for 38.717-03-02 to include n41-n66-n71

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Ericsson, T-Mobile US

Abstract:

TP for 38.717-03-02 to include n41-n66-n71

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102316	TP for 38.717-03-02 to include n25-n41-n66

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Ericsson, Bell Mobility

Abstract:

TP for 38.717-03-02 to include n25-n41-n66

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was revised to R4-2103099.

R4-2103099	TP for 38.717-03-02 to include n25-n41-n66

Type: pCR		For: Approval
					38.717-03-02 v0.2.0
					Source: Ericsson, Bell Mobility

(Replaces R4-2102316)

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

### 9.12	DC of x bands (x=1,2) LTE inter-band CA (xDL/xUL) and y bands (y=3-x) NR inter-band CA [DC_R17_xBLTE_yBNR_3DL3UL]

#### 9.12.1	Rapporteur Input (WID/TR/CR) [DC_R17_xBLTE_yBNR_3DL3UL-Core/Per]

R4-2102228	Revised WID on Rel-17 Dual Connectivity (DC) x bands (x=1,2) LTE inter-band CA (xDL/xUL) and y bands (y=3-x) NR inter-band CA

Type: WID revised		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2102229	CR to reflect the completed ENDC combinations for 3 bands DL with 3 bands UL into TS 38.101-3

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2102947	CR to reflect the completed ENDC combinations for 3 bands DL with 3 bands UL into TS 38.101-3

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0498  Cat: - (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102230	TR 37.717-33 v0.2.0

Type: draft TR		For: Agreement
					37.717-33 v0.1.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 9.12.2	UE RF [DC_R17_xBLTE_yBNR_3DL3UL-Core]

R4-2102218	TP for TR 37.717-33_DC_40A_n41A-n258A

Type: pCR		For: Approval
					37.717-33 v0.1.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

### 9.13	DC of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL) [DC_R17_xBLTE_3BNR_yDL2UL]

#### 9.13.1	Rapporteur Input (WID/TR/CR) [DC_R17_xBLTE_3BNR_yDL2UL -Core/Per]

R4-2102231	Revised WID on Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL1UL) and 3 bands NR inter-band CA (3DL1UL)

Type: WID revised		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2102232	TR 37.717-11-31_v0.3.0

Type: draft TR		For: Agreement
					37.717-11-31 v0.2.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102948	CR to reflect the completed Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL1UL) and 3 bands NR inter-band CA (3DL1UL) into TS38.101-3

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0499  Cat: B (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 9.13.2	UE RF [DC_R17_xBLTE_3BNR_yDL2UL-Core]

R4-2100669	TP for TR 37.717-11-31: EN-DC_11_n3-n28-n77

Type: pCR		For: Approval
					37.717-11-31 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100670	TP for TR 37.717-11-31: EN-DC_42_n3-n28-n77

Type: pCR		For: Approval
					37.717-11-31 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100681	TP for TR 37.717-11-31: EN-DC_1-8_n3-n28-n77

Type: pCR		For: Approval
					37.717-11-31 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100682	TP for TR 37.717-11-31: EN-DC_1-11_n3-n28-n77

Type: pCR		For: Approval
					37.717-11-31 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100685	TP for TR 37.717-11-31: EN-DC_1-42_n3-n28-n77

Type: pCR		For: Approval
					37.717-11-31 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100686	TP for TR 37.717-11-31: EN-DC_8-11_n3-n28-n77

Type: pCR		For: Approval
					37.717-11-31 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100687	TP for TR 37.717-11-31: EN-DC_8-42_n3-n28-n77

Type: pCR		For: Approval
					37.717-11-31 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100690	TP for TR 37.717-11-31: EN-DC_1-8-11_n3-n28-n77

Type: pCR		For: Approval
					37.717-11-31 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100691	TP for TR 37.717-11-31: EN-DC_1-8-42_n3-n28-n77

Type: pCR		For: Approval
					37.717-11-31 v0.1.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101019	TP for DC_1A_n28A-n77A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101020	TP for DC_1A_n28A-n78A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101021	TP for DC_3A_n1A-n77A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101022	TP for DC_3A_n1A-n78A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101023	TP for DC_3A_n28A-n77A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101024	TP for DC_3A_n28A-n78A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101025	TP for DC_19A_n1A-n77A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101026	TP for DC_19A_n1A-n78A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101027	TP for DC_21A_n1A-n77A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101028	TP for DC_21A_n1A-n78A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101029	TP for DC_21A_n28A-n77A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101030	TP for DC_21A_n28A-n78A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101031	TP for DC_1A-3A_n28A-n77A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101032	TP for DC_1A-3A_n28A-n78A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101033	TP for DC_1A-21A_n28A-n77A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101034	TP for DC_1A-21A_n28A-n78A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101035	TP for DC_3A-21A_n1A-n77A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101036	TP for DC_3A-21A_n1A-n78A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101037	TP for DC_3A-21A_n28A-n77A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101038	TP for DC_3A-21A_n28A-n78A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101039	TP for DC_19A-42A_n1A-n77A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101040	TP for DC_19A-42A_n1A-n78A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101186	TP for DC_42A_n1A-n77A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.1.0
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101187	TP for DC_42A_n1A-n78A-n79A for TR37.717-11-31

Type: pCR		For: Approval
					37.717-11-31 v0.1.0
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2102219	TP for 37.717-11-31_ DC_8A_n39A-n40A-n41A

Type: pCR		For: Approval
					37.717-11-31 v0.2.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

### 9.14	NR inter-band Carrier Aggregation and Dual connectivity for DL 4 bands and 2UL bands [NR_CADC_R17_4BDL_2BUL]

#### 9.14.1	Rapporteur Input (WID/TR/CR) [NR_CADC_R17_4BDL_2BUL -Core/Per]

R4-2100990	CR on introduction of completed NR CA/DC combs with 4DL/2UL within FR1

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0636  Cat: B (Rel-17)

					Source: Samsung

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was revised to R4-2103255.

R4-2103255	CR on introduction of completed NR CA/DC combs with 4DL/2UL within FR1

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0636 rev 1 Cat: B (Rel-17)

					Source: Samsung

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2100991	CR on introduction of completed NR CA/DC combs with 4DL/2UL including FR2

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0454  Cat: B (Rel-17)

					Source: Samsung

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was revised to R4-2103407.

R4-2103407	CR on introduction of completed NR CA/DC combs with 4DL/2UL including FR2

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0454 rev 1  Cat: B (Rel-17)

					Source: Samsung

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2100992	Revised WID on NR CA/DC with 4DL/2UL

Type: WID revised		For: Information
					Source: Samsung

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2101482	TR38.717-04-02 update version 0.3.0

Type: draft TR		For: Agreement
					38.717-04-02 v0.2.0
					Source: Samsung R&D Institute India

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 9.14.2	UE RF [NR_CADC_R17_4BDL_2BUL -Core]

R4-2100701	Draft CR for TS 38.101-3: Support of UL CA in CA_n3-n28-n77-n257

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was endorsed.

R4-2100971	TP for TR 38.717-04-02: CA_n3-n28-n41-n77

Type: pCR		For: Approval
					38.717-04-02 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2100972	TP for TR 38.717-04-02: CA_n3-n28-n41-n78

Type: pCR		For: Approval
					38.717-04-02 v0.2.0
					Source: Samsung, KDDI

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101905	TP for TR 38.717-04-02 to include CA_n41-n66-n71-n77

Type: pCR		For: Approval
					38.717-04-02 v0.2.0
					Source: Ericsson, T-Mobile US

Abstract:

TP for TR 38.717-04-02 to include CA_n41-n66-n71-n77

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

R4-2101906	TP for TR 38.717-04-02 to include CA_n25-n41-n66-n71

Type: pCR		For: Approval
					38.717-04-02 v0.2.0
					Source: Ericsson, T-Mobile US

Abstract:

TP for TR 38.717-04-02 to include CA_n25-n41-n66-n71

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was approved.

### 9.15	NR inter-band CA for 5 bands DL with x bands UL (x=1, 2) [NR_CADC_R17_5BDL_xBUL_3DL3UL]

#### 9.15.1	Rapporteur Input (WID/TR/CR) [NR_CADC_R17_5BDL_xBUL -Core/Per]

R4-2100295	Revised WID on NR inter-band CA for 5 bands DL with x bands UL (x=1, 2)

Type: WID revised		For: Endorsement
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2100296	TR 38.717-05-01 v0.2.0

Type: draft TR		For: Agreement
					38.717-05-01 v0.1.0
					Source: Huawei, HiSilicon

Abstract:

To capture the approved TPs in this meeting

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2100297	CR on Introduction of completed 5 bands inter-band CA into TS 38.101-1

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0608  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

#### 9.15.2	UE RF [NR_CADC_R17_5BDL_xBUL -Core]

### 9.16	DC of 5 bands LTE inter-band CA (5DL/1L) and 1 NR band (1DL/1UL) [DC_R17_5BLTE_1BNR_6DL2UL]

#### 9.16.1	Rapporteur Input (WID/TR/CR) [DC_R17_5BLTE_1BNR_6DL2UL-Core/Per]

R4-2100988	CR introduction completed band combinations for Dual Connectivity (DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL)

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0453  Cat: B (Rel-17)

					Source: Samsung

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2100989	Revised WID on Dual Connectivity (DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL)

Type: WID revised		For: Information
					Source: Samsung

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2101483	TR 37.717-51-11 update version 0.2.0

Type: draft TR		For: Agreement
					37.717-51-11 v0.1.0
					Source: Samsung R&D Institute India

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was withdrawn.

#### 9.16.2	UE RF [DC_R17_5BLTE_1BNR_6DL2UL-Core]

R4-2100324	Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +5LTE band) within FR1 DC_1A-3A-7A-8A-40A_n78(2A)/DC_1A-3A-7A-8A-40C_n78(2A)

Type: draftCR		For: Endorsement
					38.101-3 v17.0.0
					Source: Huawei,HiSilicon

Abstract:

3HK's new band combination

Discussion:

See email discussion summary for [98e][115] NR_Baskets_Part_1 in R4-2102963.

Decision: 		The document was endorsed.

### 9.17	DC of x bands (x=2,3,4) LTE inter-band CA (xDL/1UL) and 1 NR FR1 band (1DL/1UL) and 1 NR FR2 band (1DL/1UL) [DC_R17_xBLTE_2BNR_yDL3UL]

#### 9.17.1	Rapporteur Input (WID/TR/CR) [DC_R17_xBLTE_2BNR_yDL3UL-Core/Per]

R4-2100986	CR introduction completed band combinations for Dual Connectivity (DC) of x bands (x=2,3,4) LTE inter-band CA (xDL/1UL) and 1 NR FR1 band (1DL/1UL) and 1 NR FR2 band (1DL/1UL)

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0452  Cat: B (Rel-17)

					Source: Samsung

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2100987	Revised WID on Dual Connectivity (DC) of x bands (x=2,3,4) LTE inter-band CA (xDL/1UL) and 1 NR FR1 band (1DL/1UL) and 1 NR FR2 band (1DL/1UL)

Type: WID revised		For: Information
					Source: Samsung

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

R4-2101486	TR 37.717-21-22 update version 0.2.0

Type: draft TR		For: Agreement
					37.717-21-22 v0.1.0
					Source: Samsung R&D Institute India

Discussion:

See email discussion summary for [98e][116] NR_Baskets_Part_2 in R4-2102964.

Decision: 		The document was withdrawn.

#### 9.17.2	UE RF [DC_R17_xBLTE_2BNR_yDL3UL-Core]

### 9.18	SAR schemes for UE power class 2 (PC2) for NR inter-band Carrier Aggregation and supplemental uplink (SUL) configurations with 2 bands UL [NR_SAR_PC2_interB_SUL_2BUL]

R4-2102965	Email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL

Type: other		For: discussion
					Source: Moderator (China Telecom)

Discussion:

The contribution summarised email discussion thread [98e][117] NR_SAR_PC2_interB_SUL_2BUL. The discussion thread was moderated by Bo Liu (China Telecom Corporation Ltd.).

Decision: 		The document was revised to R4-2103305.

R4-2103305	Email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL

Type: other		For: discussion
					Source: Moderator (China Telecom)

(Replaces R4-2102965)

Discussion:

The contribution summarised email discussion thread [98e][117] NR_SAR_PC2_interB_SUL_2BUL. The discussion thread was moderated by Bo Liu (China Telecom Corporation Ltd.).

Decision: 		The document was noted.

R4-2103172	Way forward on SAR solutions for PC2 NR inter-band CA and SUL configurations

Type: other		For: discussion
					Source: China Telecom

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was approved.

R4-2103174	Way forward on increasing UE maximum power for UE equipped with two PAs

Type: other		For: discussion
					Source: Qualcomm

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

#### 9.18.1	General and Rapporteur Input (WID/TR/CR) [NR_SAR_PC2_interB_SUL_2BUL-Core/Per]

R4-2101109	MSD analysis on high power UE for CA_n41-n79

Type: discussion		For: Approval
					Source: Xiaomi, ZTE Corporation

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was approved.

R4-2101122	CR to 38.307 Release independence for UE power class 2 NR inter-band CA and SUL configurations (R15)

Type: CR		For: Agreement
					38.307 v15.7.0	  CR-0049  Cat: B (Rel-15)

					Source: China Telecom

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was not pursued.

R4-2101123	CR to 38.307 Release independence for UE power class 2 NR inter-band CA and SUL configurations (R16)

Type: CR		For: Agreement
					38.307 v16.5.0	  CR-0050  Cat: B (Rel-16)

					Source: China Telecom

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was not pursued.

R4-2101124	CR to 38.307 Release independence for UE power class 2 NR inter-band CA and SUL configurations (R17)

Type: CR		For: Agreement
					38.307 v17.0.0	  CR-0051  Cat: B (Rel-17)

					Source: China Telecom

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was revised to R4-2103173.

R4-2103173	CR to 38.307 Release independence for UE power class 2 NR inter-band CA and SUL configurations (R17)

Type: CR		For: Agreement
					38.307 v17.0.0	  CR-0051  rev 1 Cat: B (Rel-17)

					Source: China Telecom

(Replaces R4-2101124)

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was agreed.

#### 9.18.2	PC2 for inter-band CA [NR_SAR_PC2_interB_SUL_2BUL-Core]

R4-2100100	Discussion on SAR issues for PC2 NR inter-band CA and SUL configurations

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

R4-2101101	Discussion on SAR issue for HP UE inter-band UL CA

Type: other		For: Approval
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

R4-2101117	Discussion on SAR schemes for UE power class 2 NR inter-band CA with 2UL

Type: discussion		For: Discussion
					Source: China Telecom

Abstract:

.

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

R4-2101119	draft CR to 38.101-1 Introduce SAR solution for UE power class 2 NR inter-band CA with 2UL

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: China Telecom

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was not pursued.

R4-2101726	Methods for faciliating SAR compliance for inter-band UL CA

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we propose that duty-cycle reporting is not specified. Power limits combined with the P-MPR method should be used instead. We also discuss power prioritization and PHR for HPUE.

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

R4-2101752	Discussion on inter-band CA HPUE SAR

Type: discussion		For: Approval
					Source: OPPO

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

R4-2102139	Further discussion on SAR solution for NR PC2 inter-band CA

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

R4-2102190	Further discussion on SAR solution for NR PC2 inter-band CA

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

R4-2102287	On the SAR solutions for UL CA band combinations

Type: discussion		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

R4-2102712	Discussion on SAR solution for PC2 inter-band NR CA

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

#### 9.18.3	PC2 for SUL [NR_SAR_PC2_interB_SUL_2BUL-Core]

R4-2101102	Discussion on SAR issue for NR PC2 SUL

Type: other		For: Approval
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

R4-2101118	Discussion on SAR schemes for UE power class 2 NR SUL configurations

Type: discussion		For: Discussion
					Source: China Telecom

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

R4-2101120	draft CR to 38.101-1 Introduce SAR solution for UE power class 2 NR SUL configurations

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: China Telecom

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was not pursued.

R4-2102140	Further discussion on SAR solution for NR PC2 SUL

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

R4-2102191	Further discussion on SAR solution for NR PC2 SUL

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

R4-2102289	On the SAR solutions for SUL band combinations

Type: discussion		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

#### 9.18.4	Others [NR_SAR_PC2_interB_SUL_2BUL-Core]

R4-2100372	Discussion on inter-band 2UL CA Pcmax upper limit

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

R4-2101121	Discussion on release independence for UE power class 2 NR inter-band CA and SUL configurations

Type: discussion		For: Discussion
					Source: China Telecom

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

R4-2102414	Increasing UE maximum output power

Type: discussion		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL in R4-2103305.

Decision: 		The document was noted.

### 9.19	High power UE (power class 2) for NR inter-band Carrier Aggregation with 2 bands downlink and 2 bands uplink [NR_PC2_CA_R17_2BDL_2BUL]

R4-2102966	Email discussion summary for [98e][118] NR_PC2_CA_R17_2BDL_2BUL

Type: other		For: discussion
					Source: Moderator (China Telecom)

Discussion:

The contribution summarised email discussion thread [98e][118] NR_PC2_CA_R17_2BDL_2BUL. The discussion thread was moderated by Bo Liu (China Telecom Corporation Ltd.).

Decision: 		The document was revised to R4-2103306.

R4-2103306	Email discussion summary for [98e][118] NR_PC2_CA_R17_2BDL_2BUL

Type: other		For: discussion
					Source: Moderator (China Telecom)

(Replaces R4-2102966)

Discussion:

The contribution summarised email discussion thread [98e][118] NR_PC2_CA_R17_2BDL_2BUL. The discussion thread was moderated by Bo Liu (China Telecom Corporation Ltd.).

Decision: 		The document was noted.

#### 9.19.1	Rapporteur Input (WID/TR/CR) [NR_PC2_CA_R17_2BDL_2BUL-Core/Per]

R4-2101125	Draft TR 38.xxx v0.2.0: High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x =1,2)

Type: other		For: Agreement
					Source: China Telecom

Discussion:

See email discussion summary for [98e][118] NR_PC2_CA_R17_2BDL_2BUL in R4-2103306. The document was subject for post-meeting email agreement.

Decision: 		The document was not concluded.

R4-2101126	Revised WID: High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x =1,2)

Type: WID revised		For: Endorsement
					Source: China Telecom

Abstract:

update the WI code according to MCC suggestion and the target competion time

Discussion:

See email discussion summary for [98e][118] NR_PC2_CA_R17_2BDL_2BUL in R4-2103306.

Decision: 		The document was endorsed.

#### 9.19.2	UE RF [NR_PC2_CA_R17_2BDL_2BUL-Core]

R4-2100273	TP for TR38.xxx for PC2 CA_n2A-n77A

Type: discussion		For: Approval
					Source: Verizon Denmark

Discussion:

See email discussion summary for [98e][118] NR_PC2_CA_R17_2BDL_2BUL in R4-2103306.

Decision: 		The document was revised to R4-2103169.

R4-2103169	TP for TR38.xxx for PC2 CA_n2A-n77A

Type: discussion		For: Approval
					Source: Verizon Denmark

(Replaces R4-2100273)

Discussion:

See email discussion summary for [98e][118] NR_PC2_CA_R17_2BDL_2BUL in R4-2103306.

Decision: 		The document was approved.

R4-2100274	TP for TR38.xxx for PC2 CA_n5A-n77A

Type: discussion		For: Approval
					Source: Verizon Denmark

Discussion:

See email discussion summary for [98e][118] NR_PC2_CA_R17_2BDL_2BUL in R4-2103306.

Decision: 		The document was revised to R4-2103170.

R4-2103170	TP for TR38.xxx for PC2 CA_n5A-n77A

Type: discussion		For: Approval
					Source: Verizon Denmark

(Replaces R4-2100274)

Discussion:

See email discussion summary for [98e][118] NR_PC2_CA_R17_2BDL_2BUL in R4-2103306.

Decision: 		The document was approved.

R4-2100276	TP for TR38.xxx for PC2 CA_n66A-n77A

Type: discussion		For: Approval
					Source: Verizon Denmark

Discussion:

See email discussion summary for [98e][118] NR_PC2_CA_R17_2BDL_2BUL in R4-2103306.

Decision: 		The document was revised to R4-2103171.

R4-2103171	TP for TR38.xxx for PC2 CA_n66A-n77A

Type: discussion		For: Approval
					Source: Verizon Denmark

(Replaces R4-2100276)

Discussion:

See email discussion summary for [98e][118] NR_PC2_CA_R17_2BDL_2BUL in R4-2103306.

Decision: 		The document was approved.

R4-2100285	Self interference analysis and MSD results for PC2 NR inter-band CA band combinations

Type: other		For: Approval
					Source: LG Electronics France

Abstract:

Provide MSD results for PC2 NR inter-band CA band combos

Discussion:

See email discussion summary for [98e][118] NR_PC2_CA_R17_2BDL_2BUL in R4-2103306.

Decision: 		The document was noted.

R4-2102220	TP for TR38.xxx_Clarification on PC2 CA_n28A-n41A, CA_n28-n79A and CA_n40A-41A

Type: other		For: Approval
					Source: ZTE Corporation, CMCC

Discussion:

See email discussion summary for [98e][118] NR_PC2_CA_R17_2BDL_2BUL in R4-2103306.

Decision: 		The document was approved.

R4-2102221	TP for TR38.xxx_ PC2 CA_n41A-n79A

Type: other		For: Approval
					Source: ZTE Corporation, CMCC, Xiaomi

Discussion:

See email discussion summary for [98e][118] NR_PC2_CA_R17_2BDL_2BUL in R4-2103306.

Decision: 		The document was noted.

R4-2102713	Discussion on PC2 MSD for UL CA

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][118] NR_PC2_CA_R17_2BDL_2BUL in R4-2103306.

Decision: 		The document was noted.

### 9.20	High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band [ENDC_UE_PC2_R17_NR_TDD]

R4-2102967	Email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD

Type: other		For: discussion
					Source: Moderator (China Unicom)

Discussion:

The contribution summarised email discussion thread [98e][119] ENDC_UE_PC2_R17_NR_TDD. The discussion thread was moderated by Basaier Jialade (China Unicom).

Decision: 		The document was revised to R4-2103307.

R4-2103307	Email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD

Type: other		For: discussion
					Source: Moderator (China Unicom)

(Replaces R4-2102967)

Discussion:

The contribution summarised email discussion thread [98e][119] ENDC_UE_PC2_R17_NR_TDD. The discussion thread was moderated by Basaier Jialade (China Unicom).

Decision: 		The document was noted.

#### 9.20.1	Rapporteur Input (WID/TR/CR) [ENDC_UE_PC2_R17_NR_TDD -Core/Per]

R4-2100082	Big CR on introduction of completed PC2 for EN-DC with 1 LTE band + 1 NR TDD band

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0432  Cat: B (Rel-17)

					Source: China Unicom

Discussion:

See email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD in R4-2103307. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2100083	Revised WID on High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band

Type: WID revised		For: Endorsement
					Source: China Unicom

Discussion:

See email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD in R4-2103307. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed

R4-2100084	TR 37.826 v0.2.0 ENDC_UE_PC2_R17_NR_TDD

Type: draft TR		For: Agreement
					37.826 v0.2.0
					Source: China Unicom

Discussion:

See email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD in R4-2103307. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 9.20.2	UE RF [ENDC_UE_PC2_R17_NR_TDD -Core]

R4-2100266	TP for TR 37.826 for DC_2_n77

Type: discussion		For: Approval
					Source: Verizon Denmark

Discussion:

See email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD in R4-2103307.

Decision: 		The document was revised to R4-2103175.

R4-2103175	TP for TR 37.826 for DC_2_n77

Type: discussion		For: Approval
					Source: Verizon Denmark

(Replaces R4-2100266)

Discussion:

See email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD in R4-2103307.

Decision: 		The document was approved.

R4-2100268	TP for TR 37.826 for DC_5_n77

Type: discussion		For: Approval
					Source: Verizon Denmark

Discussion:

See email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD in R4-2103307.

Decision: 		The document was revised to R4-2103176.

R4-2103176	TP for TR 37.826 for DC_5_n77

Type: discussion		For: Approval
					Source: Verizon Denmark

(Replaces R4-2100268)

Discussion:

See email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD in R4-2103307.

Decision: 		The document was approved.

R4-2100269	TP for TR 37.826 for DC_13_n77

Type: discussion		For: Approval
					Source: Verizon Denmark

Discussion:

See email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD in R4-2103307.

Decision: 		The document was revised to R4-2103177.

R4-2103177	TP for TR 37.826 for DC_13_n77

Type: discussion		For: Approval
					Source: Verizon Denmark

(Replaces R4-2100269)

Discussion:

See email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD in R4-2103307.

Decision: 		The document was approved.

R4-2100271	TP for TR 37.826 for DC_66_n77

Type: discussion		For: Approval
					Source: Verizon Denmark

Discussion:

See email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD in R4-2103307.

Decision: 		The document was revised to R4-2103178.

R4-2103178	TP for TR 37.826 for DC_66_n77

Type: discussion		For: Approval
					Source: Verizon Denmark

(Replaces R4-2100271)

Discussion:

See email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD in R4-2103307.

Decision: 		The document was approved.

R4-2100286	MSD for PC2 high power NR DC (with 1 LTE FDD band + 1 NR TDD band) UE

Type: other		For: Approval
					Source: LG Electronics France

Abstract:

Provide MSD results for PC2 NR DC (with 1 LTE FDD band + 1 NR TDD band) UE in Rel-17

Discussion:

See email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD in R4-2103307.

Decision: 		The document was noted.

R4-2101184	Discussion on UE capability for improved PC2 MSD for EN-DC

Type: discussion		For: Approval
					Source: CHTTL

Discussion:

See email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD in R4-2103307.

Decision: 		The document was noted.

R4-2102415	MSD for PC2 EN-DC and UL CA

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD in R4-2103307.

Decision: 		The document was noted.

### 9.21	Adding channel bandwidth support to existing NR bands [NR_bands_R17_BWs]

R4-2102968	Email discussion summary for [98e][120] NR_bands_R17_BWs

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][120] NR_bands_R17_BWs. The discussion thread was moderated by Dominique Evereare (Ericsson).

Decision: 		The document was revised to R4-2103308.

R4-2103308	Email discussion summary for [98e][120] NR_bands_R17_BWs

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2102968)

Discussion:

The contribution summarised email discussion thread [98e][120] NR_bands_R17_BWs. The discussion thread was moderated by Dominique Evereare (Ericsson).

Decision: 		The document was noted.

R4-2103181	Way forward on adding 90 and 100MHz channel BW for UE in band n40

Type: other		For: discussion
					Source: Huawei

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was approved.

R4-2103182	Draft CR to TS 38.101-1

Type: draftCR		For: Endorsement
					Source: Nokia

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was endorsed.

R4-2103183	Draft CR to TS 38.104

Type: draftCR		For: Endorsement
					Source: Nokia

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was endorsed.

R4-2103184	Way forward on the introduction of 25, 30 and 40MHz in band n2

Type: other		For: discussion
					Source: AT&T

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was approved.

R4-2103185	Way forward on the introduction of 25 MHz in band n5

Type: other		For: discussion
					Source: AT&T

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was approved.

#### 9.21.1	General and Rapporteur Input (WID/TR/CR) [NR_bands_R17_BWs -Core/Per]

R4-2102163	Revised Basket WID on adding channel bandwidth support to existing NR bands

Type: WID revised		For: Endorsement
					Source: Ericsson

Abstract:

This contribution is the revision of the basket WI to include the new requests received before RAN4#97e meeting and update status of previous requests

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was revised to R4-2103179.

R4-2103179	Revised Basket WID on adding channel bandwidth support to existing NR bands

Type: WID revised		For: Endorsement
					Source: Ericsson

(Replaces R4-2102163)

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was endorsed.

R4-2102164	Big CR to TS 38.104 - New CBW Basket WI

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0291  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

This big CR collects all draft CRs to TS 38.104 endorsed in the scope of the new CBW basket WI

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102165	Big CR to TS 38.101-1 - New CBW Basket WI

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0670  Cat: F (Rel-17)

					Source: Ericsson

Abstract:

This big CR collects all draft CRs to TS 38.101-1 endorsed in the scope of the new CBW basket WI

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was revised to R4-2103180.

R4-2103180	Big CR to TS 38.101-1 - New CBW Basket WI

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0670  rev 1 Cat: F (Rel-17)

					Source: Ericsson

(Replaces R4-2102165)

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed

R4-2102168	Basket WID on new CBW - Rapporteur

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution is giving an update and notify about parallel discussion on the MSD analysis for CA combinations for the new CBW

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was noted.

#### 9.21.2	UE RF requirement [NR_bands_R17_BWs -Core]

R4-2101521	Adding 90 and 100MHz bandwidth for band n40

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was noted.

##### 9.21.2.1	Reference sensitivity  [NR_bands_R17_BWs -Core]

R4-2101815	Discussion on the larger channel bandwidth for band n2

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was noted.

R4-2102166	New channel BW in bands n2, n5 and n48 - A-MPR

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution is discussing the A-MPR impact when introducing the request channel BW in bands n2, n5 and n48

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was noted.

##### 9.21.2.2	MPR/A-MPR/NS signaling [NR_bands_R17_BWs -Core]

R4-2100132	n48 30 MHz A-MPR simulation results

Type: discussion		For: Discussion
					Source: Nokia

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was noted.

R4-2100133	CR introduction of 30 MHz for n48

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0592  Cat: B (Rel-17)

					Source: Nokia, Dish Network, Skyworks Inc

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was not pursued.

R4-2102167	New channel BW in bands n2, n5 and n48 - REFSENS

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution is discussing the UE REFSENS impact when introducing the request channel BW in bands n2, n5 and n48

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was noted.

R4-2102929	UE-UE Coexistence for Asynchronous n40 n41 Networks

Type: discussion		For: Approval
					38.101-1 v..
					Source: Skyworks Solutions Inc.

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was noted.

R4-2102931	UE-UE Coexistence for Asynchronous n40 n41 Networks

Type: discussion		For: Approval
					38.101-1 v..
					Source: Skyworks Solutions Inc.

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was noted.

R4-2102932	n48 30MHz A-MPR Measurements

Type: discussion		For: Approval
					38.101-1 v..
					Source: Skyworks Solutions Inc.

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was noted.

##### 9.21.2.3	others [NR_bands_R17_BWs -Core]

R4-2100166	Co-existence challenges with NR-U 100MHz channel bandwidth and other technologies

Type: Work Plan		For: Approval
					Source: Charter Communications, Inc

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was noted.

R4-2102900	NR-U Punctured Channel SEM for 100 MHz Bandwidth

Type: discussion		For: Approval
					Source: CableLabs

Discussion:

See email discussion summary for [98e][120] NR_bands_R17_BWs in R4-2103308.

Decision: 		The document was noted.

#### 9.21.3	BS RF requirement [NR_bands_R17_BWs -Core]

### 9.22	Introduction of channel bandwidths 35MHz and 45MHz for NR [NR_FR1_35MHz_45MHz_BW]

R4-2102969	Email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][121] NR_FR1_35MHz_45MHz_BW. The discussion thread was moderated by Liehai Liu (Huawei Tech.(UK) Co., Ltd).

Decision: 		The document was revised to R4-2103309.

R4-2103309	Email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW

Type: other		For: discussion
					Source: Moderator (Huawei)

(Replaces R4-2102969)

Discussion:

The contribution summarised email discussion thread [98e][121] NR_FR1_35MHz_45MHz_BW. The discussion thread was moderated by Liehai Liu (Huawei Tech.(UK) Co., Ltd).

Decision: 		The document was noted.

R4-2103186	Way forward on spectrum utilization for 35MHz and 45MHz

Type: other		For: discussion
					Source: Huawei

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was approved.

R4-2103187	Way forward on REFSENS for 35MHz and 45MHz

Type: other		For: discussion
					Source: Qualcomm

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was approved.

R4-2103188	Way forward on A-MPR for 35MHz and 45MHz

Type: other		For: discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was approved.

#### 9.22.1	General and Rapporteur Input (WID/TR/CR) [NR_FR1_35MHz_45MHz_BW-Core/Per]

R4-2101501	Consideration on the work plan on introduction of channel bandwidths 35MHz and 45MHz

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was noted.

#### 9.22.2	Spectrum utilization [NR_FR1_35MHz_45MHz_BW-Core]

R4-2100753	Spectrum utilization with channel raster and PRB grid alignment

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was noted.

R4-2102141	Further discussion on spectrum utilization for 35MHz and 45MHz

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was noted.

R4-2102192	Further discussion on spectrum utilization for 35MHz and 45MHz

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was noted.

#### 9.22.3	UE RF requirements [NR_FR1_35MHz_45MHz_BW-Core]

R4-2100516	A-MPR Proposal for n1 and 45MHz CBW

Type: discussion		For: Approval
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was noted.

R4-2100517	A-MPR Proposal for n2 and 35MHz CBW

Type: discussion		For: Approval
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was noted.

R4-2100518	A-MPR Proposal for n25 and 45MHz CBW

Type: discussion		For: Approval
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was noted.

R4-2100703	REFSENS of n8 and n71 for 35MHz channel bandwidth

Type: discussion		For: Approval
					Source: Murata Manufacturing Co Ltd.

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was noted.

R4-2100705	REFSENS of n25 for 45MHz channel bandwidth

Type: discussion		For: Approval
					Source: Murata Manufacturing Co Ltd.

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was noted.

R4-2101159	REFSENS evaluation of n8 and n71 for 35MHz channel bandwidth

Type: discussion		For: Approval
					38.101-1 v..
					Source: Mediatek India Technology Pvt.

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was noted.

R4-2101177	35MHz 45MHz AMPR, MPR, REFSENS for n8, n71, and n25.

Type: other		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was noted.

R4-2101502	UE REFSENS for 35 MHz and 45 MHz

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was noted.

R4-2101503	CR for TS 38.101: introduction of channel bandwidths 35MHz and 45MHz

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was not pursued.

R4-2102142	Introduction of 35MHz and 45 MHz bandwidths to TS38.101-1

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0666  Cat: B (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was withdrawn.

R4-2102193	Introduction of 35MHz and 45 MHz bandwidths to TS38.101-1

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0671  Cat: B (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was revised to R4-2103189.

R4-2103189	Introduction of 35MHz and 45 MHz bandwidths to TS38.101-1

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0671  rev 1 Cat: B (Rel-17)

					Source: ZTE Corporation

(Replaces R4-2102193)

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was noted.

R4-2102592	MSD considering asymmetric UL/DL for bands n8 and n71

Type: discussion		For: Approval
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was noted.

R4-2102606	CR for TS 38.101-1: UE RF requirements table simplification

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0704  Cat: F (Rel-17)

					Source: Apple

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was not pursued.

R4-2102927	35MHz 45MHz REFSENS

Type: discussion		For: Approval
					Source: Skyworks Solutions Inc.

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was noted.

#### 9.22.4	BS RF requirements [NR_FR1_35MHz_45MHz_BW-Core]

R4-2101504	CR for TS 38.104: introduction of channel bandwidths 35MHz and 45MHz

Type: draftCR		For: Endorsement
					38.104 v17.0.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was revised to R4-2103190.

R4-2103190	CR for TS 38.104: introduction of channel bandwidths 35MHz and 45MHz

Type: draftCR		For: Endorsement
					38.104 v17.0.0
					Source: Huawei, HiSilicon

(Replaces R4-2101504)

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was endorsed.

R4-2101505	CR for TS 37.141: introduction of channel bandwidths 35MHz and 45MHz

Type: draftCR		For: Endorsement
					37.141 v17.0.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was revised to R4-2103191.

R4-2103191	CR for TS 37.141: introduction of channel bandwidths 35MHz and 45MHz

Type: draftCR		For: Endorsement
					37.141 v17.0.0
					Source: Huawei, HiSilicon

(Replaces R4-2101505)

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was endorsed.

R4-2101506	CR for TS 37.145-2: introduction of channel bandwidths 35MHz and 45MHz

Type: draftCR		For: Endorsement
					37.145-2 v17.0.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was revised to R4-2103192.

R4-2103192	CR for TS 37.145-2: introduction of channel bandwidths 35MHz and 45MHz

Type: draftCR		For: Endorsement
					37.145-2 v17.0.0
					Source: Huawei, HiSilicon

(Replaces R4-2101506)

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was endorsed.

R4-2101559	CR to TS 37.105: Introduction of CBWs 35 MHz and 45 MHz

Type: CR		For: Agreement
					37.105 v17.0.0	  CR-0219  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

BS RF requirements for 35 MHz and 45 MHz channel bandwidths were added

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was revised to R4-2103193.

R4-2103193	CR to TS 37.105: Introduction of CBWs 35 MHz and 45 MHz

Type: CR		For: Agreement
					37.105 v17.0.0	  CR-0219  rev 1 Cat: B (Rel-17)

					Source: Ericsson

(Replaces R4-2101559)

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was endorsed.

R4-2101560	CR to TS 38.141-1: Introduction of CBWs 35 MHz and 45 MHz

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0191  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

BS RF requirements for 35 MHz and 45 MHz channel bandwidths were added

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was revised to R4-2103194.

R4-2103194	CR to TS 38.141-1: Introduction of CBWs 35 MHz and 45 MHz

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0191  rev 1 Cat: B (Rel-17)

					Source: Ericsson

(Replaces R4-2101560)

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was endorsed.

R4-2101986	CR to TS 38.141-2: Introduction of 35MHz and 45MHz

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0197  Cat: B (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was not pursued.

R4-2103354	CR to TS 38.141-2: Introduction of 35MHz and 45MHz

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0205  Cat: B (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was endorsed.

R4-2103195	CR to TS 38.141-2: Introduction of 35MHz and 45MHz

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0197  rev 1 Cat: B (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was withdrawn.

R4-2101987	CR to 37.145-1: Introduction of 35MHz and 45MHz

Type: CR		For: Agreement
					37.145-1 v17.0.0	  CR-0242  Cat: B (Rel-17)

					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was revised to R4-2103196.

R4-2103196	CR to 37.145-1: Introduction of 35MHz and 45MHz

Type: CR		For: Agreement
					37.145-1 v17.0.0	  CR-0242  rev 1 Cat: B (Rel-17)

					Source: ZTE Corporation

(Replaces R4-2101987)

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was endorsed.

R4-2102484	CR to 37.104: Introduction of requirements for 35 and 45MHz channel bandwidths

Type: CR		For: Agreement
					37.104 v17.0.0	  CR-0933  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was revised to R4-2103197.

R4-2103197	CR to 37.104: Introduction of requirements for 35 and 45MHz channel bandwidths

Type: CR		For: Agreement
					37.104 v17.0.0	  CR-0933  rev 1 Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102484)

Discussion:

See email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW in R4-2103309.

Decision: 		The document was endorsed.

#### 9.22.5	Others [NR_FR1_35MHz_45MHz_BW-Core]

### 9.23	Band combinations for Uu and V2X con-current operation [NR_LTE_V2X_PC5_combos]

R4-2102970	Email discussion summary for [98e][122] NR_LTE_V2X_PC5_combos

Type: other		For: discussion
					Source: Moderator (CATT)

Discussion:

The contribution summarised email discussion thread [98e][122] NR_LTE_V2X_PC5_combos. The discussion thread was moderated by Yuan Gao (Morningcore Technology Co.,Ltd).

Decision: 		The document was revised to R4-2103310.

R4-2103310	Email discussion summary for [98e][122] NR_LTE_V2X_PC5_combos

Type: other		For: discussion
					Source: Moderator (CATT)

(Replaces R4-2102970)

Discussion:

The contribution summarised email discussion thread [98e][122] NR_LTE_V2X_PC5_combos. The discussion thread was moderated by Yuan Gao (Morningcore Technology Co.,Ltd).

Decision: 		The document was noted.

R4-2103198	Updated TR 37.875 on band combinations for con-current operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band

Type: draft TR		For: Agreement
					Source: CATT

Discussion:

See email discussion summary for [98e][122] NR_LTE_V2X_PC5_combos in R4-2103310. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 9.23.1	General and Rapporteur Input (WID/TR/CR) [NR_LTE_V2X_PC5_combos-Core/Per]

R4-2100412	TP on V2X_n41A-n47A coexistence study

Type: pCR		For: Approval
					37.875 v0.0.0
					Source: CATT

Discussion:

See email discussion summary for [98e][122] NR_LTE_V2X_PC5_combos in R4-2103310.

Decision: 		The document was revised to R4-2103199.

R4-2103199	TP on V2X_n41A-n47A coexistence study

Type: pCR		For: Approval
					37.875 v0.0.0
					Source: CATT

(Replaces R4-2100412)

Discussion:

See email discussion summary for [98e][122] NR_LTE_V2X_PC5_combos in R4-2103310.

Decision: 		The document was approved.

R4-2100413	CR for TS 38.101-1, Introduce new band combination of V2X_n41A-n47A

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0616  Cat: B (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][122] NR_LTE_V2X_PC5_combos in R4-2103310.

Decision: 		The document was revised to R4-2103200.

R4-2103200	CR for TS 38.101-1, Introduce new band combination of V2X_n41A-n47A

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0616  rev 1 Cat: B (Rel-17)

					Source: CATT

(Replaces R4-2100413)

Discussion:

See email discussion summary for [98e][122] NR_LTE_V2X_PC5_combos in R4-2103310.

Decision: 		The document was agreed.

R4-2100414	CR for TS 38.101-3, Introduce new band combination of V2X_41A-n47A and V2X_n41A-47A

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0441  Cat: B (Rel-17)

					Source: CATT

Discussion:

See email discussion summary for [98e][122] NR_LTE_V2X_PC5_combos in R4-2103310.

Decision: 		The document was revised to R4-2103201.

R4-2103201	CR for TS 38.101-3, Introduce new band combination of V2X_41A-n47A and V2X_n41A-47A

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0441  rev 1 Cat: B (Rel-17)

					Source: CATT

(Replaces R4-2100414)

Discussion:

See email discussion summary for [98e][122] NR_LTE_V2X_PC5_combos in R4-2103310.

Decision: 		The document was agreed.

R4-2100502	TR 37.875 on band combinations for con-current operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band

Type: draft TR		For: Agreement
					37.875 v0.0.1
					Source: CATT

Discussion:

See email discussion summary for [98e][122] NR_LTE_V2X_PC5_combos in R4-2103310.

Decision: 		The document was agreed.

R4-2101290	Revised WID for V2X band combination

Type: WID revised		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][122] NR_LTE_V2X_PC5_combos in R4-2103310. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

#### 9.23.2	UE RF requirement for concurrent operation between NR Uu band and NR PC5 band [NR_LTE_V2X_PC5_combos-Core]

R4-2100617	Revision of inter-band V2X con-currency table for V2X_n39A-n47A and V2X_n40A-n47A

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0625  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

Removes n47 from protected band list for V2X_n39A-n47A and V2X_n40A-n47A

Discussion:

See email discussion summary for [98e][122] NR_LTE_V2X_PC5_combos in R4-2103310.

Decision: 		The document was agreed.

#### 9.23.3	UE RF requirement for concurrent operation between LTE Uu band and NR PC5 band [NR_LTE_V2X_PC5_combos-Core]

R4-2100618	Revision of inter-band V2X con-currency table for V2X_39_n47 and V2X_40_n47

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0446  Cat: F (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

Removes n47 from protected band list for V2X_39_n47 and V2X_40_n47

Discussion:

See email discussion summary for [98e][122] NR_LTE_V2X_PC5_combos in R4-2103310.

Decision: 		The document was agreed.

#### 9.23.4	UE RF requirement for concurrent operation between NR Uu band and LTE PC5 band [NR_LTE_V2X_PC5_combos-Core]

#### 9.23.5	UE RF requirement for concurrent operation of LTE/NR CA/DC band combinations + PC5 V2X [NR_LTE_V2X_PC5_combos-Core]

### 9.24	Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258  [NR_FR2_FWA_Bn257_Bn258]

R4-2103467	Email discussion summary: [98e][228] NR_FR2_FWA_Bn257_Bn258_RRM

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][228] NR_FR2_FWA_Bn257_Bn258_RRM. The discussion thread was moderated by Li Hong (HiSilicon Technologies Co. Ltd).

Decision: 		The document was revised to R4-2103709.

R4-2103709	Email discussion summary: [98e][228] NR_FR2_FWA_Bn257_Bn258_RRM

Type: other		For: discussion
					Source: Moderator (Huawei)

(Replaces R4-2103467)

Discussion:

The contribution summarised email discussion thread [98e][228] NR_FR2_FWA_Bn257_Bn258_RRM. The discussion thread was moderated by Li Hong (HiSilicon Technologies Co. Ltd).

Decision: 		The document was noted.

#### 9.24.1	UE RF (38.101-2) [NR_FR2_FWA_Bn257_Bn258-Core]

R4-2102971	Email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258

Type: other		For: discussion
					Source: Moderator (SoftBank)

Discussion:

The contribution summarised email discussion thread [98e][123] NR_FR2_FWA_Bn257_Bn258. The discussion thread was moderated by Masashi Fushiki (SoftBank Corp.).

Decision: 		The document was revised to R4-2103311.

R4-2103311	Email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258

Type: other		For: discussion
					Source: Moderator (SoftBank)

(Replaces R4-2102971)

Discussion:

The contribution summarised email discussion thread [98e][123] NR_FR2_FWA_Bn257_Bn258. The discussion thread was moderated by Masashi Fushiki (SoftBank Corp.).

Decision: 		The document was noted.

R4-2103202	Way forward on Beam correspondence for FR2 FWA

Type: other		For: discussion
					Source: SoftBank

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was approved.

R4-2100566	Views on RF requirement for FWA

Type: other		For: Approval
					Source: Sony, Ericsson

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was noted.

R4-2100692	Proposals on FR2 PC5 beam correspondence

Type: discussion		For: Approval
					Source: MediaTek Beijing Inc.

Abstract:

Proposal1: If FR2 power class 5 beam correspondence is required, both beam correspondence bit-0 and bit-1 requirement shall be defined.

Proposal2: As Table2, UE beam correspondence tolerance for FR2 power class 5:

•	n257 = [3.0] dB at 85th %-tile ?EIRPBC

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was noted.

R4-2101282	Beam correspondence requirements for FWA UE devices

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was noted.

R4-2101753	Discussion on Rel-17 FWA beam correspondence

Type: discussion		For: Approval
					Source: OPPO

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was noted.

R4-2102560	Beam correspondence of FWA device

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was noted.

R4-2102631	on new FWA UE beam correspondence requirement

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was noted.

R4-2102669	On PC5 beam correspondence requirement

Type: other		For: Agreement
					Source: Qualcomm Incorporated

Abstract:

PC5 BC requirement proposal

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was noted.

R4-2102670	CR to 38.807: Update for FR2 PC5

Type: CR		For: Agreement
					38.101-2 v15.12.0	  CR-0338  Cat: F (Rel-15)

					Source: Qualcomm Incorporated

Abstract:

Add PC5 to existing list of power classes

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was withdrawn.

R4-2102671	CR to 38.807: Update for FR2 PC5

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0339  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Abstract:

Add PC5 to existing list of power classes

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was withdrawn.

R4-2102672	CR to 38.807: Update for FR2 PC5

Type: CR		For: Agreement
					38.101-2 v17.0.0	  CR-0340  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

Add PC5 to existing list of power classes

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was withdrawn.

R4-2102688	CR for FR2 FWA RF requirements

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0711  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was revised to R4-2103203.

R4-2103203	CR for FR2 FWA RF requirements

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0711  rev 1 Cat: B (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R4-2102688)

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was agreed.

R4-2102700	CR to 38.807: Update for FR2 PC5

Type: CR		For: Agreement
					38.307 v15.7.0	  CR-0058  Cat: F (Rel-15)

					Source: Qualcomm Incorporated

Abstract:

Add PC5 to existing list of power classes

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was not pursued.

R4-2102701	CR to 38.807: Update for FR2 PC5

Type: CR		For: Agreement
					38.307 v16.5.0	  CR-0059  Cat: F (Rel-16)

					Source: Qualcomm Incorporated

Abstract:

Add PC5 to existing list of power classes

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was not pursued.

R4-2102702	CR to 38.807: Update for FR2 PC5

Type: CR		For: Agreement
					38.307 v17.0.0	  CR-0060  Cat: A (Rel-17)

					Source: Qualcomm Incorporated

Abstract:

Add PC5 to existing list of power classes

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was withdrawn.

#### 9.24.2	RRM Core requirements (38.133) [NR_FR2_FWA_Bn257_Bn258-Core]

#### 9.24.3	RRM Perf. requirements (38.133) [NR_FR2_FWA_Bn257_Bn258-Perf]

R4-2103664	WF on FR2 new FWA UE RRM performance requirements

Type: other		For: discussion
					Source: Huawei

Discussion:

See email discussion summary for [98e][228] NR_FR2_FWA_Bn257_Bn258_RRM in R4-2103709.

Decision: 		The document was approved.

R4-2101683	Discussion on conditions for FR2 new FWA UE

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][228] NR_FR2_FWA_Bn257_Bn258_RRM in R4-2103709.

Decision: 		The document was noted.

R4-2101684	CR on condition requirements for UE power class 5 in TS38.133

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1646  Cat: F (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][228] NR_FR2_FWA_Bn257_Bn258_RRM in R4-2103709.

Decision: 		The document was revised to R4-2103665.

R4-2103665	CR on condition requirements for UE power class 5 in TS38.133

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1646  rev 1 Cat: F (Rel-17)

					Source: Huawei, HiSilicon

(Replaces R4-2101684)

Discussion:

See email discussion summary for [98e][228] NR_FR2_FWA_Bn257_Bn258_RRM in R4-2103709.

Decision: 		The document was agreed.

#### 9.24.4	Others  [NR_FR2_FWA_Bn257_Bn258-Core/Perf]

R4-2100709	CR for 38.307: Introduction of power class 5 for FR2

Type: CR		For: Agreement
					38.307 v17.0.0	  CR-0042  Cat: B (Rel-17)

					Source: SoftBank Corp.

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was agreed.

R4-2101423	Introduction of Noc for PC5 in n257/n258

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0155  Cat: B (Rel-17)

					Source: Ericsson, SoftBank

Abstract:

This CR introduces Noc for PC5 in n257/n258.

Discussion:

See email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 in R4-2103311.

Decision: 		The document was agreed.

### 9.25	Introduction of NR 47 GHz band [NR_47GHz_Band]

#### 9.25.1	UE RF (38.101-2) [NR_47GHz_Band-Core]

R4-2102972	Email discussion summary for [98e][124] NR_47GHz_Band

Type: other		For: discussion
					Source: Moderator (Nokia)

Discussion:

The contribution summarised email discussion thread [98e][124] NR_47GHz_Band. The discussion thread was moderated by Hisashi Onozawa (Nokia Japan).

Decision: 		The document was revised to R4-2103312.

R4-2103312	Email discussion summary for [98e][124] NR_47GHz_Band

Type: other		For: discussion
					Source: Moderator (Nokia)

(Replaces R4-2102972)

Discussion:

The contribution summarised email discussion thread [98e][124] NR_47GHz_Band. The discussion thread was moderated by Hisashi Onozawa (Nokia Japan).

Decision: 		The document was noted.

##### 9.25.1.1	Peak EIRP and EIRP spherical coverage  [NR_47GHz_Band-Core]

R4-2100094	Multi-band relaxation for band n262

Type: discussion		For: Discussion
					Source: Murata Manufacturing Co., Ltd.

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was noted.

R4-2100567	Peak EIRP and EIRP spherical coverage for PC3 for n262

Type: other		For: Approval
					Source: Sony, Ericsson

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was noted.

R4-2100748	EIRP requirements for n262

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was noted.

R4-2100913	UE RF requirements for 47 GHz band

Type: discussion		For: Approval
					Source: Samsung

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was noted.

R4-2101831	Discussion on MOP for Band n262

Type: other		For: Approval
					Source: vivo

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was noted.

R4-2102590	Peak EIRP and EIRP Spherical coverage for n262

Type: discussion		For: Approval
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was noted.

R4-2102668	On EIRP spherical coverage requirements for n262

Type: other		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

peak gain, spherical coverage of gain discussed

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was noted.

R4-2102906	EIRP requirements of band n262

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was noted.

##### 9.25.1.2	Other UE TX requirements  [NR_47GHz_Band-Core]

R4-2100568	Multiband relaxations for PC3 for n262

Type: other		For: Approval
					Source: Sony, Ericsson

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was noted.

R4-2100749	TP to TR 38.847: UE Tx requirement for n262

Type: pCR		For: Approval
					38.847 v0.0.2
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was revised to R4-2103204.

R4-2103204	TP to TR 38.847: UE Tx requirement for n262

Type: pCR		For: Approval
					38.847 v0.0.2
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100749)

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was approved.

R4-2100750	Introduction of n262 UE RF requirements

Type: CR		For: Agreement
					38.101-2 v17.0.0	  CR-0325  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was revised to R4-2103205.

R4-2103205	Introduction of n262 UE RF requirements

Type: CR		For: Agreement
					38.101-2 v17.0.0	  CR-0325  rev 1 Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100750)

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was revised to R4-2103392.

R4-2103392	Introduction of n262 UE RF requirements

Type: CR		For: Agreement
					38.101-2 v17.0.0	  CR-0325  rev 2 Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell, Apple

(Replaces R4-2103205)

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was endorsed.

R4-2102667	On EVM requirements for n262

Type: other		For: Agreement
					Source: Qualcomm Incorporated

Abstract:

Next steps to investigate if deviation from existing FR2 EVM side conditions is justified for n262

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was noted.

##### 9.25.1.3	REFSENS and EIS spherical coverage  [NR_47GHz_Band-Core]

R4-2100569	REFSENS and EIS spherical coverage for PC3 for n262

Type: other		For: Approval
					Source: Sony, Ericsson

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was noted.

R4-2100751	EIS requirements for n262

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was noted.

R4-2101832	Discussion on REFSENS and EIS spherical coverage for Band n262

Type: other		For: Approval
					Source: vivo

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was noted.

R4-2102591	Peak EIS and EIS Spherical coverage for n262

Type: discussion		For: Approval
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was noted.

R4-2102907	EIS requirements of band n262

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was noted.

##### 9.25.1.4	Other UE RX requirements [NR_47GHz_Band-Core]

R4-2100752	TP to TR 38.847: UE Rx requirement for n262

Type: pCR		For: Approval
					38.847 v0.0.2
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was revised to R4-2103206.

R4-2103206	TP to TR 38.847: UE Rx requirement for n262

Type: pCR		For: Approval
					38.847 v0.0.2
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2100752)

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was approved.

#### 9.25.2	BS RF (38.104) [NR_47GHz_Band-Core]

R4-2103748	Email discussion summary for [98e][309] NR_47GHz_Band_BSRF

Type: other		For: discussion
					Source: Moderator (Nokia)

Discussion:

The contribution summarised email discussion thread [98e][309] NR_47GHz_Band_BSRF. The discussion thread was moderated by Iwajlo Angelow (Nokia Italy).

Decision: 		The document was revised to R4-2103944.

R4-2103944	Email discussion summary for [98e][309] NR_47GHz_Band_BSRF

Type: other		For: discussion
					Source: Moderator (Nokia)

(Replaces R4-2103748)

Discussion:

The contribution summarised email discussion thread [98e][309] NR_47GHz_Band_BSRF. The discussion thread was moderated by Iwajlo Angelow (Nokia Italy).

Decision: 		The document was noted.

R4-2102159	CR to TS 38.104 - n262 introduction

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0290  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

This CR specifies band n262 (47GHz band) in TS 38.104

Discussion:

See email discussion summary for [98e][309] NR_47GHz_Band_BSRF in 	R4-2103944.

Decision: 		The document was postponed.

#### 9.25.3	RRM (38.133) [NR_47GHz_Band-Core]

R4-2103468	Email discussion summary: [98e][229] NR_47GHz_Band_RRM

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][229] NR_47GHz_Band_RRM. The discussion thread was moderated by Muhammad Kazmi (Ericsson).

Decision: 		The document was revised to R4-2103710.

R4-2103710	Email discussion summary: [98e][229] NR_47GHz_Band_RRM

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2103468)

Discussion:

The contribution summarised email discussion thread [98e][229] NR_47GHz_Band_RRM. The discussion thread was moderated by Muhammad Kazmi (Ericsson).

Decision: 		The document was noted.

R4-2102653	Analysis of RRM requirements for band n62

Type: other		For: Discussion
					Source: Ericsson

Abstract:

This document analysis RRM requirements for new band on 47 GHz (n62)

Discussion:

See email discussion summary for [98e][229] NR_47GHz_Band_RRM in R4-2103710.

Decision: 		The document was noted.

R4-2102654	RRM core requirements for band n62

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1740  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

The CR on RRM requirements for new band on 47 GHz

Discussion:

See email discussion summary for [98e][229] NR_47GHz_Band_RRM in R4-2103710.

Decision: 		The document was revised to R4-2103666.

R4-2103666	RRM core requirements for band n62

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1740  rev 1 Cat: B (Rel-17)

					Source: Ericsson

(Replaces R4-2102654)

Discussion:

See email discussion summary for [98e][229] NR_47GHz_Band_RRM in R4-2103710.

Decision: 		The document was agreed.

#### 9.25.4	Others  [NR_47GHz_Band-Core/Perf]

##### 9.25.4.1	BS conformance (38.141)  [NR_47GHz_Band-Perf]

R4-2102049	47GHz band TT for NR BS RF requirement

Type: discussion		For: Agreement
					Source: Keysight Technologies UK Ltd

Discussion:

See email discussion summary for [98e][309] NR_47GHz_Band_BSRF in 	R4-2103944.

Decision: 		The document was noted.

R4-2102160	47GHz band - Measurement uncertainties for BS requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses the measurement uncertainties for BS requirements at 47GHz

Discussion:

See email discussion summary for [98e][309] NR_47GHz_Band_BSRF in 	R4-2103944.

Decision: 		The document was noted.

R4-2102445	TP to TR 38.847: BS conformance aspects

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][309] NR_47GHz_Band_BSRF in 	R4-2103944.

Decision: 		The document was revised to R4-2103874.

R4-2103874	TP to TR 38.847: BS conformance aspects

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102445)

Discussion:

See email discussion summary for [98e][309] NR_47GHz_Band_BSRF in 	R4-2103944.

Decision: 		The document was approved.

R4-2102446	CR to 38.141-2: Introduction of n262

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0309  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][309] NR_47GHz_Band_BSRF in 	R4-2103944.

Decision: 		The document was not pursued.

##### 9.25.4.2	UE Demod (38.101-4)  [NR_47GHz_Band-Perf]

R4-2103766	Email discussion summary for [98e][327] NR_R17_SpectrumWI_Demod

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][327] NR_47GHz_Band_Demod. The discussion thread was moderated by Kazuyoshi Uesaka (Ericsson).

Decision: 		The document was revised to R4-2103945.

R4-2103945	Email discussion summary for [98e][327] NR_R17_SpectrumWI_Demod

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2103766)

Discussion:

The contribution summarised email discussion thread [98e][327] NR_47GHz_Band_Demod. The discussion thread was moderated by Kazuyoshi Uesaka (Ericsson).

Decision: 		The document was noted.

R4-2103875	Way forward on UE/BS demodulation on NR 47GHz band

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][327] NR_R17_SpectrumWI_Demod in R4-2103945.

Decision: 		The document was approved.

R4-2102100	CR to TS 38.101-4: n262 demodulation requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0163  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

CR to introduce demodulation requirements for n262

Discussion:

See email discussion summary for [98e][327] NR_R17_SpectrumWI_Demod in R4-2103945.

Decision: 		The document was postponed.

R4-2102101	pCR to 38.847: UE performance requirements

Type: pCR		For: Approval
					38.847 v0.0.2
					Source: Ericsson

Abstract:

Captures information and rationale behind decision for demodulation requirements

Discussion:

See email discussion summary for [98e][327] NR_R17_SpectrumWI_Demod in R4-2103945.

Decision: 		The document was postponed.

R4-2102933	Discussion on NR UE demodulation performance for n262

Type: discussion		For: (not specified)
					Source: HUAWEI TECHNOLOGIES Co. Ltd.

Discussion:

See email discussion summary for [98e][327] NR_R17_SpectrumWI_Demod in R4-2103945.

Decision: 		The document was noted.

##### 9.25.4.3	BS Demod (38.104)  [NR_47GHz_Band-Perf]

R4-2100565	On 47GHZ OTA link budget in Demodulation requirement testing

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution we have expressed our views on the 47GHZ band OTA link budget for demodulation requirement testing.

Discussion:

See email discussion summary for [98e][327] NR_R17_SpectrumWI_Demod in R4-2103945.

Decision: 		The document was noted.

R4-2102102	pCR to TR 38.847: BS demodulation requirements

Type: pCR		For: Approval
					38.847 v0.0.2
					Source: Ericsson

Abstract:

Captures information and rationale behind decision for demodulation requirements

Discussion:

See email discussion summary for [98e][327] NR_R17_SpectrumWI_Demod in R4-2103945.

Decision: 		The document was postponed.

R4-2102935	Discussion on NR BS demodulation performance for n262

Type: discussion		For: Discussion
					Source: HUAWEI TECHNOLOGIES Co. Ltd.

Discussion:

See email discussion summary for [98e][327] NR_R17_SpectrumWI_Demod in R4-2103945.

Decision: 		The document was noted.

##### 9.25.4.4	Others [NR_47GHz_Band-Core/Perf]

R4-2102158	TR 38.847 Introduction of NR Band 262 (47GHz band)

Type: draft TR		For: Agreement
					38.847 v0.1.0
					Source: Ericsson

Abstract:

Updated TR to capture the work done when specifying the new NR FR2 47GHz band

Discussion:

See email discussion summary for [98e][124] NR_47GHz_Band in R4-2103312.

Decision: 		The document was agreed.

### 9.26	Introduction of NR band n24 [NR_band_n24]

R4-2102973	Email discussion summary for [98e][125] NR_LTE_band_n24

Type: other		For: discussion
					Source: Moderator (Ligado Networks)

Discussion:

The contribution summarised email discussion thread [98e][125] NR_LTE_band_n24. The discussion thread was moderated by Ojas Choksi (Ligado Networks).

Decision: 		The document was revised to R4-2103313.

R4-2103313	Email discussion summary for [98e][125] NR_LTE_band_n24

Type: other		For: discussion
					Source: Moderator (Ligado Networks)

(Replaces R4-2102973)

Discussion:

The contribution summarised email discussion thread [98e][125] NR_LTE_band_n24. The discussion thread was moderated by Ojas Choksi (Ligado Networks).

Decision: 		The document was noted.

#### 9.26.1	UE RF (38.101-1) [NR_band_n24-Core]

R4-2100134	Simulation results for n24 / band 24 A-MPR

Type: discussion		For: Discussion
					Source: Nokia

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was noted.

R4-2100135	n24 / band 24 A-MPR proposal

Type: other		For: Approval
					Source: Nokia, Skyworks Inc

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103209.

R4-2103209	n24 / band 24 A-MPR proposal

Type: other		For: Approval
					Source: Nokia, Skyworks Inc

(Replaces R4-2100135)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was noted.

R4-2100246	CR for TS 38.101-1 introduction of NR band n24

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0604  Cat: B (Rel-17)

					Source: Ligado Networks, Nokia, Skyworks Solutions Inc.

Abstract:

Updates to 38.101-1 to introduce NR band n24

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103210.

R4-2103210	CR for TS 38.101-1 introduction of NR band n24

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0604  rev 1 Cat: B (Rel-17)

					Source: Ligado Networks, Nokia, Skyworks Solutions Inc.

(Replaces R4-2100246)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103378.

R4-2103378	CR for TS 38.101-1 introduction of NR band n24

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0604  rev 2 Cat: B (Rel-17)

					Source: Ligado Networks, Nokia, Skyworks Solutions Inc.

(Replaces R4-2103210)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100545	Band 24, n24 and n99 A-MPR

Type: discussion		For: Approval
					38.101-1 v..
					Source: Skyworks Solutions Inc.

Abstract:

This contribution provided measurements for band n24 AMPR proposal. It is also proposing AMPR for LTE band 24 and NR SUl n99 and thus must be considered in related agendas.

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was approved.

#### 9.26.2	BS RF (38.104) [NR_band_n24-Core]

R4-2102451	CR to 38.104: Introduction of n24

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0295  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

#### 9.26.3	RRM (38.133) [NR_band_n24-Core]

R4-2100247	CR for TS 38.133 introduction of NR band n24

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1469  Cat: B (Rel-17)

					Source: Ligado Networks

Abstract:

Updates to TS 38.133 to introduce NR band n24

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

#### 9.26.4	Others  [NR_band_n24-Core/Perf]

R4-2100248	CR for TS 37.105 introduction of NR band n24

Type: CR		For: Agreement
					37.105 v17.0.0	  CR-0214  Cat: B (Rel-17)

					Source: Ligado Networks

Abstract:

Updates to TS 37.105 to introduce NR band n24

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100249	CR for TS 37.145-1 introduction of NR band n24

Type: CR		For: Agreement
					37.145-1 v17.0.0	  CR-0234  Cat: B (Rel-17)

					Source: Ligado Networks

Abstract:

Updates to TS 37.145-1 to introduce NR band n24

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100250	CR for TS 37.145-2 introduction of NR band n24

Type: CR		For: Agreement
					37.145-2 v17.0.0	  CR-0268  Cat: B (Rel-17)

					Source: Ligado Networks

Abstract:

Updates to TS 37.145-2 to introduce NR band n24

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100251	CR for TS 38.141-1 introduction of NR band n24

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0168  Cat: B (Rel-17)

					Source: Ligado Networks

Abstract:

Updates to TS 38.141-1 to introduce NR band n24

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100252	CR for TS 38.141-2 introduction of NR band n24

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0259  Cat: B (Rel-17)

					Source: Ligado Networks

Abstract:

Updates to TS 38.141-2 to introduce NR band n24

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102447	CR to 36.104: Introduction of n24 requirements

Type: CR		For: Agreement
					36.104 v17.0.0	  CR-4927  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102448	CR to 36.141: Introduction of n24 requirements

Type: CR		For: Agreement
					36.141 v17.0.0	  CR-1295  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102449	CR to 37.104: Introduction of n24 requirements

Type: CR		For: Agreement
					37.104 v17.0.0	  CR-0924  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102450	CR to 37.141: Introduction of n24 requirements

Type: CR		For: Agreement
					37.141 v17.0.0	  CR-0964  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

### 9.27	Introduction of 1.6 GHz NR SUL band with same uplink frequency range of Band 24 [NR_SUL_UL_n24]

#### 9.27.1	UE RF (38.101-1) [NR_SUL_UL_n24-Core]

R4-2100332	Discussion on new SUL band n99 UE requirements

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Abstract:

SUL n99

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was noted.

R4-2100334	CR to 38101-1 on introducing new SUL band n99

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0609  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Abstract:

SUL n99

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103211.

R4-2103211	CR to 38101-1 on introducing new SUL band n99

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0609  rev 1 Cat: B (Rel-17)

					Source: Huawei,HiSilicon

(Replaces R4-2100334)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103351.

R4-2103351	CR to 38101-1 on introducing new SUL band n99

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0609  rev 2 Cat: B (Rel-17)

					Source: Huawei,HiSilicon

(Replaces R4-2103211)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

#### 9.27.2	BS RF (38.104) [NR_SUL_UL_n24-Core]

R4-2100333	Discussion on new SUL band n99 BS requirements

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Abstract:

SUL n99

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was noted.

R4-2100335	CR to 38104 on introducing new SUL band n99

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0262  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Abstract:

SUL n99

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103212.

R4-2103212	CR to 38104 on introducing new SUL band n99

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0262  rev 1 Cat: B (Rel-17)

					Source: Huawei,HiSilicon

(Replaces R4-2100335)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103352.

R4-2103352	CR to 38104 on introducing new SUL band n99

Type: CR		For: Agreement
					38.104 v17.0.0	  CR-0262  rev 2 Cat: B (Rel-17)

					Source: Huawei,HiSilicon

(Replaces R4-2103212)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100336	CR to 36104 on introducing new SUL band n99

Type: CR		For: Agreement
					36.104 v17.0.0	  CR-4920  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Abstract:

SUL n99

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100337	CR to 38141-1 on introducing new SUL band n99

Type: CR		For: Agreement
					38.141-1 v17.0.0	  CR-0169  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Abstract:

SUL n99

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100338	CR to 38141-2 on introducing new SUL band n99

Type: CR		For: Agreement
					38.141-2 v17.0.0	  CR-0260  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Abstract:

SUL n99

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100339	CR to 36141 on introducing new SUL band n99

Type: CR		For: Agreement
					36.141 v17.0.0	  CR-1288  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Abstract:

SUL n99

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100340	CR to 37104 on introducing new SUL band n99

Type: CR		For: Agreement
					37.104 v17.0.0	  CR-0920  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Abstract:

SUL n99

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100341	CR to 37141 on introducing new SUL band n99

Type: CR		For: Agreement
					37.141 v17.0.0	  CR-0959  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Abstract:

SUL n99

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100342	CR to 37105 on introducing new SUL band n99

Type: CR		For: Agreement
					37.105 v17.0.0	  CR-0216  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Abstract:

SUL n99

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100343	CR to 37145-1 on introducing new SUL band n99

Type: CR		For: Agreement
					37.145-1 v17.0.0	  CR-0236  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Abstract:

SUL n99

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103213.

R4-2103213	CR to 37145-1 on introducing new SUL band n99

Type: CR		For: Agreement
					37.145-1 v17.0.0	  CR-0236  rev 1 Cat: B (Rel-17)

					Source: Huawei,HiSilicon

(Replaces R4-2100343)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100344	CR to 37145-2 on introducing new SUL band n99

Type: CR		For: Agreement
					37.145-2 v17.0.0	  CR-0270  Cat: B (Rel-17)

					Source: Huawei,HiSilicon

Abstract:

SUL n99

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103214.

R4-2103214	CR to 37145-2 on introducing new SUL band n99

Type: CR		For: Agreement
					37.145-2 v17.0.0	  CR-0270  rev 1 Cat: B (Rel-17)

					Source: Huawei,HiSilicon

(Replaces R4-2100344)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

#### 9.27.3	RRM (38.133) [NR_SUL_UL_n24-Core]

#### 9.27.4	Others  [NR_SUL_UL_n24-Core/Perf]

### 9.28	Introduction of NR band n67 [NR_n67]

R4-2102974	Email discussion summary for [98e][126] NR_n67

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][126] NR_n67. The discussion thread was moderated by Dominique Evereare (Ericsson).

Decision: 		The document was revised to R4-2103314.

R4-2103314	Email discussion summary for [98e][126] NR_n67

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2102974)

Discussion:

The contribution summarised email discussion thread [98e][126] NR_n67. The discussion thread was moderated by Dominique Evereare (Ericsson).

Decision: 		The document was noted.

R4-2103218	Way forward on the introduction of band n67

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][126] NR_n67 in R4-2103314.

Decision: 		The document was approved.

R4-2102491	Introduction of band n67

Type: discussion		For: Approval
					Source: Huawei, HiSilicon

Abstract:

discussion about introducing band n67

Discussion:

See email discussion summary for [98e][126] NR_n67 in R4-2103314.

Decision: 		The document was noted.

#### 9.28.1	UE RF (38.101-1) [NR_n67-Core]

R4-2102170	New NR band n67 - UE RF impacts

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution is analyzing the UE RF impacts when adding the new refarmed band n67

Discussion:

See email discussion summary for [98e][126] NR_n67 in R4-2103314.

Decision: 		The document was noted.

#### 9.28.2	BS RF (38.104) [NR_n67-Core]

R4-2102169	New NR band n67 - BS RF impacts

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution is analyzing the BS RF impacts when adding the new refarmed band n67

Discussion:

See email discussion summary for [98e][126] NR_n67 in R4-2103314.

Decision: 		The document was noted.

#### 9.28.3	RRM (38.133) [NR_n67-Core]

#### 9.28.4	Others  [NR_n67-Core/Perf]

### 9.29	Introduction of NR band n85 [NR_n85]

R4-2102975	Email discussion summary for [98e][127] NR_n85

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][127] NR_n85. The discussion thread was moderated by Dominique Evereare (Ericsson).

Decision: 		The document was revised to R4-2103315.

R4-2103315	Email discussion summary for [98e][127] NR_n85

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2102975)

Discussion:

The contribution summarised email discussion thread [98e][127] NR_n85. The discussion thread was moderated by Dominique Evereare (Ericsson).

Decision: 		The document was noted.

R4-2103219	Way forward on the introduction of band n85

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][127] NR_n85 in R4-2103315.

Decision: 		The document was approved.

R4-2102509	Introduction of band n85

Type: discussion		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][127] NR_n85 in R4-2103315.

Decision: 		The document was noted.

#### 9.29.1	UE RF (38.101-1) [NR_n85-Core]

R4-2102172	New NR band n85 - UE RF impacts

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution is analyzing the UE RF impacts when adding the new refarmed band n85

Discussion:

See email discussion summary for [98e][127] NR_n85 in R4-2103315.

Decision: 		The document was noted.

#### 9.29.2	BS RF (38.104) [NR_n85-Core]

R4-2102171	New NR band n85 - BS RF impacts

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution is analyzing the BS RF impacts when adding the new refarmed band n85

Discussion:

See email discussion summary for [98e][127] NR_n85 in R4-2103315.

Decision: 		The document was noted.

#### 9.29.3	RRM (38.133) [NR_n85-Core]

#### 9.29.4	Others  [NR_n85-Core/Perf]

### 9.30	Introduction of bandwidth combination set 4 (BCS4) for NR [NR_BCS4]

R4-2102976	Email discussion summary for [98e][128] NR_BCS4

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][128] NR_BCS4. The discussion thread was moderated by Per Lindell (Ericsson).

Decision: 		The document was revised to R4-2103316.

R4-2103316	Email discussion summary for [98e][128] NR_BCS4

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2102976)

Discussion:

The contribution summarised email discussion thread [98e][128] NR_BCS4. The discussion thread was moderated by Per Lindell (Ericsson).

Decision: 		The document was revised to R4-2103375.

R4-2103375	Email discussion summary for [98e][128] NR_BCS4

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2103316)

Discussion:

The contribution summarised email discussion thread [98e][128] NR_BCS4. The discussion thread was moderated by Per Lindell (Ericsson).

Decision: 		The document was noted.

R4-2103271	Way forward on BCS4 for NR

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][128] NR_BCS4 in R4-2103375.

Decision: 		The document was approved.

#### 9.30.1	General and Rapporteur Input (WID/TR/CR) [NR_BCS4-Core]

R4-2101817	General discussion on introduction of BCS4

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][128] NR_BCS4 in R4-2103375.

Decision: 		The document was noted.

R4-2102136	Templates for BCS4 configurations for inter-band NR CA

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][128] NR_BCS4 in R4-2103375.

Decision: 		The document was withdrawn.

R4-2102187	Templates for BCS4 configurations for inter-band NR CA

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][128] NR_BCS4 in R4-2103375.

Decision: 		The document was noted.

#### 9.30.2	UE RF requirements [NR_BCS4-Core]

R4-2102928	Cross-band MSD for ENDC and NR-CA BCS4

Type: discussion		For: Approval
					38.101-1 v..
					Source: Skyworks Solutions Inc.

Discussion:

See email discussion summary for [98e][128] NR_BCS4 in R4-2103375.

Decision: 		The document was noted.

##### 9.30.2.1	MSD  [NR_BCS4-Core]

R4-2101816	Discussion on how to simplify MSD definition using bandwidth-agnostic approach

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][128] NR_BCS4 in R4-2103375.

Decision: 		The document was noted.

R4-2102150	Discussion on BCS4

Type: discussion		For: Approval
					Source: T-Mobile USA

Discussion:

See email discussion summary for [98e][128] NR_BCS4 in R4-2103375.

Decision: 		The document was noted.

R4-2102151	Draft CR for 38.101-1: Introduction of BCS4

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: T-Mobile USA, MediaTek

Discussion:

See email discussion summary for [98e][128] NR_BCS4 in R4-2103375.

Decision: 		The document was revised to R4-2103376.

R4-2103376	Draft CR for 38.101-1: Introduction of BCS4

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: T-Mobile USA, MediaTek

(Replaces R4-2102151)

Discussion:

See email discussion summary for [98e][128] NR_BCS4 in R4-2103375.

Decision: 		The document was revised to R4-2103394.

R4-2103394	Draft CR for 38.101-1: Introduction of BCS4

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: T-Mobile USA, MediaTek

(Replaces R4-2103376)

Discussion:

See email discussion summary for [98e][128] NR_BCS4 in R4-2103375. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

##### 9.30.2.2	Others (in case MPR/A-MPR is needed) [NR_BCS4-Core]

#### 9.30.3	Signalling [NR_BCS4-Core]

R4-2100088	Required changes to the original BCS4 idea

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution discuss the most suitable UE capabilities signalling methods to enable BCS4 support  among captured methods in a corresponding WID.

Discussion:

See email discussion summary for [98e][128] NR_BCS4 in R4-2103375.

Decision: 		The document was noted.

R4-2101371	The signalling for BCS4

Type: discussion		For: Approval
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][128] NR_BCS4 in R4-2103375.

Decision: 		The document was noted.

R4-2102137	Discussion on UE capabilities signalling to enable BCS4

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][128] NR_BCS4 in R4-2103375.

Decision: 		The document was noted.

R4-2102188	Discussion on UE capabilities signalling to enable BCS4

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][128] NR_BCS4 in R4-2103375.

Decision: 		The document was noted.

R4-2102502	Discussion on candidate methods for BCS4

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][128] NR_BCS4 in R4-2103375.

Decision: 		The document was noted.

### 9.31	Band combination specific requirements for NR intra band UL Carrier Aggregation []

R4-2102977	Email discussion summary for [98e][129] NR_req_Intra_UL_CA

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][129] NR_req_Intra_UL_CA. The discussion thread was moderated by Zhang Qian (HiSilicon Technologies Co. Ltd).

Decision: 		The document was revised to R4-2103317.

R4-2103317	Email discussion summary for [98e][129] NR_req_Intra_UL_CA

Type: other		For: discussion
					Source: Moderator (Huawei)

(Replaces R4-2102977)

Discussion:

The contribution summarised email discussion thread [98e][129] NR_req_Intra_UL_CA. The discussion thread was moderated by Zhang Qian (HiSilicon Technologies Co. Ltd).

Decision: 		The document was noted.

#### 9.31.1	General and Rapporteur Input (WID/TR/CR) [-Core]

R4-2102621	TR skeleton 38.XXX V001 NR_HPUE_intra_Band_R17

Type: other		For: Endorsement
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][129] NR_req_Intra_UL_CA in R4-2103317.

Decision: 		The document was revised to R4-2103220.

R4-2103220	TR skeleton 38.XXX V001 NR_HPUE_intra_Band_R17

Type: other		For: Endorsement
					Source: Huawei, HiSilicon

(Replaces R4-2102621)

Discussion:

See email discussion summary for [98e][129] NR_req_Intra_UL_CA in R4-2103317.

Decision: 		The document was agreed.

#### 9.31.2	PC2 UE RF requirements [-Core]

##### 9.31.2.1	Maximum output power  [-Core]

##### 9.31.2.2	A-MPR [-Core]

##### 9.31.2.3	others [-Core]

#### 9.31.3	PC3 UE RF requirements [-Core]

### 9.32	Additional NR bands for UL-MIMO [NR_bands_UL_MIMO_PC3_R17]

R4-2102978	Email discussion summary for [98e][130] NR_bands_UL_MIMO_PC3_R17

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][130] NR_bands_UL_MIMO_PC3_R17. The discussion thread was moderated by Ye Liu (Huawei Technologies France).

Decision: 		The document was noted.

#### 9.32.1	General and Rapporteur Input (WID/TR/CR) [NR_bands_UL_MIMO_PC3_R17-Core]

R4-2100099	Introduce NR SUL bands to PC3 UL-MIMO configuration

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0587  Cat: B (Rel-17)

					Source: CMCC

Decision: 		The document was not pursued.

R4-2102394	CR for TS 38.101-1 Introduce NR SUL bands to PC3 UL-MIMO configuration

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0688  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Decision: 		The document was agreed.

#### 9.32.2	MPR/A-MPR requirement [NR_bands_UL_MIMO_PC3_R17-Core]

#### 9.32.3	Others [NR_bands_UL_MIMO_PC3_R17-Core/Perf]

R4-2102393	draftCR to introduce UL MIMO configurations for band n84

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: Huawei, HiSilicon

Decision: 		The document was endorsed.

### 9.33	Down link interruption for band combinations to conduct dynamic Tx Switching [DL_intrpt_combos_TxSW_R17]

R4-2102979	Email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17

Type: other		For: discussion
					Source: Moderator (China Telecom)

Discussion:

The contribution summarised email discussion thread [98e][131] DL_intrpt_combos_TxSW_R17. The discussion thread was moderated by Bo Liu (China Telecom Corporation Ltd.).

Decision: 		The document was revised to R4-2103318.

R4-2103318	Email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17

Type: other		For: discussion
					Source: Moderator (China Telecom)

(Replaces R4-2102979)

Discussion:

The contribution summarised email discussion thread [98e][131] DL_intrpt_combos_TxSW_R17. The discussion thread was moderated by Bo Liu (China Telecom Corporation Ltd.).

Decision: 		The document was noted.

#### 9.33.1	General and Rapporteur Input (WID/TR/CR) [DL_intrpt_combos_TxSW_R17-Core]

R4-2100373	TR skeleton for Downlink interruption for band combinations to conduct dynamic Tx Switching

Type: draft TR		For: Agreement
					Source: CATT

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318.

Decision: 		The document was agreed.

R4-2100374	TR 37.xxx 0.1.0 for Downlink interruption for band combinations to conduct dynamic Tx Switching

Type: draft TR		For: Agreement
					Source: CATT

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2101127	Work plan on downlink interruption for band combinations to conduct dynamic Tx switching

Type: other		For: Approval
					Source: China Telecom

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318.

Decision: 		The document was approved.

R4-2101128	Revised WID: Downlink interruption for NR and EN-DC band combinations to conduct dynamic Tx Switching in Uplink

Type: WID revised		For: Endorsement
					Source: China Telecom

Abstract:

Update the WI title, code and TR remarks according to MCC suggestion

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318.

Decision: 		The document was endorsed.

#### 9.33.2	Determination of inter-band uplink CA and EN-DC combinations for which DL interruption is not allowed [DL_intrpt_combos_TxSW_R17-Core]

R4-2100806	TP on DL applicability of CA_n3-n40-n41 for 37.xxx

Type: other		For: Approval
					Source: CMCC

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318.

Decision: 		The document was revised to R4-2103221.

R4-2103221	TP on DL applicability of CA_n3-n40-n41 for 37.xxx

Type: other		For: Approval
					Source: CMCC

(Replaces R4-2100806)

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318.

Decision: 		The document was approved.

R4-2100807	TP on DL applicability of CA_n3-n41-n79 for 37.xxx

Type: other		For: Approval
					Source: CMCC

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318.

Decision: 		The document was revised to R4-2103222.

R4-2103222	TP on DL applicability of CA_n3-n41-n79 for 37.xxx

Type: other		For: Approval
					Source: CMCC

(Replaces R4-2100807)

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318.

Decision: 		The document was approved.

R4-2100808	TP on DL applicability of CA_n8-n39-n41for 37.xxx

Type: other		For: Approval
					Source: CMCC

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318.

Decision: 		The document was revised to R4-2103223.

R4-2103223	TP on DL applicability of CA_n8-n39-n41for 37.xxx

Type: other		For: Approval
					Source: CMCC

(Replaces R4-2100808)

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318.

Decision: 		The document was approved.

R4-2100809	TP on DL applicability of CA_n8-n41-n79 for 37.xxx

Type: other		For: Approval
					Source: CMCC

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318.

Decision: 		The document was revised to R4-2103224.

R4-2103224	TP on DL applicability of CA_n8-n41-n79 for 37.xxx

Type: other		For: Approval
					Source: CMCC

(Replaces R4-2100809)

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318.

Decision: 		The document was approved.

R4-2100810	TP on DL applicability of CA_n39-n41-n79 for 37.xxx

Type: other		For: Approval
					Source: CMCC

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318.

Decision: 		The document was revised to R4-2103225.

R4-2103225	TP on DL applicability of CA_n39-n41-n79 for 37.xxx

Type: other		For: Approval
					Source: CMCC

(Replaces R4-2100810)

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318.

Decision: 		The document was approved.

R4-2100811	TP on DL applicability of CA_n40-n41-n79 for 37.xxx

Type: other		For: Approval
					Source: CMCC

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318.

Decision: 		The document was revised to R4-2103226.

R4-2103226	TP on DL applicability of CA_n40-n41-n79 for 37.xxx

Type: other		For: Approval
					Source: CMCC

(Replaces R4-2100811)

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318.

Decision: 		The document was approved.

#### 9.33.3	Others [DL_intrpt_combos_TxSW_R17-Core/Perf]

R4-2100812	Discussion on DL interruption applicability for inter-band CA with 3 bands

Type: discussion		For: Decision
					Source: CMCC

Discussion:

See email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 in R4-2103318.

Decision: 		The document was noted.

### 9.34	High-power UE operation for use cases in Band n77 and n78 [HPUE_PC1_5_n77_n78]

R4-2102980	Email discussion summary for [98e][132] HPUE_PC1_5_n77_n78

Type: other		For: discussion
					Source: Moderator (Qualcomm)

Discussion:

The contribution summarised email discussion thread [98e][132] HPUE_PC1_5_n77_n78. The discussion thread was moderated by Gene Fong (Qualcomm).

Decision: 		The document was revised to R4-2103319.

R4-2103319	Email discussion summary for [98e][132] HPUE_PC1_5_n77_n78

Type: other		For: discussion
					Source: Moderator (Qualcomm)

(Replaces R4-2102980)

Discussion:

The contribution summarised email discussion thread [98e][132] HPUE_PC1_5_n77_n78. The discussion thread was moderated by Gene Fong (Qualcomm).

Decision: 		The document was noted.

R4-2103227	Way forward on assumptions for PC1.5 in Band n77 and n78

Type: other		For: discussion
					Source: Qualcomm

Discussion:

See email discussion summary for [98e][132] HPUE_PC1_5_n77_n78 in R4-2103319.

Decision: 		The document was approved.

R4-2103228	Way forward on FWA MPE handling for n77/n78

Type: other		For: discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][132] HPUE_PC1_5_n77_n78 in R4-2103319.

Decision: 		The document was approved.

#### 9.34.1	General [HPUE_PC1_5_n77_n78-Core]

R4-2100515	Considerations for PC1.5 with n77 and n78

Type: discussion		For: Approval
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][132] HPUE_PC1_5_n77_n78 in R4-2103319.

Decision: 		The document was noted.

R4-2100912	Regulatory information on RF exposure for FWA devices

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][132] HPUE_PC1_5_n77_n78 in R4-2103319.

Decision: 		The document was noted.

R4-2102283	Consideration on adding PC 1.5 for n77 and n78

Type: discussion		For: Agreement
					Source: Huawei, HiSilicon

Abstract:

Changes in UE spec needed for adding PC 1.5 for n77 and n78

Discussion:

See email discussion summary for [98e][132] HPUE_PC1_5_n77_n78 in R4-2103319.

Decision: 		The document was noted.

#### 9.34.2	PC1.5 UE RF requirements [HPUE_PC1_5_n77_n78-Core]

R4-2102930	Discussion on band n77 PC1.5 operation

Type: discussion		For: Discussion
					38.101-1 v..
					Source: Skyworks Solutions Inc.

Abstract:

In this contribution we discuss PC1.5 for TDD bands n77 and n78 cases compared with the band n41 case already specified in Release 16 .

Discussion:

See email discussion summary for [98e][132] HPUE_PC1_5_n77_n78 in R4-2103319.

Decision: 		The document was noted.

##### 9.34.2.1	A-MPR [HPUE_PC1_5_n77_n78-Core]

R4-2100287	Consideration for RF architecture for n77/n78 PC1.5 UE

Type: other		For: Approval
					Source: LG Electronics France

Abstract:

propose basekine RF architecture and MPR/A-MPR simulation assumptions

Discussion:

See email discussion summary for [98e][132] HPUE_PC1_5_n77_n78 in R4-2103319.

Decision: 		The document was noted.

##### 9.34.2.2	others [HPUE_PC1_5_n77_n78-Core]

R4-2102417	PC 1.5 for bands n77 and n78

Type: discussion		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][132] HPUE_PC1_5_n77_n78 in R4-2103319.

Decision: 		The document was noted.

### 9.35	Introduction of lower 6GHz NR unlicensed operation for Europe [NR_6GHz_unlic_EU]

R4-2102981	Email discussion summary for [98e][133] NR_6GHz_unlic_EU

Type: other		For: discussion
					Source: Moderator (Nokia)

Discussion:

The contribution summarised email discussion thread [98e][133] NR_6GHz_unlic_EU. The discussion thread was moderated by Johannes Hejselbaek (Nokia Denmark).

Decision: 		The document was revised to R4-2103320.

R4-2103320	Email discussion summary for [98e][133] NR_6GHz_unlic_EU

Type: other		For: discussion
					Source: Moderator (Nokia)

(Replaces R4-2102981)

Discussion:

The contribution summarised email discussion thread [98e][133] NR_6GHz_unlic_EU. The discussion thread was moderated by Johannes Hejselbaek (Nokia Denmark).

Decision: 		The document was noted.

R4-2103229	Way forward on introduction of lower 6GHz NR unlicensed operation for Europe

Type: other		For: discussion
					Source: Nokia

Discussion:

See email discussion summary for [98e][133] NR_6GHz_unlic_EU in R4-2103320.

Decision: 		The document was approved.

R4-2100514	Band plan for lower 6GHz NR unlicensed operation for Europe

Type: discussion		For: Decision
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][133] NR_6GHz_unlic_EU in R4-2103320.

Decision: 		The document was noted.

#### 9.35.1	General [NR_6GHz_unlic_EU-Core]

R4-2100546	NRU in 6GHz EU spectrum: Band Definition and Related Emission Requirements

Type: discussion		For: Approval
					38.101-1 v..
					Source: Skyworks Solutions Inc.

Abstract:

In this contribution, we analyze the available regulations and propose to reuse the n96 band definition restricted to the channels in the 5945-6425MHz frequency range using NS mechanism as already used for n46 sub-bands or FCC compliant indoor/outdoor ope

Discussion:

See email discussion summary for [98e][133] NR_6GHz_unlic_EU in R4-2103320.

Decision: 		The document was noted.

R4-2101927	Skeleton TR 38.849 v0.0.0

Type: draft TR		For: Agreement
					38.849 v0.0.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Draft skeleton for the internal TR 38.849 for the WID on Introduction of lower 6GHz NR unlicensed operation for Europe

Discussion:

See email discussion summary for [98e][133] NR_6GHz_unlic_EU in R4-2103320.

Decision: 		The document was agreed.

R4-2101928	draft TR 38.849 v0.1.0

Type: draft TR		For: Agreement
					38.849 v0.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Inclusion of agreements and TPs provided at RAN4#98 to TR 38.849

Discussion:

See email discussion summary for [98e][133] NR_6GHz_unlic_EU in R4-2103320. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2101929	Work plan for Introduction of lower 6GHz NR unlicensed operation for Europe

Type: Work Plan		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][133] NR_6GHz_unlic_EU in R4-2103320.

Decision: 		The document was approved.

R4-2101965	Discussion on Europe unlicensed 6GHz for NR-U

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][133] NR_6GHz_unlic_EU in R4-2103320.

Decision: 		The document was noted.

#### 9.35.2	UE RF requirements [NR_6GHz_unlic_EU-Core]

R4-2101930	On UE RF aspects for the lower 6GHz NR unlicensed operation

Type: discussion		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][133] NR_6GHz_unlic_EU in R4-2103320.

Decision: 		The document was noted.

R4-2102416	UE requirements for EU NR-U 6 GHz band

Type: discussion		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][133] NR_6GHz_unlic_EU in R4-2103320.

Decision: 		The document was noted.

#### 9.35.3	BS RF requirements [NR_6GHz_unlic_EU-Core]

R4-2101931	On BS RF aspects for the lower 6GHz NR unlicensed operation

Type: discussion		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][133] NR_6GHz_unlic_EU in R4-2103320.

Decision: 		The document was noted.

R4-2101966	Discussion on BS RF requirements for Europe unlicensed 6GHz

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][133] NR_6GHz_unlic_EU in R4-2103320.

Decision: 		The document was noted.

R4-2101967	draft CR for introduction of Europe unlicensed 6GHz.

Type: draftCR		For: Endorsement
					38.104 v17.0.0
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][133] NR_6GHz_unlic_EU in R4-2103320.

Decision: 		The document was not pursued.

#### 9.35.4	Others [NR_6GHz_unlic_EU-Core]

## 10	Reply to ITU-R LS (RP-200042)

### 10.1	Study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz [FS_6425_10500MHz _NR]

R4-2102982	Email discussion summary for [98e][134] FS_6425_10500MHz_NR

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][134] FS_6425_10500MHz _NR. The discussion thread was moderated by Dominique Evereare (Ericsson).

Decision: 		The document was revised to R4-2103321.

R4-2103321	Email discussion summary for [98e][134] FS_6425_10500MHz_NR

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2102982)

Discussion:

The contribution summarised email discussion thread [98e][134] FS_6425_10500MHz _NR. The discussion thread was moderated by Dominique Evereare (Ericsson).

Decision: 		The document was noted.

R4-2103103	Way forward on WF on remaining BS and UE parameters for 6.425-7.125 and 10.0-10.5 GHz

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was revised to R4-2103396.

R4-2103396	Way forward on WF on remaining BS and UE parameters for 6.425-7.125 and 10.0-10.5 GHz

Type: other		For: discussion
					Source: Ericsson, ZTE, Nokia, Nokia Shanghai Bell, CATT, Qualcomm

(Replaces R4-2103103)

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was approved.

R4-2101494	TR 38.921 V 0.3.0

Type: draft TR		For: Agreement
					38.921 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was agreed.

R4-2101500	Reply LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)

Type: LS out		For: Approval
					to ITU-R WP5D, cc RAN
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2102840	Draft LS to WP5D on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23

Type: LS out		For: Approval
					to ITU-R WP5D, cc TSG RAN
					Source: Ericsson

Abstract:

The LS gives feedback to WP5D on the parameters and fills in the final ones.

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was revised to R4-2103104.

R4-2103104	LS to WP5D on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23

Type: LS out		For: Approval
					to ITU-R WP5D, cc TSG RAN
					Source: RAN4

(Replaces R4-2102840)

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321. The document was subject for post-meeting email approval.

Decision: 		The document was approved.

#### 10.1.1	UE parameters

R4-2100488	Fuurther discussion on UE parameters for 6.425-7.125GHz, 7.025-7.125GHz and 10.0-10.5GHz

Type: discussion		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2101495	TP to TR 38.921: UE remaining parameters

Type: pCR		For: Approval
					38.921 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2101791	Proposals of UE Parameters for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution provides the proposals of the open UE parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz according to the downlink and uplink coexistence simulation results provided.

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2101948	TP to TR 38.921 UE transmitter requirements

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was revised to R4-2103105.

R4-2103105	TP to TR 38.921 UE transmitter requirements

Type: other		For: Approval
					Source: ZTE Corporation

(Replaces R4-2101948)

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was revised to R4-2103397.

R4-2103397	TP to TR 38.921 UE transmitter requirements

Type: other		For: Approval
					Source: ZTE Corporation, Qualcomm, Huawei, CATT, Nokia, Nokia Shanghai Bell

(Replaces R4-2103105)

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321. The document was subject for post-meeting email approval.

Decision: 		The document was revised to R4-2103408.

R4-2103408	TP to TR 38.921 UE transmitter requirements

Type: other		For: Approval
					Source: ZTE Corporation, Qualcomm, Huawei, CATT, Nokia, Nokia Shanghai Bell

(Replaces R4-2103105)

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321. The document was subject for post-meeting email approval.

Decision: 		The document was approved.

R4-2102157	SI on IMT parameters - Remaining UE parameters

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution is discussing remaining UE parameters for the SI on IMT parameters

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2102501	Discussion on UE SEM for 6.425-7.125GHz and 10-10.5GHz

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

#### 10.1.2	BS parameters

R4-2100489	further discussion on BS parameters for  6.425-7.125GHz, 7.025-7.125GHz and 10.0-10.5GHz

Type: discussion		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2100823	Discussion on remaining Tx requirements of BS for 6425-7125MHz

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2101496	TP to TR 38.921: BS remaining parameters

Type: pCR		For: Approval
					38.921 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was revised to R4-2103106.

R4-2103106	TP to TR 38.921: BS remaining parameters

Type: pCR		For: Approval
					38.921 v0.2.0
					Source: Huawei, HiSilicon

(Replaces R4-2101496)

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was revised to R4-2103398.

R4-2103398	TP to TR 38.921: BS remaining parameters

Type: pCR		For: Approval
					38.921 v0.2.0
					Source: Huawei, HiSilicon, CATT, Nokia, ZTE

(Replaces R4-2103106)

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321. The document was subject for post-meeting email approval.

Decision: 		The document was not concluded.

R4-2101792	Proposals of BS Parameters for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution provides the proposals of the open BS parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz according to the downlink and uplink coexistence simulation results provided.

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2101949	TP to TR 38.921 BS requirements

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2102156	SI on IMT parameters - Remaining BS parameters

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution is discussing remaining BS parameters for the SI on IMT parameters

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

#### 10.1.3	Coexistence study

R4-2101499	TP for Clause 4.3 co-existence simulation results

Type: pCR		For: Approval
					38.921 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2101950	TP to TR 38.921 summary of simulation results

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was revised to R4-2103107.

R4-2103107	TP to TR 38.921 summary of simulation results

Type: other		For: Approval
					Source: ZTE Corporation, Nokia, Nokia Shanghai Bell, CATT, Qualcomm

(Replaces R4-2101950)

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was approved.

##### 10.1.3.1	Simulation assumptions

R4-2101793	TP to TR 38.921: Clarification of BS maximum transmit power on system level simulation assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz

Type: pCR		For: Approval
					38.921 v0.2.0
					Source: Nokia, Nokia Shanghai Bell, ZTE

Abstract:

This contribution proposes to remove the reference to Note 3 for the indoor “BS max TX power in dBm” to avoid the ambiguity that 24dBm is defined per polarization.

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was approved.

R4-2101953	TP to TR 38.921 Maintenance for simulation assumption

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2102500	TP to TR 38.921: Clarification of beamforming pattern modelling for multiple UL schedued UEs

Type: pCR		For: Approval
					38.921 v0.3.0
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2103108	TP to TR 38.921: Clarification of beamforming pattern modelling for multiple UL schedued UEs

Type: pCR		For: Approval
					38.921 v0.3.0
					Source: Qualcomm Incorporated

(Replaces R4-2102500)

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was withdrawn.

##### 10.1.3.2	Downlink

R4-2100490	Downlin simulation results for 6425-7125MHz and 10-10.5GHz - indoor scenario

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2101497	Simulation results on indoor DL co-existence for 6.425-7.125GHz, 10.0-10.5 GHz

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2101794	Downlink Indoor Hotspot Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz

Type: other		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution provides the downlink indoor hotspot coexistence simulation results according to the agreed assumptions.

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2101951	DL simulation results for 6425-7125MHz and 10-10.5GHz

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2102154	SI on IMT parameters - DL simulations results

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution is providing coexistence simulations results in DL for the 6-7GHz and 10GHz bands

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2102498	Downlink co-existence simulation results for indoor scenario

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

##### 10.1.3.3	Uplink

R4-2100491	Uuplink simulation results for 6425-7125MHz and 10-10.5GHz indoor scenario- indoor scenario

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2101498	Simulation results on indoor UL co-existence for 6.425-7.125GHz, 10.0-10.5 GHz

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2101795	Uplink Indoor Hotspot Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz

Type: other		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution provides the uplink indoor hotspot coexistence simulation results according to the agreed assumptions.

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2101952	UL simulation results for 6425-7125MHz and 10-10.5GHz

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2102155	SI on IMT parameters - UL simulations results

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution is providing coexistence simulations results in UL for the 6-7GHz and 10GHz bands

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2102499	Uplink co-existence simulation results for indoor scenario

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

#### 10.1.4	Antenna characteristics

R4-2101182	TP to TR 38.921: Addition of in-door antenna parameters and correction to model in subclause 8.1

Type: pCR		For: Approval
					38.921 v0.2.0
					Source: Ericsson

Abstract:

At the end of this contribution a text proposal is attached for approval. The text proposal consists of two parts; Addition of antenna parameters for in-door deployment scenario and correction of antenna parameter definition in subclause 8.1.

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was revised to R4-2103109.

R4-2103109	TP to TR 38.921: Addition of in-door antenna parameters and correction to model in subclause 8.1

Type: pCR		For: Approval
					38.921 v0.2.0
					Source: Ericsson

(Replaces R4-2101182)

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was approved.

R4-2101796	TP to TR 38.921: Proposals of Indoor BS Antenna Characteristics for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz

Type: pCR		For: Approval
					38.921 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution provides the proposals of indoor BS antenna characteristics for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz according to the downlink and uplink coexistence simulation results provided

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

R4-2101954	TP to TR 38.921 Antenna configurations

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

#### 10.1.5	Relevant information for the sharing and compatibility studies

R4-2101797	Proposals of Relevant Information for the ITU-R WP5D Sharing and Compatibility Studies for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution provides the proposals on how the relevant information may be considered in the ITU-R WP5D sharing and compatibility studies for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz.

Discussion:

See email discussion summary for [98e][134] FS_6425_10500MHz_NR in R4-2103321.

Decision: 		The document was noted.

## 11	Rel-17 non-spectrum related work items for NR

### 11.1	Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs [NR_MIMO_OTA]

#### 11.1.1	General  [NR_MIMO_OTA]

R4-2103768	Email discussion summary for [98e][329] NR_MIMO_OTA

Type: other		For: discussion
					Source: Moderator (CAICT)

Discussion:

The contribution summarised email discussion thread [98e][329] NR_MIMO_OTA. The discussion thread was moderated by Siting Zhu (CAICT).

Decision: 		The document was revised to R4-2103946.

R4-2103946	Email discussion summary for [98e][329] NR_MIMO_OTA

Type: other		For: discussion
					Source: Moderator (CAICT)

(Replaces R4-2103768)

Discussion:

The contribution summarised email discussion thread [98e][329] NR_MIMO_OTA. The discussion thread was moderated by Siting Zhu (CAICT).

Decision: 		The document was noted.

R4-2103913	WF on NR MIMO OTA

Type: other		For: discussion
					Source: vivo, CAICT

Decision: 		The document was approved.

R4-2103914	WF on TP work split and Work Plan

Type: other		For: discussion
					Source: vivo, CAICT, OPPO

Decision: 		The document was approved.

R4-2103915	WF on FR2 MIMO OTA simulation assumptions

Type: other		For: discussion
					Source: Huawei, HiSilicon

Decision: 		The document was approved.

R4-2101822	TS 38.151 v0.2.0 NR MIMO OTA requirements

Type: draft TS		For: Agreement
					38.151 v0.2.0
					Source: vivo

Abstract:

New version TS

Decision: 		The document was agreed.

R4-2101825	TP to TS 38.151 v0.1.0 on Performance metrics for NR MIMO OTA requirements

Type: pCR		For: Approval
					38.151 v0.1.0
					Source: vivo, CAICT

Decision: 		The document was revised to R4-2103971.

R4-2103971	TP to TS 38.151 v0.1.0 on Performance metrics for NR MIMO OTA requirements

Type: pCR		For: Approval
					38.151 v0.1.0
					Source: vivo, CAICT

(Replaces R4-2101825)

Decision: 		The document was approved.

#### 11.1.2	Performance Requirements  [NR_MIMO_OTA-Core]

##### 11.1.2.1	Performance Requirements for FR1  [NR_MIMO_OTA-Core]

R4-2101827	Discussions on Channel models mapping for FR1 MIMO OTA requirement

Type: discussion		For: Approval
					Source: vivo

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2101941	FR1 MIMO OTA measurement results and channel model mapping

Type: discussion		For: Approval
					Source: CAICT,Keysight,ETS-Lindgren

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

##### 11.1.2.2	Performance Requirements for FR2 [NR_MIMO_OTA-Core]

R4-2101758	Consideration on how to treat the missing orientations for FR2

Type: discussion		For: Approval
					Source: OPPO

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2101824	TP to TS 38.151 v0.1.0 on FR2 test system for requirements

Type: pCR		For: Approval
					38.151 v0.1.0
					Source: vivo, CAICT

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was revised to R4-2103970.

R4-2103970	TP to TS 38.151 v0.1.0 on FR2 test system for requirements

Type: pCR		For: Approval
					38.151 v0.1.0
					Source: vivo, CAICT

(Replaces R4-2101824)

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was approved.

R4-2102497	Discussion on FR2 MIMO OTA performance requirements

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2102719	Simulation assumption summary for NR FR2 MIMO OTA

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

#### 11.1.3	Testing methodologies [NR_MIMO_OTA-Core]

##### 11.1.3.1	Testing parameters for Performance  [NR_MIMO_OTA-Core]

R4-2100892	Discussion on FR1 test parameters and Figure of Merit

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2101826	Discussion on testing parameters for NR MIMO OTA requirement

Type: discussion		For: Approval
					Source: vivo

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2102729	Consideration on FR2 MIMO OTA UE requirement

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

##### 11.1.3.2	Optimization of test methodologies [NR_MIMO_OTA-Core]

R4-2101757	Consideration on 3D-MPAC probe locations configuration for FR2 MIMO OTA

Type: discussion		For: Approval
					Source: OPPO

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2102400	Analysis on number of test points vs uncertainty of FR2 MIMO OTA requirements

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

##### 11.1.3.3	Channel model validation [NR_MIMO_OTA-Core]

R4-2100845	Channel Model Validation Bounds

Type: other		For: Approval
					Source: Spirent Communications

Abstract:

Proposal 1: FR1 PDP Power = [±0.8 dB]. PDP excess delay = [±11ns].

Proposal 2 and 4: Bounds for FR1 and FR2 Autocorrelation

0.5?, [NonPolarized value +/- 0.1 capped at 1]

1 ?, [NonPolarized value +/- 0.2]

1.5?, [NonPolarized value +/- 0.25]

2?, [NonPolar

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2101823	TP to TS 38.151 v0.1.0 on FR2 Channel model and RMC

Type: pCR		For: Approval
					38.151 v0.1.0
					Source: vivo, CAICT

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was revised to R4-2103969.

R4-2103969	TP to TS 38.151 v0.1.0 on FR2 Channel model and RMC

Type: pCR		For: Approval
					38.151 v0.1.0
					Source: vivo, CAICT

(Replaces R4-2101823)

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was approved.

R4-2102613	gNB Beams Usage Criteria for NR FR1 MIMO OTA Channel Model Validation

Type: other		For: Approval
					Source: Keysight Technologies UK Ltd, China Mobile

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

### 11.2	RF requirements enhancement for NR frequency range 1 (FR1) [NR_RF_FR1_enh]

#### 11.2.1	General and work plan [NR_RF_FR1_enh-Core]

R4-2102983	Email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][135] NR_RF_FR1_enh_Part_1. The discussion thread was moderated by Qian Zhang (HiSilicon Technologies Co. Ltd).

Decision: 		The document was revised to R4-2103322.

R4-2103322	Email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1

Type: other		For: discussion
					Source: Moderator (Huawei)

(Replaces R4-2102983)

Discussion:

The contribution summarised email discussion thread [98e][135] NR_RF_FR1_enh_Part_1. The discussion thread was moderated by Qian Zhang (HiSilicon Technologies Co. Ltd).

Decision: 		The document was noted.

R4-2103231	Way forward on RF architecture and requirements for intra-band UL contiguous CA

Type: other		For: discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was approved.

R4-2103232	Way forward on MPR and AMPR for intra-band UL contiguous CA

Type: other		For: discussion
					Source: Skyworks

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was approved.

R4-2103233	Way forward on 4Rx requirement for CA_n77(3A) and CA_77(4A)

Type: other		For: discussion
					Source: SoftBank

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was approved.

R4-2102627	additional work plan for Rel-17 FR1 UE RF enhancement

Type: Work Plan		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was revised to R4-2103230.

R4-2103230	additional work plan for Rel-17 FR1 UE RF enhancement

Type: Work Plan		For: Approval
					Source: Huawei, HiSilicon

(Replaces R4-2102627)

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was approved.

#### 11.2.2	RF core requirements [NR_RF_FR1_enh-Core]

R4-2102284	n77(3/4A) DL CA UE Architecture, Regional Needs and 4x4 DL MIMO Support

Type: discussion		For: Approval
					38.101-1 v..
					Source: Skyworks Solutions Inc.

Abstract:

In this contribution, we further discuss architecture and 4x4 DL MIMO support for n77(3A/4A) and regional needs and make proposals in order to introduce these combinations in Release 17.

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was noted.

##### 11.2.2.1	UL MIMO configuration for SUL band configurations  [NR_RF_FR1_enh-Core]

R4-2100799	CR on introducing NR SUL bands n80 to UL-MIMO configuration

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0631  Cat: B (Rel-17)

					Source: CMCC

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was agreed.

R4-2101854	Draft CR to TS 38.101-1 on switching time between SUL and NUL

Type: draftCR		For: Endorsement
					38.101-1 v17.0.0
					Source: ZTE Wistron Telecom AB

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was not pursued.

R4-2101855	Switching time for UL-MIMO enabled SUL band combination

Type: discussion		For: Decision
					Source: ZTE Wistron Telecom AB

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was noted.

##### 11.2.2.2	2Tx switching between carrier 1 and carrier 2 [NR_RF_FR1_enh-Core]

R4-2102984	Email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2

Type: other		For: discussion
					Source: Moderator (China Telecom)

Discussion:

The contribution summarised email discussion thread [98e][136] NR_RF_FR1_enh_Part_2. The discussion thread was moderated by Yang Shan (China Telecomunication Corp.).

Decision: 		The document was revised to R4-2103323.

R4-2103323	Email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2

Type: other		For: discussion
					Source: Moderator (China Telecom)

(Replaces R4-2102984)

Discussion:

The contribution summarised email discussion thread [98e][136] NR_RF_FR1_enh_Part_2. The discussion thread was moderated by Yang Shan (China Telecomunication Corp.).

Decision: 		The document was noted.

R4-2103234	LS on Rel-17 Tx switching enhancements

Type: LS out		For: Approval
					to RAN1, RAN2
					Source: RAN4

Discussion:

See email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2 in R4-2103323.

Decision: 		The document was approved.

R4-2103235	Way forward on RF requirements for Rel-17 Tx switching enhancements

Type: other		For: discussion
					Source: China Telecom

Discussion:

See email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2 in R4-2103323.

Decision: 		The document was approved.

R4-2100496	Discussion on 2Tx switching

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2 in R4-2103323.

Decision: 		The document was noted.

R4-2100600	Discussion on power boosting for 2Tx switching

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2 in R4-2103323.

Decision: 		The document was withdrawn.

R4-2100790	Remaining issues on Tx switching enhancement and draft LS

Type: discussion		For: Discussion
					Source: China Telecom

Discussion:

See email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2 in R4-2103323.

Decision: 		The document was noted.

R4-2100791	Switching time mask for 2Tx-2Tx switching between two carriers and 1Tx-2Tx/2Tx-2Tx switching between two bands in Rel-17

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0628  Cat: B (Rel-17)

					Source: China Telecom

Discussion:

See email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2 in R4-2103323.

Decision: 		The document was revised to R4-2103236.

R4-2103236	Switching time mask for 2Tx-2Tx switching between two carriers and 1Tx-2Tx/2Tx-2Tx switching between two bands in Rel-17

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0628  rev 1 Cat: B (Rel-17)

					Source: China Telecom

(Replaces R4-2100791)

Discussion:

See email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2 in R4-2103323.

Decision: 		The document was agreed.

R4-2100800	Discussion on 2Tx switching between carrier 1 and carrier 2

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2 in R4-2103323.

Decision: 		The document was noted.

R4-2101754	Discussion on Rel-17 power boosting in switched UL transmission

Type: discussion		For: Approval
					Source: OPPO

Discussion:

See email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2 in R4-2103323.

Decision: 		The document was noted.

R4-2102706	Further discussion on Tx Switching enhancment

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2 in R4-2103323.

Decision: 		The document was noted.

##### 11.2.2.3	Tx switching between 1 carrier on band A and 2 contiguous aggregated carriers on band B [NR_RF_FR1_enh-Core]

R4-2100801	Discussion on Tx switching between 1 carrier on band A and 2 contiguous aggregated carriers on band B

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2 in R4-2103323.

Decision: 		The document was noted.

R4-2101104	Discussion on remaining issue on UL Tx switching enhancement in R17

Type: other		For: Approval
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2 in R4-2103323.

Decision: 		The document was noted.

R4-2101851	Discussion on 2Tx UL switching between two bands

Type: discussion		For: Discussion
					Source: ZTE Wistron Telecom AB

Discussion:

See email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2 in R4-2103323.

Decision: 		The document was noted.

R4-2102397	On 2Tx - 2Tx UE uplink switch

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2 in R4-2103323.

Decision: 		The document was noted.

##### 11.2.2.4	HPUE for TDD intra-band contiguous UL CA [NR_RF_FR1_enh-Core]

R4-2100288	MPR/A-MPR initial simulation results according to candidate RF architectures

Type: other		For: Approval
					Source: LG Electronics France

Abstract:

Provide initial MPR/A-MPR simulations results for PC2 NR intra-band contiguous CA

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was noted.

R4-2100544	PC2 Class C UL CA UE Architecture and MPR/A-MPR evaluation

Type: discussion		For: Approval
					38.101-1 v..
					Source: Skyworks Solutions Inc.

Abstract:

This contribution discusses the transmitter architecture options for PC2 contiguous UL CA and reuses the measured data to make proposals for PC2 class B and C UL CA MPR and NS04 A-MPR.

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was noted.

R4-2101103	Discussion on HP UE for TDD intra-band contiguous UL CA

Type: other		For: Approval
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was noted.

R4-2101160	HPUE TDD+TDD MPR and AMPR

Type: other		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was noted.

R4-2101755	Discussion on Rel-17 FR1 UL CA

Type: discussion		For: Approval
					Source: OPPO

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was noted.

R4-2102133	Discussion on PC2 intra-band contiguous NR CA RF requirements

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was noted.

R4-2102184	Discussion on PC2 intra-band contiguous NR CA RF requirements

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was noted.

R4-2102657	on intra-band CA HPUE RF architecture and MPR

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 in R4-2103322.

Decision: 		The document was noted.

##### 11.2.2.5	HPUE for TDD intra-band non-contiguous UL CA [NR_RF_FR1_enh-Core]

R4-2102985	Email discussion summary for [98e][137] NR_RF_FR1_enh_Part_3

Type: other		For: discussion
					Source: Moderator (Qualcomm)

Discussion:

The contribution summarised email discussion thread [98e][137] NR_RF_FR1_enh_Part_3. The discussion thread was moderated by Ville Vintola (Qualcomm).

Decision: 		The document was revised to R4-2103324.

R4-2103324	Email discussion summary for [98e][137] NR_RF_FR1_enh_Part_3

Type: other		For: discussion
					Source: Moderator (Qualcomm)

(Replaces R4-2102985)

Discussion:

The contribution summarised email discussion thread [98e][137] NR_RF_FR1_enh_Part_3. The discussion thread was moderated by Ville Vintola (Qualcomm).

Decision: 		The document was noted.

R4-2103237	Way forward on NC UL CA PC2 evaluation assumptions and scenarios

Type: other		For: discussion
					Source: Skyworks

Discussion:

See email discussion summary for [98e][137] NR_RF_FR1_enh_Part_3 in R4-2103324.

Decision: 		The document was revised to R4-2103371.

R4-2103371	Way forward on NC UL CA PC2 evaluation assumptions and scenarios

Type: other		For: discussion
					Source: Skyworks

(Replaces R4-2103237)

Discussion:

See email discussion summary for [98e][137] NR_RF_FR1_enh_Part_3 in R4-2103324.

Decision: 		The document was approved.

R4-2103238	Way forward on NC UL CA PC2 requirements

Type: other		For: discussion
					Source: ZTE

Discussion:

See email discussion summary for [98e][137] NR_RF_FR1_enh_Part_3 in R4-2103324.

Decision: 		The document was approved.

R4-2100289	MPR/A-MPR initial simulation assumptions for PC2 NR intra-band NC CA

Type: other		For: Approval
					Source: LG Electronics France, LG Uplus

Abstract:

Propose baseline RF architecture and basic simulation assumptions for MPR requirements for PC2 intra-band NC CA

Discussion:

See email discussion summary for [98e][137] NR_RF_FR1_enh_Part_3 in R4-2103324.

Decision: 		The document was noted.

R4-2100572	PC2 non-contiguous UL CA UE Architecture and MPR/A-MPR evaluation

Type: discussion		For: Approval
					38.101-1 v..
					Source: Skyworks Solutions Inc.

Abstract:

This contribution discusses the transmitter architecture for PC2 non-contiguous Ul CA and provides the evaluation assumptions for n77 and n41 MPR and NS04 A-MPR.

Discussion:

See email discussion summary for [98e][137] NR_RF_FR1_enh_Part_3 in R4-2103324.

Decision: 		The document was noted.

R4-2102134	Discussion on PC2 intra-band non-contiguous NR CA

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][137] NR_RF_FR1_enh_Part_3 in R4-2103324.

Decision: 		The document was withdrawn.

R4-2102185	Discussion on PC2 intra-band non-contiguous NR CA

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][137] NR_RF_FR1_enh_Part_3 in R4-2103324.

Decision: 		The document was noted.

R4-2102707	Discussion on SAR control scheme for TDD intra-band non-contiguous UL CA HPUE

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][137] NR_RF_FR1_enh_Part_3 in R4-2103324.

Decision: 		The document was noted.

### 11.3	NR RF requirement enhancements for frequency range 2 (FR2)  [NR_RF_FR2_req_enh2]

R4-2103469	Email discussion summary: [98e][230] NR_RF_FR2_req_enh2_RRM

Type: other		For: discussion
					Source: Moderator (Nokia)

Discussion:

The contribution summarised email discussion thread [98e][230] NR_RF_FR2_req_enh2_RRM. The discussion thread was moderated by Lei Du (Nokia).

Decision: 		The document was revised to R4-2103711.

R4-2103711	Email discussion summary: [98e][230] NR_RF_FR2_req_enh2_RRM

Type: other		For: discussion
					Source: Moderator (Nokia)

(Replaces R4-2103469)

Discussion:

The contribution summarised email discussion thread [98e][230] NR_RF_FR2_req_enh2_RRM. The discussion thread was moderated by Lei Du (Nokia).

Decision: 		The document was noted.

GTW session (February 03, 2021)

Issue 1-1-2: UE assumptions for CBM

- 1st round tentative agreements:

  - For CBM capable UE, UE is assumed to receive with one beam at a time, i.e. similar to Rel-15 baseline UE assumption

- Proposals

  - Option 1: Similar to Rel-15 baseline UE assumption i.e. UE can receive with one panel at a time.

  - Option 2: The implementation assumptions for antenna panel and RF architecture for CBM UE needs RF inputs.

- Discussion

  - Huawei: prefer not to mention panels

  - QC: what Huawei wanted to say is that we don’t need to make assumptions on the panels. It was not explicitly mentioned in the spec how many panels we have

  - Apple: Option 1 does not imply single panel implementation. For Rel-17 we can assume that UE is using a single panel at a time.

  - Huawei: this means that UE cannot use 2 panels for same direction in one time.

  - E///: we already have 1st round tentative agreement

  - Nokia: 1st round tentative agreement is ok. How UE forms the beam is up to implementation. The key is that UE can receive with one beam at a time which is same as Rel-15

  - Intel: number of panels is up to RF session and in RRM we need to focus on number of beams. Can remove the wording “similar to Rel-15”

  - LGE: Support 1st round tentative agreement. For RRM we don’t consider the number of panels. We think there should be no impact from number of panels.

  - Apple: it is a bit dangerous to assume that UE can form 1 beam using multiple panels. In this case we’ll need to revisit RF requirements (e.g. spherical coverage). We can go with Option 1 or wait RF session conclusions. We prefer not to leave any ambiguity here.

  - Nokia: the question can be split into beam and panel discussions

  - QC: if UE forms single beam using multiple panels, then it will be seen as a single panel. RF session should discuss this.

  - MTK: we should not consider advanced UE implementations (>1 panel)

  - E///: number of panels is contentious.

  - OPPO: Number of panels will not have impact on RRM requirements and can be up to UE implementation.

  - Session chair: does the number of panels have impact on requirements.

    - Apple: Potentially, yes. For instance, for 2 panels we may need to consider additional activation time.

- Agreements

  - For CBM capable UE

    - UE is assumed to make reception with one beam at a time, i.e. similar to Rel-15 baseline UE assumption

    - FFS for number of panels UE can use for CBM and it is up to RF session conclusions. At least one active panel at a time can be assumed as baseline for RRM requirements definition.

Issue 1-1-4: UE assumptions for IBM

- Proposals

  - Option 1: Capture that it is baseline UE requirement for an IBM capable UE, with more than 1 panel, to be able to have multiple panels active simultaneously.

  - Option 2: Baseline requirement should be based on R15 assumption, and it should allow UE to receive 2 bands with 1 panel

  - Option 3: Any requirements have not been specified with assumption of multiple panels active simultaneously. For consistency, one panel active from more than 1 panel needs to be kept

  - Option 4: IBM UE is assumed to be only capable of receiving signals for FR2 inter-bands CA with different beam directions (Huawei)

  - Option 5: Discuss in RF session

- Discussion

  - Huawei: focus on beams.

  - Nokia: Option 1 or 4. Prefer to avoid panel discussion. Still we can make some assumptions on IBM

  - E///: Option 4 is fine for us. Can limit discussion to beam-level

  - MTK: Option 2. Option 4 is fine for us.

  - NEC: For Option 4 does it include different beam directions active at the same time?

    - Huawei: yes, we can include it

  - LGE: RF session is discussing CBM and IBM definitions. After RF session conclusions, we can follow these conclusions.

  - Apple: both Option 1 and 4 are ok. Option 1 is more accurate. For number of panels this may not have impact on requirements. IBM was introduced in Rel-16. In Rel-16 EIS requirements imply at least 2 panels active at a time (spherical coverage is broader than for Rel-15 UE which can be achieved via using multi-panel implementation).

  - Xiaomi: Ok with Option 4. Do we need to limit the number of directions which UE can receive at the same time (e.g. 2 directions)?

  - QC: IBM UE requires 2 separate resources for beam management (for each band). UE finds the best beam for each band. For CBM UE uses resources in one band to form the beam in another band. No need to discuss number of panels.

- Agreements

  - IBM capable UE is assumed to be capable of receiving signals for FR2 inter-band CA with different beam directions at the same time

Issue 1-2-2: How to determine MRTD for FR2 inter-band CA for CBM?

- Proposals

  - Option 1: Reuse FR2 intra-band MRTD i.e. 260ns (Apple, Intel, OPPO, MTK, LG, QC, Xiaomi)

  - Option 2: 3us (NEC, Nokia, E///)

  - Option 3: 3us MRTD requirements can be applied for co-located deployment and >3us MRTD requirements can be applied for non-co-located deployment (Huawei)

- Discussion

  - Nokia: from RF session conclusions there are no restrictions on co-located and non-co-located deployments

  - Apple: no conclusion in RF session. MRTD > CP will result in performance degradation. Suggest to limit to co-located case in RRM session.

  - E///: Option 2. Also ok with Option 3. For co-located case the TAE will result in 3us.

  - Huawei: same view as E/// for co-located case. For non-co-located case we need to consider propagation delay.

  - Xiaomi: for RF session – from deployment perspective both co-located and non-co-located deployments are supported. For UE requirements – they are derived based on co-located case only.

  - Nokia: CBM should not be limited to co-located. Same view as E/// and Huawei. Option 2 and 3 are ok for us.

  - QC: Need to have separate discussion for deployment and MRTD. We cannot preclude non-co-located scenarios, but the MRTD requirements may not necessarily take this into account. There will be performance degradation.

  - vivo: Option 1.

  - LGE: RF session conclusions are limited to the UE RF requirements.

  - Intel: Option 1. Same time MRTD depends on TAE which is 3us based on current specs. We would like to check if it is feasible to achieve better TAE (e.g. intra-band TAE)

  - Nokia: tentative agreement is fine for us

  - E///: non-co-located deployment will require additional component

  - Huawei: prefer to include non-co-located deployments

  - ZTE: prefer not to preclude non-co-located deployments

  - Intel: for non-co-located deployment Option 3 may not work. We can have a single set of requirements. UE does not know the deployment characteristics. It is up to network to decide whether to use CBM depending on the actual deployment. There still may be IBM UEs which can have better characteristics. There is no harm to have tighter requirements. If we define high MRTD then we have a risk that CBM will simply not be implemented.

    - Apple: agree

  - Apple: we cannot purely rely on RF session decisions. Co-location assumption have impact on : 1) timing (RRM aspect) and 2) power imbalance (RF aspect). We can further align with RF but think the decision can be made in RRM room for RRM requirements.

  - QC/vivo/Xiaomi: same view as Apple and Intel.

  - vivo: RF session agreements may not take into account the RRM constraints. Non-co-located scenarios are not precluded.

  - Xiaomi: need to have analysis on performance impacts for different MRTD

  - E///: disagree to preclude non-co-located deployments.

  - Apple: E/// was fine with Option 2 which implies co-located deployment. What has changed? It is not abnormal in RAN4 to define requirements for a subset of scenarios.

    - E///: we are fine with 3us for co-located case. Non-co-located case will require more than 3us due to additional RF propagation.

  - E///: tentative agreement is ok for co-located case. Need to additionally consider non-co-located. Disagree with tentative agreement.

- Tentative agreements

  - Inter-band MRTD is FFS

    - MRTD requirements are derived under assumption of co-located deployments

      - Note: this does not preclude using co-located or non-co-located deployments in the field

    - MRTD value

      - Option 1: 260ns (i.e. FR2 intra-band MRTD)

      - Option 2: 3us

      - Other options are not precluded

    - Companies are encouraged to evaluate the impact on the performance in case of using MRTD larger than CP

- Session chair: No consensus reached. Continue the discussion.

R4-2103671	WF on RRM requirements for FR2 Inter-band DL CA and UL CA

Type: other		For: discussion
					Source: Nokia

Discussion:

See email discussion summary for [98e][230] NR_RF_FR2_req_enh2_RRM in R4-2103711.

Decision: 		The document was revised to R4-2104095.

R4-2104095	WF on RRM requirements for FR2 Inter-band DL CA and UL CA

Type: other		For: discussion
					Source: Nokia

(Replaces R4-2103671)

Discussion:

See email discussion summary for [98e][230] NR_RF_FR2_req_enh2_RRM in R4-2103711.

Decision: 		The document was approved.

R4-2100241	On MRTD for CBM for FR2 interband CA

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][230] NR_RF_FR2_req_enh2_RRM in R4-2103711.

Decision: 		The document was noted.

R4-2100640	Discussion on MRTD and MTTD requirements on CBM and IBM for FR2 inter-band CA

Type: discussion		For: Discussion
					Source: LG Electronics

Abstract:

It discusses MRTD and MTTD requirements on CBM and IBM for FR2 inter-band CA.

Discussion:

See email discussion summary for [98e][230] NR_RF_FR2_req_enh2_RRM in R4-2103711.

Decision: 		The document was noted.

#### 11.3.1	General and work plan [NR_RF_FR2_req_enh2-Core]

R4-2102986	Email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1

Type: other		For: discussion
					Source: Moderator (Nokia)

Discussion:

The contribution summarised email discussion thread [98e][138] NR_RF_FR2_req_enh2_Part_1. The discussion thread was moderated by Petri Vasenkari (Nokia).

Decision: 		The document was revised to R4-2103325.

R4-2103325	Email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1

Type: other		For: discussion
					Source: Moderator (Nokia)

(Replaces R4-2102986)

Discussion:

The contribution summarised email discussion thread [98e][138] NR_RF_FR2_req_enh2_Part_1. The discussion thread was moderated by Petri Vasenkari (Nokia).

Decision: 		The document was noted.

R4-2103110	Way forward on introduction of new FR2 CA BW classes

Type: other		For: discussion
					Source: Verizon

Discussion:

See email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1 in R4-2103325.

Decision: 		The document was revised to R4-2103359.

R4-2103359	Way forward on introduction of new FR2 CA BW classes

Type: other		For: discussion
					Source: Verizon

(Replaces R4-2103110)

Discussion:

See email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1 in R4-2103325.

Decision: 		The document was approved.

R4-2100264	Release 17 FR2 bandwidth class

Type: discussion		For: Decision
					Source: Verizon Denmark

Discussion:

See email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1 in R4-2103325.

Decision: 		The document was noted.

R4-2100693	Status overview and proposals on FR2 inter-band CA discussion

Type: discussion		For: Approval
					Source: MediaTek Beijing Inc.

Abstract:

Proposal1: For “feasibility study stage”, RAN4 shall converge inter-band DL CA discussion firstly, before start to do inter-band UL CA feasibility study.

Proposal2: For “UE requirement discussion stage”, RAN4 shall specify exact band combination demand f

Discussion:

See email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1 in R4-2103325.

Decision: 		The document was noted.

R4-2101727	On the inter-band UL CA study and change of scope too include improved BC

Type: other		For: Approval
					Source: Ericsson, Sony

Abstract:

In this contribution we propose that the scope of the UL inter-band study is reduced to give room for further work on beam correspondence

Discussion:

See email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1 in R4-2103325.

Decision: 		The document was noted.

#### 11.3.2	RF core requirements [NR_RF_FR2_req_enh2-Core]

R4-2102987	Email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2

Type: other		For: discussion
					Source: Moderator (Qualcomm)

Discussion:

The contribution summarised email discussion thread [98e][139] NR_RF_FR2_req_enh2_Part_2. The discussion thread was moderated by Sumant Iyer (Qualcomm).

Decision: 		The document was revised to R4-2103326.

R4-2103326	Email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2

Type: other		For: discussion
					Source: Moderator (Qualcomm)

(Replaces R4-2102987)

Discussion:

The contribution summarised email discussion thread [98e][139] NR_RF_FR2_req_enh2_Part_2. The discussion thread was moderated by Sumant Iyer (Qualcomm).

Decision: 		The document was noted.

R4-2103111	Way forward on FR2 UEs that support inter-band DL CA with IBM

Type: other		For: discussion
					Source: Sony

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was approved.

R4-2103112	Way forward on FR2 UEs that support inter-band DL CA with CBM

Type: other		For: discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was revised to R4-2103399.

R4-2103399	Way forward on FR2 UEs that support inter-band DL CA with CBM

Type: other		For: discussion
					Source: Samsung

(Replaces R4-2103112)

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was approved.

R4-2103113	Way forward on FR2 UEs that support inter-band UL CA

Type: other		For: discussion
					Source: MediaTek

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was approved.

##### 11.3.2.1	Inter-band DL CA enhancements [NR_RF_FR2_req_enh2-Core]

###### 11.3.2.1.1	Applicability of CBM/IBM for different CA configurations [NR_RF_FR2_req_enh2-Core]

R4-2100143	Specification differences between IBM and CBM

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2100570	Views on Applicability of CBM/IBM for different CA configurations

Type: other		For: Approval
					Source: Sony, Ericsson

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2100598	Applicability for CBM and IBM for FR2 inter-band CA band combos

Type: other		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2100636	Discussion on applicability of CBM and IBM for different CA configurations

Type: discussion		For: Discussion
					Source: LG Electronics

Abstract:

It discusses applicability of CBM and IBM for different CA configurations for both UL CA and DL CA.

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2101284	Frequency separation class consideration for inter-band CA based on CBM

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2101373	Frequency separation class for inter-band CA within the same frequency group based on CBM

Type: discussion		For: Approval
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2101756	Discussion on Rel-17 FR2 CBM IBM

Type: discussion		For: Approval
					Source: OPPO

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2102131	Discussion on FR2 Inter-band DL CA enhancements

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2102182	Discussion on FR2 Inter-band DL CA enhancements

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2102511	Discussion on the CBM/IBM applicability of Rel-17 FR2 inter-band CA

Type: discussion		For: Approval
					Source: Google Inc.

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2102676	on CBM and IBM for FR2 inter-band DL CA

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

###### 11.3.2.1.2	UE requirements for CA configurations CA_n258A-n260A and CA_n257A-n259A based on IBM [NR_RF_FR2_req_enh2-Core]

R4-2100620	RF specifications for DLCA n260A_n258A and n259A_n257A based on IBM

Type: other		For: Approval
					38.101-2 v..
					Source: Qualcomm Incorporated

Abstract:

Provides RF specifications for DLCA n260A_n258A and n259A_n257A

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2100695	Proposals on PC3 RIB of CA_n258A-n260A and CA_n257A-n259A

Type: discussion		For: Approval
					Source: MediaTek Beijing Inc.

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2100747	FR2 inter-band CA for different frequency band groups with IBM

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, the general framework for FR2 inter-band DL CA for different frequency group based on IBM is discussed.

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2101199	Band specific requirements for FR2 DL Inter-band CA of n257n259 with IBM

Type: other		For: Approval
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2102138	Discussion on UE requirements for CA configurations based on IBM

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was withdrawn.

R4-2102189	Discussion on UE requirements for CA configurations based on IBM

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2102607	CA_n258A-n260A and CA_n257A-n259A based on IBM

Type: other		For: Discussion
					38.101-2 v..
					Source: Apple

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

###### 11.3.2.1.3	UE requirements for CA configurations within the same frequency group based on CBM [NR_RF_FR2_req_enh2-Core]

R4-2100142	IBM RF requirements for CA configurations within same frequency group

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2100621	CBM requirements for DLCA band combinations from the same frequency group

Type: other		For: Approval
					38.101-2 v..
					Source: Qualcomm Incorporated

Abstract:

Provides a framework for CBM requirements for DLCA band combinations from the same frequency group

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2102608	Inter-band DL CA within same frequency group based on CBM

Type: other		For: Approval
					38.101-2 v..
					Source: Apple

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

##### 11.3.2.2	Inter-band UL CA [NR_RF_FR2_req_enh2-Core]

###### 11.3.2.2.1	UE requirements for CA configuration CA_n257A-n259A based on IBM [NR_RF_FR2_req_enh2-Core]

R4-2100697	Views on inter-band UL CA power class definition

Type: discussion		For: Approval
					Source: MediaTek Beijing Inc.

Abstract:

Proposal1: For min requirement of “simultaneous uplink in multiple bands”, “per UE” concept shall be applied.

Proposal2: Clarify whether apply power sharing mechanism for FR2 inter-band UL CA firstly, before discuss the details about based on “EIRP” or “

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2101202	UE requirements for FR2 UL Inter-band CA

Type: other		For: Approval
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2101283	Consideration on inter-band UL CA RF requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

R4-2101374	The MOP and Tx requirements for inter-band UL CA in FR2

Type: discussion		For: Approval
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 in R4-2103326.

Decision: 		The document was noted.

#### 11.3.3	Feasibility study [NR_RF_FR2_req_enh2-Core]

R4-2100637	Discussion on feasibility for inter-band CA  configurations

Type: discussion		For: Discussion
					Source: LG Electronics

Abstract:

It discusses feasibility for inter-band CA  configurations.

Discussion:

See email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1 in R4-2103325.

Decision: 		The document was noted.

##### 11.3.3.1	Inter-band DL CA enhancements [NR_RF_FR2_req_enh2-Core]

R4-2102714	Simulation and analysis of FR2 inter-band DL CA based on CBM/IBM

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1 in R4-2103325.

Decision: 		The document was noted.

###### 11.3.3.1.1	Feasibility study for CA configurations within same frequency group based on IBM [NR_RF_FR2_req_enh2-Core]

R4-2100893	Discussion on IBM inter-band CA within same frequency group

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1 in R4-2103325.

Decision: 		The document was noted.

R4-2101375	The IBM UE capability for inter-band CA within the same frequency group

Type: discussion		For: Approval
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1 in R4-2103325.

Decision: 		The document was noted.

###### 11.3.3.1.2	Feasibility study for CA configurations between different frequency groups based on CBM [NR_RF_FR2_req_enh2-Core]

R4-2100240	On the feasibility of CBM for FR2 inter-band CA cross different frequency groups

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1 in R4-2103325.

Decision: 		The document was noted.

R4-2101376	The CBM UE capability for inter-band CA between different frequency groups

Type: discussion		For: Approval
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1 in R4-2103325.

Decision: 		The document was noted.

##### 11.3.3.2	Inter-band UL CA [NR_RF_FR2_req_enh2-Core]

R4-2102715	Discussion on FR2 inter-band UL CA

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1 in R4-2103325.

Decision: 		The document was noted.

###### 11.3.3.2.1	Feasibility study for CA configurations within same frequency group based on IBM and CBM [NR_RF_FR2_req_enh2-Core]

R4-2100619	Definition of TRP and EIRP for FR2 ULCA

Type: other		For: Approval
					38.101-2 v..
					Source: Qualcomm Incorporated

Abstract:

Provides definitions for TRP and EIRP for FR2 ULCA

Discussion:

See email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1 in R4-2103325.

Decision: 		The document was noted.

###### 11.3.3.2.2	Feasibility study for CA configurations between different frequency groups based on CBM [NR_RF_FR2_req_enh2-Core]

#### 11.3.4	UL gaps for self-calibration and monitoring [NR_RF_FR2_req_enh2-Core]

R4-2102988	Email discussion summary for [98e][140] NR_RF_FR2_req_enh2_Part_3

Type: other		For: discussion
					Source: Moderator (Apple)

Discussion:

The contribution summarised email discussion thread [98e][140] NR_RF_FR2_req_enh2_Part_3. The discussion thread was moderated by Yang Tang (Apple).

Decision: 		The document was revised to R4-2103327.

R4-2103327	Email discussion summary for [98e][140] NR_RF_FR2_req_enh2_Part_3

Type: other		For: discussion
					Source: Moderator (Apple)

(Replaces R4-2102988)

Discussion:

The contribution summarised email discussion thread [98e][140] NR_RF_FR2_req_enh2_Part_3. The discussion thread was moderated by Yang Tang (Apple).

Decision: 		The document was noted.

R4-2103114	Way forward on UL gap for FR2 RF

Type: other		For: discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][140] NR_RF_FR2_req_enh2_Part_3 in R4-2103327.

Decision: 		The document was approved.

R4-2100825	Performance evaluation for calibration

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][140] NR_RF_FR2_req_enh2_Part_3 in R4-2103327.

Decision: 		The document was noted.

##### 11.3.4.1	Gap use cases and performance evaluation [NR_RF_FR2_req_enh2-Core]

R4-2100218	UL gaps for Tx power management

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][140] NR_RF_FR2_req_enh2_Part_3 in R4-2103327.

Decision: 		The document was noted.

R4-2101129	Discussion on the UL Gap

Type: discussion		For: Discussion
					Source: LG Electronics Finland

Abstract:

Use cases and metrics are further discussed in this contribution.

Discussion:

See email discussion summary for [98e][140] NR_RF_FR2_req_enh2_Part_3 in R4-2103327.

Decision: 		The document was noted.

R4-2101200	Consideration on FR2 UL gap for self calibration and monitering

Type: other		For: Approval
					Source: NTT DOCOMO INC.

Discussion:

See email discussion summary for [98e][140] NR_RF_FR2_req_enh2_Part_3 in R4-2103327.

Decision: 		The document was noted.

R4-2101467	Discussion on UL gap for self-calibration and monitoring

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][140] NR_RF_FR2_req_enh2_Part_3 in R4-2103327.

Decision: 		The document was noted.

R4-2102623	on gaps for self-calibration and monitoring

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][140] NR_RF_FR2_req_enh2_Part_3 in R4-2103327.

Decision: 		The document was noted.

R4-2102680	Power calibration gap UE improvement requirements

Type: discussion		For: Agreement
					Source: Ericsson, Sony

Abstract:

This paper disusses the performance iprovements required for UEs supporting PCGs

Discussion:

See email discussion summary for [98e][140] NR_RF_FR2_req_enh2_Part_3 in R4-2103327.

Decision: 		The document was noted.

##### 11.3.4.2	Others [NR_RF_FR2_req_enh2-Core]

R4-2100144	FR2 UL gaps for self-calibration and monitoring

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][140] NR_RF_FR2_req_enh2_Part_3 in R4-2103327.

Decision: 		The document was noted.

R4-2100217	UL gaps for tranceiver calibration

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][140] NR_RF_FR2_req_enh2_Part_3 in R4-2103327.

Decision: 		The document was noted.

R4-2100599	UL calibration gap continuation

Type: other		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][140] NR_RF_FR2_req_enh2_Part_3 in R4-2103327.

Decision: 		The document was noted.

#### 11.3.5	RRM core requirements  [NR_RF_FR2_req_enh2-Core]

##### 11.3.5.1	Inter-band DL CA enhancements [NR_RF_FR2_req_enh2-Core]

R4-2101077	Discussion on FR2 inter-band DL CA enhancements

Type: discussion		For: Approval
					Source: NEC

Abstract:

We provide our views on some of the FR2 enhancements and MRTD requirement for FR2 inter-band CA

Discussion:

See email discussion summary for [98e][230] NR_RF_FR2_req_enh2_RRM in R4-2103711.

Decision: 		The document was noted.

R4-2101266	RRM requirements for inter-band DL CA in NR FR2

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][230] NR_RF_FR2_req_enh2_RRM in R4-2103711.

Decision: 		The document was noted.

R4-2101540	Discussion on RRM requirements for FR2 inter-band DL CA enhancements

Type: discussion		For: Discussion
					Source: OPPO

Discussion:

See email discussion summary for [98e][230] NR_RF_FR2_req_enh2_RRM in R4-2103711.

Decision: 		The document was noted.

R4-2101686	Discussion on RRM impacts for FR2 inter-band DL CA enhancement in Rel-17

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][230] NR_RF_FR2_req_enh2_RRM in R4-2103711.

Decision: 		The document was noted.

R4-2101867	Support up to 3 us MRTD

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we develop why at least 3us MRTD is feasible from both from a network perspective and a UE perspective, for co-located deployments.

Discussion:

See email discussion summary for [98e][230] NR_RF_FR2_req_enh2_RRM in R4-2103711.

Decision: 		The document was noted.

R4-2101868	Updates on MRTD requirements for FR2 inter-band DL CA

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1688  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

Updates on MRTD requirements for FR2 inter-band DL CA

Discussion:

See email discussion summary for [98e][230] NR_RF_FR2_req_enh2_RRM in R4-2103711.

Decision: 		The document was postponed.

R4-2102267	Discussion on FR2 RF RRM

Type: discussion		For: Discussion
					38.133 v..
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][230] NR_RF_FR2_req_enh2_RRM in R4-2103711.

Decision: 		The document was noted.

##### 11.3.5.2	Inter-band UL CA [NR_RF_FR2_req_enh2-Core]

R4-2101687	Discussion on RRM impacts for  FR2 inter-band UL CA in Rel-17

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][230] NR_RF_FR2_req_enh2_RRM in R4-2103711.

Decision: 		The document was noted.

R4-2101869	Updates on MTTD requirements for FR2 inter-band DL CA

Type: CR		For: Agreement
					38.133 v17.0.0	  CR-1689  Cat: B (Rel-17)

					Source: Ericsson

Abstract:

Updates on MTTD requirements for FR2 inter-band DL CA

Discussion:

See email discussion summary for [98e][230] NR_RF_FR2_req_enh2_RRM in R4-2103711.

Decision: 		The document was postponed.

### 11.4	Further RRM enhancement for NR and MR-DC  [NR_RRM_enh2]

R4-2103470	Email discussion summary: [98e][231] NR_RRM_enh2_1

Type: other		For: discussion
					Source: Moderator (Apple)

Discussion:

The contribution summarised email discussion thread [98e][231] NR_RRM_enh2_1. The discussion thread was moderated by Jerry Cui (Apple).

Decision: 		The document was revised to R4-2103712.

R4-2103712	Email discussion summary: [98e][231] NR_RRM_enh2_1

Type: other		For: discussion
					Source: Moderator (Apple)

(Replaces R4-2103470)

Discussion:

The contribution summarised email discussion thread [98e][231] NR_RRM_enh2_1. The discussion thread was moderated by Jerry Cui (Apple).

Decision: 		The document was noted.

GTW session (February 01, 2021)

SRS antenna port switching

Issue 1-1-2: whether delay requirement would be defined in RRM for SRS antenna port switching

- Proposals

  - Option 2 (OPPO, NEC, Apple, Xiaomi, Intel, CATT, QC, Huawei?): No need to define SRS antenna port switching delay requirement in RRM.

  - Option 3 (Ericsson, MTK, Nokia): FFS: whether delay requirement would be defined in RRM for SRS antenna port switching

- Discussion

  - E///: this is the first meeting and prefer to keep it open and further double check

  - Nokia: we do not see necessity to define delay requirements. Wondering of other companies see any additional components. We are fine with Option 2.

  - MTK: we are ok to support Option 2.

  - E///: Typically we define delay and interruption requirements. Companies refer to SRS carrier switching requirements. Delay can be important for SRS antenna port switching.

  - QC: SRS antenna switching delay is already defined in RF standards. No additional requirements we need to define in RRM.

  - Apple: same view as QC. The delay is quite small and we cannot test it in RAN4.

  - Chair: is there is anything in RAN1?

    - Apple: RAN1 specified guard symbols.

  - E///: RF includes retuning time. RAN1 guard symbol is another thing. We would like to check if there is anything else. The delay will include at least RF retuning time.

  - QC: RF specs have a lot of ON/OFF masks which are not included in the RRM specs.

- Agreements

  - RRM delay requirement for SRS antenna port switching is FFS

    - Option 1: Do not define SRS antenna port switching delay requirement in RRM.

    - Option 2: Define SRS antenna port switching delay requirement same as RF retuning time.

    - Option 3: Define SRS antenna port switching delay requirement. FFS for the value. At least RF retuning time shall be included.

Issue 1-1-3: Impact of SRS antenna port switching to other RRM requirements

- Proposals

  - Option 1 (Apple, Huawei, OPPO, NEC, Xiaomi, Intel, CATT, QC, MTK): Take the SRS carrier switching as the starting point to identify the impact on other RRM requirements.

  - Option 2 (Ericsson): RAN4 to discuss the impact of SRS antenna port switching on timing measurements (e.g., UE Rx-Tx time difference, gNB Rx-Tx time difference, and UL RTOA measurements) and corresponding measurement requirements.

  - Option 3 (Ericsson):

- Take the SRS carrier switching as the starting point to identify the impact on other RRM requirements.

  - FFS: RAN4 to discuss the impact of SRS antenna port switching on timing measurements (e.g., UE Rx-Tx time difference, gNB Rx-Tx time difference, and UL RTOA measurements) and corresponding measurement requirements

  - Option 4 (Nokia): FFS on whether take the SRS carrier switching as the starting point to identify the impact on other RRM requirements

- Discussion

  - E///: we can combine Option 1 and 2

  - Nokia: SRS carrier switching and antenna port switching are different and not sure we can use SRS carrier switching can be used as a starting point

  - QC: using SRS carrier switching as baseline is fine. Need to identify first and define requirements as the second step.

  - Apple: agree with E/// suggestion

  - Intel: what is the impact on gNB requirements from UE SRS antenna port switching?

    - E///: UE is making transmission then gNB measurements may be affected.

    - Intel: need more to check if any enhancement to gNB requirements are needed. We need to be careful on the wording.

  - NEC: for timing measurements – is it only from the perspective of positioning measurements

    - E///: not necessarily related to positioning only

- Agreements

  - Further identify impact of SRS antenna port switching on RRM requirements, e.g.

    - Timing measurements and corresponding measurement requirements

    - Other RRM requirements

Issue 1-1-4: RAN4 defines the requirement only for SRS antenna port switching in FR1 or in both FR1 and FR2

- Proposals

  - Option 1 (QC, OPPO): only SRS antenna port switching in FR1 is considered

  - Option 2 (Apple, Xiaomi, MTK, Intel, NEC, E///): SRS antenna port switching in FR1 and FR2 are considered

  - Option 3 (Nokia, OPPO, Apple, LG, Xiaomi, vivo, Intel, CATT, QC, MTK): define the RRM requirements at SRS antenna switching only for FR1 unless the transient period in FR2 gets clarified in RF session (the scope of “RRM requirements” here depends on the conclusions from issue 1-1-1 and issue 1-1-2)

  - Option 4 (Huawei): wait for the conclusion from RAN1 and RF

- Discussion

  - NEC: Initially we considered FR1 and FR2. If there is no conclusion from RAN1 then we can descope FR2.

  - Huawei: There is some ongoing discussion in RAN1 for multi-panel case for FR2.

  - QC: we can extend Option 3 to include RAN1 conclusions.

  - E///: Prefer Option 2.

  - Chair: there is no RF scope in this WI. How do we trigger discussion on FR2?

    - Apple: Encourage delegates to check with RF experts. For RAN1 – also it is not clear what they discuss, since SRS antenna port switching was defined in Rel-15.

  - Apple: need to make decision to include at least for FR1.

  - QC: agree with Apple

  - NEC: we prefer not to delay the requirements for FR2.

  - QC: how do we start the discussion for FR2 if we don’t have RF

- Agreements

  - Define the RRM requirements for SRS antenna port switching for FR1.

  - FFS for FR2 SRS antenna port switching requirements:

    - Further identify the applicability of the existing RF transient period for SRS antenna port switching.

HO with PSCell

Issue 2-1-1: Scenarios for RRM requirement of HO with PSCell

- Proposals

  - Option 1(Apple, CMCC, NEC, Xiaomi, QC): RAN4 specifies RRM requirement for HO with PSCell for following scenarios:

    - from NR SA to EN-DC

    - from EN-DC to EN-DC

    - from NE-DC to NE-DC

    - from NR-DC to NR-DC

  - Option 2(Ericsson): RAN4 specifies RRM requirement for HO with PSCell for following scenarios:

    - from EN-DC to EN-DC

    - from NE-DC to NE-DC

    - from NR-DC to NR-DC

    - Use case for handover with PSCell between NR SA and EN-DC needs to be clarified and justified. Particularly, it needs to be clarified whether it is applicable to SA to EN-DC, SA to NGEN-DC, or both, and priority for development of RRM requirements shall be thereafter.

  - Option 3(HW): Consider the feasible scenarios for HO with PSCell configurations for:

    - NR to EN-DC （NR HO to LTE with NR PSCell configuration）

    - NR to NE-DC （NR HO to NR with LTE PSCell configuration）

    - NR to NR-DC（NR HO to LTE with NR PSCell configuration）

    - LTE to EN-DC （LTE HO to LTE with NR PSCell configuration）

    - NE-DC to NE-DC（NR HO to NR with LTE PSCell configuration）

    - NR-DC to NR-DC（NR HO to NR with NR PSCell configuration）

    - EN-DC to EN-DC（LTE HO to LTE with NR PSCell configuration）

  - Option 4 (HW, OPPO, Apple, Xiaomi, Intel, CATT, Ericsson, DCM, MTK, Nokia): RAN4 specifies RRM requirement for HO with PSCell for following scenarios:

    - from EN-DC to EN-DC

    - from NE-DC to NE-DC

    - from NR-DC to NR-DC

    - FFS on other scenarios

- Discussion

  - E///: “NR SA to EN-DC” is quite specific scenario. We prefer to look more into this.

  - CMCC: we support Option 1. SA – EN-DC scenario is already supported by RAN2 and they have already sent LS to RAN4. We simply need to specify the requirements.

    - E///: we are ok

  - CATT: for NR SA to EN-DC, what do we mean by “with PSCell”. Is it like HO with PSCell addition?

    - Apple: yes. Definition introduced by RAN2.

    - Huawei: “with PSCell” means that PSCell configuration comes in the same IE as HO

- Agreements

  - Define RRM requirement for HO with PSCell for following scenarios:

    - from NR SA to EN-DC

    - from EN-DC to EN-DC

    - from NE-DC to NE-DC

    - from NR-DC to NR-DC

    - FFS on other scenarios

Issue 2-2-3: timeline for HO with PSCell

- Proposals

  - Option 1 (ZTE, Apple, Xiaomi): PCell HO and PSCell addition is performed in a sequential order.

  - Option 2 (CATT, CMCC, QC, Huawei, OPPO, Intel, DCM, MTK, Nokia): PCell HO and PSCell addition is performed in parallel.

  - Option 2a (NEC): cell search can be performed in parallel and TA acquisition and application on PCell, RRC reconfig complete on PCell and RACH to PSCell can be in sequential order

  - Option 3 (Huawei): RAN4 should discuss whether the procedures could be performed in parallel based on the existing requirements.

- Discussion

  - Apple: Prefer Option 1. This is the worst case.

  - QC: it was RAN2 intention that PCell HO and PSCell addition can be performed in parallel

  - Huawei: there are multiple procedures included in HO and PSCell addition and need to discuss case by case

  - Xiaomi: Option 1. Do not consider additional RRC processing time. Synch procedure may need sequential processing.

  - NEC: we are ok with Option 2. Option 2a is a compromise between Option 1 and 2.

  - Nokia: UE should perform procedures in parallel in general. For some components we can further check if sequential processing is needed.

  - E///: Agree with Huawei.

  - OPPO: the motivation is to shorten timeline. No additional RRC processing time is needed. At least same RRC processing time is needed. Ok with Huawei proposal.

  - DOCOMO: Prefer Option 2.

- Agreement

  - Identify the detailed components of “HO with PSCell” procedure

    - Further discuss whether the procedures could be performed in parallel or sequentially based on the existing requirements.

R4-2103471	Email discussion summary: [98e][232] NR_RRM_enh2_2

Type: other		For: discussion
					Source: Moderator (CATT)

Discussion:

The contribution summarised email discussion thread [98e][232] NR_RRM_enh2_2. The discussion thread was moderated by Qiuge Guo (CATT).

Decision: 		The document was revised to R4-2103713.

R4-2103713	Email discussion summary: [98e][232] NR_RRM_enh2_2

Type: other		For: discussion
					Source: Moderator (CATT)

(Replaces R4-2103471)

Discussion:

The contribution summarised email discussion thread [98e][232] NR_RRM_enh2_2. The discussion thread was moderated by Qiuge Guo (CATT).

Decision: 		The document was noted.

GTW session (February 01, 2021)

Issue 1-1-0: Whether CSI report of PUCCH SCell is transmitted on PCell or PUCCH SCell to be activated?

- Proposals:

  - Option 1:

    - PUCCH of PCell.

  - Option 2:

    - PUCCH of PUCCH SCell to be activated.

- Discussion

  - Apple: It is up to network. Both Option 1 and 2 are possible

  - Huawei: It depends on how we define the ending point. Option 2 is more reasonable

  - E///: Both options are possible.

  - Nokia: Does the “CSI report” refer to the first valid report? The first valid report shall be transmitted in PUCCH SCell

  - QC: we have same understanding as Apple and Huawei. We need to discuss details of both.

  - CATT: Agree with Huawei. Both Option 1 and 2 are possible.

  - CMCC: Same question as Nokia. If this refers to the valid CSI report and it is transmitted in PCell, then do we need to consider invalid TA case?

    - Apple: PUCCH on PCell – UE will report out of range and when UE completes the procedure UE will report a valid value. For PUCCH on SCell – it is different and UE will transmit a valid CQI from the first occasion.

- Agreements

  - Further study the procedures for the following 2 scenarios

    - CSI report of PUCCH SCell is transmitted on PUCCH PCell

    - CSI report of PUCCH SCell is transmitted on PUCCH SCell to be activated

    - FFS whether to define requirements for both cases.

Issue 1-1-4: Whether the beam information is needed for NW to initiate the RA for TA updating by a PDCCH order?

- Proposals:

  - Option 1: (Huawei, Apple, Qualcomm, OPPO, NTT DOCOMO, MTK, Nokia)

    - The beam information is needed for NW to initiate the RA for TA updating by a PDCCH order

  - Option 2:  (Ericsson)

    - Depends on what activation sequence we are assuming.

- Discussion

  - E///: we can agree with Option 1. But it depends on how the NW gets this information

  - Nokia: RA procedure is initiate by UE.

  - NEC: it depends on the ending point for issues 1-1-0. If CSI report is sent on PCell then we do not need RA at all.

    - QC: if TA is invalid then UE needs to establish UL synch first. This is not related to where UE makes CSI report.

    - Apple: If CQI is on PCell we still needed RACH on target SCell. We need to discuss the ending point.

    - CATT: do we have common understanding on the ending point?

    - Apple: need further discussion

  - QC: the definition of beam information is unclear.

    - Huawei: this is SSB index. SSB index should be explicitly indicated by the NW

- Agreements

  - The beam information (SSB index) is needed for NW to initiate the PDCCH order to trigger RA

#### 11.4.1	General and work plan [NR_RRM_enh2-Core]

#### 11.4.2	RRM core requirements  [NR_RRM_enh2-Core]

##### 11.4.2.1	SRS antenna port switching  [NR_RRM_enh2-Core]

R4-2103672	WF on further RRM enhancement for NR and MR-DC - SRS antenna port switching

Type: other		For: discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was approved.

R4-2104048	LS on SRS antenna port switching

Type: LS out		For: Approval
					to -
					Source: Nokia

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2100192	On SRS antenna port switching

Type: discussion		For: Discussion
					38.133 v..
					Source: Apple

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2100400	Discussion on SRS antenna port switching

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2100634	SRS antenna switch discussion

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2100655	Discussion on interruption due to SRS antenna port switching

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2100726	Discussion on SRS antenna switching RRM requirements

Type: discussion		For: Discussion
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2101078	Discussion on SRS antenna port switching

Type: discussion		For: Approval
					Source: NEC

Abstract:

We analyze the interruption requirements for SRS antenna port switching

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2101223	Discussion on SRS antenna port switching

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2101379	Considerations on SRS antenna port switching

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2101392	Discussion on the interruption requirements at SRS antenna port switching

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2101414	Discussion on SRS antenna port switching

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2101656	Discussion on requirements for SRS antenna switching

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2102534	On RRM requirements for SRS antenna port switching

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On RRM requirements for SRS antenna port switching

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

##### 11.4.2.2	HO with PSCell [NR_RRM_enh2-Core]

R4-2103673	WF on further RRM enhancement for NR and MR-DC

Type: other		For: discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was approved.

R4-2103674	LS on handover with PSCell

Type: LS out		For: Approval
					to RAN2
					Source: RAN4

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was approved.

R4-2100114	Discussion on handover with PSCell

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2100193	On RRM requirement for handover with PSCell

Type: discussion		For: Discussion
					38.133 v..
					Source: Apple

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2100401	Discussion on HO with PSCell

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2100710	Discussion on RRM requirements for handover with PSCell

Type: discussion		For: Discussion
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2100867	Discussion on HO with PSCell

Type: discussion		For: Approval
					Source: CMCC

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2101079	Discussion on PSCell HO

Type: discussion		For: Approval
					Source: NEC

Abstract:

We analyze the requirements for HO with PSCell

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2101112	Views on HO with PSCell

Type: discussion		For: Discussion
					38.133 v..
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2101415	Discussion on HO with PSCell

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2101657	Discussion on requirements for HO with PSCell

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2102364	On handover with PSCell

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on handover with PSCell

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

R4-2102625	Views on specifying the requirements for HO with PSCell

Type: discussion		For: (not specified)
					38.133 v..
					Source: Qualcomm CDMA Technologies

Abstract:

Views on what to consider when discussing the requirements for HO with PSCell

Discussion:

See email discussion summary for [98e][231] NR_RRM_enh2_1 in R4-2103712.

Decision: 		The document was noted.

##### 11.4.2.3	PUCCH SCell activation/deactivation [NR_RRM_enh2-Core]

R4-2103675	WF on further RRM enhancement for NR and MR-DC - PUCCH SCell activation/deactivation requirements

Type: other		For: discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][232] NR_RRM_enh2_2 in R4-2103713.

Decision: 		The document was approved.

R4-2100194	On PUCCH SCell activation and deactivation

Type: discussion		For: Discussion
					38.133 v..
					Source: Apple

Discussion:

See email discussion summary for [98e][232] NR_RRM_enh2_2 in R4-2103713.

Decision: 		The document was noted.

R4-2100402	Discussion on PUCCH SCell activationdeactivation

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][232] NR_RRM_enh2_2 in R4-2103713.

Decision: 		The document was noted.

R4-2100711	Discussion on SCell activation and deactication requirements for PUCCH Scell

Type: discussion		For: Discussion
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][232] NR_RRM_enh2_2 in R4-2103713.

Decision: 		The document was noted.

R4-2100872	Discussion on PUCCH SCell activation/deactivation

Type: discussion		For: Approval
					Source: CMCC

Discussion:

See email discussion summary for [98e][232] NR_RRM_enh2_2 in R4-2103713.

Decision: 		The document was noted.

R4-2101046	Discussions on PUCCH SCell Activation/Deactivation delay requirements

Type: discussion		For: Discussion
					38.133 v..
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][232] NR_RRM_enh2_2 in R4-2103713.

Decision: 		The document was noted.

R4-2101080	Discussion on PUCCH SCell activation

Type: discussion		For: Approval
					Source: NEC

Abstract:

We analyze the requirements for PUCCH SCell activation/deactivation for single and multiple SCells

Discussion:

See email discussion summary for [98e][232] NR_RRM_enh2_2 in R4-2103713.

Decision: 		The document was noted.

R4-2101380	Considerations on PUCCH SCell activation and deactivation

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][232] NR_RRM_enh2_2 in R4-2103713.

Decision: 		The document was noted.

R4-2101391	Discussion on the activation and deactivation delay requirements for PUCCH SCell

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][232] NR_RRM_enh2_2 in R4-2103713.

Decision: 		The document was noted.

R4-2101536	Views on RRM requirements for PUCCH SCell Activation/Deactivation

Type: discussion		For: Discussion
					Source: OPPO

Discussion:

See email discussion summary for [98e][232] NR_RRM_enh2_2 in R4-2103713.

Decision: 		The document was noted.

R4-2101658	Discussion on requirements for PUCCH SCell activation

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][232] NR_RRM_enh2_2 in R4-2103713.

Decision: 		The document was noted.

R4-2102365	On SCell (de)activation with PUCCH

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on SCell activation and deactivation for PUCCH SCell.

Discussion:

See email discussion summary for [98e][232] NR_RRM_enh2_2 in R4-2103713.

Decision: 		The document was noted.

R4-2102892	Discussion on PUCCH SCell Activation

Type: discussion		For: Discussion
					38.133 v..
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][232] NR_RRM_enh2_2 in R4-2103713.

Decision: 		The document was noted.

### 11.5	NR and MR-DC measurement gap enhancements [NR_MG_enh]

R4-2103472	Email discussion summary: [98e][233] NR_MG_enh_1

Type: other		For: discussion
					Source: Moderator (MediaTek)

Discussion:

The contribution summarised email discussion thread [98e][233] NR_MG_enh_1. The discussion thread was moderated by Ato Yu (ZTE).

Decision: 		The document was revised to R4-2103714.

R4-2103714	Email discussion summary: [98e][233] NR_MG_enh_1

Type: other		For: discussion
					Source: Moderator (MediaTek)

(Replaces R4-2103472)

Discussion:

The contribution summarised email discussion thread [98e][233] NR_MG_enh_1. The discussion thread was moderated by Ato Yu (ZTE).

Decision: 		The document was noted.

1st round email discussion conclusions

Issue 1-1: Work plan

Session chair: Rapporteurs are encouraged to reflect extended Rel-17 timelines in the WID in the next plenary. The work plan can be updated afterwards.

GTW session (February 02, 2021)

Issue 2-1: Definition of concurrent gaps

- Proposals

  - Option 1: (ZTE)

    - Two MGs are considered concurrent if they overlap with each other partly or completely

  - Option 2: (LGE)

    - Multiple same MG pattern IDs with different MG offset

    - Different MG pattern IDs with different MG offset

    - Single MG pattern ID with multiple MG offsets

  - Option 3: (Intel, Huawei)

    - The gap patterns defined in Rel16 can be reused for the gap instances being included in the multiple concurrent gap pattern.

- Recommended WF for the 2nd round

  - Concurrent gaps are multiple MG patterns that are configured during a common period of time,

    - Gap patterns are selected from at least Rel-16 gap patterns #0 to #23.

    - Note: The definition can be further revised in the future based on consensus

- Discussion

  - E///: We have 26 gaps. #24 and #25 were added in the NR Pos WI. Prefer not to specify the exact set of gaps.

  - Nokia: Also need to take into account whether the MGs are actively used by UE to perform the measurements. It is early to preclude any MG patterns.

  - Intel: Concurrent pattern = multiple configured patterns with concurrent measurements cycles. MG patterns for PRS measurements shall not be precluded.

  - MTK: Agree with MTK, E///, Nokia to keep all MG patterns. To Nokia – our understanding that multiple configured MGs are used for measurements.

  - QC: need to clarify that the MGs shall be active (i.e. UE is performing the measurements in the gaps)

  - NEC: Need to include offset into the definition. Multiple MGs have different offsets

  - Huawei: this is also relevant to the next issue (independent gaps). Agree with NEC that offset shall be considered. Remove set of patterns. All configured MGs are active and no need to mention this.

  - LGE: Need to reduce performance degradation due to multiple MGs. This is the reason to preclude some MG patterns.

  - Apple: we need to guarantee the some parameters for the configured MGs shall be different.

  - CATT: This is also relevant to independent gap definition. What is the “common period of time”. Does this mean overlapping / non-overlapping?

  - vivo: Agree with CATT that concurrent/independent MGs shall be discussed jointly. Need to clarify what concurrent means (e.g. MGL overlap). MG for NR positioning should be kept.

  - OPPO: Rel-16 MG patterns can be reused. Pre-configured gaps are not precluded.

- Agreements

  - Concurrent MG definition

    - Concurrent MGs are multiple MGs that are configured for measurements during a common period of time

      - Exact definition of common period of time is FFS

      - UE behavior for non-overlapping, partially or fully overlapped cases is irrelevant to the definition and will be discussed separately.

      - Note 1: current definition does not address pre-configured MG patterns and NCSG. FFS how to address pre-configured MG patterns and NCSG.

  - Concurrent MG patterns

    - MG patterns are selected from Rel-16 gap patterns #0 to #25.

R4-2103473	Email discussion summary: [98e][234] NR_MG_enh_2

Type: other		For: discussion
					Source: Moderator (Intel Corporation)

Discussion:

The contribution summarised email discussion thread [98e][234] NR_MG_enh_2. The discussion thread was moderated by Rui Huang (Intel).

Decision: 		The document was revised to R4-2103715.

R4-2103715	Email discussion summary: [98e][234] NR_MG_enh_2

Type: other		For: discussion
					Source: Moderator (Intel Corporation)

(Replaces R4-2103473)

Discussion:

The contribution summarised email discussion thread [98e][234] NR_MG_enh_2. The discussion thread was moderated by Rui Huang (Intel).

Decision: 		The document was noted.

GTW session (February 02, 2021)

Issue 1-0-0: General procedures of pre-configured MG

- Proposals

  - Option 1. The basic procedures for pre-configured MG include:

    - Step 1. Configuration of the pre-configured MG

    - Step 2. Activation the pre-configured MG when BWP switching

    - Step 3. Deactivation the pre-configured MG

- Discussion

  - OPPO: Concern on the 2nd step. Activation can happen without BWP switching.

  - E///: BWP switching should trigger activation. Removing BWP switching will broaden the scope.

  - Huawei: Does this imply any specific order?

    - Intel: no specific order

  - MTK: Is pre-configured MG same or different to legacy MG? For activation we need to consider MO configuration

  - Xiaomi: do we need another procedure for activation of the pre-configured MG after the RRC configuration

- Agreements

  - Further study the following procedures for pre-configured MGs:

    - 1. (Re)Configuration of the pre-configured MG

      - FFS if specific procedure for activation after the RRC configuration is needed

    - 2. Activation the pre-configured MG following a DCI or timer-based BWP switch

    - 3. Deactivation the pre-configured MG following a DCI or timer-based BWP switch

    - Note 1: The conditions and details of each procedure are FFS

    - Note 2: MG activation in this context means that both NW and UE assume that the pre-configured MG will be used for measurements.

    - Note 3: MG deactivation in this context means that both NW and UE assume that the pre-configured MG will not be used for measurements and UE should be able to receive scheduled data.

Issue 2-1-1 NCSG scenarios

- Proposals

  - Option 1 (Intel): RAN4 can prioritize the following NR NCSG using scenario: Eliminate/reduce interruptions to the serving carriers due to RF chain states transition when measuring the deactivated SCells

  - Option 1a (Qualcomm, Intel): use cases of NCSG can be two scenarios,

    - When measurement gap is not configured at all, NCSG can be explicitly provided to UE for minimal interruptions on a serving carrier while an idle chain is employed for measurement.

    - When measurement gap is configured for some carriers but not the others, NCSG can be implicitly configured on serving carriers, where the serving carrier can be PCC or SCC.

  - Option 1b (Ericsson, Intel, Nokia):

    - Case 1: If NCSG is configured then the interruptions on PCell, PSCell or activated SCell(s) due to measurements on PCell, PSCell, activated SCell, deactivated SCell, SCell with dormant BWP or unused RF chain shall not occur outside the visible interruption length before measurement (VIL1) and the visible interruption length after measurement (VIL2).

    - Case 2: If NCSG is pre-configured then after switching from non-dormant BWP to dormant BWP on a SCell, then interruptions on PCell, PSCell or activated SCell(s) due to measurements on the SCell with dormant BWP shall not occur outside the visible interruption length before measurement (VIL1) and the visible interruption length after measurement (VIL2).

    - Case 3: For UE capable of per UE gaps, NCSG pattern can be configured to avoid interruptions provided that the UE is not configured with any legacy gap pattern defined in Table 9.1.2-1, TS 38.133.

    - Case 4: For UE capable of per FR gaps:

      - NCSG pattern cannot be configured in parallel with any legacy gap pattern (defined in Table 9.1.2-1, TS 38.133) on the same FR.

      - NCSG pattern can be configured on an FR to avoid interruptions provided that on the same FR the UE is not configured with any legacy gap pattern defined in Table 9.1.2-1, TS 38.133.

    - Case 5: If UE capable of NCSG and per UE gaps is configured with any legacy gap pattern defined in Table 9.1.2-1, TS 38.133 and there is no inter-frequency or inter-RAT carrier to monitor, then the UE shall not cause any interruption PCell, PSCell or activated SCells due to measurements on PCell, PSCell or SCells.

    - Case 6: If UE capable of NCSG and per FR gaps is configured with any legacy gap pattern defined in Table 9.1.2-1, TS 38.133 on an FR and there is no inter-frequency or inter-RAT carrier to monitor on that FR, then the UE shall not cause any interruption PCell, PSCell or activated SCells on that FR due to measurements on PCell, PSCell or SCells of that FR.

  - Option 2. (MTK): Intra-frequency measurements with MG, inter-frequency measurements with MG or Inter-RAT measurements may use NCSG instead of MG when UE supports related band combination and have additional RF chains during the measurements

  - Option 3. (Huawei): Support per UE and per FR NCSG for RRM measurement based on UE capability reporting (e.g. extension to Rel-16 NeedForGap signaling).

  - Option 4. (Apple):  When UE is to measure an intra-band target cell, it is sometimes feasible for UE to enlarge BW of the RF chain to cover target SSB

  - Option 5. (Nokia): NCSG should address similar aspects as was addressed in LTE

- Discussion

  - MTK: This is should be very similar to LTE scenarios. It can be used as baseline

  - E///: This can be applicable to different periodic measurements to avoid interruption on Serving cell. This also includes measurements on SCell Dormancy

  - Huawei: Prefer to utilize to minimize interruptions due to MG-based measurements. For SCell dormancy CQI measurements, we need to have more time to check

  - Apple: Prefer not to include any assumptions to the RF chain. There may be different UE implementations. Need to further discuss if per-CC gap is in the scope

  - Intel: Need to have a high-level criteria 1) UE performs MG-based measurements in another CC 2) UE has capability to perform additional measurements using another RF chain (or can use the same chain to make additional measurements)

  - QC: the main use case is to avoid interruptions on the serving cell. UE has spare RF chain depending on the BC. Per-CC can be discussed separately.

  - Nokia: LTE should be the baseline. Option 1b includes SCell dormancy and should be further considered.

R4-2103676	WF on R17 NR MG enhancements - Multiple concurrent and independent MG patterns

Type: other		For: discussion
					Source: MediaTek

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was revised to R4-2104096.

R4-2104096	WF on R17 NR MG enhancements - Multiple concurrent and independent MG patterns

Type: other		For: discussion
					Source: MediaTek

(Replaces R4-2103676)

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was approved.

R4-2103677	WF on R17 NR MG enhancements

Type: other		For: discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was approved.

#### 11.5.1	General and work plan [NR_MG_enh-Core]

R4-2101061	Work plan of R17 NR and MR-DC measurement gap enhancements WI

Type: discussion		For: Approval
					Source: MediaTek Inc., Intel Corporation

Discussion:

See email discussion summary for [98e][233] NR_MG_enh_1 in R4-2103714.

Decision: 		The document was noted.

#### 11.5.2	RRM core requirements  [NR_MG_enh-Core]

##### 11.5.2.1	Pre-configured MG pattern(s)  [NR_MG_enh-Core]

R4-2100221	Consideration on preconfigured measurement gap patterns

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2100454	Initial discussion on pre-configured MG pattern

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2100712	Discussion on pre-configured MG pattern for NR

Type: discussion		For: Discussion
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2100871	Discussion on pre-configured MG pattern(s)

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2101062	Pre-configured MG pattern(s) per configured BWP

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2101269	Discussion on pre-configured measurement gap

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2101381	Considerations on pre-configured MG patterns

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2101537	Views on pre-configured MG pattern(s) for NR_MG_enh

Type: discussion		For: Discussion
					Source: OPPO

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2102268	Discussion on Pre-configured MG pattern(s)

Type: discussion		For: Discussion
					38.133 v..
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2102622	Views on pre-configured MG patterns

Type: discussion		For: (not specified)
					38.133 v..
					Source: Qualcomm CDMA Technologies

Abstract:

Views on necessary issues for clarifications on preconfigured MG

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2102655	Overview of requirements for pre-configured measurement gaps

Type: other		For: Discussion
					Source: Ericsson

Abstract:

This document analysis RRM requirements for pre-configured MG in NR and MR-DC

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2102810	Initial discussion on (de)activation of pre-configured MGs

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

##### 11.5.2.2	Multiple concurrent and independent MG patterns [NR_MG_enh-Core]

R4-2100113	Discussion on independent and concurrent MGs

Type: discussion		For: Discussion
					Source: ZTE Corporation

Abstract:

This paper discusses some conceptual issues related to concurrent MGs.

Discussion:

See email discussion summary for [98e][233] NR_MG_enh_1 in R4-2103714.

Decision: 		The document was noted.

R4-2100222	Discussion on multiple concurrent and independent MG patterns

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][233] NR_MG_enh_1 in R4-2103714.

Decision: 		The document was noted.

R4-2100455	Initial discussion on multiple concurrent and independent MG patterns

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][233] NR_MG_enh_1 in R4-2103714.

Decision: 		The document was noted.

R4-2100641	Discussion on multiple concurrent and independent MG patterns

Type: discussion		For: Discussion
					Source: LG Electronics

Abstract:

It discusses multiple concurrent and independent MG patterns.

Discussion:

See email discussion summary for [98e][233] NR_MG_enh_1 in R4-2103714.

Decision: 		The document was noted.

R4-2100713	Discussion on multiple concurrent and independent MG patterns for NR

Type: discussion		For: Discussion
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][233] NR_MG_enh_1 in R4-2103714.

Decision: 		The document was noted.

R4-2100870	Discussion on multiple concurrent and independent MG patterns

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][233] NR_MG_enh_1 in R4-2103714.

Decision: 		The document was noted.

R4-2101063	Multiple concurrent and independent gap patterns

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][233] NR_MG_enh_1 in R4-2103714.

Decision: 		The document was noted.

R4-2101081	Discussion on NR measurement gap enhancements requirements

Type: discussion		For: Approval
					Source: NEC

Abstract:

We discuss the principle for defining requirements for multiple concurrent and independent MG patterns

Discussion:

See email discussion summary for [98e][233] NR_MG_enh_1 in R4-2103714.

Decision: 		The document was noted.

R4-2101270	Discussion on multiple and independent concurrent measurement gaps in NR

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][233] NR_MG_enh_1 in R4-2103714.

Decision: 		The document was noted.

R4-2101538	Views on Multiple concurrent and independent MG patterns for NR_MG_enh

Type: discussion		For: Discussion
					Source: OPPO

Discussion:

See email discussion summary for [98e][233] NR_MG_enh_1 in R4-2103714.

Decision: 		The document was noted.

R4-2102269	Discussion on multiple concurrent and independent MG patterns

Type: discussion		For: Discussion
					38.133 v..
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][233] NR_MG_enh_1 in R4-2103714.

Decision: 		The document was noted.

R4-2102297	On requirements for mulitple concurrent and independent MG patterns

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][233] NR_MG_enh_1 in R4-2103714.

Decision: 		The document was noted.

R4-2102535	On parallel measurement gap patterns

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On parallel measurement gap patterns

Discussion:

See email discussion summary for [98e][233] NR_MG_enh_1 in R4-2103714.

Decision: 		The document was noted.

R4-2102811	Initial discussion on multiple concurrent MGs

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][233] NR_MG_enh_1 in R4-2103714.

Decision: 		The document was noted.

##### 11.5.2.3	Network Controlled Small Gap [NR_MG_enh-Core]

R4-2100223	On network controlled small gap

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2100456	Initial discussion on Network Controlled Small Gap (NCSG)

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2100460	CR on NCSG in 38.133

Type: draftCR		For: Endorsement
					38.133 v17.0.0
					Source: CATT

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was postponed.

R4-2101064	Network Controlled Small Gap

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2101271	Discussion on NCSG in NR

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2101382	Considerations on network controlled small gap

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2101539	Views on pre-configured MG pattern(s) for NR_MG_enh

Type: discussion		For: Discussion
					Source: OPPO

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2102611	Discussion on network controlled small gap

Type: discussion		For: (not specified)
					38.133 v..
					Source: Qualcomm CDMA Technologies

Abstract:

Views on requirements and issues suggested for clarifications on NCSG

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2102656	Overview of requirements for network controlled small gap

Type: other		For: Discussion
					Source: Ericsson

Abstract:

This document analysis RRM requirements for NCSG in NR and MR-DC

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2102689	On Introduction of Network Controlled Small Gaps for NR

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Discussion on introduction of NCSG for NR

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

R4-2102812	Initial discussion on NCSG

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][234] NR_MG_enh_2 in R4-2103715.

Decision: 		The document was noted.

### 11.6	Enhancement for NR high speed train scenario in FR1 [NR_HST_FR1_enh-Core]

#### 11.6.1	General and work plan [NR_HST_FR1_enh-Core]

#### 11.6.2	RRM core requirements  [NR_HST_FR1_enh-Core]

R4-2103474	Email discussion summary: [98e][235] NR_HST_FR1_enh_RRM

Type: other		For: discussion
					Source: Moderator (CMCC)

Discussion:

The contribution summarised email discussion thread [98e][235] NR_HST_FR1_enh_RRM. The discussion thread was moderated by Jingjing Chen (China Mobile Com. Corporation).

Decision: 		The document was revised to R4-2103716.

R4-2103716	Email discussion summary: [98e][235] NR_HST_FR1_enh_RRM

Type: other		For: discussion
					Source: Moderator (CMCC)

(Replaces R4-2103474)

Discussion:

The contribution summarised email discussion thread [98e][235] NR_HST_FR1_enh_RRM. The discussion thread was moderated by Jingjing Chen (China Mobile Com. Corporation).

Decision: 		The document was noted.

GTW session (February 03, 2021)

Issue 2-1-2: enhancement on PSS/SSS detection for deactivated Scell, intra-frequency measurement without measurement gap

- Proposals

  - Option 1: similar enhancement to PCell measurement in R16 HST can be used as baseline for PSS/SSS detection enhancement on deactivated Scell.

  - Option 2:

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | 5 x measCycleSCell x CSSFintra |
| With DRX | Max(800ms, 5 x max(measCycleSCell, DRX cycle)) x CSSFintra |

- Discussion

  - QC: Originally proposed Option 2 but can support Option 1.

  - E///: Option 1.

  - Huawei: Will Kp issue be fixed in Option 1? Will we apply same agreements for HST?

  - Nokia: Kp factor is under discussion in Rel-16.

  - CMCC: Kp is under discussion in R16 maintenance. Need to check on Kp.

  - Apple: support Huawei view that Kp is necessary.

- Agreement

  - Similar enhancement to PCell measurement in R16 HST can be used as baseline for PSS/SSS detection enhancement on deactivated Scell.

    - FFS whether Rel-16 Kp requirements modifications (if any) shall also apply for R17 HST

Issue 2-1-3: enhancement on time index detection for deactivated Scell, intra-frequency measurement without measurement gap

- Proposals

  - Option 1: similar enhancement to PCell measurement in R16 HST can be used as baseline for time index detection enhancement on deactivated Scell.

  - Option 2:

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | 3 x measCycleSCell x CSSFintra |
| With DRX | 3 x max(measCycleSCell, DRX cycle)) x CSSFintra |

- Discussion

  - QC: ok with Option 1

- Agreement

  - Similar enhancement to PCell measurement in R16 HST can be used as baseline for time index detection enhancement on deactivated Scell.

    - FFS whether Rel-16 Kp requirements modifications (if any) shall also apply for R17 HST

Issue 2-1-4: enhancement on measurement period for deactivated Scell, intra-frequency measurement without measurement gap

- Proposals

  - Option 1: similar enhancement to PCell measurement in R16 HST can be used as baseline for measurement period enhancement on deactivated Scell.

  - Option 2 (Qualcomm):

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | 5 x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Max(800ms, 5 x max(measCycleSCell, DRX cycle)) x CSSFintra |
| DRX cycle> 320ms | 3 x max(measCycleSCell, DRX cycle) x CSSFintra |

- Discussion

  - QC: New option is to add scaling factor of M2 in DRX cycle less or equal to 320ms. Replace 3 by Y for DRX cycle > 320 ms. M2 and Y are defined for PCell measurements.

  - Apple: Is it correct understanding that M2 does not apply for No DRX? If so we are fine with QC proposal

    - QC: yes

  - CMCC: not ok with QC proposal. For 160ms DRX in R16 HST WI we have 4 samples. Based on QC proposal the number of samples is relaxed to 5.

  - E///: ok with Option 1. Open to further evaluate QC proposal.

  - Nokia: need to check

- Agreements

  - Further study the following solutions

    - Option 1: similar enhancement to PCell measurement in R16 HST can be used as baseline for measurement period enhancement on deactivated Scell.

    - Option 2:

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | 5 x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | ceil(M2 x 5) x max(measCycleSCell, DRX cycle) x CSSFintra |
| DRX cycle> 320ms | Y x max(measCycleSCell, DRX cycle) x CSSFintra |

  - FFS whether Rel-16 Kp requirements modifications (if any) shall also apply for R17 HST

Issue 2-2-1: if the issue on cell identification requirements for SCell for non-HST scenario in R15 exists (as described in R4-2101707), for Rel-17 FR1 HST, how to specify the PSS/SSS detection, time index detection, and measurement period for deactivated SCell for the scenario of intra-f measurement with MG to support HST.

- Background: The issue on cell identification requirements for deactivated SCell with MG for non-HST scenario is discussed in Rel-15 maintenance AI. Based on the 1st round discussion of email thread #201, companies share the same understanding that deactivated SCell measurement does not belong to intra-frequency measurement with MG, since deactivated SCell does not have active BWP.  In the GTW discussion of email thread #201, following agreement was reached:

  - Not to introduce the intra-frequency de-activated SCell measurement requirement with MG.

- Recommended WF

  - Companies are encouraged to check whether following suggestion is OK.

    - Following the same approach in non-HST scenario, for deactivated SCell, no need to consider the case of intra-frequency measurement with measurement gap.

- Discussion

- Agreements

  - Do not introduce the intra-frequency de-activated SCell measurement requirement with MG.

Issue 4-2-1: except the requirements related with Scell activation/deactivation, other requirements to be discussed on whether to be enhanced to support HST with CA

- Proposals

  - Option 1: The requirements of timing, interruption, UL carrier RRC reconfiguration delay, link recovery (BFD/CBD), CSSF, L1-RSRP measurement and measurement accuracy for L1-SINR measurement shall be further discussed in the work item of HST with CA.

- Discussion

  - QC: would like to hear more justifications

  - MTK: Timing, Interruption, L1-RSRP, RRC reconfiguration delay requirements can be reused. Some requirements cover multiple carriers and should be analyzed (CSSF, link recovery for SCell, L1-SINR measurements).

  - Huawei: Would like to hear more justification on timing? Timing is related to velocity and UL BW. No impact from CA.

    - MTK: timing can be reused

  - Apple: do not see good justification but open to discuss and encourage companies to bring more analysis.

R4-2103678	WF on FR1 HST RRM requirements

Type: other		For: discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][235] NR_HST_FR1_enh_RRM in R4-2103716.

Decision: 		The document was approved.

##### 11.6.2.1	UE RRM core requirements for CA scenario [NR_HST_FR1_enh-Core]

R4-2100224	On R17 FR1 HST RRM measurement requirement

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][235] NR_HST_FR1_enh_RRM in R4-2103716.

Decision: 		The document was noted.

R4-2100475	Discussion on CA for NR FR1 HST

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][235] NR_HST_FR1_enh_RRM in R4-2103716.

Decision: 		The document was noted.

R4-2100633	FR1 HST RRM discussion

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Discussion:

See email discussion summary for [98e][235] NR_HST_FR1_enh_RRM in R4-2103716.

Decision: 		The document was noted.

R4-2100857	General discussion on NR HST RRM enhancement for FR1 CA scenario

Type: discussion		For: Approval
					Source: CMCC

Discussion:

See email discussion summary for [98e][235] NR_HST_FR1_enh_RRM in R4-2103716.

Decision: 		The document was noted.

R4-2101140	On SCell RRM enhancement for NR high speed train scenario in FR1

Type: discussion		For: Discussion
					38.133 v..
					Source: Ericsson

Abstract:

Discuss Scell  RRM enhancement for high speed train in FR1

Discussion:

See email discussion summary for [98e][235] NR_HST_FR1_enh_RRM in R4-2103716.

Decision: 		The document was noted.

R4-2101141	On SCell RRM enhancement for NR high speed train scenario in FR1

Type: draftCR		For: Endorsement
					38.133 v17.0.0
					Source: Ericsson

Abstract:

Discuss Scell  RRM enhancement for high speed train in FR1

Discussion:

See email discussion summary for [98e][235] NR_HST_FR1_enh_RRM in R4-2103716.

Decision: 		The document was postponed.

R4-2101173	Discussion on high speed train for CA in FR1

Type: discussion		For: Discussion
					38.133 v..
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][235] NR_HST_FR1_enh_RRM in R4-2103716.

Decision: 		The document was noted.

R4-2101707	Discussion on Enhancement for NR high speed train scenario in FR1

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][235] NR_HST_FR1_enh_RRM in R4-2103716.

Decision: 		The document was noted.

R4-2102863	On RRM enhancements for HST scenarios for FR1 CA

Type: other		For: Discussion
					38.133 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

The document discusses the measurement requirement for SCells under HST scenarios.

Discussion:

See email discussion summary for [98e][235] NR_HST_FR1_enh_RRM in R4-2103716.

Decision: 		The document was noted.

#### 11.6.3	UE demodulation requirements (38.101-4) [NR_HST_FR1_enh-Perf]

##### 11.6.3.1	General  [NR_HST_FR1_enh-Perf]

R4-2103767	Email discussion summary for [98e][328] NR_HST_FR1_Demod

Type: other		For: discussion
					Source: Moderator (CMCC)

Discussion:

The contribution summarised email discussion thread [98e][328] NR_HST_FR1_Demod. The discussion thread was moderated by Xiaoran Zhang (China Mobile Com. Corporation).

Decision: 		The document was revised to R4-2103947.

R4-2103947	Email discussion summary for [98e][328] NR_HST_FR1_Demod

Type: other		For: discussion
					Source: Moderator (CMCC)

(Replaces R4-2103767)

Discussion:

The contribution summarised email discussion thread [98e][328] NR_HST_FR1_Demod. The discussion thread was moderated by Xiaoran Zhang (China Mobile Com. Corporation).

Decision: 		The document was noted.

R4-2103876	WF on FR1 HST demodulation

Type: other		For: discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][329] in R4-2103947.

Decision: 		The document was approved.

##### 11.6.3.2	PDSCH requirements for CA scenarios [NR_HST_FR1_enh-Perf]

R4-2100858	General discussion on NR HST UE demodulation for FR1 CA scenario

Type: discussion		For: Approval
					Source: CMCC

Discussion:

See email discussion summary for [98e][329] in R4-2103947.

Decision: 		The document was noted.

R4-2101260	Views on NR HST CA PDSCH performance requirements

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][329] in R4-2103947.

Decision: 		The document was noted.

R4-2101308	Discussion on PDSCH CA scenarios for NR UE HST FR1 performance requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][329] in R4-2103947.

Decision: 		The document was noted.

R4-2101370	Views on HST CA tests for FR1

Type: discussion		For: Discussion
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][329] in R4-2103947.

Decision: 		The document was noted.

R4-2101439	PDSCH demodulation requirements for CA with HST-SFN scenario

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution shows our view on the PDSCH demodulation requirements for CA with HST-SFN scenario.

Discussion:

See email discussion summary for [98e][329] in R4-2103947.

Decision: 		The document was noted.

##### 11.6.3.3	Enhanced transmission schemes  [NR_HST_FR1_enh-Perf]

R4-2100859	Discussion on NR HST UE demodulation for enhanced transmission scheme

Type: discussion		For: Approval
					Source: CMCC

Discussion:

See email discussion summary for [98e][329] in R4-2103947.

Decision: 		The document was noted.

R4-2101261	Views on NR HST PDSCH performance requirements for multi-DCI based Tx scheme

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][329] in R4-2103947.

Decision: 		The document was noted.

R4-2101309	Discussion on enhanced transmission schemes for NR HST demodulation

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][329] in R4-2103947.

Decision: 		The document was noted.

R4-2101440	PDSCH demodulation requirements with enhanced transmission schemes in HST scenario

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution shows our view on the PDSCH demodulation requirements with enhanced transmission schemes in HST scenario.

Discussion:

See email discussion summary for [98e][329] in R4-2103947.

Decision: 		The document was noted.

### 11.7	NR support for high speed train scenario in FR2 [NR_HST_FR2_enh]

R4-2102989	Email discussion summary for [98e][141] NR_HST_FR2_enh

Type: other		For: discussion
					Source: Moderator (Samsung)

Discussion:

The contribution summarised email discussion thread [98e][141] NR_HST_FR2_enh. The discussion thread was moderated by He Wang (Samsung Electronics Czech).

Decision: 		The document was revised to R4-2103328.

R4-2103328	Email discussion summary for [98e][141] NR_HST_FR2_enh

Type: other		For: discussion
					Source: Moderator (Samsung)

(Replaces R4-2102989)

Discussion:

The contribution summarised email discussion thread [98e][141] NR_HST_FR2_enh. The discussion thread was moderated by He Wang (Samsung Electronics Czech).

Decision: 		The document was noted.

R4-2103240	Way forward on Deployment Scenario and UE RF Requirement for FR2 HST

Type: other		For: discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was approved.

#### 11.7.1	General and work plan [NR_HST_FR2_enh-Core]

R4-2102103	HST FR2 general aspects

Type: other		For: Approval
					Source: Ericsson

Abstract:

Considers general work needed to determine supportable speed, including demod

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was noted.

R4-2102266	NR support for high speed train scenario in frequency range 2 (FR2)

Type: other		For: Approval
					38.133 v..
					Source: Nokia, Nokia Shanghai Bell, Samsung

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was revised to R4-2103239.

R4-2103239	NR support for high speed train scenario in frequency range 2 (FR2)

Type: other		For: Approval
					38.133 v..
					Source: Nokia, Nokia Shanghai Bell, Samsung

(Replaces R4-2102266)

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was approved.

#### 11.7.2	High speed train deployment scenario in FR2 [NR_HST_FR2_enh-Core]

R4-2100631	FR2 HST general discussion

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was noted.

R4-2100915	Further discussion on high speed train deployment scenario in FR2

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was noted.

R4-2100916	Simulation results and analysis on HST deployment scenario in FR2

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was noted.

R4-2101267	Discussion on deployment scenarios for NR HST in FR2

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was noted.

R4-2101368	Discussion on deployment scenarios for FR2 HST

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was noted.

R4-2101856	Deployment scenarios for HST FR2

Type: discussion		For: Discussion
					Source: ZTE Wistron Telecom AB

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was noted.

R4-2102093	Simulation results for HST in FR2

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was noted.

R4-2102099	On high-speed train deployment scenario in FR2

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was noted.

R4-2102104	HST FR2 deployment aspects

Type: other		For: Approval
					Source: Ericsson

Abstract:

Considers antenna, link budget, deployment etc.

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was noted.

#### 11.7.3	UE RF core requirements [NR_HST_FR2_enh-Core]

R4-2100918	Discussion on UE RF requirement for FR2 HST

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was noted.

R4-2102561	Power Class 4 for HST

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was noted.

R4-2102679	Consideration on UE requirements for FR2 HST

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This paper discusses the need for spherical coverage requirements for HST FR2 UEs as well as max output pwr

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was noted.

R4-2100632	FR2 HST RRM discussion

Type: discussion		For: Discussion
					Source: Qualcomm, Inc.

Discussion:

See email discussion summary for [98e][141] NR_HST_FR2_enh in R4-2103328.

Decision: 		The document was not treated.

#### 11.7.4	RRM core requirements [NR_HST_FR2_enh-Core]

R4-2103475	Email discussion summary: [98e][236] NR_HST_FR2_enh_RRM

Type: other		For: discussion
					Source: Moderator (Nokia)

Discussion:

The contribution summarised email discussion thread [98e][236] NR_HST_FR2_enh_RRM. The discussion thread was moderated by Dmitry Petrov (Nokia).

Decision: 		The document was revised to R4-2103717.

R4-2103717	Email discussion summary: [98e][236] NR_HST_FR2_enh_RRM

Type: other		For: discussion
					Source: Moderator (Nokia)

(Replaces R4-2103475)

Discussion:

The contribution summarised email discussion thread [98e][236] NR_HST_FR2_enh_RRM. The discussion thread was moderated by Dmitry Petrov (Nokia).

Decision: 		The document was noted.

GTW session (February 02, 2021)

Issue 2-1: Idle/inactive mode

- Proposals

  - Option 1 (Nokia, QC, Samsung): IDLE/INACTIVE state mobility is not applicable for Rel-17 HST FR2 deployment.

  - Option 2 (Ericsson): Enhancement in idle/inactive mode maybe is not prioritized.

  - Option 3: Need to consider IDLE/INACTIVE mode in HST FR2 deployment.

- Discussion

  - Samsung: Based on WID this WI focuses on train-mounted CPE. Power saving is not critical and we can preclude IDLE/INACTIVE modes.

  - Intel: Agree that most of time UE will be in a CONNECTED mode. How do we skip the procedures for IDLE/INACTIVE? What would be UE behavior and requirements in case it is still in IDLE/INACTIVE mode? What is UE behavior in case of connection failure?

  - E///: IDLE mode may not be typical but still can happen. Agree with Intel that UE behavior needs to be clarified. If UE looses the connection then UE needs to get to the IDLE mode. Need to further discuss how to handle this.

  - QC: Do not fully agree with Option 1. IDLE/INACTIVE modes can happen but the key question is whether we need to discuss any enhancements. We prefer no enhancement for IDLE/INACTIVE mode mobility requirements.

  - Huawei: For Option 1 does it mean that we don’t have any enhancements or does it mean that we don’t apply existing requirements?

  - Nokia: QC option seem to be a reasonable WF.

  - Samsung: we don’t plan to change the basic NR design. We are talking on the RRM requirements and their applicability. Option 1 wording may cause some confusion. We can consider to reuse the R16 requirements but not define any enhancements. Some requirements should be in place.

  - E///: At least we need to support existing requirements. Prefer not to preclude enhancements at this moment.

  - Intel: In case we keep Rel-16 requirements, would it mean that UE will be able to reconnect only in low mobility conditions?

    - Samsung: yes

- Agreements

  - Idle/Inactive mode requirements

    - Option 1: Reuse existing Rel-16 requirements

    - Option 2: Study and define enhancements to support FR2 HST conditions

Issue 2-2: DRX mode

- Proposals

  - Option 1 (Nokia, Ericsson, Huawei, CATT, Samsung): Do not include DRX mode in the requirements for a CPE operating in HST mode in FR2.

  - Option2 (Intel): DRX mode in the requirements for a CPE operating in HST mode in FR2 might be needed.

- Discussion

  - Huawei: There may be 2 interpretations for Option 1 - A) define requirements for the case of no DRX for connected mode B) define requirements for DRX but without any enhancements. We prefer “1A”.

  - Intel: We are ok 1A.

  - QC: DRX mode is not very common use case. No enhancements for DRX mode are needed. We prefer not preclude DRX mode but Rel-16 requirements would apply.

  - Samsung: 1A. The question is whether network is allowed to configure the DRX mode. We may need to clarify what happens in this case.

  - E///: We can still consider short DRX. We prefer not to restrict NW configuration.

  - Nokia: DRX is related to Connected mode. Option 1 in our interpretation means that DRX is not configured by the network.

  - Samsung: Not sure why we need short DRX and long DRX. DRX is not needed for power saving.

  - Nokia: what is the purpose of short DRX? Is it power saving?

- Agreements

  - RRC CONNECTED mode requirements for DRX

    - Option 1: Do not define enhanced requirements for the case DRX is configured

      - Option 1A: Legacy NR R16 requirements (non-HST) will apply for the case DRX is configured

      - Option 1B: No RRM requirements will be defined for the case DRX is configured

    - Option 2: Define requirements for the short DRX configurations (e.g. up to 80ms).

R4-2103679	WF on Rel-17 NR HST FR2 RRM Requirements

Type: other		For: discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][236] NR_HST_FR2_enh_RRM in R4-2103717.

Decision: 		The document was approved.

R4-2100220	Discussion on RRM requirement for high speed train scenario in FR2

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][236] NR_HST_FR2_enh_RRM in R4-2103717.

Decision: 		The document was noted.

R4-2100476	Discussion on NR FR2 HST

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][236] NR_HST_FR2_enh_RRM in R4-2103717.

Decision: 		The document was noted.

R4-2100477	Initial performance evaluation for cell identification in NR FR2 HST scenario

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][236] NR_HST_FR2_enh_RRM in R4-2103717.

Decision: 		The document was noted.

R4-2100866	General discussion on RRM requirements for FR2 HST

Type: discussion		For: Approval
					Source: CMCC

Discussion:

See email discussion summary for [98e][236] NR_HST_FR2_enh_RRM in R4-2103717.

Decision: 		The document was noted.

R4-2100917	Discussion on RRM requirement for FR2 HST

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][236] NR_HST_FR2_enh_RRM in R4-2103717.

Decision: 		The document was noted.

R4-2101138	On expected RRM impact for HST in FR2

Type: discussion		For: Discussion
					Source: Nokia

Abstract:

Discussion about RRM sections to have potential impact due to the introduciton of HST in FR2.

Discussion:

See email discussion summary for [98e][236] NR_HST_FR2_enh_RRM in R4-2103717.

Decision: 		The document was noted.

R4-2101142	Overview of RRM requirements for NR high speed train scenario in FR2

Type: discussion		For: Discussion
					38.133 v..
					Source: Ericsson

Abstract:

an overview of which spec. should be considered in order to support FR2 HST scenario.

Discussion:

See email discussion summary for [98e][236] NR_HST_FR2_enh_RRM in R4-2103717.

Decision: 		The document was noted.

R4-2101268	RRM requirements for NR HST in FR2

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][236] NR_HST_FR2_enh_RRM in R4-2103717.

Decision: 		The document was noted.

R4-2101708	Preliminary discussion on NR support for high speed train scenario in FR2

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][236] NR_HST_FR2_enh_RRM in R4-2103717.

Decision: 		The document was noted.

### 11.8	Solutions for NR to support non-terrestrial networks (NTN) [NR_NTN_solutions]

#### 11.8.1	General and work plan [NR_NTN_solutions-Core]

R4-2103749	Email discussion summary for [98e][310] NTN_Solutions_Part1

Type: other		For: discussion
					Source: Moderator (Thales)

Discussion:

The contribution summarised email discussion thread [98e][310] NTN_Solutions_Part1. The discussion thread was moderated by Dorin Panaitopol (THALES).

Decision: 		The document was revised to R4-2103948.

R4-2103948	Email discussion summary for [98e][310] NTN_Solutions_Part1

Type: other		For: discussion
					Source: Moderator (Thales)

(Replaces R4-2103749)

Discussion:

The contribution summarised email discussion thread [98e][310] NTN_Solutions_Part1. The discussion thread was moderated by Dorin Panaitopol (THALES).

Decision: 		The document was noted.

R4-2103877	WF for NTN general part

Type: other		For: discussion
					Source: Thales

Discussion:

See email discussion summary for [98e][310] NTN_Solutions_Part1 in R4-2103948.

Decision: 		The document was agreed.

R4-2103750	Email discussion summary for [98e][311] NTN_Solutions_Part2

Type: other		For: discussion
					Source: Moderator (Samsung)

Discussion:

The contribution summarised email discussion thread [98e][311] NTN_Solutions_Part2. The discussion thread was moderated by Yiran Jin (Samsung).

Decision: 		The document was revised to R4-2103949.

R4-2103949	Email discussion summary for [98e][311] NTN_Solutions_Part2

Type: other		For: discussion
					Source: Moderator (Samsung)

(Replaces R4-2103750)

Discussion:

The contribution summarised email discussion thread [98e][311] NTN_Solutions_Part2. The discussion thread was moderated by Yiran Jin (Samsung).

Decision: 		The document was noted.

------------GTW Note on Feb.5th-------------

Thales: Both options fine for all, but we prefer to cover more mixed scenarios as much as possible.

E///: We would like to focus on Rual/Urban macro and then evaluate what’s the worst cases?

Nokia: We need to identify the worst cases. We prefer to also consider HAPS scenario.

Samsung: We prefer to GEO/LEO-600 which can be worst cases and also rual/urban macro cases.

Thales: We are fine to Samsung proposal.

CATT: The purpose was for alignment?  Probably need to down-scope of different satellite types without simulation.

Hughes/Echostar: We still believe to keep LEO 1200km cases.

QC:  We agree to use phase by phase approach. We didn’t what’s the worst cases.

Huawei: in FR1, we assume handled UE type, we are not clear for GEO.

Thales: In TR 38.821, there are some results we probably can be considered.

Agreements:

RAN4 agreed to take following scenarios for initial simulation alignment purpose in Q2 2021:

TN deployment: NR only with Rual, Urban Macro

NTN deployment: GEO, LEO-600, LEO-1200, HAPS

Satellite Set: Set1

The detailed set as following:

| No. | Frq. | TN | TN scenario | NTN | Prioritize (Selected set for simulator alignment in Q2’ 2021) |
| --- | --- | --- | --- | --- | --- |
| 1 | 2GHz | NR | Rural | GEO | THALES, Nokia |
| 2 | 2GHz | NR | Rural | LEO 600km | THALES, Nokia |
| 3 | 2GHz | NR | Rural | LEO 1200km |  |
| 4 | 2GHz | NR | Urban macro | GEO | THALES, Nokia |
| 5 | 2GHz | NR | Urban macro | LEO 600km | THALES, Nokia |
| 6 | 2GHz | NR | Urban macro | LEO 1200km |  |
| 25 | 2GHz | NR | Rural | HAPS | Nokia |
| 26 | 2GHz | NR | Urban macro | HAPS | Nokia |

| No. | Frq. | NTN | NTN |
| --- | --- | --- | --- |
| 1 | 2GHz | GEO | GEO |
| 2 | 2GHz | GEO | LEO 600km |
| 5 | 2GHz | LEO 600km | LEO 600km |
| 9 | 2GHz | LEO 1200km | LEO 1200km |
| 10 | 2GHz | HAPS | HAPS |

Companies are encouraged to bring simulation results at least for above cases in Q2’ 2021 for simulator alignment purpose.

-------------------------End------------------------

R4-2103878	WF for NTN co-existence study

Type: other		For: discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was approved.

R4-2103965	Simulation assumption for NTN co-existence study

Type: other		For: discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was revised to R4-2103998.

R4-2103998	Simulation assumption for NTN co-existence study

Type: other		For: discussion
					Source: CATT

(Replaces R4-2103965)

Discussion:

Session Chair Note: This simulation assumption was for simulation alignment purpose; companies are encouraged to follow such simulation assumption to provide results in April RAN4 meeting.

See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was noted.

R4-2101813	Discussion on exemplary bands for NTN topic

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][310] NTN_Solutions_Part1 in R4-2103948.

Decision: 		The document was noted.

R4-2102175	NTN Reference model

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution is proposing specification structure for the introduction of NTN

Discussion:

See email discussion summary for [98e][310] NTN_Solutions_Part1 in R4-2103948.

Decision: 		The document was noted.

#### 11.8.2	Use cases, deployment scenarios, and regulatory information [NR_NTN_solutions-Core]

R4-2100399	Discussion on frequency band and scenarios for NTN

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][310] NTN_Solutions_Part1 in R4-2103948.

Decision: 		The document was noted.

R4-2100824	Examplary bands for NTN

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][310] NTN_Solutions_Part1 in R4-2103948.

Decision: 		The document was noted.

R4-2100905	Views on NTN exemplary bands

Type: discussion		For: Agreement
					Source: Samsung

Discussion:

See email discussion summary for [98e][310] NTN_Solutions_Part1 in R4-2103948.

Decision: 		The document was noted.

R4-2101814	General discussion on Network structure on NTN topics

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][310] NTN_Solutions_Part1 in R4-2103948.

Decision: 		The document was noted.

R4-2101858	Criteria for Choosing FR1 Exemplary Band

Type: discussion		For: Decision
					Source: THALES

Abstract:

In order to select a proper exemplary MSS FR1 band (in terms in bandwidth, throughput and regulatory requirements) this document proposes the criteria to be taken into account for exemplary FR1 band selection, to be considered by RAN4 work.

Discussion:

See email discussion summary for [98e][310] NTN_Solutions_Part1 in R4-2103948.

Decision: 		The document was noted.

R4-2101933	NTN - On use cases and deployment scenarios

Type: discussion		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][310] NTN_Solutions_Part1 in R4-2103948.

Decision: 		The document was noted.

R4-2102173	NTN - Regulatory and spectrum aspects

Type: other		For: Approval
					Source: Ericsson

Abstract:

Based on Radio Regulations analysis, this contribution is discussing NTN spectrum aspects

Discussion:

See email discussion summary for [98e][310] NTN_Solutions_Part1 in R4-2103948.

Decision: 		The document was noted.

R4-2102374	Discussion on satellite bands outside FR1/FR2 range for NR based satellite networks

Type: discussion		For: Discussion
					Source: HUGHES Network Systems, Thales, Inmarsat, Intelsat, Fraunhofer, ESA

Discussion:

See email discussion summary for [98e][310] NTN_Solutions_Part1 in R4-2103948.

Decision: 		The document was noted.

R4-2100111	NTN architecture aspects

Type: discussion		For: Decision
					Source: THALES

Discussion:

Session Chair: Move to this AI from 11.8.3 See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was noted.

#### 11.8.3	Coexistence aspects [NR_NTN_solutions-Core]

R4-2101859	NTN FR1 Coexistence Scenarios and Related Core Requirements

Type: discussion		For: Decision
					Source: THALES

Abstract:

This contribution refers to MSS FR1 band possible interference situations to be considered by RAN4 coexistence studies in adjacent bands.

Discussion:

See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was noted.

R4-2101880	Simulations for NTN FR1 Coexistence Cases

Type: discussion		For: Decision
					Source: THALES

Abstract:

This contribution proposes to down-scope coexistence scenarios to be considered for simulations, e.g. consider only NTN extreme cases.

Discussion:

See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was noted.

##### 11.8.3.1	Simulation assumptions [NR_NTN_solutions-Core]

R4-2100486	Simulaiton assumptions for NTN co-existence

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was noted.

R4-2100904	Simulation assumption for FR1 coexistence study

Type: discussion		For: Agreement
					Source: Samsung

Discussion:

See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was noted.

R4-2101105	Coexistence study on NR to support non-terrestrial networks

Type: other		For: Approval
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was noted.

R4-2101812	General discussion on NTN simulation assumptions

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was noted.

R4-2101934	NTN - HAPS simulation assumptions for co-existence study

Type: discussion		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was noted.

R4-2101964	Discussion on simulation assumptions for NTN coexistence study

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was noted.

R4-2102174	NTN Simulations assumptions discussion

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution provides an overview of the needed simulations for NTN and initiates related discussions

Discussion:

See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was noted.

R4-2102508	Simulation assumptions for NR NTN co-existence study

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was noted.

##### 11.8.3.2	UE requirements aspects [NR_NTN_solutions-Core]

##### 11.8.3.3	BS requirements aspects [NR_NTN_solutions-Core]

R4-2100487	Consideration on BS requirement impact for NTN

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was noted.

R4-2101935	NTN - HAPS adjacent channel coexistence

Type: discussion		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was noted.

R4-2102176	NTN - BS requirements overview

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution is giving an overview of candidate BS RF requirements for NTN

Discussion:

See email discussion summary for [98e][311] NTN_Solutions_Part2 in R4-2103949.

Decision: 		The document was noted.

#### 11.8.4	RRM core requirements [NR_NTN_solutions-Core]

R4-2103476	Email discussion summary: [98e][237] NR_NTN_solutions_RRM

Type: other		For: discussion
					Source: Moderator (Fraunhofer HHI)

Discussion:

The contribution summarised email discussion thread [98e][237] NR_NTN_solutions_RRM. The discussion thread was moderated by Mathis Schmieder (Fraunhofer HHI).

Decision: 		The document was revised to R4-2103718.

R4-2103718	Email discussion summary: [98e][237] NR_NTN_solutions_RRM

Type: other		For: discussion
					Source: Moderator (Fraunhofer HHI)

(Replaces R4-2103476)

Discussion:

The contribution summarised email discussion thread [98e][237] NR_NTN_solutions_RRM. The discussion thread was moderated by Mathis Schmieder (Fraunhofer HHI).

Decision: 		The document was noted.

GTW session (February 01, 2021)

Issue 1-1: Send information LS to RAN1 regarding reference point to be considered for time and frequency synchronization

- Proposals

  - R4-2101864 (Ericsson): Proposal 1: Sent information LS to RAN1 with RAN4 implications for different reference points.

  - Proposal for GTW: Ericsson should clarify content of LS

- Discussion

  - Moderator: most companies prefer to wait.

  - E///: We provided LS draft.

  - Chair: is RAN1 aware that they will need to define reference point

    - E///: Yes. The idea is provide information on possible RAN4 implications.

  - MTK: LS needs further discussion to identify on possible RAN4 impacts.

  - Xiaomi: Prefer not to send LS to RAN1. These are purely RAN1 issues.

  - Apple: Same view as MTK

  - CMCC: same view with MTK.

  - QC: this is already addressed in RAN1 discussion.

  - Session chair: Defer the LS. Further discuss the impacts of different reference points on RRM requirements and inform RAN1 in case any common observations are identified.

Issue 1-2: Possibility of using satellite and gNB as time and frequency reference

- Proposals

  - R4-2101865 (Ericsson): Proposal 1: RAN4 to investigate the impact on existing gNB requirements for the cases when satellite and gNB is time and frequency reference.

  - Options considered in 1st round discussion:

    - Option A: satellite and gNB is time and frequency reference

    - Option B: satellite is frequency and gNB is time reference

    - Option C: satellite is time reference and gNB is frequency reference

  - Proposal: RAN4 should discuss which of the options to investigate, and send a LS to RAN1 with RAN4 input if necessary.

- Discussion

  - E///: this depends on 1-1. We are ok to discuss.

  - QC: having reference point at gNB is not always possible.

  - Thales: for frequency the satellite can be chosen as the reference point, for timing – it can be either gNB or satellite.

  - Thales: For time RAN1 is still discussing this. For frequency – it is not decided yet.

  - QC: RAN1 already introduced some decisions on reference points which allow different scenarios for timing. For frequency – satellite will always be the reference.

  - E///: In our understanding RAN1 has not concluded yet.

  - Huawei: which exactly gNB requirements we need to check?

    - Xiaomi: gNB RRM requirements are out of scope of WID.

    - E///: nominal DL and UL timing delay.

- Agreements

  - Further investigate the impact of different timing and frequency reference points based on RAN1 design on the RRM requirements. Inform RAN1 if any issues are identified.

Issue 4-3: Impact of delay compensation on TA error

- Proposals

  - R4-2101541 (OPPO): Proposal 2: NTN delay compensation has impact on TA error.

  - Supporting companies should elaborate further in GTW

- Discussion

  - OPPO: TA error depends on delay compensation error.

  - CMCC: NTN delay compensation is UE specific TA estimation

  - MTK: In the requirements we have TA adjustment accuracy. Compensation error is always there and should be accounted in Te.

  - Huawei: this is related to other issues on Te and TA

  - Xiaomi: Same view as MTK. This is related to Te. RAN1 is discussing on the timing drift and already addressing it.

  - QC: we are ok with proposal but we need to discuss more details

  - Apple: NTN delay compensation will affect autonomous TA error (Te)

  - Thales: agree with MTK

  - Ericsson: Same view as MTK

  - OPPO: agree that it should be Te

  - Thales: we should differentiate Connected and Idle modes.

  - Huawei: suggest to replace “NTN delay compensation” with “UE-specific TA estimation”

  - Xiaomi: “UE-specific TA estimation” is a different issue.

  - Thales: “NTN delay compensation” we can replace with “NTN full TA estimation”

  - Huawei: does it mean that “UE-specific TA estimation” is a part of “NTN full TA estimation”

  - Thales: NTN full TA estimation includes “UE-specific TA estimation” + “Common TA”

  - Xiaomi: need to further check on the terminology

  - Thales: in the end we are talking that NTN timing compensation accuracy has impact on the UE timing accuracy

- Tentative agreements

  - NTN timing compensation accuracy has impact on Te timing error requirements for CONNECTED mode. FFS for IDLE mode.

Issue 6-3: Use of propagation delay information

- Proposals

  - R4-2100646 (LGE): Proposal 3: Consider propagation delay information from satellite/HAPS to configure SMTC or MG, and FFS for detail procedure.

  - Proposal for GTW: Further discuss if this is relevant to RAN4 and/or if a LS to RAN2 is necessary.

- Discussion

  - LGE: Due to propagation delay UE can miss the SMTC from the neighboring cell. Such information can be helpful for UE to perform neighbor cell measurements

  - QC: This is well-known issue in RAN2 and it is already under discussion. This is not in RAN4 discussion scope.

  - Xiaomi: It is up to RAN2 design.

  - MTK: Need more from RAN2.

  - LGE: we can further investigate in RAN4 and send LS to RAN2.

  - Session chair: recommend to wait for RAN2 conclusions.

R4-2103680	WF on NTN RRM requirements

Type: other		For: discussion
					Source: Fraunhofer

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was approved.

R4-2103681	WF on NTN RRM timing related requirements

Type: other		For: discussion
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was approved.

R4-2103682	WF on NTN RRM measurement requirements

Type: other		For: discussion
					Source: Qualcomm

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was approved.

R4-2101882	NTN PVT Accuracy Aspects

Type: discussion		For: Information
					Source: THALES

Abstract:

The goal of this document is to recall some basic principles required for the PVT (Position Velocity and Time) computation and distribution to UEs.

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was noted.

##### 11.8.4.1	General [NR_NTN_solutions-Core]

R4-2100780	Discussion on UE Pre-compensation for UL synchronization for in NTN

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was noted.

R4-2101864	Architecture and reference point

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion with draft LS for information from RAN4 to RAN1 about impact of different reference points.

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was noted.

R4-2102813	Discussion on general issues for NTN RRM

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was noted.

R4-2102893	Discussion on RRM in NTN Systems

Type: discussion		For: Discussion
					38.133 v..
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was noted.

##### 11.8.4.2	Timing requirements [NR_NTN_solutions-Core]

R4-2100647	Discussion on timing requirements for NTN

Type: discussion		For: Discussion
					Source: LG Electronics UK

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was noted.

R4-2100714	Discussion on timing requirements for NTN

Type: discussion		For: Discussion
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was noted.

R4-2100819	Discussion on NTN timing requirements

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was noted.

R4-2101541	Discussion on timing requirements for NR NTN RRM

Type: discussion		For: Discussion
					Source: OPPO

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was noted.

R4-2101865	RRM Timing requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

RRM timing requirements discussion and analysis.

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was noted.

R4-2102814	Discussion on NTN timing related requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was noted.

##### 11.8.4.3	Measurement requirements [NR_NTN_solutions-Core]

R4-2100646	Discussion on measurement requirements for NTN

Type: discussion		For: Discussion
					Source: LG Electronics UK

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was noted.

R4-2100715	Discussion on measurement requirements for NTN

Type: discussion		For: Discussion
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was noted.

R4-2100802	Discussion on NTN RRM measurement requirements

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was noted.

R4-2101712	Discussion on NTN measurement

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was noted.

R4-2101866	RRM Measurement Requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

RRM measurements requirements discussion and analysis.

Discussion:

See email discussion summary for [98e][237] NR_NTN_solutions_RRM in R4-2103718.

Decision: 		The document was noted.

### 11.9	UE Power Saving Enhancements [NR_UE_pow_sav_enh]

R4-2103477	Email discussion summary: [98e][238] NR_UE_pow_sav_enh_RRM

Type: other		For: discussion
					Source: Moderator (MediaTek)

Discussion:

The contribution summarised email discussion thread [98e][238] NR_UE_pow_sav_enh_RRM. The discussion thread was moderated by Hsuanli Lin (MediaTek).

Decision: 		The document was revised to R4-2103719.

R4-2103719	Email discussion summary: [98e][238] NR_UE_pow_sav_enh_RRM

Type: other		For: discussion
					Source: Moderator (MediaTek)

(Replaces R4-2103477)

Discussion:

The contribution summarised email discussion thread [98e][238] NR_UE_pow_sav_enh_RRM. The discussion thread was moderated by Hsuanli Lin (MediaTek).

Decision: 		The document was noted.

GTW session (February 03, 2021)

Issue 2-2-1: Confirmation on beneficial Scenarios, from UE power saving gain perspective

- Proposals

  - Option 1 (MTK): RAN4 to confirm that from UE power saving gain perspective, it is beneficial to relax SSB-based RLM/BFD measurement and CSI-RS based RLM/BFD measurement in both FR1 and FR2.

  - Option 2 (Vivo): RAN4 to conclude the exact power saving gain if RLM/BFD are relaxed in low mobility and/or high/medium SINR region.

    - RAN4 should strive to identify the scenarios that can achieve power saving gain when RLM/BFD are relaxed. (Vivo)

    - The RSs for RLM/BFD, especially the periodicity/bandwidth of these RSs and the relation to RSs for RRM, need careful consideration in R17 RLM/BFD relaxation. (Vivo)

  - Option 3 (Huawei): RAN4 to study the power saving gain level at which RLM/BFD measurement relaxation is considered as feasible.

    - When the power saving gain level is higher than the threshold, it is considered that the benefit is big enough to support RLM/BFD measurement relaxation.

    - To study the threshold is useful for RAN4 to identify the feasible scenarios.

  - Recommended WF:

    - To capture the summary of simulation results for power saving gain in WF

- 1st round summary

  - All companies are fine to capture the summary of simulation results for power saving gain in WF. However, some clarification questions were raised as follows.

    - Q1: Is the simulation results for delta PDCCH?

    - Q2: To clarify if in their simulations they have relaxed also RRM measurements and/or L1-RSRP measurements or not

- Discussion

  - Nokia: not sure how extended RRM measurement period should be handled in this WI. What is the justification to do it? It should be very clearly clarified on the respective assumptions in the simulation assumptions. It has major impact on power saving gains.

  - vivo: Extending L1 RRM measurements is up to UE implementation. For L3 measurement requirements UE can use 5 samples which is required for low SINR. For this item we consider higher SNR and there will be no impact on requirements.

  - E///: RRM measurement relaxation is out of scope. We did not consider this.

  - Huawei: Same view as E///. RRM and L1-RSRP relaxations are out of scope.

  - MTK: agree with vivo’s observation. We do not extend agreed RRM measurement period.

  - Apple: same view as vivo and MTK. As long as measurement accuracy can be satisfied, the exact period can be reduced. What delta PDCCH means?

  - QC: it make sense to consider both RLM and RRM relaxations. It seems a reasonable assumption for low mobility case

  - CMCC: Same view as E/// and Huawei. Need to evaluate the accuracy for proposed RRM relaxations.

  - E///: need to clarify WI scope. RRM measurement relaxations are out of scope.

  - QC: we can consider Rel-16 power saving relaxations

    - E///: Rel-16 focus on different case

  - E///: should we consider impact on PDCCH monitoring due to BFD relxation

    - MTK: this is under discussion in RAN1

- Agreements

  - Further evaluate UE power saving gains for the following UE implementations:

    - UE meets Rel-15 RRM measurement period and accuracy requirements

    - Option 1:

      - UE uses all L1 samples for RRM measurements based on Rel-15 assumptions

    - Option 2:

      - How many L1 samples UE applies for RRM measurements is up to UE implementation (e.g. UE can use lower number of measurement samples for RRM measurements)

      - Further discuss how many samples to use for evaluations

      - Companies shall evaluate RRM measurements accuracy for the proposed number of samples.

    - FFS whether Option 2 can be considered for requirements definition

    - Further assess impact on PDCCH monitoring due to relax UE measurements for RLM/BFD

#### 11.9.1	General and work plan [NR_UE_pow_sav_enh-Core]

R4-2101221	Work plan of Rel-17 Power Saving Enhancements

Type: Work Plan		For: Approval
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was revised to R4-2103669.

R4-2103669	Work plan of Rel-17 Power Saving Enhancements

Type: Work Plan		For: Approval
					Source: MediaTek inc.

(Replaces R4-2101221)

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was approved.

#### 11.9.2	UE measurements relaxation for RLM and/or BFD [NR_UE_pow_sav_enh-Core]

R4-2103670	WF on R17 UE power saving RLM/BM relaxation

Type: other		For: discussion
					Source: MediaTek

Discussion:

E///: For issue 2-5-4 – it contradicts to the GTW agreements

Agreement: Issue 2-5-4 Relaxation on PDCCH monitoring is kept as FFS.

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was approved.

R4-2100043	On RLM and RLF relaxation for UE power saving

Type: discussion		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was noted.

R4-2100219	UE measurements relaxation for RLM and/or BFD

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was noted.

R4-2100474	Discussion on RLM/BFD relaxation factor

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was noted.

R4-2100478	Initial performance evaluation for for RLM/BFD relaxation factor

Type: discussion		For: Discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was noted.

R4-2100725	Discussion on relaxation of RLM/BFD measurements

Type: discussion		For: Discussion
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was noted.

R4-2100821	Discussion on RLM/BFD relaxation for NR power saving enhancement

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was noted.

R4-2101139	Discussion and simulation results for RLM/BFD measurement relaxation

Type: discussion		For: Discussion
					Source: Nokia

Abstract:

System level and power saving evaluation based on simulation assumptions, and other discussion related to RLM/BFD measurement relaxation.

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was noted.

R4-2101222	Evaluation on Rel-17 RLM/BFD measurement relaxation

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was noted.

R4-2101461	Updated evaluation assumptions for R17 RLM/BFD relaxation

Type: other		For: Approval
					Source: vivo, MediaTek

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was revised to R4-2103724.

R4-2103724	Updated evaluation assumptions for R17 RLM/BFD relaxation

Type: other		For: Approval
					Source: vivo, MediaTek

(Replaces R4-2101461)

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was revised to R4-2104066.

R4-2104066	Updated evaluation assumptions for R17 RLM/BFD relaxation

Type: other		For: Approval
					Source: vivo, MediaTek

(Replaces R4-2103724)

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was approved.

R4-2101462	Discussion on R17 RLM/BFD relaxation

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was noted.

R4-2101463	Simulation results for R17 RLM/BFD relaxation

Type: other		For: Information
					Source: vivo

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was noted.

R4-2101542	Discussion on UE measurement relaxation for RLM and/or BFD

Type: discussion		For: Discussion
					Source: OPPO

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was noted.

R4-2101685	Discussion on feasibility of RLM/BFD measurement relaxation scheme for power saving enhancements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was noted.

R4-2102239	Updated simulation assumptions for evaluating UE power saving for RLM and BM

Type: other		For: Approval
					Source: Ericsson

Abstract:

In contribution contains updated simulation assumptions for evluating UE power saving for RLM and BM.

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was noted.

R4-2102240	Simulation results on UE power saving for RLM and BM

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we present the SINR difference (delta SINR) for RLM-RS based on SSB for different relaxation factors and UE speeds as in agreed in previous meeting.

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was noted.

R4-2102241	Discussions on UE power saving for RLM and BM

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we continue the discussions on release 17 UE power saving based on the identified issues from last meeting.

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was noted.

R4-2102587	Discussion on RLM/BFD Relaxation

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

According to the Work Plan in [4], in the upcoming RAN4 meeting beneficial relaxation method and corresponding criteria for UE to enter the relaxation mode should be discussed. In the following sections our company’s views on the issues to be studied are

Discussion:

See email discussion summary for [98e][238] NR_UE_pow_sav_enh_RRM in R4-2103719.

Decision: 		The document was noted.

### 11.10	NR Sidelink enhancement [NRSL_enh]

#### 11.10.1	General and work plan [NRSL_enh]

R4-2102990	Email discussion summary for [98e][142] NRSL_enh_Part_1

Type: other		For: discussion
					Source: Moderator (LG Electronics)

Discussion:

The contribution summarised email discussion thread [98e][142] NRSL_enh_Part_1. The discussion thread was moderated by Suhwan Lim (LG Electronics France).

Decision: 		The document was revised to R4-2103329.

R4-2103329	Email discussion summary for [98e][142] NRSL_enh_Part_1

Type: other		For: discussion
					Source: Moderator (LG Electronics)

(Replaces R4-2102990)

Discussion:

The contribution summarised email discussion thread [98e][142] NRSL_enh_Part_1. The discussion thread was moderated by Suhwan Lim (LG Electronics France).

Decision: 		The document was noted.

R4-2103241	Way forward on coexistence evaluation for NR SL enhancement in Rel-17

Type: other		For: discussion
					Source: LGE

Discussion:

See email discussion summary for [98e][142] NRSL_enh_Part_1 in R4-2103329.

Decision: 		The document was approved.

R4-2103242	Way forward on system parameters and operating CBW in n14 for NR SL enhancement

Type: other		For: discussion
					Source: CATT, AT&T

Discussion:

See email discussion summary for [98e][142] NRSL_enh_Part_1 in R4-2103329.

Decision: 		The document was approved.

R4-2100282	TR38.xxx v0.0.1 TR Skeleton for SL enhancement in Rel-17

Type: other		For: Agreement
					Source: LG Electronics France

Abstract:

Provide Draft TR skeleton for SL enhancement in Rel-17

Discussion:

See email discussion summary for [98e][142] NRSL_enh_Part_1 in R4-2103329.

Decision: 		The document was revised to R4-2103243.

R4-2103243	TR38.xxx v0.0.1 TR Skeleton for SL enhancement in Rel-17

Type: other		For: Agreement
					Source: LG Electronics France

(Replaces R4-2100282)

Discussion:

See email discussion summary for [98e][142] NRSL_enh_Part_1 in R4-2103329.

Decision: 		The document was agreed.

R4-2101938	Discussion on the adjacent channel coexistence simulation between SL and Uu in license band

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][142] NRSL_enh_Part_1 in R4-2103329.

Decision: 		The document was noted.

R4-2102342	Bandwidth for SL operating in n14

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on bandwidth support for n14.

Discussion:

See email discussion summary for [98e][142] NRSL_enh_Part_1 in R4-2103329.

Decision: 		The document was noted.

#### 11.10.2	Spectrum request for SL operation [NRSL_enh-Core]

R4-2101857	Additional Information for SL Operation in NR Band n14

Type: discussion		For: Approval
					Source: AT&T

Abstract:

This contribution provides the additional information for SL operation in NR Band n14 that was requested in the approved WF in R4-2016923.

Discussion:

See email discussion summary for [98e][142] NRSL_enh_Part_1 in R4-2103329.

Decision: 		The document was revised to R4-2103005.

R4-2103005	Additional Information for SL Operation in NR Band n14

Type: discussion		For: Approval
					Source: AT&T, FirstNet

(Replaces R4-2101857)

Discussion:

See email discussion summary for [98e][142] NRSL_enh_Part_1 in R4-2103329.

Decision: 		The document was noted.

#### 11.10.3	UE RF requirements for NR SL enhancement  [NRSL_enh-Core]

##### 11.10.3.1	TX requirements [NRSL_enh-Core]

R4-2100418	Discussion on UE Tx RF requirement for NR SL enhancement

Type: discussion		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][142] NRSL_enh_Part_1 in R4-2103329.

Decision: 		The document was noted.

R4-2101937	Discussion on n47 PC2 MPR simulation of Rel-17 SL enhancement

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][142] NRSL_enh_Part_1 in R4-2103329.

Decision: 		The document was noted.

R4-2102346	SL UE Timing mask for Partially used SL operation with NR Uu operating bands

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on timing mask for partially used SL operation with NR Uu operating bands in licensed band operation.

Discussion:

See email discussion summary for [98e][142] NRSL_enh_Part_1 in R4-2103329.

Decision: 		The document was noted.

##### 11.10.3.2	RX requirements [NRSL_enh-Core]

R4-2100419	Discussion on UE Rx RF requirement for NR SL enhancement

Type: discussion		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][142] NRSL_enh_Part_1 in R4-2103329.

Decision: 		The document was noted.

#### 11.10.4	Partially used SL operation with NR Uu operating bands [NRSL_enh-Core]

R4-2102991	Email discussion summary for [98e][143] NRSL_enh_Part_2

Type: other		For: discussion
					Source: Moderator (CATT)

Discussion:

The contribution summarised email discussion thread [98e][143] NRSL_enh_Part_2. The discussion thread was moderated by Yuan Gao (Morningcore Technology Co.,Ltd).

Decision: 		The document was revised to R4-2103330.

R4-2103330	Email discussion summary for [98e][143] NRSL_enh_Part_2

Type: other		For: discussion
					Source: Moderator (CATT)

(Replaces R4-2102991)

Discussion:

The contribution summarised email discussion thread [98e][143] NRSL_enh_Part_2. The discussion thread was moderated by Yuan Gao (Morningcore Technology Co.,Ltd).

Decision: 		The document was noted.

R4-2103244	Way forward on TDM operation for SL and Uu in licensed band

Type: other		For: discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][143] NRSL_enh_Part_2 in R4-2103330.

Decision: 		The document was approved.

R4-2103245	Way forward on FDM operation for SL and Uu in licensed band

Type: other		For: discussion
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][143] NRSL_enh_Part_2 in R4-2103330.

Decision: 		The document was approved.

R4-2103246	Way forward on MPR/A-MPR simulation assumption for intra-band V2X con-current operation

Type: other		For: discussion
					Source: LGE

Discussion:

See email discussion summary for [98e][143] NRSL_enh_Part_2 in R4-2103330.

Decision: 		The document was approved.

R4-2103247	Way forward on synchronous operation between Uu and SL in licensed band

Type: other		For: discussion
					Source: Ericsson

Discussion:

See email discussion summary for [98e][143] NRSL_enh_Part_2 in R4-2103330.

Decision: 		The document was approved.

R4-2103248	LS on synchronous operation between Uu and SL in licensed band

Type: LS out		For: Approval
					to -
					Source: CATT

Discussion:

See email discussion summary for [98e][143] NRSL_enh_Part_2 in R4-2103330.

Decision: 		The document was withdrawn.

##### 11.10.4.1	Operating scenarios for partially used SL operation [NRSL_enh-Core]

R4-2100415	Discussion on operating scenarios for partial used SL operation

Type: discussion		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][143] NRSL_enh_Part_2 in R4-2103330.

Decision: 		The document was noted.

R4-2100784	General issues about licensed bands partially used for SL

Type: discussion		For: Approval
					Source: vivo

Discussion:

See email discussion summary for [98e][143] NRSL_enh_Part_2 in R4-2103330.

Decision: 		The document was noted.

R4-2101875	on operating scenarios for partially used SL operation

Type: discussion		For: (not specified)
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][143] NRSL_enh_Part_2 in R4-2103330.

Decision: 		The document was noted.

R4-2102343	Operating scenarios for partially used SL operation

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on operation scenarios in licensed band operation.

Discussion:

See email discussion summary for [98e][143] NRSL_enh_Part_2 in R4-2103330.

Decision: 		The document was noted.

##### 11.10.4.2	Synchronous operation between NR Uu and NR SL in an operating band [NRSL_enh-Core]

R4-2100283	Consideration on partial usage operation with PC5 and Uu in a licensed band

Type: other		For: Approval
					Source: LG Electronics France

Abstract:

propose as follow

Proposal 1: RAN4 allow TDM operation between PC5 and Uu operation in a licensed TDD band.

Proposal 2: RAN4 can specify the con-current V2X operation in TDD intra-band without in-device coexistence study. For the FDD intra-band con-curre

Discussion:

See email discussion summary for [98e][143] NRSL_enh_Part_2 in R4-2103330.

Decision: 		The document was noted.

R4-2100416	Discussion on synchronous operation between NR Uu and NR SL

Type: discussion		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][143] NRSL_enh_Part_2 in R4-2103330.

Decision: 		The document was noted.

R4-2101877	Synchronous operation between NR Uu and NR SL in an operating band

Type: discussion		For: (not specified)
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][143] NRSL_enh_Part_2 in R4-2103330.

Decision: 		The document was noted.

R4-2102345	SL UE synchronization issue for licensed operation

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on timing alignment issue in licensed band operation.

Discussion:

See email discussion summary for [98e][143] NRSL_enh_Part_2 in R4-2103330.

Decision: 		The document was noted.

##### 11.10.4.3	Others [NRSL_enh-Core]

#### 11.10.5	High power UE(PC2) for SL [NRSL_enh-Core]

R4-2102992	Email discussion summary for [98e][144] NRSL_enh_Part_3

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][144] NRSL_enh_Part_3. The discussion thread was moderated by Ye Liu (Huawei Technologies France).

Decision: 		The document was revised to R4-2103331.

R4-2103331	Email discussion summary for [98e][144] NRSL_enh_Part_3

Type: other		For: discussion
					Source: Moderator (Huawei)

(Replaces R4-2102992)

Discussion:

The contribution summarised email discussion thread [98e][144] NRSL_enh_Part_3. The discussion thread was moderated by Ye Liu (Huawei Technologies France).

Decision: 		The document was noted.

R4-2103249	Way forward on issues related to PC2 NR V2X

Type: other		For: discussion
					Source: Huawei

Discussion:

See email discussion summary for [98e][144] NRSL_enh_Part_3 in R4-2103331.

Decision: 		The document was approved.

R4-2103250	Way forward on simulation assumptions for PC2 NR V2X

Type: other		For: discussion
					Source: Huawei

Discussion:

See email discussion summary for [98e][144] NRSL_enh_Part_3 in R4-2103331.

Decision: 		The document was approved.

R4-2103251	Way forward on co-existence simulation assumptions for PC2 NR V2X

Type: other		For: discussion
					Source: CATT

Discussion:

See email discussion summary for [98e][144] NRSL_enh_Part_3 in R4-2103331.

Decision: 		The document was approved.

R4-2100420	Discussion on PC2 for SL enhancement

Type: discussion		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][144] NRSL_enh_Part_3 in R4-2103331.

Decision: 		The document was noted.

R4-2101874	on HPUE signalling issue

Type: discussion		For: (not specified)
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][144] NRSL_enh_Part_3 in R4-2103331.

Decision: 		The document was noted.

##### 11.10.5.1	TX requirements [NRSL_enh-Core]

R4-2100284	PC2 MPR/A-MPR simulation assumptions for NR V2X UE in n47

Type: other		For: Approval
					Source: LG Electronics France

Abstract:

Propose detail MPR/A-MPR simulation assumptions for PC2 NR V2X UE in Rel-17

Discussion:

See email discussion summary for [98e][144] NRSL_enh_Part_3 in R4-2103331.

Decision: 		The document was noted.

R4-2100785	Discussion on HPUE for NR sidelink enhancement in R17

Type: discussion		For: Approval
					Source: vivo

Discussion:

See email discussion summary for [98e][144] NRSL_enh_Part_3 in R4-2103331.

Decision: 		The document was noted.

R4-2101873	on HPUE for V2X RF requirements

Type: discussion		For: (not specified)
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][144] NRSL_enh_Part_3 in R4-2103331.

Decision: 		The document was noted.

##### 11.10.5.2	RX requirements [NRSL_enh-Core]

#### 11.10.6	Other RF/general requirements for New SL enhancement [NRSL_enh-Core]

R4-2100417	Discussion on system parameters for newly introduced SL bands

Type: discussion		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][142] NRSL_enh_Part_1 in R4-2103329.

Decision: 		The document was noted.

R4-2102344	coexisting simulation assumption for public safety UC

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on coexisting simulation assumption for public safety UC in band n14  in licensed operation.

Discussion:

See email discussion summary for [98e][142] NRSL_enh_Part_1 in R4-2103329.

Decision: 		The document was noted.

### 11.11	NR repeater

#### 11.11.1	General and work plan [NR_repeaters-Core]

R4-2103751	Email discussion summary for [98e][312] NR_Repeater_General

Type: other		For: discussion
					Source: Moderator (Qualcomm)

Discussion:

The contribution summarised email discussion thread [98e][312] NR_Repeater_General. The discussion thread was moderated by Valentin Gheorghiu (Qualcomm).

Decision: 		The document was revised to R4-2103950.

R4-2103950	Email discussion summary for [98e][312] NR_Repeater_General

Type: other		For: discussion
					Source: Moderator (Qualcomm)

(Replaces R4-2103751)

Discussion:

The contribution summarised email discussion thread [98e][312] NR_Repeater_General. The discussion thread was moderated by Valentin Gheorghiu (Qualcomm).

Decision: 		The document was noted.

-------------------------------Agreements-----------------------

RRM is out of scope based on current WID.

The TU request will be updated to align with work plan and WID.

Include all bands in the scope:

-Take the existing Rel-16 bands in the specification as basis

- Further discuss the multi-bands supporting issue

Baseline is that passband is pre-configured. FFS whether configurable passband should be supported

Fixed antenna gain and pattern is assumed

Network controlled gain is out of scope

------------------------End--------------------------

R4-2103881	WF for TDD Repeater

Type: other		For: discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][312] NR_Repeater_General in R4-2103950.

Decision: 		The document was revised to R4-2103999.

R4-2103999	WF for TDD Repeater

Type: other		For: discussion
					Source: CMCC

(Replaces R4-2103881)

Discussion:

See email discussion summary for [98e][312] NR_Repeater_General in R4-2103950.

Decision: 		The document was approved.

R4-2103752	Email discussion summary for [98e][313] NR_Repeater_RF

Type: other		For: discussion
					Source: Moderator (CMCC)

Discussion:

The contribution summarised email discussion thread [98e][313] NR_Repeater_RF. The discussion thread was moderated by Chunxia Guo (China Mobile Group Device Co).

Decision: 		The document was revised to R4-2103951.

R4-2103951	Email discussion summary for [98e][313] NR_Repeater_RF

Type: other		For: discussion
					Source: Moderator (CMCC)

(Replaces R4-2103752)

Discussion:

The contribution summarised email discussion thread [98e][313] NR_Repeater_RF. The discussion thread was moderated by Chunxia Guo (China Mobile Group Device Co).

Decision: 		The document was noted.

R4-2103882	WF for NR repeater RF requirements

Type: other		For: discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was approved.

R4-2100375	Discussion on NR repeater general issues

Type: other		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][312] NR_Repeater_General in R4-2103950.

Decision: 		The document was noted.

R4-2100635	Discussion on Requirements for NR Repeaters

Type: discussion		For: Approval
					Source: CommScope Technologies AG

Abstract:

This NR_repeaters-Core WI aims to specify the RF and EMC requirements for NR repeaters, including repeaters operating in both the FR1 and FR2 bands, and repeaters that support both FDD and TDD operation.

Discussion:

See email discussion summary for [98e][312] NR_Repeater_General in R4-2103950.

Decision: 		The document was noted.

R4-2100832	General parts of NR repeater

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][312] NR_Repeater_General in R4-2103950.

Decision: 		The document was noted.

R4-2101041	Views on candidate bands for NR repeater

Type: other		For: Approval
					Source: NTT DOCOMO, INC.

Discussion:

See email discussion summary for [98e][312] NR_Repeater_General in R4-2103950.

Decision: 		The document was noted.

R4-2101156	General views on NR repeater

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][312] NR_Repeater_General in R4-2103950.

Decision: 		The document was noted.

R4-2101963	Discussion on NR based repeater

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][312] NR_Repeater_General in R4-2103950.

Decision: 		The document was noted.

R4-2102108	Repeaters WI general considerations

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

General considerations, especially test set-up

Discussion:

See email discussion summary for [98e][312] NR_Repeater_General in R4-2103950.

Decision: 		The document was noted.

R4-2102580	Consideration of duplex mode for the frequency bands for NR repeater

Type: discussion		For: Agreement
					Source: Huawei

Abstract:

In this contribution we provide analysis on the duplex mode for the NR Repeater frequency bands.

Discussion:

See email discussion summary for [98e][312] NR_Repeater_General in R4-2103950.

Decision: 		The document was noted.

R4-2102829	Common understanding of repeaters

Type: discussion		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][312] NR_Repeater_General in R4-2103950.

Decision: 		The document was noted.

R4-2102834	Work plan for NR Repeaters

Type: Work Plan		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][312] NR_Repeater_General in R4-2103950.

Decision: 		The document was revised to R4-2103880.

R4-2103880	Work plan for NR Repeaters

Type: Work Plan		For: Approval
					Source: Qualcomm Incorporated

(Replaces R4-2102834)

Discussion:

See email discussion summary for [98e][312] NR_Repeater_General in R4-2103950.

Decision: 		The document was approved.

R4-2102018	NR repeater considerations

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

Session Chair: Move this AI from AI 11.1

Decision: 		The document was noted.

#### 11.11.2	Conductive RF core requirements [NR_repeaters-Core]

R4-2102418	NR repeater requirements

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Study of the existing FDD and TDD repeater specification and how the requirements may be applied to an NR repeater.

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

##### 11.11.2.1	Transmitted power related requirements [NR_repeaters-Core]

R4-2100376	Discussion on NR repeater conducted transmitted power

Type: other		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

R4-2100828	Power related conducted requirements for repeaters

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

R4-2102109	Conducted transmit power

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

FR1 power

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

##### 11.11.2.2	Emission requirements [NR_repeaters-Core]

R4-2100377	Discussion on NR repeater emission requirements for FR1/FR2

Type: other		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

R4-2100830	Emission related conducted requirements for repeaters

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

R4-2102110	Conducted unwanted emissions

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

FR1 emissions

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

##### 11.11.2.3	Others  [NR_repeaters-Core]

R4-2100378	Discussion on NR repeater EVM and frequency stability for FR1/FR2

Type: other		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

R4-2100827	Discussion on RF architecture and  RF requirements scope of repeater

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

R4-2102111	Conducted other requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Other FR1 issues

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

R4-2102831	Conducted requirements for NR FR1 repeaters

Type: discussion		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

#### 11.11.3	Radiated RF core requirements

R4-2102419	Considerations for NR repeaters and AAS

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

Discussion on the differences between a RF repeater deployed in a AAS BS network compared to the non-AAS BBS network

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

##### 11.11.3.1	Transmitted power related requirements [NR_repeaters-Core]

R4-2100379	Discussion on NR repeater radiated transmitted power

Type: other		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

R4-2100829	Power related radiated requirements for repeaters

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

R4-2102112	Radiated transmit power

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

FR2 power

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

##### 11.11.3.2	Emission requirements [NR_repeaters-Core]

R4-2100831	Emission related radiated requirements for repeaters

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

R4-2102113	Radiated unwanted emissions

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

FR2 emissions

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

##### 11.11.3.3	Others  [NR_repeaters-Core]

R4-2102114	Radiated other requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

other FR2 issues

Discussion:

See email discussion summary for [98e][313] NR_Repeater_RF in R4-2103951.

Decision: 		The document was noted.

#### 11.11.4	EMC core requirements  [NR_repeaters-Core]

R4-2103879	WF for NR repeater EMC requirements

Type: other		For: discussion
					Source: ZTE

Discussion:

The contents in slide 3, slide 4 of R4-2103879 are agreed. See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was noted.

R4-2100361	Discussion on EMC requirements for NR Repeater

Type: other		For: Discussion
					Source: Ericsson

Abstract:

Discussion on EMC requirements for NR Repeater

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was noted.

R4-2102129	Proposal on the skeleton of NR Repeaters EMC

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was noted.

R4-2102130	Discussion on NR repeaters EMC

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was noted.

R4-2102180	Proposal on the skeleton of NR Repeaters EMC

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was noted.

R4-2102181	Discussion on NR repeaters EMC

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was noted.

R4-2102579	EMC core requirements for NR repeater

Type: discussion		For: Agreement
					Source: Huawei

Abstract:

In this contribution we provide initial analysis of the expected work for EMC requirements for Rel-17 NR Repeaters.

Discussion:

See email discussion summary for [98e][303] NR_EMC in R4-2103922.

Decision: 		The document was noted.

## 12	Rel-17 Study Items for NR

### 12.1	Study on enhanced test methods for FR2 in NR [FS_FR2_enhTestMethods]

#### 12.1.1	General [FS_FR2_enhTestMethods]

R4-2103769	Email discussion summary for [98e][330] FR2_enhTestMethods

Type: other		For: discussion
					Source: Moderator (Apple)

Discussion:

The contribution summarised email discussion thread [98e][330] FR2_enhTestMethods. The discussion thread was moderated by Anatoliy Ioffe (Apple).

Decision: 		The document was revised to R4-2103952.

R4-2103952	Email discussion summary for [98e][330] FR2_enhTestMethods

Type: other		For: discussion
					Source: Moderator (Apple)

(Replaces R4-2103769)

Discussion:

The contribution summarised email discussion thread [98e][330] FR2_enhTestMethods. The discussion thread was moderated by Anatoliy Ioffe (Apple).

Decision: 		The document was noted.

R4-2103985	Draft TR38.884 Study on enhanced test methods for FR2 NR UEs v0.2.0

Type: draft TR		For: Agreement
					Source: Apple, vivo

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2103918	WF on high DL power and low UL power test cases (objective1) and band n262 testability (objective7)

Type: other		For: discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was approved.

R4-2103919	WF on polarization basis mismatch (objective2)

Type: other		For: discussion
					Source: MediaTek

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was approved.

R4-2103920	WF on ETC (objective4) and test time reduction(objective6)

Type: other		For: discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was approved.

#### 12.1.2	Test methodology for high DL power and low UL power test cases [FS_FR2_enhTestMethods]

R4-2100525	TP to TR38.884 on High DL and Low UL power test cases

Type: other		For: Approval
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was merged.

R4-2103966	TP to TR38.884 on High DL and Low UL power test cases

Type: other		For: discussion
					Source: Keysight, Rohde & Schwarz

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was approved.

R4-2101485	Overview of the Impact of phase variation for Direct NF Method

Type: other		For: Approval
					Source: MVG Industries, Sony

Abstract:

During RAN4#e-97, a WF was agreed [1] for enhanced test methods for NR FR2. Specifically, it was agreed on studying further DNF (direct NF). This contribution provides further simulation results for the DNF test method with the aim of comparing 4x1, 8x2 a

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2102616	On Test methodology for high DL power and low UL power test cases

Type: other		For: Approval
					Source: Keysight Technologies UK Ltd

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2102620	NF based solutions and Enhancement of permitted methods

Type: discussion		For: Approval
					38.884 v..
					Source: ROHDE & SCHWARZ

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2100526	TP to TR38.884 on polarization mismatch

Type: other		For: Approval
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was merged.

R4-2100571	Views on solutions to minimize the impact of polarization basis mismatch

Type: other		For: Approval
					Source: Sony, Ericsson

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2100664	Discussion on enhanced test method for polarization basis mismatch

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2100699	Practical TPMI and 2-port CSI-RS for FR2 SISO test enhancement

Type: discussion		For: Approval
					Source: MediaTek Beijing Inc.

Abstract:

Observation1: “practical TPMI” is aligned with network’s capability, and it can further enhance UE performance.

Proposal1: For “TPMI method”, “practical TPMI” shall be further applied.

Observation2: “2-port CSI-RS” is a feasible test method and aligned w

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2100894	Discussion on FR2 EIRP measurement enhancement

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2101759	Solution to minimize the impact of polarization basis mismatch

Type: discussion		For: Approval
					Source: OPPO

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2101830	TP to TR38.884 v0.1.0 on polarization basis mismatch

Type: pCR		For: Approval
					38.884 v0.1.0
					Source: vivo

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was revised to R4-2103967.

R4-2103967	TP to TR38.884 v0.1.0 on polarization basis mismatch

Type: pCR		For: Approval
					38.884 v0.1.0
					Source: vivo

(Replaces R4-2101830)

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was approved.

R4-2102090	Discussion on FR2 UL demodulation measurements

Type: discussion		For: Approval
					Source: Rohde & Schwarz

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2102674	FR2 testability enhancement for polarization mismatch

Type: other		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

We discuss test mode, 2 port CSIRS and enhancement 'coverage hole'

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

#### 12.1.3	Polarization basis mismatch  [FS_FR2_enhTestMethods]

#### 12.1.4	Enhanced test methods for inter-band (FR2+FR2) CA [FS_FR2_enhTestMethods]

R4-2100096	Impact of offset antenna to quiet zone in FR2 OTA chamber

Type: discussion		For: Discussion
					Source: Anritsu corporation

Abstract:

In this contribution we discuss some open issues such as an impact of the offset antenna on quality of the quiet zone (QoQZ), UE beam forming, and also the applicability of this system to CBM UE.

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2100097	TP to TR 38.884 on Inter-band DL CA in FR2

Type: pCR		For: Approval
					38.884 v0.1.0
					Source: Anritsu corporation

Abstract:

TP to TR 38.884 on Inter-band DL CA in FR2

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was revised to R4-2103968.

R4-2103968	TP to TR 38.884 on Inter-band DL CA in FR2

Type: pCR		For: Approval
					38.884 v0.1.0
					Source: Anritsu corporation

(Replaces R4-2100097)

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was approved.

R4-2100527	Further analysis of the impact of AoA offset on inter-band CA PSD difference

Type: discussion		For: Discussion
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2102673	On impact of non-co-located test antennae for FR2 inter-band testing

Type: other		For: Agreement
					Source: Qualcomm Incorporated

Abstract:

We investigate the impact of non-co-located test antennae on CBM UEs

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

#### 12.1.5	Extreme temperature conditions [FS_FR2_enhTestMethods]

R4-2100098	DUT repositioning during ETC measurement in FR2

Type: discussion		For: Approval
					Source: Anritsu corporation

Abstract:

In this contribution we discuss restrictions of DUT measurement procedure during the 3D scan under the extreme temperature condition (ETC). We also discuss some proposals which are related to a test time reduction.

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2100528	Impact of ET on measurement uncertainty and test tolerance of spherical coverage EIRP and EIS

Type: discussion		For: Discussion
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2101828	Discussions on FR2 Extreme temperature conditions

Type: discussion		For: Approval
					Source: vivo

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2102617	On extreme temperature condition testing

Type: other		For: Approval
					Source: Keysight Technologies UK Ltd

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2102675	FR2 testability in ETC

Type: other		For: Agreement
					Source: Qualcomm Incorporated

Abstract:

We discuss TE requirements for ETC testing

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

#### 12.1.6	Enhanced test methods for FR2 DL 256QAM RF [FS_FR2_enhTestMethods]

#### 12.1.7	Test time reduction [FS_FR2_enhTestMethods]

R4-2100161	Test time reduction in FR2 using beam sweeping

Type: discussion		For: Discussion
					Source: Fraunhofer HHI, Fraunhofer IIS

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2100245	Test time reduction in OTA measurement

Type: discussion		For: Approval
					Source: Anritsu Corporation

Abstract:

In this contribution we discuss a way to reduce test time of a beam peak search in FR2.

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2100665	Discussion on enhance test method to reduce FR2 OTA test time

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2100895	Discussion on FR2 test time reduction

Type: discussion		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2101829	Discussions on Test Time Reduction for NR FR2 RF

Type: discussion		For: Approval
					Source: vivo

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2102088	Discussion on test time reduction methods

Type: discussion		For: Approval
					Source: Rohde & Schwarz

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2102401	Analysis on reducing test time

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

R4-2102618	On Test Time Enhancements based on different Antenna Array Assumptions

Type: other		For: Approval
					Source: Keysight Technologies UK Ltd

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

#### 12.1.8	Testability for band n262  [FS_FR2_enhTestMethods]

##### 12.1.8.1	Extension of frequency applicability of permitted methods in 38.810 [FS_FR2_enhTestMethods]

R4-2100529	Extending the applicability of permitted methods to band n262

Type: discussion		For: Discussion
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was withdrawn.

R4-2102619	On Testability for band n262

Type: other		For: Approval
					Source: Keysight Technologies UK Ltd

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was revised to R4-2103917.

R4-2103917	On Testability for band n262

Type: other		For: Approval
					Source: Keysight Technologies UK Ltd

(Replaces R4-2102619)

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was noted.

##### 12.1.8.2	Extension of frequency applicability of enhancement objectives 1-6 [FS_FR2_enhTestMethods]

R4-2100530	TP to TR38.884 on structure updates related to band n262

Type: other		For: Approval
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][330] FR2_enhTestMethods in R4-2103952.

Decision: 		The document was approved.

### 12.2	Study on supporting NR from 52.6 GHz to 71 GHz [FS_NR_52_to_71GHz]

#### 12.2.1	Numerology, Channel BW [FS_NR_52_to_71GHz]

R4-2102993	Email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1

Type: other		For: discussion
					Source: Moderator (Qualcomm)

Decision: 		The document was revised to R4-2103332.

R4-2103332	Email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1

Type: other		For: discussion
					Source: Moderator (Qualcomm)

(Replaces R4-2102993)

Discussion:

The contribution summarised email discussion thread [98e][145] FS_NR_52_to_71GHz_Part_1. The discussion thread was moderated by Philip Coan (Qualcomm).

Decision: 		The document was noted.

R4-2103253	Way forward on NR 52

Type: other		For: discussion
					Source: Qualcomm

Discussion:

The contribution summarised email discussion thread [98e][145] FS_NR_52_to_71GHz_Part_1. The discussion thread was moderated by Philip Coan (Qualcomm).

Decision: 		The document was approved.

R4-2100364	Discussion on CBW and FR name for above 52.6 GHz

Type: other		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was noted.

R4-2101561	TP to TR 38.808: Numerology and Channel Bandwidths

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Ericsson

Abstract:

Adding numerology and channel bandwidth to RAN4 part of TR

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was revised to R4-2103259.

R4-2103259	TP to TR 38.808: Numerology and Channel Bandwidths

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Ericsson

(Replaces R4-2101561)

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was revised to R4-2103374.

R4-2103374	TP to TR 38.808: Numerology and Channel Bandwidths

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Ericsson

(Replaces R4-2103259)

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was approved.

R4-2102569	TP to TR 38.808: capturing WF on the min/max CHBW and SCS

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Huawei

Abstract:

Based on the WF on minimum and maximum channel bandwidths for 52.6 - 71 GHz range as approved last meeting, it is proposed to capture the WF agreements in the TR 38.808.

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was noted.

R4-2102730	Discussion on minimum and maximum channel bandwidth for 52.6 GHz to 71 GHz

Type: discussion		For: Approval
					Source: Huawei, HiSilicon

Abstract:

In this contribution, we provide further consideration on minimum and maximum channel bandwidth for 52.6 GHz to 71 GHz.

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was noted.

##### 12.2.1.1	General [FS_NR_52_to_71GHz]

R4-2100519	Further considerations on the numerology and channel bandwidth sizes for 60GHz frequency range

Type: discussion		For: Decision
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was noted.

R4-2100781	Discussion on the minimum and maximum channel bandwidth for B52.6GHz

Type: discussion		For: Approval
					Source: vivo

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was noted.

R4-2100803	Discussion on band definition and channel BW for NR in 52.6GHz

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was noted.

R4-2101281	On numerology and channel bandwidth in 52.6

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was noted.

R4-2102006	Numerology and channel bandwidth discussion for NR beyond 52.6 GHz

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was noted.

R4-2102609	Views on UL TPC for NR in 60 GHz and above frequency ranges

Type: other		For: Approval
					Source: Apple

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was noted.

##### 12.2.1.2	General [FS_NR_52_to_71GHz]

R4-2101280	On improved transient period for NR 52.6 - 71 GHz

Type: discussion		For: Discussion
					Source: Intel Corporation

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was noted.

R4-2101863	TP for NR Rel-17 TR 38.808: Time and synchronization impact

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Ericsson

Abstract:

Analysis of transient cases.

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was noted.

R4-2101955	TP to TR 38.808  Further considerations on timing for 52.6-71GHz

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was noted.

R4-2102009	TP to TR 38.808: Timing considerations for operation between 52.6 and 71 GHz

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was revised to R4-2103260.

R4-2103260	TP to TR 38.808: Timing considerations for operation between 52.6 and 71 GHz

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102009)

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was approved.

##### 12.2.1.3	Phase noise  [FS_NR_52_to_71GHz]

R4-2100782	Comparison of different PN models for B52.6G

Type: discussion		For: Approval
					Source: vivo

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was noted.

R4-2101181	TP to TR 38.808: Addition of a set of phase noise models in subclause 4.2.3

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Ericsson

Abstract:

The text proposal covers not only the phase noise model but also more detailed technical information on how the model was derived in described in Annex C.

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was revised to R4-2103261.

R4-2103261	TP to TR 38.808: Addition of a set of phase noise models in subclause 4.2.3

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Ericsson

(Replaces R4-2101181)

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was approved.

R4-2102008	TP to TR 38.808: Phase noise considerations

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was noted.

R4-2102839	Text proposal PTRS

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 in R4-2103332.

Decision: 		The document was noted.

#### 12.2.2	BS aspect [FS_NR_52_to_71GHz]

R4-2102994	Email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2

Type: other		For: discussion
					Source: Moderator (Intel)

Discussion:

The contribution summarised email discussion thread [98e][146] FS_NR_52_to_71GHz_Part_2. The discussion thread was moderated by Jiwoo Kim (Intel).

Decision: 		The document was revised to R4-2103333.

R4-2103333	Email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2

Type: other		For: discussion
					Source: Moderator (Intel)

(Replaces R4-2102994)

Discussion:

The contribution summarised email discussion thread [98e][146] FS_NR_52_to_71GHz_Part_2. The discussion thread was moderated by Jiwoo Kim (Intel).

Decision: 		The document was noted.

R4-2103256	TP to TR 38.808: BS antenna array

Type: pCR		For: Approval
					Source: Nokia

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was approved.

R4-2103257	TP to TR 38.808: BS architecture

Type: pCR		For: Approval
					Source: Huawei

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was approved.

R4-2103258	Way forward on NR 52

Type: other		For: discussion
					Source: Intel

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was approved.

R4-2103356	TP to TR 38.808: UE aspects

Type: pCR		For: Approval
					Source: Intel

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was approved.

R4-2100383	Discussion on the BS requirements for 52.6-71GHz

Type: other		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

R4-2101183	TP to TR 38.808: Addition of technical background for BS in clause 2 and subclause 4.2.5

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Ericsson

Abstract:

At the end of this contribution a text proposal to TR 38.808, clause 2 and subclause 4.2.5 is attached for approval. The text proposal adds missing parts of technical information to be captured in TR 38.808 to be able to conclude the Study Item.

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

R4-2102007	TP to TR 38.808: BS RF for NR beyond 52.6 GHz

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

R4-2102570	TP to TR 38.808: BS aspects

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Huawei

Abstract:

This contribution provides TP to TR 38.808 on selected BS aspects for 52.6 – 71 GHz range, including BS architecture and BS classes and additional examples of BS antenna array.

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

#### 12.2.3	UE aspect [FS_NR_52_to_71GHz]

R4-2102010	UE RF for NR beyond 52.6 GHz

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

R4-2102681	TP to TR 38.808: Addition of UE aspects

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Ericsson

Abstract:

Impact on SCS's on number of transient affected symbols for UE

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

R4-2102862	Text proposal UE power amplifier and antenna array

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

#### 12.2.4	Others [FS_NR_52_to_71GHz]

R4-2100384	Discussion on co-existence for 52.6-71GHz

Type: other		For: Approval
					Source: CATT

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

R4-2100520	Regulatory update for the 60GHz frequency range

Type: discussion		For: Decision
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

R4-2100521	Regulatory update for the 60GHz frequency range

Type: draftCR		For: Endorsement
					38.807 v16.0.0
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

R4-2100531	Views on 60 GHz testability

Type: discussion		For: Discussion
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

R4-2100911	How to incorperate 52.6-71GHz in specification

Type: other		For: Discussion
					Source: Samsung

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

R4-2101833	Views on Study the testability of 60GHz

Type: discussion		For: Approval
					Source: vivo Communication Technology

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

R4-2101956	Discussion on frequency range definition for 52.6-71GHz

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

R4-2102043	TP to TR 38.808 on Spectrum regulatory situation

Type: pCR		For: Approval
					38.808 v1.0.0
					Source: Ericsson

Abstract:

TP to TR 38.808 on Spectrum regulatory situation within 52.6 - 71 GHz

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

R4-2102571	Inputs to the discussion on the FR2-extension vs. FR3 introduction for NR operation in 52.6 - 71 GHz range

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution we collect observations on the RAN4 specification impact (as well as selected RAN1 and RAN2 aspects) considering FR2-extension and FR3 introduction alternatives for the NR operation in 52.6 – 71 GHz range. This is provided to ease dec

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

R4-2102866	Specifying 60 GHz as part of FR2

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 in R4-2103333.

Decision: 		The document was noted.

### 12.3	Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths [FS_NR_eff_BW_util]

R4-2102995	Email discussion summary for [98e][147] FS_NR_eff_BW_util

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][147] FS_NR_eff_BW_util. The discussion thread was moderated by Esther Sienkiewicz (Ericsson Inc.).

Decision: 		The document was revised to R4-2103334.

R4-2103334	Email discussion summary for [98e][147] FS_NR_eff_BW_util

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2102995)

Discussion:

The contribution summarised email discussion thread [98e][147] FS_NR_eff_BW_util. The discussion thread was moderated by Esther Sienkiewicz (Ericsson Inc.).

Decision: 		The document was noted.

R4-2103263	Way forward on use of larger CBW

Type: other		For: discussion
					Source: Skyworks

Discussion:

See email discussion summary for [98e][147] FS_NR_eff_BW_util in R4-2103334.

Decision: 		The document was approved.

R4-2103264	Way forward on overlapping CBW method

Type: other		For: discussion
					Source: Nokia

Discussion:

See email discussion summary for [98e][147] FS_NR_eff_BW_util in R4-2103334.

Decision: 		The document was revised to R4-2103387.

R4-2103387	Way forward on overlapping CBW method

Type: other		For: discussion
					Source: Nokia

(Replaces R4-2103264)

Discussion:

See email discussion summary for [98e][147] FS_NR_eff_BW_util in R4-2103334.

Decision: 		The document was approved.

#### 12.3.1	General and work plan [FS_NR_eff_BW_util]

R4-2100804	Discussion on the general aspects for irregular bandwidth

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][147] FS_NR_eff_BW_util in R4-2103334.

Decision: 		The document was noted.

R4-2101555	Updated TR 38.844 v0.0.2

Type: draft TR		For: Agreement
					38.844 v0.0.2
					Source: Ericsson

Abstract:

Updated TR

Discussion:

See email discussion summary for [98e][147] FS_NR_eff_BW_util in R4-2103334.

Decision: 		The document was revised to R4-2103262.

R4-2103262	Updated TR 38.844 v0.0.2

Type: draft TR		For: Agreement
					38.844 v0.0.2
					Source: Ericsson

(Replaces R4-2101555)

Discussion:

See email discussion summary for [98e][147] FS_NR_eff_BW_util in R4-2103334.

Decision: 		The document was agreed.

#### 12.3.2	Input on operator licensed channel bandwidths in FR1 that do not align with existing NR channel bandwidths [FS_NR_eff_BW_util]

#### 12.3.3	Evaluation of use of larger channel bandwidths than operator licensed bandwidth [FS_NR_eff_BW_util]

R4-2101556	Utilizing larger bandwidth than the operator licensed bandwidth

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this contribution, the goal is to focus on operator licensed bandwidths between 5 MHz and 10 MHz cases.

Discussion:

See email discussion summary for [98e][147] FS_NR_eff_BW_util in R4-2103334.

Decision: 		The document was noted.

R4-2101959	Discussion on irregular channel bandwidth for NR system

Type: other		For: Discussion
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][147] FS_NR_eff_BW_util in R4-2103334.

Decision: 		The document was noted.

#### 12.3.4	Evaluation of use of overlapping UE channel bandwidths (from both UE and network perspective) [FS_NR_eff_BW_util]

R4-2100522	Non-standard spectrum allocations for NR bands

Type: discussion		For: Decision
					Source: Apple Inc.

Discussion:

See email discussion summary for [98e][147] FS_NR_eff_BW_util in R4-2103334.

Decision: 		The document was noted.

R4-2101507	Consideration for overlapping UE channel bandwidths

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][147] FS_NR_eff_BW_util in R4-2103334.

Decision: 		The document was noted.

##### 12.3.4.1	UE perspective  [FS_NR_eff_BW_util]

R4-2100805	Discussion on overlapping UE channel bandwidths

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][147] FS_NR_eff_BW_util in R4-2103334.

Decision: 		The document was noted.

R4-2101557	Overlapping Channel Bandwidth Approach from UE Perspective

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution, the focus will be discussing overlapping bandwidth approach from UE perspective

Discussion:

See email discussion summary for [98e][147] FS_NR_eff_BW_util in R4-2103334.

Decision: 		The document was noted.

R4-2102288	Network and UE options for the support of irregular channel bandwidth

Type: discussion		For: Approval
					38.101-1 v..
					Source: Skyworks Solutions Inc.

Abstract:

This contribution discusses the different cases from a Network and UE prospective and provides an analysis of potential solutions and their related constraints.

Discussion:

See email discussion summary for [98e][147] FS_NR_eff_BW_util in R4-2103334.

Decision: 		The document was noted.

R4-2102558	On the use of overlapping channel bandwidths from UE perspective

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][147] FS_NR_eff_BW_util in R4-2103334.

Decision: 		The document was noted.

##### 12.3.4.2	Network perspective [FS_NR_eff_BW_util]

R4-2101459	Handling of Channel Bandwidths That Are Not Multiples of 5MHz

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][147] FS_NR_eff_BW_util in R4-2103334.

Decision: 		The document was noted.

R4-2101558	Overlapping Channel Bandwidth Approach from BS Perspective

Type: discussion		For: Approval
					Source: Ericsson

Abstract:

In this contribution, the focus will be discussing overlapping bandwidth approach from network perspective

Discussion:

See email discussion summary for [98e][147] FS_NR_eff_BW_util in R4-2103334.

Decision: 		The document was noted.

#### 12.3.5	Others [FS_NR_eff_BW_util]

### 12.4	Study on extended 600MHz NR band [FS_NR_600MHz_ext]

R4-2102996	Email discussion summary for [98e][148] FS_NR_600MHz_ext

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][148] FS_NR_600MHz_ext. The discussion thread was moderated by Christian Bergljung (Ericsson).

Decision: 		The document was revised to R4-2103335.

R4-2103335	Email discussion summary for [98e][148] FS_NR_600MHz_ext

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2102996)

Discussion:

The contribution summarised email discussion thread [98e][148] FS_NR_600MHz_ext. The discussion thread was moderated by Christian Bergljung (Ericsson).

Decision: 		The document was noted.

R4-2103266	Way forward on regulatory requirements for protection of other services identified for Region 3 and coexistence with Band 28/n28

Type: other		For: discussion
					Source: ZTE

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

R4-2103268	Way forward on bands plans for further study and duplex filter options

Type: other		For: discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was revised to R4-2103372.

R4-2103372	Way forward on bands plans for further study and duplex filter options

Type: other		For: discussion
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2103268)

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

#### 12.4.1	General

R4-2100055	Study on extended 600MHz NR band

Type: Work Plan		For: Decision
					Source: Spark NZ Ltd

Abstract:

The work scope and work plan are presented in this document.

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was revised to R4-2103270.

R4-2103270	Work plan of study on extended 600MHz NR band

Type: Work Plan		For: Decision
					Source: Spark NZ Ltd

(Replaces R4-2100055)

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was approved.

R4-2102589	Band Plan for 600MHz SI

Type: discussion		For: Approval
					Source: Apple

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

#### 12.4.2	Regulatory study

R4-2100744	Regulatory study for APT 600 MHz

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Regulatory backgroud of APT 600 MHz is presented.

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

R4-2102162	Extended 600MHz band - Regulatory aspects

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution is giving an overview of regulatory requirements around the extended 600 MHz band in Region 3

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

R4-2102572	Regulatory aspects for the 600MHz range in APT region

Type: discussion		For: Agreement
					Source: Huawei

Abstract:

In this contribution we provide the inputs on the regulatory overview of 600 MHz range. Related TP to TR is provided for approval.

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was revised to R4-2103267.

R4-2103267	Regulatory aspects for the 600MHz range in APT region

Type: discussion		For: Agreement
					Source: Huawei

(Replaces R4-2102572)

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was approved.

#### 12.4.3	Coexistence study

R4-2100745	Coexistence for APT 600 MHz

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell, CBN

Abstract:

Coexistence requirement of APT 600 MHz is discussed.

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

R4-2101957	Coexistence study for extended 600MHz NR band

Type: other		For: Discussion
					Source: ZTE Corporation, CBN

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

R4-2102573	Initial considerations on the coexistence studies for 600MHz SI

Type: discussion		For: Discussion
					Source: Huawei

Abstract:

In this contribution we provide initial thoughts on the potential co-existence studies required for the extended 600MHz band in Region 3.

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

#### 12.4.4	Study on frequency arrangements (such as options B1 and B2)

R4-2100056	Frequency band arrangements and duplexer options for extended 600MHz NR  band

Type: SID new		For: Discussion
					Source: Spark NZ Ltd

Abstract:

The frequency band arrangements are  presented in this contribution.

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

R4-2100057	Blank TR for extended 600MHz NR band

Type: SID new		For: Decision
					Source: Spark NZ Ltd

Abstract:

A blank TR to document the ongoing progress of this study item is enclosed

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

R4-2100090	Blank TR for extended 600MHz NR band

Type: SID new		For: Decision
					Source: Spark NZ Ltd

Abstract:

A blank TR to document the ongoing progress of this study item

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

R4-2100167	Blank TR for extended 600MHz NR band

Type: SID new		For: Decision
					Source: Spark NZ Ltd

Abstract:

A blank TR to document the ongoing progress of this study item

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

R4-2103265	Blank TR for extended 600MHz NR band

Type: draft TR		For: Agreement
					Source: Spark NZ Ltd

Decision: 		The document was agreed.

R4-2100501	Consideration on extended 600MHz NR band

Type: discussion		For: Decision
					Source: CATT,CBN

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

R4-2100542	Extended 600MHz NR Duplexer Feasibility and Band Arrangement

Type: discussion		For: Approval
					38.101-1 v..
					Source: Skyworks Solutions Inc.

Abstract:

In this contribution, we analyze the different duplexer options in light of the duplexer implementation of other FDD bands in the same frequency range like band 71 and band 28/68 and make alternative proposals for the support of envisaged 2x40MHz spectrum

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

R4-2100746	Frequency arrangements for APT 600 MHz

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

duplexer feasiblity of APT 600 is discussed

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

R4-2101372	Discussion on frequency arrangement for extended 600MHz NR Band

Type: discussion		For: Approval
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

R4-2101958	Discussions on Option B1 and B2 for extended 600MHz

Type: other		For: Discussion
					Source: ZTE Corporation, CBN

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

R4-2102161	Extended 600MHz band - frequency arrangement

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution is analyzing the 2 proposed options

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

R4-2102407	600 MHz band for Region 3

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

R4-2102574	Feasibility analysis of the frequency arrangement in 600MHz range for APT

Type: discussion		For: Agreement
					Source: Huawei, CBN

Abstract:

In this contribution we provide the requested feasibility analysis of the possible frequency arrangements for 600MHz range for APT region, based on the pre-defined options B1 and B2. Related TP to TR is provided for approval.

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was noted.

#### 12.4.5	Others

R4-2102575	[DRAFT] Reply LS on technical feasibilities for frequency arrangements for IMT in 470

Type: LS out		For: Approval
					to APT Wireless Group, cc TSG RAN
					Source: Huawei, CBN

Abstract:

In this contribution we provide draft reply LS to the APT Wireless Group LS in RP-202934, based on the discussion on technical feasibilities for frequency arrangements for IMT in 470 – 703 MHz band.

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was revised to R4-2103269.

R4-2103269	[DRAFT] Reply LS on technical feasibilities for frequency arrangements for IMT in 470

Type: LS out		For: Approval
					to APT Wireless Group, cc TSG RAN
					Source: Huawei, CBN

(Replaces R4-2102575)

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was revised to R4-2103364.

R4-2103364	[DRAFT] Reply LS on technical feasibilities for frequency arrangements for IMT in 470

Type: LS out		For: Approval
					to APT Wireless Group, cc TSG RAN
					Source: Huawei, CBN

(Replaces R4-2103269)

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335.

Decision: 		The document was revised to R4-2103406.

R4-2103406	Reply LS on technical feasibilities for frequency arrangements for IMT in 470

Type: LS out		For: Approval
					to APT Wireless Group, cc TSG RAN
					Source: RAN4

(Replaces R4-2103364)

Discussion:

See email discussion summary for [98e][148] FS_NR_600MHz_ext in R4-2103335. The document was subject for post-meeting email approval.

Decision: 		The document was approved.

### 12.5	Study on high power UE (power class 2) for one NR FDD band [FS_NR_PC2_UE_FDD]

R4-2102997	Email discussion summary for [98e][149] FS_NR_PC2_UE_FDD

Type: other		For: discussion
					Source: Moderator (China Unicom)

Discussion:

The contribution summarised email discussion thread [98e][149] FS_NR_PC2_UE_FDD. The discussion thread was moderated by Basaier Jialade (China Unicom).

Decision: 		The document was revised to R4-2103336.

R4-2103336	Email discussion summary for [98e][149] FS_NR_PC2_UE_FDD

Type: other		For: discussion
					Source: Moderator (China Unicom)

(Replaces R4-2102997)

Discussion:

The contribution summarised email discussion thread [98e][149] FS_NR_PC2_UE_FDD. The discussion thread was moderated by Basaier Jialade (China Unicom).

Decision: 		The document was noted.

R4-2103273	Way forward on PC2 for NR FDD band

Type: other		For: discussion
					Source: China Unicom

Discussion:

See email discussion summary for [98e][149] FS_NR_PC2_UE_FDD in R4-2103336.

Decision: 		The document was approved.

R4-2103274	Way forward on simulation assumptions for system performance evaluation

Type: other		For: discussion
					Source: China Unicom

Discussion:

See email discussion summary for [98e][149] FS_NR_PC2_UE_FDD in R4-2103336.

Decision: 		The document was approved.

#### 12.5.1	General

R4-2100081	Work plan for study on high power UE (power class 2) for one NR FDD band

Type: Work Plan		For: Approval
					Source: China Unicom

Discussion:

See email discussion summary for [98e][149] FS_NR_PC2_UE_FDD in R4-2103336.

Decision: 		The document was revised to R4-2103272.

R4-2103272	Work plan for study on high power UE (power class 2) for one NR FDD band

Type: Work Plan		For: Approval
					Source: China Unicom

(Replaces R4-2100081)

Discussion:

See email discussion summary for [98e][149] FS_NR_PC2_UE_FDD in R4-2103336.

Decision: 		The document was approved.

R4-2101110	Discussion on HP UE for FDD bands

Type: discussion		For: Discussion
					Source: Xiaomi

Discussion:

See email discussion summary for [98e][149] FS_NR_PC2_UE_FDD in R4-2103336.

Decision: 		The document was noted.

R4-2102391	On NR FDD HPUE

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][149] FS_NR_PC2_UE_FDD in R4-2103336.

Decision: 		The document was noted.

R4-2102610	Views on HPUE for NR FDD bands

Type: other		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][149] FS_NR_PC2_UE_FDD in R4-2103336.

Decision: 		The document was noted.

R4-2102708	Discussion on HPUE in NR FDD band

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][149] FS_NR_PC2_UE_FDD in R4-2103336.

Decision: 		The document was noted.

#### 12.5.2	Scheme(s) to comply with the SAR limits

R4-2100110	Discussion on SAR schemes for PC2 FDD band

Type: discussion		For: Discussion
					Source: CMCC

Discussion:

See email discussion summary for [98e][149] FS_NR_PC2_UE_FDD in R4-2103336.

Decision: 		The document was noted.

R4-2102135	Discussion on SAR scheme for HPUE FDD

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][149] FS_NR_PC2_UE_FDD in R4-2103336.

Decision: 		The document was noted.

R4-2102186	Discussion on SAR scheme for HPUE FDD

Type: other		For: Approval
					Source: ZTE Corporation

Discussion:

See email discussion summary for [98e][149] FS_NR_PC2_UE_FDD in R4-2103336.

Decision: 		The document was noted.

#### 12.5.3	Interference issues

#### 12.5.4	UE implementation issues

R4-2100290	Consideration of current RF components feasibility for FDD PC2 UE

Type: other		For: Approval
					Source: LG Electronics France

Abstract:

Provide current RF component state of art technology for PC2 UE in FDD band

Discussion:

See email discussion summary for [98e][149] FS_NR_PC2_UE_FDD in R4-2103336.

Decision: 		The document was noted.

R4-2100543	PC2 FDD RFFE Technology, Performance, Thermal and Power Aspects

Type: discussion		For: Approval
					38.101-1 v..
					Source: Skyworks Solutions Inc.

Abstract:

In this contribution we study the impact PC2 operation on the RE front-end in terms of performance, reliability and thermal aspects

Discussion:

See email discussion summary for [98e][149] FS_NR_PC2_UE_FDD in R4-2103336.

Decision: 		The document was noted.

#### 12.5.5	System performance evaluations

R4-2102392	System performance evaluation for FDD HPUE

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][149] FS_NR_PC2_UE_FDD in R4-2103336.

Decision: 		The document was noted.

R4-2102503	Simulation assumptions for FDD HPUE performance evaluations

Type: discussion		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][149] FS_NR_PC2_UE_FDD in R4-2103336.

Decision: 		The document was noted.

R4-2102709	Initial evaulation of PC2 UE for NR FDD

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][149] FS_NR_PC2_UE_FDD in R4-2103336.

Decision: 		The document was noted.

## 13	Rel-17 Work Items for LTE

R4-2102233	Work plan of Rel-17 enhancements for NB-IoT and LTE-MTC

Type: Work Plan		For: Approval
					Source: Huawei, Ericsson

Abstract:

Work plan of Rel-17 enhancements for NB-IoT and LTE-MTC

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was not treated.

### 13.1	LTE inter-band Carrier Aggregation for 2 bands DL with 1 band UL [LTE_CA_R17_2BDL_1BUL]

R4-2102998	Email discussion summary for [98e][150] LTE_Baskets

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][150] LTE_Baskets. The discussion thread was moderated by Per Lindell (Ericsson).

Decision: 		The document was noted.

#### 13.1.1	Rapporteur Input (WID/TR/CR) [LTE_CA_R17_2BDL_1BUL-Core/Perf]

R4-2102494	Revised WID: Rel17 LTE inter-band CA for 2 bands DL with 1 band UL

Type: WID revised		For: (not specified)
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2102495	Introduction of Rel-17 LTE inter-band CA for 2 bands DL with 1 band UL combinations in TS 36.101

Type: CR		For: Agreement
					36.101 v17.0.0	  CR-5723  Cat: B (Rel-17)

					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102496	TR 36.717-02-01 Rel-17 LTE inter-band CA for 2 bands DL and 1 band UL CA

Type: draft TR		For: Agreement
					36.717-02-01 v0.2.0
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 13.1.2	UE RF with harmonic, close proximity and isolation issues [LTE_CA_R17_2BDL_1BUL-Core]

R4-2102313	draft CR to include 2A-2A-7C, 2A-2A-7A-7A, 2A-2A-13A

Type: draftCR		For: Endorsement
					36.101 v17.0.0
					Source: Ericsson, Bell Mobility

Abstract:

draft CR to include 2A-2A-7C, 2A-2A-7A-7A, 2A-2A-13A

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was endorsed.

#### 13.1.3	UE RF without specific issues  [LTE_CA_R17_2BDL_1BUL-Core]

R4-2100151	TP to TR 36.717-02-01: Addition of CA_48-53

Type: pCR		For: Approval
					36.717-02-01 v0.1.0
					Source: Nokia, Globalstar

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2100729	TP to TR 36.717-02-01 CA_7-25

Type: other		For: Approval
					36.717-02-01 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, a text proposal to complete E-UTRA CA configurations, CA_7A-25A, CA_7A-7A-25, CA_7C-25A, CA_7A-25A-25A, CA_7A-7A-25A-25A, and CA_7C-25A-25A are provided.

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2100730	TP to TR 36.717-02-01 CA_25-66

Type: other		For: Approval
					36.717-02-01 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, a text proposal to complete E-UTRA CA configurations, CA_25-66A and CA_25A-25A-66A, is provided.

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101575	TP for TR 36.717-02-01: CA_2A-8A

Type: pCR		For: Approval
					36.717-02-01 v0.1.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101936	TP to TR 36.717-02-01 Addition of CA_46-53

Type: pCR		For: Approval
					36.717-02-01 v0.1.0
					Source: Nokia, Globalstar

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

### 13.2	LTE inter-band Carrier Aggregation for 3 bands DL with 1 band UL [LTE_CA_R17_3BDL_1BUL]

#### 13.2.1	Rapporteur Input (WID/TR/CR) [LTE_CA_R17_3BDL_1BUL-Core/Perf]

R4-2102830	Revised WID for LTE inter-band CA  for  3 bands DL with 1 bands UL

Type: WID revised		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998. The document was subject for post-meeting email agreement.

Decision: 		The document was withdrawn.

R4-2102832	Introduction of completed R17 3DL band combinations to TS 36.101

Type: CR		For: Agreement
					36.101 v17.0.0	  CR-5727  Cat: B (Rel-17)

					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102833	TR 36.717-03-01 0.2.0

Type: draft TR		For: Agreement
					36.717-03-01 v0.1.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 13.2.2	UE RF with harmonic, close proximity and isolation issues [LTE_CA_R17_3BDL_1BUL-Core]

R4-2101398	TP for TR 36.717-03-01: CA_1-8-32

Type: pCR		For: Approval
					36.717-03-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101399	TP for TR 36.717-03-01: CA_1-28-32

Type: pCR		For: Approval
					36.717-03-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was revised to R4-2103096.

R4-2103096	TP for TR 36.717-03-01: CA_1-28-32

Type: pCR		For: Approval
					36.717-03-01 v0.1.0
					Source: VODAFONE Group Plc

(Replaces R4-2101399)

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101400	TP for TR 36.717-03-01: CA_7-8-28

Type: pCR		For: Approval
					36.717-03-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101401	TP for TR 36.717-03-01: CA_7-8-32

Type: pCR		For: Approval
					36.717-03-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101402	TP for TR 36.717-03-01: CA_7-28-32

Type: pCR		For: Approval
					36.717-03-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101403	TP for TR 36.717-03-01: CA_8-20-32

Type: pCR		For: Approval
					36.717-03-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101404	TP for TR 36.717-03-01: CA_8-28-32

Type: pCR		For: Approval
					36.717-03-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101405	TP for TR 36.717-03-01: CA_20-28-32

Type: pCR		For: Approval
					36.717-03-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101907	draft CR to inlcude 7A-12A-66A-66A, 2A-2A-5A-7A, 2A-2A-7A-66A-66A, 2A-2A-7A-7A-13A, 2A-2A-7C-13A, 2A-2A-7A-13A

Type: draftCR		For: Endorsement
					36.101 v17.0.0
					Source: Ericsson, Rogers, Bell Mobility

Abstract:

draft CR to inlcude 7A-12A-66A-66A, 2A-2A-5A-7A, 2A-2A-7A-66A-66A, 2A-2A-7A-7A-13A, 2A-2A-7C-13A, 2A-2A-7A-13A

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was endorsed.

#### 13.2.3	UE RF without specific issues [LTE_CA_R17_3BDL_1BUL-Core]

R4-2100731	TP to TR 36.717-03-01 CA_7-25-66

Type: other		For: Approval
					36.717-03-01 v..
					Source: Nokia, Nokia Shanghai Bell

Abstract:

In this contribution, a text proposal to complete E-UTRA CA configurations, CA_7A-25A-66A, CA_7A-7A-25A-66A, CA_7C-25A-66A, CA_7A-25A-25A-66A, CA_7A-7A-25A-25A-66A, and CA_7C-25A-25A-66A, is provided.

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101516	DraftCR for 36.101 to add CA_5A-7A-7A-66A, CA_2A-5A-7A-7A and CA_7A-7A-13A-66A

Type: draftCR		For: Endorsement
					36.101 v17.0.0
					Source: Huawei, HiSilicon, Bell Mobility, Telus

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was endorsed.

### 13.3	LTE inter-band Carrier Aggregation for x bands DL (x=4, 5) with 1 band UL [LTE_CA_R17_xBDL_1BUL]

R4-2101522	TP for TR 36.717-04-01: CA_3-7-8-20-28

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101525	TP for TR 36.717-04-01: CA_1-3-7-8-20-28

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101544	TP for TR 36.717-04-01: CA_1-7-8-20-28-32

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

#### 13.3.1	Rapporteur Input (WID/TR/CR) [LTE_CA_R17_xBDL_1BUL-Core]

R4-2100728	Introduction of LTE inter-band Carrier Aggregation for x bands DL (x=4, 5) with 1 band UL to TS36.101

Type: CR		For: Agreement
					36.101 v17.0.0	  CR-5720  Cat: B (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Abstract:

This is a big CR for the basket work item on LTE CA 4DL/1UL and 5DL/1UL.

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2102438	Revised WID: LTE Advanced inter-band CA Rel-17 for x bands DL (x=4, 5) with 1 band UL

Type: WID revised		For: Endorsement
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2102439	Updated scope of TR: LTE inter-band CA for 4/5 bands DL with 1 band UL

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2102440	TR 36.717-04-01 v0.3.0

Type: draft TR		For: Agreement
					36.717-04-01 v0.3.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 13.3.2	UE RF with 4 LTE bands CA [LTE_CA_R17_xBDL_1BUL-Core]

R4-2101406	TP for TR 36.717-04-01: CA_1A-3A-7C-20A with UL CA_7C

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101468	TP for TR 36.717-04-01: CA_1-3-40-41

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101469	TP for TR 36.717-04-01: CA_1-7-8-28

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101470	TP for TR 36.717-04-01: CA_1-7-8-32

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101471	TP for TR 36.717-04-01: CA_1-7-28-32

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101472	TP for TR 36.717-04-01: CA_1-8-20-32

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101473	TP for TR 36.717-04-01: CA_1-8-28-32

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101474	TP for TR 36.717-04-01: CA_1-20-28-32

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101475	TP for TR 36.717-04-01: CA_3-7-8-28

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101476	TP for TR 36.717-04-01: CA_3-8-40-41

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101477	TP for TR 36.717-04-01: CA_7-8-20-28

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101478	TP for TR 36.717-04-01: CA_7-8-20-32

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101479	TP for TR 36.717-04-01: CA_7-8-28-32

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101480	TP for TR 36.717-04-01: CA_7-20-28-32

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101481	TP for TR 36.717-04-01: CA_8-20-28-32

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101517	DraftCR for 36.101 to add CA_2A-5A-7A-7A-66A and CA_2A-7A-7A-13A-66A

Type: draftCR		For: Endorsement
					36.101 v17.0.0
					Source: Huawei, HiSilicon, Bell Mobility, Telus

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was endorsed.

R4-2101908	draft CR to include 2A-7A-12A-66A-66A, 2A-2A-5A-7A-66A

Type: draftCR		For: Endorsement
					36.101 v17.0.0
					Source: Ericsson, Rogers

Abstract:

draft CR to include 2A-7A-12A-66A-66A, 2A-2A-5A-7A-66A

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was endorsed.

R4-2102624	TP to TR 36.717-04-01 Correction of CA_2-5-7-66-66

Type: pCR		For: Approval
					36.717-04-01 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

#### 13.3.3	UE RF with 5 LTE bands CA [LTE_CA_R17_xBDL_1BUL-Core]

R4-2101488	TP for TR 36.717-04-01: CA_1-3-7-8-28

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101489	TP for TR 36.717-04-01: CA_1-3-8-20-28

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101490	TP for TR 36.717-04-01: CA_1-7-8-20-28

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101491	TP for TR 36.717-04-01: CA_1-7-8-20-32

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101492	TP for TR 36.717-04-01: CA_1-7-8-28-32

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101493	TP for TR 36.717-04-01: CA_1-7-20-28-32

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101524	TP for TR 36.717-04-01: CA_7-8-20-28-32

Type: pCR		For: Approval
					36.717-04-01 v0.1.0
					Source: VODAFONE Group Plc

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101582	TP for TR 36.717-04-01: CA_1A-3A-8A-20A-38A

Type: pCR		For: Approval
					36.717-04-01 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101583	TP for TR 36.717-04-01: CA_1A-3A-7A-8A-38A

Type: pCR		For: Approval
					36.717-04-01 v0.2.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

### 13.4	LTE inter-band Carrier Aggregation for 2 bands DL with 2 band UL [LTE_CA_R17_2BDL_2BUL]

#### 13.4.1	Rapporteur Input (WID/TR/CR) [LTE_CA_R17_2BDL_2BUL-Core]

R4-2102404	Introduction of completed R17 2DL2UL band combinations to TS 36.101

Type: draftCR		For: Endorsement
					36.101 v17.0.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998. The document was subject for post-meeting email agreement.

Decision: 		The document was withdrawn.

R4-2102405	Revised WID for LTE inter-band CA  for  2 bands DL with 2 bands UL

Type: WID revised		For: Endorsement
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

#### 13.4.2	UE RF with harmonic, close proximity and isolation issues [LTE_CA_R17_2BDL_2BUL-Core]

#### 13.4.3	UE RF without specific issues [LTE_CA_R17_2BDL_2BUL-Core]

### 13.5	LTE inter-band Carrier Aggregation for x bands DL (x= 3, 4, 5) with 2 band UL [LTE_CA_R17_xBDL_2BUL]

#### 13.5.1	Rapporteur Input (WID/TR/CR) [LTE_CA_R17_xBDL_2BUL-Core]

R4-2100265	TR 36.717-03-02 v0.2.0 TR Update for LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17

Type: draft TR		For: Agreement
					36.717-03-02 v0.2.0
					Source: LG Electronics France

Abstract:

Update TR to capture approved TPs in RAN4 #98-e meeting

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

R4-2100267	Revised WID on LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17

Type: WID revised		For: Endorsement
					Source: LG Electronics France

Abstract:

Revised WID to capture new CA band combos and update status for each CA band

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998. The document was subject for post-meeting email agreement.

Decision: 		The document was endorsed.

R4-2100270	Introduction of LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL to TS36.101

Type: CR		For: Agreement
					36.101 v17.0.0	  CR-5715  Cat: B (Rel-17)

					Source: LG Electronics France

Abstract:

Big CR tO add new CA band combos for  x bands (x=3,4,5) DL with 2 bands UL in Rel-17

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

#### 13.5.2	UE RF with MSD [LTE_CA_R17_xBDL_2BUL-Core]

R4-2101576	Draft CR to 36.101 to add UL configuration CA_3A-8A for CA_3C-7A-8A

Type: draftCR		For: Endorsement
					36.101 v17.0.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was endorsed.

#### 13.5.3	UE RF without MSD [LTE_CA_R17_xBDL_2BUL-Core]

R4-2101577	Draft CR to 36.101 to add UL configuration CA_1A-3A for CA_1A-3C-38A

Type: draftCR		For: Endorsement
					36.101 v17.0.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was endorsed.

R4-2101578	Draft CR to 36.101 to add UL configuration CA_3A-8A for CA_3C-8A-38A

Type: draftCR		For: Endorsement
					36.101 v17.0.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was endorsed.

R4-2101579	TP for TR 36.717-03-02: to add UL configuration CA_1A-8A for CA_1A-8A-38A

Type: pCR		For: Approval
					36.717-03-02 v0.1.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was revised to R4-2103098.

R4-2103098	TP for TR 36.717-03-02: to add UL configuration CA_1A-8A for CA_1A-8A-38A

Type: pCR		For: Approval
					36.717-03-02 v0.1.0
					Source: Huawei, HiSilicon

(Replaces R4-2101579)

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was approved.

R4-2101580	Draft CR to 36.101 to add UL configuration CA_1A-3A for CA_1A-3C-7A-8A

Type: draftCR		For: Endorsement
					36.101 v17.0.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was endorsed.

R4-2101581	Draft CR to 36.101 to add UL configuration CA_1A-3A for CA_1A-3C-8A-38A

Type: draftCR		For: Endorsement
					36.101 v17.0.0
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][150] LTE_Baskets in R4-2102998.

Decision: 		The document was endorsed.

### 13.6	RRM for LTE CA basket WIs  [LTE_CA_R17_xxxx]

#### 13.6.1	RRM Core (36.133) [LTE_CA_R17_xxxx-Core]

#### 13.6.2	RRM Perf (36.133) [LTE_CA_R17_xxxx-Perf]

R4-2102999	Email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2

Type: other		For: discussion
					Source: Moderator (Ericsson)

Discussion:

The contribution summarised email discussion thread [98e][151] LTE_bands_R17_M1_M2_NB1_NB2. The discussion thread was moderated by Chunhui Zhang (Ericsson).

Decision: 		The document was revised to R4-2103337.

R4-2103337	Email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2

Type: other		For: discussion
					Source: Moderator (Ericsson)

(Replaces R4-2102999)

Discussion:

The contribution summarised email discussion thread [98e][151] LTE_bands_R17_M1_M2_NB1_NB2. The discussion thread was moderated by Chunhui Zhang (Ericsson).

Decision: 		The document was noted.

### 13.7	New WID on Additional LTE bands for UE category M1&M2 and/or NB1&NB2 in Rel-17 [LTE_bands_R17_M1_M2_NB1_NB2]

#### 13.7.1	Rapporteur Input (WID/TR/CR) [LTE_bands_R17_M1_M2_NB1_NB2-Core]

R4-2100604	CR of adding LTE B24 for UE category NB1 in R17

Type: CR		For: Agreement
					36.307 v13.12.0	  CR-4443  Cat: B (Rel-13)

					Source: Ericsson, Ligado Networks

Abstract:

CR of adding LTE B24 for UE category NB1 in R17

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was revised to R4-2103275.

R4-2103275	CR of adding LTE B24 for UE category NB1 in R17

Type: CR		For: Agreement
					36.307 v13.12.0	  CR-4443  rev 1 Cat: B (Rel-13)

					Source: Ericsson, Ligado Networks

(Replaces R4-2100604)

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was agreed.

R4-2100605	CR of adding LTE B24 for UE category NB1 in R17

Type: CR		For: Agreement
					36.307 v14.9.0	  CR-4444  Cat: A (Rel-14)

					Source: Ericsson, Ligado Networks

Abstract:

CR of adding LTE B24 for UE category NB1 in R17

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was agreed.

R4-2100606	CR of adding LTE B24 for UE category NB1 in R17

Type: CR		For: Agreement
					36.307 v15.6.0	  CR-4445  Cat: A (Rel-15)

					Source: Ericsson, Ligado Networks

Abstract:

CR of adding LTE B24 for UE category NB1 in R17

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was agreed.

R4-2100607	CR of adding LTE B24 for UE category NB1 in R17

Type: CR		For: Agreement
					36.307 v16.2.0	  CR-4446  Cat: A (Rel-16)

					Source: Ericsson, Ligado Networks

Abstract:

CR of adding LTE B24 for UE category NB1 in R17

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was agreed.

R4-2100608	CR of adding LTE B24 for UE category NB1/NB2 in R17

Type: CR		For: Agreement
					36.101 v17.0.0	  CR-5716  Cat: B (Rel-17)

					Source: Ericsson, Ligado Networks

Abstract:

CR of adding LTE B24 for UE category NB1/NB2 in R17

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was revised to R4-2103276.

R4-2103276	CR of adding LTE B24 for UE category NB1/NB2 in R17

Type: CR		For: Agreement
					36.101 v17.0.0	  CR-5716  rev 1 Cat: B (Rel-17)

					Source: Ericsson, Ligado Networks

(Replaces R4-2100608)

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was agreed.

R4-2100609	CR of adding LTE B24 for UE category NB1/NB2 in R17

Type: CR		For: Agreement
					36.104 v17.0.0	  CR-4921  Cat: B (Rel-17)

					Source: Ericsson, Ligado Networks

Abstract:

CR of adding LTE B24 for UE category NB1/NB2 in R17

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was revised to R4-2103277.

R4-2103277	CR of adding LTE B24 for UE category NB1/NB2 in R17

Type: CR		For: Agreement
					36.104 v17.0.0	  CR-4921  rev 1 Cat: B (Rel-17)

					Source: Ericsson, Ligado Networks

(Replaces R4-2100609)

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was agreed.

R4-2100610	CR of adding LTE B24 for UE category NB1/NB2 in R17

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7016  Cat: B (Rel-17)

					Source: Ericsson, Ligado Networks

Abstract:

CR of adding LTE B24 for UE category NB1/NB2 in R17

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was revised to R4-2103278.

R4-2103278	CR of adding LTE B24 for UE category NB1/NB2 in R17

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7016  rev 1 Cat: B (Rel-17)

					Source: Ericsson, Ligado Networks

(Replaces R4-2100610)

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was agreed.

R4-2100612	CR of adding LTE B24 for UE category NB1/NB2 in R17

Type: CR		For: Agreement
					37.104 v17.0.0	  CR-0921  Cat: B (Rel-17)

					Source: Ericsson, Ligado Networks

Abstract:

CR of adding LTE B24 for UE category NB1/NB2 in R17

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was revised to R4-2103279.

R4-2103279	CR of adding LTE B24 for UE category NB1/NB2 in R17

Type: CR		For: Agreement
					37.104 v17.0.0	  CR-0921  rev 1 Cat: B (Rel-17)

					Source: Ericsson, Ligado Networks

(Replaces R4-2100612)

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was agreed.

R4-2100614	CR of adding LTE B24 for UE category NB1/NB2 in R17

Type: CR		For: Agreement
					36.307 v14.9.0	  CR-4447  Cat: B (Rel-14)

					Source: Ericsson, Ligado Networks

Abstract:

CR of adding LTE B24 for UE category NB2 in R17

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was revised to R4-2103280.

R4-2103280	CR of adding LTE B24 for UE category NB2 in R17

Type: CR		For: Agreement
					36.307 v14.9.0	  CR-4447  rev 1 Cat: B (Rel-14)

					Source: Ericsson, Ligado Networks

(Replaces R4-2100614)

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was agreed.

R4-2100615	CR of adding LTE B24 for UE category NB2 in R17

Type: CR		For: Agreement
					36.307 v15.6.0	  CR-4448  Cat: A (Rel-15)

					Source: Ericsson, Ligado Networks

Abstract:

CR of adding LTE B24 for UE category NB2 in R17

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was agreed.

R4-2100616	CR of adding LTE B24 for UE category NB2 in R17

Type: CR		For: Agreement
					36.307 v16.2.0	  CR-4449  Cat: A (Rel-16)

					Source: Ericsson, Ligado Networks

Abstract:

CR of adding LTE B24 for UE category NB2 in R17

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was agreed.

#### 13.7.2	RF  [LTE_bands_R17_M1_M2_NB1_NB2-Core]

#### 13.7.3	Others  [LTE_bands_R17_M1_M2_NB1_NB2-Perf]

R4-2100611	CR of adding LTE B24 for UE category NB1/NB2 in R17

Type: CR		For: Agreement
					36.141 v17.0.0	  CR-1289  Cat: B (Rel-17)

					Source: Ericsson, Ligado Networks

Abstract:

CR of adding LTE B24 for UE category NB1/NB2 in R17

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was revised to R4-2103281.

R4-2103281	CR of adding LTE B24 for UE category NB1/NB2 in R17

Type: CR		For: Agreement
					36.141 v17.0.0	  CR-1289  rev 1 Cat: B (Rel-17)

					Source: Ericsson, Ligado Networks

(Replaces R4-2100611)

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was agreed.

R4-2100613	CR of adding LTE B24 for UE category NB1/NB2 in R17

Type: CR		For: Agreement
					37.141 v17.0.0	  CR-0960  Cat: B (Rel-17)

					Source: Ericsson, Ligado Networks

Abstract:

CR of adding LTE B24 for UE category NB1/NB2 in R17

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was revised to R4-2103282.

R4-2103282	CR of adding LTE B24 for UE category NB1/NB2 in R17

Type: CR		For: Agreement
					37.141 v17.0.0	  CR-0960  rev 1 Cat: B (Rel-17)

					Source: Ericsson, Ligado Networks

(Replaces R4-2100613)

Discussion:

See email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 in R4-2103337.

Decision: 		The document was agreed.

### 13.8	Modification of LTE Band 24 specifications to comply with updated regulatory emission limits [LTE_B24_mod]

#### 13.8.1	General and rapporteur input [LTE_B24_mod-Core]

#### 13.8.2	UE RF [LTE_B24_mod-Core]

R4-2100254	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v10.28.0	  CR-5714  Cat: F (Rel-10)

					Source: Ligado Networks, Nokia, Skyworks Solutions Inc.

Abstract:

Changes to TS 36.101 related to recent Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103379.

R4-2103379	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v10.28.0	  CR-5714  rev 1 Cat: F (Rel-10)

					Source: Ligado Networks, Nokia, Skyworks Solutions Inc.

(Replaces R4-2100254)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102909	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v11.25.0	  CR-5728  Cat: A (Rel-11)

					Source: Ligado Networks

Abstract:

Changes to 36.101 related to Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103380.

R4-2103380	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v11.25.0	  CR-5728  rev 1 Cat: A (Rel-11)

					Source: Ligado Networks

(Replaces R4-2102909)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102910	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v12.25.0	  CR-5729  Cat: A (Rel-12)

					Source: Ligado Networks

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103381.

R4-2103381	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v12.25.0	  CR-5729  rev 1 Cat: A (Rel-12)

					Source: Ligado Networks

(Replaces R4-2102910)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102911	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v17.0.0	  CR-5730  Cat: A (Rel-17)

					Source: Ligado Networks

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was withdrawn.

R4-2102913	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v14.17.0	  CR-5731  Cat: A (Rel-14)

					Source: Ligado Networks

Abstract:

Changes to 36.101 related to Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103382.

R4-2103382	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v14.17.0	  CR-5731  rev 1 Cat: A (Rel-14)

					Source: Ligado Networks

(Replaces R4-2102913)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102914	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v15.13.0	  CR-5732  Cat: A (Rel-15)

					Source: Ligado Networks

Abstract:

Changes to 31.101 related to Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103383.

R4-2103383	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v15.13.0	  CR-5732  rev 1 Cat: A (Rel-15)

					Source: Ligado Networks

(Replaces R4-2102914)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102915	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v16.8.0	  CR-5733  Cat: A (Rel-16)

					Source: Ligado Networks

Abstract:

Changes to 31.101 related to Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103384.

R4-2103384	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v16.8.0	  CR-5733  rev 1 Cat: A (Rel-16)

					Source: Ligado Networks

(Replaces R4-2102915)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102916	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v17.0.0	  CR-5734  Cat: A (Rel-17)

					Source: Ligado Networks

Abstract:

Changes to 31.101 related to Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103385.

R4-2103385	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v17.0.0	  CR-5734  rev 1 Cat: A (Rel-17)

					Source: Ligado Networks

(Replaces R4-2102916)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102918	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v13.19.1	  CR-5735  Cat: A (Rel-13)

					Source: Ligado Networks

Abstract:

Changes to 31.101 related to Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103386.

R4-2103386	CR for 36.101: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					36.101 v13.19.1	  CR-5735  rev 1 Cat: A (Rel-13)

					Source: Ligado Networks

(Replaces R4-2102918)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

#### 13.8.3	BS RF [LTE_B24_mod-Core]

R4-2100255	CR for 37.105: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					37.105 v15.11.0	  CR-0215  Cat: F (Rel-15)

					Source: Ligado Networks

Abstract:

Changes to TS 37.105 related to Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100532	CR for 37.105: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					37.105 v16.6.0	  CR-0217  Cat: A (Rel-16)

					Source: Ligado Networks

Abstract:

Corrections to TS 37.105 related to Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100533	CR for 37.105: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					37.105 v17.0.0	  CR-0218  Cat: A (Rel-17)

					Source: Ligado Networks

Abstract:

Changes to TS 37.105 related to Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102452	CR to 36.104: Correction to Band 24 requirements (Rel-10)

Type: CR		For: Agreement
					36.104 v10.13.0	  CR-4928  Cat: F (Rel-10)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103207.

R4-2103207	CR to 36.104: Correction to Band 24 requirements (Rel-10)

Type: CR		For: Agreement
					36.104 v10.13.0	  CR-4928  rev 1 Cat: F (Rel-10)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102452)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102453	CR to 36.104: Correction to Band 24 requirements (Rel-11)

Type: CR		For: Agreement
					36.104 v11.17.0	  CR-4929  Cat: A (Rel-11)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102454	CR to 36.104: Correction to Band 24 requirements (Rel-12)

Type: CR		For: Agreement
					36.104 v12.13.0	  CR-4930  Cat: A (Rel-12)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102455	CR to 36.104: Correction to Band 24 requirements (Rel-13)

Type: CR		For: Agreement
					36.104 v13.13.0	  CR-4931  Cat: A (Rel-13)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102456	CR to 36.104: Correction to Band 24 requirements (Rel-14)

Type: CR		For: Agreement
					36.104 v14.9.0	  CR-4932  Cat: A (Rel-14)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102457	CR to 36.104: Correction to Band 24 requirements (Rel-15)

Type: CR		For: Agreement
					36.104 v15.10.0	  CR-4933  Cat: A (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102458	CR to 36.104: Correction to Band 24 requirements (Rel-16)

Type: CR		For: Agreement
					36.104 v16.8.0	  CR-4934  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102459	CR to 36.104: Correction to Band 24 requirements (Rel-17)

Type: CR		For: Agreement
					36.104 v17.0.0	  CR-4935  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102468	CR to 37.104: Correction to Band 24 requirements (Rel-10)

Type: CR		For: Agreement
					37.104 v10.14.0	  CR-0925  Cat: F (Rel-10)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103208.

R4-2103208	CR to 37.104: Correction to Band 24 requirements (Rel-10)

Type: CR		For: Agreement
					37.104 v10.14.0	  CR-0925  rev 1 Cat: F (Rel-10)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102468)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102469	CR to 37.104: Correction to Band 24 requirements (Rel-11)

Type: CR		For: Agreement
					37.104 v11.14.0	  CR-0926  Cat: A (Rel-11)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102470	CR to 37.104: Correction to Band 24 requirements (Rel-12)

Type: CR		For: Agreement
					37.104 v12.13.0	  CR-0927  Cat: A (Rel-12)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102471	CR to 37.104: Correction to Band 24 requirements (Rel-13)

Type: CR		For: Agreement
					37.104 v13.8.0	  CR-0928  Cat: A (Rel-13)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102472	CR to 37.104: Correction to Band 24 requirements (Rel-14)

Type: CR		For: Agreement
					37.104 v14.6.0	  CR-0929  Cat: A (Rel-14)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102473	CR to 37.104: Correction to Band 24 requirements (Rel-15)

Type: CR		For: Agreement
					37.104 v15.12.0	  CR-0930  Cat: A (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102474	CR to 37.104: Correction to Band 24 requirements (Rel-16)

Type: CR		For: Agreement
					37.104 v16.8.0	  CR-0931  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102475	CR to 37.104: Correction to Band 24 requirements (Rel-17)

Type: CR		For: Agreement
					37.104 v17.0.0	  CR-0932  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

#### 13.8.4	RRM and others [LTE_B24_mod-Core/Perf]

R4-2100256	CR for 37.145-1: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					37.145-1 v13.11.0	  CR-0235  Cat: F (Rel-13)

					Source: Ligado Networks

Abstract:

Changes to TS 37.145-1 related to Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100535	CR for 37.145-1: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					37.145-1 v14.9.0	  CR-0238  Cat: A (Rel-14)

					Source: Ligado Networks

Abstract:

Changes to 37.145-1 related to Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100536	CR for 37.145-1: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					37.145-1 v15.8.0	  CR-0239  Cat: A (Rel-15)

					Source: Ligado Networks

Abstract:

Changes to 37.145-1 related to Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100537	CR for 37.145-1: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					37.145-1 v16.5.0	  CR-0240  Cat: A (Rel-16)

					Source: Ligado Networks

Abstract:

Changes to 37.145-1 related to Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100538	CR for 37.145-1: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					37.145-1 v17.0.0	  CR-0241  Cat: A (Rel-17)

					Source: Ligado Networks

Abstract:

Changes to 37.145-1 related to Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100257	CR for 37.145-2: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					37.145-2 v15.9.0	  CR-0269  Cat: F (Rel-15)

					Source: Ligado Networks

Abstract:

Changes to 37.145-2 related to Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103215.

R4-2103215	CR for 37.145-2: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					37.145-2 v15.9.0	  CR-0269  rev 1 Cat: F (Rel-15)

					Source: Ligado Networks

(Replaces R4-2100257)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100539	CR for 37.145-2: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					37.145-2 v16.6.0	  CR-0274  Cat: A (Rel-16)

					Source: Ligado Networks

Abstract:

Changes to 37.145-2 related to Band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100540	CR for 37.145-2: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					37.145-2 v17.0.0	  CR-0275  Cat: A (Rel-17)

					Source: Ligado Networks

Abstract:

Changes to 37.145-2 related to band 24 regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2100534	CR for 37.145-1: Corrections related to Band 24 regulatory updates

Type: CR		For: Agreement
					37.145-1 v13.11.0	  CR-0237  Cat: A (Rel-13)

					Source: Ligado Networks

Abstract:

Changes to 37.145-1 related to regulatory updates

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was withdrawn.

R4-2102460	CR to 36.141: Correction to Band 24 requirements (Rel-10)

Type: CR		For: Agreement
					36.141 v10.14.0	  CR-1296  Cat: F (Rel-10)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103217.

R4-2103217	CR to 36.141: Correction to Band 24 requirements (Rel-10)

Type: CR		For: Agreement
					36.141 v10.14.0	  CR-1296  rev 1 Cat: F (Rel-10)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102460)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102461	CR to 36.141: Correction to Band 24 requirements (Rel-11)

Type: CR		For: Agreement
					36.141 v11.17.0	  CR-1297  Cat: A (Rel-11)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102462	CR to 36.141: Correction to Band 24 requirements (Rel-12)

Type: CR		For: Agreement
					36.141 v12.14.0	  CR-1298  Cat: A (Rel-12)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102463	CR to 36.141: Correction to Band 24 requirements (Rel-13)

Type: CR		For: Agreement
					36.141 v13.15.0	  CR-1299  Cat: A (Rel-13)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102464	CR to 36.141: Correction to Band 24 requirements (Rel-14)

Type: CR		For: Agreement
					36.141 v14.12.0	  CR-1300  Cat: A (Rel-14)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102465	CR to 36.141: Correction to Band 24 requirements (Rel-15)

Type: CR		For: Agreement
					36.141 v15.11.0	  CR-1301  Cat: A (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102466	CR to 36.141: Correction to Band 24 requirements (Rel-16)

Type: CR		For: Agreement
					36.141 v16.8.0	  CR-1302  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102467	CR to 36.141: Correction to Band 24 requirements (Rel-17)

Type: CR		For: Agreement
					36.141 v17.0.0	  CR-1303  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102476	CR to 37.141: Correction to Band 24 requirements (Rel-10)

Type: CR		For: Agreement
					37.141 v10.14.0	  CR-0965  Cat: F (Rel-10)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was revised to R4-2103216.

R4-2103216	CR to 37.141: Correction to Band 24 requirements (Rel-10)

Type: CR		For: Agreement
					37.141 v10.14.0	  CR-0965  rev 1 Cat: F (Rel-10)

					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2102476)

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102477	CR to 37.141: Correction to Band 24 requirements (Rel-11)

Type: CR		For: Agreement
					37.141 v11.15.0	  CR-0966  Cat: A (Rel-11)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102478	CR to 37.141: Correction to Band 24 requirements (Rel-12)

Type: CR		For: Agreement
					37.141 v12.13.0	  CR-0967  Cat: A (Rel-12)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102479	CR to 37.141: Correction to Band 24 requirements (Rel-13)

Type: CR		For: Agreement
					37.141 v13.14.0	  CR-0968  Cat: A (Rel-13)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102480	CR to 37.141: Correction to Band 24 requirements (Rel-14)

Type: CR		For: Agreement
					37.141 v14.12.0	  CR-0969  Cat: A (Rel-14)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102481	CR to 37.141: Correction to Band 24 requirements (Rel-15)

Type: CR		For: Agreement
					37.141 v15.13.0	  CR-0970  Cat: A (Rel-15)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102482	CR to 37.141: Correction to Band 24 requirements (Rel-16)

Type: CR		For: Agreement
					37.141 v16.8.0	  CR-0971  Cat: A (Rel-16)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

R4-2102483	CR to 37.141: Correction to Band 24 requirements (Rel-17)

Type: CR		For: Agreement
					37.141 v17.0.0	  CR-0972  Cat: A (Rel-17)

					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][125] NR_LTE_band_n24 in R4-2103313.

Decision: 		The document was agreed.

## 14	Rel-17 Study Items for LTE

### 14.1	High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands 5 and 12 and NR band n71 [FS_LTE_NR_HPUE_FWVM]

R4-2103000	Email discussion summary for [98e][152] FS_LTE_NR_HPUE_FWVM

Type: other		For: discussion
					Source: Moderator (Nokia)

Discussion:

The contribution summarised email discussion thread [98e][152] FS_LTE_NR_HPUE_FWVM. The discussion thread was moderated by Man Hung Ng (Nokia).

Decision: 		The document was revised to R4-2103338.

R4-2103338	Email discussion summary for [98e][152] FS_LTE_NR_HPUE_FWVM

Type: other		For: discussion
					Source: Moderator (Nokia)

(Replaces R4-2103000)

Discussion:

The contribution summarised email discussion thread [98e][152] FS_LTE_NR_HPUE_FWVM. The discussion thread was moderated by Man Hung Ng (Nokia).

Decision: 		The document was noted.

R4-2103283	TR 37.880 V0.3.0: High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71

Type: draft TR		For: Agreement
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][152] FS_LTE_NR_HPUE_FWVM in R4-2103338.

Decision: 		The document was agreed.

#### 14.1.1	General

R4-2101798	Revised Work Plan for Study on High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell, T-Mobile USA, U.S. Cellular

Abstract:

This contribution provides a revised work plan according to the current progress. In view of the cancelled physical meetings and the current progress of this study item, it is proposed to request at RAN#91 to extend the study item for one quarter to compl

Discussion:

See email discussion summary for [98e][152] FS_LTE_NR_HPUE_FWVM in R4-2103338.

Decision: 		The document was approved.

R4-2101799	TR 37.880 V0.2.0: High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71

Type: draft TR		For: Agreement
					37.880 v0.2.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Updated TR for Study on High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71.

Discussion:

See email discussion summary for [98e][152] FS_LTE_NR_HPUE_FWVM in R4-2103338.

Decision: 		The document was agreed.

#### 14.1.2	Coexistence study

R4-2101800	TP to TR 37.880: Coexistence Simulation Results for High-power UE Vs NB-IoT guard band operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71

Type: pCR		For: Approval
					37.880 v0.1.0
					Source: Nokia, Nokia Shanghai Bell

Abstract:

This contribution provides the coexistence simulation results for this scenario according to the agreed assumptions and a text proposal for approval to record the simulation results and observations into TR 37.880.

Discussion:

See email discussion summary for [98e][152] FS_LTE_NR_HPUE_FWVM in R4-2103338.

Decision: 		The document was revised to R4-2103284.

R4-2103284	TP to TR 37.880: Coexistence Simulation Results for High-power UE Vs NB-IoT guard band operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71

Type: pCR		For: Approval
					37.880 v0.1.0
					Source: Nokia, Nokia Shanghai Bell

(Replaces R4-2101800)

Discussion:

See email discussion summary for [98e][152] FS_LTE_NR_HPUE_FWVM in R4-2103338.

Decision: 		The document was approved.

#### 14.1.3	UE RF

R4-2100131	HPUE for bands 5, 12 and n71 UE RF aspects.

Type: pCR		For: Approval
					37.880 v0.0.1
					Source: Nokia

Discussion:

See email discussion summary for [98e][152] FS_LTE_NR_HPUE_FWVM in R4-2103338.

Decision: 		The document was approved.

## 15	Liaison and output to other groups

### 15.1	R17 related

R4-2103478	Email discussion summary: [98e][239] R17_LS_RS_for_Scell_activation

Type: other		For: discussion
					Source: Moderator (Huawei)

Discussion:

The contribution summarised email discussion thread [98e][239] R17_LS_RS_for_Scell_activation. The discussion thread was moderated by Jing Han (Huawei).

Decision: 		The document was revised to R4-2103720.

R4-2103720	Email discussion summary: [98e][239] R17_LS_RS_for_Scell_activation

Type: other		For: discussion
					Source: Moderator (Huawei)

(Replaces R4-2103478)

Discussion:

The contribution summarised email discussion thread [98e][239] R17_LS_RS_for_Scell_activation. The discussion thread was moderated by Jing Han (Huawei).

Decision: 		The document was noted.

GTW session (February 04, 2021)

RAN1 questions (R1-2009798)

- Q1: to expedite SCell activation, RAN1 is studying whether and under which conditions (e.g. FR1/FR2, known/unknown cell, etc.), how many temporary RS bursts/symbols are required to achieve both UE AGC setting and time/frequency tracking. Does RAN4 have any information to share for these aspects?

- Q2: for AGC setting in intra-band CA comprising of a to-be-activated SCell and an activated serving cell, when a temporary RS is transmitted on the to-be-activated SCell, whether and under which conditions (e.g., FR1/FR2, known/unknown cell, etc.) the UE may require to receive another RS transmitted also on the other activated serving cell in the same band?

- Q3: does the RAN1 working assumption for temporary RS (i.e., reuse existing Rel-15/16 TRS structure) provides reduction in maximum allowed activation delay requirements (specified in subclause 8.3.2 of TS 38.133)? Also, are there any suggested changes from RAN4 perspective?

Sub-topic 1: SCell being activated is known and belongs to FR1

Sub-topic 1-1: If SCell measurement cycle is equal to or smaller than 160ms

Issue 1-1-2: How many temporary RS bursts are required for fine time tracking? (It is agreed in 1st round, the temporary RS can be used for fine time tracking)

- Background

  - In the incoming LS [R1-2009798], a burst of temporary RS is notated as

- A burst of temporary RS is notated as in S5.1.6.1.1 of TS 38.214 - “2-slot with four CSI-RSs resources (4 samples)” for FR1 - either “1-slot with two CSI-RSs resources (2 samples)” or “2-slot with four CSI-RSs resources (4 samples)” for FR2

- Proposals

  - Option 1: 1 burst (2-slot with four CSI-RSs resources)

  - Option 2: 1 burst (1-slot with two CSI-RSs resources)

- Discussion

  - MTK: If AGC gain is not needed, then we may use smaller number of resources. Option 2.

  - Apple: Need to follow RAN1 working assumption (i.e. only “2-slot with four CSI-RSs resources (4 samples)” in FR1.

  - vivo: same understanding as Apple. RAN1 is not asking RAN4 to further evaluate other options.

  - Huawei: In SSB-based SCell activation, 1 SSB sample is needed and same can apply for temporary RS. Option 2 can be formulated as 1/2 burst.

  - Intel: Although the LS says “2-slot with four CSI-RSs resources (4 samples)” for FR1, 1-slot may still be considered/configured by RAN1 spec. Need to understand how to cope with this.

  - OPPO: Same view as vivo and Huawei. The question in RAN1 LS how many RS bursts or symbols is needed. Number of slots can have impact on performance.

  - Intel: 1 burst is acceptable

- Agreements

  - Provide the following reply to RAN1

    - If SCell measurement cycle is equal to or smaller than 160ms

      - Temporary RS can be used for time/frequency tracking

        - 1 burst (2-slot with four CSI-RSs resources) is required based on RAN1 working assumptions on temporary RS design provided in the LS R1-2009798

Sub-topic 1-2: If SCell measurement cycle is larger than 160ms

- Agreements

  - Provide the following reply to RAN1

    - If SCell measurement cycle is larger than 160ms

      - temporary RS can be used for AGC

        - 1 burst (2-slot with four CSI-RSs resources) is required

      - temporary RS can be used for time/frequency tracking

        - 1 separate burst (2-slot with four CSI-RSs resources) is required in addition to the one burst required for AGC

    - The agreements above apply based on RAN1 working assumptions on temporary RS design provided in the LS R1-2009798

Sub-topic 5: Answers to Q3

Agreement: Reply to Q3

Q3: does the RAN1 working assumption for temporary RS (i.e., reuse existing Rel-15/16 TRS structure) provide reduction in maximum allowed activation delay requirements (specified in subclause 8.3.2 of TS 38.133)? Also, are there any suggested changes from RAN4 perspective?

[RAN4 Response]: RAN1 working assumption for temporary RS (i.e., reuse existing Rel-15/16 TRS structure) can provide reduction in maximum allowed activation delay at least for some cases as listed above. RAN4 will continue the discussion if there are any suggested changes from RAN4 perspective and will provide additional agreements (if any) in RAN4 #99-e.

R4-2103667	WF on temporary RS for efficient SCell activation in NR CA

Type: other		For: discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was revised to R4-2104083.

R4-2104083	WF on temporary RS for efficient SCell activation in NR CA

Type: other		For: discussion
					Source: Huawei, HiSilicon

(Replaces R4-2103667)

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was approved.

R4-2103668	Reply LS on temporary RS for efficient SCell activation in NR CA

Type: LS out		For: Approval
					to -
					Source: Huawei

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was revised to R4-2104067.

R4-2104067	Reply LS on temporary RS for efficient SCell activation in NR CA

Type: LS out		For: Approval
					to RAN1
					Source: RAN4

(Replaces R4-2103668)

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was approved.

R4-2100236	Discussion on RAN1 LS on temporary RS for efficient SCell activation in NR CA

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was noted.

R4-2101543	Reply LS on temporary RS for efficient SCell activation in NR CA

Type: LS out		For: Approval
					to RAN1
					Source: OPPO

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was noted.

R4-2101709	Discussion on temporary RS for efficient SCell activation in NR CA

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was noted.

R4-2102894	Discussion on temporary RS for efficient SCell activation in NR CA

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was noted.

R4-2103003	Email discussion summary for [98e][155] NR_reply_LS_Part_2

Type: other		For: discussion
					Source: Moderator (China Telecom)

Discussion:

The contribution summarised email discussion thread [98e][155] NR_reply_LS_Part_2. The discussion thread was moderated by Yang Shan (China Telecomunication Corp.).

Decision: 		The document was revised to R4-2103341.

R4-2103341	Email discussion summary for [98e][155] NR_reply_LS_Part_2

Type: other		For: discussion
					Source: Moderator (China Telecom)

(Replaces R4-2103003)

Discussion:

The contribution summarised email discussion thread [98e][155] NR_reply_LS_Part_2. The discussion thread was moderated by Yang Shan (China Telecomunication Corp.).

Decision: 		The document was noted.

R4-2103288	Reply LS on PUCCH and PUSCH repetition

Type: LS out		For: Approval
					to -
					Source: Qualcomm

Discussion:

See email discussion summary for [98e][155] NR_reply_LS_Part_2 in R4-2103341.

Decision: 		The document was revised to R4-2103393.

R4-2103393	Reply LS on PUCCH and PUSCH repetition

Type: LS out		For: Approval
					to RAN1
					Source: RAN4

(Replaces R4-2103288)

Discussion:

See email discussion summary for [98e][155] NR_reply_LS_Part_2 in R4-2103341.

Decision: 		The document was approved.

R4-2103289	Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition

Type: other		For: discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][155] NR_reply_LS_Part_2 in R4-2103341.

Decision: 		The document was revised to R4-2103363.

R4-2103363	Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition

Type: other		For: discussion
					Source: Huawei, HiSilicon

(Replaces R4-2103289)

Discussion:

See email discussion summary for [98e][155] NR_reply_LS_Part_2 in R4-2103341.

Decision: 		The document was approved.

R4-2100159	Discussion on phase continuity for PUSCH and PUCCH repetitions for LS reply

Type: other		For: Approval
					Source: InterDigital Communications

Abstract:

Discussion on phase continuity for PUSCH and PUCCH repetitions for LS reply. We propose answers for each question of the LS received from RAN1.

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was noted.

R4-2100889	Discussion on LS on PUCCH and PUSCH repetition

Type: other		For: Approval
					Source: Qualcomm Incorporated

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was noted.

R4-2102347	Reply LS  on PUCCH and PUSCH repetition

Type: other		For: Approval
					Source: Ericsson

Abstract:

RAN1 LS (R1-2009784) is discussed and LS reply is proposed

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was noted.

R4-2102630	on phase continuity for PUCCH and PUSCH repetition and reply LS

Type: other		For: Approval
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was noted.

R4-2100547	Discussion on LS on questions to RAN WGs on dual Radio UE (2Rx/2Tx or 2Rx/1Tx) support for simultaneous communication with both SNPN and PLMN

Type: other		For: Approval
					Source: Sony

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was noted.

R4-2103004	Email discussion summary for [98e][156] NR_reply_LS_Part_3

Type: other		For: discussion
					Source: Moderator (Samsung)

Discussion:

The contribution summarised email discussion thread [98e][156] NR_reply_LS_Part_3. The discussion thread was moderated by He Wang (Samsung).

Decision: 		The document was revised to R4-2103342.

R4-2103342	Email discussion summary for [98e][156] NR_reply_LS_Part_3

Type: other		For: discussion
					Source: Moderator (Samsung)

(Replaces R4-2103004)

Discussion:

The contribution summarised email discussion thread [98e][156] NR_reply_LS_Part_3. The discussion thread was moderated by He Wang (Samsung).

Decision: 		The document was noted.

R4-2103290	Reply LS on Beam switching gaps for Multi-TRP UL transmission

Type: LS out		For: Approval
					to RAN1
					Source: RAN4

Discussion:

See email discussion summary for [98e][156] NR_reply_LS_Part_3 in R4-2103342.

Decision: 		The document was approved.

R4-2101017	Discussion and Draft Reply LS to RAN1 on beam switching gap

Type: discussion		For: Discussion
					Source: Apple

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was noted.

R4-2101157	Discussion reply LS on Beam switching gaps for Multi-TRP UL transmission

Type: discussion		For: Discussion
					Source: MediaTek inc.

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was noted.

R4-2102389	Discussion on reply LS on Beam switching gaps for Multi-TRP UL transmission

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was noted.

R4-2102390	draft reply LS on Beam switching gaps for Multi-TRP UL transmission

Type: LS out		For: Approval
					to RAN1
					Source: Huawei, HiSilicon

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was noted.

R4-2102710	Discussion and Reply LS on Beam switching gaps for Multi-TRP UL transmission

Type: discussion		For: Discussion
					Source: vivo

Discussion:

See email discussion summary for [98e][239] R17_LS_RS_for_Scell_activation in R4-2103720.

Decision: 		The document was noted.

### 15.2	Others

## 16	Revision of the Work Plan

### 16.1	Simplification of band combinations in RAN4 specifications

R4-2103001	Email discussion summary for [98e][153] BC_simplification

Type: other		For: discussion
					Source: Moderator (NTT DOCOMO)

Discussion:

The contribution summarised email discussion thread [98e][153] BC_simplification. The discussion thread was moderated by Yuta Oguma (NTT DOCOMO INC.).

Decision: 		The document was revised to R4-2103339.

R4-2103339	Email discussion summary for [98e][153] BC_simplification

Type: other		For: discussion
					Source: Moderator (NTT DOCOMO)

(Replaces R4-2103001)

Discussion:

The contribution summarised email discussion thread [98e][153] BC_simplification. The discussion thread was moderated by Yuta Oguma (NTT DOCOMO INC.).

Decision: 		The document was noted.

R4-2103285	Way forward on Handling of agreements about band combinations

Type: other		For: discussion
					Source: ZTE

Discussion:

See email discussion summary for [98e][153] BC_simplification in R4-2103339.

Decision: 		The document was approved.

R4-2103286	Way forward on Simplification on

Type: other		For: discussion
					Source: Nokia, Nokia Shanghai Bell

Discussion:

See email discussion summary for [98e][153] BC_simplification in R4-2103339.

Decision: 		The document was approved.

R4-2100089	Handling of agreements about band combinations

Type: other		For: Approval
					Source: Nokia, Nokia Shanghai Bell

Abstract:

Continuation of discussion on RP-202863

Discussion:

See email discussion summary for [98e][153] BC_simplification in R4-2103339.

Decision: 		The document was noted.

R4-2100120	On optimization for band combination in RAN4 specifications

Type: discussion		For: Approval
					Source: ZTE Corporation

Abstract:

This paper if for further discussion on optimization for band combination in RAN4 specifications

Discussion:

See email discussion summary for [98e][153] BC_simplification in R4-2103339.

Decision: 		The document was noted.

R4-2100121	CR to TS 38.101-1 on optimization on delta TIB and RIB for band combinations (Rel-16)

Type: CR		For: Agreement
					38.101-1 v16.6.0	  CR-0590  Cat: F (Rel-16)

					Source: ZTE Corporation

Abstract:

This CR is for optimization on delta TIB and RIB for band combinations in 38.101-1

Discussion:

See email discussion summary for [98e][153] BC_simplification in R4-2103339.

Decision: 		The document was not pursued.

R4-2100122	CR to TS 38.101-1 on optimization on delta TIB and RIB for band combinations (Rel-17)

Type: CR		For: Agreement
					38.101-1 v17.0.0	  CR-0591  Cat: A (Rel-17)

					Source: ZTE Corporation

Abstract:

This CR is for optimization on delta TIB and RIB for band combinations in 38.101-1

Discussion:

See email discussion summary for [98e][153] BC_simplification in R4-2103339.

Decision: 		The document was withdrawn.

R4-2100123	CR to TS 38.101-2 on optimization on delta RIB for inter-band CA (Rel-16)

Type: CR		For: Agreement
					38.101-2 v16.6.0	  CR-0316  Cat: F (Rel-16)

					Source: ZTE Corporation

Abstract:

This CR is for optimization on delta RIB for inter-band CA in 38.101-2

Discussion:

See email discussion summary for [98e][153] BC_simplification in R4-2103339.

Decision: 		The document was not pursued.

R4-2100124	CR to TS 38.101-2 on optimization on delta RIB for inter-band CA (Rel-17)

Type: CR		For: Agreement
					38.101-2 v17.0.0	  CR-0317  Cat: A (Rel-17)

					Source: ZTE Corporation

Abstract:

This CR is for optimization on delta RIB for inter-band CA in 38.101-2

Discussion:

See email discussion summary for [98e][153] BC_simplification in R4-2103339.

Decision: 		The document was withdrawn.

R4-2100125	CR to TS 38.101-3 on optimization on delta TIB and RIB for inter-band EN-DC (Rel-16)

Type: CR		For: Agreement
					38.101-3 v16.6.0	  CR-0433  Cat: F (Rel-16)

					Source: ZTE Corporation

Abstract:

This CR is for optimization on delta TIB and RIB for inter-band EN-DC in 38.101-3

Discussion:

See email discussion summary for [98e][153] BC_simplification in R4-2103339.

Decision: 		The document was not pursued.

R4-2100126	CR to TS 38.101-3 on optimization on delta TIB and RIB for inter-band EN-DC (Rel-17)

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0434  Cat: A (Rel-17)

					Source: ZTE Corporation

Abstract:

This CR is for optimization on delta TIB and RIB for inter-band EN-DC in 38.101-3

Discussion:

See email discussion summary for [98e][153] BC_simplification in R4-2103339.

Decision: 		The document was withdrawn.

R4-2101818	LS on change of methodology for new LTE-CA REL-17 combinations

Type: LS out		For: Approval
					to RAN5
					Source: RAN4

Discussion:

See email discussion summary for [98e][153] BC_simplification in R4-2103339.

Decision: 		The document was approved.

Attachments to this outgoing LS: R4-2016940

### 16.2	R17 new proposals

#### 16.2.1	Spectrum related

R4-2100101	Motivation for new WI on air-to-ground network for NR

Type: WID new		For: Information
					Source: CMCC

Decision: 		The document was not treated.

R4-2100102	New WID on air-to-ground network for NR

Type: WID new		For: Information
					Source: CMCC

Decision: 		The document was not treated.

R4-2100887	New WID on Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n259

Type: discussion		For: Information
					Source: SoftBank Corp., Rakuten Mobile

Decision: 		The document was not treated.

R4-2100888	Motivation on new WID on Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n259

Type: discussion		For: Information
					Source: SoftBank Corp., Rakuten Mobile

Decision: 		The document was not treated.

R4-2100940	Motivation on new WI of intra-band non-contiguous NR-DC using band n77

Type: discussion		For: Information
					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2101819	New WID on Support of full bandwidth combinations for inter-band EN-DC combinations

Type: WID new		For: Information
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2102940	Motivation paper for Rail Mobile Radio (RMR)

Type: discussion		For: discussion
					Source: UIC

Decision: 		The document was not treated.

R4-2102941	WID for 900MHz spectrum block

Type: WID new		For: discussion
					Source: UIC

Decision: 		The document was not treated.

R4-2102942	WID for 1900MHz spectrum block

Type: WID new		For: discussion
					Source: UIC

Decision: 		The document was not treated.

#### 16.2.2	Non-spectrum related

R4-2100103	Motivation for new WI on PC1.5 for NR band n79

Type: WID new		For: Information
					Source: CMCC

Decision: 		The document was not treated.

R4-2100104	WID on High power UE (power class 1.5) for NR band n79

Type: WID new		For: Information
					Source: CMCC

Decision: 		The document was not treated.

R4-2100105	Motivation for new WI on PC2 for NR band n34

Type: WID new		For: Information
					Source: CMCC

Decision: 		The document was not treated.

R4-2100106	WID on High power UE (power class 2) for NR band n34

Type: WID new		For: Information
					Source: CMCC

Decision: 		The document was not treated.

R4-2100107	Motivation for new WI on PC2 for NR band n39

Type: WID new		For: Information
					Source: CMCC

Decision: 		The document was not treated.

R4-2100108	WID on High power UE (power class 2) for NR band n39

Type: WID new		For: Information
					Source: CMCC

Decision: 		The document was not treated.

R4-2100195	Proposal to extend R17 FeRRM WI scope

Type: discussion		For: Information
					38.133 v..
					Source: Apple

Decision: 		The document was not treated.

R4-2100362	Discussion on EMC Test Simplification for Rel-17 EMC enhancement

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion paper on EMC test simplification for Rel 17 EMC enhancement

Decision: 		The document was not treated.

R4-2100363	New WID proposal on RAN4 Rel-17 EMC enhancement

Type: WID new		For: Information
					Source: Ericsson, Xiaomi

Abstract:

Proposal on a WID for Rel-17 EMC enhancement

Decision: 		The document was not treated.

R4-2100941	Motivation for supporting non-colocated scenarios for band 42 and n77/n78

Type: discussion		For: Information
					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2101115	Discussion on NR demodulation performance in the scenario of LTE-NR coexist

Type: discussion		For: Discussion
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2101728	Revised WID for NR RF Enhancement for FR2

Type: WID revised		For: Information
					Source: Ericsson, Sony

Abstract:

Revised WID to include further work on beam correspondence

Decision: 		The document was not treated.

R4-2102583	Motivation for WI: Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)

Type: discussion		For: Information
					Source: vivo

Decision: 		The document was not treated.

R4-2102584	New WID: Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)

Type: WID new		For: Information
					Source: vivo, OPPO, CMCC, CAICT, Rohde&Schwarz

Decision: 		The document was not treated.

### 16.3	Others

## 17	Any other business

R4-2102236	Inclusive language review in TS 36.133

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7035  Cat: F (Rel-17)

					Source: Ericsson

Abstract:

Inclusive language review of TS 36.133 according to decision in RP-202179. The document was subject for post-meeting email agreement.

Decision: 		The document was revised to R4-2103254.

R4-2103254	Inclusive language review in TS 36.133

Type: CR		For: Agreement
					36.133 v17.0.0	  CR-7035 rev 1 Cat: F (Rel-17)

					Source: Ericsson

Abstract:

Inclusive language review of TS 36.133 according to decision in RP-202179. The document was subject for post-meeting email agreement.

Decision: 		The document was agreed.

## 18	Close of the E-meeting

R4-2100030	Incoming LSs

Type: other		For: Information
					Source: ETSI MCC

Decision: 		The document was not treated.

R4-2100031	Incoming LSs

Type: other		For: Information
					Source: ETSI MCC

Decision: 		The document was not treated.

R4-2100032	Incoming LSs

Type: other		For: Information
					Source: ETSI MCC

Decision: 		The document was not treated.

R4-2100033	Incoming LSs

Type: other		For: Information
					Source: ETSI MCC

Decision: 		The document was not treated.

R4-2100034	Incoming LSs

Type: other		For: Information
					Source: ETSI MCC

Decision: 		The document was not treated.

R4-2100035	Incoming LSs

Type: other		For: Information
					Source: ETSI MCC

Decision: 		The document was not treated.

R4-2100036	Incoming LSs

Type: other		For: Information
					Source: ETSI MCC

Decision: 		The document was not treated.

R4-2100037	Incoming LSs

Type: other		For: Information
					Source: ETSI MCC

Decision: 		The document was not treated.

R4-2100038	Incoming LSs

Type: other		For: Information
					Source: ETSI MCC

Decision: 		The document was not treated.

R4-2100039	Incoming LSs

Type: other		For: Information
					Source: ETSI MCC

Decision: 		The document was not treated.

R4-2100040	Incoming LSs

Type: other		For: Information
					Source: ETSI MCC

Decision: 		The document was not treated.

R4-2104041	Void

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0496  Cat: - (Rel-17)

					Source: Void

Decision: 		The document was withdrawn.

R4-2104040	(reserved)

Type: other		For: Agreement
					Source: ETSI MCC

Decision: 		The document was not treated.

R4-2103025	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103252	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103254	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103255	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103407	CR on introduction of completed NR CA/DC combs with 4DL/2UL including FR2

Type: CR		For: Agreement
					38.101-3 v17.0.0	  CR-0454 rev 1 Cat: B (Rel-17)
					Source: Samsung

Decision: 		The document was agreed.

R4-2103408	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103410	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103411	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103412	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103413	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103414	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103415	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103416	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103417	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103418	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103419	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103420	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103421	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103422	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103423	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103424	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103425	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103426	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103427	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103428	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103429	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103430	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103431	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103432	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103433	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103434	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103435	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103436	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103437	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103438	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103439	(reserved)

Type: other		For: discussion
					Source: Main session

Decision: 		The document was not treated.

R4-2103984	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2103992	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2103996	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2103997	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104000	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104001	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104002	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104003	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104004	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104005	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104006	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104007	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104008	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104009	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104010	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104011	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104012	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104013	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104014	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104015	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104016	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104017	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104018	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104019	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104020	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104021	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104022	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104023	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104024	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104025	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104026	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104027	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104028	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104029	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104030	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104031	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104032	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104033	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104034	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104035	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104036	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104037	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104038	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

R4-2104039	(reserved)

Type: other		For: discussion
					Source: BSRF session

Decision: 		The document was not treated.

Report prepared by: Kai-Erik Sunell

## Annex A: Contribution documents and status

### A1: List of TDocs

| Document | Title | Source | Decision | Replaces | Replaced by |
| --- | --- | --- | --- | --- | --- |
| R4-2100001 | Agenda for RAN4 #98-e | RAN4 Chairman (Apple) | approved |  |  |
| R4-2100002 | RAN4#98-e E-meeting Arrangements and Guidelines | RAN4 Chairman (Apple) | approved |  |  |
| R4-2100003 | RAN4#97-e Meeting Report | ETSI MCC | approved |  |  |
| R4-2100004 | Test methods for over-the-air TRP field measurements of unwanted emissions from IMT radio equipment utilizing active antennas | ITU Radiocommunication Study Groups Working Party 1C | noted |  |  |
| R4-2100005 | LS on updated Rel-16 RAN1 UE features lists for LTE | RAN1 | noted |  |  |
| R4-2100006 | Reply LS on definition of NR V2X con-current operation | RAN1 | noted |  |  |
| R4-2100007 | LS reply on cell-grouping UE capability for synchronous NR-DC | RAN1 | noted |  |  |
| R4-2100008 | Reply LS on DCI-based multiple BWP switch simultaneously | RAN1 | noted |  |  |
| R4-2100009 | Reply LS on UE capability xDD differentiation for SUL/SDL bands | RAN1 | noted |  |  |
| R4-2100010 | LS on updated Rel-16 RAN1 UE features lists for NR | RAN1 | noted |  |  |
| R4-2100011 | Reply LS on UE capability for V2X | RAN1 | noted |  |  |
| R4-2100012 | LS on uplink Tx switching | RAN1 | noted |  |  |
| R4-2100013 | Reply LS on Clarification of UE behavior after receiving the MAC CE deactivation command for semi-persistent CSI reporting in NR-U | RAN1 | noted |  |  |
| R4-2100014 | LS on PUCCH and PUSCH repetition | RAN1 | noted |  |  |
| R4-2100015 | LS on temporary RS for efficient SCell activation in NR CA | RAN1 | noted |  |  |
| R4-2100016 | LS on Beam switching gaps for Multi-TRP UL transmission | RAN1 | noted |  |  |
| R4-2100017 | Reply LS on power control for NR-DC | RAN2 | noted |  |  |
| R4-2100018 | LS on support of NUL and SUL during DAPS handover | RAN1 | noted |  |  |
| R4-2100019 | Reply LS on IAB-MT feature list | RAN2 | noted |  |  |
| R4-2100020 | LS on ambiguity in deciding TL,C | RAN5 | noted |  |  |
| R4-2100021 | LS on Frequency Bands for testing of A-GNSS Sensitivity requirements in NR and LTE | RAN5 | noted |  |  |
| R4-2100022 | Use of the 252-296 GHz frequency range by land-mobile service applications | ITU-R WP 5A | noted |  |  |
| R4-2100023 | LS on single UL operation | RAN | noted |  |  |
| R4-2100024 | Reply LS to APT_LS200918 = RP-202143 on Frequency Arrangements for IMT in the Band 470- 703 MHz | RAN | noted |  |  |
| R4-2100025 | LS on BCS reporting and support for intra-band EN-DC band combinations | RAN | noted |  |  |
| R4-2100026 | LS on Use of Inclusive Language in 3GPP | SA | noted |  |  |
| R4-2100027 | LS on OTA LTE UE TRP and TRS Requirements | MSG TFES | noted |  |  |
| R4-2100028 | Reply LS on questions to RAN WGs on dual Radio UE (2Rx/2Tx or 2Rx/1Tx) support for simultaneous communication with both SNPN and PLMN | RAN2 | noted |  |  |
| R4-2100029 | Spurious emission limits for AAS BS in 6.425 – 7.125 GHz and 10-10.5 GHz | CEPT ECC PT1 | not treated |  |  |
| R4-2100030 | Incoming LSs | ETSI MCC | not treated |  |  |
| R4-2100031 | Incoming LSs | ETSI MCC | not treated |  |  |
| R4-2100032 | Incoming LSs | ETSI MCC | not treated |  |  |
| R4-2100033 | Incoming LSs | ETSI MCC | not treated |  |  |
| R4-2100034 | Incoming LSs | ETSI MCC | not treated |  |  |
| R4-2100035 | Incoming LSs | ETSI MCC | not treated |  |  |
| R4-2100036 | Incoming LSs | ETSI MCC | not treated |  |  |
| R4-2100037 | Incoming LSs | ETSI MCC | not treated |  |  |
| R4-2100038 | Incoming LSs | ETSI MCC | not treated |  |  |
| R4-2100039 | Incoming LSs | ETSI MCC | not treated |  |  |
| R4-2100040 | Incoming LSs | ETSI MCC | not treated |  |  |
| R4-2100041 | On requirements involving gap patterns for IAB-MTs | ZTE Corporation | noted |  |  |
| R4-2100042 | [CR] IAB Core Maintenance | ZTE Corporation | revised |  | R4-2103539 |
| R4-2100043 | On RLM and RLF relaxation for UE power saving | ZTE Corporation | noted |  |  |
| R4-2100044 | Test cases for applicable timing for PL RS activated by MAC-CE | ZTE Corporation | noted |  |  |
| R4-2100045 | [CR] Test cases for applicable timing for PL RS activated by MAC-CE | ZTE Corporation | withdrawn |  |  |
| R4-2100046 | [draft CR] Test cases for timing for IAB-MT | ZTE Corporation | revised |  | R4-2103542 |
| R4-2100047 | Scope and work split of test cases for IAB-MTs | ZTE Corporation | noted |  |  |
| R4-2100048 | Beam configuration for gNB measurement accuracy | ZTE Corporation | noted |  |  |
| R4-2100049 | UE Rx-Tx measurements | ZTE Corporation | noted |  |  |
| R4-2100050 | Random access in NR-U | ZTE Corporation | noted |  |  |
| R4-2100051 | [CR] Add Random Access requirements under NR-U | ZTE Corporation | merged |  |  |
| R4-2100052 | CGI reading with autonomous gaps | ZTE Corporation | noted |  |  |
| R4-2100053 | CR for missing B48 references in a table and note | C Spire Wireless | agreed |  | - |
| R4-2100054 | [draft CR] Test cases for applicable timing for PL RS activated by MAC-CE | ZTE Corporation | revised |  | R4-2103597 |
| R4-2100055 | Study on extended 600MHz NR band | Spark NZ Ltd | revised |  | R4-2103270 |
| R4-2100056 | Frequency band arrangements and duplexer options for extended 600MHz NR band | Spark NZ Ltd | noted |  |  |
| R4-2100057 | Blank TR for extended 600MHz NR band | Spark NZ Ltd | noted |  |  |
| R4-2100058 | [CR] RRM test case maintenance R15 Cat F | ZTE Corporation | revised |  | R4-2103485 |
| R4-2100059 | [CR] RRM test case maintenance R16 Cat A | ZTE Corporation | agreed |  |  |
| R4-2100060 | [CR] RRM test case maintenance R17 Cat A | ZTE Corporation | agreed |  |  |
| R4-2100061 | [draft CR] Test cases for PRS-RSRP measurement accuracy | ZTE Corporation | revised |  | R4-2103733 |
| R4-2100062 | Discussions on UE transmit timing and reference cell under NR-U | ZTE Corporation | noted |  |  |
| R4-2100063 | [CR] UE behavior when cannot transmit ACK due to LBT failure for MAC-CE deactivation | ZTE Corporation | withdrawn |  |  |
| R4-2100064 | [CR] UE behavior when cannot transmit ACK due to LBT failure for MAC-CE deactivation (Cat A) | ZTE Corporation | withdrawn |  |  |
| R4-2100065 | Remaining issues in SCell activation under NR-U | ZTE Corporation | noted |  |  |
| R4-2100066 | [CR] UE behavior when cannot transmit ACK due to LBT failure for MAC-CE deactivation | ZTE Corporation | merged |  |  |
| R4-2100067 | FR2 PDSCH Reference channel and OCNG for RRM Test cases | ANRITSU LTD | noted |  |  |
| R4-2100068 | Update FR2 Reference channels and OCNG for FR2 RRM Test cases | ANRITSU LTD | revised |  | R4-2103486 |
| R4-2100069 | Update FR2 Reference channels and OCNG for FR2 RRM Test cases | ANRITSU LTD | agreed |  |  |
| R4-2100070 | Update FR2 Reference channels and OCNG for FR2 RRM Test cases | ANRITSU LTD | agreed |  |  |
| R4-2100071 | FR2 Reference channels for RRM Test cases with 240kHz SSB SCS | ANRITSU LTD | noted |  |  |
| R4-2100072 | CR to FR1 SA SS-SINR measurement TCs | ANRITSU LTD | agreed |  |  |
| R4-2100073 | CR to FR1 SA SS-SINR measurement TCs | ANRITSU LTD | agreed |  |  |
| R4-2100074 | CR to FR1 SA SS-SINR measurement TCs | ANRITSU LTD | agreed |  |  |
| R4-2100075 | CR on E-UTRA carrier for EN-DC event triggered reporting tests | ANRITSU LTD | agreed |  |  |
| R4-2100076 | CR on E-UTRA carrier for EN-DC event triggered reporting tests | ANRITSU LTD | agreed |  |  |
| R4-2100077 | CR on E-UTRA carrier for EN-DC event triggered reporting tests | ANRITSU LTD | agreed |  |  |
| R4-2100078 | Add missing FR2 Test case setups and Beam assumptions | ANRITSU LTD | agreed |  |  |
| R4-2100079 | Add missing FR2 Test case setups and Beam assumptions | ANRITSU LTD | agreed |  |  |
| R4-2100080 | Add missing FR2 Test case setups and Beam assumptions | ANRITSU LTD | agreed |  |  |
| R4-2100081 | Work plan for study on high power UE (power class 2) for one NR FDD band | China Unicom | revised |  | R4-2103272 |
| R4-2100082 | Big CR on introduction of completed PC2 for EN-DC with 1 LTE band + 1 NR TDD band | China Unicom | agreed |  |  |
| R4-2100083 | Revised WID on High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band | China Unicom | not concluded |  |  |
| R4-2100084 | TR 37.826 v0.2.0 ENDC_UE_PC2_R17_NR_TDD | China Unicom | not concluded |  |  |
| R4-2100085 | Clarification on NS_203 support by n258 | Nokia, Nokia Shanghai Bell | revised |  | R4-2103125 |
| R4-2100086 | Removal of a remaining NS_201 related requirement | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2100087 | Removal of a remaining NS_201 related requirement | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2100088 | Required changes to the original BCS4 idea | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100089 | Handling of agreements about band combinations | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100090 | Blank TR for extended 600MHz NR band | Spark NZ Ltd | noted |  |  |
| R4-2100091 | Draft CR on CA_n1-n3, CA_n1-n78, CA_n3-n78 | China Unicom | not pursued |  | R4-2103034 |
| R4-2100092 | Draft CR on CA_n1-n258, CA_n3-n258, CA_n78-n258 | China Unicom | not pursued |  | R4-2103048 |
| R4-2100093 | Draft CR on CA_n1-n3-n78 | China Unicom | not pursued |  | R4-2103076 |
| R4-2100094 | Multi-band relaxation for band n262 | Murata Manufacturing Co., Ltd. | noted |  |  |
| R4-2100095 | Remaining items on transparent Tx diversity | Anritsu corporation | noted |  |  |
| R4-2100096 | Impact of offset antenna to quiet zone in FR2 OTA chamber | Anritsu corporation | noted |  |  |
| R4-2100097 | TP to TR 38.884 on Inter-band DL CA in FR2 | Anritsu corporation | revised |  | R4-2103968 |
| R4-2100098 | DUT repositioning during ETC measurement in FR2 | Anritsu corporation | noted |  |  |
| R4-2100099 | Introduce NR SUL bands to PC3 UL-MIMO configuration | CMCC | not pursued |  |  |
| R4-2100100 | Discussion on SAR issues for PC2 NR inter-band CA and SUL configurations | CMCC | noted |  |  |
| R4-2100101 | Motivation for new WI on air-to-ground network for NR | CMCC | not treated |  |  |
| R4-2100102 | New WID on air-to-ground network for NR | CMCC | not treated |  |  |
| R4-2100103 | Motivation for new WI on PC1.5 for NR band n79 | CMCC | not treated |  |  |
| R4-2100104 | WID on High power UE (power class 1.5) for NR band n79 | CMCC | not treated |  |  |
| R4-2100105 | Motivation for new WI on PC2 for NR band n34 | CMCC | not treated |  |  |
| R4-2100106 | WID on High power UE (power class 2) for NR band n34 | CMCC | not treated |  |  |
| R4-2100107 | Motivation for new WI on PC2 for NR band n39 | CMCC | not treated |  |  |
| R4-2100108 | WID on High power UE (power class 2) for NR band n39 | CMCC | not treated |  |  |
| R4-2100109 | On NS_203/CA_NS_203 for n258 | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100110 | Discussion on SAR schemes for PC2 FDD band | CMCC | noted |  |  |
| R4-2100111 | NTN architecture aspects | THALES | noted |  |  |
| R4-2100112 | PC1 and PC3 Updates for Band n14 | AT&T | revised |  | R4-2103400 |
| R4-2100113 | Discussion on independent and concurrent MGs | ZTE Corporation | noted |  |  |
| R4-2100114 | Discussion on handover with PSCell | ZTE Corporation | noted |  |  |
| R4-2100115 | [CR] Applicability rule for 2-step RA | ZTE Corporation | merged |  |  |
| R4-2100116 | [CR] Applicability rule for 2-step RA (Cat A) | ZTE Corporation | withdrawn |  |  |
| R4-2100117 | [CR] Core maintenance for 38.133 | ZTE Corporation | agreed |  |  |
| R4-2100118 | [CR] Core maintenance for 38.133 (Cat A) | ZTE Corporation | agreed |  |  |
| R4-2100119 | PC1 and PC3 Updates for Band n14 | AT&T | agreed |  |  |
| R4-2100120 | On optimization for band combination in RAN4 specifications | ZTE Corporation | noted |  |  |
| R4-2100121 | CR to TS 38.101-1 on optimization on delta TIB and RIB for band combinations (Rel-16) | ZTE Corporation | not pursued |  |  |
| R4-2100122 | CR to TS 38.101-1 on optimization on delta TIB and RIB for band combinations (Rel-17) | ZTE Corporation | withdrawn |  |  |
| R4-2100123 | CR to TS 38.101-2 on optimization on delta RIB for inter-band CA (Rel-16) | ZTE Corporation | not pursued |  |  |
| R4-2100124 | CR to TS 38.101-2 on optimization on delta RIB for inter-band CA (Rel-17) | ZTE Corporation | withdrawn |  |  |
| R4-2100125 | CR to TS 38.101-3 on optimization on delta TIB and RIB for inter-band EN-DC (Rel-16) | ZTE Corporation | not pursued |  |  |
| R4-2100126 | CR to TS 38.101-3 on optimization on delta TIB and RIB for inter-band EN-DC (Rel-17) | ZTE Corporation | withdrawn |  |  |
| R4-2100127 | CR to TS 38.101-2 on correction to intra-band non-contiguous CA configurations (Rel-16) | ZTE Corporation | agreed |  |  |
| R4-2100128 | CR to TS 38.101-2 on correction to intra-band non-contiguous CA configurations (Rel-17) | ZTE Corporation | agreed |  |  |
| R4-2100129 | CR to TS 38.101-3 on correction to hanging paragraph in the spec (Rel-16) | ZTE Corporation | revised |  | R4-2103161 |
| R4-2100130 | CR to TS 38.101-3 on correction to hanging paragraph in the spec (Rel-17) | ZTE Corporation | agreed |  |  |
| R4-2100131 | HPUE for bands 5, 12 and n71 UE RF aspects. | Nokia | approved |  |  |
| R4-2100132 | n48 30 MHz A-MPR simulation results | Nokia | noted |  |  |
| R4-2100133 | CR introduction of 30 MHz for n48 | Nokia, Dish Network, Skyworks Inc | not pursued |  |  |
| R4-2100134 | Simulation results for n24 / band 24 A-MPR | Nokia | noted |  |  |
| R4-2100135 | n24 / band 24 A-MPR proposal | Nokia, Skyworks Inc | revised |  | R4-2103209 |
| R4-2100136 | 38.101 Void clean up R16 | Nokia, Nokia Shanghai Bell | revised |  | R4-2103343 |
| R4-2100137 | 38.101 Void clean up R17 | Nokia, Nokia Shanghai Bell | revised |  | R4-2103344 |
| R4-2100138 | Reply LS on ambiguity in deciding TL,C | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100139 | CR REL15 on ambiguity in deciding TL,C | Nokia, Nokia Shanghai Bell | not pursued |  |  |
| R4-2100140 | CR REL16 on ambiguity in deciding TL,C | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R4-2100141 | CR REL17 on ambiguity in deciding TL,C | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R4-2100142 | IBM RF requirements for CA configurations within same frequency group | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100143 | Specification differences between IBM and CBM | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100144 | FR2 UL gaps for self-calibration and monitoring | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100145 | TP to TR 37.717-21-11: DC_20-40_n78 | Nokia, Telefonica | revised |  | R4-2103006 |
| R4-2100146 | TP to TR 37.717-31-11: DC_1-20-40_n78 | Nokia, Telefonica | revised |  | R4-2103007 |
| R4-2100147 | TP to TR 37.717-41-11: DC_1-3-20-40_n78 | Nokia, Telefonica | revised |  | R4-2103008 |
| R4-2100148 | TS 38.101-3: Addition of missing lower order fallbacks R16 | Nokia, AT&T | revised |  | R4-2103162 |
| R4-2100149 | TS 38.101-3: Addition of missing lower order fallbacks R17 | Nokia, AT&T | withdrawn |  | - |
| R4-2100150 | TR 37.716-21-11: Addition of missing lower order fallbacks | Nokia, AT&T | agreed |  |  |
| R4-2100151 | TP to TR 36.717-02-01: Addition of CA_48-53 | Nokia, Globalstar | approved |  |  |
| R4-2100152 | Draft CR addition of BCS2 for CA_71B | Nokia, T-Mobile USA | endorsed |  |  |
| R4-2100153 | Draft CR Addition of BCS1 for CA_n25A-n71(2A), CA_n41(2A)-n66A and CA_n66A-n71(2A) | Nokia, T-Mobile USA | not pursued |  | R4-2103083 |
| R4-2100154 | Draft CR addition of CA_n77-n258 configurations | Nokia, T-Mobile USA | endorsed |  |  |
| R4-2100155 | Draft CR addition of CA_n41-n71 configurations | Nokia, T-Mobile USA | endorsed |  |  |
| R4-2100156 | Draft CR addition of CA_n25A-n41(2A)-n71A | Nokia, T-Mobile USA | withdrawn |  |  |
| R4-2100157 | TP to TR 38.717-03-02: CA_n25-n41-n66 | Nokia, T-Mobile USA | noted |  |  |
| R4-2100158 | TP to TR 38.717-03-02: CA_n25-n41-n71 | Nokia, T-Mobile USA | revised |  | R4-2103084 |
| R4-2100159 | Discussion on phase continuity for PUSCH and PUCCH repetitions for LS reply | InterDigital Communications | noted |  |  |
| R4-2100160 | Introduction of specific Pcmax requirements for inter-band CA category A-B combos | InterDigital Communications | not pursued |  | - |
| R4-2100161 | Test time reduction in FR2 using beam sweeping | Fraunhofer HHI, Fraunhofer IIS | noted |  |  |
| R4-2100162 | CA_n7B_REFSENS_CatF_CR | Qualcomm Incorporated | agreed |  |  |
| R4-2100163 | CR_CatF_n47_AMPR_correction | Qualcomm Incorporated | not pursued |  |  |
| R4-2100164 | IBE_mask_almost_contiguous_CR_rel15 | Qualcomm Incorporated | not pursued |  |  |
| R4-2100165 | IBE_mask_almost_contiguous_CR_rel16-mirror | Qualcomm Incorporated | withdrawn |  |  |
| R4-2100166 | Co-existence challenges with NR-U 100MHz channel bandwidth and other technologies | Charter Communications, Inc | noted |  |  |
| R4-2100167 | Blank TR for extended 600MHz NR band | Spark NZ Ltd | noted |  |  |
| R4-2100168 | CR on FDD HST Single-Tap and Multipath Fading Requirements | Qualcomm Incorporated | revised |  | R4-2103978 |
| R4-2100169 | Views on URLLC Ultra-low BLER CSI Reporting Test Cases | Qualcomm Incorporated | noted |  |  |
| R4-2100170 | Views on URLLC High BLER Demodulation Test Cases | Qualcomm Incorporated | noted |  |  |
| R4-2100171 | Correction CR on URLLC Higher BLER Performance Requirements | Qualcomm Incorporated | merged |  |  |
| R4-2100172 | Further discussion on CSSF for R15 EN-DC | Apple | noted |  |  |
| R4-2100173 | CR on CSSF for EN-DC R15 | Apple | revised |  | R4-2103729 |
| R4-2100174 | On SSB-less SCell activation | Apple | noted |  |  |
| R4-2100175 | CR on FR2 SCell activation requirement R15 | Apple | postponed |  |  |
| R4-2100176 | CR on FR2 SCell activation requirement R16 | Apple | withdrawn |  |  |
| R4-2100177 | CR on FR2 SCell activation requirement R17 | Apple | withdrawn |  |  |
| R4-2100178 | CR on smtc1 and smtc2 differentiation in intra-frequency measurement with MG R15 | Apple | postponed |  |  |
| R4-2100179 | CR on smtc1 and smtc2 differentiation in intra-frequency measurement with MG R16 | Apple | withdrawn |  |  |
| R4-2100180 | CR on smtc1 and smtc2 differentiation in intra-frequency measurement with MG R17 | Apple | withdrawn |  |  |
| R4-2100181 | CR on maintenance for inter-band FR2 CA RRM R16 | Apple | agreed |  |  |
| R4-2100182 | CR on maintenance for inter-band FR2 CA RRM R17 | Apple | agreed |  |  |
| R4-2100183 | CR on UE behavior for UE specific CBW change R16 | Apple | revised |  | R4-2103622 |
| R4-2100184 | CR on UE behavior for UE specific CBW change R17 | Apple | agreed |  |  |
| R4-2100185 | On R16 IDLE and INACTIVE RRM requirement with SMTC2-LP | Apple | noted |  |  |
| R4-2100186 | CR on IDLE/INACTIVE RRM requirement with SMTC2-LP R16 | Apple | revised |  | R4-2103506 |
| R4-2100187 | CR on IDLE/INACTIVE RRM requirement with SMTC2-LP R17 | Apple | agreed |  |  |
| R4-2100188 | On reference cell availability for NR-U | Apple | noted |  |  |
| R4-2100189 | CR on reference cell availability for NR-U R16 | Apple | postponed |  |  |
| R4-2100190 | CR on reference cell availability for NR-U R17 | Apple | withdrawn |  |  |
| R4-2100191 | On RSSI measurement in NR-U | Apple | noted |  |  |
| R4-2100192 | On SRS antenna port switching | Apple | noted |  |  |
| R4-2100193 | On RRM requirement for handover with PSCell | Apple | noted |  |  |
| R4-2100194 | On PUCCH SCell activation and deactivation | Apple | noted |  |  |
| R4-2100195 | Proposal to extend R17 FeRRM WI scope | Apple | not treated |  |  |
| R4-2100196 | Analysis of RF interferences to A-GNSS Sensitivity requirements in NR and LTE | Apple | noted |  |  |
| R4-2100197 | Discussion on demodulation requirements for NR-U | Apple | noted |  |  |
| R4-2100198 | CQI Reporting requirements for URLLC | Apple | noted |  |  |
| R4-2100199 | CR to 38-101-4 on CQI reporting requirements for URLLC | Apple | revised |  | R4-2103891 |
| R4-2100200 | UE demodulation requirements with higher BLER | Apple | noted |  |  |
| R4-2100201 | CR to 38.101-4 on requirements with slot aggreagation in FR2 | Apple | revised |  | R4-2103900 |
| R4-2100202 | RRM Core requirements maintenance for eMIMO | Apple | noted |  |  |
| R4-2100203 | CR to 38.133 on RRM requirements for multi-TRP (R16) | Apple | postponed |  |  |
| R4-2100204 | CR to 38.133 on RRM requirements for multi-TRP (R17) | Apple | withdrawn |  |  |
| R4-2100205 | CR to 38.133 on Link Recovery requirements (R16) | Apple | revised |  | R4-2104055 |
| R4-2100206 | CR to 38.133 on Link Recovery requirements (R17) | Apple | agreed |  |  |
| R4-2100207 | CR to 38.133 on Pathloss activation delay requirements (R16) | Apple | revised |  | R4-2103589 |
| R4-2100208 | CR to 38.133 on Pathloss activation delay requirements (R17) | Apple | agreed |  |  |
| R4-2100209 | RRM Performance requirements for L1-SINR Measurement Accuracy | Apple | noted |  |  |
| R4-2100210 | CR to 38.101-4 for eMIMO demod requirements - General and Applicability rule | Apple | revised |  | R4-2103826 |
| R4-2100211 | Simulation results for multi-DCI based transmission scheme | Apple | noted |  |  |
| R4-2100212 | PMI reporting requirements with eType II codebook | Apple | revised |  | R4-2103961 |
| R4-2100213 | Remaining issues for UL spatial relation switching requirements | Apple | noted |  |  |
| R4-2100214 | CR to 38.133 on UL spatial relation switch requirements (R16) | Apple | not pursued |  |  |
| R4-2100215 | CR to 38.133 on UL spatial relation switch requirements (R17) | Apple | withdrawn |  |  |
| R4-2100216 | PMI reporting requirements with larger number of TX ports | Apple | noted |  |  |
| R4-2100217 | UL gaps for tranceiver calibration | Apple | noted |  |  |
| R4-2100218 | UL gaps for Tx power management | Apple | noted |  |  |
| R4-2100219 | UE measurements relaxation for RLM and/or BFD | Apple | noted |  |  |
| R4-2100220 | Discussion on RRM requirement for high speed train scenario in FR2 | Apple | noted |  |  |
| R4-2100221 | Consideration on preconfigured measurement gap patterns | Apple | noted |  |  |
| R4-2100222 | Discussion on multiple concurrent and independent MG patterns | Apple | noted |  |  |
| R4-2100223 | On network controlled small gap | Apple | noted |  |  |
| R4-2100224 | On R17 FR1 HST RRM measurement requirement | Apple | noted |  |  |
| R4-2100225 | Test applicability for mandatory gap patterns | Apple | noted |  |  |
| R4-2100226 | TC4: E-UTRAN | Apple | revised |  | R4-2103614 |
| R4-2100227 | Activation time in direct SCell activation | Apple | noted |  |  |
| R4-2100228 | CR on activation time in direct SCell activation | Apple | revised |  | R4-2103553 |
| R4-2100229 | CR on activation time in direct SCell activation (R17) | Apple | withdrawn |  |  |
| R4-2100230 | Test case for Direct SCell Activation: EN-DC, NR spCell in FR1, SCell in FR1, SCell addition | Apple | revised |  | R4-2103557 |
| R4-2100231 | Test case for SCell Dormancy: EN-DC, NR spCell in FR1, SCell FR1, DCI 2_6 within/after 3 OFDM symbols | Apple | revised |  | R4-2103564 |
| R4-2100232 | Test case for EMR with both PCell and target cell in FR1 | Apple | revised |  | R4-2103550 |
| R4-2100233 | Interruption requirements due to measurement on SCC in NR-DC (R15) | Apple | postponed |  |  |
| R4-2100234 | Interruption requirements maintenance in NR-DC (R16) | Apple | revised |  | R4-2103507 |
| R4-2100235 | Interruption requirements maintenance in NR-DC (R17) | Apple | withdrawn |  |  |
| R4-2100236 | Discussion on RAN1 LS on temporary RS for efficient SCell activation in NR CA | Apple | noted |  |  |
| R4-2100237 | R16 HST maintenance: measurement requirements for active SCell in HST | Apple | noted |  |  |
| R4-2100238 | CR on HST core part maintenance | Apple | revised |  | R4-2103655 |
| R4-2100239 | CR on HST core part maintenance (R17) | Apple | withdrawn |  |  |
| R4-2100240 | On the feasibility of CBM for FR2 inter-band CA cross different frequency groups | Apple | noted |  |  |
| R4-2100241 | On MRTD for CBM for FR2 interband CA | Apple | noted |  |  |
| R4-2100242 | CR on CSSF with CSI-RS for L3 measurement | Apple | withdrawn |  |  |
| R4-2100243 | On remaining core issues of CSI-RS for L3 measurements | Apple | noted |  |  |
| R4-2100244 | CR for missing B48 references in a table and note | C Spire Wireless | withdrawn | - |  |
| R4-2100245 | Test time reduction in OTA measurement | Anritsu Corporation | noted |  |  |
| R4-2100246 | CR for TS 38.101-1 introduction of NR band n24 | Ligado Networks, Nokia, Skyworks Solutions Inc. | revised |  | R4-2103210 |
| R4-2100247 | CR for TS 38.133 introduction of NR band n24 | Ligado Networks | agreed |  |  |
| R4-2100248 | CR for TS 37.105 introduction of NR band n24 | Ligado Networks | agreed |  |  |
| R4-2100249 | CR for TS 37.145-1 introduction of NR band n24 | Ligado Networks | agreed |  |  |
| R4-2100250 | CR for TS 37.145-2 introduction of NR band n24 | Ligado Networks | agreed |  |  |
| R4-2100251 | CR for TS 38.141-1 introduction of NR band n24 | Ligado Networks | agreed |  |  |
| R4-2100252 | CR for TS 38.141-2 introduction of NR band n24 | Ligado Networks | agreed |  |  |
| R4-2100253 | On general aspects of IAB-MT test cases | ZTE Corporation | noted |  |  |
| R4-2100254 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks, Nokia, Skyworks Solutions Inc. | revised |  | R4-2103379 |
| R4-2100255 | CR for 37.105: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed |  |  |
| R4-2100256 | CR for 37.145-1: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed |  |  |
| R4-2100257 | CR for 37.145-2: Corrections related to Band 24 regulatory updates | Ligado Networks | revised |  | R4-2103215 |
| R4-2100258 | Requirement of band 46 reference sensitivity measurement for NR EN-DC band combinations | Verizon Denmark | revised |  | R4-2103009 |
| R4-2100259 | draftCR for Rel-17 NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL within FR1 | Verizon Denmark | revised |  | R4-2103035 |
| R4-2100260 | draftCR for Rel-17Dual Connectivity of 1LTE band (1DL/1UL) and 1NR band (1DL/1UL) within FR1 | Verizon Denmark | endorsed |  |  |
| R4-2100261 | draftCR for Rel-16 Dual Connectivity of 1LTE band (1DL/1UL) and 1NR band (1DL/1UL) with FR1 | Verizon Denmark | withdrawn |  |  |
| R4-2100262 | draftCR for Rel-17 Dual Connectivity of 1LTE band (1DL/1UL) and 1NR band (1DL/1UL) including FR2 | Verizon Denmark | endorsed |  |  |
| R4-2100263 | draftCR for Rel-17 NR inter-band CA DC combination for 2 bands DL with up to 2 bands UL between FR1 and FR2 | Verizon Denmark | endorsed |  |  |
| R4-2100264 | Release 17 FR2 bandwidth class | Verizon Denmark | noted |  |  |
| R4-2100265 | TR 36.717-03-02 v0.2.0 TR Update for LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17 | LG Electronics France | not concluded |  |  |
| R4-2100266 | TP for TR 37.826 for DC_2_n77 | Verizon Denmark | revised |  | R4-2103175 |
| R4-2100267 | Revised WID on LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17 | LG Electronics France | not concluded |  |  |
| R4-2100268 | TP for TR 37.826 for DC_5_n77 | Verizon Denmark | revised |  | R4-2103176 |
| R4-2100269 | TP for TR 37.826 for DC_13_n77 | Verizon Denmark | revised |  | R4-2103177 |
| R4-2100270 | Introduction of LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL to TS36.101 | LG Electronics France | agreed |  |  |
| R4-2100271 | TP for TR 37.826 for DC_66_n77 | Verizon Denmark | revised |  | R4-2103178 |
| R4-2100272 | TR 37.717-11-21 v0.3.0 TR update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 | LG Electronics France | not concluded |  |  |
| R4-2100273 | TP for TR38.xxx for PC2 CA_n2A-n77A | Verizon Denmark | revised |  | R4-2103169 |
| R4-2100274 | TP for TR38.xxx for PC2 CA_n5A-n77A | Verizon Denmark | revised |  | R4-2103170 |
| R4-2100275 | Revised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) DC in Rel-17 | LG Electronics France | not concluded |  |  |
| R4-2100276 | TP for TR38.xxx for PC2 CA_n66A-n77A | Verizon Denmark | revised |  | R4-2103171 |
| R4-2100277 | Introduction CR on new NR DC LTE(xDL/1UL)+ NR(2DL/1UL) band combinations in Rel-17 | LG Electronics France | agreed |  |  |
| R4-2100278 | TP on summary of self-interference analysis for new NR DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 | LG Electronics France | approved |  |  |
| R4-2100279 | MSD anlaysis results for new DC band combinations | LG Electronics France | revised |  | R4-2103050 |
| R4-2100280 | CR on editorial correction on V2X operation in TS38.101-1 in Rel-16 | LG Electronics France | revised |  | R4-2103140 |
| R4-2100281 | CR on editorial correction on V2X operation in TS38.101-1 in Rel-17 | LG Electronics France | agreed |  |  |
| R4-2100282 | TR38.xxx v0.0.1 TR Skeleton for SL enhancement in Rel-17 | LG Electronics France | revised |  | R4-2103243 |
| R4-2100283 | Consideration on partial usage operation with PC5 and Uu in a licensed band | LG Electronics France | noted |  |  |
| R4-2100284 | PC2 MPR/A-MPR simulation assumptions for NR V2X UE in n47 | LG Electronics France | noted |  |  |
| R4-2100285 | Self interference analysis and MSD results for PC2 NR inter-band CA band combinations | LG Electronics France | noted |  |  |
| R4-2100286 | MSD for PC2 high power NR DC (with 1 LTE FDD band + 1 NR TDD band) UE | LG Electronics France | noted |  |  |
| R4-2100287 | Consideration for RF architecture for n77/n78 PC1.5 UE | LG Electronics France | noted |  |  |
| R4-2100288 | MPR/A-MPR initial simulation results according to candidate RF architectures | LG Electronics France | noted |  |  |
| R4-2100289 | MPR/A-MPR initial simulation assumptions for PC2 NR intra-band NC CA | LG Electronics France, LG Uplus | noted |  |  |
| R4-2100290 | Consideration of current RF components feasibility for FDD PC2 UE | LG Electronics France | noted |  |  |
| R4-2100291 | Revised WID on Band combinations for SA NR Supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | Huawei, HiSilicon | not concluded |  |  |
| R4-2100292 | TR 37.717-00-00 v0.3.0 | Huawei, HiSilicon | not concluded |  |  |
| R4-2100293 | CR on Introduction of completed SUL band combinations into TS 38.101-1 | Huawei, HiSilicon | agreed |  |  |
| R4-2100294 | CR on Introduction of completed SUL band combinations into TS 38.101-3 | Huawei, HiSilicon | not concluded |  |  |
| R4-2100295 | Revised WID on NR inter-band CA for 5 bands DL with x bands UL (x=1, 2) | Huawei, HiSilicon | not concluded |  |  |
| R4-2100296 | TR 38.717-05-01 v0.2.0 | Huawei, HiSilicon | withdrawn |  |  |
| R4-2100297 | CR on Introduction of completed 5 bands inter-band CA into TS 38.101-1 | Huawei, HiSilicon | withdrawn |  |  |
| R4-2100298 | Discussion of MSD for 3DL2UL DC_21_n28-n79 due to UL IMD issues | Mediatek India Technology Pvt. | noted |  |  |
| R4-2100299 | Discussion of MSD for 3DL2UL DC_1-21_n28 due to UL IMD3 issue | Mediatek India Technology Pvt. | noted |  |  |
| R4-2100300 | TP for TR 37.717-11-21: to update DC_18_n3-n41 with IMD3 MSD | Mediatek India Technology Pvt. | revised |  | R4-2103051 |
| R4-2100301 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +1LTE band) within FR1 DC_40A_n78(2A)/DC_40C_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100302 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +1LTE band) within FR1 DC_8A_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100303 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +2LTE band) within FR1 DC_1A-40A_n78(2A)/DC_1A-40C_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100304 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +2LTE band) within FR1 DC_1A-8A_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100305 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +2LTE band) within FR1 DC_3A-40A_n78(2A)/DC_3A-40C_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100306 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +2LTE band) within FR1 DC_3A-8A_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100307 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +2LTE band) within FR1 DC_7A-40A_n78(2A)/DC_7A-40C_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100308 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +2LTE band) within FR1 DC_7A-8A_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100309 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +2LTE band) within FR1 DC_8A-40A_n78(2A)/DC_8A-40C_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100310 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_1A-3A-40A_n78(2A)/DC_1A-3A-40C_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100311 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_1A-3A-8A_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100312 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_1A-7A-40A_n78(2A)/DC_1A-7A-40C_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100313 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_1A-7A-8A_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100314 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_1A-8A-40A_n78(2A)/DC_1A-8A-40C_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100315 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_3A-7A-40A_n78(2A)/DC_3A-7A-40C_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100316 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_3A-7A-8A_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100317 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_3A-8A-40A_n78(2A)/DC_3A-8A-40C_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100318 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +3LTE band) within FR1 DC_7A-8A-40A_n78(2A)/DC_7A-8A-40C_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100319 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +4LTE band) within FR1 DC_1A-3A-7A-40A_n78(2A)/DC_1A-3A-7A-40C_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100320 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +4LTE band) within FR1 DC_1A-3A-7A-8A_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100321 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +4LTE band) within FR1 DC_1A-3A-8A-40A_n78(2A)/DC_1A-3A-8A-40C_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100322 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +4LTE band) within FR1 DC_1A-7A-8A-40A_n78(2A)/DC_1A-7A-8A-40C_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100323 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +4LTE band) within FR1 DC_3A-7A-8A-40A_n78(2A)/DC_3A-7A-8A-40C_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100324 | Draft CR for 38.101-3 to introduce new inter-band EN-DC (1NR band +5LTE band) within FR1 DC_1A-3A-7A-8A-40A_n78(2A)/DC_1A-3A-7A-8A-40C_n78(2A) | Huawei,HiSilicon | endorsed |  |  |
| R4-2100325 | TP for TR 37.717-11-21:DC_3A-40A_n1A-n78A/DC_3A-40C_n1A-n78A | Huawei,HiSilicon | revised |  | R4-2103052 |
| R4-2100326 | TP for TR 37.717-11-21:DC_7A-40A_n1A-n78A/DC_7A-40C_n1A-n78A | Huawei,HiSilicon | revised |  | R4-2103053 |
| R4-2100327 | TP for TR 37.717-11-21:DC_8A-40A_n1A-n78A/DC_8A-40C_n1A-n78A | Huawei,HiSilicon | revised |  | R4-2103054 |
| R4-2100328 | TP for TR 37.717-11-21:DC_3A-7A-40A_n1A-n78A/DC_3A-7A-40C_n1A-n78A | Huawei,HiSilicon | revised |  | R4-2103055 |
| R4-2100329 | TP for TR 37.717-11-21:DC_3A-8A-40A_n1A-n78A/DC_3A-8A-40C_n1A-n78A | Huawei,HiSilicon | revised |  | R4-2103056 |
| R4-2100330 | TP for TR 37.717-11-21:DC_7A-8A-40A_n1A-n78A/DC_7A-8A-40C_n1A-n78A | Huawei,HiSilicon | revised |  | R4-2103057 |
| R4-2100331 | TP for TR 37.717-11-21:DC_3A-7A-8A-40A_n1A-n78A/DC_3A-7A-8A-40C_n1A-n78A | Huawei,HiSilicon | revised |  | R4-2103058 |
| R4-2100332 | Discussion on new SUL band n99 UE requirements | Huawei,HiSilicon | noted |  |  |
| R4-2100333 | Discussion on new SUL band n99 BS requirements | Huawei,HiSilicon | noted |  |  |
| R4-2100334 | CR to 38101-1 on introducing new SUL band n99 | Huawei,HiSilicon | revised |  | R4-2103211 |
| R4-2100335 | CR to 38104 on introducing new SUL band n99 | Huawei,HiSilicon | revised |  | R4-2103212 |
| R4-2100336 | CR to 36104 on introducing new SUL band n99 | Huawei,HiSilicon | agreed |  |  |
| R4-2100337 | CR to 38141-1 on introducing new SUL band n99 | Huawei,HiSilicon | agreed |  |  |
| R4-2100338 | CR to 38141-2 on introducing new SUL band n99 | Huawei,HiSilicon | agreed |  |  |
| R4-2100339 | CR to 36141 on introducing new SUL band n99 | Huawei,HiSilicon | agreed |  |  |
| R4-2100340 | CR to 37104 on introducing new SUL band n99 | Huawei,HiSilicon | agreed |  |  |
| R4-2100341 | CR to 37141 on introducing new SUL band n99 | Huawei,HiSilicon | agreed |  |  |
| R4-2100342 | CR to 37105 on introducing new SUL band n99 | Huawei,HiSilicon | agreed |  |  |
| R4-2100343 | CR to 37145-1 on introducing new SUL band n99 | Huawei,HiSilicon | revised |  | R4-2103213 |
| R4-2100344 | CR to 37145-2 on introducing new SUL band n99 | Huawei,HiSilicon | revised |  | R4-2103214 |
| R4-2100345 | TP for TR 37.717-21-11: DC_3-18_n41 | KDDI Corporation | revised |  | R4-2103010 |
| R4-2100346 | TP for TR 37.717-11-21: DC_1-3-18_n3-n41 | KDDI Corporation | revised |  | R4-2103059 |
| R4-2100347 | TP for TR 37.717-11-21: DC_1-3-18_n28-n41 | KDDI Corporation | revised |  | R4-2103060 |
| R4-2100348 | TP for TR 37.717-11-21: DC_1-3-18_n41-n77 | KDDI Corporation | revised |  | R4-2103061 |
| R4-2100349 | TP for TR 37.717-11-21: DC_1-3-18_n41-n78 | KDDI Corporation | revised |  | R4-2103062 |
| R4-2100350 | Draft CR for 38.101-3 to add n78-n257 inter-band CA configurations | SK Telecom | endorsed |  |  |
| R4-2100351 | Draft CR for 38.101-3 to add n78C in DC_n78-n257 | SK Telecom | endorsed |  |  |
| R4-2100352 | TP for DC_21_n28 for TR 37.717-11-11 | NTT DOCOMO INC. | withdrawn |  |  |
| R4-2100353 | gNB positioning requirements | Ericsson | noted |  |  |
| R4-2100354 | CR to TS 38.113 on Performance criteria for transient phenomena, Release 15 | Ericsson | not pursued |  |  |
| R4-2100355 | CR to TS 38.113 on Performance criteria for transient phenomena, Release 16 | Ericsson | withdrawn |  |  |
| R4-2100356 | Definition of Exclusion Bands for IAB EMC nodes | Ericsson | noted |  |  |
| R4-2100357 | CR to TS 38.175 on Exclusion Bands | Ericsson | not pursued |  |  |
| R4-2100358 | Discussion on Spatial Exclusion for IAB EMC RI test | Ericsson | noted |  |  |
| R4-2100359 | CR to TS 38.175 on Spatial Exclusion for IAB EMC Radiated Immunity test | Ericsson | not pursued |  |  |
| R4-2100360 | CR to TS 38.175 on IAB EMC test configuratins and performance requirements | Ericsson | revised |  | R4-2103774 |
| R4-2100361 | Discussion on EMC requirements for NR Repeater | Ericsson | noted |  |  |
| R4-2100362 | Discussion on EMC Test Simplification for Rel-17 EMC enhancement | Ericsson | not treated |  |  |
| R4-2100363 | New WID proposal on RAN4 Rel-17 EMC enhancement | Ericsson, Xiaomi | not treated |  |  |
| R4-2100364 | Discussion on CBW and FR name for above 52.6 GHz | CATT | noted |  |  |
| R4-2100365 | Discussion on IAB-MT EVM measurement process | CATT | noted |  |  |
| R4-2100366 | Draft CR for TS 38.174: IAB-MT EVM measurement | CATT | not pursued |  |  |
| R4-2100367 | Draft CR for TR 38.809: IAB-MT EVM measurement | CATT | not pursued |  |  |
| R4-2100368 | Draft CR for TS 38.174: Correction of clause 5 | CATT | revised |  | R4-2103957 |
| R4-2100369 | Draft CR for TS 38.174: Correction of clause 6,7 and 9 | CATT | revised |  | R4-2103850 |
| R4-2100370 | Discussion on IAB-MT specfic Tx requirements test | CATT | noted |  |  |
| R4-2100371 | Discussion on IAB-MT test model | CATT | noted |  |  |
| R4-2100372 | Discussion on inter-band 2UL CA Pcmax upper limit | CATT | noted |  |  |
| R4-2100373 | TR skeleton for Downlink interruption for band combinations to conduct dynamic Tx Switching | CATT | agreed |  |  |
| R4-2100374 | TR 37.xxx 0.1.0 for Downlink interruption for band combinations to conduct dynamic Tx Switching | CATT | not concluded |  |  |
| R4-2100375 | Discussion on NR repeater general issues | CATT | noted |  |  |
| R4-2100376 | Discussion on NR repeater conducted transmitted power | CATT | noted |  |  |
| R4-2100377 | Discussion on NR repeater emission requirements for FR1/FR2 | CATT | noted |  |  |
| R4-2100378 | Discussion on NR repeater EVM and frequency stability for FR1/FR2 | CATT | noted |  |  |
| R4-2100379 | Discussion on NR repeater radiated transmitted power | CATT | noted |  |  |
| R4-2100380 | Summary of ideal and impairment results for NR HST demodulation requirements | CATT | revised |  | R4-2103972 |
| R4-2100381 | CR for TS 38.141-2: Introduction of NR PUSCH UL TA performance requirement | CATT | not pursued |  | R4-2103860 |
| R4-2100382 | CR for TS 38.141-2: Introduction of NR PUSCH UL TA performance requirement(Rel-17) | CATT | withdrawn |  |  |
| R4-2100383 | Discussion on the BS requirements for 52.6-71GHz | CATT | noted |  |  |
| R4-2100384 | Discussion on co-existence for 52.6-71GHz | CATT | noted |  |  |
| R4-2100385 | Further discussion on out-of-band CLTA definition | CATT | noted |  |  |
| R4-2100386 | CR for TS 38.141-2: Correction on definition for the out-of-band CLTA(Rel-15) | CATT | revised |  | R4-2103794 |
| R4-2100387 | CR for TS 38.141-2: Correction on definition for the out-of-band CLTA(Rel-16) | CATT, Nokia, Nokia Shanghai Bell, Huawei | agreed |  |  |
| R4-2100388 | CR for TS 38.141-2: Correction on definition for the out-of-band CLTA(Rel-17) | CATT, Nokia, Nokia Shanghai Bell, Huawei | agreed |  |  |
| R4-2100389 | CR for TS 37.145-2: Correction on definition for the out-of-band CLTA(Rel-15) | CATT | not pursued |  |  |
| R4-2100390 | CR for TS 37.145-2: Correction on definition for the out-of-band CLTA(Rel-16) | CATT | withdrawn |  |  |
| R4-2100391 | CR for TS 37.145-2: Correction on definition for the out-of-band CLTA(Rel-17) | CATT | withdrawn |  |  |
| R4-2100392 | CR for TS38 101-1 Rel-15 Correction for definition of P-MPR | CATT | agreed |  |  |
| R4-2100393 | CR for TS38 101-1 Rel-16 Correction for definition of P-MPR | CATT | agreed |  |  |
| R4-2100394 | CR for TS38 101-1 Rel-17 Correction for definition of P-MPR | CATT | agreed |  |  |
| R4-2100395 | CR for TS38 101-1 Rel-16 Correction of condition for MPR and delta MPR | CATT | revised |  | R4-2103122 |
| R4-2100396 | CR for TS38 101-1 Rel-17 Correction of condition for MPR and delta MPR | CATT | agreed |  |  |
| R4-2100397 | Discussion on power saving demodulation test | CATT | noted |  |  |
| R4-2100398 | CR for TS38.101-4, test for FR2 PDCCH DCI format 2_6 demodulation | CATT | revised |  | R4-2103822 |
| R4-2100399 | Discussion on frequency band and scenarios for NTN | CATT | noted |  |  |
| R4-2100400 | Discussion on SRS antenna port switching | CATT | noted |  |  |
| R4-2100401 | Discussion on HO with PSCell | CATT | noted |  |  |
| R4-2100402 | Discussion on PUCCH SCell activationdeactivation | CATT | noted |  |  |
| R4-2100403 | Discussion on switching period position in ITS band | CATT | noted |  |  |
| R4-2100404 | CR for TS 38.101-1, General corrections for NR V2X | CATT | not pursued |  |  |
| R4-2100405 | CR for TS 38.101-3, General corrections for NR V2X | CATT | revised |  | R4-2103141 |
| R4-2100406 | CR for 38.886, General corrections for NR V2X | CATT | agreed |  |  |
| R4-2100407 | Discussion on single link demodulation test for NR V2X | CATT | noted |  |  |
| R4-2100408 | Discussion on multiple link demodulation test for NR V2X | CATT | noted |  |  |
| R4-2100409 | Simulation results of NR V2X single link demodulation test | CATT, GOHIGH | noted |  |  |
| R4-2100410 | Simulation results of NR V2X multiple link demodulation test | CATT, GOHIGH | noted |  |  |
| R4-2100411 | DraftCR for 38.101-4, Introduce PSBCH performance requirements for NR V2X | CATT, GOHIGH | revised |  | R4-2103814 |
| R4-2100412 | TP on V2X_n41A-n47A coexistence study | CATT | revised |  | R4-2103199 |
| R4-2100413 | CR for TS 38.101-1, Introduce new band combination of V2X_n41A-n47A | CATT | revised |  | R4-2103200 |
| R4-2100414 | CR for TS 38.101-3, Introduce new band combination of V2X_41A-n47A and V2X_n41A-47A | CATT | revised |  | R4-2103201 |
| R4-2100415 | Discussion on operating scenarios for partial used SL operation | CATT | noted |  |  |
| R4-2100416 | Discussion on synchronous operation between NR Uu and NR SL | CATT | noted |  |  |
| R4-2100417 | Discussion on system parameters for newly introduced SL bands | CATT | noted |  |  |
| R4-2100418 | Discussion on UE Tx RF requirement for NR SL enhancement | CATT | noted |  |  |
| R4-2100419 | Discussion on UE Rx RF requirement for NR SL enhancement | CATT | noted |  |  |
| R4-2100420 | Discussion on PC2 for SL enhancement | CATT | noted |  |  |
| R4-2100421 | Discussion on core part maintenance open issues | CATT | noted |  |  |
| R4-2100422 | CR on CSI-RS based L3 measurement | CATT | revised |  | R4-2103631 |
| R4-2100423 | Updated simulation results for CSI-RSRP measurement | CATT | noted |  |  |
| R4-2100424 | Updated simulation results for CSI-RSRQ measurement | CATT | noted |  |  |
| R4-2100425 | Simulation results for CSI-SINR measurement | CATT | noted |  |  |
| R4-2100426 | Discussion on performance requirement for CSI-RSRP | CATT | noted |  |  |
| R4-2100427 | Discussion on performance requirement for CSI-RSRQ | CATT | noted |  |  |
| R4-2100428 | Discussion on performance requirement for CSI-SINR | CATT | noted |  |  |
| R4-2100429 | CR on performance requirement for CSI-RSRP | CATT | postponed |  |  |
| R4-2100430 | CR on performance requirement for CSI-RSRQ | CATT | revised |  | R4-2103636 |
| R4-2100431 | CR on performance requirement for CSI-SINR | CATT | postponed |  |  |
| R4-2100432 | Discussion on test case for CSI-RS based measurement | CATT | noted |  |  |
| R4-2100433 | CR on test case for CSI-RS based intra-frequency measurement for FR1 in SA | CATT | revised |  | R4-2103639 |
| R4-2100434 | CR on test case for CSI-RS based inter-frequency measurement for FR1 in SA | CATT | revised |  | R4-2103642 |
| R4-2100435 | CR on test case for CSI-RSRP measurement accuracy requirements in SA | CATT | revised |  | R4-2103645 |
| R4-2100436 | Discussion on PRS RSTD measurement requirements | CATT | noted |  |  |
| R4-2100437 | Discussion on UE Rx-Tx time difference measurement requirements | CATT | noted |  |  |
| R4-2100438 | CR on PRS RSTD measurement requirements | CATT | revised |  | R4-2103578 |
| R4-2100439 | CR on UE Rx-Tx time difference measurement requirements | CATT | merged |  |  |
| R4-2100440 | Discussion on PRS RSTD accuracy requirements | CATT | noted |  |  |
| R4-2100441 | Discussion on PRS RSRP accuracy requirements | CATT | noted |  |  |
| R4-2100442 | Discussion on UE Rx-Tx time difference accuracy requirements | CATT | noted |  |  |
| R4-2100443 | Link-level simulation results of PRS RSTD measurement | CATT | noted |  |  |
| R4-2100444 | Link-level simulation results of PRS RSRP measurement | CATT | noted |  |  |
| R4-2100445 | Link-level simulation results for UE Rx-Tx time difference measurement | CATT | noted |  |  |
| R4-2100446 | CR on PRS-RSRP accuracy requirements | CATT | revised |  | R4-2103732 |
| R4-2100447 | CR on PRS configuration for test cases | CATT | merged |  |  |
| R4-2100448 | CR on test case for PRS-RSRP measurement requirements for FR2 in SA | CATT | revised |  | R4-2103734 |
| R4-2100449 | Discussion on test cases for PRS based measurement | CATT | noted |  |  |
| R4-2100450 | Discussion on SRS-RSRP measurement requirements | CATT | noted |  |  |
| R4-2100451 | Discussion on gNB Rx-Tx time difference measurement requirements | CATT | noted |  |  |
| R4-2100452 | Discussion on UL RTOA measurement requirements | CATT | noted |  |  |
| R4-2100453 | Link-level simulation results for UL timing measurement | CATT | noted |  |  |
| R4-2100454 | Initial discussion on pre-configured MG pattern | CATT | noted |  |  |
| R4-2100455 | Initial discussion on multiple concurrent and independent MG patterns | CATT | noted |  |  |
| R4-2100456 | Initial discussion on Network Controlled Small Gap (NCSG) | CATT | noted |  |  |
| R4-2100457 | CR on NCSG in 36.133 | CATT | merged |  |  |
| R4-2100458 | CR on NCSG in 36.133 | CATT | withdrawn |  |  |
| R4-2100459 | CR on NCSG in 36.133 | CATT | withdrawn |  |  |
| R4-2100460 | CR on NCSG in 38.133 | CATT | postponed |  |  |
| R4-2100461 | CR on CSI-RS based L3 measurement | CATT | agreed |  |  |
| R4-2100462 | CR on performance requirement for CSI-RSRP | CATT | withdrawn |  |  |
| R4-2100463 | CR on performance requirement for CSI-RSRQ | CATT | withdrawn |  |  |
| R4-2100464 | CR on performance requirement for CSI-SINR | CATT | withdrawn |  |  |
| R4-2100465 | CR on test case for CSI-RS based intra-frequency measurement for FR1 in SA | CATT | withdrawn |  |  |
| R4-2100466 | CR on test case for CSI-RS based inter-frequency measurement for FR1 in SA | CATT | withdrawn |  |  |
| R4-2100467 | CR on test case for CSI-RSRP measurement accuracy requirements in SA | CATT | withdrawn |  |  |
| R4-2100468 | CR on PRS RSTD measurement requirements | CATT | agreed |  |  |
| R4-2100469 | CR on UE Rx-Tx time difference measurement requirements | CATT | withdrawn |  |  |
| R4-2100470 | CR on PRS-RSRP accuracy requirements | CATT | withdrawn |  |  |
| R4-2100471 | CR on PRS configuration for test cases | CATT | withdrawn |  |  |
| R4-2100472 | CR on test case for PRS-RSRP measurement requirements for FR2 in SA | CATT | withdrawn |  |  |
| R4-2100473 | Discussion on remaining issues for UE power saving test case | CATT | noted |  |  |
| R4-2100474 | Discussion on RLM/BFD relaxation factor | CATT | noted |  |  |
| R4-2100475 | Discussion on CA for NR FR1 HST | CATT | noted |  |  |
| R4-2100476 | Discussion on NR FR2 HST | CATT | noted |  |  |
| R4-2100477 | Initial performance evaluation for cell identification in NR FR2 HST scenario | CATT | noted |  |  |
| R4-2100478 | Initial performance evaluation for for RLM/BFD relaxation factor | CATT | noted |  |  |
| R4-2100479 | Correction to cell reselection test case | CATT | agreed |  |  |
| R4-2100480 | Correction to cell reselection test case | CATT | agreed |  |  |
| R4-2100481 | Correction to cell reselection test case | CATT | agreed |  |  |
| R4-2100482 | Correction to cell reselection test case for UE Power saving | CATT | revised |  | R4-2103574 |
| R4-2100483 | Correction to cell reselection test case for UE Power saving | CATT | agreed |  |  |
| R4-2100484 | Correction to cell reselection test case for HST | CATT | agreed |  |  |
| R4-2100485 | Correction to cell reselection test case for HST | CATT | withdrawn |  |  |
| R4-2100486 | Simulaiton assumptions for NTN co-existence | CATT | noted |  |  |
| R4-2100487 | Consideration on BS requirement impact for NTN | CATT | noted |  |  |
| R4-2100488 | Fuurther discussion on UE parameters for 6.425-7.125GHz, 7.025-7.125GHz and 10.0-10.5GHz | CATT | noted |  |  |
| R4-2100489 | further discussion on BS parameters for 6.425-7.125GHz, 7.025-7.125GHz and 10.0-10.5GHz | CATT | noted |  |  |
| R4-2100490 | Downlin simulation results for 6425-7125MHz and 10-10.5GHz - indoor scenario | CATT | noted |  |  |
| R4-2100491 | Uuplink simulation results for 6425-7125MHz and 10-10.5GHz indoor scenario- indoor scenario | CATT | noted |  |  |
| R4-2100492 | TR 38.717-03-01 on Rel-17 NR inter-band Carrier Aggregation (CA) for 3 Down Link (DL) / 1 Up Link (UL) | CATT | not concluded |  |  |
| R4-2100493 | Revised WID on Rel-17 NR inter-band CA of 3DL bands and 1UL band | CATT | not concluded |  |  |
| R4-2100494 | CR on Introducing NR inter-band CA for 3DL Bands and 1UL band for 38.101-1 | CATT | agreed |  |  |
| R4-2100495 | CR on Introducing NR inter-band CA for 3DL Bands and 1UL band for 38.101-3 | CATT | agreed |  |  |
| R4-2100496 | Discussion on 2Tx switching | CATT | noted |  |  |
| R4-2100497 | Correction on supported channel bandwidth for n79 | CATT | withdrawn |  |  |
| R4-2100498 | Correction on supported channel bandwidth for n79 | CATT | withdrawn |  |  |
| R4-2100499 | CR for TS 38.101-1, General corrections for NR V2X | CATT | withdrawn |  |  |
| R4-2100500 | CR for TS 38.101-3, General corrections for NR V2X | CATT | agreed |  |  |
| R4-2100501 | Consideration on extended 600MHz NR band | CATT,CBN | noted |  |  |
| R4-2100502 | TR 37.875 on band combinations for con-current operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band | CATT | agreed |  |  |
| R4-2100503 | TP for DC_1_n28-n79 for TR 37.717-11-21 | NTT DOCOMO, INC. MediaTek Inc., LG Electronics | withdrawn |  |  |
| R4-2100504 | TP for DC_3_n28-n79 for TR 37.717-11-21 | NTT DOCOMO, INC. MediaTek Inc., LG Electronics | withdrawn |  |  |
| R4-2100505 | TP for DC_21_n28-n77 for TR 37.717-11-21 | NTT DOCOMO, INC. MediaTek Inc., LG Electronics | withdrawn |  |  |
| R4-2100506 | TP for DC_21_n28-n78 for TR 37.717-11-21 | NTT DOCOMO, INC. MediaTek Inc., LG Electronics | withdrawn |  |  |
| R4-2100507 | TP for DC_21_n28-n79 for TR 37.717-11-21 | NTT DOCOMO, INC. MediaTek Inc., LG Electronics | withdrawn |  |  |
| R4-2100508 | TP for DC_1-21_n28 for TR 37.717-21-11 | NTT DOCOMO, INC. MediaTek Inc., LG Electronics | withdrawn |  |  |
| R4-2100509 | TP for DC_3-21_n28 for TR 37.717-21-11 | NTT DOCOMO, INC. | withdrawn |  |  |
| R4-2100510 | Remaining issues on intra and inter frequency measurements under NR-U | ZTE Corporation | noted |  |  |
| R4-2100511 | NR-U wideband operation and intra-carrier guard bands | Apple Inc. | noted |  |  |
| R4-2100512 | Corrections of NR-U wideband operation intra-carrier guard bands | Apple Inc. | not pursued |  |  |
| R4-2100513 | Corrections of NR-U wideband operation intra-carrier guard bands | Apple Inc. | withdrawn |  |  |
| R4-2100514 | Band plan for lower 6GHz NR unlicensed operation for Europe | Apple Inc. | noted |  |  |
| R4-2100515 | Considerations for PC1.5 with n77 and n78 | Apple Inc. | noted |  |  |
| R4-2100516 | A-MPR Proposal for n1 and 45MHz CBW | Apple Inc. | noted |  |  |
| R4-2100517 | A-MPR Proposal for n2 and 35MHz CBW | Apple Inc. | noted |  |  |
| R4-2100518 | A-MPR Proposal for n25 and 45MHz CBW | Apple Inc. | noted |  |  |
| R4-2100519 | Further considerations on the numerology and channel bandwidth sizes for 60GHz frequency range | Apple Inc. | noted |  |  |
| R4-2100520 | Regulatory update for the 60GHz frequency range | Apple Inc. | noted |  |  |
| R4-2100521 | Regulatory update for the 60GHz frequency range | Apple Inc. | noted |  |  |
| R4-2100522 | Non-standard spectrum allocations for NR bands | Apple Inc. | noted |  |  |
| R4-2100523 | On Tx diversity | Apple Inc. | noted |  |  |
| R4-2100524 | On simultaneous TxRx for NR-DC | Apple Inc. | noted |  |  |
| R4-2100525 | TP to TR38.884 on High DL and Low UL power test cases | Apple Inc. | merged |  |  |
| R4-2100526 | TP to TR38.884 on polarization mismatch | Apple Inc. | merged |  |  |
| R4-2100527 | Further analysis of the impact of AoA offset on inter-band CA PSD difference | Apple Inc. | noted |  |  |
| R4-2100528 | Impact of ET on measurement uncertainty and test tolerance of spherical coverage EIRP and EIS | Apple Inc. | noted |  |  |
| R4-2100529 | Extending the applicability of permitted methods to band n262 | Apple Inc. | withdrawn |  |  |
| R4-2100530 | TP to TR38.884 on structure updates related to band n262 | Apple Inc. | approved |  |  |
| R4-2100531 | Views on 60 GHz testability | Apple Inc. | noted |  |  |
| R4-2100532 | CR for 37.105: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed |  |  |
| R4-2100533 | CR for 37.105: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed |  |  |
| R4-2100534 | CR for 37.145-1: Corrections related to Band 24 regulatory updates | Ligado Networks | withdrawn |  |  |
| R4-2100535 | CR for 37.145-1: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed |  |  |
| R4-2100536 | CR for 37.145-1: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed |  |  |
| R4-2100537 | CR for 37.145-1: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed |  |  |
| R4-2100538 | CR for 37.145-1: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed |  |  |
| R4-2100539 | CR for 37.145-2: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed |  |  |
| R4-2100540 | CR for 37.145-2: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed |  |  |
| R4-2100541 | Views on UE Power Saving Test Cases | Qualcomm Incorporated | noted |  |  |
| R4-2100542 | Extended 600MHz NR Duplexer Feasibility and Band Arrangement | Skyworks Solutions Inc. | noted |  |  |
| R4-2100543 | PC2 FDD RFFE Technology, Performance, Thermal and Power Aspects | Skyworks Solutions Inc. | noted |  |  |
| R4-2100544 | PC2 Class C UL CA UE Architecture and MPR/A-MPR evaluation | Skyworks Solutions Inc. | noted |  |  |
| R4-2100545 | Band 24, n24 and n99 A-MPR | Skyworks Solutions Inc. | approved |  |  |
| R4-2100546 | NRU in 6GHz EU spectrum: Band Definition and Related Emission Requirements | Skyworks Solutions Inc. | noted |  |  |
| R4-2100547 | Discussion on LS on questions to RAN WGs on dual Radio UE (2Rx/2Tx or 2Rx/1Tx) support for simultaneous communication with both SNPN and PLMN | Sony | noted |  |  |
| R4-2100548 | CR for 38.141-1: BS demodulation synchronization in test setup | Nokia, Nokia Shanghai Bell | revised |  | R4-2103800 |
| R4-2100549 | CR for 38.141-1: BS demodulation synchronization in test setup | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2100550 | CR for 38.141-1: BS demodulation synchronization in test setup | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2100551 | CR for 38.141-1: BS demodulation different channel bandwidths applicability rules | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2100552 | CR for 38.141-1: BS demodulation different channel bandwidths applicability rules | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2100553 | CR for 38.141-1: BS demodulation different channel bandwidths applicability rules | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2100554 | CR for 38.141-2: BS demodulation different channel bandwidths applicability rules | Nokia, Nokia Shanghai Bell | revised |  | R4-2103801 |
| R4-2100555 | CR for 38.141-2: BS demodulation different channel bandwidths applicability rules | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2100556 | CR for 38.141-2: BS demodulation different channel bandwidths applicability rules | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2100557 | On BS demodulation different channel bandwidths applicability rules and synchronization in test setup | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100558 | CR for 38.104: HST PUSCH demodulation requirements | Nokia, Nokia Shanghai Bell | revised |  | R4-2103861 |
| R4-2100559 | CR for 38.104: HST PUSCH demodulation requirements | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2100560 | On NR Rel-16 BS demodulation performance requirements with higher BLER and simulation results | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100561 | CR for 38.104: Low latency FR1 BS demodulation requirements | Nokia, Nokia Shanghai Bell | revised |  | R4-2103905 |
| R4-2100562 | CR for 38.104: Low latency FR1 BS demodulation requirements | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2100563 | CR for 38.104: Ultra high reliability BS demodulation requirements | Nokia, Nokia Shanghai Bell | revised |  | R4-2103894 |
| R4-2100564 | CR for 38.104: Ultra high reliability BS demodulation requirements | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2100565 | On 47GHZ OTA link budget in Demodulation requirement testing | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100566 | Views on RF requirement for FWA | Sony, Ericsson | noted |  |  |
| R4-2100567 | Peak EIRP and EIRP spherical coverage for PC3 for n262 | Sony, Ericsson | noted |  |  |
| R4-2100568 | Multiband relaxations for PC3 for n262 | Sony, Ericsson | noted |  |  |
| R4-2100569 | REFSENS and EIS spherical coverage for PC3 for n262 | Sony, Ericsson | noted |  |  |
| R4-2100570 | Views on Applicability of CBM/IBM for different CA configurations | Sony, Ericsson | noted |  |  |
| R4-2100571 | Views on solutions to minimize the impact of polarization basis mismatch | Sony, Ericsson | noted |  |  |
| R4-2100572 | PC2 non-contiguous UL CA UE Architecture and MPR/A-MPR evaluation | Skyworks Solutions Inc. | noted |  |  |
| R4-2100573 | General issues on BS demodulation performance requirements for operation in unlicensed bands | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100574 | PUSCH Demodulation performance requirements for operation in unlicensed bands | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100575 | NR-U PUSCH simulation results | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100576 | PUCCH Demodulation performance requirements for operation in unlicensed bands | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100577 | NR-U PUCCH simulation results | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100578 | PRACH Demodulation performance requirements for operation in unlicensed bands | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100579 | NR-U PRACH simulation results | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100580 | 2-step RACH RRM performance requirements corrections | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2100581 | 2-step RACH RRM performance requirements corrections | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2100582 | Corrections on 2-step RACH demodulation requirements | Nokia, Nokia Shanghai Bell, Ericsson | revised |  | R4-2103869 |
| R4-2100583 | Corrections on 2-step RACH demodulation requirements | Nokia, Nokia Shanghai Bell, Ericsson | agreed |  |  |
| R4-2100584 | 2-step RACH simulation results | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100585 | On PMI reporting requirements for enhanced Type II codebooks | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100586 | P_min correction and P_cmax CA correction to apply from all cells | Qualcomm Incorporated | revised |  | R4-2103127 |
| R4-2100587 | P_min correction and P_cmax CA correction to apply from all cells | Qualcomm Incorporated | withdrawn |  |  |
| R4-2100588 | P_min correction and P_cmax CA correction to apply from all cells | Qualcomm Incorporated | withdrawn |  |  |
| R4-2100589 | P_cmax P_IBE wording refinement and terminology improvement | Qualcomm Incorporated | revised |  | R4-2103358 |
| R4-2100590 | P_cmax P_IBE wording refinement and termonology improvement | Qualcomm Incorporated | agreed |  |  |
| R4-2100591 | Inter-band + intra-band CA FR2 frequency separation class | Qualcomm Incorporated | noted |  |  |
| R4-2100592 | TxD open items for Rel-16 | Qualcomm Incorporated | noted |  |  |
| R4-2100593 | Power class due to TxD in Rel-15 | Qualcomm Incorporated | withdrawn |  |  |
| R4-2100594 | Introduction of Tx diversity in tor 38101-1 Cat-B CR 38.101-1 | Qualcomm Incorporated | withdrawn |  |  |
| R4-2100595 | Introduction of Tx diversity in tor 38101-1 Cat-F CR 38.101-1 | Qualcomm Incorporated | withdrawn |  |  |
| R4-2100596 | UL Switching and coherent UL MIMO Cat-F CR 38.101-1 | Qualcomm Incorporated | not pursued |  |  |
| R4-2100597 | UL Switching and coherent UL MIMO Cat-A CR 38.101-1 | Qualcomm Incorporated | withdrawn |  |  |
| R4-2100598 | Applicability for CBM and IBM for FR2 inter-band CA band combos | Qualcomm Incorporated | noted |  |  |
| R4-2100599 | UL calibration gap continuation | Qualcomm Incorporated | noted |  |  |
| R4-2100600 | Discussion on power boosting for 2Tx switching | Qualcomm Incorporated | withdrawn |  |  |
| R4-2100601 | Update of DRX configuration in FR1 Event-triggered Test cases | ANRITSU LTD | agreed |  |  |
| R4-2100602 | Update of DRX configuration in FR1 Event-triggered Test cases | ANRITSU LTD | agreed |  |  |
| R4-2100603 | Update of DRX configuration in FR1 Event-triggered Test cases | ANRITSU LTD | agreed |  |  |
| R4-2100604 | CR of adding LTE B24 for UE category NB1 in R17 | Ericsson, Ligado Networks | revised |  | R4-2103275 |
| R4-2100605 | CR of adding LTE B24 for UE category NB1 in R17 | Ericsson, Ligado Networks | agreed |  |  |
| R4-2100606 | CR of adding LTE B24 for UE category NB1 in R17 | Ericsson, Ligado Networks | agreed |  |  |
| R4-2100607 | CR of adding LTE B24 for UE category NB1 in R17 | Ericsson, Ligado Networks | agreed |  |  |
| R4-2100608 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | revised |  | R4-2103276 |
| R4-2100609 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | revised |  | R4-2103277 |
| R4-2100610 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | revised |  | R4-2103278 |
| R4-2100611 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | revised |  | R4-2103281 |
| R4-2100612 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | revised |  | R4-2103279 |
| R4-2100613 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | revised |  | R4-2103282 |
| R4-2100614 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | revised |  | R4-2103280 |
| R4-2100615 | CR of adding LTE B24 for UE category NB2 in R17 | Ericsson, Ligado Networks | agreed |  |  |
| R4-2100616 | CR of adding LTE B24 for UE category NB2 in R17 | Ericsson, Ligado Networks | agreed |  |  |
| R4-2100617 | Revision of inter-band V2X con-currency table for V2X_n39A-n47A and V2X_n40A-n47A | Qualcomm Incorporated | agreed |  |  |
| R4-2100618 | Revision of inter-band V2X con-currency table for V2X_39_n47 and V2X_40_n47 | Qualcomm Incorporated | agreed |  |  |
| R4-2100619 | Definition of TRP and EIRP for FR2 ULCA | Qualcomm Incorporated | noted |  |  |
| R4-2100620 | RF specifications for DLCA n260A_n258A and n259A_n257A based on IBM | Qualcomm Incorporated | noted |  |  |
| R4-2100621 | CBM requirements for DLCA band combinations from the same frequency group | Qualcomm Incorporated | noted |  |  |
| R4-2100622 | Views on CSI Reporting test cases for eMIMO | Qualcomm Incorporated | revised |  | R4-2103962 |
| R4-2100623 | CGI reading discussion T321 LS | Qualcomm, Inc. | noted |  |  |
| R4-2100624 | SRS carrier switching discussion | Qualcomm, Inc. | noted |  |  |
| R4-2100625 | Mandatory MG applicability rule discussion | Qualcomm, Inc. | noted |  |  |
| R4-2100626 | Mandatory MG applicability rule CR | Qualcomm, Inc. | revised |  | R4-2103618 |
| R4-2100627 | Inter frequency w/o gap power imbalance discussion | Qualcomm, Inc. | noted |  |  |
| R4-2100628 | V2X single link demod discussion | Qualcomm, Inc. | noted |  |  |
| R4-2100629 | V2X multiple link demod discussion | Qualcomm, Inc. | noted |  |  |
| R4-2100630 | V2X demod HARQ buffer CR | Qualcomm, Inc. | revised |  | R4-2103980 |
| R4-2100631 | FR2 HST general discussion | Qualcomm, Inc. | noted |  |  |
| R4-2100632 | FR2 HST RRM discussion | Qualcomm, Inc. | not treated |  |  |
| R4-2100633 | FR1 HST RRM discussion | Qualcomm, Inc. | noted |  |  |
| R4-2100634 | SRS antenna switch discussion | Qualcomm, Inc. | noted |  |  |
| R4-2100635 | Discussion on Requirements for NR Repeaters | CommScope Technologies AG | noted |  |  |
| R4-2100636 | Discussion on applicability of CBM and IBM for different CA configurations | LG Electronics | noted |  |  |
| R4-2100637 | Discussion on feasibility for inter-band CA configurations | LG Electronics | noted |  |  |
| R4-2100638 | Big CR-Introduction of NR V2X RRM performance requirements (Rel-16) | LG Electronics | revised |  | R4-2103538 |
| R4-2100639 | Big CR-Introduction of NR V2X RRM performance requirements (Rel-17) | LG Electronics | agreed |  |  |
| R4-2100640 | Discussion on MRTD and MTTD requirements on CBM and IBM for FR2 inter-band CA | LG Electronics | noted |  |  |
| R4-2100641 | Discussion on multiple concurrent and independent MG patterns | LG Electronics | noted |  |  |
| R4-2100642 | TP update for TR 37.717-21-11: EN-DC_1-11_n28 | SoftBank Corp., LG Electronics | revised |  | R4-2103011 |
| R4-2100643 | TP update for TR 37.717-11-21: EN-DC_11_n3-n77 | SoftBank Corp. | approved |  |  |
| R4-2100644 | TP update for TR 37.717-11-21: EN-DC_11_n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100645 | CR for missing B48 references in a table and note | C Spire Wireless | withdrawn |  |  |
| R4-2100646 | Discussion on measurement requirements for NTN | LG Electronics UK | noted |  |  |
| R4-2100647 | Discussion on timing requirements for NTN | LG Electronics UK | noted |  |  |
| R4-2100648 | CR for missing B48 references in a table and note | C Spire Wireless | agreed |  |  |
| R4-2100649 | TP for TR 37.717-11-21: EN-DC_3-11_n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100650 | TP for TR 37.717-11-21: EN-DC_8-11_n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100651 | CR for missing B48 references in a table and note | C Spire Wireless | agreed |  |  |
| R4-2100652 | TP for TR 37.717-31-11: EN-DC_1-8-42_n3 | SoftBank Corp. | approved |  |  |
| R4-2100653 | TP for TR 37.717-11-21: EN-DC_1-11_n3-n77 | SoftBank Corp. | approved |  |  |
| R4-2100654 | TP for TR 37.717-11-21: EN-DC_8-11_n3-n77 | SoftBank Corp. | approved |  |  |
| R4-2100655 | Discussion on interruption due to SRS antenna port switching | LG Electronics Inc. | noted |  |  |
| R4-2100656 | Draft CR for PSSCH demodulation requirements for NR V2X | LG Electronics Inc. | revised |  | R4-2103815 |
| R4-2100657 | Discussion and simulation results for PSSCH single link test in NR V2X | LG Electronics Inc. | noted |  |  |
| R4-2100658 | Discussion and simulation results for PSCCH single link test in NR V2X | LG Electronics Inc. | noted |  |  |
| R4-2100659 | Simulation results for PSBCH single link test in NR V2X | LG Electronics Inc. | noted |  |  |
| R4-2100660 | TP for TR 37.717-11-21: EN-DC_1-11_n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100661 | Simulation results for PSFCH single link test in NR V2X | LG Electronics Inc. | noted |  |  |
| R4-2100662 | Discussion and simulation results for multiple link tests in NR V2X | LG Electronics Inc. | noted |  |  |
| R4-2100663 | TP for TR 37.717-11-21: EN-DC_1-42_n3-n28 | SoftBank Corp. | approved |  |  |
| R4-2100664 | Discussion on enhanced test method for polarization basis mismatch | LG Electronics Inc. | noted |  |  |
| R4-2100665 | Discussion on enhance test method to reduce FR2 OTA test time | LG Electronics Inc. | noted |  |  |
| R4-2100666 | TP for TR 37.717-11-21: EN-DC_1-42_n3-n77 | SoftBank Corp. | approved |  |  |
| R4-2100667 | TP for TR 37.717-11-21: EN-DC_8-42_n3-n28 | SoftBank Corp. | approved |  |  |
| R4-2100668 | TP for TR 37.717-11-21: EN-DC_8-42_n3-n77 | SoftBank Corp. | approved |  |  |
| R4-2100669 | TP for TR 37.717-11-31: EN-DC_11_n3-n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100670 | TP for TR 37.717-11-31: EN-DC_42_n3-n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100671 | TP for TR 37.717-31-11: EN-DC_1-3-42_n28 | SoftBank Corp. | approved |  |  |
| R4-2100672 | TP for TR 37.717-41-11: EN-DC_1-3-8-11_n28 | SoftBank Corp. | approved |  |  |
| R4-2100673 | TP for TR 37.717-41-11: EN-DC_1-3-8-11_n77 | SoftBank Corp. | approved |  |  |
| R4-2100674 | TP for TR 37.717-11-21: EN-DC_1-8-11_n3-n28 | SoftBank Corp. | approved |  |  |
| R4-2100675 | TP for TR 37.717-11-21: EN-DC_1-8-11_n3-n77 | SoftBank Corp. | approved |  |  |
| R4-2100676 | TP for TR 37.717-11-21: EN-DC_1-3-11_n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100677 | TP for TR 37.717-11-21: EN-DC_1-8-11_n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100678 | TP for TR 37.717-11-21: EN-DC_1-8-42_n3-n28 | SoftBank Corp. | approved |  |  |
| R4-2100679 | TP for TR 37.717-11-21: EN-DC_1-8-42_n3-n77 | SoftBank Corp. | approved |  |  |
| R4-2100680 | TP for TR 37.717-11-21: EN-DC_3-8-11_n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100681 | TP for TR 37.717-11-31: EN-DC_1-8_n3-n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100682 | TP for TR 37.717-11-31: EN-DC_1-11_n3-n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100683 | Correction on supported channel bandwidth for CA_n39-n41-n79 | CATT, CMCC | noted |  |  |
| R4-2100684 | Correction on supported channel bandwidth for CA_n39-n41-n79 | CATT, CMCC | withdrawn |  |  |
| R4-2100685 | TP for TR 37.717-11-31: EN-DC_1-42_n3-n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100686 | TP for TR 37.717-11-31: EN-DC_8-11_n3-n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100687 | TP for TR 37.717-11-31: EN-DC_8-42_n3-n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100688 | TP for TR 37.717-11-21: EN-DC_1-3-42_n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100689 | TP for TR 37.717-11-21: EN-DC_1-3-8-11_n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100690 | TP for TR 37.717-11-31: EN-DC_1-8-11_n3-n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100691 | TP for TR 37.717-11-31: EN-DC_1-8-42_n3-n28-n77 | SoftBank Corp. | approved |  |  |
| R4-2100692 | Proposals on FR2 PC5 beam correspondence | MediaTek Beijing Inc. | noted |  |  |
| R4-2100693 | Status overview and proposals on FR2 inter-band CA discussion | MediaTek Beijing Inc. | noted |  |  |
| R4-2100694 | Draft CR for TS 38.101-1: Support of n77(2A) for DC_n28-n77 | SoftBank Corp. | endorsed |  |  |
| R4-2100695 | Proposals on PC3 RIB of CA_n258A-n260A and CA_n257A-n259A | MediaTek Beijing Inc. | noted |  |  |
| R4-2100696 | Draft CR for TS 38.101-3: Support of DC_3-11_n257 | SoftBank Corp. | endorsed |  |  |
| R4-2100697 | Views on inter-band UL CA power class definition | MediaTek Beijing Inc. | noted |  |  |
| R4-2100698 | Draft CR for TS 38.101-1: Support of DC_ n3-n28-n77 | SoftBank Corp. | endorsed |  |  |
| R4-2100699 | Practical TPMI and 2-port CSI-RS for FR2 SISO test enhancement | MediaTek Beijing Inc. | noted |  |  |
| R4-2100700 | Draft CR for TS 38.101-3: Support of n77(2A) in DC_ n28-n77-n257 | SoftBank Corp. | endorsed |  |  |
| R4-2100701 | Draft CR for TS 38.101-3: Support of UL CA in CA_n3-n28-n77-n257 | SoftBank Corp. | endorsed |  |  |
| R4-2100702 | Draft CR for TS 38.101-3: Support of DC_1-3-11_n257 and DC_3-8-11_n257 | SoftBank Corp. | endorsed |  |  |
| R4-2100703 | REFSENS of n8 and n71 for 35MHz channel bandwidth | Murata Manufacturing Co Ltd. | noted |  |  |
| R4-2100704 | Draft CR for TS 38.101-3: Support of DC_1-3-8-11_n257 | SoftBank Corp. | endorsed |  |  |
| R4-2100705 | REFSENS of n25 for 45MHz channel bandwidth | Murata Manufacturing Co Ltd. | noted |  |  |
| R4-2100706 | MSD evaluation for TR 37.717-11-21 | MediaTek Inc. | noted |  |  |
| R4-2100707 | Discussion on DL CA_n77(3A) support | SoftBank Corp. | noted |  |  |
| R4-2100708 | TP for TR 38.717-01-01: CA_3DL_n77(3A)_1UL_n77A | SoftBank Corp. | approved |  |  |
| R4-2100709 | CR for 38.307: Introduction of power class 5 for FR2 | SoftBank Corp. | agreed |  |  |
| R4-2100710 | Discussion on RRM requirements for handover with PSCell | Xiaomi | noted |  |  |
| R4-2100711 | Discussion on SCell activation and deactication requirements for PUCCH Scell | Xiaomi | noted |  |  |
| R4-2100712 | Discussion on pre-configured MG pattern for NR | Xiaomi | noted |  |  |
| R4-2100713 | Discussion on multiple concurrent and independent MG patterns for NR | Xiaomi | noted |  |  |
| R4-2100714 | Discussion on timing requirements for NTN | Xiaomi | noted |  |  |
| R4-2100715 | Discussion on measurement requirements for NTN | Xiaomi | noted |  |  |
| R4-2100716 | Discussion on the remaining issues of core requirement for CSI-RS L3 measurement | Xiaomi | noted |  |  |
| R4-2100717 | Discussion on the remaining issues of performance requirement for CSI-RS L3 measurement | Xiaomi | noted |  |  |
| R4-2100718 | CR on core requirement for CSI-RS L3 measurement | Xiaomi | postponed |  | R4-2103632 |
| R4-2100719 | CR on CSI-RSRP performance requirement for CSI-RS L3 measurement | Xiaomi | postponed |  |  |
| R4-2100720 | CR on CSI-RSRQ performance requirement for CSI-RS L3 measurement | Xiaomi | postponed |  |  |
| R4-2100721 | CR on CSI-SINR performance requirement for CSI-RS L3 measurement | Xiaomi | postponed |  |  |
| R4-2100722 | RRM test cases for CSI-RS L3 intra-frequency measurements | Xiaomi | revised |  | R4-2103640 |
| R4-2100723 | RRM test cases for CSI-RS L3 inter-frequency measurements | Xiaomi | revised |  | R4-2103643 |
| R4-2100724 | RRM test cases for CSI-RS L3 measurement performance requirements | Xiaomi | revised |  | R4-2103646 |
| R4-2100725 | Discussion on relaxation of RLM/BFD measurements | Xiaomi | noted |  |  |
| R4-2100726 | Discussion on SRS antenna switching RRM requirements | Xiaomi | noted |  |  |
| R4-2100727 | CR on RRM test cases for NR UE power saving | Xiaomi | merged |  |  |
| R4-2100728 | Introduction of LTE inter-band Carrier Aggregation for x bands DL (x=4, 5) with 1 band UL to TS36.101 | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2100729 | TP to TR 36.717-02-01 CA_7-25 | Nokia, Nokia Shanghai Bell | approved |  |  |
| R4-2100730 | TP to TR 36.717-02-01 CA_25-66 | Nokia, Nokia Shanghai Bell | approved |  |  |
| R4-2100731 | TP to TR 36.717-03-01 CA_7-25-66 | Nokia, Nokia Shanghai Bell | approved |  |  |
| R4-2100732 | TP to TR 37.717-11-11 DC_25_n77 | Nokia, Nokia Shanghai Bell | revised |  | R4-2103012 |
| R4-2100733 | TP to TR 37.717-11-11 DC_25_n78 | Nokia, Nokia Shanghai Bell | revised |  | R4-2103013 |
| R4-2100734 | TP to TR 37.717-21-11 DC_7-25_n77 | Nokia, Nokia Shanghai Bell | revised |  | R4-2103014 |
| R4-2100735 | TP to TR 37.717-21-11 DC_7-25_n78 | Nokia, Nokia Shanghai Bell | revised |  | R4-2103015 |
| R4-2100736 | TP to TR 37.717-21-11 DC_25-66_n77 | Nokia, Nokia Shanghai Bell | revised |  | R4-2103016 |
| R4-2100737 | TP to TR 37.717-21-11 DC_25-66_n78 | Nokia, Nokia Shanghai Bell | revised |  | R4-2103017 |
| R4-2100738 | TP to TR 38.717-03-01 CA_n5-n25-n77 | Nokia, Nokia Shanghai Bell | revised |  | R4-2103077 |
| R4-2100739 | TP to TR 38.717-03-01 CA_n25-n66-n77 | Nokia, Nokia Shanghai Bell | revised |  | R4-2103078 |
| R4-2100740 | TP to TR 38.717-03-02 CA_n5-n25-n77 | Nokia, Nokia Shanghai Bell | revised |  | R4-2103085 |
| R4-2100741 | TP to TR 38.717-03-02 CA_n25-n66-n77 | Nokia, Nokia Shanghai Bell | revised |  | R4-2103100 |
| R4-2100742 | Introduction of DC_7-25-66_n77 and DC_7-25-66_n78 | Nokia, Nokia Shanghai Bell | endorsed |  |  |
| R4-2100743 | Introduction of new BCS to CA_n5A-n66A and CA_n5A-78A | Nokia, Nokia Shanghai Bell | endorsed |  |  |
| R4-2100744 | Regulatory study for APT 600 MHz | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100745 | Coexistence for APT 600 MHz | Nokia, Nokia Shanghai Bell, CBN | noted |  |  |
| R4-2100746 | Frequency arrangements for APT 600 MHz | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100747 | FR2 inter-band CA for different frequency band groups with IBM | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100748 | EIRP requirements for n262 | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100749 | TP to TR 38.847: UE Tx requirement for n262 | Nokia, Nokia Shanghai Bell | revised |  | R4-2103204 |
| R4-2100750 | Introduction of n262 UE RF requirements | Nokia, Nokia Shanghai Bell | revised |  | R4-2103205 |
| R4-2100751 | EIS requirements for n262 | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100752 | TP to TR 38.847: UE Rx requirement for n262 | Nokia, Nokia Shanghai Bell | revised |  | R4-2103206 |
| R4-2100753 | Spectrum utilization with channel raster and PRB grid alignment | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2100754 | Introduction of test cases for BFD and link recovery procedure for Scell | MediaTek inc. | revised |  | R4-2103595 |
| R4-2100755 | Discussion on the scaling factor for SCell beam failure recovery | MediaTek inc. | noted |  |  |
| R4-2100756 | CR on the scaling factor for SCell beam failure recovery in R16 | MediaTek inc. | merged |  |  |
| R4-2100757 | CR on the scaling factor for SCell beam failure recovery in R17 | MediaTek inc. | withdrawn |  |  |
| R4-2100758 | Correction on the measurement restriction for CSI-IM resource in R16 | MediaTek inc. | agreed |  |  |
| R4-2100759 | Correction on the measurement restriction for CSI-IM resource in R17 | MediaTek inc. | agreed |  |  |
| R4-2100760 | Correction on PRACH configuration for FR2 Non-Contention based Random Access in R15 | MediaTek inc. | agreed |  |  |
| R4-2100761 | Correction on PRACH configuration for FR2 Non-Contention based Random Access in R16 | MediaTek inc. | agreed |  |  |
| R4-2100762 | Correction on PRACH configuration for FR2 Non-Contention based Random Access in R17 | MediaTek inc. | agreed |  |  |
| R4-2100763 | Correction on PRACH configuration for Beam Failure Detection and Link Recovery Test in R15 | MediaTek inc. | revised |  | R4-2104049 |
| R4-2100764 | Correction on PRACH configuration for Beam Failure Detection and Link Recovery Test in R16 | MediaTek inc. | agreed |  |  |
| R4-2100765 | Correction on PRACH configuration for Beam Failure Detection and Link Recovery Test in R17 | MediaTek inc. | agreed |  |  |
| R4-2100766 | Correction on PRACH RMC for FR1 CSI-RS based Non-Contention based Random Access for BFR in R15 | MediaTek inc. | agreed |  |  |
| R4-2100767 | Correction on PRACH RMC for FR1 CSI-RS based Non-Contention based Random Access for BFR in R16 | MediaTek inc. | agreed |  |  |
| R4-2100768 | Correction on PRACH RMC for FR1 CSI-RS based Non-Contention based Random Access for BFR in R17 | MediaTek inc. | agreed |  |  |
| R4-2100769 | Discussion on terminology for NR-U RRM requirements | MediaTek inc. | noted |  |  |
| R4-2100770 | Discussion on measurement requirements for NR-U | MediaTek inc. | noted |  |  |
| R4-2100771 | Discussion on timing requirements for NR-U | MediaTek inc. | noted |  |  |
| R4-2100772 | Discussion on general test setting for NR-U test cases | MediaTek inc. | noted |  |  |
| R4-2100773 | Discussion on RRM test cases in NR-U | MediaTek inc. | noted |  |  |
| R4-2100774 | Introduction of test cases for UE transmit timing requirements with CCA | MediaTek inc. | postponed |  |  |
| R4-2100775 | Introduction of test cases for Accuracy for NR-U inter-frequency SS-RSRP measurements | MediaTek inc. | postponed |  |  |
| R4-2100776 | Introduction of test cases for L1-RSRP measurement accuracy with CCA serving cell | MediaTek inc. | postponed |  |  |
| R4-2100777 | Discussion on Inter-band CA requirement for FR2 | MediaTek inc. | noted |  |  |
| R4-2100778 | Correction on scheduling availability and measurement restriction on FR2 inter-band CA in R16 | MediaTek inc. | revised |  | R4-2103623 |
| R4-2100779 | Correction on scheduling availability and measurement restriction on FR2 inter-band CA in R17 | MediaTek inc. | agreed |  |  |
| R4-2100780 | Discussion on UE Pre-compensation for UL synchronization for in NTN | MediaTek inc. | noted |  |  |
| R4-2100781 | Discussion on the minimum and maximum channel bandwidth for B52.6GHz | vivo | noted |  |  |
| R4-2100782 | Comparison of different PN models for B52.6G | vivo | noted |  |  |
| R4-2100783 | Further discussion on switching period in the ITS band | vivo | noted |  |  |
| R4-2100784 | General issues about licensed bands partially used for SL | vivo | noted |  |  |
| R4-2100785 | Discussion on HPUE for NR sidelink enhancement in R17 | vivo | noted |  |  |
| R4-2100786 | Remaining issues on PDSCH CA test applicability | China Telecom | noted |  |  |
| R4-2100787 | Draft CR for TS 38.307 on UE demodulation performance requirements (Rel-15) | China Telecom | revised |  | R4-2103839 |
| R4-2100788 | Draft CR for TS 38.307 on UE demodulation performance requirements (Rel-16) | China Telecom | revised |  | R4-2103840 |
| R4-2100789 | Draft CR for TS 38.307 on UE demodulation performance requirements (Rel-17) | China Telecom | agreed |  |  |
| R4-2100790 | Remaining issues on Tx switching enhancement and draft LS | China Telecom | noted |  |  |
| R4-2100791 | Switching time mask for 2Tx-2Tx switching between two carriers and 1Tx-2Tx/2Tx-2Tx switching between two bands in Rel-17 | China Telecom | revised |  | R4-2103236 |
| R4-2100792 | CR for TS 38.101-1: Correction on 1Tx-2Tx switching between two uplink carriers (Rel-16) | China Telecom | agreed |  |  |
| R4-2100793 | CR for TS 38.101-1: Correction on 1Tx-2Tx switching between two uplink carriers (Rel-17) | China Telecom | agreed |  |  |
| R4-2100794 | CR for TS 38.101-3: Correction on 1Tx-2Tx switching between two uplink carriers (Rel-16) | China Telecom | agreed |  |  |
| R4-2100795 | CR for TS 38.101-3: Correction on 1Tx-2Tx switching between two uplink carriers (Rel-17) | China Telecom | agreed |  |  |
| R4-2100796 | On BS demodulation requirements for Rel-16 PUSCH repetition type B | China Telecom | noted |  |  |
| R4-2100797 | Discussion on requirement for LTE/NR spectrum sharing and dual connectivity (DSS EN-DC) in band 1/n1 | China Telecom | noted |  |  |
| R4-2100798 | MSD due to wider BW evaluation for DC_28_n5 | MediaTek Inc. | noted |  |  |
| R4-2100799 | CR on introducing NR SUL bands n80 to UL-MIMO configuration | CMCC | agreed |  |  |
| R4-2100800 | Discussion on 2Tx switching between carrier 1 and carrier 2 | CMCC | noted |  |  |
| R4-2100801 | Discussion on Tx switching between 1 carrier on band A and 2 contiguous aggregated carriers on band B | CMCC | noted |  |  |
| R4-2100802 | Discussion on NTN RRM measurement requirements | CMCC | noted |  |  |
| R4-2100803 | Discussion on band definition and channel BW for NR in 52.6GHz | CMCC | noted |  |  |
| R4-2100804 | Discussion on the general aspects for irregular bandwidth | CMCC | noted |  |  |
| R4-2100805 | Discussion on overlapping UE channel bandwidths | CMCC | noted |  |  |
| R4-2100806 | TP on DL applicability of CA_n3-n40-n41 for 37.xxx | CMCC | revised |  | R4-2103221 |
| R4-2100807 | TP on DL applicability of CA_n3-n41-n79 for 37.xxx | CMCC | revised |  | R4-2103222 |
| R4-2100808 | TP on DL applicability of CA_n8-n39-n41for 37.xxx | CMCC | revised |  | R4-2103223 |
| R4-2100809 | TP on DL applicability of CA_n8-n41-n79 for 37.xxx | CMCC | revised |  | R4-2103224 |
| R4-2100810 | TP on DL applicability of CA_n39-n41-n79 for 37.xxx | CMCC | revised |  | R4-2103225 |
| R4-2100811 | TP on DL applicability of CA_n40-n41-n79 for 37.xxx | CMCC | revised |  | R4-2103226 |
| R4-2100812 | Discussion on DL interruption applicability for inter-band CA with 3 bands | CMCC | noted |  |  |
| R4-2100813 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | withdrawn |  |  |
| R4-2100814 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | withdrawn |  |  |
| R4-2100815 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | withdrawn |  |  |
| R4-2100816 | CR for NR PDSCH FR1 CA 2Rx performance requirements | CMCC | agreed |  |  |
| R4-2100817 | Simulation results summary for R16 power saving demodulation | CMCC | noted |  |  |
| R4-2100818 | CR for TS38.101-4, test for FR1 TDD PDCCH DCI format 2_6 demodulation | CMCC | revised |  | R4-2103823 |
| R4-2100819 | Discussion on NTN timing requirements | CMCC | noted |  |  |
| R4-2100820 | Demodulation on UE power saving | CMCC | noted |  |  |
| R4-2100821 | Discussion on RLM/BFD relaxation for NR power saving enhancement | CMCC | noted |  |  |
| R4-2100822 | Test applicability rule for NR CA PDSCH normal demodulation | CMCC | noted |  |  |
| R4-2100823 | Discussion on remaining Tx requirements of BS for 6425-7125MHz | CMCC | noted |  |  |
| R4-2100824 | Examplary bands for NTN | CMCC | noted |  |  |
| R4-2100825 | Performance evaluation for calibration | CMCC | noted |  |  |
| R4-2100826 | Discussion on EVM measurement methodology for IAB-MT | CMCC | noted |  |  |
| R4-2100827 | Discussion on RF architecture and RF requirements scope of repeater | CMCC | noted |  |  |
| R4-2100828 | Power related conducted requirements for repeaters | CMCC | noted |  |  |
| R4-2100829 | Power related radiated requirements for repeaters | CMCC | noted |  |  |
| R4-2100830 | Emission related conducted requirements for repeaters | CMCC | noted |  |  |
| R4-2100831 | Emission related radiated requirements for repeaters | CMCC | noted |  |  |
| R4-2100832 | General parts of NR repeater | CMCC | noted |  |  |
| R4-2100833 | Scope of test cases for NR-U | ZTE Corporation | noted |  |  |
| R4-2100834 | Configurations for NR-U RRM test cases | ZTE Corporation | noted |  |  |
| R4-2100835 | Update the applicability rule for 2-step RA | ZTE Corporation | noted |  |  |
| R4-2100836 | Test cases for intra and inter frequency and inter-RAT measurement requirements | ZTE Corporation | noted |  |  |
| R4-2100837 | Test cases for intra and inter frequency and inter-RAT measurement accuracy | ZTE Corporation | noted |  |  |
| R4-2100838 | Test cases for PSCell addition and release | ZTE Corporation | noted |  |  |
| R4-2100839 | On test cases for cell reselection under NR-U | ZTE Corporation | noted |  |  |
| R4-2100840 | On test cases for handover under NR-U | ZTE Corporation | noted |  |  |
| R4-2100841 | On test cases for BWP switching delay and interruptions under NR-U | ZTE Corporation | noted |  |  |
| R4-2100842 | Test cases for RRC Connection Release with Redirection in NR-U | ZTE Corporation | noted |  |  |
| R4-2100843 | Test cases for timing in NR-U | ZTE Corporation | noted |  |  |
| R4-2100844 | CR for 38.101-3 Correction on EN-DC MSD due to cross band isolation for DC_28_n5 (R16) | MediaTek Inc. | not pursued |  |  |
| R4-2100845 | Channel Model Validation Bounds | Spirent Communications | noted |  |  |
| R4-2100846 | CR for 38.101-1: Update of missing fallback NR-DC combinations Rel-16 | SoftBank Corp., ZTE Corporation | agreed |  |  |
| R4-2100847 | CR for 38.101-1: Update of missing fallback NR-DC combinations Rel-17 | SoftBank Corp., ZTE Corporation | agreed |  |  |
| R4-2100848 | CR on HST-SFN requirements for TDD | CMCC | not pursued |  | R4-2103858 |
| R4-2100849 | CR on HST RRM requirements in connected mode | CMCC | revised |  | R4-2103654 |
| R4-2100850 | CR on HST RRM requirements in connected mode | CMCC | agreed |  |  |
| R4-2100851 | Discussion on deactivated SCell measurement for intra-frequency measurement with measurement gap | CMCC | noted |  |  |
| R4-2100852 | CR on deactivated SCell measurement for intra-frequency measurement with measurement gap | CMCC | merged |  |  |
| R4-2100853 | CR on deactivated SCell measurement for intra-frequency measurement with measurement gap | CMCC | withdrawn |  |  |
| R4-2100854 | CR on release independent for Rel.16 NR HST UE demodulation requirements | CMCC | agreed |  |  |
| R4-2100855 | CR on release independent for Rel.16 NR HST UE demodulation requirements | CMCC | revised |  | R4-2103859 |
| R4-2100856 | CR on release independent for Rel.16 NR HST UE demodulation requirements | CMCC | agreed |  |  |
| R4-2100857 | General discussion on NR HST RRM enhancement for FR1 CA scenario | CMCC | noted |  |  |
| R4-2100858 | General discussion on NR HST UE demodulation for FR1 CA scenario | CMCC | noted |  |  |
| R4-2100859 | Discussion on NR HST UE demodulation for enhanced transmission scheme | CMCC | noted |  |  |
| R4-2100860 | Discussion on test cases for mandatory MG patterns | CMCC | noted |  |  |
| R4-2100861 | Discussion on performance requirements for CSI-RS based L3 measurement | CMCC | noted |  |  |
| R4-2100862 | Simulation results for CSI-RSRP | CMCC | noted |  |  |
| R4-2100863 | Simulation results for CSI-RSRQ | CMCC | noted |  |  |
| R4-2100864 | Simulation results for CSI-SINR | CMCC | noted |  |  |
| R4-2100865 | Discussion on side condition for CSI-SINR measurement | CMCC | noted |  |  |
| R4-2100866 | General discussion on RRM requirements for FR2 HST | CMCC | noted |  |  |
| R4-2100867 | Discussion on HO with PSCell | CMCC | noted |  |  |
| R4-2100868 | Simulation results for the measurement of L1-SINR | CMCC | noted |  |  |
| R4-2100869 | Discussion on L1-SINR measurement accuracy | CMCC | noted |  |  |
| R4-2100870 | Discussion on multiple concurrent and independent MG patterns | CMCC | noted |  |  |
| R4-2100871 | Discussion on pre-configured MG pattern(s) | CMCC | noted |  |  |
| R4-2100872 | Discussion on PUCCH SCell activation/deactivation | CMCC | noted |  |  |
| R4-2100873 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | withdrawn |  |  |
| R4-2100874 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | withdrawn |  |  |
| R4-2100875 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | withdrawn |  |  |
| R4-2100876 | CR for 38.101-1: Update of simultaneous Rx/Tx capability for some NR CA band combinations Rel-16 | SoftBank Corp. | not pursued |  |  |
| R4-2100877 | CR for 38.101-1: Update of simultaneous Rx/Tx capability for some NR CA band combinations Rel-17 | SoftBank Corp. | withdrawn |  |  |
| R4-2100878 | CR for 38.101-3: Update of simultaneous Rx/Tx capability for some EN-DC band combinations Rel-16 | SoftBank Corp. | not pursued |  |  |
| R4-2100879 | CR for 38.101-3: Update of simultaneous Rx/Tx capability for some EN-DC band combinations Rel-17 | SoftBank Corp. | withdrawn |  |  |
| R4-2100880 | Offline e-mail discussion summary on the TDLD30 channel simplification | China Telecom | noted |  |  |
| R4-2100881 | Simulation results for PDSCH normal demodulation for FR2 DL 256QAM | China Telecom | noted |  |  |
| R4-2100882 | On CQI reporting requirements for FR2 DL 256QAM | China Telecom | noted |  |  |
| R4-2100883 | Summary of CQI reporting simulation results for FR2 DL 256QAM (TDD) | China Telecom | noted |  |  |
| R4-2100884 | CR on adding applicability, requirements and measurement channel for FR2 DL 256QAM CQI reporting test under fading condition | China Telecom | revised |  | R4-2103837 |
| R4-2100885 | Updating on CR for SDR requirements for FR2 DL 256QAM capable band | China Telecom | noted |  |  |
| R4-2100886 | CR: Adding applicability and requirements for FR1 and FR2 CA CQI reporting test | China Telecom | agreed |  |  |
| R4-2100887 | New WID on Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n259 | SoftBank Corp., Rakuten Mobile | not treated |  |  |
| R4-2100888 | Motivation on new WID on Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n259 | SoftBank Corp., Rakuten Mobile | not treated |  |  |
| R4-2100889 | Discussion on LS on PUCCH and PUSCH repetition | Qualcomm Incorporated | noted |  |  |
| R4-2100890 | CR to TS38.124 on radiated emissions | Samsung | not pursued |  |  |
| R4-2100891 | CR to TS38.124 on radiated emissions | Samsung | withdrawn |  |  |
| R4-2100892 | Discussion on FR1 test parameters and Figure of Merit | Samsung | noted |  |  |
| R4-2100893 | Discussion on IBM inter-band CA within same frequency group | Samsung | noted |  |  |
| R4-2100894 | Discussion on FR2 EIRP measurement enhancement | Samsung | noted |  |  |
| R4-2100895 | Discussion on FR2 test time reduction | Samsung | noted |  |  |
| R4-2100896 | Introduction of PMI test cases with Rel-16 eType II codebook | Samsung | revised |  | R4-2102938 |
| R4-2100897 | Introduction of PMI test cases with Rel-15 Type II codebook | Samsung | revised |  | R4-2102939 |
| R4-2100898 | Simulation results for Single-DCI based SDM scheme | Samsung | noted |  |  |
| R4-2100899 | Simulation results for multi-DCI based transmsision scheme | Samsung | noted |  |  |
| R4-2100900 | Simulation results for URLLC schemes | Samsung | revised |  | R4-2103831 |
| R4-2100901 | Discussion and simulation results for Rel-16 eType II codebook test cases | Samsung | noted |  |  |
| R4-2100902 | Discussion and simulation results for Rel-15 Type II codebook test cases | Samsung | revised |  | R4-2103844 |
| R4-2100903 | Simulation results summary for eMIMO performance requirements | Samsung | noted |  |  |
| R4-2100904 | Simulation assumption for FR1 coexistence study | Samsung | noted |  |  |
| R4-2100905 | Views on NTN exemplary bands | Samsung | noted |  |  |
| R4-2100906 | View on IAB-MT RF Test configuration | Samsung | noted |  |  |
| R4-2100907 | Configuration on reference channel for IAB-MT | Samsung | noted |  |  |
| R4-2100908 | View on IAB manufacturer declaration | Samsung | noted |  |  |
| R4-2100909 | Draft CR to align the general clause of radiated and conducted requirement | Samsung | not pursued |  |  |
| R4-2100910 | Big CR for update on TR38.809 | Samsung | agreed |  |  |
| R4-2100911 | How to incorperate 52.6-71GHz in specification | Samsung | noted |  |  |
| R4-2100912 | Regulatory information on RF exposure for FWA devices | Samsung | noted |  |  |
| R4-2100913 | UE RF requirements for 47 GHz band | Samsung | noted |  |  |
| R4-2100914 | On the Support of Transparent Tx Diversity in Rel-16 | Samsung | noted |  |  |
| R4-2100915 | Further discussion on high speed train deployment scenario in FR2 | Samsung | noted |  |  |
| R4-2100916 | Simulation results and analysis on HST deployment scenario in FR2 | Samsung | noted |  |  |
| R4-2100917 | Discussion on RRM requirement for FR2 HST | Samsung | noted |  |  |
| R4-2100918 | Discussion on UE RF requirement for FR2 HST | Samsung | noted |  |  |
| R4-2100919 | View on PUSCH demodulation requirement for NR-U | Samsung | noted |  |  |
| R4-2100920 | View on PUCCH demodulation requirement for NR-U | Samsung | noted |  |  |
| R4-2100921 | View on PRACH demodulation requirement for NR-U | Samsung | noted |  |  |
| R4-2100922 | Updated simulation results for NR HST PUSCH | Samsung | noted |  |  |
| R4-2100923 | Discussion and simulation results for BS URLLC requirement | Samsung | noted |  |  |
| R4-2100924 | Simulation results for BS 2-step RACH requirement | Samsung | noted |  |  |
| R4-2100925 | CR on UL timing adjustment conducted performance requirement for TS 38.141-1 | Samsung | revised |  | R4-2103862 |
| R4-2100926 | CR on UL timing adjustment conducted performance requirement for TS 38.141-1 | Samsung | agreed |  |  |
| R4-2100927 | CR on PUSCH repetition type A and PUSCH mapping type B radiated performance requirement for TS 38.104 | Samsung | revised |  | R4-2103906 |
| R4-2100928 | CR on PUSCH repetition type A and PUSCH mapping type B radiated performance requirement for TS 38.104 | Samsung | agreed |  |  |
| R4-2100929 | CR on FRC for URLLC BS radiated performance requirement for TS 38.141-2 | Samsung | revised |  | R4-2103907 |
| R4-2100930 | CR on FRC for URLLC BS radiated performance requirement for TS 38.141-2 | Samsung | agreed |  |  |
| R4-2100931 | CR on MsgA PUSCH radiated performance requirement for TS 38.141-2 | Samsung | revised |  | R4-2103870 |
| R4-2100932 | CR on MsgA PUSCH radiated performance requirement for TS 38.141-2 | Samsung | agreed |  |  |
| R4-2100933 | CR to TS38.133 on L1-SINR measurement requirement | Samsung | revised |  | R4-2103590 |
| R4-2100934 | CR to TS38.133 on L1-SINR measurement requirement | Samsung | agreed |  |  |
| R4-2100935 | Remaining issues on L1-SINR measurement accuracy requirement | Samsung | noted |  |  |
| R4-2100936 | Draft CR on L1-SINR measurement accuracy tests with CSI-RS based CMR and dedicated IMR | Samsung | revised |  | R4-2103593 |
| R4-2100937 | Draft CR on CSI-RS configurations | Samsung | merged |  |  |
| R4-2100938 | Big CR: Introduction of Rel-16 NR eMIMO RRM performance requirements and test cases | Samsung | agreed |  |  |
| R4-2100939 | Big CR: Introduction of Rel-16 NR eMIMO RRM performance requirements and test cases | Samsung | agreed |  |  |
| R4-2100940 | Motivation on new WI of intra-band non-contiguous NR-DC using band n77 | SoftBank Corp. | not treated |  |  |
| R4-2100941 | Motivation for supporting non-colocated scenarios for band 42 and n77/n78 | SoftBank Corp. | not treated |  |  |
| R4-2100942 | Draft CR for 38.101-1 to introduce CA_n3A-n78(2A) and CA_n41-n78(2A) | Samsung, KDDI | revised |  | R4-2103036 |
| R4-2100943 | TP for TR 38.717-02-01: CA_n3-n18 | Samsung, KDDI | revised |  | R4-2103037 |
| R4-2100944 | TP for TR 38.717-02-01: CA_n18-n41 | Samsung, KDDI | revised |  | R4-2103038 |
| R4-2100945 | TP for TR 38.717-02-01: CA_n41-n77 | Samsung, KDDI | noted |  |  |
| R4-2100946 | TP for TR 38.717-02-01: CA_n41-n257 | Samsung, KDDI | revised |  | R4-2103049 |
| R4-2100947 | TP for TR 38.717-02-01: DC_n3-n41 | Samsung, KDDI | revised |  | R4-2103039 |
| R4-2100948 | TP for TR 38.717-02-01: DC_n28-n41 | Samsung, KDDI | noted |  |  |
| R4-2100949 | TP for TR 38.717-02-01: DC_n41-n77 | Samsung, KDDI | noted |  |  |
| R4-2100950 | TP for TR 38.717-02-01: DC_n41-n78 | Samsung, KDDI | noted |  |  |
| R4-2100951 | TP for TR 38.717-02-01: DC_n41-n257 | Samsung, KDDI | noted |  |  |
| R4-2100952 | TP for TR 38.717-03-01: CA_n3-n18-n41 | Samsung, KDDI | approved |  |  |
| R4-2100953 | TP for TR 38.717-03-01: CA_n3A-n28A-n77(2A) | Samsung, KDDI | revised |  | R4-2103079 |
| R4-2100954 | TP for TR 38.717-03-01: CA_n3A-n28A-n78(2A) | Samsung, KDDI | revised |  | R4-2103080 |
| R4-2100955 | TP for TR 38.717-03-01: CA_n3A-n41A-n77(2A) | Samsung, KDDI | approved |  |  |
| R4-2100956 | TP for TR 38.717-03-01: CA_n3A-n41A-n78(2A) | Samsung, KDDI | approved |  |  |
| R4-2100957 | TP for TR 38.717-03-01: CA_n28A-n41A-n77(2A) | Samsung, KDDI | approved |  |  |
| R4-2100958 | TP for TR 38.717-03-01: CA_n28A-n41A-n78(2A) | Samsung, KDDI | approved |  |  |
| R4-2100959 | TP for TR 38.717-03-02: CA_n3-n28-n78 | Samsung, KDDI | revised |  | R4-2103101 |
| R4-2100960 | TP for TR 38.717-03-02: DC_n3-n41-n257 | Samsung, KDDI | revised |  | R4-2103086 |
| R4-2100961 | TP for TR 38.717-03-02: DC_n28-n41-n257 | Samsung, KDDI | noted |  |  |
| R4-2100962 | TP for TR 38.717-03-02: DC_n41-n77-n257 | Samsung, KDDI | noted |  |  |
| R4-2100963 | TP for TR 38.717-03-02: DC_n41-n78-n257 | Samsung, KDDI | noted |  |  |
| R4-2100964 | TP for TR 38.717-03-02: CA_n3-n18-n41 | Samsung, KDDI | revised |  | R4-2103087 |
| R4-2100965 | TP for TR 38.717-03-02: CA_n3-n28-n41 | Samsung, KDDI | noted |  |  |
| R4-2100966 | TP for TR 38.717-03-02: CA_n3-n28-n77 | Samsung, KDDI | noted |  |  |
| R4-2100967 | TP for TR 38.717-03-02: CA_n3-n41-n77 | Samsung, KDDI | revised |  | R4-2103088 |
| R4-2100968 | TP for TR 38.717-03-02: CA_n3-n41-n78 | Samsung, KDDI | revised |  | R4-2103089 |
| R4-2100969 | TP for TR 38.717-03-02: CA_n28-n41-n77 | Samsung, KDDI | revised |  | R4-2103090 |
| R4-2100970 | TP for TR 38.717-03-02: CA_n28-n41-n78 | Samsung, KDDI | revised |  | R4-2103091 |
| R4-2100971 | TP for TR 38.717-04-02: CA_n3-n28-n41-n77 | Samsung, KDDI | approved |  |  |
| R4-2100972 | TP for TR 38.717-04-02: CA_n3-n28-n41-n78 | Samsung, KDDI | approved |  |  |
| R4-2100973 | TP for TR 38.717-02-01: CA_n25-n29 | Samsung, TELUS, Bell mobility | approved |  |  |
| R4-2100974 | TP for TR 38.717-02-01: CA_n25-n71 | Samsung, TELUS, Bell mobility | revised |  | R4-2103102 |
| R4-2100975 | TP for TR 38.717-02-01: CA_n25-n77 | Samsung, TELUS, Bell mobility | revised |  | R4-2103040 |
| R4-2100976 | TP for TR 38.717-02-01: CA_n71-n77 | Samsung, TELUS, Bell mobility | revised |  | R4-2103041 |
| R4-2100977 | TP for TR 38.717-02-01: CA_n13-n25 | Samsung, TELUS, Bell mobility | revised |  | R4-2103042 |
| R4-2100978 | TP for TR 38.717-02-01: CA_n13-n66 | Samsung, TELUS, Bell mobility | revised |  | R4-2103043 |
| R4-2100979 | TP for TR 38.717-03-01: CA_n13-n25-n66 | Samsung, TELUS, Bell mobility | revised |  | R4-2103081 |
| R4-2100980 | TP for TR 38.717-03-01: CA_n25A-n29A-n66A | Samsung, TELUS, Bell mobility | approved |  |  |
| R4-2100981 | TP for TR 38.717-03-02: CA_n25-n29-n66 | Samsung, TELUS, Bell mobility | revised |  | R4-2103092 |
| R4-2100982 | TP for TR 38.717-03-02: CA_n13-n25-n66 | Samsung, TELUS, Bell mobility | revised |  | R4-2103093 |
| R4-2100983 | TP for TR 37.717-21-11: DC_2-29_n78 | Samsung, TELUS, Bell mobility | revised |  | R4-2103018 |
| R4-2100984 | TP for TR 37.717-21-11: DC_29-66_n78 | Samsung, TELUS, Bell mobility | revised |  | R4-2103019 |
| R4-2100985 | TP for TR 37.717-31-11: DC_2-29-66_n78 | Samsung, TELUS, Bell mobility | approved |  | R4-2103020 |
| R4-2100986 | CR introduction completed band combinations for Dual Connectivity (DC) of x bands (x=2,3,4) LTE inter-band CA (xDL/1UL) and 1 NR FR1 band (1DL/1UL) and 1 NR FR2 band (1DL/1UL) | Samsung | withdrawn |  |  |
| R4-2100987 | Revised WID on Dual Connectivity (DC) of x bands (x=2,3,4) LTE inter-band CA (xDL/1UL) and 1 NR FR1 band (1DL/1UL) and 1 NR FR2 band (1DL/1UL) | Samsung | withdrawn |  |  |
| R4-2100988 | CR introduction completed band combinations for Dual Connectivity (DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | Samsung | agreed |  |  |
| R4-2100989 | Revised WID on Dual Connectivity (DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | Samsung | not concluded |  |  |
| R4-2100990 | CR on introduction of completed NR CA/DC combs with 4DL/2UL within FR1 | Samsung | revised |  | R4-2103255 |
| R4-2103255 | CR on introduction of completed NR CA/DC combs with 4DL/2UL within FR1 | Samsung | agreed |  |  |
| R4-2100991 | CR on introduction of completed NR CA/DC combs with 4DL/2UL including FR2 | Samsung | not concluded |  |  |
| R4-2100992 | Revised WID on NR CA/DC with 4DL/2UL | Samsung | not concluded |  |  |
| R4-2100993 | remove SNR brackets for HST PUSCH in TS38.141-2 | Ericsson | revised |  | R4-2103863 |
| R4-2100994 | remove SNR brackets for HST PUSCH in TS38.141-2 | Ericsson | revised |  | R4-2103953 |
| R4-2100995 | discussion on general issues in NR-U UE and CSI performance requirements | Ericsson | noted |  |  |
| R4-2100996 | discussion on NR-U PDSCH demodulation | Ericsson | noted |  |  |
| R4-2100997 | discussion on NR-U CQI report demodulation | Ericsson | noted |  |  |
| R4-2100998 | discussion on general issues in NR-U BS demodulation requirements | Ericsson | noted |  |  |
| R4-2100999 | discussion on NR-U PUSCH demodulation assumptions | Ericsson | noted |  |  |
| R4-2101000 | simulation results on NR-U PUSCH demodulation | Ericsson | noted |  |  |
| R4-2101001 | discussion on NR-U PUCCH demodulation assumptions | Ericsson | noted |  |  |
| R4-2101002 | simulation results on NR-U PUCCH demodulation | Ericsson | noted |  |  |
| R4-2101003 | discussion on NR-U PRACH demodulation assumptions | Ericsson | noted |  |  |
| R4-2101004 | simulation results on NR-U PRACH demodulation | Ericsson | noted |  |  |
| R4-2101005 | On applicability of additional emission requirement to CA/DC | SoftBank Corp. | noted |  |  |
| R4-2101006 | CR on Scell activation delay maintenance (R15) | Apple | revised |  | R4-2104043 |
| R4-2101007 | CR on Scell activation delay maintenance (R16) | Apple | agreed |  |  |
| R4-2101008 | CR on Scell activation delay maintenance (R17) | Apple | agreed |  |  |
| R4-2101009 | CR on maintenance for DAPS handover (R16) | Apple | merged |  |  |
| R4-2101010 | CR on maintenance for DAPS handover (R17) | Apple | withdrawn |  |  |
| R4-2101011 | Interruption requirements maintenance in NR-DC (R17) | Apple | agreed |  |  |
| R4-2101012 | CR on activation time in direct SCell activation (R17) | Apple | withdrawn |  |  |
| R4-2101013 | CR on HST core part maintenance (R17) | Apple | agreed |  |  |
| R4-2101014 | On PL-RS switch and CGI reading for NR-U | Apple | noted |  |  |
| R4-2101015 | TCs for RSSI and CO measurement accuracy in NR-U R16 | Apple | postponed |  |  |
| R4-2101016 | Support of Japan regulation for 2.5GHz(BWA) in NR BS | SoftBank Corp., KDDI Corporation, NEC Corporation | noted |  |  |
| R4-2101017 | Discussion and Draft Reply LS to RAN1 on beam switching gap | Apple | noted |  |  |
| R4-2101018 | CR to 38.101-4 on FRC table update for URLLC ultra low BLER requirements | Apple | agreed |  |  |
| R4-2101019 | TP for DC_1A_n28A-n77A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101020 | TP for DC_1A_n28A-n78A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101021 | TP for DC_3A_n1A-n77A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101022 | TP for DC_3A_n1A-n78A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101023 | TP for DC_3A_n28A-n77A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101024 | TP for DC_3A_n28A-n78A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101025 | TP for DC_19A_n1A-n77A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101026 | TP for DC_19A_n1A-n78A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101027 | TP for DC_21A_n1A-n77A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101028 | TP for DC_21A_n1A-n78A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101029 | TP for DC_21A_n28A-n77A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101030 | TP for DC_21A_n28A-n78A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101031 | TP for DC_1A-3A_n28A-n77A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101032 | TP for DC_1A-3A_n28A-n78A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101033 | TP for DC_1A-21A_n28A-n77A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101034 | TP for DC_1A-21A_n28A-n78A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101035 | TP for DC_3A-21A_n1A-n77A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101036 | TP for DC_3A-21A_n1A-n78A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101037 | TP for DC_3A-21A_n28A-n77A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101038 | TP for DC_3A-21A_n28A-n78A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101039 | TP for DC_19A-42A_n1A-n77A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101040 | TP for DC_19A-42A_n1A-n78A-n79A for TR37.717-11-31 | NTT DOCOMO, INC. | approved |  |  |
| R4-2101041 | Views on candidate bands for NR repeater | NTT DOCOMO, INC. | noted |  |  |
| R4-2101042 | CR for TS 38.141-1 Updates of NR PUSCH performance requirements for HST | NTT DOCOMO, INC. | revised |  | R4-2103864 |
| R4-2101043 | CR for TS 38.141-1 Updates of NR PUSCH performance requirements for HST | NTT DOCOMO, INC. | agreed |  |  |
| R4-2101044 | CR for TS 38.141-2 Updates of performance requirements of PUSCH repetition type A and PUSCH mapping type B for URLLC | NTT DOCOMO, INC. | revised |  | R4-2103908 |
| R4-2101045 | CR for TS 38.141-2 Updates of performance requirements of PUSCH repetition type A and PUSCH mapping type B for URLLC | NTT DOCOMO, INC. | agreed |  |  |
| R4-2101046 | Discussions on PUCCH SCell Activation/Deactivation delay requirements | NTT DOCOMO, INC. | noted |  |  |
| R4-2101047 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | agreed |  |  |
| R4-2101048 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | agreed |  |  |
| R4-2101049 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | agreed |  |  |
| R4-2101050 | Remaining issues on RRM in R15 | MediaTek inc. | noted |  |  |
| R4-2101051 | CR on R15 remaining issues | MediaTek inc. | revised |  | R4-2103481 |
| R4-2101052 | CR on R15 remaining issues | MediaTek inc. | agreed |  |  |
| R4-2101053 | CR on R15 remaining issues | MediaTek inc. | agreed |  |  |
| R4-2101054 | CR on V2X interruption | Mediatek Inc., LG Electronics, Qualcomm Inc. | revised |  | R4-2103536 |
| R4-2101055 | CR on V2X interruption | Mediatek Inc., LG Electronics, Qualcomm Inc. | agreed |  |  |
| R4-2101056 | Remaining issues on congestion control test case | MediaTek inc. | noted |  |  |
| R4-2101057 | draftCR on congestion control test case | MediaTek inc. | revised |  | R4-2104045 |
| R4-2101058 | Remaining Issues on multiple SCell Activation | MediaTek inc. | noted |  |  |
| R4-2101059 | draftCR on multiple SCell activation test case | MediaTek inc. | revised |  | R4-2103626 |
| R4-2101060 | Discussion on multiple BWP switch test case | MediaTek inc. | noted |  |  |
| R4-2101061 | Work plan of R17 NR and MR-DC measurement gap enhancements WI | MediaTek Inc., Intel Corporation | noted |  |  |
| R4-2101062 | Pre-configured MG pattern(s) per configured BWP | MediaTek inc. | noted |  |  |
| R4-2101063 | Multiple concurrent and independent gap patterns | MediaTek inc. | noted |  |  |
| R4-2101064 | Network Controlled Small Gap | MediaTek inc. | noted |  |  |
| R4-2101065 | Discussion on NR V2X Demod test cases | MediaTek inc. | noted |  |  |
| R4-2101066 | Simulation results for NR V2X PSBCH | MediaTek inc. | noted |  |  |
| R4-2101067 | Simulation results for NR V2X PSCCH test case | MediaTek inc. | noted |  |  |
| R4-2101068 | Simulation results for NR V2X PSSCH test case | MediaTek inc. | noted |  |  |
| R4-2101069 | draftCR on NR V2X PSFCH demodulation requirements | MediaTek inc. | revised |  | R4-2103817 |
| R4-2101070 | draftCR on the CBW change test case for adding the UL CBW configuration | MediaTek inc. | revised |  | R4-2103627 |
| R4-2101071 | CR on SSB less SCell activation for FR1 for Rel-15 | NEC | merged |  |  |
| R4-2101072 | CR on SSB less SCell activation for FR1 for Rel-16 | NEC | merged |  |  |
| R4-2101073 | Draft CR on TC for direct SCell activation during handover in NR SA for FR2 | NEC | revised |  | R4-2103562 |
| R4-2101074 | Draft CR on TC for SCell dormancy in NR SA for FR1 | NEC | revised |  | R4-2103568 |
| R4-2101075 | CR for measurement period requirements correction | NEC | revised |  | R4-2103508 |
| R4-2101076 | CR for measurement period requirements correction | NEC | agreed |  |  |
| R4-2101077 | Discussion on FR2 inter-band DL CA enhancements | NEC | noted |  |  |
| R4-2101078 | Discussion on SRS antenna port switching | NEC | noted |  |  |
| R4-2101079 | Discussion on PSCell HO | NEC | noted |  |  |
| R4-2101080 | Discussion on PUCCH SCell activation | NEC | noted |  |  |
| R4-2101081 | Discussion on NR measurement gap enhancements requirements | NEC | noted |  |  |
| R4-2101082 | CR to TS 36.104: Additions of regional requirements for band 41 in Japan, Rel-15 | NEC, SoftBank, KDDI | revised |  | R4-2103775 |
| R4-2101083 | CR to TS 36.104: Additions of regional requirements for band 41 in Japan, Rel-16 | NEC, SoftBank, KDDI, Nokia | agreed |  |  |
| R4-2101084 | CR to TS 36.104: Additions of regional requirements for band 41 in Japan, Rel-17 | NEC, SoftBank, KDDI, Nokia | agreed |  |  |
| R4-2101085 | CR to TS 36.141: Additions of regional requirements for band 41 in Japan, Rel-15 | NEC, SoftBank, KDDI | revised |  | R4-2103776 |
| R4-2101086 | CR to TS 36.141: Additions of regional requirements for band 41 in Japan, Rel-16 | NEC, SoftBank, KDDI, Nokia | agreed |  |  |
| R4-2101087 | CR to TS 36.141: Additions of regional requirements for band 41 in Japan, Rel-17 | NEC, SoftBank, KDDI, Nokia | agreed |  |  |
| R4-2101088 | CR to TS 38.104: Additions of regional requirements for n41 in Japan, Rel-15 | NEC, SoftBank, KDDI | revised |  | R4-2103777 |
| R4-2101089 | CR to TS 38.104: Additions of regional requirements for n41 and n90 in Japan, Rel-16 | NEC, SoftBank, KDDI | revised |  | R4-2103958 |
| R4-2101090 | CR to TS 38.104: Additions of regional requirements for n41 and n90 in Japan, Rel-17 | NEC, SoftBank, KDDI, Nokia | agreed |  |  |
| R4-2101091 | CR to TS 38.141-1: Additions of regional requirements for n41 in Japan, Rel-15 | NEC, SoftBank, KDDI | revised |  | R4-2103778 |
| R4-2101092 | CR to TS 38.141-1: Additions of regional requirements for n41 and n90 in Japan, Rel-16 | NEC, SoftBank, KDDI | revised |  | R4-2103779 |
| R4-2101093 | CR to TS 38.141-1: Additions of regional requirements for n41 and n90 in Japan, Rel-17 | NEC, SoftBank, KDDI, Nokia | agreed |  |  |
| R4-2101094 | CR to TS 38.141-2: Additions of regional requirements for n41 in Japan, Rel-15 | NEC, SoftBank, KDDI | revised |  | R4-2103780 |
| R4-2101095 | CR to TS 38.141-2: Additions of regional requirements for n41 and n90 in Japan, Rel-16 | NEC, SoftBank, KDDI | revised |  | R4-2103781 |
| R4-2101096 | CR to TS 38.141-2: Additions of regional requirements for n41 and n90 in Japan, Rel-17 | NEC, SoftBank, KDDI, Nokia | agreed |  |  |
| R4-2101097 | TR 37.717-11-11 v0.3.0 Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) | CHTTL | not concluded |  |  |
| R4-2101098 | Revised WID: Rel-17 Dual Connectivity (DC) of 1 band LTE (1DL/1UL) and 1 NR band (1DL/1UL) | CHTTL | not concluded |  |  |
| R4-2101099 | Big CR for Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) | CHTTL | agreed |  |  |
| R4-2101100 | [CR] Add Random Access requirements under NR-U (Cat A) | ZTE Corporation | withdrawn |  |  |
| R4-2101101 | Discussion on SAR issue for HP UE inter-band UL CA | Xiaomi | noted |  |  |
| R4-2101102 | Discussion on SAR issue for NR PC2 SUL | Xiaomi | noted |  |  |
| R4-2101103 | Discussion on HP UE for TDD intra-band contiguous UL CA | Xiaomi | noted |  |  |
| R4-2101104 | Discussion on remaining issue on UL Tx switching enhancement in R17 | Xiaomi | noted |  |  |
| R4-2101105 | Coexistence study on NR to support non-terrestrial networks | Xiaomi | noted |  |  |
| R4-2101106 | CR for 38.101-1 Rel16 corrections on exception requirements on out-of-band blocking for inter-band CA | Xiaomi | revised |  | R4-2103347 |
| R4-2101107 | CR for 38.101-1 Rel17 corrections on exception requirements on out-of-band blocking for inter-band CA | Xiaomi | withdrawn |  |  |
| R4-2101108 | Discussion on Tx diversity open issues | Xiaomi | noted |  |  |
| R4-2101109 | MSD analysis on high power UE for CA_n41-n79 | Xiaomi, ZTE Corporation | approved |  |  |
| R4-2101110 | Discussion on HP UE for FDD bands | Xiaomi | noted |  |  |
| R4-2101111 | Discussion and reply draft LS on BCS reporting and support for intra-band EN-DC band combinations | Xiaomi | revised |  | R4-2102937 |
| R4-2101112 | Views on HO with PSCell | NTT DOCOMO, INC. | noted |  |  |
| R4-2101113 | TP for TR 38.717-03-01: support of CA_n1-n78-n257 | CHTTL | approved |  |  |
| R4-2101114 | Simulation For CQI reporting requirements for FR2 DL 256QAM | ZTE Corporation | noted |  |  |
| R4-2101115 | Discussion on NR demodulation performance in the scenario of LTE-NR coexist | ZTE Corporation | not treated |  |  |
| R4-2101116 | CR on demodulation performance requirements for DL 256QAM for FR2 | ZTE Corporation | revised |  | R4-2103834 |
| R4-2101117 | Discussion on SAR schemes for UE power class 2 NR inter-band CA with 2UL | China Telecom | noted |  |  |
| R4-2101118 | Discussion on SAR schemes for UE power class 2 NR SUL configurations | China Telecom | noted |  |  |
| R4-2101119 | draft CR to 38.101-1 Introduce SAR solution for UE power class 2 NR inter-band CA with 2UL | China Telecom | not pursued |  |  |
| R4-2101120 | draft CR to 38.101-1 Introduce SAR solution for UE power class 2 NR SUL configurations | China Telecom | not pursued |  |  |
| R4-2101121 | Discussion on release independence for UE power class 2 NR inter-band CA and SUL configurations | China Telecom | noted |  |  |
| R4-2101122 | CR to 38.307 Release independence for UE power class 2 NR inter-band CA and SUL configurations (R15) | China Telecom | not pursued |  |  |
| R4-2101123 | CR to 38.307 Release independence for UE power class 2 NR inter-band CA and SUL configurations (R16) | China Telecom | not pursued |  |  |
| R4-2101124 | CR to 38.307 Release independence for UE power class 2 NR inter-band CA and SUL configurations (R17) | China Telecom | revised |  | R4-2103173 |
| R4-2101125 | Draft TR 38.xxx v0.2.0: High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x =1,2) | China Telecom | not concluded |  |  |
| R4-2101126 | Revised WID: High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x =1,2) | China Telecom | endorsed |  |  |
| R4-2101127 | Work plan on downlink interruption for band combinations to conduct dynamic Tx switching | China Telecom | approved |  |  |
| R4-2101128 | Revised WID: Downlink interruption for NR and EN-DC band combinations to conduct dynamic Tx Switching in Uplink | China Telecom | endorsed |  |  |
| R4-2101129 | Discussion on the UL Gap | LG Electronics Finland | noted |  |  |
| R4-2101130 | SCell (de)activation requirement applicability when sCellDeactivationTimer is not configured | Nokia | noted |  |  |
| R4-2101131 | Random access requirements in NR-U | Nokia | noted |  |  |
| R4-2101132 | CR to 38.133 - Introducing NR-U random access requirements | Nokia | postponed |  |  |
| R4-2101133 | Discussion on NR-U RRM test configurations | Nokia | noted |  |  |
| R4-2101134 | NR-U RRM test case list updates | Nokia | noted |  |  |
| R4-2101135 | Draft TC E-UTRAN - NR-U Handover | Nokia | postponed |  |  |
| R4-2101136 | Draft TC RRC re-establishment with CCA | Nokia | postponed |  |  |
| R4-2101137 | Draft TC NR-U inter-frequency measurements | Nokia | postponed |  |  |
| R4-2101138 | On expected RRM impact for HST in FR2 | Nokia | noted |  |  |
| R4-2101139 | Discussion and simulation results for RLM/BFD measurement relaxation | Nokia | noted |  |  |
| R4-2101140 | On SCell RRM enhancement for NR high speed train scenario in FR1 | Ericsson | noted |  |  |
| R4-2101141 | On SCell RRM enhancement for NR high speed train scenario in FR1 | Ericsson | postponed |  |  |
| R4-2101142 | Overview of RRM requirements for NR high speed train scenario in FR2 | Ericsson | noted |  |  |
| R4-2101143 | Discussion on the reply to LS on BCS for intra-band EN-DC band combinations | MediaTek inc. | noted |  |  |
| R4-2101144 | Discussion on the reply to LS on single UL operation | MediaTek inc. | noted |  |  |
| R4-2101145 | Clarification on timing difference for Tx switching in EN-DC R16 | MediaTek inc. | revised |  | R4-2103353 |
| R4-2101146 | Clarification on timing difference for Tx switching in EN-DC R17 | MediaTek inc. | agreed |  |  |
| R4-2101147 | Discussion on interruption test cases for Tx switching | MediaTek inc. | noted |  |  |
| R4-2101148 | Update on interruption test cases for Tx switching R16 | MediaTek inc. | revised |  | R4-2103600 |
| R4-2101149 | Update on interruption test cases for Tx switching R17 | MediaTek inc. | agreed |  |  |
| R4-2101150 | Maintenance CR for CSI-RS based L3 measurement requirements R16 | MediaTek inc. | revised |  | R4-2103633 |
| R4-2101151 | Maintenance CR for CSI-RS based L3 measurement requirements R17 | MediaTek inc. | agreed |  |  |
| R4-2101152 | CSI-RSRP measurement accuracy requirement | MediaTek inc. | noted |  |  |
| R4-2101153 | CSI-SINR measurement accuracy requirement | MediaTek inc. | noted |  |  |
| R4-2101154 | CR to Update timing offset in test case for CSI-SINR in SA FR2 | MediaTek inc. | postponed |  | R4-2103647 |
| R4-2101155 | Discussion on UE feature list | MediaTek inc. | noted |  |  |
| R4-2101156 | General views on NR repeater | MediaTek inc. | noted |  |  |
| R4-2101157 | Discussion reply LS on Beam switching gaps for Multi-TRP UL transmission | MediaTek inc. | noted |  |  |
| R4-2101158 | MSD evaluation for CA 3DL2UL n1-n77-n79 for TR 38.717-03-02 | Qualcomm Incorporated | noted |  |  |
| R4-2101159 | REFSENS evaluation of n8 and n71 for 35MHz channel bandwidth | Mediatek India Technology Pvt. | noted |  |  |
| R4-2101160 | HPUE TDD+TDD MPR and AMPR | Qualcomm Incorporated | noted |  |  |
| R4-2101161 | Correction on the power of the first preamble for random access in EN-DC and SA in R15 | MediaTek inc. | revised |  | R4-2103487 |
| R4-2101162 | Correction on the power of the first preamble for random access in EN-DC and SA in R16 | MediaTek inc. | agreed |  |  |
| R4-2101163 | Correction on the power of the first preamble for random access in EN-DC and SA in R17 | MediaTek inc. | agreed |  |  |
| R4-2101164 | Correction on the time for Scell activation and CSI-report in R15 | MediaTek inc. | revised |  | R4-2103488 |
| R4-2101165 | Correction on the time for Scell activation and CSI-report in R16 | MediaTek inc. | agreed |  |  |
| R4-2101166 | Correction on the time for Scell activation and CSI-report in R17 | MediaTek inc. | agreed |  |  |
| R4-2101167 | Correction on the Noc level in TS38.133 in R15 | MediaTek inc. | revised |  | R4-2103489 |
| R4-2101168 | Correction on the Noc level in TS38.133 in R16 | MediaTek inc. | agreed |  |  |
| R4-2101169 | Correction on the Noc level in TS38.133 in R17 | MediaTek inc. | agreed |  |  |
| R4-2101170 | Correction on the SS-RSRP difference value for SS-RSRP measurement TC in R15 | MediaTek inc. | postponed |  | R4-2103490 |
| R4-2101171 | Correction on the SS-RSRP difference value for SS-RSRP measurement TC in R16 | MediaTek inc. | withdrawn |  |  |
| R4-2101172 | Correction on the SS-RSRP difference value for SS-RSRP measurement TC in R17 | MediaTek inc. | withdrawn |  |  |
| R4-2101173 | Discussion on high speed train for CA in FR1 | MediaTek inc. | noted |  |  |
| R4-2101174 | IBE mask for almost contiguous allocations | Qualcomm Incorporated | noted |  |  |
| R4-2101175 | n40-n41 Coexistence | Qualcomm Incorporated | noted |  |  |
| R4-2101176 | CR to TS 38.101-3 clarification on the single uplink allowance for DC_3A_n3A | CHTTL | revised |  | R4-2103163 |
| R4-2101177 | 35MHz 45MHz AMPR, MPR, REFSENS for n8, n71, and n25. | Qualcomm Incorporated | noted |  |  |
| R4-2101178 | Intra-band contiguous ULCA Pcmax Issues | Qualcomm Incorporated | noted |  |  |
| R4-2101179 | CR to TS 38.101-3 clarification on the single uplink allowance for DC_3A_n3A | CHTTL | agreed |  |  |
| R4-2101180 | CR to TR 38.820: Correction of antenna model in subclause 7.2.4 | Ericsson | revised |  | R4-2103786 |
| R4-2101181 | TP to TR 38.808: Addition of a set of phase noise models in subclause 4.2.3 | Ericsson | revised |  | R4-2103261 |
| R4-2101182 | TP to TR 38.921: Addition of in-door antenna parameters and correction to model in subclause 8.1 | Ericsson | revised |  | R4-2103109 |
| R4-2101183 | TP to TR 38.808: Addition of technical background for BS in clause 2 and subclause 4.2.5 | Ericsson | noted |  |  |
| R4-2101184 | Discussion on UE capability for improved PC2 MSD for EN-DC | CHTTL | noted |  |  |
| R4-2101185 | Comments on remaining issues of CSI-RS based L3 measurement core requirements | Qualcomm CDMA Technologies | noted |  |  |
| R4-2101186 | TP for DC_42A_n1A-n77A-n79A for TR37.717-11-31 | NTT DOCOMO INC. | approved |  |  |
| R4-2101187 | TP for DC_42A_n1A-n78A-n79A for TR37.717-11-31 | NTT DOCOMO INC. | approved |  |  |
| R4-2101188 | TP for CA_n28-n77-n79 for TR 38.717-03-01 | NTT DOCOMO INC. | approved |  |  |
| R4-2101189 | TP for CA_n28-n78-n79 for TR 38.717-03-01 | NTT DOCOMO INC. | approved |  |  |
| R4-2101190 | TP for DC_1-21_n28-n77 for TR 37.717-11-21 | NTT DOCOMO INC. | approved |  |  |
| R4-2101191 | TP for DC_1-21_n28-n78 for TR 37.717-11-21 | NTT DOCOMO INC. | approved |  |  |
| R4-2101192 | TP for DC_1-21_n28-n79 for TR 37.717-11-21 | NTT DOCOMO INC. | approved |  |  |
| R4-2101193 | TP for DC_1-3_n28-n79 for TR 37.717-11-21 | NTT DOCOMO INC. | approved |  |  |
| R4-2101194 | TP for DC_3-21_n28-n77 for TR 37.717-11-21 | NTT DOCOMO INC. | approved |  |  |
| R4-2101195 | TP for DC_3-21_n28-n78 for TR 37.717-11-21 | NTT DOCOMO INC. | approved |  |  |
| R4-2101196 | TP for DC_3-21_n28-n79 for TR 37.717-11-21 | NTT DOCOMO INC. | approved |  |  |
| R4-2101197 | Addition of UE co-existence requirements for 40 and n40 | NTT DOCOMO, INC., SoftBank Corp., KDDI Corporation, Rakuten Mobile, Inc | revised |  | R4-2103134 |
| R4-2101198 | draft CR to TS 38.101-1 Modification of MSD values for n1-n77-n79 and n1-n78-n79 | NTT DOCOMO INC. | endorsed |  |  |
| R4-2101199 | Band specific requirements for FR2 DL Inter-band CA of n257n259 with IBM | NTT DOCOMO INC. | noted |  |  |
| R4-2101200 | Consideration on FR2 UL gap for self calibration and monitering | NTT DOCOMO INC. | noted |  |  |
| R4-2101201 | Further discussion on EESS protection | NTT DOCOMO INC. | noted |  |  |
| R4-2101202 | UE requirements for FR2 UL Inter-band CA | NTT DOCOMO INC. | noted |  |  |
| R4-2101203 | On the performance requirements of CSI-RS based L3 measurement | Qualcomm CDMA Technologies | noted |  |  |
| R4-2101204 | TDD UL-DL and DL-UL switching in DAPS handover | Ericsson | noted |  |  |
| R4-2101205 | CR on TS38.133 for dual active protocol stack handover | Ericsson | merged |  |  |
| R4-2101206 | CR on TS38.133 for dual active protocol stack handover | Ericsson | withdrawn |  |  |
| R4-2101207 | TDD UL-DL and DL-UL switching in LTE DAPS handover | Ericsson | noted |  |  |
| R4-2101208 | Correction on the synchronous condition for DAPS handover | Ericsson | postponed |  |  |
| R4-2101209 | Correction on the synchronous condition for DAPS handover | Ericsson | withdrawn |  |  |
| R4-2101210 | CR on TS38.133 for Pcell change | MediaTek inc. | revised |  | R4-2104050 |
| R4-2101211 | CR on TS38.133 for Pcell change | MediaTek inc. | agreed |  |  |
| R4-2101212 | Discussion on direct Scell activation and dormancy Scell | MediaTek inc. | noted |  |  |
| R4-2101213 | CR on TS38.133 for direct Scell activation | MediaTek inc. | postponed |  |  |
| R4-2101214 | CR on TS38.133 for direct Scell activation | MediaTek inc. | withdrawn |  |  |
| R4-2101215 | CR on TS38.133 for direct SCell activation of SCell in FR2 intra-band in ENDC mode (A.5.5.X) | MediaTek inc. | revised |  | R4-2103558 |
| R4-2101216 | CR on TS38.133 for direct SCell activation of SCell in FR2 intra-band in ENDC mode (A.5.5.X) | MediaTek inc. | withdrawn |  |  |
| R4-2101217 | CR on TS38.133 for E-UTRAN | MediaTek inc. | revised |  | R4-2103565 |
| R4-2101218 | CR on TS38.133 for E-UTRAN | MediaTek inc. | withdrawn |  |  |
| R4-2101219 | CR on TS38.133 for inter-frequency measurement requirement without gap | MediaTek inc. | revised |  | R4-2103624 |
| R4-2101220 | CR on TS38.133 for inter-frequency measurement requirement without gap | MediaTek inc. | agreed |  |  |
| R4-2101221 | Work plan of Rel-17 Power Saving Enhancements | MediaTek inc. | revised |  | R4-2103669 |
| R4-2101222 | Evaluation on Rel-17 RLM/BFD measurement relaxation | MediaTek inc. | noted |  |  |
| R4-2101223 | Discussion on SRS antenna port switching | MediaTek inc. | noted |  |  |
| R4-2101224 | TP for DC_21_n28 for TR 37.717-11-11 | NTT DOCOMO INC. | revised |  | R4-2103021 |
| R4-2101225 | TP for DC_1_n28-n79 for TR 37.717-11-21 | NTT DOCOMO INC., MediaTek Inc., LG Electronics | approved |  |  |
| R4-2101226 | TP for DC_3_n28-n79 for TR 37.717-11-21 | NTT DOCOMO INC., MediaTek Inc., LG Electronics | approved |  |  |
| R4-2101227 | TP for DC_21_n28-n77 for TR 37.717-11-21 | NTT DOCOMO INC. , MediaTek Inc., LG Electronics | approved |  |  |
| R4-2101228 | TP for DC_21_n28-n78 for TR 37.717-11-21 | NTT DOCOMO INC. , MediaTek Inc., LG Electronics | approved |  |  |
| R4-2101229 | TP for DC_21_n28-n79 for TR 37.717-11-21 | NTT DOCOMO INC. , MediaTek Inc., LG Electronics | revised |  | R4-2103063 |
| R4-2101230 | TP for DC_1-21_n28 for TR 37.717-21-11 | NTT DOCOMO INC. , MediaTek Inc., LG Electronics | revised |  | R4-2103022 |
| R4-2101231 | TP for DC_3-21_n28 for TR 37.717-21-11 | NTT DOCOMO INC. | approved |  |  |
| R4-2101232 | Discussion on NR V2X Single Link PSSCH requirements | Intel Corporation | noted |  |  |
| R4-2101233 | Discussion on NR V2X Single Link PSCCH requirements | Intel Corporation | noted |  |  |
| R4-2101234 | Draft CR on NR V2X Single Link PSCCH requirements | Intel Corporation | revised |  | R4-2103816 |
| R4-2101235 | Simulation results for NR V2X Single Link PSBCH requirements | Intel Corporation | noted |  |  |
| R4-2101236 | Simulation results for NR V2X Single Link PSFCH requirements | Intel Corporation | noted |  |  |
| R4-2101237 | Discussion on NR V2X Multiple Link Power Imbalance requirements | Intel Corporation | noted |  |  |
| R4-2101238 | Discussion on NR V2X Multiple Link HARQ soft buffer combing requirements | Intel Corporation | noted |  |  |
| R4-2101239 | Discussion on NR V2X Multiple Link PSFCH decoding capability requirements | Intel Corporation | noted |  |  |
| R4-2101240 | Discussion on NR V2X Multiple Link PSCCH/PSSCH decoding capability requirements | Intel Corporation | noted |  |  |
| R4-2101241 | Discussion on PDCCH-WUS requirements | Intel Corporation | noted |  |  |
| R4-2101242 | Discussion on URLLC Ultra-low BLER CQI requirements | Intel Corporation | noted |  |  |
| R4-2101243 | Discussion on URLLC UE demodulation requirements | Intel Corporation | noted |  |  |
| R4-2101244 | Simulation results for URLLC UE demodulation requirements | Intel Corporation | noted |  |  |
| R4-2101245 | CR on FRC for URLLC UE Higher BLER requirements | Intel Corporation | revised |  | R4-2103901 |
| R4-2101246 | Discussion on URLLC BS demodulation requirements | Intel Corporation | noted |  |  |
| R4-2101247 | Simulation results for URLLC BS demodulation requirements | Intel Corporation | noted |  |  |
| R4-2101248 | CR on FR2 requirements for PUSCH mapping Type B with low number of symbols (Rel-16) | Intel Corporation | revised |  | R4-2103909 |
| R4-2101249 | CR on FR2 requirements for PUSCH mapping Type B with low number of symbols (Rel-17) | Intel Corporation | agreed |  |  |
| R4-2101250 | Discussion on FR2 DL 256QAM UE demodulation requirements | Intel Corporation | noted |  |  |
| R4-2101251 | Summary of simulation results for FR2 DL 256QAM demodulation requirements | Intel Corporation | noted |  |  |
| R4-2101252 | CR on simplified TDL-D channel model for FR2 DL 256QAM demodulation requirements | Intel Corporation | revised |  | R4-2103835 |
| R4-2101253 | CR on applicability rules and FRC for FR2 DL 256QAM CQI requirements | Intel Corporation | revised |  | R4-2103838 |
| R4-2101254 | Discussion on NR CA UE demodulation requirements | Intel Corporation | noted |  |  |
| R4-2101255 | CR on applicability rules for Normal NR CA requirements | Intel Corporation | revised |  | R4-2103841 |
| R4-2101256 | Simulation results for single-DCI based multi-TRP SDM Tx scheme | Intel Corporation | noted |  |  |
| R4-2101257 | Simulation results for multi-DCI based multi-TRP Tx scheme | Intel Corporation | noted |  |  |
| R4-2101258 | Simulation results for single-DCI based multi-TRP Repetition Tx schemes | Intel Corporation | noted |  |  |
| R4-2101259 | CR to TS 38.101-4: Performance requirements single-DCI based multi-TRP Repetition Tx schemes | Intel Corporation, Samsung, Ericsson, Huawei, HiSilicon | revised |  | R4-2103827 |
| R4-2101260 | Views on NR HST CA PDSCH performance requirements | Intel Corporation | noted |  |  |
| R4-2101261 | Views on NR HST PDSCH performance requirements for multi-DCI based Tx scheme | Intel Corporation | noted |  |  |
| R4-2101262 | Views on NR IAB-DU demodulation performance requirements | Intel Corporation | noted |  |  |
| R4-2101263 | Views on NR IAB-MT demodulation performance requirements | Intel Corporation | noted |  |  |
| R4-2101264 | Discussion on NR-U PDSCH requirements | Intel Corporation | noted |  |  |
| R4-2101265 | Discussion on NR-U PRACH requirements | Intel Corporation | noted |  |  |
| R4-2101266 | RRM requirements for inter-band DL CA in NR FR2 | Intel Corporation | noted |  |  |
| R4-2101267 | Discussion on deployment scenarios for NR HST in FR2 | Intel Corporation | noted |  |  |
| R4-2101268 | RRM requirements for NR HST in FR2 | Intel Corporation | noted |  |  |
| R4-2101269 | Discussion on pre-configured measurement gap | Intel Corporation | noted |  |  |
| R4-2101270 | Discussion on multiple and independent concurrent measurement gaps in NR | Intel Corporation | noted |  |  |
| R4-2101271 | Discussion on NCSG in NR | Intel Corporation | noted |  |  |
| R4-2101272 | Discussion on NR PRS RSTD measurement report requirements | Intel Corporation | noted |  |  |
| R4-2101273 | Discussion on NR UE RX-TX time difference measurement requirements | Intel Corporation | noted |  |  |
| R4-2101274 | Discussion on NR PRS RSTD measurement accuracy requirements | Intel Corporation | noted |  |  |
| R4-2101275 | Discussion on NR PRS RSRP measurement accuracy requirements | Intel Corporation | noted |  |  |
| R4-2101276 | Discussion on NR UE RX-TX time difference measurement accuracy requirements | Intel Corporation | noted |  |  |
| R4-2101277 | Discussion on NR Positioning test cases configuration | Intel Corporation | noted |  |  |
| R4-2101278 | [draftCR] CR for PRS configurations for NR Pos RRM tests | Intel Corporation | revised |  | R4-2103735 |
| R4-2101279 | [draftCR] CR for the test case of RSTD measurement requirements reporting in SA | Intel Corporation | revised |  | R4-2103736 |
| R4-2101280 | On improved transient period for NR 52.6 - 71 GHz | Intel Corporation | noted |  |  |
| R4-2101281 | On numerology and channel bandwidth in 52.6 | Intel Corporation | noted |  |  |
| R4-2101282 | Beam correspondence requirements for FWA UE devices | Intel Corporation | noted |  |  |
| R4-2101283 | Consideration on inter-band UL CA RF requirements | Intel Corporation | noted |  |  |
| R4-2101284 | Frequency separation class consideration for inter-band CA based on CBM | Intel Corporation | noted |  |  |
| R4-2101285 | CR to 38.101-1 (Rel-16) fall back behaviors | Intel Corporation | not pursued |  |  |
| R4-2101286 | CR to 38.101-1 (Rel-17) fall back behaviors | Intel Corporation | withdrawn |  |  |
| R4-2101287 | CR to 38.101-2 (Rel-16) SSB based beam correspondence side condition | Intel Corporation | not pursued |  |  |
| R4-2101288 | CR to 38.101-2 (Rel-17) SSB based beam correspondence side condition | Intel Corporation | withdrawn |  |  |
| R4-2101289 | Remaining Issues on Transparent TxD | Intel Corporation | noted |  |  |
| R4-2101290 | Revised WID for V2X band combination | CATT | not concluded |  |  |
| R4-2101291 | Big CR: Introduction of Rel-16 CSI-RS based L3 measurement RRM performance requirements | CATT | endorsed |  |  |
| R4-2101292 | Big CR: Introduction of Rel-16 CSI-RS based L3 measurement RRM test cases | CATT | withdrawn |  |  |
| R4-2101293 | Discussion on NR IAB DU demodulation performance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101294 | Discussion on NR IAB MT demodulation performance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101295 | Discussion and simulation results on the performance requirements for NR power saving | Huawei, HiSilicon | noted |  |  |
| R4-2101296 | Simulation results on PDSCH requirements for NR DL 256QAM for FR2 | Huawei, HiSilicon | noted |  |  |
| R4-2101297 | CR on applicability and FRC for PDSCH normal demodulation for DL 256QAM for FR2 | Huawei, HiSilicon | agreed |  |  |
| R4-2101298 | Simulation results on CQI requirements for NR DL 256QAM for FR2 | Huawei, HiSilicon | noted |  |  |
| R4-2101299 | CR on SDR requirements for DL 256QAM for FR2 | Huawei, HiSilicon | revised |  | R4-2103836 |
| R4-2101300 | CR on update TRS and CSI-RS transmission for HST DPS requirements | Huawei, HiSilicon, Ericsson, Intel Corporation | agreed |  |  |
| R4-2101301 | Summary of ideal and impairment results for NR HST demodulation requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101302 | CR on correction 2-step RACH performance requirements for FR2 in 38.104 (Rel-16) | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell | revised |  | R4-2103959 |
| R4-2101303 | CR on correction 2-step RACH performance requirements for FR2 in 38.104 (Rel-17) | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2101304 | CR on update applicability rule for 2-step RACH in 38.141-1 (Rel-16) | Huawei, HiSilicon | revised |  | R4-2103871 |
| R4-2101305 | CR on update applicability rule for 2-step RACH in 38.141-1 (Rel-17) | Huawei, HiSilicon | agreed |  |  |
| R4-2101306 | CR on update applicability rule for 2-step RACH in 38.141-2 (Rel-16) | Huawei, HiSilicon | revised |  | R4-2103872 |
| R4-2101307 | CR on update applicability rule for 2-step RACH in 38.141-2 (Rel-17) | Huawei, HiSilicon | agreed |  |  |
| R4-2101308 | Discussion on PDSCH CA scenarios for NR UE HST FR1 performance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101309 | Discussion on enhanced transmission schemes for NR HST demodulation | Huawei, HiSilicon | noted |  |  |
| R4-2101310 | Discussion on the test metric of eType II codebook based PMI reporting test | Huawei, HiSilicon | noted |  |  |
| R4-2101311 | Simulation results for eType II codebook based PMI reporting test | Huawei, HiSilicon | revised |  | R4-2103963 |
| R4-2101312 | Simulation results of PDSCH requirements for Multi-DCI transmission scheme | Huawei, HiSilicon | noted |  |  |
| R4-2101313 | Simulation results of PDSCH requirements for Single-DCI SDM scheme | Huawei, HiSilicon | revised |  | R4-2103960 |
| R4-2101314 | Simulation results of PDSCH requirements for Single-DCI URLLC schemes | Huawei, HiSilicon | noted |  |  |
| R4-2101315 | CR for 38.101-4 Introduction of PDSCH requirement with Single-DCI based SDM scheme | Huawei, HiSilicon | revised |  | R4-2103828 |
| R4-2101316 | CR for 38.101-4 Introduction of PDSCH requirement with Multi-DCI based multi-TRP transmission schemes | Huawei, HiSilicon | revised |  | R4-2103829 |
| R4-2101317 | Simulation results for Rel-15 Type II codebook PMI reporting test | Huawei, HiSilicon | noted |  |  |
| R4-2101318 | Discussion on the test point for Rel-15 Type II codebook PMI reporting test with larger Tx ports | Huawei, HiSilicon | noted |  |  |
| R4-2101319 | CR for 38.104 Cleanup of performance requirements for NR HST PRACH under fading channel (Rel-16) | Huawei, HiSilicon | revised |  | R4-2103865 |
| R4-2101320 | CR for 38.141-1 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-16) | Huawei, HiSilicon | revised |  | R4-2103866 |
| R4-2101321 | CR for 38.141-2 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-16) | Huawei, HiSilicon | revised |  | R4-2103867 |
| R4-2101322 | CR for 38.101-4 Applicablity of PMI reporting test with Tx ports larger than 8 and up to 32 | Huawei, HiSilicon | revised |  | R4-2103842 |
| R4-2101323 | CR for 38.101-4 Applicablity of PMI reporting test of eType II codebook | Huawei, HiSilicon | revised |  | R4-2103954 |
| R4-2101324 | CR for 38.104 Cleanup of performance requirements for NR HST PRACH under fading channel (Rel-17) | Huawei, HiSilicon | agreed |  |  |
| R4-2101325 | CR for 38.141-1 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-17) | Huawei, HiSilicon | agreed |  |  |
| R4-2101326 | CR for 38.141-2 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-17) | Huawei, HiSilicon | agreed |  |  |
| R4-2101327 | Discussion on CSI requireements with ultra low-BLER | Huawei, HiSilicon | noted |  |  |
| R4-2101328 | Discussion on URLLC UE demodulation requirements with higher BLER and low latency | Huawei, HiSilicon | noted |  |  |
| R4-2101329 | Discussion on URLLC BS demodulation requirements with higher BLER and low latency | Huawei, HiSilicon | noted |  |  |
| R4-2101330 | Simulation results on UE PDSCH demodulation reuqirements with higher BLER and low latency | Huawei, HiSilicon | noted |  |  |
| R4-2101331 | Simulation for CQI reporting test | Huawei, HiSilicon | noted |  |  |
| R4-2101332 | Simulation results on FR2 PUSCH demodulation reuqirements with higher BLER and low latency | Huawei, HiSilicon | noted |  |  |
| R4-2101333 | Summary of simulation results for UE URLLC demodulation performance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101334 | Summary of simulation results for BS URLLC demodulation performance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101335 | CR to TS38.141-2 Correction of BS conformance testing for FR2 URLLC PUSCH repetition Type A (Rel-16) | Huawei, HiSilicon | revised |  | R4-2103910 |
| R4-2101336 | CR to TS38.141-2 Correction of BS conformance testing for FR2 URLLC PUSCH repetition Type A (Rel-17) | Huawei, HiSilicon | agreed |  |  |
| R4-2101337 | CR to TS 38.101-4 Correction of UE performance requirements for FR1 URLLC PDSCH repetitions over multiple slots. | Huawei, HiSilicon | revised |  | R4-2103902 |
| R4-2101338 | CR to TS38.101-4 Applicability rules for URLLC CSI requirements | Huawei, HiSilicon | revised |  | R4-2103892 |
| R4-2101339 | CR to TS38.141-1 Correction of BS conformance testing for URLLC demodulation requirements with higher BLER (Rel-16) | Huawei, HiSilicon | revised |  | R4-2103911 |
| R4-2101340 | CR to TS38.141-1 Correction of BS conformance testing for URLLC demodulation requirements with higher BLER (Rel-17) | Huawei, HiSilicon | agreed |  |  |
| R4-2101341 | CR to TS38.104 Correction of BS performance requirements for URLLC FR1 PUSCH repetition Type A (Rel-16) | Huawei, HiSilicon | revised |  | R4-2103912 |
| R4-2101342 | CR to TS38.104 Correction of BS performance requirements for URLLC FR1 PUSCH repetition Type A (Rel-17) | Huawei, HiSilicon | agreed |  |  |
| R4-2101343 | Discussion on LBT transmission burst model | Huawei, HiSilicon | noted |  |  |
| R4-2101344 | Discussion on NRU UE performance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101345 | Discussion on NRU CSI requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101346 | Simualtion results on NRU PUSCH performance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101347 | Discussion on NRU PUSCH performance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101348 | Simualtion results on NRU PUCCH performance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101349 | Discussion on NRU PUCCH performance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101350 | Simualtion results on NRU PRACH performance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101351 | Discussion on NRU PRACH performance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101352 | Discussion on PSSCH requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101353 | Simulation results on PSSCH requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101354 | Discussion and simulation results on PSCCH performance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101355 | Simulation results on PSBCH performance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101356 | Simulation results on PSFCH performance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101357 | Draft CR: Introduction on Power imbalance requirements for NR V2X | Huawei, HiSilicon | revised |  | R4-2103818 |
| R4-2101358 | Discussion on V2X Power imbalance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101359 | Discussion on V2X Soft-buffer test | Huawei, HiSilicon | noted |  |  |
| R4-2101360 | Draft CR: Introduction on PSFCH decoding capability requirements for NR V2X | Huawei, HiSilicon | revised |  | R4-2103981 |
| R4-2101361 | Discussion on V2X PSFCH decoding capability test | Huawei, HiSilicon | noted |  |  |
| R4-2101362 | Draft CR: Introduction on PSSCH/PSCCH decoding capability requirements for NR V2X | Huawei, HiSilicon | revised |  | R4-2103982 |
| R4-2101363 | Discussion on V2X PSSCH/PSCCH decoding capability test | Huawei, HiSilicon | noted |  |  |
| R4-2101364 | Discussion on V2X multilink SDR test | Huawei, HiSilicon | noted |  |  |
| R4-2101365 | Discussion on NR normal CA performance requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101366 | Discussion on test applicability rule for UE power imbalance for EN-DC | Huawei, HiSilicon | noted |  |  |
| R4-2101367 | CR: Updates to power imbalance for CA | Huawei, HiSilicon | agreed |  |  |
| R4-2101368 | Discussion on deployment scenarios for FR2 HST | Huawei, HiSilicon | noted |  |  |
| R4-2101369 | Views on 256QAM UE requirements for FR2 | NTT DOCOMO, INC. | noted |  |  |
| R4-2101370 | Views on HST CA tests for FR1 | NTT DOCOMO, INC. | noted |  |  |
| R4-2101371 | The signalling for BCS4 | Xiaomi | noted |  |  |
| R4-2101372 | Discussion on frequency arrangement for extended 600MHz NR Band | Xiaomi | noted |  |  |
| R4-2101373 | Frequency separation class for inter-band CA within the same frequency group based on CBM | Xiaomi | noted |  |  |
| R4-2101374 | The MOP and Tx requirements for inter-band UL CA in FR2 | Xiaomi | noted |  |  |
| R4-2101375 | The IBM UE capability for inter-band CA within the same frequency group | Xiaomi | noted |  |  |
| R4-2101376 | The CBM UE capability for inter-band CA between different frequency groups | Xiaomi | noted |  |  |
| R4-2101377 | Remaining issues for Scell dormancy RRM requirements | vivo | noted |  |  |
| R4-2101378 | Further considerations on test cases for BWP switch over multiple CCs | vivo | noted |  |  |
| R4-2101379 | Considerations on SRS antenna port switching | vivo | noted |  |  |
| R4-2101380 | Considerations on PUCCH SCell activation and deactivation | vivo | noted |  |  |
| R4-2101381 | Considerations on pre-configured MG patterns | vivo | noted |  |  |
| R4-2101382 | Considerations on network controlled small gap | vivo | noted |  |  |
| R4-2101383 | CR for removing K2 for R16 UE power saving | vivo | postponed |  |  |
| R4-2101384 | CR for removing K2 for R16 UE power saving | vivo | postponed |  |  |
| R4-2101385 | CR for modifications on FR1 intra-frequency UE power saving test cases | vivo | merged |  |  |
| R4-2101386 | CR for modifications on FR1 intra-frequency UE power saving test cases | vivo | postponed |  |  |
| R4-2101387 | CR on Fixed reference channel for power saving performance | vivo | revised |  | R4-2103824 |
| R4-2101388 | CR for adding capability D | vivo | not pursued |  |  |
| R4-2101389 | CR for adding capability D | vivo | not pursued |  |  |
| R4-2101390 | CR for test cases for simultaneously DCI/timer based bwp switch over mulitple CCs on FR1 in SA | vivo | endorsed |  | R4-2103604 |
| R4-2101391 | Discussion on the activation and deactivation delay requirements for PUCCH SCell | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101392 | Discussion on the interruption requirements at SRS antenna port switching | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101393 | Open issues on the CSI-RS based measurement requirements | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101394 | 38.133 CR on the CSI-RS based measurement requirements | Nokia, Nokia Shanghai Bell | revised |  | R4-2103634 |
| R4-2101395 | Discussion on the performance of CSI-RS based measurements | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101396 | 38.133 draftCR on the CSI-RSRP accuracy requirements | Nokia, Nokia Shanghai Bell | revised |  | R4-2103637 |
| R4-2101397 | Simulation results for CSI-RS based measurements | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101398 | TP for TR 36.717-03-01: CA_1-8-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101399 | TP for TR 36.717-03-01: CA_1-28-32 | VODAFONE Group Plc | revised |  | R4-2103096 |
| R4-2101400 | TP for TR 36.717-03-01: CA_7-8-28 | VODAFONE Group Plc | approved |  |  |
| R4-2101401 | TP for TR 36.717-03-01: CA_7-8-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101402 | TP for TR 36.717-03-01: CA_7-28-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101403 | TP for TR 36.717-03-01: CA_8-20-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101404 | TP for TR 36.717-03-01: CA_8-28-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101405 | TP for TR 36.717-03-01: CA_20-28-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101406 | TP for TR 36.717-04-01: CA_1A-3A-7C-20A with UL CA_7C | VODAFONE Group Plc | approved |  |  |
| R4-2101407 | Discussion on RRC based BWP switching | Intel Corporation | postponed |  |  |
| R4-2101408 | Discussion on RRC based BWP switching on multiple CC | Intel Corporation | noted |  |  |
| R4-2101409 | CR on RRC based BWP switching on multiple CCs (Rel-16) | Intel Corporation | not pursued |  |  |
| R4-2101410 | CR on RRC based BWP switching on multiple CCs (Rel-17) | Intel Corporation | withdrawn |  |  |
| R4-2101411 | Discussion on test case design for BWP switching on multiple CCs | Intel Corporation | noted |  |  |
| R4-2101412 | Draft CR on DCI-based and Timer-based simultaneous Active BWP Switch on multiple CCs on FR1 in EN-DC (section 4.5.6.x) | Intel Corporation | endorsed |  |  |
| R4-2101413 | Discussion on CSI-RS L3 measurement | Intel Corporation | noted |  |  |
| R4-2101414 | Discussion on SRS antenna port switching | Intel Corporation | noted |  |  |
| R4-2101415 | Discussion on HO with PSCell | Intel Corporation | noted |  |  |
| R4-2101416 | Big CR: Introduction of Rel-16 NR RRM enhancements WI performance requirements and test cases (Rel-16) | Intel Corporation, ZTE Corporation, Apple | revised |  | R4-2103608 |
| R4-2101417 | Big CR: Introduction of Rel-16 NR RRM enhancements WI performance requirements and test cases (Rel-17) | Intel Corporation, ZTE Corporation, Apple | agreed |  |  |
| R4-2101418 | Testing of IAB-MT Requirements | Qualcomm Incorporated | noted |  |  |
| R4-2101419 | Simulation results of PDSCH with 256QAM in FR2 | Ericsson | noted |  |  |
| R4-2101420 | Open issues on FR2 256QAM PDSCH demodulation requirements | Ericsson | noted |  |  |
| R4-2101421 | Simulation results of CQI table 2 reporting in FR2 | Ericsson | noted |  |  |
| R4-2101422 | Open issues on CQI table 2 reporting test in FR2 | Ericsson | noted |  |  |
| R4-2101423 | Introduction of Noc for PC5 in n257/n258 | Ericsson, SoftBank | agreed |  |  |
| R4-2101424 | Remaining open issues on connection mobility control in NR-U | Ericsson | noted |  |  |
| R4-2101425 | CR: Introduction of random access requirements with CCA | Ericsson | revised |  | R4-2103721 |
| R4-2101426 | CR: Introduction of random access requirements with CCA | Ericsson | agreed |  |  |
| R4-2101427 | Remaining open issues on beam management in NR-U | Ericsson | noted |  |  |
| R4-2101428 | CR: Beam management requirements with CCA | Ericsson | revised |  | R4-2103513 |
| R4-2101429 | CR: Beam management requirements with CCA | Ericsson | agreed |  |  |
| R4-2101430 | Common test parameters for NR-U RRM tests | Ericsson | noted |  |  |
| R4-2101431 | Draft CR: RMC for NR-U test cases | Ericsson | revised |  | R4-2103521 |
| R4-2101432 | Test cases on link recovery and L1-RSRP reporting in NR-U | Ericsson | noted |  |  |
| R4-2101433 | Draft CR: test cases for beam management in NR-U | Ericsson | revised |  | R4-2103524 |
| R4-2101434 | Draft CR: Section numbering for PDSCH CA demodulation requirements | Ericsson | not pursued |  |  |
| R4-2101435 | Simulation results for Rel-15 Type II PMI reporting test | Ericsson | noted |  |  |
| R4-2101436 | Evaluation of Rel-15 Type-II PMI reporting test | Ericsson | noted |  |  |
| R4-2101437 | Correction of title on 16Tx port subband PMI reporting | Ericsson | agreed |  |  |
| R4-2101438 | Simulation results for HST-DPS | Ericsson | noted |  |  |
| R4-2101439 | PDSCH demodulation requirements for CA with HST-SFN scenario | Ericsson | noted |  |  |
| R4-2101440 | PDSCH demodulation requirements with enhanced transmission schemes in HST scenario | Ericsson | noted |  |  |
| R4-2101441 | Correction of CQI test parameters and FRC for UE demodulation test | Ericsson | revised |  | R4-2103796 |
| R4-2101442 | Correction of CQI test parameters and FRC for UE demodulation test | Ericsson | agreed |  |  |
| R4-2101443 | Simulation results for Rel-16 eType II PMI reporting test | Ericsson | noted |  |  |
| R4-2101444 | Evaluation of Rel-16 eType-II PMI reporting test | Ericsson | noted |  |  |
| R4-2101445 | L1-SINR measurement accuracy | Ericsson | noted |  |  |
| R4-2101446 | Test case for SCell beam failure recovery | Ericsson | noted |  |  |
| R4-2101447 | Draft CR: Introduction of test case of link recovery with link recovery requests | Ericsson | revised |  | R4-2103596 |
| R4-2101448 | CR: FRC for eMIMO sDCI/mDCI-based PDSCH transmission | Ericsson, Huawei, HiSilicon, Intel, Samsung | revised |  | R4-2103830 |
| R4-2101449 | Simulation assumption for PDSCH requirement with mDCI/sDCI-based transmission schemes | Ericsson | approved |  |  |
| R4-2101450 | Simulation results of sDCI-based SDM transmission | Ericsson | noted |  |  |
| R4-2101451 | Simulation results of mDCI-based transmission | Ericsson | noted |  |  |
| R4-2101452 | Simulation results of sDCI-based FDM/TDM transmission | Ericsson | noted |  |  |
| R4-2101453 | CR: Correction of eMTC RLM test cases (Rel-14) | Ericsson | revised |  | R4-2103499 |
| R4-2101454 | CR: Correction of eMTC RLM test cases | Ericsson | revised |  | R4-2103500 |
| R4-2101455 | CR: Correction of eMTC RLM test cases | Ericsson | agreed |  |  |
| R4-2101456 | CR: Correction of eMTC RLM test cases | Ericsson | agreed |  |  |
| R4-2101457 | Correction Rel-16 eMTC RRM performance requirements | Ericsson | revised |  | R4-2103658 |
| R4-2101458 | Correction Rel-16 eMTC RRM performance requirements | Ericsson | agreed |  |  |
| R4-2101459 | Handling of Channel Bandwidths That Are Not Multiples of 5MHz | Qualcomm Incorporated | noted |  |  |
| R4-2101460 | Short Transient Period Testing | Qualcomm Incorporated | noted |  |  |
| R4-2101461 | Updated evaluation assumptions for R17 RLM/BFD relaxation | vivo, MediaTek | revised |  | R4-2103724 |
| R4-2101462 | Discussion on R17 RLM/BFD relaxation | vivo | noted |  |  |
| R4-2101463 | Simulation results for R17 RLM/BFD relaxation | vivo | noted |  |  |
| R4-2101464 | CR on the filter for beam failure indications in 38.133 | vivo | revised |  | R4-2103484 |
| R4-2101465 | CR on the filter for beam failure indications in 38.133 | vivo | agreed |  |  |
| R4-2101466 | CR on the filter for beam failure indications in 38.133 | vivo | agreed |  |  |
| R4-2101467 | Discussion on UL gap for self-calibration and monitoring | vivo | noted |  |  |
| R4-2101468 | TP for TR 36.717-04-01: CA_1-3-40-41 | VODAFONE Group Plc | approved |  |  |
| R4-2101469 | TP for TR 36.717-04-01: CA_1-7-8-28 | VODAFONE Group Plc | approved |  |  |
| R4-2101470 | TP for TR 36.717-04-01: CA_1-7-8-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101471 | TP for TR 36.717-04-01: CA_1-7-28-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101472 | TP for TR 36.717-04-01: CA_1-8-20-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101473 | TP for TR 36.717-04-01: CA_1-8-28-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101474 | TP for TR 36.717-04-01: CA_1-20-28-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101475 | TP for TR 36.717-04-01: CA_3-7-8-28 | VODAFONE Group Plc | approved |  |  |
| R4-2101476 | TP for TR 36.717-04-01: CA_3-8-40-41 | VODAFONE Group Plc | approved |  |  |
| R4-2101477 | TP for TR 36.717-04-01: CA_7-8-20-28 | VODAFONE Group Plc | approved |  |  |
| R4-2101478 | TP for TR 36.717-04-01: CA_7-8-20-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101479 | TP for TR 36.717-04-01: CA_7-8-28-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101480 | TP for TR 36.717-04-01: CA_7-20-28-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101481 | TP for TR 36.717-04-01: CA_8-20-28-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101482 | TR38.717-04-02 update version 0.3.0 | Samsung R&D Institute India | not concluded |  |  |
| R4-2101483 | TR 37.717-51-11 update version 0.2.0 | Samsung R&D Institute India | withdrawn |  |  |
| R4-2101484 | CR on introduction of shorter Transient Period Capability | Qualcomm Incorporated | revised |  | R4-2103152 |
| R4-2101485 | Overview of the Impact of phase variation for Direct NF Method | MVG Industries, Sony | noted |  |  |
| R4-2101486 | TR 37.717-21-22 update version 0.2.0 | Samsung R&D Institute India | withdrawn |  |  |
| R4-2101487 | CR on introduction of shorter Transient Period Capability | Qualcomm Incorporated | agreed |  |  |
| R4-2101488 | TP for TR 36.717-04-01: CA_1-3-7-8-28 | VODAFONE Group Plc | approved |  |  |
| R4-2101489 | TP for TR 36.717-04-01: CA_1-3-8-20-28 | VODAFONE Group Plc | approved |  |  |
| R4-2101490 | TP for TR 36.717-04-01: CA_1-7-8-20-28 | VODAFONE Group Plc | approved |  |  |
| R4-2101491 | TP for TR 36.717-04-01: CA_1-7-8-20-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101492 | TP for TR 36.717-04-01: CA_1-7-8-28-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101493 | TP for TR 36.717-04-01: CA_1-7-20-28-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101494 | TR 38.921 V 0.3.0 | Huawei, HiSilicon | agreed |  |  |
| R4-2101495 | TP to TR 38.921: UE remaining parameters | Huawei, HiSilicon | noted |  |  |
| R4-2101496 | TP to TR 38.921: BS remaining parameters | Huawei, HiSilicon | revised |  | R4-2103106 |
| R4-2101497 | Simulation results on indoor DL co-existence for 6.425-7.125GHz, 10.0-10.5 GHz | Huawei, HiSilicon | noted |  |  |
| R4-2101498 | Simulation results on indoor UL co-existence for 6.425-7.125GHz, 10.0-10.5 GHz | Huawei, HiSilicon | noted |  |  |
| R4-2101499 | TP for Clause 4.3 co-existence simulation results | Huawei, HiSilicon | noted |  |  |
| R4-2101500 | Reply LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz) | Huawei, HiSilicon | noted |  |  |
| R4-2101501 | Consideration on the work plan on introduction of channel bandwidths 35MHz and 45MHz | Huawei, HiSilicon | noted |  |  |
| R4-2101502 | UE REFSENS for 35 MHz and 45 MHz | Huawei, HiSilicon | noted |  |  |
| R4-2101503 | CR for TS 38.101: introduction of channel bandwidths 35MHz and 45MHz | Huawei, HiSilicon | not pursued |  |  |
| R4-2101504 | CR for TS 38.104: introduction of channel bandwidths 35MHz and 45MHz | Huawei, HiSilicon | revised |  | R4-2103190 |
| R4-2101505 | CR for TS 37.141: introduction of channel bandwidths 35MHz and 45MHz | Huawei, HiSilicon | revised |  | R4-2103191 |
| R4-2101506 | CR for TS 37.145-2: introduction of channel bandwidths 35MHz and 45MHz | Huawei, HiSilicon | revised |  | R4-2103192 |
| R4-2101507 | Consideration for overlapping UE channel bandwidths | Huawei, HiSilicon | noted |  |  |
| R4-2101508 | TR 37.717-21-11 V0.2.0 for DC of 2 LTE band and 1 NR band | Huawei, HiSilicon | not concluded |  |  |
| R4-2101509 | Revised WID: Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | Huawei, HiSilicon | not concluded |  |  |
| R4-2101510 | CR on introduction of completed EN-DC of 2 bands LTE and 1 band NR from RAN4#96e and RAN4#97e into TS 38.101-3 | Huawei, HiSilicon | agreed |  |  |
| R4-2101511 | TP for 37.717-21-11: correction of duplicated TPS for some combinations | Huawei, HiSilicon | approved |  |  |
| R4-2101512 | DraftCR for 38.101-1 to add BCS1 for CA_n41A-n66A and BCS0 for CA_n41A-n66(2A) | Huawei, HiSilicon, Bell Mobility, Telus | endorsed |  |  |
| R4-2101513 | TP for TR 38.717-03-02: CA_n66-n71-n78 | Huawei, HiSilicon, Bell Mobility, Telus | endorsed |  |  |
| R4-2101514 | TP for TR 38.717-03-02: CA_n38A-n66A-n78A | Huawei, HiSilicon, Bell Mobility, Telus | revised |  | R4-2103094 |
| R4-2101515 | TP for TR 38.717-03-02: CA_n25A-n38A-n78A | Huawei, HiSilicon, Bell Mobility, Telus | approved |  |  |
| R4-2101516 | DraftCR for 36.101 to add CA_5A-7A-7A-66A, CA_2A-5A-7A-7A and CA_7A-7A-13A-66A | Huawei, HiSilicon, Bell Mobility, Telus | endorsed |  |  |
| R4-2101517 | DraftCR for 36.101 to add CA_2A-5A-7A-7A-66A and CA_2A-7A-7A-13A-66A | Huawei, HiSilicon, Bell Mobility, Telus | endorsed |  |  |
| R4-2101518 | DraftCR for 38.101-3 to add DC_7A-7A-13A-66A_n66A, DC_2A-7A-66A-66A_n66A and DC_2A-7A-7A-66A-66A_n66A | Huawei, HiSilicon, Bell Mobility, Telus | endorsed |  |  |
| R4-2101519 | DraftCR for 38.101-3 to add DC_2A-7A-7A-13A-66A_N66a and DC_2A-5A-7A-7A-66A_n66A | Huawei, HiSilicon, Bell Mobility, Telus | endorsed |  |  |
| R4-2101520 | DC_2A-66A-66A_n66A,DC_7A-66A-66A_n66A and DC_7A-7A-66A-66A_n66A | Huawei, HiSilicon, Bell Mobility, Telus | endorsed |  |  |
| R4-2101521 | Adding 90 and 100MHz bandwidth for band n40 | Huawei, HiSilicon | noted |  |  |
| R4-2101522 | TP for TR 36.717-04-01: CA_3-7-8-20-28 | VODAFONE Group Plc | approved |  |  |
| R4-2101523 | Discussion on WRC-19 remaining issues | OPPO | noted |  |  |
| R4-2101524 | TP for TR 36.717-04-01: CA_7-8-20-28-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101525 | TP for TR 36.717-04-01: CA_1-3-7-8-20-28 | VODAFONE Group Plc | approved |  |  |
| R4-2101526 | Further discussion on maintenance for RSTD measurement requirement | OPPO | noted |  |  |
| R4-2101527 | CR to TS 38.133 on UE Rx-Tx time difference measurements (section 9.9.4) | OPPO | revised |  | R4-2103580 |
| R4-2101528 | Discussion on maintenance for UE Rx-Tx time difference measurements | OPPO | noted |  |  |
| R4-2101529 | Discussion on remaining issues for general PRS measurements | OPPO | noted |  |  |
| R4-2101530 | Maintenance CR on interruption at EUTRA SRS carrier switching in 38.133 | OPPO | revised |  | R4-2103509 |
| R4-2101531 | Maintenance CR on SCell activation delay requirement in TS38.133 | OPPO | revised |  | R4-2103510 |
| R4-2101532 | Discussion on remaining issues for CSI-RS measurement accuracy requirements | OPPO | noted |  |  |
| R4-2101533 | CR for CSI-RS L3 RRM tests | OPPO, CATT | endorsed |  |  |
| R4-2101534 | CR on EN-DC tests for NR inter-frequency neighbor cell in FR2(PScell in FR2) | OPPO | revised |  | R4-2103644 |
| R4-2101535 | CR on EN-DC tests for CSI-RSRQ accuracy | OPPO | revised |  | R4-2103648 |
| R4-2101536 | Views on RRM requirements for PUCCH SCell Activation/Deactivation | OPPO | noted |  |  |
| R4-2101537 | Views on pre-configured MG pattern(s) for NR_MG_enh | OPPO | noted |  |  |
| R4-2101538 | Views on Multiple concurrent and independent MG patterns for NR_MG_enh | OPPO | noted |  |  |
| R4-2101539 | Views on pre-configured MG pattern(s) for NR_MG_enh | OPPO | noted |  |  |
| R4-2101540 | Discussion on RRM requirements for FR2 inter-band DL CA enhancements | OPPO | noted |  |  |
| R4-2101541 | Discussion on timing requirements for NR NTN RRM | OPPO | noted |  |  |
| R4-2101542 | Discussion on UE measurement relaxation for RLM and/or BFD | OPPO | noted |  |  |
| R4-2101543 | Reply LS on temporary RS for efficient SCell activation in NR CA | OPPO | noted |  |  |
| R4-2101544 | TP for TR 36.717-04-01: CA_1-7-8-20-28-32 | VODAFONE Group Plc | approved |  |  |
| R4-2101545 | TP for TR 37.717-21-11: DC_8-20_n1 | VODAFONE Group Plc | revised |  | R4-2103023 |
| R4-2101546 | TP for TR 37.717-21-11: DC_8-20_n3 | VODAFONE Group Plc | revised |  | R4-2103024 |
| R4-2101547 | TP for TR 37.717-21-11: DC_8-20_n28 | VODAFONE Group Plc | noted |  |  |
| R4-2101548 | TP for TR 37.717-21-11: DC_8-32_n1 | VODAFONE Group Plc | approved |  |  |
| R4-2101549 | TP for TR 37.717-31-11: DC_1-8-20_n28 | VODAFONE Group Plc | noted |  |  |
| R4-2101550 | TP for TR 37.717-31-11: DC_7-8-20_n1 | VODAFONE Group Plc | noted |  |  |
| R4-2101551 | TP for TR 37.717-31-11: DC_7-8-20_n3 | VODAFONE Group Plc | noted |  |  |
| R4-2101552 | TP for TR 37.717-31-11: DC_7-8-32_n1 | VODAFONE Group Plc | approved |  |  |
| R4-2101553 | TP for TR 37.717-31-11: DC_7-20-32_n78 | VODAFONE Group Plc | approved |  |  |
| R4-2101554 | TP for TR 37.717-31-11: DC_8-20-32_n1 | VODAFONE Group Plc | noted |  |  |
| R4-2101555 | Updated TR 38.844 v0.0.2 | Ericsson | revised |  | R4-2103262 |
| R4-2101556 | Utilizing larger bandwidth than the operator licensed bandwidth | Ericsson | noted |  |  |
| R4-2101557 | Overlapping Channel Bandwidth Approach from UE Perspective | Ericsson | noted |  |  |
| R4-2101558 | Overlapping Channel Bandwidth Approach from BS Perspective | Ericsson | noted |  |  |
| R4-2101559 | CR to TS 37.105: Introduction of CBWs 35 MHz and 45 MHz | Ericsson | revised |  | R4-2103193 |
| R4-2101560 | CR to TS 38.141-1: Introduction of CBWs 35 MHz and 45 MHz | Ericsson | revised |  | R4-2103194 |
| R4-2101561 | TP to TR 38.808: Numerology and Channel Bandwidths | Ericsson | revised |  | R4-2103259 |
| R4-2101562 | TP for TR 37.717-41-11: DC_1-7-8-20_n3 | VODAFONE Group Plc | noted |  |  |
| R4-2101563 | TP for TR 37.717-41-11: DC_1-7-8-20_n28 | VODAFONE Group Plc | noted |  |  |
| R4-2101564 | Discussion on IAB test models for conformance specifications | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101565 | Discussion on IAB RF conformance test configurations | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101566 | On NR-U measurement uncertainties for BS conformance tests | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101567 | Draft CR to TS 37.107 With NR-U intorduction for performance part | Nokia, Nokia Shanghai Bell | postponed |  |  |
| R4-2101568 | Further discussion on PN23 sequence generation for NR test models | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101569 | CR to TS 38.141-1 clarification on PN23 sequence generation | Nokia, Nokia Shanghai Bell | revised |  | R4-2103789 |
| R4-2101570 | CR to TS 38.141-1 clarification on PN23 sequence generation | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2101571 | CR to TS 38.141-2 clarification on PN23 sequence generation | Nokia, Nokia Shanghai Bell | revised |  | R4-2103790 |
| R4-2101572 | CR to TS 38.141-2 clarification on PN23 sequence generation | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2101573 | TP for TR 37.717-41-11: DC_1-7-8-20_n78 | VODAFONE Group Plc | approved |  |  |
| R4-2101574 | TP for TR 37.717-41-11: DC_7-8-20-32_n1 | VODAFONE Group Plc | noted |  |  |
| R4-2101575 | TP for TR 36.717-02-01: CA_2A-8A | Huawei, HiSilicon | approved |  |  |
| R4-2101576 | Draft CR to 36.101 to add UL configuration CA_3A-8A for CA_3C-7A-8A | Huawei, HiSilicon | endorsed |  |  |
| R4-2101577 | Draft CR to 36.101 to add UL configuration CA_1A-3A for CA_1A-3C-38A | Huawei, HiSilicon | endorsed |  |  |
| R4-2101578 | Draft CR to 36.101 to add UL configuration CA_3A-8A for CA_3C-8A-38A | Huawei, HiSilicon | endorsed |  |  |
| R4-2101579 | TP for TR 36.717-03-02: to add UL configuration CA_1A-8A for CA_1A-8A-38A | Huawei, HiSilicon | revised |  | R4-2103098 |
| R4-2101580 | Draft CR to 36.101 to add UL configuration CA_1A-3A for CA_1A-3C-7A-8A | Huawei, HiSilicon | endorsed |  |  |
| R4-2101581 | Draft CR to 36.101 to add UL configuration CA_1A-3A for CA_1A-3C-8A-38A | Huawei, HiSilicon | endorsed |  |  |
| R4-2101582 | TP for TR 36.717-04-01: CA_1A-3A-8A-20A-38A | Huawei, HiSilicon | approved |  |  |
| R4-2101583 | TP for TR 36.717-04-01: CA_1A-3A-7A-8A-38A | Huawei, HiSilicon | approved |  |  |
| R4-2101584 | TP for TR 37.717-11-11: DC_12_n71 | Huawei, HiSilicon, Ericsson, US Cellular | revised |  | R4-2103026 |
| R4-2101585 | TP for TR 37.717-11-21:DC_8A_n28A-n78A | Huawei, HiSilicon | revised |  | R4-2103064 |
| R4-2101586 | TP for TR 37.717-11-21:DC_1A-8A_n28A-n78A | Huawei, HiSilicon | approved |  |  |
| R4-2101587 | TP for TR 37.717-11-21:DC_3A-8A_n28A-n78A | Huawei, HiSilicon | approved |  |  |
| R4-2101588 | TP for TR 37.717-11-21:DC_7A-8A_n28A-n78A | Huawei, HiSilicon | approved |  |  |
| R4-2101589 | TP for TR 37.717-11-21:DC_1A-3A-8A_n28A-n78A | Huawei, HiSilicon | revised |  | R4-2103065 |
| R4-2101590 | TP for TR 37.717-11-21:DC_1A-7A-8A_n28A-n78A | Huawei, HiSilicon | approved |  |  |
| R4-2101591 | TP for TR 37.717-11-21:DC_3A-7A-8A_n28A-n78A | Huawei, HiSilicon | approved |  |  |
| R4-2101592 | TP for TR 37.717-11-21:DC_1A-3A-7A-8A_n28A-n78A | Huawei, HiSilicon | approved |  |  |
| R4-2101593 | Updated TP for TR 37.717-11-21:DC_3C-20A_n1A-n78A | Huawei, HiSilicon | approved |  |  |
| R4-2101594 | Updated TP for TR 37.717-11-21:DC_3C-7A-20A_n1A-n78A | Huawei, HiSilicon | approved |  |  |
| R4-2101595 | DraftCR for 38.101-1 to add BCS1 for CA_n1A-n8A-n78A | Huawei, HiSilicon | endorsed |  |  |
| R4-2101596 | TP for TR 38.717-03-01: CA_n1A-n8A-n79A | Huawei, HiSilicon | approved |  |  |
| R4-2101597 | Updated TP for TR 38.717-03-01: to add configuration CA_n1A-n78(2A)-n79A and CA_n1A-n78A-n79A_BCS1 | Huawei, HiSilicon | approved |  |  |
| R4-2101598 | TP for TR 38.717-03-01: CA_n8A-n78A-n79A/CA_n8A-n78(2A)-n79A | Huawei, HiSilicon | approved |  |  |
| R4-2101599 | TP for TR 38.717-02-01: CA_n8A-n20A | Huawei, HiSilicon | approved |  |  |
| R4-2101600 | TP for TR 38.717-02-01: CA_n3A-n39A | Huawei, HiSilicon | noted |  | R4-2103044 |
| R4-2101601 | DraftCR for 38.101-1 to add BCS1 for CA_n1A-n7A | Huawei, HiSilicon | endorsed |  |  |
| R4-2101602 | DraftCR for 38.101-1 to add BCS1 for CA_n1A-n3A | Huawei, HiSilicon | revised |  | R4-2103045 |
| R4-2101603 | TP for TR 37.717-00-00 for CA_n3A_SUL_n78A-n80A | Huawei, HiSilicon | revised |  | R4-2103070 |
| R4-2101604 | TP for TR 37.717-00-00 for CA_n41A_SUL_n79A-n83A | Huawei, HiSilicon | revised |  | R4-2103071 |
| R4-2101605 | TP for TR 37.717-00-00 for CA_n79A_SUL_n41A-n83A | Huawei, HiSilicon | revised |  | R4-2103072 |
| R4-2101606 | DraftCR for 38.101-1 to add configuration for SUL_n41C-n95A | Huawei, HiSilicon | endorsed |  |  |
| R4-2101607 | DraftCR for 38.101-1 to add configuration for SUL_n79C-n95A | Huawei, HiSilicon | endorsed |  |  |
| R4-2101608 | TP for TR 37.717-00-00 for SUL_n41A-n97A | Huawei, HiSilicon, CMCC | noted |  | R4-2103073 |
| R4-2101609 | TP for TR 37.717-00-00 for SUL_n79A-n97A | Huawei, HiSilicon, CMCC | revised |  | R4-2103074 |
| R4-2101610 | TP for TR 37.717-00-00 for SUL_n41A-n98A | Huawei, HiSilicon, CMCC | revised |  | R4-2103075 |
| R4-2101611 | TP for TR 37.717-00-00 for SUL_n79A-n98A | Huawei, HiSilicon, CMCC | approved |  |  |
| R4-2101612 | Correction to Aperiodic CSI-RS configurations R15 | Huawei, HiSilicon | agreed |  |  |
| R4-2101613 | Correction to Aperiodic CSI-RS configurations R16 | Huawei, HiSilicon | agreed |  |  |
| R4-2101614 | Correction to Aperiodic CSI-RS configurations R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2101615 | Correction to radio link monitoring test cases R15 | Huawei, HiSilicon | agreed |  |  |
| R4-2101616 | Correction to radio link monitoring test cases R16 | Huawei, HiSilicon | agreed |  |  |
| R4-2101617 | Correction to radio link monitoring test cases R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2101618 | Correction to beam failure recovery test cases R15 | Huawei, HiSilicon | revised |  | R4-2103491 |
| R4-2101619 | Correction to beam failure recovery test cases R16 | Huawei, HiSilicon | agreed |  |  |
| R4-2101620 | Correction to beam failure recovery test cases R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2101621 | Correction to L1-RSRP reporting delay test cases R15 | Huawei, HiSilicon | revised |  | R4-2103492 |
| R4-2101622 | Correction to L1-RSRP reporting delay test cases R16 | Huawei, HiSilicon | agreed |  |  |
| R4-2101623 | Correction to L1-RSRP reporting delay test cases R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2101624 | Correction to relexed cell reselection requirements R16 | Huawei, HiSilicon | merged |  |  |
| R4-2101625 | Correction to relexed cell reselection requirements R17 | Huawei, HiSilicon | withdrawn |  |  |
| R4-2101626 | Discussion on RRM core requirements maintenance for IAB | Huawei, HiSilicon | noted |  |  |
| R4-2101627 | CR on RRM core requirements maintenance for MG for IAB | Huawei, HiSilicon | merged |  |  |
| R4-2101628 | Discussion on RRM test cases for IAB | Huawei, HiSilicon | noted |  |  |
| R4-2101629 | draftCR to introduce test configurations for IAB-MT RRM performance test | Huawei, HiSilicon | revised |  | R4-2103541 |
| R4-2101630 | draftCR to introduce test cases for RRC release with redirection for IAB-MT | Huawei, HiSilicon | revised |  | R4-2103543 |
| R4-2101631 | CR on interruption requirements of BWP switch on multiple CCs for EN-DC | Huawei, HiSilicon | agreed |  |  |
| R4-2101632 | CR on interruption requirements of BWP switch on multiple CCs for EN-DC | Huawei, HiSilicon | agreed |  |  |
| R4-2101633 | Discussion on requirements maintenance for BWP switch on multiple CCs | Huawei, HiSilicon | noted |  |  |
| R4-2101634 | Discussion on performance requirements for BWP switch on multiple CCs | Huawei, HiSilicon | noted |  |  |
| R4-2101635 | draftCR to introduce Active BWP Switch on multiple CCs TC2 | Huawei, HiSilicon | revised |  | R4-2103605 |
| R4-2101636 | Discussion on terminology updates for NR-U | Huawei, HiSilicon | noted |  |  |
| R4-2101637 | Discussion on remaining issues for random access for NR-U | Huawei, HiSilicon | noted |  |  |
| R4-2101638 | CR on requirement maintenance for SCell activation and deactivation for NR-U | Huawei, HiSilicon | merged |  |  |
| R4-2101639 | CR on requirement maintenance for SCell activation and deactivation for NR-U | Huawei, HiSilicon | withdrawn |  |  |
| R4-2101640 | Discussion on remaining issues for SCell activation and deactivation for NR-U | Huawei, HiSilicon | noted |  |  |
| R4-2101641 | CR on measurement requirements for NR-U | Huawei, HiSilicon | agreed |  |  |
| R4-2101642 | CR on measurement requirements for NR-U | Huawei, HiSilicon | agreed |  |  |
| R4-2101643 | Discussion on measurement requirements for NR-U | Huawei, HiSilicon | noted |  |  |
| R4-2101644 | CR on timing requirements for NR-U | Huawei, HiSilicon | postponed |  |  |
| R4-2101645 | CR on timing requirements for NR-U | Huawei, HiSilicon | withdrawn |  |  |
| R4-2101646 | Discussion on timing requirements for NR-U | Huawei, HiSilicon | noted |  |  |
| R4-2101647 | Discussion on RRM performance requirements for NR-U | Huawei, HiSilicon | noted |  |  |
| R4-2101648 | Discussion on RRM test configurations for NR-U | Huawei, HiSilicon | noted |  |  |
| R4-2101649 | Draft CR of test cases for HO delay and interruption for NR-U | Huawei, HiSilicon | revised |  | R4-2104060 |
| R4-2101650 | Draft CR of test cases for RRC release with redirection for NR-U | Huawei, HiSilicon | revised |  | R4-2104061 |
| R4-2101651 | Draft CR of test cases for PSCell addition and release for NR-U | Huawei, HiSilicon | revised |  | R4-2104062 |
| R4-2101652 | Draft CR of test cases for inter-RAT measurement for NR-U | Huawei, HiSilicon | revised |  | R4-2104063 |
| R4-2101653 | Draft CR of test cases for intra-frequency measurement accuracy for NR-U | Huawei, HiSilicon | revised |  | R4-2104064 |
| R4-2101654 | CR on maintenance for measurement on non-anchor carrier for NB-IoT | Huawei, HiSilicon | revised |  | R4-2103660 |
| R4-2101655 | CR on maintenance for measurement on non-anchor carrier for NB-IoT | Huawei, HiSilicon | agreed |  |  |
| R4-2101656 | Discussion on requirements for SRS antenna switching | Huawei, HiSilicon | noted |  |  |
| R4-2101657 | Discussion on requirements for HO with PSCell | Huawei, HiSilicon | noted |  |  |
| R4-2101658 | Discussion on requirements for PUCCH SCell activation | Huawei, HiSilicon | noted |  |  |
| R4-2101659 | Discussion on per-FR gap capability | Huawei, HiSilicon | noted |  |  |
| R4-2101660 | Discussion on antenna configurations for 4Rx capable UE | Huawei, HiSilicon | noted |  |  |
| R4-2101661 | CR on maintaining Antenna configurations in TS38.133 R15 | Huawei, HiSilicon | revised |  | R4-2103493 |
| R4-2101662 | CR on maintaining Antenna configurations in TS38.133 R16 | Huawei, HiSilicon | agreed |  |  |
| R4-2101663 | CR on maintaining Antenna configurations in TS38.133 R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2101664 | CR on test requirements for measurement performance tests R15 | Huawei, HiSilicon | revised |  | R4-2103494 |
| R4-2101665 | CR on test requirements for measurement performance tests R16 | Huawei, HiSilicon | agreed |  |  |
| R4-2101666 | CR on test requirements for measurement performance tests R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2101667 | Discussion on sync conditions for intra-band DAPS handover | Huawei, HiSilicon | noted |  |  |
| R4-2101668 | CR on sync conditions for intra-band DAPS handover R16 | Huawei, HiSilicon | revised |  | R4-2103535 |
| R4-2101669 | CR on sync conditions for intra-band DAPS handover R17 | Huawei, HiSilicon | withdrawn |  |  |
| R4-2101670 | Discussion on maintaining issues for L1-SINR measurent requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101671 | CR on maintaining L1-SINR measurent requirements Rel-16 | Huawei, HiSilicon | revised |  | R4-2103591 |
| R4-2101672 | CR on maintaining L1-SINR measurent requirements Rel-17 | Huawei, HiSilicon | agreed |  |  |
| R4-2101673 | Discussion on L1-SINR measurement accuracy requirements | Huawei, HiSilicon | noted |  |  |
| R4-2101674 | Discussion on IMR configuration for L1-SINR measurement tests | Huawei, HiSilicon | noted |  |  |
| R4-2101675 | DraftCR on IMR configuration for L1-SINR measurement tests | Huawei, HiSilicon | endorsed |  |  |
| R4-2101676 | DraftCR on L1-SINR measurement procedure tests with SSB CMR and dedicated IMR | Huawei, HiSilicon | endorsed |  |  |
| R4-2101677 | DraftCR on L1-SINR measurement accuracy tests with SSB CMR and dedicated IMR | Huawei, HiSilicon | revised |  | R4-2103594 |
| R4-2101678 | Discussion on testability of pathloss-RS activation delay | Huawei, HiSilicon | noted |  |  |
| R4-2101679 | DraftCR on SCell activation and deactication delay test for FR2 inter-band CA | Huawei, HiSilicon | revised |  | R4-2103628 |
| R4-2101680 | Discussion on MRTD/MTTD requirements for FR1 intra-band CA/DC | Huawei, HiSilicon | noted |  |  |
| R4-2101681 | CR on MRTD/MTTD requirements for FR1 intra-band CA/DC R16 | Huawei, HiSilicon, LG Uplus, SoftBank Corp. | postponed |  |  |
| R4-2101682 | CR on MRTD/MTTD requirements for FR1 intra-band CA/DC R17 | Huawei, HiSilicon, LG Uplus, SoftBank Corp. | withdrawn |  |  |
| R4-2101683 | Discussion on conditions for FR2 new FWA UE | Huawei, HiSilicon | noted |  |  |
| R4-2101684 | CR on condition requirements for UE power class 5 in TS38.133 | Huawei, HiSilicon | revised |  | R4-2103665 |
| R4-2101685 | Discussion on feasibility of RLM/BFD measurement relaxation scheme for power saving enhancements | Huawei, HiSilicon | noted |  |  |
| R4-2101686 | Discussion on RRM impacts for FR2 inter-band DL CA enhancement in Rel-17 | Huawei, HiSilicon | noted |  |  |
| R4-2101687 | Discussion on RRM impacts for FR2 inter-band UL CA in Rel-17 | Huawei, HiSilicon | noted |  |  |
| R4-2101688 | Discussion on SRS carrier switching test case | Huawei, HiSilicon | noted |  |  |
| R4-2101689 | Correction on interruptions of SRS carrier switching | Huawei, HiSilicon | agreed |  |  |
| R4-2101690 | Correction on interruptions of SRS carrier switching | Huawei, HiSilicon | agreed |  |  |
| R4-2101691 | Clarification on inter-frequency measurement without gap requirements | Huawei, HiSilicon | postponed |  | R4-2103625 |
| R4-2101692 | Clarification on inter-frequency measurement without gap requirements | Huawei, HiSilicon | withdrawn |  |  |
| R4-2101693 | Correction on inter-frequency measurement without gap test case | Huawei, HiSilicon | endorsed |  |  |
| R4-2101694 | UL spatial relation switching to an unknown DL RS | Huawei, HiSilicon | revised |  | R4-2103603 |
| R4-2101695 | UL spatial relation switching to an unknown DL RS | Huawei, HiSilicon | agreed |  |  |
| R4-2101696 | Correction on uplink spatial relation switching test case | Huawei, HiSilicon | endorsed |  |  |
| R4-2101697 | Clarification on asynchronous DAPS handover | Huawei, HiSilicon | revised |  | R4-2103661 |
| R4-2101698 | Clarification on asynchronous DAPS handover | Huawei, HiSilicon | agreed |  |  |
| R4-2101699 | Test cases for inter-frequency DAPS handover | Huawei, HiSilicon | revised |  | R4-2103662 |
| R4-2101700 | Test cases for inter-frequency DAPS handover | Huawei, HiSilicon | agreed |  |  |
| R4-2101701 | Correction on test cases of inter-frequency Measurements R15 | Huawei, HiSilicon | revised |  | R4-2103495 |
| R4-2101702 | Correction on test cases of inter-frequency Measurements R16 | Huawei, HiSilicon | agreed |  |  |
| R4-2101703 | Correction on test cases of inter-frequency Measurements R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2101704 | Correction on NR Pcell FR2 active TCI state switching R15 | Huawei, HiSilicon | postponed |  |  |
| R4-2101705 | Correction on NR Pcell FR2 active TCI state switching R16 | Huawei, HiSilicon | withdrawn |  |  |
| R4-2101706 | Correction on NR Pcell FR2 active TCI state switching R17 | Huawei, HiSilicon | withdrawn |  |  |
| R4-2101707 | Discussion on Enhancement for NR high speed train scenario in FR1 | Huawei, HiSilicon | noted |  |  |
| R4-2101708 | Preliminary discussion on NR support for high speed train scenario in FR2 | Huawei, HiSilicon | noted |  |  |
| R4-2101709 | Discussion on temporary RS for efficient SCell activation in NR CA | Huawei, HiSilicon | noted |  |  |
| R4-2101710 | Correction on test cases of DL interruptions at switching between two uplink carriers | Huawei, HiSilicon | revised |  | R4-2103601 |
| R4-2101711 | Correction on test cases of DL interruptions at switching between two uplink carriers | Huawei, HiSilicon | agreed |  |  |
| R4-2101712 | Discussion on NTN measurement | Huawei, HiSilicon | noted |  |  |
| R4-2101713 | Correction to applicability of simultaneous RX/TX | Ericsson | revised |  | R4-2103119 |
| R4-2101714 | Correction to applicability of simultaneous RX/TX | Ericsson | withdrawn |  |  |
| R4-2101715 | Correction to the lower limit of Pumax | Ericsson | not pursued |  |  |
| R4-2101716 | Correction to the lower limit of Pumax | Ericsson | withdrawn |  |  |
| R4-2101717 | Draft Reply LS on ambiguity in deciding TL,C | Ericsson | noted |  |  |
| R4-2101718 | Correction to applicability of simultaneous RX/TX and single-UL transmission | Ericsson | not pursued |  |  |
| R4-2101719 | Correction to applicability of simultaneous RX/TX and single-UL transmission | Ericsson | withdrawn |  |  |
| R4-2101720 | Applicability of minimum requirements for shared spectrum access | Ericsson | revised |  | R4-2103135 |
| R4-2101721 | Support of transparent TxD and the associated EVM requirements | Ericsson | noted |  |  |
| R4-2101722 | LS to RAN5 on SCell dropping behavior and verification thereof | Ericsson | noted |  |  |
| R4-2101723 | Modification of Pcmax for UL CA with uplink Tx switching capability | Ericsson | revised |  | R4-2103159 |
| R4-2101724 | Correction to modified MPR behaviour | Ericsson | revised |  | R4-2103167 |
| R4-2101725 | Requirements Type 2 UEs supporting inter-band MRDC with overlapping DL | Ericsson | revised |  | R4-2103164 |
| R4-2101726 | Methods for faciliating SAR compliance for inter-band UL CA | Ericsson | noted |  |  |
| R4-2101727 | On the inter-band UL CA study and change of scope too include improved BC | Ericsson, Sony | noted |  |  |
| R4-2101728 | Revised WID for NR RF Enhancement for FR2 | Ericsson, Sony | not treated |  |  |
| R4-2101729 | gNB Positioning UL SRS System simulation results for side conditions | Ericsson | noted |  |  |
| R4-2101730 | TS 38.141-2: Correction of additional spurious emission limits for bands 50, 51, 75, 76 | Ericsson | postponed |  | R4-2103795 |
| R4-2101731 | TS 38.141-2: Correction of additional spurious emission limits for bands 50, 51, 75, 76 | Ericsson | withdrawn |  |  |
| R4-2101732 | TS 38.141-2: Correction of additional spurious emission limits for bands 50, 51, 75, 76 | Ericsson | withdrawn |  |  |
| R4-2101733 | TS 37.145-2: Tx spurious limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U) | Ericsson | postponed |  |  |
| R4-2101734 | TS 37.145-2: Tx spurious limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U) | Ericsson | withdrawn |  |  |
| R4-2101735 | TS 38.141-2: Tx spurious limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U) | Ericsson | postponed |  |  |
| R4-2101736 | TS 38.141-2: Tx spurious limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U) | Ericsson | withdrawn |  |  |
| R4-2101737 | gNB SRS-RSRP measurement analysis | Ericsson | noted |  |  |
| R4-2101738 | Discussion on FR2 equal PSD in CA and draft LS | OPPO | noted |  |  |
| R4-2101739 | CR on FR2 equal PSD in UL CA | OPPO | not pursued |  |  |
| R4-2101740 | CR on FR2 equal PSD in UL CA (R16 mirror CR) | OPPO | withdrawn |  |  |
| R4-2101741 | CR on FR2 equal PSD in UL CA (R17 mirror CR) | OPPO | withdrawn |  |  |
| R4-2101742 | Discussion on simultaneous Tx-Rx | OPPO | noted |  |  |
| R4-2101743 | CR on simultaneous Tx-Rx for CA and SUL | OPPO | not pursued |  |  |
| R4-2101744 | CR on simultaneous Tx-Rx for CA and SUL (R16 mirror CR) | OPPO | withdrawn |  |  |
| R4-2101745 | CR on simultaneous Tx-Rx for CA and SUL (R17 mirror CR) | OPPO | withdrawn |  |  |
| R4-2101746 | CR on simultaneous Tx-Rx for EN-DC | OPPO | revised |  | R4-2103128 |
| R4-2101747 | CR on simultaneous Tx-Rx for EN-DC (R16 CR) | OPPO | revised |  | R4-2103129 |
| R4-2101748 | CR on simultaneous Tx-Rx for EN-DC (R17 mirror CR) | OPPO | withdrawn |  |  |
| R4-2101749 | Discussion on UE power class high limit | OPPO | noted |  |  |
| R4-2101750 | Discussion on BCS for intra-band EN-DC and draft reply LS | OPPO | noted |  |  |
| R4-2101751 | Discussion on Rel-16 TxD | OPPO | noted |  |  |
| R4-2101752 | Discussion on inter-band CA HPUE SAR | OPPO | noted |  |  |
| R4-2101753 | Discussion on Rel-17 FWA beam correspondence | OPPO | noted |  |  |
| R4-2101754 | Discussion on Rel-17 power boosting in switched UL transmission | OPPO | noted |  |  |
| R4-2101755 | Discussion on Rel-17 FR1 UL CA | OPPO | noted |  |  |
| R4-2101756 | Discussion on Rel-17 FR2 CBM IBM | OPPO | noted |  |  |
| R4-2101757 | Consideration on 3D-MPAC probe locations configuration for FR2 MIMO OTA | OPPO | noted |  |  |
| R4-2101758 | Consideration on how to treat the missing orientations for FR2 | OPPO | noted |  |  |
| R4-2101759 | Solution to minimize the impact of polarization basis mismatch | OPPO | noted |  |  |
| R4-2101760 | gNB SRS-RSRP measurement | Ericsson | revised |  | R4-2104052 |
| R4-2101761 | gNB Rx-Tx measurement analysis | Ericsson | noted |  |  |
| R4-2101762 | CR to 38.133 correction on SRS carrier based switching core requirements | vivo | revised |  | R4-2103611 |
| R4-2101763 | CR to 38.133 correction on SRS carrier based switching core requirements | vivo | withdrawn |  |  |
| R4-2101764 | Remaining issues for SRS carrier switching test cases | vivo | noted |  |  |
| R4-2101765 | CR to 38.133 on SRS configuration | vivo | revised |  | R4-2103615 |
| R4-2101766 | On CGI reading test cases | vivo | noted |  |  |
| R4-2101767 | Remaining issues on CSI-RS L3 measurement core requirements | vivo | noted |  |  |
| R4-2101768 | Simulation results for CSI-RSRP measurement accuracy | vivo | noted |  |  |
| R4-2101769 | Discussion on CSI-RSRP measurement accuracy requirements | vivo | noted |  |  |
| R4-2101770 | Simulation results for CSI-RSRQ measurement accuracy | vivo | noted |  |  |
| R4-2101771 | Discussion on CSI-RSRQ measurement accuracy requirements | vivo | noted |  |  |
| R4-2101772 | Simulation results for CSI-SINR measurement accuracy | vivo | noted |  |  |
| R4-2101773 | Discussion on CSI-SINR measurement accuracy requirements | vivo | noted |  |  |
| R4-2101774 | Remaining issues on CSI-RS L3 measurement test cases | vivo | noted |  |  |
| R4-2101775 | CR to 38.133 correction to test cases for EN-DC CSI-SINR measurement accuracy | vivo | revised |  | R4-2103649 |
| R4-2101776 | Discussion on PRS RSTD measurement requirements | vivo | noted |  |  |
| R4-2101777 | Discussion on UE RX-TX timing difference measurement requirements | vivo | noted |  |  |
| R4-2101778 | Discussion on CCSF for NR positioning measurements | vivo | noted |  |  |
| R4-2101779 | CR to 38.133 correction to PRS RSTD measurement requirements | vivo | merged |  |  |
| R4-2101780 | CR to 38.133 correction to PRS RSTD measurement requirements | vivo | withdrawn |  |  |
| R4-2101781 | CR to 38.133 correction on PRS-RSRP measurement requirements | vivo | merged |  |  |
| R4-2101782 | CR to 38.133 correction on PRS-RSRP measurement requirements | vivo | withdrawn |  |  |
| R4-2101783 | CR to 38.133 correction on UE Rx-Tx time difference measurement requirements | vivo | merged |  |  |
| R4-2101784 | CR to 38.133 correction on UE Rx-Tx time difference measurement requirements | vivo | withdrawn |  |  |
| R4-2101785 | CR to 38.133 correction on CCSF for NR measurements for positioning | vivo | revised |  | R4-2103582 |
| R4-2101786 | CR to 38.133 correction on CCSF for NR measurements for positioning | vivo | agreed |  |  |
| R4-2101787 | Discussion on PRS RSTD accuracy requirements | vivo | noted |  |  |
| R4-2101788 | CR to 38.133 correction on reselection of V2X synchronization reference source requirements | vivo | agreed |  |  |
| R4-2101789 | CR to 38.133 correction on reselection of V2X synchronization reference source requirements | vivo | agreed |  |  |
| R4-2101790 | gNB Rx-Tx measurement | Ericsson | postponed |  |  |
| R4-2101791 | Proposals of UE Parameters for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101792 | Proposals of BS Parameters for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101793 | TP to TR 38.921: Clarification of BS maximum transmit power on system level simulation assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz | Nokia, Nokia Shanghai Bell, ZTE | approved |  |  |
| R4-2101794 | Downlink Indoor Hotspot Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101795 | Uplink Indoor Hotspot Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101796 | TP to TR 38.921: Proposals of Indoor BS Antenna Characteristics for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101797 | Proposals of Relevant Information for the ITU-R WP5D Sharing and Compatibility Studies for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101798 | Revised Work Plan for Study on High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71 | Nokia, Nokia Shanghai Bell, T-Mobile USA, U.S. Cellular | approved |  |  |
| R4-2101799 | TR 37.880 V0.2.0: High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71 | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2101800 | TP to TR 37.880: Coexistence Simulation Results for High-power UE Vs NB-IoT guard band operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71 | Nokia, Nokia Shanghai Bell | revised |  | R4-2103284 |
| R4-2101801 | gNB UL RTOA measurement analysis | Ericsson | noted |  |  |
| R4-2101802 | CR for 36.101 to add missing spurious emissions for band 38 UE co-existence (Rel-16) | Huawei, HiSilicon, DT | agreed |  |  |
| R4-2101803 | CR for 36.101 to add missing spurious emissions for band 38 UE co-existence (Rel-17) | Huawei, HiSilicon, DT | agreed |  |  |
| R4-2101804 | CR for 38.101-3 to add the missing Tib Rib for DC_2-7-7-66_n78/ DC_2-7-66-66_n78/ DC_2-7-7-66-66_n78 (Rel-16) | Huawei, HiSilicon | agreed |  |  |
| R4-2101805 | CR for 38.101-3 to add the missing Tib Rib for DC_2-7-7-66_n78/ DC_2-7-66-66_n78/ DC_2-7-7-66-66_n78 (Rel-17) | Huawei, HiSilicon | agreed |  |  |
| R4-2101806 | Discussion on spurious emission about UE co-existence between band n40 and n41 | Huawei, HiSilicon, CMCC | noted |  |  |
| R4-2101807 | CR on spurious emission about UE co-existence between band n40 and n41(Rel-16) | Huawei, HiSilicon, CMCC | not pursued |  |  |
| R4-2101808 | CR on spurious emission about UE co-existence between band n40 and n41(Rel-17) | Huawei, HiSilicon, CMCC | withdrawn |  |  |
| R4-2101809 | CR for 38.101-1 to introduce PC2 for n40 UL MIMO(Rel-16) | Huawei, HiSilicon, Reliance Jio | not pursued |  |  |
| R4-2101810 | CR for 38.101-1 to introduce PC2 for n40 UL MIMO(Rel-17) | Huawei, HiSilicon, Reliance Jio | withdrawn |  |  |
| R4-2101811 | Discussion and reply draft LS on ambiguity in deciding TL,C | Huawei, HiSilicon | noted |  |  |
| R4-2101812 | General discussion on NTN simulation assumptions | Huawei, HiSilicon | noted |  |  |
| R4-2101813 | Discussion on exemplary bands for NTN topic | Huawei, HiSilicon | noted |  |  |
| R4-2101814 | General discussion on Network structure on NTN topics | Huawei, HiSilicon | noted |  |  |
| R4-2101815 | Discussion on the larger channel bandwidth for band n2 | Huawei, HiSilicon | noted |  |  |
| R4-2101816 | Discussion on how to simplify MSD definition using bandwidth-agnostic approach | Huawei, HiSilicon | noted |  |  |
| R4-2101817 | General discussion on introduction of BCS4 | Huawei, HiSilicon | noted |  |  |
| R4-2101818 | LS on change of methodology for new LTE-CA REL-17 combinations | RAN4 | approved |  |  |
| R4-2101819 | New WID on Support of full bandwidth combinations for inter-band EN-DC combinations | Huawei, HiSilicon | not treated |  |  |
| R4-2101820 | Further discussion on RF requirements about Multi-RAT Dual-Connectivity | Huawei, HiSilicon | noted |  |  |
| R4-2101821 | Uplink Power Control for NR MIMO OTA test | vivo | agreed |  |  |
| R4-2101822 | TS 38.151 v0.2.0 NR MIMO OTA requirements | vivo | agreed |  |  |
| R4-2101823 | TP to TS 38.151 v0.1.0 on FR2 Channel model and RMC | vivo, CAICT | revised |  | R4-2103969 |
| R4-2101824 | TP to TS 38.151 v0.1.0 on FR2 test system for requirements | vivo, CAICT | revised |  | R4-2103970 |
| R4-2101825 | TP to TS 38.151 v0.1.0 on Performance metrics for NR MIMO OTA requirements | vivo, CAICT | revised |  | R4-2103971 |
| R4-2101826 | Discussion on testing parameters for NR MIMO OTA requirement | vivo | noted |  |  |
| R4-2101827 | Discussions on Channel models mapping for FR1 MIMO OTA requirement | vivo | noted |  |  |
| R4-2101828 | Discussions on FR2 Extreme temperature conditions | vivo | noted |  |  |
| R4-2101829 | Discussions on Test Time Reduction for NR FR2 RF | vivo | noted |  |  |
| R4-2101830 | TP to TR38.884 v0.1.0 on polarization basis mismatch | vivo | revised |  | R4-2103967 |
| R4-2101831 | Discussion on MOP for Band n262 | vivo | noted |  |  |
| R4-2101832 | Discussion on REFSENS and EIS spherical coverage for Band n262 | vivo | noted |  |  |
| R4-2101833 | Views on Study the testability of 60GHz | vivo Communication Technology | noted |  |  |
| R4-2101834 | Correction on inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion | Huawei, HiSilicon | revised |  | R4-2103572 |
| R4-2101835 | Test case for cell reselection to FR2 intra-frequency NR case for UE configured with relaxed measurement | Huawei, HiSilicon | revised |  | R4-2103575 |
| R4-2101836 | Test case for cell reselection to FR2 intra-frequency NR case for UE configured with relaxed measurement | Huawei, HiSilicon | agreed |  |  |
| R4-2101837 | Discussion on remaining issues for CSI-RS based L3 measurement | Huawei, HiSilicon | noted |  |  |
| R4-2101838 | CR on CSI-RS based intra-frequency scheduling restriction | Huawei, HiSilicon | postponed |  |  |
| R4-2101839 | CR on CSI-RS based intra-frequency scheduling restriction | Huawei, HiSilicon | withdrawn |  |  |
| R4-2101840 | Correction on CSSFoutsidegap | Huawei, HiSilicon | agreed |  |  |
| R4-2101841 | Correction on CSSFoutsidegap | Huawei, HiSilicon | agreed |  |  |
| R4-2101842 | CR on CSI-RS measurement window and intra-frequency measurements | Huawei, HiSilicon | revised |  | R4-2103635 |
| R4-2101843 | CR on CSI-RS measurement window and intra-frequency measurements | Huawei, HiSilicon | withdrawn |  |  |
| R4-2101844 | Correction on inter-RAT measurement in high speed scenario | Huawei, HiSilicon | agreed |  |  |
| R4-2101845 | Correction on inter-RAT measurement in high speed scenario | Huawei, HiSilicon | agreed |  |  |
| R4-2101846 | Correction on test cases for inter-RAT cell identification in connected mode for HST | Huawei, HiSilicon | revised |  | R4-2103656 |
| R4-2101847 | Correction on test cases for inter-RAT cell identification in connected mode for HST | Huawei, HiSilicon | withdrawn |  |  |
| R4-2101848 | Simulation results on CQI requirements for NR DL 256QAM for FR2 | Huawei, HiSilicon | noted |  |  |
| R4-2101849 | CR to TS 38.104 Update on UL timing adjustment performance requirements | ZTE Wistron Telecom AB | revised |  | R4-2103868 |
| R4-2101850 | EVM for transparent TxD | ZTE Wistron Telecom AB | noted |  |  |
| R4-2101851 | Discussion on 2Tx UL switching between two bands | ZTE Wistron Telecom AB | noted |  |  |
| R4-2101852 | CR to TS 38.101-1 Operating bands for DC | ZTE Wistron Telecom AB | not pursued |  |  |
| R4-2101853 | Draft reply LS on on BCS reporting and support for intra-band EN-DC band combinations | ZTE Wistron Telecom AB | noted |  |  |
| R4-2101854 | Draft CR to TS 38.101-1 on switching time between SUL and NUL | ZTE Wistron Telecom AB | not pursued |  |  |
| R4-2101855 | Switching time for UL-MIMO enabled SUL band combination | ZTE Wistron Telecom AB | noted |  |  |
| R4-2101856 | Deployment scenarios for HST FR2 | ZTE Wistron Telecom AB | noted |  |  |
| R4-2101857 | Additional Information for SL Operation in NR Band n14 | AT&T | revised |  | R4-2103005 |
| R4-2101858 | Criteria for Choosing FR1 Exemplary Band | THALES | noted |  |  |
| R4-2101859 | NTN FR1 Coexistence Scenarios and Related Core Requirements | THALES | noted |  |  |
| R4-2101860 | MRTD and MTTD in non-contiguous CA in FR1 | Ericsson | noted |  |  |
| R4-2101861 | MRTD and MTTD in non-contiguous CA in FR1 | Ericsson | postponed |  | R4-2104051 |
| R4-2101862 | MRTD and MTTD in non-contiguous CA in FR1 | Ericsson | withdrawn |  |  |
| R4-2101863 | TP for NR Rel-17 TR 38.808: Time and synchronization impact | Ericsson | noted |  |  |
| R4-2101864 | Architecture and reference point | Ericsson | noted |  |  |
| R4-2101865 | RRM Timing requirements | Ericsson | noted |  |  |
| R4-2101866 | RRM Measurement Requirements | Ericsson | noted |  |  |
| R4-2101867 | Support up to 3 us MRTD | Ericsson | noted |  |  |
| R4-2101868 | Updates on MRTD requirements for FR2 inter-band DL CA | Ericsson | postponed |  |  |
| R4-2101869 | Updates on MTTD requirements for FR2 inter-band DL CA | Ericsson | postponed |  |  |
| R4-2101870 | CR for TS 38.101-3 switching period for V2X con-current operation Rel-16 | Xiaomi | not pursued |  |  |
| R4-2101871 | CR for TS 38.101-3 switching period for V2X con-current operation Rel-17 | Xiaomi | withdrawn |  |  |
| R4-2101872 | on FR2 UE EMC requirement | Xiaomi | noted |  |  |
| R4-2101873 | on HPUE for V2X RF requirements | Xiaomi | noted |  |  |
| R4-2101874 | on HPUE signalling issue | Xiaomi | noted |  |  |
| R4-2101875 | on operating scenarios for partially used SL operation | Xiaomi | noted |  |  |
| R4-2101876 | on switching period | Xiaomi | noted |  |  |
| R4-2101877 | Synchronous operation between NR Uu and NR SL in an operating band | Xiaomi | noted |  |  |
| R4-2101878 | CR to TS 38.141-1 clarification on PN23 sequence generation | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2101879 | CR to TS 38.141-2 clarification on PN23 sequence generation | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2101880 | Simulations for NTN FR1 Coexistence Cases | THALES | noted |  |  |
| R4-2101881 | Correction on inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion | Huawei, HiSilicon | agreed |  |  |
| R4-2101882 | NTN PVT Accuracy Aspects | THALES | noted |  |  |
| R4-2101883 | Revised WID NR Intra-band Rel-17 | Ericsson | endorsed |  |  |
| R4-2101884 | Revised WID LTE 3DL and one NR band Rel-17 | Ericsson | not concluded |  |  |
| R4-2101885 | Revised WID 4 bands NR CA Rel-17 | Ericsson | not concluded |  |  |
| R4-2101886 | CR introduction completed band combinations Rel-17 NR Intra-band - | Ericsson | agreed |  |  |
| R4-2101887 | CR introduction completed band combinations Rel-17 NR Intra-band - | Ericsson | not concluded |  |  |
| R4-2101888 | CR introduction completed band combinations LTE 3DL and one NR band - | Ericsson | agreed |  |  |
| R4-2101889 | CR introduction completed band combinations NR Inter-band 4 bands CA - | Ericsson | not concluded |  |  |
| R4-2101890 | CR introduction completed band combinations NR Inter-band 4 bands CA - | Ericsson | agreed |  |  |
| R4-2101891 | TR 38.717-01-01 v0.3.0 Rel-17 NR Intra-band | Ericsson | not concluded |  |  |
| R4-2101892 | TR 37.717-31-11 v0.3.0 Rel-17 DC combinations LTE 3DL and one NR band | Ericsson | not concluded |  |  |
| R4-2101893 | TR 38.717-04-01 v0.3.0 Rel-17 NR Inter-band 4 bands CA | Ericsson | not concluded |  |  |
| R4-2101894 | draft CR for CA_n46-n48 and DC_n46-n48 configurations | Ericsson, Charter Communication | revised |  | R4-2103046 |
| R4-2101895 | TP to TR TR 37.717-31-11 to include 3-20-40_n78 | Ericsson | approved |  |  |
| R4-2101896 | draft CR 38.101-3 to include 2_n258, 5_n258, 12_n258, 66_n258 configurations | Ericsson, US Cellular | endorsed |  |  |
| R4-2101897 | TP for TR 37.717-11-21 to include DC_3A-28A_n1A-n78A | Ericsson | approved |  |  |
| R4-2101898 | TP for TR 37.717-11-21 to include DC_7A-28A_n1A-n78A | Ericsson | approved |  |  |
| R4-2101899 | TP for TR 37.717-11-21 to include DC_3A-7A-28A_n1A-n78A | Ericsson | approved |  |  |
| R4-2101900 | TP for TR 37.717-11-21 to include DC_1A-7A-28A_n3A-n78A | Ericsson | approved |  |  |
| R4-2101901 | TP for TR 37.717-11-21 to include DC_1A-3A-7A-28A_n78A-n257A/G/H/I | Ericsson | approved |  |  |
| R4-2101902 | TP for 38.717-03-02 to include n25-n66-n71 | Ericsson, T-Mobile US | revised |  | R4-2103095 |
| R4-2101903 | TP for 38.717-03-02 to include n41-n66-n71 | Ericsson, T-Mobile US | approved |  |  |
| R4-2101904 | TP for TR 38.717-04-01 to include CA_n41-n66-n71-n77 | Ericsson, T-Mobile US | revised |  | R4-2103082 |
| R4-2101905 | TP for TR 38.717-04-02 to include CA_n41-n66-n71-n77 | Ericsson, T-Mobile US | approved |  |  |
| R4-2101906 | TP for TR 38.717-04-02 to include CA_n25-n41-n66-n71 | Ericsson, T-Mobile US | approved |  |  |
| R4-2101907 | draft CR to inlcude 7A-12A-66A-66A, 2A-2A-5A-7A, 2A-2A-7A-66A-66A, 2A-2A-7A-7A-13A, 2A-2A-7C-13A, 2A-2A-7A-13A | Ericsson, Rogers, Bell Mobility | endorsed |  |  |
| R4-2101908 | draft CR to include 2A-7A-12A-66A-66A, 2A-2A-5A-7A-66A | Ericsson, Rogers | endorsed |  |  |
| R4-2101909 | TP for TR 37.717-11-11 to include 71A_n41A | Ericsson, Rogers | revised |  | R4-2103027 |
| R4-2101910 | TP for TR 37.717-11-11 to include 7A_n2A | Ericsson, Rogers | noted |  |  |
| R4-2101911 | TP for TR 37.717-11-11 to include 71A_n2A | Ericsson, Rogers | revised |  | R4-2103028 |
| R4-2101912 | draft CR to include DC_2-7_n78 configuration | Ericsson, Rogers | endorsed |  |  |
| R4-2101913 | TP for TR 37.717-21-11 to include 12A-66A_n41A | Ericsson, Rogers | approved |  |  |
| R4-2101914 | TP for TR 37.717-21-11 to include 2A-12A_n41A, 2A-2A-12A_n41A | Ericsson, Rogers | approved |  |  |
| R4-2101915 | TP for TR 37.717-21-11 to include 66A-71A_n41A | Ericsson, Rogers | approved |  |  |
| R4-2101916 | TP for TR 37.717-21-11 to include 2A-71A_n41A, 2A-2A-71A_n41A | Ericsson, Rogers | approved |  |  |
| R4-2101917 | TP for TR 37.717-21-11 to include 7A-12A_n66A | Ericsson, Rogers | approved |  |  |
| R4-2101918 | TP for TR 37.717-21-11 to include 7A-71A_n66A | Ericsson, Rogers | approved |  |  |
| R4-2101919 | TP for TR 37.717-21-11 to include 7A-12A_n78A | Ericsson, Rogers | approved |  |  |
| R4-2101920 | TP for TR 37.717-21-11 to include 12A-66A_n78A | Ericsson, Rogers | approved |  |  |
| R4-2101921 | TP for TR 37.717-21-11 to include 2A-12A_78A, 2A-2A-12A_78A | Ericsson, Rogers | approved |  |  |
| R4-2101922 | On test cases for mandatory gap patterns | ZTE Corporation | noted |  |  |
| R4-2101923 | Frequency Bands for testing of A-GNSS Sensitivity in EN-DC | Qualcomm Incorporated | noted |  |  |
| R4-2101924 | Revised Rel-17 WID on DC of 4 bands LTE inter-band CA (4DL1UL) and 1 NR band (1DL1UL) | Nokia, Nokia Shanghai Bell | not concluded |  |  |
| R4-2101925 | CR to introduce new combinations of LTE 4band + NR 1band for TS 38.101-3 | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2101926 | draft TR 37.717-41-11 v0.3.0 | Nokia, Nokia Shanghai Bell | not concluded |  |  |
| R4-2101927 | Skeleton TR 38.849 v0.0.0 | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2101928 | draft TR 38.849 v0.1.0 | Nokia, Nokia Shanghai Bell | not concluded |  |  |
| R4-2101929 | Work plan for Introduction of lower 6GHz NR unlicensed operation for Europe | Nokia, Nokia Shanghai Bell | approved |  |  |
| R4-2101930 | On UE RF aspects for the lower 6GHz NR unlicensed operation | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101931 | On BS RF aspects for the lower 6GHz NR unlicensed operation | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101932 | NR-U - On Intra-cell guardbands | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101933 | NTN - On use cases and deployment scenarios | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101934 | NTN - HAPS simulation assumptions for co-existence study | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101935 | NTN - HAPS adjacent channel coexistence | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2101936 | TP to TR 36.717-02-01 Addition of CA_46-53 | Nokia, Globalstar | approved |  |  |
| R4-2101937 | Discussion on n47 PC2 MPR simulation of Rel-17 SL enhancement | Huawei, HiSilicon | noted |  |  |
| R4-2101938 | Discussion on the adjacent channel coexistence simulation between SL and Uu in license band | Huawei, HiSilicon | noted |  |  |
| R4-2101939 | CR for 38.101-1 to add missing spurious emissions for band n38 UE co-existence (Rel-16) | Huawei, HiSilicon, DT | agreed |  |  |
| R4-2101940 | CR for 38.101-1 to add missing spurious emissions for band n38 UE co-existence (Rel-17) | Huawei, HiSilicon, DT | agreed |  |  |
| R4-2101941 | FR1 MIMO OTA measurement results and channel model mapping | CAICT,Keysight,ETS-Lindgren | noted |  |  |
| R4-2101942 | Draft CR on General section of NR V2X requirements | Intel Corporation | revised |  | R4-2103813 |
| R4-2101943 | Discussion on NR V2X Multiple Link SDR requirements | Intel Corporation | noted |  |  |
| R4-2101944 | CR on FRC for Ultra low BLER UE CQI requirements | Intel Corporation | revised |  | R4-2103893 |
| R4-2101945 | CR on FRC for NR RI requirements (Rel-15) | Intel Corporation | revised |  | R4-2103798 |
| R4-2101946 | CR on FRC for NR RI requirements (Rel-16) | Intel Corporation | agreed |  |  |
| R4-2101947 | Simplification of n70 | Dish Network | agreed |  |  |
| R4-2101948 | TP to TR 38.921 UE transmitter requirements | ZTE Corporation | revised |  | R4-2103105 |
| R4-2101949 | TP to TR 38.921 BS requirements | ZTE Corporation | noted |  |  |
| R4-2101950 | TP to TR 38.921 summary of simulation results | ZTE Corporation | revised |  | R4-2103107 |
| R4-2101951 | DL simulation results for 6425-7125MHz and 10-10.5GHz | ZTE Corporation | noted |  |  |
| R4-2101952 | UL simulation results for 6425-7125MHz and 10-10.5GHz | ZTE Corporation | noted |  |  |
| R4-2101953 | TP to TR 38.921 Maintenance for simulation assumption | ZTE Corporation | noted |  |  |
| R4-2101954 | TP to TR 38.921 Antenna configurations | ZTE Corporation | noted |  |  |
| R4-2101955 | TP to TR 38.808 Further considerations on timing for 52.6-71GHz | ZTE Corporation | noted |  |  |
| R4-2101956 | Discussion on frequency range definition for 52.6-71GHz | ZTE Corporation | noted |  |  |
| R4-2101957 | Coexistence study for extended 600MHz NR band | ZTE Corporation, CBN | noted |  |  |
| R4-2101958 | Discussions on Option B1 and B2 for extended 600MHz | ZTE Corporation, CBN | noted |  |  |
| R4-2101959 | Discussion on irregular channel bandwidth for NR system | ZTE Corporation | noted |  |  |
| R4-2101960 | Discussion on IAB test configurations | ZTE Corporation | noted |  |  |
| R4-2101961 | Discussion on IAB test model | ZTE Corporation | noted |  |  |
| R4-2101962 | Discussion on IAB conformance testing | ZTE Corporation | noted |  |  |
| R4-2101963 | Discussion on NR based repeater | ZTE Corporation | noted |  |  |
| R4-2101964 | Discussion on simulation assumptions for NTN coexistence study | ZTE Corporation | noted |  |  |
| R4-2101965 | Discussion on Europe unlicensed 6GHz for NR-U | ZTE Corporation | noted |  |  |
| R4-2101966 | Discussion on BS RF requirements for Europe unlicensed 6GHz | ZTE Corporation | noted |  |  |
| R4-2101967 | draft CR for introduction of Europe unlicensed 6GHz. | ZTE Corporation | not pursued |  |  |
| R4-2101968 | CR to TS 38.104: system parameters maintenance for NR-U | ZTE Corporation | revised |  | R4-2103137 |
| R4-2101969 | CR to TS 38.104: system parameters maintenance for NR-U | ZTE Corporation | agreed |  |  |
| R4-2101970 | CR to TS 38.101-1: system parameters maintenance for NR-U | ZTE Corporation | revised |  | R4-2103138 |
| R4-2101971 | CR to TS 38.101-1: system parameters maintenance for NR-U | ZTE Corporation | agreed |  |  |
| R4-2101972 | CR to TS 36.104: corrections of NR-U BS RF requirements | ZTE Corporation | revised |  | R4-2103973 |
| R4-2101973 | CR to TS 36.104: corrections of NR-U BS RF requirements | ZTE Corporation | agreed |  |  |
| R4-2101974 | CR to TS 37.104: corrections of NR-U BS RF requirements | ZTE Corporation | revised |  | R4-2103974 |
| R4-2101975 | CR to TS 37.104: corrections of NR-U BS RF requirements | ZTE Corporation | agreed |  |  |
| R4-2101976 | CR to TS 37.105: corrections of NR-U BS RF requirements | ZTE Corporation | not pursued |  |  |
| R4-2101977 | CR to TS 37.105: corrections of NR-U BS RF requirements | ZTE Corporation | withdrawn |  |  |
| R4-2101978 | CR to TS 38.104: corrections of NR-U BS Tx requirements | ZTE Corporation | revised |  | R4-2103888 |
| R4-2101979 | CR to TS 38.104: corrections of NR-U BS Tx requirements | ZTE Corporation | agreed |  |  |
| R4-2101980 | CR to TS 38.104: corrections of NR-U BS Rx requirements | ZTE Corporation | revised |  | R4-2103975 |
| R4-2101981 | CR to TS 38.104: corrections of NR-U BS Rx requirements | ZTE Corporation | agreed |  |  |
| R4-2101982 | CR to TS 38.141-1: introduction of NR-U BS | ZTE Corporation | postponed |  |  |
| R4-2101983 | CR to TS 38.141-1: introduction of NR-U BS | ZTE Corporation | withdrawn |  |  |
| R4-2101984 | CR to TS 36.141: introduction of NR-U BS | ZTE Corporation | postponed |  |  |
| R4-2101985 | CR to TS 36.141: introduction of NR-U BS | ZTE Corporation | withdrawn |  |  |
| R4-2101986 | CR to TS 38.141-2: Introduction of 35MHz and 45MHz | ZTE Corporation | not pursued |  | - |
| R4-2101987 | CR to 37.145-1: Introduction of 35MHz and 45MHz | ZTE Corporation | revised |  | R4-2103196 |
| R4-2101988 | Simplification of n70 | Dish Network | agreed |  |  |
| R4-2101989 | CR for 38.307 to delete the redundant information | Huawei, HiSilicon | not pursued |  |  |
| R4-2101990 | CR for 38.307 to delete the redundant information | Huawei, HiSilicon | withdrawn |  |  |
| R4-2101991 | CR for 38.307 to delete the redundant information | Huawei, HiSilicon | withdrawn |  |  |
| R4-2101992 | Simplification of n70 | Dish Network | agreed |  |  |
| R4-2101993 | Minimum number of slots for FR1 MIMO OTA testing | CAICT,vivo,OPPO,Huawei,HiSilicon,xiaomi,Samsung | noted |  |  |
| R4-2101994 | CR to TS 38.104: EESS protection requirement correction | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2101995 | CR to TS 38.104: EESS protection requirement correction | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2101996 | CR to TS 38.104: EESS protection requirement correction | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2101997 | CR to TS 38.141-2: EESS protection requirement correction | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2101998 | CR to TS 38.141-2: EESS protection requirement correction | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2101999 | CR to TS 38.141-2: EESS protection requirement correction | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102000 | CR to TS 38.141-1: Receiver requirement corrections | Nokia, Nokia Shanghai Bell | revised |  | R4-2103793 |
| R4-2102001 | CR to TS 38.141-1: Receiver requirement corrections | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102002 | CR to TS 38.141-1: Receiver requirement corrections | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102003 | CR to TS 38.141-2: Receiver requirement corrections | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102004 | CR to TS 38.141-2: Receiver requirement corrections | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102005 | CR to TS 38.141-2: Receiver requirement corrections | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102006 | Numerology and channel bandwidth discussion for NR beyond 52.6 GHz | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102007 | TP to TR 38.808: BS RF for NR beyond 52.6 GHz | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102008 | TP to TR 38.808: Phase noise considerations | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102009 | TP to TR 38.808: Timing considerations for operation between 52.6 and 71 GHz | Nokia, Nokia Shanghai Bell | revised |  | R4-2103260 |
| R4-2102010 | UE RF for NR beyond 52.6 GHz | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102011 | DraftCR to TS 38.174: Receiver requirement corrections | Nokia, Nokia Shanghai Bell | endorsed |  |  |
| R4-2102012 | IAB EVM procedure | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102013 | Dynamic range test points | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102014 | IAB RF conformance testing burden considerations | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102015 | Test setup considerations | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102016 | Manufacturer declaration framework | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102017 | Receiver RF testing considerations | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102018 | NR repeater considerations | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102019 | TP to TR TR 37.717-31-11 to include DC_1A-7A-28A_n3A | Ericsson | approved |  |  |
| R4-2102020 | TP to TR 37.717-31-11 to include 2A-12A-66A_n41A, 2A-2A-12A-66A_n41A | Ericsson | approved |  |  |
| R4-2102021 | TP to TR 37.717-31-11 to include 2A-66A-71A_n41A, 2A-2A-66A-71A_n41A | Ericsson | approved |  |  |
| R4-2102022 | TP to TR 37.717-31-11 to include 2A-7A-12A_n66A, 2A-2A-7A-12A_n66A | Ericsson | approved |  |  |
| R4-2102023 | TP to TR 37.717-31-11 to include 2A-2A-5A-7A_n66A | Ericsson | approved |  |  |
| R4-2102024 | TP to TR 37.717-31-11 to include 2A-7A-71A_n66A, 2A-2A-7A-71A_n66A | Ericsson | approved |  |  |
| R4-2102025 | TP to TR 37.717-31-11 to include 2A-7A-12A_n78A, 2A-2A-7A-12A_n78A | Ericsson | approved |  |  |
| R4-2102026 | TP to TR 37.717-31-11 to include 2A-12A-66A_n78A, 2A-2A-12A-66A_n78A | Ericsson | approved |  |  |
| R4-2102027 | TP to TR 37.717-31-11 to include 7A-12A-66A_n78A | Ericsson | approved |  |  |
| R4-2102028 | TP to TR 37.717-31-11 to include 7A-66A-71A_n78A | Ericsson | approved |  |  |
| R4-2102029 | TP to TR 37.717-31-11 to include 2A-7A-71A_n78A, 2A-2A-7A-71A_n78A | Ericsson | approved |  |  |
| R4-2102030 | TP to TR 37.717-31-11 to include 2A-7A-66A_n2A | Ericsson | approved |  |  |
| R4-2102031 | TP to TR 37.717-31-11 to include 2A-5A-7A_n2A | Ericsson | approved |  |  |
| R4-2102032 | TP to TR 37.717-31-11 to include 5A-7A-66A_n2A | Ericsson | approved |  |  |
| R4-2102033 | TP to TR 37.717-31-11 to include 2A-7A-71A_n2A | Ericsson | approved |  |  |
| R4-2102034 | TP to TR 37.717-31-11 to include 2A-66A-71A_n2A | Ericsson | approved |  |  |
| R4-2102035 | TP to TR 37.717-31-11 to include 2A-7A-12A_n2A | Ericsson | approved |  |  |
| R4-2102036 | TP to TR 37.717-31-11 to include 7A-66A-71A_n2A | Ericsson | approved |  |  |
| R4-2102037 | 7TP to TR 37.717-31-11 to include 7A-12A-66A_n2A | Ericsson | approved |  |  |
| R4-2102038 | TP to TR 37.717-41-11 to include 2A-7A-12A-66A_n78A, 2A-2A-7A-12A-66A_n78A | Ericsson | approved |  |  |
| R4-2102039 | TP to TR 37.717-41-11 to include 2A-7A-66A-71A_n78A, 2A-2A-7A-66A-71A_n78A | Ericsson | approved |  |  |
| R4-2102040 | TP to TR 37.717-41-11 to include 2A-5A-7A-66A_n2A | Ericsson | approved |  |  |
| R4-2102041 | TP to TR 37.717-41-11 to include 2A-7A-66A-71A_n2A | Ericsson | approved |  |  |
| R4-2102042 | TP to TR 37.717-41-11 to include 2A-7A-12A-66A_n2A | Ericsson | approved |  |  |
| R4-2102043 | TP to TR 38.808 on Spectrum regulatory situation | Ericsson | noted |  |  |
| R4-2102044 | LS reply to RAN2 on power control for NR-DC | Ericsson | noted |  |  |
| R4-2102045 | draft CR 38.101-3 adding CA_n7B UL configurations for 1 LTE + 1 NR | Ericsson | revised |  | R4-2103029 |
| R4-2102046 | draft CR 38.101-3 adding CA_n7B UL configurations for 2 LTE + 1 NR | Ericsson | revised |  | R4-2103030 |
| R4-2102047 | draft CR 38.101-3 adding CA_n7B UL configurations for 3 LTE + 1 NR | Ericsson | revised |  | R4-2103032 |
| R4-2102048 | draft CR 38.101-3 adding CA_n7B UL configurations for x LTE + 2 NR | Ericsson | revised |  | R4-2103066 |
| R4-2102049 | 47GHz band TT for NR BS RF requirement | Keysight Technologies UK Ltd | noted |  |  |
| R4-2102050 | TP for TR 37.717-21-11 to include 7A-71A_n78A | Ericsson, Rogers | approved |  |  |
| R4-2102051 | draft CR to include DC_2-7-66_n71, DC_2-7-66_n78 configurations | Ericsson, Rogers | endorsed |  |  |
| R4-2102052 | TP for TR 37.717-11-21 to include 2A_n71a-n78a | Ericsson, Rogers | approved |  |  |
| R4-2102053 | TP for TR 37.717-11-21 to include 7A_n71a-n78a | Ericsson, Rogers | approved |  |  |
| R4-2102054 | TP for TR 37.717-11-21 to include 66A_n71a-n78a | Ericsson, Rogers | approved |  |  |
| R4-2102055 | TP for TR 37.717-11-21 to include 2A_n38a-n71a | Ericsson, Rogers | approved |  |  |
| R4-2102056 | TP for TR 37.717-11-21 to include 66A_n38a-n71a | Ericsson, Rogers | approved |  |  |
| R4-2102057 | TP for TR 37.717-11-21 to include 71A_n38a-n78a | Ericsson, Rogers | approved |  |  |
| R4-2102058 | TP for TR 37.717-11-21 to include 5A_n66a-n78a | Ericsson, Rogers | approved |  |  |
| R4-2102059 | TP for TR 37.717-11-21 to include 71A_n66a-n78a | Ericsson, Rogers | approved |  |  |
| R4-2102060 | TP for TR 37.717-11-21 to include 5A_n38a-n66a | Ericsson, Rogers | approved |  |  |
| R4-2102061 | TP for TR 37.717-11-21 to include 71A_n38a-n66a | Ericsson, Rogers | approved |  |  |
| R4-2102062 | TP for TR 37.717-11-21 to include 2A_n2a-n38a | Ericsson, Rogers | approved |  |  |
| R4-2102063 | TP for TR 37.717-11-21 to include 66A_n2a-n38a | Ericsson, Rogers | approved |  |  |
| R4-2102064 | TP for TR 37.717-11-21 to include 12A_n2a-n38a | Ericsson, Rogers | approved |  |  |
| R4-2102065 | TP for TR 37.717-11-21 to include 2A_n2a-n41a | Ericsson, Rogers | approved |  |  |
| R4-2102066 | TP for TR 37.717-11-21 to include 12A_n2a-n41a | Ericsson, Rogers | approved |  |  |
| R4-2102067 | TP for TR 37.717-11-21 to include 71A_n2a-n41a | Ericsson, Rogers | approved |  |  |
| R4-2102068 | TP for TR 37.717-11-21 to include 2A_n2a-n78a | Ericsson, Rogers | revised |  | R4-2103067 |
| R4-2102069 | TP for TR 37.717-11-21 to include 7A_n2a-n78a | Ericsson, Rogers | approved |  |  |
| R4-2102070 | TP for TR 37.717-11-21 to include 71A_n2a-n78a | Ericsson, Rogers | revised |  | R4-2103068 |
| R4-2102071 | TP for TR 37.717-11-21 to include 2A_n66a-n71a | Ericsson, Rogers | approved |  |  |
| R4-2102072 | TP for TR 37.717-11-21 to include 7A_n66a-n71a | Ericsson, Rogers | approved |  |  |
| R4-2102073 | TP for TR 37.717-11-21 to include 66A_n66a-n71a | Ericsson, Rogers | approved |  |  |
| R4-2102074 | TP for TR 37.717-11-21 to include 2A_n2a-n71a | Ericsson, Rogers | approved |  |  |
| R4-2102075 | TP for TR 37.717-11-21 to include 7A_n2a-n71a | Ericsson, Rogers | approved |  |  |
| R4-2102076 | TP for TR 37.717-11-21 to include 66A_n2a-n71a | Ericsson, Rogers | approved |  |  |
| R4-2102077 | TP for TR 37.717-11-21 to include 2A_n2a-n66a | Ericsson, Rogers | approved |  |  |
| R4-2102078 | TP for TR 37.717-11-21 to include 7A_n2a-n66a | Ericsson, Rogers | approved |  |  |
| R4-2102079 | TP for TR 37.717-11-21 to include 66A_n2a-n66a | Ericsson, Rogers | revised |  | R4-2103069 |
| R4-2102080 | TP for TR 37.717-11-21 to include 71A_n2a-n66a | Ericsson, Rogers | approved |  |  |
| R4-2102081 | CR to TR38.827 Number of Slots for FR1 MIMO OTA test | CAICT,vivo,OPPO,Huawei,HiSilicon,xiaomi,Samsung | withdrawn |  |  |
| R4-2102082 | Discussion on performance requirements for NR-U | MediaTek inc. | noted |  |  |
| R4-2102083 | Simulation results on PDSCH performance requirements for multi-DCI based multi-TRP transmission | MediaTek inc. | noted |  |  |
| R4-2102084 | Simulation results for URLLC pre-emption | MediaTek inc. | noted |  |  |
| R4-2102085 | Simulation results on PDCCH-WUS for power saving | MediaTek inc. | noted |  |  |
| R4-2102086 | CR for TS38.101-4, test for FR1 FDD PDCCH DCI format 2_6 demodulation | MediaTek inc. | withdrawn |  |  |
| R4-2102087 | IAB-MT conformance Test setup and Test Equipment choice | Keysight Technologies UK Ltd | noted |  |  |
| R4-2102088 | Discussion on test time reduction methods | Rohde & Schwarz | noted |  |  |
| R4-2102089 | Discussion on FR1 EVM measurements | Rohde & Schwarz | noted |  |  |
| R4-2102090 | Discussion on FR2 UL demodulation measurements | Rohde & Schwarz | noted |  |  |
| R4-2102091 | Improvement of UL RMC tables | Rohde & Schwarz | noted |  |  |
| R4-2102092 | On NR IAB-DU demodulation requirements | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102093 | Simulation results for HST in FR2 | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102094 | CR for 38.101-3 to introduce a new MSD due to the counter intermodulation interference(Rel-15) | Huawei, HiSilicon | revised |  | R4-2103357 |
| R4-2102095 | CR for 38.101-3 to introduce a new MSD due to the counter intermodulation interference(Rel-16) | Huawei, HiSilicon | agreed |  |  |
| R4-2102096 | CR for 38.101-3 to introduce a new MSD due to the counter intermodulation interference(Rel-17) | Huawei, HiSilicon | agreed |  |  |
| R4-2102097 | On NR IAB-MT testing setup and demodulation requirements | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102098 | Test frequencies for NB-IOT UE in standalone operation | Sony | noted |  |  |
| R4-2102099 | On high-speed train deployment scenario in FR2 | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102100 | CR to TS 38.101-4: n262 demodulation requirements | Ericsson | postponed |  |  |
| R4-2102101 | pCR to 38.847: UE performance requirements | Ericsson | postponed |  |  |
| R4-2102102 | pCR to TR 38.847: BS demodulation requirements | Ericsson | postponed |  |  |
| R4-2102103 | HST FR2 general aspects | Ericsson | noted |  |  |
| R4-2102104 | HST FR2 deployment aspects | Ericsson | noted |  |  |
| R4-2102105 | IAB demodulation general considerations | Ericsson | noted |  |  |
| R4-2102106 | IAB demodulation DU considerations | Ericsson | noted |  |  |
| R4-2102107 | IAB demodulation MT considerations | Ericsson | noted |  |  |
| R4-2102108 | Repeaters WI general considerations | Ericsson | noted |  |  |
| R4-2102109 | Conducted transmit power | Ericsson | noted |  |  |
| R4-2102110 | Conducted unwanted emissions | Ericsson | noted |  |  |
| R4-2102111 | Conducted other requirements | Ericsson | noted |  |  |
| R4-2102112 | Radiated transmit power | Ericsson | noted |  |  |
| R4-2102113 | Radiated unwanted emissions | Ericsson | noted |  |  |
| R4-2102114 | Radiated other requirements | Ericsson | noted |  |  |
| R4-2102115 | Simulation results for BS URLLC high BLER | Ericsson | noted |  |  |
| R4-2102116 | Discussions on URLLC UE CQI reporting requirements for CQI table 3 | Ericsson | noted |  |  |
| R4-2102117 | Simulation results on UE URLLC demodulation performance requirements with higher BLER | Ericsson | noted |  |  |
| R4-2102118 | Simulation results on URLLC UE CQI reporting requirements for CQI table 3 | Ericsson | noted |  |  |
| R4-2102119 | Discussion on UE URLLC demodulation performance requirements with higher BLER | Ericsson | noted |  |  |
| R4-2102120 | CR to TS 38.101-4: Performance requirements for URLLC PDSCH 0.001% BLER | Ericsson | revised |  | R4-2103890 |
| R4-2102121 | CR to TS 38.101-4: Performance requirements for URLLC High BLER feature tests | Ericsson | revised |  | R4-2103903 |
| R4-2102122 | CR to TS 38.141-1 Update of 0.001% BLER test requirements | Ericsson | revised |  | R4-2103895 |
| R4-2102123 | CR to TS 38.141-1 Update of 0.001% BLER test requirements | Ericsson | agreed |  |  |
| R4-2102124 | CR to TS 38.141-2 Update of 0.001% BLER test requirements | Ericsson | revised |  | R4-2103896 |
| R4-2102125 | CR to TS 38.141-2 Update of 0.001% BLER test requirements | Ericsson | agreed |  |  |
| R4-2102126 | CR to TS 38.175: Performance criteria for IAB | ZTE Corporation | withdrawn |  |  |
| R4-2102127 | CR to TS 38.113: Radiated emission test method | ZTE Corporation | withdrawn |  |  |
| R4-2102128 | CR to TS 38.113: Radiated emission test method | ZTE Corporation | withdrawn |  |  |
| R4-2102129 | Proposal on the skeleton of NR Repeaters EMC | ZTE Corporation | noted |  |  |
| R4-2102130 | Discussion on NR repeaters EMC | ZTE Corporation | noted |  |  |
| R4-2102131 | Discussion on FR2 Inter-band DL CA enhancements | ZTE Corporation | noted |  |  |
| R4-2102132 | Discussion on frequency separation class | ZTE Corporation | withdrawn |  |  |
| R4-2102133 | Discussion on PC2 intra-band contiguous NR CA RF requirements | ZTE Corporation | noted |  |  |
| R4-2102134 | Discussion on PC2 intra-band non-contiguous NR CA | ZTE Corporation | withdrawn |  |  |
| R4-2102135 | Discussion on SAR scheme for HPUE FDD | ZTE Corporation | noted |  |  |
| R4-2102136 | Templates for BCS4 configurations for inter-band NR CA | ZTE Corporation | withdrawn |  |  |
| R4-2102137 | Discussion on UE capabilities signalling to enable BCS4 | ZTE Corporation | noted |  |  |
| R4-2102138 | Discussion on UE requirements for CA configurations based on IBM | ZTE Corporation | withdrawn |  |  |
| R4-2102139 | Further discussion on SAR solution for NR PC2 inter-band CA | ZTE Corporation | noted |  |  |
| R4-2102140 | Further discussion on SAR solution for NR PC2 SUL | ZTE Corporation | noted |  |  |
| R4-2102141 | Further discussion on spectrum utilization for 35MHz and 45MHz | ZTE Corporation | noted |  |  |
| R4-2102142 | Introduction of 35MHz and 45 MHz bandwidths to TS38.101-1 | ZTE Corporation | withdrawn |  |  |
| R4-2102143 | CR to TS38.101-1: Correction on applicability of minimum requirements | ZTE Corporation | withdrawn |  |  |
| R4-2102144 | TS 37.145-2: Rx blocking limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U) | Ericsson | postponed |  |  |
| R4-2102145 | TS 37.145-2: RX blocking limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U) | Ericsson | withdrawn |  |  |
| R4-2102146 | CR for 38.101-3: Correction for CA_n66A-n260 | T-Mobile USA | agreed |  |  |
| R4-2102147 | CR for 38.101-3: Correction for CA_n66A-n260A | T-Mobile USA | agreed |  |  |
| R4-2102148 | Discussion on LS RP-202935 BCS reporting and support for intra-band EN-DC band combinations | T-Mobile USA, Bell Mobility, TELUS, Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102149 | Reply LS to RP-202935 on BCS reporting and support for intra-band EN-DC band combinations | RAN4 | approved |  |  |
| R4-2102150 | Discussion on BCS4 | T-Mobile USA | noted |  |  |
| R4-2102151 | Draft CR for 38.101-1: Introduction of BCS4 | T-Mobile USA, MediaTek | revised |  | R4-2103376 |
| R4-2102152 | CR for 38.101-1: Add CA_n25A-n41(2A)-n71A which was missing in the CR implementation | T-Mobile USA | agreed |  |  |
| R4-2102153 | CR for 38.101-1: Add CA_n25A-n41(2A)-n71A which was missing in the CR implementation | T-Mobile USA | agreed |  |  |
| R4-2102154 | SI on IMT parameters - DL simulations results | Ericsson | noted |  |  |
| R4-2102155 | SI on IMT parameters - UL simulations results | Ericsson | noted |  |  |
| R4-2102156 | SI on IMT parameters - Remaining BS parameters | Ericsson | noted |  |  |
| R4-2102157 | SI on IMT parameters - Remaining UE parameters | Ericsson | noted |  |  |
| R4-2102158 | TR 38.847 Introduction of NR Band 262 (47GHz band) | Ericsson | agreed |  |  |
| R4-2102159 | CR to TS 38.104 - n262 introduction | Ericsson | postponed |  |  |
| R4-2102160 | 47GHz band - Measurement uncertainties for BS requirements | Ericsson | noted |  |  |
| R4-2102161 | Extended 600MHz band - frequency arrangement | Ericsson | noted |  |  |
| R4-2102162 | Extended 600MHz band - Regulatory aspects | Ericsson | noted |  |  |
| R4-2102163 | Revised Basket WID on adding channel bandwidth support to existing NR bands | Ericsson | revised |  | R4-2103179 |
| R4-2102164 | Big CR to TS 38.104 - New CBW Basket WI | Ericsson | agreed |  |  |
| R4-2102165 | Big CR to TS 38.101-1 - New CBW Basket WI | Ericsson | revised |  | R4-2103180 |
| R4-2102166 | New channel BW in bands n2, n5 and n48 - A-MPR | Ericsson | noted |  |  |
| R4-2102167 | New channel BW in bands n2, n5 and n48 - REFSENS | Ericsson | noted |  |  |
| R4-2102168 | Basket WID on new CBW - Rapporteur | Ericsson | noted |  |  |
| R4-2102169 | New NR band n67 - BS RF impacts | Ericsson | noted |  |  |
| R4-2102170 | New NR band n67 - UE RF impacts | Ericsson | noted |  |  |
| R4-2102171 | New NR band n85 - BS RF impacts | Ericsson | noted |  |  |
| R4-2102172 | New NR band n85 - UE RF impacts | Ericsson | noted |  |  |
| R4-2102173 | NTN - Regulatory and spectrum aspects | Ericsson | noted |  |  |
| R4-2102174 | NTN Simulations assumptions discussion | Ericsson | noted |  |  |
| R4-2102175 | NTN Reference model | Ericsson | noted |  |  |
| R4-2102176 | NTN - BS requirements overview | Ericsson | noted |  |  |
| R4-2102177 | CR to TS 38.175: Performance criteria for IAB | ZTE Corporation | not pursued |  |  |
| R4-2102178 | CR to TS 38.113: Radiated emission test method | ZTE Corporation | revised |  | R4-2103772 |
| R4-2102179 | CR to TS 38.113: Radiated emission test method | ZTE Corporation | agreed |  |  |
| R4-2102180 | Proposal on the skeleton of NR Repeaters EMC | ZTE Corporation | noted |  |  |
| R4-2102181 | Discussion on NR repeaters EMC | ZTE Corporation | noted |  |  |
| R4-2102182 | Discussion on FR2 Inter-band DL CA enhancements | ZTE Corporation | noted |  |  |
| R4-2102183 | Discussion on frequency separation class | ZTE Corporation | noted |  |  |
| R4-2102184 | Discussion on PC2 intra-band contiguous NR CA RF requirements | ZTE Corporation | noted |  |  |
| R4-2102185 | Discussion on PC2 intra-band non-contiguous NR CA | ZTE Corporation | noted |  |  |
| R4-2102186 | Discussion on SAR scheme for HPUE FDD | ZTE Corporation | noted |  |  |
| R4-2102187 | Templates for BCS4 configurations for inter-band NR CA | ZTE Corporation | noted |  |  |
| R4-2102188 | Discussion on UE capabilities signalling to enable BCS4 | ZTE Corporation | noted |  |  |
| R4-2102189 | Discussion on UE requirements for CA configurations based on IBM | ZTE Corporation | noted |  |  |
| R4-2102190 | Further discussion on SAR solution for NR PC2 inter-band CA | ZTE Corporation | noted |  |  |
| R4-2102191 | Further discussion on SAR solution for NR PC2 SUL | ZTE Corporation | noted |  |  |
| R4-2102192 | Further discussion on spectrum utilization for 35MHz and 45MHz | ZTE Corporation | noted |  |  |
| R4-2102193 | Introduction of 35MHz and 45 MHz bandwidths to TS38.101-1 | ZTE Corporation | revised |  | R4-2103189 |
| R4-2102194 | CR to TS38.101-1: Correction on applicability of minimum requirements | ZTE Corporation | agreed |  |  |
| R4-2102195 | CR to TS38.101-1: Correction on applicability of minimum requirements | ZTE Corporation | agreed |  |  |
| R4-2102196 | CR to TS38.101-1: Correction on applicability of minimum requirements | ZTE Corporation | agreed |  |  |
| R4-2102197 | CR to TS38.101-1: Correction on the Aggregated Channel Bandwidth | ZTE Corporation | revised |  | R4-2103116 |
| R4-2102198 | CR to TS38.101-1: Correction on the Aggregated Channel Bandwidth | ZTE Corporation | agreed |  |  |
| R4-2102199 | CR to TS38.101-1: Correction on the Aggregated Channel Bandwidth | ZTE Corporation | agreed |  |  |
| R4-2102200 | CR to TS38.104: Correction on the Aggregated Channel Bandwidth | ZTE Corporation | revised |  | R4-2103117 |
| R4-2102201 | CR to TS38.104: Correction on the Aggregated Channel Bandwidth | ZTE Corporation | agreed |  |  |
| R4-2102202 | CR to TS38.104: Correction on the Aggregated Channel Bandwidth | ZTE Corporation | agreed |  |  |
| R4-2102203 | CR to TS38.101-1: Correction on configured transmitted power requirement | ZTE Corporation | agreed |  |  |
| R4-2102204 | CR to TS38.101-1: Correction on configured transmitted power requirement | ZTE Corporation | agreed |  |  |
| R4-2102205 | CR to TS38.101-3: Correction on duty cycle signalling terminology for PC2 inter-band ENDC | ZTE Corporation | agreed |  |  |
| R4-2102206 | CR to TS38.101-3: Correction on duty cycle signalling terminology for PC2 inter-band ENDC | ZTE Corporation | agreed |  |  |
| R4-2102207 | CR to TS 38.307 on the definition of the duplex-mode for the band configurations | ZTE Corporation, CHTTL | not pursued |  |  |
| R4-2102208 | CR to TS 38.307 on the definition of the duplex-mode for the band configurations | ZTE Corporation, CHTTL | withdrawn |  |  |
| R4-2102209 | CR to TS 38.307 on the definition of the duplex-mode for the band configurations | ZTE Corporation, CHTTL | withdrawn |  |  |
| R4-2102210 | TP for TR38.717-02-01_Removal of the sub-clauses under clause 7 | ZTE Corporation | approved |  |  |
| R4-2102211 | draft CR to TS38.101-3: Adding CA_n39A-n258A | ZTE Corporation | endorsed |  |  |
| R4-2102212 | TP for TR 38.716-02-00:CA_n34A-n40A | ZTE Corporation | withdrawn |  |  |
| R4-2102213 | draft CR to TS38.101-3: Adding DC_40A_n41C and DC_40A_n41(2A) | ZTE Corporation | endorsed |  |  |
| R4-2102214 | TP for 37.717-11-21_ DC_39_n40-n258 | ZTE Corporation | approved |  |  |
| R4-2102215 | TP for 37.717-11-21_ DC_39_n41-n258 | ZTE Corporation | approved |  |  |
| R4-2102216 | TP for 37.717-11-21_ DC_39_n79-n258 | ZTE Corporation | approved |  |  |
| R4-2102217 | TP for TR38.717-03-01_ CA_n8A-n39A-n41A | ZTE Corporation | approved |  |  |
| R4-2102218 | TP for TR 37.717-33_DC_40A_n41A-n258A | ZTE Corporation | approved |  |  |
| R4-2102219 | TP for 37.717-11-31_ DC_8A_n39A-n40A-n41A | ZTE Corporation | approved |  |  |
| R4-2102220 | TP for TR38.xxx_Clarification on PC2 CA_n28A-n41A, CA_n28-n79A and CA_n40A-41A | ZTE Corporation, CMCC | approved |  |  |
| R4-2102221 | TP for TR38.xxx_ PC2 CA_n41A-n79A | ZTE Corporation, CMCC, Xiaomi | noted |  |  |
| R4-2102222 | Revised WID on Rel-17 NR Inter-band CA_DC xUL_2DL (x=1,2) | ZTE Corporation | not concluded |  |  |
| R4-2102223 | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into TS 38.101-1 | ZTE Corporation | withdrawn |  | - |
| R4-2102224 | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into TS 38.101-3 | ZTE Corporation | withdrawn |  | - |
| R4-2102225 | Revised WID on Rel-17 NR Inter-band Carrier AggregationDual Connectivity for 3 bands DL with 2 bands UL | ZTE Corporation | not concluded |  |  |
| R4-2102226 | CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into TS 38.101-1 | ZTE Corporation | withdrawn |  | - |
| R4-2102227 | CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into TS 38.101-3 | ZTE Corporation | withdrawn |  | - |
| R4-2102228 | Revised WID on Rel-17 Dual Connectivity (DC) x bands (x=1,2) LTE inter-band CA (xDL/xUL) and y bands (y=3-x) NR inter-band CA | ZTE Corporation | not concluded |  |  |
| R4-2102229 | CR to reflect the completed ENDC combinations for 3 bands DL with 3 bands UL into TS 38.101-3 | ZTE Corporation | withdrawn |  | - |
| R4-2102230 | TR 37.717-33 v0.2.0 | ZTE Corporation | not concluded |  |  |
| R4-2102231 | Revised WID on Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL1UL) and 3 bands NR inter-band CA (3DL1UL) | ZTE Corporation | not concluded |  |  |
| R4-2102232 | TR 37.717-11-31_v0.3.0 | ZTE Corporation | not concluded |  |  |
| R4-2102233 | Work plan of Rel-17 enhancements for NB-IoT and LTE-MTC | Huawei, Ericsson | not treated |  |  |
| R4-2102234 | Correction to PUR requirements for NB1 UE | Ericsson | withdrawn |  |  |
| R4-2102235 | Correction to PUR requirements for NB1 UE | Ericsson | not pursued |  |  |
| R4-2102236 | Inclusive language review in TS 36.133 | Ericsson | revised |  | R4-2103254 |
| R4-2102237 | Maintenance CR for NR-U core requirements | Ericsson | agreed |  |  |
| R4-2102238 | Maintenance CR for NR-U core requirements | Ericsson | agreed |  |  |
| R4-2102239 | Updated simulation assumptions for evaluating UE power saving for RLM and BM | Ericsson | noted |  |  |
| R4-2102240 | Simulation results on UE power saving for RLM and BM | Ericsson | noted |  |  |
| R4-2102241 | Discussions on UE power saving for RLM and BM | Ericsson | noted |  |  |
| R4-2102242 | Discussions on handover test cases for NR-U | Ericsson | noted |  |  |
| R4-2102243 | Introduction of NR-U cell reselection tests | Ericsson | revised |  | R4-2103531 |
| R4-2102244 | Discussions on cell reselection test cases for NR-U | Ericsson | noted |  |  |
| R4-2102245 | Changes to cell reselection tests under power saving | Ericsson | withdrawn |  |  |
| R4-2102246 | Changes to cell reselection tests under power saving | Ericsson | postponed |  | R4-2103576 |
| R4-2102247 | Discussions on RRM maintenance issues for Rel-16 eMTC UE | Ericsson | noted |  |  |
| R4-2102248 | Clarification for performance requirements tests for euCA | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102249 | Clarification for performance requirements tests for euCA | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102250 | Correction of band group notation for FR2 | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102251 | Correction of band group notation for FR2 | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102252 | Correction to Idle Mode CA/DC Measurements for Inactive mode | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102253 | Correction to Idle Mode CA/DC Measurements for Inactive mode | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102254 | CR clarifying the UE measurement requirements for an SCell with dormant BWP | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102255 | CR clarifying the UE measurement requirements for an SCell with dormant BWP | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102256 | Discussion related to multiple SCell dormancy BWP switch | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102257 | Correction to simultaneous DCI based BWP switch delay on multiple CCs | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102258 | Correction to simultaneous DCI based BWP switch delay on multiple CCs | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102259 | Draft CR for test case 7 for Dormant SCell BWP switch delay | Nokia, Nokia Shanghai Bell | revised |  | R4-2103570 |
| R4-2102260 | Draft CR for NR FR1 Intra frequency handover with direct SCell activation | Nokia, Nokia Shanghai Bell | revised |  | R4-2103561 |
| R4-2102261 | Draft CR for Idle Mode measurements of inter-frequency CA candidate cells for early reporting | Nokia, Nokia Shanghai Bell | endorsed |  |  |
| R4-2102262 | Draft Big CR: Introduction of Rel-16 MR-DC EMR RRM performance requirements (TS 38.133) | Nokia, Nokia Shanghai Bell | revised |  | R4-2103548 |
| R4-2102263 | Draft CR for Removal of brackets for idle mode CA measurement accuracy requirements | Nokia, Nokia Shanghai Bell | revised |  | R4-2103549 |
| R4-2102264 | Measurement Performance Requirements test for MR-DC | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102265 | Test case for RRC based UL spatial relation switch associated with a known DL-RS in SA for periodic SRS | Nokia, Nokia Shanghai Bell | revised |  | R4-2103607 |
| R4-2102266 | NR support for high speed train scenario in frequency range 2 (FR2) | Nokia, Nokia Shanghai Bell, Samsung | revised |  | R4-2103239 |
| R4-2102267 | Discussion on FR2 RF RRM | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102268 | Discussion on Pre-configured MG pattern(s) | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102269 | Discussion on multiple concurrent and independent MG patterns | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102270 | General issues on BS demodulation performance requirements for operation in unlicensed bands | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R4-2102271 | PUSCH Demodulation performance requirements for operation in unlicensed bands | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102272 | NR-U PUSCH simulation results | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R4-2102273 | PUCCH Demodulation performance requirements for operation in unlicensed bands | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R4-2102274 | NR-U PUCCH simulation results | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R4-2102275 | PRACH Demodulation performance requirements for operation in unlicensed bands | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R4-2102276 | NR-U PRACH simulation results | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R4-2102277 | 2-step RACH RRM performance requirements corrections | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R4-2102278 | 2-step RACH RRM performance requirements corrections | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R4-2102279 | Corrections on 2-step RACH demodulation requirements | Nokia, Nokia Shanghai Bell, Ericsson | withdrawn |  |  |
| R4-2102280 | Corrections on 2-step RACH demodulation requirements | Nokia, Nokia Shanghai Bell, Ericsson | withdrawn |  |  |
| R4-2102281 | 2-step RACH simulation results | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R4-2102282 | On PMI reporting requirements for enhanced Type II codebooks | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102283 | Consideration on adding PC 1.5 for n77 and n78 | Huawei, HiSilicon | noted |  |  |
| R4-2102284 | n77(3/4A) DL CA UE Architecture, Regional Needs and 4x4 DL MIMO Support | Skyworks Solutions Inc. | noted |  |  |
| R4-2102285 | Correction on requirements for PUSCH repetition Type A | Nokia, Nokia Shanghai Bell | merged |  |  |
| R4-2102286 | Correction on requirements for PUSCH repetition Type A | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R4-2102287 | On the SAR solutions for UL CA band combinations | Huawei, HiSilicon | noted |  |  |
| R4-2102288 | Network and UE options for the support of irregular channel bandwidth | Skyworks Solutions Inc. | noted |  |  |
| R4-2102289 | On the SAR solutions for SUL band combinations | Huawei, HiSilicon | noted |  |  |
| R4-2102290 | On PRS-RSTD measurement requirements | Qualcomm Incorporated | noted |  |  |
| R4-2102291 | On UE Rx-Tx measurement requirements | Qualcomm Incorporated | noted |  |  |
| R4-2102292 | On general PRS measurement requirements | Qualcomm Incorporated | noted |  |  |
| R4-2102293 | On PRS-RSTD measurement accuracy requirements | Qualcomm Incorporated | noted |  |  |
| R4-2102294 | On UE Rx-Tx measurement accuracy requirements | Qualcomm Incorporated | noted |  |  |
| R4-2102295 | On design of test cases for NR positioning | Qualcomm Incorporated | noted |  |  |
| R4-2102296 | On RSS measurement requirements | Qualcomm Incorporated | noted |  |  |
| R4-2102297 | On requirements for mulitple concurrent and independent MG patterns | Qualcomm Incorporated | noted |  |  |
| R4-2102298 | Revision of PRS-RSTD measurement requirements | Qualcomm Incorporated | merged |  |  |
| R4-2102299 | Revision of PRS-RSTD measurement requirements | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102300 | Revision of PRS-RSRP measurement requirements | Qualcomm Incorporated | merged |  |  |
| R4-2102301 | Revision of PRS-RSRP measurement requirements | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102302 | Revision of UE Rx-Tx measurement requirements | Qualcomm Incorporated | merged |  |  |
| R4-2102303 | Revision of UE Rx-Tx measurement requirements | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102304 | TR 38.717-02-01 v0.3.0 | ZTE Wistron Telecom AB | not concluded |  |  |
| R4-2102305 | TR 38.717-03-02 v0.3.0 | ZTE Wistron Telecom AB | not concluded |  |  |
| R4-2102306 | CR to TS 38.133: Redundant and incorrect TCI state in tests with TRS (Rel-15) | Rohde & Schwarz | revised |  | R4-2104054 |
| R4-2102307 | CR to TS 38.133: Redundant and incorrect TCI state in tests with TRS (Rel-16) | Rohde & Schwarz | agreed |  |  |
| R4-2102308 | CR to TS 38.133: Redundant and incorrect TCI state in tests with TRS (Rel-17) | Rohde & Schwarz | agreed |  |  |
| R4-2102309 | CR to TS 38.133: Corrections to TC A.4.5.7.1 (Rel-15) | Rohde & Schwarz | revised |  | R4-2103496 |
| R4-2102310 | CR to TS 38.133: Corrections to TC A.4.5.7.1 (Rel-16) | Rohde & Schwarz | agreed |  |  |
| R4-2102311 | CR to TS 38.133: Corrections to TC A.4.5.7.1 (Rel-17) | Rohde & Schwarz | endorsed |  |  |
| R4-2102312 | Test structure and complexity for NR RSTD test cases | Rohde & Schwarz | noted |  |  |
| R4-2102313 | draft CR to include 2A-2A-7C, 2A-2A-7A-7A, 2A-2A-13A | Ericsson, Bell Mobility | endorsed |  |  |
| R4-2102314 | TP for TR 38.717-02-01 to include CA_n7-n77 | Ericsson, Bell Mobility | revised |  | R4-2103047 |
| R4-2102315 | draft CR to include CA_n7-n78, CA_n25-n41, CA_n41-n77 BCS | Ericsson, Bell Mobility | endorsed |  |  |
| R4-2102316 | TP for 38.717-03-02 to include n25-n41-n66 | Ericsson, Bell Mobility | revised |  | R4-2103099 |
| R4-2102317 | draft CR to include DC_2-7-13_n66 configurations | Ericsson, Bell Mobility | endorsed |  |  |
| R4-2102318 | draft CR to include DC_2-7_n66 configuration | Ericsson, Bell Mobility | endorsed |  |  |
| R4-2102319 | Rel-17 CR 38.101-1 for improvements Intra-band tables | Ericsson | revised |  | R4-2103033 |
| R4-2102320 | Rel-17 CR 38.101-1 for corrections NR CA 2, 3 and 4 band configuration tables | Ericsson | agreed |  |  |
| R4-2102321 | On IAB test case dependency | Ericsson | noted |  |  |
| R4-2102322 | IAB Common test issue on test Configuration-Conducted | Ericsson | noted |  |  |
| R4-2102323 | IAB Common test issue on test configuration-OTA | Ericsson | noted |  |  |
| R4-2102324 | IAB Common test issue on test model-Conducted | Ericsson | noted |  |  |
| R4-2102325 | IAB Common test issue on test model-OTA | Ericsson | noted |  |  |
| R4-2102326 | On IAB-MT synchronization test setup for conformance testing | Ericsson | noted |  |  |
| R4-2102327 | IAB conducted transmitter test | Ericsson | noted |  |  |
| R4-2102328 | TP example for Conducted receiver characteristic test | Ericsson | noted |  |  |
| R4-2102329 | General conducted test conditions and declarations | Ericsson | noted |  |  |
| R4-2102330 | On IAB-MT frequency error tests | Ericsson | noted |  |  |
| R4-2102331 | TP example for OTA receiver characteristic test | Ericsson | noted |  |  |
| R4-2102332 | General test conditions and declarations-OTA | Ericsson | noted |  |  |
| R4-2102333 | IAB-EVM procedure | Ericsson | noted |  |  |
| R4-2102334 | IAB-MT interference signal characteristic | Ericsson | noted |  |  |
| R4-2102335 | IAB-MT TX dynamic range | Ericsson | noted |  |  |
| R4-2102336 | CR on Tx Power related requirements in TS 38.174 | Ericsson | not pursued |  |  |
| R4-2102337 | CR on Transmitted signal quality in TS 38.174 | Ericsson | not pursued |  |  |
| R4-2102338 | CR on Transmitter characteristics- Others TS 38.174 | Ericsson | revised |  | R4-2103851 |
| R4-2102339 | CR on In-band selectivity and blocking requirements in TS 38.174 | Ericsson | endorsed |  |  |
| R4-2102340 | CR on Rx Charateristic other related requirements | Ericsson | revised |  | R4-2103852 |
| R4-2102341 | CR on Sensitivity and dynamic range requirements TS 38.174 | Ericsson | endorsed |  |  |
| R4-2102342 | Bandwidth for SL operating in n14 | Ericsson | noted |  |  |
| R4-2102343 | Operating scenarios for partially used SL operation | Ericsson | noted |  |  |
| R4-2102344 | coexisting simulation assumption for public safety UC | Ericsson | noted |  |  |
| R4-2102345 | SL UE synchronization issue for licensed operation | Ericsson | noted |  |  |
| R4-2102346 | SL UE Timing mask for Partially used SL operation with NR Uu operating bands | Ericsson | noted |  |  |
| R4-2102347 | Reply LS on PUCCH and PUSCH repetition | Ericsson | noted |  |  |
| R4-2102348 | CR 36.133 (A.8.16.106) Correction of test case for direct SCell activation at addition | Ericsson | agreed |  |  |
| R4-2102349 | CR 36.133 (A.8.16.106) Correction of test case for direct SCell activation at addition | Ericsson | agreed |  |  |
| R4-2102350 | CR 36.133 (A.8.16.106) Correction of test case for direct SCell activation at addition | Ericsson | agreed |  |  |
| R4-2102351 | On incremental delay for dormancy switching on multiple CCs | Ericsson | noted |  |  |
| R4-2102352 | CR 38.133 (8.2 8.6) Corrections related to SCell dormancy switching | Ericsson | merged |  |  |
| R4-2102353 | CR 38.133 (8.2 8.6) Corrections related to SCell dormancy switching | Ericsson | withdrawn |  |  |
| R4-2102354 | On DCI-based simultaneous BWP switch on multiple CCs | Ericsson | noted |  |  |
| R4-2102355 | CR 38.133 (8.6.2A) Clarification on DCI-triggered BWP switch on multiple CCs | Ericsson | revised |  | R4-2103602 |
| R4-2102356 | CR 38.133 (8.6.2A) Clarification on DCI-triggered BWP switch on multiple CCs | Ericsson | agreed |  |  |
| R4-2102357 | On TC3 for Direct SCell Activation | Ericsson | noted |  |  |
| R4-2102358 | DraftCR 38.133 TC3 Direct SCell activation | Ericsson | revised |  | R4-2103559 |
| R4-2102359 | On TCs 6 and 8 for SCell dormancy switching | Ericsson | noted |  |  |
| R4-2102360 | DraftCR 38.133 TCs 6 and 8 SCell dormancy switching | Ericsson | revised |  | R4-2103569 |
| R4-2102361 | Test case applicability for mandatory measurement gaps in R15/R16 | Ericsson | noted |  |  |
| R4-2102362 | On TC4 for simultaneous BWP switch on multiple CCs | Ericsson | noted |  |  |
| R4-2102363 | DraftCR 38.133 (A.7.5.X) TC4 BWP switching | Ericsson | revised |  | R4-2103606 |
| R4-2102364 | On handover with PSCell | Ericsson | noted |  |  |
| R4-2102365 | On SCell (de)activation with PUCCH | Ericsson | noted |  |  |
| R4-2102366 | CR 36.133 Removal of brackets for SCell dormancy | Ericsson | agreed |  |  |
| R4-2102367 | CR 36.133 Removal of brackets for SCell dormancy | Ericsson | agreed |  |  |
| R4-2102368 | On SCell interruptions and SCell (de)activation test cases | Ericsson | noted |  |  |
| R4-2102369 | DraftCR Introduction of NR-U SCell interruption and SCell (de)activation tests | Ericsson | revised |  | R4-2103525 |
| R4-2102370 | On PSCell addition release and TCI state activation TCs | Ericsson | noted |  |  |
| R4-2102371 | On TC for NR-U Inter-RAT SFTD accuracy | Ericsson | noted |  |  |
| R4-2102372 | DraftCR 38.133 NR-U Inter-RAT SFTD accuracy TC | Ericsson | revised |  | R4-2103526 |
| R4-2102373 | Simulation results for FR2 256QAM PDSCH Test Cases | Qualcomm Incorporated | noted |  |  |
| R4-2102374 | Discussion on satellite bands outside FR1/FR2 range for NR based satellite networks | HUGHES Network Systems, Thales, Inmarsat, Intelsat, Fraunhofer, ESA | noted |  |  |
| R4-2102375 | Further consideration on simultaneous RxTx UE capability | Huawei, HiSilicon | noted |  |  |
| R4-2102376 | CR for TS 38.101-1 correction CR for simultaneous TxRx operation (R15) | Huawei, HiSilicon | not pursued |  |  |
| R4-2102377 | CR for TS 38.101-1: correction CR for simultaneous Tx/Rx operation (R16) | Huawei, HiSilicon | withdrawn |  |  |
| R4-2102378 | CR for TS 38.101-3 correction CR for simultaneous TxRx operation (R15) | Huawei, HiSilicon | not pursued |  |  |
| R4-2102379 | CR for TS 38.101-3: correction CR for simultaneous Tx/Rx operation (R16) | Huawei, HiSilicon | withdrawn |  |  |
| R4-2102380 | On SL switching period | Huawei, HiSilicon | noted |  |  |
| R4-2102381 | CR for TS 38.101-3 NR V2X switching period | Huawei, HiSilicon | not pursued |  |  |
| R4-2102382 | CR for 38.101-1 correction of NR V2X FRC parameter | Huawei, HiSilicon | not pursued |  |  |
| R4-2102383 | On TxD RF requirements | Huawei, HiSilicon | noted |  |  |
| R4-2102384 | CR for TS 38.101-1: TxD requirements | Huawei, HiSilicon | revised |  | R4-2103156 |
| R4-2102385 | Discussion on Reply LS to RAN5 On EN-DC power class | Huawei, HiSilicon | noted |  |  |
| R4-2102386 | CR for TS 38.101-1: correction of Pi/2 BPSK | Huawei, HiSilicon | not pursued |  |  |
| R4-2102387 | On SUO for intra-band EN-DC | Huawei, HiSilicon | noted |  |  |
| R4-2102388 | On BCS reporting and support for intra-band EN-DC band combinations | Huawei, HiSilicon | noted |  |  |
| R4-2102389 | Discussion on reply LS on Beam switching gaps for Multi-TRP UL transmission | Huawei, HiSilicon | noted |  |  |
| R4-2102390 | draft reply LS on Beam switching gaps for Multi-TRP UL transmission | Huawei, HiSilicon | noted |  |  |
| R4-2102391 | On NR FDD HPUE | Huawei, HiSilicon | noted |  |  |
| R4-2102392 | System performance evaluation for FDD HPUE | Huawei, HiSilicon | noted |  |  |
| R4-2102393 | draftCR to introduce UL MIMO configurations for band n84 | Huawei, HiSilicon | endorsed |  |  |
| R4-2102394 | CR for TS 38.101-1 Introduce NR SUL bands to PC3 UL-MIMO configuration | Huawei, HiSilicon | agreed |  |  |
| R4-2102395 | CR for TS 38.101-3 correction of intra-band contiguous EN-DC for DC_(n)66_R16 | Huawei, HiSilicon | not pursued |  |  |
| R4-2102396 | CR for TS 38.101-3 correction of intra-band contiguous EN-DC for DC_(n)66_R17 | Huawei, HiSilicon | withdrawn |  |  |
| R4-2102397 | On 2Tx - 2Tx UE uplink switch | Huawei, HiSilicon | noted |  |  |
| R4-2102398 | CR for TS 38.124: Correction of FR1 radiated spurious emissions (R15) | Huawei, HiSilicon, Bureau Veritas | revised |  | R4-2103771 |
| R4-2102399 | CR for TS 38.124: Correction of FR1 radiated spurious emissions (R16) | Huawei, HiSilicon, Bureau Veritas, Samsung | agreed |  |  |
| R4-2102400 | Analysis on number of test points vs uncertainty of FR2 MIMO OTA requirements | Huawei, HiSilicon | noted |  |  |
| R4-2102401 | Analysis on reducing test time | Huawei, HiSilicon | noted |  |  |
| R4-2102402 | CR for TS 38.101-3: Adding delta TIB and RIB requirement for DC_2-7-7-66_n78 (R16) | Huawei, HiSilicon | agreed |  |  |
| R4-2102403 | draft CR for TS 38.101-3: Adding delta TIB and RIB requirement for DC_2-7-7-66_n78 (R17) | Huawei, HiSilicon | agreed |  |  |
| R4-2102404 | Introduction of completed R17 2DL2UL band combinations to TS 36.101 | Huawei, HiSilicon | not concluded |  |  |
| R4-2102405 | Revised WID for LTE inter-band CA for 2 bands DL with 2 bands UL | Huawei, HiSilicon | not concluded |  |  |
| R4-2102406 | Views on FR2 256QAM CQI Reporting Test Cases | Qualcomm Incorporated | noted |  |  |
| R4-2102407 | 600 MHz band for Region 3 | Qualcomm Incorporated | noted |  |  |
| R4-2102408 | Missing parent clause for NR-DC PCMAX | Qualcomm Incorporated | revised |  | R4-2103142 |
| R4-2102409 | Missing parent clause for NR-DC PCMAX | Qualcomm Incorporated | agreed |  |  |
| R4-2102410 | Corrections to PCMAX for UL CA | Qualcomm Incorporated | revised |  | R4-2103148 |
| R4-2102411 | Corrections to PCMAX for UL CA | Qualcomm Incorporated | agreed |  |  |
| R4-2102412 | Notational amendment and correction to PCMAX for EN-DC | Qualcomm Incorporated | revised |  | R4-2103165 |
| R4-2102413 | Notational amendment and correction to PCMAX for EN-DC | Qualcomm Incorporated | agreed |  |  |
| R4-2102414 | Increasing UE maximum output power | Qualcomm Incorporated | noted |  |  |
| R4-2102415 | MSD for PC2 EN-DC and UL CA | Qualcomm Incorporated | noted |  |  |
| R4-2102416 | UE requirements for EU NR-U 6 GHz band | Qualcomm Incorporated | noted |  |  |
| R4-2102417 | PC 1.5 for bands n77 and n78 | Qualcomm Incorporated | noted |  |  |
| R4-2102418 | NR repeater requirements | Huawei | noted |  |  |
| R4-2102419 | Considerations for NR repeaters and AAS | Huawei | noted |  |  |
| R4-2102420 | IAB conformance test burden | Huawei | noted |  |  |
| R4-2102421 | IAB declarations format | Huawei | noted |  |  |
| R4-2102422 | draft CR to TR 38.174 - correction to clause 6 | Huawei | endorsed |  |  |
| R4-2102423 | CR to TS 37.145-1, Corrections to conformance requirements, Rel-15 | Huawei | revised |  | R4-2103791 |
| R4-2102424 | CR to TS 37.145-1, Corrections to conformance requirements, Rel-16 | Huawei | withdrawn |  |  |
| R4-2102425 | CR to TS 37.145-1, Corrections to conformance requirements, Rel-17 | Huawei | withdrawn |  |  |
| R4-2102426 | CR to TS 37.145-2: Corrections to conformance requirements, Rel-15 | Huawei | revised |  | R4-2103792 |
| R4-2102427 | CR to TS 37.145-2: Corrections to conformance requirements, Rel-16 | Huawei | agreed |  |  |
| R4-2102428 | CR to TS 37.145-2: Corrections to conformance requirements, Rel-17 | Huawei | agreed |  |  |
| R4-2102429 | Further discussion on CLTA maximum height | Huawei | noted |  |  |
| R4-2102430 | Further discussion on co-location for adjacent bands | Huawei | noted |  |  |
| R4-2102431 | CR to TS 37.145-2 - Update CLTA definition, Rel-15 | Huawei | revised |  | R4-2103788 |
| R4-2102432 | CR to TS 37.145-2 - Update CLTA definition, Rel-16 | Huawei, CATT, Nokia | agreed |  |  |
| R4-2102433 | CR to TS 37.145-2 - Update CLTA definition, Rel-17 | Huawei, CATT, Nokia | agreed |  |  |
| R4-2102434 | CR to TS 38.141-2 - Update CLTA definition, Rel-15 | Huawei | not pursued |  |  |
| R4-2102435 | CR to TS 38.141-2 - Update CLTA definition, Rel-16 | Huawei | withdrawn |  |  |
| R4-2102436 | CR to TS 38.141-2 - Update CLTA definition, Rel-17 | Huawei | withdrawn |  |  |
| R4-2102437 | A-MPR for LTE CA_NS_04 256QAM PC2 | Huawei, HiSilicon | noted |  |  |
| R4-2102438 | Revised WID: LTE Advanced inter-band CA Rel-17 for x bands DL (x=4, 5) with 1 band UL | Nokia, Nokia Shanghai Bell | not concluded |  |  |
| R4-2102439 | Updated scope of TR: LTE inter-band CA for 4/5 bands DL with 1 band UL | Nokia, Nokia Shanghai Bell | approved |  |  |
| R4-2102440 | TR 36.717-04-01 v0.3.0 | Nokia, Nokia Shanghai Bell | not concluded |  |  |
| R4-2102441 | CR to 37.141: Correction to ACLR limit in non-contiguous spectrum (Rel-15) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102442 | CR to 37.141: Correction to ACLR limit in non-contiguous spectrum (Rel-16) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102443 | CR to 37.141: Correction to ACLR limit in non-contiguous spectrum (Rel-17) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102444 | Draft CR to 37.141: Introduction of NR-U co-existence requirements | Nokia, Nokia Shanghai Bell | postponed |  |  |
| R4-2102445 | TP to TR 38.847: BS conformance aspects | Nokia, Nokia Shanghai Bell | revised |  | R4-2103874 |
| R4-2102446 | CR to 38.141-2: Introduction of n262 | Nokia, Nokia Shanghai Bell | not pursued |  |  |
| R4-2102447 | CR to 36.104: Introduction of n24 requirements | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102448 | CR to 36.141: Introduction of n24 requirements | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102449 | CR to 37.104: Introduction of n24 requirements | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102450 | CR to 37.141: Introduction of n24 requirements | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102451 | CR to 38.104: Introduction of n24 | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102452 | CR to 36.104: Correction to Band 24 requirements (Rel-10) | Nokia, Nokia Shanghai Bell | revised |  | R4-2103207 |
| R4-2102453 | CR to 36.104: Correction to Band 24 requirements (Rel-11) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102454 | CR to 36.104: Correction to Band 24 requirements (Rel-12) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102455 | CR to 36.104: Correction to Band 24 requirements (Rel-13) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102456 | CR to 36.104: Correction to Band 24 requirements (Rel-14) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102457 | CR to 36.104: Correction to Band 24 requirements (Rel-15) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102458 | CR to 36.104: Correction to Band 24 requirements (Rel-16) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102459 | CR to 36.104: Correction to Band 24 requirements (Rel-17) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102460 | CR to 36.141: Correction to Band 24 requirements (Rel-10) | Nokia, Nokia Shanghai Bell | revised |  | R4-2103217 |
| R4-2102461 | CR to 36.141: Correction to Band 24 requirements (Rel-11) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102462 | CR to 36.141: Correction to Band 24 requirements (Rel-12) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102463 | CR to 36.141: Correction to Band 24 requirements (Rel-13) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102464 | CR to 36.141: Correction to Band 24 requirements (Rel-14) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102465 | CR to 36.141: Correction to Band 24 requirements (Rel-15) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102466 | CR to 36.141: Correction to Band 24 requirements (Rel-16) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102467 | CR to 36.141: Correction to Band 24 requirements (Rel-17) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102468 | CR to 37.104: Correction to Band 24 requirements (Rel-10) | Nokia, Nokia Shanghai Bell | revised |  | R4-2103208 |
| R4-2102469 | CR to 37.104: Correction to Band 24 requirements (Rel-11) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102470 | CR to 37.104: Correction to Band 24 requirements (Rel-12) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102471 | CR to 37.104: Correction to Band 24 requirements (Rel-13) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102472 | CR to 37.104: Correction to Band 24 requirements (Rel-14) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102473 | CR to 37.104: Correction to Band 24 requirements (Rel-15) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102474 | CR to 37.104: Correction to Band 24 requirements (Rel-16) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102475 | CR to 37.104: Correction to Band 24 requirements (Rel-17) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102476 | CR to 37.141: Correction to Band 24 requirements (Rel-10) | Nokia, Nokia Shanghai Bell | revised |  | R4-2103216 |
| R4-2102477 | CR to 37.141: Correction to Band 24 requirements (Rel-11) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102478 | CR to 37.141: Correction to Band 24 requirements (Rel-12) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102479 | CR to 37.141: Correction to Band 24 requirements (Rel-13) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102480 | CR to 37.141: Correction to Band 24 requirements (Rel-14) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102481 | CR to 37.141: Correction to Band 24 requirements (Rel-15) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102482 | CR to 37.141: Correction to Band 24 requirements (Rel-16) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102483 | CR to 37.141: Correction to Band 24 requirements (Rel-17) | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102484 | CR to 37.104: Introduction of requirements for 35 and 45MHz channel bandwidths | Nokia, Nokia Shanghai Bell | revised |  | R4-2103197 |
| R4-2102485 | CR on 36133 LTE FDD-TDD inter-F sync DAPS HO TCs | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102486 | CR on 36133 LTE FDD-TDD inter-F sync DAPS HO TCs - Cat-A | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102487 | discussion on measurement gaps for IAB RRM requirements | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102488 | CR on removing gap aspects from IAB-MT RRM requirements | Nokia, Nokia Shanghai Bell | merged |  |  |
| R4-2102489 | discussion on IAB RRM test cases | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102490 | draftCR on IAB RLM test cases | Nokia, Nokia Shanghai Bell | revised |  | R4-2103544 |
| R4-2102491 | Introduction of band n67 | Huawei, HiSilicon | noted |  |  |
| R4-2102492 | Relative calibration approach using reference receiver | ROHDE & SCHWARZ | noted |  |  |
| R4-2102493 | draft CR to TR 37.941: Relative calibration approach | ROHDE & SCHWARZ | revised |  | R4-2103848 |
| R4-2102494 | Revised WID: Rel17 LTE inter-band CA for 2 bands DL with 1 band UL | Qualcomm Incorporated | not concluded |  |  |
| R4-2102495 | Introduction of Rel-17 LTE inter-band CA for 2 bands DL with 1 band UL combinations in TS 36.101 | Qualcomm Incorporated | agreed |  |  |
| R4-2102496 | TR 36.717-02-01 Rel-17 LTE inter-band CA for 2 bands DL and 1 band UL CA | Qualcomm Incorporated | not concluded |  |  |
| R4-2102497 | Discussion on FR2 MIMO OTA performance requirements | Qualcomm Incorporated | noted |  |  |
| R4-2102498 | Downlink co-existence simulation results for indoor scenario | Qualcomm Incorporated | noted |  |  |
| R4-2102499 | Uplink co-existence simulation results for indoor scenario | Qualcomm Incorporated | noted |  |  |
| R4-2102500 | TP to TR 38.921: Clarification of beamforming pattern modelling for multiple UL schedued UEs | Qualcomm Incorporated | noted |  | R4-2103108 |
| R4-2102501 | Discussion on UE SEM for 6.425-7.125GHz and 10-10.5GHz | Qualcomm Incorporated | noted |  |  |
| R4-2102502 | Discussion on candidate methods for BCS4 | Qualcomm Incorporated | noted |  |  |
| R4-2102503 | Simulation assumptions for FDD HPUE performance evaluations | Qualcomm Incorporated | noted |  |  |
| R4-2102504 | BCS reporting and support for intra-band EN-DC band combination | Qualcomm Incorporated | noted |  |  |
| R4-2102505 | CR for 38.101-3 on applicability of minimum requirements for EN-DC (Rel-15) | Qualcomm Incorporated | not pursued |  |  |
| R4-2102506 | CR for 38.101-3 on applicability of minimum requirements for EN-DC (Rel-16) | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102507 | CR for 38.101-3 on applicability of minimum requirements for EN-DC (Rel-17) | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102508 | Simulation assumptions for NR NTN co-existence study | Qualcomm Incorporated | noted |  |  |
| R4-2102509 | Introduction of band n85 | Huawei, HiSilicon | noted |  |  |
| R4-2102510 | IBE requirement for almost contiguous allocations | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102511 | Discussion on the CBM/IBM applicability of Rel-17 FR2 inter-band CA | Google Inc. | noted |  |  |
| R4-2102512 | On remaining issues for RLM in NR-U | Ericsson | noted |  |  |
| R4-2102513 | Updates in RLM requirements for NR-U | Ericsson | revised |  | R4-2104047 |
| R4-2102514 | Updates in RLM requirements for NR-U | Ericsson | agreed |  |  |
| R4-2102515 | On remaining issues for SCell activation in NR-U | Ericsson | noted |  |  |
| R4-2102516 | Updates in SCell activation in NR-U | Ericsson | merged |  |  |
| R4-2102517 | Updates in SCell activation in NR-U | Ericsson | withdrawn |  |  |
| R4-2102518 | On SSB availability to meet NR-U requirements in DRX | Ericsson | noted |  |  |
| R4-2102519 | Terminology updates for NR-U in 38.133 | Ericsson | revised |  | R4-2103514 |
| R4-2102520 | Terminology updates for NR-U in 38.133 | Ericsson | agreed |  |  |
| R4-2102521 | Terminology updates for NR-U in 36.133 | Ericsson | revised |  | R4-2103515 |
| R4-2102522 | Terminology updates for NR-U in 36.133 | Ericsson | agreed |  |  |
| R4-2102523 | Draft Big CR: Introduction of Rel-16 NR-U RRM performance requirements | Ericsson | revised |  | R4-2103532 |
| R4-2102524 | Updated test case list for NR-U | Ericsson | revised |  | R4-2103523 |
| R4-2102525 | NR-U test cases structure | Ericsson | revised |  | R4-2103533 |
| R4-2102526 | RSSI measurement bandwidth | Ericsson | endorsed |  |  |
| R4-2102527 | On CCA model in NR-U test cases | Ericsson | noted |  |  |
| R4-2102528 | CCA model in NR-U test cases | Ericsson | revised |  | R4-2103522 |
| R4-2102529 | On RLM test cases | Ericsson | noted |  |  |
| R4-2102530 | RLM test cases | Ericsson | revised |  | R4-2103527 |
| R4-2102531 | On NR-U measurements test cases | Ericsson | noted |  |  |
| R4-2102532 | NR-U RRM, SFTD, RSSI, and CO measurements test cases | Ericsson | revised |  | R4-2103528 |
| R4-2102533 | TC2 - SA interruptions at NR SRS carrier based switching | Ericsson | revised |  | R4-2103616 |
| R4-2102534 | On RRM requirements for SRS antenna port switching | Ericsson | noted |  |  |
| R4-2102535 | On parallel measurement gap patterns | Ericsson | noted |  |  |
| R4-2102536 | On correction to inter-RAT CSSF | Ericsson | noted |  |  |
| R4-2102537 | Correction to inter-RAT CSSF | Ericsson | withdrawn |  |  |
| R4-2102538 | Correction to inter-RAT CSSF | Ericsson | withdrawn |  |  |
| R4-2102539 | On RSTD measurement requirements | Ericsson | noted |  |  |
| R4-2102540 | RSTD measurement requirements | Ericsson | merged |  |  |
| R4-2102541 | RSTD measurement requirements | Ericsson | withdrawn |  |  |
| R4-2102542 | On PRS-RSRP measurement requirements | Ericsson | noted |  |  |
| R4-2102543 | PRS-RSRP measurement requirements | Ericsson | revised |  | R4-2103579 |
| R4-2102544 | PRS-RSRP measurement requirements | Ericsson | agreed |  |  |
| R4-2102545 | On UE Rx-Tx measurement requirements | Ericsson | noted |  |  |
| R4-2102546 | UE Rx-Tx measurement requirements | Ericsson | merged |  |  |
| R4-2102547 | UE Rx-Tx measurement requirements | Ericsson | withdrawn |  |  |
| R4-2102548 | Correction to CSSF for PRS measurements | Ericsson | merged |  |  |
| R4-2102549 | Draft Big CR: Introduction of Rel-16 NR Positioning RRM performance requirements and test cases | Ericsson, Intel | revised |  | R4-2103585 |
| R4-2102550 | On RSTD measurement accuracy requirements | Ericsson | noted |  |  |
| R4-2102551 | On PRS-RSRP measurement accuracy requirements | Ericsson | noted |  |  |
| R4-2102552 | On UE Rx-Tx measurement accuracy requirements | Ericsson | noted |  |  |
| R4-2102553 | UE Rx-Tx measurement accuracy requirements | Ericsson | revised |  | R4-2103731 |
| R4-2102554 | On positioning test cases | Ericsson | noted |  |  |
| R4-2102555 | TC5 and TC6: UE Rx-Tx time difference measurement requirements for FR1 and FR2 in SA | Ericsson | revised |  | R4-2103737 |
| R4-2102556 | TC11 and TC12: UE Rx-Tx time difference measurement accuracy for FR1 and FR2 in SA | Ericsson | revised |  | R4-2103738 |
| R4-2102557 | CR for n47 AMPR - Cat A | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102558 | On the use of overlapping channel bandwidths from UE perspective | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102559 | Clarification of intra-bandENDC-Support | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102560 | Beam correspondence of FWA device | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102561 | Power Class 4 for HST | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102562 | CR to 38.101-2: correction on UL MIMO | Google Inc. | agreed |  |  |
| R4-2102563 | CR to TS 37.105: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16 | Huawei, China Unicom | revised |  | R4-2103782 |
| R4-2102564 | CR to TS 37.105: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-17 | Huawei, China Unicom | agreed |  |  |
| R4-2102565 | CR to TS 37.145-1: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16 | Huawei, China Unicom | revised |  | R4-2103783 |
| R4-2102566 | CR to TS 37.145-1: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-17 | Huawei, China Unicom | agreed |  |  |
| R4-2102567 | CR to TS 37.145-2: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16 | Huawei, China Unicom | revised |  | R4-2103784 |
| R4-2102568 | CR to TS 37.145-2: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-17 | Huawei, China Unicom | agreed |  |  |
| R4-2102569 | TP to TR 38.808: capturing WF on the min/max CHBW and SCS | Huawei | noted |  |  |
| R4-2102570 | TP to TR 38.808: BS aspects | Huawei | noted |  |  |
| R4-2102571 | Inputs to the discussion on the FR2-extension vs. FR3 introduction for NR operation in 52.6 - 71 GHz range | Huawei | noted |  |  |
| R4-2102572 | Regulatory aspects for the 600MHz range in APT region | Huawei | revised |  | R4-2103267 |
| R4-2102573 | Initial considerations on the coexistence studies for 600MHz SI | Huawei | noted |  |  |
| R4-2102574 | Feasibility analysis of the frequency arrangement in 600MHz range for APT | Huawei, CBN | noted |  |  |
| R4-2102575 | [DRAFT] Reply LS on technical feasibilities for frequency arrangements for IMT in 470 | Huawei, CBN | revised |  | R4-2103269 |
| R4-2102576 | CR to TS 38.124: correction of the lower frequency range of the RSE, Rel-15 | Huawei | not pursued |  |  |
| R4-2102577 | CR to TS 38.124: correction of the lower frequency range of the RSE, Rel-16 | Huawei | withdrawn |  |  |
| R4-2102578 | Spatial exclusion zone for IAB node | Huawei | noted |  |  |
| R4-2102579 | EMC core requirements for NR repeater | Huawei | noted |  |  |
| R4-2102580 | Consideration of duplex mode for the frequency bands for NR repeater | Huawei | noted |  |  |
| R4-2102581 | CR to TS 38.175: Spatial Exclusion for EMC RI test for IAB | Huawei | not pursued |  |  |
| R4-2102582 | CR to 38.101-2: correction on UL MIMO | Google Inc. | agreed |  |  |
| R4-2102583 | Motivation for WI: Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) | vivo | not treated |  |  |
| R4-2102584 | New WID: Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) | vivo, OPPO, CMCC, CAICT, Rohde&Schwarz | not treated |  |  |
| R4-2102585 | Discussion on NR-U PDSCH Demodulation Performance Tests and related Simulation Assumptions | Qualcomm Incorporated | noted |  |  |
| R4-2102586 | Discussion on NR-U CQI Performance Tests | Qualcomm Incorporated | noted |  |  |
| R4-2102587 | Discussion on RLM/BFD Relaxation | Qualcomm Incorporated | noted |  |  |
| R4-2102588 | CR CA_n7B REFSENS | Qualcomm Incorporated | agreed |  |  |
| R4-2102589 | Band Plan for 600MHz SI | Apple | noted |  |  |
| R4-2102590 | Peak EIRP and EIRP Spherical coverage for n262 | Apple Inc. | noted |  |  |
| R4-2102591 | Peak EIS and EIS Spherical coverage for n262 | Apple Inc. | noted |  |  |
| R4-2102592 | MSD considering asymmetric UL/DL for bands n8 and n71 | Apple Inc. | noted |  |  |
| R4-2102593 | CR for bug fixing of band combination tables for 38101-3 Rel17 | Apple | revised |  | R4-2103345 |
| R4-2102594 | CR for bug fixing of band combination tables for 38101-3 Rel16 | Apple | revised |  | R4-2103346 |
| R4-2102595 | CR for TS 38.101-1: Cleanup for spurious emissions for UE co-existence table | Apple | agreed |  |  |
| R4-2102596 | CR for TS 36.101: Cleanup for spurious emissions for UE co-existence table | Apple | revised |  | R4-2103132 |
| R4-2102597 | CR for TS 38.101-1: Correction to FR1 time mask for SRS antenna switching | Apple | revised |  | R4-2103123 |
| R4-2102598 | CR for TS 38.101-1: Correction to FR1 time mask for SRS antenna switching | Apple | agreed |  |  |
| R4-2102599 | CR for TS 38.101-1: Correction to FR1 time mask for SRS antenna switching | Apple | agreed |  |  |
| R4-2102600 | CR for TS 38.101-1: Corrections to intra-band UL NC CA requirements | Apple | revised |  | R4-2103149 |
| R4-2102601 | CR for TS 38.101-1: Corrections to intra-band UL NC CA requirements | Apple | agreed |  |  |
| R4-2102602 | CR for TS 38.101-1: Cleanup for spurious emissions for UE co-existence table | Apple | revised |  | R4-2103160 |
| R4-2102603 | CR for TS 38.101-1: Cleanup for spurious emissions for UE co-existence table | Apple | agreed |  |  |
| R4-2102604 | CR for TS 36.101: Cleanup for spurious emissions for UE co-existence table | Apple | revised |  | R4-2103133 |
| R4-2102605 | CR for TS 36.101: Cleanup for spurious emissions for UE co-existence table | Apple | agreed |  |  |
| R4-2102606 | CR for TS 38.101-1: UE RF requirements table simplification | Apple | not pursued |  |  |
| R4-2102607 | CA_n258A-n260A and CA_n257A-n259A based on IBM | Apple | noted |  |  |
| R4-2102608 | Inter-band DL CA within same frequency group based on CBM | Apple | noted |  |  |
| R4-2102609 | Views on UL TPC for NR in 60 GHz and above frequency ranges | Apple | noted |  |  |
| R4-2102610 | Views on HPUE for NR FDD bands | Apple | noted |  |  |
| R4-2102611 | Discussion on network controlled small gap | Qualcomm CDMA Technologies | noted |  |  |
| R4-2102612 | 38.133 CR on the CSI-RS based measurement requirements | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102613 | gNB Beams Usage Criteria for NR FR1 MIMO OTA Channel Model Validation | Keysight Technologies UK Ltd, China Mobile | noted |  |  |
| R4-2102614 | On Remaining Channel Model Topics | Keysight Technologies UK Ltd | noted |  |  |
| R4-2102615 | CR on Channel Model Topics | Keysight Technologies UK Ltd | agreed |  |  |
| R4-2102616 | On Test methodology for high DL power and low UL power test cases | Keysight Technologies UK Ltd | noted |  |  |
| R4-2102617 | On extreme temperature condition testing | Keysight Technologies UK Ltd | noted |  |  |
| R4-2102618 | On Test Time Enhancements based on different Antenna Array Assumptions | Keysight Technologies UK Ltd | noted |  |  |
| R4-2102619 | On Testability for band n262 | Keysight Technologies UK Ltd | revised |  | R4-2103917 |
| R4-2102620 | NF based solutions and Enhancement of permitted methods | ROHDE & SCHWARZ | noted |  |  |
| R4-2102621 | TR skeleton 38.XXX V001 NR_HPUE_intra_Band_R17 | Huawei, HiSilicon | revised |  | R4-2103220 |
| R4-2102622 | Views on pre-configured MG patterns | Qualcomm CDMA Technologies | noted |  |  |
| R4-2102623 | on gaps for self-calibration and monitoring | Huawei, HiSilicon | noted |  |  |
| R4-2102624 | TP to TR 36.717-04-01 Correction of CA_2-5-7-66-66 | Nokia, Nokia Shanghai Bell | approved |  |  |
| R4-2102625 | Views on specifying the requirements for HO with PSCell | Qualcomm CDMA Technologies | noted |  |  |
| R4-2102626 | on FR1 intra-band CA separation class | Huawei, HiSilicon | noted |  |  |
| R4-2102627 | additional work plan for Rel-17 FR1 UE RF enhancement | Huawei, HiSilicon | revised |  | R4-2103230 |
| R4-2102628 | on UE capability for intra-band ENDC | Huawei, HiSilicon | noted |  |  |
| R4-2102629 | on transient period UE capability | Huawei, HiSilicon | noted |  |  |
| R4-2102630 | on phase continuity for PUCCH and PUSCH repetition and reply LS | Huawei, HiSilicon | noted |  |  |
| R4-2102631 | on new FWA UE beam correspondence requirement | Huawei, HiSilicon | noted |  |  |
| R4-2102632 | Addition of missing async FDD-TDD and TDD-FDD LTE interband interfrquency DAPS handover tests | Ericsson | revised |  | R4-2103663 |
| R4-2102633 | Addition of missing async FDD-TDD and TDD-FDD LTE interband interfrquency DAPS handover tests | Ericsson | agreed |  |  |
| R4-2102634 | Test cases for BWP switching on multiple CCs for FR1+FR2 | Ericsson | noted |  |  |
| R4-2102635 | Analysis of measurement gaps for LA IAB-MT | Ericsson | noted |  |  |
| R4-2102636 | Measurement gaps for Local Area IAB-MT | Ericsson | not pursued |  |  |
| R4-2102637 | Big CR: IAB-MT RRM test cases in 38.174 | Ericsson | revised |  | R4-2103545 |
| R4-2102638 | Analysis of RRC re-establishment tests for LA IAB-MT | Ericsson | noted |  |  |
| R4-2102639 | RRC re-establishment tests for LA IAB-MT | Ericsson | revised |  | R4-2103546 |
| R4-2102640 | General principles for IAB RRM test cases | Ericsson | noted |  |  |
| R4-2102641 | Analysis of impact of RA with CCA on RRM requirements in NR-U | Ericsson | noted |  |  |
| R4-2102642 | Applicability of RA with CCA on RRM requirements in NR-U in 38.133 | Ericsson | agreed |  |  |
| R4-2102643 | Applicability of RA with CCA on RRM requirements in NR-U in 38.133 | Ericsson | agreed |  |  |
| R4-2102644 | Applicability of RA with CCA on RRM requirements in NR-U in 36.133 | Ericsson | agreed |  |  |
| R4-2102645 | Applicability of RA with CCA on RRM requirements in NR-U in 36.133 | Ericsson | agreed |  |  |
| R4-2102646 | On UE transmit timing under DL LBT failure in reference cell | Ericsson | noted |  |  |
| R4-2102647 | RRC re-establishment tests for NR-U | Ericsson | noted |  |  |
| R4-2102648 | RRC connetion release with re-direction tests for NR-U | Ericsson | noted |  |  |
| R4-2102649 | Analysis of UE timing tests for NR-U | Ericsson | noted |  |  |
| R4-2102650 | UE timing tests for NR-U | Ericsson | revised |  | R4-2103529 |
| R4-2102651 | Analysis of test cases on BWP switching with consistent UL LBT failures | Ericsson | noted |  |  |
| R4-2102652 | Test cases on BWP switching with consistent UL LBT failures | Ericsson | revised |  | R4-2103530 |
| R4-2102653 | Analysis of RRM requirements for band n62 | Ericsson | noted |  |  |
| R4-2102654 | RRM core requirements for band n62 | Ericsson | revised |  | R4-2103666 |
| R4-2102655 | Overview of requirements for pre-configured measurement gaps | Ericsson | noted |  |  |
| R4-2102656 | Overview of requirements for network controlled small gap | Ericsson | noted |  |  |
| R4-2102657 | on intra-band CA HPUE RF architecture and MPR | Huawei, HiSilicon | noted |  |  |
| R4-2102658 | CR to 38.101-1: UL MIMO requirements update | Qualcomm Incorporated, Lenovo, Motorola Mobility | revised |  | R4-2103355 |
| R4-2102659 | CR to 38.101-1: UL MIMO requirements update | Qualcomm Incorporated, Lenovo, Motorola Mobility | withdrawn |  |  |
| R4-2102660 | CR to 38.101-1: UL MIMO requirements update | Qualcomm Incorporated, Lenovo, Motorola Mobility | withdrawn |  |  |
| R4-2102661 | On FR1 2L UL EVM Requirement | Qualcomm Incorporated, Lenovo, Motorola Mobility | noted |  |  |
| R4-2102662 | Discussion on FR2 UE Min. Output Power Requirement | Qualcomm Finland RFFE Oy | noted |  |  |
| R4-2102663 | Completion of beam correspondence requirements for all power classes | Qualcomm Finland RFFE Oy | noted |  |  |
| R4-2102664 | CR to 38.101-2 on beam correspondence | Qualcomm, Nokia, Nokia Shanghai Bell, Samsung, Verizon, NTT Docomo, Sony, Ericsson | revised |  | R4-2103126 |
| R4-2102665 | CR to 38.101-2 on beam correspondence | Qualcomm, Nokia, Nokia Shanghai Bell, Samsung, Verizon, NTT Docomo, Sony, Ericsson | agreed |  |  |
| R4-2102666 | CR to 38.101-2 on beam correspondence | Qualcomm, Nokia, Nokia Shanghai Bell, Samsung, Verizon, NTT Docomo, Sony, Ericsson | agreed |  |  |
| R4-2102667 | On EVM requirements for n262 | Qualcomm Incorporated | noted |  |  |
| R4-2102668 | On EIRP spherical coverage requirements for n262 | Qualcomm Incorporated | noted |  |  |
| R4-2102669 | On PC5 beam correspondence requirement | Qualcomm Incorporated | noted |  |  |
| R4-2102670 | CR to 38.807: Update for FR2 PC5 | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102671 | CR to 38.807: Update for FR2 PC5 | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102672 | CR to 38.807: Update for FR2 PC5 | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102673 | On impact of non-co-located test antennae for FR2 inter-band testing | Qualcomm Incorporated | noted |  |  |
| R4-2102674 | FR2 testability enhancement for polarization mismatch | Qualcomm Incorporated | noted |  |  |
| R4-2102675 | FR2 testability in ETC | Qualcomm Incorporated | noted |  |  |
| R4-2102676 | on CBM and IBM for FR2 inter-band DL CA | Huawei, HiSilicon | noted |  |  |
| R4-2102677 | Frequency separation class clarification | Ericsson | not pursued |  |  |
| R4-2102678 | Frequency separation class clarification | Ericsson | withdrawn |  |  |
| R4-2102679 | Consideration on UE requirements for FR2 HST | Ericsson | noted |  |  |
| R4-2102680 | Power calibration gap UE improvement requirements | Ericsson, Sony | noted |  |  |
| R4-2102681 | TP to TR 38.808: Addition of UE aspects | Ericsson | noted |  |  |
| R4-2102682 | on FR1 intra-band UL CA Pcmax | Huawei, HiSilicon | noted |  |  |
| R4-2102683 | CR for TS 38.101-1 Tx diversity requirements | Huawei, HiSilicon | withdrawn |  |  |
| R4-2102684 | CR on TS 38.101-1 time mask for shorter transient | Huawei, HiSilicon | not pursued |  | R4-2103153 |
| R4-2102685 | CR on TS 38.101-1 NS_49 | Huawei, HiSilicon | agreed |  |  |
| R4-2102686 | SRS carrier switching interruption CR | Qualcomm, Inc. | postponed |  | R4-2103612 |
| R4-2102687 | SRS carrier switching interruption CR | Qualcomm, Inc. | withdrawn |  |  |
| R4-2102688 | CR for FR2 FWA RF requirements | Huawei, HiSilicon | revised |  | R4-2103203 |
| R4-2102689 | On Introduction of Network Controlled Small Gaps for NR | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102690 | On SRS configurations for gNB measurement accuracy requirements | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102691 | Link simulation results for gNB Rx-Tx time difference accuracy | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102692 | On UL RTOA requirements for NR positioning | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102693 | Correction to applicability of E-UTRAN E-CID measurements requirements for NE-DC | Nokia, Nokia Shanghai Bell | revised |  | R4-2103501 |
| R4-2102694 | Correction to applicability of E-UTRAN E-CID measurements requirements for NE-DC | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102695 | Correction to applicability of E-UTRAN E-CID measurements requirements for NE-DC | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102696 | Correction to requirements for NCSG patterns | Nokia, Nokia Shanghai Bell | revised |  | R4-2103502 |
| R4-2102697 | Correction to requirements for NCSG patterns | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102698 | Correction to requirements for NCSG patterns | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102699 | Correction to requirements for NCSG patterns | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102700 | CR to 38.807: Update for FR2 PC5 | Qualcomm Incorporated | not pursued |  |  |
| R4-2102701 | CR to 38.807: Update for FR2 PC5 | Qualcomm Incorporated | not pursued |  |  |
| R4-2102702 | CR to 38.807: Update for FR2 PC5 | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102703 | CR for TS 38.101-1: correction CR for simultaneous Tx/Rx operation (R17) | Huawei, HiSilicon | withdrawn |  |  |
| R4-2102704 | Remaining issues in Transparent Tx Diversity | vivo | noted |  |  |
| R4-2102705 | Remaining issues in Power class | vivo | noted |  |  |
| R4-2102706 | Further discussion on Tx Switching enhancment | vivo | noted |  |  |
| R4-2102707 | Discussion on SAR control scheme for TDD intra-band non-contiguous UL CA HPUE | vivo | noted |  |  |
| R4-2102708 | Discussion on HPUE in NR FDD band | vivo | noted |  |  |
| R4-2102709 | Initial evaulation of PC2 UE for NR FDD | vivo | noted |  |  |
| R4-2102710 | Discussion and Reply LS on Beam switching gaps for Multi-TRP UL transmission | vivo | noted |  |  |
| R4-2102711 | Discussion and reply LS on p-NR-FR2 | vivo | revised |  | R4-2103143 |
| R4-2102712 | Discussion on SAR solution for PC2 inter-band NR CA | vivo | noted |  |  |
| R4-2102713 | Discussion on PC2 MSD for UL CA | vivo | noted |  |  |
| R4-2102714 | Simulation and analysis of FR2 inter-band DL CA based on CBM/IBM | vivo | noted |  |  |
| R4-2102715 | Discussion on FR2 inter-band UL CA | vivo | noted |  |  |
| R4-2102716 | CR on FR2 intra-band UL CA | Huawei, HiSilicon | revised |  | R4-2103348 |
| R4-2102717 | CR for TS 38.101-3: correction CR for simultaneous Tx/Rx operation (R17) | Huawei, HiSilicon | withdrawn |  |  |
| R4-2102718 | CR on FR2 inter-band DL CA CBM and IBM | Huawei, HiSilicon | revised |  | R4-2103369 |
| R4-2102719 | Simulation assumption summary for NR FR2 MIMO OTA | Huawei, HiSilicon | noted |  |  |
| R4-2102720 | CR on Active TCI state switching for NR-U | Huawei, HiSilicon | agreed |  |  |
| R4-2102721 | CR on Active TCI state switching for NR-U (cat A) | Huawei, HiSilicon | agreed |  |  |
| R4-2102722 | CR on maintenance on BWP switch requirements on multiple CCs | Huawei, HiSilicon | agreed |  |  |
| R4-2102723 | CR on maintenance on BWP switch requirements on multiple CCs (cat A) | Huawei, HiSilicon | agreed |  |  |
| R4-2102724 | CR on PSCell Addition requirements for NR-U | Huawei, HiSilicon | postponed |  |  |
| R4-2102725 | CR on PSCell Addition requirements for NR-U (cat A) | Huawei, HiSilicon | withdrawn |  |  |
| R4-2102726 | CR on test cases for inter-RAT measurement r15 | Huawei, HiSilicon | agreed |  |  |
| R4-2102727 | CR on test cases for inter-RAT measurement r16 | Huawei, HiSilicon | agreed |  |  |
| R4-2102728 | CR on test cases for inter-RAT measurement r17 | Huawei, HiSilicon | agreed |  |  |
| R4-2102729 | Consideration on FR2 MIMO OTA UE requirement | Huawei, HiSilicon | noted |  |  |
| R4-2102730 | Discussion on minimum and maximum channel bandwidth for 52.6 GHz to 71 GHz | Huawei, HiSilicon | noted |  |  |
| R4-2102731 | CR to remove intra-frequency ECID requirements for NE-DC 36133 R15 | Huawei, HiSilicon | revised |  | R4-2104044 |
| R4-2102732 | CR to remove intra-frequency ECID requirements for NE-DC 36133 R16 | Huawei, HiSilicon | agreed |  |  |
| R4-2102733 | CR to remove intra-frequency ECID requirements for NE-DC 36133 R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2102734 | CR to idle more requirements in 36133 R15 | Huawei, HiSilicon | agreed |  |  |
| R4-2102735 | CR to idle more requirements in 36133 R16 | Huawei, HiSilicon | agreed |  |  |
| R4-2102736 | CR to idle more requirements in 36133 R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2102737 | Discussion on CSSF for inter-RAT measurement, SCell activation delay and cell identification requirements on deactivated SCell in Rel-15 | Huawei, HiSilicon | noted |  |  |
| R4-2102738 | CR on SCell activation delay, cell idenfication requirements on deactivated SCell and inter-RAT ECID requirements for NE-DC R15 | Huawei, HiSilicon | revised |  | R4-2103482 |
| R4-2102739 | CR on SCell activation delay, cell idenfication requirements on deactivated SCell and inter-RAT ECID requirements for NE-DC R16 | Huawei, HiSilicon | agreed |  |  |
| R4-2102740 | CR on SCell activation delay, cell idenfication requirements on deactivated SCell and inter-RAT ECID requirements for NE-DC R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2102741 | CR on SCell activation TCs R15 | Huawei, HiSilicon | revised |  | R4-2103497 |
| R4-2102742 | CR on SCell activation TCs R16 | Huawei, HiSilicon | agreed |  |  |
| R4-2102743 | CR on SCell activation TCs R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2102744 | Discussion on remaining issues in EMR requirements | Huawei, HiSilicon | noted |  |  |
| R4-2102745 | CR on EMR requirement maintenance in 38.133 | Huawei, HiSilicon | agreed |  |  |
| R4-2102746 | CR on EMR requirement maintenance in 38.133 R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2102747 | CR on EMR requirements in 36.133 | Huawei, HiSilicon | revised |  | R4-2103727 |
| R4-2102748 | CR on EMR requirements in 36.133 R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2102749 | CR on SCell dormancy switching | Huawei, HiSilicon | revised |  | R4-2103554 |
| R4-2102750 | CR on SCell dormancy switching R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2102751 | draftCR on accuracy requirements for EMR 38.133 | Huawei, HiSilicon | endorsed |  |  |
| R4-2102752 | Big CR: Introduction of Rel-16 MR-DC EMR RRM performance requirements (TS 36.133) | Huawei, HiSilicon | postponed |  |  |
| R4-2102753 | Big CR: Introduction of Rel-16 MR-DC EMR RRM performance requirements (TS 36.133) | Huawei, HiSilicon | withdrawn |  |  |
| R4-2102754 | draftCR to introduce TC4 for EMR | Huawei, HiSilicon | revised |  | R4-2103551 |
| R4-2102755 | draftCR to introduce TC4 for direct SCell activation | Huawei, HiSilicon | revised |  | R4-2103560 |
| R4-2102756 | Further discussion on test for SCell dormancy | Huawei, HiSilicon | noted |  |  |
| R4-2102757 | draftCR to introduce TC3 for SCell dormancy | Huawei, HiSilicon | revised |  | R4-2103566 |
| R4-2102758 | Discussion on remaining issues for RSTD measurement requirements | Huawei, HiSilicon | noted |  |  |
| R4-2102759 | CR to update RSTD measurement requirements | Huawei, HiSilicon | merged |  |  |
| R4-2102760 | CR to update RSTD measurement requirements R17 | Huawei, HiSilicon | withdrawn |  |  |
| R4-2102761 | CR to update PRS-RSRP measurement requirements | Huawei, HiSilicon | merged |  |  |
| R4-2102762 | CR to update PRS-RSRP measurement requirements R17 | Huawei, HiSilicon | withdrawn |  |  |
| R4-2102763 | Discussion on remaining issues for UE Rx-Rx time difference measurement requirements | Huawei, HiSilicon | noted |  |  |
| R4-2102764 | CR to update UE Rx-Tx time differnece measurement requirements | Huawei, HiSilicon | merged |  |  |
| R4-2102765 | CR to update UE Rx-Tx time differnece measurement requirements R17 | Huawei, HiSilicon | withdrawn |  |  |
| R4-2102766 | Discussion on measurement capability and MG for PRS measurement | Huawei, HiSilicon | noted |  |  |
| R4-2102767 | CR on CSSF and MG for PRS measurement 38.133 | Huawei, HiSilicon | revised |  | R4-2103583 |
| R4-2102768 | CR on CSSF and MG for PRS measurement 38.133 R17 | Huawei, HiSilicon | withdrawn |  |  |
| R4-2102769 | CR to introduce new measurement gap patterns for positioning in 36.133 | Huawei, HiSilicon | revised |  | R4-2103584 |
| R4-2102770 | CR to introduce new measurement gap patterns for positioning in 36.133 R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2102771 | Discussion on accuracy requirements for RSTD measurement | Huawei, HiSilicon | noted |  |  |
| R4-2102772 | draftCR to introduce accuracy requirements for RSTD measurement | Huawei, HiSilicon | revised |  | R4-2103730 |
| R4-2102773 | Discussion on accuracy requirements for PRS-RSRP measurement | Huawei, HiSilicon | noted |  |  |
| R4-2102774 | Further simulation results for PRS-RSRP | Huawei, HiSilicon | noted |  |  |
| R4-2102775 | Discussion on accuracy requirements for UE Rx-Tx time difference measurement | Huawei, HiSilicon | noted |  |  |
| R4-2102776 | Discussion on RRM test case for UE positioning requirements | Huawei, HiSilicon | noted |  |  |
| R4-2102777 | draftCR to introduce TC for PRS-RSRP measurement requirements for FR1 in SA | Huawei, HiSilicon | revised |  | R4-2103739 |
| R4-2102778 | draftCR to introduce TC for RSTD measurement accuracy for FR1 and FR2 in SA | Huawei, HiSilicon | revised |  | R4-2104042 |
| R4-2102779 | Discussion on general issues for gNB positioning measurement requirements | Huawei, HiSilicon | noted |  |  |
| R4-2102780 | Discussion on gNB Rx-Tx time difference requirements | Huawei, HiSilicon | noted |  |  |
| R4-2102781 | Link level simulation results for UL-RTOA and gNB Rx-Tx time difference | Huawei, HiSilicon | noted |  |  |
| R4-2102782 | draftCR to introduce gNB Rx-Tx time difference requirements | Huawei, HiSilicon | revised |  | R4-2104053 |
| R4-2102783 | Discussion on SRS-RSRP requirements | Huawei, HiSilicon | noted |  |  |
| R4-2102784 | Link level simulation results for SRS-RSRP | Huawei, HiSilicon | noted |  |  |
| R4-2102785 | draftCR to introduce SRS-RSRP requirements | Huawei, HiSilicon | postponed |  |  |
| R4-2102786 | Discussion on UL-RTOA requirements | Huawei, HiSilicon, CMCC | noted |  |  |
| R4-2102787 | draftCR to introduce UL-RTOA requirements | Huawei, HiSilicon | postponed |  |  |
| R4-2102788 | Discussion on remaining issues in multiple SCell activation | Huawei, HiSilicon | noted |  |  |
| R4-2102789 | CR on multiple SCell activation requirements | Huawei, HiSilicon | revised |  | R4-2103621 |
| R4-2102790 | CR on multiple SCell activation requirements R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2102791 | CR on CGI reading requirements 38.133 | Huawei, HiSilicon, MediaTek | revised |  | R4-2103613 |
| R4-2102792 | CR on CGI reading requirements 38.133 R17 | Huawei, HiSilicon, MediaTek | agreed |  |  |
| R4-2102793 | CR on CGI reading report delay 36.133 | Huawei, HiSilicon | agreed |  |  |
| R4-2102794 | CR on CGI reading report delay 36.133 R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2102795 | draftCR to CGI reading TC4 | Huawei, HiSilicon | revised |  | R4-2103617 |
| R4-2102796 | Discussion on remaining issues in RSS measurement and RRC_Inactive state | Huawei, HiSilicon | noted |  |  |
| R4-2102797 | CR on eMTC RRM requirements | Huawei, HiSilicon | revised |  | R4-2103659 |
| R4-2102798 | CR on eMTC RRM requirements R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2102799 | Discussion on CSI-RSRP accuracy requirements | Huawei, HiSilicon | noted |  |  |
| R4-2102800 | Discussion on CSI-SINR accuracy requirements | Huawei, HiSilicon | noted |  |  |
| R4-2102801 | draftCR on CSI-SINR accuracy requirements | Huawei, HiSilicon | revised |  | R4-2103638 |
| R4-2102802 | Discussion on remaining issues in CSI-RS RRM test cases | Huawei, HiSilicon | noted |  |  |
| R4-2102803 | draft CR to update TC3 and TC12 for CSI-RS accuracy test | Huawei, HiSilicon | revised |  | R4-2103650 |
| R4-2102804 | CR on TC for eMTC RSTD measurement R15 | Huawei, HiSilicon | revised |  | R4-2103503 |
| R4-2102805 | CR on TC for eMTC RSTD measurement R16 | Huawei, HiSilicon | agreed |  |  |
| R4-2102806 | CR on TC for eMTC RSTD measurement R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2102807 | CR on CRS muting for eMTC R15 | Huawei, HiSilicon | revised |  | R4-2103504 |
| R4-2102808 | CR on CRS muting for eMTC R16 | Huawei, HiSilicon | agreed |  |  |
| R4-2102809 | CR on CRS muting for eMTC R17 | Huawei, HiSilicon | agreed |  |  |
| R4-2102810 | Initial discussion on (de)activation of pre-configured MGs | Huawei, HiSilicon | noted |  |  |
| R4-2102811 | Initial discussion on multiple concurrent MGs | Huawei, HiSilicon | noted |  |  |
| R4-2102812 | Initial discussion on NCSG | Huawei, HiSilicon | noted |  |  |
| R4-2102813 | Discussion on general issues for NTN RRM | Huawei, HiSilicon | noted |  |  |
| R4-2102814 | Discussion on NTN timing related requirements | Huawei, HiSilicon | noted |  |  |
| R4-2102815 | CR on FR2 intra-band UL CA | Huawei, HiSilicon | agreed |  | - |
| R4-2102816 | CR on TS 38.101-1 NS_49 | Huawei, HiSilicon | agreed |  |  |
| R4-2102817 | CR to TR 38.827 on channel model rotations | Huawei, HiSilicon | not pursued |  |  |
| R4-2102818 | Specification Structure for CA Test Cases | Qualcomm Incorporated | noted |  |  |
| R4-2102819 | CR to 38.827on base station beamforming configuration | Huawei, HiSilicon | postponed |  |  |
| R4-2102820 | Draft test case of CSI-RS based intra-frequency test for EN-DC event triggered reporting tests without gap for NR neighbor cell in FR2 | Qualcomm CDMA Technologies | revised |  | R4-2103641 |
| R4-2102821 | Views on Type II PMI Reporting Tests | Qualcomm Incorporated | noted |  |  |
| R4-2102822 | CR on FR1 PDSCH Mapping Type B and Processing Capability 2 Requirements | Qualcomm Incorporated | revised |  | R4-2103904 |
| R4-2102823 | CR to 38.133 - Introducing NR-U random access requirements | Nokia | withdrawn |  |  |
| R4-2102824 | CR on corrections for LTE-NR Co-existence tests and OCNG pattern | Qualcomm Incorporated | revised |  | R4-2103797 |
| R4-2102825 | Draft test case of measurement performance for EN-DC CSI-RSRP measurement accuracy for NR neighbor cell in FR2 | Qualcomm CDMA Technologies | revised |  | R4-2103651 |
| R4-2102826 | CR for correction of Rel-16 Dual Connectivity of 1LTE band (1DL/1UL) and 1NR band (1DL/1UL) with FR1 | Verizon Denmark | postponed |  |  |
| R4-2102827 | Correction to inter-RAT CSSF | Ericsson | merged |  |  |
| R4-2102828 | CR on corrections for LTE-NR Co-existence tests and OCNG pattern | Qualcomm Incorporated | agreed |  |  |
| R4-2102829 | Common understanding of repeaters | Qualcomm Incorporated | noted |  |  |
| R4-2102830 | Revised WID for LTE inter-band CA for 3 bands DL with 1 bands UL | Huawei, HiSilicon | not concluded |  |  |
| R4-2102831 | Conducted requirements for NR FR1 repeaters | Qualcomm Incorporated | noted |  |  |
| R4-2102832 | Introduction of completed R17 3DL band combinations to TS 36.101 | Huawei, HiSilicon | agreed |  |  |
| R4-2102833 | TR 36.717-03-01 0.2.0 | Huawei, HiSilicon | not concluded |  |  |
| R4-2102834 | Work plan for NR Repeaters | Qualcomm Incorporated | revised |  | R4-2103880 |
| R4-2102835 | CR to TS 38.104 corrections to NR-U BS RF Tx requirements | Nokia, Nokia Shanghai Bell | revised |  | R4-2103889 |
| R4-2102836 | CR to TS 38.104 corrections to NR-U BS RF Tx requirements | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102837 | CR to TS 38.104 corrections to NR-U BS RF Rx requirements | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102838 | CR to TS 38.104 corrections to NR-U BS RF Rx requirements | Nokia, Nokia Shanghai Bell | agreed |  |  |
| R4-2102839 | Text proposal PTRS | Qualcomm Incorporated | noted |  |  |
| R4-2102840 | Draft LS to WP5D on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 | Ericsson | revised |  | R4-2103104 |
| R4-2102841 | CR to 37.145-1 on Removal of additional limit for Band 7 | Ericsson | agreed |  |  |
| R4-2102842 | CR to 37.145-1 on Removal of additional limit for Band 7 | Ericsson | agreed |  |  |
| R4-2102843 | CR to 37.145-1 on Removal of additional limit for Band 7 | Ericsson | agreed |  |  |
| R4-2102844 | CR to 37.105 on NR+UTRA support for AAS | Ericsson | revised |  | R4-2103785 |
| R4-2102845 | CR to 37.105 on NR+UTRA support for AAS | Ericsson | withdrawn |  |  |
| R4-2102846 | CR to 37.105 on NR+UTRA support for AAS | Ericsson | withdrawn |  |  |
| R4-2102847 | CR to 37.104 on OBUE table headings and applicability | Ericsson | revised |  | R4-2103883 |
| R4-2102848 | CR to 37.104 on OBUE table headings and applicability | Ericsson | agreed |  |  |
| R4-2102849 | CR to 37.104 on OBUE table headings and applicability | Ericsson | agreed |  |  |
| R4-2102850 | CR to 37.141 on OBUE table headings and applicability | Ericsson | revised |  | R4-2103884 |
| R4-2102851 | CR to 37.141 on OBUE table headings and applicability | Ericsson | agreed |  |  |
| R4-2102852 | CR to 37.141 on OBUE table headings and applicability | Ericsson | agreed |  |  |
| R4-2102853 | CR to 37.105 on OBUE table headings and applicability | Ericsson | revised |  | R4-2103885 |
| R4-2102854 | CR to 37.105 on OBUE table headings and applicability | Ericsson | withdrawn |  |  |
| R4-2102855 | CR to 37.105 on OBUE table headings and applicability | Ericsson | withdrawn |  |  |
| R4-2102856 | CR to 37.145-1 on OBUE table headings and applicability | Ericsson | revised |  | R4-2103886 |
| R4-2102857 | CR to 37.145-1 on OBUE table headings and applicability | Ericsson | withdrawn |  |  |
| R4-2102858 | CR to 37.145-1 on OBUE table headings and applicability | Ericsson | withdrawn |  |  |
| R4-2102859 | CR to 37.145-2 on OBUE table headings and applicability | Ericsson | revised |  | R4-2103887 |
| R4-2102860 | CR to 37.145-2 on OBUE table headings and applicability | Ericsson | withdrawn |  |  |
| R4-2102861 | CR to 37.145-2 on OBUE table headings and applicability | Ericsson | withdrawn |  |  |
| R4-2102862 | Text proposal UE power amplifier and antenna array | Qualcomm Incorporated | noted |  |  |
| R4-2102863 | On RRM enhancements for HST scenarios for FR1 CA | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102864 | On Rel-16 NR eMIMO multi-TRxP transmissions | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102865 | Discussions on Rel-16 NR eMIMO L1-SINR measurement Accuracy | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102866 | Specifying 60 GHz as part of FR2 | Qualcomm Incorporated | noted |  |  |
| R4-2102867 | CR to TS 38.133: Adding L1-SINR accuracy requirements | Nokia, Nokia Shanghai Bell | revised |  | R4-2103592 |
| R4-2102868 | CR to TS 38.133: Adding conditions for L1-SINR reporting (Annex B.2) | Nokia, Nokia Shanghai Bell | postponed |  |  |
| R4-2102869 | CR on correcting SSB and RACH configuration in CSI-RS based beam failure detection and link recovery tests | Qualcomm CDMA Technologies | agreed | R4-2017048 |  |
| R4-2102870 | CR to 38.101-4 on update to CSI reporting test parameters for Aperiodic reporting (R15) | Apple | revised |  | R4-2103799 |
| R4-2102871 | CR to 38.101-4 on update to CSI reporting test parameters for Aperiodic reporting (R16) | Apple | agreed |  |  |
| R4-2102872 | Cat-F CR to SSB-less SCell activation delay requirement for deactivated FR1 SCell in Rel-15 | Qualcomm Incorporated | postponed |  | R4-2103483 |
| R4-2102873 | Cat-A CR to SSB-less SCell activation delay requirement for deactivated FR1 SCell in Rel-16 | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102874 | Cat-A CR to SSB-less SCell activation delay requirement for deactivated FR1 SCell in Rel-17 | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102875 | Cat-F CR to addition of TRS Configurations in Rel-15 Test Cases | Qualcomm Incorporated | revised |  | R4-2103498 |
| R4-2102876 | Cat-A CR to addition of TRS Configurations in Rel-16 Test Cases | Qualcomm Incorporated | agreed |  |  |
| R4-2102877 | Cat-A CR to addition of TRS Configurations in Rel-17 Test Cases | Qualcomm Incorporated | agreed |  |  |
| R4-2102878 | Cat-F CR to DRX Configurations in Rel-15 | Qualcomm Incorporated | not pursued |  |  |
| R4-2102879 | Cat-A CR to DRX Configurations in Rel-16 | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102880 | Cat-A CR to DRX Configurations in Rel-17 | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102881 | Cat-F CR to Removal of brackets for SCell Dormancy and Simultaneous DCI based BWP switch delay on multiple CCs in Rel-16 | Qualcomm Incorporated | merged |  |  |
| R4-2102882 | Cat-A CR to Removal of brackets for SCell Dormancy and Simultaneous DCI based BWP switch delay on multiple CCs in Rel-17 | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102883 | Cat-F CR to Removal of brackets for SCell Dormancy and Simultaneous DCI based BWP switch delay on multiple CCs in Rel-16 LTE | Qualcomm Incorporated | merged |  |  |
| R4-2102884 | Cat-A CR to Removal of brackets for SCell Dormancy and Simultaneous DCI based BWP switch delay on multiple CCs in Rel-17 LTE | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102885 | Discussion on Cell Configuration for EMR Test Case | Qualcomm Incorporated | noted |  |  |
| R4-2102886 | DraftCR on EN-DC NR SpCell in FR1 and 2 NR SCells in FR2 for Dormant SCell switch via DCI 2_6 within and after 3 OFDM symbols | Qualcomm Incorporated | revised |  | R4-2103567 |
| R4-2102887 | Test framework for SCell dormancy performance requirements | Qualcomm Incorporated | noted |  |  |
| R4-2102888 | Updates on Test Configuration for FR2 Inter-band CA IBM UEs | Qualcomm Incorporated | noted |  |  |
| R4-2102889 | Cat-F CR to addition of TRS Configurations in Rel-16 Test Case | Qualcomm Incorporated | revised |  | R4-2103511 |
| R4-2102890 | Cat-A CR to addition of TRS Configurations in Rel-17 Test Case | Qualcomm Incorporated | agreed |  |  |
| R4-2102891 | UE feature on Simultaneous dormant BWP switching | Qualcomm Incorporated | noted |  |  |
| R4-2102892 | Discussion on PUCCH SCell Activation | Qualcomm Incorporated | noted |  |  |
| R4-2102893 | Discussion on RRM in NTN Systems | Qualcomm Incorporated | noted |  |  |
| R4-2102894 | Discussion on temporary RS for efficient SCell activation in NR CA | Qualcomm Incorporated | noted |  |  |
| R4-2102895 | CR to TS 37.145-2: Updating the orthogonal cut procedure | Nokia, Nokia Shanghai Bell | not pursued |  |  |
| R4-2102896 | CR to TS 37.145-2: Updating the orthogonal cut procedure | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R4-2102897 | On Two orthogonal cuts with pattern multiplication procedures and CLTA maximum height | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102898 | CR to TS 38.141-2: Updating the orthogonal cut procedure | Nokia, Nokia Shanghai Bell | not pursued |  |  |
| R4-2102899 | CR to TS 38.141-2: Updating the orthogonal cut procedure | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R4-2102900 | NR-U Punctured Channel SEM for 100 MHz Bandwidth | CableLabs | noted |  |  |
| R4-2102901 | CR to TR 37.941: Updating the orthogonal cut procedure | Nokia, Nokia Shanghai Bell | postponed |  |  |
| R4-2102902 | CR to TR 37.941: Updating the orthogonal cut procedure | Nokia, Nokia Shanghai Bell | withdrawn |  |  |
| R4-2102903 | CR on split band duplexer exceptions to non-default TX-RX separation | Qualcomm Incorporated | not pursued |  |  |
| R4-2102904 | Non-default RX-TX Frequency Separation Values and split band duplexers | Qualcomm Incorporated | noted |  |  |
| R4-2102905 | CR on correcting SSB and RACH configuration in CSI-RS based beam failure detection and link recovery tests | Qualcomm CDMA Technologies | agreed |  |  |
| R4-2102906 | EIRP requirements of band n262 | Intel Corporation | noted |  |  |
| R4-2102907 | EIS requirements of band n262 | Intel Corporation | noted |  |  |
| R4-2102908 | CR on correcting SSB and RACH configuration in CSI-RS based beam failure detection and link recovery tests | Qualcomm CDMA Technologies | agreed |  |  |
| R4-2102909 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | revised |  | R4-2103380 |
| R4-2102910 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | revised |  | R4-2103381 |
| R4-2102911 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | withdrawn |  |  |
| R4-2102912 | Draft test case CR on measurement performance of L1-SINR for CSI-RS-based CMR and no dedicated IMR | Qualcomm CDMA Technologies | revised | R4-2014292 | R4-2103598 |
| R4-2102913 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | revised |  | R4-2103382 |
| R4-2102914 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | revised |  | R4-2103383 |
| R4-2102915 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | revised |  | R4-2103384 |
| R4-2102916 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | revised |  | R4-2103385 |
| R4-2102917 | On the EVM Definition for Transmit Diversity and Antenna Ports | Lenovo, Motorola Mobility | noted |  |  |
| R4-2102918 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | revised |  | R4-2103386 |
| R4-2102919 | Interruptions during SCell activation in NR-U | Qualcomm Incorporated | noted |  |  |
| R4-2102920 | Random Access requirements in NR-U | Qualcomm Incorporated | noted |  |  |
| R4-2102921 | Performance requirements in NR-U | Qualcomm Incorporated | noted |  |  |
| R4-2102922 | CR on Interruptions during Scell activation in NR-U | Qualcomm Incorporated | revised |  | R4-2103516 |
| R4-2102923 | CR on Interruptions during Scell activation in NR-U | Qualcomm Incorporated | agreed |  |  |
| R4-2102924 | Discussion on FR2 UE Min. Output Power Requirement | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102925 | Completion of beam correspondence requirements for all power classes | Qualcomm Incorporated | withdrawn |  |  |
| R4-2102926 | Correction to CSSF for PRS measurements | Ericsson-LG Co., LTD | withdrawn |  |  |
| R4-2102927 | 35MHz 45MHz REFSENS | Skyworks Solutions Inc. | noted |  |  |
| R4-2102928 | Cross-band MSD for ENDC and NR-CA BCS4 | Skyworks Solutions Inc. | noted |  |  |
| R4-2102929 | UE-UE Coexistence for Asynchronous n40 n41 Networks | Skyworks Solutions Inc. | noted |  |  |
| R4-2102930 | Discussion on band n77 PC1.5 operation | Skyworks Solutions Inc. | noted |  |  |
| R4-2102931 | UE-UE Coexistence for Asynchronous n40 n41 Networks | Skyworks Solutions Inc. | noted |  |  |
| R4-2102932 | n48 30MHz A-MPR Measurements | Skyworks Solutions Inc. | noted |  |  |
| R4-2102933 | Discussion on NR UE demodulation performance for n262 | HUAWEI TECHNOLOGIES Co. Ltd. | noted |  |  |
| R4-2102934 | On PRS-RSTD measurement period definition | Nokia, Nokia Shanghai Bell | noted |  |  |
| R4-2102935 | Discussion on NR BS demodulation performance for n262 | HUAWEI TECHNOLOGIES Co. Ltd. | noted |  |  |
| R4-2102936 | IAB-RRM Conformance Testing | Qualcomm CDMA Technologies | noted |  |  |
| R4-2102937 | Discussion and reply draft LS on BCS reporting and support for intra-band EN-DC band combinations | Xiaomi | noted | R4-2101111 |  |
| R4-2102938 | Introduction of PMI test cases with Rel-16 eType II codebook | Samsung | revised | R4-2100896 | R4-2103833 |
| R4-2102939 | Introduction of PMI test cases with Rel-15 Type II codebook | Samsung | revised | R4-2100897 | R4-2103843 |
| R4-2102940 | Motivation paper for Rail Mobile Radio (RMR) | UIC | not treated | - | - |
| R4-2102941 | WID for 900MHz spectrum block | UIC | not treated | - | - |
| R4-2102942 | WID for 1900MHz spectrum block | UIC | not treated | - | - |
| R4-2102943 | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into TS 38.101-1 | ZTE Corporation | agreed | - | - |
| R4-2102944 | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into TS 38.101-3 | ZTE Corporation | agreed | - | - |
| R4-2102945 | CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into TS 38.101-1 | ZTE Corporation | agreed | - | - |
| R4-2102946 | CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into TS 38.101-3 | ZTE Corporation | agreed | - | - |
| R4-2102947 | CR to reflect the completed ENDC combinations for 3 bands DL with 3 bands UL into TS 38.101-3 | ZTE Corporation | agreed | - | - |
| R4-2102948 | CR to reflect the completed Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL1UL) and 3 bands NR inter-band CA (3DL1UL) into TS38.101-3 | ZTE Corporation | agreed | - | - |
| R4-2102949 | Email discussion summary for [98e][101] NR_NewRAT_SysParameters | Moderator (ZTE) | revised | - | R4-2103291 |
| R4-2102950 | Email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 | Moderator (Nokia) | revised | - | R4-2103292 |
| R4-2102951 | Email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 | Moderator (Apple) | revised | - | R4-2103293 |
| R4-2102952 | Email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 | Moderator (Huawei) | revised | - | R4-2103294 |
| R4-2102953 | Email discussion summary for [98e][105] LTE_Maintenance | Moderator (Skyworks) | revised | - | R4-2103295 |
| R4-2102954 | Email discussion summary for [98e][106] NR_unlic_Maintenance | Moderator (Qualcomm) | revised | - | R4-2103296 |
| R4-2102955 | Email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF | Moderator (LG Electronics) | revised | - | R4-2103297 |
| R4-2102956 | Email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF | Moderator (Ericsson) | revised | - | R4-2103298 |
| R4-2102957 | Email discussion summary for [98e][109] NR_RF_FR1_Part_1 | Moderator (Huawei) | revised | - | R4-2103299 |
| R4-2102958 | Email discussion summary for [98e][110] NR_RF_FR2_req_enh_Part_4 | Moderator (Nokia) | revised | - | R4-2103300 |
| R4-2102959 | Email discussion summary for [98e][111] NR_transient_period | Moderator (CMCC) | revised | - | R4-2103301 |
| R4-2102960 | Email discussion summary for [98e][112] NR_TxD | Moderator (vivo) | revised | - | R4-2103302 |
| R4-2102961 | Email discussion summary for [98e][113] NR_R16_Maintenance | Moderator (OPPO) | revised | - | R4-2103303 |
| R4-2102962 | Email discussion summary for [98e][114] R16_UE_ feature | Moderator (CMCC) | revised | - | R4-2103304 |
| R4-2102963 | Email discussion summary for [98e][115] NR_Baskets_Part_1 | Moderator (Nokia) | noted | - | - |
| R4-2102964 | Email discussion summary for [98e][116] NR_Baskets_Part_2 | Moderator (Nokia) | noted | - | - |
| R4-2102965 | Email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL | Moderator (China Telecom) | revised | - | R4-2103305 |
| R4-2102966 | Email discussion summary for [98e][118] NR_PC2_CA_R17_2BDL_2BUL | Moderator (China Telecom) | revised | - | R4-2103306 |
| R4-2102967 | Email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD | Moderator (China Unicom) | revised | - | R4-2103307 |
| R4-2102968 | Email discussion summary for [98e][120] NR_bands_R17_BWs | Moderator (Ericsson) | revised | - | R4-2103308 |
| R4-2102969 | Email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW | Moderator (Huawei) | revised | - | R4-2103309 |
| R4-2102970 | Email discussion summary for [98e][122] NR_LTE_V2X_PC5_combos | Moderator (CATT) | revised | - | R4-2103310 |
| R4-2102971 | Email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 | Moderator (SoftBank) | revised | - | R4-2103311 |
| R4-2102972 | Email discussion summary for [98e][124] NR_47GHz_Band | Moderator (Nokia) | revised | - | R4-2103312 |
| R4-2102973 | Email discussion summary for [98e][125] NR_LTE_band_n24 | Moderator (Ligado Networks) | revised | - | R4-2103313 |
| R4-2102974 | Email discussion summary for [98e][126] NR_n67 | Moderator (Ericsson) | revised | - | R4-2103314 |
| R4-2102975 | Email discussion summary for [98e][127] NR_n85 | Moderator (Ericsson) | revised | - | R4-2103315 |
| R4-2102976 | Email discussion summary for [98e][128] NR_BCS4 | Moderator (Ericsson) | revised | - | R4-2103316 |
| R4-2102977 | Email discussion summary for [98e][129] NR_req_Intra_UL_CA | Moderator (Huawei) | revised | - | R4-2103317 |
| R4-2102978 | Email discussion summary for [98e][130] NR_bands_UL_MIMO_PC3_R17 | Moderator (Huawei) | noted | - | - |
| R4-2102979 | Email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 | Moderator (China Telecom) | revised | - | R4-2103318 |
| R4-2102980 | Email discussion summary for [98e][132] HPUE_PC1_5_n77_n78 | Moderator (Qualcomm) | revised | - | R4-2103319 |
| R4-2102981 | Email discussion summary for [98e][133] NR_6GHz_unlic_EU | Moderator (Nokia) | revised | - | R4-2103320 |
| R4-2102982 | Email discussion summary for [98e][134] FS_6425_10500MHz_NR | Moderator (Ericsson) | revised | - | R4-2103321 |
| R4-2102983 | Email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 | Moderator (Huawei) | revised | - | R4-2103322 |
| R4-2102984 | Email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2 | Moderator (China Telecom) | revised | - | R4-2103323 |
| R4-2102985 | Email discussion summary for [98e][137] NR_RF_FR1_enh_Part_3 | Moderator (Qualcomm) | revised | - | R4-2103324 |
| R4-2102986 | Email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1 | Moderator (Nokia) | revised | - | R4-2103325 |
| R4-2102987 | Email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 | Moderator (Qualcomm) | revised | - | R4-2103326 |
| R4-2102988 | Email discussion summary for [98e][140] NR_RF_FR2_req_enh2_Part_3 | Moderator (Apple) | revised | - | R4-2103327 |
| R4-2102989 | Email discussion summary for [98e][141] NR_HST_FR2_enh | Moderator (Samsung) | revised | - | R4-2103328 |
| R4-2102990 | Email discussion summary for [98e][142] NRSL_enh_Part_1 | Moderator (LG Electronics) | revised | - | R4-2103329 |
| R4-2102991 | Email discussion summary for [98e][143] NRSL_enh_Part_2 | Moderator (CATT) | revised | - | R4-2103330 |
| R4-2102992 | Email discussion summary for [98e][144] NRSL_enh_Part_3 | Moderator (Huawei) | revised | - | R4-2103331 |
| R4-2102993 | Email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 | Moderator (Qualcomm) | revised | - | R4-2103332 |
| R4-2102994 | Email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 | Moderator (Intel) | revised | - | R4-2103333 |
| R4-2102995 | Email discussion summary for [98e][147] FS_NR_eff_BW_util | Moderator (Ericsson) | revised | - | R4-2103334 |
| R4-2102996 | Email discussion summary for [98e][148] FS_NR_600MHz_ext | Moderator (Ericsson) | revised | - | R4-2103335 |
| R4-2102997 | Email discussion summary for [98e][149] FS_NR_PC2_UE_FDD | Moderator (China Unicom) | revised | - | R4-2103336 |
| R4-2102998 | Email discussion summary for [98e][150] LTE_Baskets | Moderator (Ericsson) | noted | - | - |
| R4-2102999 | Email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 | Moderator (Ericsson) | revised | - | R4-2103337 |
| R4-2103000 | Email discussion summary for [98e][152] FS_LTE_NR_HPUE_FWVM | Moderator (Nokia) | revised | - | R4-2103338 |
| R4-2103001 | Email discussion summary for [98e][153] BC_simplification | Moderator (NTT DOCOMO) | revised | - | R4-2103339 |
| R4-2103002 | Email discussion summary for [98e][154] NR_reply_LS_Part_1 | Moderator (Xiaomi) | revised | - | R4-2103340 |
| R4-2103003 | Email discussion summary for [98e][155] NR_reply_LS_Part_2 | Moderator (China Telecom) | revised | - | R4-2103341 |
| R4-2103004 | Email discussion summary for [98e][156] NR_reply_LS_Part_3 | Moderator (Samsung) | revised | - | R4-2103342 |
| R4-2103005 | Additional Information for SL Operation in NR Band n14 | AT&T, FirstNet | noted | R4-2101857 | - |
| R4-2103006 | TP to TR 37.717-21-11: DC_20-40_n78 | Nokia, Telefonica | approved | R4-2100145 | - |
| R4-2103007 | TP to TR 37.717-31-11: DC_1-20-40_n78 | Nokia, Telefonica | approved | R4-2100146 | - |
| R4-2103008 | TP to TR 37.717-41-11: DC_1-3-20-40_n78 | Nokia, Telefonica | approved | R4-2100147 | - |
| R4-2103009 | Requirement of band 46 reference sensitivity measurement for NR EN-DC band combinations | Verizon Denmark | approved | R4-2100258 | - |
| R4-2103010 | TP for TR 37.717-21-11: DC_3-18_n41 | KDDI Corporation | approved | R4-2100345 | - |
| R4-2103011 | TP update for TR 37.717-21-11: EN-DC_1-11_n28 | SoftBank Corp., LG Electronics | approved | R4-2100642 | - |
| R4-2103012 | TP to TR 37.717-11-11 DC_25_n77 | Nokia, Nokia Shanghai Bell | approved | R4-2100732 | - |
| R4-2103013 | TP to TR 37.717-11-11 DC_25_n78 | Nokia, Nokia Shanghai Bell | approved | R4-2100733 | - |
| R4-2103014 | TP to TR 37.717-21-11 DC_7-25_n77 | Nokia, Nokia Shanghai Bell | approved | R4-2100734 | - |
| R4-2103015 | TP to TR 37.717-21-11 DC_7-25_n78 | Nokia, Nokia Shanghai Bell | approved | R4-2100735 | - |
| R4-2103016 | TP to TR 37.717-21-11 DC_25-66_n77 | Nokia, Nokia Shanghai Bell | approved | R4-2100736 | - |
| R4-2103017 | TP to TR 37.717-21-11 DC_25-66_n78 | Nokia, Nokia Shanghai Bell | approved | R4-2100737 | - |
| R4-2103018 | TP for TR 37.717-21-11: DC_2-29_n78 | Samsung, TELUS, Bell mobility | approved | R4-2100983 | - |
| R4-2103019 | TP for TR 37.717-21-11: DC_29-66_n78 | Samsung, TELUS, Bell mobility | approved | R4-2100984 | - |
| R4-2103020 | TP for TR 37.717-31-11: DC_2-29-66_n78 | Samsung, TELUS, Bell mobility | withdrawn | R4-2100985 | - |
| R4-2103021 | TP for DC_21_n28 for TR 37.717-11-11 | NTT DOCOMO INC. | approved | R4-2101224 | - |
| R4-2103022 | TP for DC_1-21_n28 for TR 37.717-21-11 | NTT DOCOMO INC. , MediaTek Inc., LG Electronics | approved | R4-2101230 | - |
| R4-2103023 | TP for TR 37.717-21-11: DC_8-20_n1 | VODAFONE Group Plc | approved | R4-2101545 | - |
| R4-2103024 | TP for TR 37.717-21-11: DC_8-20_n3 | VODAFONE Group Plc | approved | R4-2101546 | - |
| R4-2103025 | (reserved) | Main session | not treated | - | - |
| R4-2103026 | TP for TR 37.717-11-11: DC_12_n71 | Huawei, HiSilicon, Ericsson, US Cellular | approved | R4-2101584 | - |
| R4-2103027 | TP for TR 37.717-11-11 to include 71A_n41A | Ericsson, Rogers | approved | R4-2101909 | - |
| R4-2103028 | TP for TR 37.717-11-11 to include 71A_n2A | Ericsson, Rogers | approved | R4-2101911 | - |
| R4-2103029 | draft CR 38.101-3 adding CA_n7B UL configurations for 1 LTE + 1 NR | Ericsson | not pursued | R4-2102045 | - |
| R4-2103030 | draft CR 38.101-3 adding CA_n7B UL configurations for 2 LTE + 1 NR | Ericsson | not pursued | R4-2102046 | - |
| R4-2103031 | draft CR 38.101-3 adding CA_n7B UL configurations for 3 LTE + 1 NR | Ericsson | not concluded | - | - |
| R4-2103032 | draft CR 38.101-3 adding CA_n7B UL configurations for 3 LTE + 1 NR | Ericsson | not pursued | R4-2102047 | - |
| R4-2103033 | Rel-17 CR 38.101-1 for improvements Intra-band tables | Ericsson | agreed | R4-2102319 | - |
| R4-2103034 | Draft CR on CA_n1-n3, CA_n1-n78, CA_n3-n78 | China Unicom | withdrawn | R4-2100091 | - |
| R4-2103035 | draftCR for Rel-17 NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL within FR1 | Verizon Denmark | endorsed | R4-2100259 | - |
| R4-2103036 | Draft CR for 38.101-1 to introduce CA_n3A-n78(2A) and CA_n41-n78(2A) | Samsung, KDDI | endorsed | R4-2100942 | - |
| R4-2103037 | TP for TR 38.717-02-01: CA_n3-n18 | Samsung, KDDI | approved | R4-2100943 | - |
| R4-2103038 | TP for TR 38.717-02-01: CA_n18-n41 | Samsung, KDDI | approved | R4-2100944 | - |
| R4-2103039 | TP for TR 38.717-02-01: DC_n3-n41 | Samsung, KDDI | approved | R4-2100947 | - |
| R4-2103040 | TP for TR 38.717-02-01: CA_n25-n77 | Samsung, TELUS, Bell mobility | noted | R4-2100975 | - |
| R4-2103041 | TP for TR 38.717-02-01: CA_n71-n77 | Samsung, TELUS, Bell mobility | noted | R4-2100976 | - |
| R4-2103042 | TP for TR 38.717-02-01: CA_n13-n25 | Samsung, TELUS, Bell mobility | approved | R4-2100977 | - |
| R4-2103043 | TP for TR 38.717-02-01: CA_n13-n66 | Samsung, TELUS, Bell mobility | approved | R4-2100978 | - |
| R4-2103044 | TP for TR 38.717-02-01: CA_n3A-n39A | Huawei, HiSilicon | withdrawn | R4-2101600 | - |
| R4-2103045 | DraftCR for 38.101-1 to add BCS1 for CA_n1A-n3A | Huawei, HiSilicon | revised | R4-2101602 | R4-2103377 |
| R4-2103046 | draft CR for CA_n46-n48 and DC_n46-n48 configurations | Ericsson, Charter Communication | endorsed | R4-2101894 | - |
| R4-2103047 | TP for TR 38.717-02-01 to include CA_n7-n77 | Ericsson, Bell Mobility | approved | R4-2102314 | - |
| R4-2103048 | Draft CR on CA_n1-n258, CA_n3-n258, CA_n78-n258 | China Unicom | withdrawn | R4-2100092 | - |
| R4-2103049 | TP for TR 38.717-02-01: CA_n41-n257 | Samsung, KDDI | approved | R4-2100946 | - |
| R4-2103050 | MSD anlaysis results for new DC band combinations | LG Electronics France | approved | R4-2100279 | - |
| R4-2103051 | TP for TR 37.717-11-21: to update DC_18_n3-n41 with IMD3 MSD | Mediatek India Technology Pvt. | approved | R4-2100300 | - |
| R4-2103052 | TP for TR 37.717-11-21:DC_3A-40A_n1A-n78A/DC_3A-40C_n1A-n78A | Huawei,HiSilicon | approved | R4-2100325 | - |
| R4-2103053 | TP for TR 37.717-11-21:DC_7A-40A_n1A-n78A/DC_7A-40C_n1A-n78A | Huawei,HiSilicon | approved | R4-2100326 | - |
| R4-2103054 | TP for TR 37.717-11-21:DC_8A-40A_n1A-n78A/DC_8A-40C_n1A-n78A | Huawei,HiSilicon | approved | R4-2100327 | - |
| R4-2103055 | TP for TR 37.717-11-21:DC_3A-7A-40A_n1A-n78A/DC_3A-7A-40C_n1A-n78A | Huawei,HiSilicon | approved | R4-2100328 | - |
| R4-2103056 | TP for TR 37.717-11-21:DC_3A-8A-40A_n1A-n78A/DC_3A-8A-40C_n1A-n78A | Huawei,HiSilicon | approved | R4-2100329 | - |
| R4-2103057 | TP for TR 37.717-11-21:DC_7A-8A-40A_n1A-n78A/DC_7A-8A-40C_n1A-n78A | Huawei,HiSilicon | approved | R4-2100330 | - |
| R4-2103058 | TP for TR 37.717-11-21:DC_3A-7A-8A-40A_n1A-n78A/DC_3A-7A-8A-40C_n1A-n78A | Huawei,HiSilicon | approved | R4-2100331 | - |
| R4-2103059 | TP for TR 37.717-11-21: DC_1-3-18_n3-n41 | KDDI Corporation | approved | R4-2100346 | - |
| R4-2103060 | TP for TR 37.717-11-21: DC_1-3-18_n28-n41 | KDDI Corporation | approved | R4-2100347 | - |
| R4-2103061 | TP for TR 37.717-11-21: DC_1-3-18_n41-n77 | KDDI Corporation | approved | R4-2100348 | - |
| R4-2103062 | TP for TR 37.717-11-21: DC_1-3-18_n41-n78 | KDDI Corporation | approved | R4-2100349 | - |
| R4-2103063 | TP for DC_21_n28-n79 for TR 37.717-11-21 | NTT DOCOMO INC. , MediaTek Inc., LG Electronics | approved | R4-2101229 | - |
| R4-2103064 | TP for TR 37.717-11-21:DC_8A_n28A-n78A | Huawei, HiSilicon | approved | R4-2101585 | - |
| R4-2103065 | TP for TR 37.717-11-21:DC_1A-3A-8A_n28A-n78A | Huawei, HiSilicon | approved | R4-2101589 | - |
| R4-2103066 | draft CR 38.101-3 adding CA_n7B UL configurations for x LTE + 2 NR | Ericsson | not pursued | R4-2102048 | - |
| R4-2103067 | TP for TR 37.717-11-21 to include 2A_n2a-n78a | Ericsson, Rogers | approved | R4-2102068 | - |
| R4-2103068 | TP for TR 37.717-11-21 to include 71A_n2a-n78a | Ericsson, Rogers | approved | R4-2102070 | - |
| R4-2103069 | TP for TR 37.717-11-21 to include 66A_n2a-n66a | Ericsson, Rogers | approved | R4-2102079 | - |
| R4-2103070 | TP for TR 37.717-00-00 for CA_n3A_SUL_n78A-n80A | Huawei, HiSilicon | approved | R4-2101603 | - |
| R4-2103071 | TP for TR 37.717-00-00 for CA_n41A_SUL_n79A-n83A | Huawei, HiSilicon | approved | R4-2101604 | - |
| R4-2103072 | TP for TR 37.717-00-00 for CA_n79A_SUL_n41A-n83A | Huawei, HiSilicon | approved | R4-2101605 | - |
| R4-2103073 | TP for TR 37.717-00-00 for SUL_n41A-n97A | Huawei, HiSilicon, CMCC | withdrawn | R4-2101608 | - |
| R4-2103074 | TP for TR 37.717-00-00 for SUL_n79A-n97A | Huawei, HiSilicon, CMCC | approved | R4-2101609 | - |
| R4-2103075 | TP for TR 37.717-00-00 for SUL_n41A-n98A | Huawei, HiSilicon, CMCC | approved | R4-2101610 | - |
| R4-2103076 | Draft CR on CA_n1-n3-n78 | China Unicom | withdrawn | R4-2100093 | - |
| R4-2103077 | TP to TR 38.717-03-01 CA_n5-n25-n77 | Nokia, Nokia Shanghai Bell | approved | R4-2100738 | - |
| R4-2103078 | TP to TR 38.717-03-01 CA_n25-n66-n77 | Nokia, Nokia Shanghai Bell | approved | R4-2100739 | - |
| R4-2103079 | TP for TR 38.717-03-01: CA_n3A-n28A-n77(2A) | Samsung, KDDI | approved | R4-2100953 | - |
| R4-2103080 | TP for TR 38.717-03-01: CA_n3A-n28A-n78(2A) | Samsung, KDDI | approved | R4-2100954 | - |
| R4-2103081 | TP for TR 38.717-03-01: CA_n13-n25-n66 | Samsung, TELUS, Bell mobility | approved | R4-2100979 | - |
| R4-2103082 | TP for TR 38.717-04-01 to include CA_n41-n66-n71-n77 | Ericsson, T-Mobile US | approved | R4-2101904 | - |
| R4-2103083 | Draft CR Addition of BCS1 for CA_n25A-n71(2A), CA_n41(2A)-n66A and CA_n66A-n71(2A) | Nokia, T-Mobile USA | withdrawn | R4-2100153 | - |
| R4-2103084 | TP to TR 38.717-03-02: CA_n25-n41-n71 | Nokia, T-Mobile USA | approved | R4-2100158 | - |
| R4-2103085 | TP to TR 38.717-03-02 CA_n5-n25-n77 | Nokia, Nokia Shanghai Bell | approved | R4-2100740 | - |
| R4-2103086 | TP for TR 38.717-03-02: DC_n3-n41-n257 | Samsung, KDDI | approved | R4-2100960 | - |
| R4-2103087 | TP for TR 38.717-03-02: CA_n3-n18-n41 | Samsung, KDDI | approved | R4-2100964 | - |
| R4-2103088 | TP for TR 38.717-03-02: CA_n3-n41-n77 | Samsung, KDDI | approved | R4-2100967 | - |
| R4-2103089 | TP for TR 38.717-03-02: CA_n3-n41-n78 | Samsung, KDDI | approved | R4-2100968 | - |
| R4-2103090 | TP for TR 38.717-03-02: CA_n28-n41-n77 | Samsung, KDDI | approved | R4-2100969 | - |
| R4-2103091 | TP for TR 38.717-03-02: CA_n28-n41-n78 | Samsung, KDDI | approved | R4-2100970 | - |
| R4-2103092 | TP for TR 38.717-03-02: CA_n25-n29-n66 | Samsung, TELUS, Bell mobility | approved | R4-2100981 | - |
| R4-2103093 | TP for TR 38.717-03-02: CA_n13-n25-n66 | Samsung, TELUS, Bell mobility | approved | R4-2100982 | - |
| R4-2103094 | TP for TR 38.717-03-02: CA_n38A-n66A-n78A | Huawei, HiSilicon, Bell Mobility, Telus | approved | R4-2101514 | - |
| R4-2103095 | TP for 38.717-03-02 to include n25-n66-n71 | Ericsson, T-Mobile US | approved | R4-2101902 | - |
| R4-2103096 | TP for TR 36.717-03-01: CA_1-28-32 | VODAFONE Group Plc | approved | R4-2101399 | - |
| R4-2103097 | Way forward on introduction of NR intra-band UL CA as UL configuration in an inter-band combination | Skyworks | approved | - | - |
| R4-2103098 | TP for TR 36.717-03-02: to add UL configuration CA_1A-8A for CA_1A-8A-38A | Huawei, HiSilicon | approved | R4-2101579 | - |
| R4-2103099 | TP for 38.717-03-02 to include n25-n41-n66 | Ericsson, Bell Mobility | approved | R4-2102316 | - |
| R4-2103100 | TP to TR 38.717-03-02 CA_n25-n66-n77 | Nokia, Nokia Shanghai Bell | approved | R4-2100741 | - |
| R4-2103101 | TP for TR 38.717-03-02: CA_n3-n28-n78 | Samsung, KDDI | approved | R4-2100959 | - |
| R4-2103102 | TP for TR 38.717-02-01: CA_n25-n71 | Samsung, TELUS, Bell mobility | approved | R4-2100974 | - |
| R4-2103103 | Way forward on WF on remaining BS and UE parameters for 6.425-7.125 and 10.0-10.5 GHz | Ericsson | revised | - | R4-2103396 |
| R4-2103104 | LS to WP5D on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 | RAN4 | approved | R4-2102840 | - |
| R4-2103105 | TP to TR 38.921 UE transmitter requirements | ZTE Corporation | revised | R4-2101948 | R4-2103397 |
| R4-2103106 | TP to TR 38.921: BS remaining parameters | Huawei, HiSilicon | revised | R4-2101496 | R4-2103398 |
| R4-2103107 | TP to TR 38.921 summary of simulation results | ZTE Corporation, Nokia, Nokia Shanghai Bell, CATT, Qualcomm | approved | R4-2101950 | - |
| R4-2103108 | TP to TR 38.921: Clarification of beamforming pattern modelling for multiple UL schedued UEs | Qualcomm Incorporated | withdrawn | R4-2102500 | - |
| R4-2103109 | TP to TR 38.921: Addition of in-door antenna parameters and correction to model in subclause 8.1 | Ericsson | approved | R4-2101182 | - |
| R4-2103110 | Way forward on introduction of new FR2 CA BW classes | Verizon | revised | - | R4-2103359 |
| R4-2103111 | Way forward on FR2 UEs that support inter-band DL CA with IBM | Sony | approved | - | - |
| R4-2103112 | Way forward on FR2 UEs that support inter-band DL CA with CBM | Samsung | revised | - | R4-2103399 |
| R4-2103113 | Way forward on FR2 UEs that support inter-band UL CA | MediaTek | approved | - | - |
| R4-2103114 | Way forward on UL gap for FR2 RF | Apple | approved | - | - |
| R4-2103115 | Way forward on BCS reporting and support for intra-band EN-DC band combinations | T-Mobile USA | noted | - | - |
| R4-2103116 | CR to TS38.101-1: Correction on the Aggregated Channel Bandwidth | ZTE Corporation | agreed | R4-2102197 | - |
| R4-2103117 | CR to TS38.104: Correction on the Aggregated Channel Bandwidth | ZTE Corporation | agreed | R4-2102200 | - |
| R4-2103118 | LS on further clarification of simultaneous Rx/Tx capability | Apple | withdrawn | - | - |
| R4-2103119 | Correction to applicability of simultaneous RX/TX | Ericsson | not pursued | R4-2101713 | - |
| R4-2103120 | Way forward on applicability of additional emission requirement to CA/DC | SoftBank | approved | - | - |
| R4-2103121 | Way forward on WF on Improvement of UL RMC tables | Rohde & Schwarz | approved | - | - |
| R4-2103122 | CR for TS38 101-1 Rel-16 Correction of condition for MPR and delta MPR | CATT | revised | R4-2100395 | R4-2103389 |
| R4-2103123 | CR for TS 38.101-1: Correction to FR1 time mask for SRS antenna switching | Apple | revised | R4-2102597 | R4-2103388 |
| R4-2103124 | LS on SCell dropping | RAN4 | revised | - | R4-2103410 |
| R4-2103410 | LS on SCell dropping | RAN4 | approved |  |  |
| R4-2103125 | Removal of a remaining NS_201 related requirement | Nokia, Nokia Shanghai Bell | revised | R4-2100085 | R4-2103370 |
| R4-2103126 | CR to 38.101-2 on beam correspondence | Qualcomm, Nokia, Nokia Shanghai Bell, Samsung, Verizon, NTT Docomo, Sony, Ericsson | agreed | R4-2102664 | - |
| R4-2103127 | P_min correction and P_cmax CA correction to apply from all cells | Qualcomm Incorporated | not pursued | R4-2100586 | - |
| R4-2103128 | CR on simultaneous Tx-Rx for EN-DC | OPPO | not pursued | R4-2101746 | - |
| R4-2103129 | CR on simultaneous Tx-Rx for EN-DC (R16 CR) | OPPO | not pursued | R4-2101747 | - |
| R4-2103130 | Way forward on UE capability on intraBandENDC-Support | Huawei | approved | - | - |
| R4-2103131 | Way forward on LTE band 41 NS04 A-MPR for UL 256 QAM | T-Mobile USA, Huawei, HiSilicon, T-mobile USA, Qualcomm Inc | approved | - | - |
| R4-2103132 | CR for TS 36.101: Cleanup for spurious emissions for UE co-existence table | Apple | agreed | R4-2102596 | - |
| R4-2103133 | CR for TS 36.101: Cleanup for spurious emissions for UE co-existence table | Apple | agreed | R4-2102604 | - |
| R4-2103134 | Addition of UE co-existence requirements for 40 and n40 | NTT DOCOMO, INC., SoftBank Corp., KDDI Corporation, Rakuten Mobile, Inc | approved | R4-2101197 | - |
| R4-2103135 | Applicability of minimum requirements for shared spectrum access | Ericsson | not pursued | R4-2101720 | - |
| R4-2103136 | Applicability of minimum requirements for shared spectrum access | Ericsson | withdrawn | - | - |
| R4-2103137 | CR to TS 38.104: system parameters maintenance for NR-U | ZTE Corporation | agreed | R4-2101968 | - |
| R4-2103138 | CR to TS 38.101-1: system parameters maintenance for NR-U | ZTE Corporation | agreed | R4-2101970 | - |
| R4-2103139 | LS on the clarification on the name of parameters for 5G V2X UE | Qualcomm | withdrawn | - | - |
| R4-2103140 | CR on editorial correction on V2X operation in TS38.101-1 in Rel-16 | LG Electronics France | agreed | R4-2100280 | - |
| R4-2103141 | CR for TS 38.101-3, General corrections for NR V2X | CATT | agreed | R4-2100405 | - |
| R4-2103142 | Missing parent clause for NR-DC PCMAX | Qualcomm Incorporated | agreed | R4-2102408 | - |
| R4-2103143 | Discussion and reply LS on p-NR-FR2 | vivo | noted | R4-2102711 | - |
| R4-2103144 | LS to RAN2 on single-uplink operation in more than one band pair of a band combination | RAN4 | approved | - | - |
| R4-2103145 | LS to RAN2 on single-uplink operation | Huawei | revised | - | R4-2103365 |
| R4-2103146 | Way forward on intra-band UL CA Pcmax | Huawei | approved | - | - |
| R4-2103147 | Introduction of specific Pcmax requirements for inter-band CA category A-B combos | InterDigital Communications, Charter Communications | agreed | - | - |
| R4-2103148 | Corrections to PCMAX for UL CA | Qualcomm Incorporated | agreed | R4-2102410 | - |
| R4-2103149 | CR for TS 38.101-1: Corrections to intra-band UL NC CA requirements | Apple | agreed | R4-2102600 | - |
| R4-2103150 | Way forward on addition of new frequency FR2 frequency separation classes | Qualcomm, Huawei, Nokia | approved | - | - |
| R4-2103151 | Way forward on transient period | CMCC | revised | - | R4-2103361 |
| R4-2103152 | CR on introduction of shorter Transient Period Capability | Qualcomm Incorporated | agreed | R4-2101484 | - |
| R4-2103153 | CR on TS 38.101-1 time mask for shorter transient | Huawei, HiSilicon | withdrawn | R4-2102684 | - |
| R4-2103154 | Way forward on NR TxD | vivo | revised | - | R4-2103390 |
| R4-2103155 | LS on Signaling scheme of Transparent TxD | vivo | revised | - | R4-2103360 |
| R4-2103156 | CR for TS 38.101-1: TxD requirements | Huawei, HiSilicon | endorsed | R4-2102384 | - |
| R4-2103157 | Way forward on UE co-existence between n40 and n41 | Huawei | approved | - | - |
| R4-2103158 | Way forward on non-default RX-TX Frequency Separation | Qualcomm | noted | - | - |
| R4-2103159 | Modification of Pcmax for UL CA with uplink Tx switching capability | Ericsson | not pursued | R4-2101723 | - |
| R4-2103160 | CR for TS 38.101-1: Cleanup for spurious emissions for UE co-existence table | Apple | agreed | R4-2102602 | - |
| R4-2103161 | CR to TS 38.101-3 on correction to hanging paragraph in the spec (Rel-16) | ZTE Corporation | agreed | R4-2100129 | - |
| R4-2103162 | TS 38.101-3: Addition of missing lower order fallbacks R16 | Nokia, AT&T | agreed | R4-2100148 | - |
| R4-2103163 | CR to TS 38.101-3 clarification on the single uplink allowance for DC_3A_n3A | CHTTL | agreed | R4-2101176 | - |
| R4-2103164 | Requirements Type 2 UEs supporting inter-band MRDC with overlapping DL | Ericsson | agreed | R4-2101725 | - |
| R4-2103165 | Notational amendment and correction to PCMAX for EN-DC | Qualcomm Incorporated | agreed | R4-2102412 | - |
| R4-2103166 | Way forward on requirements for Type 2 UEs supporting inter-band MRDC with overlapping DL | Ericsson | approved | - | - |
| R4-2103167 | Correction to modified MPR behaviour | Ericsson | not pursued | R4-2101724 | - |
| R4-2103168 | Way forward on new per BC indication of the per-FR gap | CMCC | revised | - | R4-2103366 |
| R4-2103169 | TP for TR38.xxx for PC2 CA_n2A-n77A | Verizon Denmark | approved | R4-2100273 | - |
| R4-2103170 | TP for TR38.xxx for PC2 CA_n5A-n77A | Verizon Denmark | approved | R4-2100274 | - |
| R4-2103171 | TP for TR38.xxx for PC2 CA_n66A-n77A | Verizon Denmark | approved | R4-2100276 | - |
| R4-2103172 | Way forward on SAR solutions for PC2 NR inter-band CA and SUL configurations | China Telecom | approved | - | - |
| R4-2103173 | CR to 38.307 Release independence for UE power class 2 NR inter-band CA and SUL configurations (R17) | China Telecom | agreed | R4-2101124 | - |
| R4-2103174 | Way forward on increasing UE maximum power for UE equipped with two PAs | Qualcomm | noted | - | - |
| R4-2103175 | TP for TR 37.826 for DC_2_n77 | Verizon Denmark | approved | R4-2100266 | - |
| R4-2103176 | TP for TR 37.826 for DC_5_n77 | Verizon Denmark | approved | R4-2100268 | - |
| R4-2103177 | TP for TR 37.826 for DC_13_n77 | Verizon Denmark | approved | R4-2100269 | - |
| R4-2103178 | TP for TR 37.826 for DC_66_n77 | Verizon Denmark | approved | R4-2100271 | - |
| R4-2103179 | Revised Basket WID on adding channel bandwidth support to existing NR bands | Ericsson | endorsed | R4-2102163 | - |
| R4-2103180 | Big CR to TS 38.101-1 - New CBW Basket WI | Ericsson | agreed | R4-2102165 | - |
| R4-2103181 | Way forward on adding 90 and 100MHz channel BW for UE in band n40 | Huawei | approved | - | - |
| R4-2103182 | Draft CR to TS 38.101-1 | Nokia | endorsed | - | - |
| R4-2103183 | Draft CR to TS 38.104 | Nokia | endorsed | - | - |
| R4-2103184 | Way forward on the introduction of 25, 30 and 40MHz in band n2 | AT&T | approved | - | - |
| R4-2103185 | Way forward on the introduction of 25 MHz in band n5 | AT&T | approved | - | - |
| R4-2103186 | Way forward on spectrum utilization for 35MHz and 45MHz | Huawei | approved | - | - |
| R4-2103187 | Way forward on REFSENS for 35MHz and 45MHz | Qualcomm | approved | - | - |
| R4-2103188 | Way forward on A-MPR for 35MHz and 45MHz | Apple | approved | - | - |
| R4-2103189 | Introduction of 35MHz and 45 MHz bandwidths to TS38.101-1 | ZTE Corporation | noted | R4-2102193 | - |
| R4-2103190 | CR for TS 38.104: introduction of channel bandwidths 35MHz and 45MHz | Huawei, HiSilicon | endorsed | R4-2101504 | - |
| R4-2103191 | CR for TS 37.141: introduction of channel bandwidths 35MHz and 45MHz | Huawei, HiSilicon | endorsed | R4-2101505 | - |
| R4-2103192 | CR for TS 37.145-2: introduction of channel bandwidths 35MHz and 45MHz | Huawei, HiSilicon | endorsed | R4-2101506 | - |
| R4-2103193 | CR to TS 37.105: Introduction of CBWs 35 MHz and 45 MHz | Ericsson | endorsed | R4-2101559 | - |
| R4-2103194 | CR to TS 38.141-1: Introduction of CBWs 35 MHz and 45 MHz | Ericsson | endorsed | R4-2101560 | - |
| R4-2103195 | CR to TS 38.141-2: Introduction of 35MHz and 45MHz | ZTE Corporation | withdrawn | - | - |
| R4-2103196 | CR to 37.145-1: Introduction of 35MHz and 45MHz | ZTE Corporation | endorsed | R4-2101987 | - |
| R4-2103197 | CR to 37.104: Introduction of requirements for 35 and 45MHz channel bandwidths | Nokia, Nokia Shanghai Bell | endorsed | R4-2102484 | - |
| R4-2103198 | Updated TR 37.875 on band combinations for con-current operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band | CATT | not concluded | - | - |
| R4-2103199 | TP on V2X_n41A-n47A coexistence study | CATT | approved | R4-2100412 | - |
| R4-2103200 | CR for TS 38.101-1, Introduce new band combination of V2X_n41A-n47A | CATT | agreed | R4-2100413 | - |
| R4-2103201 | CR for TS 38.101-3, Introduce new band combination of V2X_41A-n47A and V2X_n41A-47A | CATT | agreed | R4-2100414 | - |
| R4-2103202 | Way forward on Beam correspondence for FR2 FWA | SoftBank | approved | - | - |
| R4-2103203 | CR for FR2 FWA RF requirements | Huawei, HiSilicon | agreed | R4-2102688 | - |
| R4-2103204 | TP to TR 38.847: UE Tx requirement for n262 | Nokia, Nokia Shanghai Bell | approved | R4-2100749 | - |
| R4-2103205 | Introduction of n262 UE RF requirements | Nokia, Nokia Shanghai Bell | revised | R4-2100750 | R4-2103392 |
| R4-2103206 | TP to TR 38.847: UE Rx requirement for n262 | Nokia, Nokia Shanghai Bell | approved | R4-2100752 | - |
| R4-2103207 | CR to 36.104: Correction to Band 24 requirements (Rel-10) | Nokia, Nokia Shanghai Bell | agreed | R4-2102452 | - |
| R4-2103208 | CR to 37.104: Correction to Band 24 requirements (Rel-10) | Nokia, Nokia Shanghai Bell | agreed | R4-2102468 | - |
| R4-2103209 | n24 / band 24 A-MPR proposal | Nokia, Skyworks Inc | noted | R4-2100135 | - |
| R4-2103210 | CR for TS 38.101-1 introduction of NR band n24 | Ligado Networks, Nokia, Skyworks Solutions Inc. | revised | R4-2100246 | R4-2103378 |
| R4-2103211 | CR to 38101-1 on introducing new SUL band n99 | Huawei,HiSilicon | revised | R4-2100334 | R4-2103351 |
| R4-2103212 | CR to 38104 on introducing new SUL band n99 | Huawei,HiSilicon | revised | R4-2100335 | R4-2103352 |
| R4-2103213 | CR to 37145-1 on introducing new SUL band n99 | Huawei,HiSilicon | agreed | R4-2100343 | - |
| R4-2103214 | CR to 37145-2 on introducing new SUL band n99 | Huawei,HiSilicon | agreed | R4-2100344 | - |
| R4-2103215 | CR for 37.145-2: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed | R4-2100257 | - |
| R4-2103216 | CR to 37.141: Correction to Band 24 requirements (Rel-10) | Nokia, Nokia Shanghai Bell | agreed | R4-2102476 | - |
| R4-2103217 | CR to 36.141: Correction to Band 24 requirements (Rel-10) | Nokia, Nokia Shanghai Bell | agreed | R4-2102460 | - |
| R4-2103218 | Way forward on the introduction of band n67 | Ericsson | approved | - | - |
| R4-2103219 | Way forward on the introduction of band n85 | Ericsson | approved | - | - |
| R4-2103220 | TR skeleton 38.XXX V001 NR_HPUE_intra_Band_R17 | Huawei, HiSilicon | agreed | R4-2102621 | - |
| R4-2103221 | TP on DL applicability of CA_n3-n40-n41 for 37.xxx | CMCC | approved | R4-2100806 | - |
| R4-2103222 | TP on DL applicability of CA_n3-n41-n79 for 37.xxx | CMCC | approved | R4-2100807 | - |
| R4-2103223 | TP on DL applicability of CA_n8-n39-n41for 37.xxx | CMCC | approved | R4-2100808 | - |
| R4-2103224 | TP on DL applicability of CA_n8-n41-n79 for 37.xxx | CMCC | approved | R4-2100809 | - |
| R4-2103225 | TP on DL applicability of CA_n39-n41-n79 for 37.xxx | CMCC | approved | R4-2100810 | - |
| R4-2103226 | TP on DL applicability of CA_n40-n41-n79 for 37.xxx | CMCC | approved | R4-2100811 | - |
| R4-2103227 | Way forward on assumptions for PC1.5 in Band n77 and n78 | Qualcomm | approved | - | - |
| R4-2103228 | Way forward on FWA MPE handling for n77/n78 | Samsung | approved | - | - |
| R4-2103229 | Way forward on introduction of lower 6GHz NR unlicensed operation for Europe | Nokia | approved | - | - |
| R4-2103230 | additional work plan for Rel-17 FR1 UE RF enhancement | Huawei, HiSilicon | approved | R4-2102627 | - |
| R4-2103231 | Way forward on RF architecture and requirements for intra-band UL contiguous CA | Huawei, HiSilicon | approved | - | - |
| R4-2103232 | Way forward on MPR and AMPR for intra-band UL contiguous CA | Skyworks | approved | - | - |
| R4-2103233 | Way forward on 4Rx requirement for CA_n77(3A) and CA_77(4A) | SoftBank | approved | - | - |
| R4-2103234 | LS on Rel-17 Tx switching enhancements | RAN4 | approved | - | - |
| R4-2103235 | Way forward on RF requirements for Rel-17 Tx switching enhancements | China Telecom | approved | - | - |
| R4-2103236 | Switching time mask for 2Tx-2Tx switching between two carriers and 1Tx-2Tx/2Tx-2Tx switching between two bands in Rel-17 | China Telecom | agreed | R4-2100791 | - |
| R4-2103237 | Way forward on NC UL CA PC2 evaluation assumptions and scenarios | Skyworks | revised | - | R4-2103371 |
| R4-2103238 | Way forward on NC UL CA PC2 requirements | ZTE | approved | - | - |
| R4-2103239 | NR support for high speed train scenario in frequency range 2 (FR2) | Nokia, Nokia Shanghai Bell, Samsung | approved | R4-2102266 | - |
| R4-2103240 | Way forward on Deployment Scenario and UE RF Requirement for FR2 HST | Samsung | approved | - | - |
| R4-2103241 | Way forward on coexistence evaluation for NR SL enhancement in Rel-17 | LGE | approved | - | - |
| R4-2103242 | Way forward on system parameters and operating CBW in n14 for NR SL enhancement | CATT, AT&T | approved | - | - |
| R4-2103243 | TR38.xxx v0.0.1 TR Skeleton for SL enhancement in Rel-17 | LG Electronics France | agreed | R4-2100282 | - |
| R4-2103244 | Way forward on TDM operation for SL and Uu in licensed band | CATT | approved | - | - |
| R4-2103245 | Way forward on FDM operation for SL and Uu in licensed band | Xiaomi | approved | - | - |
| R4-2103246 | Way forward on MPR/A-MPR simulation assumption for intra-band V2X con-current operation | LGE | approved | - | - |
| R4-2103247 | Way forward on synchronous operation between Uu and SL in licensed band | Ericsson | approved | - | - |
| R4-2103248 | LS on synchronous operation between Uu and SL in licensed band | CATT | withdrawn | - | - |
| R4-2103249 | Way forward on issues related to PC2 NR V2X | Huawei | approved | - | - |
| R4-2103250 | Way forward on simulation assumptions for PC2 NR V2X | Huawei | approved | - | - |
| R4-2103251 | Way forward on co-existence simulation assumptions for PC2 NR V2X | CATT | approved | - | - |
| R4-2103252 | (reserved) | Main session | not treated | - | - |
| R4-2103253 | Way forward on NR 52 | Qualcomm | approved | - | - |
| R4-2103254 | (reserved) | Main session | not treated | - | - |
| R4-2103255 | (reserved) | Main session | not treated | - | - |
| R4-2103256 | TP to TR 38.808: BS antenna array | Nokia | approved | - | - |
| R4-2103257 | TP to TR 38.808: BS architecture | Huawei | approved | - | - |
| R4-2103258 | Way forward on NR 52 | Intel | approved | - | - |
| R4-2103259 | TP to TR 38.808: Numerology and Channel Bandwidths | Ericsson | revised | R4-2101561 | R4-2103374 |
| R4-2103260 | TP to TR 38.808: Timing considerations for operation between 52.6 and 71 GHz | Nokia, Nokia Shanghai Bell | approved | R4-2102009 | - |
| R4-2103261 | TP to TR 38.808: Addition of a set of phase noise models in subclause 4.2.3 | Ericsson | approved | R4-2101181 | - |
| R4-2103262 | Updated TR 38.844 v0.0.2 | Ericsson | agreed | R4-2101555 | - |
| R4-2103263 | Way forward on use of larger CBW | Skyworks | approved | - | - |
| R4-2103264 | Way forward on overlapping CBW method | Nokia | revised | - | R4-2103387 |
| R4-2103265 | Blank TR for extended 600MHz NR band | Spark NZ Ltd | agreed | - | - |
| R4-2103266 | Way forward on regulatory requirements for protection of other services identified for Region 3 and coexistence with Band 28/n28 | ZTE | noted | - | - |
| R4-2103267 | Regulatory aspects for the 600MHz range in APT region | Huawei | approved | R4-2102572 | - |
| R4-2103268 | Way forward on bands plans for further study and duplex filter options | Nokia, Nokia Shanghai Bell | revised | - | R4-2103372 |
| R4-2103269 | [DRAFT] Reply LS on technical feasibilities for frequency arrangements for IMT in 470 | Huawei, CBN | revised | R4-2102575 | R4-2103364 |
| R4-2103270 | Work plan of study on extended 600MHz NR band | Spark NZ Ltd | approved | R4-2100055 | - |
| R4-2103271 | Way forward on BCS4 for NR | Ericsson | approved | - | - |
| R4-2103272 | Work plan for study on high power UE (power class 2) for one NR FDD band | China Unicom | approved | R4-2100081 | - |
| R4-2103273 | Way forward on PC2 for NR FDD band | China Unicom | approved | - | - |
| R4-2103274 | Way forward on simulation assumptions for system performance evaluation | China Unicom | approved | - | - |
| R4-2103275 | CR of adding LTE B24 for UE category NB1 in R17 | Ericsson, Ligado Networks | agreed | R4-2100604 | - |
| R4-2103276 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | agreed | R4-2100608 | - |
| R4-2103277 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | agreed | R4-2100609 | - |
| R4-2103278 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | agreed | R4-2100610 | - |
| R4-2103279 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | agreed | R4-2100612 | - |
| R4-2103280 | CR of adding LTE B24 for UE category NB2 in R17 | Ericsson, Ligado Networks | agreed | R4-2100614 | - |
| R4-2103281 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | agreed | R4-2100611 | - |
| R4-2103282 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | agreed | R4-2100613 | - |
| R4-2103283 | TR 37.880 V0.3.0: High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71 | Nokia, Nokia Shanghai Bell | agreed | - | - |
| R4-2103284 | TP to TR 37.880: Coexistence Simulation Results for High-power UE Vs NB-IoT guard band operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71 | Nokia, Nokia Shanghai Bell | approved | R4-2101800 | - |
| R4-2103285 | Way forward on Handling of agreements about band combinations | ZTE | approved | - | - |
| R4-2103286 | Way forward on Simplification on | Nokia, Nokia Shanghai Bell | approved | - | - |
| R4-2103287 | Way forward on frequency Bands for testing of A-GNSS Sensitivity requirements in NR and LTE | Apple | approved | - | - |
| R4-2103288 | Reply LS on PUCCH and PUSCH repetition | Qualcomm | revised | - | R4-2103393 |
| R4-2103289 | Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition | Huawei, HiSilicon | revised | - | R4-2103363 |
| R4-2103290 | Reply LS on Beam switching gaps for Multi-TRP UL transmission | RAN4 | approved | - | - |
| R4-2103291 | Email discussion summary for [98e][101] NR_NewRAT_SysParameters | Moderator (ZTE) | noted | R4-2102949 | - |
| R4-2103292 | Email discussion summary for [98e][102] NR_NewRAT_UE_RF_Part_1 | Moderator (Nokia) | noted | R4-2102950 | - |
| R4-2103293 | Email discussion summary for [98e][103] NR_NewRAT_UE_RF_Part_2 | Moderator (Apple) | noted | R4-2102951 | - |
| R4-2103294 | Email discussion summary for [98e][104] NR_NewRAT_UE_RF_Part_3 | Moderator (Huawei) | noted | R4-2102952 | - |
| R4-2103295 | Email discussion summary for [98e][105] LTE_Maintenance | Moderator (Skyworks) | noted | R4-2102953 | - |
| R4-2103296 | Email discussion summary for [98e][106] NR_unlic_Maintenance | Moderator (Qualcomm) | noted | R4-2102954 | - |
| R4-2103297 | Email discussion summary for [98e][107] 5G_V2X_NRSL_UE_RF | Moderator (LG Electronics) | noted | R4-2102955 | - |
| R4-2103298 | Email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF | Moderator (Ericsson) | noted | R4-2102956 | - |
| R4-2103299 | Email discussion summary for [98e][109] NR_RF_FR1_Part_1 | Moderator (Huawei) | noted | R4-2102957 | - |
| R4-2103300 | Email discussion summary for [98e][110] NR_RF_FR2_req_enh_Part_4 | Moderator (Nokia) | noted | R4-2102958 | - |
| R4-2103301 | Email discussion summary for [98e][111] NR_transient_period | Moderator (CMCC) | revised | R4-2102959 | R4-2103362 |
| R4-2103302 | Email discussion summary for [98e][112] NR_TxD | Moderator (vivo) | noted | R4-2102960 | - |
| R4-2103303 | Email discussion summary for [98e][113] NR_R16_Maintenance | Moderator (OPPO) | noted | R4-2102961 | - |
| R4-2103304 | Email discussion summary for [98e][114] R16_UE_ feature | Moderator (CMCC) | noted | R4-2102962 | - |
| R4-2103305 | Email discussion summary for [98e][117] NR_SAR_PC2_interB_SUL_2BUL | Moderator (China Telecom) | noted | R4-2102965 | - |
| R4-2103306 | Email discussion summary for [98e][118] NR_PC2_CA_R17_2BDL_2BUL | Moderator (China Telecom) | noted | R4-2102966 | - |
| R4-2103307 | Email discussion summary for [98e][119] ENDC_UE_PC2_R17_NR_TDD | Moderator (China Unicom) | noted | R4-2102967 | - |
| R4-2103308 | Email discussion summary for [98e][120] NR_bands_R17_BWs | Moderator (Ericsson) | noted | R4-2102968 | - |
| R4-2103309 | Email discussion summary for [98e][121] NR_FR1_35MHz_45MHz_BW | Moderator (Huawei) | noted | R4-2102969 | - |
| R4-2103310 | Email discussion summary for [98e][122] NR_LTE_V2X_PC5_combos | Moderator (CATT) | noted | R4-2102970 | - |
| R4-2103311 | Email discussion summary for [98e][123] NR_FR2_FWA_Bn257_Bn258 | Moderator (SoftBank) | noted | R4-2102971 | - |
| R4-2103312 | Email discussion summary for [98e][124] NR_47GHz_Band | Moderator (Nokia) | noted | R4-2102972 | - |
| R4-2103313 | Email discussion summary for [98e][125] NR_LTE_band_n24 | Moderator (Ligado Networks) | noted | R4-2102973 | - |
| R4-2103314 | Email discussion summary for [98e][126] NR_n67 | Moderator (Ericsson) | noted | R4-2102974 | - |
| R4-2103315 | Email discussion summary for [98e][127] NR_n85 | Moderator (Ericsson) | noted | R4-2102975 | - |
| R4-2103316 | Email discussion summary for [98e][128] NR_BCS4 | Moderator (Ericsson) | revised | R4-2102976 | R4-2103375 |
| R4-2103317 | Email discussion summary for [98e][129] NR_req_Intra_UL_CA | Moderator (Huawei) | noted | R4-2102977 | - |
| R4-2103318 | Email discussion summary for [98e][131] DL_intrpt_combos_TxSW_R17 | Moderator (China Telecom) | noted | R4-2102979 | - |
| R4-2103319 | Email discussion summary for [98e][132] HPUE_PC1_5_n77_n78 | Moderator (Qualcomm) | noted | R4-2102980 | - |
| R4-2103320 | Email discussion summary for [98e][133] NR_6GHz_unlic_EU | Moderator (Nokia) | noted | R4-2102981 | - |
| R4-2103321 | Email discussion summary for [98e][134] FS_6425_10500MHz_NR | Moderator (Ericsson) | noted | R4-2102982 | - |
| R4-2103322 | Email discussion summary for [98e][135] NR_RF_FR1_enh_Part_1 | Moderator (Huawei) | noted | R4-2102983 | - |
| R4-2103323 | Email discussion summary for [98e][136] NR_RF_FR1_enh_Part_2 | Moderator (China Telecom) | noted | R4-2102984 | - |
| R4-2103324 | Email discussion summary for [98e][137] NR_RF_FR1_enh_Part_3 | Moderator (Qualcomm) | noted | R4-2102985 | - |
| R4-2103325 | Email discussion summary for [98e][138] NR_RF_FR2_req_enh2_Part_1 | Moderator (Nokia) | noted | R4-2102986 | - |
| R4-2103326 | Email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2 | Moderator (Qualcomm) | noted | R4-2102987 | - |
| R4-2103327 | Email discussion summary for [98e][140] NR_RF_FR2_req_enh2_Part_3 | Moderator (Apple) | noted | R4-2102988 | - |
| R4-2103328 | Email discussion summary for [98e][141] NR_HST_FR2_enh | Moderator (Samsung) | noted | R4-2102989 | - |
| R4-2103329 | Email discussion summary for [98e][142] NRSL_enh_Part_1 | Moderator (LG Electronics) | noted | R4-2102990 | - |
| R4-2103330 | Email discussion summary for [98e][143] NRSL_enh_Part_2 | Moderator (CATT) | noted | R4-2102991 | - |
| R4-2103331 | Email discussion summary for [98e][144] NRSL_enh_Part_3 | Moderator (Huawei) | noted | R4-2102992 | - |
| R4-2103332 | Email discussion summary for [98e][145] FS_NR_52_to_71GHz_Part_1 | Moderator (Qualcomm) | noted | R4-2102993 | - |
| R4-2103333 | Email discussion summary for [98e][146] FS_NR_52_to_71GHz_Part_2 | Moderator (Intel) | noted | R4-2102994 | - |
| R4-2103334 | Email discussion summary for [98e][147] FS_NR_eff_BW_util | Moderator (Ericsson) | noted | R4-2102995 | - |
| R4-2103335 | Email discussion summary for [98e][148] FS_NR_600MHz_ext | Moderator (Ericsson) | noted | R4-2102996 | - |
| R4-2103336 | Email discussion summary for [98e][149] FS_NR_PC2_UE_FDD | Moderator (China Unicom) | noted | R4-2102997 | - |
| R4-2103337 | Email discussion summary for [98e][151] LTE_bands_R17_M1_M2_NB1_NB2 | Moderator (Ericsson) | noted | R4-2102999 | - |
| R4-2103338 | Email discussion summary for [98e][152] FS_LTE_NR_HPUE_FWVM | Moderator (Nokia) | noted | R4-2103000 | - |
| R4-2103339 | Email discussion summary for [98e][153] BC_simplification | Moderator (NTT DOCOMO) | noted | R4-2103001 | - |
| R4-2103340 | Email discussion summary for [98e][154] NR_reply_LS_Part_1 | Moderator (Xiaomi) | noted | R4-2103002 | - |
| R4-2103341 | Email discussion summary for [98e][155] NR_reply_LS_Part_2 | Moderator (China Telecom) | noted | R4-2103003 | - |
| R4-2103342 | Email discussion summary for [98e][156] NR_reply_LS_Part_3 | Moderator (Samsung) | noted | R4-2103004 | - |
| R4-2103343 | 38.101 Void clean up R16 | Nokia, Nokia Shanghai Bell | agreed | R4-2100136 | - |
| R4-2103344 | 38.101 Void clean up R17 | Nokia, Nokia Shanghai Bell | agreed | R4-2100137 | - |
| R4-2103345 | CR for bug fixing of band combination tables for 38101-3 Rel17 | Apple | agreed | R4-2102593 | - |
| R4-2103346 | CR for bug fixing of band combination tables for 38101-3 Rel16 | Apple | agreed | R4-2102594 | - |
| R4-2103347 | CR for 38.101-1 Rel16 corrections on exception requirements on out-of-band blocking for inter-band CA | Xiaomi | agreed | R4-2101106 | - |
| R4-2103348 | CR on FR2 intra-band UL CA | Huawei, HiSilicon | agreed | R4-2102716 | - |
| R4-2103349 | Way forward on BCS reporting and support for intra-band EN-DC band combinations | Bell Mobility | approved | - | - |
| R4-2103350 | Corrections to PCMAX for UL CA | Qualcomm Incorporated | agreed | - | - |
| R4-2103351 | CR to 38101-1 on introducing new SUL band n99 | Huawei,HiSilicon | agreed | R4-2103211 | - |
| R4-2103352 | CR to 38104 on introducing new SUL band n99 | Huawei,HiSilicon | agreed | R4-2103212 | - |
| R4-2103353 | Clarification on timing difference for Tx switching in EN-DC R16 | MediaTek inc. | agreed | R4-2101145 | - |
| R4-2103354 | CR to TS 38.141-2: Introduction of 35MHz and 45MHz | ZTE Corporation | endorsed | - | - |
| R4-2103355 | CR to 38.101-1: UL MIMO requirements update | Qualcomm Incorporated, Lenovo, Motorola Mobility | not pursued | R4-2102658 | - |
| R4-2103356 | TP to TR 38.808: UE aspects | Intel | approved | - | - |
| R4-2103357 | CR for 38.101-3 to introduce a new MSD due to the counter intermodulation interference(Rel-15) | Huawei, HiSilicon | agreed | R4-2102094 | - |
| R4-2103358 | P_cmax P_IBE wording refinement and terminology improvement | Qualcomm Incorporated | agreed | R4-2100589 | - |
| R4-2103359 | Way forward on introduction of new FR2 CA BW classes | Verizon | approved | R4-2103110 | - |
| R4-2103360 | LS on Signaling scheme of Transparent TxD | RAN4 | approved | R4-2103155 | - |
| R4-2103361 | Way forward on transient period | CMCC | approved | R4-2103151 | - |
| R4-2103362 | Email discussion summary for [98e][111] NR_transient_period | Moderator (CMCC) | noted | R4-2103301 | - |
| R4-2103363 | Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition | Huawei, HiSilicon | approved | R4-2103289 | - |
| R4-2103364 | [DRAFT] Reply LS on technical feasibilities for frequency arrangements for IMT in 470 | Huawei, CBN | revised | R4-2103269 | R4-2103406 |
| R4-2103365 | LS to RAN2 on single-uplink operation | RAN4 | noted | R4-2103145 | - |
| R4-2103366 | Way forward on new per BC indication of the per-FR gap | CMCC | approved | R4-2103168 | - |
| R4-2103367 | Updated RAN4 UE features list for Rel-16 | CMCC | approved | - | - |
| R4-2103368 | LS on updated Rel-16 RAN4 UE features lists for NR and LTE | RAN4 | approved | - | - |
| R4-2103369 | CR on FR2 inter-band DL CA CBM and IBM | Huawei, HiSilicon | revised | R4-2102718 | R4-2103405 |
| R4-2103370 | Removal of a remaining NS_201 related requirement | Nokia, Nokia Shanghai Bell | agreed | R4-2103125 | - |
| R4-2103371 | Way forward on NC UL CA PC2 evaluation assumptions and scenarios | Skyworks | approved | R4-2103237 | - |
| R4-2103372 | Way forward on bands plans for further study and duplex filter options | Nokia, Nokia Shanghai Bell | noted | R4-2103268 | - |
| R4-2103373 | Further Reply LS on power control for NR-DC | RAN4 | approved | - | - |
| R4-2103374 | TP to TR 38.808: Numerology and Channel Bandwidths | Ericsson | approved | R4-2103259 | - |
| R4-2103375 | Email discussion summary for [98e][128] NR_BCS4 | Moderator (Ericsson) | noted | R4-2103316 | - |
| R4-2103376 | Draft CR for 38.101-1: Introduction of BCS4 | T-Mobile USA, MediaTek | revised | R4-2102151 | R4-2103394 |
| R4-2103377 | DraftCR for 38.101-1 to add BCS1 for CA_n1A-n3A | Huawei, HiSilicon | endorsed | R4-2103045 | - |
| R4-2103378 | CR for TS 38.101-1 introduction of NR band n24 | Ligado Networks, Nokia, Skyworks Solutions Inc. | agreed | R4-2103210 | - |
| R4-2103379 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks, Nokia, Skyworks Solutions Inc. | agreed | R4-2100254 | - |
| R4-2103380 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed | R4-2102909 | - |
| R4-2103381 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed | R4-2102910 | - |
| R4-2103382 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed | R4-2102913 | - |
| R4-2103383 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed | R4-2102914 | - |
| R4-2103384 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed | R4-2102915 | - |
| R4-2103385 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed | R4-2102916 | - |
| R4-2103386 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | agreed | R4-2102918 | - |
| R4-2103387 | Way forward on overlapping CBW method | Nokia | approved | R4-2103264 | - |
| R4-2103388 | CR for TS 38.101-1: Correction to FR1 time mask for SRS antenna switching | Apple | agreed | R4-2103123 | - |
| R4-2103389 | CR for TS38 101-1 Rel-16 Correction of condition for MPR and delta MPR | CATT | agreed | R4-2103122 | - |
| R4-2103390 | Way forward on NR TxD | vivo | approved | R4-2103154 | - |
| R4-2103391 | CR on FR2 intra-band UL CA | Huawei, HiSilicon | agreed | - | - |
| R4-2103392 | Introduction of n262 UE RF requirements | Nokia, Nokia Shanghai Bell, Apple | endorsed | R4-2103205 | - |
| R4-2103393 | Reply LS on PUCCH and PUSCH repetition | RAN4 | approved | R4-2103288 | - |
| R4-2103394 | Draft CR for 38.101-1: Introduction of BCS4 | T-Mobile USA, MediaTek | not concluded | R4-2103376 | - |
| R4-2103395 | TS 38.101-3: Addition of missing lower order fallbacks R16 | Nokia, AT&T | agreed | - | - |
| R4-2103396 | Way forward on WF on remaining BS and UE parameters for 6.425-7.125 and 10.0-10.5 GHz | Ericsson, ZTE, Nokia, Nokia Shanghai Bell, CATT, Qualcomm | approved | R4-2103103 | - |
| R4-2103397 | TP to TR 38.921 UE transmitter requirements | ZTE Corporation, Qualcomm, Huawei, CATT, Nokia, Nokia Shanghai Bell | not concluded | R4-2103105 | - |
| R4-2103398 | TP to TR 38.921: BS remaining parameters | Huawei, HiSilicon, CATT, Nokia, ZTE | not concluded | R4-2103106 | - |
| R4-2103399 | Way forward on FR2 UEs that support inter-band DL CA with CBM | Samsung | approved | R4-2103112 | - |
| R4-2103400 | PC1 and PC3 Updates for Band n14 | AT&T | not concluded | R4-2100112 | R4-2103409 |
| R4-2103409 | PC1 and PC3 Updates for Band n14 | AT&T | agreed |  |  |
| R4-2103401 | Second Reply LS to RP-202935 = R4-2100025 on BCS reporting and support for intra-band EN-DC band combinations | RAN4 | approved | - | - |
| R4-2103402 | CR for 38.101-3: clarification of intra-band EN-DC BCS applicability | : T-Mobile USA | agreed | - | - |
| R4-2103403 | CR for 38.101-3: clarification of intra-band EN-DC BCS applicability | : T-Mobile USA | agreed | - | - |
| R4-2103404 | CR for 38.101-3: clarification of intra-band EN-DC BCS applicability | : T-Mobile USA | agreed | - | - |
| R4-2103405 | CR on FR2 inter-band DL CA CBM and IBM | Huawei, HiSilicon | agreed | R4-2103369 | - |
| R4-2103406 | Reply LS on technical feasibilities for frequency arrangements for IMT in 470 | RAN4 | approved | R4-2103364 | - |
| R4-2103407 | CR on introduction of completed NR CA/DC combs with 4DL/2UL including FR2 | Samsung | agreed | R4-2100991 | - |
| R4-2103408 | (reserved) | Main session | not treated | - | - |
| R4-2103409 | (reserved) | Main session | not treated | - | - |
| R4-2103411 | (reserved) | Main session | not treated | - | - |
| R4-2103412 | (reserved) | Main session | not treated | - | - |
| R4-2103413 | (reserved) | Main session | not treated | - | - |
| R4-2103414 | (reserved) | Main session | not treated | - | - |
| R4-2103415 | (reserved) | Main session | not treated | - | - |
| R4-2103416 | (reserved) | Main session | not treated | - | - |
| R4-2103417 | (reserved) | Main session | not treated | - | - |
| R4-2103418 | (reserved) | Main session | not treated | - | - |
| R4-2103419 | (reserved) | Main session | not treated | - | - |
| R4-2103420 | (reserved) | Main session | not treated | - | - |
| R4-2103421 | (reserved) | Main session | not treated | - | - |
| R4-2103422 | (reserved) | Main session | not treated | - | - |
| R4-2103423 | (reserved) | Main session | not treated | - | - |
| R4-2103424 | (reserved) | Main session | not treated | - | - |
| R4-2103425 | (reserved) | Main session | not treated | - | - |
| R4-2103426 | (reserved) | Main session | not treated | - | - |
| R4-2103427 | (reserved) | Main session | not treated | - | - |
| R4-2103428 | (reserved) | Main session | not treated | - | - |
| R4-2103429 | (reserved) | Main session | not treated | - | - |
| R4-2103430 | (reserved) | Main session | not treated | - | - |
| R4-2103431 | (reserved) | Main session | not treated | - | - |
| R4-2103432 | (reserved) | Main session | not treated | - | - |
| R4-2103433 | (reserved) | Main session | not treated | - | - |
| R4-2103434 | (reserved) | Main session | not treated | - | - |
| R4-2103435 | (reserved) | Main session | not treated | - | - |
| R4-2103436 | (reserved) | Main session | not treated | - | - |
| R4-2103437 | (reserved) | Main session | not treated | - | - |
| R4-2103438 | (reserved) | Main session | not treated | - | - |
| R4-2103439 | (reserved) | Main session | not treated | - | - |
| R4-2103440 | Email discussion summary: [98e][201] NR_NewRAT_RRM_Core | Moderator (Huawei) | revised | - | R4-2103683 |
| R4-2103441 | Email discussion summary: [98e][202] NR_NewRAT_RRM_Perf | Moderator (Ericsson) | revised | - | R4-2103684 |
| R4-2103442 | Email discussion summary: [98e][203] LTE_RRM_maintenance | Moderator (Ericsson) | revised | - | R4-2103685 |
| R4-2103443 | Email discussion summary: [98e][204] R16_NR_RRM_maintenance | Moderator (Apple) | revised | - | R4-2103686 |
| R4-2103444 | Email discussion summary: [98e][205] NR_unlic_RRM_1 | Moderator (Ericsson) | revised | - | R4-2103687 |
| R4-2103445 | Email discussion summary: [98e][206] NR_unlic_RRM_2 | Moderator (Nokia) | revised | - | R4-2103688 |
| R4-2103446 | Email discussion summary: [98e][207] NR_Mob_enh_RRM | Moderator (Apple) | revised | - | R4-2103689 |
| R4-2103447 | Email discussion summary: [98e][208] 5G_V2X_NRSL_RRM | Moderator (LG Electronics) | revised | - | R4-2103690 |
| R4-2103448 | Email discussion summary: [98e][209] NR_IAB_RRM | Moderator (ZTE) | revised | - | R4-2103691 |
| R4-2103449 | Email discussion summary: [98e][210] LTE_NR_DC_CA_RRM_1 | Moderator (Nokia) | revised | - | R4-2103692 |
| R4-2103450 | Email discussion summary: [98e][211] LTE_NR_DC_CA_RRM_2 | Moderator (Ericsson) | revised | - | R4-2103693 |
| R4-2103451 | Email discussion summary: [98e][212] NR_UE_pow_sav_RRM | Moderator (CATT) | revised | - | R4-2103694 |
| R4-2103452 | Email discussion summary: [98e][213] NR_pos_RRM_1 | Moderator (Huawei) | revised | - | R4-2103695 |
| R4-2103453 | Email discussion summary: [98e][214] NR_pos_RRM_2 | Moderator (Intel Corporation) | revised | - | R4-2103696 |
| R4-2103454 | Email discussion summary: [98e][215] NR_pos_RRM_3 | Moderator (Ericsson) | revised | - | R4-2103697 |
| R4-2103455 | Email discussion summary: [98e][216] NR_eMIMO_RRM | Moderator (Samsung) | revised | - | R4-2103698 |
| R4-2103456 | Email discussion summary: [98e][217] NR_RF_FR1_RRM | Moderator (Huawei) | revised | - | R4-2103699 |
| R4-2103457 | Email discussion summary: [98e][218] NR_RRM_Enh_RRM_1 | Moderator (Intel Corporation) | revised | - | R4-2103700 |
| R4-2103458 | Email discussion summary: [98e][219] NR_RRM_Enh_RRM_2 | Moderator (ZTE) | revised | - | R4-2103701 |
| R4-2103459 | Email discussion summary: [98e][220] NR_RRM_Enh_RRM_3 | Moderator (Apple) | revised | - | R4-2103702 |
| R4-2103460 | Email discussion summary: [98e][221] NR_CSIRS_L3meas_RRM_1 | Moderator (CATT) | revised | - | R4-2103703 |
| R4-2103461 | Email discussion summary: [98e][222] NR_CSIRS_L3meas_RRM_2 | Moderator (OPPO) | revised | - | R4-2103704 |
| R4-2103462 | Email discussion summary: [98e][223] NR_HST_RRM | Moderator (CMCC) | revised | - | R4-2103705 |
| R4-2103463 | Email discussion summary: [98e][224] NR_2step_RACH_RRM | Moderator (ZTE) | noted | - | - |
| R4-2103464 | Email discussion summary: [98e][225] LTE_eMTC5_RRM | Moderator (Ericsson) | revised | - | R4-2103706 |
| R4-2103465 | Email discussion summary: [98e][226] NB_IOTenh3_RRM | Moderator (Huawei) | revised | - | R4-2103707 |
| R4-2103466 | Email discussion summary: [98e][227] LTE_feMob_RRM | Moderator (Nokia) | revised | - | R4-2103708 |
| R4-2103467 | Email discussion summary: [98e][228] NR_FR2_FWA_Bn257_Bn258_RRM | Moderator (Huawei) | revised | - | R4-2103709 |
| R4-2103468 | Email discussion summary: [98e][229] NR_47GHz_Band_RRM | Moderator (Ericsson) | revised | - | R4-2103710 |
| R4-2103469 | Email discussion summary: [98e][230] NR_RF_FR2_req_enh2_RRM | Moderator (Nokia) | revised | - | R4-2103711 |
| R4-2103470 | Email discussion summary: [98e][231] NR_RRM_enh2_1 | Moderator (Apple) | revised | - | R4-2103712 |
| R4-2103471 | Email discussion summary: [98e][232] NR_RRM_enh2_2 | Moderator (CATT) | revised | - | R4-2103713 |
| R4-2103472 | Email discussion summary: [98e][233] NR_MG_enh_1 | Moderator (MediaTek) | revised | - | R4-2103714 |
| R4-2103473 | Email discussion summary: [98e][234] NR_MG_enh_2 | Moderator (Intel Corporation) | revised | - | R4-2103715 |
| R4-2103474 | Email discussion summary: [98e][235] NR_HST_FR1_enh_RRM | Moderator (CMCC) | revised | - | R4-2103716 |
| R4-2103475 | Email discussion summary: [98e][236] NR_HST_FR2_enh_RRM | Moderator (Nokia) | revised | - | R4-2103717 |
| R4-2103476 | Email discussion summary: [98e][237] NR_NTN_solutions_RRM | Moderator (Fraunhofer HHI) | revised | - | R4-2103718 |
| R4-2103477 | Email discussion summary: [98e][238] NR_UE_pow_sav_enh_RRM | Moderator (MediaTek) | revised | - | R4-2103719 |
| R4-2103478 | Email discussion summary: [98e][239] R17_LS_RS_for_Scell_activation | Moderator (Huawei) | revised | - | R4-2103720 |
| R4-2103479 | Updated RAN4 UE features list for Rel-16 | CMCC | approved | - | - |
| R4-2103480 | LS on updated Rel-16 RAN4 UE features lists for NR and LTE | RAN4 | approved | - | - |
| R4-2103481 | CR on R15 remaining issues | MediaTek inc. | agreed | R4-2101051 | - |
| R4-2103482 | CR on SCell activation delay, cell idenfication requirements on deactivated SCell and inter-RAT ECID requirements for NE-DC R15 | Huawei, HiSilicon | revised | R4-2102738 | R4-2104090 |
| R4-2103483 | Cat-F CR to SSB-less SCell activation delay requirement for deactivated FR1 SCell in Rel-15 | Qualcomm Incorporated | withdrawn | R4-2102872 | - |
| R4-2103484 | CR on the filter for beam failure indications in 38.133 | vivo | agreed | R4-2101464 | - |
| R4-2103485 | [CR] RRM test case maintenance R15 Cat F | ZTE Corporation | agreed | R4-2100058 | - |
| R4-2103486 | Update FR2 Reference channels and OCNG for FR2 RRM Test cases | ANRITSU LTD | agreed | R4-2100068 | - |
| R4-2103487 | Correction on the power of the first preamble for random access in EN-DC and SA in R15 | MediaTek inc. | agreed | R4-2101161 | - |
| R4-2103488 | Correction on the time for Scell activation and CSI-report in R15 | MediaTek inc. | revised | R4-2101164 | R4-2104082 |
| R4-2103489 | Correction on the Noc level in TS38.133 in R15 | MediaTek inc. | agreed | R4-2101167 | - |
| R4-2103490 | Correction on the SS-RSRP difference value for SS-RSRP measurement TC in R15 | MediaTek inc. | withdrawn | R4-2101170 | - |
| R4-2103491 | Correction to beam failure recovery test cases R15 | Huawei, HiSilicon | revised | R4-2101618 | R4-2104065 |
| R4-2103492 | Correction to L1-RSRP reporting delay test cases R15 | Huawei, HiSilicon | agreed | R4-2101621 | - |
| R4-2103493 | CR on maintaining Antenna configurations in TS38.133 R15 | Huawei, HiSilicon | revised | R4-2101661 | R4-2104056 |
| R4-2103494 | CR on test requirements for measurement performance tests R15 | Huawei, HiSilicon | agreed | R4-2101664 | - |
| R4-2103495 | Correction on test cases of inter-frequency Measurements R15 | Huawei, HiSilicon | agreed | R4-2101701 | - |
| R4-2103496 | CR to TS 38.133: Corrections to TC A.4.5.7.1 (Rel-15) | Rohde & Schwarz | agreed | R4-2102309 | - |
| R4-2103497 | CR on SCell activation TCs R15 | Huawei, HiSilicon | agreed | R4-2102741 | - |
| R4-2103498 | Cat-F CR to addition of TRS Configurations in Rel-15 Test Cases | Qualcomm Incorporated | revised | R4-2102875 | R4-2104086 |
| R4-2103499 | CR: Correction of eMTC RLM test cases (Rel-14) | Ericsson | agreed | R4-2101453 | - |
| R4-2103500 | CR: Correction of eMTC RLM test cases | Ericsson | agreed | R4-2101454 | - |
| R4-2103501 | Correction to applicability of E-UTRAN E-CID measurements requirements for NE-DC | Nokia, Nokia Shanghai Bell | agreed | R4-2102693 | - |
| R4-2103502 | Correction to requirements for NCSG patterns | Nokia, Nokia Shanghai Bell | agreed | R4-2102696 | - |
| R4-2103503 | CR on TC for eMTC RSTD measurement R15 | Huawei, HiSilicon | revised | R4-2102804 | R4-2104072 |
| R4-2103504 | CR on CRS muting for eMTC R15 | Huawei, HiSilicon | agreed | R4-2102807 | - |
| R4-2103505 | WF on MRTD/MTTD for intra-band non-contiguous CA and EN-DC | Apple | withdrawn | - | - |
| R4-2103506 | CR on IDLE/INACTIVE RRM requirement with SMTC2-LP R16 | Apple | agreed | R4-2100186 | - |
| R4-2103507 | Interruption requirements maintenance in NR-DC (R16) | Apple | agreed | R4-2100234 | - |
| R4-2103508 | CR for measurement period requirements correction | NEC | agreed | R4-2101075 | - |
| R4-2103509 | Maintenance CR on interruption at EUTRA SRS carrier switching in 38.133 | OPPO | agreed | R4-2101530 | - |
| R4-2103510 | Maintenance CR on SCell activation delay requirement in TS38.133 | OPPO | agreed | R4-2101531 | - |
| R4-2103511 | Cat-F CR to addition of TRS Configurations in Rel-16 Test Case | Qualcomm Incorporated | agreed | R4-2102889 | - |
| R4-2103512 | WF on NR-U RRM core | Ericsson | approved | - | - |
| R4-2103513 | CR: Beam management requirements with CCA | Ericsson | agreed | R4-2101428 | - |
| R4-2103514 | Terminology updates for NR-U in 38.133 | Ericsson | agreed | R4-2102519 | - |
| R4-2103515 | Terminology updates for NR-U in 36.133 | Ericsson | agreed | R4-2102521 | - |
| R4-2103516 | CR on Interruptions during Scell activation in NR-U | Qualcomm Incorporated | agreed | R4-2102922 | - |
| R4-2103517 | WF on LBT models for NR-U RRM performance requirements | Qualcomm | revised | - | R4-2104088 |
| R4-2103518 | WF on general test configurations for NR-U RRM performance requirements | Nokia, Nokia Shanghai Bell | approved | - | - |
| R4-2103519 | WF on NR-U RRM performance test case list | Ericsson | withdrawn | - | - |
| R4-2103520 | WF on test configurations for NR-U RRM performance requirements | Ericsson | approved | - | - |
| R4-2103521 | Draft CR: RMC for NR-U test cases | Ericsson | endorsed | R4-2101431 | - |
| R4-2103522 | CCA model in NR-U test cases | Ericsson | endorsed | R4-2102528 | - |
| R4-2103523 | Updated test case list for NR-U | Ericsson | approved | R4-2102524 | - |
| R4-2103524 | Draft CR: test cases for beam management in NR-U | Ericsson | revised | R4-2101433 | R4-2104081 |
| R4-2103525 | DraftCR Introduction of NR-U SCell interruption and SCell (de)activation tests | Ericsson | endorsed | R4-2102369 | - |
| R4-2103526 | DraftCR 38.133 NR-U Inter-RAT SFTD accuracy TC | Ericsson | endorsed | R4-2102372 | - |
| R4-2103527 | RLM test cases | Ericsson | endorsed | R4-2102530 | - |
| R4-2103528 | NR-U RRM, SFTD, RSSI, and CO measurements test cases | Ericsson | endorsed | R4-2102532 | - |
| R4-2103529 | UE timing tests for NR-U | Ericsson | revised | R4-2102650 | R4-2104079 |
| R4-2103530 | Test cases on BWP switching with consistent UL LBT failures | Ericsson | revised | R4-2102652 | R4-2104080 |
| R4-2103531 | Introduction of NR-U cell reselection tests | Ericsson | revised | R4-2102243 | R4-2104078 |
| R4-2103532 | Draft Big CR: Introduction of Rel-16 NR-U RRM performance requirements | Ericsson | endorsed | R4-2102523 | - |
| R4-2103533 | NR-U test cases structure | Ericsson | endorsed | R4-2102525 | - |
| R4-2103534 | WF on NR mobility enhancement maintenance | Apple | approved | - | - |
| R4-2103535 | CR on sync conditions for intra-band DAPS handover R16 | Huawei, HiSilicon | postponed | R4-2101668 | - |
| R4-2103536 | CR on V2X interruption | Mediatek Inc., LG Electronics, Qualcomm Inc. | agreed | R4-2101054 | - |
| R4-2103537 | WF on NR V2X RRM requirements | LG Electronics | approved | - | - |
| R4-2103538 | Big CR-Introduction of NR V2X RRM performance requirements (Rel-16) | LG Electronics | agreed | R4-2100638 | - |
| R4-2103539 | [CR] IAB Core Maintenance | ZTE Corporation | revised | R4-2100042 | R4-2104057 |
| R4-2103540 | WF on test cases for IAB-MTs | ZTE Corporation | revised | - | R4-2104069 |
| R4-2103541 | draftCR to introduce test configurations for IAB-MT RRM performance test | Huawei, HiSilicon | endorsed | R4-2101629 | - |
| R4-2103542 | [draft CR] Test cases for timing for IAB-MT | ZTE Corporation | endorsed | R4-2100046 | - |
| R4-2103543 | draftCR to introduce test cases for RRC release with redirection for IAB-MT | Huawei, HiSilicon | endorsed | R4-2101630 | - |
| R4-2103544 | draftCR on IAB RLM test cases | Nokia, Nokia Shanghai Bell | endorsed | R4-2102490 | - |
| R4-2103545 | Big CR: IAB-MT RRM test cases in 38.174 | Ericsson | endorsed | R4-2102637 | - |
| R4-2103546 | RRC re-establishment tests for LA IAB-MT | Ericsson | endorsed | R4-2102639 | - |
| R4-2103547 | WF on Test cases for MR-DC Idle mode CA measurements | Nokia, Nokia Shanghai Bell | approved | - | - |
| R4-2103548 | Draft Big CR: Introduction of Rel-16 MR-DC EMR RRM performance requirements (TS 38.133) | Nokia, Nokia Shanghai Bell | endorsed | R4-2102262 | - |
| R4-2103549 | Draft CR for Removal of brackets for idle mode CA measurement accuracy requirements | Nokia, Nokia Shanghai Bell | endorsed | R4-2102263 | - |
| R4-2103550 | Test case for EMR with both PCell and target cell in FR1 | Apple | endorsed | R4-2100232 | - |
| R4-2103551 | draftCR to introduce TC4 for EMR | Huawei, HiSilicon | endorsed | R4-2102754 | - |
| R4-2103552 | WF on replacement of measCycleSCell in core requirements for Direct SCell activation | Apple | approved | - | - |
| R4-2103553 | CR on activation time in direct SCell activation | Apple | revised | R4-2100228 | R4-2104071 |
| R4-2103554 | CR on SCell dormancy switching | Huawei, HiSilicon | agreed | R4-2102749 | - |
| R4-2103555 | Draft Big CR: Introduction of Rel-16 MR-DC EMR RRM performance requirements (TS 36.133) | Huawei, HiSilicon | withdrawn | - | - |
| R4-2103556 | Draft Big CR: Introduction of Rel-16 MR-DC Direct SCell activation and SCell dormancy RRM performance requirements (TS 38.133) | Ericsson | endorsed | - | - |
| R4-2103557 | Test case for Direct SCell Activation: EN-DC, NR spCell in FR1, SCell in FR1, SCell addition | Apple | endorsed | R4-2100230 | - |
| R4-2103558 | CR on TS38.133 for direct SCell activation of SCell in FR2 intra-band in ENDC mode (A.5.5.X) | MediaTek inc. | endorsed | R4-2101215 | - |
| R4-2103559 | DraftCR 38.133 TC3 Direct SCell activation | Ericsson | endorsed | R4-2102358 | - |
| R4-2103560 | draftCR to introduce TC4 for direct SCell activation | Huawei, HiSilicon | endorsed | R4-2102755 | - |
| R4-2103561 | Draft CR for NR FR1 Intra frequency handover with direct SCell activation | Nokia, Nokia Shanghai Bell | endorsed | R4-2102260 | - |
| R4-2103562 | Draft CR on TC for direct SCell activation during handover in NR SA for FR2 | NEC | endorsed | R4-2101073 | - |
| R4-2103563 | WF on test cases for SCell dormancy | Ericsson | approved | - | - |
| R4-2103564 | Test case for SCell Dormancy: EN-DC, NR spCell in FR1, SCell FR1, DCI 2_6 within/after 3 OFDM symbols | Apple | endorsed | R4-2100231 | - |
| R4-2103565 | CR on TS38.133 for E-UTRAN | MediaTek inc. | endorsed | R4-2101217 | - |
| R4-2103566 | draftCR to introduce TC3 for SCell dormancy | Huawei, HiSilicon | endorsed | R4-2102757 | - |
| R4-2103567 | DraftCR on EN-DC NR SpCell in FR1 and 2 NR SCells in FR2 for Dormant SCell switch via DCI 2_6 within and after 3 OFDM symbols | Qualcomm Incorporated | endorsed | R4-2102886 | - |
| R4-2103568 | Draft CR on TC for SCell dormancy in NR SA for FR1 | NEC | endorsed | R4-2101074 | - |
| R4-2103569 | DraftCR 38.133 TCs 6 and 8 SCell dormancy switching | Ericsson | endorsed | R4-2102360 | - |
| R4-2103570 | Draft CR for test case 7 for Dormant SCell BWP switch delay | Nokia, Nokia Shanghai Bell | endorsed | R4-2102259 | - |
| R4-2103571 | LS on RRM relaxation in power saving | CATT | withdrawn | - | - |
| R4-2103572 | Correction on inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion | Huawei, HiSilicon | agreed | R4-2101834 | - |
| R4-2103573 | WF on remaining issue for power saving RRM | CATT | approved | - | - |
| R4-2103574 | Correction to cell reselection test case for UE Power saving | CATT | agreed | R4-2100482 | - |
| R4-2103575 | Test case for cell reselection to FR2 intra-frequency NR case for UE configured with relaxed measurement | Huawei, HiSilicon | agreed | R4-2101835 | - |
| R4-2103576 | Changes to cell reselection tests under power saving | Ericsson | withdrawn | R4-2102246 | - |
| R4-2103577 | WF on UE PRS measurement requirements | Huawei, HiSilicon | revised | - | R4-2104076 |
| R4-2103578 | CR on PRS RSTD measurement requirements | CATT | agreed | R4-2100438 | - |
| R4-2103579 | PRS-RSRP measurement requirements | Ericsson | revised | R4-2102543 | R4-2104074 |
| R4-2103580 | CR to TS 38.133 on UE Rx-Tx time difference measurements (section 9.9.4) | OPPO | agreed | R4-2101527 | - |
| R4-2103581 | CR to TS 38.133 on UE Rx-Tx time difference measurements (section 9.9.4) | OPPO | agreed | - | - |
| R4-2103582 | CR to 38.133 correction on CCSF for NR measurements for positioning | vivo | revised | R4-2101785 | R4-2104075 |
| R4-2103583 | CR on CSSF and MG for PRS measurement 38.133 | Huawei, HiSilicon | postponed | R4-2102767 | - |
| R4-2103584 | CR to introduce new measurement gap patterns for positioning in 36.133 | Huawei, HiSilicon | agreed | R4-2102769 | - |
| R4-2103585 | Draft Big CR: Introduction of Rel-16 NR Positioning RRM performance requirements and test cases | Ericsson, Intel | endorsed | R4-2102549 | - |
| R4-2103586 | WF on NR Positioning UE performance requirements | Intel Corporation | approved | - | - |
| R4-2103587 | WF on gNB positioning measurement requirements | Ericsson | approved | - | - |
| R4-2103588 | WF on NR eMIMO RRM Performance requirements | Samsung | revised | - | R4-2104068 |
| R4-2103589 | CR to 38.133 on Pathloss activation delay requirements (R16) | Apple | agreed | R4-2100207 | - |
| R4-2103590 | CR to TS38.133 on L1-SINR measurement requirement | Samsung | agreed | R4-2100933 | - |
| R4-2103591 | CR on maintaining L1-SINR measurent requirements Rel-16 | Huawei, HiSilicon | agreed | R4-2101671 | - |
| R4-2103592 | CR to TS 38.133: Adding L1-SINR accuracy requirements | Nokia, Nokia Shanghai Bell | endorsed | R4-2102867 | - |
| R4-2103593 | Draft CR on L1-SINR measurement accuracy tests with CSI-RS based CMR and dedicated IMR | Samsung | endorsed | R4-2100936 | - |
| R4-2103594 | DraftCR on L1-SINR measurement accuracy tests with SSB CMR and dedicated IMR | Huawei, HiSilicon | endorsed | R4-2101677 | - |
| R4-2103595 | Introduction of test cases for BFD and link recovery procedure for Scell | MediaTek inc. | endorsed | R4-2100754 | - |
| R4-2103596 | Draft CR: Introduction of test case of link recovery with link recovery requests | Ericsson | endorsed | R4-2101447 | - |
| R4-2103597 | [draft CR] Test cases for applicable timing for PL RS activated by MAC-CE | ZTE Corporation | postponed | R4-2100054 | - |
| R4-2103598 | Draft test case CR on measurement performance of L1-SINR for CSI-RS-based CMR and no dedicated IMR | Qualcomm CDMA Technologies | endorsed | R4-2102912 | - |
| R4-2103599 | WF on test case for DL interruption due to Tx switching between two uplink carriers | Huawei, HiSilicon | approved | - | - |
| R4-2103600 | Update on interruption test cases for Tx switching R16 | MediaTek inc. | agreed | R4-2101148 | - |
| R4-2103601 | Correction on test cases of DL interruptions at switching between two uplink carriers | Huawei, HiSilicon | agreed | R4-2101710 | - |
| R4-2103602 | CR 38.133 (8.6.2A) Clarification on DCI-triggered BWP switch on multiple CCs | Ericsson | agreed | R4-2102355 | - |
| R4-2103603 | UL spatial relation switching to an unknown DL RS | Huawei, HiSilicon | agreed | R4-2101694 | - |
| R4-2103604 | CR for test cases for simultaneously DCI/timer based bwp switch over mulitple CCs on FR1 in SA | vivo | withdrawn | R4-2101390 | - |
| R4-2103605 | draftCR to introduce Active BWP Switch on multiple CCs TC2 | Huawei, HiSilicon | endorsed | R4-2101635 | - |
| R4-2103606 | DraftCR 38.133 (A.7.5.X) TC4 BWP switching | Ericsson | endorsed | R4-2102363 | - |
| R4-2103607 | Test case for RRC based UL spatial relation switch associated with a known DL-RS in SA for periodic SRS | Nokia, Nokia Shanghai Bell | endorsed | R4-2102265 | - |
| R4-2103608 | Big CR: Introduction of Rel-16 NR RRM enhancements WI performance requirements and test cases (Rel-16) | Intel Corporation, ZTE Corporation, Apple | agreed | R4-2101416 | - |
| R4-2103609 | WF on R16 RRM enhancement part 2 | ZTE Corporation | revised | - | R4-2104070 |
| R4-2103610 | LS on CGI reading with autonomous gaps | RAN4 | approved | - | - |
| R4-2103611 | CR to 38.133 correction on SRS carrier based switching core requirements | vivo, Qualcomm, Huawei, HiSilicon, MediaTek Inc., Apple, Nokia | postponed | R4-2101762 | - |
| R4-2103612 | SRS carrier switching interruption CR | Qualcomm, Inc. | withdrawn | R4-2102686 | - |
| R4-2103613 | CR on CGI reading requirements 38.133 | Huawei, HiSilicon, MediaTek | agreed | R4-2102791 | - |
| R4-2103614 | TC4: E-UTRAN | Apple | endorsed | R4-2100226 | - |
| R4-2103615 | CR to 38.133 on SRS configuration | vivo | endorsed | R4-2101765 | - |
| R4-2103616 | TC2 - SA interruptions at NR SRS carrier based switching | Ericsson | revised | R4-2102533 | R4-2104085 |
| R4-2103617 | draftCR to CGI reading TC4 | Huawei, HiSilicon | endorsed | R4-2102795 | - |
| R4-2103618 | Mandatory MG applicability rule CR | Qualcomm, Inc. | postponed | R4-2100626 | - |
| R4-2103619 | WF on R16 RRM enhancement part 1 | Intel Corporation | approved | - | - |
| R4-2103620 | WF on Rel-16 RRM enhancement part 3 | Apple | approved | - | - |
| R4-2103621 | CR on multiple SCell activation requirements | Huawei, HiSilicon | agreed | R4-2102789 | - |
| R4-2103622 | CR on UE behavior for UE specific CBW change R16 | Apple | agreed | R4-2100183 | - |
| R4-2103623 | Correction on scheduling availability and measurement restriction on FR2 inter-band CA in R16 | MediaTek inc. | agreed | R4-2100778 | - |
| R4-2103624 | CR on TS38.133 for inter-frequency measurement requirement without gap | MediaTek inc. | agreed | R4-2101219 | - |
| R4-2103625 | Clarification on inter-frequency measurement without gap requirements | Huawei, HiSilicon | withdrawn | R4-2101691 | - |
| R4-2103626 | draftCR on multiple SCell activation test case | MediaTek inc. | endorsed | R4-2101059 | - |
| R4-2103627 | draftCR on the CBW change test case for adding the UL CBW configuration | MediaTek inc. | endorsed | R4-2101070 | - |
| R4-2103628 | DraftCR on SCell activation and deactication delay test for FR2 inter-band CA | Huawei, HiSilicon | endorsed | R4-2101679 | - |
| R4-2103629 | WF on core part maintenance of CSI-RS based L3 measurement requirements | Apple | approved | - | - |
| R4-2103630 | WF on accuracy requirements and test cases of CSI-RS based L3 measurement | CATT, OPPO | revised | - | R4-2104089 |
| R4-2103631 | CR on CSI-RS based L3 measurement | CATT | agreed | R4-2100422 | - |
| R4-2103632 | CR on core requirement for CSI-RS L3 measurement | Xiaomi | withdrawn | R4-2100718 | - |
| R4-2103633 | Maintenance CR for CSI-RS based L3 measurement requirements R16 | MediaTek inc. | agreed | R4-2101150 | - |
| R4-2103634 | 38.133 CR on the CSI-RS based measurement requirements | Nokia, Nokia Shanghai Bell | revised | R4-2101394 | R4-2104093 |
| R4-2103635 | CR on CSI-RS measurement window and intra-frequency measurements | Huawei, HiSilicon | postponed | R4-2101842 | - |
| R4-2103636 | CR on performance requirement for CSI-RSRQ | CATT | endorsed | R4-2100430 | - |
| R4-2103637 | 38.133 draftCR on the CSI-RSRP accuracy requirements | Nokia, Nokia Shanghai Bell | endorsed | R4-2101396 | - |
| R4-2103638 | draftCR on CSI-SINR accuracy requirements | Huawei, HiSilicon | endorsed | R4-2102801 | - |
| R4-2103639 | CR on test case for CSI-RS based intra-frequency measurement for FR1 in SA | CATT | endorsed | R4-2100433 | - |
| R4-2103640 | RRM test cases for CSI-RS L3 intra-frequency measurements | Xiaomi | endorsed | R4-2100722 | - |
| R4-2103641 | Draft test case of CSI-RS based intra-frequency test for EN-DC event triggered reporting tests without gap for NR neighbor cell in FR2 | Qualcomm CDMA Technologies | endorsed | R4-2102820 | - |
| R4-2103642 | CR on test case for CSI-RS based inter-frequency measurement for FR1 in SA | CATT | endorsed | R4-2100434 | - |
| R4-2103643 | RRM test cases for CSI-RS L3 inter-frequency measurements | Xiaomi | endorsed | R4-2100723 | - |
| R4-2103644 | CR on EN-DC tests for NR inter-frequency neighbor cell in FR2(PScell in FR2) | OPPO | endorsed | R4-2101534 | - |
| R4-2103645 | CR on test case for CSI-RSRP measurement accuracy requirements in SA | CATT | endorsed | R4-2100435 | - |
| R4-2103646 | RRM test cases for CSI-RS L3 measurement performance requirements | Xiaomi | endorsed | R4-2100724 | - |
| R4-2103647 | CR to Update timing offset in test case for CSI-SINR in SA FR2 | MediaTek inc. | withdrawn | R4-2101154 | - |
| R4-2103648 | CR on EN-DC tests for CSI-RSRQ accuracy | OPPO | endorsed | R4-2101535 | - |
| R4-2103649 | CR to 38.133 correction to test cases for EN-DC CSI-SINR measurement accuracy | vivo | endorsed | R4-2101775 | - |
| R4-2103650 | draft CR to update TC3 and TC12 for CSI-RS accuracy test | Huawei, HiSilicon | endorsed | R4-2102803 | - |
| R4-2103651 | Draft test case of measurement performance for EN-DC CSI-RSRP measurement accuracy for NR neighbor cell in FR2 | Qualcomm CDMA Technologies | endorsed | R4-2102825 | - |
| R4-2103652 | Big CR: Introduction of Rel-16 CSI-RS based L3 measurement RRM test cases | CATT | withdrawn | - | - |
| R4-2103653 | Big CR: Introduction of Rel-16 CSI-RS based L3 measurement RRM performance requirements | CATT | withdrawn | - | - |
| R4-2103654 | CR on HST RRM requirements in connected mode | CMCC | agreed | R4-2100849 | - |
| R4-2103655 | CR on HST core part maintenance | Apple | agreed | R4-2100238 | - |
| R4-2103656 | Correction on test cases for inter-RAT cell identification in connected mode for HST | Huawei, HiSilicon | endorsed | R4-2101846 | - |
| R4-2103657 | LS on timing of RSS in neighbor cell for Rel-16 eMTC | RAN4 | approved | - | - |
| R4-2103658 | Correction Rel-16 eMTC RRM performance requirements | Ericsson | agreed | R4-2101457 | - |
| R4-2103659 | CR on eMTC RRM requirements | Huawei, HiSilicon | agreed | R4-2102797 | - |
| R4-2103660 | CR on maintenance for measurement on non-anchor carrier for NB-IoT | Huawei, HiSilicon | agreed | R4-2101654 | - |
| R4-2103661 | Clarification on asynchronous DAPS handover | Huawei, HiSilicon | agreed | R4-2101697 | - |
| R4-2103662 | Test cases for inter-frequency DAPS handover | Huawei, HiSilicon | agreed | R4-2101699 | - |
| R4-2103663 | Addition of missing async FDD-TDD and TDD-FDD LTE interband interfrquency DAPS handover tests | Ericsson | agreed | R4-2102632 | - |
| R4-2103664 | WF on FR2 new FWA UE RRM performance requirements | Huawei | approved | - | - |
| R4-2103665 | CR on condition requirements for UE power class 5 in TS38.133 | Huawei, HiSilicon | agreed | R4-2101684 | - |
| R4-2103666 | RRM core requirements for band n62 | Ericsson | agreed | R4-2102654 | - |
| R4-2103667 | WF on temporary RS for efficient SCell activation in NR CA | Huawei, HiSilicon | revised | - | R4-2104083 |
| R4-2103668 | Reply LS on temporary RS for efficient SCell activation in NR CA | Huawei | revised | - | R4-2104067 |
| R4-2103669 | Work plan of Rel-17 Power Saving Enhancements | MediaTek inc. | approved | R4-2101221 | - |
| R4-2103670 | WF on R17 UE power saving RLM/BM relaxation | MediaTek | approved | - | - |
| R4-2103671 | WF on RRM requirements for FR2 Inter-band DL CA and UL CA | Nokia | revised | - | R4-2104095 |
| R4-2103672 | WF on further RRM enhancement for NR and MR-DC - SRS antenna port switching | Apple | approved | - | - |
| R4-2103673 | WF on further RRM enhancement for NR and MR-DC | Apple | approved | - | - |
| R4-2103674 | LS on handover with PSCell | RAN4 | approved | - | - |
| R4-2103675 | WF on further RRM enhancement for NR and MR-DC - PUCCH SCell activation/deactivation requirements | CATT | approved | - | - |
| R4-2103676 | WF on R17 NR MG enhancements - Multiple concurrent and independent MG patterns | MediaTek | revised | - | R4-2104096 |
| R4-2103677 | WF on R17 NR MG enhancements | Intel Corporation | approved | - | - |
| R4-2103678 | WF on FR1 HST RRM requirements | CMCC | approved | - | - |
| R4-2103679 | WF on Rel-17 NR HST FR2 RRM Requirements | Nokia, Nokia Shanghai Bell | approved | - | - |
| R4-2103680 | WF on NTN RRM requirements | Fraunhofer | approved | - | - |
| R4-2103681 | WF on NTN RRM timing related requirements | Xiaomi | approved | - | - |
| R4-2103682 | WF on NTN RRM measurement requirements | Qualcomm | approved | - | - |
| R4-2103683 | Email discussion summary: [98e][201] NR_NewRAT_RRM_Core | Moderator (Huawei) | noted | R4-2103440 | - |
| R4-2103684 | Email discussion summary: [98e][202] NR_NewRAT_RRM_Perf | Moderator (Ericsson) | noted | R4-2103441 | - |
| R4-2103685 | Email discussion summary: [98e][203] LTE_RRM_maintenance | Moderator (Ericsson) | noted | R4-2103442 | - |
| R4-2103686 | Email discussion summary: [98e][204] R16_NR_RRM_maintenance | Moderator (Apple) | noted | R4-2103443 | - |
| R4-2103687 | Email discussion summary: [98e][205] NR_unlic_RRM_1 | Moderator (Ericsson) | noted | R4-2103444 | - |
| R4-2103688 | Email discussion summary: [98e][206] NR_unlic_RRM_2 | Moderator (Nokia) | noted | R4-2103445 | - |
| R4-2103689 | Email discussion summary: [98e][207] NR_Mob_enh_RRM | Moderator (Apple) | noted | R4-2103446 | - |
| R4-2103690 | Email discussion summary: [98e][208] 5G_V2X_NRSL_RRM | Moderator (LG Electronics) | noted | R4-2103447 | - |
| R4-2103691 | Email discussion summary: [98e][209] NR_IAB_RRM | Moderator (ZTE) | noted | R4-2103448 | - |
| R4-2103692 | Email discussion summary: [98e][210] LTE_NR_DC_CA_RRM_1 | Moderator (Nokia) | noted | R4-2103449 | - |
| R4-2103693 | Email discussion summary: [98e][211] LTE_NR_DC_CA_RRM_2 | Moderator (Ericsson) | noted | R4-2103450 | - |
| R4-2103694 | Email discussion summary: [98e][212] NR_UE_pow_sav_RRM | Moderator (CATT) | noted | R4-2103451 | - |
| R4-2103695 | Email discussion summary: [98e][213] NR_pos_RRM_1 | Moderator (Huawei) | noted | R4-2103452 | - |
| R4-2103696 | Email discussion summary: [98e][214] NR_pos_RRM_2 | Moderator (Intel Corporation) | noted | R4-2103453 | - |
| R4-2103697 | Email discussion summary: [98e][215] NR_pos_RRM_3 | Moderator (Ericsson) | noted | R4-2101279 | - |
| R4-2103698 | Email discussion summary: [98e][216] NR_eMIMO_RRM | Moderator (Samsung) | noted | R4-2103455 | - |
| R4-2103699 | Email discussion summary: [98e][217] NR_RF_FR1_RRM | Moderator (Huawei) | noted | R4-2103456 | - |
| R4-2103700 | Email discussion summary: [98e][218] NR_RRM_Enh_RRM_1 | Moderator (Intel Corporation) | noted | R4-2103457 | - |
| R4-2103701 | Email discussion summary: [98e][219] NR_RRM_Enh_RRM_2 | Moderator (ZTE) | noted | R4-2103458 | - |
| R4-2103702 | Email discussion summary: [98e][220] NR_RRM_Enh_RRM_3 | Moderator (Apple) | noted | R4-2103459 | - |
| R4-2103703 | Email discussion summary: [98e][221] NR_CSIRS_L3meas_RRM_1 | Moderator (CATT) | noted | R4-2103460 | - |
| R4-2103704 | Email discussion summary: [98e][222] NR_CSIRS_L3meas_RRM_2 | Moderator (OPPO) | noted | R4-2103461 | - |
| R4-2103705 | Email discussion summary: [98e][223] NR_HST_RRM | Moderator (CMCC) | noted | R4-2103462 | - |
| R4-2103706 | Email discussion summary: [98e][225] LTE_eMTC5_RRM | Moderator (Ericsson) | noted | R4-2103464 | - |
| R4-2103707 | Email discussion summary: [98e][226] NB_IOTenh3_RRM | Moderator (Huawei) | noted | R4-2103465 | - |
| R4-2103708 | Email discussion summary: [98e][227] LTE_feMob_RRM | Moderator (Nokia) | noted | R4-2103466 | - |
| R4-2103709 | Email discussion summary: [98e][228] NR_FR2_FWA_Bn257_Bn258_RRM | Moderator (Huawei) | noted | R4-2103467 | - |
| R4-2103710 | Email discussion summary: [98e][229] NR_47GHz_Band_RRM | Moderator (Ericsson) | noted | R4-2103468 | - |
| R4-2103711 | Email discussion summary: [98e][230] NR_RF_FR2_req_enh2_RRM | Moderator (Nokia) | noted | R4-2103469 | - |
| R4-2103712 | Email discussion summary: [98e][231] NR_RRM_enh2_1 | Moderator (Apple) | noted | R4-2103470 | - |
| R4-2103713 | Email discussion summary: [98e][232] NR_RRM_enh2_2 | Moderator (CATT) | noted | R4-2103471 | - |
| R4-2103714 | Email discussion summary: [98e][233] NR_MG_enh_1 | Moderator (MediaTek) | noted | R4-2103472 | - |
| R4-2103715 | Email discussion summary: [98e][234] NR_MG_enh_2 | Moderator (Intel Corporation) | noted | R4-2103473 | - |
| R4-2103716 | Email discussion summary: [98e][235] NR_HST_FR1_enh_RRM | Moderator (CMCC) | noted | R4-2103474 | - |
| R4-2103717 | Email discussion summary: [98e][236] NR_HST_FR2_enh_RRM | Moderator (Nokia) | noted | R4-2103475 | - |
| R4-2103718 | Email discussion summary: [98e][237] NR_NTN_solutions_RRM | Moderator (Fraunhofer HHI) | noted | R4-2103476 | - |
| R4-2103719 | Email discussion summary: [98e][238] NR_UE_pow_sav_enh_RRM | Moderator (MediaTek) | noted | R4-2103477 | - |
| R4-2103720 | Email discussion summary: [98e][239] R17_LS_RS_for_Scell_activation | Moderator (Huawei) | noted | R4-2103478 | - |
| R4-2103721 | CR: Introduction of random access requirements with CCA | Ericsson | agreed | R4-2101425 | - |
| R4-2103722 | Maintenance CR on interruption at EUTRA SRS carrier switching in 38.133 | OPPO | agreed | - | - |
| R4-2103723 | Maintenance CR on SCell activation delay requirement in TS38.133 | OPPO | agreed | - | - |
| R4-2103724 | Updated evaluation assumptions for R17 RLM/BFD relaxation | vivo, MediaTek | revised | R4-2101461 | R4-2104066 |
| R4-2103725 | Simulation assumptions for CSI-RS based L3 measurement | CATT | approved | - | - |
| R4-2103726 | L1-SINR simulation results summary | Samsung | noted | - | - |
| R4-2103727 | CR on EMR requirements in 36.133 | Huawei, HiSilicon | agreed | R4-2102747 | - |
| R4-2103728 | LS on RSS based RSRQ for LTE-MTC | RAN4 | approved | - | - |
| R4-2103729 | CR on CSSF for EN-DC R15 | Apple | postponed | R4-2100173 | - |
| R4-2103730 | draftCR to introduce accuracy requirements for RSTD measurement | Huawei, HiSilicon | endorsed | R4-2102772 | - |
| R4-2103731 | UE Rx-Tx measurement accuracy requirements | Ericsson | endorsed | R4-2102553 | - |
| R4-2103732 | CR on PRS-RSRP accuracy requirements | CATT | endorsed | R4-2100446 | - |
| R4-2103733 | [draft CR] Test cases for PRS-RSRP measurement accuracy | ZTE Corporation | postponed | R4-2100061 | - |
| R4-2103734 | CR on test case for PRS-RSRP measurement requirements for FR2 in SA | CATT | postponed | R4-2100448 | - |
| R4-2103735 | [draftCR] CR for PRS configurations for NR Pos RRM tests | Intel Corporation | postponed | R4-2101278 | - |
| R4-2103736 | [draftCR] CR for the test case of RSTD measurement requirements reporting in SA | Intel Corporation | postponed | R4-2101279 | - |
| R4-2103737 | TC5 and TC6: UE Rx-Tx time difference measurement requirements for FR1 and FR2 in SA | Ericsson | postponed | R4-2102555 | - |
| R4-2103738 | TC11 and TC12: UE Rx-Tx time difference measurement accuracy for FR1 and FR2 in SA | Ericsson | postponed | R4-2102556 | - |
| R4-2103739 | draftCR to introduce TC for PRS-RSRP measurement requirements for FR1 in SA | Huawei, HiSilicon | postponed | R4-2102777 | - |
| R4-2103740 | Email discussion summary for [98e][301] BSRF_Maintenance | Moderator (Ericsson) | revised | - | R4-2103923 |
| R4-2103741 | Email discussion summary for [98e][302] NR_Conformance_Maintenance | Moderator (Huawei) | revised | - | R4-2103924 |
| R4-2103742 | Email discussion summary for [98e] [303] NR_EMC | Moderator (ZTE) | revised | - | R4-2103922 |
| R4-2103743 | Email discussion summary for [98e][304] NR_unlic_BSRF_Conformance | Moderator (ZTE) | revised | - | R4-2103926 |
| R4-2103744 | Email discussion summary for [98e][305] NR_IAB_RF_Maintenance | Moderator (CATT) | revised | - | R4-2103929 |
| R4-2103745 | Email discussion summary for [98e][306] NR_IAB_Conformance_Part1 | Moderator (Nokia) | revised | - | R4-2103930 |
| R4-2103746 | Email discussion summary for [98e][307] NR_IAB_Conformance_Part2 | Moderator (Samsung) | revised | - | R4-2103931 |
| R4-2103747 | Email discussion summary for [98e][308] OTA_BS_Testing_Maintenance | Moderator (Huawei) | revised | - | R4-2103943 |
| R4-2103748 | Email discussion summary for [98e][309] NR_47GHz_Band_BSRF | Moderator (Nokia) | revised | - | R4-2103944 |
| R4-2103749 | Email discussion summary for [98e][310] NTN_Solutions_Part1 | Moderator (Thales) | revised | - | R4-2103948 |
| R4-2103750 | Email discussion summary for [98e][311] NTN_Solutions_Part2 | Moderator (Samsung) | revised | - | R4-2103949 |
| R4-2103751 | Email discussion summary for [98e][312] NR_Repeater_General | Moderator (Qualcomm) | revised | - | R4-2103950 |
| R4-2103752 | Email discussion summary for [98e][313] NR_Repeater_RF | Moderator (CMCC) | revised | - | R4-2103951 |
| R4-2103753 | Email discussion summary for [98e][314] Demod_R15_Maintenance | Moderator (Intel) | revised | - | R4-2103925 |
| R4-2103754 | Email discussion summary for [98e][315] Demod_R16_Maintenance | Moderator (ZTE) | revised | - | R4-2103941 |
| R4-2103755 | Email discussion summary for [98e][316] NR_unlic_Demod_UE | Moderator (Qualcomm) | revised | - | R4-2103934 |
| R4-2103756 | Email discussion summary for [98e][317] NR_unlic_Demod_BS | Moderator (Huawei) | revised | - | R4-2103934 |
| R4-2103757 | Email discussion summary for [98e][318] V2X_Demod_Part1 | Moderator (LGE) | revised | - | R4-2103927 |
| R4-2103758 | Email discussion summary for [98e][319] V2X_Demod_Part2 | Moderator (Intel) | revised | - | R4-2103928 |
| R4-2103759 | Email discussion summary for [98e][320] NR_IAB_Demod | Moderator (Nokia) | revised | - | R4-2103935 |
| R4-2103760 | Email discussion summary for [98e][321] NR_UE_pow_sav_Demod | Moderator (CATT) | revised | - | R4-2103936 |
| R4-2103761 | Email discussion summary for [98e][322] NR_URLLC_Demod_Part1 | Moderator (Ericsson) | revised | - | R4-2103937 |
| R4-2103762 | Email discussion summary for [98e][323] NR_URLLC_Demod_Part2 | Moderator (Huawei) | revised | - | R4-2103938 |
| R4-2103763 | Email discussion summary for [98e][324] NR_eMIMO_Demod | Moderator (Samsung) | revised | - | R4-2103939 |
| R4-2103764 | Email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod | Moderator (China Telecomm) | revised | - | R4-2103940 |
| R4-2103765 | Email discussion summary for [98e][326] NR_perf_enh_Demod | Moderator (China Telecomm) | revised | - | R4-2103942 |
| R4-2103766 | Email discussion summary for [98e][327] NR_R17_SpectrumWI_Demod | Moderator (Ericsson) | revised | - | R4-2103945 |
| R4-2103767 | Email discussion summary for [98e][328] NR_HST_FR1_Demod | Moderator (CMCC) | revised | - | R4-2103947 |
| R4-2103768 | Email discussion summary for [98e][329] NR_MIMO_OTA | Moderator (CAICT) | revised | - | R4-2103946 |
| R4-2103769 | Email discussion summary for [98e][330] FR2_enhTestMethods | Moderator (Apple) | revised | - | R4-2103952 |
| R4-2103770 | WF on single carrier NR FR2 UE EMC requirement | Xiaomi | approved | - | - |
| R4-2103771 | CR for TS 38.124: Correction of FR1 radiated spurious emissions (R15) | Huawei, HiSilicon, Bureau Veritas,Samsung | agreed | R4-2102398 | - |
| R4-2103772 | CR to TS 38.113: Radiated emission test method | ZTE Corporation | agreed | R4-2102178 | - |
| R4-2103773 | WF on IAB EMC RI testing with spatial exclusions | Huawei | approved | - | - |
| R4-2103774 | CR to TS 38.175 on IAB EMC test configuratins and performance requirements | Ericsson, ZTE | not pursued | R4-2100360 | - |
| R4-2103775 | CR to TS 36.104: Additions of regional requirements for band 41 in Japan, Rel-15 | NEC, SoftBank, KDDI, Nokia | agreed | R4-2101082 | - |
| R4-2103776 | CR to TS 36.141: Additions of regional requirements for band 41 in Japan, Rel-15 | NEC, SoftBank, KDDI, Nokia | agreed | R4-2101085 | - |
| R4-2103777 | CR to TS 38.104: Additions of regional requirements for n41 in Japan, Rel-15 | NEC, SoftBank, KDDI, Nokia | agreed | R4-2101088 | - |
| R4-2103778 | CR to TS 38.141-1: Additions of regional requirements for n41 in Japan, Rel-15 | NEC, SoftBank, KDDI, Nokia | agreed | R4-2101091 | - |
| R4-2103779 | CR to TS 38.141-1: Additions of regional requirements for n41 and n90 in Japan, Rel-16 | NEC, SoftBank, KDDI, Nokia | agreed | R4-2101092 | - |
| R4-2103780 | CR to TS 38.141-2: Additions of regional requirements for n41 in Japan, Rel-15 | NEC, SoftBank, KDDI, Nokia | agreed | R4-2101094 | - |
| R4-2103781 | CR to TS 38.141-2: Additions of regional requirements for n41 and n90 in Japan, Rel-16 | NEC, SoftBank, KDDI, Nokia | agreed | R4-2101095 | - |
| R4-2103782 | CR to TS 37.105: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16 | Huawei, China Unicom | agreed | R4-2102563 | - |
| R4-2103783 | CR to TS 37.145-1: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16 | Huawei, China Unicom | agreed | R4-2102565 | - |
| R4-2103784 | CR to TS 37.145-2: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16 | Huawei, China Unicom | agreed | R4-2102567 | - |
| R4-2103785 | CR to 37.105 on NR+UTRA support for AAS | Ericsson | agreed | R4-2102844 | - |
| R4-2103786 | CR to TR 38.820: Correction of antenna model in subclause 7.2.4 | Ericsson | agreed | R4-2101180 | - |
| R4-2103787 | WF on adjacent channel co-location | Huawei | approved | - | - |
| R4-2103788 | CR to TS 37.145-2 - Update CLTA definition, Rel-15 | Huawei, CATT, Nokia | agreed | R4-2102431 | - |
| R4-2103789 | CR to TS 38.141-1 clarification on PN23 sequence generation | Nokia, Nokia Shanghai Bell | agreed | R4-2101569 | - |
| R4-2103790 | CR to TS 38.141-2 clarification on PN23 sequence generation | Nokia, Nokia Shanghai Bell | agreed | R4-2101571 | - |
| R4-2103791 | CR to TS 37.145-1, Corrections to conformance requirements, Rel-15 | Huawei | withdrawn | R4-2102423 | - |
| R4-2103792 | CR to TS 37.145-2: Corrections to conformance requirements, Rel-15 | Huawei | agreed | R4-2102426 | - |
| R4-2103793 | CR to TS 38.141-1: Receiver requirement corrections | Nokia, Nokia Shanghai Bell | agreed | R4-2102000 | - |
| R4-2103794 | CR for TS 38.141-2: Correction on definition for the out-of-band CLTA(Rel-15) | CATT, Nokia, Nokia Shanghai Bell, Huawei | agreed | R4-2100386 | - |
| R4-2103795 | TS 38.141-2: Correction of additional spurious emission limits for bands 50, 51, 75, 76 | Ericsson | withdrawn | R4-2101730 | - |
| R4-2103796 | Correction of CQI test parameters and FRC for UE demodulation test | Ericsson | agreed | R4-2101441 | - |
| R4-2103797 | CR on corrections for LTE-NR Co-existence tests and OCNG pattern | Qualcomm Incorporated | agreed | R4-2102824 | - |
| R4-2103798 | CR on FRC for NR RI requirements (Rel-15) | Intel Corporation | agreed | R4-2101945 | - |
| R4-2103799 | CR to 38.101-4 on update to CSI reporting test parameters for Aperiodic reporting (R15) | Apple | agreed | R4-2102870 | - |
| R4-2103800 | CR for 38.141-1: BS demodulation synchronization in test setup | Nokia, Nokia Shanghai Bell | agreed | R4-2100548 | - |
| R4-2103801 | CR for 38.141-2: BS demodulation different channel bandwidths applicability rules | Nokia, Nokia Shanghai Bell | agreed | R4-2100554 | - |
| R4-2103802 | CR for 38.141-2: BS demodulation synchronization in test setup | Nokia, Nokia Shanghai Bell | agreed | - | - |
| R4-2103803 | CR for 38.141-2: BS demodulation synchronization in test setup | Nokia, Nokia Shanghai Bell | agreed | - | - |
| R4-2103804 | CR for 38.141-2: BS demodulation synchronization in test setup | Nokia, Nokia Shanghai Bell | agreed | - | - |
| R4-2103805 | Way Forward on NR-U UE demodulation requirements | QUALCOMM | revised | - | R4-2103987 |
| R4-2103806 | Way forward on NR-U BS demodulation requirements for general part and PUSCH | Huawei, HiSilicon | revised | - | R4-2103988 |
| R4-2103807 | Way Forward on NR-U PUCCH demodulation requirements | Ericsson | revised | - | R4-2103989 |
| R4-2103808 | Way forward on NR-U PRACH demodulation requirements | Nokia, Nokia Shanghai Bell | approved | - | - |
| R4-2103809 | Summary of simulation results for NR-U | Huawei, HiSilicon | noted | - | - |
| R4-2103810 | WF on single link tests for NR V2X demodulation performance | LGE | revised | - | R4-2103990 |
| R4-2103811 | Summary of simulation results for V2X single link demodulation | LGE | noted | - | - |
| R4-2103812 | Updated simulation assumption for NR V2X single link test cases | Huawei | revised | - | R4-2103991 |
| R4-2103813 | Draft CR on General section of NR V2X requirements | Intel Corporation | endorsed | R4-2101942 | - |
| R4-2103814 | DraftCR for 38.101-4, Introduce PSBCH performance requirements for NR V2X | CATT, GOHIGH | endorsed | R4-2100411 | - |
| R4-2103815 | Draft CR for PSSCH demodulation requirements for NR V2X | LG Electronics Inc. | revised | R4-2100656 | R4-2103995 |
| R4-2103816 | Draft CR on NR V2X Single Link PSCCH requirements | Intel Corporation | endorsed | R4-2101234 | - |
| R4-2103817 | draftCR on NR V2X PSFCH demodulation requirements | MediaTek inc. | endorsed | R4-2101069 | - |
| R4-2103818 | Draft CR: Introduction on Power imbalance requirements for NR V2X | Huawei, HiSilicon | endorsed | R4-2101357 | - |
| R4-2103819 | WF on multiple link tests for NR V2X demodulation performance | Intel | approved | - | - |
| R4-2103820 | Simulation assumptions for NR V2X multiple link test case | Huawei | revised | - | R4-2103993 |
| R4-2103821 | WF on remaining issue for UE power saving | CATT | approved | - | - |
| R4-2103822 | CR for TS38.101-4, test for FR2 PDCCH DCI format 2_6 demodulation | CATT | agreed | R4-2100398 | - |
| R4-2103823 | CR for TS38.101-4, test for FR1 TDD PDCCH DCI format 2_6 demodulation | CMCC | agreed | R4-2100818 | - |
| R4-2103824 | CR on Fixed reference channel for power saving performance | vivo | agreed | R4-2101387 | - |
| R4-2103825 | WF on NR eMIMO demodulation and CSI requirements | Samsung | approved | - | - |
| R4-2103826 | CR to 38.101-4 for eMIMO demod requirements - General and Applicability rule | Apple | agreed | R4-2100210 | - |
| R4-2103827 | CR to TS 38.101-4: Performance requirements single-DCI based multi-TRP Repetition Tx schemes | Intel Corporation, Samsung, Ericsson, Huawei, HiSilicon | agreed | R4-2101259 | - |
| R4-2103828 | CR for 38.101-4 Introduction of PDSCH requirement with Single-DCI based SDM scheme | Huawei, HiSilicon | agreed | R4-2101315 | - |
| R4-2103829 | CR for 38.101-4 Introduction of PDSCH requirement with Multi-DCI based multi-TRP transmission schemes | Huawei, HiSilicon | revised | R4-2101316 | R4-2103986 |
| R4-2103830 | CR: FRC for eMIMO sDCI/mDCI-based PDSCH transmission | Ericsson, Huawei, HiSilicon, Intel, Samsung | agreed | R4-2101448 | - |
| R4-2103831 | Simulation results for URLLC schemes | Samsung | noted | R4-2100900 | - |
| R4-2103832 | Simulation results for single and multi-DCI based transmission scheme | Qualcomm | noted | - | - |
| R4-2103833 | Introduction of PMI test cases with Rel-16 eType II codebook | Samsung | agreed | R4-2102938 | - |
| R4-2103834 | CR on demodulation performance requirements for DL 256QAM for FR2 | ZTE Corporation | agreed | R4-2101116 | - |
| R4-2103835 | CR on simplified TDL-D channel model for FR2 DL 256QAM demodulation requirements | Intel Corporation | agreed | R4-2101252 | - |
| R4-2103836 | CR on SDR requirements for DL 256QAM for FR2 | Huawei, HiSilicon | agreed | R4-2101299 | - |
| R4-2103837 | CR on adding applicability, requirements and measurement channel for FR2 DL 256QAM CQI reporting test under fading condition | China Telecom | agreed | R4-2100884 | - |
| R4-2103838 | CR on applicability rules and FRC for FR2 DL 256QAM CQI requirements | Intel Corporation | agreed | R4-2101253 | - |
| R4-2103839 | Draft CR for TS 38.307 on UE demodulation performance requirements (Rel-15) | China Telecom | agreed | R4-2100787 | - |
| R4-2103840 | Draft CR for TS 38.307 on UE demodulation performance requirements (Rel-16) | China Telecom | agreed | R4-2100788 | - |
| R4-2103841 | CR on applicability rules for Normal NR CA requirements | Intel Corporation | agreed | R4-2101255 | - |
| R4-2103842 | CR for 38.101-4 Applicablity of PMI reporting test with Tx ports larger than 8 and up to 32 | Huawei, HiSilicon | agreed | R4-2101322 | - |
| R4-2103843 | Introduction of PMI test cases with Rel-15 Type II codebook | Samsung | agreed | R4-2102939 | - |
| R4-2103844 | Discussion and simulation results for Rel-15 Type II codebook test cases | Samsung | noted | R4-2100902 | - |
| R4-2103845 | CR: Update on test applicability rule for EN-DC power imbalance | Huawei, HiSilicon | agreed | - | - |
| R4-2103846 | Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports | Ericsson | noted | - | - |
| R4-2103847 | Summary of simulation results of Rel-15 Type-II UE PMI test | Ericsson | noted | - | - |
| R4-2103848 | draft CR to TR 37.941: Relative calibration approach | ROHDE & SCHWARZ | postponed | R4-2102493 | - |
| R4-2103849 | WF on IAB-MT EVM measurement in core spec | CATT | approved | - | - |
| R4-2103850 | Draft CR for TS 38.174: Correction of clause 6,7 and 9 | CATT | endorsed | R4-2100369 | - |
| R4-2103851 | CR on Transmitter characteristics- Others TS 38.174 | Ericsson | endorsed | R4-2102338 | - |
| R4-2103852 | CR on Rx Charateristic other related requirements | Ericsson | endorsed | R4-2102340 | - |
| R4-2103853 | WF on IAB-MT test environment | Ericsson | approved | - | - |
| R4-2103854 | WF on test configurations, models and Rx FRC | Nokia, Nokia Shanghai Bell | approved | - | - |
| R4-2103855 | WF on manufacturer declarations | Samsung | approved | - | - |
| R4-2103856 | WF on IAB conformance specification work split and drafting guidelines | Nokia | approved | - | - |
| R4-2103857 | WF on IAB-MT test coverage and IAB node test burden reduction | Nokia | approved | - | - |
| R4-2103858 | CR on HST-SFN requirements for TDD | CMCC | withdrawn | R4-2100848 | - |
| R4-2103859 | CR on release independent for Rel.16 NR HST UE demodulation requirements | CMCC | agreed | R4-2100855 | - |
| R4-2103860 | CR for TS 38.141-2: Introduction of NR PUSCH UL TA performance requirement | CATT | withdrawn | R4-2100381 | - |
| R4-2103861 | CR for 38.104: HST PUSCH demodulation requirements | Nokia, Nokia Shanghai Bell | agreed | R4-2100558 | - |
| R4-2103862 | CR on UL timing adjustment conducted performance requirement for TS 38.141-1 | Samsung | agreed | R4-2100925 | - |
| R4-2103863 | remove SNR brackets for HST PUSCH in TS38.141-2 | Ericsson | agreed | R4-2100993 | - |
| R4-2103864 | CR for TS 38.141-1 Updates of NR PUSCH performance requirements for HST | NTT DOCOMO, INC. | agreed | R4-2101042 | - |
| R4-2103865 | CR for 38.104 Cleanup of performance requirements for NR HST PRACH under fading channel (Rel-16) | Huawei, HiSilicon | agreed | R4-2101319 | - |
| R4-2103866 | CR for 38.141-1 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-16) | Huawei, HiSilicon | agreed | R4-2101320 | - |
| R4-2103867 | CR for 38.141-2 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-16) | Huawei, HiSilicon | agreed | R4-2101321 | - |
| R4-2103868 | CR to TS 38.104 Update on UL timing adjustment performance requirements | ZTE Wistron Telecom AB | agreed | R4-2101849 | - |
| R4-2103869 | Corrections on 2-step RACH demodulation requirements | Nokia, Nokia Shanghai Bell, Ericsson | agreed | R4-2100582 | - |
| R4-2103870 | CR on MsgA PUSCH radiated performance requirement for TS 38.141-2 | Samsung | agreed | R4-2100931 | - |
| R4-2103871 | CR on update applicability rule for 2-step RACH in 38.141-1 (Rel-16) | Huawei, HiSilicon | agreed | R4-2101304 | - |
| R4-2103872 | CR on update applicability rule for 2-step RACH in 38.141-2 (Rel-16) | Huawei, HiSilicon | agreed | R4-2101306 | - |
| R4-2103873 | Updated simulation results collection on BS demodulation requirements for 2-step RACH | ZTE | noted | - | - |
| R4-2103874 | TP to TR 38.847: BS conformance aspects | Nokia, Nokia Shanghai Bell | approved | R4-2102445 | - |
| R4-2103875 | Way forward on UE/BS demodulation on NR 47GHz band | Ericsson | approved | - | - |
| R4-2103876 | WF on FR1 HST demodulation | CMCC | approved | - | - |
| R4-2103877 | WF for NTN general part | Thales | agreed | - | - |
| R4-2103878 | WF for NTN co-existence study | Samsung | approved | - | - |
| R4-2103879 | WF for NR repeater EMC requirements | ZTE | noted | - | - |
| R4-2103880 | Work plan for NR Repeaters | Qualcomm Incorporated | approved | R4-2102834 | - |
| R4-2103881 | WF for TDD Repeater | CMCC | revised | - | R4-2103999 |
| R4-2103882 | WF for NR repeater RF requirements | CMCC | approved | - | - |
| R4-2103883 | CR to 37.104 on OBUE table headings and applicability | Ericsson | agreed | R4-2102847 | - |
| R4-2103884 | CR to 37.141 on OBUE table headings and applicability | Ericsson | agreed | R4-2102850 | - |
| R4-2103885 | CR to 37.105 on OBUE table headings and applicability | Ericsson | agreed | R4-2102853 | - |
| R4-2103886 | CR to 37.145-1 on OBUE table headings and applicability | Ericsson | agreed | R4-2102856 | - |
| R4-2103887 | CR to 37.145-2 on OBUE table headings and applicability | Ericsson | agreed | R4-2102859 | - |
| R4-2103888 | CR to TS 38.104: corrections of NR-U BS Tx requirements | ZTE Corporation | agreed | R4-2101978 | - |
| R4-2103889 | CR to TS 38.104 corrections to NR-U BS RF Tx requirements | Nokia, Nokia Shanghai Bell | agreed | R4-2102835 | - |
| R4-2103890 | CR to TS 38.101-4: Performance requirements for URLLC PDSCH 0.001% BLER | Ericsson | agreed | R4-2102120 | - |
| R4-2103891 | CR to 38-101-4 on CQI reporting requirements for URLLC | Apple | agreed | R4-2100199 | - |
| R4-2103892 | CR to TS38.101-4 Applicability rules for URLLC CSI requirements | Huawei, HiSilicon | agreed | R4-2101338 | - |
| R4-2103893 | CR on FRC for Ultra low BLER UE CQI requirements | Intel Corporation | agreed | R4-2101944 | - |
| R4-2103894 | CR for 38.104: Ultra high reliability BS demodulation requirements | Nokia, Nokia Shanghai Bell | agreed | R4-2100563 | - |
| R4-2103895 | CR to TS 38.141-1 Update of 0.001% BLER test requirements | Ericsson | agreed | R4-2102122 | - |
| R4-2103896 | CR to TS 38.141-2 Update of 0.001% BLER test requirements | Ericsson | agreed | R4-2102124 | - |
| R4-2103897 | LS on Test Methodology for UE URLLC Ultra Low BLER CQI requirements | RAN4 | approved | - | - |
| R4-2103898 | Summary of simulation results for 0.001% BLER UE requirement | Ericsson | noted | - | - |
| R4-2103899 | Summary of simulation results for 0.001% BLER BS requirement | Ericsson | noted | - | - |
| R4-2103900 | CR to 38.101-4 on requirements with slot aggreagation in FR2 | Apple | agreed | R4-2100201 | - |
| R4-2103901 | CR on FRC for URLLC UE Higher BLER requirements | Intel Corporation | agreed | R4-2101245 | - |
| R4-2103902 | CR to TS 38.101-4 Correction of UE performance requirements for FR1 URLLC PDSCH repetitions over multiple slots. | Huawei, HiSilicon | agreed | R4-2101337 | - |
| R4-2103903 | CR to TS 38.101-4: Performance requirements for URLLC High BLER feature tests | Ericsson | agreed | R4-2102121 | - |
| R4-2103904 | CR on FR1 PDSCH Mapping Type B and Processing Capability 2 Requirements | Qualcomm Incorporated | agreed | R4-2102822 | - |
| R4-2103905 | CR for 38.104: Low latency FR1 BS demodulation requirements | Nokia, Nokia Shanghai Bell | agreed | R4-2100561 | - |
| R4-2103906 | CR on PUSCH repetition type A and PUSCH mapping type B radiated performance requirement for TS 38.104 | Samsung | agreed | R4-2100927 | - |
| R4-2103907 | CR on FRC for URLLC BS radiated performance requirement for TS 38.141-2 | Samsung | agreed | R4-2100929 | - |
| R4-2103908 | CR for TS 38.141-2 Updates of performance requirements of PUSCH repetition type A and PUSCH mapping type B for URLLC | NTT DOCOMO, INC. | agreed | R4-2101044 | - |
| R4-2103909 | CR on FR2 requirements for PUSCH mapping Type B with low number of symbols (Rel-16) | Intel Corporation | agreed | R4-2101248 | - |
| R4-2103910 | CR to TS38.141-2 Correction of BS conformance testing for FR2 URLLC PUSCH repetition Type A (Rel-16) | Huawei, HiSilicon | agreed | R4-2101335 | - |
| R4-2103911 | CR to TS38.141-1 Correction of BS conformance testing for URLLC demodulation requirements with higher BLER (Rel-16) | Huawei, HiSilicon | agreed | R4-2101339 | - |
| R4-2103912 | CR to TS38.104 Correction of BS performance requirements for URLLC FR1 PUSCH repetition Type A (Rel-16) | Huawei, HiSilicon | agreed | R4-2101341 | - |
| R4-2103913 | WF on NR MIMO OTA | vivo, CAICT | approved | - | - |
| R4-2103914 | WF on TP work split and Work Plan | vivo, CAICT, OPPO | approved | - | - |
| R4-2103915 | WF on FR2 MIMO OTA simulation assumptions | Huawei, HiSilicon | approved | - | - |
| R4-2103916 | CR to TR38.827 on the direction of the BS strongest beams | Huawei, HiSilicon | agreed | - | - |
| R4-2103917 | On Testability for band n262 | Keysight Technologies UK Ltd | noted | R4-2102619 | - |
| R4-2103918 | WF on high DL power and low UL power test cases (objective1) and band n262 testability (objective7) | Apple | approved | - | - |
| R4-2103919 | WF on polarization basis mismatch (objective2) | MediaTek | approved | - | - |
| R4-2103920 | WF on ETC (objective4) and test time reduction(objective6) | vivo | approved | - | - |
| R4-2103921 | WF on Rel-16 NR IAB demodulation requirements | Nokia | revised | - | R4-2103994 |
| R4-2103922 | Email discussion summary for [98e][303] NR_EMC | Moderator (ZTE) | noted | R4-2103742 | - |
| R4-2103923 | Email discussion summary for [98e][301] BSRF_Maintenance | Moderator (Ericsson) | noted | R4-2103740 | - |
| R4-2103924 | Email discussion summary for [98e][302] NR_Conformance_Maintenance | Moderator (Huawei) | noted | R4-2103741 | - |
| R4-2103925 | Email discussion summary for [98e][314] Demod_R15_Maintenance | Moderator (Intel) | noted | R4-2103753 | - |
| R4-2103926 | Email discussion summary for [98e][304] NR_unlic_BSRF_Conformance | Moderator (ZTE) | noted | R4-2103743 | - |
| R4-2103927 | Email discussion summary for [98e][318] V2X_Demod_Part1 | Moderator (LGE) | noted | R4-2103757 | - |
| R4-2103928 | Email discussion summary for [98e][319] V2X_Demod_Part2 | Moderator (Intel) | noted | R4-2103758 | - |
| R4-2103929 | Email discussion summary for [98e][305] NR_IAB_RF_Maintenance | Moderator (CATT) | noted | R4-2103744 | - |
| R4-2103930 | Email discussion summary for [98e][306] NR_IAB_Conformance_Part1 | Moderator (Nokia) | noted | R4-2103745 | - |
| R4-2103931 | Email discussion summary for [98e][307] NR_IAB_Conformance_Part2 | Moderator (Samsung) | noted | R4-2103746 | - |
| R4-2103932 | CR for TS38.101-4, test for FR1 FDD PDCCH DCI format 2_6 demodulation | MediaTek inc. | agreed | - | - |
| R4-2103933 | Email discussion summary for [98e][317] NR_unlic_Demod_BS | Moderator (Huawei) | noted | R4-2103756 | - |
| R4-2103934 | Email discussion summary for [98e][316] NR_unlic_Demod_UE | Moderator (Qualcomm) | noted | R4-2103755 | - |
| R4-2103935 | Email discussion summary for [98e][320] NR_IAB_Demod | Moderator (Nokia) | noted | R4-2103759 | - |
| R4-2103936 | Email discussion summary for [98e][321] NR_UE_pow_sav_Demod | Moderator (CATT) | noted | R4-2103760 | - |
| R4-2103937 | Email discussion summary for [98e][322] NR_URLLC_Demod_Part1 | Moderator (Ericsson) | noted | R4-2103761 | - |
| R4-2103938 | Email discussion summary for [98e][323] NR_URLLC_Demod_Part2 | Moderator (Huawei) | noted | R4-2103762 | - |
| R4-2103939 | Email discussion summary for [98e][324] NR_eMIMO_Demod | Moderator (Samsung) | noted | R4-2103763 | - |
| R4-2103940 | Email discussion summary for [98e][325] NR_DL256QAM_FR2_Demod | Moderator (China Telecomm) | noted | R4-2103764 | - |
| R4-2103941 | Email discussion summary for [98e][315] Demod_R16_Maintenance | Moderator (ZTE) | noted | R4-2103754 | - |
| R4-2103942 | Email discussion summary for [98e][326] NR_perf_enh_Demod | Moderator (China Telecomm) | noted | R4-2103765 | - |
| R4-2103943 | Email discussion summary for [98e][308] OTA_BS_Testing_Maintenance | Moderator (Huawei) | noted | R4-2103747 | - |
| R4-2103944 | Email discussion summary for [98e][309] NR_47GHz_Band_BSRF | Moderator (Nokia) | noted | R4-2103748 | - |
| R4-2103945 | Email discussion summary for [98e][327] NR_R17_SpectrumWI_Demod | Moderator (Ericsson) | noted | R4-2103766 | - |
| R4-2103946 | Email discussion summary for [98e][329] NR_MIMO_OTA | Moderator (CAICT) | noted | R4-2103768 | - |
| R4-2103947 | Email discussion summary for [98e][328] NR_HST_FR1_Demod | Moderator (CMCC) | noted | R4-2103767 | - |
| R4-2103948 | Email discussion summary for [98e][310] NTN_Solutions_Part1 | Moderator (Thales) | noted | R4-2103749 | - |
| R4-2103949 | Email discussion summary for [98e][311] NTN_Solutions_Part2 | Moderator (Samsung) | noted | R4-2103750 | - |
| R4-2103950 | Email discussion summary for [98e][312] NR_Repeater_General | Moderator (Qualcomm) | noted | R4-2103751 | - |
| R4-2103951 | Email discussion summary for [98e][313] NR_Repeater_RF | Moderator (CMCC) | noted | R4-2103752 | - |
| R4-2103952 | Email discussion summary for [98e][330] FR2_enhTestMethods | Moderator (Apple) | noted | R4-2103769 | - |
| R4-2103953 | remove SNR brackets for HST PUSCH in TS38.141-2 | Ericsson | agreed | R4-2100994 | - |
| R4-2103954 | CR for 38.101-4 Applicablity of PMI reporting test of eType II codebook | Huawei, HiSilicon | agreed | R4-2101323 | - |
| R4-2103955 | WF for URLLC UE with higher BLER | Intel | approved | - | - |
| R4-2103956 | WF for URLLC BS with higher BLER | Huawei | approved | - | - |
| R4-2103957 | Draft CR for TS 38.174: Correction of clause 5 | CATT | endorsed | R4-2100368 | - |
| R4-2103958 | CR to TS 38.104: Additions of regional requirements for n41 and n90 in Japan, Rel-16 | NEC, SoftBank, KDDI, Nokia | agreed | R4-2101089 | - |
| R4-2103959 | CR on correction 2-step RACH performance requirements for FR2 in 38.104 (Rel-16) | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell | agreed | R4-2101302 | - |
| R4-2103960 | Simulation results of PDSCH requirements for Single-DCI SDM scheme | Huawei, HiSilicon | noted | R4-2101313 | - |
| R4-2103961 | PMI reporting requirements with eType II codebook | Apple | noted | R4-2100212 | - |
| R4-2103962 | Views on CSI Reporting test cases for eMIMO | Qualcomm Incorporated | noted | R4-2100622 | - |
| R4-2103963 | Simulation results for eType II codebook based PMI reporting test | Huawei, HiSilicon | noted | R4-2101311 | - |
| R4-2103964 | CR to TS 38.104 Update on UL timing adjustment performance requirements | Moderator (China Telecomm) | revised | - | - |
| R4-2103965 | Simulation assumption for NTN co-existence study | CATT | revised | - | R4-2103998 |
| R4-2103966 | TP to TR38.884 on High DL and Low UL power test cases | Keysight, Rohde & Schwarz | approved | - | - |
| R4-2103967 | TP to TR38.884 v0.1.0 on polarization basis mismatch | vivo | approved | R4-2101830 | - |
| R4-2103968 | TP to TR 38.884 on Inter-band DL CA in FR2 | Anritsu corporation | approved | R4-2100097 | - |
| R4-2103969 | TP to TS 38.151 v0.1.0 on FR2 Channel model and RMC | vivo, CAICT | approved | R4-2101823 | - |
| R4-2103970 | TP to TS 38.151 v0.1.0 on FR2 test system for requirements | vivo, CAICT | approved | R4-2101824 | - |
| R4-2103971 | TP to TS 38.151 v0.1.0 on Performance metrics for NR MIMO OTA requirements | vivo, CAICT | approved | R4-2101825 | - |
| R4-2103972 | Summary of ideal and impairment results for NR HST demodulation requirements | CATT | noted | R4-2100380 | - |
| R4-2103973 | CR to TS 36.104: corrections of NR-U BS RF requirements | ZTE Corporation | agreed | R4-2101972 | - |
| R4-2103974 | CR to TS 37.104: corrections of NR-U BS RF requirements | ZTE Corporation | agreed | R4-2101974 | - |
| R4-2103975 | CR to TS 38.104: corrections of NR-U BS Rx requirements | ZTE Corporation | agreed | R4-2101980 | - |
| R4-2103976 | Draft Big CR: Introduction of Rel-16 NR V2X demodulation performance requirements | LGE | endorsed | - | - |
| R4-2103977 | WF on Testing aspect on IAB-MT dynamic range, power control and frequency error | Samsung | approved | - | - |
| R4-2103978 | CR on FDD HST Single-Tap and Multipath Fading Requirements | Qualcomm Incorporated | agreed | R4-2100168 | - |
| R4-2103979 | Simulation assumptions for demodulation performance requirements for IAB-MT | Huawei | noted | - | - |
| R4-2103980 | V2X demod HARQ buffer CR | Qualcomm, Inc. | endorsed | R4-2100630 | - |
| R4-2103981 | Draft CR: Introduction on PSFCH decoding capability requirements for NR V2X | Huawei, HiSilicon | endorsed | R4-2101360 | - |
| R4-2103982 | Draft CR: Introduction on PSSCH/PSCCH decoding capability requirements for NR V2X | Huawei, HiSilicon | endorsed | R4-2101362 | - |
| R4-2103983 | Big CR to TR 38.174 - correction to clause 6 | Qualcomm | agreed | - | - |
| R4-2103984 | (reserved) | BSRF session | not treated | - | - |
| R4-2103985 | Draft TR38.884 Study on enhanced test methods for FR2 NR UEs v0.2.0 | Apple, vivo | agreed | - | - |
| R4-2103986 | CR for 38.101-4 Introduction of PDSCH requirement with Multi-DCI based multi-TRP transmission schemes | Huawei, HiSilicon | agreed | R4-2103829 | - |
| R4-2103987 | Way Forward on NR-U UE demodulation requirements | QUALCOMM | approved | R4-2103805 | - |
| R4-2103988 | Way forward on NR-U BS demodulation requirements for general part and PUSCH | Huawei, HiSilicon | approved | R4-2103806 | - |
| R4-2103989 | Way Forward on NR-U PUCCH demodulation requirements | Ericsson | approved | R4-2103807 | - |
| R4-2103990 | WF on single link tests for NR V2X demodulation performance | LGE | approved | R4-2103810 | - |
| R4-2103991 | Updated simulation assumption for NR V2X single link test cases | Huawei | approved | R4-2103812 | - |
| R4-2103992 | (reserved) | BSRF session | not treated | - | - |
| R4-2103993 | Simulation assumptions for NR V2X multiple link test case | Huawei | approved | R4-2103820 | - |
| R4-2103994 | WF on Rel-16 NR IAB demodulation requirements | Nokia | approved | R4-2103921 | - |
| R4-2103995 | Draft CR for PSSCH demodulation requirements for NR V2X | LG Electronics Inc. | endorsed | R4-2103815 | - |
| R4-2103996 | (reserved) | BSRF session | not treated | - | - |
| R4-2103997 | (reserved) | BSRF session | not treated | - | - |
| R4-2103998 | Simulation assumption for NTN co-existence study | CATT | noted | R4-2103965 | - |
| R4-2103999 | WF for TDD Repeater | CMCC | approved | R4-2103881 | - |
| R4-2104000 | (reserved) | BSRF session | not treated | - | - |
| R4-2104001 | (reserved) | BSRF session | not treated | - | - |
| R4-2104002 | (reserved) | BSRF session | not treated | - | - |
| R4-2104003 | (reserved) | BSRF session | not treated | - | - |
| R4-2104004 | (reserved) | BSRF session | not treated | - | - |
| R4-2104005 | (reserved) | BSRF session | not treated | - | - |
| R4-2104006 | (reserved) | BSRF session | not treated | - | - |
| R4-2104007 | (reserved) | BSRF session | not treated | - | - |
| R4-2104008 | (reserved) | BSRF session | not treated | - | - |
| R4-2104009 | (reserved) | BSRF session | not treated | - | - |
| R4-2104010 | (reserved) | BSRF session | not treated | - | - |
| R4-2104011 | (reserved) | BSRF session | not treated | - | - |
| R4-2104012 | (reserved) | BSRF session | not treated | - | - |
| R4-2104013 | (reserved) | BSRF session | not treated | - | - |
| R4-2104014 | (reserved) | BSRF session | not treated | - | - |
| R4-2104015 | (reserved) | BSRF session | not treated | - | - |
| R4-2104016 | (reserved) | BSRF session | not treated | - | - |
| R4-2104017 | (reserved) | BSRF session | not treated | - | - |
| R4-2104018 | (reserved) | BSRF session | not treated | - | - |
| R4-2104019 | (reserved) | BSRF session | not treated | - | - |
| R4-2104020 | (reserved) | BSRF session | not treated | - | - |
| R4-2104021 | (reserved) | BSRF session | not treated | - | - |
| R4-2104022 | (reserved) | BSRF session | not treated | - | - |
| R4-2104023 | (reserved) | BSRF session | not treated | - | - |
| R4-2104024 | (reserved) | BSRF session | not treated | - | - |
| R4-2104025 | (reserved) | BSRF session | not treated | - | - |
| R4-2104026 | (reserved) | BSRF session | not treated | - | - |
| R4-2104027 | (reserved) | BSRF session | not treated | - | - |
| R4-2104028 | (reserved) | BSRF session | not treated | - | - |
| R4-2104029 | (reserved) | BSRF session | not treated | - | - |
| R4-2104030 | (reserved) | BSRF session | not treated | - | - |
| R4-2104031 | (reserved) | BSRF session | not treated | - | - |
| R4-2104032 | (reserved) | BSRF session | not treated | - | - |
| R4-2104033 | (reserved) | BSRF session | not treated | - | - |
| R4-2104034 | (reserved) | BSRF session | not treated | - | - |
| R4-2104035 | (reserved) | BSRF session | not treated | - | - |
| R4-2104036 | (reserved) | BSRF session | not treated | - | - |
| R4-2104037 | (reserved) | BSRF session | not treated | - | - |
| R4-2104038 | (reserved) | BSRF session | not treated | - | - |
| R4-2104039 | (reserved) | BSRF session | not treated | - | - |
| R4-2104040 | (reserved) | ETSI MCC | not treated | - | - |
| R4-2104041 | Void | Void | withdrawn | - | - |
| R4-2104042 | draftCR to introduce TC for RSTD measurement accuracy for FR1 and FR2 in SA | Huawei, HiSilicon | postponed | R4-2102778 | - |
| R4-2104043 | CR on Scell activation delay maintenance (R15) | Apple | revised | R4-2101006 | R4-2104084 |
| R4-2104044 | CR to remove intra-frequency ECID requirements for NE-DC 36133 R15 | Huawei, HiSilicon | agreed | R4-2102731 | - |
| R4-2104045 | draftCR on congestion control test case | MediaTek inc. | endorsed | R4-2101057 | - |
| R4-2104046 | Summary of link level simulation result of RSTD, PRS RSRP and UE DToA | Intel Corporation | revised | - | R4-2104073 |
| R4-2104047 | Updates in RLM requirements for NR-U | Ericsson | agreed | R4-2102513 | - |
| R4-2104048 | LS on SRS antenna port switching | Nokia | noted | - | - |
| R4-2104049 | Correction on PRACH configuration for Beam Failure Detection and Link Recovery Test in R15 | MediaTek inc. | agreed | R4-2100763 | - |
| R4-2104050 | CR on TS38.133 for Pcell change | MediaTek inc. | agreed | R4-2101210 | - |
| R4-2104051 | MRTD and MTTD in non-contiguous CA in FR1 | Ericsson | withdrawn | R4-2101861 | - |
| R4-2104052 | gNB SRS-RSRP measurement | Ericsson | endorsed | R4-2101760 | - |
| R4-2104053 | draftCR to introduce gNB Rx-Tx time difference requirements | Huawei, HiSilicon | endorsed | R4-2102782 | - |
| R4-2104054 | CR to TS 38.133: Redundant and incorrect TCI state in tests with TRS (Rel-15) | Rohde & Schwarz | agreed | R4-2102306 | - |
| R4-2104055 | CR to 38.133 on Link Recovery requirements (R16) | Apple, MediaTek | agreed | R4-2100205 | - |
| R4-2104056 | CR on maintaining Antenna configurations in TS38.133 R15 | Huawei, HiSilicon | agreed | R4-2103493 | - |
| R4-2104057 | [CR] IAB Core Maintenance | ZTE Corporation | endorsed | R4-2103539 | - |
| R4-2104058 | CR on core requirement for CSI-RS L3 measurement | Xiaomi | agreed | - | - |
| R4-2104059 | CR on core requirement for CSI-RS L3 measurement | Xiaomi | agreed | - | - |
| R4-2104060 | Draft CR of test cases for HO delay and interruption for NR-U | Huawei, HiSilicon | endorsed | R4-2101649 | - |
| R4-2104061 | Draft CR of test cases for RRC release with redirection for NR-U | Huawei, HiSilicon | endorsed | R4-2101650 | - |
| R4-2104062 | Draft CR of test cases for PSCell addition and release for NR-U | Huawei, HiSilicon | endorsed | R4-2101651 | - |
| R4-2104063 | Draft CR of test cases for inter-RAT measurement for NR-U | Huawei, HiSilicon | endorsed | R4-2101652 | - |
| R4-2104064 | Draft CR of test cases for intra-frequency measurement accuracy for NR-U | Huawei, HiSilicon | revised | R4-2101653 | R4-2104077 |
| R4-2104065 | Correction to beam failure recovery test cases R15 | Huawei, HiSilicon | agreed | R4-2103491 | - |
| R4-2104066 | Updated evaluation assumptions for R17 RLM/BFD relaxation | vivo, MediaTek | approved | R4-2103724 | - |
| R4-2104067 | Reply LS on temporary RS for efficient SCell activation in NR CA | RAN4 | approved | R4-2103668 | - |
| R4-2104068 | WF on NR eMIMO RRM Performance requirements | Samsung | approved | R4-2103588 | - |
| R4-2104069 | WF on test cases for IAB-MTs | ZTE Corporation | revised | R4-2103540 | R4-2104091 |
| R4-2104070 | WF on R16 RRM enhancement part 2 | ZTE Corporation | approved | R4-2103609 | - |
| R4-2104071 | CR on activation time in direct SCell activation | Apple | postponed | R4-2103553 | - |
| R4-2104072 | CR on TC for eMTC RSTD measurement R15 | Huawei, HiSilicon | agreed | R4-2103503 | - |
| R4-2104073 | Summary of link level simulation result of RSTD, PRS RSRP and UE DToA | Intel Corporation | revised | R4-2104046 | R4-2104087 |
| R4-2104074 | PRS-RSRP measurement requirements | Ericsson | agreed | R4-2103579 | - |
| R4-2104075 | CR to 38.133 correction on CCSF for NR measurements for positioning | vivo | agreed | R4-2103582 | - |
| R4-2104076 | WF on UE PRS measurement requirements | Huawei, HiSilicon | approved | R4-2103577 | - |
| R4-2104077 | Draft CR of test cases for intra-frequency measurement accuracy for NR-U | Huawei, HiSilicon | endorsed | R4-2104064 | - |
| R4-2104078 | Introduction of NR-U cell reselection tests | Ericsson | endorsed | R4-2103531 | - |
| R4-2104079 | UE timing tests for NR-U | Ericsson | endorsed | R4-2103529 | - |
| R4-2104080 | Test cases on BWP switching with consistent UL LBT failures | Ericsson | endorsed | R4-2103530 | - |
| R4-2104081 | Draft CR: test cases for beam management in NR-U | Ericsson | endorsed | R4-2103524 | - |
| R4-2104082 | Correction on the time for Scell activation and CSI-report in R15 | MediaTek inc. | agreed | R4-2103488 | - |
| R4-2104083 | WF on temporary RS for efficient SCell activation in NR CA | Huawei, HiSilicon | approved | R4-2103667 | - |
| R4-2104084 | CR on Scell activation delay maintenance (R15) | Apple | agreed | R4-2104043 | - |
| R4-2104085 | TC2 - SA interruptions at NR SRS carrier based switching | Ericsson | endorsed | R4-2103616 | - |
| R4-2104086 | Cat-F CR to addition of TRS Configurations in Rel-15 Test Cases | Qualcomm Incorporated | agreed | R4-2103498 | - |
| R4-2104087 | Summary of link level simulation result of RSTD, PRS RSRP and UE DToA | Intel Corporation | revised | R4-2104073 | R4-2104092 |
| R4-2104088 | WF on LBT models for NR-U RRM performance requirements | Qualcomm | approved | R4-2103517 | - |
| R4-2104089 | WF on accuracy requirements and test cases of CSI-RS based L3 measurement | CATT, OPPO | approved | R4-2103630 | - |
| R4-2104090 | CR on SCell activation delay, cell idenfication requirements on deactivated SCell and inter-RAT ECID requirements for NE-DC R15 | Huawei, HiSilicon | agreed | R4-2103482 | - |
| R4-2104091 | WF on test cases for IAB-MTs | ZTE Corporation | approved | R4-2104069 | - |
| R4-2104092 | Summary of link level simulation result of RSTD, PRS RSRP and UE DToA | Intel Corporation | noted | R4-2104087 | - |
| R4-2104093 | 38.133 CR on the CSI-RS based measurement requirements | Nokia, Nokia Shanghai Bell | agreed | R4-2103634 | - |
| R4-2104094 | CR on introduction of missing HST test cases | CMCC | agreed | - | - |
| R4-2104095 | WF on RRM requirements for FR2 Inter-band DL CA and UL CA | Nokia | approved | R4-2103671 | - |
| R4-2104096 | WF on R17 NR MG enhancements - Multiple concurrent and independent MG patterns | MediaTek | approved | R4-2103676 | - |

## Annex B: List of change requests

| Document | Title | Source | Spec | CR | Rev | Rel | Cat | WI | Decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R4-2100053 | CR for missing B48 references in a table and note | C Spire Wireless | 36.101 | 5713 | - | Rel-14 | F | TEI14 | agreed |
| R4-2100244 | CR for missing B48 references in a table and note | C Spire Wireless | 36.101 | 5713 | 1 | Rel-15 | A | TEI14 | agreed |
| R4-2100254 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks, Nokia, Skyworks Solutions Inc. | 36.101 | 5714 | - | Rel-10 | F | LTE_B24_mod-Core | revised |
| R4-2103379 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks, Nokia, Skyworks Solutions Inc. | 36.101 | 5714 | 1 | Rel-10 | F | LTE_B24_mod-Core | agreed |
| R4-2100270 | Introduction of LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL to TS36.101 | LG Electronics France | 36.101 | 5715 | - | Rel-17 | B | LTE_CA_R17_xBDL_2BUL-Core | agreed |
| R4-2100608 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | 36.101 | 5716 | - | Rel-17 | B | LTE_bands_R17_M1_M2_NB1_NB2-Core | revised |
| R4-2103276 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | 36.101 | 5716 | 1 | Rel-17 | B | LTE_bands_R17_M1_M2_NB1_NB2-Core | agreed |
| R4-2100645 | CR for missing B48 references in a table and note | C Spire Wireless | 36.101 | 5717 | - | Rel-15 | A | TEI14 | withdrawn |
| R4-2100648 | CR for missing B48 references in a table and note | C Spire Wireless | 36.101 | 5718 | - | Rel-16 | A | TEI14 | agreed |
| R4-2100651 | CR for missing B48 references in a table and note | C Spire Wireless | 36.101 | 5719 | - | Rel-17 | A | TEI14 | agreed |
| R4-2100728 | Introduction of LTE inter-band Carrier Aggregation for x bands DL (x=4, 5) with 1 band UL to TS36.101 | Nokia, Nokia Shanghai Bell | 36.101 | 5720 | - | Rel-17 | B | LTE_CA_R17_xBDL_1BUL-Core | agreed |
| R4-2101802 | CR for 36.101 to add missing spurious emissions for band 38 UE co-existence (Rel-16) | Huawei, HiSilicon, DT | 36.101 | 5721 | - | Rel-16 | F | TEI16 | agreed |
| R4-2101803 | CR for 36.101 to add missing spurious emissions for band 38 UE co-existence (Rel-17) | Huawei, HiSilicon, DT | 36.101 | 5722 | - | Rel-17 | A | TEI16 | agreed |
| R4-2102495 | Introduction of Rel-17 LTE inter-band CA for 2 bands DL with 1 band UL combinations in TS 36.101 | Qualcomm Incorporated | 36.101 | 5723 | - | Rel-17 | B | LTE_CA_R17_2BDL_1BUL-Core | agreed |
| R4-2102596 | CR for TS 36.101: Cleanup for spurious emissions for UE co-existence table | Apple | 36.101 | 5724 | - | Rel-15 | F | TEI | revised |
| R4-2103132 | CR for TS 36.101: Cleanup for spurious emissions for UE co-existence table | Apple | 36.101 | 5724 | 1 | Rel-15 | F | TEI | agreed |
| R4-2102604 | CR for TS 36.101: Cleanup for spurious emissions for UE co-existence table | Apple | 36.101 | 5725 | - | Rel-16 | F | TEI16 | revised |
| R4-2103133 | CR for TS 36.101: Cleanup for spurious emissions for UE co-existence table | Apple | 36.101 | 5725 | 1 | Rel-16 | F | TEI16 | agreed |
| R4-2102605 | CR for TS 36.101: Cleanup for spurious emissions for UE co-existence table | Apple | 36.101 | 5726 | - | Rel-17 | A | TEI16 | agreed |
| R4-2102832 | Introduction of completed R17 3DL band combinations to TS 36.101 | Huawei, HiSilicon | 36.101 | 5727 | - | Rel-17 | B | LTE_CA_R17_3BDL_1BUL | agreed |
| R4-2102909 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | 36.101 | 5728 | - | Rel-11 | A | LTE_B24_mod-Core | revised |
| R4-2103380 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | 36.101 | 5728 | 1 | Rel-11 | A | LTE_B24_mod-Core | agreed |
| R4-2102910 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | 36.101 | 5729 | - | Rel-12 | A | LTE_B24_mod-Core | revised |
| R4-2103381 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | 36.101 | 5729 | 1 | Rel-12 | A | LTE_B24_mod-Core | agreed |
| R4-2102911 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | 36.101 | 5730 | - | Rel-17 | A | LTE_B24_mod-Core | withdrawn |
| R4-2102913 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | 36.101 | 5731 | - | Rel-14 | A | LTE_B24_mod-Core | revised |
| R4-2103382 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | 36.101 | 5731 | 1 | Rel-14 | A | LTE_B24_mod-Core | agreed |
| R4-2102914 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | 36.101 | 5732 | - | Rel-15 | A | LTE_B24_mod-Core | revised |
| R4-2103383 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | 36.101 | 5732 | 1 | Rel-15 | A | LTE_B24_mod-Core | agreed |
| R4-2102915 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | 36.101 | 5733 | - | Rel-16 | A | LTE_B24_mod-Core | revised |
| R4-2103384 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | 36.101 | 5733 | 1 | Rel-16 | A | LTE_B24_mod-Core | agreed |
| R4-2102916 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | 36.101 | 5734 | - | Rel-17 | A | LTE_B24_mod-Core | revised |
| R4-2103385 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | 36.101 | 5734 | 1 | Rel-17 | A | LTE_B24_mod-Core | agreed |
| R4-2102918 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | 36.101 | 5735 | - | Rel-13 | A | LTE_B24_mod-Core | revised |
| R4-2103386 | CR for 36.101: Corrections related to Band 24 regulatory updates | Ligado Networks | 36.101 | 5735 | 1 | Rel-13 | A | LTE_B24_mod-Core | agreed |
| R4-2100336 | CR to 36104 on introducing new SUL band n99 | Huawei,HiSilicon | 36.104 | 4920 | - | Rel-17 | B | NR_SUL_UL_n24-Core | agreed |
| R4-2100609 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | 36.104 | 4921 | - | Rel-17 | B | LTE_bands_R17_M1_M2_NB1_NB2-Core | revised |
| R4-2103277 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | 36.104 | 4921 | 1 | Rel-17 | B | LTE_bands_R17_M1_M2_NB1_NB2-Core | agreed |
| R4-2101082 | CR to TS 36.104: Additions of regional requirements for band 41 in Japan, Rel-15 | NEC, SoftBank, KDDI | 36.104 | 4922 | - | Rel-15 | F | TEI15 | revised |
| R4-2103775 | CR to TS 36.104: Additions of regional requirements for band 41 in Japan, Rel-15 | NEC, SoftBank, KDDI, Nokia | 36.104 | 4922 | 1 | Rel-15 | F | TEI15 | agreed |
| R4-2101083 | CR to TS 36.104: Additions of regional requirements for band 41 in Japan, Rel-16 | NEC, SoftBank, KDDI, Nokia | 36.104 | 4923 | - | Rel-16 | A | TEI15 | agreed |
| R4-2101084 | CR to TS 36.104: Additions of regional requirements for band 41 in Japan, Rel-17 | NEC, SoftBank, KDDI, Nokia | 36.104 | 4924 | - | Rel-17 | A | TEI15 | agreed |
| R4-2101972 | CR to TS 36.104: corrections of NR-U BS RF requirements | ZTE Corporation | 36.104 | 4925 | - | Rel-16 | F | NR_unlic-Core | revised |
| R4-2103973 | CR to TS 36.104: corrections of NR-U BS RF requirements | ZTE Corporation | 36.104 | 4925 | 1 | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2101973 | CR to TS 36.104: corrections of NR-U BS RF requirements | ZTE Corporation | 36.104 | 4926 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2102447 | CR to 36.104: Introduction of n24 requirements | Nokia, Nokia Shanghai Bell | 36.104 | 4927 | - | Rel-17 | B | NR_band_n24-Core | agreed |
| R4-2102452 | CR to 36.104: Correction to Band 24 requirements (Rel-10) | Nokia, Nokia Shanghai Bell | 36.104 | 4928 | - | Rel-10 | F | LTE_B24_mod-Core | revised |
| R4-2103207 | CR to 36.104: Correction to Band 24 requirements (Rel-10) | Nokia, Nokia Shanghai Bell | 36.104 | 4928 | 1 | Rel-10 | F | LTE_B24_mod-Core | agreed |
| R4-2102453 | CR to 36.104: Correction to Band 24 requirements (Rel-11) | Nokia, Nokia Shanghai Bell | 36.104 | 4929 | - | Rel-11 | A | LTE_B24_mod-Core | agreed |
| R4-2102454 | CR to 36.104: Correction to Band 24 requirements (Rel-12) | Nokia, Nokia Shanghai Bell | 36.104 | 4930 | - | Rel-12 | A | LTE_B24_mod-Core | agreed |
| R4-2102455 | CR to 36.104: Correction to Band 24 requirements (Rel-13) | Nokia, Nokia Shanghai Bell | 36.104 | 4931 | - | Rel-13 | A | LTE_B24_mod-Core | agreed |
| R4-2102456 | CR to 36.104: Correction to Band 24 requirements (Rel-14) | Nokia, Nokia Shanghai Bell | 36.104 | 4932 | - | Rel-14 | A | LTE_B24_mod-Core | agreed |
| R4-2102457 | CR to 36.104: Correction to Band 24 requirements (Rel-15) | Nokia, Nokia Shanghai Bell | 36.104 | 4933 | - | Rel-15 | A | LTE_B24_mod-Core | agreed |
| R4-2102458 | CR to 36.104: Correction to Band 24 requirements (Rel-16) | Nokia, Nokia Shanghai Bell | 36.104 | 4934 | - | Rel-16 | A | LTE_B24_mod-Core | agreed |
| R4-2102459 | CR to 36.104: Correction to Band 24 requirements (Rel-17) | Nokia, Nokia Shanghai Bell | 36.104 | 4935 | - | Rel-17 | A | LTE_B24_mod-Core | agreed |
| R4-2100457 | CR on NCSG in 36.133 | CATT | 36.133 | 7013 | - | Rel-15 | F | TEI | merged |
| R4-2100458 | CR on NCSG in 36.133 | CATT | 36.133 | 7014 | - | Rel-16 | A | TEI | withdrawn |
| R4-2100459 | CR on NCSG in 36.133 | CATT | 36.133 | 7015 | - | Rel-17 | A | TEI | withdrawn |
| R4-2100610 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | 36.133 | 7016 | - | Rel-17 | B | LTE_bands_R17_M1_M2_NB1_NB2-Core | revised |
| R4-2103278 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | 36.133 | 7016 | 1 | Rel-17 | B | LTE_bands_R17_M1_M2_NB1_NB2-Core | agreed |
| R4-2101208 | Correction on the synchronous condition for DAPS handover | Ericsson | 36.133 | 7017 | - | Rel-16 | F | LTE_feMob-Core | postponed |
| R4-2101209 | Correction on the synchronous condition for DAPS handover | Ericsson | 36.133 | 7018 | - | Rel-17 | A | LTE_feMob-Core | withdrawn |
| R4-2101453 | CR: Correction of eMTC RLM test cases (Rel-14) | Ericsson | 36.133 | 7019 | - | Rel-14 | F | LTE_feMTC-Perf | revised |
| R4-2103499 | CR: Correction of eMTC RLM test cases (Rel-14) | Ericsson | 36.133 | 7019 | 1 | Rel-14 | F | LTE_feMTC-Perf | agreed |
| R4-2101454 | CR: Correction of eMTC RLM test cases | Ericsson | 36.133 | 7020 | - | Rel-15 | F | LTE_feMTC-Perf | revised |
| R4-2103500 | CR: Correction of eMTC RLM test cases | Ericsson | 36.133 | 7020 | 1 | Rel-15 | F | LTE_feMTC-Perf | agreed |
| R4-2101455 | CR: Correction of eMTC RLM test cases | Ericsson | 36.133 | 7021 | - | Rel-16 | A | LTE_feMTC-Perf | agreed |
| R4-2101456 | CR: Correction of eMTC RLM test cases | Ericsson | 36.133 | 7022 | - | Rel-17 | A | LTE_feMTC-Perf | agreed |
| R4-2101457 | Correction Rel-16 eMTC RRM performance requirements | Ericsson | 36.133 | 7023 | - | Rel-16 | F | LTE_eMTC5-Core, LTE_eMTC5-Perf | revised |
| R4-2103658 | Correction Rel-16 eMTC RRM performance requirements | Ericsson | 36.133 | 7023 | 1 | Rel-16 | F | LTE_eMTC5-Core, LTE_eMTC5-Perf | agreed |
| R4-2101458 | Correction Rel-16 eMTC RRM performance requirements | Ericsson | 36.133 | 7024 | - | Rel-17 | A | LTE_eMTC5-Perf | agreed |
| R4-2101631 | CR on interruption requirements of BWP switch on multiple CCs for EN-DC | Huawei, HiSilicon | 36.133 | 7025 | - | Rel-16 | F | NR_RRM_enh-Core | agreed |
| R4-2101632 | CR on interruption requirements of BWP switch on multiple CCs for EN-DC | Huawei, HiSilicon | 36.133 | 7026 | - | Rel-17 | A | NR_RRM_enh-Core | agreed |
| R4-2101654 | CR on maintenance for measurement on non-anchor carrier for NB-IoT | Huawei, HiSilicon | 36.133 | 7027 | - | Rel-16 | F | NB_IOTenh3-Core | revised |
| R4-2103660 | CR on maintenance for measurement on non-anchor carrier for NB-IoT | Huawei, HiSilicon | 36.133 | 7027 | 1 | Rel-16 | F | NB_IOTenh3-Core | agreed |
| R4-2101655 | CR on maintenance for measurement on non-anchor carrier for NB-IoT | Huawei, HiSilicon | 36.133 | 7028 | - | Rel-17 | A | NB_IOTenh3-Core | agreed |
| R4-2101697 | Clarification on asynchronous DAPS handover | Huawei, HiSilicon | 36.133 | 7029 | - | Rel-16 | F | LTE_feMob-Core | revised |
| R4-2103661 | Clarification on asynchronous DAPS handover | Huawei, HiSilicon | 36.133 | 7029 | 1 | Rel-16 | F | LTE_feMob-Core | agreed |
| R4-2101698 | Clarification on asynchronous DAPS handover | Huawei, HiSilicon | 36.133 | 7030 | - | Rel-17 | A | LTE_feMob-Core | agreed |
| R4-2101699 | Test cases for inter-frequency DAPS handover | Huawei, HiSilicon | 36.133 | 7031 | - | Rel-16 | B | LTE_feMob-Perf | revised |
| R4-2103662 | Test cases for inter-frequency DAPS handover | Huawei, HiSilicon | 36.133 | 7031 | 1 | Rel-16 | B | LTE_feMob-Perf | agreed |
| R4-2101700 | Test cases for inter-frequency DAPS handover | Huawei, HiSilicon | 36.133 | 7032 | - | Rel-17 | A | LTE_feMob-Perf | agreed |
| R4-2102234 | Correction to PUR requirements for NB1 UE | Ericsson | 36.133 | 7033 | - | Rel-17 | A | NB_IOTenh3-Core | withdrawn |
| R4-2102235 | Correction to PUR requirements for NB1 UE | Ericsson | 36.133 | 7034 | - | Rel-16 | F | NB_IOTenh3-Core | not pursued |
| R4-2102236 | Inclusive language review in TS 36.133 | Ericsson | 36.133 | 7035 | - | Rel-17 | F | TEI17 | revised |
| R4-2103254 | Inclusive language review in TS 36.133 | Ericsson | 36.133 | 7035 | 1 | Rel-17 | F | TEI17 | agreed |
| R4-2102248 | Clarification for performance requirements tests for euCA | Nokia, Nokia Shanghai Bell | 36.133 | 7036 | - | Rel-16 | F | LTE_euCA-Core | agreed |
| R4-2102249 | Clarification for performance requirements tests for euCA | Nokia, Nokia Shanghai Bell | 36.133 | 7037 | - | Rel-17 | A | LTE_euCA-Core | agreed |
| R4-2102348 | CR 36.133 (A.8.16.106) Correction of test case for direct SCell activation at addition | Ericsson | 36.133 | 7038 | - | Rel-15 | F | LTE_euCA-Perf | agreed |
| R4-2102349 | CR 36.133 (A.8.16.106) Correction of test case for direct SCell activation at addition | Ericsson | 36.133 | 7039 | - | Rel-16 | A | LTE_euCA-Perf | agreed |
| R4-2102350 | CR 36.133 (A.8.16.106) Correction of test case for direct SCell activation at addition | Ericsson | 36.133 | 7040 | - | Rel-17 | A | LTE_euCA-Perf | agreed |
| R4-2102366 | CR 36.133 Removal of brackets for SCell dormancy | Ericsson | 36.133 | 7041 | - | Rel-16 | F | LTE_NR_DC_CA_enh-Core | agreed |
| R4-2102367 | CR 36.133 Removal of brackets for SCell dormancy | Ericsson | 36.133 | 7042 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Core | agreed |
| R4-2102485 | CR on 36133 LTE FDD-TDD inter-F sync DAPS HO TCs | Nokia, Nokia Shanghai Bell | 36.133 | 7043 | - | Rel-16 | B | LTE_feMob-Perf | agreed |
| R4-2102486 | CR on 36133 LTE FDD-TDD inter-F sync DAPS HO TCs - Cat-A | Nokia, Nokia Shanghai Bell | 36.133 | 7044 | - | Rel-17 | A | LTE_feMob-Perf | agreed |
| R4-2102521 | Terminology updates for NR-U in 36.133 | Ericsson | 36.133 | 7045 | - | Rel-16 | F | NR_unlic-Core | revised |
| R4-2103515 | Terminology updates for NR-U in 36.133 | Ericsson | 36.133 | 7045 | 1 | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2102522 | Terminology updates for NR-U in 36.133 | Ericsson | 36.133 | 7046 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2102632 | Addition of missing async FDD-TDD and TDD-FDD LTE interband interfrquency DAPS handover tests | Ericsson | 36.133 | 7047 | - | Rel-16 | F | LTE_feMob-Perf | revised |
| R4-2103663 | Addition of missing async FDD-TDD and TDD-FDD LTE interband interfrquency DAPS handover tests | Ericsson | 36.133 | 7047 | 1 | Rel-16 | F | LTE_feMob-Perf | agreed |
| R4-2102633 | Addition of missing async FDD-TDD and TDD-FDD LTE interband interfrquency DAPS handover tests | Ericsson | 36.133 | 7048 | - | Rel-17 | A | LTE_feMob-Perf | agreed |
| R4-2102644 | Applicability of RA with CCA on RRM requirements in NR-U in 36.133 | Ericsson | 36.133 | 7049 | - | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2102645 | Applicability of RA with CCA on RRM requirements in NR-U in 36.133 | Ericsson | 36.133 | 7050 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2102693 | Correction to applicability of E-UTRAN E-CID measurements requirements for NE-DC | Nokia, Nokia Shanghai Bell | 36.133 | 7051 | - | Rel-15 | F | TEI15 | revised |
| R4-2103501 | Correction to applicability of E-UTRAN E-CID measurements requirements for NE-DC | Nokia, Nokia Shanghai Bell | 36.133 | 7051 | 1 | Rel-15 | F | TEI15 | agreed |
| R4-2102694 | Correction to applicability of E-UTRAN E-CID measurements requirements for NE-DC | Nokia, Nokia Shanghai Bell | 36.133 | 7052 | - | Rel-16 | A | TEI15 | agreed |
| R4-2102695 | Correction to applicability of E-UTRAN E-CID measurements requirements for NE-DC | Nokia, Nokia Shanghai Bell | 36.133 | 7053 | - | Rel-17 | A | TEI15 | agreed |
| R4-2102696 | Correction to requirements for NCSG patterns | Nokia, Nokia Shanghai Bell | 36.133 | 7054 | - | Rel-14 | F | TEI14 | revised |
| R4-2103502 | Correction to requirements for NCSG patterns | Nokia, Nokia Shanghai Bell | 36.133 | 7054 | 1 | Rel-14 | F | TEI14 | agreed |
| R4-2102697 | Correction to requirements for NCSG patterns | Nokia, Nokia Shanghai Bell | 36.133 | 7055 | - | Rel-15 | A | TEI14 | agreed |
| R4-2102698 | Correction to requirements for NCSG patterns | Nokia, Nokia Shanghai Bell | 36.133 | 7056 | - | Rel-16 | A | TEI14 | agreed |
| R4-2102699 | Correction to requirements for NCSG patterns | Nokia, Nokia Shanghai Bell | 36.133 | 7057 | - | Rel-17 | A | TEI14 | agreed |
| R4-2102731 | CR to remove intra-frequency ECID requirements for NE-DC 36133 R15 | Huawei, HiSilicon | 36.133 | 7058 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2104044 | CR to remove intra-frequency ECID requirements for NE-DC 36133 R15 | Huawei, HiSilicon | 36.133 | 7058 | 1 | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2102732 | CR to remove intra-frequency ECID requirements for NE-DC 36133 R16 | Huawei, HiSilicon | 36.133 | 7059 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2102733 | CR to remove intra-frequency ECID requirements for NE-DC 36133 R17 | Huawei, HiSilicon | 36.133 | 7060 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2102734 | CR to idle more requirements in 36133 R15 | Huawei, HiSilicon | 36.133 | 7061 | - | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2102735 | CR to idle more requirements in 36133 R16 | Huawei, HiSilicon | 36.133 | 7062 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2102736 | CR to idle more requirements in 36133 R17 | Huawei, HiSilicon | 36.133 | 7063 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2102747 | CR on EMR requirements in 36.133 | Huawei, HiSilicon | 36.133 | 7064 | - | Rel-16 | F | LTE_NR_DC_CA_enh-Core | revised |
| R4-2103727 | CR on EMR requirements in 36.133 | Huawei, HiSilicon | 36.133 | 7064 | 1 | Rel-16 | F | LTE_NR_DC_CA_enh-Core | agreed |
| R4-2102748 | CR on EMR requirements in 36.133 R17 | Huawei, HiSilicon | 36.133 | 7065 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Core | agreed |
| R4-2102752 | Big CR: Introduction of Rel-16 MR-DC EMR RRM performance requirements (TS 36.133) | Huawei, HiSilicon | 36.133 | 7066 | - | Rel-16 | B | LTE_NR_DC_CA_enh-Perf | postponed |
| R4-2102753 | Big CR: Introduction of Rel-16 MR-DC EMR RRM performance requirements (TS 36.133) | Huawei, HiSilicon | 36.133 | 7067 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Perf | withdrawn |
| R4-2102769 | CR to introduce new measurement gap patterns for positioning in 36.133 | Huawei, HiSilicon | 36.133 | 7068 | - | Rel-16 | F | NR_pos-Core | revised |
| R4-2103584 | CR to introduce new measurement gap patterns for positioning in 36.133 | Huawei, HiSilicon | 36.133 | 7068 | 1 | Rel-16 | F | NR_pos-Core | agreed |
| R4-2102770 | CR to introduce new measurement gap patterns for positioning in 36.133 R17 | Huawei, HiSilicon | 36.133 | 7069 | - | Rel-17 | A | NR_pos-Core | agreed |
| R4-2102793 | CR on CGI reading report delay 36.133 | Huawei, HiSilicon | 36.133 | 7070 | - | Rel-16 | F | NR_RRM_enh-Core | agreed |
| R4-2102794 | CR on CGI reading report delay 36.133 R17 | Huawei, HiSilicon | 36.133 | 7071 | - | Rel-17 | A | NR_RRM_enh-Core | agreed |
| R4-2102797 | CR on eMTC RRM requirements | Huawei, HiSilicon | 36.133 | 7072 | - | Rel-16 | F | LTE_eMTC5-Core | revised |
| R4-2103659 | CR on eMTC RRM requirements | Huawei, HiSilicon | 36.133 | 7072 | 1 | Rel-16 | F | LTE_eMTC5-Core | agreed |
| R4-2102798 | CR on eMTC RRM requirements R17 | Huawei, HiSilicon | 36.133 | 7073 | - | Rel-17 | A | LTE_eMTC5-Core | agreed |
| R4-2102804 | CR on TC for eMTC RSTD measurement R15 | Huawei, HiSilicon | 36.133 | 7074 | - | Rel-15 | F | LTE_eMTC4-Perf | revised |
| R4-2103503 | CR on TC for eMTC RSTD measurement R15 | Huawei, HiSilicon | 36.133 | 7074 | 1 | Rel-15 | F | LTE_eMTC4-Perf | revised |
| R4-2104072 | CR on TC for eMTC RSTD measurement R15 | Huawei, HiSilicon | 36.133 | 7074 | 2 | Rel-15 | F | LTE_eMTC4-Perf | agreed |
| R4-2102805 | CR on TC for eMTC RSTD measurement R16 | Huawei, HiSilicon | 36.133 | 7075 | - | Rel-16 | A | LTE_eMTC4-Perf | agreed |
| R4-2102806 | CR on TC for eMTC RSTD measurement R17 | Huawei, HiSilicon | 36.133 | 7076 | - | Rel-17 | A | LTE_eMTC4-Perf | agreed |
| R4-2102807 | CR on CRS muting for eMTC R15 | Huawei, HiSilicon | 36.133 | 7077 | - | Rel-15 | F | LTE_eMTC4-Core | revised |
| R4-2103504 | CR on CRS muting for eMTC R15 | Huawei, HiSilicon | 36.133 | 7077 | 1 | Rel-15 | F | LTE_eMTC4-Core | agreed |
| R4-2102808 | CR on CRS muting for eMTC R16 | Huawei, HiSilicon | 36.133 | 7078 | - | Rel-16 | A | LTE_eMTC4-Core | agreed |
| R4-2102809 | CR on CRS muting for eMTC R17 | Huawei, HiSilicon | 36.133 | 7079 | - | Rel-17 | A | LTE_eMTC4-Core | agreed |
| R4-2102883 | Cat-F CR to Removal of brackets for SCell Dormancy and Simultaneous DCI based BWP switch delay on multiple CCs in Rel-16 LTE | Qualcomm Incorporated | 36.133 | 7080 | - | Rel-16 | F | LTE_NR_DC_CA_enh-Core | merged |
| R4-2102884 | Cat-A CR to Removal of brackets for SCell Dormancy and Simultaneous DCI based BWP switch delay on multiple CCs in Rel-17 LTE | Qualcomm Incorporated | 36.133 | 7081 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Core | withdrawn |
| R4-2100339 | CR to 36141 on introducing new SUL band n99 | Huawei,HiSilicon | 36.141 | 1288 | - | Rel-17 | B | NR_SUL_UL_n24-Perf | agreed |
| R4-2100611 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | 36.141 | 1289 | - | Rel-17 | B | LTE_bands_R17_M1_M2_NB1_NB2-Perf | revised |
| R4-2103281 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | 36.141 | 1289 | 1 | Rel-17 | B | LTE_bands_R17_M1_M2_NB1_NB2-Perf | agreed |
| R4-2101085 | CR to TS 36.141: Additions of regional requirements for band 41 in Japan, Rel-15 | NEC, SoftBank, KDDI | 36.141 | 1290 | - | Rel-15 | F | TEI15 | revised |
| R4-2103776 | CR to TS 36.141: Additions of regional requirements for band 41 in Japan, Rel-15 | NEC, SoftBank, KDDI, Nokia | 36.141 | 1290 | 1 | Rel-15 | F | TEI15 | agreed |
| R4-2101086 | CR to TS 36.141: Additions of regional requirements for band 41 in Japan, Rel-16 | NEC, SoftBank, KDDI, Nokia | 36.141 | 1291 | - | Rel-16 | A | TEI15 | agreed |
| R4-2101087 | CR to TS 36.141: Additions of regional requirements for band 41 in Japan, Rel-17 | NEC, SoftBank, KDDI, Nokia | 36.141 | 1292 | - | Rel-17 | A | TEI15 | agreed |
| R4-2101984 | CR to TS 36.141: introduction of NR-U BS | ZTE Corporation | 36.141 | 1293 | - | Rel-16 | B | NR_unlic-Perf | postponed |
| R4-2101985 | CR to TS 36.141: introduction of NR-U BS | ZTE Corporation | 36.141 | 1294 | - | Rel-17 | A | NR_unlic-Perf | withdrawn |
| R4-2102448 | CR to 36.141: Introduction of n24 requirements | Nokia, Nokia Shanghai Bell | 36.141 | 1295 | - | Rel-17 | B | NR_band_n24-Perf | agreed |
| R4-2102460 | CR to 36.141: Correction to Band 24 requirements (Rel-10) | Nokia, Nokia Shanghai Bell | 36.141 | 1296 | - | Rel-10 | F | LTE_B24_mod-Perf | revised |
| R4-2103217 | CR to 36.141: Correction to Band 24 requirements (Rel-10) | Nokia, Nokia Shanghai Bell | 36.141 | 1296 | 1 | Rel-10 | F | LTE_B24_mod-Perf | agreed |
| R4-2102461 | CR to 36.141: Correction to Band 24 requirements (Rel-11) | Nokia, Nokia Shanghai Bell | 36.141 | 1297 | - | Rel-11 | A | LTE_B24_mod-Perf | agreed |
| R4-2102462 | CR to 36.141: Correction to Band 24 requirements (Rel-12) | Nokia, Nokia Shanghai Bell | 36.141 | 1298 | - | Rel-12 | A | LTE_B24_mod-Perf | agreed |
| R4-2102463 | CR to 36.141: Correction to Band 24 requirements (Rel-13) | Nokia, Nokia Shanghai Bell | 36.141 | 1299 | - | Rel-13 | A | LTE_B24_mod-Perf | agreed |
| R4-2102464 | CR to 36.141: Correction to Band 24 requirements (Rel-14) | Nokia, Nokia Shanghai Bell | 36.141 | 1300 | - | Rel-14 | A | LTE_B24_mod-Perf | agreed |
| R4-2102465 | CR to 36.141: Correction to Band 24 requirements (Rel-15) | Nokia, Nokia Shanghai Bell | 36.141 | 1301 | - | Rel-15 | A | LTE_B24_mod-Perf | agreed |
| R4-2102466 | CR to 36.141: Correction to Band 24 requirements (Rel-16) | Nokia, Nokia Shanghai Bell | 36.141 | 1302 | - | Rel-16 | A | LTE_B24_mod-Perf | agreed |
| R4-2102467 | CR to 36.141: Correction to Band 24 requirements (Rel-17) | Nokia, Nokia Shanghai Bell | 36.141 | 1303 | - | Rel-17 | A | LTE_B24_mod-Perf | agreed |
| R4-2100604 | CR of adding LTE B24 for UE category NB1 in R17 | Ericsson, Ligado Networks | 36.307 | 4443 | - | Rel-13 | B | LTE_bands_R17_M1_M2_NB1_NB2-Core | revised |
| R4-2103275 | CR of adding LTE B24 for UE category NB1 in R17 | Ericsson, Ligado Networks | 36.307 | 4443 | 1 | Rel-13 | B | LTE_bands_R17_M1_M2_NB1_NB2-Core | agreed |
| R4-2100605 | CR of adding LTE B24 for UE category NB1 in R17 | Ericsson, Ligado Networks | 36.307 | 4444 | - | Rel-14 | A | LTE_bands_R17_M1_M2_NB1_NB2-Core | agreed |
| R4-2100606 | CR of adding LTE B24 for UE category NB1 in R17 | Ericsson, Ligado Networks | 36.307 | 4445 | - | Rel-15 | A | LTE_bands_R17_M1_M2_NB1_NB2-Core | agreed |
| R4-2100607 | CR of adding LTE B24 for UE category NB1 in R17 | Ericsson, Ligado Networks | 36.307 | 4446 | - | Rel-16 | A | LTE_bands_R17_M1_M2_NB1_NB2-Core | agreed |
| R4-2100614 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | 36.307 | 4447 | - | Rel-14 | B | LTE_bands_R17_M1_M2_NB1_NB2-Core | revised |
| R4-2103280 | CR of adding LTE B24 for UE category NB2 in R17 | Ericsson, Ligado Networks | 36.307 | 4447 | 1 | Rel-14 | B | LTE_bands_R17_M1_M2_NB1_NB2-Core | agreed |
| R4-2100615 | CR of adding LTE B24 for UE category NB2 in R17 | Ericsson, Ligado Networks | 36.307 | 4448 | - | Rel-15 | A | LTE_bands_R17_M1_M2_NB1_NB2-Core | agreed |
| R4-2100616 | CR of adding LTE B24 for UE category NB2 in R17 | Ericsson, Ligado Networks | 36.307 | 4449 | - | Rel-16 | A | LTE_bands_R17_M1_M2_NB1_NB2-Core | agreed |
| R4-2100340 | CR to 37104 on introducing new SUL band n99 | Huawei,HiSilicon | 37.104 | 0920 | - | Rel-17 | B | NR_SUL_UL_n24-Core | agreed |
| R4-2100612 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | 37.104 | 0921 | - | Rel-17 | B | LTE_bands_R17_M1_M2_NB1_NB2-Core | revised |
| R4-2103279 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | 37.104 | 0921 | 1 | Rel-17 | B | LTE_bands_R17_M1_M2_NB1_NB2-Core | agreed |
| R4-2101974 | CR to TS 37.104: corrections of NR-U BS RF requirements | ZTE Corporation | 37.104 | 0922 | - | Rel-16 | F | NR_unlic-Core | revised |
| R4-2103974 | CR to TS 37.104: corrections of NR-U BS RF requirements | ZTE Corporation | 37.104 | 0922 | 1 | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2101975 | CR to TS 37.104: corrections of NR-U BS RF requirements | ZTE Corporation | 37.104 | 0923 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2102449 | CR to 37.104: Introduction of n24 requirements | Nokia, Nokia Shanghai Bell | 37.104 | 0924 | - | Rel-17 | B | NR_band_n24-Core | agreed |
| R4-2102468 | CR to 37.104: Correction to Band 24 requirements (Rel-10) | Nokia, Nokia Shanghai Bell | 37.104 | 0925 | - | Rel-10 | F | LTE_B24_mod-Core | revised |
| R4-2103208 | CR to 37.104: Correction to Band 24 requirements (Rel-10) | Nokia, Nokia Shanghai Bell | 37.104 | 0925 | 1 | Rel-10 | F | LTE_B24_mod-Core | agreed |
| R4-2102469 | CR to 37.104: Correction to Band 24 requirements (Rel-11) | Nokia, Nokia Shanghai Bell | 37.104 | 0926 | - | Rel-11 | A | LTE_B24_mod-Core | agreed |
| R4-2102470 | CR to 37.104: Correction to Band 24 requirements (Rel-12) | Nokia, Nokia Shanghai Bell | 37.104 | 0927 | - | Rel-12 | A | LTE_B24_mod-Core | agreed |
| R4-2102471 | CR to 37.104: Correction to Band 24 requirements (Rel-13) | Nokia, Nokia Shanghai Bell | 37.104 | 0928 | - | Rel-13 | A | LTE_B24_mod-Core | agreed |
| R4-2102472 | CR to 37.104: Correction to Band 24 requirements (Rel-14) | Nokia, Nokia Shanghai Bell | 37.104 | 0929 | - | Rel-14 | A | LTE_B24_mod-Core | agreed |
| R4-2102473 | CR to 37.104: Correction to Band 24 requirements (Rel-15) | Nokia, Nokia Shanghai Bell | 37.104 | 0930 | - | Rel-15 | A | LTE_B24_mod-Core | agreed |
| R4-2102474 | CR to 37.104: Correction to Band 24 requirements (Rel-16) | Nokia, Nokia Shanghai Bell | 37.104 | 0931 | - | Rel-16 | A | LTE_B24_mod-Core | agreed |
| R4-2102475 | CR to 37.104: Correction to Band 24 requirements (Rel-17) | Nokia, Nokia Shanghai Bell | 37.104 | 0932 | - | Rel-17 | A | LTE_B24_mod-Core | agreed |
| R4-2102484 | CR to 37.104: Introduction of requirements for 35 and 45MHz channel bandwidths | Nokia, Nokia Shanghai Bell | 37.104 | 0933 | - | Rel-17 | B | NR_FR1_35MHz_45MHz_BW-Core | revised |
| R4-2103197 | CR to 37.104: Introduction of requirements for 35 and 45MHz channel bandwidths | Nokia, Nokia Shanghai Bell | 37.104 | 0933 | 1 | Rel-17 | B | NR_FR1_35MHz_45MHz_BW-Core | endorsed |
| R4-2102847 | CR to 37.104 on OBUE table headings and applicability | Ericsson | 37.104 | 0934 | - | Rel-15 | F | TEI15 | revised |
| R4-2103883 | CR to 37.104 on OBUE table headings and applicability | Ericsson | 37.104 | 0934 | 1 | Rel-15 | F | TEI15 | agreed |
| R4-2102848 | CR to 37.104 on OBUE table headings and applicability | Ericsson | 37.104 | 0935 | - | Rel-16 | A | TEI15 | agreed |
| R4-2102849 | CR to 37.104 on OBUE table headings and applicability | Ericsson | 37.104 | 0936 | - | Rel-17 | A | TEI15 | agreed |
| R4-2100248 | CR for TS 37.105 introduction of NR band n24 | Ligado Networks | 37.105 | 0214 | - | Rel-17 | B | NR_band_n24-Core | agreed |
| R4-2100255 | CR for 37.105: Corrections related to Band 24 regulatory updates | Ligado Networks | 37.105 | 0215 | - | Rel-15 | F | LTE_B24_mod-Core | agreed |
| R4-2100342 | CR to 37105 on introducing new SUL band n99 | Huawei,HiSilicon | 37.105 | 0216 | - | Rel-17 | B | NR_SUL_UL_n24-Core | agreed |
| R4-2100532 | CR for 37.105: Corrections related to Band 24 regulatory updates | Ligado Networks | 37.105 | 0217 | - | Rel-16 | A | LTE_B24_mod-Core | agreed |
| R4-2100533 | CR for 37.105: Corrections related to Band 24 regulatory updates | Ligado Networks | 37.105 | 0218 | - | Rel-17 | A | LTE_B24_mod-Core | agreed |
| R4-2101559 | CR to TS 37.105: Introduction of CBWs 35 MHz and 45 MHz | Ericsson | 37.105 | 0219 | - | Rel-17 | B | NR_FR1_35MHz_45MHz_BW-Core | revised |
| R4-2103193 | CR to TS 37.105: Introduction of CBWs 35 MHz and 45 MHz | Ericsson | 37.105 | 0219 | 1 | Rel-17 | B | NR_FR1_35MHz_45MHz_BW-Core | endorsed |
| R4-2101976 | CR to TS 37.105: corrections of NR-U BS RF requirements | ZTE Corporation | 37.105 | 0220 | - | Rel-16 | F | NR_unlic-Core | not pursued |
| R4-2101977 | CR to TS 37.105: corrections of NR-U BS RF requirements | ZTE Corporation | 37.105 | 0221 | - | Rel-17 | A | NR_unlic-Core | withdrawn |
| R4-2102563 | CR to TS 37.105: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16 | Huawei, China Unicom | 37.105 | 0222 | - | Rel-16 | F | TEI16, AASenh_BS_LTE_UTRA-Core, MSR_GSM_UTRA_LTE_NR-Core | revised |
| R4-2103782 | CR to TS 37.105: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16 | Huawei, China Unicom | 37.105 | 0222 | 1 | Rel-16 | F | TEI16, AASenh_BS_LTE_UTRA-Core, MSR_GSM_UTRA_LTE_NR-Core | agreed |
| R4-2102564 | CR to TS 37.105: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-17 | Huawei, China Unicom | 37.105 | 0223 | - | Rel-17 | A | TEI16, AASenh_BS_LTE_UTRA-Core, MSR_GSM_UTRA_LTE_NR-Core | agreed |
| R4-2102844 | CR to 37.105 on NR+UTRA support for AAS | Ericsson | 37.105 | 0224 | - | Rel-15 | F | AASenh_BS_LTE_UTRA-Core | revised |
| R4-2103785 | CR to 37.105 on NR+UTRA support for AAS | Ericsson | 37.105 | 0224 | 1 | Rel-15 | F | AASenh_BS_LTE_UTRA-Core | agreed |
| R4-2102845 | CR to 37.105 on NR+UTRA support for AAS | Ericsson | 37.105 | 0225 | - | Rel-16 | A | AASenh_BS_LTE_UTRA-Core | withdrawn |
| R4-2102846 | CR to 37.105 on NR+UTRA support for AAS | Ericsson | 37.105 | 0226 | - | Rel-17 | A | AASenh_BS_LTE_UTRA-Core | withdrawn |
| R4-2102853 | CR to 37.105 on OBUE table headings and applicability | Ericsson | 37.105 | 0227 | - | Rel-15 | F | TEI15 | revised |
| R4-2103885 | CR to 37.105 on OBUE table headings and applicability | Ericsson | 37.105 | 0227 | 1 | Rel-15 | F | TEI15 | agreed |
| R4-2102854 | CR to 37.105 on OBUE table headings and applicability | Ericsson | 37.105 | 0228 | - | Rel-16 | A | TEI15 | withdrawn |
| R4-2102855 | CR to 37.105 on OBUE table headings and applicability | Ericsson | 37.105 | 0229 | - | Rel-17 | A | TEI15 | withdrawn |
| R4-2100341 | CR to 37141 on introducing new SUL band n99 | Huawei,HiSilicon | 37.141 | 0959 | - | Rel-17 | B | NR_SUL_UL_n24-Perf | agreed |
| R4-2100613 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | 37.141 | 0960 | - | Rel-17 | B | LTE_bands_R17_M1_M2_NB1_NB2-Perf | revised |
| R4-2103282 | CR of adding LTE B24 for UE category NB1/NB2 in R17 | Ericsson, Ligado Networks | 37.141 | 0960 | 1 | Rel-17 | B | LTE_bands_R17_M1_M2_NB1_NB2-Perf | agreed |
| R4-2102441 | CR to 37.141: Correction to ACLR limit in non-contiguous spectrum (Rel-15) | Nokia, Nokia Shanghai Bell | 37.141 | 0961 | - | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2102442 | CR to 37.141: Correction to ACLR limit in non-contiguous spectrum (Rel-16) | Nokia, Nokia Shanghai Bell | 37.141 | 0962 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2102443 | CR to 37.141: Correction to ACLR limit in non-contiguous spectrum (Rel-17) | Nokia, Nokia Shanghai Bell | 37.141 | 0963 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2102450 | CR to 37.141: Introduction of n24 requirements | Nokia, Nokia Shanghai Bell | 37.141 | 0964 | - | Rel-17 | B | NR_band_n24-Perf | agreed |
| R4-2102476 | CR to 37.141: Correction to Band 24 requirements (Rel-10) | Nokia, Nokia Shanghai Bell | 37.141 | 0965 | - | Rel-10 | F | LTE_B24_mod-Perf | revised |
| R4-2103216 | CR to 37.141: Correction to Band 24 requirements (Rel-10) | Nokia, Nokia Shanghai Bell | 37.141 | 0965 | 1 | Rel-10 | F | LTE_B24_mod-Perf | agreed |
| R4-2102477 | CR to 37.141: Correction to Band 24 requirements (Rel-11) | Nokia, Nokia Shanghai Bell | 37.141 | 0966 | - | Rel-11 | A | LTE_B24_mod-Perf | agreed |
| R4-2102478 | CR to 37.141: Correction to Band 24 requirements (Rel-12) | Nokia, Nokia Shanghai Bell | 37.141 | 0967 | - | Rel-12 | A | LTE_B24_mod-Perf | agreed |
| R4-2102479 | CR to 37.141: Correction to Band 24 requirements (Rel-13) | Nokia, Nokia Shanghai Bell | 37.141 | 0968 | - | Rel-13 | A | LTE_B24_mod-Perf | agreed |
| R4-2102480 | CR to 37.141: Correction to Band 24 requirements (Rel-14) | Nokia, Nokia Shanghai Bell | 37.141 | 0969 | - | Rel-14 | A | LTE_B24_mod-Perf | agreed |
| R4-2102481 | CR to 37.141: Correction to Band 24 requirements (Rel-15) | Nokia, Nokia Shanghai Bell | 37.141 | 0970 | - | Rel-15 | A | LTE_B24_mod-Perf | agreed |
| R4-2102482 | CR to 37.141: Correction to Band 24 requirements (Rel-16) | Nokia, Nokia Shanghai Bell | 37.141 | 0971 | - | Rel-16 | A | LTE_B24_mod-Perf | agreed |
| R4-2102483 | CR to 37.141: Correction to Band 24 requirements (Rel-17) | Nokia, Nokia Shanghai Bell | 37.141 | 0972 | - | Rel-17 | A | LTE_B24_mod-Perf | agreed |
| R4-2102850 | CR to 37.141 on OBUE table headings and applicability | Ericsson | 37.141 | 0973 | - | Rel-15 | F | TEI15 | revised |
| R4-2103884 | CR to 37.141 on OBUE table headings and applicability | Ericsson | 37.141 | 0973 | 1 | Rel-15 | F | TEI15 | agreed |
| R4-2102851 | CR to 37.141 on OBUE table headings and applicability | Ericsson | 37.141 | 0974 | - | Rel-16 | A | TEI15 | agreed |
| R4-2102852 | CR to 37.141 on OBUE table headings and applicability | Ericsson | 37.141 | 0975 | - | Rel-17 | A | TEI15 | agreed |
| R4-2100249 | CR for TS 37.145-1 introduction of NR band n24 | Ligado Networks | 37.145-1 | 0234 | - | Rel-17 | B | NR_band_n24-Perf | agreed |
| R4-2100256 | CR for 37.145-1: Corrections related to Band 24 regulatory updates | Ligado Networks | 37.145-1 | 0235 | - | Rel-13 | F | LTE_B24_mod-Perf | agreed |
| R4-2100343 | CR to 37145-1 on introducing new SUL band n99 | Huawei,HiSilicon | 37.145-1 | 0236 | - | Rel-17 | B | NR_SUL_UL_n24-Perf | revised |
| R4-2103213 | CR to 37145-1 on introducing new SUL band n99 | Huawei,HiSilicon | 37.145-1 | 0236 | 1 | Rel-17 | B | NR_SUL_UL_n24-Perf | agreed |
| R4-2100534 | CR for 37.145-1: Corrections related to Band 24 regulatory updates | Ligado Networks | 37.145-1 | 0237 | - | Rel-13 | A | NR_band_n24-Perf | withdrawn |
| R4-2100535 | CR for 37.145-1: Corrections related to Band 24 regulatory updates | Ligado Networks | 37.145-1 | 0238 | - | Rel-14 | A | LTE_B24_mod-Perf | agreed |
| R4-2100536 | CR for 37.145-1: Corrections related to Band 24 regulatory updates | Ligado Networks | 37.145-1 | 0239 | - | Rel-15 | A | LTE_B24_mod-Perf | agreed |
| R4-2100537 | CR for 37.145-1: Corrections related to Band 24 regulatory updates | Ligado Networks | 37.145-1 | 0240 | - | Rel-16 | A | LTE_B24_mod-Perf | agreed |
| R4-2100538 | CR for 37.145-1: Corrections related to Band 24 regulatory updates | Ligado Networks | 37.145-1 | 0241 | - | Rel-17 | A | LTE_B24_mod-Perf | agreed |
| R4-2101987 | CR to 37.145-1: Introduction of 35MHz and 45MHz | ZTE Corporation | 37.145-1 | 0242 | - | Rel-17 | B | NR_FR1_35MHz_45MHz_BW-Core | revised |
| R4-2103196 | CR to 37.145-1: Introduction of 35MHz and 45MHz | ZTE Corporation | 37.145-1 | 0242 | 1 | Rel-17 | B | NR_FR1_35MHz_45MHz_BW-Core | endorsed |
| R4-2102423 | CR to TS 37.145-1, Corrections to conformance requirements, Rel-15 | Huawei | 37.145-1 | 0243 | - | Rel-15 | F | TEI15, AAS_BS_LTE_UTRA-Perf | revised |
| R4-2103791 | CR to TS 37.145-1, Corrections to conformance requirements, Rel-15 | Huawei | 37.145-1 | 0243 | 1 | Rel-15 | F | TEI15, AAS_BS_LTE_UTRA-Perf | withdrawn |
| R4-2102424 | CR to TS 37.145-1, Corrections to conformance requirements, Rel-16 | Huawei | 37.145-1 | 0244 | - | Rel-16 | A | TEI15, AAS_BS_LTE_UTRA-Perf | withdrawn |
| R4-2102425 | CR to TS 37.145-1, Corrections to conformance requirements, Rel-17 | Huawei | 37.145-1 | 0245 | - | Rel-17 | A | TEI15, AAS_BS_LTE_UTRA-Perf | withdrawn |
| R4-2102565 | CR to TS 37.145-1: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16 | Huawei, China Unicom | 37.145-1 | 0246 | - | Rel-16 | F | TEI16, AASenh_BS_LTE_UTRA-Perf, MSR_GSM_UTRA_LTE_NR-Perf | revised |
| R4-2103783 | CR to TS 37.145-1: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16 | Huawei, China Unicom | 37.145-1 | 0246 | 1 | Rel-16 | F | TEI16, AASenh_BS_LTE_UTRA-Perf, MSR_GSM_UTRA_LTE_NR-Perf | agreed |
| R4-2102566 | CR to TS 37.145-1: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-17 | Huawei, China Unicom | 37.145-1 | 0247 | - | Rel-17 | A | TEI16, AASenh_BS_LTE_UTRA-Perf, MSR_GSM_UTRA_LTE_NR-Perf | agreed |
| R4-2102841 | CR to 37.145-1 on Removal of additional limit for Band 7 | Ericsson | 37.145-1 | 0248 | - | Rel-15 | F | TEI15 | agreed |
| R4-2102842 | CR to 37.145-1 on Removal of additional limit for Band 7 | Ericsson | 37.145-1 | 0249 | - | Rel-16 | A | TEI15 | agreed |
| R4-2102843 | CR to 37.145-1 on Removal of additional limit for Band 7 | Ericsson | 37.145-1 | 0250 | - | Rel-17 | A | TEI15 | agreed |
| R4-2102856 | CR to 37.145-1 on OBUE table headings and applicability | Ericsson | 37.145-1 | 0251 | - | Rel-15 | F | TEI15 | revised |
| R4-2103886 | CR to 37.145-1 on OBUE table headings and applicability | Ericsson | 37.145-1 | 0251 | 1 | Rel-15 | F | TEI15 | agreed |
| R4-2102857 | CR to 37.145-1 on OBUE table headings and applicability | Ericsson | 37.145-1 | 0252 | - | Rel-16 | A | TEI15 | withdrawn |
| R4-2102858 | CR to 37.145-1 on OBUE table headings and applicability | Ericsson | 37.145-1 | 0253 | - | Rel-17 | A | TEI15 | withdrawn |
| R4-2100250 | CR for TS 37.145-2 introduction of NR band n24 | Ligado Networks | 37.145-2 | 0268 | - | Rel-17 | B | NR_band_n24-Perf | agreed |
| R4-2100257 | CR for 37.145-2: Corrections related to Band 24 regulatory updates | Ligado Networks | 37.145-2 | 0269 | - | Rel-15 | F | LTE_B24_mod-Perf | revised |
| R4-2103215 | CR for 37.145-2: Corrections related to Band 24 regulatory updates | Ligado Networks | 37.145-2 | 0269 | 1 | Rel-15 | F | LTE_B24_mod-Perf | agreed |
| R4-2100344 | CR to 37145-2 on introducing new SUL band n99 | Huawei,HiSilicon | 37.145-2 | 0270 | - | Rel-17 | B | NR_SUL_UL_n24-Perf | revised |
| R4-2103214 | CR to 37145-2 on introducing new SUL band n99 | Huawei,HiSilicon | 37.145-2 | 0270 | 1 | Rel-17 | B | NR_SUL_UL_n24-Perf | agreed |
| R4-2100389 | CR for TS 37.145-2: Correction on definition for the out-of-band CLTA(Rel-15) | CATT | 37.145-2 | 0271 | - | Rel-15 | F | NR_newRAT-Perf | not pursued |
| R4-2100390 | CR for TS 37.145-2: Correction on definition for the out-of-band CLTA(Rel-16) | CATT | 37.145-2 | 0272 | - | Rel-16 | A | NR_newRAT-Perf | withdrawn |
| R4-2100391 | CR for TS 37.145-2: Correction on definition for the out-of-band CLTA(Rel-17) | CATT | 37.145-2 | 0273 | - | Rel-17 | A | NR_newRAT-Perf | withdrawn |
| R4-2100539 | CR for 37.145-2: Corrections related to Band 24 regulatory updates | Ligado Networks | 37.145-2 | 0274 | - | Rel-16 | A | LTE_B24_mod-Perf | agreed |
| R4-2100540 | CR for 37.145-2: Corrections related to Band 24 regulatory updates | Ligado Networks | 37.145-2 | 0275 | - | Rel-17 | A | LTE_B24_mod-Perf | agreed |
| R4-2101733 | TS 37.145-2: Tx spurious limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U) | Ericsson | 37.145-2 | 0276 | - | Rel-16 | B | NR_unlic-Perf | postponed |
| R4-2101734 | TS 37.145-2: Tx spurious limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U) | Ericsson | 37.145-2 | 0277 | - | Rel-17 | A | NR_unlic-Perf | withdrawn |
| R4-2102144 | TS 37.145-2: Rx blocking limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U) | Ericsson | 37.145-2 | 0278 | - | Rel-16 | B | NR_unlic-Perf | postponed |
| R4-2102145 | TS 37.145-2: RX blocking limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U) | Ericsson | 37.145-2 | 0279 | - | Rel-17 | A | NR_unlic-Perf | withdrawn |
| R4-2102426 | CR to TS 37.145-2: Corrections to conformance requirements, Rel-15 | Huawei | 37.145-2 | 0280 | - | Rel-15 | F | TEI15, AAS_BS_LTE_UTRA-Perf | revised |
| R4-2103792 | CR to TS 37.145-2: Corrections to conformance requirements, Rel-15 | Huawei | 37.145-2 | 0280 | 1 | Rel-15 | F | TEI15, AAS_BS_LTE_UTRA-Perf | agreed |
| R4-2102427 | CR to TS 37.145-2: Corrections to conformance requirements, Rel-16 | Huawei | 37.145-2 | 0281 | - | Rel-16 | A | TEI15, AAS_BS_LTE_UTRA-Perf | agreed |
| R4-2102428 | CR to TS 37.145-2: Corrections to conformance requirements, Rel-17 | Huawei | 37.145-2 | 0282 | - | Rel-17 | A | TEI15, AAS_BS_LTE_UTRA-Perf | agreed |
| R4-2102431 | CR to TS 37.145-2 - Update CLTA definition, Rel-15 | Huawei | 37.145-2 | 0283 | - | Rel-15 | F | TEI15, AAS_BS_LTE_UTRA-Perf | revised |
| R4-2103788 | CR to TS 37.145-2 - Update CLTA definition, Rel-15 | Huawei, CATT, Nokia | 37.145-2 | 0283 | 1 | Rel-15 | F | TEI15, AAS_BS_LTE_UTRA-Perf | agreed |
| R4-2102432 | CR to TS 37.145-2 - Update CLTA definition, Rel-16 | Huawei, CATT, Nokia | 37.145-2 | 0284 | - | Rel-16 | A | TEI15, AAS_BS_LTE_UTRA-Perf | agreed |
| R4-2102433 | CR to TS 37.145-2 - Update CLTA definition, Rel-17 | Huawei, CATT, Nokia | 37.145-2 | 0285 | - | Rel-17 | A | TEI15, AAS_BS_LTE_UTRA-Perf | agreed |
| R4-2102567 | CR to TS 37.145-2: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16 | Huawei, China Unicom | 37.145-2 | 0286 | - | Rel-16 | F | TEI16, AASenh_BS_LTE_UTRA-Perf, MSR_GSM_UTRA_LTE_NR-Perf | revised |
| R4-2103784 | CR to TS 37.145-2: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-16 | Huawei, China Unicom | 37.145-2 | 0286 | 1 | Rel-16 | F | TEI16, AASenh_BS_LTE_UTRA-Perf, MSR_GSM_UTRA_LTE_NR-Perf | agreed |
| R4-2102568 | CR to TS 37.145-2: Introduction of new BS capability set for NR+EUTRA+UTRA, Rel-17 | Huawei, China Unicom | 37.145-2 | 0287 | - | Rel-17 | A | TEI16, AASenh_BS_LTE_UTRA-Perf, MSR_GSM_UTRA_LTE_NR-Perf | agreed |
| R4-2102859 | CR to 37.145-2 on OBUE table headings and applicability | Ericsson | 37.145-2 | 0288 | - | Rel-15 | F | TEI15 | revised |
| R4-2103887 | CR to 37.145-2 on OBUE table headings and applicability | Ericsson | 37.145-2 | 0288 | 1 | Rel-15 | F | TEI15 | agreed |
| R4-2102860 | CR to 37.145-2 on OBUE table headings and applicability | Ericsson | 37.145-2 | 0289 | - | Rel-16 | A | TEI15 | withdrawn |
| R4-2102861 | CR to 37.145-2 on OBUE table headings and applicability | Ericsson | 37.145-2 | 0290 | - | Rel-17 | A | TEI15 | withdrawn |
| R4-2102895 | CR to TS 37.145-2: Updating the orthogonal cut procedure | Nokia, Nokia Shanghai Bell | 37.145-2 | 0291 | - | Rel-15 | F | NR_newRAT-Perf | not pursued |
| R4-2102896 | CR to TS 37.145-2: Updating the orthogonal cut procedure | Nokia, Nokia Shanghai Bell | 37.145-2 | 0292 | - | Rel-16 | A | NR_newRAT-Perf | withdrawn |
| R4-2100150 | TR 37.716-21-11: Addition of missing lower order fallbacks | Nokia, AT&T | 37.716-21-11 | 0001 | - | Rel-16 | B | TEI16 | agreed |
| R4-2102901 | CR to TR 37.941: Updating the orthogonal cut procedure | Nokia, Nokia Shanghai Bell | 37.941 | 0025 | - | Rel-15 | F | OTA_BS_testing-Perf | postponed |
| R4-2102902 | CR to TR 37.941: Updating the orthogonal cut procedure | Nokia, Nokia Shanghai Bell | 37.941 | 0026 | - | Rel-16 | A | OTA_BS_testing-Perf | withdrawn |
| R4-2100099 | Introduce NR SUL bands to PC3 UL-MIMO configuration | CMCC | 38.101-1 | 0587 | - | Rel-17 | B | NR_bands_UL_MIMO_PC3_R17 | not pursued |
| R4-2100112 | PC1 and PC3 Updates for Band n14 | AT&T | 38.101-1 | 0588 | - | Rel-16 | F | NR_n14-Core | revised |
| R4-2103400 | PC1 and PC3 Updates for Band n14 | AT&T | 38.101-1 | 0588 | 1 | Rel-16 | F | NR_n14-Core | not concluded |
| R4-2100119 | PC1 and PC3 Updates for Band n14 | AT&T | 38.101-1 | 0589 | - | Rel-17 | A | NR_n14-Core | not treated |
| R4-2100121 | CR to TS 38.101-1 on optimization on delta TIB and RIB for band combinations (Rel-16) | ZTE Corporation | 38.101-1 | 0590 | - | Rel-16 | F | TEI16 | not pursued |
| R4-2100122 | CR to TS 38.101-1 on optimization on delta TIB and RIB for band combinations (Rel-17) | ZTE Corporation | 38.101-1 | 0591 | - | Rel-17 | A | TEI16 | withdrawn |
| R4-2100133 | CR introduction of 30 MHz for n48 | Nokia, Dish Network, Skyworks Inc | 38.101-1 | 0592 | - | Rel-17 | B | NR_bands_R17_BWs-Core | not pursued |
| R4-2100136 | 38.101 Void clean up R16 | Nokia, Nokia Shanghai Bell | 38.101-1 | 0593 | - | Rel-16 | F | NR_newRAT-Core | revised |
| R4-2103343 | 38.101 Void clean up R16 | Nokia, Nokia Shanghai Bell | 38.101-1 | 0593 | 1 | Rel-16 | F | NR_newRAT-Core | agreed |
| R4-2100137 | 38.101 Void clean up R17 | Nokia, Nokia Shanghai Bell | 38.101-1 | 0594 | - | Rel-17 | A | NR_newRAT-Core | revised |
| R4-2103344 | 38.101 Void clean up R17 | Nokia, Nokia Shanghai Bell | 38.101-1 | 0594 | 1 | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2100139 | CR REL15 on ambiguity in deciding TL,C | Nokia, Nokia Shanghai Bell | 38.101-1 | 0595 | - | Rel-15 | F | NR_newRAT-Core | not pursued |
| R4-2100140 | CR REL16 on ambiguity in deciding TL,C | Nokia, Nokia Shanghai Bell | 38.101-1 | 0596 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2100141 | CR REL17 on ambiguity in deciding TL,C | Nokia, Nokia Shanghai Bell | 38.101-1 | 0597 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2100149 | TS 38.101-3: Addition of missing lower order fallbacks R17 | Nokia, AT&T | 38.101-1 | 0598 | - | Rel-17 | A | TEI16 | withdrawn |
| R4-2100160 | Introduction of specific Pcmax requirements for inter-band CA category A-B combos | InterDigital Communications | 38.101-1 | 0599 | - | Rel-16 | C | NR_RF_FR1-Core | not pursued |
| R4-2100162 | CA_n7B_REFSENS_CatF_CR | Qualcomm Incorporated | 38.101-1 | 0600 | - | Rel-16 | F | NR_RF_FR1-Core | agreed |
| R4-2100163 | CR_CatF_n47_AMPR_correction | Qualcomm Incorporated | 38.101-1 | 0601 | - | Rel-16 | F | TEI16 | not pursued |
| R4-2100164 | IBE_mask_almost_contiguous_CR_rel15 | Qualcomm Incorporated | 38.101-1 | 0602 | - | Rel-15 | F | NR_newRAT-Core | not pursued |
| R4-2100165 | IBE_mask_almost_contiguous_CR_rel16-mirror | Qualcomm Incorporated | 38.101-1 | 0603 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2100246 | CR for TS 38.101-1 introduction of NR band n24 | Ligado Networks, Nokia, Skyworks Solutions Inc. | 38.101-1 | 0604 | - | Rel-17 | B | NR_band_n24-Core | revised |
| R4-2103210 | CR for TS 38.101-1 introduction of NR band n24 | Ligado Networks, Nokia, Skyworks Solutions Inc. | 38.101-1 | 0604 | 1 | Rel-17 | B | NR_band_n24-Core | revised |
| R4-2103378 | CR for TS 38.101-1 introduction of NR band n24 | Ligado Networks, Nokia, Skyworks Solutions Inc. | 38.101-1 | 0604 | 2 | Rel-17 | B | NR_band_n24-Core | agreed |
| R4-2100280 | CR on editorial correction on V2X operation in TS38.101-1 in Rel-16 | LG Electronics France | 38.101-1 | 0605 | - | Rel-16 | F | 5G_V2X_NRSL-Core | revised |
| R4-2103140 | CR on editorial correction on V2X operation in TS38.101-1 in Rel-16 | LG Electronics France | 38.101-1 | 0605 | 1 | Rel-16 | F | 5G_V2X_NRSL-Core | agreed |
| R4-2100281 | CR on editorial correction on V2X operation in TS38.101-1 in Rel-17 | LG Electronics France | 38.101-1 | 0606 | - | Rel-17 | A | 5G_V2X_NRSL-Core | agreed |
| R4-2100293 | CR on Introduction of completed SUL band combinations into TS 38.101-1 | Huawei, HiSilicon | 38.101-1 | 0607 | - | Rel-17 | B | NR_SUL_combos_R17-Core | agreed |
| R4-2100297 | CR on Introduction of completed 5 bands inter-band CA into TS 38.101-1 | Huawei, HiSilicon | 38.101-1 | 0608 | - | Rel-17 | B | NR_CADC_R17_5BDL_xBUL-Core | withdrawn |
| R4-2100334 | CR to 38101-1 on introducing new SUL band n99 | Huawei,HiSilicon | 38.101-1 | 0609 | - | Rel-17 | B | NR_SUL_UL_n24-Core | revised |
| R4-2103211 | CR to 38101-1 on introducing new SUL band n99 | Huawei,HiSilicon | 38.101-1 | 0609 | 1 | Rel-17 | B | NR_SUL_UL_n24-Core | revised |
| R4-2103351 | CR to 38101-1 on introducing new SUL band n99 | Huawei,HiSilicon | 38.101-1 | 0609 | 2 | Rel-17 | B | NR_SUL_UL_n24-Core | agreed |
| R4-2100392 | CR for TS38 101-1 Rel-15 Correction for definition of P-MPR | CATT | 38.101-1 | 0610 | - | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2100393 | CR for TS38 101-1 Rel-16 Correction for definition of P-MPR | CATT | 38.101-1 | 0611 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2100394 | CR for TS38 101-1 Rel-17 Correction for definition of P-MPR | CATT | 38.101-1 | 0612 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2100395 | CR for TS38 101-1 Rel-16 Correction of condition for MPR and delta MPR | CATT | 38.101-1 | 0613 | - | Rel-16 | F | NR_newRAT-Core | revised |
| R4-2103122 | CR for TS38 101-1 Rel-16 Correction of condition for MPR and delta MPR | CATT | 38.101-1 | 0613 | 1 | Rel-16 | F | NR_newRAT-Core | revised |
| R4-2103389 | CR for TS38 101-1 Rel-16 Correction of condition for MPR and delta MPR | CATT | 38.101-1 | 0613 | 2 | Rel-16 | F | NR_newRAT-Core | agreed |
| R4-2100396 | CR for TS38 101-1 Rel-17 Correction of condition for MPR and delta MPR | CATT | 38.101-1 | 0614 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2100404 | CR for TS 38.101-1, General corrections for NR V2X | CATT | 38.101-1 | 0615 | - | Rel-16 | F | 5G_V2X_NRSL-Core | not pursued |
| R4-2100413 | CR for TS 38.101-1, Introduce new band combination of V2X_n41A-n47A | CATT | 38.101-1 | 0616 | - | Rel-17 | B | NR_LTE_V2X_PC5_combos-Core | revised |
| R4-2103200 | CR for TS 38.101-1, Introduce new band combination of V2X_n41A-n47A | CATT | 38.101-1 | 0616 | 1 | Rel-17 | B | NR_LTE_V2X_PC5_combos-Core | agreed |
| R4-2100494 | CR on Introducing NR inter-band CA for 3DL Bands and 1UL band for 38.101-1 | CATT | 38.101-1 | 0617 | - | Rel-17 | B | NR_CA_R17_3BDL_1BUL-Core | agreed |
| R4-2100499 | CR for TS 38.101-1, General corrections for NR V2X | CATT | 38.101-1 | 0618 | - | Rel-17 | A | 5G_V2X_NRSL-Core | withdrawn |
| R4-2100512 | Corrections of NR-U wideband operation intra-carrier guard bands | Apple Inc. | 38.101-1 | 0619 | - | Rel-16 | F | NR_unlic | not pursued |
| R4-2100513 | Corrections of NR-U wideband operation intra-carrier guard bands | Apple Inc. | 38.101-1 | 0620 | - | Rel-17 | A | NR_unlic | withdrawn |
| R4-2100594 | Introduction of Tx diversity in tor 38101-1 Cat-B CR 38.101-1 | Qualcomm Incorporated | 38.101-1 | 0621 | - | Rel-16 | B | TEI16 | withdrawn |
| R4-2100595 | Introduction of Tx diversity in tor 38101-1 Cat-F CR 38.101-1 | Qualcomm Incorporated | 38.101-1 | 0622 | - | Rel-17 | A | TEI16 | withdrawn |
| R4-2100596 | UL Switching and coherent UL MIMO Cat-F CR 38.101-1 | Qualcomm Incorporated | 38.101-1 | 0623 | - | Rel-16 | F | NR_RF_FR1-Core | not pursued |
| R4-2100597 | UL Switching and coherent UL MIMO Cat-A CR 38.101-1 | Qualcomm Incorporated | 38.101-1 | 0624 | - | Rel-17 | A | NR_RF_FR1-Core | withdrawn |
| R4-2100617 | Revision of inter-band V2X con-currency table for V2X_n39A-n47A and V2X_n40A-n47A | Qualcomm Incorporated | 38.101-1 | 0625 | - | Rel-17 | F | NR_LTE_V2X_PC5_combos-Core | agreed |
| R4-2100683 | Correction on supported channel bandwidth for CA_n39-n41-n79 | CATT, CMCC | 38.101-1 | 0626 | - | Rel-16 | F | NR_CA_R16_3BDL_1BUL-Core | noted |
| R4-2100684 | Correction on supported channel bandwidth for CA_n39-n41-n79 | CATT, CMCC | 38.101-1 | 0627 | - | Rel-17 | A | NR_CA_R16_3BDL_1BUL-Core | withdrawn |
| R4-2100791 | Switching time mask for 2Tx-2Tx switching between two carriers and 1Tx-2Tx/2Tx-2Tx switching between two bands in Rel-17 | China Telecom | 38.101-1 | 0628 | - | Rel-17 | B | NR_RF_FR1_enh-Core | revised |
| R4-2103236 | Switching time mask for 2Tx-2Tx switching between two carriers and 1Tx-2Tx/2Tx-2Tx switching between two bands in Rel-17 | China Telecom | 38.101-1 | 0628 | 1 | Rel-17 | B | NR_RF_FR1_enh-Core | agreed |
| R4-2100792 | CR for TS 38.101-1: Correction on 1Tx-2Tx switching between two uplink carriers (Rel-16) | China Telecom | 38.101-1 | 0629 | - | Rel-16 | F | NR_RF_FR1-Core | agreed |
| R4-2100793 | CR for TS 38.101-1: Correction on 1Tx-2Tx switching between two uplink carriers (Rel-17) | China Telecom | 38.101-1 | 0630 | - | Rel-17 | F | NR_RF_FR1-Core | agreed |
| R4-2100799 | CR on introducing NR SUL bands n80 to UL-MIMO configuration | CMCC | 38.101-1 | 0631 | - | Rel-17 | B | NR_RF_FR1_enh-Core | agreed |
| R4-2100846 | CR for 38.101-1: Update of missing fallback NR-DC combinations Rel-16 | SoftBank Corp., ZTE Corporation | 38.101-1 | 0632 | - | Rel-16 | F | TEI16 | agreed |
| R4-2100847 | CR for 38.101-1: Update of missing fallback NR-DC combinations Rel-17 | SoftBank Corp., ZTE Corporation | 38.101-1 | 0633 | - | Rel-17 | A | TEI16 | agreed |
| R4-2100876 | CR for 38.101-1: Update of simultaneous Rx/Tx capability for some NR CA band combinations Rel-16 | SoftBank Corp. | 38.101-1 | 0634 | - | Rel-16 | F | TEI16 | not pursued |
| R4-2100877 | CR for 38.101-1: Update of simultaneous Rx/Tx capability for some NR CA band combinations Rel-17 | SoftBank Corp. | 38.101-1 | 0635 | - | Rel-17 | A | TEI16 | withdrawn |
| R4-2100990 | CR on introduction of completed NR CA/DC combs with 4DL/2UL within FR1 | Samsung | 38.101-1 | 0636 | - | Rel-17 | B | NR_CADC_R17_4BDL_2BUL | revised |
| R4-2103255 | CR on introduction of completed NR CA/DC combs with 4DL/2UL within FR1 | Samsung | 38.101-1 | 0636 | 1 | Rel-17 | B | NR_CADC_R17_4BDL_2BUL | agreed |
| R4-2101106 | CR for 38.101-1 Rel16 corrections on exception requirements on out-of-band blocking for inter-band CA | Xiaomi | 38.101-1 | 0637 | - | Rel-16 | F | NR_newRAT-Core | revised |
| R4-2103347 | CR for 38.101-1 Rel16 corrections on exception requirements on out-of-band blocking for inter-band CA | Xiaomi | 38.101-1 | 0637 | 1 | Rel-16 | F | NR_newRAT-Core | agreed |
| R4-2101107 | CR for 38.101-1 Rel17 corrections on exception requirements on out-of-band blocking for inter-band CA | Xiaomi | 38.101-1 | 0638 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2101285 | CR to 38.101-1 (Rel-16) fall back behaviors | Intel Corporation | 38.101-1 | 0639 | - | Rel-16 | F | NR_RF_FR1-Core | not pursued |
| R4-2101286 | CR to 38.101-1 (Rel-17) fall back behaviors | Intel Corporation | 38.101-1 | 0640 | - | Rel-17 | A | NR_RF_FR1-Core | withdrawn |
| R4-2101484 | CR on introduction of shorter Transient Period Capability | Qualcomm Incorporated | 38.101-1 | 0641 | - | Rel-16 | B | TEI16 | revised |
| R4-2103152 | CR on introduction of shorter Transient Period Capability | Qualcomm Incorporated | 38.101-1 | 0641 | 1 | Rel-16 | B | TEI16 | agreed |
| R4-2101487 | CR on introduction of shorter Transient Period Capability | Qualcomm Incorporated | 38.101-1 | 0642 | - | Rel-17 | A | TEI16 | agreed |
| R4-2101713 | Correction to applicability of simultaneous RX/TX | Ericsson | 38.101-1 | 0643 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103119 | Correction to applicability of simultaneous RX/TX | Ericsson | 38.101-1 | 0643 | 1 | Rel-15 | F | NR_newRAT-Core | not pursued |
| R4-2101714 | Correction to applicability of simultaneous RX/TX | Ericsson | 38.101-1 | 0644 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2101715 | Correction to the lower limit of Pumax | Ericsson | 38.101-1 | 0645 | - | Rel-15 | F | NR_newRAT-Core | not pursued |
| R4-2101716 | Correction to the lower limit of Pumax | Ericsson | 38.101-1 | 0646 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2101720 | Applicability of minimum requirements for shared spectrum access | Ericsson | 38.101-1 | 0647 | - | Rel-16 | F | NR_unlic-Core | revised |
| R4-2103135 | Applicability of minimum requirements for shared spectrum access | Ericsson | 38.101-1 | 0647 | 1 | Rel-16 | F | NR_unlic-Core | not pursued |
| R4-2101723 | Modification of Pcmax for UL CA with uplink Tx switching capability | Ericsson | 38.101-1 | 0648 | - | Rel-16 | F | TEI16 | revised |
| R4-2103159 | Modification of Pcmax for UL CA with uplink Tx switching capability | Ericsson | 38.101-1 | 0648 | 1 | Rel-16 | F | TEI16 | not pursued |
| R4-2101743 | CR on simultaneous Tx-Rx for CA and SUL | OPPO | 38.101-1 | 0649 | - | Rel-15 | F | NR_newRAT-Core | not pursued |
| R4-2101744 | CR on simultaneous Tx-Rx for CA and SUL (R16 mirror CR) | OPPO | 38.101-1 | 0650 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2101745 | CR on simultaneous Tx-Rx for CA and SUL (R17 mirror CR) | OPPO | 38.101-1 | 0651 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2101807 | CR on spurious emission about UE co-existence between band n40 and n41(Rel-16) | Huawei, HiSilicon, CMCC | 38.101-1 | 0652 | - | Rel-16 | F | NR_RF_FR1-Core | not pursued |
| R4-2101808 | CR on spurious emission about UE co-existence between band n40 and n41(Rel-17) | Huawei, HiSilicon, CMCC | 38.101-1 | 0653 | - | Rel-17 | A | NR_RF_FR1-Core | withdrawn |
| R4-2101809 | CR for 38.101-1 to introduce PC2 for n40 UL MIMO(Rel-16) | Huawei, HiSilicon, Reliance Jio | 38.101-1 | 0654 | - | Rel-16 | F | TEI16 | not pursued |
| R4-2101810 | CR for 38.101-1 to introduce PC2 for n40 UL MIMO(Rel-17) | Huawei, HiSilicon, Reliance Jio | 38.101-1 | 0655 | - | Rel-17 | A | TEI16 | withdrawn |
| R4-2101852 | CR to TS 38.101-1 Operating bands for DC | ZTE Wistron Telecom AB | 38.101-1 | 0656 | - | Rel-16 | F | TEI16 | not pursued |
| R4-2101886 | CR introduction completed band combinations Rel-17 NR Intra-band - | Ericsson | 38.101-1 | 0657 | - | Rel-17 | B | NR_CA_R17_Intra-Core | agreed |
| R4-2101889 | CR introduction completed band combinations NR Inter-band 4 bands CA - | Ericsson | 38.101-1 | 0658 | - | Rel-17 | B | NR_CA_R17_4BDL_1BUL | not concluded |
| R4-2101939 | CR for 38.101-1 to add missing spurious emissions for band n38 UE co-existence (Rel-16) | Huawei, HiSilicon, DT | 38.101-1 | 0659 | - | Rel-16 | F | TEI16 | agreed |
| R4-2101940 | CR for 38.101-1 to add missing spurious emissions for band n38 UE co-existence (Rel-17) | Huawei, HiSilicon, DT | 38.101-1 | 0660 | - | Rel-17 | A | TEI16 | agreed |
| R4-2101947 | Simplification of n70 | Dish Network | 38.101-1 | 0661 | - | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2101970 | CR to TS 38.101-1: system parameters maintenance for NR-U | ZTE Corporation | 38.101-1 | 0662 | - | Rel-16 | F | NR_unlic-Core | revised |
| R4-2103138 | CR to TS 38.101-1: system parameters maintenance for NR-U | ZTE Corporation | 38.101-1 | 0662 | 1 | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2101971 | CR to TS 38.101-1: system parameters maintenance for NR-U | ZTE Corporation | 38.101-1 | 0663 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2101988 | Simplification of n70 | Dish Network | 38.101-1 | 0664 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2101992 | Simplification of n70 | Dish Network | 38.101-1 | 0665 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2102142 | Introduction of 35MHz and 45 MHz bandwidths to TS38.101-1 | ZTE Corporation | 38.101-1 | 0666 | - | Rel-17 | B | NR_FR1_35MHz_45MHz_BW-Core | withdrawn |
| R4-2102143 | CR to TS38.101-1: Correction on applicability of minimum requirements | ZTE Corporation | 38.101-1 | 0667 | - | Rel-15 | F | NR_newRAT-Core | withdrawn |
| R4-2102152 | CR for 38.101-1: Add CA_n25A-n41(2A)-n71A which was missing in the CR implementation | T-Mobile USA | 38.101-1 | 0668 | - | Rel-16 | F | NR_CA_R16_3BDL_1BUL-Core | agreed |
| R4-2102153 | CR for 38.101-1: Add CA_n25A-n41(2A)-n71A which was missing in the CR implementation | T-Mobile USA | 38.101-1 | 0669 | - | Rel-17 | A | NR_CA_R16_3BDL_1BUL-Core | agreed |
| R4-2102165 | Big CR to TS 38.101-1 - New CBW Basket WI | Ericsson | 38.101-1 | 0670 | - | Rel-17 | F | NR_bands_R17_BWs | revised |
| R4-2103180 | Big CR to TS 38.101-1 - New CBW Basket WI | Ericsson | 38.101-1 | 0670 | 1 | Rel-17 | F | NR_bands_R17_BWs | not concluded |
| R4-2102193 | Introduction of 35MHz and 45 MHz bandwidths to TS38.101-1 | ZTE Corporation | 38.101-1 | 0671 | - | Rel-17 | B | NR_FR1_35MHz_45MHz_BW-Core | revised |
| R4-2103189 | Introduction of 35MHz and 45 MHz bandwidths to TS38.101-1 | ZTE Corporation | 38.101-1 | 0671 | 1 | Rel-17 | B | NR_FR1_35MHz_45MHz_BW-Core | noted |
| R4-2102194 | CR to TS38.101-1: Correction on applicability of minimum requirements | ZTE Corporation | 38.101-1 | 0672 | - | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2102195 | CR to TS38.101-1: Correction on applicability of minimum requirements | ZTE Corporation | 38.101-1 | 0673 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2102196 | CR to TS38.101-1: Correction on applicability of minimum requirements | ZTE Corporation | 38.101-1 | 0674 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2102197 | CR to TS38.101-1: Correction on the Aggregated Channel Bandwidth | ZTE Corporation | 38.101-1 | 0675 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103116 | CR to TS38.101-1: Correction on the Aggregated Channel Bandwidth | ZTE Corporation | 38.101-1 | 0675 | 1 | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2102198 | CR to TS38.101-1: Correction on the Aggregated Channel Bandwidth | ZTE Corporation | 38.101-1 | 0676 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2102199 | CR to TS38.101-1: Correction on the Aggregated Channel Bandwidth | ZTE Corporation | 38.101-1 | 0677 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2102203 | CR to TS38.101-1: Correction on configured transmitted power requirement | ZTE Corporation | 38.101-1 | 0678 | - | Rel-16 | F | NR_newRAT-Core | agreed |
| R4-2102204 | CR to TS38.101-1: Correction on configured transmitted power requirement | ZTE Corporation | 38.101-1 | 0679 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2102319 | Rel-17 CR 38.101-1 for improvements Intra-band tables | Ericsson | 38.101-1 | 0680 | - | Rel-17 | B | NR_CA_R17_Intra | revised |
| R4-2103033 | Rel-17 CR 38.101-1 for improvements Intra-band tables | Ericsson | 38.101-1 | 0680 | 1 | Rel-17 | B | NR_CA_R17_Intra | agreed |
| R4-2102320 | Rel-17 CR 38.101-1 for corrections NR CA 2, 3 and 4 band configuration tables | Ericsson | 38.101-1 | 0681 | - | Rel-17 | F | NR_CADC_R17_2BDL_xBUL, NR_CADC_R17_3BDL_2BUL, NR_CADC_R17_4BDL_2BUL | agreed |
| R4-2102376 | CR for TS 38.101-1 correction CR for simultaneous TxRx operation (R15) | Huawei, HiSilicon | 38.101-1 | 0682 | - | Rel-15 | F | NR_newRAT-Core | not pursued |
| R4-2102377 | CR for TS 38.101-1: correction CR for simultaneous Tx/Rx operation (R16) | Huawei, HiSilicon | 38.101-1 | 0683 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2102378 | CR for TS 38.101-3 correction CR for simultaneous TxRx operation (R15) | Huawei, HiSilicon | 38.101-1 | 0684 | - | Rel-15 | F | NR_newRAT-Core | not pursued |
| R4-2102382 | CR for 38.101-1 correction of NR V2X FRC parameter | Huawei, HiSilicon | 38.101-1 | 0685 | - | Rel-16 | F | 5G_V2X_NRSL-Core | not pursued |
| R4-2102384 | CR for TS 38.101-1: TxD requirements | Huawei, HiSilicon | 38.101-1 | 0686 | - | Rel-16 | B | TEI16 | revised |
| R4-2103156 | CR for TS 38.101-1: TxD requirements | Huawei, HiSilicon | 38.101-1 | 0686 | 1 | Rel-16 | B | TEI16 | endorsed |
| R4-2102386 | CR for TS 38.101-1: correction of Pi/2 BPSK | Huawei, HiSilicon | 38.101-1 | 0687 | - | Rel-16 | F | NR_eMIMO-Core | not pursued |
| R4-2102394 | CR for TS 38.101-1 Introduce NR SUL bands to PC3 UL-MIMO configuration | Huawei, HiSilicon | 38.101-1 | 0688 | - | Rel-17 | B | NR_bands_UL_MIMO_PC3_R17-Core | agreed |
| R4-2102408 | Missing parent clause for NR-DC PCMAX | Qualcomm Incorporated | 38.101-1 | 0689 | - | Rel-16 | D | NR_newRAT-Core | revised |
| R4-2103142 | Missing parent clause for NR-DC PCMAX | Qualcomm Incorporated | 38.101-1 | 0689 | 1 | Rel-16 | D | NR_newRAT-Core | agreed |
| R4-2102409 | Missing parent clause for NR-DC PCMAX | Qualcomm Incorporated | 38.101-1 | 0690 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2102410 | Corrections to PCMAX for UL CA | Qualcomm Incorporated | 38.101-1 | 0691 | - | Rel-16 | F | NR_newRAT-Core | revised |
| R4-2103148 | Corrections to PCMAX for UL CA | Qualcomm Incorporated | 38.101-1 | 0691 | 1 | Rel-16 | F | NR_newRAT-Core | agreed |
| R4-2102411 | Corrections to PCMAX for UL CA | Qualcomm Incorporated | 38.101-1 | 0692 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2102510 | IBE requirement for almost contiguous allocations | Qualcomm Incorporated | 38.101-1 | 0693 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2102557 | CR for n47 AMPR - Cat A | Qualcomm Incorporated | 38.101-1 | 0694 | - | Rel-17 | A | TEI16 | withdrawn |
| R4-2102588 | CR CA_n7B REFSENS | Qualcomm Incorporated | 38.101-1 | 0695 | - | Rel-17 | A | NR_RF_FR1-Core | agreed |
| R4-2102595 | CR for TS 38.101-1: Cleanup for spurious emissions for UE co-existence table | Apple | 38.101-1 | 0696 | - | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2102597 | CR for TS 38.101-1: Correction to FR1 time mask for SRS antenna switching | Apple | 38.101-1 | 0697 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103123 | CR for TS 38.101-1: Correction to FR1 time mask for SRS antenna switching | Apple | 38.101-1 | 0697 | 1 | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103388 | CR for TS 38.101-1: Correction to FR1 time mask for SRS antenna switching | Apple | 38.101-1 | 0697 | 2 | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2102598 | CR for TS 38.101-1: Correction to FR1 time mask for SRS antenna switching | Apple | 38.101-1 | 0698 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2102599 | CR for TS 38.101-1: Correction to FR1 time mask for SRS antenna switching | Apple | 38.101-1 | 0699 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2102600 | CR for TS 38.101-1: Corrections to intra-band UL NC CA requirements | Apple | 38.101-1 | 0700 | - | Rel-16 | F | NR_RF_FR1-Core | revised |
| R4-2103149 | CR for TS 38.101-1: Corrections to intra-band UL NC CA requirements | Apple | 38.101-1 | 0700 | 1 | Rel-16 | F | NR_RF_FR1-Core | agreed |
| R4-2102601 | CR for TS 38.101-1: Corrections to intra-band UL NC CA requirements | Apple | 38.101-1 | 0701 | - | Rel-17 | A | NR_RF_FR1-Core | agreed |
| R4-2102602 | CR for TS 38.101-1: Cleanup for spurious emissions for UE co-existence table | Apple | 38.101-1 | 0702 | - | Rel-16 | F | TEI16 | revised |
| R4-2103160 | CR for TS 38.101-1: Cleanup for spurious emissions for UE co-existence table | Apple | 38.101-1 | 0702 | 1 | Rel-16 | F | TEI16 | agreed |
| R4-2102603 | CR for TS 38.101-1: Cleanup for spurious emissions for UE co-existence table | Apple | 38.101-1 | 0703 | - | Rel-17 | A | TEI16 | agreed |
| R4-2102606 | CR for TS 38.101-1: UE RF requirements table simplification | Apple | 38.101-1 | 0704 | - | Rel-17 | F | NR_FR1_35MHz_45MHz_BW-Core | not pursued |
| R4-2102658 | CR to 38.101-1: UL MIMO requirements update | Qualcomm Incorporated, Lenovo, Motorola Mobility | 38.101-1 | 0705 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103355 | CR to 38.101-1: UL MIMO requirements update | Qualcomm Incorporated, Lenovo, Motorola Mobility | 38.101-1 | 0705 | 1 | Rel-15 | F | NR_newRAT-Core | not pursued |
| R4-2102659 | CR to 38.101-1: UL MIMO requirements update | Qualcomm Incorporated, Lenovo, Motorola Mobility | 38.101-1 | 0706 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2102660 | CR to 38.101-1: UL MIMO requirements update | Qualcomm Incorporated, Lenovo, Motorola Mobility | 38.101-1 | 0707 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2102683 | CR for TS 38.101-1 Tx diversity requirements | Huawei, HiSilicon | 38.101-1 | 0708 | - | Rel-17 | B | TEI16 | withdrawn |
| R4-2102684 | CR on TS 38.101-1 time mask for shorter transient | Huawei, HiSilicon | 38.101-1 | 0709 | - | Rel-16 | F | NR_RF_FR1-Core | not pursued |
| R4-2103153 | CR on TS 38.101-1 time mask for shorter transient | Huawei, HiSilicon | 38.101-1 | 0709 | 1 | Rel-16 | F | NR_RF_FR1-Core | withdrawn |
| R4-2102685 | CR on TS 38.101-1 NS_49 | Huawei, HiSilicon | 38.101-1 | 0710 | - | Rel-16 | F | TEI16 | agreed |
| R4-2102688 | CR for FR2 FWA RF requirements | Huawei, HiSilicon | 38.101-1 | 0711 | - | Rel-17 | B | NR_FR2_FWA_Bn257_Bn258-Core | revised |
| R4-2103203 | CR for FR2 FWA RF requirements | Huawei, HiSilicon | 38.101-1 | 0711 | 1 | Rel-17 | B | NR_FR2_FWA_Bn257_Bn258-Core | agreed |
| R4-2102703 | CR for TS 38.101-1: correction CR for simultaneous Tx/Rx operation (R17) | Huawei, HiSilicon | 38.101-1 | 0712 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2102816 | CR on TS 38.101-1 NS_49 | Huawei, HiSilicon | 38.101-1 | 0713 | - | Rel-17 | A | TEI16 | agreed |
| R4-2102903 | CR on split band duplexer exceptions to non-default TX-RX separation | Qualcomm Incorporated | 38.101-1 | 0714 | - | Rel-15 | F | NR_FDD_bands_varduplex-Core | not pursued |
| R4-2102943 | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into TS 38.101-1 | ZTE Corporation | 38.101-1 | 0715 | - | Rel-17 | - | NR_CADC_R17_2BDL_xBUL | agreed |
| R4-2102945 | CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into TS 38.101-1 | ZTE Corporation | 38.101-1 | 0716 | - | Rel-17 | - | NR_CADC_R17_3BDL_2BUL | agreed |
| R4-2103147 | Introduction of specific Pcmax requirements for inter-band CA category A-B combos | InterDigital Communications, Charter Communications | 38.101-1 | 0717 | - | Rel-17 | B | NR_CADC_R17_2BDL_xBUL-core | agreed |
| R4-2103350 | Corrections to PCMAX for UL CA | Qualcomm Incorporated | 38.101-1 | 0718 | - | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2103136 | Applicability of minimum requirements for shared spectrum access | Ericsson | 38.101-1 | 0719 | - | Rel-17 | - | NR_unlic-Core | withdrawn |
| R4-2100085 | Clarification on NS_203 support by n258 | Nokia, Nokia Shanghai Bell | 38.101-2 | 0313 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103125 | Removal of a remaining NS_201 related requirement | Nokia, Nokia Shanghai Bell | 38.101-2 | 0313 | 1 | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103370 | Removal of a remaining NS_201 related requirement | Nokia, Nokia Shanghai Bell | 38.101-2 | 0313 | 2 | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2100086 | Removal of a remaining NS_201 related requirement | Nokia, Nokia Shanghai Bell | 38.101-2 | 0314 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2100087 | Removal of a remaining NS_201 related requirement | Nokia, Nokia Shanghai Bell | 38.101-2 | 0315 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2100123 | CR to TS 38.101-2 on optimization on delta RIB for inter-band CA (Rel-16) | ZTE Corporation | 38.101-2 | 0316 | - | Rel-16 | F | TEI16 | not pursued |
| R4-2100124 | CR to TS 38.101-2 on optimization on delta RIB for inter-band CA (Rel-17) | ZTE Corporation | 38.101-2 | 0317 | - | Rel-17 | A | TEI16 | withdrawn |
| R4-2100127 | CR to TS 38.101-2 on correction to intra-band non-contiguous CA configurations (Rel-16) | ZTE Corporation | 38.101-2 | 0318 | - | Rel-16 | F | TEI16 | agreed |
| R4-2100128 | CR to TS 38.101-2 on correction to intra-band non-contiguous CA configurations (Rel-17) | ZTE Corporation | 38.101-2 | 0319 | - | Rel-17 | A | TEI16 | agreed |
| R4-2100586 | P_min correction and P_cmax CA correction to apply from all cells | Qualcomm Incorporated | 38.101-2 | 0320 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103127 | P_min correction and P_cmax CA correction to apply from all cells | Qualcomm Incorporated | 38.101-2 | 0320 | 1 | Rel-15 | F | NR_newRAT-Core | not pursued |
| R4-2100587 | P_min correction and P_cmax CA correction to apply from all cells | Qualcomm Incorporated | 38.101-2 | 0321 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2100588 | P_min correction and P_cmax CA correction to apply from all cells | Qualcomm Incorporated | 38.101-2 | 0322 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2100589 | P_cmax P_IBE wording refinement and terminology improvement | Qualcomm Incorporated | 38.101-2 | 0323 | - | Rel-16 | F | NR_RF_FR2_req_enh-Core | revised |
| R4-2103358 | P_cmax P_IBE wording refinement and terminology improvement | Qualcomm Incorporated | 38.101-2 | 0323 | 1 | Rel-16 | F | NR_RF_FR2_req_enh-Core | agreed |
| R4-2100590 | P_cmax P_IBE wording refinement and termonology improvement | Qualcomm Incorporated | 38.101-2 | 0324 | - | Rel-17 | A | NR_RF_FR2_req_enh-Core | agreed |
| R4-2100750 | Introduction of n262 UE RF requirements | Nokia, Nokia Shanghai Bell | 38.101-2 | 0325 | - | Rel-17 | B | NR_47GHz_band-Core | revised |
| R4-2103205 | Introduction of n262 UE RF requirements | Nokia, Nokia Shanghai Bell | 38.101-2 | 0325 | 1 | Rel-17 | B | NR_47GHz_band-Core | revised |
| R4-2103392 | Introduction of n262 UE RF requirements | Nokia, Nokia Shanghai Bell, Apple | 38.101-2 | 0325 | 2 | Rel-17 | B | NR_47GHz_band-Core | endorsed |
| R4-2101287 | CR to 38.101-2 (Rel-16) SSB based beam correspondence side condition | Intel Corporation | 38.101-2 | 0326 | - | Rel-16 | F | NR_RF_FR2_req_enh-Core | not pursued |
| R4-2101288 | CR to 38.101-2 (Rel-17) SSB based beam correspondence side condition | Intel Corporation | 38.101-2 | 0327 | - | Rel-17 | A | NR_RF_FR2_req_enh-Core | withdrawn |
| R4-2101724 | Correction to modified MPR behaviour | Ericsson | 38.101-2 | 0328 | - | Rel-16 | F | TEI16 | revised |
| R4-2103167 | Correction to modified MPR behaviour | Ericsson | 38.101-2 | 0328 | 1 | Rel-16 | F | TEI16 | not pursued |
| R4-2101739 | CR on FR2 equal PSD in UL CA | OPPO | 38.101-2 | 0329 | - | Rel-15 | F | NR_newRAT-Core | not pursued |
| R4-2101740 | CR on FR2 equal PSD in UL CA (R16 mirror CR) | OPPO | 38.101-2 | 0330 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2101741 | CR on FR2 equal PSD in UL CA (R17 mirror CR) | OPPO | 38.101-2 | 0331 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2101887 | CR introduction completed band combinations Rel-17 NR Intra-band - | Ericsson | 38.101-2 | 0332 | - | Rel-17 | B | NR_CA_R17_Intra-Core | not concluded |
| R4-2102562 | CR to 38.101-2: correction on UL MIMO | Google Inc. | 38.101-2 | 0333 | - | Rel-16 | F | NR_newRAT-Core | agreed |
| R4-2102582 | CR to 38.101-2: correction on UL MIMO | Google Inc. | 38.101-2 | 0334 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2102664 | CR to 38.101-2 on beam correspondence | Qualcomm, Nokia, Nokia Shanghai Bell, Samsung, Verizon, NTT Docomo, Sony, Ericsson | 38.101-2 | 0335 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103126 | CR to 38.101-2 on beam correspondence | Qualcomm, Nokia, Nokia Shanghai Bell, Samsung, Verizon, NTT Docomo, Sony, Ericsson | 38.101-2 | 0335 | 1 | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2102665 | CR to 38.101-2 on beam correspondence | Qualcomm, Nokia, Nokia Shanghai Bell, Samsung, Verizon, NTT Docomo, Sony, Ericsson | 38.101-2 | 0336 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2102666 | CR to 38.101-2 on beam correspondence | Qualcomm, Nokia, Nokia Shanghai Bell, Samsung, Verizon, NTT Docomo, Sony, Ericsson | 38.101-2 | 0337 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2102670 | CR to 38.807: Update for FR2 PC5 | Qualcomm Incorporated | 38.101-2 | 0338 | - | Rel-15 | F | NR_FR2_FWA_Bn257_Bn258-Core | withdrawn |
| R4-2102671 | CR to 38.807: Update for FR2 PC5 | Qualcomm Incorporated | 38.101-2 | 0339 | - | Rel-16 | F | NR_FR2_FWA_Bn257_Bn258-Core | withdrawn |
| R4-2102672 | CR to 38.807: Update for FR2 PC5 | Qualcomm Incorporated | 38.101-2 | 0340 | - | Rel-17 | A | NR_FR2_FWA_Bn257_Bn258-Core | withdrawn |
| R4-2102677 | Frequency separation class clarification | Ericsson | 38.101-2 | 0341 | - | Rel-16 | F | NR_newRAT-Core | not pursued |
| R4-2102678 | Frequency separation class clarification | Ericsson | 38.101-2 | 0342 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2102716 | CR on FR2 intra-band UL CA | Huawei, HiSilicon | 38.101-2 | 0343 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103348 | CR on FR2 intra-band UL CA | Huawei, HiSilicon | 38.101-2 | 0343 | 1 | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2102718 | CR on FR2 inter-band DL CA CBM and IBM | Huawei, HiSilicon | 38.101-2 | 0344 | - | Rel-16 | F | NR_RF_FR2_req_enh2-Core | revised |
| R4-2103369 | CR on FR2 inter-band DL CA CBM and IBM | Huawei, HiSilicon | 38.101-2 | 0344 | 1 | Rel-16 | F | NR_RF_FR2_req_enh2-Core | revised |
| R4-2103405 | CR on FR2 inter-band DL CA CBM and IBM | Huawei, HiSilicon | 38.101-2 | 0344 | 2 | Rel-16 | F | NR_RF_FR2_req_enh2-Core | agreed |
| R4-2102815 | CR on FR2 intra-band UL CA | Huawei, HiSilicon | 38.101-2 | 0345 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2103391 | CR on FR2 intra-band UL CA | Huawei, HiSilicon | 38.101-2 | 0346 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2100082 | Big CR on introduction of completed PC2 for EN-DC with 1 LTE band + 1 NR TDD band | China Unicom | 38.101-3 | 0432 | - | Rel-17 | B | ENDC_UE_PC2_R17_NR_TDD | agreed |
| R4-2100125 | CR to TS 38.101-3 on optimization on delta TIB and RIB for inter-band EN-DC (Rel-16) | ZTE Corporation | 38.101-3 | 0433 | - | Rel-16 | F | TEI16 | not pursued |
| R4-2100126 | CR to TS 38.101-3 on optimization on delta TIB and RIB for inter-band EN-DC (Rel-17) | ZTE Corporation | 38.101-3 | 0434 | - | Rel-17 | A | TEI16 | withdrawn |
| R4-2100129 | CR to TS 38.101-3 on correction to hanging paragraph in the spec (Rel-16) | ZTE Corporation | 38.101-3 | 0435 | - | Rel-16 | F | TEI16 | revised |
| R4-2103161 | CR to TS 38.101-3 on correction to hanging paragraph in the spec (Rel-16) | ZTE Corporation | 38.101-3 | 0435 | 1 | Rel-16 | F | TEI16 | agreed |
| R4-2100130 | CR to TS 38.101-3 on correction to hanging paragraph in the spec (Rel-17) | ZTE Corporation | 38.101-3 | 0436 | - | Rel-17 | A | TEI16 | agreed |
| R4-2100148 | TS 38.101-3: Addition of missing lower order fallbacks R16 | Nokia, AT&T | 38.101-3 | 0437 | - | Rel-16 | B | TEI16 | revised |
| R4-2103162 | TS 38.101-3: Addition of missing lower order fallbacks R16 | Nokia, AT&T | 38.101-3 | 0437 | 1 | Rel-16 | B | TEI16 | agreed |
| R4-2100277 | Introduction CR on new NR DC LTE(xDL/1UL)+ NR(2DL/1UL) band combinations in Rel-17 | LG Electronics France | 38.101-3 | 0438 | - | Rel-17 | B | DC_R17_xBLTE_2BNR_yDL2UL-Core | agreed |
| R4-2100294 | CR on Introduction of completed SUL band combinations into TS 38.101-3 | Huawei, HiSilicon | 38.101-3 | 0439 | - | Rel-17 | B | NR_SUL_combos_R17-Core | not concluded |
| R4-2100405 | CR for TS 38.101-3, General corrections for NR V2X | CATT | 38.101-3 | 0440 | - | Rel-16 | F | 5G_V2X_NRSL-Core | revised |
| R4-2103141 | CR for TS 38.101-3, General corrections for NR V2X | CATT | 38.101-3 | 0440 | 1 | Rel-16 | F | 5G_V2X_NRSL-Core | agreed |
| R4-2100414 | CR for TS 38.101-3, Introduce new band combination of V2X_41A-n47A and V2X_n41A-47A | CATT | 38.101-3 | 0441 | - | Rel-17 | B | NR_LTE_V2X_PC5_combos-Core | revised |
| R4-2103201 | CR for TS 38.101-3, Introduce new band combination of V2X_41A-n47A and V2X_n41A-47A | CATT | 38.101-3 | 0441 | 1 | Rel-17 | B | NR_LTE_V2X_PC5_combos-Core | agreed |
| R4-2100495 | CR on Introducing NR inter-band CA for 3DL Bands and 1UL band for 38.101-3 | CATT | 38.101-3 | 0442 | - | Rel-17 | B | NR_CA_R17_3BDL_1BUL-Core | agreed |
| R4-2100497 | Correction on supported channel bandwidth for n79 | CATT | 38.101-3 | 0443 | - | Rel-16 | F | NR_CA_R16_3BDL_1BUL-Core | withdrawn |
| R4-2100498 | Correction on supported channel bandwidth for n79 | CATT | 38.101-3 | 0444 | - | Rel-17 | A | NR_CA_R16_3BDL_1BUL-Core | withdrawn |
| R4-2100500 | CR for TS 38.101-3, General corrections for NR V2X | CATT | 38.101-3 | 0445 | - | Rel-17 | A | 5G_V2X_NRSL-Core | agreed |
| R4-2100618 | Revision of inter-band V2X con-currency table for V2X_39_n47 and V2X_40_n47 | Qualcomm Incorporated | 38.101-3 | 0446 | - | Rel-17 | F | NR_LTE_V2X_PC5_combos-Core | agreed |
| R4-2100794 | CR for TS 38.101-3: Correction on 1Tx-2Tx switching between two uplink carriers (Rel-16) | China Telecom | 38.101-3 | 0447 | - | Rel-16 | F | NR_RF_FR1-Core | agreed |
| R4-2100795 | CR for TS 38.101-3: Correction on 1Tx-2Tx switching between two uplink carriers (Rel-17) | China Telecom | 38.101-3 | 0448 | - | Rel-17 | A | NR_RF_FR1-Core | agreed |
| R4-2100844 | CR for 38.101-3 Correction on EN-DC MSD due to cross band isolation for DC_28_n5 (R16) | MediaTek Inc. | 38.101-3 | 0449 | - | Rel-16 | F | LTE_NR_DC_CA_enh-Core | not pursued |
| R4-2100878 | CR for 38.101-3: Update of simultaneous Rx/Tx capability for some EN-DC band combinations Rel-16 | SoftBank Corp. | 38.101-3 | 0450 | - | Rel-16 | F | TEI16 | not pursued |
| R4-2100879 | CR for 38.101-3: Update of simultaneous Rx/Tx capability for some EN-DC band combinations Rel-17 | SoftBank Corp. | 38.101-3 | 0451 | - | Rel-17 | A | TEI16 | withdrawn |
| R4-2100986 | CR introduction completed band combinations for Dual Connectivity (DC) of x bands (x=2,3,4) LTE inter-band CA (xDL/1UL) and 1 NR FR1 band (1DL/1UL) and 1 NR FR2 band (1DL/1UL) | Samsung | 38.101-3 | 0452 | - | Rel-17 | B | DC_R17_xBLTE_2BNR_yDL3UL | withdrawn |
| R4-2100988 | CR introduction completed band combinations for Dual Connectivity (DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | Samsung | 38.101-3 | 0453 | - | Rel-17 | B | DC_R17_5BLTE_1BNR_6DL2UL | agreed |
| R4-2100991 | CR on introduction of completed NR CA/DC combs with 4DL/2UL including FR2 | Samsung | 38.101-3 | 0454 | - | Rel-17 | B | NR_CADC_R17_4BDL_2BUL | not concluded |
| R4-2101099 | Big CR for Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) | CHTTL | 38.101-3 | 0455 | - | Rel-17 | B | DC_R17_1BLTE_1BNR_2DL2UL-Core | agreed |
| R4-2101145 | Clarification on timing difference for Tx switching in EN-DC R16 | MediaTek inc. | 38.101-3 | 0456 | - | Rel-16 | F | NR_RF_FR1-Core | revised |
| R4-2103353 | Clarification on timing difference for Tx switching in EN-DC R16 | MediaTek inc. | 38.101-3 | 0456 | 1 | Rel-16 | F | NR_RF_FR1-Core | agreed |
| R4-2101146 | Clarification on timing difference for Tx switching in EN-DC R17 | MediaTek inc. | 38.101-3 | 0457 | - | Rel-17 | A | NR_RF_FR1-Core | agreed |
| R4-2101176 | CR to TS 38.101-3 clarification on the single uplink allowance for DC_3A_n3A | CHTTL | 38.101-3 | 0458 | - | Rel-16 | F | DC_R16_1BLTE_1BNR_2DL2UL | revised |
| R4-2103163 | CR to TS 38.101-3 clarification on the single uplink allowance for DC_3A_n3A | CHTTL | 38.101-3 | 0458 | 1 | Rel-16 | F | DC_R16_1BLTE_1BNR_2DL2UL | agreed |
| R4-2101179 | CR to TS 38.101-3 clarification on the single uplink allowance for DC_3A_n3A | CHTTL | 38.101-3 | 0459 | - | Rel-17 | A | DC_R16_1BLTE_1BNR_2DL2UL | agreed |
| R4-2101510 | CR on introduction of completed EN-DC of 2 bands LTE and 1 band NR from RAN4#96e and RAN4#97e into TS 38.101-3 | Huawei, HiSilicon | 38.101-3 | 0460 | - | Rel-17 | B | DC_R17_2BLTE_1BNR_3DL2UL-Core | not concluded |
| R4-2101718 | Correction to applicability of simultaneous RX/TX and single-UL transmission | Ericsson | 38.101-3 | 0461 | - | Rel-15 | F | NR_newRAT-Core | not pursued |
| R4-2101719 | Correction to applicability of simultaneous RX/TX and single-UL transmission | Ericsson | 38.101-3 | 0462 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2101725 | Requirements Type 2 UEs supporting inter-band MRDC with overlapping DL | Ericsson | 38.101-3 | 0463 | - | Rel-16 | F | TEI16 | revised |
| R4-2103164 | Requirements Type 2 UEs supporting inter-band MRDC with overlapping DL | Ericsson | 38.101-3 | 0463 | 1 | Rel-16 | F | TEI16 | agreed |
| R4-2101746 | CR on simultaneous Tx-Rx for EN-DC | OPPO | 38.101-3 | 0464 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103128 | CR on simultaneous Tx-Rx for EN-DC | OPPO | 38.101-3 | 0464 | 1 | Rel-15 | F | NR_newRAT-Core | not pursued |
| R4-2101747 | CR on simultaneous Tx-Rx for EN-DC (R16 CR) | OPPO | 38.101-3 | 0465 | - | Rel-16 | F | NR_newRAT-Core | revised |
| R4-2103129 | CR on simultaneous Tx-Rx for EN-DC (R16 CR) | OPPO | 38.101-3 | 0465 | 1 | Rel-16 | F | NR_newRAT-Core | not pursued |
| R4-2101748 | CR on simultaneous Tx-Rx for EN-DC (R17 mirror CR) | OPPO | 38.101-3 | 0466 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2101804 | CR for 38.101-3 to add the missing Tib Rib for DC_2-7-7-66_n78/ DC_2-7-66-66_n78/ DC_2-7-7-66-66_n78 (Rel-16) | Huawei, HiSilicon | 38.101-3 | 0467 | - | Rel-16 | F | DC_R16_3BLTE_1BNR_4DL2UL-Core | agreed |
| R4-2101805 | CR for 38.101-3 to add the missing Tib Rib for DC_2-7-7-66_n78/ DC_2-7-66-66_n78/ DC_2-7-7-66-66_n78 (Rel-17) | Huawei, HiSilicon | 38.101-3 | 0468 | - | Rel-17 | A | DC_R16_3BLTE_1BNR_4DL2UL-Core | agreed |
| R4-2101870 | CR for TS 38.101-3 switching period for V2X con-current operation Rel-16 | Xiaomi | 38.101-3 | 0469 | - | Rel-16 | F | 5G_V2X_NRSL-Core | not pursued |
| R4-2101871 | CR for TS 38.101-3 switching period for V2X con-current operation Rel-17 | Xiaomi | 38.101-3 | 0470 | - | Rel-17 | A | 5G_V2X_NRSL-Core | withdrawn |
| R4-2101888 | CR introduction completed band combinations LTE 3DL and one NR band - | Ericsson | 38.101-3 | 0471 | - | Rel-17 | B | DC_R17_3BLTE_1BNR_4DL2UL | agreed |
| R4-2101890 | CR introduction completed band combinations NR Inter-band 4 bands CA - | Ericsson | 38.101-3 | 0472 | - | Rel-17 | B | NR_CA_R17_4BDL_1BUL | agreed |
| R4-2101925 | CR to introduce new combinations of LTE 4band + NR 1band for TS 38.101-3 | Nokia, Nokia Shanghai Bell | 38.101-3 | 0473 | - | Rel-17 | B | DC_R17_4BLTE_1BNR_5DL2UL | agreed |
| R4-2102094 | CR for 38.101-3 to introduce a new MSD due to the counter intermodulation interference(Rel-15) | Huawei, HiSilicon | 38.101-3 | 0474 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103357 | CR for 38.101-3 to introduce a new MSD due to the counter intermodulation interference(Rel-15) | Huawei, HiSilicon | 38.101-3 | 0474 | 1 | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2102095 | CR for 38.101-3 to introduce a new MSD due to the counter intermodulation interference(Rel-16) | Huawei, HiSilicon | 38.101-3 | 0475 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2102096 | CR for 38.101-3 to introduce a new MSD due to the counter intermodulation interference(Rel-17) | Huawei, HiSilicon | 38.101-3 | 0476 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2102146 | CR for 38.101-3: Correction for CA_n66A-n260 | T-Mobile USA | 38.101-3 | 0477 | - | Rel-16 | F | NR_CADC_R16_2BDL_xBUL-Core | agreed |
| R4-2102147 | CR for 38.101-3: Correction for CA_n66A-n260A | T-Mobile USA | 38.101-3 | 0478 | - | Rel-17 | A | NR_CADC_R16_2BDL_xBUL-Core | agreed |
| R4-2102205 | CR to TS38.101-3: Correction on duty cycle signalling terminology for PC2 inter-band ENDC | ZTE Corporation | 38.101-3 | 0479 | - | Rel-16 | F | ENDC_UE_PC2_FDD_TDD-Core | agreed |
| R4-2102206 | CR to TS38.101-3: Correction on duty cycle signalling terminology for PC2 inter-band ENDC | ZTE Corporation | 38.101-3 | 0480 | - | Rel-17 | A | ENDC_UE_PC2_FDD_TDD-Core | agreed |
| R4-2102379 | CR for TS 38.101-3: correction CR for simultaneous Tx/Rx operation (R16) | Huawei, HiSilicon | 38.101-3 | 0481 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2102381 | CR for TS 38.101-3 NR V2X switching period | Huawei, HiSilicon | 38.101-3 | 0482 | - | Rel-16 | B | 5G_V2X_NRSL-Core | not pursued |
| R4-2102395 | CR for TS 38.101-3 correction of intra-band contiguous EN-DC for DC_(n)66_R16 | Huawei, HiSilicon | 38.101-3 | 0483 | - | Rel-16 | F | TEI16 | not pursued |
| R4-2102396 | CR for TS 38.101-3 correction of intra-band contiguous EN-DC for DC_(n)66_R17 | Huawei, HiSilicon | 38.101-3 | 0484 | - | Rel-17 | A | TEI16 | withdrawn |
| R4-2102402 | CR for TS 38.101-3: Adding delta TIB and RIB requirement for DC_2-7-7-66_n78 (R16) | Huawei, HiSilicon | 38.101-3 | 0485 | - | Rel-16 | F | DC_R16_3BLTE_1BNR_4DL2UL-Core | agreed |
| R4-2102403 | draft CR for TS 38.101-3: Adding delta TIB and RIB requirement for DC_2-7-7-66_n78 (R17) | Huawei, HiSilicon | 38.101-3 | 0486 | - | Rel-17 | A | DC_R16_3BLTE_1BNR_4DL2UL-Core | agreed |
| R4-2102412 | Notational amendment and correction to PCMAX for EN-DC | Qualcomm Incorporated | 38.101-3 | 0487 | - | Rel-16 | F | NR_newRAT-Core | revised |
| R4-2103165 | Notational amendment and correction to PCMAX for EN-DC | Qualcomm Incorporated | 38.101-3 | 0487 | 1 | Rel-16 | F | NR_newRAT-Core | agreed |
| R4-2102413 | Notational amendment and correction to PCMAX for EN-DC | Qualcomm Incorporated | 38.101-3 | 0488 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2102505 | CR for 38.101-3 on applicability of minimum requirements for EN-DC (Rel-15) | Qualcomm Incorporated | 38.101-3 | 0489 | - | Rel-15 | F | NR_newRAT-Core | not pursued |
| R4-2102506 | CR for 38.101-3 on applicability of minimum requirements for EN-DC (Rel-16) | Qualcomm Incorporated | 38.101-3 | 0490 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2102507 | CR for 38.101-3 on applicability of minimum requirements for EN-DC (Rel-17) | Qualcomm Incorporated | 38.101-3 | 0491 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2102593 | CR for bug fixing of band combination tables for 38101-3 Rel17 | Apple | 38.101-3 | 0492 | - | Rel-17 | F | NR_newRAT-Core | revised |
| R4-2103345 | CR for bug fixing of band combination tables for 38101-3 Rel17 | Apple | 38.101-3 | 0492 | 1 | Rel-17 | F | NR_newRAT-Core | agreed |
| R4-2102594 | CR for bug fixing of band combination tables for 38101-3 Rel16 | Apple | 38.101-3 | 0493 | - | Rel-16 | F | NR_newRAT-Core | revised |
| R4-2103346 | CR for bug fixing of band combination tables for 38101-3 Rel16 | Apple | 38.101-3 | 0493 | 1 | Rel-16 | F | NR_newRAT-Core | agreed |
| R4-2102717 | CR for TS 38.101-3: correction CR for simultaneous Tx/Rx operation (R17) | Huawei, HiSilicon | 38.101-3 | 0494 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2102826 | CR for correction of Rel-16 Dual Connectivity of 1LTE band (1DL/1UL) and 1NR band (1DL/1UL) with FR1 | Verizon Denmark | 38.101-3 | 0495 | - | Rel-16 | F | DC_R16_1BLTE_1BNR_2DL2UL-Core | postponed |
| R4-2104041 | Void | Void | 38.101-3 | 0496 | - | Rel-17 | - | NR_CADC_R17_2BDL_xBUL | withdrawn |
| R4-2102946 | CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into TS 38.101-3 | ZTE Corporation | 38.101-3 | 0497 | - | Rel-17 | - | NR_CADC_R17_3BDL_2BUL | not concluded |
| R4-2102947 | CR to reflect the completed ENDC combinations for 3 bands DL with 3 bands UL into TS 38.101-3 | ZTE Corporation | 38.101-3 | 0498 | - | Rel-17 | - | DC_R17_xBLTE_yBNR_3DL3UL | agreed |
| R4-2102948 | CR to reflect the completed Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL1UL) and 3 bands NR inter-band CA (3DL1UL) into TS38.101-3 | ZTE Corporation | 38.101-3 | 0499 | - | Rel-17 | B | DC_R17_xBLTE_3BNR_yDL2UL-Core | agreed |
| R4-2102944 | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into TS 38.101-3 | ZTE Corporation | 38.101-3 | 0500 | - | Rel-17 | - | NR_CADC_R17_2BDL_xBUL | agreed |
| R4-2103395 | TS 38.101-3: Addition of missing lower order fallbacks R16 | Nokia, AT&T | 38.101-3 | 0501 | - | Rel-17 | A | TEI16 | agreed |
| R4-2103402 | CR for 38.101-3: clarification of intra-band EN-DC BCS applicability | : T-Mobile USA | 38.101-3 | 0502 | - | Rel-15 | - | - | agreed |
| R4-2103403 | CR for 38.101-3: clarification of intra-band EN-DC BCS applicability | : T-Mobile USA | 38.101-3 | 0503 | - | Rel-16 | - | - | agreed |
| R4-2103404 | CR for 38.101-3: clarification of intra-band EN-DC BCS applicability | : T-Mobile USA | 38.101-3 | 0504 | - | Rel-17 | - | - | agreed |
| R4-2100168 | CR on FDD HST Single-Tap and Multipath Fading Requirements | Qualcomm Incorporated | 38.101-4 | 0124 | - | Rel-16 | F | NR_HST-Perf | revised |
| R4-2103978 | CR on FDD HST Single-Tap and Multipath Fading Requirements | Qualcomm Incorporated | 38.101-4 | 0124 | 1 | Rel-16 | F | NR_HST-Perf | agreed |
| R4-2100171 | Correction CR on URLLC Higher BLER Performance Requirements | Qualcomm Incorporated | 38.101-4 | 0125 | - | Rel-16 | F | NR_L1enh_URLLC-Perf | merged |
| R4-2100199 | CR to 38-101-4 on CQI reporting requirements for URLLC | Apple | 38.101-4 | 0126 | - | Rel-16 | B | NR_L1enh_URLLC-Perf | revised |
| R4-2103891 | CR to 38-101-4 on CQI reporting requirements for URLLC | Apple | 38.101-4 | 0126 | 1 | Rel-16 | B | NR_L1enh_URLLC-Perf | agreed |
| R4-2100201 | CR to 38.101-4 on requirements with slot aggreagation in FR2 | Apple | 38.101-4 | 0127 | - | Rel-16 | F | NR_L1enh_URLLC-Perf | revised |
| R4-2103900 | CR to 38.101-4 on requirements with slot aggreagation in FR2 | Apple | 38.101-4 | 0127 | 1 | Rel-16 | F | NR_L1enh_URLLC-Perf | agreed |
| R4-2100210 | CR to 38.101-4 for eMIMO demod requirements - General and Applicability rule | Apple | 38.101-4 | 0128 | - | Rel-16 | B | NR_eMIMO-Perf | revised |
| R4-2103826 | CR to 38.101-4 for eMIMO demod requirements - General and Applicability rule | Apple | 38.101-4 | 0128 | 1 | Rel-16 | B | NR_eMIMO-Perf | agreed |
| R4-2100398 | CR for TS38.101-4, test for FR2 PDCCH DCI format 2_6 demodulation | CATT | 38.101-4 | 0129 | - | Rel-16 | B | NR_UE_pow_sav-Perf | revised |
| R4-2103822 | CR for TS38.101-4, test for FR2 PDCCH DCI format 2_6 demodulation | CATT | 38.101-4 | 0129 | 1 | Rel-16 | B | NR_UE_pow_sav-Perf | agreed |
| R4-2100816 | CR for NR PDSCH FR1 CA 2Rx performance requirements | CMCC | 38.101-4 | 0130 | - | Rel-16 | F | NR_perf_enh-Perf | agreed |
| R4-2100818 | CR for TS38.101-4, test for FR1 TDD PDCCH DCI format 2_6 demodulation | CMCC | 38.101-4 | 0131 | - | Rel-16 | B | NR_UE_pow_sav-Perf | revised |
| R4-2103823 | CR for TS38.101-4, test for FR1 TDD PDCCH DCI format 2_6 demodulation | CMCC | 38.101-4 | 0131 | 1 | Rel-16 | B | NR_UE_pow_sav-Perf | agreed |
| R4-2100848 | CR on HST-SFN requirements for TDD | CMCC | 38.101-4 | 0132 | - | Rel-16 | F | NR_HST-Perf | not pursued |
| R4-2103858 | CR on HST-SFN requirements for TDD | CMCC | 38.101-4 | 0132 | 1 | Rel-16 | F | NR_HST-Perf | withdrawn |
| R4-2100884 | CR on adding applicability, requirements and measurement channel for FR2 DL 256QAM CQI reporting test under fading condition | China Telecom | 38.101-4 | 0133 | - | Rel-16 | B | NR_DL256QAM_FR2-Perf | revised |
| R4-2103837 | CR on adding applicability, requirements and measurement channel for FR2 DL 256QAM CQI reporting test under fading condition | China Telecom | 38.101-4 | 0133 | 1 | Rel-16 | B | NR_DL256QAM_FR2-Perf | agreed |
| R4-2100886 | CR: Adding applicability and requirements for FR1 and FR2 CA CQI reporting test | China Telecom | 38.101-4 | 0134 | - | Rel-16 | B | NR_perf_enh-Perf | agreed |
| R4-2100896 | Introduction of PMI test cases with Rel-16 eType II codebook | Samsung | 38.101-4 | 0135 | - | Rel-16 | B | NR_eMIMO-Perf | revised |
| R4-2102938 | Introduction of PMI test cases with Rel-16 eType II codebook | Samsung | 38.101-4 | 0135 | 1 | Rel-16 | B | NR_eMIMO-Perf | revised |
| R4-2103833 | Introduction of PMI test cases with Rel-16 eType II codebook | Samsung | 38.101-4 | 0135 | 2 | Rel-16 | B | NR_eMIMO-Perf | agreed |
| R4-2100897 | Introduction of PMI test cases with Rel-15 Type II codebook | Samsung | 38.101-4 | 0136 | - | Rel-16 | B | NR_perf_enh-Perf | revised |
| R4-2102939 | Introduction of PMI test cases with Rel-15 Type II codebook | Samsung | 38.101-4 | 0136 | 1 | Rel-16 | B | NR_perf_enh-Perf | revised |
| R4-2103843 | Introduction of PMI test cases with Rel-15 Type II codebook | Samsung | 38.101-4 | 0136 | 2 | Rel-16 | B | NR_perf_enh-Perf | agreed |
| R4-2101018 | CR to 38.101-4 on FRC table update for URLLC ultra low BLER requirements | Apple | 38.101-4 | 0137 | - | Rel-16 | F | NR_L1enh_URLLC-Perf | agreed |
| R4-2101116 | CR on demodulation performance requirements for DL 256QAM for FR2 | ZTE Corporation | 38.101-4 | 0138 | - | Rel-16 | F | NR_DL256QAM_FR2-Perf | revised |
| R4-2103834 | CR on demodulation performance requirements for DL 256QAM for FR2 | ZTE Corporation | 38.101-4 | 0138 | 1 | Rel-16 | F | NR_DL256QAM_FR2-Perf | agreed |
| R4-2101245 | CR on FRC for URLLC UE Higher BLER requirements | Intel Corporation | 38.101-4 | 0139 | - | Rel-16 | B | NR_L1enh_URLLC-Perf | revised |
| R4-2103901 | CR on FRC for URLLC UE Higher BLER requirements | Intel Corporation | 38.101-4 | 0139 | 1 | Rel-16 | B | NR_L1enh_URLLC-Perf | agreed |
| R4-2101252 | CR on simplified TDL-D channel model for FR2 DL 256QAM demodulation requirements | Intel Corporation | 38.101-4 | 0140 | - | Rel-16 | B | NR_DL256QAM_FR2-Perf | revised |
| R4-2103835 | CR on simplified TDL-D channel model for FR2 DL 256QAM demodulation requirements | Intel Corporation | 38.101-4 | 0140 | 1 | Rel-16 | B | NR_DL256QAM_FR2-Perf | agreed |
| R4-2101253 | CR on applicability rules and FRC for FR2 DL 256QAM CQI requirements | Intel Corporation | 38.101-4 | 0141 | - | Rel-16 | B | NR_DL256QAM_FR2-Perf | revised |
| R4-2103838 | CR on applicability rules and FRC for FR2 DL 256QAM CQI requirements | Intel Corporation | 38.101-4 | 0141 | 1 | Rel-16 | B | NR_DL256QAM_FR2-Perf | agreed |
| R4-2101255 | CR on applicability rules for Normal NR CA requirements | Intel Corporation | 38.101-4 | 0142 | - | Rel-16 | B | NR_perf_enh-Perf | revised |
| R4-2103841 | CR on applicability rules for Normal NR CA requirements | Intel Corporation | 38.101-4 | 0142 | 1 | Rel-16 | B | NR_perf_enh-Perf | agreed |
| R4-2101259 | CR to TS 38.101-4: Performance requirements single-DCI based multi-TRP Repetition Tx schemes | Intel Corporation, Samsung, Ericsson, Huawei, HiSilicon | 38.101-4 | 0143 | - | Rel-16 | B | NR_eMIMO-Perf | revised |
| R4-2103827 | CR to TS 38.101-4: Performance requirements single-DCI based multi-TRP Repetition Tx schemes | Intel Corporation, Samsung, Ericsson, Huawei, HiSilicon | 38.101-4 | 0143 | 1 | Rel-16 | B | NR_eMIMO-Perf | agreed |
| R4-2101297 | CR on applicability and FRC for PDSCH normal demodulation for DL 256QAM for FR2 | Huawei, HiSilicon | 38.101-4 | 0144 | - | Rel-16 | B | NR_DL256QAM_FR2-Perf | agreed |
| R4-2101299 | CR on SDR requirements for DL 256QAM for FR2 | Huawei, HiSilicon | 38.101-4 | 0145 | - | Rel-16 | B | NR_DL256QAM_FR2-Perf | revised |
| R4-2103836 | CR on SDR requirements for DL 256QAM for FR2 | Huawei, HiSilicon | 38.101-4 | 0145 | 1 | Rel-16 | B | NR_DL256QAM_FR2-Perf | agreed |
| R4-2101300 | CR on update TRS and CSI-RS transmission for HST DPS requirements | Huawei, HiSilicon, Ericsson, Intel Corporation | 38.101-4 | 0146 | - | Rel-16 | F | NR_HST-Perf | agreed |
| R4-2101315 | CR for 38.101-4 Introduction of PDSCH requirement with Single-DCI based SDM scheme | Huawei, HiSilicon | 38.101-4 | 0147 | - | Rel-16 | B | NR_eMIMO-Perf | revised |
| R4-2103828 | CR for 38.101-4 Introduction of PDSCH requirement with Single-DCI based SDM scheme | Huawei, HiSilicon | 38.101-4 | 0147 | 1 | Rel-16 | B | NR_eMIMO-Perf | agreed |
| R4-2101316 | CR for 38.101-4 Introduction of PDSCH requirement with Multi-DCI based multi-TRP transmission schemes | Huawei, HiSilicon | 38.101-4 | 0148 | - | Rel-16 | B | NR_eMIMO-Perf | revised |
| R4-2103829 | CR for 38.101-4 Introduction of PDSCH requirement with Multi-DCI based multi-TRP transmission schemes | Huawei, HiSilicon | 38.101-4 | 0148 | 1 | Rel-16 | B | NR_eMIMO-Perf | revised |
| R4-2103986 | CR for 38.101-4 Introduction of PDSCH requirement with Multi-DCI based multi-TRP transmission schemes | Huawei, HiSilicon | 38.101-4 | 0148 | 2 | Rel-16 | B | NR_eMIMO-Perf | agreed |
| R4-2101322 | CR for 38.101-4 Applicablity of PMI reporting test with Tx ports larger than 8 and up to 32 | Huawei, HiSilicon | 38.101-4 | 0149 | - | Rel-16 | B | NR_perf_enh-Perf | revised |
| R4-2103842 | CR for 38.101-4 Applicablity of PMI reporting test with Tx ports larger than 8 and up to 32 | Huawei, HiSilicon | 38.101-4 | 0149 | 1 | Rel-16 | B | NR_perf_enh-Perf | agreed |
| R4-2101323 | CR for 38.101-4 Applicablity of PMI reporting test of eType II codebook | Huawei, HiSilicon | 38.101-4 | 0150 | - | Rel-16 | B | NR_perf_enh-Perf | revised |
| R4-2103954 | CR for 38.101-4 Applicablity of PMI reporting test of eType II codebook | Huawei, HiSilicon | 38.101-4 | 0150 | 1 | Rel-16 | B | NR_perf_enh-Perf | agreed |
| R4-2101337 | CR to TS 38.101-4 Correction of UE performance requirements for FR1 URLLC PDSCH repetitions over multiple slots. | Huawei, HiSilicon | 38.101-4 | 0151 | - | Rel-16 | F | NR_L1enh_URLLC-Perf | revised |
| R4-2103902 | CR to TS 38.101-4 Correction of UE performance requirements for FR1 URLLC PDSCH repetitions over multiple slots. | Huawei, HiSilicon | 38.101-4 | 0151 | 1 | Rel-16 | F | NR_L1enh_URLLC-Perf | agreed |
| R4-2101338 | CR to TS38.101-4 Applicability rules for URLLC CSI requirements | Huawei, HiSilicon | 38.101-4 | 0152 | - | Rel-16 | B | NR_L1enh_URLLC-Perf | revised |
| R4-2103892 | CR to TS38.101-4 Applicability rules for URLLC CSI requirements | Huawei, HiSilicon | 38.101-4 | 0152 | 1 | Rel-16 | B | NR_L1enh_URLLC-Perf | agreed |
| R4-2101367 | CR: Updates to power imbalance for CA | Huawei, HiSilicon | 38.101-4 | 0153 | - | Rel-16 | F | NR_perf_enh-Perf | agreed |
| R4-2101387 | CR on Fixed reference channel for power saving performance | vivo | 38.101-4 | 0154 | - | Rel-16 | F | NR_UE_pow_sav-Perf | revised |
| R4-2103824 | CR on Fixed reference channel for power saving performance | vivo | 38.101-4 | 0154 | 1 | Rel-16 | F | NR_UE_pow_sav-Perf | agreed |
| R4-2101423 | Introduction of Noc for PC5 in n257/n258 | Ericsson, SoftBank | 38.101-4 | 0155 | - | Rel-17 | B | NR_FR2_FWA_Bn257_Bn258-Perf | agreed |
| R4-2101437 | Correction of title on 16Tx port subband PMI reporting | Ericsson | 38.101-4 | 0156 | - | Rel-16 | F | NR_perf_enh-Perf | agreed |
| R4-2101441 | Correction of CQI test parameters and FRC for UE demodulation test | Ericsson | 38.101-4 | 0157 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103796 | Correction of CQI test parameters and FRC for UE demodulation test | Ericsson | 38.101-4 | 0157 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2101442 | Correction of CQI test parameters and FRC for UE demodulation test | Ericsson | 38.101-4 | 0158 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2101448 | CR: FRC for eMIMO sDCI/mDCI-based PDSCH transmission | Ericsson, Huawei, HiSilicon, Intel, Samsung | 38.101-4 | 0159 | - | Rel-16 | B | NR_eMIMO-Perf | revised |
| R4-2103830 | CR: FRC for eMIMO sDCI/mDCI-based PDSCH transmission | Ericsson, Huawei, HiSilicon, Intel, Samsung | 38.101-4 | 0159 | 1 | Rel-16 | B | NR_eMIMO-Perf | agreed |
| R4-2101944 | CR on FRC for Ultra low BLER UE CQI requirements | Intel Corporation | 38.101-4 | 0160 | - | Rel-16 | B | NR_L1enh_URLLC-Perf | revised |
| R4-2103893 | CR on FRC for Ultra low BLER UE CQI requirements | Intel Corporation | 38.101-4 | 0160 | 1 | Rel-16 | B | NR_L1enh_URLLC-Perf | agreed |
| R4-2101945 | CR on FRC for NR RI requirements (Rel-15) | Intel Corporation | 38.101-4 | 0161 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103798 | CR on FRC for NR RI requirements (Rel-15) | Intel Corporation | 38.101-4 | 0161 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2101946 | CR on FRC for NR RI requirements (Rel-16) | Intel Corporation | 38.101-4 | 0162 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2102100 | CR to TS 38.101-4: n262 demodulation requirements | Ericsson | 38.101-4 | 0163 | - | Rel-17 | B | NR_47GHz_band-Perf | postponed |
| R4-2102120 | CR to TS 38.101-4: Performance requirements for URLLC PDSCH 0.001% BLER | Ericsson | 38.101-4 | 0164 | - | Rel-16 | F | NR_L1enh_URLLC-Perf | revised |
| R4-2103890 | CR to TS 38.101-4: Performance requirements for URLLC PDSCH 0.001% BLER | Ericsson | 38.101-4 | 0164 | 1 | Rel-16 | F | NR_L1enh_URLLC-Perf | agreed |
| R4-2102121 | CR to TS 38.101-4: Performance requirements for URLLC High BLER feature tests | Ericsson | 38.101-4 | 0165 | - | Rel-16 | F | NR_L1enh_URLLC-Perf | revised |
| R4-2103903 | CR to TS 38.101-4: Performance requirements for URLLC High BLER feature tests | Ericsson | 38.101-4 | 0165 | 1 | Rel-16 | F | NR_L1enh_URLLC-Perf | agreed |
| R4-2102822 | CR on FR1 PDSCH Mapping Type B and Processing Capability 2 Requirements | Qualcomm Incorporated | 38.101-4 | 0166 | - | Rel-16 | F | NR_L1enh_URLLC-Perf | revised |
| R4-2103904 | CR on FR1 PDSCH Mapping Type B and Processing Capability 2 Requirements | Qualcomm Incorporated | 38.101-4 | 0166 | 1 | Rel-16 | F | NR_L1enh_URLLC-Perf | agreed |
| R4-2102824 | CR on corrections for LTE-NR Co-existence tests and OCNG pattern | Qualcomm Incorporated | 38.101-4 | 0167 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103797 | CR on corrections for LTE-NR Co-existence tests and OCNG pattern | Qualcomm Incorporated | 38.101-4 | 0167 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2102828 | CR on corrections for LTE-NR Co-existence tests and OCNG pattern | Qualcomm Incorporated | 38.101-4 | 0168 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2102870 | CR to 38.101-4 on update to CSI reporting test parameters for Aperiodic reporting (R15) | Apple | 38.101-4 | 0169 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103799 | CR to 38.101-4 on update to CSI reporting test parameters for Aperiodic reporting (R15) | Apple | 38.101-4 | 0169 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2102871 | CR to 38.101-4 on update to CSI reporting test parameters for Aperiodic reporting (R16) | Apple | 38.101-4 | 0170 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2103932 | CR for TS38.101-4, test for FR1 FDD PDCCH DCI format 2_6 demodulation | MediaTek inc. | 38.101-4 | 0171 | - | Rel-16 | F | NR_UE_pow_sav-Perf | agreed |
| R4-2103845 | CR: Update on test applicability rule for EN-DC power imbalance | Huawei, HiSilicon | 38.101-4 | 0172 | - | Rel-16 | F | NR_perf_enh-Perf | agreed |
| R4-2100335 | CR to 38104 on introducing new SUL band n99 | Huawei,HiSilicon | 38.104 | 0262 | - | Rel-17 | B | NR_SUL_UL_n24-Core | revised |
| R4-2103212 | CR to 38104 on introducing new SUL band n99 | Huawei,HiSilicon | 38.104 | 0262 | 1 | Rel-17 | B | NR_SUL_UL_n24-Core | revised |
| R4-2103352 | CR to 38104 on introducing new SUL band n99 | Huawei,HiSilicon | 38.104 | 0262 | 2 | Rel-17 | B | NR_SUL_UL_n24-Core | agreed |
| R4-2100558 | CR for 38.104: HST PUSCH demodulation requirements | Nokia, Nokia Shanghai Bell | 38.104 | 0263 | - | Rel-16 | F | NR_HST-Perf | revised |
| R4-2103861 | CR for 38.104: HST PUSCH demodulation requirements | Nokia, Nokia Shanghai Bell | 38.104 | 0263 | 1 | Rel-16 | F | NR_HST-Perf | agreed |
| R4-2100559 | CR for 38.104: HST PUSCH demodulation requirements | Nokia, Nokia Shanghai Bell | 38.104 | 0264 | - | Rel-17 | A | NR_HST-Perf | agreed |
| R4-2100561 | CR for 38.104: Low latency FR1 BS demodulation requirements | Nokia, Nokia Shanghai Bell | 38.104 | 0265 | - | Rel-16 | F | NR_L1enh_URLLC-Perf | revised |
| R4-2103905 | CR for 38.104: Low latency FR1 BS demodulation requirements | Nokia, Nokia Shanghai Bell | 38.104 | 0265 | 1 | Rel-16 | F | NR_L1enh_URLLC-Perf | agreed |
| R4-2100562 | CR for 38.104: Low latency FR1 BS demodulation requirements | Nokia, Nokia Shanghai Bell | 38.104 | 0266 | - | Rel-17 | A | NR_L1enh_URLLC-Perf | agreed |
| R4-2100563 | CR for 38.104: Ultra high reliability BS demodulation requirements | Nokia, Nokia Shanghai Bell | 38.104 | 0267 | - | Rel-16 | F | NR_L1enh_URLLC-Perf | revised |
| R4-2103894 | CR for 38.104: Ultra high reliability BS demodulation requirements | Nokia, Nokia Shanghai Bell | 38.104 | 0267 | 1 | Rel-16 | F | NR_L1enh_URLLC-Perf | agreed |
| R4-2100564 | CR for 38.104: Ultra high reliability BS demodulation requirements | Nokia, Nokia Shanghai Bell | 38.104 | 0268 | - | Rel-17 | A | NR_L1enh_URLLC-Perf | agreed |
| R4-2100927 | CR on PUSCH repetition type A and PUSCH mapping type B radiated performance requirement for TS 38.104 | Samsung | 38.104 | 0269 | - | Rel-16 | B | NR_L1enh_URLLC-Perf | revised |
| R4-2103906 | CR on PUSCH repetition type A and PUSCH mapping type B radiated performance requirement for TS 38.104 | Samsung | 38.104 | 0269 | 1 | Rel-16 | B | NR_L1enh_URLLC-Perf | agreed |
| R4-2100928 | CR on PUSCH repetition type A and PUSCH mapping type B radiated performance requirement for TS 38.104 | Samsung | 38.104 | 0270 | - | Rel-17 | A | NR_L1enh_URLLC-Perf | agreed |
| R4-2101088 | CR to TS 38.104: Additions of regional requirements for n41 in Japan, Rel-15 | NEC, SoftBank, KDDI | 38.104 | 0271 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103777 | CR to TS 38.104: Additions of regional requirements for n41 in Japan, Rel-15 | NEC, SoftBank, KDDI, Nokia | 38.104 | 0271 | 1 | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2101089 | CR to TS 38.104: Additions of regional requirements for n41 and n90 in Japan, Rel-16 | NEC, SoftBank, KDDI | 38.104 | 0272 | - | Rel-16 | F | NR_newRAT-Core | revised |
| R4-2103958 | CR to TS 38.104: Additions of regional requirements for n41 and n90 in Japan, Rel-16 | NEC, SoftBank, KDDI, Nokia | 38.104 | 0272 | 1 | Rel-16 | F | NR_newRAT-Core | agreed |
| R4-2101090 | CR to TS 38.104: Additions of regional requirements for n41 and n90 in Japan, Rel-17 | NEC, SoftBank, KDDI, Nokia | 38.104 | 0273 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2101302 | CR on correction 2-step RACH performance requirements for FR2 in 38.104 (Rel-16) | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell | 38.104 | 0274 | - | Rel-16 | F | NR_2step_RACH-Perf | revised |
| R4-2103959 | CR on correction 2-step RACH performance requirements for FR2 in 38.104 (Rel-16) | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell | 38.104 | 0274 | 1 | Rel-16 | F | NR_2step_RACH-Perf | agreed |
| R4-2101303 | CR on correction 2-step RACH performance requirements for FR2 in 38.104 (Rel-17) | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell | 38.104 | 0275 | - | Rel-17 | A | NR_2step_RACH-Perf | agreed |
| R4-2101319 | CR for 38.104 Cleanup of performance requirements for NR HST PRACH under fading channel (Rel-16) | Huawei, HiSilicon | 38.104 | 0276 | - | Rel-16 | F | NR_HST-Perf | revised |
| R4-2103865 | CR for 38.104 Cleanup of performance requirements for NR HST PRACH under fading channel (Rel-16) | Huawei, HiSilicon | 38.104 | 0276 | 1 | Rel-16 | F | NR_HST-Perf | agreed |
| R4-2101324 | CR for 38.104 Cleanup of performance requirements for NR HST PRACH under fading channel (Rel-17) | Huawei, HiSilicon | 38.104 | 0277 | - | Rel-17 | A | NR_HST-Perf | agreed |
| R4-2101341 | CR to TS38.104 Correction of BS performance requirements for URLLC FR1 PUSCH repetition Type A (Rel-16) | Huawei, HiSilicon | 38.104 | 0278 | - | Rel-16 | F | NR_L1enh_URLLC-Perf | revised |
| R4-2103912 | CR to TS38.104 Correction of BS performance requirements for URLLC FR1 PUSCH repetition Type A (Rel-16) | Huawei, HiSilicon | 38.104 | 0278 | 1 | Rel-16 | F | NR_L1enh_URLLC-Perf | agreed |
| R4-2101342 | CR to TS38.104 Correction of BS performance requirements for URLLC FR1 PUSCH repetition Type A (Rel-17) | Huawei, HiSilicon | 38.104 | 0279 | - | Rel-17 | A | NR_L1enh_URLLC-Perf | agreed |
| R4-2101849 | CR to TS 38.104 Update on UL timing adjustment performance requirements | ZTE Wistron Telecom AB | 38.104 | 0280 | - | Rel-16 | F | NR_HST-Perf | revised |
| R4-2103868 | CR to TS 38.104 Update on UL timing adjustment performance requirements | ZTE Wistron Telecom AB | 38.104 | 0280 | 1 | Rel-16 | F | NR_HST-Perf | agreed |
| R4-2101968 | CR to TS 38.104: system parameters maintenance for NR-U | ZTE Corporation | 38.104 | 0281 | - | Rel-16 | F | NR_unlic-Core | revised |
| R4-2103137 | CR to TS 38.104: system parameters maintenance for NR-U | ZTE Corporation | 38.104 | 0281 | 1 | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2101969 | CR to TS 38.104: system parameters maintenance for NR-U | ZTE Corporation | 38.104 | 0282 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2101978 | CR to TS 38.104: corrections of NR-U BS Tx requirements | ZTE Corporation | 38.104 | 0283 | - | Rel-16 | F | NR_unlic-Core | revised |
| R4-2103888 | CR to TS 38.104: corrections of NR-U BS Tx requirements | ZTE Corporation | 38.104 | 0283 | 1 | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2101979 | CR to TS 38.104: corrections of NR-U BS Tx requirements | ZTE Corporation | 38.104 | 0284 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2101980 | CR to TS 38.104: corrections of NR-U BS Rx requirements | ZTE Corporation | 38.104 | 0285 | - | Rel-16 | F | NR_unlic-Core | revised |
| R4-2103975 | CR to TS 38.104: corrections of NR-U BS Rx requirements | ZTE Corporation | 38.104 | 0285 | 1 | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2101981 | CR to TS 38.104: corrections of NR-U BS Rx requirements | ZTE Corporation | 38.104 | 0286 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2101994 | CR to TS 38.104: EESS protection requirement correction | Nokia, Nokia Shanghai Bell | 38.104 | 0287 | - | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2101995 | CR to TS 38.104: EESS protection requirement correction | Nokia, Nokia Shanghai Bell | 38.104 | 0288 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2101996 | CR to TS 38.104: EESS protection requirement correction | Nokia, Nokia Shanghai Bell | 38.104 | 0289 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2102159 | CR to TS 38.104 - n262 introduction | Ericsson | 38.104 | 0290 | - | Rel-17 | B | NR_47GHz_band | postponed |
| R4-2102164 | Big CR to TS 38.104 - New CBW Basket WI | Ericsson | 38.104 | 0291 | - | Rel-17 | B | NR_bands_R17_BWs | agreed |
| R4-2102200 | CR to TS38.104: Correction on the Aggregated Channel Bandwidth | ZTE Corporation | 38.104 | 0292 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103117 | CR to TS38.104: Correction on the Aggregated Channel Bandwidth | ZTE Corporation | 38.104 | 0292 | 1 | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2102201 | CR to TS38.104: Correction on the Aggregated Channel Bandwidth | ZTE Corporation | 38.104 | 0293 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2102202 | CR to TS38.104: Correction on the Aggregated Channel Bandwidth | ZTE Corporation | 38.104 | 0294 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2102451 | CR to 38.104: Introduction of n24 | Nokia, Nokia Shanghai Bell | 38.104 | 0295 | - | Rel-17 | B | NR_band_n24-Core | agreed |
| R4-2102835 | CR to TS 38.104 corrections to NR-U BS RF Tx requirements | Nokia, Nokia Shanghai Bell | 38.104 | 0296 | - | Rel-16 | F | NR_unlic-Core | revised |
| R4-2103889 | CR to TS 38.104 corrections to NR-U BS RF Tx requirements | Nokia, Nokia Shanghai Bell | 38.104 | 0296 | 1 | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2102836 | CR to TS 38.104 corrections to NR-U BS RF Tx requirements | Nokia, Nokia Shanghai Bell | 38.104 | 0297 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2102837 | CR to TS 38.104 corrections to NR-U BS RF Rx requirements | Nokia, Nokia Shanghai Bell | 38.104 | 0298 | - | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2102838 | CR to TS 38.104 corrections to NR-U BS RF Rx requirements | Nokia, Nokia Shanghai Bell | 38.104 | 0299 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2103964 | CR to TS 38.104 Update on UL timing adjustment performance requirements | Moderator (China Telecomm) | 38.104 | 0300 | - | Rel-17 | A | NR_HST-Perf | revised |
| R4-2100354 | CR to TS 38.113 on Performance criteria for transient phenomena, Release 15 | Ericsson | 38.113 | 0031 | - | Rel-15 | F | NR_newRAT-Perf | not pursued |
| R4-2100355 | CR to TS 38.113 on Performance criteria for transient phenomena, Release 16 | Ericsson | 38.113 | 0032 | - | Rel-16 | A | NR_newRAT-Perf | withdrawn |
| R4-2102127 | CR to TS 38.113: Radiated emission test method | ZTE Corporation | 38.113 | 0033 | - | Rel-15 | F | NR_newRAT-Core | withdrawn |
| R4-2102128 | CR to TS 38.113: Radiated emission test method | ZTE Corporation | 38.113 | 0034 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2102178 | CR to TS 38.113: Radiated emission test method | ZTE Corporation | 38.113 | 0035 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103772 | CR to TS 38.113: Radiated emission test method | ZTE Corporation | 38.113 | 0035 | 1 | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2102179 | CR to TS 38.113: Radiated emission test method | ZTE Corporation | 38.113 | 0036 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2100890 | CR to TS38.124 on radiated emissions | Samsung | 38.124 | 0029 | - | Rel-15 | F | NR_newRAT-Core | not pursued |
| R4-2100891 | CR to TS38.124 on radiated emissions | Samsung | 38.124 | 0030 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2102398 | CR for TS 38.124: Correction of FR1 radiated spurious emissions (R15) | Huawei, HiSilicon, Bureau Veritas | 38.124 | 0031 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103771 | CR for TS 38.124: Correction of FR1 radiated spurious emissions (R15) | Huawei, HiSilicon, Bureau Veritas,Samsung | 38.124 | 0031 | 1 | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2102399 | CR for TS 38.124: Correction of FR1 radiated spurious emissions (R16) | Huawei, HiSilicon, Bureau Veritas, Samsung | 38.124 | 0032 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2102576 | CR to TS 38.124: correction of the lower frequency range of the RSE, Rel-15 | Huawei | 38.124 | 0033 | - | Rel-15 | F | NR_newRAT-Core | not pursued |
| R4-2102577 | CR to TS 38.124: correction of the lower frequency range of the RSE, Rel-16 | Huawei | 38.124 | 0034 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2102869 | CR on correcting SSB and RACH configuration in CSI-RS based beam failure detection and link recovery tests | Qualcomm CDMA Technologies | 38.133 | 1404 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2100045 | [CR] Test cases for applicable timing for PL RS activated by MAC-CE | ZTE Corporation | 38.133 | 1414 | - | Rel-16 | B | NR_eMIMO-Perf | withdrawn |
| R4-2100051 | [CR] Add Random Access requirements under NR-U | ZTE Corporation | 38.133 | 1415 | - | Rel-16 | B | NR_unlic-Core | merged |
| R4-2100058 | [CR] RRM test case maintenance R15 Cat F | ZTE Corporation | 38.133 | 1416 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103485 | [CR] RRM test case maintenance R15 Cat F | ZTE Corporation | 38.133 | 1416 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2100059 | [CR] RRM test case maintenance R16 Cat A | ZTE Corporation | 38.133 | 1417 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2100060 | [CR] RRM test case maintenance R17 Cat A | ZTE Corporation | 38.133 | 1418 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2100063 | [CR] UE behavior when cannot transmit ACK due to LBT failure for MAC-CE deactivation | ZTE Corporation | 38.133 | 1419 | - | Rel-16 | B | NR_unlic-Core | withdrawn |
| R4-2100064 | [CR] UE behavior when cannot transmit ACK due to LBT failure for MAC-CE deactivation (Cat A) | ZTE Corporation | 38.133 | 1420 | - | Rel-17 | A | NR_unlic-Core | withdrawn |
| R4-2100066 | [CR] UE behavior when cannot transmit ACK due to LBT failure for MAC-CE deactivation | ZTE Corporation | 38.133 | 1421 | - | Rel-16 | F | NR_unlic-Core | merged |
| R4-2100068 | Update FR2 Reference channels and OCNG for FR2 RRM Test cases | ANRITSU LTD | 38.133 | 1422 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103486 | Update FR2 Reference channels and OCNG for FR2 RRM Test cases | ANRITSU LTD | 38.133 | 1422 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2100069 | Update FR2 Reference channels and OCNG for FR2 RRM Test cases | ANRITSU LTD | 38.133 | 1423 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2100070 | Update FR2 Reference channels and OCNG for FR2 RRM Test cases | ANRITSU LTD | 38.133 | 1424 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2100072 | CR to FR1 SA SS-SINR measurement TCs | ANRITSU LTD | 38.133 | 1425 | - | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2100073 | CR to FR1 SA SS-SINR measurement TCs | ANRITSU LTD | 38.133 | 1426 | - | Rel-16 | F | NR_newRAT-Perf | agreed |
| R4-2100074 | CR to FR1 SA SS-SINR measurement TCs | ANRITSU LTD | 38.133 | 1427 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2100075 | CR on E-UTRA carrier for EN-DC event triggered reporting tests | ANRITSU LTD | 38.133 | 1428 | - | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2100076 | CR on E-UTRA carrier for EN-DC event triggered reporting tests | ANRITSU LTD | 38.133 | 1429 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2100077 | CR on E-UTRA carrier for EN-DC event triggered reporting tests | ANRITSU LTD | 38.133 | 1430 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2100078 | Add missing FR2 Test case setups and Beam assumptions | ANRITSU LTD | 38.133 | 1431 | - | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2100079 | Add missing FR2 Test case setups and Beam assumptions | ANRITSU LTD | 38.133 | 1432 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2100080 | Add missing FR2 Test case setups and Beam assumptions | ANRITSU LTD | 38.133 | 1433 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2100115 | [CR] Applicability rule for 2-step RA | ZTE Corporation | 38.133 | 1434 | - | Rel-16 | F | NR_2step_RACH-Core | merged |
| R4-2100116 | [CR] Applicability rule for 2-step RA (Cat A) | ZTE Corporation | 38.133 | 1435 | - | Rel-17 | A | NR_2step_RACH-Core | withdrawn |
| R4-2100117 | [CR] Core maintenance for 38.133 | ZTE Corporation | 38.133 | 1436 | - | Rel-16 | F | TEI16 | agreed |
| R4-2100118 | [CR] Core maintenance for 38.133 (Cat A) | ZTE Corporation | 38.133 | 1437 | - | Rel-17 | A | TEI16 | agreed |
| R4-2100173 | CR on CSSF for EN-DC R15 | Apple | 38.133 | 1438 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103729 | CR on CSSF for EN-DC R15 | Apple | 38.133 | 1438 | 1 | Rel-15 | F | NR_newRAT-Core | postponed |
| R4-2100175 | CR on FR2 SCell activation requirement R15 | Apple | 38.133 | 1439 | - | Rel-15 | F | NR_newRAT-Core | postponed |
| R4-2100176 | CR on FR2 SCell activation requirement R16 | Apple | 38.133 | 1440 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2100177 | CR on FR2 SCell activation requirement R17 | Apple | 38.133 | 1441 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2100178 | CR on smtc1 and smtc2 differentiation in intra-frequency measurement with MG R15 | Apple | 38.133 | 1442 | - | Rel-15 | F | NR_newRAT-Core | postponed |
| R4-2100179 | CR on smtc1 and smtc2 differentiation in intra-frequency measurement with MG R16 | Apple | 38.133 | 1443 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2100180 | CR on smtc1 and smtc2 differentiation in intra-frequency measurement with MG R17 | Apple | 38.133 | 1444 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2100181 | CR on maintenance for inter-band FR2 CA RRM R16 | Apple | 38.133 | 1445 | - | Rel-16 | F | NR_RRM_enh-Core | agreed |
| R4-2100182 | CR on maintenance for inter-band FR2 CA RRM R17 | Apple | 38.133 | 1446 | - | Rel-17 | A | NR_RRM_enh-Core | agreed |
| R4-2100183 | CR on UE behavior for UE specific CBW change R16 | Apple | 38.133 | 1447 | - | Rel-16 | F | NR_RRM_enh-Core | revised |
| R4-2103622 | CR on UE behavior for UE specific CBW change R16 | Apple | 38.133 | 1447 | 1 | Rel-16 | F | NR_RRM_enh-Core | agreed |
| R4-2100184 | CR on UE behavior for UE specific CBW change R17 | Apple | 38.133 | 1448 | - | Rel-17 | A | NR_RRM_enh-Core | agreed |
| R4-2100186 | CR on IDLE/INACTIVE RRM requirement with SMTC2-LP R16 | Apple | 38.133 | 1449 | - | Rel-16 | B | TEI16 | revised |
| R4-2103506 | CR on IDLE/INACTIVE RRM requirement with SMTC2-LP R16 | Apple | 38.133 | 1449 | 1 | Rel-16 | B | TEI16 | agreed |
| R4-2100187 | CR on IDLE/INACTIVE RRM requirement with SMTC2-LP R17 | Apple | 38.133 | 1450 | - | Rel-17 | A | TEI16 | agreed |
| R4-2100189 | CR on reference cell availability for NR-U R16 | Apple | 38.133 | 1451 | - | Rel-16 | F | NR_unlic-Core | postponed |
| R4-2100190 | CR on reference cell availability for NR-U R17 | Apple | 38.133 | 1452 | - | Rel-17 | A | NR_unlic-Core | withdrawn |
| R4-2100203 | CR to 38.133 on RRM requirements for multi-TRP (R16) | Apple | 38.133 | 1453 | - | Rel-16 | B | NR_eMIMO-Core | postponed |
| R4-2100204 | CR to 38.133 on RRM requirements for multi-TRP (R17) | Apple | 38.133 | 1454 | - | Rel-17 | A | NR_eMIMO-Core | withdrawn |
| R4-2100205 | CR to 38.133 on Link Recovery requirements (R16) | Apple | 38.133 | 1455 | - | Rel-16 | F | NR_eMIMO-Core | revised |
| R4-2104055 | CR to 38.133 on Link Recovery requirements (R16) | Apple, MediaTek | 38.133 | 1455 | 1 | Rel-16 | F | NR_eMIMO-Core | agreed |
| R4-2100206 | CR to 38.133 on Link Recovery requirements (R17) | Apple | 38.133 | 1456 | - | Rel-17 | A | NR_eMIMO-Core | agreed |
| R4-2100207 | CR to 38.133 on Pathloss activation delay requirements (R16) | Apple | 38.133 | 1457 | - | Rel-16 | F | NR_eMIMO-Core | revised |
| R4-2103589 | CR to 38.133 on Pathloss activation delay requirements (R16) | Apple | 38.133 | 1457 | 1 | Rel-16 | F | NR_eMIMO-Core | agreed |
| R4-2100208 | CR to 38.133 on Pathloss activation delay requirements (R17) | Apple | 38.133 | 1458 | - | Rel-17 | A | NR_eMIMO-Core | agreed |
| R4-2100214 | CR to 38.133 on UL spatial relation switch requirements (R16) | Apple | 38.133 | 1459 | - | Rel-16 | F | NR_RRM_enh-Core | not pursued |
| R4-2100215 | CR to 38.133 on UL spatial relation switch requirements (R17) | Apple | 38.133 | 1460 | - | Rel-17 | A | NR_RRM_enh-Core | withdrawn |
| R4-2100228 | CR on activation time in direct SCell activation | Apple | 38.133 | 1461 | - | Rel-16 | F | LTE_NR_DC_CA_enh-Core | revised |
| R4-2103553 | CR on activation time in direct SCell activation | Apple | 38.133 | 1461 | 1 | Rel-16 | F | LTE_NR_DC_CA_enh-Core | revised |
| R4-2104071 | CR on activation time in direct SCell activation | Apple | 38.133 | 1461 | 2 | Rel-16 | F | LTE_NR_DC_CA_enh-Core | postponed |
| R4-2100229 | CR on activation time in direct SCell activation (R17) | Apple | 38.133 | 1462 | - | Rel-16 | A | LTE_NR_DC_CA_enh-Core | withdrawn |
| R4-2100233 | Interruption requirements due to measurement on SCC in NR-DC (R15) | Apple | 38.133 | 1463 | - | Rel-15 | F | NR_newRAT-Core | postponed |
| R4-2100234 | Interruption requirements maintenance in NR-DC (R16) | Apple | 38.133 | 1464 | - | Rel-16 | F | TEI16 | revised |
| R4-2103507 | Interruption requirements maintenance in NR-DC (R16) | Apple | 38.133 | 1464 | 1 | Rel-16 | F | TEI16 | agreed |
| R4-2100235 | Interruption requirements maintenance in NR-DC (R17) | Apple | 38.133 | 1465 | - | Rel-16 | A | TEI16 | withdrawn |
| R4-2100238 | CR on HST core part maintenance | Apple | 38.133 | 1466 | - | Rel-16 | F | NR_HST-Core | revised |
| R4-2103655 | CR on HST core part maintenance | Apple | 38.133 | 1466 | 1 | Rel-16 | F | NR_HST-Core | agreed |
| R4-2100239 | CR on HST core part maintenance (R17) | Apple | 38.133 | 1467 | - | Rel-16 | A | NR_HST-Core | withdrawn |
| R4-2100242 | CR on CSSF with CSI-RS for L3 measurement | Apple | 38.133 | 1468 | - | Rel-16 | F | NR_CSIRS_L3meas-Core | withdrawn |
| R4-2100247 | CR for TS 38.133 introduction of NR band n24 | Ligado Networks | 38.133 | 1469 | - | Rel-17 | B | NR_band_n24-Core | agreed |
| R4-2100422 | CR on CSI-RS based L3 measurement | CATT | 38.133 | 1470 | - | Rel-16 | F | NR_CSIRS_L3meas-Core | revised |
| R4-2103631 | CR on CSI-RS based L3 measurement | CATT | 38.133 | 1470 | 1 | Rel-16 | F | NR_CSIRS_L3meas-Core | agreed |
| R4-2100429 | CR on performance requirement for CSI-RSRP | CATT | 38.133 | 1471 | - | Rel-16 | B | NR_CSIRS_L3meas-Perf | postponed |
| R4-2100430 | CR on performance requirement for CSI-RSRQ | CATT | 38.133 | 1472 | - | Rel-16 | B | NR_CSIRS_L3meas-Perf | revised |
| R4-2103636 | CR on performance requirement for CSI-RSRQ | CATT | 38.133 | 1472 | 1 | Rel-16 | B | NR_CSIRS_L3meas-Perf | endorsed |
| R4-2100431 | CR on performance requirement for CSI-SINR | CATT | 38.133 | 1473 | - | Rel-16 | B | NR_CSIRS_L3meas-Perf | postponed |
| R4-2100433 | CR on test case for CSI-RS based intra-frequency measurement for FR1 in SA | CATT | 38.133 | 1474 | - | Rel-16 | B | NR_CSIRS_L3meas-Perf | revised |
| R4-2103639 | CR on test case for CSI-RS based intra-frequency measurement for FR1 in SA | CATT | 38.133 | 1474 | 1 | Rel-16 | B | NR_CSIRS_L3meas-Perf | endorsed |
| R4-2100434 | CR on test case for CSI-RS based inter-frequency measurement for FR1 in SA | CATT | 38.133 | 1475 | - | Rel-16 | B | NR_CSIRS_L3meas-Perf | revised |
| R4-2103642 | CR on test case for CSI-RS based inter-frequency measurement for FR1 in SA | CATT | 38.133 | 1475 | 1 | Rel-16 | B | NR_CSIRS_L3meas-Perf | endorsed |
| R4-2100435 | CR on test case for CSI-RSRP measurement accuracy requirements in SA | CATT | 38.133 | 1476 | - | Rel-16 | B | NR_CSIRS_L3meas-Perf | revised |
| R4-2103645 | CR on test case for CSI-RSRP measurement accuracy requirements in SA | CATT | 38.133 | 1476 | 1 | Rel-16 | B | NR_CSIRS_L3meas-Perf | endorsed |
| R4-2100438 | CR on PRS RSTD measurement requirements | CATT | 38.133 | 1477 | - | Rel-16 | F | NR_pos-Core | revised |
| R4-2103578 | CR on PRS RSTD measurement requirements | CATT | 38.133 | 1477 | 1 | Rel-16 | F | NR_pos-Core | agreed |
| R4-2100439 | CR on UE Rx-Tx time difference measurement requirements | CATT | 38.133 | 1478 | - | Rel-16 | F | NR_pos-Core | merged |
| R4-2100446 | CR on PRS-RSRP accuracy requirements | CATT | 38.133 | 1479 | - | Rel-16 | B | NR_pos-Perf | revised |
| R4-2103732 | CR on PRS-RSRP accuracy requirements | CATT | 38.133 | 1479 | 1 | Rel-16 | B | NR_pos-Perf | endorsed |
| R4-2100447 | CR on PRS configuration for test cases | CATT | 38.133 | 1480 | - | Rel-16 | B | NR_pos-Perf | merged |
| R4-2100448 | CR on test case for PRS-RSRP measurement requirements for FR2 in SA | CATT | 38.133 | 1481 | - | Rel-16 | B | NR_pos-Perf | revised |
| R4-2103734 | CR on test case for PRS-RSRP measurement requirements for FR2 in SA | CATT | 38.133 | 1481 | 1 | Rel-16 | B | NR_pos-Perf | postponed |
| R4-2100461 | CR on CSI-RS based L3 measurement | CATT | 38.133 | 1482 | - | Rel-17 | A | NR_CSIRS_L3meas-Core | agreed |
| R4-2100462 | CR on performance requirement for CSI-RSRP | CATT | 38.133 | 1483 | - | Rel-17 | A | NR_CSIRS_L3meas-Perf | withdrawn |
| R4-2100463 | CR on performance requirement for CSI-RSRQ | CATT | 38.133 | 1484 | - | Rel-17 | A | NR_CSIRS_L3meas-Perf | withdrawn |
| R4-2100464 | CR on performance requirement for CSI-SINR | CATT | 38.133 | 1485 | - | Rel-17 | A | NR_CSIRS_L3meas-Perf | withdrawn |
| R4-2100465 | CR on test case for CSI-RS based intra-frequency measurement for FR1 in SA | CATT | 38.133 | 1486 | - | Rel-17 | A | NR_CSIRS_L3meas-Perf | withdrawn |
| R4-2100466 | CR on test case for CSI-RS based inter-frequency measurement for FR1 in SA | CATT | 38.133 | 1487 | - | Rel-17 | A | NR_CSIRS_L3meas-Perf | withdrawn |
| R4-2100467 | CR on test case for CSI-RSRP measurement accuracy requirements in SA | CATT | 38.133 | 1488 | - | Rel-17 | A | NR_CSIRS_L3meas-Perf | withdrawn |
| R4-2100468 | CR on PRS RSTD measurement requirements | CATT | 38.133 | 1489 | - | Rel-17 | A | NR_pos-Core | agreed |
| R4-2100469 | CR on UE Rx-Tx time difference measurement requirements | CATT | 38.133 | 1490 | - | Rel-17 | A | NR_pos-Core | withdrawn |
| R4-2100470 | CR on PRS-RSRP accuracy requirements | CATT | 38.133 | 1491 | - | Rel-17 | A | NR_pos-Perf | withdrawn |
| R4-2100471 | CR on PRS configuration for test cases | CATT | 38.133 | 1492 | - | Rel-17 | A | NR_pos-Perf | withdrawn |
| R4-2100472 | CR on test case for PRS-RSRP measurement requirements for FR2 in SA | CATT | 38.133 | 1493 | - | Rel-17 | A | NR_pos-Perf | withdrawn |
| R4-2100479 | Correction to cell reselection test case | CATT | 38.133 | 1494 | - | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2100480 | Correction to cell reselection test case | CATT | 38.133 | 1495 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2100481 | Correction to cell reselection test case | CATT | 38.133 | 1496 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2100482 | Correction to cell reselection test case for UE Power saving | CATT | 38.133 | 1497 | - | Rel-16 | F | NR_UE_pow_sav-Perf | revised |
| R4-2103574 | Correction to cell reselection test case for UE Power saving | CATT | 38.133 | 1497 | 1 | Rel-16 | F | NR_UE_pow_sav-Perf | agreed |
| R4-2100483 | Correction to cell reselection test case for UE Power saving | CATT | 38.133 | 1498 | - | Rel-17 | A | NR_UE_pow_sav-Perf | agreed |
| R4-2100484 | Correction to cell reselection test case for HST | CATT | 38.133 | 1499 | - | Rel-16 | F | NR_HST-Perf | agreed |
| R4-2100485 | Correction to cell reselection test case for HST | CATT | 38.133 | 1500 | - | Rel-17 | A | NR_HST-Perf | withdrawn |
| R4-2100580 | 2-step RACH RRM performance requirements corrections | Nokia, Nokia Shanghai Bell | 38.133 | 1501 | - | Rel-16 | F | NR_2step_RACH-Perf | agreed |
| R4-2100581 | 2-step RACH RRM performance requirements corrections | Nokia, Nokia Shanghai Bell | 38.133 | 1502 | - | Rel-17 | A | NR_2step_RACH-Perf | agreed |
| R4-2100601 | Update of DRX configuration in FR1 Event-triggered Test cases | ANRITSU LTD | 38.133 | 1503 | - | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2100602 | Update of DRX configuration in FR1 Event-triggered Test cases | ANRITSU LTD | 38.133 | 1504 | - | Rel-16 | F | NR_newRAT-Perf | agreed |
| R4-2100603 | Update of DRX configuration in FR1 Event-triggered Test cases | ANRITSU LTD | 38.133 | 1505 | - | Rel-17 | F | NR_newRAT-Perf | agreed |
| R4-2100638 | Big CR-Introduction of NR V2X RRM performance requirements (Rel-16) | LG Electronics | 38.133 | 1506 | - | Rel-16 | B | 5G_V2X_NRSL-Perf | revised |
| R4-2103538 | Big CR-Introduction of NR V2X RRM performance requirements (Rel-16) | LG Electronics | 38.133 | 1506 | 1 | Rel-16 | B | 5G_V2X_NRSL-Perf | agreed |
| R4-2100639 | Big CR-Introduction of NR V2X RRM performance requirements (Rel-17) | LG Electronics | 38.133 | 1507 | - | Rel-17 | A | 5G_V2X_NRSL-Perf | agreed |
| R4-2100756 | CR on the scaling factor for SCell beam failure recovery in R16 | MediaTek inc. | 38.133 | 1508 | - | Rel-16 | F | NR_eMIMO-Core | merged |
| R4-2100757 | CR on the scaling factor for SCell beam failure recovery in R17 | MediaTek inc. | 38.133 | 1509 | - | Rel-17 | A | NR_eMIMO-Core | withdrawn |
| R4-2100758 | Correction on the measurement restriction for CSI-IM resource in R16 | MediaTek inc. | 38.133 | 1510 | - | Rel-16 | F | NR_eMIMO-Core | agreed |
| R4-2100759 | Correction on the measurement restriction for CSI-IM resource in R17 | MediaTek inc. | 38.133 | 1511 | - | Rel-17 | A | NR_eMIMO-Core | agreed |
| R4-2100760 | Correction on PRACH configuration for FR2 Non-Contention based Random Access in R15 | MediaTek inc. | 38.133 | 1512 | - | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2100761 | Correction on PRACH configuration for FR2 Non-Contention based Random Access in R16 | MediaTek inc. | 38.133 | 1513 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2100762 | Correction on PRACH configuration for FR2 Non-Contention based Random Access in R17 | MediaTek inc. | 38.133 | 1514 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2100763 | Correction on PRACH configuration for Beam Failure Detection and Link Recovery Test in R15 | MediaTek inc. | 38.133 | 1515 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2104049 | Correction on PRACH configuration for Beam Failure Detection and Link Recovery Test in R15 | MediaTek inc. | 38.133 | 1515 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2100764 | Correction on PRACH configuration for Beam Failure Detection and Link Recovery Test in R16 | MediaTek inc. | 38.133 | 1516 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2100765 | Correction on PRACH configuration for Beam Failure Detection and Link Recovery Test in R17 | MediaTek inc. | 38.133 | 1517 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2100766 | Correction on PRACH RMC for FR1 CSI-RS based Non-Contention based Random Access for BFR in R15 | MediaTek inc. | 38.133 | 1518 | - | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2100767 | Correction on PRACH RMC for FR1 CSI-RS based Non-Contention based Random Access for BFR in R16 | MediaTek inc. | 38.133 | 1519 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2100768 | Correction on PRACH RMC for FR1 CSI-RS based Non-Contention based Random Access for BFR in R17 | MediaTek inc. | 38.133 | 1520 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2100778 | Correction on scheduling availability and measurement restriction on FR2 inter-band CA in R16 | MediaTek inc. | 38.133 | 1521 | - | Rel-16 | F | NR_RRM_enh-Core | revised |
| R4-2103623 | Correction on scheduling availability and measurement restriction on FR2 inter-band CA in R16 | MediaTek inc. | 38.133 | 1521 | 1 | Rel-16 | F | NR_RRM_enh-Core | agreed |
| R4-2100779 | Correction on scheduling availability and measurement restriction on FR2 inter-band CA in R17 | MediaTek inc. | 38.133 | 1522 | - | Rel-17 | A | NR_RRM_enh-Core | agreed |
| R4-2100813 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | 38.133 | 1523 | - | Rel-15 | F | TEI | withdrawn |
| R4-2100814 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | 38.133 | 1524 | - | Rel-16 | F | TEI | withdrawn |
| R4-2100815 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | 38.133 | 1525 | - | Rel-17 | F | TEI | withdrawn |
| R4-2100849 | CR on HST RRM requirements in connected mode | CMCC | 38.133 | 1526 | - | Rel-16 | F | NR_HST-Core | revised |
| R4-2103654 | CR on HST RRM requirements in connected mode | CMCC | 38.133 | 1526 | 1 | Rel-16 | F | NR_HST-Core | agreed |
| R4-2100850 | CR on HST RRM requirements in connected mode | CMCC | 38.133 | 1527 | - | Rel-17 | A | NR_HST-Core | agreed |
| R4-2100852 | CR on deactivated SCell measurement for intra-frequency measurement with measurement gap | CMCC | 38.133 | 1528 | - | Rel-15 | F | NR_newRAT-Core | merged |
| R4-2100853 | CR on deactivated SCell measurement for intra-frequency measurement with measurement gap | CMCC | 38.133 | 1529 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2100873 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | 38.133 | 1530 | - | Rel-15 | F | NR_newRAT-Core | withdrawn |
| R4-2100874 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | 38.133 | 1531 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2100875 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | 38.133 | 1532 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2100933 | CR to TS38.133 on L1-SINR measurement requirement | Samsung | 38.133 | 1533 | - | Rel-16 | F | NR_eMIMO-Core | revised |
| R4-2103590 | CR to TS38.133 on L1-SINR measurement requirement | Samsung | 38.133 | 1533 | 1 | Rel-16 | F | NR_eMIMO-Core | agreed |
| R4-2100934 | CR to TS38.133 on L1-SINR measurement requirement | Samsung | 38.133 | 1534 | - | Rel-17 | A | NR_eMIMO-Core | agreed |
| R4-2100938 | Big CR: Introduction of Rel-16 NR eMIMO RRM performance requirements and test cases | Samsung | 38.133 | 1535 | - | Rel-16 | B | NR_eMIMO-Perf | agreed |
| R4-2100939 | Big CR: Introduction of Rel-16 NR eMIMO RRM performance requirements and test cases | Samsung | 38.133 | 1536 | - | Rel-17 | A | NR_eMIMO-Perf | agreed |
| R4-2101006 | CR on Scell activation delay maintenance (R15) | Apple | 38.133 | 1537 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2104043 | CR on Scell activation delay maintenance (R15) | Apple | 38.133 | 1537 | 1 | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2104084 | CR on Scell activation delay maintenance (R15) | Apple | 38.133 | 1537 | 2 | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2101007 | CR on Scell activation delay maintenance (R16) | Apple | 38.133 | 1538 | - | Rel-16 | F | NR_newRAT-Core | agreed |
| R4-2101008 | CR on Scell activation delay maintenance (R17) | Apple | 38.133 | 1539 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2101009 | CR on maintenance for DAPS handover (R16) | Apple | 38.133 | 1540 | - | Rel-16 | F | NR_Mob_enh-Core | merged |
| R4-2101010 | CR on maintenance for DAPS handover (R17) | Apple | 38.133 | 1541 | - | Rel-17 | A | NR_Mob_enh-Core | withdrawn |
| R4-2101011 | Interruption requirements maintenance in NR-DC (R17) | Apple | 38.133 | 1542 | - | Rel-17 | A | TEI16 | agreed |
| R4-2101012 | CR on activation time in direct SCell activation (R17) | Apple | 38.133 | 1543 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Core | withdrawn |
| R4-2101013 | CR on HST core part maintenance (R17) | Apple | 38.133 | 1544 | - | Rel-17 | A | NR_HST-Core | agreed |
| R4-2101047 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | 38.133 | 1545 | - | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2101048 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | 38.133 | 1546 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2101049 | CR for test requirements correction of SA event triggered reporting tests for FR1 inter-frequency measurements with SSB time index detection when DRX is used | CMCC | 38.133 | 1547 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2101051 | CR on R15 remaining issues | MediaTek inc. | 38.133 | 1548 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103481 | CR on R15 remaining issues | MediaTek inc. | 38.133 | 1548 | 1 | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2101052 | CR on R15 remaining issues | MediaTek inc. | 38.133 | 1549 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2101053 | CR on R15 remaining issues | MediaTek inc. | 38.133 | 1550 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2101054 | CR on V2X interruption | Mediatek Inc., LG Electronics, Qualcomm Inc. | 38.133 | 1551 | - | Rel-16 | B | 5G_V2X_NRSL-Core | revised |
| R4-2103536 | CR on V2X interruption | Mediatek Inc., LG Electronics, Qualcomm Inc. | 38.133 | 1551 | 1 | Rel-16 | B | 5G_V2X_NRSL-Core | agreed |
| R4-2101055 | CR on V2X interruption | Mediatek Inc., LG Electronics, Qualcomm Inc. | 38.133 | 1552 | - | Rel-17 | A | 5G_V2X_NRSL-Core | agreed |
| R4-2101071 | CR on SSB less SCell activation for FR1 for Rel-15 | NEC | 38.133 | 1553 | - | Rel-15 | F | NR_newRAT-Core | merged |
| R4-2101072 | CR on SSB less SCell activation for FR1 for Rel-16 | NEC | 38.133 | 1554 | - | Rel-16 | F | NR_newRAT-Core | merged |
| R4-2101075 | CR for measurement period requirements correction | NEC | 38.133 | 1555 | - | Rel-16 | F | TEI16, NR_newRAT-Core | revised |
| R4-2103508 | CR for measurement period requirements correction | NEC | 38.133 | 1555 | 1 | Rel-16 | F | TEI16, NR_newRAT-Core | agreed |
| R4-2101076 | CR for measurement period requirements correction | NEC | 38.133 | 1556 | - | Rel-17 | A | TEI16, NR_newRAT-Core | agreed |
| R4-2101100 | [CR] Add Random Access requirements under NR-U (Cat A) | ZTE Corporation | 38.133 | 1557 | - | Rel-17 | A | NR_unlic-Core | withdrawn |
| R4-2101132 | CR to 38.133 - Introducing NR-U random access requirements | Nokia | 38.133 | 1558 | - | Rel-16 | B | NR_unlic-Core | postponed |
| R4-2101148 | Update on interruption test cases for Tx switching R16 | MediaTek inc. | 38.133 | 1559 | - | Rel-16 | F | NR_RF_FR1-Perf | revised |
| R4-2103600 | Update on interruption test cases for Tx switching R16 | MediaTek inc. | 38.133 | 1559 | 1 | Rel-16 | F | NR_RF_FR1-Perf | agreed |
| R4-2101149 | Update on interruption test cases for Tx switching R17 | MediaTek inc. | 38.133 | 1560 | - | Rel-17 | A | NR_RF_FR1-Perf | agreed |
| R4-2101150 | Maintenance CR for CSI-RS based L3 measurement requirements R16 | MediaTek inc. | 38.133 | 1561 | - | Rel-16 | F | NR_CSIRS_L3meas-Core | revised |
| R4-2103633 | Maintenance CR for CSI-RS based L3 measurement requirements R16 | MediaTek inc. | 38.133 | 1561 | 1 | Rel-16 | F | NR_CSIRS_L3meas-Core | agreed |
| R4-2101151 | Maintenance CR for CSI-RS based L3 measurement requirements R17 | MediaTek inc. | 38.133 | 1562 | - | Rel-17 | A | NR_CSIRS_L3meas-Core | agreed |
| R4-2101161 | Correction on the power of the first preamble for random access in EN-DC and SA in R15 | MediaTek inc. | 38.133 | 1563 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103487 | Correction on the power of the first preamble for random access in EN-DC and SA in R15 | MediaTek inc. | 38.133 | 1563 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2101162 | Correction on the power of the first preamble for random access in EN-DC and SA in R16 | MediaTek inc. | 38.133 | 1564 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2101163 | Correction on the power of the first preamble for random access in EN-DC and SA in R17 | MediaTek inc. | 38.133 | 1565 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2101164 | Correction on the time for Scell activation and CSI-report in R15 | MediaTek inc. | 38.133 | 1566 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103488 | Correction on the time for Scell activation and CSI-report in R15 | MediaTek inc. | 38.133 | 1566 | 1 | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2104082 | Correction on the time for Scell activation and CSI-report in R15 | MediaTek inc. | 38.133 | 1566 | 2 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2101165 | Correction on the time for Scell activation and CSI-report in R16 | MediaTek inc. | 38.133 | 1567 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2101166 | Correction on the time for Scell activation and CSI-report in R17 | MediaTek inc. | 38.133 | 1568 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2101167 | Correction on the Noc level in TS38.133 in R15 | MediaTek inc. | 38.133 | 1569 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103489 | Correction on the Noc level in TS38.133 in R15 | MediaTek inc. | 38.133 | 1569 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2101168 | Correction on the Noc level in TS38.133 in R16 | MediaTek inc. | 38.133 | 1570 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2101169 | Correction on the Noc level in TS38.133 in R17 | MediaTek inc. | 38.133 | 1571 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2101170 | Correction on the SS-RSRP difference value for SS-RSRP measurement TC in R15 | MediaTek inc. | 38.133 | 1572 | - | Rel-15 | F | NR_newRAT-Perf | postponed |
| R4-2103490 | Correction on the SS-RSRP difference value for SS-RSRP measurement TC in R15 | MediaTek inc. | 38.133 | 1572 | 1 | Rel-15 | F | NR_newRAT-Perf | withdrawn |
| R4-2101171 | Correction on the SS-RSRP difference value for SS-RSRP measurement TC in R16 | MediaTek inc. | 38.133 | 1573 | - | Rel-16 | A | NR_newRAT-Perf | withdrawn |
| R4-2101172 | Correction on the SS-RSRP difference value for SS-RSRP measurement TC in R17 | MediaTek inc. | 38.133 | 1574 | - | Rel-17 | A | NR_newRAT-Perf | withdrawn |
| R4-2101205 | CR on TS38.133 for dual active protocol stack handover | Ericsson | 38.133 | 1575 | - | Rel-16 | F | NR_Mob_enh-Core | merged |
| R4-2101206 | CR on TS38.133 for dual active protocol stack handover | Ericsson | 38.133 | 1576 | - | Rel-17 | A | NR_Mob_enh-Core | withdrawn |
| R4-2101210 | CR on TS38.133 for Pcell change | MediaTek inc. | 38.133 | 1577 | - | Rel-16 | F | NR_Mob_enh-Core | revised |
| R4-2104050 | CR on TS38.133 for Pcell change | MediaTek inc. | 38.133 | 1577 | 1 | Rel-16 | F | NR_Mob_enh-Core | agreed |
| R4-2101211 | CR on TS38.133 for Pcell change | MediaTek inc. | 38.133 | 1578 | - | Rel-17 | A | NR_Mob_enh-Core | agreed |
| R4-2101213 | CR on TS38.133 for direct Scell activation | MediaTek inc. | 38.133 | 1579 | - | Rel-16 | F | LTE_NR_DC_CA_enh-Core | postponed |
| R4-2101214 | CR on TS38.133 for direct Scell activation | MediaTek inc. | 38.133 | 1580 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Core | withdrawn |
| R4-2101215 | CR on TS38.133 for direct SCell activation of SCell in FR2 intra-band in ENDC mode (A.5.5.X) | MediaTek inc. | 38.133 | 1581 | - | Rel-16 | B | LTE_NR_DC_CA_enh-Perf | revised |
| R4-2103558 | CR on TS38.133 for direct SCell activation of SCell in FR2 intra-band in ENDC mode (A.5.5.X) | MediaTek inc. | 38.133 | 1581 | 1 | Rel-16 | B | LTE_NR_DC_CA_enh-Perf | endorsed |
| R4-2101216 | CR on TS38.133 for direct SCell activation of SCell in FR2 intra-band in ENDC mode (A.5.5.X) | MediaTek inc. | 38.133 | 1582 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Perf | withdrawn |
| R4-2101217 | CR on TS38.133 for E-UTRAN | MediaTek inc. | 38.133 | 1583 | - | Rel-16 | B | LTE_NR_DC_CA_enh-Perf | revised |
| R4-2103565 | CR on TS38.133 for E-UTRAN | MediaTek inc. | 38.133 | 1583 | 1 | Rel-16 | B | LTE_NR_DC_CA_enh-Perf | endorsed |
| R4-2101218 | CR on TS38.133 for E-UTRAN | MediaTek inc. | 38.133 | 1584 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Perf | withdrawn |
| R4-2101219 | CR on TS38.133 for inter-frequency measurement requirement without gap | MediaTek inc. | 38.133 | 1585 | - | Rel-16 | F | NR_RRM_enh-Core | revised |
| R4-2103624 | CR on TS38.133 for inter-frequency measurement requirement without gap | MediaTek inc. | 38.133 | 1585 | 1 | Rel-16 | F | NR_RRM_enh-Core | agreed |
| R4-2101220 | CR on TS38.133 for inter-frequency measurement requirement without gap | MediaTek inc. | 38.133 | 1586 | - | Rel-17 | A | NR_RRM_enh-Core | agreed |
| R4-2101291 | Big CR: Introduction of Rel-16 CSI-RS based L3 measurement RRM performance requirements | CATT | 38.133 | 1587 | - | Rel-16 | B | NR_CSIRS_L3meas-Perf | endorsed |
| R4-2101292 | Big CR: Introduction of Rel-16 CSI-RS based L3 measurement RRM test cases | CATT | 38.133 | 1588 | - | Rel-16 | B | NR_CSIRS_L3meas-Perf | withdrawn |
| R4-2101383 | CR for removing K2 for R16 UE power saving | vivo | 38.133 | 1589 | - | Rel-16 | F | NR_UE_pow_sav-Core | postponed |
| R4-2101384 | CR for removing K2 for R16 UE power saving | vivo | 38.133 | 1590 | - | Rel-17 | A | NR_UE_pow_sav-Core | postponed |
| R4-2101385 | CR for modifications on FR1 intra-frequency UE power saving test cases | vivo | 38.133 | 1591 | - | Rel-16 | F | NR_UE_pow_sav-Core | merged |
| R4-2101386 | CR for modifications on FR1 intra-frequency UE power saving test cases | vivo | 38.133 | 1592 | - | Rel-17 | A | NR_UE_pow_sav-Core | postponed |
| R4-2101388 | CR for adding capability D | vivo | 38.133 | 1593 | - | Rel-16 | F | LTE_NR_DC_CA_enh-Core | not pursued |
| R4-2101389 | CR for adding capability D | vivo | 38.133 | 1594 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Core | not pursued |
| R4-2101390 | CR for test cases for simultaneously DCI/timer based bwp switch over mulitple CCs on FR1 in SA | vivo | 38.133 | 1595 | - | Rel-16 | B | NR_RRM_enh-Perf | endorsed |
| R4-2103604 | CR for test cases for simultaneously DCI/timer based bwp switch over mulitple CCs on FR1 in SA | vivo | 38.133 | 1595 | 1 | Rel-16 | B | NR_RRM_enh-Perf | withdrawn |
| R4-2101394 | 38.133 CR on the CSI-RS based measurement requirements | Nokia, Nokia Shanghai Bell | 38.133 | 1596 | - | Rel-16 | F | NR_CSIRS_L3meas-Core | revised |
| R4-2103634 | 38.133 CR on the CSI-RS based measurement requirements | Nokia, Nokia Shanghai Bell | 38.133 | 1596 | 1 | Rel-16 | F | NR_CSIRS_L3meas-Core | revised |
| R4-2104093 | 38.133 CR on the CSI-RS based measurement requirements | Nokia, Nokia Shanghai Bell | 38.133 | 1596 | 2 | Rel-16 | F | NR_CSIRS_L3meas-Core | agreed |
| R4-2101409 | CR on RRC based BWP switching on multiple CCs (Rel-16) | Intel Corporation | 38.133 | 1597 | - | Rel-16 | B | NR_RRM_enh-Core | not pursued |
| R4-2101410 | CR on RRC based BWP switching on multiple CCs (Rel-17) | Intel Corporation | 38.133 | 1598 | - | Rel-17 | A | NR_RRM_enh-Core | withdrawn |
| R4-2101416 | Big CR: Introduction of Rel-16 NR RRM enhancements WI performance requirements and test cases (Rel-16) | Intel Corporation, ZTE Corporation, Apple | 38.133 | 1599 | - | Rel-16 | B | NR_RRM_enh-Perf | revised |
| R4-2103608 | Big CR: Introduction of Rel-16 NR RRM enhancements WI performance requirements and test cases (Rel-16) | Intel Corporation, ZTE Corporation, Apple | 38.133 | 1599 | 1 | Rel-16 | B | NR_RRM_enh-Perf | agreed |
| R4-2101417 | Big CR: Introduction of Rel-16 NR RRM enhancements WI performance requirements and test cases (Rel-17) | Intel Corporation, ZTE Corporation, Apple | 38.133 | 1600 | - | Rel-17 | A | NR_RRM_enh-Perf | agreed |
| R4-2101425 | CR: Introduction of random access requirements with CCA | Ericsson | 38.133 | 1601 | - | Rel-16 | B | NR_unlic-Core | revised |
| R4-2103721 | CR: Introduction of random access requirements with CCA | Ericsson | 38.133 | 1601 | 1 | Rel-16 | B | NR_unlic-Core | agreed |
| R4-2101426 | CR: Introduction of random access requirements with CCA | Ericsson | 38.133 | 1602 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2101428 | CR: Beam management requirements with CCA | Ericsson | 38.133 | 1603 | - | Rel-16 | F | NR_unlic-Core | revised |
| R4-2103513 | CR: Beam management requirements with CCA | Ericsson | 38.133 | 1603 | 1 | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2101429 | CR: Beam management requirements with CCA | Ericsson | 38.133 | 1604 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2101464 | CR on the filter for beam failure indications in 38.133 | vivo | 38.133 | 1605 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103484 | CR on the filter for beam failure indications in 38.133 | vivo | 38.133 | 1605 | 1 | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2101465 | CR on the filter for beam failure indications in 38.133 | vivo | 38.133 | 1606 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2101466 | CR on the filter for beam failure indications in 38.133 | vivo | 38.133 | 1607 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2101527 | CR to TS 38.133 on UE Rx-Tx time difference measurements (section 9.9.4) | OPPO | 38.133 | 1608 | - | Rel-16 | F | NR_pos-Core | revised |
| R4-2103580 | CR to TS 38.133 on UE Rx-Tx time difference measurements (section 9.9.4) | OPPO | 38.133 | 1608 | 1 | Rel-16 | F | NR_pos-Core | agreed |
| R4-2101530 | Maintenance CR on interruption at EUTRA SRS carrier switching in 38.133 | OPPO | 38.133 | 1609 | - | Rel-16 | F | TEI16 | revised |
| R4-2103509 | Maintenance CR on interruption at EUTRA SRS carrier switching in 38.133 | OPPO | 38.133 | 1609 | 1 | Rel-16 | F | TEI16 | agreed |
| R4-2101531 | Maintenance CR on SCell activation delay requirement in TS38.133 | OPPO | 38.133 | 1610 | - | Rel-16 | F | TEI16 | revised |
| R4-2103510 | Maintenance CR on SCell activation delay requirement in TS38.133 | OPPO | 38.133 | 1610 | 1 | Rel-16 | F | TEI16 | agreed |
| R4-2101533 | CR for CSI-RS L3 RRM tests | OPPO, CATT | 38.133 | 1611 | - | Rel-16 | F | NR_CSIRS_L3meas-Perf | endorsed |
| R4-2101534 | CR on EN-DC tests for NR inter-frequency neighbor cell in FR2(PScell in FR2) | OPPO | 38.133 | 1612 | - | Rel-16 | F | NR_CSIRS_L3meas-Perf | revised |
| R4-2103644 | CR on EN-DC tests for NR inter-frequency neighbor cell in FR2(PScell in FR2) | OPPO | 38.133 | 1612 | 1 | Rel-16 | F | NR_CSIRS_L3meas-Perf | endorsed |
| R4-2101535 | CR on EN-DC tests for CSI-RSRQ accuracy | OPPO | 38.133 | 1613 | - | Rel-16 | F | NR_CSIRS_L3meas-Perf | revised |
| R4-2103648 | CR on EN-DC tests for CSI-RSRQ accuracy | OPPO | 38.133 | 1613 | 1 | Rel-16 | F | NR_CSIRS_L3meas-Perf | endorsed |
| R4-2101612 | Correction to Aperiodic CSI-RS configurations R15 | Huawei, HiSilicon | 38.133 | 1614 | - | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2101613 | Correction to Aperiodic CSI-RS configurations R16 | Huawei, HiSilicon | 38.133 | 1615 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2101614 | Correction to Aperiodic CSI-RS configurations R17 | Huawei, HiSilicon | 38.133 | 1616 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2101615 | Correction to radio link monitoring test cases R15 | Huawei, HiSilicon | 38.133 | 1617 | - | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2101616 | Correction to radio link monitoring test cases R16 | Huawei, HiSilicon | 38.133 | 1618 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2101617 | Correction to radio link monitoring test cases R17 | Huawei, HiSilicon | 38.133 | 1619 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2101618 | Correction to beam failure recovery test cases R15 | Huawei, HiSilicon | 38.133 | 1620 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103491 | Correction to beam failure recovery test cases R15 | Huawei, HiSilicon | 38.133 | 1620 | 1 | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2104065 | Correction to beam failure recovery test cases R15 | Huawei, HiSilicon | 38.133 | 1620 | 2 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2101619 | Correction to beam failure recovery test cases R16 | Huawei, HiSilicon | 38.133 | 1621 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2101620 | Correction to beam failure recovery test cases R17 | Huawei, HiSilicon | 38.133 | 1622 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2101621 | Correction to L1-RSRP reporting delay test cases R15 | Huawei, HiSilicon | 38.133 | 1623 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103492 | Correction to L1-RSRP reporting delay test cases R15 | Huawei, HiSilicon | 38.133 | 1623 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2101622 | Correction to L1-RSRP reporting delay test cases R16 | Huawei, HiSilicon | 38.133 | 1624 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2101623 | Correction to L1-RSRP reporting delay test cases R17 | Huawei, HiSilicon | 38.133 | 1625 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2101624 | Correction to relexed cell reselection requirements R16 | Huawei, HiSilicon | 38.133 | 1626 | - | Rel-16 | F | NR_UE_pow_sav-Core | merged |
| R4-2101625 | Correction to relexed cell reselection requirements R17 | Huawei, HiSilicon | 38.133 | 1627 | - | Rel-17 | A | NR_UE_pow_sav-Core | withdrawn |
| R4-2101638 | CR on requirement maintenance for SCell activation and deactivation for NR-U | Huawei, HiSilicon | 38.133 | 1628 | - | Rel-16 | F | NR_unlic-Core | merged |
| R4-2101639 | CR on requirement maintenance for SCell activation and deactivation for NR-U | Huawei, HiSilicon | 38.133 | 1629 | - | Rel-17 | A | NR_unlic-Core | withdrawn |
| R4-2101641 | CR on measurement requirements for NR-U | Huawei, HiSilicon | 38.133 | 1630 | - | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2101642 | CR on measurement requirements for NR-U | Huawei, HiSilicon | 38.133 | 1631 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2101644 | CR on timing requirements for NR-U | Huawei, HiSilicon | 38.133 | 1632 | - | Rel-16 | F | NR_unlic-Core | postponed |
| R4-2101645 | CR on timing requirements for NR-U | Huawei, HiSilicon | 38.133 | 1633 | - | Rel-17 | A | NR_unlic-Core | withdrawn |
| R4-2101661 | CR on maintaining Antenna configurations in TS38.133 R15 | Huawei, HiSilicon | 38.133 | 1634 | - | Rel-15 | F | NR_RRM_enh-Perf | revised |
| R4-2103493 | CR on maintaining Antenna configurations in TS38.133 R15 | Huawei, HiSilicon | 38.133 | 1634 | 1 | Rel-15 | F | NR_RRM_enh-Perf | revised |
| R4-2104056 | CR on maintaining Antenna configurations in TS38.133 R15 | Huawei, HiSilicon | 38.133 | 1634 | 2 | Rel-15 | F | NR_RRM_enh-Perf | agreed |
| R4-2101662 | CR on maintaining Antenna configurations in TS38.133 R16 | Huawei, HiSilicon | 38.133 | 1635 | - | Rel-16 | A | NR_RRM_enh-Perf | agreed |
| R4-2101663 | CR on maintaining Antenna configurations in TS38.133 R17 | Huawei, HiSilicon | 38.133 | 1636 | - | Rel-17 | A | NR_RRM_enh-Perf | agreed |
| R4-2101664 | CR on test requirements for measurement performance tests R15 | Huawei, HiSilicon | 38.133 | 1637 | - | Rel-15 | F | NR_RRM_enh-Perf | revised |
| R4-2103494 | CR on test requirements for measurement performance tests R15 | Huawei, HiSilicon | 38.133 | 1637 | 1 | Rel-15 | F | NR_RRM_enh-Perf | agreed |
| R4-2101665 | CR on test requirements for measurement performance tests R16 | Huawei, HiSilicon | 38.133 | 1638 | - | Rel-16 | A | NR_RRM_enh-Perf | agreed |
| R4-2101666 | CR on test requirements for measurement performance tests R17 | Huawei, HiSilicon | 38.133 | 1639 | - | Rel-17 | A | NR_RRM_enh-Perf | agreed |
| R4-2101668 | CR on sync conditions for intra-band DAPS handover R16 | Huawei, HiSilicon | 38.133 | 1640 | - | Rel-16 | F | NR_Mob_enh-Core | revised |
| R4-2103535 | CR on sync conditions for intra-band DAPS handover R16 | Huawei, HiSilicon | 38.133 | 1640 | 1 | Rel-16 | F | NR_Mob_enh-Core | postponed |
| R4-2101669 | CR on sync conditions for intra-band DAPS handover R17 | Huawei, HiSilicon | 38.133 | 1641 | - | Rel-17 | A | NR_Mob_enh-Core | withdrawn |
| R4-2101671 | CR on maintaining L1-SINR measurent requirements Rel-16 | Huawei, HiSilicon | 38.133 | 1642 | - | Rel-16 | F | NR_eMIMO-Core | revised |
| R4-2103591 | CR on maintaining L1-SINR measurent requirements Rel-16 | Huawei, HiSilicon | 38.133 | 1642 | 1 | Rel-16 | F | NR_eMIMO-Core | agreed |
| R4-2101672 | CR on maintaining L1-SINR measurent requirements Rel-17 | Huawei, HiSilicon | 38.133 | 1643 | - | Rel-17 | A | NR_eMIMO-Core | agreed |
| R4-2101681 | CR on MRTD/MTTD requirements for FR1 intra-band CA/DC R16 | Huawei, HiSilicon, LG Uplus, SoftBank Corp. | 38.133 | 1644 | - | Rel-16 | F | TEI16 | postponed |
| R4-2101682 | CR on MRTD/MTTD requirements for FR1 intra-band CA/DC R17 | Huawei, HiSilicon, LG Uplus, SoftBank Corp. | 38.133 | 1645 | - | Rel-17 | A | TEI16 | withdrawn |
| R4-2101684 | CR on condition requirements for UE power class 5 in TS38.133 | Huawei, HiSilicon | 38.133 | 1646 | - | Rel-17 | F | NR_FR2_FWA_Bn257_Bn258-Perf | revised |
| R4-2103665 | CR on condition requirements for UE power class 5 in TS38.133 | Huawei, HiSilicon | 38.133 | 1646 | 1 | Rel-17 | F | NR_FR2_FWA_Bn257_Bn258-Perf | agreed |
| R4-2101689 | Correction on interruptions of SRS carrier switching | Huawei, HiSilicon | 38.133 | 1647 | - | Rel-16 | F | NR_RRM_enh-Core | agreed |
| R4-2101690 | Correction on interruptions of SRS carrier switching | Huawei, HiSilicon | 38.133 | 1648 | - | Rel-17 | A | NR_RRM_enh-Core | agreed |
| R4-2101691 | Clarification on inter-frequency measurement without gap requirements | Huawei, HiSilicon | 38.133 | 1649 | - | Rel-16 | F | NR_RRM_enh-Core | postponed |
| R4-2103625 | Clarification on inter-frequency measurement without gap requirements | Huawei, HiSilicon | 38.133 | 1649 | 1 | Rel-16 | F | NR_RRM_enh-Core | withdrawn |
| R4-2101692 | Clarification on inter-frequency measurement without gap requirements | Huawei, HiSilicon | 38.133 | 1650 | - | Rel-17 | A | NR_RRM_enh-Core | withdrawn |
| R4-2101694 | UL spatial relation switching to an unknown DL RS | Huawei, HiSilicon | 38.133 | 1651 | - | Rel-16 | F | NR_RRM_enh-Core | revised |
| R4-2103603 | UL spatial relation switching to an unknown DL RS | Huawei, HiSilicon | 38.133 | 1651 | 1 | Rel-16 | F | NR_RRM_enh-Core | agreed |
| R4-2101695 | UL spatial relation switching to an unknown DL RS | Huawei, HiSilicon | 38.133 | 1652 | - | Rel-17 | A | NR_RRM_enh-Core | agreed |
| R4-2101701 | Correction on test cases of inter-frequency Measurements R15 | Huawei, HiSilicon | 38.133 | 1653 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103495 | Correction on test cases of inter-frequency Measurements R15 | Huawei, HiSilicon | 38.133 | 1653 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2101702 | Correction on test cases of inter-frequency Measurements R16 | Huawei, HiSilicon | 38.133 | 1654 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2101703 | Correction on test cases of inter-frequency Measurements R17 | Huawei, HiSilicon | 38.133 | 1655 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2101704 | Correction on NR Pcell FR2 active TCI state switching R15 | Huawei, HiSilicon | 38.133 | 1656 | - | Rel-15 | F | NR_newRAT-Perf | postponed |
| R4-2101705 | Correction on NR Pcell FR2 active TCI state switching R16 | Huawei, HiSilicon | 38.133 | 1657 | - | Rel-16 | A | NR_newRAT-Perf | withdrawn |
| R4-2101706 | Correction on NR Pcell FR2 active TCI state switching R17 | Huawei, HiSilicon | 38.133 | 1658 | - | Rel-17 | A | NR_newRAT-Perf | withdrawn |
| R4-2101710 | Correction on test cases of DL interruptions at switching between two uplink carriers | Huawei, HiSilicon | 38.133 | 1659 | - | Rel-16 | F | NR_RF_FR1-Perf | revised |
| R4-2103601 | Correction on test cases of DL interruptions at switching between two uplink carriers | Huawei, HiSilicon | 38.133 | 1659 | 1 | Rel-16 | F | NR_RF_FR1-Perf | agreed |
| R4-2101711 | Correction on test cases of DL interruptions at switching between two uplink carriers | Huawei, HiSilicon | 38.133 | 1660 | - | Rel-17 | A | NR_RF_FR1-Perf | agreed |
| R4-2101762 | CR to 38.133 correction on SRS carrier based switching core requirements | vivo | 38.133 | 1661 | - | Rel-16 | F | NR_RRM_enh-Core | revised |
| R4-2103611 | CR to 38.133 correction on SRS carrier based switching core requirements | vivo, Qualcomm, Huawei, HiSilicon, MediaTek Inc., Apple, Nokia | 38.133 | 1661 | 1 | Rel-16 | F | NR_RRM_enh-Core | postponed |
| R4-2101763 | CR to 38.133 correction on SRS carrier based switching core requirements | vivo | 38.133 | 1662 | - | Rel-17 | A | NR_RRM_enh-Core | withdrawn |
| R4-2101779 | CR to 38.133 correction to PRS RSTD measurement requirements | vivo | 38.133 | 1663 | - | Rel-16 | F | NR_pos-Core | merged |
| R4-2101780 | CR to 38.133 correction to PRS RSTD measurement requirements | vivo | 38.133 | 1664 | - | Rel-17 | A | NR_pos-Core | withdrawn |
| R4-2101781 | CR to 38.133 correction on PRS-RSRP measurement requirements | vivo | 38.133 | 1665 | - | Rel-16 | F | NR_pos-Core | merged |
| R4-2101782 | CR to 38.133 correction on PRS-RSRP measurement requirements | vivo | 38.133 | 1666 | - | Rel-17 | A | NR_pos-Core | withdrawn |
| R4-2101783 | CR to 38.133 correction on UE Rx-Tx time difference measurement requirements | vivo | 38.133 | 1667 | - | Rel-16 | F | NR_pos-Core | merged |
| R4-2101784 | CR to 38.133 correction on UE Rx-Tx time difference measurement requirements | vivo | 38.133 | 1668 | - | Rel-17 | A | NR_pos-Core | withdrawn |
| R4-2101785 | CR to 38.133 correction on CCSF for NR measurements for positioning | vivo | 38.133 | 1669 | - | Rel-16 | F | NR_pos-Core | revised |
| R4-2103582 | CR to 38.133 correction on CCSF for NR measurements for positioning | vivo | 38.133 | 1669 | 1 | Rel-16 | F | NR_pos-Core | revised |
| R4-2104075 | CR to 38.133 correction on CCSF for NR measurements for positioning | vivo | 38.133 | 1669 | 2 | Rel-16 | F | NR_pos-Core | agreed |
| R4-2101786 | CR to 38.133 correction on CCSF for NR measurements for positioning | vivo | 38.133 | 1670 | - | Rel-17 | A | NR_pos-Core | agreed |
| R4-2101788 | CR to 38.133 correction on reselection of V2X synchronization reference source requirements | vivo | 38.133 | 1671 | - | Rel-16 | F | 5G_V2X_NRSL-Core | agreed |
| R4-2101789 | CR to 38.133 correction on reselection of V2X synchronization reference source requirements | vivo | 38.133 | 1672 | - | Rel-17 | A | 5G_V2X_NRSL-Core | agreed |
| R4-2101834 | Correction on inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion | Huawei, HiSilicon | 38.133 | 1673 | - | Rel-16 | F | NR_UE_pow_sav-Core | revised |
| R4-2103572 | Correction on inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion | Huawei, HiSilicon | 38.133 | 1673 | 1 | Rel-16 | F | NR_UE_pow_sav-Core | agreed |
| R4-2101835 | Test case for cell reselection to FR2 intra-frequency NR case for UE configured with relaxed measurement | Huawei, HiSilicon | 38.133 | 1674 | - | Rel-16 | F | NR_UE_pow_sav-Perf | revised |
| R4-2103575 | Test case for cell reselection to FR2 intra-frequency NR case for UE configured with relaxed measurement | Huawei, HiSilicon | 38.133 | 1674 | 1 | Rel-16 | F | NR_UE_pow_sav-Perf | agreed |
| R4-2101836 | Test case for cell reselection to FR2 intra-frequency NR case for UE configured with relaxed measurement | Huawei, HiSilicon | 38.133 | 1675 | - | Rel-17 | A | NR_UE_pow_sav-Perf | agreed |
| R4-2101838 | CR on CSI-RS based intra-frequency scheduling restriction | Huawei, HiSilicon | 38.133 | 1676 | - | Rel-16 | F | NR_CSIRS_L3meas-Core | postponed |
| R4-2101839 | CR on CSI-RS based intra-frequency scheduling restriction | Huawei, HiSilicon | 38.133 | 1677 | - | Rel-17 | A | NR_CSIRS_L3meas-Core | withdrawn |
| R4-2101840 | Correction on CSSFoutsidegap | Huawei, HiSilicon | 38.133 | 1678 | - | Rel-16 | F | NR_CSIRS_L3meas-Core | agreed |
| R4-2101841 | Correction on CSSFoutsidegap | Huawei, HiSilicon | 38.133 | 1679 | - | Rel-17 | A | NR_CSIRS_L3meas-Core | agreed |
| R4-2101842 | CR on CSI-RS measurement window and intra-frequency measurements | Huawei, HiSilicon | 38.133 | 1680 | - | Rel-16 | F | NR_CSIRS_L3meas-Core | revised |
| R4-2103635 | CR on CSI-RS measurement window and intra-frequency measurements | Huawei, HiSilicon | 38.133 | 1680 | 1 | Rel-16 | F | NR_CSIRS_L3meas-Core | postponed |
| R4-2101843 | CR on CSI-RS measurement window and intra-frequency measurements | Huawei, HiSilicon | 38.133 | 1681 | - | Rel-17 | A | NR_CSIRS_L3meas-Core | withdrawn |
| R4-2101844 | Correction on inter-RAT measurement in high speed scenario | Huawei, HiSilicon | 38.133 | 1682 | - | Rel-16 | F | NR_HST-Core | agreed |
| R4-2101845 | Correction on inter-RAT measurement in high speed scenario | Huawei, HiSilicon | 38.133 | 1683 | - | Rel-17 | A | NR_HST-Core | agreed |
| R4-2101846 | Correction on test cases for inter-RAT cell identification in connected mode for HST | Huawei, HiSilicon | 38.133 | 1684 | - | Rel-16 | F | NR_HST-Perf | revised |
| R4-2103656 | Correction on test cases for inter-RAT cell identification in connected mode for HST | Huawei, HiSilicon | 38.133 | 1684 | 1 | Rel-16 | F | NR_HST-Perf | endorsed |
| R4-2101847 | Correction on test cases for inter-RAT cell identification in connected mode for HST | Huawei, HiSilicon | 38.133 | 1685 | - | Rel-17 | A | NR_HST-Perf | withdrawn |
| R4-2101861 | MRTD and MTTD in non-contiguous CA in FR1 | Ericsson | 38.133 | 1686 | - | Rel-16 | F | TEI16 | postponed |
| R4-2104051 | MRTD and MTTD in non-contiguous CA in FR1 | Ericsson | 38.133 | 1686 | 1 | Rel-16 | F | TEI16 | withdrawn |
| R4-2101862 | MRTD and MTTD in non-contiguous CA in FR1 | Ericsson | 38.133 | 1687 | - | Rel-17 | A | TEI16 | withdrawn |
| R4-2101868 | Updates on MRTD requirements for FR2 inter-band DL CA | Ericsson | 38.133 | 1688 | - | Rel-17 | B | NR_RF_FR2_req_enh2-Core | postponed |
| R4-2101869 | Updates on MTTD requirements for FR2 inter-band DL CA | Ericsson | 38.133 | 1689 | - | Rel-17 | B | NR_RF_FR2_req_enh2-Core | postponed |
| R4-2101881 | Correction on inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion | Huawei, HiSilicon | 38.133 | 1690 | - | Rel-17 | A | NR_UE_pow_sav-Core | agreed |
| R4-2102237 | Maintenance CR for NR-U core requirements | Ericsson | 38.133 | 1691 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2102238 | Maintenance CR for NR-U core requirements | Ericsson | 38.133 | 1692 | - | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2102243 | Introduction of NR-U cell reselection tests | Ericsson | 38.133 | 1693 | - | Rel-16 | B | NR_unlic-Perf | revised |
| R4-2103531 | Introduction of NR-U cell reselection tests | Ericsson | 38.133 | 1693 | 1 | Rel-16 | B | NR_unlic-Perf | revised |
| R4-2104078 | Introduction of NR-U cell reselection tests | Ericsson | 38.133 | 1693 | 2 | Rel-16 | B | NR_unlic-Perf | endorsed |
| R4-2102245 | Changes to cell reselection tests under power saving | Ericsson | 38.133 | 1694 | - | Rel-17 | A | NR_UE_pow_sav-Perf | withdrawn |
| R4-2102246 | Changes to cell reselection tests under power saving | Ericsson | 38.133 | 1695 | - | Rel-16 | F | NR_UE_pow_sav-Perf | postponed |
| R4-2103576 | Changes to cell reselection tests under power saving | Ericsson | 38.133 | 1695 | 1 | Rel-16 | F | NR_UE_pow_sav-Perf | withdrawn |
| R4-2102250 | Correction of band group notation for FR2 | Nokia, Nokia Shanghai Bell | 38.133 | 1696 | - | Rel-16 | F | TEI16 | agreed |
| R4-2102251 | Correction of band group notation for FR2 | Nokia, Nokia Shanghai Bell | 38.133 | 1697 | - | Rel-17 | A | TEI16 | agreed |
| R4-2102252 | Correction to Idle Mode CA/DC Measurements for Inactive mode | Nokia, Nokia Shanghai Bell | 38.133 | 1698 | - | Rel-16 | F | LTE_NR_DC_CA_enh-Core | agreed |
| R4-2102253 | Correction to Idle Mode CA/DC Measurements for Inactive mode | Nokia, Nokia Shanghai Bell | 38.133 | 1699 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Core | agreed |
| R4-2102254 | CR clarifying the UE measurement requirements for an SCell with dormant BWP | Nokia, Nokia Shanghai Bell | 38.133 | 1700 | - | Rel-16 | F | LTE_NR_DC_CA_enh-Core | agreed |
| R4-2102255 | CR clarifying the UE measurement requirements for an SCell with dormant BWP | Nokia, Nokia Shanghai Bell | 38.133 | 1701 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Core | agreed |
| R4-2102257 | Correction to simultaneous DCI based BWP switch delay on multiple CCs | Nokia, Nokia Shanghai Bell | 38.133 | 1702 | - | Rel-16 | F | LTE_NR_DC_CA_enh-Core | agreed |
| R4-2102258 | Correction to simultaneous DCI based BWP switch delay on multiple CCs | Nokia, Nokia Shanghai Bell | 38.133 | 1703 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Core | agreed |
| R4-2102277 | 2-step RACH RRM performance requirements corrections | Nokia, Nokia Shanghai Bell | 38.133 | 1704 | - | Rel-16 | F | NR_2step_RACH-Perf | withdrawn |
| R4-2102278 | 2-step RACH RRM performance requirements corrections | Nokia, Nokia Shanghai Bell | 38.133 | 1705 | - | Rel-17 | A | NR_2step_RACH-Perf | withdrawn |
| R4-2102298 | Revision of PRS-RSTD measurement requirements | Qualcomm Incorporated | 38.133 | 1706 | - | Rel-16 | F | NR_pos-Core | merged |
| R4-2102299 | Revision of PRS-RSTD measurement requirements | Qualcomm Incorporated | 38.133 | 1707 | - | Rel-17 | A | NR_pos-Core | withdrawn |
| R4-2102300 | Revision of PRS-RSRP measurement requirements | Qualcomm Incorporated | 38.133 | 1708 | - | Rel-16 | F | NR_pos-Core | merged |
| R4-2102301 | Revision of PRS-RSRP measurement requirements | Qualcomm Incorporated | 38.133 | 1709 | - | Rel-17 | A | NR_pos-Core | withdrawn |
| R4-2102302 | Revision of UE Rx-Tx measurement requirements | Qualcomm Incorporated | 38.133 | 1710 | - | Rel-16 | F | NR_pos-Core | merged |
| R4-2102303 | Revision of UE Rx-Tx measurement requirements | Qualcomm Incorporated | 38.133 | 1711 | - | Rel-17 | A | NR_pos-Core | withdrawn |
| R4-2102306 | CR to TS 38.133: Redundant and incorrect TCI state in tests with TRS (Rel-15) | Rohde & Schwarz | 38.133 | 1712 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2104054 | CR to TS 38.133: Redundant and incorrect TCI state in tests with TRS (Rel-15) | Rohde & Schwarz | 38.133 | 1712 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2102307 | CR to TS 38.133: Redundant and incorrect TCI state in tests with TRS (Rel-16) | Rohde & Schwarz | 38.133 | 1713 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2102308 | CR to TS 38.133: Redundant and incorrect TCI state in tests with TRS (Rel-17) | Rohde & Schwarz | 38.133 | 1714 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2102309 | CR to TS 38.133: Corrections to TC A.4.5.7.1 (Rel-15) | Rohde & Schwarz | 38.133 | 1715 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103496 | CR to TS 38.133: Corrections to TC A.4.5.7.1 (Rel-15) | Rohde & Schwarz | 38.133 | 1715 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2102310 | CR to TS 38.133: Corrections to TC A.4.5.7.1 (Rel-16) | Rohde & Schwarz | 38.133 | 1716 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2102311 | CR to TS 38.133: Corrections to TC A.4.5.7.1 (Rel-17) | Rohde & Schwarz | 38.133 | 1717 | - | Rel-17 | A | NR_newRAT-Perf | endorsed |
| R4-2102352 | CR 38.133 (8.2 8.6) Corrections related to SCell dormancy switching | Ericsson | 38.133 | 1718 | - | Rel-16 | F | LTE_NR_DC_CA_enh-Core | merged |
| R4-2102353 | CR 38.133 (8.2 8.6) Corrections related to SCell dormancy switching | Ericsson | 38.133 | 1719 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Core | withdrawn |
| R4-2102355 | CR 38.133 (8.6.2A) Clarification on DCI-triggered BWP switch on multiple CCs | Ericsson | 38.133 | 1720 | - | Rel-16 | F | NR_RRM_enh-Core | revised |
| R4-2103602 | CR 38.133 (8.6.2A) Clarification on DCI-triggered BWP switch on multiple CCs | Ericsson | 38.133 | 1720 | 1 | Rel-16 | F | NR_RRM_enh-Core | agreed |
| R4-2102356 | CR 38.133 (8.6.2A) Clarification on DCI-triggered BWP switch on multiple CCs | Ericsson | 38.133 | 1721 | - | Rel-17 | A | NR_RRM_enh-Core | agreed |
| R4-2102513 | Updates in RLM requirements for NR-U | Ericsson | 38.133 | 1722 | - | Rel-16 | F | NR_unlic-Core | revised |
| R4-2104047 | Updates in RLM requirements for NR-U | Ericsson | 38.133 | 1722 | 1 | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2102514 | Updates in RLM requirements for NR-U | Ericsson | 38.133 | 1723 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2102516 | Updates in SCell activation in NR-U | Ericsson | 38.133 | 1724 | - | Rel-16 | F | NR_unlic-Core | merged |
| R4-2102517 | Updates in SCell activation in NR-U | Ericsson | 38.133 | 1725 | - | Rel-17 | A | NR_unlic-Core | withdrawn |
| R4-2102519 | Terminology updates for NR-U in 38.133 | Ericsson | 38.133 | 1726 | - | Rel-16 | F | NR_unlic-Core | revised |
| R4-2103514 | Terminology updates for NR-U in 38.133 | Ericsson | 38.133 | 1726 | 1 | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2102520 | Terminology updates for NR-U in 38.133 | Ericsson | 38.133 | 1727 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2102537 | Correction to inter-RAT CSSF | Ericsson | 38.133 | 1728 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2102538 | Correction to inter-RAT CSSF | Ericsson | 38.133 | 1729 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2102540 | RSTD measurement requirements | Ericsson | 38.133 | 1730 | - | Rel-16 | F | NR_pos-Core | merged |
| R4-2102541 | RSTD measurement requirements | Ericsson | 38.133 | 1731 | - | Rel-17 | A | NR_pos-Core | withdrawn |
| R4-2102543 | PRS-RSRP measurement requirements | Ericsson | 38.133 | 1732 | - | Rel-16 | F | NR_pos-Core | revised |
| R4-2103579 | PRS-RSRP measurement requirements | Ericsson | 38.133 | 1732 | 1 | Rel-16 | F | NR_pos-Core | revised |
| R4-2104074 | PRS-RSRP measurement requirements | Ericsson | 38.133 | 1732 | 2 | Rel-16 | F | NR_pos-Core | agreed |
| R4-2102544 | PRS-RSRP measurement requirements | Ericsson | 38.133 | 1733 | - | Rel-17 | A | NR_pos-Core | agreed |
| R4-2102546 | UE Rx-Tx measurement requirements | Ericsson | 38.133 | 1734 | - | Rel-16 | F | NR_pos-Core | merged |
| R4-2102547 | UE Rx-Tx measurement requirements | Ericsson | 38.133 | 1735 | - | Rel-17 | A | NR_pos-Core | withdrawn |
| R4-2102548 | Correction to CSSF for PRS measurements | Ericsson | 38.133 | 1736 | - | Rel-16 | F | NR_pos-Core | merged |
| R4-2102612 | 38.133 CR on the CSI-RS based measurement requirements | Nokia, Nokia Shanghai Bell | 38.133 | 1737 | - | Rel-17 | A | NR_CSIRS_L3meas-Core | agreed |
| R4-2102642 | Applicability of RA with CCA on RRM requirements in NR-U in 38.133 | Ericsson | 38.133 | 1738 | - | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2102643 | Applicability of RA with CCA on RRM requirements in NR-U in 38.133 | Ericsson | 38.133 | 1739 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2102654 | RRM core requirements for band n62 | Ericsson | 38.133 | 1740 | - | Rel-17 | B | NR_47GHz_band-Core | revised |
| R4-2103666 | RRM core requirements for band n62 | Ericsson | 38.133 | 1740 | 1 | Rel-17 | B | NR_47GHz_band-Core | agreed |
| R4-2102686 | SRS carrier switching interruption CR | Qualcomm, Inc. | 38.133 | 1741 | - | Rel-16 | F | NR_RRM_enh | postponed |
| R4-2103612 | SRS carrier switching interruption CR | Qualcomm, Inc. | 38.133 | 1741 | 1 | Rel-16 | F | NR_RRM_enh | withdrawn |
| R4-2102687 | SRS carrier switching interruption CR | Qualcomm, Inc. | 38.133 | 1742 | - | Rel-17 | A | NR_RRM_enh | withdrawn |
| R4-2102720 | CR on Active TCI state switching for NR-U | Huawei, HiSilicon | 38.133 | 1743 | - | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2102721 | CR on Active TCI state switching for NR-U (cat A) | Huawei, HiSilicon | 38.133 | 1744 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2102722 | CR on maintenance on BWP switch requirements on multiple CCs | Huawei, HiSilicon | 38.133 | 1745 | - | Rel-16 | F | NR_RRM_enh-Core | agreed |
| R4-2102723 | CR on maintenance on BWP switch requirements on multiple CCs (cat A) | Huawei, HiSilicon | 38.133 | 1746 | - | Rel-17 | A | NR_RRM_enh-Core | agreed |
| R4-2102724 | CR on PSCell Addition requirements for NR-U | Huawei, HiSilicon | 38.133 | 1747 | - | Rel-16 | F | NR_unlic-Core | postponed |
| R4-2102725 | CR on PSCell Addition requirements for NR-U (cat A) | Huawei, HiSilicon | 38.133 | 1748 | - | Rel-17 | A | NR_unlic-Core | withdrawn |
| R4-2102726 | CR on test cases for inter-RAT measurement r15 | Huawei, HiSilicon | 38.133 | 1749 | - | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2102727 | CR on test cases for inter-RAT measurement r16 | Huawei, HiSilicon | 38.133 | 1750 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2102728 | CR on test cases for inter-RAT measurement r17 | Huawei, HiSilicon | 38.133 | 1751 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2102738 | CR on SCell activation delay, cell idenfication requirements on deactivated SCell and inter-RAT ECID requirements for NE-DC R15 | Huawei, HiSilicon | 38.133 | 1752 | - | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2103482 | CR on SCell activation delay, cell idenfication requirements on deactivated SCell and inter-RAT ECID requirements for NE-DC R15 | Huawei, HiSilicon | 38.133 | 1752 | 1 | Rel-15 | F | NR_newRAT-Core | revised |
| R4-2104090 | CR on SCell activation delay, cell idenfication requirements on deactivated SCell and inter-RAT ECID requirements for NE-DC R15 | Huawei, HiSilicon | 38.133 | 1752 | 2 | Rel-15 | F | NR_newRAT-Core | agreed |
| R4-2102739 | CR on SCell activation delay, cell idenfication requirements on deactivated SCell and inter-RAT ECID requirements for NE-DC R16 | Huawei, HiSilicon | 38.133 | 1753 | - | Rel-16 | A | NR_newRAT-Core | agreed |
| R4-2102740 | CR on SCell activation delay, cell idenfication requirements on deactivated SCell and inter-RAT ECID requirements for NE-DC R17 | Huawei, HiSilicon | 38.133 | 1754 | - | Rel-17 | A | NR_newRAT-Core | agreed |
| R4-2102741 | CR on SCell activation TCs R15 | Huawei, HiSilicon | 38.133 | 1755 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103497 | CR on SCell activation TCs R15 | Huawei, HiSilicon | 38.133 | 1755 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2102742 | CR on SCell activation TCs R16 | Huawei, HiSilicon | 38.133 | 1756 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2102743 | CR on SCell activation TCs R17 | Huawei, HiSilicon | 38.133 | 1757 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2102745 | CR on EMR requirement maintenance in 38.133 | Huawei, HiSilicon | 38.133 | 1758 | - | Rel-16 | F | LTE_NR_DC_CA_enh-Core | agreed |
| R4-2102746 | CR on EMR requirement maintenance in 38.133 R17 | Huawei, HiSilicon | 38.133 | 1759 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Core | agreed |
| R4-2102749 | CR on SCell dormancy switching | Huawei, HiSilicon | 38.133 | 1760 | - | Rel-16 | F | LTE_NR_DC_CA_enh-Core | revised |
| R4-2103554 | CR on SCell dormancy switching | Huawei, HiSilicon | 38.133 | 1760 | 1 | Rel-16 | F | LTE_NR_DC_CA_enh-Core | agreed |
| R4-2102750 | CR on SCell dormancy switching R17 | Huawei, HiSilicon | 38.133 | 1761 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Core | agreed |
| R4-2102759 | CR to update RSTD measurement requirements | Huawei, HiSilicon | 38.133 | 1762 | - | Rel-16 | F | NR_pos-Core | merged |
| R4-2102760 | CR to update RSTD measurement requirements R17 | Huawei, HiSilicon | 38.133 | 1763 | - | Rel-17 | A | NR_pos-Core | withdrawn |
| R4-2102761 | CR to update PRS-RSRP measurement requirements | Huawei, HiSilicon | 38.133 | 1764 | - | Rel-16 | F | NR_pos-Core | merged |
| R4-2102762 | CR to update PRS-RSRP measurement requirements R17 | Huawei, HiSilicon | 38.133 | 1765 | - | Rel-17 | A | NR_pos-Core | withdrawn |
| R4-2102764 | CR to update UE Rx-Tx time differnece measurement requirements | Huawei, HiSilicon | 38.133 | 1766 | - | Rel-16 | F | NR_pos-Core | merged |
| R4-2102765 | CR to update UE Rx-Tx time differnece measurement requirements R17 | Huawei, HiSilicon | 38.133 | 1767 | - | Rel-17 | A | NR_pos-Core | withdrawn |
| R4-2102767 | CR on CSSF and MG for PRS measurement 38.133 | Huawei, HiSilicon | 38.133 | 1768 | - | Rel-16 | F | NR_pos-Core | revised |
| R4-2103583 | CR on CSSF and MG for PRS measurement 38.133 | Huawei, HiSilicon | 38.133 | 1768 | 1 | Rel-16 | F | NR_pos-Core | postponed |
| R4-2102768 | CR on CSSF and MG for PRS measurement 38.133 R17 | Huawei, HiSilicon | 38.133 | 1769 | - | Rel-17 | A | NR_pos-Core | withdrawn |
| R4-2102789 | CR on multiple SCell activation requirements | Huawei, HiSilicon | 38.133 | 1770 | - | Rel-16 | F | NR_RRM_enh-Core | revised |
| R4-2103621 | CR on multiple SCell activation requirements | Huawei, HiSilicon | 38.133 | 1770 | 1 | Rel-16 | F | NR_RRM_enh-Core | agreed |
| R4-2102790 | CR on multiple SCell activation requirements R17 | Huawei, HiSilicon | 38.133 | 1771 | - | Rel-17 | A | NR_RRM_enh-Core | agreed |
| R4-2102791 | CR on CGI reading requirements 38.133 | Huawei, HiSilicon, MediaTek | 38.133 | 1772 | - | Rel-16 | F | NR_RRM_enh-Core | revised |
| R4-2103613 | CR on CGI reading requirements 38.133 | Huawei, HiSilicon, MediaTek | 38.133 | 1772 | 1 | Rel-16 | F | NR_RRM_enh-Core | agreed |
| R4-2102792 | CR on CGI reading requirements 38.133 R17 | Huawei, HiSilicon, MediaTek | 38.133 | 1773 | - | Rel-17 | A | NR_RRM_enh-Core | agreed |
| R4-2102823 | CR to 38.133 - Introducing NR-U random access requirements | Nokia | 38.133 | 1774 | - | Rel-17 | A | NR_unlic-Core | withdrawn |
| R4-2102827 | Correction to inter-RAT CSSF | Ericsson | 38.133 | 1775 | - | Rel-15 | F | NR_newRAT-Core | merged |
| R4-2102872 | Cat-F CR to SSB-less SCell activation delay requirement for deactivated FR1 SCell in Rel-15 | Qualcomm Incorporated | 38.133 | 1776 | - | Rel-15 | F | NR_newRAT-Core | postponed |
| R4-2103483 | Cat-F CR to SSB-less SCell activation delay requirement for deactivated FR1 SCell in Rel-15 | Qualcomm Incorporated | 38.133 | 1776 | 1 | Rel-15 | F | NR_newRAT-Core | withdrawn |
| R4-2102873 | Cat-A CR to SSB-less SCell activation delay requirement for deactivated FR1 SCell in Rel-16 | Qualcomm Incorporated | 38.133 | 1777 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2102874 | Cat-A CR to SSB-less SCell activation delay requirement for deactivated FR1 SCell in Rel-17 | Qualcomm Incorporated | 38.133 | 1778 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2102875 | Cat-F CR to addition of TRS Configurations in Rel-15 Test Cases | Qualcomm Incorporated | 38.133 | 1779 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103498 | Cat-F CR to addition of TRS Configurations in Rel-15 Test Cases | Qualcomm Incorporated | 38.133 | 1779 | 1 | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2104086 | Cat-F CR to addition of TRS Configurations in Rel-15 Test Cases | Qualcomm Incorporated | 38.133 | 1779 | 2 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2102876 | Cat-A CR to addition of TRS Configurations in Rel-16 Test Cases | Qualcomm Incorporated | 38.133 | 1780 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2102877 | Cat-A CR to addition of TRS Configurations in Rel-17 Test Cases | Qualcomm Incorporated | 38.133 | 1781 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2102878 | Cat-F CR to DRX Configurations in Rel-15 | Qualcomm Incorporated | 38.133 | 1782 | - | Rel-15 | F | NR_newRAT-Perf | not pursued |
| R4-2102879 | Cat-A CR to DRX Configurations in Rel-16 | Qualcomm Incorporated | 38.133 | 1783 | - | Rel-16 | A | NR_newRAT-Perf | withdrawn |
| R4-2102880 | Cat-A CR to DRX Configurations in Rel-17 | Qualcomm Incorporated | 38.133 | 1784 | - | Rel-17 | A | NR_newRAT-Perf | withdrawn |
| R4-2102881 | Cat-F CR to Removal of brackets for SCell Dormancy and Simultaneous DCI based BWP switch delay on multiple CCs in Rel-16 | Qualcomm Incorporated | 38.133 | 1785 | - | Rel-16 | F | LTE_NR_DC_CA_enh-Core | merged |
| R4-2102882 | Cat-A CR to Removal of brackets for SCell Dormancy and Simultaneous DCI based BWP switch delay on multiple CCs in Rel-17 | Qualcomm Incorporated | 38.133 | 1786 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Core | withdrawn |
| R4-2102889 | Cat-F CR to addition of TRS Configurations in Rel-16 Test Case | Qualcomm Incorporated | 38.133 | 1787 | - | Rel-16 | F | TEI16 | revised |
| R4-2103511 | Cat-F CR to addition of TRS Configurations in Rel-16 Test Case | Qualcomm Incorporated | 38.133 | 1787 | 1 | Rel-16 | F | TEI16 | agreed |
| R4-2102890 | Cat-A CR to addition of TRS Configurations in Rel-17 Test Case | Qualcomm Incorporated | 38.133 | 1788 | - | Rel-17 | A | TEI16 | agreed |
| R4-2102905 | CR on correcting SSB and RACH configuration in CSI-RS based beam failure detection and link recovery tests | Qualcomm CDMA Technologies | 38.133 | 1789 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2102908 | CR on correcting SSB and RACH configuration in CSI-RS based beam failure detection and link recovery tests | Qualcomm CDMA Technologies | 38.133 | 1790 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2102922 | CR on Interruptions during Scell activation in NR-U | Qualcomm Incorporated | 38.133 | 1791 | - | Rel-16 | F | NR_unlic-Core | revised |
| R4-2103516 | CR on Interruptions during Scell activation in NR-U | Qualcomm Incorporated | 38.133 | 1791 | 1 | Rel-16 | F | NR_unlic-Core | agreed |
| R4-2102923 | CR on Interruptions during Scell activation in NR-U | Qualcomm Incorporated | 38.133 | 1792 | - | Rel-17 | A | NR_unlic-Core | agreed |
| R4-2102926 | Correction to CSSF for PRS measurements | Ericsson-LG Co., LTD | 38.133 | 1793 | - | Rel-17 | A | NR_pos-Core | withdrawn |
| R4-2103653 | Big CR: Introduction of Rel-16 CSI-RS based L3 measurement RRM performance requirements | CATT | 38.133 | 1794 | - | Rel-17 | A | NR_CSIRS_L3meas-Perf | withdrawn |
| R4-2104058 | CR on core requirement for CSI-RS L3 measurement | Xiaomi | 38.133 | 1795 | - | Rel-16 | F | NR_CSIRS_L3meas-Core | agreed |
| R4-2104059 | CR on core requirement for CSI-RS L3 measurement | Xiaomi | 38.133 | 1796 | - | Rel-17 | A | NR_CSIRS_L3meas-Core | agreed |
| R4-2103722 | Maintenance CR on interruption at EUTRA SRS carrier switching in 38.133 | OPPO | 38.133 | 1797 | - | Rel-17 | A | TEI16 | agreed |
| R4-2103723 | Maintenance CR on SCell activation delay requirement in TS38.133 | OPPO | 38.133 | 1798 | - | Rel-17 | - | TEI16 | agreed |
| R4-2103581 | CR to TS 38.133 on UE Rx-Tx time difference measurements (section 9.9.4) | OPPO | 38.133 | 1799 | - | Rel-17 | - | NR_pos-Core | agreed |
| R4-2103652 | Big CR: Introduction of Rel-16 CSI-RS based L3 measurement RRM test cases | CATT | 38.133 | 1800 | - | Rel-17 | A | NR_CSIRS_L3meas-Perf | withdrawn |
| R4-2104094 | CR on introduction of missing HST test cases | CMCC | 38.133 | 1801 | - | Rel-17 | F | NR_HST-Core | agreed |
| R4-2100251 | CR for TS 38.141-1 introduction of NR band n24 | Ligado Networks | 38.141-1 | 0168 | - | Rel-17 | B | NR_band_n24-Perf | agreed |
| R4-2100337 | CR to 38141-1 on introducing new SUL band n99 | Huawei,HiSilicon | 38.141-1 | 0169 | - | Rel-17 | B | NR_SUL_UL_n24-Perf | agreed |
| R4-2100548 | CR for 38.141-1: BS demodulation synchronization in test setup | Nokia, Nokia Shanghai Bell | 38.141-1 | 0170 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103800 | CR for 38.141-1: BS demodulation synchronization in test setup | Nokia, Nokia Shanghai Bell | 38.141-1 | 0170 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2100549 | CR for 38.141-1: BS demodulation synchronization in test setup | Nokia, Nokia Shanghai Bell | 38.141-1 | 0171 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2100550 | CR for 38.141-1: BS demodulation synchronization in test setup | Nokia, Nokia Shanghai Bell | 38.141-1 | 0172 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2100551 | CR for 38.141-1: BS demodulation different channel bandwidths applicability rules | Nokia, Nokia Shanghai Bell | 38.141-1 | 0173 | - | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2100552 | CR for 38.141-1: BS demodulation different channel bandwidths applicability rules | Nokia, Nokia Shanghai Bell | 38.141-1 | 0174 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2100553 | CR for 38.141-1: BS demodulation different channel bandwidths applicability rules | Nokia, Nokia Shanghai Bell | 38.141-1 | 0175 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2100582 | Corrections on 2-step RACH demodulation requirements | Nokia, Nokia Shanghai Bell, Ericsson | 38.141-1 | 0176 | - | Rel-16 | F | NR_2step_RACH-Perf | revised |
| R4-2103869 | Corrections on 2-step RACH demodulation requirements | Nokia, Nokia Shanghai Bell, Ericsson | 38.141-1 | 0176 | 1 | Rel-16 | F | NR_2step_RACH-Perf | agreed |
| R4-2100583 | Corrections on 2-step RACH demodulation requirements | Nokia, Nokia Shanghai Bell, Ericsson | 38.141-1 | 0177 | - | Rel-17 | A | NR_2step_RACH-Perf | agreed |
| R4-2100925 | CR on UL timing adjustment conducted performance requirement for TS 38.141-1 | Samsung | 38.141-1 | 0178 | - | Rel-16 | F | NR_HST-Perf | revised |
| R4-2103862 | CR on UL timing adjustment conducted performance requirement for TS 38.141-1 | Samsung | 38.141-1 | 0178 | 1 | Rel-16 | F | NR_HST-Perf | agreed |
| R4-2100926 | CR on UL timing adjustment conducted performance requirement for TS 38.141-1 | Samsung | 38.141-1 | 0179 | - | Rel-17 | A | NR_HST-Perf | agreed |
| R4-2101042 | CR for TS 38.141-1 Updates of NR PUSCH performance requirements for HST | NTT DOCOMO, INC. | 38.141-1 | 0180 | - | Rel-16 | F | NR_HST-Perf | revised |
| R4-2103864 | CR for TS 38.141-1 Updates of NR PUSCH performance requirements for HST | NTT DOCOMO, INC. | 38.141-1 | 0180 | 1 | Rel-16 | F | NR_HST-Perf | agreed |
| R4-2101043 | CR for TS 38.141-1 Updates of NR PUSCH performance requirements for HST | NTT DOCOMO, INC. | 38.141-1 | 0181 | - | Rel-17 | A | NR_HST-Perf | agreed |
| R4-2101091 | CR to TS 38.141-1: Additions of regional requirements for n41 in Japan, Rel-15 | NEC, SoftBank, KDDI | 38.141-1 | 0182 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103778 | CR to TS 38.141-1: Additions of regional requirements for n41 in Japan, Rel-15 | NEC, SoftBank, KDDI, Nokia | 38.141-1 | 0182 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2101092 | CR to TS 38.141-1: Additions of regional requirements for n41 and n90 in Japan, Rel-16 | NEC, SoftBank, KDDI | 38.141-1 | 0183 | - | Rel-16 | F | NR_newRAT-Perf | revised |
| R4-2103779 | CR to TS 38.141-1: Additions of regional requirements for n41 and n90 in Japan, Rel-16 | NEC, SoftBank, KDDI, Nokia | 38.141-1 | 0183 | 1 | Rel-16 | F | NR_newRAT-Perf | agreed |
| R4-2101093 | CR to TS 38.141-1: Additions of regional requirements for n41 and n90 in Japan, Rel-17 | NEC, SoftBank, KDDI, Nokia | 38.141-1 | 0184 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2101304 | CR on update applicability rule for 2-step RACH in 38.141-1 (Rel-16) | Huawei, HiSilicon | 38.141-1 | 0185 | - | Rel-16 | F | NR_2step_RACH-Perf | revised |
| R4-2103871 | CR on update applicability rule for 2-step RACH in 38.141-1 (Rel-16) | Huawei, HiSilicon | 38.141-1 | 0185 | 1 | Rel-16 | F | NR_2step_RACH-Perf | agreed |
| R4-2101305 | CR on update applicability rule for 2-step RACH in 38.141-1 (Rel-17) | Huawei, HiSilicon | 38.141-1 | 0186 | - | Rel-17 | A | NR_2step_RACH-Perf | agreed |
| R4-2101320 | CR for 38.141-1 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-16) | Huawei, HiSilicon | 38.141-1 | 0187 | - | Rel-16 | F | NR_HST-Perf | revised |
| R4-2103866 | CR for 38.141-1 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-16) | Huawei, HiSilicon | 38.141-1 | 0187 | 1 | Rel-16 | F | NR_HST-Perf | agreed |
| R4-2101325 | CR for 38.141-1 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-17) | Huawei, HiSilicon | 38.141-1 | 0188 | - | Rel-17 | A | NR_HST-Perf | agreed |
| R4-2101339 | CR to TS38.141-1 Correction of BS conformance testing for URLLC demodulation requirements with higher BLER (Rel-16) | Huawei, HiSilicon | 38.141-1 | 0189 | - | Rel-16 | F | NR_L1enh_URLLC-Perf | revised |
| R4-2103911 | CR to TS38.141-1 Correction of BS conformance testing for URLLC demodulation requirements with higher BLER (Rel-16) | Huawei, HiSilicon | 38.141-1 | 0189 | 1 | Rel-16 | F | NR_L1enh_URLLC-Perf | agreed |
| R4-2101340 | CR to TS38.141-1 Correction of BS conformance testing for URLLC demodulation requirements with higher BLER (Rel-17) | Huawei, HiSilicon | 38.141-1 | 0190 | - | Rel-17 | A | NR_L1enh_URLLC-Perf | agreed |
| R4-2101560 | CR to TS 38.141-1: Introduction of CBWs 35 MHz and 45 MHz | Ericsson | 38.141-1 | 0191 | - | Rel-17 | B | NR_FR1_35MHz_45MHz_BW-Core | revised |
| R4-2103194 | CR to TS 38.141-1: Introduction of CBWs 35 MHz and 45 MHz | Ericsson | 38.141-1 | 0191 | 1 | Rel-17 | B | NR_FR1_35MHz_45MHz_BW-Core | endorsed |
| R4-2101569 | CR to TS 38.141-1 clarification on PN23 sequence generation | Nokia, Nokia Shanghai Bell | 38.141-1 | 0192 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103789 | CR to TS 38.141-1 clarification on PN23 sequence generation | Nokia, Nokia Shanghai Bell | 38.141-1 | 0192 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2101570 | CR to TS 38.141-1 clarification on PN23 sequence generation | Nokia, Nokia Shanghai Bell | 38.141-1 | 0193 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2101878 | CR to TS 38.141-1 clarification on PN23 sequence generation | Nokia, Nokia Shanghai Bell | 38.141-1 | 0194 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2101982 | CR to TS 38.141-1: introduction of NR-U BS | ZTE Corporation | 38.141-1 | 0195 | - | Rel-16 | B | NR_unlic-Perf | postponed |
| R4-2101983 | CR to TS 38.141-1: introduction of NR-U BS | ZTE Corporation | 38.141-1 | 0196 | - | Rel-17 | A | NR_unlic-Perf | withdrawn |
| R4-2101986 | CR to TS 38.141-2: Introduction of 35MHz and 45MHz | ZTE Corporation | 38.141-1 | 0197 | - | Rel-17 | B | NR_FR1_35MHz_45MHz_BW-Core | not pursued |
| R4-2103195 | CR to TS 38.141-2: Introduction of 35MHz and 45MHz | ZTE Corporation | 38.141-1 | 0197 | 1 | Rel-17 | B | NR_FR1_35MHz_45MHz_BW-Core | withdrawn |
| R4-2102000 | CR to TS 38.141-1: Receiver requirement corrections | Nokia, Nokia Shanghai Bell | 38.141-1 | 0198 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103793 | CR to TS 38.141-1: Receiver requirement corrections | Nokia, Nokia Shanghai Bell | 38.141-1 | 0198 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2102001 | CR to TS 38.141-1: Receiver requirement corrections | Nokia, Nokia Shanghai Bell | 38.141-1 | 0199 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2102002 | CR to TS 38.141-1: Receiver requirement corrections | Nokia, Nokia Shanghai Bell | 38.141-1 | 0200 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2102122 | CR to TS 38.141-1 Update of 0.001% BLER test requirements | Ericsson | 38.141-1 | 0201 | - | Rel-16 | F | NR_L1enh_URLLC-Perf | revised |
| R4-2103895 | CR to TS 38.141-1 Update of 0.001% BLER test requirements | Ericsson | 38.141-1 | 0201 | 1 | Rel-16 | F | NR_L1enh_URLLC-Perf | agreed |
| R4-2102123 | CR to TS 38.141-1 Update of 0.001% BLER test requirements | Ericsson | 38.141-1 | 0202 | - | Rel-17 | A | NR_L1enh_URLLC-Perf | agreed |
| R4-2102279 | Corrections on 2-step RACH demodulation requirements | Nokia, Nokia Shanghai Bell, Ericsson | 38.141-1 | 0203 | - | Rel-16 | F | NR_2step_RACH-Perf | withdrawn |
| R4-2102280 | Corrections on 2-step RACH demodulation requirements | Nokia, Nokia Shanghai Bell, Ericsson | 38.141-1 | 0204 | - | Rel-17 | A | NR_2step_RACH-Perf | withdrawn |
| R4-2103354 | CR to TS 38.141-2: Introduction of 35MHz and 45MHz | ZTE Corporation | 38.141-1 | 0205 | - | Rel-17 | B | NR_FR1_35MHz_45MHz_BW-Core | endorsed |
| R4-2100252 | CR for TS 38.141-2 introduction of NR band n24 | Ligado Networks | 38.141-2 | 0259 | - | Rel-17 | B | NR_band_n24-Perf | agreed |
| R4-2100338 | CR to 38141-2 on introducing new SUL band n99 | Huawei,HiSilicon | 38.141-2 | 0260 | - | Rel-17 | B | NR_SUL_UL_n24-Perf | agreed |
| R4-2100381 | CR for TS 38.141-2: Introduction of NR PUSCH UL TA performance requirement | CATT | 38.141-2 | 0261 | - | Rel-16 | F | NR_HST-Perf | not pursued |
| R4-2103860 | CR for TS 38.141-2: Introduction of NR PUSCH UL TA performance requirement | CATT | 38.141-2 | 0261 | 1 | Rel-16 | F | NR_HST-Perf | withdrawn |
| R4-2100382 | CR for TS 38.141-2: Introduction of NR PUSCH UL TA performance requirement(Rel-17) | CATT | 38.141-2 | 0262 | - | Rel-17 | A | NR_HST-Perf | withdrawn |
| R4-2100386 | CR for TS 38.141-2: Correction on definition for the out-of-band CLTA(Rel-15) | CATT | 38.141-2 | 0263 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103794 | CR for TS 38.141-2: Correction on definition for the out-of-band CLTA(Rel-15) | CATT, Nokia, Nokia Shanghai Bell, Huawei | 38.141-2 | 0263 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2100387 | CR for TS 38.141-2: Correction on definition for the out-of-band CLTA(Rel-16) | CATT, Nokia, Nokia Shanghai Bell, Huawei | 38.141-2 | 0264 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2100388 | CR for TS 38.141-2: Correction on definition for the out-of-band CLTA(Rel-17) | CATT, Nokia, Nokia Shanghai Bell, Huawei | 38.141-2 | 0265 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2100554 | CR for 38.141-2: BS demodulation different channel bandwidths applicability rules | Nokia, Nokia Shanghai Bell | 38.141-2 | 0266 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103801 | CR for 38.141-2: BS demodulation different channel bandwidths applicability rules | Nokia, Nokia Shanghai Bell | 38.141-2 | 0266 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2100555 | CR for 38.141-2: BS demodulation different channel bandwidths applicability rules | Nokia, Nokia Shanghai Bell | 38.141-2 | 0267 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2100556 | CR for 38.141-2: BS demodulation different channel bandwidths applicability rules | Nokia, Nokia Shanghai Bell | 38.141-2 | 0268 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2100929 | CR on FRC for URLLC BS radiated performance requirement for TS 38.141-2 | Samsung | 38.141-2 | 0269 | - | Rel-16 | B | NR_L1enh_URLLC-Perf | revised |
| R4-2103907 | CR on FRC for URLLC BS radiated performance requirement for TS 38.141-2 | Samsung | 38.141-2 | 0269 | 1 | Rel-16 | B | NR_L1enh_URLLC-Perf | agreed |
| R4-2100930 | CR on FRC for URLLC BS radiated performance requirement for TS 38.141-2 | Samsung | 38.141-2 | 0270 | - | Rel-17 | A | NR_L1enh_URLLC-Perf | agreed |
| R4-2100931 | CR on MsgA PUSCH radiated performance requirement for TS 38.141-2 | Samsung | 38.141-2 | 0271 | - | Rel-16 | F | NR_2step_RACH-Perf | revised |
| R4-2103870 | CR on MsgA PUSCH radiated performance requirement for TS 38.141-2 | Samsung | 38.141-2 | 0271 | 1 | Rel-16 | F | NR_2step_RACH-Perf | agreed |
| R4-2100932 | CR on MsgA PUSCH radiated performance requirement for TS 38.141-2 | Samsung | 38.141-2 | 0272 | - | Rel-17 | A | NR_2step_RACH-Perf | agreed |
| R4-2100993 | remove SNR brackets for HST PUSCH in TS38.141-2 | Ericsson | 38.141-2 | 0273 | - | Rel-16 | F | NR_HST-Perf | revised |
| R4-2103863 | remove SNR brackets for HST PUSCH in TS38.141-2 | Ericsson | 38.141-2 | 0273 | 1 | Rel-16 | F | NR_HST-Perf | agreed |
| R4-2100994 | remove SNR brackets for HST PUSCH in TS38.141-2 | Ericsson | 38.141-2 | 0274 | - | Rel-17 | A | NR_HST-Perf | revised |
| R4-2103953 | remove SNR brackets for HST PUSCH in TS38.141-2 | Ericsson | 38.141-2 | 0274 | 1 | Rel-17 | A | NR_HST-Perf | agreed |
| R4-2101044 | CR for TS 38.141-2 Updates of performance requirements of PUSCH repetition type A and PUSCH mapping type B for URLLC | NTT DOCOMO, INC. | 38.141-2 | 0275 | - | Rel-16 | B | NR_L1enh_URLLC-Perf | revised |
| R4-2103908 | CR for TS 38.141-2 Updates of performance requirements of PUSCH repetition type A and PUSCH mapping type B for URLLC | NTT DOCOMO, INC. | 38.141-2 | 0275 | 1 | Rel-16 | B | NR_L1enh_URLLC-Perf | agreed |
| R4-2101045 | CR for TS 38.141-2 Updates of performance requirements of PUSCH repetition type A and PUSCH mapping type B for URLLC | NTT DOCOMO, INC. | 38.141-2 | 0276 | - | Rel-17 | A | NR_L1enh_URLLC-Perf | agreed |
| R4-2101094 | CR to TS 38.141-2: Additions of regional requirements for n41 in Japan, Rel-15 | NEC, SoftBank, KDDI | 38.141-2 | 0277 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103780 | CR to TS 38.141-2: Additions of regional requirements for n41 in Japan, Rel-15 | NEC, SoftBank, KDDI, Nokia | 38.141-2 | 0277 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2101095 | CR to TS 38.141-2: Additions of regional requirements for n41 and n90 in Japan, Rel-16 | NEC, SoftBank, KDDI | 38.141-2 | 0278 | - | Rel-16 | F | NR_newRAT-Perf | revised |
| R4-2103781 | CR to TS 38.141-2: Additions of regional requirements for n41 and n90 in Japan, Rel-16 | NEC, SoftBank, KDDI, Nokia | 38.141-2 | 0278 | 1 | Rel-16 | F | NR_newRAT-Perf | agreed |
| R4-2101096 | CR to TS 38.141-2: Additions of regional requirements for n41 and n90 in Japan, Rel-17 | NEC, SoftBank, KDDI, Nokia | 38.141-2 | 0279 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2101248 | CR on FR2 requirements for PUSCH mapping Type B with low number of symbols (Rel-16) | Intel Corporation | 38.141-2 | 0280 | - | Rel-16 | F | NR_L1enh_URLLC-Perf | revised |
| R4-2103909 | CR on FR2 requirements for PUSCH mapping Type B with low number of symbols (Rel-16) | Intel Corporation | 38.141-2 | 0280 | 1 | Rel-16 | F | NR_L1enh_URLLC-Perf | agreed |
| R4-2101249 | CR on FR2 requirements for PUSCH mapping Type B with low number of symbols (Rel-17) | Intel Corporation | 38.141-2 | 0281 | - | Rel-17 | A | NR_L1enh_URLLC-Perf | agreed |
| R4-2101306 | CR on update applicability rule for 2-step RACH in 38.141-2 (Rel-16) | Huawei, HiSilicon | 38.141-2 | 0282 | - | Rel-16 | F | NR_2step_RACH-Perf | revised |
| R4-2103872 | CR on update applicability rule for 2-step RACH in 38.141-2 (Rel-16) | Huawei, HiSilicon | 38.141-2 | 0282 | 1 | Rel-16 | F | NR_2step_RACH-Perf | agreed |
| R4-2101307 | CR on update applicability rule for 2-step RACH in 38.141-2 (Rel-17) | Huawei, HiSilicon | 38.141-2 | 0283 | - | Rel-17 | A | NR_2step_RACH-Perf | agreed |
| R4-2101321 | CR for 38.141-2 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-16) | Huawei, HiSilicon | 38.141-2 | 0284 | - | Rel-16 | F | NR_HST-Perf | revised |
| R4-2103867 | CR for 38.141-2 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-16) | Huawei, HiSilicon | 38.141-2 | 0284 | 1 | Rel-16 | F | NR_HST-Perf | agreed |
| R4-2101326 | CR for 38.141-2 Cleanup of conformance testing for NR HST PRACH under fading channel (Rel-17) | Huawei, HiSilicon | 38.141-2 | 0285 | - | Rel-17 | A | NR_HST-Perf | agreed |
| R4-2101335 | CR to TS38.141-2 Correction of BS conformance testing for FR2 URLLC PUSCH repetition Type A (Rel-16) | Huawei, HiSilicon | 38.141-2 | 0286 | - | Rel-16 | F | NR_L1enh_URLLC-Perf | revised |
| R4-2103910 | CR to TS38.141-2 Correction of BS conformance testing for FR2 URLLC PUSCH repetition Type A (Rel-16) | Huawei, HiSilicon | 38.141-2 | 0286 | 1 | Rel-16 | F | NR_L1enh_URLLC-Perf | agreed |
| R4-2101336 | CR to TS38.141-2 Correction of BS conformance testing for FR2 URLLC PUSCH repetition Type A (Rel-17) | Huawei, HiSilicon | 38.141-2 | 0287 | - | Rel-17 | A | NR_L1enh_URLLC-Perf | agreed |
| R4-2101571 | CR to TS 38.141-2 clarification on PN23 sequence generation | Nokia, Nokia Shanghai Bell | 38.141-2 | 0288 | - | Rel-15 | F | NR_newRAT-Perf | revised |
| R4-2103790 | CR to TS 38.141-2 clarification on PN23 sequence generation | Nokia, Nokia Shanghai Bell | 38.141-2 | 0288 | 1 | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2101572 | CR to TS 38.141-2 clarification on PN23 sequence generation | Nokia, Nokia Shanghai Bell | 38.141-2 | 0289 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2101730 | TS 38.141-2: Correction of additional spurious emission limits for bands 50, 51, 75, 76 | Ericsson | 38.141-2 | 0290 | - | Rel-15 | F | NR_newRAT-Perf | postponed |
| R4-2103795 | TS 38.141-2: Correction of additional spurious emission limits for bands 50, 51, 75, 76 | Ericsson | 38.141-2 | 0290 | 1 | Rel-15 | F | NR_newRAT-Perf | withdrawn |
| R4-2101731 | TS 38.141-2: Correction of additional spurious emission limits for bands 50, 51, 75, 76 | Ericsson | 38.141-2 | 0291 | - | Rel-16 | A | NR_newRAT-Perf | withdrawn |
| R4-2101732 | TS 38.141-2: Correction of additional spurious emission limits for bands 50, 51, 75, 76 | Ericsson | 38.141-2 | 0292 | - | Rel-17 | A | NR_newRAT-Perf | withdrawn |
| R4-2101735 | TS 38.141-2: Tx spurious limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U) | Ericsson | 38.141-2 | 0293 | - | Rel-16 | B | NR_unlic-Perf | postponed |
| R4-2101736 | TS 38.141-2: Tx spurious limits for co-existence and co-location with of NR-based access to unlicensed spectrum (NR-U) | Ericsson | 38.141-2 | 0294 | - | Rel-17 | A | NR_unlic-Perf | withdrawn |
| R4-2101879 | CR to TS 38.141-2 clarification on PN23 sequence generation | Nokia, Nokia Shanghai Bell | 38.141-2 | 0295 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2101997 | CR to TS 38.141-2: EESS protection requirement correction | Nokia, Nokia Shanghai Bell | 38.141-2 | 0296 | - | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2101998 | CR to TS 38.141-2: EESS protection requirement correction | Nokia, Nokia Shanghai Bell | 38.141-2 | 0297 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2101999 | CR to TS 38.141-2: EESS protection requirement correction | Nokia, Nokia Shanghai Bell | 38.141-2 | 0298 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2102003 | CR to TS 38.141-2: Receiver requirement corrections | Nokia, Nokia Shanghai Bell | 38.141-2 | 0299 | - | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2102004 | CR to TS 38.141-2: Receiver requirement corrections | Nokia, Nokia Shanghai Bell | 38.141-2 | 0300 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2102005 | CR to TS 38.141-2: Receiver requirement corrections | Nokia, Nokia Shanghai Bell | 38.141-2 | 0301 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2102124 | CR to TS 38.141-2 Update of 0.001% BLER test requirements | Ericsson | 38.141-2 | 0302 | - | Rel-16 | F | NR_L1enh_URLLC-Perf | revised |
| R4-2103896 | CR to TS 38.141-2 Update of 0.001% BLER test requirements | Ericsson | 38.141-2 | 0302 | 1 | Rel-16 | F | NR_L1enh_URLLC-Perf | agreed |
| R4-2102125 | CR to TS 38.141-2 Update of 0.001% BLER test requirements | Ericsson | 38.141-2 | 0303 | - | Rel-17 | A | NR_L1enh_URLLC-Perf | agreed |
| R4-2102285 | Correction on requirements for PUSCH repetition Type A | Nokia, Nokia Shanghai Bell | 38.141-2 | 0304 | - | Rel-16 | F | NR_L1enh_URLLC-Perf | merged |
| R4-2102286 | Correction on requirements for PUSCH repetition Type A | Nokia, Nokia Shanghai Bell | 38.141-2 | 0305 | - | Rel-17 | A | NR_L1enh_URLLC-Perf | withdrawn |
| R4-2102434 | CR to TS 38.141-2 - Update CLTA definition, Rel-15 | Huawei | 38.141-2 | 0306 | - | Rel-15 | F | NR_newRAT-Perf | not pursued |
| R4-2102435 | CR to TS 38.141-2 - Update CLTA definition, Rel-16 | Huawei | 38.141-2 | 0307 | - | Rel-16 | A | NR_newRAT-Perf | withdrawn |
| R4-2102436 | CR to TS 38.141-2 - Update CLTA definition, Rel-17 | Huawei | 38.141-2 | 0308 | - | Rel-17 | A | NR_newRAT-Perf | withdrawn |
| R4-2102446 | CR to 38.141-2: Introduction of n262 | Nokia, Nokia Shanghai Bell | 38.141-2 | 0309 | - | Rel-17 | B | NR_47GHz_band-Perf | not pursued |
| R4-2102898 | CR to TS 38.141-2: Updating the orthogonal cut procedure | Nokia, Nokia Shanghai Bell | 38.141-2 | 0310 | - | Rel-15 | F | NR_newRAT-Perf | not pursued |
| R4-2102899 | CR to TS 38.141-2: Updating the orthogonal cut procedure | Nokia, Nokia Shanghai Bell | 38.141-2 | 0311 | - | Rel-16 | A | NR_newRAT-Perf | withdrawn |
| R4-2103802 | CR for 38.141-2: BS demodulation synchronization in test setup | Nokia, Nokia Shanghai Bell | 38.141-2 | 0312 | - | Rel-15 | F | NR_newRAT-Perf | agreed |
| R4-2103803 | CR for 38.141-2: BS demodulation synchronization in test setup | Nokia, Nokia Shanghai Bell | 38.141-2 | 0313 | - | Rel-16 | A | NR_newRAT-Perf | agreed |
| R4-2103804 | CR for 38.141-2: BS demodulation synchronization in test setup | Nokia, Nokia Shanghai Bell | 38.141-2 | 0314 | - | Rel-17 | A | NR_newRAT-Perf | agreed |
| R4-2100042 | [CR] IAB Core Maintenance | ZTE Corporation | 38.174 | 0007 | - | Rel-16 | F | NR_IAB-Core | revised |
| R4-2103539 | [CR] IAB Core Maintenance | ZTE Corporation | 38.174 | 0007 | 1 | Rel-16 | F | NR_IAB-Core | revised |
| R4-2104057 | [CR] IAB Core Maintenance | ZTE Corporation | 38.174 | 0007 | 2 | Rel-16 | F | NR_IAB-Core | endorsed |
| R4-2101627 | CR on RRM core requirements maintenance for MG for IAB | Huawei, HiSilicon | 38.174 | 0008 | - | Rel-16 | F | NR_IAB-Core | merged |
| R4-2102488 | CR on removing gap aspects from IAB-MT RRM requirements | Nokia, Nokia Shanghai Bell | 38.174 | 0009 | - | Rel-16 | F | NR_IAB-Core | merged |
| R4-2102636 | Measurement gaps for Local Area IAB-MT | Ericsson | 38.174 | 0010 | - | Rel-16 | F | NR_IAB-Core | not pursued |
| R4-2103983 | Big CR to TR 38.174 - correction to clause 6 | Qualcomm | 38.174 | 0011 | - | Rel-16 | F | NR_IAB-Core | agreed |
| R4-2100357 | CR to TS 38.175 on Exclusion Bands | Ericsson | 38.175 | 0008 | - | Rel-16 | B | NR_IAB-Core | not pursued |
| R4-2100359 | CR to TS 38.175 on Spatial Exclusion for IAB EMC Radiated Immunity test | Ericsson | 38.175 | 0009 | - | Rel-16 | B | NR_IAB-Core | not pursued |
| R4-2100360 | CR to TS 38.175 on IAB EMC test configuratins and performance requirements | Ericsson | 38.175 | 0010 | - | Rel-16 | B | NR_IAB-Perf | revised |
| R4-2103774 | CR to TS 38.175 on IAB EMC test configuratins and performance requirements | Ericsson, ZTE | 38.175 | 0010 | 1 | Rel-16 | B | NR_IAB-Perf | not pursued |
| R4-2102126 | CR to TS 38.175: Performance criteria for IAB | ZTE Corporation | 38.175 | 0011 | - | Rel-16 | F | NR_IAB-Core | withdrawn |
| R4-2102177 | CR to TS 38.175: Performance criteria for IAB | ZTE Corporation | 38.175 | 0012 | - | Rel-16 | F | NR_IAB-Core | not pursued |
| R4-2102581 | CR to TS 38.175: Spatial Exclusion for EMC RI test for IAB | Huawei | 38.175 | 0013 | - | Rel-16 | B | NR_IAB-Perf | not pursued |
| R4-2100709 | CR for 38.307: Introduction of power class 5 for FR2 | SoftBank Corp. | 38.307 | 0042 | - | Rel-17 | B | NR_FR2_FWA_Bn257_Bn258-Perf | agreed |
| R4-2100787 | Draft CR for TS 38.307 on UE demodulation performance requirements (Rel-15) | China Telecom | 38.307 | 0043 | - | Rel-15 | B | NR_perf_enh-Perf | revised |
| R4-2103839 | Draft CR for TS 38.307 on UE demodulation performance requirements (Rel-15) | China Telecom | 38.307 | 0043 | 1 | Rel-15 | B | NR_perf_enh-Perf | agreed |
| R4-2100788 | Draft CR for TS 38.307 on UE demodulation performance requirements (Rel-16) | China Telecom | 38.307 | 0044 | - | Rel-16 | B | NR_perf_enh-Perf | revised |
| R4-2103840 | Draft CR for TS 38.307 on UE demodulation performance requirements (Rel-16) | China Telecom | 38.307 | 0044 | 1 | Rel-16 | B | NR_perf_enh-Perf | agreed |
| R4-2100789 | Draft CR for TS 38.307 on UE demodulation performance requirements (Rel-17) | China Telecom | 38.307 | 0045 | - | Rel-17 | A | NR_perf_enh-Perf | agreed |
| R4-2100854 | CR on release independent for Rel.16 NR HST UE demodulation requirements | CMCC | 38.307 | 0046 | - | Rel-15 | F | NR_HST-Perf | agreed |
| R4-2100855 | CR on release independent for Rel.16 NR HST UE demodulation requirements | CMCC | 38.307 | 0047 | - | Rel-16 | F | NR_HST-Perf | revised |
| R4-2103859 | CR on release independent for Rel.16 NR HST UE demodulation requirements | CMCC | 38.307 | 0047 | 1 | Rel-16 | F | NR_HST-Perf | agreed |
| R4-2100856 | CR on release independent for Rel.16 NR HST UE demodulation requirements | CMCC | 38.307 | 0048 | - | Rel-17 | A | NR_HST-Perf | agreed |
| R4-2101122 | CR to 38.307 Release independence for UE power class 2 NR inter-band CA and SUL configurations (R15) | China Telecom | 38.307 | 0049 | - | Rel-15 | B | NR_SAR_PC2_interB_SUL_2BUL-Perf | not pursued |
| R4-2101123 | CR to 38.307 Release independence for UE power class 2 NR inter-band CA and SUL configurations (R16) | China Telecom | 38.307 | 0050 | - | Rel-16 | B | NR_SAR_PC2_interB_SUL_2BUL-Perf | not pursued |
| R4-2101124 | CR to 38.307 Release independence for UE power class 2 NR inter-band CA and SUL configurations (R17) | China Telecom | 38.307 | 0051 | - | Rel-17 | B | NR_SAR_PC2_interB_SUL_2BUL-Perf | revised |
| R4-2103173 | CR to 38.307 Release independence for UE power class 2 NR inter-band CA and SUL configurations (R17) | China Telecom | 38.307 | 0051 | 1 | Rel-17 | B | NR_SAR_PC2_interB_SUL_2BUL-Perf | agreed |
| R4-2101989 | CR for 38.307 to delete the redundant information | Huawei, HiSilicon | 38.307 | 0052 | - | Rel-15 | F | NR_newRAT-Core | not pursued |
| R4-2101990 | CR for 38.307 to delete the redundant information | Huawei, HiSilicon | 38.307 | 0053 | - | Rel-16 | A | NR_newRAT-Core | withdrawn |
| R4-2101991 | CR for 38.307 to delete the redundant information | Huawei, HiSilicon | 38.307 | 0054 | - | Rel-17 | A | NR_newRAT-Core | withdrawn |
| R4-2102207 | CR to TS 38.307 on the definition of the duplex-mode for the band configurations | ZTE Corporation, CHTTL | 38.307 | 0055 | - | Rel-15 | F | LTE_NR_DC_CA_enh-Core | not pursued |
| R4-2102208 | CR to TS 38.307 on the definition of the duplex-mode for the band configurations | ZTE Corporation, CHTTL | 38.307 | 0056 | - | Rel-16 | A | LTE_NR_DC_CA_enh-Core | withdrawn |
| R4-2102209 | CR to TS 38.307 on the definition of the duplex-mode for the band configurations | ZTE Corporation, CHTTL | 38.307 | 0057 | - | Rel-17 | A | LTE_NR_DC_CA_enh-Core | withdrawn |
| R4-2102700 | CR to 38.807: Update for FR2 PC5 | Qualcomm Incorporated | 38.307 | 0058 | - | Rel-15 | F | NR_FR2_FWA_Bn257_Bn258-Core | not pursued |
| R4-2102701 | CR to 38.807: Update for FR2 PC5 | Qualcomm Incorporated | 38.307 | 0059 | - | Rel-16 | F | NR_FR2_FWA_Bn257_Bn258-Core | not pursued |
| R4-2102702 | CR to 38.807: Update for FR2 PC5 | Qualcomm Incorporated | 38.307 | 0060 | - | Rel-17 | A | NR_FR2_FWA_Bn257_Bn258-Core | withdrawn |
| R4-2100910 | Big CR for update on TR38.809 | Samsung | 38.809 | 0002 | - | Rel-16 | F | NR_IAB-Core | agreed |
| R4-2101180 | CR to TR 38.820: Correction of antenna model in subclause 7.2.4 | Ericsson | 38.820 | 0002 | - | Rel-16 | F | TEI16 | revised |
| R4-2103786 | CR to TR 38.820: Correction of antenna model in subclause 7.2.4 | Ericsson | 38.820 | 0002 | 1 | Rel-16 | F | TEI16 | agreed |
| R4-2101821 | Uplink Power Control for NR MIMO OTA test | vivo | 38.827 | 0008 | - | Rel-16 | F | FS_NR_MIMO_OTA_test | agreed |
| R4-2102081 | CR to TR38.827 Number of Slots for FR1 MIMO OTA test | CAICT,vivo,OPPO,Huawei,HiSilicon,xiaomi,Samsung | 38.827 | 0009 | - | Rel-16 | F | FS_NR_MIMO_OTA_test | withdrawn |
| R4-2102615 | CR on Channel Model Topics | Keysight Technologies UK Ltd | 38.827 | 0010 | - | Rel-16 | F | FS_NR_MIMO_OTA_test | agreed |
| R4-2102817 | CR to TR 38.827 on channel model rotations | Huawei, HiSilicon | 38.827 | 0011 | - | Rel-16 | F | FS_NR_MIMO_OTA_test | not pursued |
| R4-2102819 | CR to 38.827on base station beamforming configuration | Huawei, HiSilicon | 38.827 | 0012 | - | Rel-16 | F | FS_NR_MIMO_OTA_test | postponed |
| R4-2103916 | CR to TR38.827 on the direction of the BS strongest beams | Huawei, HiSilicon | 38.827 | 0013 | - | Rel-16 | F | FS_NR_MIMO_OTA_test | agreed |
| R4-2100406 | CR for 38.886, General corrections for NR V2X | CATT | 38.886 | 0006 | - | Rel-16 | F | 5G_V2X_NRSL-Core | agreed |

Annex C: Lists of liaisons

### C1: Incoming liaison statements

| Document | Original | Title | From | Decision | Reply TDoc |
| --- | --- | --- | --- | --- | --- |
| R4-2100004 |  | Test methods for over-the-air TRP field measurements of unwanted emissions from IMT radio equipment utilizing active antennas | ITU Radiocommunication Study Groups Working Party 1C | noted | (none) |
| R4-2100005 | R1-2009351 | LS on updated Rel-16 RAN1 UE features lists for LTE | RAN1 | noted | (none) |
| R4-2100006 | R1-2009491 | Reply LS on definition of NR V2X con-current operation | RAN1 | noted | (none) |
| R4-2100007 | R1-2009570 | LS reply on cell-grouping UE capability for synchronous NR-DC | RAN1 | noted | (none) |
| R4-2100008 | R1-2009575 | Reply LS on DCI-based multiple BWP switch simultaneously | RAN1 | noted | (none) |
| R4-2100009 | R1-2009576 | Reply LS on UE capability xDD differentiation for SUL/SDL bands | RAN1 | noted | (none) |
| R4-2100010 | R1-2009586 | LS on updated Rel-16 RAN1 UE features lists for NR | RAN1 | noted | (none) |
| R4-2100011 | R1-2009635 | Reply LS on UE capability for V2X | RAN1 | noted | (none) |
| R4-2100012 | R1-2009676 | LS on uplink Tx switching | RAN1 | noted | (none) |
| R4-2100013 | R1-2009689 | Reply LS on Clarification of UE behavior after receiving the MAC CE deactivation command for semi-persistent CSI reporting in NR-U | RAN1 | noted | (none) |
| R4-2100014 | R1-2009784 | LS on PUCCH and PUSCH repetition | RAN1 | noted | (none) |
| R4-2100015 | R1-2009798 | LS on temporary RS for efficient SCell activation in NR CA | RAN1 | noted | (none) |
| R4-2100016 | R1-2009807 | LS on Beam switching gaps for Multi-TRP UL transmission | RAN1 | noted | (none) |
| R4-2100017 | R2-2011246 | Reply LS on power control for NR-DC | RAN2 | noted | (none) |
| R4-2100018 | R1-2009682 | LS on support of NUL and SUL during DAPS handover | RAN1 | noted | (none) |
| R4-2100019 | R2-2011273 | Reply LS on IAB-MT feature list | RAN2 | noted | (none) |
| R4-2100020 | R5-206676 | LS on ambiguity in deciding TL,C | RAN5 | noted | (none) |
| R4-2100021 | R5-206900 | LS on Frequency Bands for testing of A-GNSS Sensitivity requirements in NR and LTE | RAN5 | noted | (none) |
| R4-2100022 |  | Use of the 252-296 GHz frequency range by land-mobile service applications | ITU-R WP 5A | noted | (none) |
| R4-2100023 | RP-202932 | LS on single UL operation | RAN | noted | (none) |
| R4-2100024 | RP-202934 | Reply LS to APT_LS200918 = RP-202143 on Frequency Arrangements for IMT in the Band 470- 703 MHz | RAN | noted | (none) |
| R4-2100025 | RP-202935 | LS on BCS reporting and support for intra-band EN-DC band combinations | RAN | noted | (none) |
| R4-2100026 | SP-201143 | LS on Use of Inclusive Language in 3GPP | SA | noted | (none) |
| R4-2100027 |  | LS on OTA LTE UE TRP and TRS Requirements | MSG TFES | noted | (none) |
| R4-2100028 | R2-2011270 | Reply LS on questions to RAN WGs on dual Radio UE (2Rx/2Tx or 2Rx/1Tx) support for simultaneous communication with both SNPN and PLMN | RAN2 | noted | (none) |
| R4-2100029 |  | Spurious emission limits for AAS BS in 6.425 – 7.125 GHz and 10-10.5 GHz | CEPT ECC PT1 | not treated | (none) |

### C2: Outgoing liaison statements

| Document | Title | To | Cc | reply to i/c LS |
| --- | --- | --- | --- | --- |
| R4-2101818 | LS on change of methodology for new LTE-CA REL-17 combinations | RAN5 | - |  |
| R4-2102149 | Reply LS to RP-202935 on BCS reporting and support for intra-band EN-DC band combinations | RAN, RAN2 | - | RP-202935 |
| R4-2103144 | LS to RAN2 on single-uplink operation in more than one band pair of a band combination | RAN2 | - |  |
| R4-2103234 | LS on Rel-17 Tx switching enhancements | RAN1, RAN2 | - |  |
| R4-2103290 | Reply LS on Beam switching gaps for Multi-TRP UL transmission | RAN1 | - | R1-2009807 |
| R4-2103360 | LS on Signaling scheme of Transparent TxD | RAN2 | RAN1, RAN5 |  |
| R4-2103368 | LS on updated Rel-16 RAN4 UE features lists for NR and LTE | RAN2 | RAN1 |  |
| R4-2103373 | Further Reply LS on power control for NR-DC | RAN2 | RAN1 | R2-2011246 |
| R4-2103393 | Reply LS on PUCCH and PUSCH repetition | RAN1 | - |  |
| R4-2103401 | Second Reply LS to RP-202935 = R4-2100025 on BCS reporting and support for intra-band EN-DC band combinations | RAN, RAN2 | - |  |
| R4-2103480 | LS on updated Rel-16 RAN4 UE features lists for NR and LTE | RAN2 | RAN1 |  |
| R4-2103610 | LS on CGI reading with autonomous gaps | RAN2 | - |  |
| R4-2103657 | LS on timing of RSS in neighbor cell for Rel-16 eMTC | RAN1, RAN2 | - |  |
| R4-2103674 | LS on handover with PSCell | RAN2 | - |  |
| R4-2103728 | LS on RSS based RSRQ for LTE-MTC | RAN2 | RAN1 |  |
| R4-2103897 | LS on Test Methodology for UE URLLC Ultra Low BLER CQI requirements | RAN5 | - |  |
| R4-2104067 | Reply LS on temporary RS for efficient SCell activation in NR CA | RAN1 | - | R1-2009798 |

Annex D: List of agreed/approved new and revised Work Items

| Document | Title | Source | new/revised |
| --- | --- | --- | --- |

Annex E: List of draft Technical Specifications and Reports

| Document | Spec | vers | Doc title |
| --- | --- | --- | --- |
| R4-2100084 | 37.826 | 0.2.0 | TR 37.826 v0.2.0 ENDC_UE_PC2_R17_NR_TDD |
| R4-2100265 | 36.717-03-02 | 0.2.0 | TR 36.717-03-02 v0.2.0 TR Update for LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17 |
| R4-2100272 | 37.717-11-21 | 0.3.0 | TR 37.717-11-21 v0.3.0 TR update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 |
| R4-2100292 | 37.717-00-00 | 0.2.0 | TR 37.717-00-00 v0.3.0 |
| R4-2100296 | 38.717-05-01 | 0.1.0 | TR 38.717-05-01 v0.2.0 |
| R4-2100373 | - | .. | TR skeleton for Downlink interruption for band combinations to conduct dynamic Tx Switching |
| R4-2100374 | - | .. | TR 37.xxx 0.1.0 for Downlink interruption for band combinations to conduct dynamic Tx Switching |
| R4-2100492 | 38.717-03-01 | 0.2.0 | TR 38.717-03-01 on Rel-17 NR inter-band Carrier Aggregation (CA) for 3 Down Link (DL) / 1 Up Link (UL) |
| R4-2100502 | 37.875 | 0.0.1 | TR 37.875 on band combinations for con-current operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band |
| R4-2101097 | 37.717-11-11 | 0.1.0 | TR 37.717-11-11 v0.3.0 Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) |
| R4-2101482 | 38.717-04-02 | 0.2.0 | TR38.717-04-02 update version 0.3.0 |
| R4-2101483 | 37.717-51-11 | 0.1.0 | TR 37.717-51-11 update version 0.2.0 |
| R4-2101486 | 37.717-21-22 | 0.1.0 | TR 37.717-21-22 update version 0.2.0 |
| R4-2101494 | 38.921 | 0.2.0 | TR 38.921 V 0.3.0 |
| R4-2101508 | 37.717-21-11 | 0.3.0 | TR 37.717-21-11 V0.2.0 for DC of 2 LTE band and 1 NR band |
| R4-2101555 | 38.844 | 0.0.2 | Updated TR 38.844 v0.0.2 |
| R4-2101799 | 37.880 | 0.2.0 | TR 37.880 V0.2.0: High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71 |
| R4-2101822 | 38.151 | 0.2.0 | TS 38.151 v0.2.0 NR MIMO OTA requirements |
| R4-2101891 | 38.717-01-01 | 0.2.0 | TR 38.717-01-01 v0.3.0 Rel-17 NR Intra-band |
| R4-2101892 | 37.717-31-11 | 0.2.0 | TR 37.717-31-11 v0.3.0 Rel-17 DC combinations LTE 3DL and one NR band |
| R4-2101893 | 38.717-04-01 | 0.2.0 | TR 38.717-04-01 v0.3.0 Rel-17 NR Inter-band 4 bands CA |
| R4-2101926 | 37.717-41-11 | 0.2.0 | draft TR 37.717-41-11 v0.3.0 |
| R4-2101927 | 38.849 | 0.0.0 | Skeleton TR 38.849 v0.0.0 |
| R4-2101928 | 38.849 | 0.1.0 | draft TR 38.849 v0.1.0 |
| R4-2102158 | 38.847 | 0.1.0 | TR 38.847 Introduction of NR Band 262 (47GHz band) |
| R4-2102230 | 37.717-33 | 0.1.0 | TR 37.717-33 v0.2.0 |
| R4-2102232 | 37.717-11-31 | 0.2.0 | TR 37.717-11-31_v0.3.0 |
| R4-2102304 | 38.717-02-01 | 0.3.0 | TR 38.717-02-01 v0.3.0 |
| R4-2102305 | 38.717-03-02 | 0.3.0 | TR 38.717-03-02 v0.3.0 |
| R4-2102440 | 36.717-04-01 | 0.3.0 | TR 36.717-04-01 v0.3.0 |
| R4-2102496 | 36.717-02-01 | 0.2.0 | TR 36.717-02-01 Rel-17 LTE inter-band CA for 2 bands DL and 1 band UL CA |
| R4-2102833 | 36.717-03-01 | 0.1.0 | TR 36.717-03-01 0.2.0 |
| R4-2103198 | - | .. | Updated TR 37.875 on band combinations for con-current operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band |
| R4-2103262 | 38.844 | 0.0.2 | Updated TR 38.844 v0.0.2 |
| R4-2103265 | - | .. | Blank TR for extended 600MHz NR band |
| R4-2103283 | - | .. | TR 37.880 V0.3.0: High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71 |
| R4-2103985 | - | .. | Draft TR38.884 Study on enhanced test methods for FR2 NR UEs v0.2.0 |

Annex F: List of action items

| Meeting/Number | Agenda item | Document | Details | Responsible | Due by |
| --- | --- | --- | --- | --- | --- |

Annex G: List of decisions

| Meeting/Number | Agenda item | Document | Details |
| --- | --- | --- | --- |

Annex H: List of participants

| Name | Representing | Status (OP) |
| --- | --- | --- |
| ABDELGELIL, Mahmoud | Samsung Electronics Co., Ltd | 3GPPMEMBER (TTA) |
| ABDI ABYANEH, mohammad | Huawei Technologies France | 3GPPMEMBER (ETSI) |
| AGATA, Mikiya | KDDI Corporation | 3GPPMEMBER (ARIB) |
| AHMADI, Sassan | Xilinx Ireland | 3GPPMEMBER (ETSI) |
| ALIANI, Maqbool | Ligado Networks | 3GPPMEMBER (ETSI) |
| AMURU, Saidhiraj | Indian Institute of Tech (H) | 3GPPMEMBER (TSDSI) |
| ANGELOW, Iwajlo | Nokia Italy | 3GPPMEMBER (ETSI) |
| ARTYOM, Putilin | Intel Technology Poland SP Zoo | 3GPPMEMBER (ETSI) |
| AUSTIN, Shane | Southern Linc. | 3GPPMEMBER (ATIS) |
| AXMON, Joakim | Ericsson España S.A. | 3GPPMEMBER (ETSI) |
| AZCUY, Frank | Charter Communications, Inc | 3GPPMEMBER (ATIS) |
| BACON, Peter | Murata Manufacturing Co Ltd. | 3GPPMEMBER (ARIB) |
| BAI, Lu | CHENGDU TD TECH LTD. | 3GPPMEMBER (CCSA) |
| BAKER, Matthew | Nokia UK | 3GPPMEMBER (ETSI) |
| BASAIER, JIALADE | China Unicom | 3GPPMEMBER (CCSA) |
| BELOV, Dmitry | Intel Corporation SAS | 3GPPMEMBER (ETSI) |
| BEN TOVIM, Erez | Sequans Communications | 3GPPMEMBER (ETSI) |
| BERGLJUNG, Christian | L.M. Ericsson Limited | 3GPPMEMBER (ETSI) |
| BHAT, Prakash | CKH IOD UK LIMITED | 3GPPMEMBER (ETSI) |
| BHAUMIK, Saswata | NXP Semiconductors Netherlands | 3GPPMEMBER (ETSI) |
| BOIXADERA, Francesc | MediaTek Inc. | 3GPPMEMBER (ETSI) |
| BOLOTIN, Ilya | Intel Belgium SA/NV | 3GPPMEMBER (ETSI) |
| BORSATO, Ronald | AT&T | 3GPPMEMBER (ATIS) |
| BRIA, Aurelian | Ericsson LM | 3GPPMEMBER (ETSI) |
| BRUNEL, Dominique | Skyworks Solutions Inc. | 3GPPMEMBER (ETSI) |
| CABRERA-MERCADER, Carlos | Qualcomm Incorporated | 3GPPMEMBER (ATIS) |
| CALIGURI, Steven | PHY Wireless | 3GPPMEMBER (ATIS) |
| CAO, Aijun | ZTE Wistron Telecom AB | 3GPPMEMBER (ETSI) |
| CAPORAL, Samantha | Nokia Denmark | 3GPPMEMBER (ETSI) |
| CARRION, Inma | CommScope Technologies AG | 3GPPMEMBER (ETSI) |
| CATMUR, Richard | Spirent Communications | 3GPPMEMBER (ETSI) |
| CAUDURO DIAS DE PAIVA, Rafael | Nokia Belgium | 3GPPMEMBER (ETSI) |
| CEROVIC, Stefan | Orange Spain | 3GPPMEMBER (ETSI) |
| CHABRAK, Karim | Deutsche Telekom AG | 3GPPMEMBER (ETSI) |
| CHAN, William | Spirent Communications | 3GPPMEMBER (ETSI) |
| CHAN, Yee Sin | Facebook | 3GPPMEMBER (ETSI) |
| CHAPMAN, Thomas | Ericsson Hungary Ltd | 3GPPMEMBER (ETSI) |
| CHEN, jingjing | China Mobile (Hangzhou) Inf. | 3GPPMEMBER (CCSA) |
| CHEN, Mengzhu | ZONSON | 3GPPMEMBER (CCSA) |
| CHEN, Xiang (Steven) | Apple (UK) Limited | 3GPPMEMBER (ETSI) |
| CHEN, Yueji | Nokia Belgium | 3GPPMEMBER (ETSI) |
| CHEN, Zeyan | SmarterMicro Inc. | 3GPPMEMBER (CCSA) |
| CHENG, Qian | Futurewei Technologies | 3GPPMEMBER (ATIS) |
| CHENGJUN, Sun | Samsung Electronics Benelux BV | 3GPPMEMBER (ETSI) |
| CHERVYAKOV, Andrey | Intel | 3GPPMEMBER (ATIS) |
| CHHIBBER, Sachin | Ligado Networks | 3GPPMEMBER (ETSI) |
| CHIBA, Tsunehiko | Rakuten Mobile, Inc | 3GPPMEMBER (ARIB) |
| CHOKSI, Ojas | Ligado Networks | 3GPPMEMBER (ETSI) |
| CHUANGXIN, Jiang | CALTTA | 3GPPMEMBER (CCSA) |
| CHUBERRE, Nicolas | THALES | 3GPPMEMBER (ETSI) |
| CIONI, Stefano | ESA | 3GPPMEMBER (ETSI) |
| CLASSON, Brian | Classon Consulting | 3GPPMEMBER (ATIS) |
| COAN, Philip | QUALCOMM Europe Inc. - Italy | 3GPPMEMBER (ETSI) |
| COMSA, Virgil | InterDigital Communications | 3GPPMEMBER (ATIS) |
| COOPER, Paul | Qorvo | 3GPPMEMBER (ETSI) |
| CUI, Jie | Apple Poland Sp. z.o.o. | 3GPPMEMBER (ETSI) |
| DALSGAARD, Lars | Nokia Corporation | 3GPPMEMBER (ETSI) |
| DESAI, Vipul | Futurewei Technologies | 3GPPMEMBER (ATIS) |
| DIMNIK, Riikka | Nokia Solutions & Networks (I) | 3GPPMEMBER (TSDSI) |
| DOHOPOLSKI, Joe | BROADCOM CORPORATION | 3GPPMEMBER (ETSI) |
| DOOLEY, John | GLOBALSTAR Inc. | 3GPPMEMBER (ETSI) |
| DU, Hao | vivo Mobile Communication Co., | 3GPPMEMBER (CCSA) |
| DU, Lei | Nokia Korea | 3GPPMEMBER (TTA) |
| DUTTA, Santanu | Ligado Networks | 3GPPMEMBER (ETSI) |
| EDWARDS, Keith | Eutelsat S.A. | 3GPPMEMBER (ETSI) |
| EIREINER, Klaus | Andrew Wireless Systems GmbH | 3GPPMEMBER (ETSI) |
| EL MOUMOUHI, Sanaa | Eutelsat S.A. | 3GPPMEMBER (ETSI) |
| ELFSTROM, Torbjorn | Ericsson India Private Limited | 3GPPMEMBER (TSDSI) |
| EMARA, Mustafa | Qualcomm CDMA Technologies | 3GPPMEMBER (ETSI) |
| EVERAERE, Dominique | Ericsson Limited | 3GPPMEMBER (ETSI) |
| FABIEN, Jean-Aicard | NIST | 3GPPMEMBER (ATIS) |
| FENG, Sanjun | vivo Mobile Communication (H) | 3GPPMEMBER (CCSA) |
| FERNANDO, Chan | Qualcomm Incorporated | 3GPPMEMBER (ATIS) |
| FERRUS, Ramon | Sateliot | 3GPPMEMBER (ETSI) |
| FISCHER, Sven | Qualcomm CDMA Technologies | 3GPPMEMBER (ETSI) |
| FLORDELIS, Jose | Sony Corporation | 3GPPMEMBER (ARIB) |
| FONG, Gene | Qualcomm Korea | 3GPPMEMBER (TTA) |
| FORTES LOPEZ, Jose M. | ROHDE & SCHWARZ | 3GPPMEMBER (ETSI) |
| FRANK, Colin | Motorola Mobility UK Ltd. | 3GPPMEMBER (ETSI) |
| FRANK, Dominik | Ericsson LM | 3GPPMEMBER (ETSI) |
| FU, Huanren | Shanghai Chen Si Electronics | 3GPPMEMBER (CCSA) |
| FU, Ting | Beijing Xiaomi Software Tech | 3GPPMEMBER (CCSA) |
| FU, Yanze | Datang Linktester Technology | 3GPPMEMBER (CCSA) |
| FUSHIKI, Masashi | SoftBank Corp. | 3GPPMEMBER (ARIB) |
| FUTAKI, Hisashi | NEC Corporation | 3GPPMEMBER (ARIB) |
| GAN, Jiansong | HuaWei Technologies Co., Ltd | 3GPPMEMBER (CCSA) |
| GAO, Yang | ABP, NRTA | 3GPPMEMBER (CCSA) |
| GAO, YUAN | Datang Mobile Com. Equipment | 3GPPMEMBER (CCSA) |
| GAO, zhanyuan | SAICT | 3GPPMEMBER (CCSA) |
| GEORGEAUX, Eric | Airbus | 3GPPMEMBER (ETSI) |
| GHEORGHIU, Valentin | QUALCOMM JAPAN LLC. | 3GPPMEMBER (ARIB) |
| GOLEBIOWSKI, Bartlomiej | Nokia Poland | 3GPPMEMBER (ETSI) |
| GONUGUNTLA, Venkatarao | NEC Europe Ltd | 3GPPMEMBER (ETSI) |
| GONZALEZ G., David | Continental Automotive GmbH | 3GPPMEMBER (ETSI) |
| GRANT, Marc | AT&T GNS Belgium SPRL | 3GPPMEMBER (ETSI) |
| GUADALUPI, Marco | Sateliot | 3GPPMEMBER (ETSI) |
| GUO, Chunxia | China Mobile (Suzhou) Software | 3GPPMEMBER (CCSA) |
| GUO, Li | Dongguan OPPO Precision Elec. | 3GPPMEMBER (CCSA) |
| GUO, Qiuge | CATT | 3GPPMEMBER (ETSI) |
| GUO, Shengxiang | Beijing Xiaomi Mobile Software | 3GPPMEMBER (CCSA) |
| GUO, zhiyu | Guangdong OPPO Mobile Telecom. | 3GPPMEMBER (CCSA) |
| GUPTA, Ashish | Reliance Jio | 3GPPMEMBER (TSDSI) |
| GUPTA, SHEIFALI | Qualcomm Technologies Int | 3GPPMEMBER (ETSI) |
| GURELLI, Mehmet | Qualcomm Incorporated | 3GPPMEMBER (ATIS) |
| HAN, Bin | Qualcomm Wireless GmbH | 3GPPMEMBER (ETSI) |
| HAN, Jing | Huawei Technologies R&D UK | 3GPPMEMBER (ETSI) |
| HANSON, Van | CommScope Technologies AG | 3GPPMEMBER (ETSI) |
| HARPER, Colby | Pivotal Commware | 3GPPMEMBER (ATIS) |
| HARRIS, Paul | VODAFONE Group Plc | 3GPPMEMBER (ETSI) |
| HASEGAWA, Yoshiaki | Murata Manufacturing Co Ltd. | 3GPPMEMBER (ARIB) |
| HAUSTEIN, Thomas | Fraunhofer HHI | 3GPPMEMBER (ETSI) |
| HE, Michael | Qualcomm CDMA Technologies | 3GPPMEMBER (ETSI) |
| HEJSELBAEK, Johannes | Nokia Denmark | 3GPPMEMBER (ETSI) |
| HERTEL, Thorsten | Keysight Technologies UK Ltd | 3GPPMEMBER (ETSI) |
| HIGUCHI, Shoichi | NTT DOCOMO INC. | 3GPPMEMBER (ARIB) |
| HITOMI, Shinya | Murata Manufacturing Co Ltd. | 3GPPMEMBER (ARIB) |
| HO, Kai-Hsiang | MediaTek Inc. | 3GPPMEMBER (ETSI) |
| HOFMANN, Alexander | Fraunhofer IIS | 3GPPMEMBER (ETSI) |
| HOFMANN, Juergen | Nokia Germany | 3GPPMEMBER (ETSI) |
| HSIEH, Bo-Han | CHTTL | 3GPPMEMBER (ETSI) |
| HSIEH, Ming-Yu | Mediatek India Technology Pvt. | 3GPPMEMBER (TSDSI) |
| HSU, Hsin-Yueh | MediaTek Inc. | 3GPPMEMBER (ETSI) |
| HU, Roy | Chengdu OPPO Mobile Com. corp. | 3GPPMEMBER (CCSA) |
| HU, Ziquan | Xiaomi Communications | 3GPPMEMBER (CCSA) |
| HUANG, Chu-Hsiang | Qualcomm communications-France | 3GPPMEMBER (ETSI) |
| HUANG, Clement | Google Inc. | 3GPPMEMBER (ATIS) |
| HUANG, Dinhwa | MediaTek (Shenzhen) Inc. | 3GPPMEMBER (CCSA) |
| HUANG, Rui | Intel Technology India Pvt Ltd | 3GPPMEMBER (TSDSI) |
| HWANG, Jin-yup | LG Electronics Inc. | 3GPPMEMBER (TTA) |
| HWANG, Youngho | Qorvo | 3GPPMEMBER (ETSI) |
| IKEDA, Tetsu | NEC Corporation | 3GPPMEMBER (ETSI) |
| ILIC SAVOIA, Andjela | Pivotal Commware | 3GPPMEMBER (ATIS) |
| IMMONEN, Antti | Dish Network | 3GPPMEMBER (ATIS) |
| IOFFE, Anatoliy | Apple Italia S.R.L. | 3GPPMEMBER (ETSI) |
| IYER, Sumant | Qualcomm India Pvt Ltd | 3GPPMEMBER (TSDSI) |
| JAFARIAN, Javad | Bell Mobility | 3GPPMEMBER (ETSI) |
| JAFFAR, Munira | HUGHES Network Systems Ltd | 3GPPMEMBER (ETSI) |
| JHA, Vivek | Altiostar | 3GPPMEMBER (ETSI) |
| JIA, Qichen | ZTE Corporation | 3GPPMEMBER (CCSA) |
| JIANG, Shouning | GOHIGH DATA NETWORKS TECH. | 3GPPMEMBER (CCSA) |
| JIHOON, Choi | TTA | 3GPPORG_REP (TTA) |
| JIN, Yiran | Harman GmbH | 3GPPMEMBER (ETSI) |
| KANG, Ting-Wei | MediaTek Beijing Inc. | 3GPPMEMBER (CCSA) |
| KARAJANI, Bledar | ROHDE & SCHWARZ | 3GPPMEMBER (ETSI) |
| KAZMI, Muhammad | Nanjing Ericsson Panda Com Ltd | 3GPPMEMBER (CCSA) |
| KELLY, Ivy | C Spire Wireless | 3GPPMEMBER (ATIS) |
| KIHARA, Kenichi | SoftBank Corp. | 3GPPMEMBER (ARIB) |
| KIM, Donghyeon | SK Telecom | 3GPPMEMBER (TTA) |
| KIM, Jiwoo | Intel | 3GPPMEMBER (ATIS) |
| KIM, Joonwoong | SK Telecom | 3GPPMEMBER (TTA) |
| KIM, Pansoo | ETRI | 3GPPMEMBER (TTA) |
| KIM, Taekhoon | Samsung Research America | 3GPPMEMBER (ATIS) |
| KIM, Yunsung | LG Uplus | 3GPPMEMBER (TTA) |
| KITAGAWA, Koichiro | Sumitomo Elec. Industries, Ltd | 3GPPMEMBER (ARIB) |
| KITAGAWA, Ryu | NTT DOCOMO INC. | 3GPPMEMBER (TTC) |
| KO, Minbeom | Samsung Electronics Romania | 3GPPMEMBER (ETSI) |
| KOBAYASHI, Ryosuke | Fujitsu Limited | 3GPPMEMBER (TTC) |
| KONG, Lingyu | HiSilicon Technologies Co. Ltd | 3GPPMEMBER (CCSA) |
| KUO, Chun-ming | MediaTek Inc. | 3GPPMEMBER (ETSI) |
| KUO, Godeman | MediaTek Inc. | 3GPPMEMBER (ETSI) |
| KUUSISTO, Jussi | Dish Network | 3GPPMEMBER (ATIS) |
| KYBETT, Richard | Huawei Tech.(UK) Co.. Ltd | 3GPPMEMBER (ETSI) |
| LAHTEENSUO, Toni | Nokia Hungary | 3GPPMEMBER (ETSI) |
| LARSSON, Magnus | Oy LM Ericsson AB | 3GPPMEMBER (ETSI) |
| LAVAN, Ethan | Eutelsat S.A. | 3GPPMEMBER (ETSI) |
| LEATHER, Paul | INFINEON TECHNOLOGIES | 3GPPMEMBER (ETSI) |
| LEE, Jae Seung | ETRI | 3GPPMEMBER (TTA) |
| LEE, Jeongheum | Samsung Electronics Romania | 3GPPMEMBER (ETSI) |
| LEE, Junghoon | ETRI | 3GPPMEMBER (TTA) |
| LEE, Sang-wook | LG Electronics Deutschland | 3GPPMEMBER (ETSI) |
| LEE, Young-Taek | Murata Manufacturing Co Ltd. | 3GPPMEMBER (ARIB) |
| LEHTINEN, Vesa | Nokia Corporation | 3GPPMEMBER (ETSI) |
| LEISSE, Volker | CableLabs | 3GPPMEMBER (ETSI) |
| LEO, Richie | ZTE Photonics | 3GPPMEMBER (CCSA) |
| LEVITAN, Ben | TIA | 3GPPOBSERV (OTHER) |
| LI, Bozhi | Samsung Electronics France SA | 3GPPMEMBER (ETSI) |
| LI, HONG | Huawei Technologies (Korea) | 3GPPMEMBER (TTA) |
| LI, hua | Intel China Ltd. | 3GPPMEMBER (CCSA) |
| LI, Ming | Ericsson India Private Limited | 3GPPMEMBER (TSDSI) |
| LI, Nan | ZTE Corporation | 3GPPMEMBER (ETSI) |
| LI, NING | Meizu Technology | 3GPPMEMBER (CCSA) |
| LI, Qiming | Apple GmbH | 3GPPMEMBER (ETSI) |
| LI, Ruyue Yu-Ngok | ShenZhen Zhongxing Shitong | 3GPPMEMBER (CCSA) |
| LI, Shuang | CBN | 3GPPMEMBER (CCSA) |
| LI, Tricia | HUAWEI TECHNOLOGIES Co. Ltd. | 3GPPMEMBER (ETSI) |
| LI, Yankun | Samsung Electronics GmbH | 3GPPMEMBER (ETSI) |
| LI, Yanwei | KDDI Corporation | 3GPPMEMBER (ARIB) |
| LI, Yuanliu | SAMSUNG R&D INSTITUTE JAPAN | 3GPPMEMBER (ARIB) |
| LIM, Suhwan | LG Electronics France | 3GPPMEMBER (ETSI) |
| LIN, Hsuanli | MediaTek (Wuhan) Inc. | 3GPPMEMBER (CCSA) |
| LIN, hui | Huawei Telecommunication India | 3GPPMEMBER (TSDSI) |
| LIN, Licheng | MediaTek Inc. | 3GPPMEMBER (ETSI) |
| LINDELL, Per | Ericsson Telecomunicazioni SpA | 3GPPMEMBER (ETSI) |
| LIU, Bo | China Telecom Corporation Ltd. | 3GPPMEMBER (CCSA) |
| LIU, Liehai | HUAWEI TECH. GmbH | 3GPPMEMBER (ETSI) |
| LIU, Qifei | Guangdong OPPO Mobile Telecom. | 3GPPMEMBER (CCSA) |
| LIU, Yajian | Huawei Tech.(UK) Co.. Ltd | 3GPPMEMBER (ETSI) |
| LIU, Ye | Huawei Technologies France | 3GPPMEMBER (ETSI) |
| LIYUN, zhang | China Unicom | 3GPPMEMBER (CCSA) |
| LO, Anthony | Nokia UK | 3GPPMEMBER (ETSI) |
| LODIGIANI, Luca | Inmarsat | 3GPPMEMBER (ETSI) |
| LU, Sylvia | u-blox AG | 3GPPMEMBER (ETSI) |
| MA, Jinwen | Verizon Switzerland AG | 3GPPMEMBER (ETSI) |
| MA, Jun | Qualcomm CDMA Technologies | 3GPPMEMBER (ETSI) |
| MA, Yujuan | CAICT | 3GPPMEMBER (CCSA) |
| MA, Zhifeng | Jetflow | 3GPPMEMBER (CCSA) |
| MAJMUNDAR, Milap | AT&T GNS Belgium SPRL | 3GPPMEMBER (ETSI) |
| MALEKAFZALIARDAKANI, Reihaneh | Ericsson GmbH, Eurolab | 3GPPMEMBER (ETSI) |
| MALJEVIC, Ivo | TELUS | 3GPPMEMBER (ATIS) |
| MARTINEZ, Luis | Ericsson Inc. | 3GPPMEMBER (ATIS) |
| MARTTILA, Jaakko | Nokia Corporation | 3GPPMEMBER (ETSI) |
| MARUMO, Ryo | DOCOMO Communications Lab. | 3GPPMEMBER (ETSI) |
| MASAL, Abhijeet | CEWiT | 3GPPMEMBER (TSDSI) |
| MATSUMOTO, Yusuke | Murata Manufacturing Co Ltd. | 3GPPMEMBER (ARIB) |
| MAX, Sun | Google Inc. | 3GPPMEMBER (ATIS) |
| MAZZARESE, David | HuaWei Technologies Co., Ltd | 3GPPMEMBER (CCSA) |
| MENDIVIL, Edwin | ETS-Lindgren Europe | 3GPPMEMBER (ETSI) |
| MIYAKE, Takao | Keysight Technologies UK Ltd | 3GPPMEMBER (ETSI) |
| MUELLER, Axel | Nokia Shanghai Bell | 3GPPMEMBER (CCSA) |
| MUELLER, William | BROADCOM CORPORATION | 3GPPMEMBER (ETSI) |
| MUHAMMAD, awn | Rakuten Mobile, Inc | 3GPPMEMBER (ARIB) |
| NAHLER, Achim | National Instruments Corp. | 3GPPMEMBER (ETSI) |
| NAKATOGAWA, Tsuyoshi | NHK | 3GPPMEMBER (ARIB) |
| NANGIA, Vijay | Motorola Mobility UK Ltd. | 3GPPMEMBER (ETSI) |
| NATHOO, Rahim | TELUS | 3GPPMEMBER (ATIS) |
| NEBESNY, Paul | Rogers Communications Canada | 3GPPMEMBER (ETSI) |
| NG, Chenghock | NEC Telecom MODUS Ltd. | 3GPPMEMBER (ETSI) |
| NG, Man Hung | Nokia France | 3GPPMEMBER (ETSI) |
| NGUYEN, Serge | Ligado Networks | 3GPPMEMBER (ETSI) |
| NIELSEN, Sari | Nokia Corporation | 3GPPMEMBER (ETSI) |
| NIGAM, Gaurav | QUALCOMM Europe Inc. - Spain | 3GPPMEMBER (ETSI) |
| NIU, Huaning | Apple Hungary Kft. | 3GPPMEMBER (ETSI) |
| NOEL, Laurent | Skyworks Solutions Inc. | 3GPPMEMBER (ETSI) |
| NOGAMI, Toshizo | SHARP Corporation | 3GPPMEMBER (ARIB) |
| NOVLAN, Thomas | AT&T | 3GPPMEMBER (ATIS) |
| OGUMA, Yuta | NTT DOCOMO INC. | 3GPPMEMBER (TTC) |
| OH, ChoongKeun | TTA | 3GPPORG_REP (TTA) |
| OH, Kyoungseok | TTA | 3GPPORG_REP (TTA) |
| OHIRA, Hidefumi | Murata Manufacturing Co Ltd. | 3GPPMEMBER (ARIB) |
| OLESEN, Robert | Intelsat | 3GPPMEMBER (ATIS) |
| OLFAT, Masoud | Federated Wireless | 3GPPMEMBER (ETSI) |
| ONOZAWA, Hisashi | Nokia Japan | 3GPPMEMBER (ARIB) |
| OUCHI, Mikihiro | Panasonic Corporation | 3GPPMEMBER (ARIB) |
| OZAN, Waseem | MediaTek Inc. | 3GPPMEMBER (ETSI) |
| PACKER, Clive | Ligado Networks | 3GPPMEMBER (ETSI) |
| PANAITOPOL, Dorin | THALES | 3GPPMEMBER (ETSI) |
| PARK, CH | Qualcomm Technologies Int | 3GPPMEMBER (ETSI) |
| PARK, Jin Woong | LG Electronics UK | 3GPPMEMBER (ETSI) |
| PARK, Justin | U.S. Department of Defense | 3GPPMEMBER (ATIS) |
| PETROV, Dmitry | Nokia Germany | 3GPPMEMBER (ETSI) |
| PETROVIC, Niels | ROHDE & SCHWARZ | 3GPPMEMBER (ETSI) |
| PETTERSSON, Markus | LG Electronics Finland | 3GPPMEMBER (ETSI) |
| PINHEIRO, Melissa | Bell Mobility | 3GPPMEMBER (ETSI) |
| PITTAMPALLI, Eshwar | FirstNet | 3GPPMEMBER (ATIS) |
| POPP, Daniel | Apple Switzerland AG | 3GPPMEMBER (ETSI) |
| PRIALE, Camila | Apple GmbH | 3GPPMEMBER (ETSI) |
| PU, Xiaolin | Ericsson-LG Co., LTD | 3GPPMEMBER (TTA) |
| QIU, Haijie | Samsung Electronics Co., Ltd | 3GPPMEMBER (TTA) |
| RAGHAVAN, Manasa | Apple GmbH | 3GPPMEMBER (ETSI) |
| RAMAMURTHI, Vishwanath | Verizon Sweden | 3GPPMEMBER (ETSI) |
| RASCHKOWSKI, Leszek | Fraunhofer HHI | 3GPPMEMBER (ETSI) |
| ROBSON, Julius | Eutelsat S.A. | () |
| RODRIGUEZ-HERRERA, Alfonso | Spirent Communications | 3GPPMEMBER (ETSI) |
| RONG, Zhigang | Futurewei | 3GPPMEMBER (ETSI) |
| ROSE, Ian | ANRITSU LTD | 3GPPMEMBER (ETSI) |
| SASAKI, Takahiro | NEC Corporation | 3GPPMEMBER (TTC) |
| SAVAGLIO, Frank | Telstra Corporation Limited | 3GPPMEMBER (ETSI) |
| SAYENKO, Alexander | Apple GmbH | 3GPPMEMBER (ETSI) |
| SCANNAVINI, Alessandro | MVG Industries | 3GPPMEMBER (ETSI) |
| SCHADE, Holger | Qorvo | 3GPPMEMBER (ETSI) |
| SCHMID, Peter | Andrew Wireless Systems GmbH | 3GPPMEMBER (ETSI) |
| SCHMIEDER, Mathis | Fraunhofer HHI | 3GPPMEMBER (ETSI) |
| SEIBERT, Cristina | NextNav | 3GPPMEMBER (ATIS) |
| SHAFI, Mansoor | Spark NZ Ltd | 3GPPMEMBER (ATIS) |
| SHAH, Manan | Murata Manufacturing Co Ltd. | 3GPPMEMBER (ARIB) |
| SHAN, Huiping | Fiberhome Technologies Group | 3GPPMEMBER (CCSA) |
| SHAN, Yang | China Telecomunication Corp. | 3GPPMEMBER (CCSA) |
| SHAO, Xuanbo | MediaTek Inc. | 3GPPMEMBER (ETSI) |
| SHAO, Zhe | China Mobile Group Device Co. | 3GPPMEMBER (CCSA) |
| SHARMA, Prashant | Qualcomm Austria RFFE GmbH | 3GPPMEMBER (ETSI) |
| SHEN, Songhui | Samsung Electronics Nordic AB | 3GPPMEMBER (ETSI) |
| SHEN, Zhongyi | Huawei Technologies France | 3GPPMEMBER (ETSI) |
| SHEN, Zukang | HuaWei Technologies Co., Ltd | 3GPPMEMBER (CCSA) |
| SHI, Jiakai | HuaWei Technologies Co., Ltd | 3GPPMEMBER (CCSA) |
| SHI, Zhihua | Hangzhou Douku | 3GPPMEMBER (CCSA) |
| SHIMODAIRA, Hidekazu | NTT DOCOMO INC. | 3GPPMEMBER (ARIB) |
| SHIPLEY, Trevor | C Spire Wireless | 3GPPMEMBER (ATIS) |
| SHITOMI, Takuya | NHK | 3GPPMEMBER (ARIB) |
| SHVODIAN, Bill | T-Mobile USA Inc. | 3GPPMEMBER (ATIS) |
| SIAFARIKAS, Dimitris | Apple GmbH | 3GPPMEMBER (ETSI) |
| SIENKIEWICZ, Esther | Ericsson Inc. | 3GPPMEMBER (ATIS) |
| SIMMONS, Paul | Eutelsat S.A. | 3GPPMEMBER (ETSI) |
| SIOMINA, Iana | Ericsson-LG Co., LTD | 3GPPMEMBER (TTA) |
| SKOLD, Johan | Ericsson France S.A.S | 3GPPMEMBER (ETSI) |
| SONG, Lei | Verizon UK Ltd | 3GPPMEMBER (ETSI) |
| SONG, Yuexia | CATT | 3GPPMEMBER (CCSA) |
| STEFANIK, Joerg | Andrew Wireless Systems GmbH | 3GPPMEMBER (ETSI) |
| SU, Frank Chih-Wei | III | 3GPPMEMBER (ETSI) |
| SUN, Roy | CableLabs | 3GPPMEMBER (ETSI) |
| SUN, Sang | ZTE Corporation | 3GPPMEMBER (CCSA) |
| SUN, Siyang | CCSA | 3GPPORG_REP (CCSA) |
| SUN, Yanliang | vivo Mobile Communication (S) | 3GPPMEMBER (CCSA) |
| SUNDSTRÖM, Fredrik | Ericsson France S.A.S | 3GPPMEMBER (ETSI) |
| SUNELL, Kai-Erik | ETSI | 3GPPORG_REP (ETSI) |
| SVENDSEN, Simon | Nokia Denmark | 3GPPMEMBER (ETSI) |
| SZYDELKO, Michal | Huawei Technologies Sweden AB | 3GPPMEMBER (ETSI) |
| TABATA, Fumie | SoftBank Corp. | 3GPPMEMBER (ARIB) |
| TAKAMIYA, Kotaro | NTT DOCOMO INC. | 3GPPMEMBER (TTC) |
| TAKEDA, Hiroki | KDDI Corporation | 3GPPMEMBER (ARIB) |
| TANG, Zhixun | MediaTek (Chengdu) Inc. | 3GPPMEMBER (CCSA) |
| TANNO, Takahito | Murata Manufacturing Co Ltd. | 3GPPMEMBER (ARIB) |
| TAO, Xuhua | Beijing Xiaomi Mobile Software | 3GPPMEMBER (CCSA) |
| TAYLOR, Carolyn | Samsung R&D Institute UK | 3GPPMEMBER (ETSI) |
| THALANANY, Sebastian | US Cellular Corporation | 3GPPMEMBER (ETSI) |
| THANGARASA, Santhan | Ericsson Hungary Ltd | 3GPPMEMBER (ETSI) |
| TKATCH, Alex | ROHDE & SCHWARZ | 3GPPMEMBER (ETSI) |
| TOSATO, Filippo | Nokia Italy | 3GPPMEMBER (ETSI) |
| TRIKHA, Pushp | Qualcomm Incorporated | 3GPPMEMBER (ATIS) |
| TROGOLO, Alessandro | TELECOM ITALIA S.p.A. | 3GPPMEMBER (ETSI) |
| TRUELOVE, Stephen | BT plc | 3GPPMEMBER (ETSI) |
| TSEYTLIN, Michael | Facebook | 3GPPMEMBER (ETSI) |
| UCHIDA, Daisuke | Toshiba | 3GPPMEMBER (ETSI) |
| UESAKA, Kazuyoshi | Ericsson Japan K.K. | 3GPPMEMBER (ARIB) |
| UMEDA, Hiromasa | Nokia Japan | 3GPPMEMBER (ARIB) |
| VAIDYA, Maulik | Charter Communications, Inc | 3GPPMEMBER (ATIS) |
| VALLESE, Pierpaolo | Qualcomm Technologies Ireland | 3GPPMEMBER (ETSI) |
| VASENKARI, Petri | Nokia | 3GPPMEMBER (ATIS) |
| VENKATAKRISHNAN, Aniruddh | Indian Institute of Tech (M) | 3GPPMEMBER (TSDSI) |
| VERA, Aida | Intel Finland Oy | 3GPPMEMBER (ETSI) |
| VINTOLA, Ville | Qualcomm Finland RFFE Oy | 3GPPMEMBER (ETSI) |
| WACHTER, Andreas | Polaris Wireless | 3GPPMEMBER (ATIS) |
| WAGNER, Elmar | Apple GmbH | 3GPPMEMBER (ETSI) |
| WALA, Philip | CommScope Technologies AG | 3GPPMEMBER (ETSI) |
| WANG, He | Samsung Electronics Czech | 3GPPMEMBER (ETSI) |
| WANG, Hung-hsiang | ITRI | 3GPPMEMBER (ETSI) |
| WANG, James | Apple (UK) Limited | 3GPPMEMBER (ETSI) |
| WANG, Jin | Huawei Technologies R&D UK | 3GPPMEMBER (ETSI) |
| WANG, Miao | Spreadtrum Communications | 3GPPMEMBER (CCSA) |
| WANG, Ruixin | vivo Communication Technology | 3GPPMEMBER (CCSA) |
| WANG, Shiyuan | China Mobile Com. Corporation | 3GPPMEMBER (CCSA) |
| WANG, ZHOU | Huawei Device Co., Ltd | 3GPPMEMBER (CCSA) |
| WARDER, Philip | Qorvo | 3GPPMEMBER (ETSI) |
| WEI, Xingguang | ZTE Corporation | 3GPPMEMBER (ETSI) |
| WEI, xusheng | vivo Communication Technology | 3GPPMEMBER (CCSA) |
| WENDLER, Ingo | Union Inter. Chemins de Fer | 3GPPMEMBER (ETSI) |
| WENHAO, liu | ZTE Corporation | 3GPPMEMBER (CCSA) |
| WIESEL, Eli | Azimuth | 3GPPMEMBER (ETSI) |
| WOLF, Bernd | BMWi | 3GPPMEMBER (ETSI) |
| WU, Jingzhou | China Telecom Corporation Ltd. | 3GPPMEMBER (CCSA) |
| XIE, Yuming | Sanechips | 3GPPMEMBER (CCSA) |
| XING, Jinqiang | OnePlus | 3GPPMEMBER (CCSA) |
| XIZENG, Dai | HuaWei Technologies Co., Ltd | 3GPPMEMBER (CCSA) |
| XU, Zhikun | Spreadtrum Communications | 3GPPMEMBER (CCSA) |
| XUE, Fei | ZTE Corporation | 3GPPMEMBER (CCSA) |
| YAMASHITA, Osamu | Anritsu Corporation | 3GPPMEMBER (ARIB) |
| YAN, Liang | Samsung R&D Institute UK | 3GPPMEMBER (ETSI) |
| YAN, Nathan | Apple Computer Trading Co. Ltd | 3GPPMEMBER (CCSA) |
| YANG, ChihKai | MediaTek (Shenzhen) Inc. | 3GPPMEMBER (CCSA) |
| YANG, Qian | vivo Mobile Communication (S) | 3GPPMEMBER (CCSA) |
| YANG, Tang | Apple AB | 3GPPMEMBER (ETSI) |
| YANG, xiaohang | CAICT | 3GPPMEMBER (CCSA) |
| YANG, YOONOH | LG Electronics Polska | 3GPPMEMBER (ETSI) |
| YANG, Yunchuan | Samsung Guangzhou Mobile R&D | 3GPPMEMBER (CCSA) |
| YOKOKAWA, Tomoki | NTT DOCOMO INC. | 3GPPMEMBER (TTC) |
| YOON, Daejung | Nokia Solutions & Networks (I) | 3GPPMEMBER (TSDSI) |
| YOON, Min-Geun | SK Telecom | 3GPPMEMBER (TTA) |
| YU, Tsang-wei | MediaTek (Hefei) Inc. | 3GPPMEMBER (CCSA) |
| ZANDER, Olof | Sony Corporation | 3GPPMEMBER (ARIB) |
| ZENG, Yong | Murata Manufacturing Co Ltd. | 3GPPMEMBER (ARIB) |
| ZHANG, Chenchen | ZTE Corporation | 3GPPMEMBER (ETSI) |
| ZHANG, Chunhui | Nanjing Ericsson Panda Com Ltd | 3GPPMEMBER (CCSA) |
| ZHANG, Dawei | Apple France | 3GPPMEMBER (ETSI) |
| ZHANG, jian | CCSA | () |
| ZHANG, Jinyu | Guangdong OPPO Mobile Telecom. | 3GPPMEMBER (CCSA) |
| ZHANG, juan | Beijing Xiaomi Software Tech | 3GPPMEMBER (CCSA) |
| ZHANG, Li | Huawei Technologies Japan K.K. | 3GPPMEMBER (TTC) |
| ZHANG, Meng | Intel | 3GPPMEMBER (ATIS) |
| ZHANG, Peng | HUAWEI Technologies Japan K.K. | 3GPPMEMBER (ARIB) |
| ZHANG, Qian | HiSilicon Technologies Co. Ltd | 3GPPMEMBER (CCSA) |
| ZHANG, Shichang | Hangzhou Mengyuxiang | 3GPPMEMBER (CCSA) |
| ZHANG, Xiaoran | China Mobile Com. Corporation | 3GPPMEMBER (CCSA) |
| ZHANG, Yiyan | Samsung Electronics Iberia SA | 3GPPMEMBER (ETSI) |
| ZHANG, Yuanyuan | Samsung Electronics Co., Ltd | 3GPPMEMBER (TTA) |
| ZHANG, Yufeng | CAICT | 3GPPMEMBER (CCSA) |
| ZHANG, Zhi | Orope Germany GmbH | 3GPPMEMBER (ETSI) |
| ZHAO, Dong | Samsung Electronics Polska | 3GPPMEMBER (ETSI) |
| ZHAO, Kun | Sony Mobile Communications | 3GPPMEMBER (ARIB) |
| ZHAO, Yichen | China Mobile Group Device Co. | 3GPPMEMBER (CCSA) |
| ZHAO, Zheng | Verizon Denmark | 3GPPMEMBER (ETSI) |
| ZHAO, Zhenshan | Guangdong OPPO Mobile Telecom. | 3GPPMEMBER (CCSA) |
| ZHOU, Hai | Huawei Tech.(UK) Co.. Ltd | 3GPPMEMBER (ETSI) |
| ZHOU, Rui | Beijing Xiaomi Electronics | 3GPPMEMBER (CCSA) |
| ZHOU, Shuai | vivo Mobile Com. (Chongqing) | 3GPPMEMBER (CCSA) |
| ZHOU, Wubin | ShenZhen Zhongxing Shitong | 3GPPMEMBER (CCSA) |
| ZHOU, Xutao | BEIJING SAMSUNG TELECOM R&D | 3GPPMEMBER (CCSA) |
| ZHU, Siting | CAICT | 3GPPMEMBER (CCSA) |
| ZIQI, Liu | VIVO TECH GmbH | 3GPPMEMBER (ETSI) |

Annex I: List of future meetings

| Title | Start date | End date (OP) | Town | Country | Reference |
| --- | --- | --- | --- | --- | --- |
| RAN4#98 | 2021-03-01 | 2021-03-05 | Athens | GR | R4-98 |
| RAN4#98-bis | 2021-04-19 | 2021-04-23 | China | CN | R4-98 |
| RAN4#99 | 2021-05-24 | 2021-05-28 | US | US | R4-99 |
| RAN4#100 | 2021-08-23 | 2021-08-27 | EU | EU | R4-100 |
| RAN4#100-bis | 2021-10-11 | 2021-10-15 | China | CN | R4-100 |
| RAN4#111 | 2021-11-15 | 2021-11-19 | US | US | R4-111 |

Annexes to report prepared by: Kai-Erik Sunell