3GPP TSG-RAN WG4 Meeting # 98-bis-e	R4-2104482

Electronic Meeting, Apr. 12-20, 2021

Source:	ZTE Corporation

Title:	On IAB test cases

Agenda item:	5.3.3.1

Document for:	Discussion

# 1	Introduction

In RAN4 98-e, companies discussed about the general aspects and the scope of IAB-MT test cases. Companies had slightly different views on the test cases / configurations and some points for further study were captured in the WF [1] as shown below.

- This paper provides our view on the topic.

2	Discussion

The first point to be discussed is whether to define TCs for WA IAB-MTs. In our view this is unnecessary. First of all, TCs are defined to verify that the devices can meet the core requirements in typical scenarios. Thus, the test set up shall simulate the typical scenario as much as possible. For WA IAB-MTs, this might not be the case. If test cases for LA IAB-MTs are directly re-used for WA IAB-MTs, as suggested by some companies during the last meeting, the TC configurations cannot represent the typical deployment of WA IAB-MTs.

For WA IAB-MTs, since the IAB nodes are deployed in a carefully planned way, it is reasonable to assume that channel blockage will happen very rarely. But in the test cases, for BFD or RLM, the requirements for the IAB-MTs to measure and report the measurement results are within several seconds, which is far from the typical deployment scenario for WA IAB-MTs. Forcing WA IAB-MTs to measure and report measurement results in such a frequent way doesn’t consider the typical deployment scenario and is a huge waste of energy and resources.

- Proposal 1: Do not define test cases for WA IAB-MTs.

As to whether to define TCs for timing advance, we understand that once deployed, the TA would hardly change for LA and WA IAB-MTs. Thus, we can support not to define such TCs.

- Proposal 2: Do not define test cases of timing advance for both LA IAB-MTs and WA IAB-MTs.

3	Conclusion

- Proposal 1: Do not define test cases for WA IAB-MTs.

- Proposal 2: Do not define test cases of timing advance for both LA IAB-MTs and WA IAB-MTs.

References

- R4-2104091