3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2104516
Electronic Meeting, Apr. 12-20, 2021

Agenda item:	7.27.1.1

Source:	vivo

Title:	Discussion on EIRP for PC1, PC2 and PC4

Document for:	Approval

1	Introduction

The EIRP and spherical coverage requirements for n262 PC3 have been defined in RAN4#98e meeting [1]. It was also agreed that the MOP requirements for other PCs should also be finalized during the WI timeline [2]. Some analysis results for other PCs have been shared [3] [4].

In this contribution, we share our proposals on finalizing the requirement of EIRP and Spherical coverage for PC1, PC2 and PC4.

2	Discussion

In [5], the following PC3 EIRP requirements have been defined for all the FR2 bands:

Table 1 (Table 6.2.1.3-1 in TS 38.101-2 v17.0.0): UE minimum peak EIRP for power class 3

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 22.4 |
| n258 | 22.4 |
| n259 | 18.7 |
| n260 | 20.6 |
| n261 | 22.4 |
| n262 | 16.0 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance NOTE 2: Void |  |

Table 2 (Table 6.2.1.3-3 in TS 38.101-2 v17.0.0): UE spherical coverage for power class 3

| Operating band | Min EIRP at 50 %-tile CDF (dBm) |
| --- | --- |
| n257 | 11.5 |
| n258 | 11.5 |
| n259 | 5.8 |
| n260 | 8 |
| n261 | 11.5 |
| n262 | 2.9 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance NOTE 2: Void NOTE 3: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. |  |

Observation 1: For PC3, the gain drops between n257/258/261 and n262 is 6.4 dB and 8.6dB, for Min peak EIRP and spherical coverage, respectively.

To be consistent with the single percentile point assigned per power class, the spherical coverage for n262 at PC1/2/4 should be defined with the same percentile point of each Power class.

Proposal 1: For PC1, PC2 and PC4, n262 adopt the same percentile value, to ensure the consistent with the single percentile point assigned per power class.

If we examine the defined EIRP and spherical requirements for PC1, PC2 and PC4, we can observe that only band n257, n258 and n261 have been fully with EIRP and spherical requirements at each power class. Thus, the requirements at those bands for each Power class can be selected as the reference, while we can reuse this same drop for band n262 from PC3.

Proposal 2: Select the requirements at band n257/258/261 for each power class as the reference, adopt the same gain drop of n262 at PC3, to derive the EIRP and spherical requirements for PC1, PC2 and PC4.

We propose to assume the same degradation level for other PCs, thus the proposed values are listed in the table below:

Table 3: proposals for EIRP and spherical coverage for PC1, PC2 and PC4

| Power Class | EIRP (dBm) | n257 | n258 | n259 | n260 | n261 | n262 | Proposals for n262 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC1 | min peak | 40 | 40 | - | 38 | 40 | TBD | 33.6 |
| PC1 | spherical | 32 | 32 | - | 30 | 32 | TBD | 23.4 |
| PC2 | min peak | 29 | 29 | - | - | 29 | TBD | 22.6 |
| PC2 | spherical | 18 | 18 | - | - | 18 | TBD | 9.4 |
| PC3 | min peak | 22.4 | 22.4 | 18.7 | 20.6 | 22.4 | 16.0 | / |
| PC3 | spherical | 11.5 | 11.5 | 5.8 | 8 | 11.5 | 2.9 | / |
| PC4 | min peak | 34 | 34 | - | 31 | 34 | TBD | 27.6 |
| PC4 | spherical | 25 | 25 | - | 19 | 25 | TBD | 16.4 |

Proposal 3: For PC1, PC2 and PC4, min peak EIRP and EIRP spherical coverage requirements for n262 are derived assuming 6.4 dB and 8.6dB degradation, respectively, from n257/258/n261. The final values are listed in the table above.

- Conclusion

This paper shares our proposals on finalizing the requirement of EIRP and Spherical coverage for PC1, PC2 and PC4.

Observation 1: For PC3, the gain drops between n257/258/261 and n262 is 6.4 dB and 8.6dB, for Min peak EIRP and spherical coverage, respectively.

Proposal 1: For PC1, PC2 and PC4, n262 adopt the same percentile value, to ensure the consistent with the single percentile point assigned per power class.

Proposal 2: Select the requirements at band n257/258/261 for each power class as the reference, adopt the same gain drop of n262 at PC3, to derive the EIRP and spherical requirements for PC1, PC2 and PC4.

Proposal 3: For PC1, PC2 and PC4, min peak EIRP and EIRP spherical coverage requirements for n262 are derived assuming 6.4 dB and 8.6dB degradation, respectively, from n257/258/n261. The final values are listed in the table above.

4	References

- R4-2103392,	“Introduction of n262 UE RF requirements”, Nokia, RAN4#98-e, Jan 2021

- R4-2103312, “Email discussion summary for [98e][124] NR_47GHz_Band”, Moderator (Nokia), RAN4#98-e, Jan 2021

- R4-2100748, “EIRP requirements for n262 and TP to TR 38.847”, Nokia, RAN4#98-e, Jan 2021

- R4-2102906, “EIRP requirements of band n262”, Intel, RAN4#98-e, Jan 2021

- 3GPP TS 38.101-2, v17.0.0