3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2104546

Electronic meeting, 12th – 20th April, 2021

Source:	Ericsson

Title:	Simulation results for NR-U PDSCH demodulation requirement

Agenda item:	5.1.4.2

Document for:	information

# Introduction

In RAN4#98-e meeting, companies got some agreements on PDSCH simulation assumptions [1] and further discussion is delivered in our contribution [2]. In this contribution our simulation results for PDSCH are showed based on WF and a part of our analysis.

# Simulation assumptions and results for PDSCH

Also adding simulation results for 10 DL burst slots and 14 PDSCH symbols in the last slot with LBT failure=0 as comparison.

Table 2-1	PDSCH demodulation parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Active DL BWP index |  |  | 1 |
| Slot Pattern |  |  | According to DL Transmission Model |
| LBT failure probability |  |  | 0.5 or 0.25 |
| Common serving cell parameters | Physical Cell ID |  | 0 |
|  | SSB position in burst |  | The first SSB |
|  | SSB Q factor |  | 8 |
| PDCCH configuration | TCI state |  | TCI state #0 |
| CSI-RS for tracking |  |  | Not configured |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | According to DL Transmission Model |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| RB Allocation |  |  | Full BW |
| MIMO Layers |  |  | 2 |
| Number of HARQ Processes |  |  | 8 |
| TX EVM |  |  | 6% |

Table 2-2	DL transmission model

| DL Transmission Model | Maximum DL COT Duration | ms | 4 |
| --- | --- | --- | --- |
|  | DL Transmission Model Period/ Fixed Frame Period (Note 1) | ms | 5 |
|  | Probability of LBT Failure pLBT |  | 0.5 or 0.25 |
|  | UL COT start time within each FFP | ms | 4 for maximum DL burst length <=7 slots |
|  | UL COT duration | ms | 0.9 for maximum DL burst length <=7 slots |
|  | Idle Time after UL COT | ms | 0.1 |
|  | Number of slots between PDSCH and corresponding HARQ-ACK information |  | 8 if mod(i,10)=0 7 if mod(i,10)=1 6 if mod(i,10)=2 5 if mod(i,10)=3 4 if mod(i,10)=4 3 if mod(i,10)=5 2 if mod(i,10)=6 |
|  | Duration of the Downlink burst (S1) | Slots | {2, 3, 5, 6} |
|  | PDSCH Allocation in the last Slot of the Downlink burst (S2) | Symbols | {6, 9, 12, 14} (no TRS is configured) |
| Notes: - The Fixed Frame Period denomination applies only for ChannelAccessType-r16 = ‘semistatic’. For ChannelAccessType-r16 = ‘dynamic’ this parameter is identified only as DL Transmission Model Period. |  |  |  |

- 20MHz

- 40MHz

- 60MHz

- 80MHz

Figure 2-1 Simulation results for PDSCH

The configuration comparison between TRS configured and no TRS configured showed as below, and the performance have no difference. The burst model configuration is :

- DL burst slot length: 10 slots

- PDSCH symbols in the last burst slot: 14

- LBT failure probability: 0

Figure 2-2 TRS configured vs. no TRS configured

# Conclusion

In this contribution, we deliver our simulation results based on the WF in the last meeting and our discussion paper [2].

# References

[1]	R4-2103987, “Way forward on NR-U UE demodulation requirements”, RAN4#98-e, Qualcomm

[2]	R4-2104545, “Discussion on NR-U PDSCH demodulation requirements”, RAN4#98-bis-e, Ericsson

[3]