3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2104550

Electronic meeting, 12th – 20th April, 2021

Source:	Ericsson

Title:	Discussion on remain issue of NR-U PUCCH demodulation requirements

Agenda item:	5.1.4.4.3

Document for:	Discussion

# Introduction

In RAN4#98-e meeting, companies got agreements on almost all assumptions but there only one issue about test metric of format 3.

- Test metric for enhanced PUCCH format 3

  - Option 1: SNR@ UCI block BLER <= 10-2

  - Option 2: SNR@ACK missed <= 10-2, SNR@Prob(NACK→ACK)≤10^(−3) and SNR@Prob(DTX->ACK)≤ 10-2

In this contribution, this issue will be further discussed.

# Discussion

In Rel-15 specification, there is no clear definition for information bit pattern but only “random codeword selection” is mentioned in some cases. There is no “random bits” configuration on signal generator during the real test. To avoid confusing, it would be better to use fixed bits pattern for information bits for at least interlacing PUCCH requirements.

Furthermore, the UCI information content should also be clarified in the specification according to the bit length and real situation.

Proposal: Use fixed bit pattern for information bits for interlacing PUCCH requirements. Consider following bit pattern and content.

|  | Bit length | Bit pattern | Information content |
| --- | --- | --- | --- |
| ePF0 | 1 | [0] | HARQ-ACK only |
| ePF1 | 2 | [0 1] | HARQ-ACK only |
| ePF2 | 22 | [0 1 0 1… 0 1] | HARQ-ACK and CSI part 1 |
| ePF3 | 4 | [0 1 0 1] | HARQ-ACK only |

In the previous meeting, companies agreed that only use 4bits information bits and 4 OFDM symbols for simulation. Based on this agreement, the RM channel coding will be used for this case. In Rel-15, only CSI is included in the format 3 requirement, it would good to test HARQ-ACK performance for interlacing structure considering the coverage. Thus, the ACK miss and/or NACK ACK metric would be preferred.

Based on our simulation results [2], ACK miss have around 1.2dB higher SNR than NACK ACK. It is a big gap and is reasonable to have only ACK miss as metric as SNR@ACK missed <= 10-2.

Proposal: Define enhanced PUCCH format 3 requirement test metric as SNR@ACK missed <= 10-2 with SNR@Prob(DTX->ACK)≤ 10-2.

# Conclusion

Proposal: Use fixed bit pattern for information bits for interlacing PUCCH requirements. Consider following bit pattern and content.

|  | Bit length | Bit pattern | Information content |
| --- | --- | --- | --- |
| ePF0 | 1 | [0] | HARQ-ACK only |
| ePF1 | 2 | [0 1] | HARQ-ACK only |
| ePF2 | 22 | [0 1 0 1… 0 1] | HARQ-ACK and CSI part 1 |
| ePF3 | 4 | [0 1 0 1] | HARQ-ACK only |

Proposal: Define enhanced PUCCH format 3 requirement test metric as SNR@ACK missed <= 10-2 with SNR@Prob(DTX->ACK)≤ 10-2

# References

[1]	R4-2103989, “Way Forward on NR-U PUCCH demodulation requirements”, Ericsson, RAN4#98-e

[2]	R4-2104551, “Simulation results for NR-U PUCCH demodulation”, Ericsson, RAN4#98-bis-e