3GPP TSG-RAN WG4 Meeting #98-bis-e                	 R4-2104577

Electronic Meeting, Apr. 12-20, 2021

Agenda Item:	5.7.2.2.1

Source:	MediaTek Inc.

Title:	CSI-RSRP measurement accuracy requirements

Document for:		Discussion

# Introduction

In this paper, we provide the simulation results of CSI-RSRP measurement accuracy and discuss requirement.

# Discussion

The simulation results are given in Table 1, according to the follow simulation assumptions:

- SCS: 30kHz

- Es/Iot: -6, 0, 6, 10, 12, 15, 18, 20, 25dB

- Configuration: Density=3, RB=48

- Channel: STATIC

- # of samples for L1 filtering: 5

The RSRP span in the table is calculated at the 90%-tile CDF of the absolute value of the difference between ideal CSI-RSRP and estimated CSI-RSRP. We also compared the measurement accuracy under different timing offset (TΔ) between UE’s FFT window and the CSI-RS to be measured, where

TΔ = target CSI-RS timing – FFT window timing.

Example for TΔ =±CP is illustrated in Figure 1. Positive TΔ means late target CSI-RS timing, while negative TΔ means early target CSI-RS timing. Note we adopted a different way for FFT window boundary selection, compared to our previous submission [1] in last meeting. Here, the FFT window is selected to be start from the middle of the CP duration. In our view, this method is more robust to UE timing estimation error at serving cell and it can bring a symmetric results w.r.t. positive and negative TΔ.

Figure 1. TΔ =±CP with FFT window started from the middle of CP

Table 1. Absolute measurement accuracy of CSI-RSRP

| RSRP span of 90%-tile CDF of ¦ideal RSRP-estimated RSRP¦ |  | Timing offset TΔ (target CSI-RS – FFT window) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | -CP | -0.9*CP | -CP/2 | 0 | +CP/2 | +0.9*CP | +CP |
| Es/Iot | -6dB | 1.28 | 1.27 | 1.26 | 1.14 | 1.25 | 1.23 | 1.35 |
|  | 0dB | 0.81 | 0.38 | 0.30 | 0.28 | 0.29 | 0.39 | 0.83 |
|  | 6dB | 0.76 | 0.29 | 0.15 | 0.13 | 0.14 | 0.31 | 0.74 |
|  | 10dB | 0.72 | 0.26 | 0.09 | 0.08 | 0.09 | 0.27 | 0.70 |
|  | 12dB | 0.71 | 0.26 | 0.07 | 0.07 | 0.07 | 0.27 | 0.70 |
|  | 15dB | 0.70 | 0.27 | 0.05 | 0.05 | 0.05 | 0.26 | 0.70 |
|  | 18dB | 0.71 | 0.27 | 0.04 | 0.03 | 0.04 | 0.27 | 0.70 |
|  | 20dB | 0.70 | 0.28 | 0.03 | 0.03 | 0.03 | 0.26 | 0.69 |
|  | 25dB | 0.69 | 0.27 | 0.02 | 0.02 | 0.02 | 0.27 | 0.69 |
| Note |  | ISI |  | ISI free |  |  | ISI |  |

From Table 1, we have the following observations

- The performance for |TΔ |≤ CP/2 are almost identical because they are all ISI free. One thing to be noted here is that all simulations were conducted under static channel. For other channel types with larger delay spread, we expect to see degraded performance due to ISI even for |TΔ |≤ CP/2.

- For other TΔ values, we can see the accuracy gets degraded as the absolute value of the timing offset increases. Nevertheless, the performance is still acceptable.

Based on the observations, we suggest to define the accuracy requirements with timing offset |TΔ |≤CP.

Proposal 1: Specify CSI-RSRP accuracy requirement with the absolute timing offset between UE’s FFT window and the target CSI-RS no larger than CP.

# Summary

In this paper, we provide the simulation results of CSI-RSRP measurement accuracy and discuss requirements. We have the following proposals.

Proposal 1: Specify CSI-RSRP accuracy requirement with the absolute timing offset between UE’s FFT window and the target CSI-RS no larger than CP.

# Reference

[1] R4-2101152, “CSI-RSRP measurement accuracy requirements”, MediaTek inc.