3GPP TSG-RAN WG4 Meeting # 98-bis-e                	R4-2104622

Electronic Meeting, Apr. 12-Apr. 20, 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 	NR-U PUSCH simulation results

Agenda item: 	5.1.4.4.2

Document for: 	Discussion

Introduction

This paper presents simulation results for interlaced PUSCH including GC-UCI considering the agreed simulation parameters on [1] and further results for analysis in the companion discussion paper [3].

PUSCH Simulation results

Preliminary simulation results are presented for interlaced PUSCH in this section. The simulation parameters are shown in Table 1, and the results are shown in Table 2.

Table 1 Simulation assumptions for interlaced and contiguous PUSCH analysis

| Parameter | Value |
| --- | --- |
| Waveform | CP-OFDM |
| Allocation length | 14 symbols |
| Number interlaces | 1 |
| BW | 20 MHz |
| SCS | 15 and 30 kHz |
| TDD pattern | 7D2S1U S=6D:4G:4U for 30kHz SCS 3D1S1U S=10D:2G:2U for 15kHz SCS |
| DM-RS configuration | 1+1 |
| Mapping type | Type A and Type B |
| MCS (Table 1, namely “qam64”) | MCS 20 |
| Propagation condition | TDLA30-10 |
| Antenna config | 1x2 |
| Maximum number HARQ retransmissions | 4 |
| RV sequence | {0,2,3,1} |
| Test metric | SNR@70% TPUT |

Table 2 Simulation results for interlaced PUSCH for MCS 20

| Bandwidth | DMRS Type | SCS | RV seq. | SNR@70% TPUT |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ideal | Impairment |
| 20 | TypeA | 15 | {0.2.3.1} | 9.9 | 12.4 |
|  |  | 30 | {0.2.3.1} | 9.8 | 12.3 |
|  | TypeB | 15 | {0.2.3.1} | 9.9 | 12.4 |
|  |  | 30 | {0.2.3.1} | 9.8 | 12.3 |

GC-UCI simulation results

Preliminary simulation results are presented for GC-UCI in this section. The simulation parameters are shown in Table 3 and the simulation results are presented in Tables 4 and 5 for MCS20 and MCS16, respectively.

Table 3 CG-UCI simulation parameters

| Parameter |  | Value |
| --- | --- | --- |
| Channel model |  | TDLA30-10 |
| Bandwidth |  | 20 MHz |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern |  | 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| MCS |  | MCS16, MCS 20 |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID=0 |
| Time domain | PUSCH mapping type | A,B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | 1 interlace |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
|  | Number GC-UCI information bit payload | 18 |
|  | Scaling | 1 |
| UCI | betaOffsetGC-UCI | TBD |
|  | UCI partition for frequency hopping | Disabled |

Table 4 CG-UCI simulation results for MCS 20

| MCS | SCS | Mapping type | IoffsetCG-UCI | CG-UCI SNR@BLER=1% |  | PUSCH SNR@BLER=10% |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ideal | impairment | ideal | impairment |
| MCS 20 | 15 kHz | A | 0 | 13.7 | 16.2 | 13.7 | 16.2 |
|  |  |  | 1 | 9.3 | 11.8 | 13.9 | 16.4 |
|  |  |  | 2 | 8.3 | 10.8 | 13.9 | 16.4 |
|  |  |  | 3 | 6.9 | 9.4 | 14.0 | 16.5 |
|  |  |  | 8 | 3.2 | 5.7 | 14.2 | 16.7 |
|  |  |  | 11 | 0.2 | 2.7 | 14.8 | 17.3 |
|  |  | B | 0 | 13.6 | 16.1 | 13.7 | 16.2 |
|  |  |  | 1 | 8.9 | 11.4 | 13.9 | 16.4 |
|  |  |  | 2 | 8.2 | 10.7 | 13.9 | 16.4 |
|  |  |  | 3 | 6.6 | 9.1 | 13.9 | 16.4 |
|  |  |  | 8 | 3.3 | 5.8 | 14.2 | 16.7 |
|  |  |  | 11 | 0.4 | 2.9 | 14.8 | 17.3 |
|  | 30 kHz | A | 0 | 14.6 | 17.1 | 14.3 | 16.8 |
|  |  |  | 1 | 10.3 | 12.8 | 14.4 | 16.9 |
|  |  |  | 2 | 9.3 | 11.8 | 14.4 | 16.9 |
|  |  |  | 3 | 8.0 | 10.5 | 14.5 | 17 |
|  |  |  | 8 | 3.8 | 6.3 | 14.9 | 17.4 |
|  |  |  | 11 | 1.0 | 3.5 | 15.3 | 17.8 |
|  |  | B | 0 | 14.6 | 17.1 | 14.3 | 16.8 |
|  |  |  | 1 | 10.8 | 13.3 | 14.4 | 16.9 |
|  |  |  | 2 | 9.6 | 12.1 | 14.4 | 16.9 |
|  |  |  | 3 | 8.1 | 10.6 | 14.5 | 17 |
|  |  |  | 8 | 3.9 | 6.4 | 14.8 | 17.3 |
|  |  |  | 11 | 1.3 | 3.8 | 15.3 | 17.8 |

Table 5 CG-UCI simulation results for MCS 16

| MCS | SCS | Mapping type | IoffsetCG-UCI | CG-UCI SNR@BLER=1% |  | PUSCH SNR@BLER=10% |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ideal | impairment | ideal | impairment |
| MCS 16 | 15 kHz | A | 0 | 11.5 | 14 | 10.7 | 13.2 |
|  |  |  | 3 | 4.6 | 7.1 | 10.8 | 13.3 |
|  |  |  | 6 | 2.4 | 4.9 | 11.1 | 13.6 |
|  |  |  | 9 | -0.1 | 2.4 | 11.6 | 14.1 |
|  |  |  | 11 | -1.4 | 1.1 | 12.1 | 14.6 |
|  |  | B | 0 | 11.4 | 13.9 | 10.7 | 13.2 |
|  |  |  | 3 | 4.7 | 7.2 | 10.8 | 13.3 |
|  |  |  | 6 | 2.3 | 4.8 | 11.1 | 13.6 |
|  |  |  | 9 | -0.3 | 2.2 | 11.5 | 14 |
|  |  |  | 11 | -1.5 | 1 | 12.1 | 14.6 |
|  | 30 kHz | A | 0 | 12.5 | 15 | 11.2 | 13.7 |
|  |  |  | 3 | 5.7 | 8.2 | 11.4 | 13.9 |
|  |  |  | 6 | 3.3 | 5.8 | 11.6 | 14.1 |
|  |  |  | 9 | 1.2 | 3.7 | 12.1 | 14.6 |
|  |  |  | 11 | -0.3 | 2.2 | 12.6 | 15.1 |
|  |  | B | 0 | 12.3 | 14.8 | 11.2 | 13.7 |
|  |  |  | 3 | 5.6 | 8.1 | 11.3 | 13.8 |
|  |  |  | 6 | 3.2 | 5.7 | 11.5 | 14 |
|  |  |  | 9 | 1.2 | 3.7 | 12.1 | 14.6 |
|  |  |  | 11 | -0.5 | 2 | 12.6 | 15.1 |

Conclusion

In this contribution we provided interlaced PUSCH simulation results for NR-U.

References

- R4-2017688, Way forward on NR-U BS demodulation requirements for general part and PUSCH, 3GPP TSG-RAN WG4 Meeting #98-e

- R4-2104621, PUSCH Demodulation performance requirements for operation in unlicensed bands, 3GPP TSG-RAN WG4 Meeting #98-bis-e, Nokia