3GPP TSG-RAN WG4 Meeting # 98-bis-e                	R4-2104626

Electronic Meeting, Apr. 12-20, 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 	NR-U PRACH simulation results

Agenda item: 	5.1.4.4.4

Document for: 	Discussion

Introduction

This paper presents simulation results considering some of the previously agreed parameters in the outcome of RAN4 #98-e meeting on that topic [1].

Simulation results

Simulation results are presented for different PRACH formats in this section. The simulation parameters are shown in Table 1. Besides, time error tolerances and related PRACH performance evaluations are shown in Tables 2 and 3, respectively.

Table 1 Simulations parameters for PRACH sequences in NR-U

| Parameter | Value |
| --- | --- |
| PRACH formats | A2, B4 and C2 |
| SCS | 15 kHz (LRA = 1151) and 30 kHz (LRA = 571) |
| Antenna configuration | 1x2 |
| Propagation conditions | AWGN TDLA30-10 |
| Frequency offset | 400Hz |
| Ncs | 164 for LRA=1151 and 190 for LRA=571 |
| Test metric | The false alarm probability < 0.1% The probability of detection > 99% |
| Carrier frequency | 5 GHz |
| Channel bandwidth | 20 MHz |

Table 2 Time error tolerance based on Table 8.4.2.1-1 of 38.104 [2].

| PRACH SCS | AWGN | TDLC 300 | TDLA 30Note1 |
| --- | --- | --- | --- |
| 15 kHz | 0.52 us | 2.03 us | 0.67 us |
| 30 kHz | 0.26 us | 1.77 us | 0.41 us |
| Note 1: The time error tolerance for TDLA 30 is not available in 38.104 [2] and was calculated as TA command resolution + delay for the second last tap of the channel model in Table G.2.1.1-2 of [2] as agreed in the last RAN4 meeting [1] |  |  |  |

Table 3 Simulation results for PRACH in unlicensed bands

| PRACH format | SCS (LRA) | Propagation condition | Frequency offset (Hz) | Ideal (dB) | Impairment (dB) |
| --- | --- | --- | --- | --- | --- |
| A2 | 15 kHz | AWGN | 0 | -23.6 | -21.1 |
|  | (1151) | TDLA30-10 | 400 | -17.5 | -15.0 |
|  | 30 kHz | AWGN | 0 | -20.6 | -18.1 |
|  | (571) | TDLA30-10 | 400 | -13.7 | -11.2 |
| B4 | 15 kHz | AWGN | 0 | -27.3 | -24.8 |
|  | (1151) | TDLA30-10 | 400 | -21.1 | -18.6 |
|  | 30 kHz | AWGN | 0 | -24.5 | -22.0 |
|  | (571) | TDLA30-10 | 400 | -17.4 | -14.9 |
| C2 | 15 kHz | AWGN | 0 | -23.3 | -20.8 |
|  | (1151) | TDLA30-10 | 400 | -17.4 | -14.9 |
|  | 30 kHz | AWGN | 0 | -20.6 | -18.1 |
|  | (571) | TDLA30-10 | 400 | -13.6 | -11.1 |

Conclusion

In this contribution we provided PRACH simulation results for NR-U.

References

- R4-2103808,	Way forward on PRACH demodulation requirements, Nokia, 3GPP TSG-RAN WG4 Meeting # 98-e

- TS 38.104 V16.6.0, NR; Base Station (BS) radio transmission and reception