3GPP TSG-RAN WG4 Meeting #98-bis-e                                                         R4-2104641

Online, 12th April - 20th April, 2021

Source:	Vodafone

Title:	TP for TR 38.717-02-01: CA_n3-n20

Agenda item:			7.2.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-02-01 to include CA_n3-n20 according to the request in [1].

# 2. Reference

- RP-202199, Revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2).

# 3. Text Proposal

<Start of Text Proposal>

## 6.x	CA_n3-n20

### 6.x.1		Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1		Operating bands for CA

Table 6.x.1.1-1: CA band combination of band n3+n20

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |

#### 6.x.1.2		Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n3+n20

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n20A | - | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

#### 6.x.1.3		Co-existence studies

Table 6.X.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n3-n20.

Table 6.X.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 |
| n20 | 832 | 862 | 791 | 821 | 1664 | 1724 | 2496 | 2586 | 3328 | 3448 |

Based on above table, no harmonic for CA_n3-n20

Table 6.X.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 |
| n20 | 832 | 862 | 791 | 821 | 1582 | 1456 | 2151 | 2184 | 2868 | 2912 |

Based on above table, no harmonic for CA_n3-n20.

#### 6.X.1.4		∆TIB and ∆RIB values

For CA_n3-n20 , the ΔTIB,c and ΔRIB,c values are given in the tables below. The values same as CA_3-20 specified in TS 36.101.

Table 6.X.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n20 | n3 | 0.3 |
|  | n20 | 0.3 |

Table 6.X.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n20 | n3 | 0 |
|  | n20 | 0 |

#### 6.X.1.5 REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

#### 6.X.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

<End of Text Proposal>