3GPP TSG-RAN WG4 Meeting # 98-bis-e                                                        R4-2104649

Electronic Meeting, 12th April – 20th April, 2021

Source:	Vodafone

Title:	TP for TR 36.717-02-02 to add UL configuration CA_8A-28A for CA_8A-28A

Agenda item:	10.4.2

Document for:	Approval

# Background

This contribution provides text proposal on the LTE UL configuration CA_8A-28A for CA_8A-28A as defined in the revised WID [1].

# Reference

[1] RP-201732 Revised WID proposal Rel-17 LTE inter-band CA for 2 bands DL with 2 band UL.

# Text Proposal

##### ---Start of changes---

## 6.X	LTE-A inter-band CA: Band 8 and Band 28 DL with 2 bands UL

### 6.X.1	List of specific combination issues

#### 6.X.1.1	Channel bandwidths per operating band for CA

Table 6.X.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_8A-28A | CA_8A-28A | 8 |  | Yes | Yes | Yes |  |  | 30 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

#### 6.X.1.2	Co-existence studies for LTE-A UL CA_8A-28A and DL CA_8A-28A

For 2UL / 2DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.X.1.2-1.

Table 6.X.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fn_low | fn_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fn_low | 2* fn_high |
| 2nd harmonics frequency limits (MHz) | 1760 – 1830 |  | 1406 – 1496 |  |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fn_low | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2640 – 2745 |  | 2109 – 2244 |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |
| IMD frequency limits (MHz) | 132 – 212 |  | 1583 – 1663 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |
| IMD frequency limits (MHz) | 1012 – 1127 |  | 491 – 616 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |
| IMD frequency limits (MHz) | 2463 – 2578 |  | 2286 – 2411 |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) | (fx_high + max BW fn) | (fn_low – max BW fx) | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 860 – 935 |  | 693 – 758 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1892 – 2042 |  | 1194 – 1364 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 264 – 424 |  | 3166 – 3326 |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3343 – 3493 |  | 2989 – 3159 |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1897 – 2112 |  | 2772 – 2957 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 279 – 484 |  | 1144 – 1339 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3692 – 3907 |  | 4223 – 4408 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3869 – 4074 |  | 4046 – 4241 |  |

Based on Table 6.X.1.2-1, there is an IMD3 hit on the Rx of band 8.

#### 6.X.1.3	MSD

There are no additional MSD requirements for this combination.

#### 6.X.1.4	∆TIB and ∆RIB values

The following ∆TIB and ∆RIB values are drawn from DC_8A_n28A in TS 38101-3:

Table 6.X.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_8-281 | 8 | 0.6 |
|  | 28 | 0.5 |
| NOTE 1: The frequency range in band 28 is restricted for this CA band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |

Note 14 that exists for CA_8-28 Tib in 36101 is not applicable (states “only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8”).

Table 6.X.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_8-28 | 8 | 0.2 |
|  | 28 | 0.1 |

Note 13 that exists for CA_8-28 Rib in 36101 is not applicable (states “only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8”).

##### ---End of changes---